| # Liaison Statement |  |  |
| --- | --- | --- |
| Title: |  | Transmitter spurious emissions: co-existence requirements limits for high frequency bands |
| Date: |  | 17.09.2024 |
| From (source): |  | ETSI TC MSG/TFES, Dominique Everaere |
| Contact(s): |  | (dominique.everaere@erisson.com); MSGTFESsupport@etsi.org; |
| To: |  | 3GPP RAN WG4, 3GPPliaison@etsi.org |
| Copy to: |  | 3GPP TSG RAN |
| Response to: (if applicable) |  | - |
| Attachments: (if applicable) | - |  |

1. Overall description:

During discussion on Rel-17 versions of Harmonized Standards for BS specifications (i.e. E-UTRA BS, MSR BS, AAS BS, NR BS) one of the topics discussed was co-existence requirement for TX spurious emissions, for 3GPP operating bands introduced in Rel-17/18 versions of 3GPP specifications.

During the discussion it was brought to the attention of ETSI MSG TFES that some limits for the TX spurious emission co-existence requirements (e.g. as captured in Table 6.6.5.5.1.3-1 on Additional spurious emissions requirements in TS 38.141-1 specification) may require further verification. More specifically, re-using legacy TX spurious emissions co-existence limits for the high frequency bands was discussed.

Referring to the co-existence limits captured in the table below (extracted from Table 6.6.5.5.1.3-1 in TS 38.141-1), one can notice that the same basic limit of -52 dBm is used for 450 MHz frequency bands, as well as for the high frequency bands above 3.5 GHz and up to 7 GHz. This has triggered discussion the on relevancy of those requirements for such a wide frequencies range, considering as well the various BS classes and deployment scenarios to be considered for each band.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limit | Measurement bandwidth |
| --- | --- | --- | --- |
| E-UTRA Band 31 or NR Band n31 | 462.5 -467.5 MHz | -52 dBm | 1 MHz |
|  | 452.5 -457.5 MHz | -49 dBm | 1 MHz |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -52 dBm | 1 MHz |
| E-UTRA Band 47 | 5855 – 5925 MHz | -52 dBm | 1 MHz |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -52 dBm | 1 MHz |
| NR Band n102 | 5925 – 6425 MHz | -52 dBm | 1 MHz |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm | 1 MHz |

2. Actions:

ETSI MSG TFES respectfully asks 3GPP RAN WG4 to consider the above information in their discussions and to re-evaluate if the current limits are appropriate for all bands, considering the different conditions relevant for each band. ETSI MSG TFES would also appreciate if RAN4 could share the assumptions taken for the derivation of TX spurious emissions co-existence limits in BS specifications, for both conducted and radiated requirements.

3. Date of next meetings of the originator:

MSG TFES#82		Sophia Antipolis, France		15-16.01.2025

MSG TFES#83		Paris, France			17-18.03.2025