pCR for text inclusion in TR 45.820 :

C-UNB Battery Life Evaluation

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the battery life evaluation of the C-UNB solution.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

### Uplink physical layer design

### Link layer aspects

### Radio resource management

### Concept evaluation

#### Latency evaluation

#### Battery life evaluation

##### Evaluation methodology

###### Common assumptions

The battery life evaluation for C-UNB MS is based on methodology detailed in annex hereunder. Because of the specificity of the C-UNB radio access technology, two assumptions are used in the evaluation:

MAR are supposed to be transmitted with two repetitions at physical layer,

no acknowledgement is transmitted in DL.

These assumptions are valid as long as the collision rate is less than 10%, which is the case in all performance evaluations because of multiple receptions given by sector overlaps.

###### MAR segmentation

In the present document, the battery life evaluation assumes two lengths for the MAR: 50 and 200 bytes at SNDCP level. In C-UNB, such data packets are segmented and encapsulated as stated in table 7.4.6.2. MAC-PDU size and total number of transmission give the inputs for power consumption.

Table 7.4.6.2: MAR mapping in C-UNB PHY-PDU  (all sizes in bytes)

| MAR size | Number of segments | Segment size | LLC-PDU size | MAC-PDU size | Total # of PHY-PDU | PHY-PDU duration |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | 2 | 25 | 26 | 42,5 | 4 | 1 360ms |
| 200 | 7 | 29 | 30 | 46,5 | 14 | 1 488ms |

###### Energy consumption profile

Table 7.4.6.3 gives electrical parameters for energy consumption analysis. Figures for Tx and Rx power are those from off-the-shelves sub-gigaHertz chipsets or what is expected in the next generation of chipsets where architecture and power consumption will be optimized for ultra narrow band transmission. For example, a power consumption of 450mW at +24dBm is expected to be achievable the following way: 100mW for transceiver and 350mW for Power Amplifier (PA) with 71% efficiency, which is expected to be available with UNB optimization.

Table 7.4.6.3: Key electrical parameters for energy consumption analysis

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power in Processing (mW) | (5) Battery power in Power Save (mW) |
| --- | --- | --- | --- | --- |
| 5 | 450 @ +24dBm 100 @ +14dBm 30 @ +4dBm | 40 | 10 | 0,015 |

Table 7.4.6.4: Key timing parameters for energy consumption analysis (in ms)

| (6) trigger event | (7) Processing time from PS to Tx mode | (8) Beacon reception window | (9) Processing time between two Tx (ms) | (10) Transmit time for one PHY-PDU |
| --- | --- | --- | --- | --- |
| 0 | 50 | 70 to 200 | 25 | see table 1 |

In table 7.4.6.4, the duration of the reception window for beacon checking takes into account the real time clock drift between two consecutive transmissions of periodic MAR.

Figure 7.4.6.2: Energy consumption model for C-UNB devices

##### Results

Table 7.4.6.5 gives the battery life of C-UNB devices with a combination of two packet sizes, two reporting intervals and three coverage levels.

Table 7.4.6.5: Battery life (in years)

End of Changes