pCR for text inclusion in TR 45.820 :

C-UNB general description

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.3.1.

The proposed text is an introduction, which is intended to provide the reader with an overview of the key design principles for the C-UNB candidate solution. Sections, dealing with technical description and concept evaluation, are detailed in other pCRs.

It is a revision of pCR GP-150998.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

Cellular IoT devices are a new category of MSs, which require extended coverage, improved battery lifetime and low complexity compared to what is available in 2G networks. Cooperative Ultra Narrow Band (C-UNB) is a clean slate approach that answers these requests.

In C-UNB, the extended coverage is achieved by means of ultra narrow band communications, which are possible because of the low volume of data transmitted by mobile stations.

#### Random uplink transmission

In order to keep the complexity of C-UNB modems as low as possible, the C-UNB radio access technology (RAT) is based on random transmissions by mobile devices. Uplink access is a grant-free protocol, where mobile stations don't need to ask for a resource allocation before transmitting their radio packets. This kind of medium access is equivalent to ALOHA, which is known for its simplicity. Its drawback is packet collisions that hamper the overall efficiency when the network load becomes high. To overcome this issue, the C-UNB RAT implements two mitigation techniques that are frequency diversity and space diversity.

#### Ad'hoc micro-channels

Ultra narrow band transmission occupies a very small portion of spectrum in a 200kHz block, leaving space for other transmissions. This feature is the well-known channelization implemented in frequency division multiple access (FDMA). In the case of C-UNB, uplink channel bandwidth is a few hundreds of Hertz; this value is equivalent and sometimes lower than the frequency drift of crystal oscillators. Hence, the center frequency of an uplink transmission is difficult to set precisely without tight synchronization to the network.

To overcome this issue, there is no channelization of the 200kHz block: the center frequency of each transmission is selected randomly by a transmitting MS in a 200kHz block. In fact, the useful bandwidth is 180kHz because of two 10kHz guard bands. The random selection of carrier frequency is a kind of "frequency diversity" that helps reducing the radio packet collisions. This uplink implementation is called "ad'hoc micro-channel" in C-UNB solution.

When needed, multiple transmissions improve the quality of service. Figure 7.4.1-1 illustrates frequency diversity of uplink transmissions, in case of 3 repetitions per radio packet.

Figure 7.4.1-1: Frequency diversity in C-UNB

#### Cooperative reception

The second feature, used to cope with collision on the air interface, is the "space diversity" given by the multiple receptions of radio packets in uplink. All base stations listen for the same 200kHz block continuously. As there are large overlaps in base station coverage, several base stations may receive the same radio packet (i.e. the same PHY-PDU). If an UL radio packet experiences collision or interference during reception in a given base station, the probability to have bad reception in all surrounding base stations is limited. De-duplication of received packets is performed in a dedicated C-UNB server, located in the core network. Figure 7.4.1-2 illustrates space diversity and packet reconstruction in core network.

Figure 7.4.1-2: Radio packet collision managed by space diversity

Ad'hoc micro-channels and multiple receptions by overlapping base stations help achieving the requirement of low complexity as stated in the present document.

#### System architecture

The C-UNB system architecture differs from the legacy Gb architecture but can connect it (see figure 7.4.1-3). It consists in base stations and a C-UNB server.

In the C-UNB solution, the base station run a specific software update, which has three main features:

it implements the C-UNB air interface,

for any MS seen to be attached to this base station, it makes the gateway between the Gb- related messages of this MS and the C-UNB server,

it makes the gateway between the C-UNB air interface and the C-UNB server.

The last two features use a tunnel set up by the C-UNB software between the corresponding base station and the C-UNB server; this tunnel is, in fact, a permanent PDP-context.

The C-UNB server is unique in a given C-UNB network. It has three main features:

in uplink: C-UNB radio packet deduplication, link layer management and security check,

in downlink: message storage until a MS wakes up, and all features related to DL transmission (segmentation, authentication, base station selection)

termination of the Gb-related messages of MSs seen by the network.

Figure 7.4.1-3: C-UNB solution in Gb architecture

Figure 7.4.1-4 illustrates how the C-UNB software in base stations and the C-UNB server cooperate together in order to make to liaison between Gb-related messages and the C-UNB messages.

Figure 7.4.1-4: C-UNB and Gb-related message streams

#### Beacon channel

##### Beacon channel concept

The C-UNB solution assumes that all base stations, of a given MNO in a given area, listen for the same 200kHz frequency block and transmit in the corresponding DL frequency block. Apart from user data packets, each sector of a base station transmits a beacon channel that has the following purpose:

to help MSs discover the 200kHz block allocated to C-UNB (by reading the C-UNB network ID)

to broadcast system information elements.

As beacon channels of a given cluster use the same frequency (i.e. the center frequency of the 200KHz block allocated to CIoT), a periodic cycle prevents collision of beacon packets.

The beacon channel can be use also for roaming purpose. Detailed implementation of roaming procedures is FFS.

##### Network discovery

Before deployment, mobile stations are configured with a preferred C-UNB network identifier.

When deployed, a mobile station listens for the full set of 200kHz block until it receives a beacon channel with its C-UNB network ID. Once proper network is discovered, the mobile station uses the system information available in the beacon channel to set its radio parameters.

In C-UNB solution, beacon channel check is done prior to each uplink transmission sequence to prevent mobile stations transmitting at the wrong power level or in a wrong 200kHz block if they have moved.

#### Downlink transmission

In legacy cellular networks, the transmission of network-triggered downlink packets is supported by the paging procedure. This procedure implements various timers and messages in order to keep the downlink latency under tight control.

As such, there is no paging mechanism in the C-UNB solution. In C-UNB, the downlink transmissions occur only after an uplink message, i.e. they are device triggered.

When a mobile station has regular transmission of user plane packets, it can use these packets for retrieving DL messages. If not, it is encouraged by the C-UNB network to transmit periodic poll messages. The maximum delay between two consecutive poll messages is a system information element.

DL messages are stored in the C-UNB server until the corresponding MS wakes up for UL transmission or regular check.

#### Security

The C-UNB solution implements two types of security mechanisms: one SIM card based mechanism and one simplified mechanism over the air interface. As explained in the following subclauses, this split of security mechanism helps reducing the complexity of the C-UNB modem and the number of control messages over the air.

##### Radio access level

C-UNB radio access technology implements a specific security mechanism over the radio interface by means of authentication field added to every radio packet. A hash algorithm computes the authentication field from the following elements: MS identity, MS secret key, a sequence number and the payload content. The authentication field is added to each MAC-PDU sent over UL and DL.

##### Core network level

The security features of the Gb architecture are implemented in the C-UNB solution between the legacy core network and the C-UNB server. They use soft SIM cards that are stored in the C-UNB server. For each MS connected to a given MNO, the C-UNB server instantiates the credentials of the corresponding soft SIM card.

Although this implementation gives full compatibility with the legacy Gb interfaces, the split of security features between network and mobile stations differ from legacy system. It has to be submitted to 3GPP SA3 group for validation.

##### User data encryption

Encryption of user data over the air interface can reuse existing credentials already available in the mobile stations, such as MS identifier, MS secrete key and sequence number. Detailed implementation is FFS.

End of Changes