# Impact on legacy services in a reduced BCCH spectrum allocation
(update of GP-151183)

# Introduction

At GERAN#67 a new work item named Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things (WI code: CIoT_EC_GSM) was approved, see [1]. One of the objectives of the work item is the following:

“Support for extended coverage GSM deployment in a reduced BCCH spectrum allocation, provided it is shown to be feasible, from 4/12 (2.4 MHz) frequency re-use to 3/9 (1.8 MHz) or 1/3 (600 kHz) frequency re-use, where legacy CS users might not be supported, and add respective normative changes, if any.”

The BCCH layer serves many purposes, which may be negatively impacted by a tightened reuse, including the following:

- Frequency and time synchronization (FCCH, SCH)

- Cell identification (reading BSIC on SCH) and received signal level measurements e.g. for cell reselection purposes

- System information acquisition (BCCH)

- Common control signaling (RACH, PCH, AGCH)

- User data traffic

In this contribution, item 5 is investigated for legacy services for BCCH layer spectrum allocations using 1.8 MHz down to 600 kHz. Item 1 is investigated in [2].

# Assumptions

## Traffic generation

MTC traffic is generated according to the MAR periodic reporting and Network Command traffic models [3]. The split between these is 80 % MAR periodic and 20 % Network command.

## RACH interference

Interference from RACH without power control has been modelled.

## BCCH Power Savings

BCCH power savings can be used to reduce interference on the BCCH frequency layer.  With tighter BCCH frequency re-use the importance of this functionality increases. BCCH power savings can be used with various levels of reduction and selections of what timeslots and channels it should be applied to. For simulator implementation a simple implementation for BCCH PS was used with a reduction of 6 dB for Time Slots not used for PDTCH or PACCH. In case PDTCH or PACCH is used on the DL, no power regulation is used. The simulator implementation was improved compared to the version used to develop the results in GP-151183 [4]. Timeslots TS0 (carrying BCCH, FCCH, SCH, CCCH) and TS1 (carrying EC-BCCH, EC-CCCH, EC-SCH) are now excluded from BCCH Power Savings.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [5] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [5]

| Parameter | Value |
| --- | --- |
| General |  |
| Simulation time | 100 s |
| System size | 108 cells (all frequency re-uses) |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | Varied from 9, 8, …, 4, 3 |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Cell radius | 577.33 m |
| Legacy GPRS MTC parameters |  |
| PDTCH timeslots per cell | 6 PDCH(note 1) |
| Legacy GPRS MTC arrival rate per cell and second | 6.81 (100%)(note 2) |
| Fixed UL allocation | On |
| Coding schemes | CS-1 |
| GPRS L2S model | Approximated by EGPRS L2S (MCS-1) without incremental redundancy, see [6] |
| Minimum delay between subsequent transmissions on PDTCH and PACCH | 1 radio block |
| Incremental Redundancy | Off |
| Power control | Off. Power savings 6 dB DL if nothing to transmit on BCCH TS2-TS7. |
| IP header compression | Off |
| Device output power | 33 dBm (100%) |
| BPL model(note 3) | No BPL is applied to the Legacy GPRS MTC users |
| Device timeout | 20 seconds |
| NOTE 1: The system simulator uses a network wide timeslot alignment with a random timeslot offset between cells. NOTE 2: Derived from traffic models in [3]. 6.81 reports/commands per cell and second corresponds to the targeted number of devices per sector in the study. NOTE 3: According to model in [5]. |  |

### Cell selection and coding scheme selection

Cell selection was based on the calculated path gain and a N(0,2) dB measurement error. All devices have 0 velocity in the simulations so there will be no cell re-selection.

In the simulations no link adaptation was used. Instead the coding scheme was always selected to CS-1 and remained the same throughout the duration of the TBF.

### Control signaling

Packet uplink ACK/NACK (PUAN) is sent on PACCH/D to (negatively) acknowledge data sent in the UL and assign fixed allocations to the MS. In the simulations its performance is modeled with EGPRS MCS-1. If a PUAN is unsuccessfully received the negative acknowledged blocks will not be transmitted, i.e. the allocated MS will not transmit anything, but the radio block resources are consumed.

Packet downlink ACK/NACK (PDAN) is sent on PACCH/U to (negatively) acknowledge data sent in the DL. In the simulations its performance is modeled with EGPRS MCS-1.

### Simulated scenarios

Table 2 summarizes the simulated scenarios and clarifies the legends in the figures presented in section 3.2. The frequency planning for the re-uses between 8 and 4 has been done with an Automatic Frequency Planner that, based on path gain relationships between cells, tries to minimize co- and adjacent channel interference on the downlink.

Table 2: Simulated scenarios

| Legend text | BCCH Re-use | Frequency planning | BCCH Power Saving |
| --- | --- | --- | --- |
| Re-use = 9 | 9 | 3/9 cluster re-use pattern | 6 dB |
| Re-use = 8 | 8 | AFP generated plan | 6 dB |
| Re-use = 7 | 7 | AFP generated plan | 6 dB |
| Re-use = 6 | 6 | AFP generated plan | 6 dB |
| Re-use = 5 | 5 | AFP generated plan | 6 dB |
| Re-use = 4 | 4 | AFP generated plan | 6 dB |
| Re-use = 3 | 3 | 1/3 cluster re-use pattern | 6 dB |

## Results

The results presented are:

- Resource (TS) Usage (section 3.2.1)

  - This represents the average amount of PDTCH DL and UL TS resources required per cell in the system, for the different scenarios, see Table 3.

- Latency of MAR periodic reports (section 3.2.2)

  - The latency includes time to transfer the message.

  - The results are presented as CDFs of the delay at the target traffic load (6.81 users per cell and second), see Figure 1.

  - Failed attempts are not included in the statistics (following the agreed methodology).

- Latency of DL application Ack (section 3.2.3)

  - Latency is measured from the time an application layer DL ACK is received at the base station till the time when the device has successfully received the application layer DL ACK

  - The results are presented as CDFs of the delay at the target traffic load (6.81 users per cell and second), see Figure 2.

- Failed attempts (section 3.2.4)

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds.

- Uplink capacity (section 3.2.5)

  - Uplink capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. Results are shown in Table 5.

### TS Usage

The TS Usage is shown in Table 3 for the downlink and uplink respectively. On the downlink, the TS Usage increases from 0.48 to 0.62 when the reuse is changed from 9 to 3. This is an increase of ~30% while the frequency reuse is increased 3 times (67% smaller spectrum).

Table 3. TS Usage for the downlink and uplink

| BCCH Re-use | TS usage DL [#TS] | TS usage UL [#TS] |
| --- | --- | --- |
| 9 | 0.48 | 0.75 |
| 8 | 0.50 | 0.76 |
| 7 | 0.52 | 0.78 |
| 6 | 0.53 | 0.79 |
| 5 | 0.55 | 0.81 |
| 4 | 0.59 | 0.85 |
| 3 | 0.62 | 0.89 |

For the uplink, the TS Utilization increases from ~0.75 to ~0.9 when the reuse is changed from 9 to 3. This is an increase of ~20% with 67% fewer frequencies.

### Latency of MAR periodic reports

The latency of MAR periodic reports is represented by the latency of the data transfer, i.e. the common control signaling delay, see [3], is not included. A few users will experience an increased delay as seen in Figure 1. The delays are increasing with tighter frequency re-use.

Figure 1: Uplink Transmission Delay

### Latency of Downlink Application Ack

A few users will experience an increased delay as seen in Figure 2. Also in this case, the delay is increased with tighter frequency re-use.

Figure 2: Downlink Application Ack Delay

### Failed Attempts

At the traffic load 6.81 users per cell and second, the percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is found to be 0 % in the 9, 8 and 7 re-use scenarios. For the tighter reuses the percentage is still very close to 0 %. All failed attempts are shown in Table 4.

Table 4: Failed attempts

| BCCH Re-use | Failed attempts [%] |
| --- | --- |
| 9 | 0 |
| 8 | 0 |
| 7 | 0 |
| 6 | 0.0015% |
| 5 | 0.0030% |
| 4 | 0.0089% |
| 3 | 0.0163% |

### Capacity

In [3] the capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. This definition is made with a standalone CIoT system in mind. Since the system in this evaluation serves only one traffic type (MTC traffic), the capacity definition is more meaningful in this case than in the previous EC-EGPRS investigations in which mixed services were assumed. Still it should be noted that the measure is not really a capacity measure since it does not reflect the capacity limit of the system but rather at an assumed fixed load.

Capacity is here calculated as

(#sent reports per sector per hour)*(1 - failed attempts)/reuse

The capacity is shown in Table 5 for the simulated scenarios.

Table 5: Capacity

| BCCH Re-use | BCCH Power Saving | Capacity [reports/200kHz/hour] |
| --- | --- | --- |
| 9 | 6 dB | 2725 |
| 8 | 6 dB | 3065 |
| 7 | 6 dB | 3503 |
| 6 | 6 dB | 4087 |
| 5 | 6 dB | 4904 |
| 4 | 6 dB | 6130 |
| 3 | 6 dB | 8173 |

As can be seen from the table, the 3-reuse scenario has three times capacity than the 9-reuse scenario, as expected considering the change in re-use factor, and the fact that almost no reports fails to be delivered.

# Discussion and conclusions

This paper shows that legacy MTC services may be accommodated on as low BCCH spectrum allocations as 600 kHz with less than 30% increase in TS utilization compared to 1.8 MHz. The transmission delays are increased for some devices; the effect is however rather small compared to the 67% reduction of the required frequency spectrum, corresponding to three times the spectral efficiency.

# References

GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

GP-151124, ”GPRS synchronization performance in a tight frequency reuse scenario”, source Ericsson LM. GERAN#68.

3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things V13.0.0.

GP-151183, “Impact on legacy services in a reduced BCCH spectrum allocation”, source Ericsson LM. GERAN#68.

GP-151104, ”Intended scope for reduced spectrum allocation on BCCH evaluation (update of GPE150043)”, source Ericsson LM. GERAN#68.

GP-150762, “EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations (update of GPC150441)”, source Ericsson LM. GERAN#67.

In reality, the MS would likely send preemptive retransmissions, but this is not modelled in the simulator.

It should be noted that this is rather an increase in spectral efficiency than capacity since the same absolute number of reports are delivered in both scenarios.