3GPP TSG RAN WG1 Meeting #90                                 		  R1-1715262

21st  – 25th  August 2017

Prague, Czech Republic

Agenda item:	6.1.2.3.3

Source: 	Qualcomm

Title: 	Remaining issues on NR DM-RS

Document for:	Discussion and Decision

# Introduction

This document provides a list with remaining issues on DM-RS for NR based on the previous agreements captured in the Rel-14 SI [1] and the RAN1#86 to RAN1 AH2 Chairman’s notes shown in Appendix [2]-[9].

# Remaining issues on NR DM-RS

The remaining issues for NR DM-RS, at least based on the “study, or FFS, or consider” agreements [2]-[9], are summarized below:

- DMRS configuration for broadcast PDSCH channel

- Location of one additional DMRS

  - For DL CP-OFDM, if one additional DMRS exists,

    - Location of additional DMRS for non-self-contained ACK/NAK slots

      - FFS: Additional DMRS position for 14-symbol slot

        - Consider symbol 12th, 11th, 10th, 9th

- For self-contained ACK/NAK, 7-symbol and 14-symbol slot, downselect

  - For 7-symbol slot. down selection should be done in next meeting

    - Alt.1:   Only support front loaded DMRS

    - Alt.2 :  Support  front loaded DMRS +additional DMRS

    - Alt.3 :  Configurable between Alt.1 and Alt.2

  - For  14-symbol slot, down selection should be done in next meeting

    - Alt.1:   Only support front loaded DMRS

    - Alt.2 :  Support  front loaded DMRS +additional DMRS

    - Alt.3 :  Configurable between Alt.1 and Alt.2

- Additional DL DMRS locations, if more than one additional DMRS exists, and maximum number of additional DMRS that can be configured.

  - Note: Additional DMRS for UL is captured as a remaining issue later in this list.

- Regarding DL DMRS configurability mechanisms

  - Decide whether to use mechanism for UE-assisted DMRS configuration

  - Agree on details related to signaling aspects of the DMRS configuration

- In case of DL DMRS bundling in time domain, decide whether to support further overhead reduction of DMRS

- Agree on details for the supported PN sequences for CP-OFDM.

- Remaining issues related to UL DMRS:

  - At least for CP-OFDM, decide a common DMRS structure for DL and UL where the exact DMRS location, DMRS pattern, and scrambling sequence can be the same or different

  - Decide the DMRS pattern for UL DFT-S-OFDM waveform

  - Details for the supported sequence for UL CP-OFDM

  - Details for the supported ZC-sequences for UL DFT-S-OFDM DMRS

  - Details of additional DMRS for UL

- DMRS design for 60 KHz ECP

  - DMRS pattern for ECP slot

  - Decide whether to configure different DM-RS patterns for a UE

- DMRS design for mini-slots

  - Decide which DMRS format/structure/configuration for slot-level data channel is re-used for mini slot-level data channel

- Decide whether all DMRS patterns should include front-loaded DMRS or not

- DM-RS power boosting: UE assumption on DM-RS to PDSCH EPRE ratio

- Control signalling for DMRS antenna port indication

- Collision handling of DMRS with PSS/SSS, CSI-RS (if any collision is possible).

# Summary of proposals for some of the remaining topics

## DMRS for broadcast PDSCH

The following agreement was made:

- For broadcast/multicast PDSCH (other than PBCH), the PDSCH is transmitted with a single DMRS port:

  - Support using only one front-load DMRS Configuration from Configuration 1 and Configuration 2. To down-select from:

    - Alt 1: Configuration 1 (FFS 1 or 2 symbols)

    - Alt 2: Configuration 2 with 2 symbols

  - Support additional DM-RS. To down-select from:

    - Alt 1: Additional DMRS is always present

    - Alt 2: Additional DMRS is configurable

    - FFS the number of additional DMRS symbol(s)

From the contributions, at least the following positions can be found:

On the Configuration type of DMRS for broadcast/multicast PDSCH: - Support Alt 1: Configuration 1 with 1 symbol - ZTE, Samsung, Qualcomm, Ericsson, NTT DOCOMO, LGE, ATT - Support of Alt 2 - Huawei, HiSilicon, Mitsubishi - Support the pattern with all of the REs in 1 symbol - Huawei, HiSilicon

Proposal: For broadcast/multicast PDSCH (other than PBCH), the PDSCH is transmitted with a single DMRS port: Support using only front-load DMRS Configuration 1.

Support: Samsung, Qualcomm, NTT DOCOMO, Nokia, NSB, ATT, LGE

Object: Huawei, HiSilicon, Mitsubishi

## Additional DMRS for non-self-contained ACK/NAK slots

## Maximum number of additional DMRS for DL and UL

- For a 14-symbol slot, for 2-symbol front-load DMRS, - no additional DMRS may be configured - ZTE, LGE - up to two more 2-symbol additional DMRS may be configured - Huawei, HiSilicon - up to one more 2-symbol additional DMRS may be configured - Ericsson, Qualcomm, Nokia, NSB, Mediatek, Samsung, ATT, Intel, CATT - For a 14-symbol slot, for 1-symbol front-load DMRS, - up to two 1-symbol additional DMRS may be configured - Mediatek, Spreadtrum, ITL - up to three 1-symbol additional DMRS may be configured Qualcomm, Huawei, HiSilicon, Ericsson, ZTE, Nokia, NSB, NTT DOCOMO, Samsung, ATT, Intel, CATT, LGE - Note: The list above is a starting point for discussion on the maximum number of additional DMRS that may be configured in NR, and additional details are needed on which scenarios and slot formats/duration, the maximum number of DMRS should be used.

## Location of additional DMRS for DL for non-self-contained ACK/NAK slots

At least the following proposals on specific locations have been made from different companies based on the contributions.

DL DMRS for 14-symbol slot-based scheduling with counting starting at 0:

| Company | Using 1-symbol front-load |
| --- | --- |
| Qualcomm | Potential position is every 3*k, or 4*k, or 5*k symbol starting from front-load |
| ZTE | {3,10}, {3,6}, {3,6,10,13} |
| Mediatek | {2, 7}, {2, 11}, {2, 7, 11} |
| Ericsson | {2,10}, {2,5,8}, {2,5,8,11} |
| Nokia | {2,9}, {2,5,9,12}, {2,5,9} |
| Huawei, HiSilicon | {3,6,9,12}, {3,7,11} |
| vivo | {2(3), 8}, {2(3),10} |
| NTT DOCOMO | 4th, 8th and 10th symbol counted from front-load: i.e.: {2,10},{2,6,10},{2,6,10,12} {3,11},{3,7,11}, {3,7,11,13} |

Note also that following design principles of additional DMRS has been proposed:

- The last symbol of additional DMRS in a slot is no later than the last data symbol in the slot for both UL and DL - ZTE

## DL DMRS bundling in time domain

There has been an agreement in RAN1#88 on DMRS bundling in time domain:

  - Support DMRS bundling in time domain

    - At least time domain bundling with slot aggregation of DL-only slots is supported

    - DMRS pattern within the first slot is not impacted by the time domain DMRS bundling

    - FFS: Consider further overhead reduction of DMRS in case of bundling in time domain

NR supports the following for the scenario of DMRS bundling in time domain:

- In scenarios of DMRS bundling in time domain, DMRS patterns of the bundled slots is not impacted. - Nokia, NSB, Samsung, Qualcomm, Ericsson - For DMRS design in slot aggregation scenario, time and frequency domain DMRS port(s) reduction can be considered. - vivo, Huawei, HiSilicon

Proposal: In scenarios of DMRS bundling in time domain, DMRS patterns of the bundled slots are not impacted.

## DL DMRS pattern for self-contained ACK/NAK slots

The following conclusion has been made:

Conclusion:

- When ACK/NACK feedback is configured in the same slot with corresponding DL data transmission

  - For 7-symbol slot. down selection should be done in next meeting

    - Alt.1:   Only support front loaded DMRS

    - Alt.2 :  Support  front loaded DMRS +additional DMRS

    - Alt.3 :  Configurable between Alt.1 and Alt.2

  - For  14-symbol slot, down selection should be done in next meeting

    - Alt.1:   Only support front loaded DMRS

    - Alt.2 :  Support  front loaded DMRS +additional DMRS

    - Alt.3 :  Configurable between Alt.1 and Alt.2

  - Companies are encouraged to provide simulation results for additional DMRS position for mobility scenarios and the assumed number of symbols for front loaded DMRS and additional DMRS, and consider the minimum time to support fast demodulation for DM-RS pattern related design

- For 14-symbol DL slot, - front loaded DMRS is only used for SC ACK/NAK slots - ZTE, Qualcomm - 4th symbol counted from front-loaded DM-RS symbol - NTT DOCOMO - For 7-symbol DL slot, SC ACK/NAK slot is not supported - ZTE, Qualcomm

Proposal: When ACK/NACK feedback is configured in the same slot with corresponding DL data transmission

- For 14-symbol DL slot, only front-loaded DMRS is supported in Rel. 15.

## Power boosting on DL DM-RS

Further progress is needed on the DM-RS to PDSCH EPRE ratio. For example, if DMRS/PDSCH are not FDMed on the same symbol, what will be the UE’s assumption on the DM-RS to PDSCH EPRE ratio? Based on the contributions, it seems that the majority of the companies consider DMRS RE power boosting to be supported in NR:

- NR supports DMRS RE power boosting - China Telecom, CATT, Intel, LGE, Nokia, NSB, Ericsson, KT corp, Qualcomm - NR does not support DMRS RE power boosting - Mediatek - Note: Further discussions and specific proposals are needed to be agreed on details for DMRS RE power boosting (how much power boosting, e.g., 3 dB, 4.77 dB, whether signalling is needed and if yes what signalling). This discussion is also strongly related to the topic of DMRS/PDSCH FDMing and transparent/non-transparent MU-MIMO topic shown below.

## MU-MIMO considerations

Several details need to be agreed for MU-MIMO. Agreeing on the following three topics would help to converge on the main principles on NR MU-MIMO.

## Max MU-MIMO number of ports per UE

- NR supports up 2 ports per UE in MU-MIMO - Intel, ITL(for config-1) - NR supports up 3 ports per UE in MU-MIMO - ITL(for config-2) - NR supports up 4 ports per UE in MU-MIMO - Samsung, Huawei, HiSilicon, Qualcomm, Mitsubishi

Proposal: NR supports up 4 ports per UE in MU-MIMO

## Transparent or Non-transparent MU-MIMO

Several companies provided their proposals on whether to support transparent or non-transparent MU (e.g., UE may be signalled additional information on max or actual number of ports scheduled in a scheduling unit, which comb/CDM-groups are used, whether it is SU-MIMO or MU-MIMO, etc).

- NR supports transparent MU-MIMO - Spreadtrum, ZTE, KT corp, China Telecom - NR supports non-transparent MU-MIMO - Huawei, HiSilicon, Qualcomm, Ericsson, CATT, Intel

A related topic to transparent/non-transparent MU-MIMO is the DMPRS/PDSCH FDMing for MU-MIMO:

## Number of DM-RS port indication tables

Several companies have identified that due to high configurability of the DMRS patterns, more than one DMRS port indication table are needed to be designed to save DCI overhead, especially for MU-MIMO.

- Multiple DCI tables of DMRS port indication at least based on different DMRS configuration type are supported in NR - Agree: Huawei, HiSilicon, Qualcomm, LGE, Intel, Ericsson - Disagree:

## DM-RS configuration signalling aspects

- A UE is configured for PUSCH and PDSCH DMRS with the following signaling method: - Number of additional DMRS - Semi-static: ZTE, Qualcomm, Samsung, NTT DOCOMO, Ericsson - Dynamic: - Location of additional DMRS - Semi-static: ZTE, Qualcomm, Samsung, NTT DOCOMO, Ericsson - Dynamic: - Maximum number of DMRS orthogonal antenna ports - Semi-static: Intel, Qualcomm, Ericsson - Dynamic: - Number of front-load DMRS symbols - Semi-static: Qualcomm, Ericsson - Dynamic : Huawei, HiSilicon, LGE, Panasonic, ZTE

There is a remaining issue whether the UL and the DL will have an independently configured DMRS type:

- The configuration type for UL and DL DMRS is independently configured to a UE - NTT DOCOMO, Ericsson, Qualcomm

Proposal: The configuration type for UL and DL DMRS is independently configured to a UE

## DM-RS port indication tables

## Design principles/ordering

Several companies provided DMRS port indication tables. However, it would be good to initially agree/discussion on a few principles to create these tables.

- For port scheduling ordering principles in DL the following have been proposed: - In MU-MIMO, NR designs DMRS port indication tables such that a UE is expected to be assigned - CDMed ports first, then FDMed ports - Huawei, HiSilicon, Qualcomm, vivo, Intel - FDMed ports first, then CDMed ports - Both may be supported - Samsung, Lenovo, Spreadtrum - In SU-MIMO, NR designs DMRS port indication tables such that a UE is expected to be assigned - CDMed ports first, then FDMed ports - vivo, Intel - FDMed ports first, then CDMed ports - Ericsson - Both may be supported - Qualcomm, Samsung, Lenovo, Spreadtrum - Note: Spreadtrum considers “FDM first” as a first priority for the DMRS table design.

## PTRS and TD-OCC usage

It was agreed in RAN1#AH2 that it should be possible to schedule up to 4 and 6 ports without using both {1,1} and {1,-1} for configuration 1 and configuration 2 respectively. Based on this, the following proposal has been made at least from the following companies:

- When PTRS is configured, TD-OCC is not being used for port multiplexing. - ZTE, Ericsson, Spreadtrum

## DMRS PN sequence for CP-OFDM for DL and UL

At least the following aspects on the DMRS PN sequence for DL and UL have been discussed in a few contributions:

- QPSK modulated DMRS is derived from the - length-31 Gold sequence (as in LTE) - Ericsson, Qualcomm - consider larger degree polynomial (e.g. 63) for PN sequence generator - Intel, ITL

- PN sequence generator depends on the - {VCID, slot index, subcarrier spacing, SCID} - Intel - {VCID, symbol index, SCID} - Qualcomm

- The SCID may take up to - Two values - Spreadtrum, Samsung, Qualcomm - Four values - Ericsson - reconsider whether SCID is needed for NR - Huawei, HiSilicon - Note: Some companies consider SCID to have more than one value for only the one-symbol DMRS configurations.

## ZC-sequence DFT-S-OFDM sequence

- For DFT-S-OFDM, reuse the LTE rel.14 UL DMRS design and associated higher-layer configuration signaling. - Ericsson, Qualcomm - Downselect from circularly extend ZC design and truncation based ZC design determined by resource allocation positions. - Huawei, HiSilicon - the DM-RS sequence for DFT-s-OFDM waveform should be generated by one or multiple concatenated ZC sequences - Intel

## DMRS & SS-block collision

- DM-RS collision with SS-block is handled by DMRS puncturing - Nokia, NSB, ITL - Front-load DMRS is shifted, while additional DMRS is punctured - Intel

## OCC and power imbalance problem

It has been discussed in the reflector the issue of OCC and power imbalance. A few companies have provide some proposals on how this needs to be addressed.

- Specification-based solution to handle the power imbalance of TD-OCC - Mediatek, Speadtrum, LGE - Specification-transparent solution to handle the power imbalance of TD-OCC - Samsung

## DMRS design for URLLC

- The symbol location of front loaded DMRS for URLLC can be adaptive, dependent on whether PDSCH shares DMRS with PDCCH.

  - Huawei, HiSilicon

- The DMRS for first transmission can be used by consequent DL URLLC

  - Huawei, HiSilicon

## UL and DL common DMRS design

- Support same symbol location for DL DMRS and UL DMRS with equal DMRS symbol number for CP-OFDM

  - Huawei, HiSilicon

- RE muting can be considered for orthogonal configuration between DL and UL DMRS

  - Huawei, HiSilicon

- Timing alignment between UL and DL DMRS symbols should be considered to orthogonal configuration

- Huawei, HiSilicon

# Current list of way forwards

R1-1714663, WF on DM-RS for Broadcast PDSCH, Qualcomm

R1-17xxxxx WF on DMRS port mapping ZTE

R1-1714836 WF on DMRS and data multiplexing, LGE, Huawei, HiSi, Ericsson

R1-17xxxxx WF on DMRS port numbering/indexing sequence, Panasonic

R1-1715231 WF on DMRS port mapping table, Panasonic

R1-1714785 WF on signalling DMRS configuration, LGE, ZTE

R1-1714815, WF on configurable density for DL DMRS, Mitsubishi Electric, ETRI

R1-1715195, WF of DMRS for ECP, Huawei, HiSilicon, Intel, MediaTek, vivo

R1-1714883, WF on DMRS port mapping, Huawei, HiSilicon, CATT, Spreadtrum

R1-17xxxxx, WF on Remaining issues on DMRS, Qualcomm

# Contributions on DMRS

# R1-1712243 Design of DL DMRS for data transmission, Huawei, HiSilicon

# R1-1712305 Discussion on downlink DMRS design, ZTE

# R1-1712337 On layer mapping and signaling aspects of front-loaded DMRS for CP-OFDM, Panasonic

# R1-1712383 On DMRS design for DL, CATT

# R1-1712384 On DMRS design for UL, CATT

# R1-1712559 On the remaining details of DL DM-RS, Intel

# R1-1712715, Remaining Details of DM-RS for NR PDSCH, ATT

- R1-1712842, Details on DMRS design, vivo

- R1-1712882, DMRS design considerations for mini-slots, TDL Communications

- R1-1712999, Discussion on DMRS configurations, NEC

- R1-1713052, Considerations on DMRS for CP-OFDM, Spreadtrum

- R1-1713156, On DL and UL DMRS design, LGE

- R1-1713603, Remaining details on DL DMRS, Samsung

- R1-1713712, Views on the remaining issues of downlink DMRS, Mediatek

- R1-1713809, Discussion on downlink DMRS design, ETRI

- R1-1713923, Views on DM-RS, NTT DOCOMO

- R1-1714127, Multiplexing between front-load DMRS and data for CP-OFDM, China Telecom

- R1-1714255, On remaining issues of DL DM-RS for NR physical data channels, Nokia, NSB

- R1-1714310, On DL DMRS design, Ericsson

- R1-1714312, On UL DMRS design, Ericsson

- R1-1714464, Remaining details on DMRS configuration for NR, ITL

- R1-1714467, Discussion on details of DMRS design, KT

- R1-1710196, Discussion on uplink DMRS design, ZTE

- R1-1712307, UL DMRS design for data transmission, Huawei, HiSilicon

- R1-1712560, On the remaining details of UL DM-RS, Intel

- R1-1712680, Discussion on DL DMRS indication, Lenovo, Motorola Mobility

- R1-1714144, Remaining Issues on DM-RS, InterDigital, Inc.

- R1-1704191 Updated simulation results on ECP, Huawei, HiSilicon

# Appendix

## Agreements in RAN1#86

Agreements: - DL DMRS and UL DMRS based spatial multiplexing (SU-MIMO/MU-MIMO) is supported - FFS: Necessity of sidelink spatial multiplexing - At least 8 orthogonal DL DMRS ports is supported for SU-MIMO scheduling - At least 8 orthogonal DL DMRS ports is supported for MU-MIMO scheduling Agreements: - The following QCL assumptions for DM-RS antenna ports in NR are for further study - QCL across DM-RS antenna ports - All the DMRS antenna ports are QCL-ed with each other - Not all the DMRS antenna ports are QCL-ed with each other - QCL across scheduled PRBs for DM-RS antenna port - QCL among DM-RS antenna port groups - QCL of DM-RS antenna ports with antenna ports of other reference signals (RS to be defined in NR) - Other assumptions are not precluded - Note: The antenna ports of demodulation reference signal (DM-RS) in NR are used to transmit at least physical data and, possibly, control channels and used at the UE for demodulation

## Agreements in RAN1#86bis

Agreements: - At least the following RSs are supported for NR downlink - … - DM-RS: Reference signal with main functionalities of data and control demodulation - FFS: channel state information estimation and interference estimation - FFS: beam management - Reference signal for phase tracking - FFS: Whether DM-RS extension can be applied or not - FFS whether new RS or RS for other functionalities can be used - At least the following RSs are supported for NR uplink - … - DM-RS: Reference signal with main functionalities of data and control demodulation - FFS: beam management - Reference signal for phase tracking - FFS: Whether DM-RS extension can be applied or not - FFS whether new RS or RS for other functionalities can be used Agreements: - Study variable/configurable DL/UL RS pattern for demodulation - For data channel and control channel - At least density can be configurable - FFS: other configurability - The applicable scenarios need to be studied - Study multi-set DL/UL RS for control and/or data demodulation - The first set is front-loaded (i.e. loaded in the front of RB) - Other set(s) can be configured for different purposes - Details FFS (e.g. higher frequency/time density, Rx beam detection, RSRP/CSI-reporting, phase noise compensation) Agreements: - Study design of demodulation RS for broadcast channel, control channel and data channel - Separate vs. joint design - Study on design of demodulation RS for data channel - Whether or not the same principle for UL and DL RS pattern design - How to map DM RS in symbols of a slot - Max # of orthogonal DL DMRS ports for SU/MU-MIMO scheduling Agreements: - For DM-RS antenna ports, NR supports: - All ports are QCL-ed - Not all ports are QCL-ed - FFS on details

## Agreements in RAN1#87

Agreements: - Support variable/configurable DMRS pattern for data demodulation - FFS: Time and/or frequency domain density can be configured - FFS: RE location can be configured - At least one configuration supports front-loaded DMRS pattern Conclusions: - Study necessity of supporting 16 orthogonal DMRS ports in DL and UL considering symmetric design both for DL and UL Agreements: - Organize two email discussions i) to collect companies proposals on DMRS design for DL data channel for NR to facilitate evaluation for the next RAN1 meetings till 8th Dec. and ii) to discussion/agree on simulation assumptions till 15th Dec. – Hyunsoo (LGE) - The companies are encouraged to provide DM-RS patterns including DM-RS design details such as - Time domain density (per antenna port) for different ranks (e.g., number of DMRS symbols in slot, symbol position, etc.) - Frequency domain density (per antenna port) for different ranks - The number of maximum orthogonal ports - Multiplexing of DMRS ports (e.g., TDM, FDM, CDM, etc.) - DM-RS sequence (e.g., PN, Zadoff-Chu, etc.) - For the RAN1 NR Ad-hoc meeting, companies are encouraged to provide performance evaluation and to clarify the following aspects used for evaluation: - Channel estimation method (e.g. IFFT type, MMSE type and LS type including assumptions on pipe-line or interpolation/extrapolation for data demodulation, etc.) - PRB bundling assumption - Considering of noise limited and interference limited scenarios - Companies to state phase noise assumptions

## Agreements in RAN1 NR-AH

Agreements: - For the design of front-loaded DMRS, Alt. 1 is agreed as a working assumption. - Alt. 1: Front-loaded DMRS is mapped over 1 or 2 adjacent OFDM symbols. - FFS: Further down-selection between 1 and 2, if necessary - Companies are encouraged to propose further details - Companies are encouraged to provide comparison between Alt. 1 and Alt. 2. - Alt. 2: Design in R1-1700352 (Front-loaded DMRS is mapped over 3 or 4 adjacent OFDM symbols). - Additional DMRS can be configured for the later part of the slot. - FFS: Density reduction compared to front-loaded DMRS - DMRS configuration can be up to the max. number of DMRS ports. Note: DMRS design should take into account channel estimation accuracy for low-to-high SINR scenario, throughput performance for SU and MU-MIMO, DMRS overhead, receiver complexity, receiver implementation, support of the use cases/features, UL/DL symmetry, etc. - FFS: Relative timing relationship between front-loaded DMRS and NR-PDSCH - Option 1: The first symbol of front-loaded DM-RS is fixed regardless of the first symbol of NR-PDSCH. - Option 2: The first symbol of front-loaded DM-RS is no later than the first symbol of NR-PDSCH. Agreements: - Study the following options: - Option1: with 16 orthogonal DMRS ports or - Option2: with non-orthogonal DMRS ports in addition to 8 orthogonal ports - Other options are not precluded. Conclusion: - Companies are encouraged to provide additional evaluation results in next meeting. - Observations are to be drawn based on additional evaluation results with further aligned simulation assumptions - FTP traffic model is baseline, full-buffer traffic model is optional - Each company is to report assumption of the traffic model Agreements: - NR supports at least following functionalities - … - At least for DL data scheduled for a slot, the DL data DMRS location in time is not dynamically varying relative to the start of slot Agreements: - Take the following into account for designing slot-level channels/signals/procedures: - … - At least one of DMRS format/structure/configuration for slot-level data channel is re-used for mini-slot-level data channel - Take the following into account as starting point for designing mini-slot-level channels/signals/procedures: - … - DMRS for mini-slot-level data channel is just a re-use of that for slot-level data channel - …

## Agreements in RAN1#88

Agreements: - At least for CP-OFDM, NR supports a common DMRS structure for DL and UL - DMRS for same or different links can be configured to be orthogonal to each other. - FFS exact DMRS location, DMRS pattern, and, scrambling sequence for the common DMRS structure. Agreements: - Confirm working assumption with some updates: - Front-loaded DMRS is mapped over 1 or 2 adjacent OFDM symbols - NR aims for performance at least comparable to DM-RS of LTE in scenarios where applicable for both LTE and NR Agreements: - For DL DMRS port multiplexing, FDM (including comb), CDM (including OCC and Cyclic shift) and TDM should be considered - For the CDM of DMRS ports in time and/or frequency domain - FFS for OCC based or cycling based - FFS: supporting CDM across adjacent REs - FFS: supporting cyclic shift across non-adjacent REs - FFS OCC size - Support PN sequence for CP-OFDM - FFS: ZC-sequence for CP-OFDM - FFS: For the case front-loaded DMRS pattern with 4 ports, 1 OFDM symbol is supported - FFS: For the case of front-loaded DMRS pattern with 8 ports, two adjacent OFDM symbols are supported - For high Doppler scenario, down selects from the followings - Additional DMRS with reduced density in frequency domain compared to front loaded DMRS - Additional DMRS with same density in frequency domain compared to front loaded DMRS - Note that: Front loaded DMRS can be configured with low density - Note: the complementary use of PT-RS for high Doppler channel estimation can be considered when determining the number of the additional DMRS. - Other option is not precluded - Support DMRS bundling in time domain - At least time domain bundling with slot aggregation of DL-only slots is supported - DMRS pattern within the first slot is not impacted by the time domain DMRS bundling - FFS: Consider further overhead reduction of DMRS in case of bundling in time domain - Consider whether to use mechanism of UE-assisted DMRS configuration. - Consider whether to use UE-assisted configuration of PRG size Working assumption: - Support at least the following design of DL DM-RS for data channels - Support the maximal 12 orthogonal DL DMRS ports for MU-MIMO - Companies are encouraged to perform SLS especially assuming practical channel and interference estimations Agreements: - Study further DMRS configuration(s) for CP-OFDM (DL&UL) and DMRS configuration(s) for DFT-s-OFDM (UL) for a given number of antenna ports, considering at least: - DMRS pattern/position, multiplexing scheme, MU-MIMO (within CP-OFDM UEs, between CP-OFDM&DFT-s-OFDM UEs), etc. - Whether or not to have the same number configuration(s) in DL and UL for CP-OFDM - Possible frequency domain configurations considering: - DMRS overhead - Channel estimation performance - Possible time domain configurations assuming the following scenarios - Low, Medium, high, & very high mobility - Carrier frequency - Latency

## Agreements in RAN1#88bis

Agreements: - Confirm the following working assumption: - Support at least the following design of DL DM-RS for data channels - Support the maximal 12 orthogonal DL DMRS ports for MU-MIMO Agreements: - At least for slot, the location of front-loaded DL DMRS is fixed regardless of the first symbol location of PDSCH - FFS: Mini-slot case - Support ZC-sequence for UL DFT-S-OFDM DMRS Conclusions: - Continue discussions/evaluations until the next meeting about following DMRS port multiplexing schemes for 2 adjacent front-loaded DMRS symbols in the time domain, and RAN1 will definitely conclude this down selection in the next meeting - Alt. 1: OCC - Alt. 2: TDM - Alt. 3: Frequency domain multiplexing only with the time domain repetition/ with a pattern shift - Alt. 4: Configure between Alt. 1 and Alt. 2 - Consider phase noise impact in the high frequency band - Alt. 5: Configure between Alt. 1 and Alt. 3 Agreements: - For CP-OFDM, if one additional DMRS exists - At least for non-self-contained ACK/NAK slot, the time distance between the additional DMRS and front loaded DMRS for 14-symbol slot is larger than that for 7-symbol slot. - FFS additional DMRS position for 14-symbol slot - Consider symbol 12th, 11th, 10th, 9th - Study the location of additional DMRS for self-contained ACK/NAK slots - Evaluations are encouraged for next meeting Conclusion: - Consider the issue of collision between DC subcarrier and DMRS. - Evaluate and analyze whether it can be solved by implementation or if DMRS design needs to take DC subcarrier into account Agreements: - Companies are encouraged to perform further evaluations on additional DM-RS symbols, using same or lower density compared with front loaded DM-RS, and also identifying use cases associated with the operation - Aim to decide in the next meeting whether to support same density only, or lower density only, or both - FFS at least CP-OFDM, frequency domain density of front loaded DMRS is configurable. Agreements: - For 60kHz ECP in the case with WA will be confirmed - … - NR study DMRS patterns for ECP slot - NR may support to configure different DM-RS patterns for a UE

## Agreements in RAN1#89

Agreements: - Support additional DMRS symbols with same density in frequency domain compared to front loaded DMRS - FFS: Necessity of reduced DMRS density in additional DMRS symbols Conclusion: - When ACK/NACK feedback is configured in the same slot with corresponding DL data transmission - For 7-symbol slot. down selection should be done in next meeting - Alt.1: Only support front loaded DMRS - Alt.2 : Support front loaded DMRS +additional DMRS - Alt.3 : Configurable between Alt.1 and Alt.2 - For 14-symbol slot, down selection should be done in next meeting - Alt.1: Only support front loaded DMRS - Alt.2 : Support front loaded DMRS +additional DMRS - Alt.3 : Configurable between Alt.1 and Alt.2 - Companies are encouraged to provide simulation results for additional DMRS position for mobility scenarios and the assumed number of symbols for front loaded DMRS and additional DMRS, and consider the minimum time to support fast demodulation for DM-RS pattern related design Working assumptions: - For DMRS port multiplexing schemes on 2 adjacent front-loaded DMRS symbols in the time domain: - Support DMRS configurations with two alternatives - Time domain OCC (TD-OCC) of antenna ports is supported. In addition, for the other alternative only one of following is supported: - Option 1A: TDM of antenna ports - Option 1B: Time domain repetition of all antenna ports - Notes: - This is for both SU-MIMO and MU-MIMO - If 1B is chosen then whether and/or not pattern shift is used must be decided Objected by Huawei, HiSilicon Working assumption: - UEs in a cell are higher layer configured with 2 DMRS configurations for the front-load DMRS for UL/DL CP-OFDM - Front-load DMRS Configuration 1: Supports up to 8 ports - IFDM based pattern with Comb [2] and/or [4] w cyclic shifts (CS) - One OFDM symbol: - To be down selected to 1 Alt: - Alt 1: Comb 2 + 2 CS, up to 4 ports - Alt 2: Comb 4 + 2 CS, up to 8 ports - Two OFDM symbols: - To be down selected to 2 Alts: - Alt. 1: Comb 2 + 2 CS + TD-OCC ({1 1} and {1 -1}), up to 8 ports - Alt. 2: Comb 2 + 4 CS + TD-OCC ({1 1}), up to 8 ports - Alt. 3: Comb 4 + 2 CS + TD-OCC ({1 1}), up to 8 ports - Front-load DMRS Configuration 2: Supports up to 12 ports - FD-OCC pattern with adjacent REs in the frequency domain - One OFDM symbol: - To be down selected to 1 Alt: - Alt. 1: 2-FD-OCC across adjacent REs in the frequency domain up to 6 ports - Alt. 2: 2-FD-OCC across adjacent REs in the frequency domain up to 4 ports - Alt. 3: 2-FD-OCC across adjacent REs in the frequency domain up to 2 ports - Two OFDM symbols: - 2-FD-OCC across adjacent REs in the frequency domain + TDM up to 12 ports - 2-FD-OCC across adjacent REs in the frequency domain + TD-OCC (both {1,1} and {1,-1}) up to 12 ports - FFS: DMRS pattern before configuration, e.g., SIB1

## Agreements in RAN1 AH2

Agreements: - Both in DL and UL DMRS for CP-OFDM, only PN sequence is supported Agreements: - The working assumption made in RAN1#89 for DM-RS is updated and agreed as follows for CP-OFDM: - A UE is configured by higher layers with DMRS pattern either from the front-loaded DMRS Configuration type 1 or from the front-loaded DMRS Configuration type 2 for DL/UL: - Configuration type 1: - One symbol: - Comb 2 + 2 CS, up to 4 ports - Two symbols: - Comb 2 + 2 CS + TD-OCC ({1 1} and {1 -1}), up to 8 ports - Note: It should be possible to schedule up to 4 ports without using both {1,1} and {1,-1}. - Configuration type 2: - One symbol: - 2-FD-OCC across adjacent REs in the frequency domain, up to 6 ports - Two symbols: - 2-FD-OCC across adjacent REs in the frequency domain + TD-OCC (both {1,1} and {1,-1}) up to 12 ports - Note: It should be possible to schedule up to 6 ports without using both {1,1} and {1,-1}. - From UE perspective, frequency domain CDMed DMRS ports are QCLed. - FFS: Whether the front-load DMRS configuration type for a UE for UL and DL can be different or not. - Note: If there are significant complexity/performance issues involved in the above agreements, down-selection can still be discussed Agreements: - For DFT-S-OFDM based PUSCH DMRS - DMRS are mapped to resource elements using a comb structure (IFDMA). - DMRS and associated PUSCH are multiplexed in time domain - At least the following (repetition factor, CS) combinations is supported - (2, 2) - Front load DMRS is allocated to 1 or 2 OFDM symbols - When 2 OFDM symbols are allocated, TD-OCC ({1 1} and {1 -1}) are supported for orthogonal DMRS port multiplexing. - FFS the detailed applicability of 1 vs. 2 OFDM symbols - FFS the location of the DMRS symbol(s) Agreements: - For broadcast/multicast PDSCH (other than PBCH), the PDSCH is transmitted with a single DMRS port: - Support using only one front-load DMRS Configuration from Configuration 1 and Configuration 2. To down-select from: - Alt 1: Configuration 1 (FFS 1 or 2 symbols) - Alt 2: Configuration 2 with 2 symbols - Support additional DM-RS. To down-select from: - Alt 1: Additional DMRS is always present - Alt 2: Additional DMRS is configurable - FFS the number of additional DMRS symbol(s) Agreements: - Study further aspects related to DMRS and data multiplexing in DL and UL considering 14 and 7 symbol slots/mini-slots, 1 vs. 2 front loaded DM-RS symbols, additional DM-RS, etc. - Study further aspects related to possibly power boosting DM-RS (performance, complexity, spec impact) Agreements: - The number of front-load DMRS symbols can be 1 or 2 when the number of DMRS ports allocated to UE is equal or less than N - N is 4 for Configuration 1 and 6 for Configuration 2. - FFS the details to determine 1 or 2 symbols

# References

- 3GPP, Study on new radio access technology physical layer aspects, TR 38.802 V14.0.0

- 3GPP, RAN1 #86, Chairman’s Notes

- 3GPP, RAN1 #86bis, Chairman’s Notes

- 3GPP, RAN1 #87, Chairman’s Notes

- 3GPP, RAN1 NR-AH, Chairman’s Notes

- 3GPP, RAN1 #88, Chairman’s Notes

- 3GPP, RAN1 #88bis, Chairman’s Notes

- 3GPP, RAN1 #89, Chairman’s Notes

- 3GPP, RAN1 AH2, Chairman’s Notes