3GPP TSG-CT WG4 Meeting #94	C4-194165

# Portoroz, Slovenia; 07th – 11th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0109 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS privacy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019.9.20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 23.273 clause 5.4.2 and clause 7.1, UE LCS Privacy Profile provides information for LCS privacy classes and Location Provacy Indication (LPI), it is a kind of subscription data which is handle by UDM. GLMC / NEF gets the UE LCS Privacy Profile from UDM during procedures as below and control the access user location initiated by AFs / CLS clients: - 5GC-MT-LR Procedure for the commercial location service(see 23.573 clause 6.1.2 step2) - Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events(see 23.573 clause 6.3.1 step2) - Unified Location Service Exposure Procedure (see 23.573 clause 6.5.1 step7) - Bulk Operation of LCS Service Request Targeting to Multiple UEs(see 23.573 clause 6.8 step3) UDM shall store the LCS Privacy Profile in UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The revisions include: - Revise the 5.2.1 Overview to add new LcsPrivacySubscriptionData - Add clause to define new resource LcsPrivacySubscriptionData and get operation - Add LcsPrivacyData in Table 5.4.1-2: Nudr re-used Data Types and Correct some typos in the table. - Add attribute lcsPrivacyData in type ProvisionedDataSets - Add value LCS_PRIVACY in DataSetName - Revise the yaml file accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | To store the LCS Privacy subscription Data for UE is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.x(new), 5.2.x.1(new), 5.2.x.2(new), 5.2.x.3(new), 5.4.1, 5.4.2.8, 5.4.3.4, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... CR0253 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR will introduce backward compatible new features in the OpenAPI specification file of Nudr_DataRepository API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********************The start of changes********************

5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources and applicable HTTP methods.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AuthenticationSubscription | /subscription-data/{ueId}/authentication-data/authentication-subscription | GET | Retrieve a UE's authentication subscription data |
|  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) |
|  |  | GET | Retrieve UE's SoR acknowledgement information |
| AuthenticationUPU | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status | PUT | Store a UE's authentication status |
|  |  | GET | Retrieve a UE's authentication status |
| ProvisionedData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data /am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | GET | Retrieve the UE's subscribed SM Subscription Data |
| LcsPrivacySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-privacy-data | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| ContextData | /subscription-data/{ueId}/context-data | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration | subscription-data/{ueId}/context-data/amf-3gpp-access | PUT | Create and Update the AMF registration for 3GPP access |
|  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration | subscription-data/{ueId}/ context-data/amf-non-3gpp-access | PUT | Update the AMF registration for non 3GPP access |
|  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations | subscription-data/{ueId}/context-data/smf-registrations | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration | subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  | DELETE | Delete an individual SMF registration |
|  |  | GET | Retrieve individual SMF registration |
| OperatorSpecificData | /subscription-data/{ueId}/operator-specific-data | GET | retrieve the operator specific subscription data of a UE |
|  |  | PATCH | modify the operator specific subscription data of a UE |
| OperatorDeterminedBarringData | /subscription-data/{ueId}/operator-determined-barring-data | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration | subscription-data/{ueId}/context-data /smsf-3gpp-access | PUT | Create or Update the SMSF registration |
|  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration | subscription-data/{ueId}/context-data /smsf-non-3gpp-access | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| SdmSubscriptions | /subscription-data/{ueId}/context-data/sdm-subscriptions | GET | Retrieve SDM subscriptions |
|  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | PUT | Update an individual SDM subscription |
|  |  | DELETE | Delete an individual SDM subscription |
|  |  | PATCH | Update an individual SDM Subscription |
| EeSubscriptions | /subscription-data/{ueId}/context-data/ee-subscriptions | GET | Retrieve EE subscriptions |
|  |  | POST | Create an EE subscription |
| IndividualEeSubscription | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription |
|  |  | DELETE | Delete an individual EE subscription |
|  |  | PATCH | Update an individual EE subscription |
| AmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | PUT | Store information related the a received Amf-EE-Subscription response |
|  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  | GET | Retrieve AMF-subscriptions |
|  |  | PATCH | Update AMF-subscriptions |
| EeProfileData | /subscription-data/{ueId}/ ee-profile-data | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData | /subscription-data/{ueId}/pp-data | PATCH | Update of provisioned parameters. |
|  |  | GET | Retrieves the UE's provisioned parameters. |
| SMSSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions | /subscription-data/subs-to-notify | GET | Retrieve existing subscriptions |
|  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription | /subscription-data/subs-to-notify/{subsId} | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
| EeGroupSubscriptions | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | GET | Retrieve EE subscriptions for groups of UEs |
|  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription for a group of UEs |
|  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  | PATCH | Update an individual EE subscription for a group of UEs |
| TraceData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data /trace-data | GET | Retrieve the UE's trace configuration data |
| IdentityData | /subscription-data/{ueId}/identity-data | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | GET | Retrieve shared data |
| GroupIdentifiers | /group-data/group-identifiers | GET | Retrieve group identifiers |
| 5GVnGroups | /group-data/5g-vn-groups | GET | Retrieve 5G VN Groups |
| Individual5GVnGroup | /group-data/5g-vn-groups/{externalGroupId} | PUT | Create a 5G VN Group |
|  |  | PATCH | Update a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |

********************Next change********************

5.2.x	Resource: LcsPrivacySubscriptionData

5.2.x.1	Description

This resource represents the subscribed LcsPrivacySubscriptionData for a SUPI for use in a serving PLMN. It is queried by the HGMLC or NEF via the UDM.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

5.2.x.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-privacy-data

This resource shall support the resource URI variables defined in table 5.2.x.2-1.

# Table 5.2.x.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 1 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: "^(imsi-[0-9]{5,15}¦nai-.+¦msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦.+)$" |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

5.2.x.3	Resource Standard Methods

5.2.x.3.1	GET

This method shall support the URI query parameters specified in table 5.2.x.3.1-1.

# Table 5.2.x.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 1 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.x.3.1-2 and the response data structures and response codes specified in table 5.2.x.3.1-3.

# Table 5.2.x.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.x.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LcsPrivacyData | M | 1 | 200 OK | Upon success, a response body containing the LCS Privacy Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

********************Next change********************

5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 |  |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 |  |
| AuthMethod | 5.4.3.3 |  |
| DataSetName | 5.4.3.4 |  |
| ConextDataSetName | 5.4.3.6 |  |
| SqnScheme | 5.4.3.7 |  |
| Sign | 5.4.3.8 |  |
| UeUpdateStatus | 5.4.3.9 |  |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| ` | Reference | Comments |
| --- | --- | --- |
| AccessAndMobilitySubscriptionData | 3GPP TS 29.503 [6] | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 3GPP TS 29.503 [6] | SMF Selection Subscription Data |
| SessionManagementSubscriptionData | 3GPP TS 29.503 [6] | Session Management Subscription Data |
| Amf3GppAccessRegistration | 3GPP TS 29.503 [6] |  |
| LcsPrivacyData | 3GPP TS 29.503 [6] | LCS Privacy Subscription Data |
| AmfNon3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |
| SmfRegistration | 3GPP TS 29.503 [6] |  |
| SmsfRegistration | 3GPP TS 29.503 [6] |  |
| SmsManagementSubscriptionData | 3GPP TS 29.503 [6] |  |
| SupportedFeatures | 3GPP TS 29.571 [3] | Used to negotiate the applicability of the optional features |
| ProblemDetails | 3GPP TS 29.571 [3] | Detailed information about the status code. |
| PpData | 3GPP TS 29.503 [6] |  |
| SmsSubscriptionData | 3GPP TS 29.503 [6] |  |
| PatchItem | 3GPP TS 29.571 [3] | Data structure used for JSON patch. |
| SdmSubscription | 3GPP TS 29.503 [6] |  |
| EeSubscription | 3GPP TS 29.503 [6] |  |
| Uri | 3GPP TS 29.571 [3] |  |
| TraceData | 3GPP TS 29.571 [3] |  |
| Dnn | 3GPP TS 29.571 [3] | Data Network Name |
| Snssai | 3GPP TS 29.571 [3] | Single NSSAI |
| VarUeId | 3GPP TS 29.571 [3] | String represents the SUPI or GPSI. |
| NfInstanceId | 3GPP TS 29.571 [3] |  |
| NotifyItem | 3GPP TS 29.571 [3] | Common data type used for data change notification. |
| OdbData | 3GPP TS 29.571 [3] | Operator Determined Barring Data |
| EventType | 3GPP TS 29.503 [6] |  |
| ExtGroupId | 3GPP TS 29.503 [6] | External Group Identifier |
| PduSessionId | 3GPP TS 29.571 [3] |  |
| DateTime | 3GPP TS 29.571 [3] |  |
| UpuMac | 3GPP TS 29.509 [15] |  |
| 5GVnGroupConfiguration | 3GPP TS 29.503 [6] |  |

********************Next change********************

5.4.2.8	Type: ProvisionedDataSets

# Table 5.4.2.8-1: ProvisionedDataSets

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amData | AccessAndMobilitySubscriptionData | O | 0..1 | Access and Mobility Subscription Data |
| smfSelData | SmfSelectionSubscriptionData | O | 0..1 | SMF Selection Subscription Data |
| smsSubsData | SmsSubscriptionData | O | 0..1 | SMS Subscription Data |
| smData | array(SessionManagementSubscriptionData) | O | 1..N | Session Management Subscription Data |
| traceData | TraceData | O | 0..1 | Trace Data. The Null value indicates that trace is not active. |
| smsMngData | SmsManagementSubscriptionData | O | 0..1 | SMS Management Subscription Data |
| lcsPrivacyData | LcsPrivacyData | O | 0..1 | LCS Privacy Subscription Data |

********************Next change********************

5.4.3.4	Enumeration: DataSetName

# Table 5.4.3.4-1: Enumeration DataSetName

| Enumeration value | Description |
| --- | --- |
| "AM" | Access and Mobility Subscription Data |
| "SMF_SEL" | SMF Selection Subscription Data |
| "SMS_SUB" | SMS Subscription Data |
| "SM" | Session Management Subscription Data |
| "TRACE" | Trace Data |
| "SMS_MNG" | SMS Management Subscription Data |
| "LCS_PRIVACY" | LCS Privacy Subscription Data |

********************Next change********************

A.2	Nudr_DataRepository API for Subscription Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29505_Subscription_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: 'Unified Data Repository Service API file for subscription data'

description: |

Unified Data Repository Service (subscription data).

The API version is defined in 3GPP TS 29.504.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

Skipped for Clarity

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data:

get:

summary: Retrieves the Session Management subscription data of a UE

operationId: QuerySmData

tags:

- Session Management Subscription Data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: single-nssai

in: query

content:

application/json:

schema:

$ref: '#/components/schemas/VarSnssai'

description: single NSSAI

required: false

- name: dnn

in: query

description: DNN

required: false

schema:

$ref: '#/components/schemas/Dnn'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

minItems: 1

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-privacy-data:

get:

summary: Retrieves the LCS Privacy subscription data of a UE

operationId: QueryLcsPrivacyData

tags:

- LCS Privacy Subscription Data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/LcsPrivacyData'

minItems: 1

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/subscription-data/{ueId}/context-data/amf-3gpp-access:

get:

summary: Retrieves the AMF context data of a UE using 3gpp access

operationId: QueryAmfContext3gpp

tags:

- AMF 3GPP Access Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

put:

summary: To store the AMF context data of a UE using 3gpp access in the UDR

operationId: CreateAmfContext3gpp

tags:

- AMF 3GPP Access Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

patch:

summary: To modify the AMF context data of a UE using 3gpp access in the UDR

operationId: AmfContext3gpp

tags:

- AMF 3GPP Access Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

Skipped for Clarity

components:

schemas:

AuthenticationSubscription:

type: object

required:

- authenticationMethod

properties:

authenticationMethod:

$ref: '#/components/schemas/AuthMethod'

encPermanentKey:

type: string

protectionParameterId:

type: string

sequenceNumber:

$ref: '#/components/schemas/SequenceNumber'

authenticationManagementField:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

algorithmId:

type: string

encOpcKey:

type: string

encTopcKey:

type: string

SequenceNumber:

type: object

properties:

sqnScheme:

$ref: '#/components/schemas/SqnScheme'

sqn:

type: string

pattern: '^[A-Fa-f0-9]{12}$'

lastIndexes:

type: object

additionalProperties:

type: integer

minimum: 0

indLength:

type: integer

minimum: 0

difSign:

$ref: '#/components/schemas/Sign'

SqnScheme:

anyOf:

- type: string

enum:

- GENERAL

- NON_TIME_BASED

- TIME_BASED

- type: string

Sign:

type: string

enum:

- POSITIVE

- NEGATIVE

VarPlmnId:

type: string

pattern: '^[0-9]{5,6}$'

DatasetNames:

type: array

items:

$ref: '#/components/schemas/DataSetName'

minItems: 1

uniqueItems: true

DataSetName:

anyOf:

- type: string

enum:

- AM

- SMF_SEL

- SMS_SUB

- SM

- TRACE

- SMS_MNG

- LCS_PRIVACY

- type: string

ProvisionedDataSets:

type: object

properties:

amData:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

smfSelData:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

smsSubsData:

$ref: '#/components/schemas/SmsSubscriptionData'

smData:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

smsMngData:

$ref: '#/components/schemas/SmsManagementSubscriptionData'

lcsPrivacyData:

$ref: '#/components/schemas/LcsPrivacyData'

AccessAndMobilitySubscriptionData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/AccessAndMobilitySubscriptionData'

SmfSelectionSubscriptionData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmfSelectionSubscriptionData'

VarSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

Dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

SessionManagementSubscriptionData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SessionManagementSubscriptionData'

Amf3GppAccessRegistration:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/Amf3GppAccessRegistration'

AmfNon3GppAccessRegistration:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/AmfNon3GppAccessRegistration'

SmfRegistration:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/SmfRegistration'

SmsfRegistration:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/SmsfRegistration'

SmsManagementSubscriptionData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmsManagementSubscriptionData'

SmsSubscriptionData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmsSubscriptionData'

LcsPrivacyData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsPrivacyData'

OperatorSpecificDataContainer:

type: object

required:

- dataType

- value

properties:

dataType:

type: string

enum:

- string

- integer

- number

- boolean

- object

dataTypeDefinition:

type: string

value:

oneOf:

- type: string

- type: integer

- type: number

- type: boolean

- type: object

AuthMethod:

anyOf:

- type: string

enum:

- 5G_AKA

- EAP_AKA_PRIME

- EAP_TLS

- type: string

Skipped for Clarity

********************The end of changes********************