3GPP TSG-CT WG4 Meeting #94	C4-194288

# Portoroz, Slovenia; 07th – 11th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0252 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NID for Standalone NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A standalone Non-Public Network is identified by a PLMN ID and a NID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NID is added to UeContextTransferReqData to identify the SNPN of the NF Service Consumer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.23, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the OpenAPI specification file for the Namf_Communication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Information within AMFStatusChangeSubscribe |  |
| AmfStatusChangeNotification | 6.1.6.2.3 | Information within AMFStatusChangeNotify |  |
| AmfStatusInfo | 6.1.6.2.4 | Information within AMFStatusChangeNotify |  |
| AssignEbiData | 6.1.6.2.5 | Represents information needed for AMF to assign EBIs. |  |
| AssignedEbiData | 6.1.6.2.6 | Represents successful assignment of EBI(s). |  |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |  |
| UEContextRelease | 6.1.6.2.8 | Information within ReleaseUeContext |  |
| N2InformationTransferReqData | 6.1.6.2.9 | N2 information requested to be transferred to 5G AN. |  |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Subscription information for non UE specific N2 information notification. |  |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | The created subscription for non UE specific N2 information notification. |  |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Subscription information for UE specific N1 and/or N2 information notification. |  |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | The created subscription for UE specific N1 and/or N2 information notification. |  |
| N2InformationNotification | 6.1.6.2.14 | N2 information for notification. |  |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |  |
| N1MessageNotification | 6.1.6.2.16 | N1 message notification data structure. |  |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container |  |
| N1N2MessageTransferReqData | 6.1.6.2.18 | N1/N2 message container |  |
| N1N2MessageTransferRspData | 6.1.6.2.19 | N1/N2 message transfer response |  |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |  |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |  |
| UeContextTransferReqData | 6.1.6.2.23 | Represents to start transferring of an individual ueContext resource from old AMF to new AMF. |  |
| UeContextTransferRspData | 6.1.6.2.24 | Indicates the transferring of the individual ueContext resource is started successfully. |  |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |  |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |  |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |  |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |  |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |  |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | N1/N2 Message Transfer Failure Notification |  |
| N1N2MessageTransferError | 6.1.6.2.31 | N1/N2 Message Transfer Error |  |
| N2N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |  |
| N2InformationTransferRspData | 6.1.6.2.33 | Indicates a successful delivery of N2 Information to the AN. |  |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |  |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |  |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |  |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |  |
| NssaiMapping | 6.1.6.2.38 | Represents a map of a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |  |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Provides information on the UE registration completion at a target AMF. |  |
| AssignEbiError | 6.1.6.3.40 | Represents the details regarding EBI assignment failure. |  |
| UeContextCreateData | 6.1.6.2.41 | Indicates a request to create an individual ueContext resource |  |
| UeContextCreatedData | 6.1.6.2.42 | Indicates a successful creation of an individual ueContext resource |  |
| UeContextCreateError | 6.1.6.2.43 | Represents an error when creating a UE context |  |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |  |
| N2InformationTransferError | 6.1.6.2.45 | Error within NonUeN2MessageTransfer response |  |
| PWSResponseData | 6.1.6.2.46 | Represents the type of PWS |  |
| PWSErrorData | 6.1.6.2.47 | Represents the type of PWS error |  |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 1 33.501 [27]) |  |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 1 33.501 [27]). |  |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |  |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Provides the status of UE context transfer status update at a source AMF. |  |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |  |
| N2InfoNotificationRspData | 6.1.6.2.54 | N2 information notification response data |  |
| SmallDataRateStatusInfo | 6.1.6.2.55 | Represents the small data rate status |  |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |  |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |  |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |  |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |  |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |  |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |  |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |  |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |  |
| MSClassmark2 | 6.1.6.3.2 | Indicates the MS Classmark 2 of a 5G SRVCC UE |  |
| SupportedCodec | 6.1.6.3.2 | Indicates the supported codec of a 5G SRVCC UE |  |
| StatusChange | 6.1.6.3.3 |  |  |
| N2InformationClass | 6.1.6.3.4 |  |  |
| N1MessageClass | 6.1.6.3.5 |  |  |
| N1N2MessageTransferCause | 6.1.6.3.6 |  |  |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |  |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |  |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |  |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |  |
| SmsSupport | 6.1.6.3.11 | Indicates the supported SMS delivery of a UE. |  |
| ScType | 6.1.6.3.12 | Indicates the security context type. |  |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |  |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |  |
| AMPolicyReqTrigger | 6.1.6.3.15 | AM Policy Request Triggers |  |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |  |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |  |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |  |
| SmfChangeIndication | 6.1.6.3.19 | Indicates the I-SMF, change or removal, or the V-SMF change | DTSSA |

Table 6.1.6.1-2 specifies data types re-used by the Namf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf service based interface.

Table 6.1.6.1-2: Namf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Snssai | 3GPP TS 29.571 [6] |  |  |
| Arp | 3GPP TS 29.571 [6] |  |  |
| PduSesisonId | 3GPP TS 29.571 [6] |  |  |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |  |
| AmfName | 3GPP TS 29.571 [6] | The name of the AMF |  |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |  |
| IndicationFlags | 3GPP TS 29.502 [16] | Indication Flags |  |
| Cause | 3GPP TS 29.571 [6] | 5G-AN Cause |  |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |  |
| supportedFeatures | 3GPP TS 29.571 [6] | Supported Features |  |
| TimeZone | 3GPP TS 29.571 [6] |  |  |
| UserLocation | 3GPP TS 29.571 [6] |  |  |
| AccessType | 3GPP TS 29.571 [6] |  |  |
| AllowedNssai | 3GPP TS 29.531 [18] |  |  |
| NfInstanceId | 3GPP TS 29.571 [6] |  |  |
| Uri | 3GPP TS 29.571 [6] |  |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |  |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |  |
| Uint16 | 3GPP TS 29.571 [6] |  |  |
| 5Qi | 3GPP TS 29.571 [6] | 5G QoS Identifier |  |
| CorrelationID | 3GPP TS 29.572 [25] | LCS Correlation ID |  |
| Pei | 3GPP TS 29.571 [6] |  |  |
| Dnn | 3GPP TS 29.571 [6] |  |  |
| Gpsi | 3GPP TS 29.571 [6] |  |  |
| GroupId | 3GPP TS 29.571 [6] |  |  |
| PlmnId | 3GPP TS 29.571 [6] |  |  |
| RfspIndex | 3GPP TS 29.571 [6] |  |  |
| EbiArpMapping | 3GPP TS 29.502 [16] | EBI - ARP mapping |  |
| NsiId | 3GPP TS 29.531 [18] |  |  |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |  |
| ConfiguredSnssai | 3GPP TS 29.531 [18] |  |  |
| NgApCause | 3GPP TS 29.571 [6] | Represents the NG AP cause IE |  |
| Area | 3GPP TS 29.571 [6] |  |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [6] |  |  |
| CoreNetworkType | 3GPP TS 29.571 [6] |  |  |
| Ambr | 3GPP TS 29.571 [6] |  |  |
| GlobalRanNodeId | 3GPP TS 29.571 [6] |  |  |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |  |
| DurationSec | 3GPP TS 29.571 [6] |  |  |
| StnSr | 3GPP TS 29.571 [6] | Session Transfer Number for SRVCC |  |
| CMsisdn | 3GPP TS 29.571 [6] | Correlation MSISDN |  |
| DateTime | 3GPP TS 29.571 [6] |  |  |
| SmallDataRateStatus | 3GPP TS 29.571 [6] |  |  |
| Nid | 3GPP TS 29.571 [6] |  |  |

* * * Next Change * * * *

##### 6.1.6.2.23	Type: UeContextTransferReqData

Table 6.1.6.2.23-1: Definition of type UeContextTransferReqData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reason | TransferReason | M | 1 | Indicate the reason for the UEContextTransfer service request |
| accessType | AccessType | M | 1 | This IE shall contain the access type of the UE. |
| plmnId | PlmnId | O | 0..1 | If present, this IE shall contain the PLMN ID of the NF service consumer (e.g target AMF). |
| nid | Nid | C | 0..1 | This IE shall be present if the NF service consumer pertains to a Standalone Non-Public Network. When present, it shall contain the Network Identifier, which together with the PLMN ID identifies the SNPN of the NF Service consumer. |
| regRequest | N1MessageContainer | O | 0..1 | If present, this IE shall refer to the registration request message which triggers the UE Context Transfer. The message class shall be "5GMM" and message content shall be reference to N1 Message Content binary data, See clause 6.1.6.4.2. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |

* * * Next Change * * * *

## A.2	Namf_Communication API

openapi: 3.0.0

info:

version: 1.1.0.alpha-2

title: Namf_Communication

description: |

AMF Communication Service

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

UeContextTransferReqData:

type: object

properties:

reason:

$ref: '#/components/schemas/TransferReason'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Nid'

regRequest:

$ref: '#/components/schemas/N1MessageContainer'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- reason

- accessType

[…]

* * * End of Changes * * * *