# GSM Evolution for cellular IoT - SCH design, performance and mapping

# Introduction

In this contribution the Synchronization Channel (SCH) performance in extended coverage is investigated for stationary devices. The target of the investigation is to support “extended coverage of 20 dB compared to legacy GPRS (Non EGPRS)” [1] for the GSM Evolution solution using repetition based transmission schemes.

In its current form the SCH conveys a Reduced TDMA frame Number (RFN) that signals the position of the SCH burst in the 51-multiframe structure. This means that the content of the SCH changes for each transmission. As the repetition based schemes investigated in this contribution requires that the content of the SCH remains constant over the repetition period a redesign of the SCH information is presented as part of the investigations.

This contribution is an update of [11] presented at GERAN#63, to take into account recent agreements on legacy GPRS performance. Major updates have in the following been highlighted using red font color.

# Coverage improvment

## Maximum Coupling Loss

The aim of the feasibility study FS_IoT_LC [1] is to improve coverage by 20 dB compared to legacy GPRS.  At the 5th Teleconference on FS_IoT_LC a common understanding was reached on a legacy GPRS MCL reference value of 144 dB given a MS Noise figure of 5 dB.  Adding 20 dB of coverage extension to the MCL of 144 dB leads to a requirement of 164 dB. As the SCH is a synchronization channel it is important that it reaches beyond e.g. the packet data traffic channels with a certain margin. The target of this investigation is to explore the feasibility of extending SCH coverage beyond a MCL of 164 dB.

## Repetition based transmission

To achieve a coverage improvement beyond 20 dB it is obvious that the receiving end must cope with negative signal to noise ratios (SNR). Without redefining the channel coding of the SCH the channel can be made more robust trough repetition based transmission. It will be shown that 16 or 32 repetitions are needed to reach beyond the ambitious target of 20 dB coverage enhancement. At the transmitter side the repetition is straightforward, the BTS sends the same SCH burst over the air on the defined SCH positions in the 51-multiframe. At the receiving side the MS is in this investigation accumulates partitions of the repeated bursts in the IQ domain coherently, and sends the aggregated signal from each partition to the equalizer that produces soft bits. The soft bits from all partitions are combined and forwarded to the channel decoder.

Important to remember is that there will be mobiles requiring less than the full set of repetitions to read the SCH. However since the SCH is a broadcast channel it must be designed to support mobiles in the worst expected coverage. So an aspect to take note of is that while the BSS will transmit the SCH over e.g. 32 repetitions, the mobiles in a network will be able to decode the SCH after anywhere from one up to 32 repetitions. It can further be noted that if the MS is not in extended coverage the legacy procedure for FCCH and SCH on TS0 on the BCCH carrier can be followed.

## Frequency synchronization

In [4] it is shown that a mobile receiver in extended coverage with high likelihood will meet the frequency accuracy requirement of 0.1 ppm [9] already after reading the FCCH for coverage extensions up to 20 dB. Further investigation is needed to see the resulting performance of FCCH beyond 20 dB. It is believed this can be realized by increased acquisition time, but is left FFS.

The SCH can be used to further improve the frequency estimate from the FCCH. To study the feasibility of estimating the SCH carrier frequency a low complexity method for frequency tracking has been investigated as part of this contribution. The method is based on the observation that a frequency offset leads to a phase drift in the receiver that can be estimated by studying the received samples in consecutive bursts.

# SCH definition

## Mapping on 51-multiframe structure

The legacy SCH is defined as a single burst radio block, which is mapped on time slot (TS) 0 and TDMA frame 1, 11, 21, 32 and 41 in the 51-multiframe on of the Broadcast control channel (BCCH) frequency [3].

It will be shown in section 4.2 that 16 or 32 repetitions are needed to achieve coverage improvements beyond 20 dB. To not disturb GSM legacy operation it is proposed to map the SCH design for extended coverage onto TS1. At the same time it is indicated in [4] that operation of the FCCH is feasible already with today’s mapping on TS 0, and no support for FCCH is expected to be needed on TS 1. So one straightforward redesign of the SCH mapping on TS 1 would be to use a compact mapping where the complete set of SCH blocks is transmitted over the first eight, or 16, TDMA frames in the beginning of two consecutive 51-multiframes, as depicted in Figure 1, to achieve 16 or 32 repetitions, respectively.

Figure 1 Compact mapping of SCH on 51-multiframe on TS1.

## Redesign of SCH

The legacy SCH contains per its current definition a 19 bit RFN defined as follows [3];

“T1		(11 bits)	range 0 to 2047	= FN div ( 26 x 51)

T2		(5 bits)	range 0 to 25		= FN mod 26

T3' 	(3 bits)	range 0 to 4		= (T3 - 1) div 10

where

T3		(6 bits)	range 0 to 50		= FN mod 51”.

As T1 defines the position of the GSM superframe within the hyperframe, it remains unaffected by the SCH mapping alternatives presented in section 3.1.

T2 defines the position of the 51-multiframe within the T1 superframe. When repeating a single instance of the SCH over two consecutive 51-multiframes T2 can be reduced to signal on which pair of 51-multiframes in the superframe the instance of the SCH is transmitted. T2 can hence be defined as;

T2		(4 bits)	range 0 to 12		= FN mod 13

As a mobile requiring repetitions only over one 51-mutliframe still need to be able to identify if the SCH is transmitted over an even or an odd numbered 51-multiframe. Also for mobiles requiring repetitions over two 51-multiframes, an indication of between which 51-multiframe borders the SCH information is changing is needed to avoid blindly decoding two different SCH mapping options.

It is therefore proposed to modulate the SCH using a modified GMSK modulation with a negative modulation index h equal to -½ on odd numbered frames. This can be compared with today’s GMSK implementation where a modulation index of ½ is used [5]. By detecting the modulation index, using de-rotation of /2 or -/2, a mobile can determine if a SCH burst is transmitted over an even or odd numbered 51-multiframe. It is expected that this detection can be done with sufficient accuracy, but a detailed investigation on performance is for further study. Figure 2 exemplifies this proposal using the 16 repetitions alternative.

Figure 2 Implementing GMSK with a negative modulation index on odd 51-multiframes.

In the case of mobiles performing combination in the IQ domain over two 51-multiframes the mobile can just apply the complex conjugate to the instances of the SCH burst received in the odd numbered 51-multiframe to achieve coherent combination over all repetitions.

From the BTS perspective implementation of using negative modulation index is simplistic, and can be achieved e.g. through applying the complex conjugate to the output of an already existing GMSK modulator.

T3’ is finally proposed to be removed, as a mobile device in extended coverage is expected to detect which TDMA frame number (FN) a SCH instance in the 51-multiframes is configured on when considering  the uneven distribution of FCCH and SCH blocks over the 51-multiframe on TS 0 and 1, respectively.

With this redefinition the RFN will be signaled using 15 bits, effectively creating four spare bits. Usage of these four spare bits is for further study, interesting applications could however be to move the signaling of the Implicit Reject PS, and the BCCH_CHANGE_MARK messages to the SCH [6]. In case of the BCCH_CHANGE_MARK a single bit information would probably be sufficient given the compressed system information proposed in [10] for Internet of Things.  This would mean that mobiles do no longer have to read the Paging Channel (PCH) to realize that they are restricted from network access, or the BCCH to understand that the system information must be reread. This information will be at hand already when synchronizing to the cell.

# Simulations

SCH simulation has been carried out for stationary devices using 16 or 32 repetitions using the compact mappings presented in section 3.1. The below sections highlight some important simulation assumptions and results.

## Simulation assumptions

To facilitate link level simulations the required coverage improvement has been translated into an Es/N0 requirement using the following MCL calculation as baseline;

MCL=PBTS-N0+NFMS+EsN0dB

Where;

- The base station output power BTS=43 dBm

- The  mobile noise figure NFMS=5 dB

- The thermal noise is defined as N0=-174+10log10BWsim dBm, where the simulator bandwidth BWsim=13*106/48 kHz.

If in addition to this requiring MCL to go beyond 164 dB implies that Es/N0 shall be less than;

EsN0=43+119.7-5-164=- 6.3 dB

One can either use the simulator bandwidth, i.e. the bandwidth over which the used simulator defines Es/N0, or the mobile receiver bandwidth to calculate the thermal noise. If the latter definition is used then the definition of Es/N0, or rather the SNR after receive filtering needs to be corrected accordingly.

Other simulation settings are presented in below table. Of especial importance are the channel propagation conditions, i.e. the channel and the speed of the mobile. Settings in Table 1 are assumed to model a stationary mobile, but this assumption is discussed in more detail in [8]. The assumed frequency offset was set to 90 Hz, based on the requirement on carrier frequency accuracy in [9] and the results in [4] where it is shown that also devices in extended coverage can meet the specified requirement after reading the FCCH.

Table 1.  Simulation assumptions.

| Parameter | Value | Comment |
| --- | --- | --- |
| MCS | SCH | Redefinition of RFN assumed. |
| Channel | Extended Pedestrian A [2] | According to agreed assumption in FS_LC_MTC_LTE feasibility study. Taps are fading according to the Classical Doppler spectrum. |
| Mobile speed | 1.2 km/h | Assumed to model stationary mobile in somewhat mobile environment. Possible to use for performance evaluation as agreed at FS_IoT_LC telco#2 Corresponds to max Doppler shift of 1Hz. |
| Band | GSM 900 | The 0.1ppm frequency accuracy requirement corresponds to 90 Hz [9]. |
| Impairments | Typical RX and TX. Frequency offset fixed at 90 Hz. | See e.g. [7] for details on typical settings. Frequency offset chosen based on results in [4]. |
| AFC | Range of possible frequency offsets detected [-100,100] Hz |  |
| Number of repetitions | 16 or 32 | Using a compact mapping over 2 51-multiframes. |

## Simulated results

Results are shown in Figure 3 where the MCL requirement of 164 dB is equivalent to an Es/N0 of -6.3 dB. At 16 repetitions an Es/N0 of -6.7 dB, and at 32 repetitions an Es/N0 of -9.1    is achieved at 10% Data BLER. In both cases of the IQ accumulation was done over groups of SCH bursts, followed by equalization and soft combining. This confirms the feasibility of network synchronization in case of the GSM evolution track in the FS_IoT_LC study.

Figure 3. SCH BLER performance for 16 and 32 repetitions.

In parallel to the equalization the mobile receiver continuously estimates the SCH carrier frequency over the full set of repetitions. Figure 4 contains a PDF over the frequency estimation error logged during a simulation where the Es/N0 was set to -10 dB to correspond to a MCL of 167.7 dB. It is seen that the frequency accuracy after reading the repeated SCH can be assumed to be of an accuracy ±10 Hz with a very high probability, and that a normal distribution characterized by a 3 Hz standard deviation can be used to model the accuracy. Accurate frequency estimate after synchronization is important as the error in the estimated frequency can be mapped to a frequency offset in the mobile transmitter which must be dealt with by the BTS during later packet uplink transfer, at least if some level of coherent accumulation of IQ samples assumed performed.

This confirms that further investigations on uplink performance in the GSM evolution track in the FS_IoT_LC study can be performed with stringent assumption on typical UL carrier frequency offsets.

Figure 4 Frequency estimation error after reading SCH at Es/N0 -10 dB.

# Summary

This contribution has shown that network synchronization at a MCL of roughly 167 dB is achievable in case of the GSM evolution track in the FS_IoT_LC study. During the synchronization, frequency accuracy within ±10 Hz is achievable with high probability, which is important to take into account when studying UL performance in the FS_IoT_LC study.

The mentioned coverage extension is achieved through up to 32 repeated transmissions of the SCH over two consecutive 51-multiframes configured on TS1 of the BCCH carrier. To support the repeated transmission scheme redefinitions of the SCH mapping over the 51-multiframe, the SCH information content and the SCH modulation have been proposed. The SCH block definition in terms of channel coding used and block size is however left as currently defined.

# References

GP-140241, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, GERAN#62, source VODAFONE Group Plc.

TR 36.888, v12.0.0, “Study on provision of low-cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”, 3GPP RAN1

TS 45.002, v12.0.0, “Multiplexing and multiple access on the radio path, 3GG GERAN1

GP-140601, “GSM Evolution for cellular IoT - FCCH overview”, GERAN#63, source Ericsson

TS 45.004, v11.0.0, Modulation, 3GPP GERAN1

TS 44.018, v12.0.0, Radio Resource Control, 3GPP GERAN2

TR 45.860, v11.5.0, “Signal Precoding Enhancements for EGPRS2 DL”. 3GPP GERAN1

GP-140608, “Radio propagation channel for cellular IoT”, GERAN#63, source Ericsson

TS 45.010, v12.0.0, Radio Subsystem Synchronization, 3GPP GERAN1

GP-140603, “GSM Evolution for cellular IoT - BCCH overview”, GERAN#63, source Ericsson

GP-140602, “GSM Evolution for cellular IoT – SCH overview”, GERAN#63, source Ericsson