# EC-GSM – Link-Level Performance of EC-PACCH

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One of the main objectives in FS_IoT_LC is to increase the coverage compared to existing GPRS services. In case of evolving GSM, the most straight forward way is to make use of blind repetitions. This is what has been proposed for the Extended Coverage GSM (EC-GSM) concept (earlier known as GSM Evolution), see [2].

In this contribution, the Link-Level performance of EC-PACCH down link with blind repetitions is provided, and it shows current repetition scheme can fulfill the -6.3 dB SNR performance requirement.

# EC-PACCH down link burst mapping with blind repetition

Figure 1. EC-PACCH down link mapping. CC1,CC2 (top),CC3, …, CC6 (bottom)

As Figure 1 shows, burst mapping is different for different coverage class [3].To achieve the -6.3 dB SNR performance requirement, 16 TDMA frames as shown in burst mapping of CC6 should be used.

Figure 2 elaborates the burst mapping of EC-PACCH down link [4] which occupies time slot 0 to time slot 3 in more details. Each radio block of EC-PACCH down link consists of four bursts. The four bursts of one radio block are mapped to the same time slot of four consecutive TDMA frames. Each burst will be repeated four times within one TDMA frame. So IQ accumulation can be applied to the four bursts which belong to the same TDMA frame.

Figure 2. Repetition pattern of EC-PACCH down link

# Simulations

## Simulation assumptions

Simulations have been run for EC-PACCH down link using 16 blind transmissions of one radio block.

The simulation assumptions from [3] have been followed.

Some of the parameters are listed in Table 1.

| Parameter | Value |
| --- | --- |
| MCS | EC-PACCH down link |
| Interference | Reference sensitivity |
| Channel | TU |
| Mobile speed | 1.2 km/h |
| Frequency hopping | Ideal frequency hopping1 |
| Band | GSM 900 |
| Impairments | Typical RX and TX. Frequency offset=0 Hz, 5Hz, 10Hz |
| FEFC | Range of possible frequency offsets detected [-108,108]Hz |
| Number of repetitions | 162 |
| Blind modulation detection | Ideal |
| CC | 6 |
| Note 1: Ideal frequency hopping means that frequency hopping happens every TDMA frame. Note 2: The unit of repetition is radio block. Each radio block of EC-PACCH down link consists of 4 bursts, so 16 repetitions need 64 time slots. |  |

Table 1. Simulation assumptions

For EC-PACCH down link, four time slots are used within one TDMA frame. The content of these four time slots are completely the same. So, IQ accumulation within one TDMA frame is used to improve the performance.

## Results

Figure 2. Performance of EC-PACCH down link

As can be seen from figure 2, when there’s a frequency offset of 3 Hz, the performance is around 3 dB better than the requirement; when the frequency offset is 30 Hz, there is negligible performance degradation; when the frequency offset is increased to 90 Hz, there’s around 0.7 dB performance degradation but the requirement can still be fulfilled with around 2 dB margin. Compared with EC-PCH/EC-AGCH, the performance of EC-PACCH down link is not so sensitive to frequency offset since only the four bursts within one TDMA frame are accumulated. Besides, continuous phase is guaranteed in the four accumulated bursts since they should be transmitted and received continuously at TX/RX side.

# Conclusions

For EC-PACCH down link, with current coding scheme and burst mapping scheme, the -6.3 dB SNR requirement can be fulfilled. The performance is not very sensitive to frequency offset, so the requirement for FEFC is not as stringent as the requirement for EC-PCH/EC-AGCH.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-140882, “GSM Evolution for cellular IoT – On using blind repetitions”, source Ericsson LM. GERAN#64.

GP-150317, “3GPP TR 45.820 V1.0.0 (2015-03)”.

TR 45 820 v121.