3GPP TSG-GERAN #69 	GP-160085

# Malta, 15-18 February, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 43.064 | CR | XXXX | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS, Definitions and general feature description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-01-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The basic feature blocks and definitions of EC-EGPRS are described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2b(new), 3.3.1, 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 CR 0003 TS 43.022 CR 0036 TS 43.064 CR 0091, 0092, 0093 TS 44.018 CR 1027, 1026 TS 44.060 CR 1617,1610,1614,1612,1613,1615 TS 45.001 CR 0079,0080, 0081 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 TS 45.010 CR 0065 TS 48.058, CR 0022 |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.060: "General Packet Radio Service (GPRS); Stage 2".

[3]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[4]	3GPP TS 44.004: "Digital cellular telecommunications system; Layer 1; General requirements".

[5]	3GPP TS 24.007: "Mobile radio interface signalling layer 3 General aspects"

[6]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol"

7	3GPP TS 44.060: "Radio Link Control/Medium Access Control (RLC/MAC) protocol".

8	3GPP TS 44.064: "General Packet Radio Service (GPRS); Logical Link Control (LLC)".

9	3GPP TS 44.065: "General Packet Radio Service (GPRS); Subnetwork Dependent Convergence Protocol (SNDCP)".

[10]	3GPP TS 45.001: "Physical layer on the radio path, General description".

[11]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[12]	3GPP TS 45.003: "Channel coding".

[13]	3GPP TS 45.004: "Modulation".

[14]	3GPP TS 45.005: "Radio transmission and reception".

[15]	3GPP TS 45.008: "Radio subsystem link control".

[16]	3GPP TS 45.010: "Radio subsystem synchronisation".

[17]	3GPP TS 43.246: "Multimedia Broadcast Multicast Service (MBMS) in the GERAN; Stage 2".

[18]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”

[19]	3GPP TS 24.008: “Mobile radio interface Layer 3 specification; Core network protocols; Stage 3”

2nd modification

## 3.1	Abbreviations

In addition to abbreviations in 3GPP TR 21.905 [1] and 3GPP TS 22.060 [2] the following abbreviations apply:

ARQ	Automatic Repeat reQuest

BCS	Block Check Sequence

BEC	Backward Error Correction

BH	Block Header

BTTI	Basic Transmission Time Interval

CC	Coverage Class

CCN	Cell Change Notification

CFCCH	Compact Frequency Correction Channel

CPAGCH	Compact Packet Access Grant Channel

CPBCCH	Compact Packet Broadcast Control Channel

CPCCCH	Compact Packet Common Control Channel

CPPCH	Compact Packet Paging Channel

CPRACH	Compact Packet Random Access Channel

CSCH	Compact Synchronization Channel

CS-i	GPRS Coding Scheme i

CU	Cell Update

DAS-i	EGPRS2 Downlink level A modulation and coding Scheme i

DBS-i	EGPRS2 Downlink level B modulation and coding Scheme i

DLMC	Downlink Multi Carrier

DTM	Dual Transfer Mode

EGPRS	Enhanced GPRS

EGPRS2	Enhanced GPRS phase 2

EC-EGPRS	Extended Coverage EGPRS

EC-SI	EC-EGPRS System Information

eTFI	Extended Temporary Flow Identity

FANR	Fast Ack/Nack Reporting

FBI	Final Block Indicator

FH	Frame Header

GGSN	Gateway GPRS Support Node

HCS	Header Check Sequence

HSR	Higher Symbol Rate

IR	Incremental Redundancy

LLC	Logical Link Control

MAC	Medium Access Control

MBMS	Multimedia Broadcast/Multicast Service

MCS-i	EGPRS Modulation and Coding Scheme i

MPRACH	MBMS Packet Random Access Channel

NSS	Network and Switching Subsystem

PACCH	Packet Associate Control Channel

PAGCH	Packet Access Grant Channel

PAN	Piggy-backed Ack/Nack

PBCCH	Packet Broadcast Control Channel

PC	Power Control

PCCCH	Packet Common Control Channel

PCS	PAN Check Sequence

PDCH	Packet Data Channel

PDTCH	Packet Data Traffic Channel

PDU	Protocol Data Unit

PFC	Packet Flow Context

PFI	Packet Flow Indentifier

PL	Physical Link

PPCH	Packet Paging Channel

PRACH	Packet Random Access Channel

PSI	Packet System Information

PTCCH	Packet Timing Advance Control Channel

p-t-m	point-to-multipoint

RLC	Radio Link Control

RTTI	Reduced Transmission Time Interval

SGSN	Serving GPRS Support Node

SNDC	Subnetwork Dependent Convergence

TA	Timing Advance

TBF	Temporary Block Flow

TFI	Temporary Flow Identity

TTI	Transmission Time Interval

UAS-i	EGPRS2 Uplink level A modulation and coding Scheme i

UBS-i	EGPRS2 Uplink level B modulation and coding Scheme i

USF	Uplink State Flag

3rd modification

### 3.2b	Definitions

Blind Physical Layer Transmissions: Predetermined number of logical channel specific blind physical layer transmissions used by Extended Coverage logical channels, EC-channels, to support extended coverage. The repetitions are sent blindly, without feedback from the receiving end. To maximize the processing gain at the receiver, phase coherency at the transmitter, between repetitions, are required. The requirement on phase coherency can differ between logical channels and also depending on the use of frequency hopping, see 3GPP TS 45.005 [14].

Coverage Class: Defines a coverage range interval supported in EC-EGPRS operation. [Four] Coverage Classes are defined where Coverage Class 1 (CC1) corresponds to the approximate coverage range interval of GPRS/EGPRS. To achieve the extended coverage range intervals associated with Coverage Class 2 (CC2), Coverage Class 3 (CC3) and Coverage Class 4 (CC4) a fixed predefined number of blind physical layer transmissions is applied per logical channel, see 3GPP TS 45.002 [11] and 3GPP TS 45.003 [12]. This number of blind physical layer transmissions may differ between logical channels for the same coverage class.

EC-EGPRS operation: An EC-EGPRS capable MS in a cell supporting EC-EGPRS is said to have enabled EC-EGPRS operation and shall always use EC channels.

Extended coverage: Coverage level exceeding the reference sensitivity and reference interference performance of GPRS/EGPRS, see 3GPP TS 45.005 [14].

Fixed Uplink Allocation: Static allocation (and assignment) of limited resources in the uplink over one or more TTIs, using one or more PDCH, that does not make use of USF based allocation.

4th modification

## 3.3	Network and mobile station capabilities

### 3.3.1	General

In addition to GPRS specific definitions which can be found in 3GPP TS 22.060 [2] and 3GPP TS 23.060 [3] the following apply.

When referring to radio resources (i.e. physical channels) provided by the network to the mobile station, the term "assignment" refers to granting of resources on a semi-static basis, whereas "allocation" refers to the dynamically changing permission to use those resources that have been "assigned" to it and are shared with other users.  An exception applies when granting resources using Fixed Uplink Allocation where only the term “allocation” is used. In this case, resources are assigned and allocated by the same message.

Multislot Capability: the capability of the mobile station to support Multislot Configurations.

Multislot Class: a value which implicitly determines the Multislot Capability of the mobile station.

Multislot Configuration: the set of receive and transmit timeslots assigned to the MS.

### 3.3.2	EGPRS mobile station

An EGPRS mobile station is a GPRS mobile station with additional capabilities for new radio access protocol features and new modulation and coding schemes. An EGPRS mobile station shall comply with GPRS requirements and the additional requirements defined for an EGPRS mobile station. The support of EGPRS is optional for the mobile station and the network.

An EGPRS mobile station may additionally indicate support for EGPRS2 in uplink and/or downlink direction. In this case an EGPRS mobile station supports additional modulation and coding schemes, and may also support higher symbol rate, see sub-clause 3.3.6. The support of EGPRS2 is optional for the mobile station and the network.

An EGPRS mobile station may additionally indicate the support of Reduced Latency. In this case an EGPRS mobile station may be assigned a TBF with FANR activated either in BTTI configuration or in RTTI configuration, see sub-clause 3.3.5. The support of Reduced Latency is optional for the mobile station and the network.

### 3.3.3	Dual Transfer Mode

In dual transfer mode, the mobile station is assigned resources providing an RR connection and one or more Temporary Block Flows on one or more physical channels. This feature is optional for the mobile station and the network. It is only applicable for a mobile station supporting GPRS, EGPRS or EGPRS2. Dual transfer mode is a subset of class A mode of operation, which is only possible if there is radio resource assignment co-ordination in the network.

### 3.3.4	Downlink dual carrier configuration

In a downlink dual carrier configuration, one or more PDCHs are assigned to a single MS on each of two different radio frequency channels on either the uplink or downlink, or both. On the downlink, radio blocks may be allocated on both radio frequency channels in any radio block period. On the uplink, radio blocks shall not be allocated on both radio frequency channels in any given radio block period.

NOTE:	A radio frequency channel in this context is defined by the frequency parameter(s) ARFCN for a non-hopping radio frequency channel or MA, MAIO and HSN for a hopping radio frequency channel.

A downlink dual carrier configuration shall support multislot configurations either for packet switched connections or dual transfer mode. For a Dual Transfer Mode capable MS which supports Downlink Dual Carrier, support of Downlink Dual Carrier configurations for Dual Transfer Mode is optional.

Downlink dual carrier is not supported in GPRS mode.

### 3.3.5	Reduced Latency TBF

A TBF applying Reduced Latency shall operate according to all EGPRS/EGPRS2 requirements, unless otherwise stated, with the Fast Ack/Nack Reporting procedure (see sub-clause 3.3.5.1). In addition, a TBF applying Reduced Latency is characterized by either RTTI configuration or BTTI configuration (see sub-clause 3.3.5.2).

#### 3.3.5.1	Fast Ack/Nack Reporting procedure

The Fast Ack/Nack reporting procedure (FANR) refers to the possibility to include, in a radio block for data transfer sent in one direction, piggy-backed ack/nack information relative to a TBF with FANR activated in the other direction.

This is achieved by inserting a fixed-size piggy-backed ack/nack (PAN) field in the radio block. When a PAN field is inserted, a suitable Puncturing Scheme variant for the modulation and coding schemes in use is chosen, so that the RLC data field and the PAN field fit together in the radio block along with the RLC/MAC header. The presence of the PAN field, is signalled by the PAN indicator bit in the RLC/MAC header. When this bit is set the receiver shall use the corresponding Puncturing Scheme variant of the CPS indicated in the RLC/MAC header to decode the RLC data field.

If a mobile station is assigned a TBF with FANR activated, all concurrent TBFs assigned to the mobile station shall have FANR activated irrespective of the RLC/MAC mode of each TBF (acknowledged mode, non-persistent mode or unacknowledged mode).

Fast Ack/Nack reporting is not supported in GPRS TBF mode.

#### 3.3.5.2	RTTI configuration

In RTTI configuration, a radio block consisting of four bursts is sent using two PDCHs, i.e. a PDCH-pair, in each of two consecutive TDMA frames. In RTTI configuration, the time to transmit a radio block is half of a basic radio block period.

NOTE: 	The term “basic radio block period” refers the time needed to transmit a radio block on one PDCH using BTTI configuration i.e. four TDMA frames, while the term “reduced radio block period” refers the time needed to transmit a radio block on a PDCH-pair using RTTI configuration, i.e. two TDMA frames.

In RTTI configuration, a downlink TBF assignment consists of a number of PDCH-pairs, each PDCH-pair comprising two PDCHs. In a downlink dual carrier configuration (see sub-clause 3.3.4), up to 8 PDCH-pairs may be assigned per TBF. In a single carrier configuration up to 4 PDCH-pairs may be assigned per TBF. Two PDCHs constituting a downlink PDCH-pair need not be contiguous.

In RTTI configuration, an uplink TBF assignment consists of up to four PDCH-pairs. The two PDCHs constituting an uplink PDCH-pair need not be contiguous.

The PACCH shall have the same TTI configuration as the TBF with which it is associated.

In each direction PDCH-pairs cannot be assigned so that they are partially overlapped, i.e. two different PDCH-pairs cannot have one PDCH in common.

For an uplink TBF in RTTI configuration USFs can be sent in one of two ways:

-	RTTI USFs: a USF is sent in one reduced radio block period, i.e. a USF is mapped on four bursts transmitted on a downlink PDCH-pair during two consecutive TDMA frames. The USF allocates resources for one or four uplink radio blocks in the next reduced radio block period(s), depending on the value of USF_GRANULARITY.

-	BTTI USFs: two USFs are sent in a basic radio block period. One USF is mapped on four bursts transmitted on the first PDCH of a downlink PDCH-pair during four consecutive TDMA frames. This USF allocates resources for one or four uplink radio blocks in the first 2 TDMA frames of the next basic radio block period(s), depending on the value of USF_GRANULARITY. A second USF is mapped on four bursts transmitted on the second PDCH of a downlink PDCH-pair during four consecutive TDMA frames. This USF allocates resources for one or four uplink radio blocks in the second 2 TDMA frames of the next basic radio block period(s), depending on the value of USF_GRANULARITY.

For each TBF, the network signals the corresponding TTI configuration (i.e. either basic or reduced) at TBF establishment/ reconfiguration. In case of RTTI configuration, the network also signals at uplink TBF establishment/reconfiguration the USF mode (i.e. either RTTI or BTTI) to be used for that TBF.

For each assigned uplink PDCH-pair, the network may signal at TBF establishment/reconfiguration the “corresponding downlink PDCH-pair”, i.e. the DL PDCH-pair where USFs and PACCH/D are monitored. The timeslot numbers of the PDCHs constituting an uplink PDCH-pair may be different from those of the PDCHs constituting the corresponding downlink PDCH-pair. If no indication is provided, the corresponding downlink PDCH-pair will be the one with the same timeslot numbers as the uplink PDCH-pair.

On a given downlink PDCH-pair all USFs shall be sent with the same USF mode.

For a given mobile station, in each direction, the same TTI configuration shall be used for all TBFs (and PACCHs) assigned with one or more PDCH(s) in common in that direction. If a TBF is assigned on completely different PDCH(s) the TTI used may be different.

For a given mobile station, different uplink TBFs may use different USF modes if assigned on completely different PDCH(s).

A PDCH may be shared between RTTI TBF and BTTI TBF (assigned to different mobile stations). Alternatively, both PDCHs forming a PDCH-pair may be assigned to only support RTTI TBFs.

RTTI configuration is not supported in GPRS TBF mode.

### 3.3.6	EGPRS2 mobile station

An EGPRS2 mobile station is an EGPRS mobile station supporting additional modulation and coding schemes on the downlink and/or the uplink. An EGPRS2 mobile station shall comply with GPRS requirements and the additional requirements defined for an EGPRS mobile station. The support of EGPRS2 is optional for the mobile station and the network.

EGPRS2 features can be supported independently in the downlink and in the uplink.

An EGPRS2 mobile station may additionally indicate the support of Reduced Latency. In this case an EGPRS2 mobile station may be assigned a TBF applying Reduced Latency, see sub-clause 3.3.5. The support of Reduced Latency is optional for the mobile station and the network.

#### 3.3.6.1	EGPRS2 in the downlink

A mobile station supporting EGPRS2 in the downlink supports additional modulation and coding schemes on the downlink.

##### 3.3.6.1.1	EGPRS2-A and EGPRS2-B in the downlink.

Two levels of support are defined for the EGPRS2 feature in the downlink: EGPRS2-A and EGPRS2-B. EGPRS2-A includes additional modulation and coding schemes. EGPRS2-B includes additional modulation and coding schemes as well as a higher symbol rate. For a MS, support of EGPRS2-B in the downlink implies support of EGPRS2-A in the downlink.

#### 3.3.6.2	EGPRS2 in the uplink

A mobile station supporting EGPRS2 in the uplink supports additional modulation and coding schemes on the uplink.

##### 3.3.6.2.1	EGPRS2-A and EGPRS2-B in the uplink

Two levels of support are defined for the EGPRS2 feature in the uplink: EGPRS2-A and EGPRS2-B. EGPRS2-A includes additional modulation and coding schemes. EGPRS2-B includes additional modulation and coding schemes as well as a higher symbol rate. For a MS, support of EGPRS2-B in the uplink implies support of EGPRS2-A in the uplink.

### 3.3.7	Downlink multi carrier configuration

In a downlink multi carrier configuration, one or more PDCHs are assigned to a single MS on each of up to 16 different downlink radio frequency channels, depending on the MS capability. On the downlink, radio blocks may be allocated on all assigned radio frequency channels in any given radio block period. On the uplink, radio blocks shall only be allocated on one radio frequency channel in any given radio block period.

NOTE:	A radio frequency channel in this context is defined by the frequency parameter(s) ARFCN for a non-hopping radio frequency channel or MA, MAIO and HSN for a hopping radio frequency channel.

The MS shall monitor the assigned PDCHs on the assigned and selected radio frequency channels in a DLMC configuration. Which radio frequency channels that are selected depends on the maximum carrier separation supported by the MS and the ARFCNs of the radio frequency channels in a given radio block period.

A MS indicating support for downlink multi carrier may optionally indicate support for non-contigous intra-band reception and /or inter-band receception. In case of inter-band reception radio frequency channels are assigned in two different frequency bands. Inter band reception is only supported in multiband operatation. In non-contigous intra-band reception the assigned radio frequency channels are grouped into two separate groups during each TDMA frame, where the carrier separation indicated by the maximum bandwidth applies in each group.For a MS in Downlink Multi Carrier configuration, fallback to reception of a single radio frequency channel, irrespective of the number of assigned radio frequency channels is performed with regular periodicity.

A downlink multi carrier configuration shall support multislot configurations for packet swiched connections.

Dual Transfer Mode is not supported in downlink multi carrier configurations.

Downlink multi carrier is not supported in GPRS mode.

### 3.3.9 	Extended Coverage EGPRS (EC-EGPRS)

#### 3.3.9.1	General

EC-EGPRS is an evolution of EGPRS providing a streamlined protocol implementation, reducing device complexity while supporting energy efficient operation with extended coverage compared to GPRS/EGPRS operation. EC-EGPRS also mandates the use of an improved security framework by both the network and the mobile station.

EC-EGPRS makes use of Fixed Uplink Allocation for allocating uplink resources for EC-PDTCHs and hence does not support USF based uplink allocation.

No simultaneous uplink and downlink packet transfer is supported.

A MS in EC-EGPRS operation always make use of EC-channels (e.g. EC-PDTCH), except for the FCCH. An EC-EGPRS MS shall also support RACH and AGCH, in addition to EC-RACH and EC-AGCH. RACH and AGCH can be used by the MS when in GPRS/EGPRS coverage range (CC1, see subclause 3.3.x.2) if indicated by the network. EC-channels are used in idle mode as well as in packet transfer mode.

To minimize the impact on uplink capacity from blind physical layer transmissions, up to four mobile stations, can be assigned to simultaneously transmit on the same physical channel on the uplink. The technique is referred to as overlaid CDMA and can be used on all uplink channels (EC-RACH, EC-PDTCH and EC-PACCH). Orthogonality between MSs is achieved through orthogonal codes, corresponding to a set of phase shifts to be applied on a per transmitted burst basis. The phase shifts shall, in sequence, be applied to the transmitted burst. Another approach to realize orthogonaility between MSs is through predefined frequency offsets. Different predefined frequency offsets are assigned to different MSs. The accumulation of the predefined frequency offset introduces desired phase shifts bewteen bursts. Overlaid CDMA shall be supported by the mobile station. The use of overlaid CDMA on EC-RACH is broadcasted in System Information (EC-SI).

An EC-EGPRS mobile station need not comply with GPRS requirements, but shall comply with EGPRS requirements unless otherwise stated.

An EC-EGPRS mobile station need only support EC-EGPRS operation. The mobile station can optionally also support GPRS, EGPRS and/or EGPRS2.

EC-EGPRS supports overload control using the Implicit Reject Status field within the EC-SCH (see 3GPP TS 45.002 [11]) which applies for the case where a mobile station has enabled EC-EGPRS operation and is accessing the network for the purpose of NAS signalling low priority (see 3GPP TS 24.008 [19]).

EC-EGPRS shall not be operated in reduced latency TBF mode, see subclause 3.3.5, nor shall it support the Fast Ack/Nack reporting procedure.

DTM is not supported in EC-EGPRS operation.

The EGPRS modulation and coding schemes for PDTCH are reused for EC-PDTCH. The EC-BCCH re-uses the same coding scheme as the BCCH. Modified coding schemes, or specific coding schemes, are defined for EC-SCH, EC-PCH, EC-AGCH and EC-PACCH, see 3GPP TS 45.003 [12].

The support of EC-EGPRS is optional for the mobile station and the network.

The presence of EC-SCH indicates that a cell supports EC-CCCH and therefore allows an EC-EGPRS mobile station to camp on it.

#### 3.3.9.2	Extended coverage

Extended coverage is achieved by blind physical layer transmissions in both uplink and downlink, and is defined as improved MS and BTS sensitivity and interference performance. The sensitivity level and interference performance is improved by [20 dB] compared to GPRS/EGPRS operation, see 3GPP TS 45.005 [11].

For some logical channels, the number of blind physical layer transmissions can vary depending on the coverage extension required. [Four] different Coverage Classes are defined, each one approximated with a level of extended coverage compared to GPRS/EGPRS operation (see table 3.3.X.2-1), denoted as CC1, CC2, CC3 and CC4 respectively. CC1 corresponds approximately to the coverage range of GPRS/EGPRS. In case of extended coverage (coverage beyond that of GPRS/EGPRS), a fixed predefined number of blind physical layer transmissions is applied per logical channel. This number of blind physical layer transmissions, used by CC2, CC3 and CC4, may differ between logical channels for the same Coverage Class. Different Coverage Classes can be used on uplink and downlink. Logical channels supporting operation in extended coverage are referred to as EC-channels.

Table 3.3.x.2-1. Approximate coverage level per Coverage Class compared to GPRS/EGPRS operation

| Coverage class | Approximate extended coverage level compared to GPRS/EGPRS [dB] |
| --- | --- |
| CC1 | 0 |
| CC2 | [0-TBD] |
| CC3 | [TBD-TBD] |
| CC4 | [TBD-20] |

It is mandatory for the MS to support all defined Coverage Classes for all EC-channels. The network shall support at least CC1 and CC4. The set of coverage classes supported on EC-CCCH is broadcasted in System Information (EC-SI).

#### 3.3.9.3	Energy efficient operation

Energy efficient operation is enabled by the use of relaxed mobility related requirements and optionally eDRX (see 3GPP TS 45.002 [11]) or Power Saving Mode (see 3GPP TS 23.682 [18]), PSM. A mobile station supporting EC-EGPRS may support eDRX and/or PSM, and shall support the use of relaxed mobility related requirements.

#### 3.3.9.4	Improved security

EC-EGPRS mandates the use of an improved security framework by both the network and the mobile station.

End of modifications