3GPP TSG-SA WG1 Meeting #72	S1-154343

Anaheim, California, 16-20 November 2015	(revision of S1-154145)

Title:	CriC: Ultra-reliable communications

Agenda Item:	8.3

Source:	Qualcomm Incorporated

Contact:	Jack Nasielski (jackn at qti dot qualcomm dot com)

Abstract: Text for SMARTER TR for Critical Communications

# Proposal

This document proposes text to fill in the skeleton of the Critical Communications (CriC) TR.  Specifically it provides text for the “Higher reliability and lower latency” section.

--- START MODIFIED CLAUSE ---

## 5.1	Higher reliability and lower latency

### 5.1.1	Description

### 5.1.2	Traffic scenarios

### 5.1.2.x	Traffic scenario - Ultra-reliable communications

In order to enable certain services related to ultra-highly reliable communications, a minimum level of reliability and latency is required to guarantee the user experience and/or enable the service initially. This is especially important in areas like eHealth or for critical infrastructure communications.

Mission critical communication services require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

Examples of mission critical services include:

- Industrial control systems (from sensor to actuator, very low latency for some applications)

- Mobile Health Care, remote monitoring, diagnosis and treatment (high rates and availability)

- Real time control of vehicles, road traffic, accident prevention (location, vector, context, low Round Trip Time RTT)

- Wide area monitoring and control systems for smart grids

- Communication of a critical information with preferential handling for public safety scenarios

Overall, mission critical services are expected to require significant improvements in end-to-end latency, ubiquity, security, robustness, and availability/reliability compared to UMTS/LTE/WiFi.

The 3GPP system shall support efficient multiplexing of mission critical traffic and nominal traffic.

The 3GPP system shall limit the duration of service interruption for mission critical traffic.

The 3GPP system shall support improved reliability and latency as defined in table 5.x.x.

Subject to regional regulatory requirements, the 3GPP system shall support a mechanism to provide end-to-end integrity and confidentiality protection for user data,

The 3GPP system shall provide significant improvements in end-to-end latency, ubiquity, security, and availability/reliability compared to UMTS/EPS/WiFi.

Table 5.x.x: Example mission critical use cases

| Sample use case | Description | Critical Requirements |
| --- | --- | --- |
| Substation protection and control | - Automates fault detection and isolation to prevent large scale power outage - For example, Merging Units (MUs) perform periodic measurements of power system components, and send sampled measurement data to a Protection Relay. When the Protection Relay detects a fault, it sends signals to trip circuit breakers. | - Latency: as low as 1 ms end-to-end - Packet loss rate: as low as 1e-04 - Transmission frequency: 80 samples/cycle for protection applications. 256 samples/ cycle for quality analysis & recording - Data rate: ~12.5Mbps per MU at 256 samples/cycle - Range: provide coverage to the substation |
| Smart grid system with distributed sensors and management | - A smart grid system aims at improving the efficiency of energy distribution and requires prompt reaction in reconfiguring the smart grid network in response to unforeseen events. | Performance requirements are derived from EC FP7 project METIS Deliverable D.1.1: - Throughput: from 200 to 1521 bytes reliably delivered in 8 ms, - One trip time latency between any two communicating points should be less than 8 ms for event-triggered message that may occur anytime. - Device density: dense urban hundreds of UEs per square km; urban: around 15 UEs per square km; populated rural: max 1 UE per squared km. |
| Public Safety | - Operation of first responders in case of fire or other kind of emergency situation. | Public Safety requires preferential handling of its traffic. |

--- END NEW CLAUSE ---