3GPP TSG-SA2 Meeting #134	S2-1907315

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1539 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SM context transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document is to address 1) Adress editor's notes: Editors note: It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF. Editors note: It is FFS if steps 6 and 9 can be optimised or changed to a modification of the N4 session. 2) align flows with CTSF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated SMF Service Context Transfer procedures in which the SM Context Transfer is performed indirectly via CTSF - Removed SM Context ID from step 4 Clarification for Nsmf_PDUSession_SMContextStatusNotify |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor's notes not resolved; not aligend with |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26.5, 5.2.8.2.8, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** FIRST CHANGE ***

### 4.26.5	SMF Service Context Transfer procedures

#### 4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts).

#### 4.26.5.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_SMContextStatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

#### 4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID.

2.	From Old SMF to AMF Nsmf_PDUSession_SMContextStatusNotify (SMF transfer indication, Old SMF ID, New SMF ID, PDU Session ID, SUPI, SM Context ID)

3.	The AMF waits until UE is in state CM-IDLE or CM-CONNECTED with RRC inactive state.

4.	AMF uses New SMF ID to select New SMF and sends Nsmf_PDU_Session_CreateSMContext request (PDU Session ID, Old SMF ID, SM Context ID in Old SMF, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication). The same PDU Session ID as received in step 1 is used.

5.	From New SMF to Old SMF SMF Nsmf_PDUSessionContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

6.	The Old SMF releases the N4 session.

7.	At the end of this step, New SMF has obtained Old SMF’s SM Context. The SM Context includes the IP address(es) in case PDU session is of type IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet.
7A: [Conditional] If the SM Context is directly transferred, from Old SMF to New SMF Nsmf_PDUSessionContextRequest response (SM Context). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet.
7B: [Conditional] If the SM Context is transferred via a CTSF, 7B.a, 7B.b, 7B.c, 7B.d apply
7B.a [Conditional]: Old SMF pushes the SM Context into the CTSF
7B.b [Conditional]: From Old SMF to New SMF Nsmf_PDUSessionContextRequest response (SM Context location)
7B.c [Conditional]: New SMF requests the SM Context from CTSF with the data received in step 7B.c 
7B.d [Conditional]: New SMF receives the SM Context from CTSF

8.	[Condtitional] If dynamic PCC is used for the PDU Session, the new SMF sets up a new policy association towards PCF

9.	New SMF establishes N4 session. The parameters from step 7 and, if applicable, step 8 are used.

10.		New SMF notifies Old SMF by sending Nsmf_PDUSession_SMContextStatusNotify about the status of the SMF context transfer.

11. From New SMF to AMF: Nsmf_PDU_Session_CreateSMContext response

12.	New SMF registers to UDM,

13.	New SMF subscribes to subscription changes for the UE

14.	[Conditional] Old SMF removes its policy association with PCF (triggered by step 7) Any changes to the QoS rules need to be sent to the UE when it becomes active. After Step 13 (Step 7 if there is no policy assiciation) the old SMF can relase any internal resources for the PDU session.

*** NEXT CHANGE ***

##### 5.2.8.2.8	Nsmf_PDUSession_SMContextStatusNotify service operation

Service operation name: Nsmf_PDUSession_SMContextStatusNotify.

Description: This service operation is used by the SMF to notify its consumers about the status of an SM context related to a PDU Session (e.g. PDU Session release due to local reasons within the SMF, PDU Session handover to a different system or access type, SM Context transfer triggered). The SMF may use this service operation to update the SMF derived CN assisted RAN parameters tuning in the AMF.

Input, Required: Status information.

Input, Optional: Cause, SMF derived CN assisted RAN parameters tuning.

Output, Required: Result Indication.

Output, Optional: None.

*** NEXT CHANGE ***

##### 5.2.8.2.10	Nsmf_PDUSession_ContextRequest service operation

Service operation name: Nsmf_PDUSession_ContextRequest.

Description: This service operation is used by the NF Consumer to request for SM Context (e.g. during EPS IWK, HO, SM Context transfer), or during mobility procedure with I-SMF changes or may be triggered by OAM.

Input, Required: SM Context ID, SM context type.

Input, Optional: Target MME Capability, PDU Session ID (include PDU Session ID when available).

Output, Required: SM Context Container.

Output, Optional:Small Data Rate Control Status.

The SM context type indicates the type of SM context to be requrested, e.g. PDN Connection Context ,5G SM Context or both. If the SM context type is PDN Connection Context, the SM Context included in the SM Context container is the PDN Connection Context. If the SM context type is all, the SM Context included in the SM Context container includes both the PDN Connection Context and the 5G SM Context.

Table 5.2.8.2.10-1 illustrates the SM Context that may be transferred between I-SMF(s) or between V-SMF(s) in home-routed roaming case.

Table 5.2.8.2.10-1: SM Context that may be transferred between I-SMF(s) or between V-SMF(s)

| Field | Description |
| --- | --- |
| SUPI | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |
| Trace reference | Identifies a record or a collection of records for a particular trace. |
| Trace type | Indicates the type of trace |
| OMC identity | Identifies the OMC that shall receive the trace record(s). |
| S-NSSAI | The requested S-NSSAI for the serving PLMN to the PDU Session. |
| HPLMN S-NSSAI | Requested S-NSSAI for the HPLMN to the Home-Routed PDU Session |
| Network Slice Instance id | The network Slice Instance information for the PDU Session |
| DNN | The associated DNN for the PDU Session. |
| AMF Information | The associated AMF instance identifier and GUAMI. |
| Access Type | The current access type for this PDU Session. |
| RAT Type | RAT Type used by the UE. |
| PDU Session ID | The identifier of the PDU Session. |
| H-SMF Information | The associated H-SMF identifier and H-SMF address for the HR PDU Session.( applies only for a V-SMF) |
| For each QoS Flow in the PDU Session: |  |
| 5G QoS Identifier (5QI) | Identifier for the authorized QoS parameters for the service data flow. |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability |
| GFBR | Guaranteed Flow Bit Rate (GFBR) - UL and DL |
| MFBR | Maximum Flow Bit Rate (MFBR) - UL and DL |
| Priority Level | Indicates a priority in scheduling resources among QoS Flows. |
| Averaging Window | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |
| Maximum Data Burst Volume | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |
| Reflective QoS Control | Indicates to apply reflective QoS for the SDF in the TFT. |
| QoS Notification Control (QNC) | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |
| Maximum Packet Loss Rate | Maximum Packet Loss Rate - UL and DL. |
| Mapped EPS Bearer Context for Each QFI to support interworking with EPS: |  |
| EPS Bearer Id | An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN |
| Mapped EPS Bearer QoS | ARP, GBR, MBR, QCI. |
| PGW-U tunnel Information | PGW-U S5/S8 GTP-U tunnel IP address and TEID information. |
| TFT | Traffic Flow Template |

Editor's note:	This table is known to be incomplete and to require updates. Some items listed in the table may be removed. This table requires also updates to cover all kind of SM contexts not limited to I-SMF and V-SMF.

*** END CHANGES ***