SA WG2 Meeting #S2-136AH	S2-2000355

13 - 17 January, 2020, Incheon Korea

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 72 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Unicast Link ID Changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.287 section 6.3.3.2 defines the procedure of link identifier update for a unicast link. In current version, it is not clear what are action of UE-1 after sending out link identifier update request. Option 1 is UE-1 shall stop sending any traffic using the old Layer-2 ID after sending out the link identifier update request, then when UE-2 receives the update request, UE-2 can just stop receiving traffic with the old Layer-2 ID. Option 2 is UE-1 can keep sending traffic using the old link identifier until it receives the link identifer update response fron UE-2. That would require UE-2 to keep receiving traffic with the old Layer-2 ID from UE-1 until it receives traffic with the new Layer-2 ID from UE-1. Recommendation is to go with option two since it is more efficient, better performance and already needs to be supported during the process of the update procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two sentences are added in section 6.3.3.2 to clarify the behaviors of UE-1 and UE-2 regarding sending and receiving traffic with the old link identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification that can lead to misleading implementation/stage 3 work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of Change * * * *

#### 6.3.3.2	Link identifier update for a unicast link

Figure 6.3.3.2-1 shows the link identifier update procedure for a unicast link. Due to the privacy requirements, identifiers used for unicast mode of V2X communication over PC5 reference point (e.g. Application Layer ID, Source Layer-2 ID and IP address/prefix) shall be changed over time as specified in clauses 5.6.1.1 and 5.6.1.4. This procedure is used to update the peer UE for a unicast link of the impending change of the identifiers used for this link before the identifier changes happen, to prevent service interruptions.

If a UE has multiple unicast links using the same Application Layer IDs or Layer-2 IDs, the UE needs to perform the link identifier update procedure over each of the unicast link.

Figure 6.3.3.2-1: Link identifier update procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.3.3.1.

1.	UE-1 decides the change of identifiers, e.g. due to the Application Layer ID change or upon expiry of a timer, and send a Link Identifier Update Request message to UE-2 before it changes the identifiers. After sending the Link Identifier Update request, UE-1 keeps sending data traffic to UE-2 with the old identifier until UE-1receives the Link Identifier Update Response from UE-2.

The Link Identifier Update Request message includes the new identifiers to use (including the new Application Layer ID, new Layer-2 ID, new IP address/prefix if IP communication is used). The new identifiers should be cyphered to protect privacy.

NOTE:	The timer is running on per Source Layer-2 ID.

2.	UE-2 responds with a Link Identifier Update Response message. Upon reception of the message, UE-1 and UE-2 start to use the new identifiers for the data traffic. UE-1 shall receive traffic on its old Layer-2 ID until it receives the Link Identifier Update Response message from UE-2. UE-2 continues to receive traffic with the old Layer-2 ID from UE-1 until UE-2 receives traffic with the new Layer-2 ID from UE-1.

The V2X layer of each UE passes the PC5 Link Identifier for the unicast link and the updated Layer-2 ID (i.e. source Layer-2 ID for UE-1 while destination Layer-2 ID for UE-2) down to the AS layer. This enables the AS layer to update the provided Layer-2 ID for the unicast link.

# * * * * End Change * * * *