3GPP TSG-SA WG2 Meeting #136-AH	S2-2001054

# Incheon, Korea, January 13-17 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2066 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UCMF provisioning correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification of the UCMF provisioning is incorrect and inconsistent. See also S2-2000837. |  |  |  |  |  |  |  |  |
| Summary of change: |  | provisioning of UCMF corrected, and added the essential capability to add new TAC values of a UE radio capability ID (as the UE models may require more TAC values if they become more and more successful, or the Ue manufacturer reuses the ID for more UE models). Note that the "modify " behaviour is already supported at stage 3 (29.522/29.122) for NEF/SCEF based provisioning of UCMF and also in 23.682. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | overall incorrect and incomplte and incoherent specification and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First proposed Changes

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF, SMF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_NetworkStatus | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |

More proposed Changes

#### 5.2.6.17	Nnef_UCMFProvisioning service

##### 5.2.6.17.1	General

This service is for allowing external party to provision the UCMF with UCMF dictionary entries for Manufacturer-assigned UE Radio Capability IDs.

##### 5.2.6.17.2	Nnef_UCMFProvisioning_Create operation

Service operation name: Nnef_UCMFProvisioning_Create

Description: The consumer creates a UCMF dictionary entry (or more entries) for a Manufacturer-assigned UE Radio Capability ID via the NEF. For each UE Radio Capability ID provided, a UE radio access capability set and the related UE model(s) TAC value(s) the UE radio capability ID applies to is provided.

Inputs (required): (list of) [UE Radio Capability ID, UE radio access capability set, (List of) TAC value(s)].

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 5.2.6.17.3	Nnef_UCMFProvisioning_Delete operation

Service operation name: Nnef_UCMF_Provisioning_Delete

Description: The consumer deletes a UCMF dictionary entry for a Manufacturer-assigned UE Radio Capability ID via the NEF. The consumer provides a (list of) UE radio capability ID value(s) to be deleted or it may provide the TAC values for which the associated UE radio capability ID entries shall be no longer used.

Inputs (required): UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be deleted or Type Allocation Code(s) that no longer use associated UE radio Capability ID(s).

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 5.2.6.17.x	Nnef_UCMFProvisioning_Update operation

Service operation name: Nnef_UCMF_Provisioning_Update

Description: The consumer updates the list of TAC values a UCMF dictionary entry(or a list of entries) applies to for a Manufacturer-assigned UE Radio Capability ID via the NEF. For each UE Radio Capability ID provided, (a list of) UE model(s) TAC value(s) to be added or removed to the related UCMF entry is provided.

Inputs (required): Update Type (one of “Add TAC Values” or “Remove TAC Values”) and:

- If Update Type is “Add TAC Values”, the (list of) UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be updated and the related additional (list of) Type Allocation Code(s), or

- If Update Type is “Remove TAC Values”, the (list of) UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be updated and the related (list of) Type Allocation Code(s) to be removed.

Inputs (optional): None

Outputs (required): None.

Outputs (optional): None.

More proposed Changes

#### 5.2.18.1	General

The following table illustrates the UCMF Services.

Table 5.2.18.1-1: NF services provided by UCMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nucmf_Provisioning | Create | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
| Nucmf_UECapabilityManagement | Resolve | Request/Response | AMF |
|  | Assign | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF |
|  | Unsubscribe | Subscribe/Notify | AMF |
|  | Notify | Subscribe/Notify | AMF |

More proposed Changes

#### 5.2.18.2	Nucmf_Provisioning service

##### 5.2.18.2.1	Nucmf_Provisioning_Create service operation

Service operation name: Nucmf_Provisioning_Create.

Description: The consumer creates a UCMF dictionary entry(or more entries) for a Manufacturer-assigned UE Radio Capability ID(s). For each UE Radio Capability ID provided, a UE radio access capability set and the related UE model(s) TAC value(s) the UE radio capability ID applies to is provided.

Inputs (required): (list of) [UE Radio Capability ID(s), set(s) of UE radio access capability set, (list of) Type Allocation Code(s) value(s)].

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 5.2.18.2.2	Nucmf_Provisioning_Delete service operation

Service operation name: Nucmf_Provisioning_Delete.

Description: The consumer deletes a UCMF dictionary entry(s) for a Manufacturer-assigned UE Radio Capability ID(s). The consumer provides a (list of) UE radio capability ID value(s) to be deleted or it may provide the TAC values for which the associated UE radio capability ID entries shall be no longer used.

Inputs (required): UE Radio Capability ID(s) of the UCMF dictionary entry(s) to be deleted or Type Allocation Code(s) that no longer use associated UE radio Capability ID(s).

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 5.2.18.2.1	Nucmf_Provisioning_Update service operation

Service operation name: Nucmf_Provisioning_Update.

Description: The consumer updates the list of TAC values a UCMF dictionary entry (or a list of entries) applies to for a Manufacturer-assigned UE Radio Capability ID. For each UE Radio Capability ID provided, (a list of) UE model(s) TAC value(s) to be added or removed to the related UCMF entry is provided.

Inputs (required): Update Type (one of “Add TAC Values” or “Remove TAC Values”) and:

- If Update Type is “Add TAC Values”, the (list of) UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be updated and the related additional (list of) Type Allocation Code(s), or

- If Update Type is “Remove TAC Values”, the (list of) UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be updated and the related (list of) Type Allocation Code(s) to be removed.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

End of proposed Changes