3GPP TSG-SA/WG2 Meeting #136-AH	S2-2001715

# Incheon, Korea, 2020-01-13 -- 2020-01-17	(e-mail revision 4 of S2-2001555)

Source:	ZTE

Title:	Solution to KI#1: Discovery of Edge Application Server

Document for:	Agreement

Agenda Item: 	8.5

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes solution to discover the address of Edge Application Server by using existing DNS solution.

# 1	Introduction

This contribution proposes a solution for discover the address of Edge Application Server by using existing DNS solution. In this solution, the AF request information is sent to AMF so the AMF can insert or relocate to a proper I-SMF supporting the requested DNAI, so the SMF can insert an I-UPF in data plane path before the UE initiate the DNS discovery. The I-UPF detect the DNS query towards the identified FQDN and route the DNS query message to Edge Hosting Environment to discover the address of Edge Application Server. There is no enhancement on DNS discovery mechanism.

# 2	Proposal

It is proposed to agree the following changes in TR 23.748.

************* Start Changes *************

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| Solution #X: SMF/I-SMF selection based on DNAI | X |  |  |  |

************* Next Changes *************

## 6.X	Solution #X: SMF/I-SMF selection based on DNAI

### 6.x.1	Solution description

#### 6.x.1.1	General

This solution addresses the Key Issue #1: Discovery of Edge Application Server. This solution is applicable for both Session Breakout Connection Model and Multiple PDU sessions Connection Model defined in clause 4.2.

In this solution it is assumed DNS server is deployed in Edge Hosting Environment. The DNS server is used to discover the Edge Application Server in the Edge Hosting Environment.

The Application Functions uses AF influence traffic mechanism to activate the traffic routing towards the Edge Hosting Environment. The AF request information is stored in the UDR as DataSet "Application Data" and Data Subset " AF traffic influence request information" as described in TS 23.502 [X]. In this solution the UDR notifies AM-PCF about the AF traffic influence request information and AM-PCF further sends the AF traffic influence request information to AMF. So the AMF can select SMF or I-SMF based on the requested DNAI(s).

Editor’s note:	It is FFS why the AMF would need the DNAI to select a (I-)SMF whereas per R16 it can select the (I-)SMF based on the TA currently serving the UE.

The SMF then configures the I-UPF to route the DNS query message towards the Edge Hosting Environment.

The AF traffic influence request information includes FQDN or DNS server address as traffic descriptor. The SMF configures the I-UPF so when the UE sends DNS query with the target FQDN or target DNS server address, the I-UPF can route the DNS query message towards the DNS server in Edge Hosting Environment to discover the EAS IP address.

Editor’s note:	How to support the case when an edge computing hosting environment has no connectivity with the central data network.

### 6.X.2	Procedures

Figure 6.x.2-1 SMF selection based on AF influence Request

This procedure is example to illustrate how to select an SMF based on AF influence traffic routing request. After the PDU Session is established the SMF activate the traffic routing towards the Edge Hosting Environment. When the UE sends DNS query request, the I-UPF can detect the DNS query message targeting particular FQDN or DNS Server address and route the DNS query request towards Edge Hosting Environment.

- The AM-PCF subscribe DataSet "Application Data" and Data Subset "AF traffic influence request information" in the UDR.

- The AF traffic influence request information storage in the UDR is created/modified/released by the NEF per AF request. The Application Function can be deployed in the Edge Hosting Environment or in the Central Environment.

- The UDR sends the Nudr_DM_Notify to AM-PCF, including the AF traffic influence request information. The AM-PCF stores the AF traffic influence request information locally.

- The UE perform registration procedure in the network.

- The AMF establishes AM association towards the AM-PCF. If the UE is the targeting of AF request the AM-PCF sends Npcf_AMPolicyControl_Create Response includes the AF traffic influence request information. The AF traffic influence request information contains the S-NSSAI, DNN, the DNAI list requested by AF, etc.

- The UE establishes PDU Session with the S-NSSAI and the DNN.

- The AMF performs SMF selection for the PDU Session, taking into account the DNAI(s) requested by AF. The NRF provides the candidate SMF profiles for S-NSSAI and the DNN to AMF. The SMF profiles includes the supported DNAI list. The AMF selects an SMF which supports the DNAI(s) requested by the AF. If AF requests a list of DNAI, the AMF may determine which DNAI(s) needs to be taken into account based on the UE location. In case of ETSUN, the AMF selects an I-SMF which supports the DNAI(s) requested by AF.

- The AMF continue the PDU Session establishment procedure. For Multiple PDU Session connectivity model the SMF selects the PSA UPF based on the DNAI(s) of interest for this PDU Session.

- The SMF activates the traffic routing influence towards the Edge Hosting Environment. For Session Breakout Connection Model the SMF selects I-UPF acting as UL-CL or BP based on the DNAI(s) of interest for this PDU Session. The I-UPF is configured with rules to route the DNS query message including the requested FQDN or requested DNS Server address towards the Edge Hosting Environment.

Editor’s note:	It is FFS whether “The I-UPF is configured with rules to route the DNS query message including the requested FQDN” requires changes to N4.

- The UE sends DNS query request message over the user plane path.

- The I-UPF detects based on Traffic Descriptor the DNS query request message including the requested FQDN or requested DNS Server address and routes the DNS query request message towards the DNS server in Edge Hosting Environment.

In case of the Traffic Descriptor is FQDN the I-UPF/local PSA replaces the original DNS server address with the address of DNS server in Edge Hosting Environment. The I-UPF/local PSA keeps a context during the DNS request handling in order to be able when sending the DNS answer to the UE to regenerate the address of the DNS server that the UE had put in its request.

In case of the Traffic Descriptor is DNS Server address the I-UPF/local PSA need not replace the target DNS server address.

- The DNS server responds with address information of the Edge Application Server. If DNS Server Address had been replaced in step 11, the I-UPF/local PSA replaces the source address back with the original DNS server address that had been used by the UE.

- The I-UPF forwards the DNS query response to the UE.

If the AF has not provided traffic description information of the selected Edge Application Servers in step 2, the AF may further update the AF traffic influence request information in order to activate the traffic routing in I-UPF towards the selected Edge Application Server.

### 6.X.3	Impacts on Existing Nodes and Functionality

AM-PCF:

- Subscribe the AF traffic influence request modification in UDR.

- Receives the AF traffic influence request from the UDR.

- Sends the new/updated AF traffic influence request to AMF.

AMF:

- Receives the new/updated AF traffic influence request from the AM-PCF.

- Select I-SMF or SMF supporting the requested DNAI(s) based on AF traffic influence information received from the AM-PCF.

NRF:

- Store the supported DNAI in the service profile of the I-SMF/SMF.

UPF:

- When Traffic Descriptor is FQDN the I-UPF/local PSA replaces the original DNS server address in DNS query message with the address of DNS server in Edge Hosting Environment. This implies N4 and SMF Change.

- The I-UPF/local PSA replaces the source address in DNS query response with the original DNS server address.

*************** End Changes ***************