3GPP TSG-SA2 Meeting #137-E	S2-2001816

# Online, , 24th Feb 2020 - 27th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1231 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EPS Fallback event transporting within IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson (?),Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  |  | Date: |  | 2019-11-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT3 send LS (S2-2000010/ C3-194438) on separating scenarios for IMS Callback. CT3 requested SA2 to study possible solution for IMS layer to identify EPS Fallback. Discussion paper S2-2001813, proposes that the P-CSCF indicates EPS fallback failure as cause when terminating SIP sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the PCF reports that resources were released because an EPS fallback failure occurred, the P-CSCF will terminate the related SIP session and may, based on local configuration, include EPS fallback failure as termination cause in related outgoing SIP messages towards other IMS nodes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for the S-CSCF to identify EPS fallback failures as cause for call failures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex Y.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Rev3: Updates according to proposals in discussion paper S2-2001813. |  |  |  |  |  |  |  |  |

***** Start of Change *****

# Y.X	Reporting of EPS Fallback Failures

If the PCF reports that resources were released because an EPS fallback failure occurred (using procedures defined in TS 23.503 [95]), the P-CSCF will terminate the related SIP session and may, based on local configuration, include EPS fallback failure as termination cause in related outgoing SIP messages towards other IMS nodes.

***** End of Changes *****