3GPP TSG SA WG6 Meeting #3	S6-150202

San Jose del Cabo, Mexico, 13-17 April 2015

Title:	Considerations for MCPTT between public safety and LTE commercial Network

Agenda Item:	6.2

Source:	KT Corp.

Contact :	Sangkwon Ahn. kwon.ahn@kt.com

Chungwoo Hwang. cwhwang@kt.com

Document for:	Approval

Abstract: This document outlines the considerations MCPTT services between public safety network and LTE commercial network with or without eMBMS support. For the architecture of IMS  or SIP core based, this paper proposes that MCPTT architecture shall consider PTT service between public safety and LTE commercial network.

# Introduction

Public safety network may interwork with commercial network because of coverage issue and network redundancy. In this case MCPTT shall be serviced in public safety network as well as LTE commercial network. The typical methods such as roaming and RAN sharing are for the interworking with both networks. However, if public safety network have its own access and core network, the roaming will be prefer method for interworking between public safety and commercial network. The roaming type may be in many forms depending on the service provider’s environment. But regardless of roaming type, it is necessary to consider the architecture of the MCPTT service available.

# MCPTT Scenarios

## MCPTT service in Public Safety network

In public safety network, each UE in relevant area communicates with broadcast and UEs outside relevant area can communicate with unicast.

The following functionalities and capabilities are expected to be needed for MCPTT in public safety network ;

- BMSC : establish eMBMS session of the  relevant area

- Dispatcher : Establish MCPTT Group session

- PTT Server : Manages receiving(communication)  mode of each UE by session information

Broadcast media burst(voice and  video) to UE within relevant area

- UE : Make a decision of exsisting eMBMS session,  then notify to PTT server when communication

mode changes

## MCPTT service interworking with eMBMS in PS network

When MCPTT service is provide between public safety network having eMBMS and commercial network not having eMBMS, MBMS gateway in public safety network can accommodate eNodeB in commercial network.

MCPTT operations with eMBMS are as following ;

- eNB in visited network can link with MBMS gateway  in public safety network.

- Establish eMBMS session for MCPTT broadcast in public safety network.

- Likewise the preceding process as 2.1, MCPTT can be broadcasted in visited network.

## MCPTT service non-interworking with eMBMS in PS network

If MBMS gateway can’t accommodate directly eNodeB in commercial network, MCPTT with unicast mode  is services by way of  linking P-GW and S-GW.

- PGW in home network link with SGW in visited network.

- UE in visited network can received MCPTT by unicast  mode

# Proposal

Currently under discussion in relation to the architecture which is the IMS based or SIP Core based,

It shall include

- the ability to provide smoothly MCPTT service between public safety network and commercial network.

- and MCPTT architecture considering the roaming environment of service providers.