3GPP TSG-CT WG1 Meeting #129-e	C1-21mint

- Electronic meeting; 19 – 23 April 2021

Source:	LG Electronics (Rapporteur)

Title:	FS_MINT-CT moderated e-mail discussion

Agenda item:	17.2.9

Document for:	Discussion

# 1. Introduction

This document includes a request for companies to provide their opinion on the Key Issues of FS_MINT-CT. The detailed analysis and rationale of each questions are described in a discussion paper in C1-210952.

The purpose of the moderated discussion is that gathering company’s view on the key questions regarding the Key Issues and solutions addressed in 3GPP TR 24.811, and finding a potential way forward based on the companies’ preferences on the conclusion efficiently.

Tentative time plan for the moderated e-mail discussion is shown below:

| Moderated e-mail discussion starts | 29 March (Monday) 15:00 UTC |
| --- | --- |
| Moderated e-mail discussion ends | 2 April (Friday) 15:00 UTC |
| Document including the summary submitted by rapporteur | 5 April (Monday) |
| MINT CC#2 for discussing the result and any other contributions to CT1#129e | TBD (between 5 Apr and 9 Apr) |

# 2.	Discussion

NOTE:	the examples listed in the questions hereafter are non-exhaustive, and other options are not excluded.

## 2.1	Overall way forwards

Q.1: Please indicate whether you support the RAN sharing based approach to solve the issues in case of Disaster Condition, and if you do support it, please indicate whether you think that other enhancements are needed.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson does not support the RAN sharing based approach to solve the issues in case of Disaster Condition. Reasons: The RAN sharing based approach requires that PLMN without Disaster Condition needs to understand core network deployment of PLMN where the Disaster Condition might happen in future, and PLMN with Disaster Condition needs to understand RAN deployment of PLMN without Disaster Condition, since: - to setup the N2 connection to appropriate AMF, NG-RAN node of PLMN without Disaster Condition needs to know AMF(s) of PLMN where the Disaster Condition might happen in future serving the area of the NG-RAN node; and - to trigger appropriate RAN nodes to become shared RAN nodes, AMF of PLMN with Disaster Condition needs to know which area the RAN nodes of PLMN without Disaster Condition serve. Given that the disaster roaming use case addresses PLMNs not normally cooperating (PLMN without Disaster Condition in the list of forbidden PLMNs of the UE of PLMN with Disaster Condition), it is not reasonable to expect such deployment information sharing, and updates of configuration of the network nodes, between the involved PLMNs, solely for disaster roaming functionality. |
| vivo | No Reason： We believe RAN sharing based approach is an implementation based solution to cope with DC，i.e. the people who manage the PLMN could trigger appropriate RAN nodes to become shared RAN node when DC happens. |
| OPPO | Do not support AN sharing as a mandatory solution Rationale: There is no specific requirement. And if a number of OPerators wish to do that it is still within optional possibilities. |

Q.2: Please indicate whether or not the pre-configuration and/or provision of information on the UE are needed for disaster roaming even before the Disaster Condition occurs (e.g. Information needed for network selection)

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that pre-configuration and provision of information on the UE is not needed for disaster roaming even before the Disaster Condition occurs. Reasons: Some use cases cannot be addressed solely by pre-configuration and provision of information before the Disaster Condition occurs, e.g.: Use-case-A) A UE of HPLMN D powers up for the 1st time with USIM which contains no or obsolete (pre)configured information related to Disaster Condition (e.g. because the USIM has not been used for some time), does not discover any available and allowable PLMN but discovers that PLMN D has Disaster Condition. There is only one available PLMN (called PLMN A) and PLMN A is in UE's list of forbidden PLMNs. PLMN A offers Disaster Roaming to UEs of PLMN D. However, the UE is unaware of PLMN A offering Disaster Roaming to UEs of PLMN D since such information is not (pre)configured in the UE and thus the UE does not use disaster roaming in PLMN A in automatic selection. Use-case-B) A UE of HPLMN H arrives into a new country. UE's USIM is not configured with information applicable in the country yet. Upon landing, the user powers up the UE. The UE discovers one available and allowable PLMN (called PLMN A). The UE selects PLMN A, attempts to register for non-disaster roaming and is rejected with non-integrity protected 5GMM cause #11. The UE adds PLMN A into list of forbidden PLMN. PLMN A offers Disaster Roaming to UEs of PLMN D which is allowable PLMN for the UE. However, the UE is unaware of PLMN A offering Disaster Roaming to UEs of PLMN D since such information is not (pre)configured in the UE and thus the UE does not use disaster roaming in PLMN A in automatic selection. It is better to rely on usage of information broadcast by the PLMN without Disaster Condition, which: - enables the use cases enabled by the pre-configuration and provision of information before the Disaster Condition occurs; and - also enables the use cases above. |
| vivo | Yes |
| OPPO | Support having pre-configuration info but pre-configuration should only be for home country. |

Q.3: Please indicate whether the PLMN with Disaster Condition or the PLMN without Disaster Condition can provide information regarding Disaster Condition via non-3GPP access, if non-3GPP access is available.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson does not support providing information regarding Disaster Condition via non-3GPP access, neither by he PLMN with Disaster Condition nor by the PLMN without Disaster Condition. Reasons: The Disaster Condition impacts 3GPP access (as TR 24.811 states "The network functions except one or more RAN nodes of the PLMN with Disaster Condition can be assumed to be still operational. One or more RAN nodes of the PLMN with Disaster Condition are non-operational."). Thus, UEs impacted by the Disaster Condition support 3GPP access. Such UEs do not necessarily support non-3GPP access. Therefore, the solution has to be provided via 3GPP access. |
| vivo | No. Reason: the UE with available non-3GPP access is not in the scope of MINT. |
| OPPO | Do not support use of non-3GPP access in any circumstances when there is Disaster Condition In any case if UE can keep the existing services over N3GPP access if N3GPP access is available. |

Q.4: Please indicate whether the higher priority PLMN search should be suppressed or modified when the UE is camped on a PLMN providing disaster roaming.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view that the higher priority PLMN search should not be suppressed when the UE is camped on a PLMN providing disaster roaming. Ericsson is OK to fix periodicity of the higher priority PLMN search. |
| vivo | Same as Ericsson. |
| OPPO | NO, search for higher priority PLMN should continue |

## 2.2	Issues regarding notification of Disaster Condition

Q.5 (KI#1): In addition to the indication that Disaster Condition applies to the PLMN with Disaster Condition, please indicate whether and what additional information is provided to the UE (by the PLMN with Disaster Condition, or by a PLMN without Disaster Condition) when the indication that Disaster Condition applies to the PLMN with Disaster Condition, is provided.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is: - the PLMN without Disaster Condition (called PLMN A) indicates to the UEs in SIB or PWS message, one or more PLMN IDs of PLMNs with Disaster Condition for which PLMN A is able to provide disaster roaming; and - the PLMN with Disaster Condition does not provide to the UE any information; when the indication that Disaster Condition applies to the PLMN with Disaster Condition, is provided. |
| vivo | Yes The recommended disaster roaming PLMN list shall be provided to the UE along with the indication that Disaster Condition applies to the PLMN with Disaster Condition. |
| OPPO | OPPO supports pre-configuration information, pre-configured by HPLMN only. PLMN with Disaster Condition (DC) could not provide any info, via broadcast or otherwise as PLMN is already in DC. VPLMN that accepts inbound roamers, i.e supports Disaster Roaminh, can provide information to inbound roamers, if VPLMN considers necessary. |

Q.6 (KI#2): Please indicate whether the notification of Disaster Condition between PLMN with Disaster Condition and PLMN providing disaster roaming should be left out of 3GPP scope?

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is: - when the notification of Disaster Condition between PLMN with Disaster Condition and PLMN providing disaster roaming is provided directly, this should be left out of scope of 3GPP. - when the notification of Disaster Condition between PLMN with Disaster Condition and PLMN providing disaster roaming is provided via CBE, the communication between the PLMN with Disaster Condition and CBE should be left out of scope of 3GPP. However, it needs to be specified which entity is responsible for involvement of the government in decision on initiation and termination of the Disaster Condition, as expected in Disaster Condition definition in TS 22.261: ------------- Disaster Condition: This is the condition that a government decides when to initiate and terminate, e.g. a natural disaster. When this condition applies, users may have the opportunity to mitigate service interruptions and failures. ------------- |
| vivo | Yes |
| OPPO | YES, notification of Disaster Condition between PLMN with Disaster Condition and PLMN providing disaster roaming is out of scope of 3GPP. |

Q.7 (KI#3): Please indicate what information does the UE receive from a PLMN providing disaster roaming? (e.g. list of PLMNs with Disaster Condition) And how this information is provided? (e.g. as explicit indication in SIB, access identity)

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that the UE receives from a PLMN providing disaster roaming (called PLMN A) a list of one or more PLMN IDs of PLMNs with Disaster Condition for which PLMN A is able to provide disaster roaming. The information is included in SIB or PWS message. |
| vivo | The PLMN providing Disaster Roaming Service shall provide a timer to indicate the wait time value before attempting to register to the PLMN that is recovered from DC and the recommended disaster roaming PLMN list to the UEs. The information can be provided via NAS signalling or RRC or SIB. |
| OPPO | - minimal indications over SIBs as that is least secure; - Once inbound roamers are accepted by the VPLMN, the VPLMN can provide more info and whatever is necessary (tbd). Such info should be valid only for that VPLMN. |

Q.8 (KI#6): Please indicate whether the UE needs to be deregistered from the PLMN providing disaster roaming when Disaster Condition is over. In other words, Please indicate whether inter-PLMN mobility from PLMN providing disaster roaming to PLMN previously with Disaster Condition can be supported or not.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that inter-PLMN mobility from PLMN providing disaster roaming to PLMN previously with Disaster Condition needs to be supported. Reason: - inter-PLMN mobility enables preserving the PDU sessions and thus no disruption to services used by the UE and less 5GSM signalling. |
| vivo | Please indicate whether the UE needs to be deregistered from the PLMN providing disaster roaming when Disaster Condition is over: Yes In other words, Please indicate whether inter-PLMN mobility from PLMN providing disaster roaming to PLMN previously with Disaster Condition can be supported or not.: No |
| OPPO | NO, UE does not need to de-register. Keep it simple. Serving VPLMN can reject UE on next request for resources or next registration update. UE will then naturally attempt automatic PLMN reselection and get back to its highest priority or original PLMN. |

Q.9 (KI#6): Please indicate whether the UE in idle mode should return to the PLMN previously with Disaster Condition immediately or not. (i.e. whether needs to be paged in order to return immediately, or return later by searching higher priority PLMN)

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that it is sufficient that the UE in idle mode returns to the PLMN previously with Disaster Condition: - when searching for higher priority PLMN; or - when attempting to transit from 5GMM-IDLE mode to 5GMM-CONNECTED mode. NOTE: Usage of paging to trigger the UE to transit from 5GMM-IDLE mode to 5GMM-CONNECTED mode is possible but up to decision of PLMN providing disaster roaming. |
| vivo | Prefer “return later by searching higher priority PLMN” Can live with paging. |
| OPPO | NO, same answer as for Q8 |

## 2.3	Issues regarding network selection / registration

Q.10 (KI#4): Does the AMF of the PLMN providing disaster roaming need to distinguish between a normal registration and a registration for disaster roaming? If yes, is this distinction achieved via an explicit indication from the UE, or using based on PLMN ID in GUTI, SUCI, or other indication)?

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that the AMF of the PLMN providing disaster roaming needs to distinguish between a normal registration and a registration for disaster roaming. The distinction is achieved based on: - the UE providing PLMN ID identifying a PLMN with Disaster Condition. The UE provides the PLMN ID in 5G-GUTI, in SUCI or in a new IE. NOTE: The new IE is needed when the UE has not registered in the PLMN with Disaster Condition. - AMF checking that the PLMN ID in 5G-GUTI, in SUCI or in the new IE identifies a PLMN with Disaster Condition in the area served by the NG-RAN node serving the UE. |
| vivo | No |
| OPPO | NO. Serving PLMN that accept Disaster roamers should accept registration request as a) that PLMN is acting with the other PLMN experiencing disaster condition. PLMN ID in GUTI/SUCI can be used to distinguish the disaster roaming UEs. |

Q.11 (KI#4): Please indicate what level of granularity of area with Disaster Condition, if any, should be provided to the UE? (i.e. cell level, TA level, polygon coordinates..)

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that no information on area with Disaster Condition needs to be provided to the UE. Reason: There is no stage-1 requirement requiring providing the disaster area to the UE. Furthermore, it is possible to solve the stage-1 requirements without providing the disaster area to the UE, by the disaster area of PLMN D being used by PLMN A as follows: - if a UE of PLMN D is inside the disaster area of PLMN D, PLMN A provides the disaster roaming to the UE. - if a UE of PLMN D is outside the disaster area of PLMN D, PLMN A does not provide the disaster roaming to the UE and triggers the UE to move back to PLMN D. |
| vivo | cell level or TA level |
| OPPO | None. Just let UE be in VPLMN as an inbound roamer. So normal registration update procedures takes it course. The serving VPLMN should know over which area it can or cannot (will or will not) provide service eto inbound roamers. |

Q.12 (KI#4): Please indicate whether the new mechanism is needed to confine the UE to the area of disaster roaming service, or the existing mechanisms (e.g. service area restriction) can be utilised?

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that the AMF of PLMN providing disaster roaming needs to ensure that the UE is in the area of disaster roaming service at least: - upon first registration in PLMN providing disaster roaming; and - when the UE registers in a new TAI not in UE's TAI list. As long as the above is ensured, the UE can be confined to the area of disaster roaming service by providing the UE with TAI list where each TAI contains at least part of the area of disaster roaming service. |
| vivo | The existing mechanisms (e.g. service area restriction) can be utilised. |
| OPPO | No new mechanisms needed nor time spent studying new mechanisms. Existing mechanism is sufficient, unless demanded by SA1. |

Q.13 (KI#4): Please indicate whether a UE can detect that a Disaster Condition applies to PLMN D even if the UE is not registered in PLMN D and if so, whether such UE can register on a PLMN offering disaster roaming to Disaster Inbound Roamers from PLMN D?

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that UE can detect that a Disaster Condition applies to PLMN D even if the UE is not registered in PLMN D and if so, such UE can register on a PLMN offering disaster roaming to Disaster Inbound Roamers from PLMN D. Reason: The above is needed for providing service to the UE e.g. in following use cases: Use-case-A) A UE of HPLMN D powers up for the 1st time, does not discover any available and allowable PLMN but discovers that PLMN D has Disaster Condition. There is only one available PLMN (called PLMN A) and PLMN A is in UE's list of forbidden PLMNs. PLMN A offers Disaster Roaming to UEs of PLMN D. Use-case-B) A UE of HPLMN H arrives into a new country. UE's USIM is not configured with information applicable in the country yet. Upon landing, the user powers up the UE. The UE discovers one available and allowable PLMN (called PLMN A). The UE selects PLMN A, attempts to register for non-disaster roaming and is rejected with non-integrity protected 5GMM cause #11. The UE adds PLMN A into list of forbidden PLMN. PLMN A offers Disaster Roaming to UEs of PLMN D which is allowable PLMN for the UE. |
| vivo | No, only a UE that was registered to PLMN D can register on a PLMN offering disaster roaming to Disaster Inbound Roamers from PLMN D. |
| OPPO | NO. There is no need to (over) complicate this feature. Besides there are no such service requirements. |

Q.14 (KI#5): Please indicate how the UE handles PLMNs for disaster roaming that are in the list of forbidden PLMNs.

NOTE:	stage 1 specifies that PLMN in the forbidden list can be selected for disaster roamin if no other PLMN is available. This question is about how the UE manages these PLMNs internally.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson 's view is that if the UE registers for disaster roaming in PLMN offering disaster roaming, the UE does not remove the PLMN from UE's list of forbidden PLMNs. |
| vivo | Same as Ericsson |
| OPPO | FPLMN list can be untouched or can be specifically updated (i.e remove that specific PLMN ID of the VPLMN that has now accepted the UE as inbound roamer) Then when DC is over and VPLMN no longer accepts Disaster Roaming, the inbound romaer will be rejected at next registration update with #11 if needed and then PLMN ID gets put back into FPLMN. |

Q.15 (KI#9): Please indicate whether the NAS layer should provide additional information whether any PLMN supports disaster roaming to the upper layer during manual network selection

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that: - if there is no available and allowable PLMN, the NAS layer can provide additional information whether any PLMN supports disaster roaming to the upper layer during manual network selection; and - if there is an available and allowable PLMN, the NAS layer does not provide additional information whether any PLMN supports disaster roaming to the upper layer during manual network selection. Reason: If there is an available and allowable PLMN, information that a PLMN supports disaster roaming is irrelevant since disaster roaming cannot be used according to stage-1 requirement "The 3GPP system shall be able to provide means to enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list.". |
| vivo | Leave up to UE implementation. |
| OPPO | Even today much of manual selection, so leave manual selection during DC to UE implementation. |

Q.16 (KI#4): Please indicate whether PLMN with DC and PLMN providing disaster roaming should have an agreement on disaster roaming before the PLMN providing disaster roaming starts providing the disaster roaming.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that need of a PLMN with Disaster Condition and a PLMN providing disaster roaming to have an agreement on disaster roaming before the PLMN providing disaster roaming starts providing the disaster roaming, depends on regulation in the country where the PLMNs reside. In absence of a regulation, a PLMN with Disaster Condition and a PLMN providing disaster roaming need to have an agreement on disaster roaming before the PLMN providing disaster roaming starts providing the disaster roaming. The agreement can also be set up after the Disaster Condition started applying in the PLMN with Disaster Condition. |
| vivo | Yes, but the agreement is Out of 3GPP scope. |
| OPPO | This and such kind of agreements between Operators are out of scope of 3GPP, certainly out of CT1 scope. |

## 2.4	Issues regarding congestion mitigation

Q.17 (KI#7): For 5GMM layer congestion mitigation, please indicate whether a new mechanism for restricting access attempts at the UE to avoid potential overload/congestion is needed? Or are the currently available mechanisms for mitigation of overload/congestion (e.g. existing NAS level congestion control) enough?

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that a new mechanism for restricting access attempts at the UE to avoid potential overload/congestion, is not needed. The currently available mechanisms for mitigation of overload/congestion, with extension of unified access control to enable restriction on first registrations of disaster inbound roamers in the PLMN offering disaster roaming, are sufficient. |
| vivo | Existing NAS level congestion control with new cause value optionally. New wait time before attempting to register to PLMN providing DC roaming service. |
| OPPO | No new mechanism needed. Leave to PLMN offering Disaster Roaming to adjust its UAC parameters. |

Q.18 (KI#7, 8): Please indicate whether it is preferable to explicitly signal the wait time value (or the range of time) to the UE, or to have the UE computes the wait time (optionally based on parameters pre-configured or signalled to the UE), for staggering UEs changing PLMN

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that the UE uses unified access control and computes the wait time using the existing TS 38.331 mechanism associated with T390 and uac-BarringTime. |
| vivo | It is preferable to pre-configure or explicitly signal the wait time value (or the range of time) to the UE. |
| OPPO | Implicit random implementation way(s) |

Q.19 (KI#7, 8): Please indicate whether any enhancements to existing mechanisms for congestion/overaload mitigation (NAS level congestion control, RAN overload control, UAC) are needed.

NOTE:	using Access Identity 3 can be considered as an existing mechanism since SA1 already introduced it.

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that the unified access control needs to be extended to enable restriction on first registrations of disaster inbound roamers in the PLMN offering disaster roaming. |
| vivo | Existing NAS level congestion control with new cause value optionally. |
| OPPO | No new methods required other than using Access Identity 3. |

Q.20 (KI#7): For 5GSM layer congestion mitigation, please indicate whether it should be possible to limit the number of PDU sessions for the disaster inbound roaming UE during registration? Or are the currently available mechanisms for mitigation of overload/congestion (e.g. existing NAS level congestion control) enough?

| Company name | Comments |
| --- | --- |
| Ericsson | Ericsson's view is that there is no need to limit the number of PDU sessions for the disaster inbound roaming UE during registration, and that the currently available mechanisms for mitigation of overload/congestion (e.g. existing NAS level congestion control) are enough. NOTE: The disaster inbound roamer property can be taken into account when invoking the currently available mechanisms for mitigation of overload/congestion to, if preferred, apply stricter congestion levels for disaster inbound roamers. Reasons: - It is not expected that Inbound disaster roamers have different characteristics compared to normal users at requesting PDU sessions. - 5GS is dimensioned to handle 5GSM load caused by the registered UEs. Thus, as long as the rate and number of accepted inbound disaster roamers is controlled, the 5GSM level needs no additional mechanism. - PDU session limitation will risk negative side effects and impact to UE service without gain on NW congestion handling. |
| vivo | No, no SA1 requirement and the limitation of the number of PDU sessions can be achieved implictly via existing 5GSM congestion control mechanism. |
| OPPO | Nothing new needed. Leave the VPLMN to allow or not allow however many VPLMN feels it can give. Make it simple |