| 3GPP TS 29.cde V0.1.0 (2023-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Service Enabler Architecture Layer for Verticals (SEAL); Network Slice Capability Exposure (NSCE) Server Services; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	10

3	Definitions, symbols and abbreviations	11

3.1	Definitions	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Overview	12

5	Services offered by the NSCE Server	13

5.1	Introduction	13

5.2	<Service 1> Service	13

5.2.1	Service Description	13

5.2.2	Service Operations	13

5.2.2.1	Introduction	13

5.2.2.2	<Service operation 1>	13

5.2.2.2.1	General	13

5.2.2.2.2	<Procedure 1 using service operation 1 of service 1>	13

5.2.2.2.3	<Procedure 2 using service operation 1 of service 1>	13

5.2.2.3	<Service operation 2>	14

5.3	NSCE_NSOptimization	15

5.3.1	Service Description	15

5.3.2	Service Operations	15

5.3.2.1	Introduction	15

5.3.2.2	NSCE_NSOptimization_Subscribe	15

5.3.2.2.1	General	15

5.3.2.2.2	Network Slice Optimization Subscription Creation	15

5.3.2.2.3	Network Slice Optimization Subscription Update	16

5.3.2.2.4	Network Slice Optimization Subscription Deletion	17

5.3.2.3	NSCE_NSOptimization_Notify	17

5.3.2.3.1	General	17

5.3.2.3.2	Network Slice Optimization Notification	17

5.6	NSCE_PolicyManagement	18

5.6.1	Service Description	18

5.6.2.1	Introduction	18

5.6.2.2	NSCE_PolicyManagement_Create	19

5.6.2.2.1	General	19

5.6.2.2.2	Policy Provisioning Creation	19

5.6.2.3	NSCE_PolicyManagement_Update	19

5.6.2.3.1	General	19

5.6.2.3.2	Policy Provisioning Update	19

5.6.2.4	NSCE_PolicyManagement_Delete	20

5.6.2.4.1	General	20

5.6.2.4.3	Policy Provisioning Deletion	20

5.6.2.5	NSCE_PolicyManagement_Subscribe	21

5.6.2.5.1	General	21

5.6.2.5.2	Policy Usage Subscription Creation	21

5.6.2.5.3	Policy Usage Subscription Update	22

5.6.2.5.4	Policy Usage Subscription Deletion	22

5.6.2.7	NSCE_PolicyManagement_Notify	23

5.6.2.7.1	General	23

5.6.2.7.2	Policy Usage Notification	23

6	API Definitions	25

6.1	< Service 1> Service API	25

6.1.1	Introduction	25

6.1.2	Usage of HTTP and common API related aspects	25

6.1.3	Resources	25

6.1.3.1	Overview	25

6.1.3.2	Resource: <resource 1>	26

6.1.3.2.1	Description	26

6.1.3.2.2	Resource Definition	27

6.1.3.2.3	Resource Standard Methods	27

6.1.3.2.4	Resource Custom Operations	28

6.1.3.3	Resource: <resource 2>	29

6.1.4	Custom Operations without associated resources	30

6.1.4.1	Overview	30

6.1.4.2	Operation: <operation 1>	30

6.1.4.2.1	Description	30

6.1.4.2.2	Operation Definition	30

6.1.4.3	Operation: < operation 2>	30

6.1.5	Notifications	31

6.1.5.1	General	31

6.1.5.2	<notification 1>	31

6.1.5.2.1	Description	31

6.1.5.2.2	Target URI	31

6.1.5.2.3	Standard Methods	31

6.1.5.3	<notification 2>	32

6.1.6	Data Model	32

6.1.6.1	General	32

6.1.6.2	Structured data types	32

6.1.6.2.1	Introduction	32

6.1.6.2.2	Type: <TypeName 1>	33

6.1.6.2.3	Type: <TypeName 2>	33

6.1.6.3	Simple data types and enumerations	33

6.1.6.3.1	Introduction	33

6.1.6.3.2	Simple data types	33

6.1.6.3.3	Enumeration: <EnumType1>	33

6.1.6.3.4	Enumeration: <EnumType2>	33

6.1.6.4	Data types describing alternative data types or combinations of data types	34

6.1.6.4.1	Type: <TypeName 1>	34

6.1.6.4.2	Type: <TypeName 2>	34

6.1.6.5	Binary data	34

6.1.6.5.1	Binary Data Types	34

6.1.6.5.2	< Binary Data 1 >	34

6.1.7	Error Handling	34

6.1.7.1	General	35

6.1.7.2	Protocol Errors	35

6.1.7.3	Application Errors	35

6.1.8	Feature negotiation	35

6.1.9	Security	35

6.2	NSCE_NSOptimization API	36

6.2.1	Introduction	36

6.2.2	Usage of HTTP	36

6.2.3	Resources	36

6.2.3.1	Overview	36

6.2.3.2	Resource: Network Slice Optimization Subscriptions	37

6.2.3.2.1	Description	37

6.2.3.2.2	Resource Definition	37

6.2.3.2.3	Resource Standard Methods	37

6.2.3.2.3.1	POST	37

6.2.3.2.4	Resource Custom Operations	38

6.2.3.3	Resource: Individual Network Slice Optimization Subscription	38

6.2.3.3.1	Description	38

6.2.3.3.2	Resource Definition	38

6.2.3.3.3	Resource Standard Methods	38

6.2.3.3.3.1	GET	38

6.2.3.3.3.2	PUT	39

6.2.3.3.3.3	DELETE	40

6.2.3.3.4	Resource Custom Operations	41

6.2.4	Custom Operations without associated resources	41

6.2.5	Notifications	42

6.2.5.1	General	42

6.2.5.2	Network Slice Optimization Notification	42

6.2.5.2.1	Description	42

6.2.5.2.2	Target URI	42

6.2.5.2.3	Standard Methods	42

6.2.5.2.3.1	POST	42

6.2.6	Data Model	43

6.2.6.1	General	43

6.2.6.2	Structured data types	44

6.2.6.2.1	Introduction	44

6.2.6.2.2	Type: NetSliceOptSubsc	44

6.2.6.2.3	Type: NetSliceOptNotif	45

6.2.6.2.4	Type: NetSliceInformation	45

6.2.6.3	Simple data types and enumerations	45

6.2.6.3.1	Introduction	45

6.2.6.3.2	Simple data types	45

6.2.6.4	Data types describing alternative data types or combinations of data types	45

6.2.6.5	Binary data	45

6.2.6.5.1	Binary Data Types	45

6.2.7	Error Handling	46

6.2.7.1	General	46

6.2.7.2	Protocol Errors	46

6.2.7.3	Application Errors	46

6.2.8	Feature negotiation	46

6.2.9	Security	46

6.4	NSCE_PolicyManagement API	46

6.4.1	Introduction	46

6.4.2	Usage of HTTP	47

6.4.3	Resources	47

6.4.3.1	Overview	47

6.4.3.2	Resource: Policy Provisionings	48

6.4.3.2.1	Description	48

6.4.3.2.2	Resource Definition	48

6.4.3.2.3	Resource Standard Methods	48

6.4.3.2.3.1	POST	48

6.4.3.2.4	Resource Custom Operations	49

6.4.3.3	Resource: Individual Policy Provisioning	49

6.4.3.3.1	Description	49

6.4.3.3.2	Resource Definition	49

6.4.3.3.3	Resource Standard Methods	49

6.4.3.3.3.1	GET	49

6.4.3.3.3.2	PUT	50

6.4.3.3.3.3	PATCH	52

6.4.3.3.3.4	DELETE	53

6.4.3.3.4	Resource Custom Operations	54

6.4.3.4	Resource: Policy Usage Subscriptions	54

6.4.3.4.1	Description	54

6.4.3.4.2	Resource Definition	54

6.4.3.4.3	Resource Standard Methods	54

6.4.3.4.3.2	POST	54

6.4.3.4.4	Resource Custom Operations	55

6.4.3.5	Resource: Individual Policy Usage Subscription	55

6.4.3.5.1	Description	55

6.4.3.5.2	Resource Definition	55

6.4.3.5.3	Resource Standard Methods	55

6.4.3.5.3.1	GET	55

6.4.3.5.3.2	PUT	56

6.4.3.5.3.3	PATCH	57

6.4.3.5.3.4	DELETE	58

6.4.3.5.4	Resource Custom Operations	59

6.4.4	Custom Operations without associated resources	59

6.4.5	Notifications	60

6.4.5.1	General	60

6.4.5.2	Policy Usage Notification	60

6.4.5.2.1	Description	60

6.4.5.2.2	Target URI	60

6.4.5.2.3	Standard Methods	60

6.4.6	Data Model	61

6.4.6.1	General	61

6.4.6.2	Structured data types	62

6.4.6.2.1	Introduction	62

6.4.6.2.2	Type: PolicyProv	63

6.4.6.2.3	Type: PolicyProvPatch	64

6.4.6.2.4	Type: Policy	64

6.4.6.2.5	Type: PolUsageSubsc	64

6.4.6.2.6	Type: PolUsageSubscPatch	65

6.4.6.2.7	Type: ReqPolRep	65

6.4.6.2.8	Type: PolUsageNotif	65

6.4.6.2.9	Type: PolRepData	65

6.4.6.3	Simple data types and enumerations	65

6.4.6.3.1	Introduction	65

6.4.6.3.2	Simple data types	66

6.4.6.4	Data types describing alternative data types or combinations of data types	66

6.4.6.5	Binary data	66

6.4.6.5.1	Binary Data Types	66

6.4.7	Error Handling	66

6.4.7.1	General	66

6.4.7.2	Protocol Errors	66

6.4.7.3	Application Errors	66

6.4.8	Feature negotiation	66

6.4.9	Security	67

7	Using Common API Framework	68

Annex A (normative): OpenAPI specification	69

A.1	General	69

A.2	<Service 1> API	70

A.3	<Service 2> API	74

A.6	NSCE_PolicyManagement API	74

Annex B (informative): Withdrawn API versions	83

B.1	General	83

B.2	<Service 1> API	83

B.3	<Service 2> API	83

Annex <X> (informative): Change history	84

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

The present document specifies the stage 3 protocol and data model for the Network Slice Capability Exposure (NSCE) Server Services, for enabling the support of Network Slice Capability Exposure (NSCE) Server services for vertical applications. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the NSCE Serve.

The stage 2 architecture and procedures are specified in 3GPP TS 23.435 [14]>.

The common protocol and interface aspects for API definition are specified in clause 5.2 of 3GPP TS 29.122 [2].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 9113: "HTTP/2".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 9457: "Problem Details for HTTP APIs".

[13]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[14]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[15]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NSCE	Network Slice Capability Exposure

SEAL	Service Enabler Architecture Layer for Verticals

4	Overview

The Network Slice Capability Exposure (NSCE) Server forms part of the SEAL Enabler Layer defined in 3GPP TS 23.434 [23434] and aims to ensure the efficient use and deployment of network slice capability exposure capabilities to vertical applications. The NSCE server services provides additional functionality (e.g. network slice related performance and analytics monitoring) and exposes slice capabilities based on 5GS management system services (e.g MnS services) and 5GS network services (e.g. NEF APIs, NWDAF APIs, NSACF APIs).The NCSE Server supports for this purpose, among other functionalities defined in 3GPP TS 23.435 [14], the following functionalities:

-	NSCE application layer support functions to VAL Servers over the NSCE-S reference point, i.e.:

-	NSCE application layer support functions to other NSCE Servers over the NSCE-E reference point, i.e.:

Editor's Note:	The exact list can be introduced for each API when introduced.

Figure 4-1 shows the reference model of the NSCE Enabler Layer, with a focus on the NSCE Server:

Figure 4-1: NSCE Enabler Layer functional model

5	Services offered by the NSCE Server

5.1	Introduction

This clause will list the different services produced by the <NF or Entity, e.g. UAE Server>.

Table 5.1-x summarizes the corresponding APIs defined for this specification.

Table 5.1-x: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| <service name> | <ref clause> | <short description as included in the OpenAPI file> | <file name> | <apiName in the URI> | <ref Annex> |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the NSCE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

5.2	<Service 1> Service

One clause per service, where <service 1> is to be replaced by the service name (e.g. UAE_C2OperationModeManagement).

5.2.1	Service Description

This clause will provide a general description of the related service, include a description of the functional elements involved in the invocation of the service, i.e. Service Producer and Service Consumer(s), and list the service operations it supports.

5.2.2	Service Operations

One clause per service operation.

This clause will include a description of the different service operations supported by the service. For RESTful service operations, the service operations depict the resources and the methods they support.

5.2.2.1	Introduction

This clause will contain a generic introduction of the service operations described in the following clauses.

5.2.2.2	<Service operation 1>

5.2.2.2.1	General

This clause provides a general description of the service operation.

5.2.2.2.2	<Procedure 1 using service operation 1 of service 1>

5.2.2.2.3	<Procedure 2 using service operation 1 of service 1>

And so on if there are more than 2 procedures that need to be described for the service.

Clauses 5.2.2.2.x are optional to include. They can be specified e.g. if a service operation is implemented using different combinations of resources and methods.

5.2.2.3	<Service operation 2>

And so on if there are more than 2 service operations to be described for the service.

5.3	NSCE_NSOptimization

5.3.1	Service Description

The NSCE_NSOptimization service exposed by the NSCE Server enables a service consumer (e.g. VAL Server) to:

-	Create/delete the Network Slice Optimization Subscription;

-	Receive the Network Slice Optimization notifications; and

-	Retrieve the Network Slice Optimization reports.

5.3.2	Service Operations

5.3.2.1	Introduction

The service operations defined for the NSCE_NSOptimization service are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: NSCE_NSOptimization Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| NSCE_NSOptimization_Subscribe | This service operation enables a service consumer to create/update/delete the Network Slice Optimization Subscription. | e.g., VAL Server |
| NSCE_NSOptimization_Notify | This service operation enables a service consumer to receive Network Slice Optimization notifications. | e.g., VAL Server |

5.3.2.2	NSCE_NSOptimization_Subscribe

5.3.2.2.1	General

This service operation is used by a service consumer (e.g. VAL Server) to request the creation/update/deletion of a Network Slice Optimization Subscription at the NSCE Server.

The following procedures are supported by the "NSCE_NSOptimization_Subscribe" service operation:

-	Network Slice Optimization Subscription Creation;

-	Network Slice Optimization Subscription Update;

-	Network Slice Optimization Subscription Deletion.

Editor's Note:	The definition of Retrieve Network Slice Optimization report(s) is FFS.

5.3.2.2.2	Network Slice Optimization Subscription Creation

Figure 5.3.2.2.2-1 depicts a scenario where a a service consumer (e.g. VAL Server) sends a request to the NSCE Server to request the creation of a Network Slice Optimization Subscription (as defined in clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.3.2.2.2-1: Procedure for Network Slice Optimization Subscription Creation

1.	In order to subscribe to network slice optimization reporting, the service consumer (e.g. VAL Server) shall send an HTTP POST request to the NSCE Server targeting the URI of the "Network Slice Optimization Subscriptions" collection resource, with the request body including the NetSliceOptSubsc data structure.

2a.	Upon success, the NSCE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Network Slice Optimization Subscriptions" resource within the NetSliceOptSubsc data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.7.

5.3.2.2.3	Network Slice Optimization Subscription Update

Figure 5.3.2.2.3-1 depicts a scenario where a service consumer (e.g. VAL Server) sends a request to the NSCE Server to request the update of an existing Network Slice Optimization Subscription (as defined in clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.3.2.2.3-1: Procedure for Network Slice Optimization Subscription Update

1.	In order to update an existing network slice optimization subscription, the service consumer (e.g. VAL Server) shall send an HTTP PUT request to the NSCE Server, targeting the URI of the corresponding "Individual Network Slice Optimization Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the NetSliceOptSubsc data structure, in case the HTTP PUT method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the NSCE Server shall update the targeted "Individual Network Slice Optimization Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Network Slice Optimization Subscriptions" resource within the NetSliceOptSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT response body, as specified in clause 6.2.7.

Editor's Note:	The definition of this procedure(e.g. PATCH request) is FFS.

5.3.2.2.4	Network Slice Optimization Subscription Deletion

Figure 5.3.2.2.4-1 depicts a scenario where a service consumer (e.g. VAL Server) sends a request to the NSCE Server to delete an existing Network Slice Optimization Subscription (as defined in clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.3.2.2.4-1: Procedure for Network Slice Optimization Subscription Deletion

1.	In order to request the deletion of an existing network slice optimization subscription, the service consumer (e.g. VAL Server) shall send an HTTP DELETE request to the NSCE Server targeting the corresponding "Individual Network Slice Optimization Subscriptions" resource.

2a.	Upon success, the NSCE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.2.7.

5.3.2.3	NSCE_NSOptimization_Notify

5.3.2.3.1	General

This service operation is used by a NSCE Server to notify a previously subscribed service consumer (e.g. VAL Server) on:

-	Network Slice Optimization report(s).

The following procedures are supported by the "NSCE_NSOptimization_Notify" service operation:

-	Network Slice Optimization Notification.

5.3.2.3.2	Network Slice Optimization Notification

Figure 5.3.2.3.2-1 depicts a scenario where the NSCE Server sends a request to notify a previously subscribed service consumer (e.g. VAL Server) on Network Slice Optimization report(s) (as defined in clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.3.2.3.2-1: Procedure for Network Slice Optimization Notification

1.	In order to notify a previously subscribed service consumer (e.g. VAL Server) on Network Slice Optimization report(s), the NSCE Server shall send an HTTP POST request to the service consumer (e.g. VAL Server) with the request URI set to "{notifUri}", where the "notifUri" is set to the value received from the service consumer (e.g. VAL Server) during the creation of the corresponding Network Slice Optimization Subscription using the procedures defined in clause 5.3.2.2, and the request body including the NetSliceOptNotif data structure.

2a.	Upon success, the service consumer (e.g. VAL Server) shall respond to the NSCE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.7.

5.6	NSCE_PolicyManagement

5.6.1	Service Description

The NSCE_PolicyManagement service exposed by the NSCE Server enables a service consumer to:

-	create/update/delete a Policy Provisioning;

-	create/update/delete a Policy Usage Subscription; and

-	receive Policy Usage Notifications.

5.6.2.1	Introduction

The service operations defined for the NSCE_PolicyManagement service are shown in table 5.6.2.1-1.

Table 5.6.2.1-1: NSCE_PolicyManagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| NSCE_PolicyManagement_Create | This service operation enables a service consumer to request the creation of a Policy Provisioning at the NSCE Server. | e.g., VAL Server |
| NSCE_PolicyManagement_Update | This service operation enables a service consumer to request the update/modification of a Policy Provisioning at the NSCE Server. | e.g., VAL Server |
| NSCE_PolicyManagement_Delete | This service operation enables a service consumer to request the deletion of a Policy Provisioning at the NSCE Server. | e.g., VAL Server |
| NSCE_PolicyManagement_Subscribe | This service operation enables a service consumer to request the creation/update/deletion of a Policy Usage Subscription. | e.g., VAL Server |
| NSCE_PolicyManagement_Notify | This service operation enables a service consumer to receive Policy Usage Notifications. | NSCE Server |

5.6.2.2	NSCE_PolicyManagement_Create

5.6.2.2.1	General

This service operation is used by a service consumer to request the creation of a Policy Provisioning at the NSCE Server.

The following procedures are supported by the "NSCE_PolicyManagement_Create" service operation:

-	Policy Provisioning Creation.

5.6.2.2.2	Policy Provisioning Creation

Figure 5.6.2.2.2-1 depicts a scenario where a a service consumer sends a request to the NSCE Server to request the creation of a Policy Provisioning (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.2.2-1: Procedure for Policy Provisioning Creation

1.	In order to create or reserve a new Policy Provisioning, the service consumer shall send an HTTP POST request to the NSCE Server targeting the URI of the "Policy Provisionings" collection resource, with the request body including the PolicyProv data structure.

2a.	Upon success, the NSCE Server shall respond with an HTTP "201 Created" status code, with the response body containing a representation of the created "Individual Policy Provisioning" resource within the PolicyProv data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

5.6.2.3	NSCE_PolicyManagement_Update

5.6.2.3.1	General

This service operation is used by a service consumer to request the update of an existing Policy Provisioning at the NSCE Server.

The following procedures are supported by the "NSCE_PolicyManagement_Update" service operation:

-	Policy Provisioning Update.

5.6.2.3.2	Policy Provisioning Update

Figure 5.6.2.3.2-1 depicts a scenario where a service consumer sends a request to the NSCE Server to request the update of an existing Policy Provisioning (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.3.2-1: Procedure for Policy Provisioning Update

1.	In order to update an existing Policy Provisioning, the service consumer shall send an HTTP PUT/PATCH request to the NSCE Server, targeting the URI of the corresponding "Individual Policy Provisioning" resource, with the request body including either:

-	the updated representation of the resource within the PolicyProv data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the PolicyProvPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the NSCE Server shall update the targeted "Individual Policy Provisioning" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Policy Provisioning" resource within the PolicyProv data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.4.7.

5.6.2.4	NSCE_PolicyManagement_Delete

5.6.2.4.1	General

This service operation is used by a service consumer to request the deletion of an existing Policy Provisioning at the NSCE Server.

The following procedures are supported by the "NSCE_PolicyManagement_Delete" service operation:

-	Policy Provisioning Deletion.

5.6.2.4.3	Policy Provisioning Deletion

Figure 5.6.2.4.3-1 depicts a scenario where a service consumer sends a request to the NSCE Server to request the deletion of an existing Policy Provisioning (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.4.3-1: Procedure for Policy Provisioning Deletion

1.	In order to request the deletion of an existing Policy Provisioning, the service consumer shall send an HTTP DELETE request to the NSCE Server targeting the corresponding "Individual Policy Provisioning" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the NSCE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.4.7.

5.6.2.5	NSCE_PolicyManagement_Subscribe

5.6.2.5.1	General

This service operation is used by a service consumer to request the creation/update/deletion of a Policy Usage Subscription at the NSCE Server.

The following procedures are supported by the "NSCE_PolicyManagement_Subscribe" service operation:

-	Policy Usage Subscription Creation.

-	Policy Usage Subscription Update.

-	Policy Usage Subscription Deletion.

5.6.2.5.2	Policy Usage Subscription Creation

Figure 5.6.2.5.2-1 depicts a scenario where a a service consumer sends a request to the NSCE Server to request the creation of a Policy Usage Subscription (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.5.2-1: Procedure for Policy Usage Subscription Creation

1.	In order to request the creation of a new Policy Usage Subscription, the service consumer shall send an HTTP POST request to the NSCE Server targeting the URI of the "Policy Usage Subscriptions" collection resource, with the request body including the PolUsageSubsc data structure.

2a.	Upon success, the NSCE Server shall respond with an HTTP "201 Created" status code, with the response body containing a representation of the created "Individual Policy Usage Subscription" resource within the PolUsageSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

5.6.2.5.3	Policy Usage Subscription Update

Figure 5.6.2.5.3-1 depicts a scenario where a service consumer sends a request to the NSCE Server to request the update of an existing Policy Usage Subscription (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.5.3-1: Procedure for Policy Usage Subscription Update

1.	In order to update an existing Policy Usage Subscription, the service consumer shall send an HTTP PUT/PATCH request to the NSCE Server, targeting the URI of the corresponding "Individual Policy Usage Subscription" resource, with the request body including either:

-	the updated representation of the resource within the PolUsageSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the PolUsageSubscPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the NSCE Server shall update the targeted "Individual Policy Usage Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Policy Usage Subscription" resource within the PolUsageSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.4.7.

5.6.2.5.4	Policy Usage Subscription Deletion

Figure 5.6.2.5.4-1 depicts a scenario where a service consumer sends a request to the NSCE Server to request the deletion of an existing Policy Usage Subscription (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.5.4-1: Procedure for Policy Usage Subscription Deletion

1.	In order to request the deletion of an existing Policy Usage Subscription, the service consumer shall send an HTTP DELETE request to the NSCE Server targeting the corresponding "Individual Policy Usage Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the NSCE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.4.7.

5.6.2.7	NSCE_PolicyManagement_Notify

5.6.2.7.1	General

This service operation is used by a NSCE Server to notify a previously subscribed service consumer on:

-	Policy Usage event(s).

The following procedures are supported by the "NSCE_PolicyManagement_Notify" service operation:

-	Policy Usage Notification.

5.6.2.7.2	Policy Usage Notification

Figure 5.6.2.7.2-1 depicts a scenario where the NSCE Server sends a request to notify a previously subscribed service consumer on Policy Usage event(s) (see also clause 9.5 of 3GPP°TS°23.435°[14]).

Figure 5.6.2.7.2-1: Policy Usage Notification

1.	In order to notify a previously subscribed service consumer on Policy Usage event(s), the NSCE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" is set to the value received from the service consumer during the creation/update of the corresponding Policy Usage Subscription using the procedures defined in clause 5.6.2.5, and the request body including the DataDelNotif data structure.

2a.	Upon success, the service consumer shall respond to the NSCE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

6	API Definitions

6.1	< Service 1> Service API

One clause per service, where <service 1> is to be replaced by the service name (e.g. UAE_C2OperationModeManagement).

6.1.1	Introduction

This clause specifies the API Name and Version.

The <Service 1> shall use the <Service 1> API.

The API URI of the <Service 1> API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "<service 1 API name>".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. <NF or Entity, e.g. UAE Server>) takes the role of the SCEF and the service consumer (i.e. <provide examples of service consumers>) takes the role of the SCS/AS.

6.1.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2 of 3GPP TS 29.122 [2] shall apply for the <Service 1> API.

6.1.3	Resources

6.1.3.1	Overview

This clause will describe the structure for the Resource URIs and the resources and methods used for the service.

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the <Service1> API.

Example:

Figure 6.1.3.1-1: Resource URI structure of the <xyz > API

Figure 6.1.3.1-1 illustrates resource URI structure with an example. Clause 5.2.1-1 in 3GPP TS 29.501 [3] specifies graphical conventions used in the resource URI structures.

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| <Resource name> | <relative path after API URI> | GET | <Operation executed by GET> |
|  |  | PUT | <Operation executed by PUT> |
|  |  | PATCH | <Operation executed by PATCH> |
|  |  | POST | <Operation executed by POST> |
|  |  | DELETE | <Operation executed by DELETE> |
|  |  | Custom operation | <Operation executed by custom operation> |

6.1.3.2	Resource: <resource 1>

Where <resource 1> is to be replaced by the resource name, e.g. "Real-time UAV Status Subscriptions".

6.1.3.2.1	Description

This clause will specify what the resource represents or what it is used for.

6.1.3.2.2	Resource Definition

This clause will describe the Resource URI and the supported resource variables. "<apiName>" and "<apiVersion>" are placeholders (not resource URI variables) which are ultimately replaced by their values defined in clause 6.1.1. The "<apiName>" placeholder should hence be replaced by its value (e.g. "uae-c2opmode-mngt") as defined in clause 6.1.1. However, for the sake of simplicity and in order to avoid to have to update all the occurrences of the API version when the latter is changed, the "<apiVersion>" should be kept in the resource URI when present in the core of the specification.

Resource URI: {apiRoot}/<apiName>/<apiVersion>/xxx

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| <name> | <type> | <definition> |

6.1.3.2.3	Resource Standard Methods

The following clauses will specify the standard methods supported by the resource.

It will describe, for each method, the use of the method, the URI query parameters supported by the method, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

6.1.3.2.3.1	< method 1 >

This clause will specify the meaning of the method applied on the resource.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the <method 1> method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the <method 1> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.3.2.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the HTTP <method 1> method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the <e.g. GET> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 6.1.3.2.3.1-5: Headers supported by the <e.g. 200> response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 6.1.3.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| <link name> e.g. search | <resource 1> e.g. Stored Search (Document) | <method 1> e.g. GET | <parameter> e.g. searchId | <description of the link> |

6.1.3.2.3.2	< method 2 >

And so on if there are more than two methods supported by the resource. Same structure as in clause 6.1.3.2.3.1.

6.1.3.2.4	Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

6.1.3.2.4.1	Overview

Table 6.1.3.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

6.1.3.2.4.2	Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

6.1.3.2.4.2.1	Description

This clause will describe the custom operation and what it is used for, and the custom operation's URI.

6.1.3.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 6.1.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2.2-2.

Table 6.1.3.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.3.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. HTTP POST> method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

6.1.3.2.4.3	Operation: < operation 2 >

And so on if there are more than two operations supported by the resource. Same structure as in clause 6.1.3.2.4.1.

6.1.3.3	Resource: <resource 2>

And so on if there are more than two resources supported by the service. Same structure as in clause 6.1.3.2.

6.1.4	Custom Operations without associated resources

6.1.4.1	Overview

This clause will specify custom operations without any associated resource (i.e. RPC) supported by this API.

Table 6.1.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

6.1.4.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Initiate, Allocate.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

6.1.4.2.1	Description

This clause will describe the custom operation and what it is used for, and the custom operation's URI.

6.1.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 6.1.4.2.2-1 and 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. HTTP POST> method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

6.1.4.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 6.1.4.2.

6.1.5	Notifications

6.1.5.1	General

This clause will specify the use of notifications and corresponding protocol details if required for the specific service. When notifications are supported by the API, it will include a reference to the general description of notifications support over the Northbound and Application Layer Interfaces/APIs (NBI) specified in clause 5.2.5 of 3GPP TS 29.122 [2].

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [2].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| <notification 1> e.g. Status Change Notification | < Callback URI > e.g. {StatusCallbackUri} | e.g POST | e.g. Notify Event |

6.1.5.2	<notification 1>

6.1.5.2.1	Description

The Event Notification is used by the NF service producer to report one or several observed Events to a NF service consumer that has subscribed to such Notifications.

6.1.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Callback Uri |

6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

6.1.5.3	<notification 2>

And so on if there are more than one notifications supported by the service. Same structure as in clause 6.1.5.2.

6.1.6	Data Model

6.1.6.1	General

This clause specifies the application data model supported by the API.

Data types that may be common to multiple APIs (offered by the same or different NFs) should be specified in a new separate TS (similar approach as for TS 29.230 for Diameter AVPs).

Table 6.1.6.1-1 specifies the data types defined for the <Service 1> API.

Table 6.1.6.1-1: <Service 1> API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |

Table 6.1.6.1-2 specifies data types re-used by the <Service 1> API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the <Service 1> API.

Table 6.1.6.1-2: <Service 1> API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

6.1.6.2	Structured data types

This clause will specify the structured data types.

6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.6.2.2	Type: <TypeName 1>

Table 6.1.6.2.2-1: Definition of type <TypeName 1>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <attribute name> | "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N" | <only if applicable> |  |

6.1.6.2.3	Type: <TypeName 2>

And so on if there are more types to specify.

6.1.6.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

6.1.6.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

6.1.6.3.4	Enumeration: <EnumType2>

And so on if there are more enumerations to define.

6.1.6.4	Data types describing alternative data types or combinations of data types

6.1.6.4.1	Type: <TypeName 1>

The data types describing alternative data types or combinations of data types shall represent an OpenAPI schema object using the "oneOf", "anyOf" or "allOf"  keyword to list alternative or to be combined data types (see the OpenAPI specification [4] and https://swagger.io/docs/specification/data-models/oneof-anyof-allof-not/).

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches , i.e. a list of mutually exclusive alternative data types., as described using the OpenAPI "oneOf" keyword, if the instance matches against one and only one of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of non-exclusive alternative data types, as described using the OpenAPI "anyOf" keyword, if the instance matches against one or more of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of to be combined data types, as described using the OpenAPI "allOf" keyword, if the instance matches against all of the to be combined data types."Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON elements in a JSON object.

Table 6.1.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

6.1.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

6.1.6.5	Binary data

6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| < Binary Data 1 > e.g. N1 SM Message | < clause > e.g. 6.1.6.5.2 | <content type> e.g. vnd.3gpp.5gnas |

6.1.6.5.2	< Binary Data 1 >

And so on if there are more binary data to specify

6.1.7	Error Handling

This clause will include a reference to the general error handling principles specified in TS 29.122, and further specify any general error handling aspect specific to the API, if any Error handling specific to each method (and resource) is specified in clauses 6.1.3. and 6.1.4.

6.1.7.1	General

For the <API Name> API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the <API Name> API.

6.1.7.2	Protocol Errors

No specific procedures for the <API name> API are specified.

Or add specific information for the API if applicable.

6.1.7.3	Application Errors

The application errors defined for the <API name> API are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the <API name> API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

The feature number is a unique integer number within the API designating the feature. The first feature obtains the number 1, and subsequent features obtain the next numbers (2,3 …).

The feature name is unique name within the API used to designate the feature e.g. in "Applicability" columns of various tables within the API definition.

In the Description column, a description of the feature is provided. A possible withdrawal of the feature is also indicated in the description column; the deficits leading to the withdrawal of the feature are then also explained in that column.

6.1.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the <API name> API.

6.2	NSCE_NSOptimization API

6.2.1	Introduction

The NSCE_NSOptimization service shall use the NSCE_NSOptimization API.

The API URI of the NSCE_NSOptimization Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "nsce-ns-optimization".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.2, the service producer (i.e. NSCE Server) takes the role of the SCEF and the service consumer (e.g., VAL Server) takes the role of the SCS/AS.

6.2.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the NSCE_NSOptimization API.

6.2.3	Resources

6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the NSCE_NSOptimization API.

Figure 6.2.3.1-1: Resource URIs structure of the NSCE_NSOptimization API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods for the NSCE_NSOptimization API.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Network Slice Optimization Subscriptions | /subscriptions | POST | Request the creation of a Network Slice Optimization Subscription. |
| Individual Network Slice Optimization Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual Network Slice Optimization Subscription". |
|  |  | PUT | Request the fully update of an existing "Individual Network Slice Optimization Subscription". |
|  |  | DELETE | Request the deletion of an existing "Individual Network Slice Optimization Subscription". |

Editor's Note:	The HTTP PATCH method is needed or not is FFS.

6.2.3.2	Resource: Network Slice Optimization Subscriptions

6.2.3.2.1	Description

This resource represents the collection of Network Slice Optimization Subscriptions managed by the NSCE Server.

6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nsce-ns-optimization/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |

6.2.3.2.3	Resource Standard Methods

6.2.3.2.3.1	POST

The HTTP POST method allows a service consumer (e.g. VAL Server) to request the creation of a Network Slice Optimization Subscription at the NSCE Server.

This method shall support the URI query parameters specified in the table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NetSliceOptSubsc | M | 1 | Create a new service switch information subscription. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NetSliceOptSubsc | M | 1 | 201 Created | Successful case. The Network Slice Optimization Subscription is successfully created and a representation of the created "Individual Network Slice Optimization Subscription" resource shall be returned. An HTTP "Location" header that contains the resource URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsce-ns-optimization/<apiVersion>/subscriptions/{subscriptionId} |

6.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.2.3.3	Resource: Individual Network Slice Optimization Subscription

6.2.3.3.1	Description

This resource represents a Network Slice Optimization Subscription managed by the NSCE Server.

6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/nsce-ns-optimization/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| subscriptionId | string | Represents the identifier of the "Individual Network Slice Optimization Subscription" resource. |

6.2.3.3.3	Resource Standard Methods

6.2.3.3.3.1	GET

The HTTP GET method allows a service consumer (e.g. VAL Server) to retrieve an existing "Individual Network Slice Optimization Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in the table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NetSliceOptSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Network Slice Optimization Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NSCE server. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NSCE server. |

6.2.3.3.3.2	PUT

The HTTP PUT method allows a service consumer (e.g. VAL Server) to request the update of an existing "Individual Network Slice Optimization Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in the table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NetSliceOptSubsc | M | 1 | Represents the updated representation of the "Individual Network Slice Optimization Subscription" resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NetSliceOptSubsc | M | 1 | 200 OK | Successful case. The "Individual Network Slice Optimization Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Network Slice Optimization Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NSCE server. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NSCE server. |

6.2.3.3.3.3	DELETE

The HTTP DELETE method allows a service consumer (e.g. VAL Server) to request the deletion of an existing "Individual Network Slice Optimization Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.4-3.

Table 6.2.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Network Slice Optimization Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NSCE Server. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NSCE Server. |

6.2.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.2.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

6.2.5	Notifications

6.2.5.1	General

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Network Slice Optimization Notification | {notifUri} | POST | This service operation enables a NSCE Server to notify a previously subscribed service consumer (e.g. VAL Server) on Network Slice Optimization reports. |

6.2.5.2	Network Slice Optimization Notification

6.2.5.2.1	Description

The Network Slice Optimization Notification is used by a NSCE Server to notify a previously subscribed service consumer on Network Slice Optimization reports.

6.2.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | String formatted as a URI containing the Callback URI. The notification URI is provided as part of the corresponding Network Slice Optimization Subscription creation/update request. |

6.2.5.2.3	Standard Methods

6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-2.

Table 6.2.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NetSliceOptNotif | M | 1 | Represents a Network Slice Optimization notification. |

Table 6.2.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Network Slice Optimization Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 6.2.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

6.2.6	Data Model

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the NSCE_NSOptimization API.

Table 6.2.6.1-1: NSCE_NSOptimization API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| NetSliceInformation | 6.2.6.2.4 | Contains the network slice information with network slice identifier. |  |
| NetSliceOptNotif | 6.2.6.2.3 | Represents a Network Slice Optimization notification. |  |
| NetSliceOptSubsc | 6.2.6.2.2 | Represents a Network Slice Optimization subscription. |  |

Table 6.2.6.1-2 specifies data types re-used by the NSCE_NSOptimization API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the NSCE_NSOptimization API.

Table 6.2.6.1-2: NSCE_NSOptimization API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTimeRo | 3GPP TS 29.122 [2] | Represents a date and a time with the read-only property. |  |
| Dnn | 3GPP TS 29.571 [y] | Identifies a DNN. |  |
| DurationSec | 3GPP TS 29.571 [y] | Identifies a period of time in units of seconds. |  |
| Snssai | 3GPP TS 29.571 [y] | Represents an S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [y] | Used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

Editor's Note:	The definition of the OpenAPI file is FFS and will be updated based on the conclusions agreed upon at CT3#131 meeting.

6.2.6.2	Structured data types

6.2.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.2.6.2.2	Type: NetSliceOptSubsc

Table 6.2.6.2.2-1: Definition of type NetSliceOptSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the URI via which notifications shall be provided. |  |
| netSlicIds | array(Snssai) | O | 1..N | Contains the identifier(s) for the network slice. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| policyId | string | O | 0..1 | Identifies the VAL server policy. |  |
| expTime | DateTimeRo | O | 0..1 | Indicates the time at which the network slice optimization subscription shall expire. This attribute may only be present in Network Slice Optimization subscription creation/update responses. If this attribute is absent, this means that the Network Slice Optimization subscription shall not expire, until explicitly deleted by the service consumer (e.g. VAL Server). |  |
| secPolicId | string | O | 0..1 | Contains the identifier of the secondary policy for the network slice optimization in the case of a failed network slice optimization. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.2.8. This attribute shall be provided if feature negotiation shall take place. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

Editor's Note:	The need of the security credentials is FFS.

Editor's Note:	The encoding of the netSlicIds attribute is FFS

6.2.6.2.3	Type: NetSliceOptNotif

Table 6.2.6.2.3-1: Definition of type NetSliceOptNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsId | string | M | 1 | Identifies the Network slice optimization subscribe event. |  |
| netSliceInfo | NetSliceInformation | M | 1 | Contains the network slice information with network slice identifier. |  |
| optTime | DurationSec | O | 0..1 | Indicates time spent for slice optimization by the NSCE Server. |  |
| enforPolId | string | O | 0..1 | Indicates the policy used for slice optimization in the case of the failed network slice optimization. |  |

6.2.6.2.4	Type: NetSliceInformation

Table 6.2.6.2.4-1: Definition of type NetSliceInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nest | string | M | 1 | Contains the network slice information. |  |
| snssai | string | M | 1 | Contains the network slice identifier. |  |

Editor's Note:	The encoding of the "nest" and "snssai" attribute are FFS.

6.2.6.3	Simple data types and enumerations

6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

6.2.6.5	Binary data

6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

6.2.7	Error Handling

6.2.7.1	General

For the NSCE_NSOptimization API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the NSCE_NSOptimization API.

6.2.7.2	Protocol Errors

No specific protocol errors for the NSCE_NSOptimization API are specified.

6.2.7.3	Application Errors

The application errors defined for the NSCE_NSOptimization API are listed in Table 6.4.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the NSCE_NSOptimization API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

6.2.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the NSCE_NSOptimization API.

6.4	NSCE_PolicyManagement API

6.4.1	Introduction

The NSCE_PolicyManagement service shall use the NSCE_PolicyManagement API.

The API URI of the NSCE_PolicyManagement Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "nsce-pm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.4, the NSCE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

6.4.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the NSCE_PolicyManagement API.

6.4.3	Resources

6.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.4.3.1-1 depicts the resource URIs structure for the NSCE_PolicyManagement API.

Figure 6.4.3.1-1: Resource URIs structure of the NSCE_PolicyManagement API

Table 6.4.3.1-1 provides an overview of the resources and applicable HTTP methods for the NSCE_PolicyManagement API.

Table 6.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Policy Provisionings | /provisionings | POST | Request the creation of a Policy Provisioning. |
| Individual Policy Provisioning | /provisionings/{provId} | GET | Retrieve an existing Policy Provisioning. |
|  |  | PUT | Request the update of an existing Policy Provisioning. |
|  |  | PATCH | Request the modification of an existing Policy Provisioning. |
|  |  | DELETE | Request the deletion of an existing Policy Provisioning. |
| Policy Usage Subscriptions | /subscriptions | POST | Request the creation of a Policy Usage Subscription. |
| Individual Policy Usage Subscription | /sbscriptions/{subscriptionId} | GET | Retrieve an existing Policy Usage Subscription. |
|  |  | PUT | Request the update of an existing Policy Usage Subscription. |
|  |  | PATCH | Request the modification of an existing Policy Usage Subscription. |
|  |  | DELETE | Request the deletion of an existing Policy Usage Subscription. |

6.4.3.2	Resource: Policy Provisionings

6.4.3.2.1	Description

This resource represents the collection of Policy Provisionings managed by the NSCE Server.

6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/nsce-pm/<apiVersion>/provisionings

This resource shall support the resource URI variables defined in table 6.4.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1. |

6.4.3.2.3	Resource Standard Methods

6.4.3.2.3.1	POST

The HTTP POST method allows a service consumer to request the creation of a Policy Provisioning at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyProv | M | 1 | Represents the parameters to request the creation of a Policy Provisioning resource. |

Table 6.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyProv | M | 1 | 201 Created | Successful case. The Policy Provisioning is successfully created and a representation of the created "Individual Policy Provisioning" resource shall be returned. An HTTP "Location" header that contains the resource URI of the created resource shall also be included. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsce-pm/<apiVersion>/provisionings/{provId} |

6.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.3.3	Resource: Individual Policy Provisioning

6.4.3.3.1	Description

This resource represents a Policy Provisioning managed by the NSCE Server.

6.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/nsce-pm/<apiVersion>/provisionings/{provId}

This resource shall support the resource URI variables defined in table 6.4.3.3.2-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1. |
| provId | string | Represents the identifier of the "Individual Policy Provisioning" resource. |

6.4.3.3.3	Resource Standard Methods

6.4.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Policy Provisioning" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.1-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.4.3.3.3.1-3.

Table 6.4.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyProv | M | 1 | 200 OK | Successful case. The requested "Individual Policy Provisioning" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual Policy Provisioning" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.2-1.

Table 6.4.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.2-2 and the response data structures and response codes specified in table 6.4.3.3.3.2-3.

Table 6.4.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyProv | M | 1 | Represents the updated representation of the "Individual Policy Provisioning" resource. |

Table 6.4.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyProv | M | 1 | 200 OK | Successful case. The "Individual Policy Provisioning" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Provisioning" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual Policy Provisioning" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.3-1.

Table 6.4.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.3-2 and the response data structures and response codes specified in table 6.4.3.3.3.3-3.

Table 6.4.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyProvPatch | M | 1 | Represents the parameters to request the modification of the "Individual Policy Provisioning" resource. |

Table 6.4.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyProv | M | 1 | 200 OK | Successful case. The "Individual Policy Provisioning" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Provisioning" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual Policy Provisioning" resource at the NSCE Server.

Editor's Note:	The full definition of this operation and whether a resource custom operation is more appropriate to support it is FFS.

This method shall support the URI query parameters specified in table 6.4.3.3.3.4-1.

Table 6.4.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.4-2 and the response data structures and response codes specified in table 6.4.3.3.3.4-3.

Table 6.4.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Provisioning" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.3.4	Resource: Policy Usage Subscriptions

6.4.3.4.1	Description

This resource represents the collection of Policy Usage Subscriptions managed by the NSCE Server.

6.4.3.4.2	Resource Definition

Resource URI: {apiRoot}/nsce-pm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.4.3.4.2-1.

Table 6.4.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1. |

6.4.3.4.3	Resource Standard Methods

6.4.3.4.3.2	POST

The HTTP POST method allows a service consumer to request the creation of a Policy Usage Subscription at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.4.3.2-1.

Table 6.4.3.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.4.3.2-2 and the response data structures and response codes specified in table 6.4.3.4.3.2-3.

Table 6.4.3.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolUsageSubsc | M | 1 | Represents the parameters to request the creation of a Policy Usage Subscription resource. |

Table 6.4.3.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolUsageSubsc | M | 1 | 201 Created | Successful case. The Policy Usage Subscription is successfully created and a representation of the created "Individual Policy Usage Subscription" resource shall be returned. An HTTP "Location" header that contains the resource URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.4.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsce-pm/<apiVersion>/subscriptions/{subscriptionId} |

6.4.3.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.3.5	Resource: Individual Policy Usage Subscription

6.4.3.5.1	Description

This resource represents a Policy Usage Subscription managed by the NSCE Server.

6.4.3.5.2	Resource Definition

Resource URI: {apiRoot}/nsce-pm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.4.3.5.2-1.

Table 6.4.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1. |
| subscriptionId | string | Represents the identifier of the "Individual Policy Usage Subscription" resource. |

6.4.3.5.3	Resource Standard Methods

6.4.3.5.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Policy Usage Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.5.3.1-1.

Table 6.4.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.5.3.1-2 and the response data structures and response codes specified in table 6.4.3.5.3.1-3.

Table 6.4.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolUsageSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Policy Usage Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.5.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual Policy Usage Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.5.3.2-1.

Table 6.4.3.5.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.5.3.2-2 and the response data structures and response codes specified in table 6.4.3.5.3.2-3.

Table 6.4.3.5.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolUsageSubsc | M | 1 | Represents the updated representation of the "Individual Policy Usage Subscription" resource. |

Table 6.4.3.5.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolUsageSubsc | M | 1 | 200 OK | Successful case. The "Individual Policy Usage Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Usage Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.5.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual Policy Usage Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.5.3.3-1.

Table 6.4.3.5.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.5.3.3-2 and the response data structures and response codes specified in table 6.4.3.5.3.3-3.

Table 6.4.3.5.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolUsageSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual Policy Usage Subscription" resource. |

Table 6.4.3.5.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolUsageSubsc | M | 1 | 200 OK | Successful case. The "Individual Policy Usage Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Usage Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.5.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.5.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.5.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual Policy Usage Subscription" resource at the NSCE Server.

This method shall support the URI query parameters specified in table 6.4.3.5.3.4-1.

Table 6.4.3.5.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.5.3.4-2 and the response data structures and response codes specified in table 6.4.3.5.3.4-3.

Table 6.4.3.5.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.5.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Usage Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NSCE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.5.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

Table 6.4.3.5.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NSCE Server. |

6.4.3.5.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

6.4.5	Notifications

6.4.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [15].

Table 6.4.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Policy Usage Notification | {notifUri} | POST | This service operation enables a NSCE Server to notify a previously subscribed service consumer on Policy Usage event(s). |

6.4.5.2	Policy Usage Notification

6.4.5.2.1	Description

The Policy Usage Notification is used by the NSCE Server to notify a previously subscribed service consumer on Policy Usage event(s).

6.4.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.4.5.2.2-1.

Table 6.4.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

6.4.5.2.3	Standard Methods

6.4.5.2.3.1	POST

This method shall support the request data structures specified in table 6.4.5.2.3.1-1 and the response data structures and response codes specified in table 6.4.5.2.3.1-2.

Table 6.4.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolUsageNotif | M | 1 | Represents the Policy Usage Notification. |

Table 6.4.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Policy Usage Notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.4.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 6.4.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

6.4.6	Data Model

6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the NSCE_PolicyManagement API.

Table 6.4.6.1-1: NSCE_PolicyManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| Policy | 6.4.6.2.4 | Represents a policy. |  |
| PolicyProv | 6.4.6.2.2 | Represents a Policy Provisioning. |  |
| PolicyProvPatch | 6.4.6.2.3 | Represents the requested modifications to a Policy Provisioning. |  |
| PolRepData | 6.4.6.2.9 | Represents policy usage reporting data. |  |
| PolUsageNotif | 6.4.6.2.8 | Represents a Policy Usage Notification. |  |
| PolUsageSubsc | 6.4.6.2.5 | Represents a Policy Usage Subscription. |  |
| PolUsageSubscPatch | 6.4.6.2.6 | Represents the requested modifications to a Policy Usage Subscription. |  |
| ReqPolRep | 6.4.6.2.7 | Represents the requested policy usage reporting information. |  |

Table 6.4.6.1-2 specifies data types re-used by the NSCE_PolicyManagement API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the NSCE_PolicyManagement API.

Table 6.4.6.1-2: NSCE_PolicyManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| Dnn | 3GPP TS 29.571 [18] | Represents a DNN. |  |
| DurationSec | 3GPP TS 29.122 [2] | Represents a time duration in seconds. |  |
| ProblemDetails | 3GPP TS 29.122 [2] | Represents error related information. |  |
| Snssai | 3GPP TS 29.571 [18] | Represents an S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uinteger | 3GPP TS 29.571 [18] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

6.4.6.2	Structured data types

6.4.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.4.6.2.2	Type: PolicyProv

Table 6.4.6.2.2-1: Definition of type PolicyProv

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| netSliceId | Snssai | O | 0..1 | Contains the identifier for the concerned network slice. |  |
| reqDnn | Dnn | O | 0..1 | Contains the requested DNN. |  |
| polHarmInd | boolean | O | 0..1 | Contains the policy harmonization indication. It indicates whether policy harmonization is requested or not, i.e.: - "true" means that policy harmonization is requested. - "false" means that policy harmonization is not requested. - The default value when omitted is "false". |  |
| policy | Policy | O | 0..1 | Contains the provisioned policy. |  |
| defaultPolInd | boolean | O | 0..1 | Contains the default policy indication. It indicates whether or not the provisioned policy shall be used as a default policy for the network slices provisioned without any policy, i.e.: - "true" means that the provisioned policy shall be used as a default policy for the network slices provisioned without any policy. - "false" means that the provisioned policy shall not be used as a default policy for the network slices provisioned without any policy. - The default value when omitted is "false". |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.4.8. This attribute shall be present only if feature negotiation needs to take place. |  |

Editor's Note:	The full content of this data structure and whether the provisioning can be done for more than one network slice is FFS.

Editor's Note:	The encoding of the "netSliceId" attribute is FFS.

6.4.6.2.3	Type: PolicyProvPatch

Table 6.4.6.2.3-1: Definition of type PolicyProvPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| netSliceId | Snssai | O | 0..1 | Contains the identifier for the concerned network slice. |  |
| reqDnn | Dnn | O | 0..1 | Contains the requested DNN. |  |
| policy | Policy | O | 0..1 | Contains the updated policy. |  |
| defaultPolInd | boolean | O | 0..1 | Contains the default policy indication. It indicates whether or not the provisioned policy shall be used as a default policy for the network slices provisioned without any policy, i.e.: - "true" means that the provisioned policy shall be used as a default policy for the network slices provisioned without any policy. - "false" means that the provisioned policy shall not be used as a default policy for the network slices provisioned without any policy. - The default value when omitted is "false". |  |

Editor's Note:	The full content of this data structure is FFS.

Editor's Note:	The encoding of the "netSliceId" attribute is FFS.

6.4.6.2.4	Type: Policy

Table 6.4.6.2.4-1: Definition of type ReservRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The definition of this data structure is FFS.

6.4.6.2.5	Type: PolUsageSubsc

Table 6.4.6.2.5-1: Definition of type PolUsageSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Contains the URI via which the Policy Usage Notifications shall be delivered. |  |
| netSliceId | Snssai | M | 1 | Contains the identifier for the requested network slice. |  |
| reqPolicyRep | ReqPolRep | M | 1 | Contains the requested policy usage reporting information. |  |
| repPeriodicity | DurationSec | O | 0..1 | Contains the reporting periodicity (i.e., reporting interval). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.4.8. This attribute shall be present only if feature negotiation needs to take place. |  |

Editor's Note:	The encoding of the "netSliceId" attribute is FFS.

6.4.6.2.6	Type: PolUsageSubscPatch

Table 6.4.6.2.6-1: Definition of type PolUsageSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which the Policy Usage Notifications shall be delivered. |  |
| reqPolicyRep | ReqPolRep | O | 0..1 | Contains the updated requested policy usage reporting information. |  |
| repPeriodicity | DurationSec | O | 0..1 | Contains the updated reporting periodicity (i.e., reporting interval). |  |

6.4.6.2.7	Type: ReqPolRep

Table 6.4.6.2.7-1: Definition of type ReqPolRep

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| policyId | string | M | 1 | Contains the identifier of the targeted policy. |  |
| startTime | DateTime | M | 1 | Contains the start time of the requested policy usage reporting. |  |
| endTime | DateTime | M | 1 | Contains the end time of the requested policy usage reporting. |  |

6.4.6.2.8	Type: PolUsageNotif

Table 6.4.6.2.8-1: Definition of type PolUsageNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Contains the identifier of the subscription to which the Policy Usage Notification is related. |  |
| reports | array(PolRepData) | M | 1..N | Contains the reported policy usage data. |  |

6.4.6.2.9	Type: PolRepData

Table 6.4.6.2.9-1: Definition of type PolRepData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| policyId | string | M | 1 | Contains the identifier of the policy to which the policy usage report is related. |  |
| count | Uinteger | M | 1 | Contains the number of times the policy identified by the "policyId" attribute is active/used. |  |
| timeSpent | DurationSec | M | 1 | Contains the usage time duration of the policy identified by the "policyId" attribute. |  |
| preEmptCount | Uinteger | O | 0..1 | Contains the number of times the policy is pre-empted by another policy. |  |
| preEmptPolId | string | O | 0..1 | Contains the identifier of the policy that is used for pre-emption. |  |

6.4.6.3	Simple data types and enumerations

6.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.4.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

6.4.6.5	Binary data

6.4.6.5.1	Binary Data Types

Table 6.4.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

6.4.7	Error Handling

6.4.7.1	General

For the NSCE_PolicyManagement API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [15].

In addition, the requirements in the following clauses are applicable for the NSCE_PolicyManagement API.

6.4.7.2	Protocol Errors

No specific protocol errors for the NSCE_PolicyManagement API are specified.

6.4.7.3	Application Errors

The application errors defined for the NSCE_PolicyManagement API are listed in Table 6.4.7.3-1.

Table 6.4.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| INVALID_POLICY | 403 Forbidden | Indicates that the Policy Provisioning is rejected because the provided policy in not valid. |  |

Editor's Note:	The complete definition of the error handling cases is FFS.

6.4.8	Feature negotiation

The optional features listed in table 6.4.8-1 are defined for the NSCE_PolicyManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [15].

Table 6.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.4.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [15] shall apply for the NSCE_PolicyManagement API.

7	Using Common API Framework

The provisions of clause 8 of 3GPP TS 29.549 [15] shall apply for the NSCE Server APIs defined in this specification.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [3] and clause 5B of 3GPP TR 21.900 [5]).

A.2	<Service 1> API

Where <Service 1> is to be replaced by the name of the Service (e.g. UAE_C2OperationModeManagement).

One clause is introduced per Service, with the corresponding OpenAPI 3.0.0 Document.

openapi: 3.0.0

info:

title: <API Name>

version: 1.0.0-alpha.1

description: |

<API Name> Service.

© <2023>, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.<xxx> V<x.y.z>; <TS Name>.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.xxx/

servers:

- url: '{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

# API specific definitions, below is an example

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/<version>/subscriptions/{subId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<yyy>'

responses:

'204':

description: No Content, Notification was succesfull.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: update subscription

operationId: ReplaceIndividualSubcription

tags:

- IndividualSubscription (Document)

tags:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'204':

description: No Content. Resource was succesfully modified

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# API specific definitions

A.3	<Service 2> API

And so on if there are more than two services supported by the NF.

A.6	NSCE_PolicyManagement API

openapi: 3.0.0

info:

title: NSCE Server Policy Management Service

version: 1.0.0-alpha.1

description: |

NSCE Server Policy Management Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.5xx V0.0.0; Service Enabler Architecture Layer for Verticals (SEAL);

Network Slice Capability Exposure (NSCE) Server Service(s); Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.5xx/

servers:

- url: '{apiRoot}/nsce-pm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/provisionings:

post:

summary: Request the creation of a Policy Provisioning.

operationId: CreatePolProv

tags:

- Policy Provisionings (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyProv'

responses:

'201':

description: >

Created. The Policy Provisioning is successfully created and a representation of the

created Individual Policy Provisioning resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyProv'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Provisioning resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/provisionings/{provId}:

parameters:

- name: provId

in: path

description: >

Represents the identifier of the Individual Policy Provisioning resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Provisioning resource.

operationId: GetIndPolProv

tags:

- Individual Policy Provisioning (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Provisioning resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyProv'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Provisioning resource.

operationId: UpdateIndDPolProv

tags:

- Individual Policy Provisioning (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyProv'

responses:

'200':

description: >

OK. The Individual Policy Provisioning resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyProv'

'204':

description: >

No Content. The Individual Policy Provisioning resource is successfully updated and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Provisioning resource.

operationId: ModifyIndPolProv

tags:

- Individual Policy Provisioning (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolicyProvPatch'

responses:

'200':

description: >

OK. The Individual Policy Provisioning resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyProv'

'204':

description: >

No Content. The Individual Policy Provisioning resource is successfully modified and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Provisioning resource.

operationId: DeleteIndPolProv

tags:

- Individual Policy Provisioning (Document)

responses:

'204':

description: >

No Content. The Individual Policy Provisioning resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Request the creation of a Policy Usage Subscription.

operationId: CreatePolUsageSubsc

tags:

- Policy Usage Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageSubsc'

responses:

'201':

description: >

Created. The Policy Usage Subscription is successfully created and a representation

of the created Individual Policy Usage Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Usage Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

PolUsageNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageNotif'

responses:

'204':

description: >

No Content. The Policy Usage Notification is successfully received and

Acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Policy Usage Subscription resource.

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Usage Subscription resource.

operationId: GetIndPolUsageSubsc

tags:

- Individual Policy Usage Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Usage Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Usage Subscription resource.

operationId: UpdateIndPolUsageSubsc

tags:

- Individual Policy Usage Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageSubsc'

responses:

'200':

description: >

OK. The Individual Policy Usage Subscription resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageSubsc'

'204':

description: >

No Content. The Individual Policy Usage Subscription resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Usage Subscription resource.

operationId: ModifyIndPolUsageSubsc

tags:

- Individual Policy Usage Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolUsageSubscPatch'

responses:

'200':

description: >

OK. The Individual Policy Usage Subscription resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolUsageSubsc'

'204':

description: >

No Content. The Individual Policy Usage Subscription resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Usage Subscription resource.

operationId: DeleteIndPolUsageSubsc

tags:

- Individual Policy Usage Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Policy Usage Subscription resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

PolicyProv:

description: >

Represents a Policy Provisioning.

type: object

properties:

netSliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

reqDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

polHarmInd:

type: boolean

default: false

description: >

Contains the policy harmonization indication. It indicates whether policy harmonization

is requested or not.

true means that policy harmonization is requested.

false means that policy harmonization is not requested.

The default value when omitted is "false".

defaultPolInd:

type: boolean

default: false

description: >

Contains the default policy indication. It indicates whether or not the provisioned

policy shall be used as a default policy for the network slices provisioned without any

policy.

true means that the provisioned policy shall be used as a default policy for the network

slices provisioned without any policy.

false means that the provisioned policy shall not be used as a default policy for the

network slices provisioned without any policy.

The default value when omitted is "false".

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

PolicyProvPatch:

description: >

Represents the requested modifications to a Policy Provisioning.

type: object

properties:

netSliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

reqDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

defaultPolInd:

type: boolean

default: false

description: >

Contains the default policy indication. It indicates whether or not the provisioned

policy shall be used as a default policy for the network slices provisioned without any

policy.

true means that the provisioned policy shall be used as a default policy for the network

slices provisioned without any policy.

false means that the provisioned policy shall not be used as a default policy for the

network slices provisioned without any policy.

The default value when omitted is "false".

PolUsageSubsc:

description: >

Represents a Policy Usage Subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

netSliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

reqPolicyRep:

$ref: '#/components/schemas/ReqPolRep'

repPeriodicity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- netSliceId

- reqPolicyRep

PolUsageSubscPatch:

description: >

Represents the requested modifications to a Policy Usage Subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reqPolicyRep:

$ref: '#/components/schemas/ReqPolRep'

repPeriodicity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

ReqPolRep:

description: >

Represents the requested policy usage reporting information.

type: object

properties:

policyId:

type: string

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- policyId

- startTime

- endTime

PolUsageNotif:

description: >

Represents a Policy Usage Notification.

type: object

properties:

subscriptionId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/PolRepData'

minItems: 1

required:

- subscriptionId

- reports

PolRepData:

description: >

Represents data access statistics.

type: object

properties:

policyId:

type: string

count:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeSpent:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

preEmptCount:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

preEmptPolId:

type: string

required:

- policyId

- count

- timeSpent

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

Annex B (informative):
Withdrawn API versions

B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [3] clause 4.3.1.6 describes the withdrawal of API versions.

B.2	<Service 1> API

Where <Service 1> is to be replaced by the name of the Service (e.g. UAE_C2OperationModeManagement).

One clause is introduced per Service.

The API versions listed in table B.2-1 are withdrawn for the <Service 1> API.

Table B.2-1: Withdrawn API versions of the <Service 1> service

| API version number | Remarks |
| --- | --- |

In the Remarks column, the deficits leading to the withdrawal of an API version are explained.

B.3	<Service 2> API

And so on if there are more than two services supported by the NF.

Annex <X> (informative):
Change history

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | CT3#131 | C3-235577 |  |  |  | TS skeleton. | 0.0.0 |
| 2023-11 | CT3#131 | C3-235466 |  |  |  | Including C3-235630, C3-235631, C3-235632, C3-235550, C3-235551, C3-235578, C3-235579 and C3-235580. | 0.0.0 |