3GPP TS 29.525 V18.7.0 (2024-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; UE Policy Control Service;

Stage 3

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, 5G System

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	10

2	References	10

3	Definitions and abbreviations	12

3.1	Definitions	12

3.2	Abbreviations	12

4	UE Policy Control Service	13

4.1	Service Description	13

4.1.1	Overview	13

4.1.2	Service Architecture	14

4.1.3	Network Functions	15

4.1.3.1	Policy Control Function (PCF)	15

4.1.3.2	NF Service Consumers	17

4.2	Service Operations	18

4.2.1	Introduction	18

4.2.2	Npcf_UEPolicyControl_Create Service Operation	18

4.2.2.1	General	18

4.2.2.2	UE Policy	24

4.2.2.2.1	Overview	24

4.2.2.2.1.0	General	24

4.2.2.2.1.1	Provisioning of the UE Access Network discovery and selection policies and UE Route Selection Policy	27

4.2.2.2.1.1a	Provisioning of URSP in EPS	28

4.2.2.2.1.2	Provisioning of Vehicle-to-Everything Policy	29

4.2.2.2.1.3	Provisioning of ProSe Policy	30

4.2.2.2.1.4	Provisioning of Aircraft-to-Everything Policy	30

4.2.2.2.1.5	Provisioning of Ranging and Sidelink Positioning Policy	30

4.2.2.2.2	UE Access Network discovery and selection policies (ANDSP)	31

4.2.2.2.3	UE Route Selection Policy (URSP)	31

4.2.2.2.3.1	General	31

4.2.2.2.3.2	Provisioning of VPLMN-specific URSP Rules	35

4.2.2.2.4	Vehicle-to-Everything Policy (V2XP)	36

4.2.2.2.5	Proximity based Services Policy (ProSeP)	36

4.2.2.2.6	Aircraft-to-Everything Policy (A2XP)	36

4.2.2.2.7	Ranging and Sidelink Positioning Policy (RSLPP)	37

4.2.2.3	V2X N2 PC5 Policy	37

4.2.2.4	5G ProSe N2 PC5 Policy	37

4.2.2.5	A2X N2 PC5 Policy	37

4.2.2.6	Ranging/SL N2 PC5 Policy	38

4.2.3	Npcf_UEPolicyControl_Update Service Operation	38

4.2.3.1	General	38

4.2.3.2	Policy Control Request Triggers	43

4.2.3.3	Encoding of updated policy	44

4.2.3.4	Feature renegotiation during AMF relocation	45

4.2.4	Npcf_UEPolicyControl_UpdateNotify Service Operation	46

4.2.4.1	General	46

4.2.4.2	Policy update notification	46

4.2.4.3	Request for termination of the policy association	48

4.2.4.4	URSP provisioning for Background Data Transfer policy	50

4.2.4.5	UE policy provisioning for V2X communication over PC5 and Uu reference points	50

4.2.4.6	UE policy provisioning for 5G ProSe	50

4.2.4.7	UE policy provisioning for AF-influenced URSP	50

4.2.4.8	UE policy provisioning for A2X communication over PC5 and A2X communication over Uu reference point	51

4.2.4.9	URSP provisioning in EPS	51

4.2.4.10	UE policy provisioning for Ranging/SL	52

4.2.5	Npcf_UEPolicyControl_Delete Service Operation	52

5	Npcf_UEPolicyControl API	54

5.1	Introduction	54

5.2	Usage of HTTP	54

5.2.1	General	54

5.2.2	HTTP standard headers	54

5.2.2.1	General	54

5.2.2.2	Content type	54

5.2.3	HTTP custom headers	54

5.3	Resources	55

5.3.1	Resource Structure	55

5.3.2	Resource: UE Policy Associations	55

5.3.2.1	Description	55

5.3.2.2	Resource definition	55

5.3.2.3	Resource Standard Methods	56

5.3.2.3.1	POST	56

5.3.3	Resource: Individual UE Policy Association	56

5.3.3.1	Description	56

5.3.3.2	Resource definition	56

5.3.3.3	Resource Standard Methods	57

5.3.3.3.1	GET	57

5.3.3.3.2	DELETE	58

5.3.3.4	Resource Custom Operations	59

5.3.3.4.1	Overview	59

5.3.3.4.2	Operation: Update	59

5.3.3.4.2.1	Description	59

5.3.3.4.2.2	Operation Definition	59

5.4	Custom Operations without associated resources	60

5.5	Notifications	61

5.5.1	General	61

5.5.2	Policy Update Notification	61

5.5.2.1	Description	61

5.5.2.2	Operation Definition	61

5.5.3	Request for termination of the UE policy association	62

5.5.3.1	Description	62

5.5.3.2	Operation Definition	62

5.6	Data Model	63

5.6.1	General	63

5.6.2	Structured data types	66

5.6.2.1	Introduction	66

5.6.2.2	Type PolicyAssociation	67

5.6.2.3	Type PolicyAssociationRequest	69

5.6.2.4	Type PolicyAssociationUpdateRequest	72

5.6.2.5	Type PolicyUpdate	76

5.6.2.6	Type TerminationNotification	77

5.6.2.7	Type UePolicyTransferFailureNotification	78

5.6.2.8	Type UeRequestedValueRep	79

5.6.2.9	Type UePolicyParameters	80

5.6.2.10	Type LboRoamingInformation	80

5.6.2.11	Type UrspEnforcementPduSession	81

5.6.2.12	Type UePolicyNotification	81

5.6.3	Simple data types and enumerations	81

5.6.3.1	Introduction	81

5.6.3.2	Simple data types	82

5.6.3.3	Enumeration: RequestTrigger	82

5.6.3.4	Enumeration: PolicyAssociationReleaseCause	84

5.6.3.5	Enumeration: Pc5Capability	84

5.6.3.6	Enumeration: ProSeCapability	84

5.6.3.8	Void	85

5.6.3.9	Enumeration: N1N2MessTransferErrorReply	85

5.6.3.10	Enumeration: RangSLCapability	85

5.6.3.11	Enumeration: PolicyStatus	86

5.6.3.12	Enumeration: A2xCapability	86

5.6.4	Data types describing alternative data types or combinations of data types	86

5.6.4.1	Type: UePolicyTransferFailureCause	86

5.7	Error handling	87

5.7.1	General	87

5.7.2	Protocol Errors	87

5.7.3	Application Errors	87

5.8	Feature negotiation	88

5.9	Security	90

Annex A (normative): OpenAPI specification	91

A.1	General	91

A.2	Npcf_UEPolicyControl API	91

Annex B (normative): Wireless and wireline convergence access support	106

B.1	Scope	106

B.2	Npcf_UEPolicyControl Service	106

B.2.1	Service Description	106

B.2.1.1	Overview	106

B.2.1.2	Service Architecture	107

B.2.1.3	Network Functions	107

B.2.1.3.1	Policy Control Function (PCF)	107

B.2.1.3.2	NF Service Consumers	107

B.3	Service Operations	107

B.3.1	Introduction	107

B.3.2	Npcf_UEPolicyControl_Create Service Operation	108

B.3.2.1	General	108

B.3.3	Npcf_UEPolicyControl_Update Service Operation	108

B.3.3.1	General	108

B.3.4	Npcf_UEPolicyControl_UpdateNotify Service	108

B.3.4.1	General	108

B.3.5	Npcf_UEPolicyControl_Delete Service Operation	109

B.3.5.1	General	109

Annex C (informative): Withdrawn API versions	110

Annex D (informative): Change history	111

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification provides the stage 3 definition of the UE Policy Control Service (Npcf_UEPolicyControl) of the 5G System.

The stage 2 definition and procedures of UE Policy Control Service are contained in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4]. The 5G System Architecture is defined in 3GPP TS 23.501 [2].

Stage 3 call flows are provided in 3GPP TS 29.513 [7].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition of the 5G System are specified in 3GPP TS 29.500 [5] and 3GPP TS 29.501 [6].

The UE Policy Control Service is provided by the Policy Control Function (PCF). This service provides UE policies and N2 PC5 policy.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[8]	IETF RFC 9113: "HTTP/2".

[9]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[10]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[11]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[12]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[13]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[14]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[15]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[16]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[17]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[18]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[19]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[20]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[21]	IETF RFC 9457: "Problem Details for HTTP APIs".

[22]	3GPP TR 21.900: "Technical Specification Group working methods".

[23]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[24]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[25]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[26]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository service for Subscription Data; Stage 3".

[27]	3GPP TS 29.504:"5G System; Unified Data Repository Services; Stage 3".

[28]	3GPP TS 24.554: "Proximity based services (ProSe) in 5G system (5GS) protocol aspects; Stage 3".

[29]	3GPP TS 24.555: "Proximity based services (ProSe) in 5G system (5GS); User Equipment (UE) policies; Stage 3".

[30]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[31]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[32]	3GPP TS 24.577: "Aircraft-to-Everything (A2X) services in 5G System (5GS) protocol aspects; Stage 3".

[33]	3GPP TS 24.588: "Aircraft-to-Everything (A2X) services in 5G System (5GS); UE policies".

[34]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3"

[35]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[36]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[37]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[38]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[39]	3GPP TS 29.594: "5G System; Spending Limit Control Service; Stage 3".

[40]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[41]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[42]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system(5GS); Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.503 [4], clause 3.1 and 3GPP TS 23.501 [2], clause 3.1 apply:

VPLMN specific URSP rules

Configured NSSAI

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G-RG	5G Residential Gateway

5G-VN	5G Virtual Network

A2X	Aircraft-to-Everything

A2XP	Aircraft-to-Everything Policy

AMF	Access and Mobility Management Function

ANDSP	Access Network Discovery and Selection Policy

API	Application Programming Interface

CHF	Charging Function

DNN	Data Network Name

EPS	Evolved Packet Core System

FN-RG	Fixed Network Residential Gateway

FN-BRG	Fixed Network Broadband Residential Gateway

FN-CRG	Fixed Network Cable Residential Gateway

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coaxial

HTTP	Hypertext Transfer Protocol

H-PCF	Home Policy Control Function

JSON	JavaScript Object Notation

N3AN	Non-3GPP access network

N3IWF	Non-3GPP InterWorking Function

NID	Network Identifier

NF	Network Function

NRF	Network Repository Function

NSWO	Non-Seamless WLAN Offload

OS	Operating System

OSId	Operating System Identity

PCF	Policy Control Function

PDU	Packet Data Unit

PEI	Permanent Equipment Identifier

PIN	Personal IoT Network

PRA	Presence Reporting Area

ProSeP	5G ProSe Policy

PTI	Procedure Transaction Identity

RSLPP	Ranging and Sidelink Positioning Policy

RSN	Redundancy Sequence Number

SL	Sidelink

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

SSC	Service and Session Continuity

SUPI	Subscription Permanent Identifier

TNGF	Trusted Non-3GPP Gateway Function

UDR	Unified Data Repository

UPSC	UE policy section code

UPSI	UE policy section identifier

URSP	UE Route Selection Policy

V2X	Vehicle-to-Everything

V2XP	Vehicle-to-Everything Policy

V-PCF	Visited Policy Control Function

VPS	VPLMN Specific

W-5GAN	Wireline 5G Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# 4	UE Policy Control Service

## 4.1	Service Description

### 4.1.1	Overview

The UE Policy Control Service, as defined in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4], is provided by the Policy Control Function (PCF).

This service is used as part of the provisioning of UE policies (e.g. ANDSP, URSP, V2XP, A2XP, ProSeP, RSLPP) determined by the PCF to the UE via the AMF and as part of the provisioning of N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL determined by the PCF to the NG-RAN via the AMF. In case of URSP provisioning in EPS this service may be used as part of the provisioning of URSP determined by the PCF to the UE via a PCF for a PDU session. This service hence offers the following functionalities:

-	creation of a UE Policy Association as requested by the NF service consumer (e.g. AMF);

-	provisioning of policy control request trigger(s) to the NF service consumer (e.g. AMF);

-	provisioning of the UE policy (e.g. ANDSP, URSP, V2XP, A2XP,ProSeP, RSLPP) to the V-PCF by the H-PCF in the roaming case;

-	provisioning of the N2 PC5 policy for V2X communications and/or  A2X communications and/or 5G ProSe and/or Ranging/SL to the V-PCF by the H-PCF in the roaming case;

-	update of a UE Policy Association as requested by the NF service consumer (e.g. AMF);

-	reporting of the met policy control request trigger(s) by the NF service consumer;

-	update of policy control request trigger(s) by the PCF to the NF service consumer (e.g. AMF);

-	deletion of a UE Policy Association as requested by the NF service consumer (e.g. AMF);

-	enable the PCF to request the termination of a UE Policy Association to the NF service consumer (e.g. AMF);

-	provisioning of the URSP to a PCF for a PDU session in case of URSP provisioning in EPS; and

-	provisioning of slice-based N3IWF/TNGF selection policies based on the UE subscribed/configured S-NSSAI(s).

### 4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture is also described in 3GPP TS 29.513 [7].

The UE Policy Control Service (Npcf_UEPolicyControl) is part of the Npcf service-based interface exhibited by the Policy Control Function (PCF).

The known NF service consumers of the Npcf_UEPolicyControl service are the Access and Mobility Management Function (AMF) and the Visited Policy Control Function (V-PCF).

The AMF accesses the UE Policy Control Service at the PCF via the N15 reference point. In case of URSP delivery in EPS, when the PCF for the PDU session and the PCF for the UE are different, the PCF for the PDU session accesses the UE Policy Control Service at the PCF via the N43 reference point,

In the roaming scenario, the N15 reference point is located between the V-PCF in the visited network and the AMF. The V-PCF accesses the UE Policy Control Service at the Home Policy Control Function (H-PCF) via the N24 Reference point.

Figure 4.1.2-1: Reference Architecture for the Npcf_UEPolicyControl Service; SBI representation

Figure 4.1.2-2: Non-roaming Reference Architecture for the Npcf_UEPolicyControlService; reference point representation

NOTE 1:	When the N43 reference point exists, i.e. when the PCF is a NF service consumer of the Npcf_UEPolicyControl service, the PCF for the PDU session interacts with the PCF for the UE, and the non-roaming and home routed roaming architecture are the same.

Figure 4.1.3-2: Roaming reference Architecture for the Npcf_UEPolicyControlService; reference point representation

NOTE 2:	In LBO roaming scenarios, the V-PCF for the PDU session interacts with the V-PCF for the UE (i.e., the V-PCF for the PDU session is a NF service consumer of the Npcf_UEPolicyControl service offered by the V-PCF of the UE).

### 4.1.3	Network Functions

#### 4.1.3.1	Policy Control Function (PCF)

For non-roaming scenarios, the Policy Control Function (PCF):

-	supports unified policy framework to govern network behaviour;

-	provides UE policy, including Access Network Discovery and Selection Policy (ANDSP), UE Route Selection Policy (URSP), Vehicle-to-Everything Policy (V2XP), Aircraft-to-Everything Policy (A2XP), 5G ProSe Policy (ProSeP) and/or Ranging and Sidelink Positioning Policy (RSLPP) via the AMF transparently to the UE;

-	provides policy control request trigger(s) to the AMF;

NOTE 1:	The PCF invokes the Namf_Communication service specified in 3GPP TS 29.518 [14] to provide the UE Policy.

-	provides N2 PC5 policy, containing the PC5 QoS parameters used by NG-RAN for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL via the AMF to the NG-RAN;

NOTE 2:	The PCF invokes the Namf_Communication service specified in 3GPP TS 29.518 [14] to provide the N2 PC5 Policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL.

-	provides URSP via a PCF for a PDU session transparently to the UE in case of URSP provisioning in EPS;

-	provides policy control request trigger(s) to a PCF for a PDU session in case of URSP provisioning in EPS; and

-	provides slice-based N3IWF/TNGF selection policies based on the UE subscribed S-NSSAI(s).

For roaming scenarios, the Visited Policy Control Function (V-PCF):

-	provides policy control request trigger(s) to the AMF;

-	provides the ANDSP of the VPLMN via the AMF transparently to the UE;

-	forwards the ANDSP, URSP, V2XP, A2XP, ProSeP and/or RSLPP received from the H-PCF via the AMF to the UE;

NOTE 3:	The V-PCF invokes the Namf_Communication service specified in 3GPP TS 29.518 [14] to provide the UE Policy.

-	forwards the N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL received from the H-PCF via the AMF to the NG-RAN;

NOTE 4:	The V-PCF invokes the Namf_Communication service specified in 3GPP TS 29.518 [14] to provide the N2 PC5 Policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL.

-	in case of AF guidance of VPLMN-specific URSP rules for inbound roamers:

a.	provides the AF guidance for VPLMN-specific URSP rules to the H-PCF; and

b.	if requested by the AF, notifies the NEF in the VPLMN about the outcome of the delivery of VPLMN-specific URSP rules;

-	provides slice-based N3IWF/TNGF selection policies based on the UE Configured NSSAI; and

-	for the LBO roaming scenarios, and in case of URSP provisioning in EPS:

a.	provides policy control request trigger(s) received from the H-PCF to a V-PCF for a PDU session; and

b.	forwards to the UE the URSP received from the H-PCF using a V-PCF for a PDU session.

For roaming scenarios, the Home Policy Control Function (H-PCF):

-	provides policy control request trigger(s) to the V-PCF;

-	provides the UE policy (e.g. ANDSP, URSP, V2XP, A2XP, ProSeP or RSLPP) of the HPLMN to the V-PCF for forwarding to the UE via the the AMF;

-	provides the N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL to the V-PCF for forwarding to the NG-RAN via the AMF;

-	in case of AF guidance of VPLMN-Specific URSP rules:

a.	provides VPLMN-specific URSP rules to capable UE(s) using AF guidance for VPLMN-specific URSP rules retrieved from the HPLMN UDR and/or received from the V-PCF; and

b.	notifies the NEF in the HPLMN and the V-PCF about the outcome of the VPLMN-specific URSP rules delivery to the UE;

-	in case of URSP provisioning in EPS:

a.	for the LBO roaming scenarios, provides URSP to the V-PCF for forwarding to the UE via a V-PCF for a PDU session.

b.	for the Home Routed scenarios, provides URSP to the PCF for the PDU session in the HPLMN, for forwarding to the UE via the H-SMF.

The policy decisions made by the PCF may also be based on one or more of the following:

-	Information obtained from the UDR (e.g., UE Policy Subscription data and/or Service Parameter Data provided by the AF/NEF via the UDR);

-	Information obtained from the AMF, e.g., UE related and access related information;

-	Information obtained from the NWDAF;

-	Information from the CHF about spending limits control, e.g., status of each relevant policy counter and optional pending policy counter statuses; and

NOTE 5:	In this release of the specification, policy decisions based on spending limits control apply to URSP only.

-	PCF pre-configured policy context.

#### 4.1.3.2	NF Service Consumers

The known NF service consumers of the Npcf_UEPolicyControl are the AMF, the V-PCF in the roaming case, and a PCF for a PDU session in case of URSP provisioning in EPS.

The Access and Mobility Management function (AMF) performs:

-	registration management;

-	connection management;

-	reachability management;

-	mobility Management;

-	forwarding of UE Policy towards the served UE;

-	reporting of the UE state to the (V-)PCF;

-	forwarding of the UE policy enforcement result received from the UE to the (V-)PCF; and

NOTE:	The AMF invokes the Namf_Communication service specified in 3GPP TS 29.518 [14] to report the UE policy enforcement result.

-	forwarding of the N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL towards the NG-RAN.

The Visited Policy Control Function (V-PCF) provides the functions described in clause 4.1.3.1 towards the visited network as NF service producer and acts as NF Service consumer toward the H-PCF, performing the following functions:

-	receiving policy control request trigger(s) and/or UE policy (e.g. ANDSP, URSP, V2XP, A2XP, ProSeP, RSLPP) from the H-PCF;

-	receiving the N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe and/or Ranging/SL from the H-PCF; and

-	reporting of the UE state and UE policy enforcement result to the H-PCF.

-	providing the URSP rule enforcement information received from the UE to the H-PCF, if requested by the H-PCF as described in clause 4.2.2.2.3.

The PCF for a PDU session in case of URSP provisioning in EPS performs:

-	forwarding of URSP towards the served UE and the URSP rule provisioning outcome from the UE; and

-	forwarding towards/from the selected PDN connection the provisioning/report of the policy control request trigger(s).

## 4.2	Service Operations

### 4.2.1	Introduction

Table 4.2.1-1: Operations of the Npcf_UEPolicyControl Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Npcf_UEPolicyControl_Create | Creates a UE Policy Association. | NF service consumer (e.g. AMF, V-PCF in roaming case) |
| Npcf_UEPolicyControl_Update | Updates a UE Policy Association and provides the corresponding policies to the NF service consumer when policy control request trigger(s) is/are met or the AMF is relocated due to the UE mobility and the old PCF is selected. | NF service consumer (e.g. AMF, V-PCF in roaming case) |
| Npcf_UEPolicyControl_UpdateNotify | - Provides the updated policy control request trigger(s) and/or applicable indications to the AMF by the (V-)PCF in the non-roaming or roaming case; - Provides the updated UE policy, applicable indications, and/or policy control request trigger(s) to the V-PCF by the H-PCF; or - Initiates the UE Policy association termination towards the NF service consumer by the NF service producer. | PCF (H-PCF and V-PCF in roaming case) |
| Npcf_UEPolicyControl_Delete | Provides means for the NF service consumer to delete a UE Policy Association. | NF service consumer (e.g. AMF, V-PCF in roaming case) |

### 4.2.2	Npcf_UEPolicyControl_Create Service Operation

#### 4.2.2.1	General

The procedure in the present clause is applicable when the NF service consumer creates a UE policy association in the following cases:

-	UE performs initial registration to the network, as defined in clause 5.5.1.2.2 of 3GPP TS 24.501 [15];

-	UE performs a mobility registration, if the UE operating in single-registration mode performs inter-system change from S1 mode to N1 mode, as defined in clause 5.5.1.3.2 of 3GPP TS 24.501 [15], and there is no existing UE Policy Association between AMF and PCF for this UE; and

-	the AMF is relocated (between the different AMF sets) and the new AMF selects a new PCF. The procedure for the case where the AMF is relocated and the new AMF selects the old PCF is defined in clause 4.2.3.1.

To support the delivery of URSP in EPS, the procedure in the present clause is also applicable when:

-	When the UE triggers a BEARER RESOURCE MODIFICATION REQUEST message with a UE policy container IE after the UE performs ePCO capability negotiation during PDN connection establishment procedure (during the Initial Attach with default PDN connection establishment or during the first PDN connection establishment or during PDN connection modification without QoS update or during new PDN connection establishment when no other existing PDN connection indicates support of URPS provisioning in EPS) as defined in 3GPP TS 24.301 [33], and both, the UE and the network support URSP provisioning in EPS PCO; and

-	5GS to EPS handover or 5GS to EPS Idle Mode mobility (both referred as 5GS to EPS mobility in the present document) as defined in 3GPP TS 24.501 [15] and if the UE and at least one of the PDN connection(s) supports URSP delivery in EPS as specified in 3GPP TS 29.512 [31].

The creation of a UE policy association only applies for normally registered UEs, i.e. it does not apply for emergency-registered UEs.

Figure 4.2.2.1-1 illustrates the procedure used for the creation of a policy association.

Figure 4.2.2.1-1: Creation of a UE policy association

NOTE 1:	For the roaming scenario, the PCF represents the V-PCF, if the NF service consumer is an AMF, and the PCF represents the H-PCF, if the NF service consumer is a V-PCF.

When a UE registers to the network and a UE context is being established, if the AMF obtains from the UE a UE policy delivery protocol message as defined in Annex D of 3GPP TS 24.501 [15] and/or the authorized PC5 capability for 5G ProSe, and/or the authorized PC5 capability for V2X communications and/or A2X communications, and/or the authorized PC5 capability for Ranging/SL, the AMF shall establish a UE policy association with the (V-)PCF, in case there is no existing UE policy association for the UE; otherwise, the AMF may establish a UE Policy Association with the (V-)PCF based on AMF local configuration.

NOTE 2:	In the roaming scenario, the visited AMF's local configuration can indicate whether UE Policy delivery is needed based on the roaming agreement with the home PLMN of the UE.

During UE Initial Attach with default PDN connection or the establishment of the first PDN connection in EPS or a new PDN connection when no other existing PDN connection indicates the support of URSP provisioning in EPS, if the UE and the SMF+PGW support URSP provisioning in EPS PCO, and the "EpsUrsp" feature is supported between the SMF+PGW-C and the PCF for the PDU session, the PCF for a PDU session associated with the SMF+PGW-C serving the PDN connection obtains from the UE a UE policy container in a Npcf_SMPolicyControl_Update procedure triggered by a bearer resource modification procedure as described in 3GPP TS 29.512 [31]. Then, if the "EpsUrsp" feature described in clause 5.8 is supported, the PCF for a PDU session shall establish a UE policy association with the (V-)PCF for the UE for the delivery of URSP only.

During 5GS to EPS mobility with N26, and if the "EpsUrsp" feature described in clause 5.8 is supported, the PCF for the PDU session determines whether 5GS to EPS mobility applies based on the received RAT and/or Access-Type change event as described in 3GPP TS 29.512 [31]. Then, for non-roaming and Home Routed roaming scenarios, the PCF for a PDU session shall determine whether the UE supports URSP provisioning in EPS by checking the UE Policy Set information in UDR as specified in 3GPP TS 29.519 [17], and if supported, shall establish a UE policy association with the PCF for the UE that is handling the UE policy association with the source AMF. For LBO roaming scenarios, the V-PCF for the PDU session determines based on local configuration whether to establish a UE Policy Association towards the V-PCF for the UE.

NOTE 3:	The PCF for the PDU session discovers the address of the PCF for the UE handling the UE policy association with the source AMF by querying the BSF as described in 3GPP TS 29.521 [22].

NOTE 4:	If during the 5GS to EPS mobility there are more than one PCF for the PDU session maintaining PDN connections for the UE, every PCF for the PDU session establishes a UE Policy Association towards the PCF for the UE. In LBO scenarios, the V-PCF for the UE will handle only one UE Policy Association towards the H-PCF for the UE.

To establish a UE policy association with the PCF, the NF service consumer (e.g. AMF) shall send an HTTP POST request with "{apiRoot}/npcf-ue-policy-control/v1/policies" as Resource URI and the PolicyAssociationRequest data structure as request body, which shall include:

-	the Notification URI encoded as "notificationUri" attribute;

-	the SUPI encoded as "supi" attribute; and

-	the features supported by the NF service consumer encoded as "suppFeat" attribute,

shall also include, when available:

-	the GPSI encoded as "gpsi" attribute;

-	the Access type encoded as "accessType" attribute;

-	the Permanent Equipment Identifier (PEI) encoded as "pei" attribute;

-	the User Location Information encoded as "userLoc" attribute;

-	the UE Time Zone encoded as "timeZone" attribute;

-	the identifier of the serving network (the PLMN Identifier or the SNPN Identifier), encoded as "servingPlmn" attribute;

NOTE 5:	The SNPN Identifier consists of the PLMN Identifier and the NID.

-	the RAT type encoded as "ratType" attribute;

-	the received UE policy delivery protocol message defined in Annex D of 3GPP TS 24.501 [15] encoded as "uePolReq" attribute;

-	for the roaming scenario, if the NF service consumer is an AMF, the H-PCF ID encoded as "hPcfId" attribute, and if the "EnhEstRoaming" feature is supported, the H-PCF URI encoded as the "hPcfUri" attribute and the H-PCF Set ID encoded as "hPcfSetId" attribute;

-	the Internal Group Identifier(s) encoded as "groupIds" attribute;

-	the PC5 capability for V2X encoded as "pc5Capab" attribute if the "V2X" feature defined in clause 5.8 is supported;

-	the 5G ProSe capability within the "proSeCapab" attribute, if the "ProSe" feature defined in clause 5.8 is supported;

-	the Ranging/SL capability within the "rangSlCapab" attribute, if the "Ranging_SL" feature defined in clause 5.8 is supported;

-	if the NF service consumer is an AMF, the GUAMI encoded as "guami" attribute;

-	if the NF service consumer is an AMF, the serving AMF Id encoded as "servingNfId" attribute;

NOTE 6:	If the PCF received the "servingNfId" attribute, the PCF can use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] to retrieve the NF profile of the Namf_Communication service available in the indicated AMF instance Id.

-	if the NF service consumer is an AMF and the "SliceAwareANDSP" feature is supported:

-	if the UE indicated the support of slice-based N3IWF and/or TNGF selection as specified in 3GPP TS 24.501 [15], the AMF may provide information about these UE indications within the "sliceN3gNodeSelCap" attribute;

-	if the AMF has determined that the UE has selected a non-3gpp access node (i.e. TNGF or N3IWF) that is not compatible with the allowed S-NSSAI(s), and the UE indicated the support of slice-based N3IWF and/or TNGF selection as specified in 3GPP TS 24.501 [15], the wrongly selected type of non-3gpp access node encoded as "n3gNodeReSel" attribute, and, in the roaming case, also the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute;

-	if the NF service consumer is an AMF, the Satellite Backhaul Category encoded as "satBackhaulCategory" attribute, if the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported;

-	if the NF service consumer is the PCF for the PDU session, and the "EpsUrsp" feature defined in clause 5.8 is supported, the indication that the trigger for the UE Policy Association Establishment is the 5GS to EPS mobility scenario encoded as the "5gsToEpsMob" attribute;

-	for the roaming scenario, if the NF service consumer is an AMF and the "NssaiChange" feature is supported, the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute and optionally the mapped each S-NSSAI value of home network corresponding to the configured S-NSSAI values in the serving PLMN encoded as "mappedHomeSnssai" attribute within the "confSnssais" attribute;

-	the A2X capability encoded as "a2xCapab" attribute if the "A2X" feature defined in clause 5.8 is supported;

-	if the feature "AccessChange" is supported, the NF service consumer shall include:

a)	the "accessTypes" attribute indicating registration in the 3GPP access, in the non-3GPP access, or in both 3GPP and non-3GPP access, if available; and

b)	the RAT type entry corresponding to the 3GPP access and/or the RAT type entry corresponding to the non-3GPP access encoded in the "ratTypes" attribute, if available.

NOTE 7:	If the feature "AccessChange" is not supported or it is not known yet whether it is supported in the PCF, the NF service consumer can also provide the "accessType" attribute and the "ratType" attribute, if available, with one available access type and RAT type.

NOTE 8:	When the UE is simultaneously connected to the 5G Core Network of a PLMN/SNPN over a 3GPP access and a non-3GPP access, the UE is served by the same AMF, as specified in 3GPP TS 23.501 [2]. In this case, the UE Policy Association contains both, 3GPP and non-3GPP accesses.
When the UE is simultaneously connected to 5G Core Network over 3GPP access and non-3GPP access in different PLMN(s)/SNPN(s), the UE is served by different AMFs. In this case, there can be two UE Policy Associations, each with the corresponding access type.

-	for the roaming scenario, if the NF service consumer is a V-PCF and the "VPLMNSpecificURSP" feature is supported, the AF guidance on VPLMN-specific URSP rules related information, if applicable, within the "vpsUePolGuidance" attribute, that shall contain for each related AF:

a.	the AF guidance on VPLMN-Specific URSP rules within the "urspGuidance" attribute; and

b.	if the AF requested to the VPLMN notifications about the delivery of UE Policies, the "deliveryEvents" attribute including the "SUCCESS_UE_POL_DEL_SP" and/or "UNSUCCESS_UE_POL_DEL_SP" events; and

-	for the roaming scenario, if the NF service consumer is an AMF, and the "VPLMNSpecificURSP" feature is supported, LBO information within the "lboRoamInfo" attribute.

and may include:

-	if the NF service consumer is an AMF, the name of a service produced by the AMF that expects to receive information via the Npcf_UEPolicyControl_UpdateNotify service operation encoded as "serviceName" attribute;

-	if the NF service consumer is an AMF, the alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

-	if the NF service consumer is an AMF, the alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

-	if the NF service consumer is an AMF, the alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

-	if the NF service consumer is an AMF, the alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute.

Upon the reception of the HTTP POST request,

-	the (V-)(H-)PCF shall assign a UE policy association ID;

-	for the roaming scenario and based on operator policy, the V-PCF (as the NF service consumer) should send to the H-PCF a request for the Creation of a UE policy association as described in the present clause;

-	the (V-)(H-)PCF shall determine the applicable UE policy as detailed in clause 4.2.2.2. For the V-PCF, any policy received from the H-PCF in the reply to the possible request for the Creation of a policy association should be taken into consideration;

-	if the (V-)PCF determines that UE policy needs to be provisioned, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall subscribe to the AMF to notifications on N1 messages for UE Policy Delivery Results using the Namf_Communication_N1N2MessageSubscribe service operation;

(ii)	the (V-)PCF shall send the determined UE policy (e.g. ANDSP, URSP, V2XP, A2XP, ProSeP, RSLPP) using Namf_Communication_N1N2MessageTransfer service operation(s); and

(iii) the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the AMF and/or subsequent UE policy requests (e.g. for V2XP and/or A2XP and/or ProSeP and/or RSLPP) within the Namf_Communication_N1MessageNotify service operation. For the V-PCF, if the received UE Policy Delivery results relate to UE policy sections provided by the H-PCF, the V-PCF shall use the Npcf_UEPolicyControl_Update Service Operation defined in clause 4.2.3 to send those UE Policy Delivery results to the H-PCF;

-	if the UE indicates the support of V2X communications over PC5 reference point and the "V2X" feature is supported, the (H-)PCF shall determine the applicable V2XP, as detailed in clause 4.2.2.2.1.2, and V2X N2 PC5 policy, as detailed in clause 4.2.2.3 and based on the operator's policy;

-	if the UE indicates the support of 5G ProSe and the "ProSe" feature is supported, the (H-)PCF shall determine the applicable ProSeP, as detailed in clause 4.2.2.2.1.3, and 5G ProSe N2 PC5 policy, as detailed in clause 4.2.2.4 and based on the operator's policy;

-	if the UE indicates the support of Ranging/SL and the "Ranging_SL" feature is supported, the (H-)PCF shall determine the applicable RSLPP, as detailed in clause 4.2.2.2.1.5, and Ranging/SL N2 PC5 policy, as detailed in clause 4.2.2.7, and based on the operator's policy;

-	if the PCF determines that N2 PC5 policy (e.g., for V2X communications, for 5G ProSe, for Ranging/SL) needs to be provisioned, including the case of the V-PCF when receiving the N2 PC5 policy from the H-PCF, the PCF shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the N2 PC5 policy according to clause 4.2.2.3 and/or clause 4.2.2.4;

-	if the UE indicates support for URSP provisionng in EPS, the "EpsUrsp" feature is supported, and the (V-)PCF determines that UE policy needs to be provisioned via a PCF for a PDU session, the (V-)PCF shall select a UE Policy Association and shall provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall send a UE policy container with the determined URSP using Npcf_UEPolicyControl_Create response service operation(s); and

(ii) the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the PCF for a PDU session. The PCF for a PDU session shall use the Npcf_UEPolicyControl_Update service operation defined in clause 4.2.3 to send those UE Policy Delivery results to the (V-)PCF;

-	if the UE indicates the support of A2X communications over PC5 reference point and the "A2X" feature is supported, the (H-)PCF shall determine the applicable A2XP, as detailed in clause 4.2.2.2.1.4, and A2X N2 PC5 policy, as detailed in clause 4.2.2.5 and based on the operator's policy;

for the successful case, the (V-)(H-)PCF shall send a HTTP "201 Created" response with the URI for the created resource in the "Location" header field.

NOTE 9:	The assigned policy association ID is part of the URI for the created resource and is thus associated with the SUPI.

and the PolicyAssociation data type as response body, including:

-	mandatorily, the negotiated supported features encoded as "suppFeat" attribute;

-	optionally, the information provided by the NF service consumer when requesting the creation of this policy association encoded as "request" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, N2 PC5 policy (see clause 4.2.2.3 and/or clause 4.2.2.4 and/or clause 4.2.2.5 and/or clause 4.2.2.6) encoded as "n2Pc5Pol" attribute (for V2X communications) and/or "n2Pc5PolA2x" attribute (for A2X communications) and/or "n2Pc5ProSePol" attribute (for 5G ProSe) and/or "n2Pc5RsppPol" attribute (for Ranging/SL);

-	optionally, for the H-PCF as service producer communicating with the V-PCF, and when the feature "UECapabilityIndication" is supported, if the H-PCF did not receive from the UE information about ANDSP support and the information is available and reliable in the UDR (see clause 4.2.2.2.1.1), the ANDSP support indication retrieved from UDR encoded as "andspInd" attribute;

-	optionally, for the (V-)PCF communicating with the AMF, and if the "URSPEnforcement" feature is supported, the request to the AMF to be notified about the PDU session established/terminated events by providing the PCF for the UE callback information within the "pcfUeInfo" attribute, and the DNN and S-NSSAI combination of the concerned PDU session(s) within the "matchPdus" attribute.

-	optionally, one or several of the following Policy Control Request Trigger(s) encoded as "triggers" attribute (see clause 4.2.3.2):

a)	Location change (tracking area);

b)	Change of UE presence in PRA;

c)	Change of PLMN, if the "PlmnChange" feature is supported;

d)	Change of UE connectivity state, if the "ConnectivityStateChange" feature is supported;

e)	URSP rule enforcement information, if the "URSPEnforcement" feature is supported;

f)	Change of Satellite Backhaul Category, if the "EnSatBackhaulCategoryChg" feature is supported;

g)	Change of Access Type and RAT Type, if the "AccessChange" feature is supported;

h)	LBO information change, applicable to roaming scenarios, if the "VPLMNSpecificURSP" feature is supported and the NF service consumer is an AMF; and

i)	Change of Configured NSSAI, in roaming scenarios, if the "NssaiChange" feature is supported and the NF service consumer is the AMF;

-	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the presence reporting areas for which reporting is required encoded as "pras" attribute;

-	if the Policy Control Request Trigger "LBO information change" is provided, optionally, the DNNs(s) and S-NSSAI(s) for which LBO information is required encoded as "pduSessions" attribute;

-	if the NF service consumer is an AMF and the "SLAMUP" feature is supported, based on the operator policies the H-PCF indicates that the AMF should select the same CHF that is selected by the H-PCF for a UE, the charging address(es) information encoded in the "chfInfo" attribute.

NOTE 10:		If the PCF uses a Presence Reporting Area identifier referring to a Set of Core Network predefined Presence Reporting Areas as defined in 3GPP TS 23.501 [2], the PCF includes the identifier of this Presence Reporting Area set within the "praId" attribute.

-	if the "SliceAwareANDSP" feature is supported, the PCF received the "n3gNodeReSel" attribute and the PCF has successfully delivered to the UE the ANDSP/WLANSP with the slice selection information for the corresponding non-3gpp node, the indication of the successful UE configuration by providing the "andspDelInd" attribute with the value "CONFIGURED". The PCF may delay the indication of the configuration result to a subsequent Npcf_UEPolicyControl_UpdateNotify request, as described in clause 4.2.4.2.

-	if errors occur when processing the HTTP POST request, the (V-)(H-)PCF shall apply error handling procedures as specified in clause 5.7 and according to the following provisions:

-	if the user information received within the "supi" attribute is unknown, the (V-)(H-)PCF shall reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "USER_UNKNOWN"; and

-	if the (V-)(H-)PCF is, due to incomplete, erroneous or missing information in the request, not able to provision a UE policy decision, the (V-)(H-)PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

If the (V-)PCF received a GUAMI, the (V-)PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF (service) set.

When the "SliceAwareANDSP" feature is supported, and the AMF receives the "andspDelInd" attribute, the AMF, based on operator's policies, may reject the UE Registration request, and may provide a valid target N3IWF/TNGF within the Registration Reject message as specified in clause 5.5.1.3.5 of 3GPP TS 24.501 [15]. In this case, the AMF terminates the UE Policy Association as described in clause 4.2.5 (if the UE is not registered over 3GPP access).

#### 4.2.2.2	UE Policy

##### 4.2.2.2.1	Overview

###### 4.2.2.2.1.0	General

The UE policy consists of

-	UE Access Network discovery and selection policies (ANDSP). It is used by the UE for selecting non-3GPP accesses networks. The encoding of ANDSP is defined in 3GPP TS 24.526 [16];

-	UE Route Selection Policy (URSP). This UE policy is used by the UE to determine how to route outgoing traffic. Traffic can be routed to an established PDU Session, offloaded to non-3GPP access outside a PDU Session, can be routed via a ProSe Layer-3 UE-to-Network Relay outside a PDU session or trigger the establishment of a new PDU Session. The encoding of URSP is defined in 3GPP TS 24.526 [16];

-	UE Vehicle-to-Everything Policy (V2XP). This UE policy provides configuration information to the UE for V2X communications over PC5 reference point or over Uu reference point or both. The encoding of V2XP is defined in 3GPP TS 24.588 [25];

-	UE 5G Proximity based Services Policy (ProSeP). This UE policy provides configuration information to the UE for 5G ProSe direct discovery, 5G ProSe direct communications, 5G ProSe UE-to-network relay, 5G ProSe usage reporting configuration and rules and/or 5G ProSe UE-to-UE relay; and

-	UE Aircraft-to-Everything Policy (A2XP). This UE policy provides configuration information to the UE for A2X communications over PC5 reference point or A2X communications over Uu reference point or both. The encoding of A2XP is defined in 3GPP TS 24.578 [33];

-	UE Ranging and Sidelink Positioning Policy (RSLPP). The UE policy provides configuration information to the UE for Ranging/SL over PC5 reference point. The encoding of RSLPP is defined in 3GPP TS 24.514 [42];

The (V-)(H-)PCF determines the UE Policies that apply to the UE based on the received information from the UE about the list of UE Policies stored in the UE and UE policy classmark information as described in Annex D of 3GPP TS 24.501 [15], the information available in UDR as described in 3GPP TS 29.519 [17], inputs received from the NF service consumer and local policies as described in clause 4.2.2.2.

NOTE 1:	There is a possibility of misalignment between the UE Policies determined by the PCF and the UE Policies stored at the UE (which can be caused e.g. in AMF relocation with PCF reselection scenario). When the PCF, based on configuration and implementation specific means, detects this possibility, the PCF can avoid it if the PCF first removes all the UE Policies in the UE as per the list of UPSI(s) stored in UDR/PCF and then provides to the UE all determined UE Policies as per currently specified procedures.

The UE Policy is transferred to the UE using the UE policy delivery protocol defined in Annex D of 3GPP TS 24.501 [15]. The (V-)(H-)PCF shall send UE policy using the "MANAGE UE POLICY COMMAND" message and will receive the "MANAGE UE POLICY COMPLETE"or the "MANAGE UE POLICY COMMAND REJECT" messages in the response. Those messages are transparently forwarded by the AMF.

The (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send "MANAGE UE POLICY COMMAND" messages to the UE and use the Namf_Communication_N1MessageNotify service operation defined in clause 5.2.2.3.5 of 3GPP TS 29.518 [14] to receive "MANAGE UE POLICY COMPLETE" and "MANAGE UE POLICY COMMAND REJECT" messages from the UE. The (V-)PCF shall only send "MANAGE UE POLICY COMMAND" messages below a predefined size limit.

The H-PCF shall use service operations as defined in the present specification to receive "MANAGE UE POLICY COMPLETE" and "MANAGE UE POLICY COMMAND REJECT" messages from the V-PCF and to send MANAGE UE POLICY COMMAND" messages to the V-PCF. The H-PCF shall encode the "MANAGE UE POLICY COMMAND" message in a "uePolicy" attribute. The H-PCF shall only send "MANAGE UE POLICY COMMAND" messages below a predefined size limit.

The (V-)(H-)PCF may deliver the UE policy to the UE in several "MANAGE UE POLICY COMMAND" messages.

For the purpose of such fragmented delivery and subsequent partial updates of UE policies, the UE policy is divided into policy sections. Such policy sections may be predefined in the (V-)(H-)PCF, may be retrieved by the (V-)(H-)PCF from the UDR as specified in 3GPP TS 29.519 [17], or may be dynamically generated by the (V-)(H-)PCF, but shall comply to the rules detailed below. The (V-)(H-)PCF may combine several policy sections into one "MANAGE UE POLICY COMMAND" message, if the predefined size limit is observed.

The following rules apply to policy sections:

-	The size shall be below the predefined size limit.

-	The policy section shall only contain complete URSP rule(s), WLANSP rule(s), N3AN node configuration information, V2XP, A2XP, ProSeP and/or RSLPP info content, but no fractions of such rules, configuration information, or info contents.

-	To ease a subsequent partial update of UE policies, policy sections should only contain a small number of policies, e.g. URSP rule(s), and/or WLANSP rule(s).

-	The entire content of a policy section shall be provided by a single PLMN.

A PCF shall only determine policy sections of its own PLMN(s). However, a V-PCF may forward UE policy sections received from the H-PCF to the UE.

Each UE policy section is identified by a UE policy section identifier (UPSI). The UPSI is composed of two parts:

a)	a PLMN ID part containing the PLMN ID of the PLMN or SNPN of the PCF which provides the UE policies (i.e, the PLMN ID derived from the SUPI); and

b)	a UE policy section code (UPSC) containing a unique value within the PLMN or SNPN selected by the PCF.

NOTE 2:	When the UE is operating in SNPN access operation mode, the UE associates the PLMN ID with the NID of the SNPN to differentiate between PLMN UPSI(s) and SNPN UPSI(s).

The (V-)(H-)PCF provides an UPSI when providing a new UE policy section and may then identify that policy section using that UPSI when requesting that that UE policy section is modified or deleted, as specified in Annex D of 3GPP TS 24.501 [15].

If the (V-)(H-)PCF determines that changes are required and/or the V-PCF receives possible new or modified policy sections determined by the H-PCF in the roaming case, it shall send the determined new, updated or deleted policy sections using one or several "MANAGE UE POLICY COMMAND" messages towards the NF service consumer. In the roaming case, the V-PCF may either combine policy sections received from the H-PCF and policy sections the V-PCF selected in the same "MANAGE UE POLICY COMMAND" (as long as the predefined size limit is observed), or use separate "MANAGE UE POLICY COMMAND" messages; however, the V-PCF shall not distribute the policy sections received in one "MANAGE UE POLICY COMMAND" from the H-PCF into several "MANAGE UE POLICY COMMAND" messages as long as the predefined size limit is observed for the policy sections received from the H-PCF. The V-PCF shall allocate a new PTI for the "MANAGE UE POLICY COMMAND" sent by the V-PCF and store the mapping between the new PTI and the PTI within the "MANAGE UE POLICY COMMAND" received from the H-PCF.

After sending a "MANAGE UE POLICY COMMAND" messages, the (V-)(H-)PCF shall wait for a related confirmation in a "MANAGE UE POLICY COMPLETE" messages or failure indication in a "MANAGE UE POLICY COMMAND REJECT" message. When receiving no such message until the expiry of a supervision timer specified in Annex D of 3GPP TS 24.501 [15], or when receiving a failure indication, the PCF should re-send related instructions for the policy sections. In the roaming case, the H-PCF and the V-PCF shall each be responsible for resending those policy sections that it originally supplied. In the case that the V-PCF combined policy sections received from the H-PCF and policy sections the V-PCF selected in the same "MANAGE UE POLICY COMMAND" described below, the V-PCF shall wait for the H-PCF to resend the policy sections of HPLMN, and then resend the combined policy sections. The (V-)(H-)PCF shall always include the initially supplied policy sections when resending the UE policy.

The (V-)(H-)PCF shall determine that a received "MANAGE UE POLICY COMPLETE" message or "MANAGE UE POLICY COMMAND REJECT" message is related to the result of a "MANAGE UE POLICY COMMAND" based on the PTI within that message. In the roaming case, the V-PCF shall determine that the received message is related to the result of the UE policy provided by the H-PCF if the PTI within the message belongs to one of the stored PTI mapping(s).

If the V-PCF combined policy sections received from the H-PCF and policy sections the V-PCF selected in the same "MANAGE UE POLICY COMMAND", upon reception of a "MANAGE UE POLICY COMPLETE" message or "MANAGE UE POLICY COMMAND REJECT" message the V-PCF shall:

-	forward the corresponding "MANAGE UE POLICY COMPLETE" message to the H-PCF;

-	if a "MANAGE UE POLICY COMMAND REJECT" message with UPSI(s) of the HPLMN is received, forward the parts of the "MANAGE UE POLICY COMMAND REJECT" message that relate to the UPSI(s) of the HPLMN to the H-PCF;

-	if a "MANAGE UE POLICY COMMAND REJECT" message without UPSI(s) of the HPLMN is received, send a "MANAGE UE POLICY COMPLETE" message to the H-PCF; and

-	provide the stored PTI received from the HPLMN in the corresponding "MANAGE UE POLICY COMMAND" within the "MANAGE UE POLICY COMPLETE" message or "MANAGE UE POLICY COMMAND REJECT" message towards the H-PCF.

If the V-PCF sent a separate "MANAGE UE POLICY COMMAND" containing only the policy sections received from the H-PCF, the V-PCF shall forward the corresponding "MANAGE UE POLICY COMPLETE" or "MANAGE UE POLICY COMMAND REJECT" message to the H-PCF and provide the stored PTI received from the HPLMN in the corresponding "MANAGE UE POLICY COMMAND" within the "MANAGE UE POLICY COMPLETE" message or "MANAGE UE POLICY COMMAND REJECT" message towards the H-PCF.If the V-PCF distributed the policy sections received in one "MANAGE UE POLICY COMMAND" from the H-PCF into several "MANAGE UE POLICY COMMAND" messages to the UE (because the predefined size limit of the VPLMN was exceeded), the V-PCF shall aggregate all corresponding "MANAGE UE POLICY COMPLETE" or "MANAGE UE POLICY COMMAND REJECT" messages received from the UE into one "MANAGE UE POLICY COMPLETE" or "MANAGE UE POLICY COMMAND REJECT" message towards the H-PCF.

When the (V-)PCF receives an Namf_Communication_N1N2MessageTransfer failure response as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14], or an N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14], the (V-)PCF shall stop the supervision timer specified in Annex D of 3GPP TS 24.501 [15] corresponding to the affected PTIs. If the "retryAfter" attribute is received, the (V-)PCF should not initiate new UE Policy Delivery request until the timer expires. For the N1N2 Transfer Failure Notification case, the (V-)PCF determines the affected PTIs allocated by the V-PCF based on the resource URI within the "n1n2MsgDataUri" attribute of the N1N2MsgTxfrFailureNotification data structure as defined in clause 6.1.6.2.30 of 3GPP TS 29.518 [14].

NOTE 3:	The (V-)PCF correlates the Namf_Communication_N1N2MessageTransfer request and the corresponding N1N2 Transfer Failure Notification based on the resource URI within the "Location" header included in the response HTTP status code "202 Accepted" of the Namf_Communication_N1N2MessageTransfer response and the resource URI within the "n1n2MsgDataUri" attribute of and N1N2 Transfer Failure Notification. And then the V-PCF determines the affected PTIs related with the resource URI.

For the non-roaming case or the roaming case when the V-PCF determines that the affected UE Policy is related to the V-PLMN, the (V-)PCF may provision the policy control request trigger "CON_STATE_CH" if not provisioned yet. Upon receiving the notification of UE connectivity state change indicating that the UE enters the CM-Connected state, the (V-)PCF may retry to deliver the UE Policy.

For the roaming case and if the V-PCF determines that the affected UE policy is related with the UE policy delivered by the H-PCF, the V-PCF shall send a POST message as defined in clause 4.2.3.1 to notify the H-PCF of the failure of UE policy transfer by including the "uePolTransFailNotif" attribute within the PolicyAssociationUpdateRequest data structure. Within the UePolicyTransferFailureNotification data structure, the V-PCF shall include the cause of the UE Policy Transfer Failure within the "cause" attribute and the PTI(s) allocated by the H-PCF corresponding to the PTI(s) allocated by the V-PCF within the "ptis" attribute. The H-PCF shall stop the supervision timer corresponding to the affected PTIs. In this case, the H-PCF may provision the policy control request trigger "CON_STATE_CH" if not provisioned yet. Upon receiving the notification of UE connectivity state change indicating that the UE enters the CM-Connected state, the H-PCF may retry to deliver the UE Policy. If the feature "EnErrorHandling" is supported and the "retryAfter" attribute is received, the H-PCF should not initiate new UE Policy Delivery request until the timer expires.

When the (H-)PCF receives the "MANAGE UE POLICY COMPLETE" or the "MANAGE UE POLICY COMMAND REJECT" message and determines that this message indicates a UE Policy Delivery outcome to which an NF service consumer has subscribed via a request for service specific parameters, the (H-)PCF shall invoke the Npcf_EventExposure_Notify service operation as defined in clause 4.2.4.2 of 3GPP TS 29.523 [30].

###### 4.2.2.2.1.1	Provisioning of the UE Access Network discovery and selection policies and UE Route Selection Policy

During Initial Registration and 5GS Registration during UE mobility from EPS to 5GS, and when:

a)	the UE has one or more stored UE policy sections corresponding to the serving PLMN/SNPN or HPLMN;or

b)	the UE does not have any stored UE policy section corresponding to the serving PLMN/SNPN or HPLMN and the UE needs to send a UE policy container to the network;

Then the UE includes the "UE STATE INDICATION" message as defined in clause D.5.4.1 of 3GPP TS 24.501 [15], which is transferred transparently by the AMF within the "uePolReq" attribute during the creation of a policy association, as described in clause 4.2.2.1.

The (H-)PCF, or the PCF of the SNPN for the UEs subscribed to the SNPN, may store in the UDR, as specified in 3GPP TS 29.519 [17]:

a)	UPSCs and related UE policy sections of the own PLMN or SNPN it provided to a UE;

b)	the PEI received from the NF service consumer (e.g. AMF), if available;

c)	the OSId(s) received from the UE within the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available;

d)	the indication of UE's support for ANDSP included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available;

e)	if the "EpsUrsp" feature defined in 3GPP TS 29.519 [17] is supported, the indication of UE's support for URSP provisioning in EPS included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available;

f)	if the "URSPEnforcement" feature defined in 3GPP TS 29.519 [17] is supported, the indication of UE's support for reporting URSP rule enforcement included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available; and

g)	if the "VPLMNSpecificURSP" feature defined 3GPP TS 29.519 [17] is supported, the indication of UE's support for VPLMN-Specific URSP included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available.

The PCF shall retrieve from UDR the information previously stored in UDR, if not locally available, for URSP/ANDSP rule determination as specified in 3GPP TS 29.519 [17].

The V-PCF may retrieve UPSCs and related UE policy sections applicable for all UEs from a HPLMN from the V-UDR, using the HPLMN ID as key as specified in 3GPP TS 29.519 [17]. The PCF of the serving SNPN has locally configured the UPSCs and related UE policy sections applicable for all UEs other than the UEs subscribed to the SNPN.

When receiving the "UE STATE INDICATION" message, the (V-)(H-)PCF or the PCF of the serving SNPN, shall determine, based on the UPSIs indicated in that message, if available, the ANDSP support indication and the OSId(s) indicated in that message, if available, the reporting URSP rule enforcement support in that message, if available, the UE Policy Sections and UPSCs stored in the UDR, if available, the policy subscription data, if available, application data, if available, inputs received from the NF service consumer,and local policy, as specified in clauses 4.2.2.2.2 and 4.2.2.2.3, whether any new UE policy section(s) need to be installed and whether any existing UE policy section(s) need to be updated or deleted. Based on local configuration, the (H-)PCF or the PCF of the serving SNPN (for the SNPN-subscribed UEs), may indicate to the UE to accept/not accept URSP rules signalled by non-subscribed SNPNs within the UE policy network classmark IE in a MANAGE UE POLICY COMMAND message as described in Annex D of 3GPP TS 24.501 [15].

NOTE 1:	When an SNPN-enabled UE registers in a SNPN using credentials from a Credentials Holder (CH) but the UE is not subscribed in that SNPN, the PCF of the non-subscribed SNPN, based on local policies, can provision the UE with URSP rules and/or ANDSP rules for the SNPN. For the provisioned ANDSP rules, the UE gives priority to the valid ANDSP from the registered SNPN.

When the received "UE STATE INDICATION" message indicated that the UE supports VPLMN-specific URSP rules as specified in Annex D of 3GPP TS 24.501 [15], the (H-)PCF may determine URSP rules specific per VPLMN as specified in clause 4.2.2.2.3.2. In this case, the (H-)PCF shall provide to the UE within the "MANAGE UE POLICY COMMAND" the URSP rules to be applied in VPLMN(s) in specific UE policy section(s) and the VPS URSP configuration IE as specified in clause D.6.8 of 3GPP TS 24.501 [15].

NOTE 2:	The VPS URSP configuration IE includes zero or more tuples, each tuple containing a tuple Id, VPLMN ID(s) and a list of UPSC(s) (of HPLMN's UE policy sections) with UE policies with URSP rules applicable to the VPLMN(s) and its equivalent PLMN(s).

###### 4.2.2.2.1.1a	Provisioning of URSP in EPS

When the UE initially attaches in EPS and establishes the default PDN connection or establishes the first PDN connection in EPS or when the UE establishes a new PDN connection and no other existing PDN connection indicates the support of URSP provisioning in EPS, if the "EpsUrsp" feature is supported as described in 3GPP TS 29.512 [31], and both the UE and the network support URSP provisioning in EPS PCO, the UE includes the UE policy container IE with the "UE STATE INDICATION" message as defined in clause D.5.4.1 of 3GPP TS 24.501 [15] in the BEARER RESOURCE MODIFICATION REQUEST message as defined in 3GPP TS 24.301 [36]. The UE policy container is then transferred transparently by the PCF for the PDU session within the "uePolReq" attribute during the creation of a UE policy association, as described in clause 4.2.2.1.

The (H-)PCF for the UE, may store in the UDR, as specified in 3GPP TS 29.519 [17]:

a)	UPSCs and related URSP sections of the own PLMN it provided to a UE;

b)	the PEI received from the NF service consumer, if available;

c)	the OSId(s) received from the UE within the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available;

d)	if the "EpsUrsp" feature defined in 3GPP TS 29.519 [17] is supported, the indication of UE's support for URSP provisioning in EPS included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available; and

f)	if the "URSPEnforcement" feature defined in 3GPP TS 29.519 [17] is supported, the indication of UE's support for reporting URSP rule enforcement included in the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available.

The (H-)PCF shall retrieve from UDR the information previously stored in UDR, if not locally available, for URSP rule determination as specified in 3GPP TS 29.519 [17].

NOTE 1:	URSP provisioning in EPS is supported in Home Routed roaming scenarios as it is supported in non-roaming scenarios. In Home Routed roaming scenarios the H-PCF corresponds with the PCF.

When receiving the "UE STATE INDICATION" message, the (H-)PCF, shall determine, based on the UPSIs indicated in that message, if available, the OSId(s) indicated in that message, if available, the reporting URSP rule enforcement support in that message, if available, the UE Policy Sections and UPSCs stored in the UDR, if available, the policy subscription data, if available, application data, if available, and local policy, as specified in clauses 4.2.2.2.2 and 4.2.2.2.3, whether any new URSP section(s) need to be installed and whether any existing URSP section(s) need to be updated or deleted.

During 5GS to EPS mobility with N26, when the "EpsUrsp" feature is supported and PCF for the PDU session establishes a UE Policy Association with the PCF for the UE as described in clause 4.2.2.1, the PCF for the UE shall determine whether the 5GS to EPS mobility with N26 scenario applies based on the "5gsToEpsMob" attribute. If it applies, the PCF for the UE shall recover from the UE Policy Association previously established with the AMF:

-	UE Policy Section related information, i.e.:

a)	UPSCs and related URSP sections of the own PLMN it provided to the UE;

b)	if the "URSPEnforcement" feature defined in 3GPP TS 29.519 [17] is supported, the indication of UE's support for reporting URSP rule enforcement received from the UE within the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available; and

c)	the OSId(s) received from the UE within the "UE STATE INDICATION" message as described in the Annex D of 3GPP TS 24.501 [15], if available; and

-	the subscribed Policy Control Triggers with the AMF, if available.

NOTE 2:	At 5GS to EPS mobility with N26, the guard timer in the AMF (as specified in clause 4.11.1.2.1 and clause 4.11.1.3.2 of TS 23.502 [3]) ensures that the UE Policy Association remains until the PCF for the UE detects that a UE Policy Association establishment is received from a PCF for the PDU Session indicating 5GS to EPS mobility.

When receiving the 5GS to EPS mobility indication, the PCF for the UE, shall determine, based on the UE Policy Sections and the OSId(s) recovered from the former UE Policy Association in 5GS, if available, the policy subscription data, if available, application data, if available, and local policy, as specified in clauses 4.2.2.2.2 and 4.2.2.2.3, whether any new UE Policy section(s) with URSP need to be installed and whether any existing UE Policy section(s) with URSP need to be updated or deleted.

In the scenarios above, initial attach and/or first PDN connection establishment in EPS and/or new PDN connection establishment in EPS when no other existing PDN connection indicates the support of URSP provisioning in EPS scenarios, and 5GS to EPS mobility scenario, the determined URSP is transferred to the UE as specified in 4.2.2.2.1.0 with the following differences:

-	the messages of the UE policy delivery protocol defined in Annex D of 3GPP TS 24.501 [15] are transparently forwarded to the UE by a PCF for a PDU session;

-	the (V-)(H-)PCF shall use the Npcf_UEPolicyControl_Create/Update response and the Npcf_UEPolicyControl_UpdateNotify request to send "MANAGE UE POLICY COMMAND" messages to the UE in a "uePolicy" attribute and use the Npcf_UEPolicyControl_Update service operation to receive "MANAGE UE POLICY COMPLETE" and "MANAGE UE POLICY COMMAND REJECT" messages from the UE via a PCF for a PDU session in a "uePolDelResult" attribute; and

-	in the 5GS to EPS mobility scenario, the (V-)PCF selects one of the UE Policy Association(s) with the (V-)PCF for the PDU session to send "MANAGE UE POLICY COMMAND" message (and receive the "MANAGE UE POLICY COMPLETE" and "MANAGE UE POLICY COMMAND REJECT" messages from the UE) and to provision the applicable policy control request triggers.

###### 4.2.2.2.1.2	Provisioning of Vehicle-to-Everything Policy

When the UE registers to the network, if the AMF receives from the UE the PC5 capability for V2X communications in the Registration Request message, the UE is authorized to use V2X service based on the UE's subscription information and the "V2X" feature is supported, the AMF further reports to the PCF the PC5 capability for V2X communications within the "pc5Capab" attribute as defined in clause 4.2.2.1. The PCF may determine the V2XP over PC5 interface based on the received UE's PC5 capability for V2X, the Service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

After UE registration, if the UE supports V2X communication and it does not have valid V2XP, the UE includes the "UE POLICY PROVISIONING REQUEST" message as defined in 3GPP TS 24.587 [24] during the NAS transport procedure. The PCF may reject the request by sending back a "UE POLICY PROVISIONING REJECT" message as defined in clause 7.2.2 of 3GPP TS 24.587 [24] or provision the policy, as defined in clause 4.2.2.2.1, based on the service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

For both scenarios mentioned above, in the roaming case, the H-PCF may include the V2XP within the "uePolicy" attribute in the policy association create or update response to the V-PCF and in the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send the V2XP to the UE.

###### 4.2.2.2.1.3	Provisioning of ProSe Policy

When the UE registers to the network and the UE supports 5G ProSe, if the AMF receives from the UE the 5G ProSe Capability in the Registration Request message, the UE is authorized to use 5G ProSe service based on the UE's subscription information and the "ProSe" feature defined in clause 5.8 is supported, the AMF further reports to the PCF this 5G ProSe Capability of the UE within the "proSeCapab" attribute, as per the procedures defined in clause 4.2.2.1. When the UE disables/enables a 5G ProSe capability, the AMF further reports to the PCF the updated 5G ProSe capabilities of the UE within the "proSeCapab" attribute, as per the procedures defined in clause 4.2.3.1. The PCF may determine the support of 5G ProSe based on the received UE's 5G ProSe Capability, the service specific parameter information retrieved from the UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

After UE registration, if the UE does not have valid ProSeP, the UE includes a "UE POLICY PROVISIONING REQUEST" message defined in clause 7.2.1.1 of 3GPP TS 24.554 [28] during theNAS transport procedure. The PCF may either reject the request by sending back a "UE POLICY PROVISIONING REJECT" message defined in clause 7.2.2.1 of 3GPP TS 24.587 [24] or provision the policy, as defined in clause 4.2.2.2.1, based on the service specific parameter information retrieved from the UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

For both scenarios mentioned above, in the roaming case, the H-PCF may include the ProSeP within the "uePolicy" attribute in the policy association create and update response to the V-PCF and in the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send the ProSeP to the UE.

###### 4.2.2.2.1.4	Provisioning of Aircraft-to-Everything Policy

When the UE registers to the network, if the AMF receives from the UE the capability for A2X communications in the Registration Request message, the UE is authorized to use A2X service based on the UE's subscription information and the "A2X" feature is supported, the AMF further reports to the PCF the UEcapability for A2X communications within the "a2xCapab" attribute as defined in clause 4.2.2.1. The PCF may determine the A2XP based on the received UE's capability for A2X, the Service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

After UE registration, if the UE supports A2X communication and it does not have valid A2XP, the UE includes the "UE POLICY PROVISIONING REQUEST" message as defined in 3GPP TS 24.577 [32] during the NAS transport procedure. The PCF may reject the request by sending back a "UE POLICY PROVISIONING REJECT" message as defined in 3GPP TS 24.577 [32] or provision the policy, as defined in clause 4.2.2.2.1, based on the service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

For both scenarios mentioned above, in the roaming case, the H-PCF may include the A2XP within the "uePolicy" attribute in the policy association create or update response to the V-PCF and in the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send the A2XP to the UE.

###### 4.2.2.2.1.5	Provisioning of Ranging and Sidelink Positioning Policy

When the UE registers to the network and the UE supports Ranging/SL, if the AMF receives from the UE the Ranging/SL Capability in the Registration Request message, the UE is authorized to use Ranging/SL service based on the UE's subscription information and the "Ranging_SL" feature defined in clause 5.8 is supported, the AMF further reports to the PCF this Ranging/SL Capability of the UE within the "rangSlCapab" attribute, as per the procedures defined in clause 4.2.2.1. The PCF may determine the RSLPP over PC5 interface based on the received UE's PC5 capability for Ranging/SL, the service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

After UE registration, if the UE does not have valid RSLPP, the UE includes a "UE POLICY PROVISIONING REQUEST" message defined in 3GPP TS 24.514 [42] during the NAS transport procedure. The PCF may either reject the request by sending back a "UE POLICY PROVISIONING REJECT" message defined in 3GPP TS 24.514 [42] or provision the policy, as defined in clause 4.2.2.2.1, based on the service specific parameter information retrieved from the UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

For both scenarios mentioned above, in the roaming case, the H-PCF may include the RSLPP within the "uePolicy" attribute in the policy association create and update response to the V-PCF and in the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send the RSLPP to the UE.

##### 4.2.2.2.2	UE Access Network discovery and selection policies (ANDSP)

UE Access Network discovery and selection policies are used by the UE to select non-3GPP accesses and to decide how to route traffic between the selected 3GPP and non 3GPP accesses.

In this release of the specification, the Access Network Discovery & Selection policy shall contain only rules that aid the UE in selecting a WLAN access network. Rules for selecting other types of non-3GPP access networks are not specified.

The WLAN access network selected by the UE with the use of Access Network Discovery & Selection policy may be used for direct traffic offload (i.e. sending traffic to the WLAN outside of a PDU Session) and for registering to 5GC using the non-3GPP access network selection information.

The Access Network Discovery & Selection policy shall contain one or more WLAN Selection Policy (WLANSP) rules and and may contain Non-3GPP access network (N3AN) node selection information and configuration information.

N3AN node selection information and configuration information is used to control UE behaviour related to selection of N3IWF, or ePDG for accessing 5GC via untrusted non-3GPP access.

To support N3IWF selection based on the S-NSSAI(s) allowed for the UE, the ANDSP contains the S-NSSAIs supported by the N3IWF(s) and slice-specific configuration, as described in 3GPP TS 24.526 [16]. To support TNGF selection based on the S-NSSAI(s) allowed for the UE, the extended WLANSP information contains the association of the S-NNSAI(s) and SSID(s) supported by the TNGF(s) as described in TS 3GPP TS 24.526 [16]. The (H-)PCF takes into account the UE's subscribed S-NSSAIs and the V-PCF the UE's Configured NSSAI, in both cases together with the UE indication of support of slice-based N3IWF and/or TNGF selection, to provide the ANDSP/WLANSP with slice specific information.

UE Access Network discovery and selection policies are encoded as defined in 3GPP TS 24.526 [16].

UE Access Network discovery and selection policies may be provided by a V-PCF and/or a H-PCF.

If the UE has indicated in the "UE STATE INDICATION" message it does not support ANDSP, or, when the feature "UECapabilityIndication" is supported, the V-PCF receives from the H-PCF the "andspInd" attribute to false, i.e. the UE does not support non-3GPP access, the (V-)(H-)PCF shall not send any Access Network discovery and selection policies to the UE.

##### 4.2.2.2.3	UE Route Selection Policy (URSP)

###### 4.2.2.2.3.1	General

The UE Route Selection Policy is used by the UE to determine how to route outgoing traffic.

The UE Route Selection Policy shall consist of one or several URSP rules. The PCF determines whether URSP rule(s) have to be provisioned based on input parameters received from the NF service consumer, the received list of UPSIs from the UE, if available, the UE Policy Sections stored in the UDR, if available, other received UE parameters, if available, the policy subscription and application data retrieved from UDR, if available, analytics information received from NWDAF, if available, and local policies.

URSP rules are encoded as defined in 3GPP TS 24.526 [16].

UE Route Selection Policy may only be provided by a H-PCF or the PCF of the SNPN, but shall not be provided by a V-PCF. However, UE Route Selection Policy determined and provided by the H-PCF may be retrieved by a V-PCF from the H-PCF and forwarded to a UE.

The (H-)PCF shall use the UE policy subscription data stored in UDR as specified in 3GPP TS 29.519 [17] to ensure the values included in the Route Selection Descriptor of the generated URSP rules are always supported by subscription.

For the received list of internal group Ids, the (H-)PCF retrieves the corresponding 5G VN group configuration data stored from the UDR as specified in 3GPP TS 29.504[27] and 3GPP TS 29.505 [26], if available. For each available 5G VN group, the (H-)PCF may use the retrieved 5G VN group configuration values to encode the values for the Route Selection Descriptor and the values for the Traffic Descriptor of the generated URSP rules.

If the "EnhancedBackgroundDataTransfer" feature is supported, the (H-)PCF may retrieve the Background Data Transfer Reference ID(s) by retrieving the UE's Application Data from the UDR as defined in clause 6.2.9 of 3GPP TS 29.519 [17]. In this case, the PCF shall retrieve the transfer policy corresponding to the Background Data Transfer Reference ID(s) as defined in clause 5.2.8 of 3GPP TS 29.519 [17] and then may create the URSP rules including the Route Selection Validation Criteria for the UE as defined in clause 6.6.2.1 of 3GPP TS 23.503 [4]. If the (H-)PCF provisions the URSP rules including the Route Selection Validation Criteria for the UE, it shall use the associated S-NSSAI and DNN to store in the UDR the Background Data Transfer Reference ID(s) in the UE's session management policy data as specified in 3GPP TS 29.519 [17].

If the (H-)PCF retrieves the BDT policy and corresponding related information (e.g. network area information, the volume of data to be transferred per UE, etc.) within the BdtData data type, and the "bdtpStatus" attribute within the BdtData data type is set to value "INVALID", the (H-)PCF shall not provision the URSP rules based on the invalid BDT policy. When the BDT policy re-negotiation is completed the PCF may:

-	if the new BDT Policy is determined, create or update the applicable URSP rules based on the new BDT policy; or

-	if the invalid BDT policy is removed, remove applicable URSP rules.

If the "AfGuideURSP" feature is supported by the Nudr_DataRepository service, the (H-)PCF may receive Service specific parameter information that contains data for AF guidance information on the URSP determination as defined in clause 6.4.2.15 of 3GPP TS 29.519 [17]. In this case, the (H-)PCF may also use this AF guidance information as input to determine the URSP that will be provisioned to the UE. If the received AF guidance information is not consistent with the UE subscription data, or the local operator policy does not allow the specific S-NSSAI and DNN provided by the AF guidance information, the corresponding AF guidance information shall not be used to determine the URSP rules. The PCF may also determine not to use AF guidance based on the analytics info received from the NWDAF.

When the (H-)PCF decides to provide URSP rules based on the AF guidance information, it shall derive the information as follows:

-	Application traffic descriptor within the "trafficDesc" attribute is used to set the Traffic Descriptor of URSP rule (defined in Figure 5.2.2 of 3GPP TS 24.526 [16]).

-	Each route selection parameter set within the "routeSelParamSets" attribute of the UrspRuleRequest data type is used to determine a Route selection descriptor (defined in Figure 5.2.2 of 3GPP TS 24.526 [16]) as follows:

-	DNN (within the "dnn" attribute of the RouteSelectionParameterSet data type) and S-NSSAI (within the "snssai" attribute of the RouteSelectionParameterSet data type) from the route selection parameter set are used to set the Route selection descriptor contents (defined in Figure 5.2.4 of 3GPP TS 24.526 [16]);

-	Route selection precedence (within the "precedence" attribute of the RouteSelectionParameterSet data type) is used to set the Precedence value of route selection descriptor (defined in Figure 5.2.4 of 3GPP TS 24.526 [16]); and

-	the spatial validity condition (within the "spatialValidityTais" attribute of the RouteSelectionParameterSet data type) is used to set the Location criteria of the route selection descriptor (defined in Figure 5.2.5 of 3GPP TS 24.526 [16]).

-	The PCF may use the requested PDU Session type provided within the "pduSessType" attribute of the RouteSelectionParameterSet data structure to derive the PDU Session type of the route selection descriptors of the URSP rule.

-	The precedence for the generated URSP rule is determined by the (H-)PCF. The (H-)PCF may use the "relatPrecedence" attribute within the "UrspRuleRequest" data type to derive the relative precedence of the URSP rule for a request coming from the same AF.

URSP rules based on AF guidance should not be set as the URSP rules with the "match all" application traffic descriptor.

The (H-)PCF may obtain the information about the UE's OS from the UE as described in the Annex D of 3GPP TS 24.501 [15] or it may derive the information about the UE's OS from the PEI provided by the NF service consumer (e.g. AMF).

If the (H-)PCF is required to provide UE policies to the UE that includes application descriptors then:

a)	If the (H-)PCF has been provided with one UE's OS Id by the UE, the (H-)PCF shall use either the traffic descriptor "OS App Id type" or the traffic descriptor "OS Id + OS App Id type" as defined in 3GPP TS 24.526 [16].

NOTE 1:	The (H-)PCF uses the traffic descriptor "OS Id + OS App Id type" when the (H-)PCF does not take the received UE's OS Id into account.

b)	If the (H-)PCF has been provided with more than one UE's OS Id by the UE,

-	the (H-)PCF shall use the traffic descriptor "OS Id + OS App Id type" for the UE's OS Id provided by the UE as defined in 3GPP TS 24.526 [16]; and

-	the (H-)PCF shall not use the traffic descriptor "OS App Id type" as defined in 3GPP TS 24.526 [16].

c)	If the (H-)PCF has not been provided with the UE's OS Id by the UE,

-	the (H-)PCF shall use the traffic descriptor "OS Id + OS App Id type" as defined in 3GPP TS 24.526 [16]; and

-	the (H-)PCF shall not use the traffic descriptor "OS App Id type" as defined in 3GPP TS 24.526 [16].

d)	If the (H-)PCF has been provided with the UE's OS Id by the UE and the (H-)PCF has derived the UE's OS Id from the PEI and if there is an inconsistency between the OS Id provided by the UE and the OS Id derived from the PEI, the (H-)PCF shall use the OS Id provided by the UE for providing UE policies to the UE that include application descriptors.

URSP rules may be used to support end to end redundant user plane paths by establishing two redundant PDU sessions. PCF configuration based on e.g. deployment, terminal implementation or policies per group of UE(s) may be used by the PCF to determine whether the URSP Rules shall include PDU Session Pair ID and RSN to indicate that they refer to redundant PDU sessions or whether the UE will determine these values instead.

NOTE 2:	When the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported, the received satellite or non-satellite backhaul category can be used as input to provision or update URSP rules to enable appropriate PDU session capabilities. E.g., when satellite backhaul category is indicated by the AMF, the (H-)PCF can take it into account to determine, based on operator policies, an appropriate Route Selection Descriptor for the URSP rule and the services deployed on the satellite, (e.g., the provisioning or update of URSP rules to indicate the specific DNN for services deployed on-board satellites).

If the AF provided the (H-)PCF with Personal IoT Network identifier (PIN ID) associated with a DNN and S-NSSAI, and the received DNN and S-NSSAI corresponds to a subscribed DNN and S-NSSAI combination in the UE Policy Context as described in 3GPP TS 29.519 [17], the (H-)PCF shall include the PIN ID within the traffic descriptor of the URSP Rule attribute as defined in 3GPP TS 24.526 [16] for UE to choose an appropriate PIN to establish the PDU session.

NOTE 3:	The PCF can provide two distinct URSP rules to support end to end redundant user plane paths using Dual Connectivity for the duplicated traffic of an application. Duplicated traffic from the UE application is differentiated by two distinct traffic descriptors (different DNNs, and for IP traffic, different IP descriptors or non-IP descriptors), each one defined in a different URSP rule, so that the two redundant PDU sessions are matched to the specific Route Selection Descriptors of distinct URSP rules. These Route Selection Descriptors of distinct URSP rules may include corresponding RSNs and PDU Session Pair IDs as defined in 3GPP TS 24.526 [16]. The Route Selection Descriptors share the same PDU Session Pair ID, if included, to denote the two traffic are redundant with each other.

NOTE 4:	For backward compatibility, PCF can provide a Route Selection Descriptor with PDU Session Pair ID and RSN and a Route Selection Descriptor without PDU Session Pair ID and RSN in the URSP rule. In this case, the Route Selection Descriptor with PDU Session Pair ID and RSN has a lower precedence value (i.e. higher prioritised) than the one without PDU Session Pair ID. It allows that if a non-supporting UE receives the Route Selection Descriptor containing PDU Session Pair ID, it ignores this Route Selection Descriptor.

NOTE 5:	PIN ID and other traffic descriptor components are mutually exclusive, i.e., if PIN ID is included in a URSP rule, then no other traffic descriptor components are supported in the same URSP rule.

The PCF may adjust the URSP rules when needed, based on awareness of URSP rule enforcement for an application by using the following mechanisms:

A.	Awareness of URSP rule enforcement with UE assistance:

-	Based on operator policies, and if the UE included in the UE STATE INDICATION message the indication of UE's support of reporting URSP rule enforcement as specified in the Annex D of 3GPP TS 24.501 [15], the PCF may indicate in a URSP rule sent to the UE to send reporting of URSP rule enforcement, as specified in 3GPP TS 24.526 [16]. For this URSP rule, the UE reports URSP rule enforcement information to the SMF if Connection Capabilities are included in the traffic descriptor, as specified in the Annex D of 3GPP TS 24.501 [15] and in 3GPP TS 24.526 [16]. When several URSP rules for multiple applications associated to a PDU session are enforced, several URSP rule enforcement reports are included within the URSP rule enforcement information. The SMF reports URSP rule enforcement information received from the UE and its PDU session parameters (e.g. requested DNN, SSC mode, S-NSSAI of the HPLMN, PDU Session Type) to the PCF for the PDU session as specifed in 3GPP TS 29.512 [31].

-	For LBO roaming session case, if the feature "URSPEnforcement" is supported, the H-PCF for the UE may send the "URSP_ENF_INFO" Policy Control Request Trigger to the V-PCF for the UE during the UE Policy Association Establishment or Modification procedures. When the V-PCF receives URSP rule enforcement information and the PDU session parameters as described above, the V-PCF shall invoke the UE Policy Association Update Modification procedure as described in clause 4.2.3.1.

-	If the (V-)(H-)PCF for a UE and the PCF for a PDU session are different, then the (V-)(H-)PCF for a UE may subscribe to the PCF for a PDU session to receive the reporting of URSP rule enforcement information as defined in 3GPP TS 29.514 [37] and the (V-)(H-)PCF for a UE may obtain UE reporting of URSP rule enforcement information and the PDU session parameters from the PCF for a PDU session as defined in 3GPP TS 29.514 [37], where the V-PCF for a UE interacts with the PCF for a PDU session in the VPLMN and the H-PCF for a UE interacts with the PCF for a PDU session in the HPLMN. The (V-)(H-)PCF for the UE discovers the PCF for the PDU session via subscription with the BSF as specified in 3GPP TS 29.521 [22] or, when the feature "URSPEnforcement" is supported, via the request to the AMF to be notified about whether the PCF for the PDU session is available as specified in clauses 4.2.2.1, 4.2.3.1 and 4.2.4.2.

-	Based on the received URSP rule enforcement information and the PDU session parameters, the (H-)PCF may adjust the URSP rules e.g. when the (H-)PCF determines that the UE does not have up-to-date URSP rules. To identify the enforced URSP rule, the PCF may compare the reported Connection Capabilities with the Connection Capabilities of the URSP rules with the indication of "reporting of URSP rule enforcement" and, to identify the Route Selection Descriptor of the enforced URSP rule, the PCF may compare the received PDU session parameters with the parameters in the Route Selection Descriptor components. If there are more than one URSP rule(s) that match the reported Connection Capabilities, the PCF may identify the enforced URSP rule(s) based on implementation specific means. If there are inconsistencies between the received PDU session parameters and the parameters in the Route Selection Descriptor components of the enforced URSP rule, the PCF may perform appropriate actions (e.g. if the S-NSSAI does not match, the PCF may initiate slice replacement procedure).

-	In this release of the specification, the received URSP rule enforcement information shall contain for each URSP rule enforcement report, the one or more Connection Capabilities contained in the traffic descriptor of the concerned URSP rule. If the URSP rule enforcement report does not include connection capabilities, based on local policies, the (H-)PCF for the UE may ignore the received URSP rule enforcement report.

NOTE 6:	A UE supporting the report of URSP rule enforcement reports URSP rule enforcement when the UE (based on the application information matching the traffic descriptor of the URSP rule) associates a newly detected application to a new PDU session or to an existing PDU session and when based on URSP re-evaluation the UE changes the association of an existing application to a PDU session. The UE does not report URSP rule enforcement while the UE is in EPS, delaying the report to when the UE moves from EPS to 5GS.

B.	Awareness of URSP rule enforcement without UE assistance: The PCF may subscribe to or request the PDU Session Traffic analytics statistics using the Nnwdaf_EventsSubscription_Subscribe service operation or Nnwdaf_AnalyticsInfo_Request service operation including the "PDU_SESSION_TRAFFIC" event for traffic monitoring of known traffic according to provisioned PDU Session Traffic requirements of corresponding URSP rule(s) at the NWDAF as defined in 3GPP TS 29.520 [38]. If the PCF is notified or responded with traffic that does not match Traffic Descriptor provided that is the traffic which is not expected according to a URSP rule, the PCF may adjust the URSP rules when unexpected application traffic is detected.

NOTE 7:	The PCF can combine the UE reporting of URSP rule enforcement with the analytics information together to adjust the URSP rules.

###### 4.2.2.2.3.2	Provisioning of VPLMN-specific URSP Rules

When the UE supports VPLMN-specific URSP rules, the H-PCF may provision VPLMN-specific URSP rules to the UE as described in clause 4.2.2.2.1.1 for the purpose to route traffic to the VPLMN or to route traffic to the Home PLMN based on the VPLMN. The H-PCF provides VPLMN specific URSP rules that contains HPLMN values; but the RSD(s) may contain values based on agreements with the VPLMN or parameters received from the VPLMN (e.g., Location Criteria).

NOTE 1:	 For network slice information, the VPLMN-specific URSP rule contains HPLMN NSSAI values. For DNN information, the VPLMN-specific URSP rule contains DNN values according to the subscribed DNNs for which LBO roaming is allowed, as specified in 3GPP TS 29.519 [17].

The (H-)PCF may use AF guidance on URSP determination as input for VPLMN-specific URSP rule determination as specified in clause 4.2.2.2.3.1. The (H-)PCF retrieves from the UDR at the HPLMN the AF guidance for the VPLMN-specific URSP rules for a UE, group of UEs or any UE as specified in 3GPP TS 29.519 [17]. The H-PCF may provide the VPLMN-specific URSP rules to the UE before the UE roams into the VPLMN.

In case of roaming and if the feature "VPLMNSpecificURSP" is supported, the H-PCF may receive from the V-PCF the AF-guidance on VPLMN specific URSP rules within the "vpsUePolGuidance" attribute as specified in clauses 4.2.2.1, and 4.2.3.1. The V-PCF receives from the UDR at the VPLMN the V-AF guidance for the VPLMN specific URSP rules for all roaming UEs of a HPLMN as specified in 3GPP TS 29.519 [17]. The V-PCF determines based on LBO information received from the AMF whether the received V-AF-guidance on VPLMN specific URSP rules may apply for this UE, and if it is so, the V-PCF forwards the related information to the H-PCF within the "vpsUePolGuidance" attribute as specified in clause 4.2.2.1.

For a UE for which AF guidance on VPLMN specific URSP rules is forwarded to the H-PCF within the "vpsUePolGuidance" attribute, the V-PCF:

-	maps the S-NSSAI of the VPLMN (indicated by the AF and retrieved from the UDR, if available) into the S-NSSAI of the HPLMN. The V-PCF uses the Configured NSSAI for the Serving PLMN and mapping of each S-NSSAI of the Configured NSSAI to corresponding HPLMN S-NSSAI values provided by the AMF within the "confSnssais" attribute as specified in clauses 4.2.2.1, and 4.2.3.1. The V-PCF shall subscribe to the "CONF_NSSAI_CH" policy control request trigger. Then, for each URSP rule included within the "urspGuidance" attribute, the V-PCF sends the mapped application guidance on URSP determination including the HPLMN S-NSSAI values to the H-PCF within the "snssai" attribute included within the corresponding "routeSelParamSets" entry; and

-	indicates to the H-PCF to notify about the result of the delivery of UE policies (if it was requested by the AF to the VPLMN) using the "deliveryEvents" attribute as specified in clauses 4.2.2.1, and 4.2.3.1. The H-PCF notifies about the result of the delivery of UE policies using the "delivReport" attribute as specified in clauses 4.2.4.2 and 4.2.4.7.

The H-PCF generates new or updated VPLMN-specific URSP rules using the received application guidance on the URSP rule determination, where the VPLMN ID(s) included in the (H-)AF and/or V-PCF request is used to indicate to the UE that this URSP rule applies when the UE is registered in the VPLMN ID. The H-PCF provides URSP rules for the received AF-guidance parameter values that are within the subscribed values defined in the UE Policy Data Set, as specified in 3GPP TS 29.519 [17]. The VPLMN ID(s) received in the (H-)(V-)AF request, as specified in 3GPP TS 29.522[39], and/or received in the V-PCF request, and provided by the H-PCF within the VPLMN-specific URSP rule, as specified in 3GPP TS 24.501 [15], may contain one or more specific values for the MCC and MNC and/or may indicate any MNC for a MCC.

The H-PCF, based on operator policies, may set the precedence in the URSP Rules to ensure that the UE checks the VPLMN ID(s) that contain one or more specific values for the MCC and MNC before any URSP Rule related the VPLMN ID(s) that contain non-specific values for MCC and/or MNC. The H-PCF should also set the precedence in the URSP rules to ensure that the UE checks any VPLMN-specific URSP rule related to the serving PLMN before any non-VPLMN specific URSP rules.

If the UE does not indicate the support for VPLMN specific URSP rules, the H-PCF generates new or updated URSP rules using the VPLMN ID related information retrieved from the UDR and/or received from the V-PCF, upon receiving a notification that the UE has registered in the VPLMN.

If the UE does not indicate the support for VPLMN specific URSP rules, the H-PCF generates new or updated URSP rules using the VPLMN ID related information retrieved from the UDR and/or received from the V-PCF.

NOTE 2:	To avoid the UE stores obsolete information about VPLMN-specific URSP rules, the H-PCF could delete those determined based on V-AF guidance and once the UE has left the VPLMN.

##### 4.2.2.2.4	Vehicle-to-Everything Policy (V2XP)

V2XP includes the V2XP over PC5 and over Uu interfaces.

The V2XP over PC5 are defined in clause 5.2.3 of 3GPP TS 24.587 [24] and the corresponding encoding is defined in clause 5.3.1 of 3GPP TS 24.588 [25].

The V2XP over Uu are defined in clause 5.2.4 of 3GPP TS 24.587 [24] and the corresponding encoding is defined in clause 5.3.2 of 3GPP TS 24.588 [25].

##### 4.2.2.2.5	Proximity based Services Policy (ProSeP)

The ProSeP includes:

-	ProSeP for 5G ProSe direct discovery defined in clause 5.3 of 3GPP TS 24.555 [29];

-	ProSeP for 5G ProSe direct communications defined in clause 5.4 of 3GPP TS 24.555 [29];

-	ProSeP for 5G ProSe UE-to-network relay, including:

-	ProSeP for 5G ProSe UE-to-network relay UE defined in clause 5.5 of 3GPP TS 24.555 [29]; and/or

-	ProSeP for 5G ProSe Remote UE defined in clause 5.6 of 3GPP TS 24.555 [29];

-	ProSeP for 5G ProSe usage reporting configuration and rules defined in clause 5.7 of 3GPP TS 24.555 [29] ;

and/or

-	ProSeP for 5G ProSe UE-to-UE relay, including:

-	ProSeP for 5G ProSe UE-to-UE relay UE defined in clause 5.8 of 3GPP TS 24.555 [29]; and/or

-	ProSeP for 5G ProSe End UE defined in clause 5.9 of 3GPP TS 24.555 [29].

##### 4.2.2.2.6	Aircraft-to-Everything Policy (A2XP)

A2XP includes the A2X Policy over PC5 interface or A2X Policy over Uu reference point or both.

The A2XP over PC5 or A2XP over Uu reference point or both are defined in 3GPP TS 24.577 [32] and the corresponding encoding is defined in 3GPP TS 24.578 [33].

##### 4.2.2.2.7	Ranging and Sidelink Positioning Policy (RSLPP)

RSLPP includes the Ranging/SL Policy over the PC5 interface. The RSLPP over the PC5 interface is defined in clause 12 of 3GPP TS 24.514 [42].

#### 4.2.2.3	V2X N2 PC5 Policy

The V2X N2 PC5 policy consists of V2X PC5 QoS parameters used by the NG-RAN.

When the (H-)PCF derives the UE policy for V2X communications over PC5 reference point as defined in clause 4.2.2.2.4, the (H-)PCF shall derive the corresponding V2X PC5 QoS parameters used by the NG-RAN.

In the roaming case, the H-PCF:

-	if PC5 UE capabilities and UE Policy Provisioning request messages are received, and V2X policies are derived, shall include the V2X N2 PC5 Policy within the "n2Pc5Pol" attribute in the policy association creation response  towards the V-PCF; or

-	shall include the V2X N2 PC5 Policy within the "n2Pc5Pol" attribute, if changes apply, in the policy association update response towards the V-PCF; or

-	may include the V2X N2 PC5 Policy within the "n2Pc5Pol" attribute in the the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send V2X N2 PC5 policy to the NG-RAN.

#### 4.2.2.4	5G ProSe N2 PC5 Policy

The 5G ProSe N2 PC5 policy consists of 5G ProSe PC5 QoS parameters used by the NG-RAN.

When the (H-)PCF derives the UE policy for 5G ProSe as defined in clause 4.2.2.2.5, the (H-)PCF shall derive the corresponding 5G ProSe N2 PC5 QoS parameters used by the NG-RAN.

In the roaming case, the H-PCF:

-	if the 5G ProSe capabilities and the UE Policy Provisioning request message are received, and 5G ProSe policies are derived, shall include the N2 PC5 Policy for 5G ProSe within the "n2Pc5ProSePol" attribute in the of policy association creation response towards the V-PCF; or

-	shall include the N2 PC5 Policy for 5G ProSe within the "n2Pc5ProSePol" attribute, if changes apply, in the response of the policy association update response towards the V-PCF; or

-	may include the N2 PC5 Policy for 5G ProSe within the "n2Pc5ProSePol" attribute in the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send 5G ProSe N2 PC5 policy to the NG-RAN.

#### 4.2.2.5	A2X N2 PC5 Policy

The A2X N2 PC5 policy consists of A2X PC5 QoS parameters used by the NG-RAN.

When the (H-)PCF derives the UE policy for A2X communications over PC5 reference point as defined in clause 4.2.2.2.6, the (H-)PCF shall derive the corresponding A2X PC5 QoS parameters used by the NG-RAN.

In the roaming case, the H-PCF:

-	if PC5 UE capabilities and UE Policy Provisioning request messages are received, and A2X policies are derived, shall include the A2X N2 PC5 Policy within the "n2Pc5PolA2x" attribute in the policy association creation response towards the V-PCF; or

-	shall include the A2X N2 PC5 Policy within the "n2Pc5PolA2x" attribute, if changes apply, in the policy association update response towards the V-PCF; or

-	may include the A2X N2 PC5 Policy within the "n2Pc5PolA2x" attribute in the the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send A2X N2 PC5 policy to the NG-RAN.

#### 4.2.2.6	Ranging/SL N2 PC5 Policy

The Ranging/SL N2 PC5 policy consists of Ranging/SL PC5 QoS parameters used by the NG-RAN.

When the (H-)PCF derives the UE policy for Ranging/SL over the PC5 reference point as defined in clause 4.2.2.2.7, the (H-)PCF shall derive the corresponding Ranging/SL PC5 QoS parameters used by the NG-RAN.

In the roaming case, the H-PCF:

-	if the PC5 UE capabilities and UE Policy Provisioning request messages are received, and Ranging/SL policies are derived, shall include the Ranging/SL N2 PC5 Policy within the "n2Pc5RsppPol" attribute in the policy association creation response towards the V-PCF;

-	shall include the Ranging/SL N2 PC5 Policy within the "n2Pc5RsppPol" attribute, if changes apply, in the policy association update response towards the V-PCF; or

-	may include the Ranging/SL N2 PC5 Policy within the "n2Pc5RsppPol" attribute in the the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send Ranging/SL N2 PC5 policy to the NG-RAN.

### 4.2.3	Npcf_UEPolicyControl_Update Service Operation

#### 4.2.3.1	General

The procedure in the present clause is applicable when the NF service consumer modifies an existing UE policy association (including the case where the AMF is relocated and the new AMF selects to maintain the policy association with the old PCF and to update the Notification URI).

Figure 4.2.3.1-1 illustrates the update of a policy association.

Figure 4.2.3.1-1: Update of a UE policy association

NOTE 1:	For the roaming case, the PCF represents the V-PCF if the NF service consumer is an AMF and the PCF represents the H-PCF if the NF service consumer is a V-PCF.

The AMF, as NF service consumer, invokes this procedure when a subscribed policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that requires the subscription as defined in table 5.6.3.3-1 (e.g. LOC_CH trigger) occurs, the NF service consumer (AMF) shall only invoke this procedure if the PCF has explicitly subscribed to that event trigger. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. GROUP_ID_LIST_CHG trigger) occurs, the NF service consumer (AMF) shall always invoke the procedure.

NOTE 2:	The AMF uses the Namf_Communication_N1MessageNotify service operation specified in 3GPP TS 29.518 [14] to send to the V-PCF a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] or a "UE POLICY PROVISIONING REQUEST" message as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24].

If an AMF as NF service consumer knows by implementation specific means that the UE context has been transferred to an AMF with another GUAMI within the AMF set, it may also invoke this procedure to update the Notification URI.

NOTE 3:	Either the old or the new AMF can invoke this procedure.

During the AMF relocation, if the new AMF received the resource URI of the individual UE Policy from the old AMF and selects the old PCF, the new AMF shall also invoke this procedure to update the Notification URI. The new AMF may also update the alternate or backup IP addresses, and if service discovery via NRF applies, the AMF Id. If the feature "FeatureRenegotiation" is supported, the new AMF may perform feature renegotiation, as described in clause 4.2.3.4.

NOTE 4:	During inter-AMF mobility, the N1N2 Individual Subscription context is transferred from the source AMF to the target AMF as specified in 3GPP TS 29.518 [14]. When the target AMF determines to reuse the UE Policy Association indicated by the source AMF, the PCF can keep the N1N2 Individual Subscription context and, for subsequent interactions, replace in the request URI the {apiRoot} of the N1N2 Individual Subscription resource with the one of the target AMF.

The V-PCF, as NF service consumer, invokes this procedure when a policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. UE_POLICY trigger) occurs, the V-PCF shall always invoke the procedure. When a policy control request trigger that requires the subscription as defined in table 5.6.3.3-1 (e.g. LOC_CH trigger) occurs, the V-PCF shall only invoke this procedure if the H-PCF has subscribed to that event trigger.

To request policies (e.g. policy control request trigger(s) is/are met) from the PCF, to update the Notification URI, to renegotiate features, to update the trace control configuration or to request the termination of trace, the NF Service Consumer shall request the update of the associated UE Policy Association by providing the relevant parameters about the UE context in an HTTP POST request with "{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}/update" as Resource URI and the PolicyAssociationUpdateRequest data structure as request body that shall include:

-	at least one of the following:

1.	a new Notification URI encoded in the "notificationUri" attribute;

2.	observed Policy Control Request Trigger(s) (see clause 4.2.3.2) encoded as "triggers" attribute;

3.	if a UE location change occurred, the UE location encoded as "userLoc" attribute;

4.	if a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message of the UE policy delivery protocol defined in Annex D of 3GPP TS 24.501 [15] has been received by the V-PCF as NF service consumer, and at least parts of the contents relate to UPSIs of the HPLMN, the parts of that message that relate to UPSIs of the HPLMN encoded as "uePolDelResult" attribute;

5.	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the current presence status of the UE for the presence reporting areas for which reporting was requested, if not previously provided, or the presence reporting areas for which reporting was requested and the status has changed encoded as "praStatuses" attribute;

NOTE 5:		If the PCF included the identifer of a Core Network predefined Presence Reporting Area Set within the "praId" attribute during the subscription to changes of UE presence in PRA, the AMF only provides the presence reporting area information corresponding to the concerned individual Presence Reporting Area Identifier(s) within the Set. The "praId" attribute within each returned "PresenceInfo" data type hence includes the identifier of the concerned individual Presence Reporting Area.

6.	if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

7.	if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

8.		if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

9.  for AMF relocation scenarios, the GUAMI encoded as "guami" attribute;

NOTE 6:	An alternate NF service consumer than the one that requested the generation of the subscription resource can send the request. For instance, an AMF as service consumer can change;

10.	if the NF service consumer is an AMF, for AMF relocation scenarios, the new serving AMF Id encoded in the "servingNfId" attribute;

NOTE 7:	If the PCF received the "servingNfId" attribute, the PCF can use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] to retrieve the NF profile of the Namf_Communication service available in the indicated AMF instance Id.

11.	if a UE PLMN change occurred and the "PlmnChange" feature defined in clause 5.8 is supported, the PLMN Identifier or the SNPN Identifier of the new serving network encoded as "plmnId" attribute;

NOTE 8:	The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 9:	When the UE moves between PLMNs, the trigger reports changes of equivalent PLMNs.

NOTE 10:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

12. if a "UE POLICY PROVISIONING REQUEST" message defined in clause 7.2.1.1 of 3GPP TS 24.587 [24] has been received by the V-PCF as NF service consumer and respectively the "V2X" feature and/or the "A2X" feature and/or the "ProSe" feature and/or the "Ranging_SL" feature defined in clause 5.8 is/are supported, the message encoded as "uePolReq" attribute;

13.	if a UE Internal Group Identifier(s) change occurred and the "GroupIdListChange" feature defined in clause 5.8 is supported, the Internal Group Identifier(s) of the served UE encoded as "groupIds" attribute;

14.	if a change of PC5 capablity for 5G ProSe occurred and the "ProSe" feature defined in clause 5.8 is supported, the PC5 capability for 5G ProSe encoded as "proSeCapab" attribute;

14a.	if a change of the Ranging/SL Capability occurred and the "Ranging_SL" feature defined in clause 5.8 is supported, the Ranging/SL Capability encoded as "rangSlCapab" attribute; and/or

15.	if a change of the connectivity state of the UE occurred and the "ConnectivityStateChange" feature defined in clause 5.8 is supported, the connectivity state of the served UE encoded as "connectState" attribute;

16.	when a response with HTTP status code 4xx or 5xx as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14] or a N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14] is received by the V-PCF after provisioning the UE policy by invoking the Namf_Communication_N1N2MessageTransfer service operation to the AMF, this UE policy transfer failure notification encoded as "uePolTransFailNotif" attribute;

17.		if the NF service consumer is an AMF, the "SliceAwareANDSP" feature is supported, and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute, the wrongly selected type of non-3gpp access node encoded as "n3gNodeReSel" attribute, and, in the roaming case, also the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute;

18.		if satellite backhaul category change occurred and the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported, the satellite backhaul category or non-satellite backhaul encoded as "satBackhaulCategory" attribute;

19.	for the roaming scenario, if the NF service consumer is an AMF, Configured NSSAI change occurred and the "NssaiChange" feature is supported, the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute and optionally the mapped each S-NSSAI value of home network corresponding to the configured S-NSSAI values in the serving PLMN encoded as "mappedHomeSnssai" attribute within the "confSnssais" attribute;

20	for the roaming scenario, if the NF service consumer is a V-PCF, the "URSPEnforcement" feature is supported, and the "URSP_ENF_INFO" policy control request trigger is met, the URSP rule enforcement information within the "urspEnfReport" attribute;

21.	for the roaming scenario, if the NF service consumer is a V-PCF the "VPLMNSpecificURSP" feature is supported, the new/modified/deleted AF guidance on VPLMN-specific URSP rules related information within the "vpsUePolGuidance" attribute, that shall contain for each related AF:

a.	the AF guidance on VPLMN-Specific URSP rules within the "urspGuidance" attribute, if the AF updated/provided this information; and/or

b.	if the AF requested to the VPLMN notifications about the delivery of UE Policies or the update of the subscription to notification information previously provided, the "deliveryEvents" attribute including the"SUCCESS_UE_POL_DEL_SP" and/or "UNSUCCESS_UE_POL_DEL_SP" events;

22.	for the roaming scenario, if the NF service consumer is an AMF, the "VPLMNSpecificURSP" feature is supported and the "LBO_INFO_CH" policy control request trigger is met, the LBO roaming information within the "lboRoamInfo" attribute; and/or

23.		if an access type change occurred and the "AccessChange" feature defined in clause 5.8 is supported, the access type(s) where the UE is registered encoded within the "accessTypes" attribute and the corresponding RAT Type(s), if available, in the "ratTypes" attribute.

Upon the reception of the HTTP POST request:

-	if the PCF is a V-PCF and the V-PCF has an established policy association with the H-PCF, the V-PCF shall determine based on the contents of a potentially received "uePolDelResult" attribute to be sent to the H-PCF (see above) and requested event triggers of the H-PCF whether to send as the NF service consumer towards the H-PCF a request for the update of the policy association as described in the present clause;

-	the (V-)(H-)PCF shall determine the applicable UE policy based on the contents of the received HTTP POST request, the UE Policy Sections stored in UDR, local policy and, for the H-PCF, taking into consideration the information received within the UE policy delivery protocol encoded in the "uePolReq" attribute, if available, and for the V-PCF, taking into consideration any policy received from the H-PCF encoded in the "uePolicy" attribute in the reply to the possible request for the update of the associated policy association. When the "ProSe" feature is supported, the H-PCF shall determine the applicable ProSeP based on the received PC5 capability for 5G ProSe. When the UE disables a 5G ProSe capability the PCF may stop updating the corresponding ProSeP, and when the UE enables a 5G ProSe capability the PCF may update the corresponding ProSeP;

-	if the UE indicated the support of A2X communications over PC5 reference point, "A2X" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable A2XP and A2X N2 PC5 policy as detailed in clauses 4.2.2.2.1.4 and 4.2.2.5, based on the operator's policy;

-	if the UE indicates the support of 5G ProSe communications over PC5 reference point, the "ProSe" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message with the requested 5G ProSe policies was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable ProSeP and 5G ProSe N2 PC5 policy, as detailed in clauses 4.2.2.2.1.3 and 4.2.2.4, based on the operator's policy;

-	if the UE indicated the support of V2X communications over PC5 reference point, "V2X" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable V2XP and V2X N2 PC5 policy as detailed in clauses 4.2.2.2.1.2 and 4.2.2.3, based on the operator's policy;

-	if the UE indicated the support of Ranging/SL over the PC5 reference point, the "Ranging_SL" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable RSLPP and Ranging/SL N2 PC5 policy as detailed in clauses 4.2.2.2.1.5 and 4.2.2.6 based on the operator's policy;

-	for the succesfull case, the (V-)(H-)PCF shall send a HTTP "200 OK" response with the PolicyUpdate data type as response body with the possibly updated of UE policy (for the H-PCF), and/or ProSe N2 PC5 policy (for the H-PCF) as specified in clause 4.2.2.4, N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe (for the H-PCF), as specified in clause 4.2.2.3, and/or the Ranging/SL N2 PC5 policy (for the H-PCF), as specified in clause 4.2.2.6, and/or Policy Control Request Trigger(s) encoded as described in clause 4.2.3.3;

-	if the (V-)PCF determines that UE policy needs to be updated, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall send the determined UE policy using Namf_Communication_N1N2MessageTransfer service operation(s); and

(ii)	the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the AMF within the Namf_Communication_N1MessageNotify service operation, and for the V-PCF, if the received UE Policy Delivery results relate to UE policy sections provided by the H-PCF, the V-PCF shall use the Npcf_UEPolicyControl_Update Service Operation to send those UE Policy Delivery results to the H-PCF; and

NOTE 11:	A PolicyUpdate data structure with only mandatory attribute(s) is included in the "200 OK" response when the PCF decides not to update the policies.

-	if the PCF determines that the V2XP and N2 PC5 policy (e.g. for V2X communications, for 5G ProSe) for V2X communications need to be updated, and for the V-PCF when receiving the updated V2XP and N2 PC5 policy for V2X communications from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the V2XP to the UE and the V2X N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.2 and 4.2.2.3;

-	if the PCF determines that the A2XP (e.g. for A2X communications) for A2X communications need to be updated, and for the V-PCF when receiving the updated A2XP and N2 PC5 policy for A2X communications from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the A2XP to the UE and the A2X N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.4 and 4.2.2.5;

-	if the PCF determines that ProSeP and 5G ProSe N2 PC5 policy needs to be updated, and for the V-PCF when receiving the updated ProSeP and 5G ProSe N2 PC5 policy from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the ProSeP to the UE and 5G ProSe N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.3 and 4.2.2.4;

-	if the PCF determines that RSLPP and Ranging/SL N2 PC5 policy needs to be updated, and for the V-PCF when receiving the updated RSLPP and Ranging/SL N2 PC5 policy from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the RSLPP to the UE and Ranging/SL N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.5 and 4.2.2.6;

-	if the "SliceAwareANDSP" feature is supported, the PCF received the "NON_3GPP_NODE_RESELECTION" trigger, and the PCF has successfully delivered to the UE the ANDSP/WLANSP with the slice selection information for the corresponding non-3gpp node, the indication of the successful UE configuration by providing the "andspDelInd" attribute with the value "CONFIGURED". The PCF may delay the indication of the configuration result to a subsequent Npcf_UEPolicyControl_UpdateNotify request as described in clause 4.2.4.2; and

-	optionally, for the (V-)PCF communicating with the AMF, if the "URSPEnforcement" feature is supported, and if not previously provided, the request to the AMF to be notified about the PDU session established/terminated events by providing the PCF for the UE callback information within the "pcfUeInfo" attribute, and the DNN and S-NSSAI of the concerned PDU session(s) within the "matchPdus" attribute. If previously provided, the (V-)PCF may update the complete list of DNN and S-NSSAI combination(s) of the concerned PDU session(s) within the "matchPdus" attribute and/or update the PCF for the UE callback information within the "pcfUeInfo" attribute.

-	if errors occur when processing the HTTP POST request, the (V-)(H-)PCF shall:

-	send an HTTP error response as specified in clause 5.7; or

-	if the feature "ES3XX" is supported, and the (V-)(H-)PCF determines the received HTTP POST request needs to be redirected, send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5];

according to the following provisions:

-	if the (V-)(H-)PCF is, due to incomplete, erroneous or missing information in the request not able to provision a UE policy decision, the PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

If the PCF received a new GUAMI, the PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF set.

When the "SliceAwareANDSP" feature is supported, and the AMF receives the "andspDelInd" attribute with the outcome of the UE configuration with slice aware ANDSP/WLANSP, the AMF proceeds as described in clause 4.2.2.1.

When the "URSPEnforcement" feature is supported and the AMF receives the "matchPdus" attribute, the AMF shall update the affected established PDU session(s), by forwarding the received PCF for the UE callback information for the PDU session(s) matching the new S-NSSAI and DNN combination(s) to the SMF, and removing the previously provided PCF for the UE information for the PDU session(s) matching the removed S-NSSAI and DNN combination(s) from the SMF as defined in 3GPP TS 29.502 [31]. When the AMF receives the "pcfUeInfo" attribute with updated SBA binding indication, the AMF shall apply the updated PCF for the UE callback information to the new PDU sessions only, i.e., already established PDU sessions are not affected.

#### 4.2.3.2	Policy Control Request Triggers

The following Policy Control Request Triggers are defined:

-	"LOC_CH", i.e. location change (tracking area): the tracking area of the UE has changed;

-	"PRA_CH", i.e. change of UE presence in PRA: the UE is entering/leaving a Presence Reporting Area. This includes reporting the initial status at the time the request for this reporting is initiated;

-	"UE_POLICY", i.e. a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] has been received by the V-PCF and is being forwarded to the H-PCF, or has been received by a PCF for a PDU session (in case for URSP provisioning in EPS) and is being forwarded to the (V-)PCF, or a "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24] has been received by the V-PCF and is being forwarded to the H-PCF;

-	"PLMN_CH", i.e. PLMN change: the serving network (PLMN or SNPN) of the UE has changed;

NOTE 1:	The "PLMN_CH" trigger only applies if the "PlmnChange" feature is supported.

NOTE 2:	When the UE is moving between PLMNs, the trigger reports changes of equivalent PLMNs.

NOTE 3:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

-	"CON_STATE_CH", i.e. connectivity state change: the connectivity state of the UE has changed;

NOTE 4:	The "CON_STATE_CH" trigger only applies if the "ConnectivityStateChange" feature is supported.

-	"GROUP_ID_LIST_CHG", i.e. UE Internal Group Identifier(s) change: the UDM provided list of group Ids has changed;

NOTE 5:	The "GROUP_ID_LIST_CHG" trigger only applies if the "GroupIdListChange" feature is supported. This Policy Control Request Trigger does not require an explicit subscription from the PCF.

-	"UE_CAP_CH", i.e. UE Capabilities change: the UE provided 5G ProSe capabilities have changed;

NOTE 6:	The "UE_CAP_CH" trigger only applies if the "ProSe" feature is supported. This Policy Control Request Trigger does not require a subscription.

-	"SAT_CATEGORY_CHG", i.e. Satellite Backhaul Category change: the AMF has detected a change between different satellite category, or non-satellite backhaul;

NOTE 7:	The "SAT_CATEGORY_CHG" trigger only applies if the "EnSatBackhaulCategoryChg" feature is supported.

-	"NON_3GPP_NODE_RESELECTION", i.e. wrong TNGF or N3IWF: the UE indicated to the AMF the support of slice-based N3IWF and/or TNGF selection as specified in 3GPP TS 24.501 [15] and the AMF determined that the UE has connected to a non-3GPP access node that is not compatible with the allowed S-NSSAI(s);

NOTE 8:	The "NON_3GPP_NODE_RESELECTION" trigger only applies if the "SliceAwareANDSP" feature is supported. This Policy Control Request Trigger does not require expilict subscription by the PCF.

-	"CONF_NSSAI_CH", i.e. Configured NSSAI change: the Configured NSSAI has changed;

NOTE 9:	The "CONF_NSSAI_CH" trigger only applies if the "NssaiChange" feature is supported.

-	"URSP_ENF_INFO", i.e. URSP rule enforcement Information: The V-PCF has received URSP rule enforcement information about the enforced URSP rule(s) in one or more PDU sessions;

NOTE 10:	The "URSP_ENF_INFO" trigger only applies to the V-PCF in LBO roaming scenarios and if the "URSPEnforcement" feature is supported.

-	"LBO_INFO_CH", i.e. LBO information change: The AMF reports LBO roaming allowed or not allowed for the requested DNN(s) and S-NSSAI(s);

NOTE 11:	The "LBO_INFO_CH" trigger only applies to the AMF and when the "VPLMNSpecificURSP" feature is supported.

-	"ACCESS_TYPE_CH", i.e. access type change: The registered access type and RAT type has changed, an access type and RAT type is added or removed; and

NOTE 12:	The "ACCESS_TYPE_CH" trigger only applies when the "AccessChange" feature is supported.

-	"FEAT_RENEG", i.e. the target AMF determines feature re-negotiation is required.

NOTE 13:	The "FEAT_RENEG" trigger only applies if the "FeatureRenegotiation" feature is supported during AMF relocation.

#### 4.2.3.3	Encoding of updated policy

Updated policies shall be encoded within the PolicyUpdate data type that may include:

-	only when the updated policy is supplied by the H-PCF in the roaming scenario, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute, and N2 PC5 policy for V2X communications (see clause 4.2.2.3) encoded as "n2Pc5Pol" attribute and/or the N2 PC5 policy for A2X communications (see clause 4.2.2.5) encoded as "n2Pc5PolA2x" attribute and/or the N2 PC5 policy for 5G ProSe (see clause 4.2.2.4) encoded as "n2Pc5ProSePo" attribute and/or the N2 PC5 policy for Ranging/SL (see clause 4.2.2.6) encoded using the "n2Pc5RsppPol" attribute;

-	when the updated policy is supplied via PCF of a PDU session by the (V-)PCF in case of URSP provisioning in EPS, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute;

-	updated Policy Control Request Trigger(s) (see clause 4.2.3.2) encoded as "triggers" attribute, i.e.:

1)	either a new complete list of applicable Policy Control Request Trigger(s) including one or several of the following:

a)	Location change (tracking area);

b)	Change of UE presence in PRA;

c)	Change of PLMN, if the "PlmnChange" feature is supported;

d)	Change of UE connectivity state, if the "ConnectivityStateChange" feature is supported;

e)	Change of Satellite Backhaul Category, if the "EnSatBackhaulCategoryChg" feature is supported;

f)	Change of Configured NSSAI, in roaming scenarios, if the "NssaiChange" feature is supported and the NF service consumer is the AMF;

g)	LBO information change, applicable to roaming scenarios, if the "VPLMNSpecificURSP" feature is supported and the NF service consumer is an AMF; or

h)	Change of Access type and RAT type, if the "AccessChange" feature is supported;

2)	a "NULL" value to request the removal of all previously installed Policy Control Request Trigger(s); and

-	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided or if that trigger was already set but the requested presence reporting areas need to be changed, the presence reporting areas for which reporting is required encoded as "pras" attribute encoded as follows:

a)	A new entry shall be added by supplying a new identifier as key and the corresponding PresenceInfo data type instance with complete contents as value as an entry within the map.

b)	An existing entry shall be modified by supplying the existing identifier as key and the PresenceInfo data type instance with complete contents as value as an entry within the map.

c)	An existing entry shall be deleted by supplying the existing identifier as key and "NULL" as value as an entry within the map.

d)	For an unmodified entry, no entry needs to be provided within the map.

-	if the Policy Control Request Trigger "LBO information change" is provided or if that trigger was already set but the requested LBO information needs to be changed, the requested LBO information encoded in the "pduSessions" attribute, a list of DNN and S-NSSAI combinations previously provided is updated by providing either a new complete list within the"pduSessions" attribute or by setting it to "NULL". If the "pduSessions" attribute is not provided or the previously provided "pduSessions" attribute is deleted, the LBO information change policy control request trigger applies to any S-NSSAI and DNN combination.

#### 4.2.3.4	Feature renegotiation during AMF relocation

During the AMF relocation, if the new AMF received the resource URI of the individual UE Policy from the old AMF and selects the old (V-)PCF, and the feature "FeatureRenegotiation" is supported, the new AMF shall invoke the update of the UE policy association as described in clause 4.2.3.1 with the following differences:

-	The new AMF shall include in the PolicyAssociationUpdateRequest data structure sent in the HTTP POST request:

a.	the "FEAT_RENEG" policy control request trigger within the "triggers" attribute;

b.	the "suppFeat" attribute with the AMF supported features; and

c.	for each supported feature, the required feature information elements as specified in clauses 4.2.2.1 and 4.2.3.1, if applicable.

NOTE 1:	When the new AMF received from the old AMF the subscribedpolicy control request trigger(s) that depend on feature control, and a policy control request trigger is met, the required feature information included in the update request contains the report of the met policy control request trigger within the "triggers" attribute and the associated information in the corresponding attribute as described in clause 4.2.3.1, when applicable. If the new AMF supports features not previously supported by the old AMF, the new AMF will include the available information associated to the supported features, if applicable, as described in clause 4.2.2.1.

-	Upon reception of the HTTP POST request, the (V-)PCF shall update the "Individual UE Policy Association" resource, determine the applicable policy and include in the PolicyUpdate data structure sent in the HTTP POST response:

NOTE 2:	The determination of the applicable policy can consider the features supported by the new AMF.

a.	the "suppFeat" attribute with the negotiated supported features; and

b.	the complete "Individual UE Policy Association" resource representation, as specified in clause 4.2.2.1.

### 4.2.4	Npcf_UEPolicyControl_UpdateNotify Service Operation

#### 4.2.4.1	General

The (V-)(H)-PCF may decide to update policy control request triggers, and in the roaming case, the H-PCF may decide to update the UE Policy, the V2X N2 PC5 policy, if the "V2X" feature is supported, and/or the A2X N2 PC5 policy, if the "A2X" feature is supported, and/or the 5G ProSe N2 PC5 policy, if the "ProSe" feature is supported, and/or the Ranging/SL N2 PC5 policy, if the "Ranging_SL" feature is supported. The PCF (H-PCF in the roaming case) may decide to request the termination of the policy association.

If the "EpsUrsp" feature is supported, and the NF consumer is a PCF for a PDU session the PCF (H-PCF in the LBO roaming scenario) may decide to update policy control request triggers and/or to update the URSP. The PCF (H-PCF in the LBO roaming scenario) may decide to request the termination of the policy association.

The(V-)(H-)PCF shall then use an Npcf_UEPolicyControl_UpdateNotify service operation.

The following procedures using the Npcf_UEPolicyControl_UpdateNotify service operation are supported:

-	Policy update notification.

-	Request the termination of the UE policy association.

-	URSP provisioning for background Data Transfer policy.

-	UE policy provisioning for V2X communications over PC5 and Uu reference points.

-	UE policy provisioning for 5G ProSe.

-	UE policy provisioning for Ranging/SL.

-	N2 PC5 Policy (e.g. for V2X communications, for A2X communications, for 5G ProSe, for Ranging/SL) provisioning.

-	UE policy provisioning for A2X communications over PC5 reference point or A2X communications over Uu reference point or both.

NOTE:	The PCF derives the UE policy information and invokes the Namf_Communication_N1N2MessageTransfer service operation to provision it to the UE.

-	URSP provisioning in EPS.

#### 4.2.4.2	Policy update notification

Figure 4.2.4.2-1 illustrates the policy update notification.

Figure 4.2.4.2-1: policy update notification

NOTE:	For the roaming case, the PCF represents the V-PCF if the NF service consumer is an AMF and the PCF represents the H-PCF if the NF service consumer is a V-PCF.

The (V-)(H)-PCF may decide to update, based on external triggers (e.g. notifications received from UDR about new or updated service parameter data as described in 3GPP TS 29.519 [17]) or internal triggers (e.g., the activation of a pending policy counter provided via the Nchf_SpendingLimitControl Service as described in 3GPP TS 29.594 [33]) policy control request trigger(s) and in the roaming case, the H-PCF may also decide to update the UE Policy, the N2 PC5 policy for V2X communications if the "V2X" feature is supported and/or the N2 PC5 policy for A2X communications if the "A2X" feature is supported and/or the N2 PC5 policy for 5G ProSe if the "ProSe" feature is supported and/or the N2 PC5 policy for Ranging/SL if the "Ranging_SL" feature is supported.

NOTE:	In this release of the specification, policy decisions based on policy counters provided via Nchf_SpendingLimitControl service apply only for non-roaming cases and UE policies refer to URSP only.

If the "EpsUrsp" feature is supported and the NF consumer is a PCF for a PDU session the PCF (H-PCF in the LBO roaming scenario) may decide to update policy control request triggers and/or to update the URSP.

If the "SliceAwareANDSP" feature is supported, the PCF received the indication of wrong NI3WF or TNGF selection during UE Policy Association creation as described in clause 4.2.2.1 or during UE Policy Association modification as described in clause 4.2.3.1, and the PCF determines that the UE needs to be configured with ANDSP/WLANSP with slice selection information and the configuration result is to be indicated within a Npcf_UEPolicyControl_UpdateNotify request then:

-	when the PCF has successfully delivered to the UE the updated ANDSP/WLANSP with the slice selection information for the corresponding type of non-3gpp node, the PCF notifies to the NF service consumer about the successful delivery providing the "andspDelInd" attribute set to value "CONFIGURED".

-	if the UE update with the ANDSP/WLANSP with the slice selection information for the corresponding type of non-3GPP node fails, the PCF provides the "andspDelInd" attribute set to value "NOT_CONFIGURED".

If the "VPLMNSpecificURSP" feature is supported, the NF consumer is the V-PCF and the H-PCF received the subscription to notification about the delivery outcome of VPLMN-specific URSP rules within the "deliveryEvents" attribute as specified in clauses 4.2.2.1, and 4.2.3.1, the H-PCF notifies about the result of the delivery of UE policies using the "delivReport" attribute as described in clause 4.2.4.7.

For the (V-)PCF communicating with the AMF, if the "URSPEnforcement" feature is supported, and if not previously provided, the (V-)PCF may decide to request to the AMF to be notified about the PDU session established/terminated events by providing the PCF for the UE callback information within the "pcfUeInfo" attribute, and the DNN and S-NSSAI of the concerned PDU session(s) within the "matchPdus" attribute. Alternatively, the (V-)PCF may provide the updated complete list of DNN and S-NSSAI combination(s) of the concerned PDU sessions within the "matchPdus" attribute and/or updated PCF for the UE callback information within the "pcfUeInfo" attribute.

The (V-)(H-)PCF shall then send an HTTP POST request with "{notificationUri}/update" as URI (where the Notification URI was previously supplied by the NF service consumer) to the NF service consumer and the PolicyUpdate data structure as request body encoded as described in clause 4.2.3.3.

Upon the reception of the HTTP POST request, the NF service consumer:

-	if the V-PCF is the NF service consumer, shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to send "MANAGE UE POLICY COMMAND" message(s) with the received UE policy to the UE via the AMF and/or with the received N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe to the NG-RAN via the AMF;

-	if the V-PCF is the NF service consumer, shall provision the received policy control requested trigger(s) to the AMF, if applicable, using the Npcf_UEPolicyControl_UpdateNotify service operation according to the present clause;

-	if the AMF is the NF service consumer, shall enforce the received policy control request trigger(s);

-	if the "EpsUrsp" feature is supported and a PCF for a PDU session is the NF service consumer, shall behave as specified in clause 4.2.4.9;

-	if the "VPLMNSpecificURSP" feature is supported, and the V-PCF is the NF service consumer, may trigger the notification(s) about the result of the delivery of UE policies as specified in clause 4.2.4.7;

-	shall either send a successful response indicating the success of the enforcement or an appropriate failure response, for the V-PCF as the NF service consumer taking into consideration a reply received from the possible Namf_Communication Service service operation and from the possible Npcf_UEPolicyControl_UpdateNotify service operation according to the previous bullets. In case of a successful response:

a.	if the feature "ImmediateReport" is supported and the PCF provisioned policy control request triggers (applicable triggers are as defined in Table 5.6.2.8-1), a "200 OK" response code and a response body with the corresponding available information in the "UeRequestedValueRep" data structure shall be returned in the response;

b.-	otherwise, a "204 No Content" response code shall be returned in the response; and

-	if errors occur when processing the HTTP POST request, shall send an HTTP error response as specified in clause 5.7; or

-	if the feature "ES3XX" is supported, and the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

When the "URSPEnforcement" feature is supported and the AMF receives the "matchPdus" attribute, the AMF shall update the affected established PDU sesssion(s), by forwarding the received PCF for the UE callback information for the PDU session(s) matching the new S-NSSAI and DNN combination(s) to the SMF, and removing the previously provided PCF for the UE callback information for the PDU session(s) matching the removed S-NSSAI and DNN combination(s) from the SMF as defined in 3GPP TS 29.502 [31]. When the AMF receives "pcfUeInfo" attribute with updated SBA binding indication, the AMF shall apply the updated PCF for the UE callback information to the new PDU sessions only, i.e., already established PDU sessions are not affected.

If the feature "ErrorResponse" is supported and if the AMF as NF service consumer is not able to handle the notification but another unknown AMF could possibly handle the notification, it shall reply with an HTTP "404 Not found" error response.

If the (V-)PCF receives a "307 Temporary Redirect" response, the (V-)PCF shall resend the failed policy update notification request using the received URI in the Location header field as Notification URI. Subsequent policy update notifications, triggered after the failed one, shall be sent to the Notification URI provided by the NF service consumer during the corresponding policy association creation/update.

If the (V-)PCF becomes aware that a new AMF is requiring notifications (e.g. via the "404 Not found" response or via Namf_Communication service AMFStatusChange Notifications, see 3GPP TS 29.518 [14], or via link level failures), and the (V-)PCF knows alternate or backup IPv4,  Ipv6 Addess(es) or FQDN(s) where to send Notifications (e.g. via "altNotifIpv4Addrs",  "altNotifIpv6Addrs" or "altNotifFqdns" attributes received when the policy association was created or via AMFStatusChange Notifications, or via the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the service name and GUAMI obtained during the creation of the subscription) to query the other AMFs within the AMF set), the (V-)PCF shall exchange the authority part of the corresponding Notification URI with one of those addresses and shall use that URI in any subsequent communication.

If the (V-)PCF received a "404 Not found" response, the (V-)PCF should resend the failed policy update notification request to that URI.

#### 4.2.4.3	Request for termination of the policy association

Figure 4.2.4.3-1 illustrates the request for a termination of the policy association.

Figure 4.2.4.3-1: request for a termination of the UE policy association

NOTE:	For the roaming case, the PCF represents the V-PCF if the NF service consumer is an AMF and the PCF represents the H-PCF if the NF service consumer is a V-PCF.

The (V-)(H-)PCF may request the termination of the UE policy association and shall then send an HTTP POST request with "{notificationUri}/terminate" as URI (where the Notification URI was previously supplied by the NF service consumer) and the TerminationNotification data structure as request body that shall include:

-	the resource URI of the concerned individual UE policy association (including the policy association ID) encoded as "resourceUri" attribute; and

-	the cause why the (V-)(H-)PCF requests the termination of the policy association encoded as "cause" attribute.

Upon the reception of the HTTP POST request, the NF service consumer:

-	if the V-PCF is the NF service consumer, shall send as NF service producer for the corresponding policy association (towards the AMF as NF service consumer) a request for a termination of the policy association according to the present clause;

-	shall either send an HTTP "204 No Content" response for the succesfull processing of the HTTP POST request or an appropriate failure response, for the V-PCF as the NF service consumer taking into consideration a reply received for the possible corresponding policy association termination request according to the previous bullet; and

-	if errors occur when processing the HTTP POST request, shall send an HTTP error response as specified in clause 5.7; or

-	if the feature "ES3XX" is supported, and the NF service consumer determines that the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

After the succesfull processing of the HTTP POST request, any NF service consumer except for the V-PCF shall invoke the Npcf_UEPolicyControl_Delete Service Operation defined in clause 4.2.5 to terminate the policy association.

If the AMF as NF service consumer is not able to handle the notification but knows by implementation specific means that another AMF is able to handle the notification, it shall reply with an HTTP "307 Temporary Redirect" response pointing to the URI of the new AMF. If the AMF as NF service consumer is not able to handle the notification but another unknown AMF could possibly handle the notification, it shall reply with an HTTP "404 Not found" error response.

If the (V-)PCF receives a "307 Temporary Redirect" response, the PCF shall resend the failed request for termination of the policy association using the received URI in the Location header field as Notification URI.

If the (V-)PCF becomes aware that a new NF service consumer (AMF) is requiring notifications (e.g. via the "404 Not found" response or via Namf_Communication service AMFStatusChange Notifications, see 3GPP TS TS 29.518 [14], or via link level failures), and the (V-)PCF knows alternate or backup Ipv4, Ipv6 Addess(es) or FQDN(s) where to send Notifications (e.g. via "altNotifIpv4Addrs", "altNotifIpv6Addrs" or "altNotifFqdns" attributes received when the policy association was created or via AMFStatusChange Notifications, or via the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the service name and GUAMI obtained during the creation of the subscription) to query the other AMFs within the AMF set), the (V-)PCF shall exchange the authority part of the corresponding Notification URI with one of those addresses and shall resend the failed request for termination of the policy association to that URI.

If the (V-)PCF received a "404 Not found" response, the (V-)PCF should resend the failed request for termination of the policy association to that URI.

#### 4.2.4.4	URSP provisioning for Background Data Transfer policy

If the "EnhancedBackgroundDataTransfer" feature is supported, after the UE policy association establishment, the (H-)PCF may receive the Background Data Transfer Reference ID(s) notified by the UDR for the change of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case, the (H-)PCF shall retrieve the transfer policy corresponding to the Background Data Transfer Reference ID(s) as defined in clause 5.2.8 of 3GPP TS 29.519 [17] and derive the URSP including the Route Selection Validation Criteria for the UE as defined in clause 6.2.2.1 of 3GPP TS 23.503 [4]. The H-PCF shall provision the URSP to the V-PCF as defined in clause 4.2.4.2 and then the V-PCF shall invoke the Namf_Communication_N1N2MessageTransfer service operation to provision it to the UE. The (H-)PCF shall use the associated S-NSSAI and DNN to store in the UDR the Background Data Transfer Reference ID(s) in the UE's session management policy data as specified in 3GPP TS 29.519 [17].

#### 4.2.4.5	UE policy provisioning for V2X communication over PC5 and Uu reference points

After the UE policy association establishment and if the "V2X" feature is supported, the (H-)PCF may receive the service specific parameter information notified by the UDR for the change of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case:

-	for the roaming case, the H-PCF shall derive the V2XP and provision it to the V-PCF as defined in clause 4.2.4.2; and/or

-	for the roaming and non-roaming case, the (V-)PCF shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to send "MANAGE UE POLICY COMMAND" message(s) with the V2XP to the UE via the AMF.

#### 4.2.4.6	UE policy provisioning for 5G ProSe

After the UE policy association establishment and if the "ProSe" feature is supported, the (H-)PCF may receive the service specific parameter information via a notification on the change of UE's Application Data from the UDR, as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case:

-	for the roaming case, the H-PCF shall derive the ProSeP and provision it to the V-PCF as defined in clause 4.2.4.2; and/or

-	for the roaming and non-roaming case, the (H-)PCF shall derive the ProSeP and the (V-)PCF shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to convey it to the UE via the AMF by sending "MANAGE UE POLICY COMMAND" message(s) as defined in 3GPP TS 24.554 [28].

#### 4.2.4.7	UE policy provisioning for AF-influenced URSP

If the "AfGuideURSP" feature is supported by the Nudr_DataRepository service, after the UE policy association establishment, the (H-)PCF may be informed that service specific parameter information that contains data for AF guidance on the URSP determination has been created, modified or removed via a notification by the UDR for the change or removal of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case, the H-PCF may derive new URSP(s), modify existing URSP(s) or remove existing URSP(s) by using the information received from the UDR (see clause 4.2.2.2.1.1 and 4.2.2.2.3 for the description of how the (H-)PCF may use this information, stored UPSC(s), policy subscription information, analytics information received from NWDAF and local operator policy to determine the URSP that will be provisioned to the UE).

If the "VPLMNSpecificURSP" feature is supported by the Nudr_DataRepository service, the (H-)PCF may be informed about changes on service parameter data in the HPLMN that contain AF guidance on VPLMN-specific URSP rule determination as defined in 3GPP TS 29.519 [17]. For the roaming case and when the feature "VPLMNSpecificURSP" is supported, the H-PCF may be informed about changes on VPLMN-specific URSP from the V-PCF as defined in clause 4.2.3.1. Based on the received information, the (H-)PCF determines the VPLMN-specific URSP rules as specified in clause 4.2.2.2.3.2 and the new UE Policy Sections and VPS Configuration as defined in clause 4.2.2.2.1.1.

The (H-)PCF shall:

-	for the roaming case, provision the derived new UE Policy Sections, and/or update and/or remove existing UE Policy Sections to the V-PCF as defined in clause 4.2.4.2 and then the V-PCF shall invoke the Namf_Communication_N1N2MessageTransfer service operation to provision the received UE Policy Sections  to the UE; or

-	for the non-roaming case, use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to convey the derived new UE Policy Sections and/or to update and/or remove existing UE Policy Sections to the UE via the AMF within "MANAGE UE POLICY COMMAND" message(s).

In the roaming case, when the AMF informs the V-PCF that the UE is temporarily unreachable (see 3GPP TS 29.518 [18]), the V-PCF notifies the H-PCF accordingly (including the "uePolTransFailNotif" attribute within the PolicyAssociationUpdateRequest data structure, as described in clause 4.2.2.2.1.0).

When the (H-)PCF receives the "MANAGE UE POLICY COMPLETE" or the "MANAGE UE POLICY COMMAND REJECT" message and/or the PCF deducts that the UE is temporarily unreachable, and the PCF determines that the received message or the internal deduction indicates a UE Policy Delivery outcome event is matched:

-	if an NF service consumer has subscribed via a request for service specific parameters to the HPLMN, the (H-)PCF shall invoke the Npcf_EventExposure_Notify service operation as defined in clause 4.2.4.2 of 3GPP TS 29.523 [30]; or

-	if the "VPLMNSpecificURSP" is supported and the V-PCF has subscribed with the H-PCF as specified in clauses 4.2.2.1 and/or 4.2.3.1 because an AF has subscribed via a request for service parameters to the VPLMN, the H-PCF shall invoke the Npcf_UEPolicyControl_UpdateNotify as specified in this clause to notify about the result of the delivery of UE policies using the "delivReport" attribute. The "delivReport" attribute is a map of "eventNotifs" attributes, where:

a.	the key of the map represents the related AF; and

b.	each "eventNotifs" entry shall contain the reported event(s) ("SUCCESS_UE_POL_DEL_SP" or "UNSUCCESS_UE_POL_DEL_SP") within the "event" attribute and in case of delivery failure, the "delivFailure" attribute with the corresponding failure reason;

the V-PCF, based on the information received in the "delivReport" attribute and the notification information retrieved from the UDR in the VPLMN, shall invoke the Npcf_EventExposure_Notify service operation as defined in clause 4.2.4.2 of 3GPP TS 29.523 [30].

When the AMF (non roaming case) or the V-PCF (roaming case) informs the (H-)PCF that the UE is temporarily unreachable (see 3GPP TS 29.518 [18]), the (H-) PCF may subscribe to "CON_STATE_CH" trigger if not done before and reattempt the provisioning of URSP(s) when the UE becomes reachable.

#### 4.2.4.8	UE policy provisioning for A2X communication over PC5 and A2X communication over Uu reference point

After the UE policy association establishment and if the "A2X" feature is supported, the (H-)PCF may receive the service specific parameter information notified by the UDR for the change of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case:

-	for the roaming case, the H-PCF shall derive the A2XP and provision it to the V-PCF as defined in clause 4.2.4.2; and/or

-	for the roaming and non-roaming case, the (V-)PCF shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to send "MANAGE UE POLICY COMMAND" message(s) with the A2XP to the UE via the AMF.

#### 4.2.4.9	URSP provisioning in EPS

When the "EpsUrsp" feature is supported and a PCF for a PDU session is the NF service consumer, the PCF for the UE may provide a UE Policy Container (with a "MANAGE UE POLICY COMMAND" message(s) with the UE policy to send to the UE via the PCF for the PDU session) and/or an update in the Policy Control Request Triggers applicable to the UE as described in clause 4.2.4.2 using the selected UE Policy Association.

1)	When the PCF for the PDU session receives a UE Policy Container from the PCF for the UE, the PCF for the PDU session first shall select one of the ongoing PDN connections for the related UE in EPC and forward the UE Policy Container to the UE as follows:

-	if the PCF for the PDU session has previously received a UE Policy Container from the UE over an ongoing PDN connection, the PCF for the PDU session shall select that PDN connection; or

-	otherwise when 5GS to EPS mobility applies, the PCF for the PDU session shall select an ongoing PDN connection that supports URSP provisioning in EPS (i.e. a PDN connection served by the associated SMF+PGW-C supporting the transfer of UE Policy Container using ePCO, where the URSP provisioning in EPS support indicator was provided during PDN connection establishment or PDU session establishment as described in 3GPP TS 29.512 [31]). If there are multiple ongoing PDN connections that support URSP provisioning in EPS the PCF for the PDU session shall select one of those based on operator policy or implementation specific means; and

-	the PCF for the PDU session shall use the Npcf_SMPolicyControl_UpdateNotify service operation defined in 3GPP TS 29.512 [31] to forward to the UE via SMF+PGW-C the UE Policy Container with the "MANAGE UE POLICY COMMAND" message(s) with the received UE policy.

2)	When the PCF for the PDU session receives an update in the Policy Control Request Triggers applicable to the UE, the PCF for the PDU session shall determine whether an update on the current Policy Control Triggers need to be sent to the SMF+PGW-C. In that case, the PCF for the PDU session shall select one of the ongoing PDN connection(s) for the related UE in EPC, and shall provision the received policy control requested trigger(s) to the SMF+PGW-C as follows:

-	if the PCF for the PDU session has previously received a UE Policy Container from the UE over an ongoing PDN connection, the PCF for the PDU session shall select that PDN connection; or

-	otherwise when 5GS to EPS mobility applies, the PCF for the PDU session shall select the PDN connection that supports URSP provisioning in EPS as described in bullet 1) if there is no PDN connection selected yet; otherwise, it uses the PDN connection selected to transfer UE Policy Containers to the UE; and

-	the PCF for the PDU session shall use the Npcf_SMPolicyControl_UpdateNotify service operation to provision the received policy control requested trigger(s) according to 3GPP TS 29.512 [31].

When there are multiple ongoing UE Policy Associations that enable the URSP provisioning in EPS, the PCF for the UE shall select one of those for both, URSP and/or Policy Control Request Trigger provisioning, based on operator policy or implementation specific means.

#### 4.2.4.10	UE policy provisioning for Ranging/SL

After the UE policy association establishment and if the "Ranging_SL" feature is supported, the (H-)PCF may receive the service specific parameter information via a notification on the change of UE's Application Data from the UDR, as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case:

-	for the roaming case, the H-PCF shall derive the RSLPP and provision it to the V-PCF as defined in clause 4.2.4.2; and/or

-	for the roaming and non-roaming case, the (H-)PCF shall derive the RSLPP and the (V-)PCF shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to convey it to the UE via the AMF by sending "MANAGE UE POLICY COMMAND" message(s).

### 4.2.5	Npcf_UEPolicyControl_Delete Service Operation

Figure 4.2.5-1 illustrates the deletion of a policy association.

Figure 4.2.5-1: Deletion of a policy association

NOTE:	For the roaming case, the PCF represents the V-PCF if the NF service consumer is an AMF and the PCF represents the H-PCF if the NF service consumer is a V-PCF.

The AMF as NF service consumer requests that the policy association is deleted when the corresponding UE context is terminated, e.g. during UE de-registration from the network. In roaming scenarios, the V-PCF requests to the H-PCF the deletion of the UE policy association when the V-PCF determines that the UE context is terminated in the AMF, e.g., with the reception of the policy association deletion request or the reception of UE context not found reply to the UE Policy delivery request.

During the AMF relocation, the old AMF shall invoke this procedure when:

-	the resource URI of the individual UE Policy Association resource is not transferred to the new AMF; or

-	the new AMF informs the old AMF that the individual UE Policy Association resource is not being reused.

The PCF for the PDU session as NF service consumer requests that the UE policy association is deleted when:

-	all the PDU sessions related with the UE policy association are terminated; or

-	the PCF for the PDU session receives an indication of RAT type change from the SMF+PGW-C (from any of the related PDU sessions) and determines the EPS to 5GS mobility scenario applies.

To request that the UE policy association is deleted, the NF service consumer (e.g. AMF) shall send an HTTP DELETE request with "{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}" as Resource URI.

Upon the reception of the HTTP DELETE request,

-	the (V-)(H-)PCF shall delete the policy association;

-	if the PCF is a V-PCF and has an established corresponding policy association towards the H-PCF, the V-PCF shall send as the NF service consumer towards the H-PCF a request for the deletion of that policy association as described in the present clause;

-	the (V-)(H-)PCF shall send either an HTTP "204 No Content" response indicating the success of the deletion or an appropriate failure response, for the V-PCF as PCF taking into consideration a reply received for the possible policy association deletion request according to the previous bullet; and

-	the (V-)(H-)PCF shall if errors occur when processing the HTTP DELETE request, send an HTTP error response as specified in clause 5.7; or

-	if the feature ES3XX is supported, and the (V-)(H-)PCF determines the received HTTP DELETE request needs to be redirected, the (V-)(H-)PCF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

Once the UE policy association is deleted, to unsubscribe to notifications of N1 message for UE Policy Delivery Result, the (V-)PCF shall trigger the Namf_Communication_N1N2MessageUnsubscribe service operation towards the N1N2 Individual Subscription resource as specified in 3GPP TS 29.518 [14].

# 5	Npcf_UEPolicyControl API

## 5.1	Introduction

The Access and Mobility Policy Control Service shall use the Npcf_UEPolicyControl API.

The API URI of the Npcf_UEPolicyControl API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the PCF shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "npcf-ue-policy-control".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.3.

## 5.2	Usage of HTTP

### 5.2.1	General

HTTP/2, IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [10] specification of HTTP messages and content bodies for the Npcf_UEPolicyControl is contained in Annex A.

### 5.2.2	HTTP standard headers

#### 5.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

#### 5.2.2.2	Content type

JSON, IETF RFC 8259 [9], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [21].

### 5.2.3	HTTP custom headers

The Npcf_UEPolicyControl API shall support HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [5] and may support HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [5].

In this Release of the specification, no specific custom headers are defined for the Npcf_UEPolicyControl API.

## 5.3	Resources

### 5.3.1	Resource Structure

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 5.3.1-1 depicts the resource URIs structure for the Npcf_UEPolicyControl API.

Figure 5.3.1-1: Resource URI structure of the Npcf_UEPolicyControl API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| UE Policy Associations | /policies | POST | Create a new Individual UE policy association resource. |
| Individual UE Policy Association | /policies/{polAssoId} | GET | Read an Individual UE Policy Association resource. |
|  |  | DELETE | Delete an Individual UE Policy Association resource. |
|  | /policies/{polAssoId}/update | update (POST) | Report observed event trigger and obtain updated UE policies. |

### 5.3.2	Resource: UE Policy Associations

#### 5.3.2.1	Description

This resource represents a collection of UE policy associations.

#### 5.3.2.2	Resource definition

Resource URI: {apiRoot}/npcf-ue-policy-control/v1/policies

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1 |

#### 5.3.2.3	Resource Standard Methods

##### 5.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyAssociationRequest | M | 1 | Input parameters for the creation of a policy association. |

Table 5.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PolicyAssociation |  | M |  | 1 |  | 201 Created |  | Policy association was created and policies are being provided. |  |
| ProblemDetails |  | O |  | 0..1 |  | 400 Bad Request |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: Failure cases are described in clause 5.7. |  |  |  |  |  |  |  |  |  |

Table 5.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId} |

### 5.3.3	Resource: Individual UE Policy Association

#### 5.3.3.1	Description

This resource represents an individual UE policy association.

#### 5.3.3.2	Resource definition

Resource URI: {apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1. |
| polAssoId | string | Identifier of a policy association. |

#### 5.3.3.3	Resource Standard Methods

##### 5.3.3.3.1	GET

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PolicyAssociation |  | M |  | 1 |  | 200 OK |  |  |  |
| RedirectResponse |  | O |  | 0..1 |  | 307 Temporary Redirect |  | Temporary redirection, during Individual UE policy retrieval. Applicable if the feature "ES3XX" is supported. (NOTE 2) |  |
| RedirectResponse |  | O |  | 0..1 |  | 308 Permanent Redirect |  | Permanent redirection, during Individual UE policy retrieval. Applicable if the feature "ES3XX" is supported. (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP/SEPP (refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]). |  |  |  |  |  |  |  |  |  |

Table 5.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative PCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PCF (service) instance towards which the request is redirected. |

Table 5.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative PCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PCF (service) instance towards which the request is redirected. |

##### 5.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 5.3.3.3.2-1.

Table 5.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.3.3.2-2 and the response data structures and response codes specified in table 5.3.3.3.2-3.

Table 5.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  | 204 No Content |  | The policy association was successfully deleted. |  |
| RedirectResponse |  | O |  | 0..1 |  | 307 Temporary Redirect |  | Temporary redirection, during Individual UE policy deletion. Applicable if the feature "ES3XX" is supported. (NOTE 2) |  |
| RedirectResponse |  | O |  | 0..1 |  | 308 Permanent Redirect |  | Permanent redirection, during Individual UE policy deletion. Applicable if the feature "ES3XX" is supported. (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP/SEPP (refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]). |  |  |  |  |  |  |  |  |  |

Table 5.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative PCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PCF (service) instance towards which the request is redirected. |

Table 5.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative PCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PCF (service) instance towards which the request is redirected. |

#### 5.3.3.4	Resource Custom Operations

##### 5.3.3.4.1	Overview

Table 5.3.3.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Update | /policies/{polAssoId}/update | POST | Report observed event trigger and obtain updated policies. |

##### 5.3.3.4.2	Operation: Update

###### 5.3.3.4.2.1	Description

The update custom operation allows an NF service consumer to report the occurrence on one or more policy request trigger(s) and to obtain related updated policies.

###### 5.3.3.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 5.3.3.4.2.2-1 and the response data structure and response codes specified in table 5.3.3.4.2.2-2.

Table 5.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyAssociationUpdateRequest | M | 1 | Describes the observed policy control request trigger(s). |

Table 5.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PolicyUpdate |  | M |  | 1 |  | 200 OK |  | Describes updated policies. |  |
| RedirectResponse |  | O |  | 0..1 |  | 307 Temporary Redirect |  | Temporary redirection, during Individual UE policy modification. Applicable if the feature "ES3XX" is supported. (NOTE 3) |  |
| RedirectResponse |  | O |  | 0..1 |  | 308 Permanent Redirect |  | Permanent redirection, during Individual UE policy modification. Applicable if the feature "ES3XX" is supported. (NOTE 3) |  |
| ProblemDetails |  | O |  | 0..1 |  | 400 Bad Request |  | (NOTE 2) |  |  |
| ProblemDetails |  | O |  | 0..1 |  | 404 Not Found |  | (NOTE 2) |  |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: Failure cases are described in clause 5.7. NOTE 3: The RedirectResponse data structure may be provided by an SCP/SEPP (refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]). |  |  |  |  |  |  |  |  |  |

Table 5.3.3.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative PCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PCF (service) instance towards which the request is redirected. |

Table 5.3.3.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative PCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target PCF (service) instance towards which the request is redirected. |

## 5.4	Custom Operations without associated resources

None.

## 5.5	Notifications

### 5.5.1	General

Table 5.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Policy Update Notification | {notificationUri}/update | update (POST) | Policy Update Notification. |
| Request for termination of the UE policy association | {notificationUri}/terminate | termminate (POST) | Request for termination of the policy association. |

### 5.5.2	Policy Update Notification

#### 5.5.2.1	Description

This notification is used by the H-PCF to provide updates of UE policies to the V-PCF as NF service consumer, and used by the V-PCF to provide updates of policy control request triggers to the AMF as NF service consumer.

#### 5.5.2.2	Operation Definition

This operation shall support the request data structures specified in table 5.5.2.2-1 and the response data structure and response codes specified in table 5.5.2.2-2.

Table 5.5.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyUpdate | M | 1 | Updated policies. |

Table 5.5.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UeRequestedValueRep |  | M |  | 1 |  | 200 OK |  | The current applicable values corresponding to the policy control request trigger are reported. |  |
| n/a |  |  |  |  |  | 204 No Content |  | The policies were successfully updated. |  |
| RedirectResponse |  | O |  | 0..1 |  | 307 Temporary Redirect |  | Temporary redirection, during UE policy notification. Applicable if the feature "ES3XX" is supported. (NOTE 3) |  |
| RedirectResponse |  | O |  | 0..1 |  | 308 Permanent Redirect |  | Permanent redirection, during UE policy notification. Applicable if the feature "ES3XX" is supported. (NOTE 3) |  |
| ProblemDetails |  | O |  | 0..1 |  | 404 Not Found |  | The NF service consumer can use this response when the notification can be sent to another unknown host. Applicable if the feature "ErrorResponse" is supported. |  |
| ProblemDetails |  | O |  | 0..1 |  | 400 Bad Request |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: Failure cases are described in clause 5.7. NOTE 3: The RedirectResponse data structure may be provided by an SCP/SEPP (refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]). |  |  |  |  |  |  |  |  |  |

Table 5.5.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NF service consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. May be included if the feature "ES3XX" is supported. |

Table 5.5.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

### 5.5.3	Request for termination of the UE policy association

#### 5.5.3.1	Description

This notification is used by the PCF to request the termination of a UE policy association.

#### 5.5.3.2	Operation Definition

This operation shall support the request data structures specified in table 5.5.3.2-1 and the response data structure and response codes specified in table 5.5.3.2-2.

Table 5.5.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TerminationNotification | M | 1 | Request to terminate the policy association. |

Table 5.5.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  | 204 No Content |  | The request for policy association termination was received. |  |
| RedirectResponse |  | O |  | 0..1 |  | 307 Temporary Redirect |  | Temporary redirection, during UE policy notification. Applicable if the feature "ES3XX" is supported. (NOTE 2) |  |
| RedirectResponse |  | O |  | 0..1 |  | 308 Permanent Redirect |  | Permanent redirection, during UE policy notification. Applicable if the feature "ES3XX" is supported. (NOTE 2) |  |
| ProblemDetails |  | O |  | 0..1 |  | 404 Not Found |  | The NF service consumer can use this response when the notification can be sent to another unknown host. |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP/SEPP (refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]). |  |  |  |  |  |  |  |  |  |

Table 5.5.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. It may be included if the feature "ES3XX" is supported. |

Table 5.5.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resources located in an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP/SEPP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

## 5.6	Data Model

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_UEPolicyControl service based interface protocol.

Table 5.6.1-1: Npcf_UEPolicyControl specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A2xCapability |  | 5.6.3.12 |  | Indicates the A2X capabilities |  | A2X |  |
| LboRoamingInformation |  | 5.6.2.10 |  | LBO roaming information for a DNN and S-NSSAI |  | VPLMNSpecificURSP |  |
| N1N2MessTransferErrorReply |  | 5.6.3.8 |  | Error the V-PCF may send to the H-PCF when the V-PCF receives from the AMF an error to the N1N2MessageTransfer request. |  | EnErrorHandling |  |
| Non3gppAccess |  | 5.6.3.7 |  | Represents a Non-3gpp access node. |  | SliceAwareANDSP |  |
| Pc5Capability |  | 5.6.3.5 |  | Indicates the specific PC5 RAT(s) which the UE supports for V2X communications and/or A2X communications over PC5 reference point. |  | V2X |  |
| ProSeCapability |  | 5.6.3.6 |  | Indicates the 5G ProSe capabilities. |  | ProSe |  |
| PolicyAssociation |  | 5.6.2.2 |  | Description of a policy association that is returned by the PCF when a policy Association is created, updated, or read. |  |  |  |
| PolicyAssociationReleaseCause |  | 5.6.3.4 |  | The cause why the PCF requests the termination of the policy association. |  |  |  |
| PolicyAssociationRequest |  | 5.6.2.3 |  | Information that NF service consumer provides when requesting the creation of a policy association. |  |  |  |
| PolicyAssociationUpdateRequest |  | 5.6.2.4 |  | Information that NF service consumer provides when requesting the update of a policy association. |  |  |  |
| PolicyStatus |  | 5.6.3.10 |  | Represents the configuration status of a UE Policy in the UE. |  | SliceAwareANDSP |  |
| PolicyUpdate |  | 5.6.2.5 |  | Updated policies that the PCF provides in a notification or in the reply to an Update Request. |  |  |  |
| RangSLCapability |  | 5.6.3.10 |  | Represents the Ranging/SL capabilities. |  | Ranging_SL |  |
| RequestTrigger |  | 5.6.3.3 |  | Enumeration of possible Request Triggers. |  |  |  |
| SliceSpecificN3gNodeSelectionCapability |  | 5.6.3.13 |  | Represents the UE capabilities with regard to slice-specific non-3gpp node selection. |  | SliceAwareANDSP |  |
| TerminationNotification |  | 5.6.2.6 |  | Request to terminate a policy Association that the PCF provides in a notification. |  |  |  |
| UeRequestedValueRep |  | 5.6.2.8 |  | Contains the current applicable values corresponding to the policy control request triggers. |  | ImmediateReport |  |
| UePolicy |  | 5.6.3.2 |  | UE Policies |  |  |  |
| UePolicyDeliveryResult |  | 5.6.3.2 |  | UE Policy delivery Result |  |  |  |
| UePolicyNotification |  | 5.6.2.12 |  | Contains the delivery outcome of VPLMN-Specific URSP rules |  | VPLMNSpecificURSP |  |
| UePolicyParameters |  | 5.6.2.9 |  | Contains the service parameters used to guide the VPLMN-specific URSP rule determination. |  | VPLMNSpecificURSP |  |
| UePolicyRequest |  | 5.6.3.2 |  | Request for UE Policies |  |  |  |
| UePolicyTransferFailureCause |  | 5.6.4.1 |  | UE Policy Transfer Failure Cause |  | EnErrorHandling |  |
| UePolicyTransferFailureNotification |  | 5.6.2.7 |  | Information that the UE policy is failure to be transferred to the UE. |  |  |  |
| UrspEnforcementPduSession |  | 5.6.2.11 |  | Represents URSP rule enforcement information for a PDU session. |  | URSPEnforcement |  |

Table 5.6.1-2 specifies data types re-used by the Npcf_UEPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_UEPolicyControl service based interface.

Table 5.6.1-2: Npcf_UEPolicyControl re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AccessType |  | 3GPP TS 29.571 [11] |  | Represents an Access Type. |  |  |  |
| Bytes |  | 3GPP TS 29.571 [11] |  | String with format "byte". |  |  |  |
| ChargingInformation |  | 3GPP TS 29.512 [31] |  | Contains the addresses, and if available, the instance ID and set ID, of the charging functions. |  | SLAMUP |  |
| ConfiguredSnssai |  | 3GPP TS 29.531 [34] |  | Contains the configured S-NSSAI and optionally mapped home S-NSSA. |  | SliceAwareANDSP, NssaiChange |  |
| CmState |  | 3GPP TS 29.518 [14] |  | Connectivity state of UE |  | ConnectivityStateChange |  |
| Dnn |  | 3GPP TS 29.571 [11] |  | Represents a DNN. |  | VPLMNSpecificURSP URSPEnforcement |  |
| Event |  | 3GPP TS 29.522 [41] |  | Subscription to notification about delivery of VPLMN specific URSP rule. |  | VPLMNSpecificURSP |  |
| Fqdn |  | 3GPP TS 29.571 [11] |  | FQDN |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [11] |  | Generic Public Subscription Identifier |  |  |  |
| GroupId |  | 3GPP TS 29.571 [11] |  | Represents a UE Group Identifier |  |  |  |
| Guami |  | 3GPP TS 29.571 [11] |  | Globally Unique AMF Identifier |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [11] |  | Represents an Ipv4 address. |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [11] |  | Represents an Ipv6 address. |  |  |  |
| N1N2MessageTransferCause |  | 3GPP TS 29.518 [14] |  | Contains an error cause for an N1 or N2 message transfer. |  |  |  |
| N2InfoContent |  | 3GPP TS 29.518 [14] |  | Represents a transparent N2 information content to be relayed by AMF. |  | V2X, A2X, ProSe, Ranging_SL |  |
| NfInstanceId |  | 3GPP TS 29.571 [11] |  | Represents an NF instance identifier |  |  |  |
| NfSetId |  | 3GPP TS 29.571 [11] |  | Represents an NF set identifier |  | EnhEstRoaming |  |
| PcEventNotification |  | 3GPP TS 29.523 [30] |  | Represents notification about UE Policy Delivery outcome |  | VPLMNSpecificURSP |  |
| PcfUeCallbackInfo |  | 3GPP TS 29.571 [11] |  | Contains the PCF for the UE callback information necessary for the PCF for the PDU session to send Establishment and Termination event. |  | URSPEnforcement |  |
| PduSessionInfo |  | 3GPP TS 29.571 [11] |  | Contains a DNN and SNSSAI combination |  | VPLMNSpecificURSP URSPEnforcement |  |
| PduSessionType |  | 3GPP TS 29.571 [13] |  | Contains the PDU Session Type |  | URSPEnforcement |  |
| Pei |  | 3GPP TS 29.571 [11] |  | Permanent Equipment Identifier |  |  |  |
| PlmnId |  | 3GPP TS 29.571 [11] |  | Represents a PLMN identifier. |  |  |  |
| PlmnIdNid |  | 3GPP TS 29.571 [11] |  | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [11] |  | Presence reporting area information |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [11] |  | Contains detailed information about an error response. |  |  |  |
| RatType |  | 3GPP TS 29.571 [11] |  | Represents a RAT type. |  |  |  |
| RedirectResponse |  | 3GPP TS 29.571 [11] |  | Contains redirection related information. |  | ES3XX |  |
| ServiceName |  | 3GPP TS 29.510 [13] |  | Name of the service instance. |  |  |  |
| SatelliteBackhaulCategory |  | 3GPP TS 29.571 [11] |  | Indicates the satellite backhaul category or non-satellite backhaul. |  | EnSatBackhaulCategoryChg |  |
| Snssai |  | 3GPP TS 29.571 [11] |  | Represents an S-NSSAI |  | SliceAwareANDSP |  |
| SscMode |  | 3GPP TS 29.571 [11] |  | Service and session continuity mode. |  | URSPEnforcement |  |
| Supi |  | 3GPP TS 29.571 [11] |  | Subscription Permanent Identifier |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [11] |  | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [11] |  | Represents a time zone. |  |  |  |
| Uinteger |  | 3GPP TS 29.571 [11] |  | Unsigned integer. |  |  |  |
| Uri |  | 3GPP TS 29.571 [11] |  | Represents a URI. |  |  |  |
| UrspEnforcementInfo |  | 3GPP TS 29.512 [31] |  | URSP rule enforcement information as received from the UE. |  | URSPEnforcement |  |
| UrspRuleRequest |  | 3GPP TS 29.522 [41] |  | URSP rule guidance information |  | VPLMNSpecificURSP |  |
| UserLocation |  | 3GPP TS 29.571 [11] |  | Contains User Location information. |  |  |  |

### 5.6.2	Structured data types

#### 5.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

#### 5.6.2.2	Type PolicyAssociation

Table 5.6.2.2-1: Definition of type PolicyAssociation

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| request |  | PolicyAssociationRequest |  | O |  | 0..1 |  | The information provided by the NF service consumer when requesting the creation of a policy association |  |  |  |
| uePolicy |  | UePolicy |  | O |  | 0..1 |  | The UE policy as determined by the H-PCF (for the H-PCF as NF service producer). |  |  |  |
| n2Pc5Pol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for V2X communications as determined by the H-PCF. |  | V2X |  |
| n2Pc5PolA2x |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for A2X communications as determined by the H-PCF. |  | A2X |  |
| n2Pc5ProSePol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for 5G ProSe as determined by the PCF. |  | ProSe |  |
| triggers |  | array(RequestTrigger) |  | O |  | 1..N |  | Request Triggers to which the PCF subscribes. (NOTE 1) |  |  |  |
| pras |  | map(PresenceInfo) |  | C |  | 1..N |  | If the Request Trigger "PRA_CH" is provided, the presence reporting area(s) for which reporting is requested shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" and the "additionalPraId" attributes within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of either a presence reporting area or a presence reporting area set. |  |  |  |
| andspDelInd |  | PolicyStatus |  | O |  | 0..1 |  | Information about whether the updated ANDSP/WLANSP has been successfully delivered to the UE. |  | SliceAwareANDSP |  |
| andspInd |  | boolean |  | O |  | 0..1 |  | Indication of UE support of ANDSP. True: The UE supports ANDSP; False: The UE does not support ANDSP. |  | UECapabilityIndication |  |
| pduSessions |  | array(PduSessionInfo) |  | O |  | 1..N |  | Contains the DNNs and S-NSSAIs for which LBO information is being requested. It may be provided when the "LBO_INFO_CH" request trigger is provided. |  | VPLMNSpecificURSP |  |
| chfInfo |  | ChargingInformation |  | O |  | 0..1 |  | Contains the CHF address(es), and if available, the associated CHF instance ID(s) and CHF set ID(s). (NOTE 3) |  | SLAMUP |  |
| suppFeat |  | SupportedFeatures |  | M |  | 1 |  | Indicates the negotiated supported features. |  |  |  |
| n2Pc5RsppPol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for Ranging/SL as determined by the H-PCF. |  | Ranging_SL |  |
| pcfUeInfo |  | PcfUeCallbackInfo |  | O |  | 0..1 |  | Contains the PCF for the UE information necessary for the PCF for the PDU session to send established/terminated events notifications to the PCF for the UE. |  | URSPEnforcement |  |
| matchPdus |  | array(PduSessionInfo) |  | C |  | 1..N |  | Indicates the matched PDU session(s) for which the AMF shall forward the PCF for the UE callback information in the "pcfUeInfo" attribute to the SMF. It shall be present when the "pcfUeInfo" attribute is present. (NOTE 2) |  | URSPEnforcement |  |
| NOTE 1: Only the RequestTrigger enumeration values corresponding to PCRTs that require explicit subscription as defined in clause 5.6.3.3 shall be applicable within the "triggers" attribute. NOTE 2: The DNN encoded within the PduSessionInfo element(s) of the "matchPdus" array may contain a full DNN or only the DNN Network Identifier. When the DNN contains the Network Identifier only, the AMF shall match a PDU session for the received Network Identifier and for any value of the Operator Identifier. NOTE 3: This attribute may only be supplied by the PCF in the response to the initial POST request that requested the creation of an individual UE policy resource. |  |  |  |  |  |  |  |  |  |  |  |

#### 5.6.2.3	Type PolicyAssociationRequest

Table 5.6.2.3-1: Definition of type PolicyAssociationRequest

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri |  | Uri |  | M |  | 1 |  | Identifies the recipient of Notifications sent by the PCF. |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  | O |  | 1..N |  | Alternate or backup IPv4 Addess(es) where to send Notifications. |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  | O |  | 1..N |  | Alternate or backup IPv6 Addess(es) where to send Notifications. |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  | O |  | 1..N |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |
| supi |  | Supi |  | M |  | 1 |  | Subscription Permanent Identifier. |  |  |  |
| gpsi |  | Gpsi |  | C |  | 0..1 |  | Generic Public Subscription Identifier. Shall be provided when available. |  |  |  |
| accessType |  | AccessType |  | C |  | 0..1 |  | The Access Type where the served UE is camping. Shall be provided when available. |  |  |  |
| accessTypes |  | array(AccessType) |  | C |  | 1..N |  | The Access Type(s) where the served UE is camping. Shall be provided when available. |  | AccessChange |  |
| pei |  | Pei |  | C |  | 0..1 |  | The Permanent Equipment Identifier of the served UE. Shall be provided when available. |  |  |  |
| userLoc |  | UserLocation |  | C |  | 0..1 |  | The location of the served UE. Shall be provided when available. |  |  |  |
| timeZone |  | TimeZone |  | C |  | 0..1 |  | The time zone of the network where the served UE is camping. Shall be provided when available. |  |  |  |
| servingPlmn |  | PlmnIdNid |  | C |  | 0..1 |  | The serving network (a PLMN or an SNPN) where the served UE is camping. For the SNPN the NID together with the PLMN ID identifies the SNPN. Shall be provided when available. |  |  |  |
| ratType |  | RatType |  | C |  | 0..1 |  | The RAT Type where the served UE is camping. Shall be provided when available. |  |  |  |
| ratTypes |  | array(RatType) |  | C |  | 1..N |  | The RAT Type(s) where the served UE is camping. Shall be provided when available. |  | AccessChange |  |
| groupIds |  | array(GroupId) |  | C |  | 1..N |  | Internal Group Identifier(s) of the served UE. Shall be provided when available. |  |  |  |
| hPcfId |  | NfInstanceId |  | C |  | 0..1 |  | H-PCF Identifier. Shall be provided by the AMF in roaming scenarios when available. |  |  |  |
| hPcfUri |  | Uri |  | C |  | 0..1 |  | H-PCF URI. It shall be provided by the AMF in roaming scenarios, if available. When present, it shall contain the API URI of the Npcf_UEPolicyControl service of the H-PCF ID indicated in the "hPcfId" attribute. The API URI shall take the form specified in clause 5.1. |  | EnhEstRoaming |  |
| hPcfSetId |  | NfSetId |  | C |  | 0..1 |  | H-PCF Set Identifier of the H-PCF instance indicated in the "hPcfId" attribute. It shall be provided by the AMF in roaming scenarios, if available. |  | EnhEstRoaming |  |
| uePolReq |  | UePolicyRequest |  | C |  | 0..1 |  | A request for UE Policies. Shall be provided when the AMF receives an "UE STATE INDICATION" message, as defined in Annex D.5.4 of 3GPP TS 24.501 [15]. |  |  |  |
| guami |  | Guami |  | C |  | 0..1 |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer. |  |  |  |
| serviceName |  | ServiceName |  | O |  | 0..1 |  | If the NF service consumer is an AMF, it should provide the name of a service produced by the AMF that makes use of information received within the Npcf_UEPolicyControl_UpdateNotify service operation. |  |  |  |
| servingNfId |  | NfInstanceId |  | C |  | 0..1 |  | If the NF service consumer is an AMF, it shall contain the identifier of the serving AMF. |  |  |  |
| pc5Capab |  | Pc5Capability |  | C |  | 0..1 |  | Indicates the PC5 Capability for V2X communications supported by the UE. It shall be provided when available at the NF service consumer. |  | V2X |  |
| a2xCapab |  | array(A2xCapability) |  | C |  | 1..N |  | Indicates the A2X Capabilities for A2X communications supported by the UE. It shall be provided when available at the NF service consumer. |  | A2X |  |
| proSeCapab |  | array(ProSeCapability) |  | C |  | 1..N |  | Indicates whether the UE is capable of one or more of the the following 5G ProSe Capabilities: 5G ProSe Direct Discovery, 5G ProSe Direct Communication, Layer-2 and/or Layer 3 5G ProSe UE-to-Network Relay and Layer-2 and/or Layer 3 5G ProSe Remote UE, and when the "ProSe_Ph2" feature is supported, Layer-2 and/or Layer-3 5G ProSe UE-to-UE Relay and Layer-2 and/or Layer-3 5G ProSe End UE. It shall be provided when available at the NF service consumer. |  | ProSe |  |
| confSnssais |  | array(ConfiguredSnssai) |  | C |  | 1..N |  | The Configured NSSAI for the serving PLMN, and optionally the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN. When the feature SliceAwareANDSP is supported, it shall be provided in the roaming case when available at the NF service consumer and the "n3gNodeReSel" attribute is present. If the feature NssaiChange is supported, it shall be provided in the roaming case. (NOTE 1) |  | SliceAwareANDSP, NssaiChange |  |
| n3gNodeReSel |  | Non3gppAccess |  | O |  | 0..1 |  | A wrongly selected non-3gpp access node. It shall be provided when the UE has selected a non-3gpp access node that is not compatible with the Allowed NSSAI. |  | SliceAwareANDSP |  |
| sliceN3gNodeSelCap |  | SliceSpecificN3gNodeSelectionCapability |  | O |  | 0..1 |  | Indicates whether the UE supports N3IWF/TNGF selection based on the slices the UE wishes to use over untrusted/trusted non-3GPP access. |  | SliceAwareANDSP |  |
| satBackhaulCategory |  | SatelliteBackhaulCategory |  | C |  | 0..1 |  | Indicates types of the satellite backhaul based on satellite types (when satellite backhaul is used) or non-satellite backhaul (when satellite backhaul is not used). The default value "NON_SATELLITE" shall apply if the attribute is not present. |  | EnSatBackhaulCategoryChg |  |
| vpsUePolGuidance |  | map(UePolicyParameters) |  | O |  | 1..N |  | Contains the service parameter used to guide the VPLMN-specific URSP and may contain the subscription to VPLMN-specific URSP delivery outcome. The key of the map represents the AF request to guide the VPLMN-specific URSP rules. This attribute only applies in roaming and when the V-PCF is the NF service consumer. |  | VPLMNSpecificURSP |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  | O |  | 1..N |  | Contains LBO roaming information for DNN and S-NSSAI combination(s). This attribute only applies in roaming and when the AMF is the NF service consumer. |  | VPLMNSpecificURSP |  |
| 5gsToEpsMob |  | boolean |  | O |  | 0..1 |  | When it is set to true, it indicates the UE Policy Association creation is triggered by a 5GS to EPS mobility scenario. Default value is false. |  | EpsUrsp |  |
| s uppFeat |  | SupportedFeatures |  | M |  | 1 |  | Indicates the features supported by the service consumer. |  |  |  |
| rangSlCapab |  | array(RangSLCapability) |  | C |  | 1..N |  | Contains the Ranging/SL related UE capabilities. It shall be provided when available at the NF service consumer. |  | Ranging_SL |  |
| NOTE 1: The "mappedHomeSnssai" attribute within the ConfiguredSnssai data type may only be provided if the "NssaiChange" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |

#### 5.6.2.4	Type PolicyAssociationUpdateRequest

Table 5.6.2.4-1: Definition of type PolicyAssociationUpdateRequest

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri |  | Uri |  | O |  | 0..1 |  | Identifies the recipient of Notifications sent by the PCF. |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  | O |  | 1..N |  | Alternate or backup IPv4 Address(es) where to send Notifications. |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  | O |  | 1..N |  | Alternate or backup IPv6 Address(es) where to send Notifications. |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  | O |  | 1..N |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |
| triggers |  | array(RequestTrigger) |  | C |  | 1..N |  | Request Triggers that the NF service consumer observes. |  |  |  |
| praStatuses |  | map(PresenceInfo) |  | C |  | 1..N |  | If the Trigger "PRA_CH" is reported, the UE presence status for tracking area for which changes of the UE presence occurred shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" attribute within the PresenceInfo data type shall be supplied. The "additionalPraId" attribute within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of an individual presence reporting area. |  |  |  |
| userLoc |  | UserLocation |  | C |  | 0..1 |  | The location of the served UE shall be provided for trigger "LOC_CH". |  |  |  |
| uePolDelResult |  | UePolicyDeliveryResult |  | C |  | 0..1 |  | UE Policy Delivery Result. Shall be provided together with trigger "UE_POLICY" when a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15], has been received by the V-PCF and is being forwarded to the H-PCF. |  |  |  |
| uePolTransFailNotif |  | UePolicyTransferFailureNotification |  | C |  | 0..1 |  | The UE policy transfer failure notification. Shall be the provided together with trigger "UE_POLICY" when a response with HTTP status code 4xx or 5xx as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14] or a N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14] is received after the V-PCF provisioned the UE policy by invoking the Namf_Communication_N1N2MessageTransfer service operation to the AMF and is notifying the H-PCF. |  |  |  |
| uePolReq |  | UePolicyRequest |  | C |  | 0..1 |  | A request for UE Policies. Shall be provided together with trigger "UE_POLICY" when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24], if the "V2X" feature is supported, and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for 5G ProSe, as defined in clause 10.4.1 of 3GPP TS 24.554 [28], if the "ProSe" feature is supported and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for A2X, as defined 3GPP TS 24.577 [32], if the "A2X" feature is supported and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for Ranging/SL, as defined 3GPP TS 24.514 [42], if the "Ranging_SL" feature is supported.. |  | V2X, A2X, ProSe, Ranging_SL |  |
| guami |  | Guami |  | C |  | 0..1 |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer during the AMF relocation. |  |  |  |
| servingNfId |  | NfInstanceId |  | C |  | 0..1 |  | It shall contain the identifier of the new AMF during the AMF relocation. |  |  |  |
| plmnId |  | PlmnIdNid |  | C |  | 0..1 |  | The serving network identity (a PLMN or an SNPN) of the served UE shall be provided for trigger "PLMN_CH". |  | PlmnChange |  |
| connectState |  | CmState |  | C |  | 0..1 |  | The connectivity state of the served UE shall be provided for trigger "CON_STATE_CH". |  | ConnectivityStateChange |  |
| groupIds |  | array(GroupId) |  | C |  | 1..N |  | Internal Group Identifier(s) of the served UE. Shall be provided for trigger "GROUP_ID_LIST_CHG". |  | GroupIdListChange |  |
| pc5Capab |  | Pc5Capability |  | C |  | 0..1 |  | Indicates the PC5 Capability for V2X communications supported by the UE. It shall be provided when available at the NF service consumer. It shall be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "V2X" feature is supported. |  | FeatureRenegotiation |  |
| a2xCapab |  | array(A2xCapability) |  | C |  | 1..N |  | Indicates the A2X capabilities supported by the UE. It shall be provided when available at the NF service consumer. It shall be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "A2X" feature is supported. |  | FeatureRenegotiation |  |
| proSeCapab |  | array(ProSeCapability) |  | O |  | 1..N |  | Indicates whether the UE is capable of one or more of the the following 5G ProSe Capabilities: 5G ProSe Direct Discovery, 5G ProSe Direct Communication, Layer-2 and/or Layer 3 5G ProSe UE-to-Network Relay and Layer-2 and/or Layer 3 5G ProSe Remote UE, and when the "ProSe_Ph2" feature is supported, Layer-2 and/or Layer-3 5G ProSe UE-to-UE Relay and Layer-2 and/or Layer-3 5G ProSe End UE. |  | ProSe |  |
| confSnssais |  | array(ConfiguredSnssai) |  | C |  | 1..N |  | The Configured NSSAI for the serving PLMN, and optionally the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN. It shall be provided in case of roaming for trigger "CONF_NSSAI_CH" or for trigger "NON_3GPP_NODE_RESELECTION". (NOTE) |  | SliceAwareANDSP, NssaiChange |  |
| n3gNodeReSel |  | Non3gppAccess |  | O |  | 0..1 |  | A wrongly selected non-3gpp access node. It shall be provided when available at the NF service consumer and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute. |  | SliceAwareANDSP |  |
| sliceN3gNodeSelCap |  | SliceSpecificN3gNodeSelectionCapability |  | O |  | 0..1 |  | Indicates whether the UE supports N3IWF/TNGF selection based on the slices the UE wishes to use over untrusted/trusted non-3GPP access. It may be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "SliceAwareANDSP" feature is supported. |  | FeatureRenegotiation |  |
| satBackhaulCategory |  | SatelliteBackhaulCategory |  | C |  | 0..1 |  | Indicates types of the satellite backhaul based on satellite types (when satellite backhaul is used) or non-satellite backhaul (when satellite backhaul is not used). It shall be provided for trigger "SAT_CATEGORY_CHG". |  | EnSatBackhaulCategoryChg |  |
| urspEnfReport |  | map(UrspEnforcementPduSession) |  | O |  | 1..N |  | Represents information about the enforced URSP rule(s) in one or more PDU sessions for the affected UE. The key of the map is a character string that represents an integer value (it may correspond with a PDU session identifier). It shall be present when the notified policy control request trigger is "URSP_ENF_INFO". |  | URSPEnforcement |  |
| vpsUePolGuidance |  | map(UePolicyParameters) |  | O |  | 1..N |  | Contains the service parameter used to guide the VPLMN-specific URSP rule determination and may contain the subscription to VPLMN-specific URSP delivery outcome. The key of the map represents the AF request to guide VPLMN-specific URSP rules. This attribute only applies in roaming and when the V-PCF is the NF service consumer. |  | VPLMNSpecificURSP |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  | O |  | 1..N |  | Contains LBO roaming information for a DNN and S-NSSAI combination(s). This attribute only applies in roaming and when the AMF is the NF service consumer. |  | VPLMNSpecificURSP |  |
| accessTypes |  | array(AccessType) |  | C |  | 1..N |  | The Access Type(s) where the served UE is camping. It shall be provided for trigger "ACCESS_TYPE_CH" when the access type(s) changes or when the access type(s) is initially reported as consequence of the provisioning of the trigger. |  | AccessChange |  |
| ratTypes |  | array(RatType) |  | C |  | 1..N |  | The RAT Type(s), if available, for the reported "accessTypes" where the served UE is camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH" when the access type(s) changes or when the access type(s) is initially reported as consequence of the provisioning of the trigger. |  | AccessChange |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the features supported by the NF service consumer. It shall be included by the target AMF in inter-AMF mobility scenarios for trigger "FEAT_RENEG". |  | FeatureRenegotiation |  |
| rangSlCapab |  | array(RangSLCapability) |  | O |  | 1..N |  | Contains the Ranging/SL related UE capabilities. |  | Ranging_SL |  |
| NOTE: The "mappedHomeSnssai" attribute within the ConfiguredSnssai data type may only be provided if the "NssaiChange" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |

#### 5.6.2.5	Type PolicyUpdate

Table 5.6.2.5-1: Definition of type PolicyUpdate

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| resourceUri |  | Uri |  | M |  | 1 |  | The resource URI of the individual UE policy association related to the notification. (NOTE 2) |  |  |  |
| uePolicy |  | UePolicy |  | O |  | 0..1 |  | The UE policy as determined by the H-PCF. |  |  |  |
| n2Pc5Pol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for V2X communications as determined by the H-PCF. |  | V2X |  |
| n2Pc5PolA2x |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for A2X communications as determined by the H-PCF. |  | A2X |  |
| n2Pc5ProSePol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for 5G ProSe as determined by the PCF. |  | ProSe |  |
| triggers |  | array(RequestTrigger) |  | O |  | 1..N |  | Request Triggers that the PCF subscribes. (NOTE 1) |  |  |  |
| pras |  | map(PresenceInfoRm) |  | C |  | 1..N |  | If the Trigger "PRA_CH" is provided or if that trigger was already set but the requested presence reporting areas need to be changed, the presence reporting area(s) for which reporting is requested shall be provided. The "praId" attribute within the PresenceInfoRm data type shall also be the key of the map. The "presenceState" attribute within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of either a presence reporting area or a presence reporting area set. |  | PresenceInfo |  |
| andspDelInd |  | PolicyStatus |  | O |  | 0..1 |  | Information about whether the updated ANDSP/WLANSP has been successfully delivered to the UE. |  | SliceAwareANDSP |  |
| delivReport |  | map(UePolicyNotification) |  | O |  | 1..N |  | Contains the delivery outcome of VPLMN-Specific URSP rules. It may be included if the V-PCF indicated the subscription to delivery outcome events as described in clause 4.2.2.2.3.2. The key of the map represents the AF request of the corresponding subscription, i.e. its value shall match the key that was previously provided by the V-PCF in the “vpsUePolGuidance“ attribute. |  | VPLMNSpecificURSP |  |
| pduSessions |  | array(PduSessionInfo) |  | O |  | 1..N |  | Contains the list of the DNN and SNSSAI pairs for which LBO information is being requested. It may be provided when the "LBO_INFO_CH" request trigger is provided. |  | VPLMNSpecificURSP |  |
| pcfUeInfo |  | PcfUeCallbackInfo |  | O |  | 0..1 |  | Contains the PCF for the UE callback information necessary for the PCF for the PDU session to send established/terminated events notifications to the PCF for the UE. |  | URSPEnforcement |  |
| matchPdus |  | array(PduSessionInfo) |  | O |  | 1..N |  | Indicates the matched PDU session(s) for which the AMF shall forward the PCF for the UE information in the "pcfUeInfo" attribute to the SMF. It shall be present when the "pcfUeInfo" attribute is present. (NOTE 3) |  | URSPEnforcement |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the negotiated supported features. It shall be included in the HTTP POST response when the NF service consumer provided the supported features in the HTTP POST request. |  | FeatureRenegotiation |  |
| n2Pc5RsppPol |  | N2InfoContent |  | O |  | 0..1 |  | The N2 PC5 policy for Ranging/SL as determined by the H-PCF. |  | Ranging_SL |  |
| NOTE 1: Only the RequestTrigger enumeration values corresponding to PCRTs that require explicit subscription as defined in clause 5.6.3.3 shall be applicable within the "triggers" attribute. NOTE 2: When the PolicyUpdate data type is used in a policy update notify service operation, either the complete resource URI included in the "resourceUri" attribute or the "apiSpecificResourceUriPart" component (see clause 5.1) of the resource URI included in the "resourceUri" attribute may be used by the NF service consumer (e.g. AMF) for the identification of the Individual UE Policy Association resource related to the notification. NOTE 3: The DNN encoded within the PduSessionInfo element(s) of the "matchPdus" array may contain a full DNN or only the DNN Network Identifier. When the DNN contains the Network Identifier only, the AMF shall match a PDU session for the received Network Identifier and for any value of the Operator Identifier. |  |  |  |  |  |  |  |  |  |  |  |

#### 5.6.2.6	Type TerminationNotification

Table 5.6.2.6-1: Definition of type TerminationNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceUri | Uri | M | 1 | The resource URI of the individual UE policy association related to the notification. (NOTE) |  |
| cause | PolicyAssociationReleaseCause | M | 1 | The cause why the PCF requests the termination of the policy association. |  |
| NOTE: Either the complete resource URI included in the "resourceUri" attribute or the "apiSpecificResourceUriPart" component (see clause 5.1) of the resource URI included in the "resourceUri" attribute may be used by the NF service consumer (e.g. AMF) for the identification of the Individual UE Policy Association resource related to the termination notification. |  |  |  |  |  |

#### 5.6.2.7	Type UePolicyTransferFailureNotification

Table 5.6.2.7-1: UEPolicyTransferFailureNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cause | UePolicyTransferFailureCause | M | 1 | Indicates the reason why the UE policy could not be transferred by the AMF. When the feature "EnErrorHandling" is supported, the "cause" attribute may include the enumeration values defined in the "N1N2MessTransferErrorReply" data type. |  |
| retryAfter | Uinteger | O | 0..1 | The V-PCF may include this IE if the AMF requests to stop sending the N1N2MessageTransfer before timeout of the indicated time period. | EnErrorHandling |
| ptis | array(Uinteger) | M | 1..N | Contains a list of PTI assigned by the H-PCF corresponding to the UE policy(s) which could not be transferred by the AMF. |  |

#### 5.6.2.8	Type UeRequestedValueRep

Table 5.6.2.8-1: Definition of type UeRequestedValueRep

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| userLoc |  | UserLocation |  | C |  | 0..1 |  | The location of the served UE is camping shall be provided for trigger "LOC_CH". |  |  |  |
| praStatuses |  | map(PresenceInfo) |  | C |  | 1..N |  | The UE presence statuses for tracking areas shall be provided for trigger "PRA_CH". The "praId" attribute within the PresenceInfo data type shall also be the key of the map. |  |  |  |
| plmnId |  | PlmnIdNid |  | C |  | 0..1 |  | The serving network identity (a PLMN or an SNPN) of the served UE shall be provided for trigger "PLMN_CH". |  | PlmnChange |  |
| connectState |  | CmState |  | C |  | 0..1 |  | The connectivity state of the served UE. It shall be provided for trigger "CON_STATE_CH". |  | ConnectivityStateChange |  |
| satBackhaulCategory |  | SatelliteBackhaulCategory |  | C |  | 0..1 |  | Indicates types of the satellite backhaul based on satellite types (when satellite backhaul is used) or non-satellite backhaul (when satellite backhaul is not used). It shall be provided for trigger "SAT_CATEGORY_CHG". |  | EnSatBackhaulCategoryChg |  |
| urspEnfReport |  | map(UrspEnforcementPduSession) |  | C |  | 1..N |  | Represents information about the enforced URSP rule(s) in one or more PDU sessions for the affected UE. The key of the map is a character string that represents an integer value (it may correspond with a PDU session identifier). It shall be present when the notified policy control request trigger is "URSP_ENF_INFO". |  | URSPEnforcement |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  | C |  | 1..N |  | Contains a list of LBO roaming information for a DNN and S-NSSAI combination. It shall be provided for trigger "LBO_INFO_CH". |  | VPLMNSpecificURSP |  |
| confSnssais |  | array(ConfiguredSnssai) |  | C |  | 1..N |  | The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN. It shall be provided for trigger "CONF_NSSAI_CH". |  | NssaiChange |  |
| accessTypes |  | array(AccessType) |  | C |  | 1..N |  | The Access Type(s) where the served UE is camping. Shall be provided for trigger "ACCESS_TYPE_CH". |  | AccessChange |  |
| ratTypes |  | array(RatType) |  | C |  | 1..N |  | The RAT Type(s), if available, for the reported "accessTypes" where the served UE is camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH. |  | AccessChange |  |

#### 5.6.2.9	Type UePolicyParameters

Table 5.6.2.9-1: Definition of type UePolicyParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| urspGuidance | array(UrspRuleRequest) | O | 1..N | Contains the service parameter used to guide the VPLMN-specific URSP. |  |
| deliveryEvents | array(Event) | O | 1..N | Identifies the AF subscribed event(s) related to AF provisioned guidance for VPLMN-specific URSP rules. (NOTE) |  |
| NOTE: In this release of the specification, only the "SUCCESS_UE_POL_DEL_SP" and "UNSUCCESS_UE_POL_DEL_SP" events apply. |  |  |  |  |  |

#### 5.6.2.10	Type LboRoamingInformation

Table 5.6.2.10-1: Definition of type LboRoamingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lboRoamAllowed | boolean | O | 0..1 | Indicates whether local breakout for the DNN and S-NSSAI is allowed when roaming. true: allowed false: not allowed. If the attribute is absent it means not allowed. |  |
| dnn | Dnn | M | 1 | Data Network Name with Network Identifier only. |  |
| snssai | Snssai | M | 1 | S-NSSAI. |  |

#### 5.6.2.11	Type UrspEnforcementPduSession

Table 5.6.2.11-1: Definition of type UrspEnforcementPduSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| urspEnfInfo | UrspEnforcementInfo | M | 1 | Represents UE provided information about the enforced URSP rule(s) in one PDU session. |  |
| sscMode | SscMode | C | 0..1 | SSC Mode of the PDU session. It shall be provided when URSP rule enforcement information is provided for the first time. |  |
| ueReqDnn | Dnn | C | 0..1 | UE requested DNN. It shall be provided when URSP rule enforcement information is provided for the first time, if available and different from the selected DNN. |  |
| ueReqPduSessionType | PduSessionType | O | 0..1 | UE requested PDU session Type. It shall be provided when URSP rule enforcement information is provided for the first time. |  |
| dnn | Dnn | C | 0..1 | Selected DNN. It shall be provided when URSP rule enforcement information is provided for the first time. |  |
| snssai | Snssai | C | 0..1 | S-NSSAI of the HPLMN. It shall be provided when URSP rule enforcement information is provided for the first time. |  |

#### 5.6.2.12	Type UePolicyNotification

Table 5.6.2.12-1: Definition of type UePolicyNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventNotifs | array(PcEventNotification) | M | 1..N | Represents the events to be reported according to the subscription to delivery outcome events as described in clause 4.2.2.2.3.2. (NOTE) |  |
| NOTE: In this release of the specification, only the "SUCCESS_UE_POL_DEL_SP" and "UNSUCCESS_UE_POL_DEL_SP" events apply for the "events" attribute within the PcEventNotification data type. |  |  |  |  |  |

### 5.6.3	Simple data types and enumerations

#### 5.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

#### 5.6.3.2	Simple data types

The simple data types defined in table 5.6.3.2-1 shall be supported.

Table 5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| UePolicy | Bytes | "MANAGE UE POLICY COMMAND" message content, as defined in Table D.5.1.1.1 of 3GPP TS 24.501 [15] |  |
| UePolicyDeliveryResult | Bytes | "MANAGE UE POLICY COMPLETE" message content, as defined in Table D.5.2.1.1 of 3GPP TS 24.501 [15], or "MANAGE UE POLICY COMMAND REJECT" message content, as defined in Table D.5.3.1.1 of 3GPP TS 24.501 [15] |  |
| UePolicyRequest | Bytes | "UE STATE INDICATION" message content, as defined in Table D.5.4.1.1 of 3GPP TS 24.501 [15] or "UE POLICY PROVISIONING REQUEST" message content, as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24]. |  |

#### 5.6.3.3	Enumeration: RequestTrigger

The enumeration RequestTrigger represents the possible Policy Control Request Triggers. It shall comply with the provisions defined in table 5.6.3.3-1.

Table 5.6.3.3-1: Enumeration RequestTrigger

| Enumeration value |  |  | Description |  |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LOC_CH |  |  | Location change (tracking area): the tracking area of the UE has changed. (NOTE) |  |  |  |  |  |
| PRA_CH |  |  | Change of UE presence in PRA: the AMF reports the current presence status of the UE in a Presence Reporting Area, and notifies that the UE enters/leaves the Presence Reporting Area. (NOTE) |  |  |  |  |  |
| UE_POLICY |  |  | A "MANAGE UE POLICY COMPLETE" message, a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] has been received by the V-PCF and is being forwarded to the H-PCF, or has been received by a PCF for a PDU session and is being forwarded to the (V-)PCF (and then from the V-PCF to the H-PCF) when the "EpsUrsp" feature is supported. A Namf_Communication_N1N2MessageTransfer failure response as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14], an N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14], a UE Policy transfer failure is notifying to the H-PCF, or a UE Policy transfer failure is notifying to the (V-)PCF when the "EpsUrsp" feature is supported. When the "ProSe" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in clause 10.4 of 3GPP TS 24.554 [28] has been received by the V-PCF and is being forwarded to the H-PCF. When the "V2X" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2 of 3GPP TS 24.587 [24] has been received by the V-PCF and is being forwarded to the H-PCF. When the "A2X" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in 3GPP TS 24.577 [32] has been received by the V-PCF and is being forwarded to the H-PCF. When the "Ranging_SL" feature is supported it indicates that a "UE POLICY PROVISIONING REQUEST" message, as defined in 3GPP TS 24.514 [42] has been received by the V-PCF and is being forwarded to the H-PCF. This event does not require a subscription and is only applicable for the V-PCF as NF service consumer and the H-PCF as NF service producer or a PCF for a PDU session as NF service consumer and the (V-)PCF as NF service producer when the “EpsUrsp” feature is supported. |  |  |  |  |  |
| PLMN_CH |  |  | PLMN change: the serving network (a PLMN or an SNPN) of UE has changed. (NOTE) |  |  | PlmnChange |  |  |
| CON_STATE_CH |  |  | Connectivity state change: the connectivity state of UE has changed. (NOTE) |  |  | ConnectivityStateChange |  |  |
| GROUP_ID_LIST_CHG |  |  | UE Internal Group Identifier(s) has changed: the AMF reports that UDM provided list of group Ids has changed. This policy control request trigger does not require a subscription. |  |  | GroupIdListChange |  |  |
| UE_CAP_CH |  |  | UE Capabilities change: the UE provided 5G ProSe capabilities have changed. This policy control request trigger does not require subscription. |  |  | ProSe |  |  |
| SAT_CATEGORY_CHG |  |  | Satellite Backhaul Category change: the AMF has detected a change between different satellite backhaul category, or a change between satellite and non-satellite backhaul. (NOTE) |  |  | EnSatBackhaulCategoryChg |  |  |
| CONF_NSSAI_CH |  |  | Configured NSSAI change: the configured NSSAI has changed. This policy control request trigger only applies in roaming scenarios when the NF service consumer is the AMF. (NOTE) |  |  | NssaiChange |  |  |
| NON_3GPP_NODE_RESELECTION |  |  | Wrong TNGF or N3IWF: the UE has connected to a wrong non-3GPP access node that does not match its subscribed S-NSSAI(s). This policy control request trigger does not require a subscription. |  |  | SliceAwareANDSP |  |  |
| FEAT_RENEG |  |  | The target AMF determines feature re-negotiation is required. This policy control request trigger does not require subscription. |  |  | FeatureRenegotiation |  |  |
| URSP_ENF_INFO |  |  | The V-PCF has received URSP rule enforcement information about the enforced URSP rule(s) in one or more PDU sessions. This trigger only applies in roaming scenarios and to the V-PCF. (NOTE) |  |  | URSPEnforcement |  |  |
| LBO_INFO_CH |  |  | LBO information change: The AMF reports LBO roaming allowed or not allowed for the requested DNN(s) and S-NSSAI(s). This policy control request trigger only applies in roaming scenarios when the NF service consumer is the AMF. (NOTE) |  |  | VPLMNSpecificURSP |  |  |
| ACCESS_TYPE_CH |  |  | Access Type change: The registered access type and RAT type has changed, an access type and RAT type is added or removed. (NOTE) |  |  | AccessChange |  |  |
| NOTE: The report of this trigger includes reporting the current value at the time the trigger is provisioned during the update or update notification of the policy association. |  |  |  |  |  |  |  |  |

#### 5.6.3.4	Enumeration: PolicyAssociationReleaseCause

The enumeration PolicyAssociationReleaseCause represents the cause why the PCF requests the termination of the policy association. It shall comply with the provisions defined in table 5.6.3.4-1.

Table 5.6.3.4-1: Enumeration PolicyAssociationReleaseCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNSPECIFIED | This value is used for unspecified reasons. |  |
| UE_SUBSCRIPTION | This value is used to indicate that the policy association needs to be terminated because the subscription of UE has changed (e.g. was removed). |  |
| INSUFFICIENT_RES | This value is used to indicate that the server is overloaded and needs to abort the policy association. |  |

#### 5.6.3.5	Enumeration: Pc5Capability

The enumeration Pc5Capability indicates the specific PC5 RAT(s) which the UE supports for V2X communication over PC5 reference point. It shall comply with the provisions defined in table 5.6.3.5-1.

Table 5.6.3.5-1: Enumeration Pc5Capability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LTE_PC5 | This value is used to indicate that the UE supports PC5 LTE RAT for V2X communication over PC5 reference point. |  |
| NR_PC5 | This value is used to indicate that the UE supports PC5 LTE RAT for V2X communication over PC5 reference point. |  |
| LTE_NR_PC5 | This value is used to indicate that the UE supports both PC5 LTE and NR RAT for V2X communication over PC5 reference point. |  |

#### 5.6.3.6	Enumeration: ProSeCapability

This enumeration indicates the 5G ProSe capabilities. It shall comply with the provisions defined in table 5.6.3.6-1.

Table 5.6.3.6-1: Enumeration ProSeCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PROSE_DD | This value is used to indicate that 5G ProSe Direct Discovery is supported by the UE. |  |
| PROSE_DC | This value is used to indicate that 5G ProSe Direct Communication is supported by the UE. |  |
| PROSE_L2_U2N_RELAY | This value is used to indicate that Layer-2 5G ProSe UE-to-Network Relay is supported by the UE. |  |
| PROSE_L3_U2N_RELAY | This value is used to indicate that Layer-3 5G ProSe UE-to-Network Relay is supported by the UE. |  |
| PROSE_L2_REMOTE_UE | This value is used to indicate that Layer-2 5G ProSe Remote UE is supported by the UE. |  |
| PROSE_L3_REMOTE_UE | This value is used to indicate that Layer-3 5G ProSe Remote UE is supported by the UE. |  |
| PROSE_L2_U2U_RELAY | This value is used to indicate that Layer-2 5G ProSe UE-to-UE Relay is supported by the UE. | ProSe_Ph2 |
| PROSE_L3_U2U_RELAY | This value is used to indicate that Layer-3 5G ProSe UE-to-UE Relay is supported by the UE. | ProSe_Ph2 |
| PROSE_L2_END_UE | This value is used to indicate that Layer-2 5G ProSe End UE is supported by the UE. | ProSe_Ph2 |
| PROSE_L3_END_UE | This value is used to indicate that Layer-3 5G ProSe End UE is supported by the UE. | ProSe_Ph2 |

#### 5.6.3.7	Enumeration: Non3gppAccess

The enumeration Non3gppAccess represents the possible Non-3gpp access nodes. It shall comply with the provisions defined in table 5.6.3.7-1.

Table 5.6.3.7-1: Non3gppAccess

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| N3IWF | Non-3gpp Interworking Function |  |
| TNGF | Trusted Non-3GPP Gateway Function |  |

#### 5.6.3.8	Void

#### 5.6.3.9	Enumeration: N1N2MessTransferErrorReply

The enumeration N1N2MessTransferErrorReply represents the possible errors the V-PCF may send to the H-PCF when the V-PCF receives from the AMF an error reply to the N1N2MessageTransfer request. It shall comply with the provisions defined table 5.6.3.9-1.

Table 5.6.3.9-1: N1N2MessTransferErrorReply

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_NOT_REACHABLE | The UE is not reachable for paging. |  |
| UNSPECIFIED | Unspecified error. |  |

#### 5.6.3.10	Enumeration: RangSLCapability

The enumeration RangSLCapability represents the Ranging and Sidelink Capability. It shall comply with the provisions defined in table 5.6.3.10-1.

Table 5.6.3.10-1: Enumeration RangSLCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PC5_RANGING_SL | Indicates that the PC5 Capability for Ranging and Sidelink is supported by the UE. |  |

#### 5.6.3.11	Enumeration: PolicyStatus

The enumeration PolicyStatus represents the configuration status of a UE Policy in the UE. It shall comply with the provisions defined in table 5.6.3.11-1.

Table 5.6.3.11-1: PolicyStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CONFIGURED | The UE Policy is configured in the UE. |  |
| NOT_CONFIGURED | The UE Policy is not configured in the UE. |  |

#### 5.6.3.12	Enumeration: A2xCapability

The enumeration A2xCapability indicates the A2X capabilities the UE supports for A2X communication. It shall comply with the provisions defined in table 5.6.3.12-1.

Table 5.6.3.12-1: Enumeration A2xCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EUTRA_PC5 | This value is used to indicate that the UE supports PC5 E-UTRA RAT for A2X communication over PC5 reference point. |  |
| NR_PC5 | This value is used to indicate that the UE supports PC5 NR RAT for A2X communication over PC5 reference point. |  |
| UU | This value is used to indicate that the UE supports A2X communication over Uu reference point. |  |

#### 5.6.3.13	Enumeration: SliceSpecificN3gNodeSelectionCapability

Table 5.6.3.13-1: SliceSpecificN3gNodeSelectionCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ONLY_N3IWF_SS_SEL | Indicates that the UE supports N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access. |  |
| ONLY_TNGF_SS_SEL | Indicates that the UE supports TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access. |  |
| TNGF_N3IWF_SS_SEL | Indicates that the UE supports N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access and TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access. |  |

### 5.6.4	Data types describing alternative data types or combinations of data types

#### 5.6.4.1	Type: UePolicyTransferFailureCause

Table 5.6.4.1-1: Definition of type UePolicyTransferFailureCause as a list of non-exclusive alternatives

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| N1N2MessageTransferCause | 0..1 | The failure causes notified by the AMF within the N1 Message Transfer Failure notification. |  |
| N1N2MessTransferErrorReply | 0..1 | Error reply the AMF may indicate within the response to N1N2MessageTransfer request. |  |

## 5.7	Error handling

### 5.7.1	General

For the Npcf_UEPolicyControl API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [6].

Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

In addition, the requirements in the following clauses are applicable for the Npcf_UEPolicyControl API.

### 5.7.2	Protocol Errors

No specific protocol errorsfor the Npcf_UEPolicyControl API are specified.

### 5.7.3	Application Errors

The application errors defined for the Npcf_UEPolicyControl service are listed in Table 5.7.3-1 and Table 5.7.3-2.

Table 5.7.3-1: Application errors when PCF acts as a server

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_UNKNOWN | 400 Bad Request | The HTTP request is rejected because the end user specified in the request is unknown to the PCF. |
| ERROR_REQUEST_PARAMETERS | 400 Bad Request | The HTTP request is rejected because the set of information needed by the PCF for UE Policy selection is incomplete or erroneous or not available for the decision to be made. |
| PENDING_TRANSACTION | 400 Bad Request | This error shall be used when the PendingTransaction feature is supported and the PCF receives an incoming request on a policy association while it has an ongoing transaction on the same policy association and cannot handle the request as described in clause 9.2 of 3GPP TS 29.513 [7]. |
| POLICY_ASSOCIATION_NOT_FOUND | 404 Not Found | The HTTP request is rejected because no UE policy association corresponding to the request exists in the PCF. |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |

Table 5.7.3-2: Application errors when NF service consumer acts as a server to receive a notification

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| PENDING_TRANSACTION | 400 Bad Request | This error shall be used when the PendingTransaction feature is supported and the NF service consumer receives an incoming request on a policy association while it has an ongoing transaction on the same policy association and cannot handle the request as described in clause 9.2 of 3GPP TS 29.513 [7]. (NOTE 1) |
| NOTE 1: This application error is included in the response to the Policy Update Notification HTTP POST request. NOTE 2: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Npcf_UEPolicyControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number |  |  | Feature Name |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | PendingTransaction |  |  | This feature indicates support for the race condition handling as defined in 3GPP TS 29.513 [7]. |  |  |
| 2 |  |  | PlmnChange |  |  | This feature indicates support for the change of PLMN trigger handling. |  |  |
| 3 |  |  | ConnectivityStateChange |  |  | This feature indicates support for the UE connectivity state change trigger handling. |  |  |
| 4 |  |  | V2X |  |  | This feature indicates support for the UE policy provisioning and N2 information provisioning for V2X communications. |  |  |
| 5 |  |  | GroupIdListChange |  |  | This feature indicates the support for the notification of changes in the list of internal group identifiers. |  |  |
| 6 |  |  | ImmediateReport |  |  | This feature indicates the support of the current applicable values report corresponding to the policy control request triggers for policy update notification. |  |  |
| 7 |  |  | ErrorResponse |  |  | This feature indicates support for "404 Not Found" error response code for policy update notification between AMF and (V-)PCF. |  |  |
| 8 |  |  | ES3XX |  |  | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |  |  |
| 9 |  |  | ProSe |  |  | This feature indicates support of UE policy and N2 information provisioning for 5G ProSe. |  |  |
| 10 |  |  | FeatureRenegotiation |  |  | This feature indicates the support of feature renegotiation during the update of a policy association triggered by UE mobility with AMF change. |  |  |
| 11 |  |  | SliceAwareANDSP |  |  | This feature indicates the support of ANDSP/WLANSP policies that consider the slices supported by the UE. |  |  |
| 12 |  |  | EpsUrsp |  |  | This feature indicates support of URSP provisioning in EPS and is only applicable in the case of of 5GC and EPC interworking. |  |  |
| 13 |  |  | EnSatBackhaulCategoryChg |  |  | This feature indicates the support of notification of a change between different satellite backhaul categories, or dynamic satellite backhaul categories, or between satellite backhaul and non-satellite backhaul. |  |  |
| 14 |  |  | UECapabilityIndication |  |  | This feature indicates the support of the provisioning by the H-PCF to the V-PCF of the UE Capability for UE Policy, when the UE Capability is not received from the UE and the information is available and reliable in the UDR. |  |  |
| 15 |  |  | A2X |  |  | This feature indicates support of A2X communications. |  |  |
| 16 |  |  | NssaiChange |  |  | This feature indicates support for the change of Configured NSSAI trigger handling. |  |  |
| 17 |  |  | ProSe_Ph2 |  |  | This feature indicates the support of UE policy and N2 information provisioning for 5G ProSe UE-to-UE Relay function. This feature requires that the ProSe feature is also supported. |  |  |
| 18 |  |  | PresenceInfo |  |  | The feature indicates the support of policy update to remove the existing presence reporting areas entry. |  |  |
| 19 |  |  | URSPEnforcement |  |  | This feature indicates the support of the report of URSP rule enforcement information by the V-PCF to the H-PCF. |  |  |
| 20 |  |  | VPLMNSpecificURSP |  |  | This feature indicates the support of AF guidance on VPLMN-specific URSP rules. It requires the support of NssaiChange feature. |  |  |
| 21 |  |  | Ranging_SL |  |  | This feature indicates the support of the ranging and sidelink positioning functionality. The following functionalities are supported: - Support for the UE policy provisioning and N2 information provisioning for Ranging and sidelink positioning. |  |  |
| 22 |  |  | AccessChange |  |  | This feature indicates the support of the reporting of an access type and RAT type changes, the addition of an access type and RAT type or the removal of an existing access type and RAT type. |  |  |
| 23 |  |  | EnErrorHandling |  |  | This feature indicates the support of the indication from the V-PCF to the H-PCF of the received AMF error response to the UE Policy Delivery transfer request. |  |  |
| 24 |  |  | SLAMUP |  |  | This feature indicates the support of the provisioning to the AMF of the CHF information of the CHF selected by the PCF for UE policy. |  |  |
| 25 |  |  | EnhEstRoaming |  |  | The feature enhanced UE Policy Establishment procedure in roaming scenarios indicates the provisioning of the AMF to the V-PCF of the H-PCF Set Id and H-PCF URI of the selected H-PCF ID. |  |  |

## 5.9	Security

As indicated in 3GPP TS 33.501 [19] and 3GPP TS 29.500 [5], the access to the Npcf_UEPolicyControl API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [20]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF service consumer, prior to consuming services offered by the Npcf_UEPolicyControl API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF service consumer used for discovering the Npcf_UEPolicyControl service.

The Npcf_UEPolicyControl API defines a single scope "npcf-ue-policy-control" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

The present Annex contains an OpenAPI [10] specification of HTTP messages and content bodies used by the Npcf_UEPolicyControl API.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [22] and clause 5.3.1 of the 3GPP TS 29.501 [6] for further information).

# A.2	Npcf_UEPolicyControl API

openapi: 3.0.0

info:

version: 1.3.1

title: Npcf_UEPolicyControl

description: |

UE Policy Control Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.525 V18.7.0; 5G System; UE Policy Control Service.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.525/'

servers:

- url: '{apiRoot}/npcf-ue-policy-control/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npcf-ue-policy-control

paths:

/policies:

post:

operationId: CreateIndividualUEPolicyAssociation

summary: Create individual UE policy association.

tags:

- UE Policy Associations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociationRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociation'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

policyUpdateNotification:

'{$request.body#/notificationUri}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyUpdate'

responses:

'200':

description: >

OK. The current applicable values corresponding to the policy control request

trigger is reported

content:

application/json:

schema:

$ref: '#/components/schemas/UeRequestedValueRep'

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

policyAssocitionTerminationRequestNotification:

'{$request.body#/notificationUri}/terminate':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TerminationNotification'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policies/{polAssoId}:

get:

operationId: ReadIndividualUEPolicyAssociation

summary: Read individual UE policy association.

tags:

- Individual UE Policy Association (Document)

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualUEPolicyAssociation

summary: Delete individual UE policy association.

tags:

- Individual UE Policy Association (Document)

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policies/{polAssoId}/update:

post:

operationId: ReportObservedEventTriggersForIndividualUEPolicyAssociation

summary: >

Report observed event triggers and possibly obtain updated policies for an individual UE

policy association.

tags:

- Individual UE Policy Association (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociationUpdateRequest'

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'200':

description: OK. Updated policies are returned

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyUpdate'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-ue-policy-control: Access to the Npcf_UEPolicyControl API

schemas:

PolicyAssociation:

description: >

Contains the description of a policy association that is returned by the PCF when a policy

Association is created, updated, or read.

type: object

properties:

request:

$ref: '#/components/schemas/PolicyAssociationRequest'

uePolicy:

$ref: '#/components/schemas/UePolicy'

n2Pc5Pol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5PolA2x:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5ProSePol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

description: >

Request Triggers that the PCF subscribes.

pras:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfoRm'

minProperties: 1

description: >

Contains the presence reporting area(s) for which reporting was requested.

The praId attribute within the PresenceInfoRm data type is the key of the map.

andspDelInd:

$ref: '#/components/schemas/PolicyStatus'

andspInd:

description: >

Indication of UE support of ANDSP. When set to true, it indicates the UE supports ANDSP,

when set to false it indicates the UE does not support ANDSP.

type: boolean

pduSessions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

description: Combination of DNN and S-NSSAIs for which LBO information is requested.

chfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

n2Pc5RsppPol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

pcfUeInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

matchPdus:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

required:

- suppFeat

PolicyAssociationRequest:

description: >

Represents information that the NF service consumer provides when requesting the creation of

a policy association.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: Alternate or backup IPv4 Address(es) where to send Notifications.

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: Alternate or backup IPv6 Address(es) where to send Notifications.

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

accessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

description: >

The Access Type(s) where the served UE is camping.

It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

servingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ratTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

description: >

The RAT Type(s), if available, for the reported "accessTypes" where the served UE is

camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

groupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

hPcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

hPcfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hPcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

uePolReq:

$ref: '#/components/schemas/UePolicyRequest'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pc5Capab:

$ref: '#/components/schemas/Pc5Capability'

a2xCapab:

type: array

items:

$ref: '#/components/schemas/A2xCapability'

minItems: 1

proSeCapab:

type: array

items:

$ref: '#/components/schemas/ProSeCapability'

minItems: 1

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

description: >

The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home

network corresponding to the configured S-NSSAI in the serving PLMN.

n3gNodeReSel:

$ref: '#/components/schemas/Non3gppAccess'

sliceN3gNodeSelCap:

$ref: '#/components/schemas/SliceSpecificN3gNodeSelectionCapability'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

5gsToEpsMob:

type: boolean

description: >

It indicates the UE Policy Association is triggered by a 5GS to EPS mobility

scenario.

vpsUePolGuidance:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyParameters'

minProperties: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP and may contain

the subscription to VPLMN-specific URSP delivery outcome.

The key of the map represents the AF request to guide VPLMN-specific URSP rules.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

This attribute only applies in roaming and when the AMF is the NF service consumer.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

rangSlCapab:

type: array

items:

$ref: '#/components/schemas/RangSLCapability'

minItems: 1

required:

- notificationUri

- suppFeat

- supi

PolicyAssociationUpdateRequest:

description: >

Represents Information that the NF service consumer provides when requesting the update of

a policy association.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: Alternate or backup IPv4 Address(es) where to send Notifications.

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: Alternate or backup IPv6 Address(es) where to send Notifications.

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

description: Request Triggers that the NF service consumer observes.

praStatuses:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

description: >

Contains the UE presence status for tracking area for which changes of the UE presence

occurred. The praId attribute within the PresenceInfo data type is the key of the map.

minProperties: 1

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uePolDelResult:

$ref: '#/components/schemas/UePolicyDeliveryResult'

uePolTransFailNotif:

$ref: '#/components/schemas/UePolicyTransferFailureNotification'

uePolReq:

$ref: '#/components/schemas/UePolicyRequest'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

connectState:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

groupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

pc5Capab:

$ref: '#/components/schemas/Pc5Capability'

a2xCapab:

type: array

items:

$ref: '#/components/schemas/A2xCapability'

minItems: 1

proSeCapab:

type: array

items:

$ref: '#/components/schemas/ProSeCapability'

minItems: 1

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

description: >

The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home

network corresponding to the configured S-NSSAI in the serving PLMN.

n3gNodeReSel:

$ref: '#/components/schemas/Non3gppAccess'

sliceN3gNodeSelCap:

$ref: '#/components/schemas/SliceSpecificN3gNodeSelectionCapability'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

urspEnfRep:

type: object

additionalProperties:

$ref: '#/components/schemas/UrspEnforcementPduSession'

description: >

Contains information about the enforced URSP rule(s) in one or more PDU sessions.

The key of the map is a character string that represents an integer value.

minProperties: 1

vpsUePolGuidance:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyParameters'

minProperties: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP and may contain

the subscription to VPLMN-specific URSP delivery outcome.

The key of the map represents the AF request to guide VPLMN-specific URSP rules.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

nullable: true

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

This attribute only applies in roaming and when the AMF is the NF service consumer.

accessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

description: >

The Access Type(s) where the served UE is camping.

It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

ratTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

description: >

The RAT Type(s), if available, for the reported "accessTypes" where the served UE is

camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

rangSlCapab:

type: array

items:

$ref: '#/components/schemas/RangSLCapability'

minItems: 1

description: >

Contains the Ranging/SL related UE capabilities.

PolicyUpdate:

description: >

Represents updated policies that the PCF provides in a notification or in the reply to an

Update Request.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

uePolicy:

$ref: '#/components/schemas/UePolicy'

n2Pc5Pol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5PolA2x:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5ProSePol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

nullable: true

description: >

Request Triggers that the PCF subscribes.

pras:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

description: >

Contains the presence reporting area(s) for which reporting was requested.

The praId attribute within the PresenceInfo data type is the key of the map.

minProperties: 1

nullable: true

andspDelInd:

$ref: '#/components/schemas/PolicyStatus'

delivReport:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyNotification'

minProperties: 1

description: >

Contains the delivery outcome of the VPLMN-specific URSP.

The key of the map represents the AF request of the corresponding subscription, i.e. its

value shall match the key that was previously provided by the V-PCF in the

vpsUePolGuidance attribute.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

pduSessions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

description: >

Combination of DNN and S-NSSAIs for which LBO information is requested.

nullable: true

pcfUeInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

matchPdus:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

nullable: true

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

n2Pc5RsppPol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

required:

- resourceUri

TerminationNotification:

description: >

Represents a request to terminate a policy association that the PCF provides in a

notification.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

cause:

$ref: '#/components/schemas/PolicyAssociationReleaseCause'

required:

- resourceUri

- cause

UePolicyTransferFailureNotification:

description: >

Represents information on the failure of a UE policy transfer to the UE because the UE is

not reachable.

type: object

properties:

cause:

$ref: '#/components/schemas/UePolicyTransferFailureCause'

retryAfter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

ptis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

description: >

This contains a list of PTI assigned by the H-PCF corresponding to the UE policy(s)

which could not be transferred by the AMF.

required:

- cause

- ptis

UeRequestedValueRep:

description: >

Contains the current applicable values corresponding to the policy control request triggers.

type: object

properties:

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

praStatuses:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains the UE presence statuses for tracking areas. The praId attribute within the

PresenceInfo data type is the key of the map.

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

connectState:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

description: >

The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home

network corresponding to the configured S-NSSAI in the serving PLMN.

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

urspEnfRep:

type: object

additionalProperties:

$ref: '#/components/schemas/UrspEnforcementPduSession'

description: >

Contains information about the enforced URSP rule(s) in one or more PDU sessions.

The key of the map is a character string that represents an integer value.

minProperties: 1

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

accessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

description: >

The Access Type(s) where the served UE is camping.

It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

ratTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

description: >

The RAT Type(s), if available, for the reported "accessTypes" where the served UE is

camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

UePolicyParameters:

description: >

Contains the service parameters used to guide the VPLMN-specific URSP rule determination.

type: object

properties:

urspGuidance:

type: array

items:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/UrspRuleRequest'

minItems: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP.

deliveryEvents:

type: array

items:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/Event'

minItems: 1

description: >

AF subscribed event(s) notifications related to AF provisioned guidance

for VPLMN-specific URSP rules.

nullable: true

nullable: true

LboRoamingInformation:

description: >

Contains LBO roaming information for a DNN and S-NSSAI.

type: object

properties:

lboRoamAllowed:

type: boolean

description: >

Indicates whether LBO for the DNN and S-NSSAI is allowed when roaming.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- dnn

- snssai

UrspEnforcementPduSession:

description: >

Represents URSP rule enforcement information for a PDU session.

type: object

required:

- urspEnfInfo

properties:

urspEnfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/UrspEnforcementInfo'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

ueReqDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

ueReqPduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

UePolicyNotification:

description: >

Contains the delivery outcome of VPLMN-specific URSP rules.

type: object

required:

- eventNotifs

properties:

eventNotifs:

type: array

items:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/PcEventNotification'

minItems: 1

description: >

Represents the events to be reported according to the subscription to notifications

of VPLMN-specific URSP delivery outcome events.

UePolicy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

UePolicyDeliveryResult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

UePolicyRequest:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

RequestTrigger:

anyOf:

- type: string

enum:

- LOC_CH

- PRA_CH

- UE_POLICY

- PLMN_CH

- CON_STATE_CH

- GROUP_ID_LIST_CHG

- UE_CAP_CH

- SAT_CATEGORY_CHG

- NON_3GPP_NODE_RESELECTION

- CONF_NSSAI_CH

- LBO_INFO_CH

- FEAT_RENEG

- URSP_ENF_INFO

- ACCESS_TYPE_CH

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the possible request triggers.

Possible values are:

- LOC_CH: Location change (tracking area). The tracking area of the UE has changed.

- PRA_CH: Change of UE presence in PRA. The AMF reports the current presence status

of the UE in a Presence Reporting Area, and notifies that the UE enters/leaves the

Presence Reporting Area.

- UE_POLICY: A MANAGE UE POLICY COMPLETE message or a MANAGE UE POLICY COMMAND REJECT

message, as defined in Annex D.5 of 3GPP TS 24.501 or a "UE POLICY PROVISIONING REQUEST"

message, as defined in clause 7.2.1.1 of 3GPP TS 24.587, has been received by the AMF

and is being forwarded.

- PLMN_CH: PLMN change. the serving PLMN of UE has changed.

- CON_STATE_CH: Connectivity state change: the connectivity state of UE has changed.

- GROUP_ID_LIST_CHG: UE Internal Group Identifier(s) has changed. This policy

control request

trigger does not require a subscription.

- UE_CAP_CH: UE Capabilities change: the UE provided 5G ProSe capabilities have changed.

This policy control request trigger does not require subscription.

- SAT_CATEGORY_CHG: Indicates that the AMF has detected a change between different satellite

category, or non-satellite backhaul.

- NON_3GPP_NODE_RESELECTION: The UE has connected to a wrong non-3GPP access node that

does not match its subscribed S-NSSAI(s). This policy control request trigger does not

require a subscription.

- CONF_NSSAI_CH: Configured NSSAI change. Indicates that the configured NSSAI has changed.

- LBO_INFO_CH: LBO information change. The AMF reports LBO roaming allowed or not allowed

for the requested DNN(s) and S-NSSAI(s). This policy control request trigger only applies

in roaming scenarios when the NF service consumer is the AMF.

- FEAT_RENEG: The NF service consumer notifies that the target AMF is requesting feature

re-negotiation.

- URSP_ENF_INFO: The V-PCF has received URSP rule enforcement information about the enforced

URSP rule(s) in one or more PDU sessions. This trigger applies in roaming scenarios and

to the V-PCF.

- ACCESS_TYPE_CH: Access Type change. The registered access type and RAT type

has changed, an access type and RAT type is added or removed.

PolicyAssociationReleaseCause:

anyOf:

- type: string

enum:

- UNSPECIFIED

- UE_SUBSCRIPTION

- INSUFFICIENT_RES

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the cause why the PCF requests the policy association termination.

Possible values are:

- UNSPECIFIED: This value is used for unspecified reasons.

- UE_SUBSCRIPTION: This value is used to indicate that the policy association needs to be

terminated because the subscription of UE has changed (e.g. was removed).

- INSUFFICIENT_RES: This value is used to indicate that the server is overloaded and needs

to abort the policy association.

Pc5Capability:

anyOf:

- type: string

enum:

- LTE_PC5

- NR_PC5

- LTE_NR_PC5

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the specific PC5 RAT(s) which the UE supports for V2X communications over

PC5 reference point.

Possible values are:

- LTE_PC5: This value is used to indicate that UE supports PC5 LTE RAT for V2X

communications over the PC5 reference point

- NR_PC5: This value is used to indicate that UE supports PC5 NR RAT for V2X communications

over the PC5 reference point.

- LTE_NR_PC5: This value is used to indicate that UE supports both PC5 LTE and NR RAT for

V2X communications over the PC5 reference point.

ProSeCapability:

anyOf:

- type: string

enum:

- PROSE_DD

- PROSE_DC

- PROSE_L2_U2N_RELAY

- PROSE_L3_U2N_RELAY

- PROSE_L2_REMOTE_UE

- PROSE_L3_REMOTE_UE

- PROSE_L2_U2U_RELAY

- PROSE_L3_U2U_RELAY

- PROSE_L2_END_UE

- PROSE_L3_END_UE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

the content defined in the present version of this API.

description: |

Represents the 5G ProSe capabilities.

Possible values are:

- PROSE_DD: This value is used to indicate that 5G ProSe Direct Discovery is supported

by the UE.

- PROSE_DC: This value is used to indicate that 5G ProSe Direct Communication is supported

by the UE.

- PROSE_L2_U2N_RELAY: This value is used to indicate that Layer-2 5G ProSe UE-to-Network

Relay is supported by the UE.

- PROSE_L3_U2N_RELAY: This value is used to indicate that Layer-3 5G ProSe UE-to-Network

Relay is supported by the UE.

- PROSE_L2_REMOTE_UE: This value is used to indicate that Layer-2 5G ProSe Remote UE is

supported by the UE.

- PROSE_L3_REMOTE_UE: This value is used to indicate that Layer-3 5G ProSe Remote UE is

supported by the UE.

- PROSE_L2_U2U_RELAY: This value is used to indicate that Layer-2 5G ProSe UE-to-UE

Relay is supported by the UE.

- PROSE_L3_U2U_RELAY: This value is used to indicate that Layer-3 5G ProSe UE-to-UE

Relay is supported by the UE.

- PROSE_L2_END_UE: This value is used to indicate that Layer-2 5G ProSe End UE is

supported by the UE.

- PROSE_L3_END_UE: This value is used to indicate that Layer-3 5G ProSe End UE is

supported by the UE.

Non3gppAccess:

anyOf:

- type: string

enum:

- N3IWF

- TNGF

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents a non-3gpp access node.

Possible values are:

- N3IWF: Non-3gpp Interworking Function.

- TNGF: Trusted Non-3gpp Gateway Function.

N1N2MessTransferErrorReply:

anyOf:

- type: string

enum:

- UE_NOT_REACHABLE

- UNSPECIFIED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents an N1N2 Message Transfer error.

Possible values are:

- UE_NOT_REACHABLE: The UE is not reachable for paging.

- UNSPECIFIED: Unspecified error.

RangSLCapability:

anyOf:

- type: string

enum:

- PC5_RANGING_SL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Indicates the Ranging and Sidelink Capability.

Possible values are:

- PC5_RANGING_SL: Indicates that the PC5 Capability for Ranging and Sidelink is supported

by the UE.

PolicyStatus:

anyOf:

- type: string

enum:

- CONFIGURED

- NOT_CONFIGURED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the configuration status of a UE Policy in the UE.

Possible values are:

- CONFIGURED: The UE Policy is configured in the UE.

- NOT_CONFIGURED: The UE Policy is not configured in the UE.

A2xCapability:

anyOf:

- type: string

enum:

- EUTRA_PC5

- NR_PC5

- UU

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the A2X capabilities the UE supports for A2X communication.

Possible values are:

- EUTRA_PC5: This value is used to indicate that the UE supports PC5 EUTRA RAT for A2X

communications over the PC5 reference point

- NR_PC5: This value is used to indicate that the UE supports PC5 NR RAT for A2X

communications over the PC5 reference point.

- UU: This value is used to indicate that UE supports A2X communications over the PC5

reference point.

SliceSpecificN3gNodeSelectionCapability:

anyOf:

- type: string

enum:

- ONLY_N3IWF_SS_SEL

- ONLY_TNGF_SS_SEL

- TNGF_N3IWF_SS_SEL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the UE capabilities with regard to slice-specific non-3gpp node selection.

Possible values are:

- ONLY_N3IWF_SS_SEL: Indicates that the UE supports N3IWF selection based on the slices

the UE wishes to use over untrusted non-3GPP access.

- ONLY_TNGF_SS_SEL: Indicates that the UE supports TNGF selection based on the slices

the UE wishes to use over trusted non-3GPP access.

- TNGF_N3IWF_SS_SEL: Indicates that the UE supports N3IWF selection based on the slices

the UE wishes to use over untrusted non-3GPP access and TNGF selection based on the

slices the UE wishes to use over trusted non-3GPP access.

#

UePolicyTransferFailureCause:

description: UE Policy Transfer Failure Cause.

anyOf:

- $ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N1N2MessageTransferCause'

- $ref: '#/components/schemas/N1N2MessTransferErrorReply'

Annex B (normative):
Wireless and wireline convergence access support

# B.1	Scope

This annex provides the stage 3 definition of the UE Policy Control Service for wireless and wireline convergence access support for 5GS.

The stage 2 definition and procedures of the UE Policy Control Service for wireless and wireline convergence access support for 5GS are contained in 3GPP TS 23.316 [23].

# B.2	Npcf_UEPolicyControl Service

## B.2.1	Service Description

### B.2.1.1	Overview

Clause 4.1.1 applies with the modification that the 5G-RG and FN-RG replace the UE. Only URSP policies are applicable for a 5G-RG or an FN-RG connected via wireline access.

NOTE:	The URSPs related to the FN-RG are delivered to the W-AGF, which is acting as a UE towards the 5GC on behalf of the FN-RG.

### B.2.1.2	Service Architecture

Clause 4.1.2 applies with the exception that roaming functionality (V-PCF and H-PCF specific functionality) shall not apply in this Release of the specification for UE policy control for 5G-RG connecting via W-5GAN and FN-RG. Roaming architecture is only applicable to a 5G-RG connecting to the 5GC via NG RAN.

### B.2.1.3	Network Functions

#### B.2.1.3.1	Policy Control Function (PCF)

The PCF functionality defined in clause 4.1.3.1 shall apply with the following differences:

-	Only URSP policies are applicable. The PCF should not provide any UE policy other than URSP for a 5G-RG connected via wireline access.

-	If the PCF provides the URSP policy to the 5G-RG or FN-RG, the PCF should only provide the following URSP policy information:

-	Rule Precedence.

-	Traffic Descriptor, including only: Application descriptors (not applicable for FN-RG), IP descriptors, Domain descriptors (not applicable for FN-RG), Non-IP descriptors, DNN (not applicable for FN-RG), Connection Capabilities (not applicable for FN-RG).

-	Route Selection Descriptor Precedence.

-	Route selection components, including only: SSC Mode Selection, Network Slice Selection, DNN Selection, PDU Session Type Selection.

-	Route Selection Validation Criteria, including only: Time Window

-	The Visited Policy Control Function (V-PCF) shall not apply for 5G-RG connecting via wireline access and FN-RG.

-	The PCF provides the UE access selection and PDU session selection policy control as described in this Annex.

#### B.2.1.3.2	NF Service Consumers

The NF service consumer functionality shall apply as defined in clause 4.1.3.2 with the differences described in this Annex.

# B.3	Service Operations

## B.3.1	Introduction

Clause 4.2.1 is applied with the following differences:

-	UE is replaced by the 5G-RG or or FN-RG.

- 	Update of an UE Policy Association for the case that the AMF is relocated due to the UE mobility and the old PCF is selected is not applicable when the 5G-RG or FN-RG connects the 5GC via wireline access.

-	Roaming scenario is not applicable when the 5G-RG or FN-RG connects the 5GC via wireline access in this release of specification.

## B.3.2	Npcf_UEPolicyControl_Create Service Operation

### B.3.2.1	General

Clause 4.2.2.1 is applied with the following differences:

-	UE is replaced by the 5G-RG or FN-RG. The 5G-RG connecting via wireline access and the FN-RG do not indicate the support of the URSP rule enforcement capability as described in clause 4.13 of 3GPP TS 24.501 [15].

-	The PEI that may be included within the "pei" attribute shall have one of the following representations:

i.	If the 5G-BRG supports only wireline access, the PEI shall be the 5G-BRG MAC address.

ii.	If the 5G-CRG supports only wireline access, the PEI shall be the cable modem MAC address.

iii.	If the 5G-RG supports at least one 3GPP access technology, the PEI shall be the allocated IMEI or IMEISV.

iv.	For the FN-BRG and FN-CRG, the PEI shall be the FN-RG MAC address.

NOTE:	When the PEI includes an indication that the MAC address cannot be used as Equipment identifier of the of the FN-RG, the PEI cannot be trusted for regulatory purposes and cannot be used for equipment based policy evaluation.

-	When the 5G-BRG or FN-BRG connects the 5GC via W-5GBAN, the "n3gaLocation" attribute shall be included in the "ueLoc" attribute and:

-	the Global Line Identifier shall be included in the "gli" attribute; and

-	the "w5gbanLineType" attribute to indicate whether the W-5GBAN access is DSL or PON may be included.

-	The HFC Node Identifier is encoded in the "hfcNodeId" attribute of the "n3gaLocation" attribute included in the "userLoc" attribute within the PolicyAssociationRequest data structure when the 5G-CRG or FN-CRG connects to the 5GC via W-5GCAN.

-	Roaming scenario is not applicable when the 5G-RG or FN-RG connects the 5GC via wireline access in this release of specification.

-	The PCF should neither include NSWO indication nor any policies other than URSP in the UE Policy.

## B.3.3	Npcf_UEPolicyControl_Update Service Operation

### B.3.3.1	General

Clause 4.2.3.1 is applied with the following differences:

-	UE is replaced by the 5G-RG or FN-RG.

-	Roaming scenario is not applicable when the 5G-RG or FN-RG connects the 5GC via wireline access in this release of specification.

-	The PCF should neither include NSWO indication nor any policies other than URSP in the UE Policy.

## B.3.4	Npcf_UEPolicyControl_UpdateNotify Service

### B.3.4.1	General

Clause 4.2.4.1 is applied with the following differences:

-	UE is replaced by the 5G-RG or FN-RG.

-	Roaming scenario is not applicable when the 5G-RG or FN-RG connects the 5GC via wireline access in this release of specification.

-	The PCF should neither include NSWO indication nor any policies other than URSP in the UE Policy.

## B.3.5	Npcf_UEPolicyControl_Delete Service Operation

### B.3.5.1	General

Clause 4.2.5 is applied with the following differences:

-	UE is replaced by the 5G-RG or FN-RG.

-	Roaming scenario is not applicable when the 5G-RG or FN-RG connects the 5GC via wireline access in this release of specification.

Annex C (informative):
Withdrawn API versions

This Annex list withdrawn API versions of the Npcf_UEPolicyControl API defined in the present specification. 3GPP TS 3GPP TS 29.501 [6] clause 4.3.1.6 describes the withdrawal of API versions.

The API versions listed in table C-1 are withdrawn for the Npcf_UEPolicyControl API.

Table C-1: Withdrawn API versions of the Npcf_UEPolicyControl service

| API version number | Remarks |
| --- | --- |
| 1.0.0 | Deficits in: - SUPI not mandatory (Unnecessary support of Emergency registration). - Missing AMF instance id in Policy Association request |

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  | Meeting |  | TDoc |  | CR |  | Rev |  | Cat |  | Subject/Comment |  | New version |  |
| 2018-10 |  | CT3#98-Bis |  | C3-186282 |  |  |  |  |  |  |  | First TS version created based on suitable parts of TS 29.507v15.1.0 |  | 0.1.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187094 |  |  |  |  |  |  |  | API Version |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187532 |  |  |  |  |  |  |  | ExternalDocs OpenAPI field |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187096 |  |  |  |  |  |  |  | Location header field in OpenAPI |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187533 |  |  |  |  |  |  |  | Security |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187098 |  |  |  |  |  |  |  | supported content types |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187534 |  |  |  |  |  |  |  | HTTP Error responses |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187673 |  |  |  |  |  |  |  | Alternate IP address in Npcf_UEPolicyControl_Update |  | 0.2.0 |  |
| 2018-12 |  | CT3#99 |  | C3-187673 |  |  |  |  |  |  |  | Corrections on Protocol and Application errors |  | 0.2.0 |  |
| 2018-12 |  | CP#82 |  | CP-183130 |  |  |  |  |  |  |  | TS sent to plenary for information and approval |  | 1.0.0 |  |
| 2018-12 |  | CP#82 |  | CP-183175 |  |  |  |  |  |  |  | PCR 29.xyz Corrections of Cardinality in OpenAPI |  | 1.1.0 |  |
| 2018-12 |  | CP#82 |  | CP-183250 |  |  |  |  |  |  |  | TS number assigned for approval at plenary |  | 1.1.0 |  |
| 2018-12 |  | CP#82 |  | CP-183252 |  |  |  |  |  |  |  | TS approved by plenary |  | 15.0.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0001 |  | 1 |  | F |  | Usage of the Namf_Communication Service by V-PCF |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0002 |  | 1 |  | F |  | Allignment with TS 24.501 changes on UE STATE INDICATION message |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0005 |  |  |  | F |  | OpenAPI version Update |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0006 |  |  |  | F |  | Correction to the overview |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0007 |  |  |  | F |  | Correction to the descriptions of network functions |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0008 |  | 1 |  | F |  | Correction to the service operation introduction |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0011 |  | 3 |  | F |  | Correction to the Npcf_UEPolicyControl_UpdateNotify operation |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0012 |  |  |  | F |  | Correction to the PresenceInfo data type |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0013 |  |  |  | F |  | UE Policy Control support for Emergency Registration |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0014 |  |  |  | F |  | Correction to the group identifier |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0017 |  | 1 |  | F |  | Adding AMF instance id in Policy Association request |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190114 |  | 0018 |  | 3 |  | F |  | V-PCF Interworking procedures for UE policy delivery service |  | 15.1.0 |  |
| 2019-03 |  | CP#83 |  | CP-190214 |  | 0019 |  | 3 |  | F |  | Correction on the handling of URSP and ANDSP policies |  | 15.1.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0021 |  | 1 |  | F |  | ANDSP correction |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0022 |  | 2 |  | F |  | Correction to PolicyAssociationReleaseCause data type |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0023 |  | 1 |  | F |  | Resending the UE policy |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0024 |  | 2 |  | F |  | Correction to the service operation procedure |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0028 |  | 2 |  | F |  | Withdrawing API version |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0029 |  | 1 |  | F |  | Precedence of OpenAPI file |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0030 |  | 1 |  | F |  | API version Update |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191082 |  | 0031 |  |  |  | F |  | Correction to the serviceName attribute |  | 15.2.0 |  |
| 2019-06 |  | CT#84 |  | CP-191160 |  | 0034 |  | 2 |  | F |  | Copyright Note in YAML file |  | 15.2.0 |  |
| 2019-06 |  | CP#84 |  | CP-191089 |  | 0027 |  | 1 |  | F |  | Correction on Policy Association Termination |  | 16.0.0 |  |
| 2019-06 |  | CP#84 |  | CP-191089 |  | 0032 |  | 1 |  | B |  | Race condition handling |  | 16.0.0 |  |
| 2019-06 |  | CP#84 |  | CP-191101 |  | 0035 |  | 1 |  | F |  | API version Update |  | 16.0.0 |  |
| 2019-09 |  | CP#85 |  | CP-192178 |  | 0036 |  |  |  | B |  | Adding NID as input for policy decisions |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192148 |  | 0038 |  |  |  | A |  | UE policy correction in AMF |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192152 |  | 0040 |  | 1 |  | B |  | Support of wireline and wireless access convergence, Annex Skeleton |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192176 |  | 0041 |  | 1 |  | B |  | Support of wireline and wireless access convergence, NFs |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192224 |  | 0043 |  | 3 |  | A |  | Message transfer failure notification |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192171 |  | 0044 |  | 3 |  | B |  | URSP rule provisioning for supporting xBDT |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192148 |  | 0046 |  | 1 |  | A |  | GUAMI included in the Update operation |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192160 |  | 0047 |  | 1 |  | B |  | PLMN change for V2X |  | 16.1.0 |  |
| 2019-09 |  | CP#85 |  | CP-192173 |  | 0048 |  |  |  | F |  | OpenAPI version update for TS 29.525 Rel-16 |  | 16.1.0 |  |
| 2019-12 |  | CP#86 |  | CP-193197 |  | 0050 |  | 1 |  | F |  | Data type of the "serviceName" attribute |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193223 |  | 0051 |  |  |  | F |  | Correcting references related to xBDT support |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193189 |  | 0053 |  | 1 |  | A |  | Correction to the trigger of UE policy association establishment |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193223 |  | 0054 |  | 3 |  | B |  | URSP provisioning for xBDT |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193197 |  | 0055 |  | 1 |  | B |  | Format of hPcfId attribute |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193197 |  | 0057 |  | 1 |  | B |  | Subscription to UE Connectivity state changes |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193197 |  | 0058 |  |  |  | F |  | Removal of TABs from OpenAPI file |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193202 |  | 0059 |  | 1 |  | F |  | correction to PLMN change trigger |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193223 |  | 0060 |  | 1 |  | B |  | store BDT reference ID in SMPolicyData |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193189 |  | 0064 |  |  |  | A |  | Correction to PolicyUpdate |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193189 |  | 0066 |  | 1 |  | A |  | Correction on 307 error |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193191 |  | 0067 |  | 1 |  | B |  | Clarification of PEI format, TS 29.525 |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193227 |  | 0068 |  | 2 |  | B |  | Wireline Location information |  | 16.2.0 |  |
| 2019-12 |  | CP#86 |  | CP-193212 |  | 0069 |  |  |  | F |  | Update of API version and TS version in OpenAPI file |  | 16.2.0 |  |
| 2020-03 |  | CT#87e |  | CP-200223 |  | 0071 |  |  |  | B |  | Correction on UE Policy Association Establishment |  | 16.3.0 |  |
| 2020-03 |  | CT#87e |  | CP-200212 |  | 0072 |  | 1 |  | B |  | Network function enhancement for V2X communication |  | 16.3.0 |  |
| 2020-03 |  | CT#87e |  | CP-200212 |  | 0073 |  | 1 |  | B |  | UE Policy for V2XARC |  | 16.3.0 |  |
| 2020-03 |  | CT#87e |  | CP-200262 |  | 0074 |  | 2 |  | B |  | N2 PC5 Policy for V2XARC |  | 16.3.0 |  |
| 2020-03 |  | CT#87e |  | CP-200203 |  | 0075 |  | 1 |  | B |  | Complete the procedure for WWC |  | 16.3.0 |  |
| 2020-03 |  | CT#87e |  | CP-200207 |  | 0076 |  |  |  | B |  | Completing the description of "PLMN_CH" and "CON_STATE_CH" triggers. |  | 16.3.0 |  |
| 2020-03 |  | CT#87e |  | CP-200216 |  | 0078 |  | 1 |  | B |  | Update of OpenAPI version and TS version in externalDocs field |  | 16.3.0 |  |
| 2020-06 |  | CT#88e |  | CP-201224 |  | 0080 |  | 1 |  | A |  | Location Header of 307 status code |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201224 |  | 0082 |  | 1 |  | A |  | Notification URI |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201233 |  | 0083 |  |  |  | B |  | FQDN of alternative AMF |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201224 |  | 0085 |  |  |  | A |  | Description of scopes field and presenceStatus attribute |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201228 |  | 0086 |  |  |  | F |  | Removal of MAC address |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201244 |  | 0087 |  |  |  | F |  | Removal of unbreakable spaces |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201228 |  | 0088 |  | 2 |  | B |  | Untrusted FN-RG PEI |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201244 |  | 0089 |  | 1 |  | F |  | Storage of YAML files in ETSI Forge |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0090 |  | 1 |  | B |  | Correction to the UE policy definition |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0091 |  | 1 |  | B |  | Correction to the V2X Policy provisioning |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0093 |  | 1 |  | B |  | Remove editor's node |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201256 |  | 0094 |  | 1 |  | F |  | URI of the Npcf_UEPolicyControl service |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0095 |  |  |  | F |  | AF-based service parameter provisioning |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0096 |  |  |  | F |  | Complete service description for V2X |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0097 |  |  |  | F |  | Corrections on N2 PC5 policy |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0098 |  |  |  | F |  | Include V2XP info contents into policy section |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201213 |  | 0099 |  | 1 |  | B |  | Support of Dual Connectivity end to end Redundant User Plane Paths |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201238 |  | 0100 |  |  |  | F |  | Correction to 4.2.4.1 |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201244 |  | 0101 |  |  |  | F |  | Optionality of ProblemDetails |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201244 |  | 0102 |  | 1 |  | F |  | Supported headers, Resource Data type, Operation Name |  | 16.4.0 |  |
| 2020-06 |  | CT#88e |  | CP-201255 |  | 0105 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs field |  | 16.4.0 |  |
| 2020-09 |  | CT#89e |  | CP-202069 |  | 0106 |  |  |  | F |  | Include N2 PC5 policy in update response |  | 16.5.0 |  |
| 2020-09 |  | CT#89e |  | CP-202069 |  | 0107 |  |  |  | F |  | Remove the dependency of subscription data in UDR for V2X |  | 16.5.0 |  |
| 2020-09 |  | CT#89e |  | CP-202079 |  | 0108 |  | 1 |  | F |  | report initial presence status for PRA |  | 17.0.0 |  |
| 2020-09 |  | CT#89e |  | CP-202073 |  | 0109 |  | 1 |  | B |  | Successful Response |  | 17.0.0 |  |
| 2020-09 |  | CT#89e |  | CP-202073 |  | 0110 |  |  |  | B |  | Error status code |  | 17.0.0 |  |
| 2020-09 |  | CT#89e |  | CP-202085 |  | 0112 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs field |  | 17.0.0 |  |
| 2020-12 |  | CT#90e |  | CP-203139 |  | 0115 |  | 1 |  | A |  | Essential corrections and alignments |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203139 |  | 0117 |  | 1 |  | A |  | Storage of YAML files in 3GPP Forge |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203143 |  | 0119 |  | 1 |  | A |  | Correction to PRA |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203129 |  | 0121 |  | 1 |  | A |  | Correction to the BDT policy re-negotiation |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203119 |  | 0126 |  | 1 |  | A |  | Correction to Policy Update Notification |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203148 |  | 0127 |  | 1 |  | F |  | Report current value in Update for location related triggers |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203148 |  | 0128 |  | 1 |  | B |  | Adding 200OK response for UpdateNotify |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203148 |  | 0129 |  | 2 |  | B |  | Support of 307&404 response codes for Policy update notification |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203148 |  | 0130 |  |  |  | F |  | "400 Bad Request" response on notification |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203150 |  | 0132 |  | 1 |  | A |  | Correction to URSP rules, support of 5G VN services |  | 17.1.0 |  |
| 2020-12 |  | CT#90e |  | CP-203153 |  | 0136 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs fieldFilename: draft29525-h10-rm-v0.doc |  | 17.1.0 |  |
| 2021-03 |  | CT#91e |  | CP-210191 |  | 0139 |  | 1 |  | A |  | Support of stateless NFs |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210210 |  | 0141 |  | 1 |  | A |  | Correction to N2 PC5 policy |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210218 |  | 0142 |  |  |  | F |  | Adding "description" field for map data types |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210218 |  | 0143 |  |  |  | F |  | OpenAPI reference |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210219 |  | 0144 |  |  |  | F |  | Clarification on optional HTTP custom headers |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210227 |  | 0146 |  | 1 |  | F |  | Clarification of update operation |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210221 |  | 0147 |  | 1 |  | F |  | Ambiguous concept of NF service consumer terminology |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210221 |  | 0148 |  | 1 |  | F |  | Adding some missing description fields to data type definitions in OpenAPI specification files |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210191 |  | 0150 |  | 1 |  | A |  | Correction to resource identifiers descriptions used in notifications |  | 17.2.0 |  |
| 2021-03 |  | CT#91e |  | CP-210240 |  | 0152 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs field |  | 17.2.0 |  |
| 2021-06 |  | CT#92e |  | CP-211133 |  | 0153 |  | 4 |  | B |  | 5G ProSe related updates to the Npcf_UEPolicyControl_Create Service Operation |  | 17.3.0 |  |
| 2021-06 |  | CT#92e |  | CP-211245 |  | 0155 |  | 1 |  | B |  | GLI report |  | 17.3.0 |  |
| 2021-06 |  | CT#92e |  | CP-211200 |  | 0157 |  | 1 |  | A |  | Temporary and Permanent Redirection |  | 17.3.0 |  |
| 2021-06 |  | CT#92e |  | CP-211218 |  | 0158 |  | 1 |  | B |  | Support of UE policy updates for AF influence on URSP |  | 17.3.0 |  |
| 2021-06 |  | CT#92e |  | CP-211265 |  | 0160 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs field |  | 17.3.0 |  |
| 2021-09 |  | CT#93e |  | CP-212220 |  | 0161 |  | 1 |  | F |  | Correction of URI structure |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212250 |  | 0163 |  | 1 |  | A |  | Correction to V2X Policy Provisioning Request |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212188 |  | 0164 |  | 1 |  | F |  | Correction to ProSe Policy Provisioning Request |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212188 |  | 0165 |  | 1 |  | F |  | Separation of 5G ProSe N2 PC5 and V2X N2 PC5 policies. |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212188 |  | 0166 |  |  |  | B |  | Update of URSP definition |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212188 |  | 0167 |  | 1 |  | B |  | Notification of 5G ProSe capability changes |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212224 |  | 0168 |  | 1 |  | F |  | Correction to the reused data types |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212224 |  | 0169 |  |  |  | F |  | Correction to immediate PRA report |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212188 |  | 0170 |  | 1 |  | B |  | Removal of some 5G ProSe related ENs |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212224 |  | 0171 |  | 1 |  | F |  | Miscellaneous corrections to the Npcf_UEPolicyControl service |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212190 |  | 0174 |  |  |  | A |  | Correction of URI related attribute for the termination notification |  | 17.4.0 |  |
| 2021-09 |  | CT#93e |  | CP-212223 |  | 0175 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs field |  | 17.4.0 |  |
| 2021-12 |  | CT#94e |  | CP-213229 |  | 0177 |  |  |  | F |  | Direct access to SNPN |  | 17.5.0 |  |
| 2021-12 |  | CT#94e |  | CP-213243 |  | 0178 |  |  |  | F |  | Correction to Update procedure |  | 17.5.0 |  |
| 2021-12 |  | CT#94e |  | CP-213213 |  | 0179 |  | 1 |  | B |  | Updates to ProSeP for 5G ProSe UE-to-network relay |  | 17.5.0 |  |
| 2021-12 |  | CT#94e |  | CP-213223 |  | 0180 |  |  |  | B |  | Handling of retrieved URSP policies from the UDR |  | 17.5.0 |  |
| 2021-12 |  | CT#94e |  | CP-213248 |  | 0181 |  |  |  | B |  | Support of RSN and PDU Session Pair ID in the URSP Rule |  | 17.5.0 |  |
| 2021-12 |  | CT#94e |  | CP-213244 |  | 0182 |  | 1 |  | F |  | Error handling when no UE Policy Association exists |  | 17.5.0 |  |
| 2021-12 |  | CT#94e |  | CP-213246 |  | 0183 |  |  |  | F |  | Update of OpenAPI version and TS version in externalDocs field |  | 17.5.0 |  |
| 2022-03 |  | CT#95e |  | CP-220206 |  | 0184 |  | 1 |  | B |  | PCF checking of redundant PDU session applicability |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220176 |  | 0185 |  | 1 |  | A |  | Alignment of "Application Errors" clause with SBI TS template |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220186 |  | 0186 |  | 1 |  | B |  | Resolutions related to URSP guidance handling at the PCF |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220185 |  | 0187 |  | 1 |  | F |  | Handling of supported features for Edge Computing |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220185 |  | 0188 |  | 1 |  | F |  | Removal of Editor's Notes related to AF guidance of URSP determination |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220167 |  | 0191 |  | 1 |  | A |  | Handling of error responses |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220174 |  | 0193 |  | 1 |  | A |  | Description of a "307 Temporary Redirect" response |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220193 |  | 0195 |  | 1 |  | F |  | Description of a "307 Temporary Redirect" response in table 5.5.2.2-2 |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220196 |  | 0196 |  |  |  | F |  | Removing the remaining ENs on ProSeP definition2022-03 |  | 17.6.0 |  |
| 2022-03 |  | CT#95e |  | CP-220194 |  | 0200 |  |  |  | F |  | Update of info and externalDocs fields |  | 17.6.0 |  |
| 2022-06 |  | CT#96 |  | CP-221154 |  | 0201 |  | 1 |  | F |  | Formatting of description fields |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221154 |  | 0202 |  | - |  | F |  | Using the common data type for FQDN |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221126 |  | 0203 |  | - |  | F |  | Validation of guidance information |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221126 |  | 0204 |  | - |  | F |  | Limitation of URSP derived based guidance information |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221126 |  | 0205 |  | 1 |  | F |  | Notification of outcome of URSP provisioning |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221116 |  | 0208 |  | 1 |  | F |  | Correction to ProSe related triggers |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221253 |  | 0209 |  | 2 |  | F |  | Correction to GROUP_ID_LIST_CHG trigger |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221138 |  | 0210 |  | 1 |  | B |  | UE Policies support in SNPN |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221254 |  | 0211 |  | 2 |  | A |  | Request of ProSeP/V2XP during NAS Transport procedure |  | 17.7.0 |  |
| 2022-06 |  | CT#96 |  | CP-221151 |  | 0215 |  | - |  | F |  | Update of info and externalDocs fields |  | 17.7.0 |  |
| 2022-09 |  | CT#97e |  | CP-222123 |  | 0217 |  | 1 |  | F |  | Alignment with the SBI template |  | 17.8.0 |  |
| 2022-09 |  | CT#97e |  | CP-222133 |  | 0218 |  | 1 |  | F |  | Correction in the handling of precedence for URSP rules |  | 17.8.0 |  |
| 2022-09 |  | CT#97e |  | CP-224217 |  | 0220 |  |  |  | A |  | Correction of policy update procedures |  | 17.8.0 |  |
| 2022-09 |  | CT#97e |  | CP-222178 |  | 0221 |  | 1 |  | F |  | Correction of UE Policy Association management for URSP and ANDSP |  | 17.8.0 |  |
| 2022-09 |  | CT#97e |  | CP-224322 |  | 0222 |  |  |  | F |  | Correction to UE policy provisioning for AF-influenced URSP |  | 17.8.0 |  |
| 2022-12 |  | CT#98e |  | CP-223162 |  | 0223 |  | 2 |  | F |  | Removal of the request of V2XP and/or ProSeP during registration |  | 17.9.0 |  |
| 2022-12 |  | CT#98e |  | CP-223177 |  | 0229 |  |  |  | F |  | Corrections to PLMN change trigger |  | 17.9.0 |  |
| 2022-12 |  | CT#98e |  | CP-223177 |  | 0231 |  |  |  | F |  | Correction to UE Policies determination in a serving SNPN |  | 17.9.0 |  |
| 2022-12 |  | CT#98e |  | CP-223188 |  | 0232 |  |  |  | F |  | Update of info and externalDocs fields |  | 17.9.0 |  |
| 2022-12 |  | CT#98e |  | CP-223201 |  | 0225 |  | 1 |  | B |  | Feature awareness during UE mobility with AMF change |  | 18.0.0 |  |
| 2022-12 |  | CT#98e |  | CP-223191 |  | 0227 |  |  |  | F |  | Adding the mandatory error code 502 Bad Gateway |  | 18.0.0 |  |
| 2022-12 |  | CT#98e |  | CP-223199 |  | 0228 |  | 1 |  | F |  | Correction on the handling of UE policy delivery errors |  | 18.0.0 |  |
| 2022-12 |  | CT#98e |  | CP-223178 |  | 0230 |  |  |  | B |  | SNPN mobility |  | 18.0.0 |  |
| 2022-12 |  | CT#98e |  | CP-223189 |  | 0233 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.0.0 |  |
| 2023-03 |  | CT#99 |  | CP-230133 |  | 0234 |  | 1 |  | B |  | Adding Configured NSSAI to UE Policy Control inputs |  | 18.1.0 |  |
| 2023-03 |  | CT#99 |  | CP-230166 |  | 0236 |  |  |  | F |  | Correction of the description fields in enumerations |  | 18.1.0 |  |
| 2023-03 |  | CT#99 |  | CP-230147 |  | 0237 |  | 1 |  | B |  | NWDAF-assisted for URSP rules determination |  | 18.1.0 |  |
| 2023-03 |  | CT#99 |  | CP-230152 |  | 0238 |  | 1 |  | B |  | URSP provisioning in EPS |  | 18.1.0 |  |
| 2023-03 |  | CT#99 |  | CP-230181 |  | 0239 |  | 1 |  | F |  | Feature negotiation in roaming scenarios |  | 18.1.0 |  |
| 2023-03 |  | CT#99 |  | CP-230130 |  | 0240 |  | 1 |  | B |  | Support of the satellite backhaul category |  | 18.1.0 |  |
| 2023-03 |  | CT#99 |  | CP-230161 |  | 0241 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.1.0 |  |
| 2023-06 |  | CT#100 |  | CP-231174 |  | 0242 |  | 1 |  | B |  | Support of Non-3GPP access for SNPN scenarios |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231165 |  | 0243 |  | 1 |  | B |  | Trigger slice-aware ANDSP/WLANSP determination |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231165 |  | 0244 |  | 1 |  | B |  | ANDSP delivery notifications |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231145 |  | 0246 |  | 1 |  | B |  | Representation of N43 reference point |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231133 |  | 0247 |  | 1 |  | F |  | ANDSP support indication to V-PCF |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231182 |  | 0248 |  | 1 |  | B |  | Support of PIN ID in URSP |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231156 |  | 0249 |  | 1 |  | B |  | Support for A2X service authorization and policy provisioning in Npcf_UEPolicyControl Service Operation |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231131 |  | 0251 |  | 1 |  | D |  | Miscellaneouse changes |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231173 |  | 0252 |  | 1 |  | B |  | Complete the feature negotiation during the AMF relocation |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231145 |  | 0253 |  | 3 |  | B |  | Support for URSP awareness |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231145 |  | 0254 |  | 2 |  | B |  | Support of the configured NSSAI change |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231165 |  | 0255 |  | 1 |  | B |  | Support of TNGF selection for S-NSSAI |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231132 |  | 0258 |  | 1 |  | F |  | HTTP redirection clause correction |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231184 |  | 0259 |  | 1 |  | B |  | Support UE policies for 5G ProSe UE-to-UE relay |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231145 |  | 0260 |  | 1 |  | B |  | URSP Provisioning in EPS roaming support |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231134 |  | 0261 |  | 1 |  | B |  | URSP Provisioning in EPS corrections regarding the delivery of the initial UE policy container with the UE STATE INDICATION message |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231145 |  | 0262 |  | 1 |  | B |  | Provisioning of VPLMN specific URSP rules |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231145 |  | 0263 |  | 1 |  | B |  | Completion of URSP provisioning in EPS |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231173 |  | 0264 |  | 1 |  | B |  | Solving remaining ENs on feature re-negotiation |  | 18.2.0 |  |
| 2023-06 |  | CT#100 |  | CP-231141 |  | 0265 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.2.0 |  |
| 2023-09 |  | CT#101 |  | CP-232098 |  | 0266 |  | 1 |  | F |  | Update on PresenceInfo for PolicyUpdate |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232104 |  | 0267 |  |  |  | F |  | EN resolution for A2XP related UE policy encoding |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232104 |  | 0268 |  |  |  | F |  | EN resolution for A2X subscription |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232108 |  | 0269 |  |  |  | B |  | UE policy based on subscribed DNN/S-NSSAI for PIN scenarios |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232092 |  | 0270 |  | 1 |  | B |  | Enhancement to Npcf_UEPolicyControl service for URSP rule enforcement |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232099 |  | 0271 |  | 1 |  | B |  | Spending limits report for UE Policy |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232121 |  | 0272 |  | 1 |  | B |  | Completion of the reporting of Satellite Backhaul changes |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232092 |  | 0273 |  | 1 |  | B |  | URSP rule enforcement |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232240 |  | 0274 |  | 1 |  | B |  | Completion of the Provisioning of VPLMN specific URSP |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232092 |  | 0275 |  | 1 |  | B |  | Solving URSP delivery in EPS ENs related to roaming scenarios |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232092 |  | 0276 |  | 3 |  | B |  | Completion of the reporting of configured NSSAI changes |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232110 |  | 0277 |  | 1 |  | B |  | N1N2MessageSubscribe during AMF relocation |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232110 |  | 0278 |  |  |  | B |  | Discovery of Namf_Communication service |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232110 |  | 0279 |  | 1 |  | B |  | Non-subscribed SNPN signalled URSP |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232104 |  | 0280 |  | 1 |  | B |  | Support of A2X policy provisioning for A2X communication over Uu reference point |  | 18.3.0 |  |
| 2023-09 |  | CT#101 |  | CP-232085 |  | 0281 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.3.0 |  |
| 2023-12 |  | CT#102 |  | CP-233245 |  | 0282 |  | 2 |  | B |  | Support the change of the PDU Session Type for a 5G VN group |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0285 |  | 1 |  | B |  | Remove the EN for the details that the PCF receives the report of URSP rule enforcement info from NWDAF |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233202 |  | 0286 |  | 1 |  | F |  | Correction to the report of URSP rule enforcement info |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0287 |  |  |  | F |  | Wrong attribute name |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233256 |  | 0288 |  | 2 |  | D |  | Incorrect feature name |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233256 |  | 0289 |  | 1 |  | F |  | Correction in error handling in roaming scenarios |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0290 |  | 1 |  | B |  | Enhancement to Npcf_UEPolicyControl service for URSP rule enforcement in EPS |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233268 |  | 0293 |  | 1 |  | B |  | Update UE Policy Control Service for Ranging_SL |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0295 |  | 1 |  | F |  | Miscellaneous changes |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0297 |  | 1 |  | B |  | Subscription to the outcome of the provisioning of VPLMN specific URSP |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0298 |  | 1 |  | B |  | URSP provisioning in EPS in Home Routed scenarios |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233172 |  | 0299 |  | 2 |  | B |  | Same UE Policy Association shared by 3GPP and non-3GPP |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0300 |  | 1 |  | F |  | Corrections on PCRT for immediate report |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0301 |  |  |  | F |  | definition of term Configured NSSAI |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233244 |  | 0302 |  | 1 |  | B |  | Update for the URSP rule enforcement without UE assistance |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233230 |  | 0304 |  | 1 |  | F |  | HTTP RFC uplifting |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233253 |  | 0305 |  | 1 |  | B |  | Support for services deployed on GEO satellite |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233283 |  | 0306 |  |  |  | B |  | Non-subscribed SNPN signalled WLANSP |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233256 |  | 0307 |  | 1 |  | B |  | Completion of error handling functionality |  | 18.4.0 |  |
| 2023-12 |  | CT#102 |  | CP-233237 |  | 0309 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.4.0 |  |
| 2024-03 |  | CT#103 |  | CP-240183 |  | 0292 |  | 2 |  | F |  | EN resolution on selecting a PDU session for URSP provisioning in EPS |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240202 |  | 0311 |  |  |  | F |  | Resolve ENs related to A2XP encoding. |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240188 |  | 0315 |  | 1 |  | F |  | Missing applicable feature for suppFeat attribute |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240180 |  | 0316 |  |  |  | F |  | Error handling update and miscellaneous changes |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240190 |  | 0317 |  | 1 |  | B |  | Support of CHF address(es) information |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240181 |  | 0318 |  | 1 |  | F |  | UE Policy restrictions and differences for Wireline Access |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240183 |  | 0319 |  |  |  | F |  | Data type correction |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240183 |  | 0320 |  |  |  | F |  | Re-used data types |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240179 |  | 0321 |  | 1 |  | B |  | Various corrections and alignments |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240173 |  | 0322 |  | 1 |  | F |  | Correction on 200 OK response |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240181 |  | 0323 |  | 1 |  | B |  | Slice based TNGF and N3IWF selection |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240183 |  | 0324 |  |  |  | F |  | Miscellaneous Corrections |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240242 |  | 0325 |  | 2 |  | F |  | UE Policy Association over 3GPP and non-3GPP access |  | 18.5.0 |  |
| 2024-03 |  | CT#103 |  | CP-240166 |  | 0326 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.5.0 |  |
| 2024-06 |  | CT#104 |  | CP-241107 |  | 0327 |  | 1 |  | F |  | Miscellaneous changes |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0328 |  | 2 |  | B |  | Resolve EN on URSP rule enforcement when the UE moves from EPS to 5GS |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241093 |  | 0330 |  | 1 |  | F |  | Corrections on the trigger description |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0331 |  | 1 |  | B |  | URSP rule enforcement |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0332 |  |  |  | B |  | URSP over EPS, SM and UE Policy Association selection |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241113 |  | 0333 |  | 1 |  | F |  | Clarification to UPSI determination and Access Type Change |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0335 |  | 1 |  | B |  | Missing PDU Session Type in URSP rule enforcement information |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241093 |  | 0337 |  | 1 |  | F |  | Description update and miscellaneous changes |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241113 |  | 0339 |  | 1 |  | F |  | completion of 4.2.3.2 |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241113 |  | 0340 |  | 1 |  | F |  | Correction of AMF Relocation scenario |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0341 |  |  |  | F |  | Corrections on UE policy Notification |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0342 |  |  |  | F |  | Corrections on VPLMN Specific URSP |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241098 |  | 0343 |  | 1 |  | F |  | Removal of reported parameters in URSP rule enforcement information |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241097 |  | 0344 |  |  |  | F |  | URSP Enforcement information and PDU session parameters |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241098 |  | 0345 |  |  |  | B |  | Support of PCF Binding information |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241121 |  | 0346 |  |  |  | F |  | Correction to A2XP determination |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241113 |  | 0347 |  |  |  | F |  | Misalignment between UE Policies stored in the UE and UE Policies stored in 5GC |  | 18.6.0 |  |
| 2024-06 |  | CT#104 |  | CP-241085 |  | 0348 |  |  |  | F |  | Update of info and externalDocs fields |  | 18.6.0 |  |
| 2024-08 |  | CT#104 |  |  |  |  |  |  |  |  |  | Correction of order of clauses 5.6.3.12 and 5.6.4 |  | 18.6.1 |  |
| 2024-09 |  | CT#105 |  | CP-242151 |  | 0353 |  | 1 |  | F |  | Indication of slice-related N3GPP node selection capability |  | 18.7.0 |  |
| 2024-09 |  | CT#105 |  | CP-242151 |  | 0354 |  | 1 |  | F |  | Missing attribute |  | 18.7.0 |  |
| 2024-09 |  | CT#105 |  | CP-242137 |  | 0357 |  | 1 |  | F |  | Corrections to the provisioning of VPLMN Specific URSP rules |  | 18.7.0 |  |
| 2024-09 |  | CT#105 |  | CP-242140 |  | 0358 |  | 1 |  | F |  | Enhanced H-PCF reselection |  | 18.7.0 |  |
| 2024-09 |  | CT#105 |  | CP-242145 |  | 0362 |  | 1 |  | F |  | Correction on CHF selection |  | 18.7.0 |  |
| 2024-09 |  | CT#105 |  | CP-242120 |  | 0364 |  | - |  | F |  | Update of info and externalDocs fields |  | 18.7.0 |  |