3GPP TS 29.522 V18.8.0 (2024-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	33

2	References	33

3	Definitions and abbreviations	35

3.1	Definitions	35

3.2	Abbreviations	36

4	NEF Northbound Interface	37

4.1	Overview	37

4.2	Reference model	40

4.3	Functional elements	40

4.3.1	NEF	40

4.3.2	AF	41

4.4	Procedures over NEF Northbound Interface	41

4.4.1	Introduction	41

4.4.2	Procedures for Monitoring	41

4.4.3	Procedures for Device Triggering	49

4.4.4	Procedures for resource management of Background Data Transfer	50

4.4.5	Procedures for CP Parameters Provisioning	50

4.4.6	Procedures for PFD Management	52

4.4.7	Procedures for Traffic Influence	52

4.4.7.1	General	52

4.4.7.2	AF request identified by UE address	53

4.4.7.3	AF request not identified by UE address	54

4.4.7.4	Handling of UP path management event notification	54

4.4.7.5	Processing AF requests to influence traffic routing for HR-SBO session	55

4.4.8	Procedures for changing the chargeable party at session set up or during the session	56

4.4.9	Procedures for AF required QoS	57

4.4.9.1	General	57

4.4.9.2	Procedures for AF setting up an AF session with required QoS for target UE identified by UE address or for target list of UEs identified by list of UE addresses	57

4.4.9.3	Procedures for AF requested QoS for a target UE or group of UE(s) not identified by UE address(es)	67

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	67

4.4.11	Procedures for Network Configuration Parameters Provisioning	68

4.4.12	Procedures for Non-IP data delivery	69

4.4.12.1	General	69

4.4.12.2	NIDD configuration Triggered by the NEF	69

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	69

4.4.13	Procedures for RACS Parameter Provisioning	70

4.4.14	Procedures for analytics information exposure	70

4.4.14.1	Subscription/unsubscription to notification of analytics information	70

4.4.14.2	Fetch analytics information	72

4.4.15	Procedures for 5G LAN Parameter Provisioning	72

4.4.15.1	General	72

4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning	73

4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning	73

4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning	73

4.4.15.5	5G LAN parameter provisioning event notification	73

4.4.16	Procedures for applying BDT policy	74

4.4.17	Procedures for Enhanced Coverage Restriction Control	74

4.4.18	Procedures for IPTV Configuration	75

4.4.20	Procedures for service specific parameter provisioning	76

4.4.21	Procedures for ACS configuration parameter provisioning	79

4.4.22	Procedures for Mobile Originated Location Request	80

4.4.22.1	General	80

4.4.22.2	Location Update Notification triggered by UE	80

4.4.23	Procedures for AKMA	80

4.4.23.1	General	80

4.4.23.2	AKMA Application Key Request	80

4.4.23.3	AKMA Service Disablement Notification	81

4.4.24	Procedures for Time Synchronization Exposure	81

4.4.24.0	General	81

4.4.24.1	Subscription and unsubscription to notification of Time Synchronization Capabilites	81

4.4.24.2	Time Synchronization Exposure Configuration	83

4.4.24.3	Management of 5G access stratum time distribution	84

4.4.25	Procedures for ECS address Provisioning	85

4.4.26	Procedures for AM Policy Authorization	86

4.4.26.1	General	86

4.4.26.2	Creation of a new Individual Application AM Context	86

4.4.26.3	Modification of an existing individual Application AM Context	87

4.4.26.4	Deletion of an existing individual Application AM Context	87

4.4.26.5	Create or modify subscription to notification of AM policy event	87

4.4.26.6	Unsubscription to notification of AM policy event	88

4.4.26.7	Notification of AM policy event	88

4.4.27	Procedures for AF triggered Access and Mobility Influence	88

4.4.27.1	General	88

4.4.27.2	Create the AM Influence Subscription	88

4.4.27.3	Modifiy the AM Influence Subscription	89

4.4.27.4	Delete the AM Influence Subscription	89

4.4.27.5	Notification of service area coverage outcome events	89

4.4.28	Procedures for Northbound EAS Deployment Information management	89

4.4.28.1	General	89

4.4.28.2	Creation of a new Individual EAS Deployment information resource	90

4.4.28.3	Modification of an existing individual EAS Deployment Information resource	90

4.4.28.4	Deletion of an existing individual EAS Deployment Information resource	91

4.4.28.5	Deletion of EAS Deplyoment Information based on given criteria	91

4.4.29	Procedures for MBS Management	91

4.4.29.1	General	91

4.4.29.2	Procedures for MBS TMGI management	92

4.4.29.2.1	General	92

4.4.29.2.2	Procedure for MBS TMGI(s) allocation or MBS TMGI(s) expiry time refresh	92

4.4.29.2.3	Procedure for MBS TMGI(s) deallocation	93

4.4.29.2.4	Procedure for MBS TMGI(s) timer expiry notification	93

4.4.29.3	Procedures for MBS session management	94

4.4.29.3.1	General	94

4.4.29.3.2	Procedure for MBS session creation	94

4.4.29.3.3	Procedure for MBS session update	96

4.4.29.3.4	Procedure for MBS session deletion	97

4.4.29.3.5	Procedure for MBS session status subscription	97

4.4.29.3.6	Procedure for MBS session status unsubscription	98

4.4.29.3.7	Procedure for MBS session status notification	98

4.4.29.4	Procedures for MBS Parameters Provisioning	99

4.4.29.4.1	General	99

4.4.29.4.2	Procedure for multicast MBS Session Authorization information provisioning	99

4.4.29.4.3	Procedure for multicast MBS Session Assistance information provisioning	100

4.4.29.5	Procedures for MBS User Service management	101

4.4.29.5.1	General	101

4.4.29.5.2	Procedure for MBS User Service creation	101

4.4.29.5.3	Procedure for MBS User Service retrieval	101

4.4.29.5.4	Procedure for MBS User Service update/modification	101

4.4.29.5.5	Procedure for MBS User Service deletion	102

4.4.29.6	Procedures for MBS User Data Ingest Session management	102

4.4.29.6.1	General	102

4.4.29.6.2	Procedure for MBS User Data Ingest Session creation	102

4.4.29.6.3	Procedure for MBS User Data Ingest Session retrieval	103

4.4.29.6.4	Procedure for MBS User Data Ingest Session update/modification	103

4.4.29.6.5	Procedure for MBS User Data Ingest Session deletion	104

4.4.29.6.6	Procedure for MBS User Data Ingest Session Status Subscription	104

4.4.29.6.7	Procedure for MBS User Data Ingest Session Status update/modification	104

4.4.29.6.8	Procedure for MBS User Data Ingest Session Status Unsubscription	105

4.4.29.6.9	Procedure for MBS User Data Ingest Session Status Notification	105

4.4.29.7	Procedures for MBS Group Message Delivery Management	106

4.4.29.7.1	General	106

4.4.29.7.2	Procedure for MBS Group Message Delivery Creation	106

4.4.29.7.3	Procedure for MBS Group Message Delivery Update	106

4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion	107

4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification	107

4.4.30	Procedures for Data Reporting	107

4.4.30.1	General	107

4.4.30.2	Procedure for Data Reporting Session Management	108

4.4.30.3	Procedure for Data Report	108

4.4.31	Procedures for Data Reporting Provisioning	108

4.4.31.1	General	108

4.4.31.2	Procedure for Data Reporting Provisioning Session Management	108

4.4.31.3	Procedure for Data Reporting Configuration management	109

4.4.32	Procedures for UE ID retrieval	110

4.4.32.1	General	110

4.4.32.2	Retrieve AF specific UE ID service operation	110

4.4.32.3	Retrieve UE ID in the form of MSISDN	111

4.4.32.4	UE ID Mapping Information Provisioning	112

4.4.33	Procedures for Media Streaming Event Exposure	113

4.4.33.1	General	113

4.4.33.2	Procedure for Media Streaming Event Exposure Subscription Creation	113

4.4.33.3	Procedure for Media Streaming Event Exposure Subscription Update	114

4.4.33.4	Procedure for Media Streaming Event Exposure Unsubscription	114

4.4.33.5	Procedure for Media Streaming Event Exposure Notification	114

4.4.34	Procedures for DNAI Mapping	115

4.4.34.1	General	115

4.4.34.2	Creation of a new subscription for DNAI Mapping	115

4.4.34.3	Deletion of an existing individual DNAI Mapping subscription	115

4.4.34.4	Notification for updated DNAI Mapping information	116

4.4.35	Procedures for negotiation of planned data transfer with QoS requirements	116

4.4.36	Procedures for Member UE Selection Assistance	117

4.4.36.1	General	117

4.4.36.2	Procedure for Member UE Selection Assistance Subscription Creation	117

4.4.36.3	Procedure for Member UE Selection Assistance Subscription Update	117

4.4.36.4	Procedure for Member UE Selection Assistance Subscription Unsubscription	118

4.4.36.5	Procedure for Member UE Selection Assistance Notification	118

4.4.37	Procedures for Group Parameters Provisioning	118

4.4.37.1	General	118

4.4.37.2	Procedures for DNN and S-NSSAI specific Group Parameters Provisioning	118

4.4.38	Procedures for Network Slice Parameters Provisioning	119

4.4.38.1	General	119

4.4.38.2	Procedures for Network Slice Usage Control Parameters Provisioning	120

4.4.39	Procedures for UE Address Retrieval	121

4.4.39.1	General	121

4.4.39.2	Procedures for UE Address Retrieval	121

4.4.40	Procedures for ECS Address Configuration Information provisioning in roaming	122

4.4.40.1	General	122

4.4.40.2	Creation of new ECS Address Configuration Information Set	122

4.4.40.3	Modification of existing ECS Address Configuration Information Set	122

4.4.40.4	Deletion of existing Individual ECS Address Configuration Information Set	123

4.4.40.5	Deletion of ECS Address Configuration Information based on given criteria	123

4.4.41.1	General	123

4.4.41.2	Procedures for RSLPPI Parameters Provisioning	124

5	NEF Northbound APIs	125

5.1	Introduction	125

5.2	Information applicable to several APIs	128

5.3	Reused APIs	128

5.4	TrafficInfluence API	129

5.4.0	Introduction	129

5.4.1	Resources	130

5.4.1.1	Overview	130

5.4.1.2	Resource: Traffic Influence Subscription	130

5.4.1.2.1	Introduction	130

5.4.1.2.2	Resource Definition	130

5.4.1.2.3	Resource Methods	131

5.4.1.2.3.1	General	131

5.4.1.2.3.2	GET	131

5.4.1.2.3.3	POST	132

5.4.1.3	Resource: Individual Traffic Influence Subscription	132

5.4.1.3.1	Introduction	132

5.4.1.3.2	Resource Definition	132

5.4.1.3.3	Resource Methods	133

5.4.1.3.3.1	General	133

5.4.1.3.3.2	GET	133

5.4.1.3.3.3	PUT	134

5.4.1.3.3.4	PATCH	134

5.4.1.3.3.5	DELETE	135

5.4.1A	Custom Operations without associated resources	136

5.4.2	Notifications	136

5.4.2.1	Introduction	136

5.4.2.2	Event Notification	137

5.4.2.2.1	Description	137

5.4.2.2.2	Target URI	137

5.4.2.2.3	Operation Definition	137

5.4.2.3	Acknowledgement of event notification	138

5.4.2.3.1	Description	138

5.4.2.3.2	Target URI	138

5.4.2.3.3	Operation Definition	139

5.4.2.3.3.1	Notification via HTTP POST	139

5.4.3	Data Model	139

5.4.3.1	General	139

5.4.3.2	Reused data types	140

5.4.3.3	Structured data types	141

5.4.3.3.1	Introduction	141

5.4.3.3.2	Type: TrafficInfluSub	141

5.4.3.3.3	Type: TrafficInfluSubPatch	148

5.4.3.3.4	Type: EventNotification	153

5.4.3.3.5	Type: AfResultInfo	156

5.4.3.3.6	Type AfAckInfo	156

5.4.3.3.7	Type TrafficDataSet	156

5.4.3.3.8	Type TrafficDataSetRm	157

5.4.3.4	Simple data types and enumerations	157

5.4.3.4.1	Introduction	157

5.4.3.4.2	Simple data types	157

5.4.3.4.3	Enumeration: SubscribedEvent	157

5.4.3.4.4	Enumeration: AfResultStatus	157

5.4.4	Used Features	158

5.4.5	Error handling	158

5.4.5.1	General	158

5.4.5.2	Protocol Errors	159

5.4.5.3	Application Errors	159

5.5	NiddConfigurationTrigger API	159

5.5.0	Introduction	159

5.5.1	Resources	159

5.5.1A	Custom Operations without associated resources	159

5.5.2	Notifications	159

5.5.2.1	Introduction	159

5.5.2.2	Event Notification	160

5.5.2.3	Operation Definition	160

5.5.2.3.1	Notification via HTTP POST	160

5.5.2.3.2	Notification via Websocket	161

5.5.3	Data Model	161

5.5.3.1	General	161

5.5.3.2	Reused data types	161

5.5.3.3	Structured data types	161

5.5.3.3.1	Introduction	161

5.5.3.3.2	Type: NiddConfigurationTrigger	161

5.5.3.3.3	Type: NiddConfigurationTriggerReply	161

5.5.3.4	Simple data types and enumerations	162

5.5.3.4.1	Introduction	162

5.5.3.4.2	Simple data types	162

5.5.4	Used Features	162

5.5.5	Error handling	162

5.5.5.1	General	162

5.5.5.2	Protocol Errors	162

5.5.5.3	Application Errors	162

5.6	AnalyticsExposure API	163

5.6.1	Resources	163

5.6.0	Introduction	163

5.6.1.1	Overview	163

5.6.1.2	Resource: Analytics Exposure Subscriptions	164

5.6.1.2.1	Introduction	164

5.6.1.2.2	Resource Definition	164

5.6.1.2.3	Resource Methods	164

5.6.1.2.3.1	General	164

5.6.1.2.3.2	GET	164

5.6.1.2.3.3	POST	165

5.6.1.3	Resource: Individual Analytics Exposure Subscription	166

5.6.1.3.1	Introduction	166

5.6.1.3.2	Resource Definition	166

5.6.1.3.3	Resource Methods	166

5.6.1.3.3.1	General	166

5.6.1.3.3.2	GET	166

5.6.1.3.3.3	PUT	167

5.6.1.3.3.4	DELETE	168

5.6.1A	Custom Operations without associated resources	169

5.6.1A.1	Overview	169

5.6.1A.2	Operation: fetch	170

5.6.1A.2.1	Description	170

5.6.1A.2.2	Operation Definition	170

5.6.2	Notifications	171

5.6.2.1	Introduction	171

5.6.2.2	Event Notification	171

5.6.2.3	Operation Definition	171

5.6.2.3.1	Notification via HTTP POST	171

5.6.2.3.2	Notification via Websocket	172

5.6.3	Data Model	172

5.6.3.1	General	172

5.6.3.2	Reused data types	173

5.6.3.3	Structured data types	177

5.6.3.3.1	Introduction	177

5.6.3.3.2	Type: AnalyticsExposureSubsc	177

5.6.3.3.3	Type: AnalyticsEventNotification	179

5.6.3.3.4	Type: AnalyticsEventNotif	180

5.6.3.3.5	Type: AnalyticsEventSubsc	184

5.6.3.3.6	Type: AnalyticsEventFilterSubsc	185

5.6.3.3.7	Type TargetUeId	191

5.6.3.3.8	Void	192

5.6.3.3.9	Type UeMobilityExposure	192

5.6.3.3.10	Type UeLocationInfo	193

5.6.3.3.11	Void	193

5.6.3.3.12	Type: AnalyticsRequest	193

5.6.3.3.13	Type AnalyticsEventFilter	194

5.6.3.3.14	Type AnalyticsData	199

5.6.3.3.15	Type AbnormalExposure	202

5.6.3.3.16	Type CongestInfo	202

5.6.3.3.17	Type CongestionAnalytics	203

5.6.3.3.18	Type QosSustainabilityExposure	204

5.6.3.3.19	Type NetworkPerfExposure	205

5.6.3.3.20	Type AnalyticsFailureEventInfo	205

5.6.3.3.21	Type WlanPerformInfo	206

5.6.3.4	Simple data types and enumerations	206

5.6.3.4.1	Introduction	206

5.6.3.4.2	Simple data types	206

5.6.3.4.3	Enumeration: AnalyticsEvent	206

5.6.3.4.4	Enumeration: AnalyticsFailureCode	207

5.6.4	Used Features	208

5.6.5	Error handling	211

5.6.5.1	General	211

5.6.5.2	Protocol Errors	211

5.6.5.3	Application Errors	212

5.7	5GLANParameterProvision API	212

5.7.0	Introduction	212

5.7.1	Resources	212

5.7.1.1	Overview	212

5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions	213

5.7.1.2.1	Introduction	213

5.7.1.2.2	Resource Definition	213

5.7.1.2.3	Resource Methods	213

5.7.1.2.3.1	General	213

5.7.1.2.3.2	GET	214

5.7.1.2.3.3	POST	214

5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription	215

5.7.1.3.1	Introduction	215

5.7.1.3.2	Resource Definition	215

5.7.1.3.3	Resource Methods	215

5.7.1.3.3.1	General	215

5.7.1.3.3.2	GET	215

5.7.1.3.3.3	PUT	216

5.7.1.3.3.4	DELETE	217

5.7.1.3.3.5	PATCH	218

5.7.1A	Custom Operations without associated resources	219

5.7.1B	Notifications	219

5.7.1B.1	General	219

5.7.1B.2	5G LAN Parameter Provisioning Event Notification	219

5.7.1B.2.1	Description	219

5.7.1B.2.2	Target URI	219

5.7.1B.2.3	Operation Definition	220

5.7.1B.2.3.1	Notification via HTTP POST	220

5.7.1B.2.3.2	Void	220

5.7.2	Data Model	220

5.7.2.1	General	220

5.7.2.2	Reused data types	221

5.7.2.3	Structured data types	222

5.7.2.3.1	Introduction	222

5.7.2.3.2	Type: 5GLanParametersProvision	223

5.7.2.3.3	Type: 5GLanParameters	223

5.7.2.3.4	Type: AppDescriptor	226

5.7.2.3.5	Type: 5GLanParametersProvisionPatch	227

5.7.2.3.6	Type: 5GLanParametersPatch	227

5.7.2.3.7	Type: AppDescriptorRm	228

5.7.2.3.8	Void	228

5.7.2.3.9	Type: CpParams	228

5.7.2.3.10	Type: NpConfigParams	229

5.7.2.3.11	Type: LpiParams	229

5.7.2.3.12	Type: AcsParams	229

5.7.2.3.13	Type: ECSAddrParams	229

5.7.2.3.14	Type: DnnSnssaiParams	230

5.7.2.3.15	Type: 5GLanParamProvNotif	230

5.7.2.3.16	Type: NpConfigNotif	230

5.7.2.3.17	Type: MaxGrpDataRateInfo	230

5.7.2.4	Simple data types and enumerations	231

5.7.2.4.1	Introduction	231

5.7.2.4.2	Simple data types	231

5.7.2.4.3	Enumeration: AaaUsage	231

5.7.3	Used Features	231

5.7.4	Error handling	231

5.7.4.1	General	231

5.7.4.2	Protocol Errors	232

5.7.4.3	Application Errors	232

5.8	ApplyingBdtPolicy API	232

5.8.0	Introduction	232

5.8.1	Resources	232

5.8.1.1	Overview	232

5.8.1.2	Resource: Applied BDT Policy Subscriptions	233

5.8.1.2.1	Introduction	233

5.8.1.2.2	Resource Definition	233

5.8.1.2.3	Resource Methods	233

5.8.1.2.3.1	General	233

5.8.1.2.3.2	GET	233

5.8.1.2.3.3	POST	234

5.8.1.3	Resource: Individual Applied BDT Policy Subscription	235

5.8.1.3.1	Introduction	235

5.8.1.3.2	Resource Definition	235

5.8.1.3.3	Resource Methods	235

5.8.1.3.3.1	General	235

5.8.1.3.3.2	GET	235

5.8.1.3.3.3	PATCH	236

5.8.1.3.3.4	DELETE	237

5.8.1A	Custom Operations without associated resources	238

5.8.2	Notifications	238

5.8.3	Data Model	238

5.8.3.1	General	238

5.8.3.2	Reused data types	238

5.8.3.3	Structured data types	239

5.8.3.3.1	Introduction	239

5.8.3.3.2	Type: AppliedBdtPolicy	239

5.8.3.3.3	Type: AppliedBdtPolicyPatch	239

5.8.3.4	Simple data types and enumerations	239

5.8.3.4.1	Introduction	239

5.8.3.4.2	Simple data types	240

5.8.4	Used Features	240

5.8.5	Error handling	240

5.8.5.1	General	240

5.8.5.2	Protocol Errors	240

5.8.5.3	Application Errors	240

5.9	IPTVConfiguration API	240

5.9.0	Introduction	240

5.9.1	Resources	241

5.9.1.1	Overview	241

5.9.1.2	Resource: IPTV Configurations	241

5.9.1.2.1	Introduction	241

5.9.1.2.2	Resource Definition	241

5.9.1.2.3	Resource Methods	242

5.9.1.2.3.1	General	242

5.9.1.2.3.2	GET	242

5.9.1.2.3.3	POST	243

5.9.1.3	Resource: Individual IPTV Configuration	243

5.9.1.3.1	Introduction	243

5.9.1.3.2	Resource Definition	243

5.9.1.3.3	Resource Methods	244

5.9.1.3.3.1	General	244

5.9.1.3.3.2	GET	244

5.9.1.3.3.3	PUT	245

5.9.1.3.3.4	DELETE	245

5.9.1.3.3.5	PATCH	246

5.9.1A	Custom Operations without associated resources	247

5.9.1B	Notifications	247

5.9.2	Data Model	247

5.9.2.1	General	247

5.9.2.2	Reused data types	248

5.9.2.3	Structured data types	248

5.9.2.3.1	Introduction	248

5.9.2.3.2	Type: IptvConfigData	249

5.9.2.3.3	Type: MulticastAccessControl	249

5.9.2.3.4	Type: IptvConfigDataPatch	250

5.9.2.4	Simple data types and enumerations	250

5.9.2.4.1	Introduction	250

5.9.2.4.2	Simple data types	250

5.9.2.4.3	Enumeration: AccessRightStatus	250

5.9.3	Used Features	250

5.9.4	Error handling	250

5.9.4.1	General	250

5.9.4.2	Protocol Errors	251

5.9.4.3	Application Errors	251

5.10	LpiParameterProvision API	251

5.10.0	Introduction	251

5.10.1	Resources	251

5.10.1.1	Overview	251

5.10.1.2	Resource: LPI Parameters Provisionings	252

5.10.1.2.1	Introduction	252

5.10.1.2.2	Resource Definition	252

5.10.1.2.3	Resource Methods	252

5.10.1.2.3.1	General	252

5.10.1.2.3.2	GET	252

5.10.1.2.3.3	POST	253

5.10.1.3	Resource: Individual LPI Parameters Provisioning	254

5.10.1.3.1	Introduction	254

5.10.1.3.2	Resource Definition	254

5.10.1.3.3	Resource Methods	254

5.10.1.3.3.1	General	254

5.10.1.3.3.2	GET	254

5.10.1.3.3.3	PUT	255

5.10.1.3.3.3A	PATCH	256

5.10.1.3.3.4	DELETE	257

5.10.1A	Custom Operations without associated resources	258

5.10.1B	Notifications	258

5.10.2	Data Model	258

5.10.2.1	General	258

5.10.2.2	Reused data types	258

5.10.2.3	Structured data types	259

5.10.2.3.1	Introduction	259

5.10.2.3.2	Type: LpiParametersProvision	259

5.10.2.3.3	Type: LpiParametersProvisionPatch	260

5.10.2.4	Simple data types and enumerations	260

5.10.2.4.1	Introduction	260

5.10.2.4.2	Simple data types	260

5.10.3	Used Features	260

5.10.4	Error handling	260

5.10.4.1	General	260

5.10.4.2	Protocol Errors	260

5.10.4.3	Application Errors	261

5.11	ServiceParameter API	261

5.11.0	Introduction	261

5.11.1	Resources	261

5.11.1.1	Overview	261

5.11.1.2	Resource: Service Parameter Subscriptions	262

5.11.1.2.1	Introduction	262

5.11.1.2.2	Resource Definition	262

5.11.1.2.3	Resource Methods	262

5.11.1.2.3.1	General	262

5.11.1.2.3.2	GET	262

5.11.1.2.3.3	POST	264

5.11.1.3	Resource: Individual Service Parameter Subscription	264

5.11.1.3.1	Introduction	264

5.11.1.3.2	Resource Definition	264

5.11.1.3.3	Resource Methods	264

5.11.1.3.3.1	General	264

5.11.1.3.3.2	GET	265

5.11.1.3.3.3	PUT	265

5.11.1.3.3.4	DELETE	266

5.11.1.3.3.5	PATCH	267

5.11.1A	Notifications	268

5.11.1A.1	Introduction	268

5.11.1A.2	AF Notifications	268

5.11.1A.2.1	Description	268

5.11.1A.2.2	Target URI	268

5.11.1A.3	Operation Definition	269

5.11.1A.3.1	Notification via HTTP POST	269

5.11.1A.3.2	Notification via Websocket	270

5.11.1B	Custom Operations without associated resources	270

5.11.2	Data Model	270

5.11.2.1	General	270

5.11.2.2	Reused data types	273

5.11.2.3	Structured data types	273

5.11.2.3.1	Introduction	273

5.11.2.3.2	Type: ServiceParameterData	274

5.11.2.3.3	Type: ServiceParameterDataPatch	277

5.11.2.3.4	Type: UrspRuleRequest	278

5.11.2.3.5	Type: RouteSelectionParameterSet	279

5.11.2.3.6	Type: AfNotification	279

5.11.2.3.7	Type: EventInfo	280

5.11.2.3.8	Type: TrafficDescriptorComponents	281

5.11.2.3.9	Type: NetworkDescription	282

5.11.2.3.10	Void	282

5.11.2.3.11	Void	282

5.11.2.4	Simple data types and enumerations	282

5.11.2.4.1	Introduction	282

5.11.2.4.2	Simple data types	282

5.11.2.4.3	Enumeration: Event	285

5.11.2.4.4	Enumeration: AuthorizationResult	285

5.11.2.4.5	Enumeration: Failure	285

5.11.2.4.6	Enumeration: ConnectionCapabilities	285

5.11.3	Used Features	286

5.11.4	Error handling	287

5.11.4.1	General	287

5.11.4.2	Protocol Errors	287

5.11.4.3	Application Errors	287

5.12	ACSParameterProvision API	288

5.12.0	Introduction	288

5.12.1	Resources	288

5.12.1.1	Overview	288

5.12.1.2	Resource: ACS Configuration Subscriptions	289

5.12.1.2.1	Introduction	289

5.12.1.2.2	Resource Definition	289

5.12.1.2.3	Resource Methods	289

5.12.1.2.3.1	General	289

5.12.1.2.3.2	GET	289

5.12.1.2.3.3	POST	290

5.12.1.3	Resource: Individual ACS Configuration Subscription	291

5.12.1.3.1	Introduction	291

5.12.1.3.2	Resource Definition	291

5.12.1.3.3	Resource Methods	291

5.12.1.3.3.1	General	291

5.12.1.3.3.2	GET	291

5.12.1.3.3.3	PUT	292

5.12.1.3.3.3A	PATCH	293

5.12.1.3.3.4	DELETE	294

5.12.1A	Custom Operations without associated resources	295

5.12.1B	Notifications	295

5.12.2	Data Model	295

5.12.2.1	General	295

5.12.2.2	Reused data types	295

5.12.2.3	Structured data types	295

5.12.2.3.1	Introduction	295

5.12.2.3.2	Type: AcsConfigurationData	296

5.12.2.3.3	Type: AcsConfigurationDataPatch	296

5.12.2.4	Simple data types and enumerations	296

5.12.2.4.1	Introduction	296

5.12.2.4.2	Simple data types	296

5.12.3	Used Features	297

5.12.4	Error handling	297

5.12.4.1	General	297

5.12.4.2	Protocol Errors	297

5.12.4.3	Application Errors	297

5.13	MoLcsNotify API	297

5.13.0	Introduction	297

5.13.1	Resources	297

5.13.1A	Custom Operations without associated resources	298

5.13.2	Notifications	298

5.13.2.1	Introduction	298

5.13.2.2	Event Notification	298

5.13.2.3	Operation Definition	298

5.13.2.3.1	Notification via HTTP POST	298

5.13.3	Data Model	299

5.13.3.1	General	299

5.13.3.2	Reused data types	299

5.13.3.3	Structured data types	299

5.13.3.3.1	Introduction	299

5.13.3.3.2	Type: LocUpdateData	299

5.13.3.3.3	Type: LocUpdateDataReply	300

5.13.3.4	Simple data types and enumerations	300

5.13.3.4.1	Introduction	300

5.13.3.4.2	Simple data types	300

5.13.4	Used Features	300

5.13.5	Error handling	301

5.13.5.1	General	301

5.13.5.2	Protocol Errors	301

5.13.5.3	Application Errors	301

5.14	AKMA API	301

5.14.1	Introduction	301

5.14.2	Resources	301

5.14.3	Custom Operations without associated resources	301

5.14.3.1	Overview	301

5.14.3.2	Operation: Retrieve	302

5.14.3.2.1	Description	302

5.14.3.2.2	Operation Definition	302

5.14.4	Notifications	303

5.14.4.1	General	303

5.14.4.2	AKMA Service Disablement Notification	303

5.14.4.2.1	Description	303

5.14.4.2.2	Target URI	303

5.14.4.2.3	Operation Definition	303

5.14.5	Data Model	304

5.14.5.1	General	304

5.14.5.2	Reused data types	305

5.14.5.3	Structured data types	305

5.14.5.3.1	Introduction	305

5.14.5.3.2	Type: AkmaAfKeyRequest	305

5.14.5.3.3	Type: AkmaAfKeyData	306

5.14.5.3.4	Type: ServiceDisableNotif	306

5.14.5.4	Simple data types and enumerations	306

5.14.5.4.1	Introduction	306

5.14.5.4.2	Simple data types	306

5.14.6	Used Features	307

5.14.7	Error handling	307

5.14.7.1	General	307

5.14.7.2	Protocol Errors	307

5.14.7.3	Application Errors	307

5.15	TimeSyncExposure API	308

5.15.0	Introduction	308

5.15.1	Resources	308

5.15.1.1	Overview	308

5.15.1.2	Resource: Time Synchronization Exposure Subscriptions	309

5.15.1.2.1	Introduction	309

5.15.1.2.2	Resource Definition	310

5.15.1.2.3	Resource Methods	310

5.15.1.2.3.1	General	310

5.15.1.2.3.2	GET	310

5.15.1.2.3.3	POST	311

5.15.1.3	Resource: Individual Time Synchronization Exposure Subscription	312

5.15.1.3.1	Introduction	312

5.15.1.3.2	Resource Definition	312

5.15.1.3.3	Resource Methods	312

5.15.1.3.3.1	General	312

5.15.1.3.3.2	GET	312

5.15.1.3.3.3	PUT	313

5.15.1.3.3.4	DELETE	314

5.15.1.4	Resource: Time Synchronization Exposure Configurations	315

5.15.1.4.1	Introduction	315

5.15.1.4.2	Resource Definition	315

5.15.1.4.3	Resource Methods	316

5.15.1.4.3.1	General	316

5.15.1.4.3.2	GET	316

5.15.1.4.3.3	POST	317

5.15.1.5	Resource: Individual Time Synchronization Exposure Configuration	317

5.15.1.5.1	Introduction	317

5.15.1.5.2	Resource Definition	317

5.15.1.5.3	Resource Methods	318

5.15.1.5.3.1	General	318

5.15.1.5.3.2	GET	318

5.15.1.5.3.3	PUT	319

5.15.1.5.3.4	DELETE	320

5.15.2	Custom Operations without associated resources	321

5.15.3	Notifications	321

5.15.3.1	Introduction	321

5.15.3.2	Time Synchronization Capability Notification	321

5.15.3.2.1	Description	321

5.15.3.2.2	Callback URI	321

5.15.3.2.3	Operation Definition	321

5.15.3.2.3.1	Notification via HTTP POST	321

5.15.3.2.3.2	Notification via Websocket	322

5.15.3.3	Time Synchronization Configuration Notification	322

5.15.3.3.1	Description	322

5.15.3.3.2	Callback URI	322

5.15.3.3.3	Operation Definition	323

5.15.3.3.3.1	Notification via HTTP POST	323

5.15.3.3.3.2	Void	324

5.15.4	Data Model	324

5.15.4.1	General	324

5.15.4.2	Reused data types	324

5.15.4.3	Structured data types	325

5.15.4.3.1	Introduction	325

5.15.4.3.2	Type: TimeSyncExposureSubsc	326

5.15.4.3.3	Type: TimeSyncCapability	328

5.15.4.3.4	Void	329

5.15.4.3.5	Void	329

5.15.4.3.6	Type: TimeSyncExposureConfig	329

5.15.4.3.7	Type: TimeSyncExposureSubsNotif	331

5.15.4.3.8	Type SubsEventNotification	331

5.15.4.3.9	Type: TimeSyncExposureConfigNotif	331

5.15.4.3.10	Type: EventFilter	331

5.15.4.3.11	Type: PtpCapabilitiesPerUe	332

5.15.4.3.12	Type: PtpInstance	332

5.15.4.3.13	Void	333

5.15.4.3.14	Void	333

5.15.4.3.15	Void	333

5.15.4.3.16	Void	333

5.15.4.3.17	Type: StateOfConfiguration	333

5.15.4.3.18	Type: ConfigForPort	334

5.15.4.3.19	Type: StateOfDstt	336

5.15.4.3.20	Void	337

5.15.4.3.21	Void	337

5.15.4.3.22	Type: AvailReport	337

5.15.4.4	Simple data types and enumerations	337

5.15.4.4.1	Introduction	337

5.15.4.4.2	Simple data types	337

5.15.4.4.3	Void	337

5.15.4.4.4	Enumeration: Protocol	337

5.15.4.4.5	Enumeration: GmCapable	337

5.15.4.4.6	Enumeration: SubscribedEvent	338

5.15.4.4.7	Enumeration: InstanceType	338

5.15.4.4.8	Enumeration: AsTimeResource	338

5.15.4.4.9	Enumeration: AcceptanceCriteriaResultIndication	338

5.15.4.4.10	Enumeration: AvailStatus	339

5.15.4.5	Data types describing alternative data types or combinations of data types	339

5.15.4.5.1	Type: ReportedCabability	339

5.15.5	Used Features	339

5.15.6	Error handling	339

5.15.6.1	General	339

5.15.6.2	Protocol Errors	339

5.15.6.3	Application Errors	340

5.16	EcsAddressProvision API	340

5.16.0	Introduction	340

5.16.1	Resources	340

5.16.1.1	Overview	340

5.16.1.2	Resource: ECS Address Provision Configurations	341

5.16.1.2.1	Introduction	341

5.16.1.2.2	Resource Definition	341

5.16.1.2.3	Resource Methods	341

5.16.1.2.3.1	General	341

5.16.1.2.3.2	GET	341

5.16.1.2.3.3	POST	342

5.16.1.3	Resource: Individual ECS Address Provision Configuration	343

5.16.1.3.1	Introduction	343

5.16.1.3.2	Resource Definition	343

5.16.1.3.3	Resource Methods	343

5.16.1.3.3.1	General	343

5.16.1.3.3.2	GET	343

5.16.1.3.3.3	PUT	344

5.16.1.3.3.4	DELETE	345

5.16.1A	Custom Operations without associated resources	346

5.16.1B	Notifications	346

5.16.2	Data Model	346

5.16.2.1	General	346

5.16.2.2	Reused data types	346

5.16.2.3	Structured data types	347

5.16.2.3.1	Introduction	347

5.16.2.3.2	Type: EcsAddressProvision	347

5.16.2.4	Simple data types and enumerations	348

5.16.2.4.1	Introduction	348

5.16.2.4.2	Simple data types	348

5.16.3	Used Features	348

5.16.4	Error handling	348

5.16.4.1	General	348

5.16.4.2	Protocol Errors	348

5.16.4.3	Application Errors	348

5.17	AMPolicyAuthorization API	349

5.17.0	Introduction	349

5.17.1	Resources	349

5.17.1.1	Overview	349

5.17.1.2	Resource: Application AM Contexts	350

5.17.1.2.1	Introduction	350

5.17.1.2.2	Resource Definition	350

5.17.1.2.3	Resource Methods	350

5.17.1.2.3.1	General	350

5.17.1.2.3.2	POST	350

5.17.1.3	Resource: Individual Application AM Context	351

5.17.1.3.1	Introduction	351

5.17.1.3.2	Resource Definition	351

5.17.1.3.3	Resource Methods	351

5.17.1.3.3.1	General	351

5.17.1.3.3.2	GET	351

5.17.1.3.3.3	PATCH	352

5.17.1.3.3.4	DELETE	354

5.17.1.4	Resource: AM Policy Events Subscription	355

5.17.1.4.1	Introduction	355

5.17.1.4.2	Resource Definition	355

5.17.1.4.3	Resource Methods	355

5.17.1.4.3.1	General	355

5.17.1.4.3.2	PUT	355

5.17.1.4.3.3	DELETE	357

5.17.1A	Custom Operations without associated resources	357

5.17.2	Notifications	358

5.17.2.1	Introduction	358

5.17.2.2	AM Event Notification	358

5.17.2.2.1	Description	358

5.17.2.2.2	Callback URI	358

5.17.2.2.3	Operation Definition	358

5.17.2.2.3.1	Notification via HTTP POST	358

5.17.2.2.3.2	Notification via Websocket	359

5.17.3	Data Model	359

5.17.3.1	General	359

5.17.3.2	Reused data types	360

5.17.3.3	Structured data types	360

5.17.3.3.1	Introduction	360

5.17.3.3.2	Type: AppAmContextExpData	361

5.17.3.3.3	Type: AppAmContextExpUpdateData	362

5.17.3.3.4	Type: GeographicalArea	362

5.17.3.4	Simple data types and enumerations	362

5.17.3.4.1	Introduction	362

5.17.3.4.2	Simple data types	362

5.17.3.5	Data types describing alternative data types or combinations of data types	362

5.17.3.5.1	Type: AppAmContextExpRespData	362

5.17.4	Used Features	363

5.17.5	Error handling	363

5.17.5.1	General	363

5.17.5.2	Protocol Errors	363

5.17.5.3	Application Errors	363

5.18	AMInfluence API	363

5.18.0	Introduction	363

5.18.1	Resources	364

5.18.1.1	Overview	364

5.18.1.2	Resource: AM Influence Subscription	364

5.18.1.2.1	Introduction	364

5.18.1.2.2	Resource Definition	365

5.18.1.2.3	Resource Methods	365

5.18.1.2.3.1	General	365

5.18.1.2.3.2	GET	365

5.18.1.2.3.3	POST	366

5.18.1.3	Resource: Individual AM Influence Subscription	366

5.18.1.3.1	Introduction	366

5.18.1.3.2	Resource Definition	366

5.18.1.3.3	Resource Methods	367

5.18.1.3.3.1	General	367

5.18.1.3.3.2	GET	367

5.18.1.3.3.3	PUT	368

5.18.1.3.3.4	PATCH	368

5.18.1.3.3.5	DELETE	369

5.18.1A	Custom Operations without associated resources	370

5.18.2	Notifications	370

5.18.2.1	Introduction	370

5.18.2.2	Event Notification	371

5.18.2.2.1	Description	371

5.18.2.2.2	Target URI	371

5.18.2.2.3	Operation Definition	371

5.18.3	Data Model	372

5.18.3.1	General	372

5.18.3.2	Reused data types	372

5.18.3.3	Structured data types	373

5.18.3.3.1	Introduction	373

5.18.3.3.2	Type: AmInfluSub	373

5.18.3.3.3	Type: AmInfluSubPatch	376

5.18.3.3.4	Type: AmInfluEventNotif	377

5.18.3.3.5	Type: DnnSnssaiInformation	378

5.18.3.4	Simple data types and enumerations	378

5.18.3.4.1	Introduction	378

5.18.3.4.2	Simple data types	378

5.18.3.4.3	Enumeration: AmInfluEvent	378

5.18.4	Used Features	378

5.18.5	Error handling	379

5.18.5.1	General	379

5.18.5.2	Protocol Errors	379

5.18.5.3	Application Errors	379

5.19	MBSTMGI API	379

5.19.1	Introduction	379

5.19.2	Resources	379

5.19.3	Custom Operations without associated resources	379

5.19.3.1	Overview	379

5.19.3.2	Operation: Allocate	380

5.19.3.2.1	Description	380

5.19.3.2.2	Operation Definition	380

5.19.3.3	Operation: Deallocate	381

5.19.3.3.1	Description	381

5.19.3.3.2	Operation Definition	381

5.19.4	Notifications	382

5.19.4.1	General	382

5.19.4.2	Notification of Allocated MBS TMGI(s) Timer Expiry	382

5.19.4.2.1	Description	382

5.19.4.2.2	Target URI	382

5.19.4.2.3	Operation Definition	383

5.19.4.2.3.1	Notification via HTTP POST	383

5.19.4.2.3.2	Notification via Websocket	383

5.19.5	Data Model	384

5.19.5.1	General	384

5.19.5.2	Structured data types	384

5.19.5.2.1	Introduction	384

5.19.5.2.2	Type: TmgiAllocRequest	385

5.19.5.2.3	Type: TmgiAllocResponse	386

5.19.5.2.4	Type: TmgiDeallocRequest	386

5.19.5.2.5	Type: ExpiryNotif	386

5.19.5.2.6	Type: ReducedMbsServArea	387

5.19.5.3	Simple data types and enumerations	387

5.19.5.3.1	Introduction	387

5.19.5.3.2	Simple data types	387

5.19.5.4	Data types describing alternative data types or combinations of data types	388

5.19.5.4.1	Type: ProblemDetailsTmgiAlloc	388

5.19.6	Used Features	388

5.19.7	Error handling	388

5.19.7.1	General	388

5.19.7.2	Protocol Errors	388

5.19.7.3	Application Errors	388

5.20	MBSSession API	389

5.20.1	Introduction	389

5.20.2	Resources	389

5.20.2.1	Overview	389

5.20.2.2	Resource: MBS sessions	391

5.20.2.2.1	Introduction	391

5.20.2.2.2	Resource Definition	391

5.20.2.2.3	Resource Methods	392

5.20.2.2.3.1	POST	392

5.20.2.2.4	Resource Custom Operations	392

5.20.2.3	Resource: Individual MBS Session	392

5.20.2.3.1	Introduction	392

5.20.2.3.2	Resource Definition	392

5.20.2.3.3	Resource Standard Methods	393

5.20.2.3.3.1	PATCH	393

5.20.2.3.3.3	DELETE	394

5.20.2.3.4	Resource Custom Operations	395

5.20.2.4	Resource: MBS Session Subscriptions	395

5.20.2.4.1	Introduction	395

5.20.2.4.2	Resource Definition	395

5.20.2.4.3	Resource Methods	395

5.20.2.4.3.1	GET	395

5.20.2.4.3.2	POST	396

5.20.2.4.4	Resource Custom Operations	397

5.20.2.5	Resource: Individual MBS Session Subscription	397

5.20.2.5.1	Introduction	397

5.20.2.5.2	Resource Definition	397

5.20.2.5.3	Resource Methods	397

5.20.2.5.3.1	GET	397

5.20.2.5.3.2	DELETE	398

5.20.2.5.4	Resource Custom Operations	399

5.20.2.6	Resource: MBS Parameters Provisionings	399

5.20.2.6.1	Introduction	399

5.20.2.6.2	Resource Definition	399

5.20.2.6.3	Resource Methods	400

5.20.2.6.3.1	GET	400

5.20.2.6.3.2	POST	401

5.20.2.6.4	Resource Custom Operations	401

5.20.2.7	Resource: Individual MBS Parameters Provisioning	401

5.20.2.7.1	Introduction	401

5.20.2.7.2	Resource Definition	402

5.20.2.7.3	Resource Methods	402

5.20.2.7.3.1	GET	402

5.20.2.7.3.2	PUT	403

5.20.2.7.3.3	PATCH	404

5.20.2.7.3.4	DELETE	405

5.20.2.7.4	Resource Custom Operations	406

5.20.3	Custom Operations without associated resources	406

5.20.4	Notifications	406

5.20.4.1	General	406

5.20.4.2	MBS Session Status Notification	406

5.20.4.2.1	Description	406

5.20.4.2.2	Target URI	406

5.20.4.2.3	Operation Definition	407

5.20.4.2.3.1	Notification via HTTP POST	407

5.20.5	Data Model	407

5.20.5.1	General	407

5.20.5.2	Structured data types	408

5.20.5.2.1	Introduction	408

5.20.5.2.2	Type: MbsSessionCreateReq	409

5.20.5.2.3	Type: MbsSessionCreateRsp	409

5.20.5.2.4	Type: MbsSessionSubsc	409

5.20.5.2.5	Type: MbsSessionStatusNotif	409

5.20.5.2.6	Type: MbsPpData	410

5.20.5.2.7	Type: MbsSessAuthData	410

5.20.5.2.8	Type: MbsPpDataPatch	411

5.20.5.2.9	Type: MbsSessAssistInfo	411

5.20.5.2.10	Type: MbsSessionUpdateResp	411

5.20.5.3	Simple data types and enumerations	411

5.20.5.3.1	Introduction	411

5.20.5.3.2	Simple data types	412

5.20.6	Used Features	412

5.20.7	Error handling	412

5.20.7.1	General	412

5.20.7.2	Protocol Errors	412

5.20.7.3	Application Errors	412

5.21	EASDeployment API	413

5.21.0	Introduction	413

5.21.1	Resources	413

5.21.1.1	Overview	413

5.21.1.2	Resource: EAS Deployment Information	414

5.21.1.2.1	Introduction	414

5.21.1.2.2	Resource Definition	414

5.21.1.2.3	Resource Methods	415

5.21.1.2.3.1	General	415

5.21.1.2.3.2	GET	415

5.21.1.2.3.3	POST	416

5.21.1.3	Resource: Individual EAS Deployment Information	416

5.21.1.3.1	Introduction	416

5.21.1.3.2	Resource Definition	416

5.21.1.3.3	Resource Methods	416

5.21.1.3.3.1	General	416

5.21.1.3.3.2	GET	417

5.21.1.3.3.3	PUT	417

5.21.1.3.3.4	DELETE	418

5.21.2	Custom Operations without associated resources	419

5.21.2.1	Overview	419

5.21.2.2	Operation: remove-edis	420

5.21.2.2.1	Description	420

5.21.2.2.2	Operation Definition	420

5.21.3	Notifications	420

5.21.4	Data Model	420

5.21.4.1	General	420

5.21.4.2	Reused data types	421

5.21.4.3	Structured data types	421

5.21.4.3.1	Introduction	421

5.21.4.3.2	Type: EasDeployInfo	422

5.21.4.3.3	Type: DnaiInformation	422

5.21.4.3.4	Type: DnsServerIdentifier	423

5.21.4.3.5	Type: EdiDeleteCriteria	423

5.21.4.4	Simple data types and enumerations	423

5.21.4.4.1	Introduction	423

5.21.4.4.2	Simple data types	423

5.21.5	Used Features	423

5.21.6	Error handling	423

5.21.6.1	General	423

5.21.6.2	Protocol Errors	424

5.21.6.3	Application Errors	424

5.22	ASTI API	424

5.22.0	Introduction	424

5.22.1	Resources	424

5.22.1.1	Overview	424

5.22.1.2	Resource: ASTI Configurations	425

5.22.1.2.1	Introduction	425

5.22.1.2.2	Resource Definition	425

5.22.1.2.3	Resource Methods	426

5.22.1.2.3.1	General	426

5.22.1.2.3.2	GET	426

5.22.1.2.3.3	POST	427

5.22.1.2.4	Resource Custom Operations	427

5.22.1.2.4.1	Overview	427

5.22.1.2.4.2	Operation: retrieve	427

5.22.1.2.4.2.1	Description	427

5.22.1.2.4.2.2	Operation Definition	428

5.22.1.3	Resource: Individual ASTI Configuration	428

5.22.1.3.1	Introduction	428

5.22.1.3.2	Resource Definition	428

5.22.1.3.3	Resource Methods	428

5.22.1.3.3.1	General	428

5.22.1.3.3.2	GET	428

5.22.1.3.3.3	PUT	429

5.22.1.3.3.4	DELETE	430

5.22.2	Custom Operations without associated resources	431

5.22.3	Notifications	431

5.22.3.1	General	431

5.22.3.2	ASTI Notification	431

5.22.3.2.1	Description	431

5.22.3.2.2	Target URI	431

5.22.3.2.3	Operation Definition	432

5.22.3.2.3.1	Notification via HTTP POST	432

5.22.3.2.3.2	Void	433

5.22.4	Data Model	433

5.22.4.1	General	433

5.22.4.2	Reused data types	433

5.22.4.3	Structured data types	433

5.22.4.3.1	Introduction	433

5.22.4.3.2	Type: AccessTimeDistributionData	434

5.22.4.3.3	Type: StatusRequestData	434

5.22.4.3.4	Type: StatusResponseData	435

5.22.4.3.5	Type: ActiveUe	435

5.22.4.3.6	Type AstiConfigNotification	435

5.22.4.3.7	Type AstiConfigStateNotification	435

5.22.4.4	Simple data types and enumerations	435

5.22.4.4.1	Introduction	435

5.22.4.4.2	Simple data types	436

5.22.5	Used Features	436

5.22.6	Error handling	436

5.22.6.1	General	436

5.22.6.2	Protocol Errors	436

5.22.6.3	Application Errors	436

5.23	DataReporting API	437

5.23.1	Introduction	437

5.23.2	Resources	437

5.23.2.1	Overview	437

5.23.2.2	Resource: Data Reporting Sessions	438

5.23.2.2.1	Introduction	438

5.23.2.2.2	Resource definition	438

5.23.2.2.3	Resource Methods	438

5.23.2.2.3.1	POST	438

5.23.2.3	Resource: Individual Data Reporting Session	439

5.23.2.3.1	Introduction	439

5.23.2.3.2	Resource Definition	439

5.23.2.3.3	Resource standard methods	439

5.23.2.3.3.1	GET	439

5.23.2.3.3.2	PUT	440

5.23.2.3.3.3	DELETE	442

5.23.2.3.4	Resource custom operations	443

5.23.3	Custom Operations without associated resources	444

5.23.4	Notifications	444

5.23.5	Data Model	444

5.23.3.1	General	444

5.23.6	Used Features	444

5.23.7	Error handling	444

5.23.7.1	General	444

5.23.7.2	Protocol Errors	444

5.23.7.3	Application Errors	444

5.24	DataReportingProvisioning API	445

5.24.1	Introduction	445

5.24.2	Resources	445

5.24.2.1	Overview	445

5.24.2.2	Resource: Data Reporting Provisioning Sessions	447

5.24.2.2.1	Introduction	447

5.24.2.2.2	Resource definition	448

5.24.2.2.3	Resource Methods	448

5.24.2.2.3.1	POST	448

5.24.2.3	Resource: Individual Data Reporting Provisioning Session	449

5.24.2.3.1	Introduction	449

5.24.2.3.2	Resource Definition	449

5.24.2.3.3	Resource standard methods	449

5.24.2.3.3.1	GET	449

5.24.2.3.3.2	Void	450

5.24.2.3.3.3	DELETE	450

5.24.2.4	Resource: Data Reporting Configurations	451

5.24.2.4.1	Introduction	451

5.24.2.4.2	Resource definition	452

5.24.2.4.3	Resource Methods	452

5.24.2.4.3.1	POST	452

5.24.2.5	Resource: Individual Data Reporting Configuration	453

5.24.2.5.1	Introduction	453

5.24.2.5.2	Resource Definition	453

5.24.2.5.3	Resource standard methods	453

5.24.2.5.3.2	GET	453

5.24.2.5.3.3	PUT	454

5.24.2.5.3.3A	PATCH	456

5.24.2.5.3.4	DELETE	457

5.24.3	Custom Operations without associated resources	457

5.24.4	Notifications	458

5.24.5	Data Model	458

5.24.5.1	General	458

5.24.6	Used Features	458

5.24.7	Error handling	458

5.24.7.1	General	458

5.24.7.2	Protocol Errors	458

5.24.7.3	Application Errors	459

5.25	UEId API	459

5.25.1	Introduction	459

5.25.2	Resources	459

5.25.2.1	Overview	459

5.25.2.2	Resource: UE ID Mapping Information Provisionings	460

5.25.2.2.1	Introduction	460

5.25.2.2.2	Resource Definition	460

5.25.2.2.3	Resource Methods	460

5.25.2.2.3.1	GET	460

5.25.2.2.3.2	POST	461

5.25.2.2.4	Resource Custom Operations	462

5.25.2.3	Resource: Individual UE ID Mapping Provisioning	462

5.25.2.3.1	Introduction	462

5.25.2.3.2	Resource Definition	462

5.25.2.3.3	Resource Methods	463

5.25.2.3.3.1	GET	463

5.25.2.3.3.2	PUT	464

5.25.2.3.3.3	PATCH	465

5.25.2.3.3.4	DELETE	466

5.25.3	Custom Operations without associated resources	467

5.25.3.1	Overview	467

5.25.3.2	Operation: Retrieve	468

5.25.3.2.1	Description	468

5.25.3.2.2	Operation Definition	468

5.25.3.2.3	Operation Definition	469

5.25.4	Notifications	470

5.25.5	Data Model	470

5.25.5.1	General	470

5.25.5.2	Structured data types	471

5.25.5.2.1	Introduction	471

5.25.5.2.2	Type: UeIdReq	472

5.25.5.2.3	Type: UeIdInfo	472

5.25.5.2.4	Type: MsisdnReq	473

5.25.5.2.5	Type: MsisdnInfo	473

5.25.5.2.6	Type: UeIdMappingInfo	474

5.25.5.2.7	Type: RangSlUeIdMappInfo	474

5.25.5.2.8	Type: UeIdMappingInfoPatch	474

5.25.5.3	Simple data types and enumerations	474

5.25.5.3.1	Introduction	474

5.25.5.3.2	Simple data types	474

5.25.6	Used Features	475

5.25.7	Error handling	475

5.25.7.1	General	475

5.25.7.2	Protocol Errors	475

5.25.7.3	Application Errors	475

5.25.8	Security	475

5.26	MBSUserService API	475

5.26.1	Introduction	475

5.26.2	Resources	476

5.26.2.1	Overview	476

5.26.2.2	Resource: MBS User Services	477

5.26.2.2.1	Introduction	477

5.26.2.2.2	Resource Definition	477

5.26.2.2.3	Resource Standard Methods	477

5.26.2.2.3.1	GET	477

5.26.2.2.3.2	POST	478

5.26.2.2.4	Resource Custom Operations	478

5.26.2.3	Resource: Individual MBS User Service	479

5.26.2.3.1	Introduction	479

5.26.2.3.2	Resource Definition	479

5.26.2.3.3	Resource Standard Methods	479

5.26.2.3.3.1	GET	479

5.26.2.3.3.2	PUT	480

5.26.2.3.3.3	PATCH	481

5.26.2.3.3.4	DELETE	482

5.26.2.3.4	Resource Custom Operations	483

5.26.3	Custom Operations without associated resources	483

5.26.4	Notifications	483

5.26.5	Data Model	483

5.26.5.1	General	483

5.26.5.2	Structured data types	484

5.26.5.2.1	Introduction	484

5.26.5.3	Simple data types and enumerations	484

5.26.5.3.1	Introduction	484

5.26.5.3.2	Simple data types	484

5.26.6	Used Features	484

5.26.7	Error handling	484

5.26.7.1	General	484

5.26.7.2	Protocol Errors	485

5.26.7.3	Application Errors	485

5.27	MBSUserDataIngestSession API	485

5.27.1	Introduction	485

5.27.2	Resources	485

5.27.2.1	Overview	485

5.27.2.2	Resource: MBS User Data Ingest Sessions	487

5.27.2.2.1	Introduction	487

5.27.2.2.2	Resource Definition	487

5.27.2.2.3	Resource Standard Methods	487

5.27.2.2.3.1	GET	487

5.27.2.2.3.2	POST	488

5.27.2.2.4	Resource Custom Operations	489

5.27.2.3	Resource: Individual MBS User Data Ingest Session	489

5.27.2.3.1	Introduction	489

5.27.2.3.2	Resource Definition	489

5.27.2.3.3	Resource Standard Methods	489

5.27.2.3.3.1	GET	489

5.27.2.3.3.2	PUT	490

5.27.2.3.3.3	PATCH	491

5.27.2.3.3.4	DELETE	493

5.27.2.3.4	Resource Custom Operations	493

5.27.2.4	Resource: MBS User Data Ingest Session Status Subscriptions	494

5.27.2.4.1	Introduction	494

5.27.2.4.2	Resource Definition	494

5.27.2.4.3	Resource Standard Methods	494

5.27.2.4.3.1	GET	494

5.27.2.4.3.2	POST	495

5.27.2.4.4	Resource Custom Operations	496

5.27.2.5	Resource: Individual MBS User Data Ingest Session Status Subscription	496

5.27.2.5.1	Introduction	496

5.27.2.5.2	Resource Definition	496

5.27.2.5.3	Resource Standard Methods	496

5.27.2.5.3.1	GET	496

5.27.2.5.3.2	PUT	497

5.27.2.5.3.3	PATCH	498

5.27.2.5.3.4	DELETE	499

5.27.2.5.4	Resource Custom Operations	500

5.27.3	Custom Operations without associated resources	500

5.27.4	Notifications	500

5.27.4.1	General	500

5.27.4.2	MBS User Data Ingest Session Status Change Notification	501

5.27.4.2.1	Description	501

5.27.4.2.2	Target URI	501

5.27.4.2.3	Operation Definition	501

5.27.4.2.3.1	Notification via HTTP POST	501

5.27.4.2.3.2	Void	502

5.27.5	Data Model	502

5.27.5.1	General	502

5.27.5.2	Structured data types	503

5.27.5.2.1	Introduction	503

5.27.5.3	Simple data types and enumerations	503

5.27.5.3.1	Introduction	503

5.27.5.3.2	Simple data types	503

5.27.6	Used Features	503

5.27.7	Error handling	504

5.27.7.1	General	504

5.27.7.2	Protocol Errors	504

5.27.7.3	Application Errors	504

5.28	MSEventExposure API	505

5.28.1	Introduction	505

5.28.2	Resources	506

5.28.2.1	Overview	506

5.28.2.2	Resource: Media Streaming Event Exposure Subscriptions	506

5.28.2.2.1	Introduction	506

5.28.2.2.2	Resource Definition	507

5.28.2.2.3	Resource Standard Methods	507

5.28.2.2.3.1	GET	507

5.28.2.2.3.2	POST	508

5.28.2.2.4	Resource Custom Operations	508

5.28.2.3	Resource: Individual Media Streaming Event Exposure Subscription	508

5.28.2.3.1	Introduction	508

5.28.2.3.2	Resource Definition	509

5.28.2.3.3	Resource Standard Methods	509

5.28.2.3.3.1	GET	509

5.28.2.3.3.2	PUT	510

5.28.2.3.3.3	DELETE	511

5.28.2.3.4	Resource Custom Operations	512

5.28.3	Custom Operations without associated resources	512

5.28.4	Notifications	512

5.28.4.1	General	512

5.28.4.2	Media Streaming Event Exposure Notification	512

5.28.4.2.1	Description	512

5.28.4.2.2	Target URI	512

5.28.4.2.3	Operation Definition	512

5.28.4.2.3.1	Notification via HTTP POST	512

5.28.4.2.3.2	Void	513

5.28.5	Data Model	513

5.28.5.1	General	513

5.28.5.2	Structured data types	514

5.28.5.2.1	Introduction	514

5.28.5.3	Simple data types and enumerations	514

5.28.5.3.1	Introduction	514

5.28.5.3.2	Simple data types	514

5.28.6	Used Features	514

5.28.7	Error handling	514

5.28.7.1	General	514

5.28.7.2	Protocol Errors	515

5.28.7.3	Application Errors	515

5.29	MBSGroupMsgDelivery API	515

5.29.1	Introduction	515

5.29.2	Resources	515

5.29.2.1	Overview	515

5.29.2.2	Resource: MBS Group Message Deliveries	516

5.29.2.2.1	Introduction	516

5.29.2.2.2	Resource Definition	516

5.29.2.2.3	Resource Standard Methods	517

5.29.2.2.3.1	GET	517

5.29.2.2.3.2	POST	518

5.29.2.2.4	Resource Custom Operations	518

5.29.2.3	Resource: Individual MBS Group Message Delivery	518

5.29.2.3.1	Introduction	518

5.29.2.3.2	Resource Definition	518

5.29.2.3.3	Resource Standard Methods	519

5.29.2.3.3.1	GET	519

5.29.2.3.3.2	PATCH	520

5.29.2.3.3.3	DELETE	521

5.29.2.3.4	Resource Custom Operations	522

5.29.3	Custom Operations without associated resources	522

5.29.4	Notifications	522

5.29.4.1	General	522

5.29.4.2	MBS Group Message Delivery Notification	522

5.29.4.2.1	Description	522

5.29.4.2.2	Target URI	522

5.29.4.2.3	Operation Definition	523

5.29.5	Data Model	523

5.29.5.1	General	523

5.29.5.2	Structured data types	524

5.29.5.2.1	Introduction	524

5.29.5.2.2	Void	525

5.29.5.2.3	Type: MbsGroupMsgDel	525

5.29.5.2.4	Void	527

5.29.5.2.5	Type: MbsGroupMsgDelPatch	527

5.29.5.2.6	Type: MbsGroupMsgDelStatusNotif	527

5.29.5.3	Simple data types and enumerations	527

5.29.5.3.1	Introduction	527

5.29.5.3.2	Simple data types	527

5.29.5.4	Data types describing alternative data types or combinations of data types	528

5.29.5.4.1	Type: MbsServArea	528

5.29.6	Used Features	528

5.29.7	Error handling	528

5.29.7.1	General	528

5.29.7.2	Protocol Errors	528

5.29.7.3	Application Errors	528

5.30	DNAIMapping API	528

5.30.2	Resources	529

5.30.2.1	Overview	529

5.30.2.2	Resource: DNAI Mapping Subscriptions	530

5.30.2.2.1	Introduction	530

5.30.2.2.2	Resource Definition	530

5.30.2.2.3	Resource Standard Methods	530

5.30.2.2.3.1	Void	530

5.30.2.2.3.2	GET	530

5.30.2.2.3.3	POST	531

5.30.2.2.4	Resource Custom Operations	532

5.30.2.3	Resource: Individual DNAI Mapping Subscription	532

5.30.2.3.1	Introduction	532

5.30.2.3.2	Resource Definition	532

5.30.2.3.3	Resource Standard Methods	532

5.30.2.3.3.1	Void	532

5.30.2.3.3.2	GET	532

5.30.2.3.3.3	DELETE	533

5.30.2.3.4	Resource Custom Operations	534

5.30.4	Notifications	535

5.30.4.1	General	535

5.30.4.2	DNAI Mapping Information Update Notification	535

5.30.4.2.1	Description	535

5.30.4.2.2	Target URI	535

5.30.4.2.3	Operation Definition	535

5.30.4.2.3.1	Notification via HTTP POST	535

5.30.4.2.3.2	Void	536

5.30.5	Data Model	536

5.30.5.1	General	536

5.30.5.2	Structured data types	537

5.30.5.2.1	Introduction	537

5.30.5.2.2	Type: DnaiMapSub	538

5.30.5.2.3	Type: DnaiMapUpdateNotif	539

5.30.5.2.4	Void	539

5.30.5.3	Simple data types and enumerations	539

5.30.5.3.1	Introduction	539

5.30.5.3.2	Simple data types	539

5.30.5.4	Data types describing alternative data types or combinations of data types	539

5.30.6	Used Features	539

5.30.7	Error handling	540

5.30.7.1	General	540

5.30.7.2	Protocol Errors	540

5.30.7.3	Application Errors	540

5.31	PdtqPolicyNegotiation API	540

5.31.0	Introduction	540

5.31.1	Resources	540

5.31.1.1	Overview	540

5.31.1.2	Resource: PDTQ Policy Subscriptions	541

5.31.1.2.1	Introduction	541

5.31.1.2.2	Resource Definition	541

5.31.1.2.3	Resource Standard Methods	542

5.31.1.2.3.1	Void	542

5.31.1.2.3.2	GET	542

5.31.1.2.3.3	POST	543

5.31.1.2.4	Resource Custom Operations	543

5.31.1.3	Resource: Individual PDTQ Policy Subscription	543

5.31.1.3.1	Introduction	543

5.31.1.3.2	Resource Definition	544

5.31.1.3.3	Resource Standard Methods	544

5.31.1.3.3.1	Void	544

5.31.1.3.3.2	GET	544

5.31.1.3.3.3	PATCH	545

5.31.1.3.3.4	DELETE	546

5.31.1.3.4	Resource Custom Operations	547

5.31.1A	Custom Operations without associated resources	547

5.31.2	Notifications	547

5.31.2.1	General	547

5.31.2.2	PDTQ Warning Notification	548

5.31.2.2.1	Description	548

5.31.2.2.2	Target URI	548

5.31.2.2.3	Operation Definition	548

5.31.2.2.3.1	Notification via HTTP POST	548

5.31.3	Data Model	549

5.31.3.1	General	549

5.31.3.2	Void	549

5.31.3.3	Structured data types	549

5.31.3.3.1	Introduction	549

5.31.3.3.2	Type: Pdtq	550

5.31.3.3.3	Type: PdtqPatch	552

5.31.3.3.4	Type: PdtqNotification	552

5.31.3.4	Simple data types and enumerations	552

5.31.3.4.1	Introduction	552

5.31.3.4.2	Simple data types	552

5.31.3.4	Data types describing alternative data types or combinations of data types	553

5.31.4	Used Features	553

5.31.5	Error handling	553

5.31.5.1	General	553

5.31.5.2	Protocol Errors	553

5.31.5.3	Application Errors	553

5.32	MemberUESelectionAssistance API	553

5.32.1	Introduction	553

5.32.2	Resources	554

5.32.2.1	Overview	554

5.32.2.2	Resource: Member UE Selection Assistance Subscriptions	555

5.32.2.2.1	Introduction	555

5.32.2.2.2	Resource definition	555

5.32.2.2.3	Resource Standard Methods	556

5.32.2.2.3.1	POST	556

5.32.2.2.3.2	GET	556

5.32.2.2.4	Resource Custom Operations	557

5.32.2.3	Resource: Individual Member UE Selection Assistance Subscription	557

5.32.2.3.1	Introduction	557

5.32.2.3.2	Resource Definition	558

5.32.2.3.3	Resource Standard Methods	558

5.32.2.3.3.1	GET	558

5.32.2.3.3.2	PUT	559

5.32.2.3.3.3	PATCH	560

5.32.2.3.3.4	DELETE	561

5.32.2.3.4	Resource Custom Operations	562

5.32.3	Custom Operations without associated resources	562

5.32.4	Notifications	562

5.32.5	Data Model	564

5.32.5.1	General	564

5.32.5.2	Structured data types	566

5.32.5.2.1	Introduction	566

5.32.5.2.2	Type: MemUeSelectAssistSubsc	567

5.32.5.2.3	Type: MemUeSeletAssistNotif	569

5.32.5.2.4	Type: QoSFilterCriteria	570

5.32.5.2.5	Type: AccessRatTypeFilterCriteria	570

5.32.5.2.6	Type: E2ETransTimeFilterCriteria	571

5.32.5.2.7	Type: UeLocFilterCriteria	571

5.32.5.2.8	Type: UeHisLocFilterCriteria	572

5.32.5.2.9	Type: UeDirectionFilterCriteria	572

5.32.5.2.10	Type: UeDistanceFilterCriteria	572

5.32.5.2.11	Type: ServiceExpFilterCriteria	573

5.32.5.2.12	Type: DnnFilterCriteria	573

5.32.5.2.13	Type: CandiUeListInfo	574

5.32.5.2.14	Type: MemUeSeletReport	574

5.32.5.2.15	Type: MemUeSelectAssistSubscPatch	575

5.32.5.2.16	Type: CandUeInfo	576

5.32.5.3	Simple data types and enumerations	576

5.32.5.3.1	Introduction	576

5.32.5.3.2	Simple data types	576

5.32.5.3.3	Enumeration: FilterCriterionType	577

5.32.5.4	Data types describing alternative data types or combinations of data types	577

5.32.6	Used Features	577

5.32.7	Error handling	577

5.32.7.1	General	577

5.32.7.2	Protocol Errors	577

5.32.7.3	Application Errors	577

5.33	GroupParametersProvisioning API	578

5.33.1	Introduction	578

5.33.2	Resources	578

5.33.2.1	Overview	578

5.33.2.2	Resource: Group Parameters Provisionings	579

5.33.2.2.1	Introduction	579

5.33.2.2.2	Resource Definition	579

5.33.2.2.3	Resource Methods	579

5.33.2.2.3.1	GET	579

5.33.2.2.3.2	POST	580

5.33.2.2.4	Resource Custom Operations	581

5.33.2.3	Resource: Individual Group Parameters Provisioning	581

5.33.2.3.1	Introduction	581

5.33.2.3.2	Resource Definition	581

5.33.2.3.3	Resource Methods	582

5.33.2.3.3.1	GET	582

5.33.2.3.3.2	PUT	583

5.33.2.3.3.3	PATCH	584

5.33.2.3.3.4	DELETE	585

5.33.2.3.4	Resource Custom Operations	586

5.33.3	Custom Operations without associated resources	586

5.33.4	Notifications	586

5.33.5	Data Model	586

5.33.5.1	General	586

5.33.5.2	Structured data types	587

5.33.5.2.1	Introduction	587

5.33.5.2.2	Type: GrpPpData	587

5.33.5.2.3	Type: GrpPpDataPatch	587

5.33.5.2.4	Type: DnnSnssaiGrpData	588

5.33.5.2.5	Type: AfReqDefaultQoS	588

5.33.5.2.6	Type: LadnServArea	588

5.33.5.3	Simple data types and enumerations	588

5.33.5.3.1	Introduction	588

5.33.5.3.2	Simple data types	589

5.33.5.4	Data types describing alternative data types or combinations of data types	589

5.33.6	Used Features	589

5.33.7	Error handling	589

5.33.7.1	General	589

5.33.7.2	Protocol Errors	589

5.33.7.3	Application Errors	589

5.34	SliceParamProvision API	589

5.34.1	Introduction	589

5.34.2	Resources	590

5.34.2.1	Overview	590

5.34.2.2	Resource: Slice Parameters Provisionings	591

5.34.2.2.1	Introduction	591

5.34.2.2.2	Resource Definition	591

5.34.2.2.3	Resource Methods	591

5.34.2.2.3.1	GET	591

5.34.2.2.3.2	POST	592

5.34.2.2.4	Resource Custom Operations	593

5.34.2.3	Resource: Individual Slice Parameters Provisioning	593

5.34.2.3.1	Introduction	593

5.34.2.3.2	Resource Definition	593

5.34.2.3.3	Resource Methods	594

5.34.2.3.3.1	GET	594

5.34.2.3.3.2	PUT	595

5.34.2.3.3.3	PATCH	596

5.34.2.3.3.4	DELETE	597

5.34.2.3.4	Resource Custom Operations	598

5.34.3	Custom Operations without associated resources	598

5.34.4	Notifications	598

5.34.5	Data Model	598

5.34.5.1	General	598

5.34.5.2	Structured data types	599

5.34.5.2.1	Introduction	599

5.34.5.2.2	Type: SlicePpData	599

5.34.5.2.3	Type: SlicePpDataPatch	599

5.34.5.3	Simple data types and enumerations	599

5.34.5.3.1	Introduction	599

5.34.5.3.2	Simple data types	599

5.34.6	Used Features	600

5.34.7	Error handling	600

5.34.7.1	General	600

5.34.7.2	Protocol Errors	600

5.34.7.3	Application Errors	600

5.35	UeAddress API	600

5.35.1	Introduction	600

5.35.2	Resources	601

5.35.3	Custom Operations without associated resources	601

5.35.3.1	Overview	601

5.35.3.2	Operation: Retrieve	601

5.35.3.2.1	Description	601

5.35.3.2.2	Operation Definition	601

5.35.4	Notifications	602

5.35.5	Data Model	602

5.35.5.1	General	602

5.35.5.2	Structured data types	603

5.35.5.2.1	Introduction	603

5.35.5.2.2	Type: UeAddressReq	603

5.35.5.2.3	Type: UeAddressInfo	603

5.35.5.3	Simple data types and enumerations	603

5.35.5.3.1	Introduction	603

5.35.5.3.2	Simple data types	604

5.35.5.4	Data types describing alternative data types or combinations of data types	604

5.35.6	Used Features	604

5.35.7	Error handling	604

5.35.7.1	General	604

5.35.7.2	Protocol Errors	604

5.35.7.3	Application Errors	604

5.36	ECSAddress API	604

5.36.1	Introduction	604

5.36.2	Resources	605

5.36.2.1	Overview	605

5.36.2.2	Resource: ECS Address Configuration Information Sets	606

5.36.2.2.1	Introduction	606

5.36.2.2.2	Resource Definition	606

5.36.2.2.3	Resource Standard Methods	606

5.36.2.2.3.1	Void	606

5.36.2.2.3.2	GET	606

5.36.2.2.3.3	POST	607

5.36.2.2.4	Resource Custom Operations	608

5.36.2.3	Resource: Individual ECS Address Configuration Information Set	608

5.36.2.3.1	Introduction	608

5.36.2.3.2	Resource Definition	608

5.36.2.3.3	Resource Standard Methods	609

5.36.2.3.3.1	Void	609

5.36.2.3.3.2	GET	609

5.36.2.3.3.3	PUT	610

5.36.2.3.3.3A	PATCH	611

5.36.2.3.3.4	DELETE	612

5.36.2.3.4	Resource Custom Operations	613

5.36.3	Custom Operations without associated resources	613

5.36.3.1	Overview	613

5.36.3.2	Operation: RemoveEcsAddr	614

5.36.3.2.1	Description	614

5.36.3.2.2	Operation Definition	614

5.36.4	Notifications	615

5.36.5	Data Model	615

5.36.5.1	General	615

5.36.5.2	Void	616

5.36.5.3	Structured data types	616

5.36.5.3.1	Introduction	616

5.36.5.3.2	Type: EcsAddrInfo	616

5.36.5.3.3	Type: EcsAddrDeleteCriteria	617

5.36.5.3.4	Type: EcsAddrInfoPatch	617

5.36.5.4	Simple data types and enumerations	617

5.36.5.4.1	Introduction	617

5.36.5.4.2	Simple data types	618

5.36.5.4	Data types describing alternative data types or combinations of data types	618

5.36.6	Used Features	618

5.36.7	Error handling	618

5.36.7.1	General	618

5.36.7.2	Protocol Errors	618

5.36.7.3	Application Errors	618

5.37	RSLPPIParametersProvisioning API	618

5.37.1	Introduction	618

5.37.2	Resources	619

5.37.2.1	Overview	619

5.37.2.2	Resource: RSLPPI Parameters Provisionings	620

5.37.2.2.1	Introduction	620

5.37.2.2.2	Resource Definition	620

5.37.2.2.3	Resource Methods	620

5.37.2.2.3.1	GET	620

5.37.2.2.3.2	POST	621

5.37.2.2.4	Resource Custom Operations	622

5.37.2.3	Resource: Individual RSLPPI Parameters Provisioning	622

5.37.2.3.1	Introduction	622

5.37.2.3.2	Resource Definition	622

5.37.2.3.3	Resource Methods	623

5.37.2.3.3.1	GET	623

5.37.2.3.3.2	PUT	624

5.37.2.3.3.3	PATCH	625

5.37.2.3.3.4	DELETE	626

5.37.2.3.4	Resource Custom Operations	627

5.37.3	Custom Operations without associated resources	627

5.37.4	Notifications	627

5.37.5	Data Model	627

5.37.5.1	General	627

5.37.5.2	Structured data types	628

5.37.5.2.1	Introduction	628

5.37.5.2.2	Type: RslppiData	628

5.37.5.2.3	Type: RslppiPpData	628

5.37.5.2.4	Type: RslppiPpDataPatch	628

5.37.5.3	Simple data types and enumerations	628

5.37.5.3.1	Introduction	628

5.37.5.3.2	Simple data types	629

5.37.5.4	Data types describing alternative data types or combinations of data types	629

5.37.6	Used Features	629

5.37.7	Error handling	629

5.37.7.1	General	629

5.37.7.2	Protocol Errors	629

5.37.7.3	Application Errors	629

6	Security	629

7	Using Common API Framework	630

7.1	General	630

7.2	Security	630

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	631

A.1	General	631

A.2	TrafficInfluence API	631

A.3	NiddConfigurationTrigger API	642

A.4	AnalyticsExposure API	644

A.5	5GLANParameterProvision API	661

A.6	ApplyingBdtPolicy API	670

A.7	IPTVConfiguration API	674

A.8	LpiParameterProvision API	679

A.9	ServiceParameter API	684

A.10	ACSParameterProvision API	697

A.11	MoLcsNotify API	702

A.12	AKMA API	703

A.13	TimeSyncExposure API	706

A.14	EcsAddressProvision API	720

A.15	AMPolicyAuthorization API	725

A.16	AMInfluence API	732

A.17	MBSTMGI API	738

A.18	MBSSession API	742

A.19	EASDeployment API	752

A.20	ASTI API	757

A.21	DataReporting API	764

A.22	DataReportingProvisioning API	767

A.23	UEId API	773

A.24	MBSUserService API	780

A.25	MBSUserDataIngestSession API	784

A.26	MSEventExposure API	793

A.27	MBSGroupMsgDelivery API	798

A.28	DNAIMapping API	803

A.29	PDTQPolicyNegotiation API	807

A.30	MemberUESelectionAssistance API	812

A.31	GroupParametersProvisioning API	823

A.32	SliceParamProvision API	829

A.33	UEAddress API	833

A.34	ECSAddress API	835

A.35	RSLPPIParametersProvisioning API	841

Annex B (informative):	Change history	846