| 3GPP TS 29.558 V0.5.0 (2021-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	12

1	Scope	14

2	References	14

3	Definitions of terms, symbols and abbreviations	14

3.1	Terms	15

3.2	Symbols	15

3.3	Abbreviations	15

4	Overview	16

5	Services offered by Edge Enabler Server	16

5.1	Introduction	16

5.2	Eees_EASRegistration Service	17

5.2.1	Service Description	17

5.2.2	Service Operations	17

5.2.2.1	Introduction	17

5.2.2.2	Eees_EASRegistration_Request	17

5.2.2.2.1	General	17

5.2.2.2.2	EAS registering to EES using Eees_EASRegistration_Request operation	17

5.2.2.3	Eees_EASRegistration_Update	18

5.2.2.3.1	General	18

5.2.2.3.2	EAS updating registration information using Eees_EASRegistration_Update operation	18

5.2.2.4	Eees_EASRegistration_Deregister	18

5.2.2.4.1	General	18

5.2.2.4.2	EAS deregistering from EES using Eees_EASRegistration_Deregister operation	18

5.3	Eees_UELocation Service	19

5.3.1	Service Description	19

5.3.2	Service Operations	19

5.3.2.1	Introduction	19

5.3.2.2	Eees_UELocation_Get	19

5.3.2.2.1	General	19

5.3.2.2.2	EAS obtaining UE location information from EES using Eees_UELocation_Get operation	19

5.3.2.3	Eees_UELocation_Subscribe	20

5.3.2.3.1	General	20

5.3.2.3.2	EAS subscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Subscribe operation	20

5.3.2.4	Eees_UELocation_Notify	20

5.3.2.4.1	General	20

5.3.2.4.2	EES notifying the UE(s) location reporting to EAS using Eees_UELocation_Notify operation	20

5.3.2.5	Eees_UELocation_UpdateSubscription	21

5.3.2.5.1	General	21

5.3.2.5.2	EAS updating continuous UE(s) location reporting subscription at EES using Eees_UELocation_UpdateSubscribe operation	21

5.3.2.6	Eees_UELocation_Unsubscribe	21

5.3.2.6.1	General	21

5.3.2.6.2	EAS unsubscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Unsubscribe operation	21

5.4	Eees_UEIdentifier Service	22

5.4.1	Service Description	22

5.4.2	Service Operations	22

5.4.2.1	Introduction	22

5.4.2.2	Eees_UEIdentifier_Get	22

5.4.2.2.1	General	22

5.4.2.2.2	EAS obtaining UE identifier from EES using Eees_UEIdentifier_Get operation	22

5.5	Eees_AppClientInformation Service	23

5.5.1	Service Description	23

5.5.2	Service Operations	23

5.5.2.1	Introduction	23

5.5.2.2	Eees_AppClientInformation_Subscribe	23

5.5.2.2.1	General	23

5.5.2.2.2	EAS subscribing to AC information reporting from EES using Eees_AppClientInformation_Subscribe operation	23

5.5.2.3	Eees_AppClientInformation_Notify	24

5.5.2.3.1	General	24

5.5.2.3.2	EES notifying the AC information to EAS using Eees_AppClientInformation_Notify operation	24

5.5.2.4	Eees_AppClientInformation_UpdateSubscription	24

5.5.2.4.1	General	24

5.5.2.4.2	EAS updating AC information reporting subscription at EES using Eees_AppClientInformation_UpdateSubscribe operation	24

5.5.2.5	Eees_AppClientInformation_Unsubscribe	25

5.5.2.5.1	General	25

5.5.2.5.2	EAS unsubscribing to AC information reporting from EES using Eees_AppClientInformation_Unsubscribe operation	25

5.6	Eees_SessionWithQoS Service	25

5.6.1	Service Description	25

5.6.2	Service Operations	25

5.6.2.1	Introduction	25

5.6.2.2	Eees_SessionWithQoS_Create	26

5.6.2.2.1	General	26

5.6.2.2.2	EAS requesting reservation of resources for a data session between AC and EAS with specific QoS using Eees_SessionWithQoS operation	26

5.6.2.3	Eees_SessionWithQoS_Update	27

5.6.2.3.1	General	27

5.6.2.3.2	EAS updating QoS of a data session between AC and EAS using Eees_SessionWithQoS_Update operation	27

5.6.2.4	Eees_SessionWithQoS_Revoke	27

5.6.2.4.1	General	27

5.6.2.4.2	EAS revoking QoS of a data session between AC and EAS using Eees_SessionWithQoS_Revoke operation	27

5.6.2.5	Eees_SessionWithQoS_Notify	28

5.6.2.5.1	General	28

5.6.2.5.2	EAS notifying QoS of a data session between AC and EAS using Eees_SessionWithQoS_Notify operation	28

5.7	Eees_TargetEASDiscovery Service	28

5.7.1	Service Description	28

5.7.2	Service Operations	28

5.7.2.1	Introduction	28

5.7.2.2	Eees_TargetEASDiscovery_Request	28

5.7.2.2.1	General	28

5.7.2.2.2	EAS and EES fetching the T-EAS information from the EES using Eees_TargetEASDiscovery_Request operation	28

5.8	Eees_ACRManagementEvent Service	29

5.8.1	Service Description	29

5.8.2	Service Operations	29

5.8.2.1	Introduction	29

5.8.2.2	Eees_ACRManagementEvent_Subscribe	29

5.8.2.2.1	General	29

5.8.2.2.2	EAS requesting to get notifications of ACR management events using Eees_ACRManagementEvent_Subscribe service operation	29

5.8.2.3	Eees_ACRManagementEvent_UpdateSubscription	30

5.8.2.3.1	General	30

5.8.2.3.2	EAS updating an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_UpdateSubscription service operation	30

5.8.2.4	Eees_ACRManagementEvent_Unsubscribe	30

5.8.2.4.1	General	30

5.8.2.4.2	EAS revoking an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_Unsubscribe service operation	30

5.8.2.5	Eees_ACRManagementEvent_Notify	31

5.8.2.5.1	General	31

5.8.2.5.2	EAS notifying ACR management events using Eees_ACRManagementEvent_Notify operation	31

5.9	Eees_SelectedTargetEAS Service	31

5.9.1	Service Description	31

5.9.2	Service Operations	31

5.9.2.1	Introduction	31

5.9.2.2	Eees_SelectedTargetEAS_Declare	31

5.9.2.2.1	General	31

5.9.2.2.2	S-EAS informing the S-EES about the selected T-EAS using Eees_SelectedTargetEAS_Declare operation	32

5.x	<Eees_xxx> Service	32

5.x.1	Service Description	32

5.x.2	Service Operations	32

5.x.2.1	Introduction	32

5.x.2.2	<Service operation 1>	32

5.x.2.2.1	General	32

5.x.2.2.2	<Description> <Service Operation Name> operation	32

5.x.2.3	<Service operation 2>	32

6	Services offered by Edge Configuration Server	33

6.1	Introduction	33

6.2	Eecs_EESRegistration Service	33

6.2.1	Service Description	33

6.2.2	Service Operations	33

6.2.2.1	Introduction	33

6.2.2.2	Eecs_EESRegistration_Request	34

6.2.2.2.1	General	34

6.2.2.2.2	EES registering to ECS using Eecs_EESRegistration_Request operation	34

6.2.2.3	Eecs_EESRegistration_Update	34

6.2.2.3.1	General	34

6.2.2.3.2	EES updating registration information using Eecs_EESRegistration_Update operation	34

6.2.2.4	Eecs_EESRegistration_Deregister	35

6.2.2.4.1	General	35

6.2.2.4.2	EES deregistering from ECS using Eecs_EESRegistration_Deregister operation	35

6.3	Eecs_TargetEESDiscovery Service	35

6.3.1	Service Description	35

6.3.2	Service Operations	35

6.3.2.1	Introduction	35

6.3.2.2	Eecs_TargetEESDiscovery_Request	35

6.3.2.2.1	General	35

6.3.2.2.2	EES fetching the T-EES information from the ECS using Eecs_TargetEESDiscovery_Request operation	36

6.x	<Eecs_xxx> Service	36

6.x.1	Service Description	36

6.x.2	Service Operations	36

6.x.2.1	Introduction	36

6.x.2.2	<Service operation 1>	36

6.x.2.2.1	General	36

6.x.2.2.2	<Description> <Service Operation Name> operation	37

6.x.2.3	<Service operation 2>	37

7	Information applicable to several APIs	37

7.1	General	37

7.2	Data Types	37

7.2.1	General	37

7.2.2	Referenced structured data types	37

7.2.3	Referenced simple data types and enumerations	38

7.3	Usage of HTTP	38

7.4	Content type	38

7.5	URI structure	38

7.5.1	Resource URI structure	38

7.5.2	Custom operations URI structure	38

7.6	Notifications	38

7.7	Error handling	39

7.8 	Feature negotiation	39

7.9	HTTP headers	39

7.10	Conventions for Open API specification files	39

8	Edge Enabler Server API Definitions	39

8.1	Eees_EASRegistration API	39

8.1.1	API URI	39

8.1.2	Resources	40

8.1.2.1	Overview	40

8.1.2.2	Resource: EAS Registrations	40

8.1.2.2.1	Description	40

8.1.2.2.2	Resource Definition	40

8.1.2.2.3	Resource Standard Methods	41

8.1.2.2.3.1	POST	41

8.1.2.2.4	 Resource Custom Operations	42

8.1.2.3	Resource: Individual EAS Registration	42

8.1.2.3.1	Description	42

8.1.2.3.2	Resource Definition	42

8.1.2.3.3	Resource Standard Methods	42

8.1.2.3.3.1	GET	42

8.1.2.3.3.2	PUT	43

8.1.2.3.3.3	DELETE	44

8.1.2.3.3.4	PATCH	45

8.1.2.3.4	 Resource Custom Operations	45

8.1.3	Custom Operations without associated resources	45

8.1.4	Notifications	46

8.1.5	Data Model	46

8.1.5.1	General	46

8.1.5.2	Structured data types	47

8.1.5.2.1	Introduction	47

8.1.5.2.2	Type: EASRegistration	47

8.1.5.2.3	Type: EASProfile	48

8.1.5.2.4	Type: EASServiceKPI	49

8.1.5.2.5	Type: EndPoint	49

8.1.5.2.6	Type: EASRegistrationPatch	49

8.1.5.3	Simple data types and enumerations	49

8.1.6	Error Handling	49

8.1.7	Feature negotiation	50

8.2	Eees_UELocation API	50

8.2.1	API URI	50

8.2.2	Resources	50

8.2.2.1	Overview	50

8.2.2.2	Resource: Location Information Subscriptions	51

8.2.2.2.1	Description	51

8.2.2.2.2	Resource Definition	51

8.2.2.2.3	Resource Standard Methods	51

8.2.2.2.3.1	POST	51

8.2.2.2.4	 Resource Custom Operations	52

8.2.2.3	Resource: Individual Location Information Subscription	52

8.2.2.3.1	Description	52

8.2.2.3.2	Resource Definition	52

8.2.2.3.3	Resource Standard Methods	53

8.2.2.3.3.1	GET	53

8.2.2.3.3.2	PATCH	54

8.2.2.3.3.3	PUT	54

8.2.2.3.3.4	DELETE	55

8.2.2.3.4	 Resource Custom Operations	56

8.2.3	Custom Operations without associated resources	56

8.2.3.1	Overview	56

8.2.3.2	Operation: Fetch	57

8.2.3.2.1	Description	57

8.2.3.2.2	Operation Definition	57

8.2.4	Notifications	58

8.2.4.1	General	58

8.2.4.2	Location Information Notification	58

8.2.4.2.1	Description	58

8.2.4.2.2	Notification definition	58

8.2.5	Data Model	59

8.2.5.1	General	59

8.2.5.2	Structured data types	61

8.2.5.2.1	Introduction	61

8.2.5.2.2	Type: LocationSubscription	61

8.2.5.2.3	Type: LocationSubscriptionPatch	62

8.2.5.2.4	Type: LocationNotification	62

8.2.5.2.5	Type: LocationEvent	62

8.2.5.2.6	Type: LocationRequest	62

8.2.5.3	Simple data types and enumerations	62

8.2.6	Error Handling	63

8.2.7	Feature negotiation	63

8.3	Eees_UEIdentifier API	63

8.3.1	API URI	63

8.3.2	Resources	63

8.3.2.1	Overview	63

8.3.2.2	Resource: Identifier Information of UEs	64

8.3.2.2.1	Description	64

8.3.2.2.2	Resource Definition	64

8.3.2.2.3	Resource Standard Methods	64

8.3.2.2.4	 Resource Custom Operations	64

8.3.2.2.4.1	 Overview	64

8.3.2.2.4.2	 Operation: Fetch	64

8.3.2.2.4.2.1	Description	64

8.3.2.2.4.2.2	Operation Definition	64

8.3.3	Custom Operations without associated resources	65

8.3.4	Notifications	65

8.3.5	Data Model	65

8.3.5.1	General	65

8.3.5.2	Structured data types	66

8.3.5.2.1	Introduction	66

8.3.5.2.2	Type: UserInformation	66

8.3.5.3	Simple data types and enumerations	66

8.3.6	Error Handling	66

8.3.7	Feature negotiation	66

8.4	Eees_AppClientInformation API	66

8.4.1	API URI	66

8.4.2	Resources	67

8.4.2.1	Overview	67

8.4.2.2	Resource: Application Client Information Subscriptions	67

8.4.2.2.1	Description	67

8.4.2.2.2	Resource Definition	67

8.4.2.2.3	Resource Standard Methods	67

8.4.2.2.3.1	POST	67

8.4.2.2.4	 Resource Custom Operations	67

8.4.2.3	Resource: Individual Application Client Information Subscription	67

8.4.2.3.1	Description	67

8.4.2.3.2	Resource Definition	67

8.4.2.3.3	Resource Standard Methods	67

8.4.2.3.3.1	GET	67

8.4.2.3.3.2	PATCH	67

8.4.2.3.3.3	PUT	67

8.4.2.3.3.4	DELETE	67

8.4.2.3.4	 Resource Custom Operations	67

8.4.3	Custom Operations without associated resources	67

8.4.4	Notifications	67

8.4.4.1	General	67

8.4.4.2	AC Information Notification	67

8.4.4.2.1	Description	67

8.4.4.2.2	Notification definition	67

8.4.5	Data Model	67

8.4.5.1	General	67

8.4.5.2	Structured data types	67

8.4.5.2.1	Introduction	67

8.4.5.2.2	Type: ACInfoSubscription	67

8.4.5.2.3	Type: ACInfoSubscriptionPatch	67

8.4.5.2.4	Type: ACFilters	67

8.4.5.2.5	Type: ACInfoNotification	67

8.4.5.2.6	Type: ACInformation	67

8.4.5.3	Simple data types and enumerations	67

8.4.6	Error Handling	67

8.4.7	Feature negotiation	67

8.5	Eees_SessionWithQoS API	67

8.5.1	API URI	67

8.5.2	Resources	67

8.5.2.1	Overview	67

8.5.2.2	Resource: Sessions with QoS	67

8.5.2.2.1	Description	67

8.5.2.2.2	Resource Definition	67

8.5.2.2.3	Resource Standard Methods	67

8.5.2.2.3.1	POST	67

8.5.2.2.3.2	GET	67

8.5.2.2.4	 Resource Custom Operations	67

8.5.2.3	Resource: Individual Session with QoS	67

8.5.2.3.1	Description	67

8.5.2.3.2	Resource Definition	67

8.5.2.3.3	Resource Standard Methods	67

8.5.2.3.3.1	PATCH	67

8.5.2.3.3.2	PUT	67

8.5.2.3.3.3	DELETE	67

8.5.2.3.3.4	GET	67

8.5.2.3.4	 Resource Custom Operations	67

8.5.3	Custom Operations without associated resources	67

8.5.4	Notifications	67

8.5.4.1	General	67

8.5.4.2	User Plane Event Notification	67

8.5.4.2.1	Description	67

8.5.4.2.2	Notification definition	67

8.5.5	Data Model	67

8.5.5.1	General	67

8.5.5.2	Structured data types	67

8.5.5.2.1	Introduction	67

8.5.5.2.2	Type: SessionWithQoS	67

8.5.5.2.3	Type: SessionWithQoSPatch	67

8.5.5.2.4	Type: UserPlaneEventNotification	67

8.5.5.3	Simple data types and enumerations	67

8.5.6	Error Handling	67

8.5.7	Feature negotiation	67

8.6	Eees_TargetEASDiscovery API	67

8.6.1	API URI	67

8.6.2	Resources	67

8.6.2.1	Overview	67

8.6.2.2	Resource: EAS Profiles	67

8.6.2.2.1	Description	67

8.6.2.2.2	Resource Definition	67

8.6.2.2.3	Resource Standard Methods	67

8.6.2.2.3.1	GET	67

8.6.2.2.4	 Resource Custom Operations	67

8.6.3	Custom Operations without associated resources	67

8.6.4	Notifications	67

8.6.5	Data Model	67

8.6.5.1	General	67

8.6.5.2	Structured data types	67

8.6.5.3	Simple data types and enumerations	67

8.6.6	Error Handling	67

8.6.7	Feature negotiation	67

8.7	Eees_ACRManagementEvent API	67

8.7.1	API URI	67

8.7.2	Resources	67

8.7.2.1	Overview	67

8.7.2.2	Resource: ACR Management Events Subscriptions	67

8.7.2.2.1	Description	67

8.7.2.2.2	Resource Definition	67

8.7.2.2.3	Resource Standard Methods	67

8.7.2.2.3.1	POST	67

8.7.2.2.3.2	GET	67

8.7.2.2.4	 Resource Custom Operations	67

8.7.2.3	Resource: Individual ACR Management Events Subscription	67

8.7.2.3.1	Description	67

8.7.2.3.2	Resource Definition	67

8.7.2.3.3	Resource Standard Methods	67

8.7.2.3.3.1	PATCH	67

8.7.2.3.3.2	PUT	67

8.7.2.3.3.3	DELETE	67

8.7.2.3.3.4	GET	67

8.7.2.3.4	 Resource Custom Operations	67

8.7.3	Custom Operations without associated resources	67

8.7.4	Notifications	67

8.7.4.1	General	67

8.7.4.2	ACR Management Events Notification	67

8.7.4.2.1	Description	67

8.7.4.2.2	Notification definition	67

8.7.5	Data Model	67

8.7.5.1	General	67

8.7.5.2	Structured data types	67

8.7.5.2.1	Introduction	67

8.7.5.2.2	Type: AcrMgntEventsSubscription	67

8.7.5.2.3	Type: AcrMgntEventSubsc	67

8.7.5.2.4	Type: AcrMgntEventsSubscriptionPatch	67

8.7.5.2.5	Type: AcrMgntEventsNotification	67

8.7.5.2.6	Type: AcrMgntEventReport	67

8.7.5.2.7	Type: FailureAcrMgntEventInfo	67

8.7.5.2.8	Type: TargetUeIdentification	67

8.7.5.3	Simple data types and enumerations	67

8.7.5.3.1	Introduction	67

8.7.5.3.2	Simple data types	67

8.7.5.3.3	Enumeration: AcrMgntEvent	67

8.7.6	Error Handling	67

8.7.7	Feature negotiation	67

8.8	Eees_SelectedTargetEAS API	67

8.8.1	API URI	67

8.8.2	Resources	67

8.8.3	Custom Operations without associated resources	67

8.8.3.1	Overview	67

8.8.3.2	Operation: Declare	67

8.8.3.2.1	Description	67

8.8.3.2.2	Operation Definition	67

8.8.4	Notifications	67

8.8.5	Data Model	67

8.8.5.1	General	67

8.8.5.2	Structured data types	67

8.8.5.2.1	Introduction	67

8.8.5.2.2	Type: SelectedEAS	67

8.8.5.3	Simple data types and enumerations	67

8.8.6	Error Handling	67

8.8.7	Feature negotiation	67

8.x	<API Name – Eees_xxx> API	67

8.x.1	API URI	67

8.x.2	Resources	67

8.x.2.1	Overview	67

8.x.2.2	Resource: <Resource name>	67

8.x.2.2.1	Description	67

8.x.2.2.2	Resource Definition	67

8.x.2.2.3	Resource Standard Methods	67

8.x.2.2.3.1	<Method Name>	67

8.x.2.2.4	 Resource Custom Operations	67

8.x.2.2.4.1	 Overview	67

8.x.2.2.4.2	 Operation: < operation 1 >	67

8.x.2.2.4.2.1	Description	67

8.x.2.2.4.2.2	Operation Definition	67

8.x.3	Custom Operations without associated resources	67

8.x.3.1	Overview	67

8.x.3.2	Operation: <operation 1>	67

8.x.3.2.1	Description	67

8.x.3.2.2	Operation Definition	67

8.x.3.3	Operation: < operation 2>	67

8.x.4	Notifications	67

8.x.4.1	General	67

8.x.4.2	<notification 1>	67

8.x.4.2.1	Description	67

8.x.4.2.2	Notification definition	67

8.x.5	Data Model	67

8.x.5.1	General	67

8.x.5.2	Structured data types	67

8.x.5.2.1	Introduction	67

8.x.5.2.2	Type: <Data type name>	67

8.x.5.3	Simple data types and enumerations	67

8.x.5.3.1	Introduction	67

8.x.5.3.2	Simple data types	67

8.x.5.3.3	Enumeration: <EnumType1>	67

8.x.6	Error Handling	67

8.x.7	Feature negotiation	67

9	Edge Configuration Server API Definitions	67

9.1	Eecs_EESRegistration API	67

9.1.1	API URI	67

9.1.2	Resources	67

9.1.2.1	Overview	67

9.1.2.2	Resource: EES Registrations	67

9.1.2.2.1	Description	67

9.1.2.2.2	Resource Definition	67

9.1.2.2.3	Resource Standard Methods	67

9.1.2.2.3.1	POST	67

9.1.2.2.4	 Resource Custom Operations	67

9.1.2.3	Resource: Individual EES Registration	67

9.1.2.3.1	Description	67

9.1.2.3.2	Resource Definition	67

9.1.2.3.3	Resource Standard Methods	67

9.1.2.3.3.1	GET	67

9.1.2.3.3.2	PUT	67

9.1.2.3.3.3	DELETE	67

9.1.2.3.3.4	PATCH	67

9.1.2.3.4	 Resource Custom Operations	67

9.1.3	Custom Operations without associated resources	67

9.1.4	Notifications	67

9.1.5	Data Model	67

9.1.5.1	General	67

9.1.5.2	Structured data types	67

9.1.5.2.1	Introduction	67

9.1.5.2.2	Type: EESRegistration	67

9.1.5.2.3	Type: EESProfile	67

9.1.5.2.4	Type: EESRegistrationPatch	67

9.1.5.2.5	Type: ServiceArea	67

9.1.5.2.6	Type: TopologicalServiceArea	67

9.1.5.2.7	Type: GeographicalServiceArea	67

9.1.5.3	Simple data types and enumerations	67

9.1.5.3.1	Introduction	67

9.1.5.3.2	Simple data types	67

9.1.5.3.3	Enumeration: ACRScenario	67

9.1.6	Error Handling	67

9.1.7	Feature negotiation	67

9.2	Eecs_TargetEESDiscovery API	67

9.2.1	API URI	67

9.2.2	Resources	67

9.2.2.1	Overview	67

9.2.2.2	Resource: EES Profiles	67

9.2.2.2.1	Description	67

9.2.2.2.2	Resource Definition	67

9.2.2.2.3	Resource Standard Methods	67

9.2.2.2.3.1	GET	67

9.2.2.2.4	 Resource Custom Operations	67

9.2.3	Custom Operations without associated resources	67

9.2.4	Notifications	67

9.2.5	Data Model	67

9.2.5.1	General	67

9.2.5.2	Structured data types	67

9.2.5.3	Simple data types and enumerations	67

9.2.6	Error Handling	67

9.2.7	Feature negotiation	67

9.x	<API Name – Eecs_xxx> API	67

9.x.1	API URI	67

9.x.2	Resources	67

9.x.2.1	Overview	67

9.x.2.2	Resource: <Resource name>	67

9.x.2.2.1	Description	67

9.x.2.2.2	Resource Definition	67

9.x.2.2.3	Resource Standard Methods	67

9.x.2.2.3.1	<Method Name>	67

9.x.2.2.4	 Resource Custom Operations	67

9.x.2.2.4.1	 Overview	67

9.x.2.2.4.2	 Operation: < operation 1 >	67

9.x.2.2.4.2.1	Description	67

9.x.2.2.4.2.2	Operation Definition	67

9.x.3	Custom Operations without associated resources	67

9.x.3.1	Overview	67

9.x.3.2	Operation: <operation 1>	67

9.x.3.2.1	Description	67

9.x.3.2.2	Operation Definition	67

9.x.3.3	Operation: < operation 2>	67

9.x.4	Notifications	67

9.x.4.1	General	67

9.x.4.2	<notification 1>	67

9.x.4.2.1	Description	67

9.x.4.2.2	Notification definition	67

9.x.5	Data Model	67

9.x.5.1	General	67

9.x.5.2	Structured data types	67

9.x.5.2.1	Introduction	67

9.x.5.2.2	Type: <Data type name>	67

9.x.5.3	Simple data types and enumerations	67

9.x.5.3.1	Introduction	67

9.x.5.3.2	Simple data types	67

9.x.5.3.3	Enumeration: <EnumType1>	67

9.x.6	Error Handling	67

9.x.7	Feature negotiation	67

10	Using Common API Framework	67

11	Security	67

Annex A (normative): OpenAPI specification	67

A.1 General	67

A.1	Eees_ACRManagementEvent API	67

Annex B (informative): Change history	67

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This clause shall start on a new page.

The present document specifies the APIs for enabling the edge applications over 3GPP networks. The application layer architecture, functional requirements, procedures and information flows necessary for enabling edge applications over 3GPP networks are specified in 3GPP TS 23.558 [2]. The APIs are specified as RESTful APIs except for custom operations wherever required.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]	Open API: "OpenAPI Specification Version 3.0.0.", https://spec.openapis.org/oas/v3.0.0.

[4]	3GPP TR 21.900: "Technical Specification Group working methods".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[7]	IETF RFC 6455: "The Websocket Protocol".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[10]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[11]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[12]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[13]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[14]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[15]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[16]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[17]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and its definitions given in 3GPP TS 23.558 [2] shall apply:

Application Context

Application Context Relocation

Application Context Transfer

Application Server

Edge Computing Service Provider

Edge Data Network

EEC Context

Edge Hosting Environment

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

ACR	Application Context Relocation

AF	Application Function

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

EHE	Edge Hosting Environment

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

LADN	Local Area Data Network

NEF	Network Exposure Function

S-EAS	Source Edge Application Server

S-EES	Source Edge Enabler Server

SCEF	Service Capability Exposure Function

SSID	Service Set Identifier

T-EAS	Target Edge Application Server

T-EES	Target Edge Enabler Server

TAI	Tracking Area Identity

# 4	Overview

This clause will provide the overview of the EdgeApp services, along with core network capabilities utilized by the Edge Enabler layer.

3GPP TS 23.558 [2] has specified the application layer architecture, requirements, procedures, information flows and the APIs, in order to support the edge applications over the 3GPP systems. Various features are defined to ensure the efficient use and deployment of edge applications, some of which include, registration, discovery, service provisioning, capability exposure and support for service continuity.

The present document specifies the north-bound APIs in detail, needed to support the services offered by EES and ECS over EDGE-3/6/9 interfaces for enabling the edge applications over 3GPP network.

# 5	Services offered by Edge Enabler Server

## 5.1	Introduction

This clause will provide the list of Edge Enabler Server services with their respective service operations.

The table 5.1-1 lists the Edge Enabler Server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of EES Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_EASRegistration | Request | Request/Response | EAS |
|  | Update | Request/Response | EAS |
|  | Deregister | Request/Response | EAS |
| Eees_UELocation | Get | Request/Response | EAS |
|  | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_UEIdentifier | Get | Request/Response | EAS |
| Eees_AppClientInformation | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_SessionWithQoS | Create | Request/Response | EAS |
|  | Update | Request/Response | EAS |
|  | Revoke | Request/Response | EAS |
|  | Notify | Subscribe/Notify | EAS |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |
| Eees_ACRManagementEvent | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_SelectedTargetEAS | Declare | Request/Response | EAS |

Table 5.1-2 summarizes the corresponding Edge Enabler Server APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |

## 5.2	Eees_EASRegistration Service

### 5.2.1	Service Description

The Eees_EASRegistration API, as defined in 3GPP TS 23.558 [2], allows an Edge Application Server via Eees interface to register, update its registration and deregister at a given Edge Enabler Server.

Editor’s Note: Details about EAS security credentials, verification and authorization of Eees_EASRegistration Request, Update and Delete, by the EES, to be aligned with security aspects defined by SA3.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operation defined for Eees_EASRegistration API is shown in the table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the Eees_EASRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EASRegistration_Request | This service operation is used by the EAS to register itself to a given EES. | EAS |
| Eees_EASRegistration_Update | This service operation is used by the EAS to update its registration information at EES. | EAS |
| Eees_EASRegistration_Deregister | This service operation is used by the EAS to deregister itself from a given EES. | EAS |

#### 5.2.2.2	Eees_EASRegistration_Request

##### 5.2.2.2.1	General

This service operation is used by EAS to register itself to a given EES.

##### 5.2.2.2.2	EAS registering to EES using Eees_EASRegistration_Request operation

To register itself as an Edge Application Server at the EES, the EAS shall send an HTTP POST message to the Edge Enabler Server on the "EAS Registrations" collection resource. The body of the HTTP POST message shall include the EAS profile information, may include proposed expiration time for the registration, as specified in clause 8.1.2.2.3.1.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1. Process the EAS registration request information;

2. verify the identity of the Edge Application Server and check if the EAS is authorized to register itself at EES ;

3. if the EAS is authorized to register to EES, then the EES shall;

a.  store the EAS profile and create a new resource with the EAS registration information as specified in clause 8.1.2.1;

b. return the EAS registration information, the resource URI of the EAS registration information, in the response message. The response message may include expiration time to indicate when the EAS registration will automatically expire.

If the expiration time is provided, then to maintain the registration, the EAS shall send a registration update request (as described in clause 5.2.2.3) prior to the expiration time. If the registration update request is not sent before the expiry time, then the EES shall treat the EAS as deregistered and remove the corresponding EAS registration resource.

#### 5.2.2.3	Eees_EASRegistration_Update

##### 5.2.2.3.1	General

This service operation is used by EAS to update its registration information at a given EES.

##### 5.2.2.3.2	EAS updating registration information using Eees_EASRegistration_Update operation

To update the EAS registration information at the EES, the EAS shall send a HTTP PUT or PATCH message to the Edge Enabler Server on resource URI identifying the Individual EAS registration resource representation, as specified in clause 8.1.2.3.3.2 for HTTP PUT message and in clause 8.1.2.3.3.4 for HTTP PATCH message.

The HTTP PUT message shall replace all properties in the existing resource with the EAS registration information in the request. The body of the HTTP PUT message shall include the EAS profile information, may include proposed expiration time to update the registration. This request shall not replace the easId property of the existing resource.

The HTTP PATCH message includes parameters (EAS Profile, expiry time) that need to be replaced in the existing Individual EAS registration resource.  This request shall not replace the easId property of the existing resource.

Upon receiving the HTTP PUT or PATCH message from the EAS, the EES shall:

1. check the registration update message from the EAS to see if the EAS is authorized to modify the requested registration resource;

2. if the EAS is authorized to update the registration information and the easId information in the request and the resource match, then the EES shall;

a. update the resource identified by Resource URI of the EAS registration information with the updated EAS registration information received in the HTTP PUT or PATCH request message;

b. return the updated EAS registration information in the response. In the response message, the EES may provide an updated expiration time to indicate to the EAS when the updated registration will automatically expire.

If the expiration time is provided, then to maintain the registration, the EAS shall send a registration update prior to registration expiry time. If the registration update request is not sent before the expiry time, then the EES shall treat EAS as deregistered and remove the corresponding EAS registration resource.

#### 5.2.2.4	Eees_EASRegistration_Deregister

##### 5.2.2.4.1	General

This service operation is used by EAS to deregister itself from a given EES.

##### 5.2.2.4.2	EAS deregistering from EES using Eees_EASRegistration_Deregister operation

To deregister itself from the EES, the EAS shall send HTTP DELETE message to the EES, on the resource URI identifying the Individual EAS registration resource representation as specified in clause 8.1.2.3.3.3. Upon receiving the HTTP DELETE request, the EES shall:

1. verify the identity of the EAS and check if the EAS is authorized to deregister the EAS registration information;

2. if the EAS is authorized to deregister the EAS registration information, then the EES shall deregister the EAS profile from the EES and delete the resource representing EAS registration information;

3. return the "204 Not Content" message to the EAS, indicating the successful deregistration of the EAS information.

## 5.3	Eees_UELocation Service

### 5.3.1	Service Description

The Eees_UELocation API, as defined in 3GPP TS 23.558 [2], allows an Edge Application Server via Eees interface to obtain the UE location information as one time request or subscribe for continuous reporting.

Editor’s Note: Details about EAS security credentials, verification and authorization of Eees_UELocation service operations, by the EES, to be aligned with security aspects defined by SA3.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operation defined for Eees_UELocation API is shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the Eees_UELocation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_UELocation_Get | This service operation is used by the EAS to request UE location information from a given EES. | EAS |
| Eees_UELocation_Subscribe | This service operation is used by the EAS to subscribe to EES, for continuous reporting of UE location information. | EAS |
| Eees_UELocation_Notify | This service operation is used by the EES to notify the EAS about the UE location information. | EES |
| Eees_UELocation_UpdateSubscription | This service operation is used by the EAS to update its subscription at EES, for continuous reporting of UE location information. | EAS |
| Eees_UELocation_Unsubscribe | This service operation is used by the EAS to remove its subscription from EES, for continuous reporting of UE location information. | EAS |

#### 5.3.2.2	Eees_UELocation_Get

##### 5.3.2.2.1	General

This service operation is used by EAS to obtain a UE’s location information from a given EES.

##### 5.3.2.2.2	EAS obtaining UE location information from EES using Eees_UELocation_Get operation

To obtain an UE’s location information from the EES, the EAS shall send an HTTP POST message to the EES on the URI "{apiRoot}/eees-uelocation/<apiVersion>/fetch" as specified in clause 8.2.3.2. The POST request includes:

-	the identifier of the UE for which location information is requested;

-	the accuracy of the requested location in terms of granularity and location QoS.

Upon reception of the HTTP POST request from the EAS, the EES shall:

1. process the EAS UE location information request;

2. verify the identity of the Edge Application Server and check if the EAS is authorized to obtain UE location information;

3. if the EAS is authorized to obtain the UE’s location information, then the EES shall:

a. consider the location granularity information received in the request message to obtain the UE’s location information;

b. check if a valid locally cached UE location information is available, and if available then the EES shall return the UE location information in the format requested by the EAS along with the location accuracy and its timestamp;

c. if valid UE location information is not available in local cache, then the EES shall obtain the UE location information by consuming the 3GPP core network capabilities. The EES shall return the UE location information to EAS in the format requested by the EAS along with the location accuracy and its timestamp.

#### 5.3.2.3	Eees_UELocation_Subscribe

##### 5.3.2.3.1	General

This service operation is used by the EAS to subscribe for continuous UE(s) location reporting.

##### 5.3.2.3.2	EAS subscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Subscribe operation

To subscribe to continuous UE(s) location information reporting at the EES, the EAS shall send a HTTP POST message to the EES on the "Location Information Subscriptions" resource. The body of the POST message shall include EAS identifier, the identifier of the UE or the identifier of the group uniquely identifying a group of UEs, Notification Destination URI and may include location format that is understood by EAS, location QoS, proposed expiry time of the subscription and reporting requirements, as specified in clause 8.2.2.2.3.1.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1. Process the EAS UE location information subscription request;

2. verify the identity of the Edge Application Server and check if the EAS is authorized to subscribe for the continuous UE(s) location reporting;

3. if the EAS is authorized to subscribe for the continuous UE(s) location information reporting, then the EES shall;

a.  create a new resource with the Location Information Subscription as specified in clause 8.2.2.1;

b.  return the EAS’s location subscription information, the resource URI of the EAS location subscription, in the response message. The response message may include expiration time to indicate when the location information subscription will automatically expire;

EES shall obtain the UE location information by consuming the 3GPP core network capabilities from NEF as specified in 3GPP TS 29.522 [10], from SCEF as specified in 3GPP TS 29.122 [6] or from LMF as specified in 3GPP TS 29.572 [11]. The EES may also consume the UE mobility analytics from NEF as specified in 3GPP TS 29.522 [10] or from NWDAF as specified in 3GPP TS 29.520 [12].

If the expiration time is provided, then to maintain the registration, the EAS shall send a subscription update request (as described in clause 5.3.2.5) prior to the expiration time. If the subscription update request is not sent before the expiry time, then the EES shall treat the subscription as unsubscribed and remove the corresponding EAS’s Individual Location Information Subscription resource.

#### 5.3.2.4	Eees_UELocation_Notify

##### 5.3.2.4.1	General

This service operation is used by the EES to send UE(s) location information notifications to the EAS.

##### 5.3.2.4.2	EES notifying the UE(s) location reporting to EAS using Eees_UELocation_Notify operation

The EES determines to notify the EAS with the UE location information, when the UE location information is available either locally cached or from the 3GPP core network.

To notify the UE(s) location information events, the EES shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include LocationNotification. LocationNotification includes location information of each UE with accuracy, timestamp and type. The location information of each UE may be actual location change or predictive location report from the UE mobility analytics report from NEF as specified in 3GPP TS 29.522 [10] or from NWDAF as specified in 3GPP TS 29.520 [12]. The EES may modify the UE location information in the format requested by the EAS in the subscription request.

Upon receiving the HTTP POST message, the EAS shall process the Location Notification.

#### 5.3.2.5	Eees_UELocation_UpdateSubscription

##### 5.3.2.5.1	General

This service operation is used by the EAS to update its location information subscription at the EES.

##### 5.3.2.5.2	EAS updating continuous UE(s) location reporting subscription at EES using Eees_UELocation_UpdateSubscribe operation

To update continuous UE(s) location information reporting subscription at the EES, the EAS shall send a HTTP PATCH or PUT message to the EES on resource URI identifying the "Individual Location Information" Subscription resource representation, as specified in clause 8.2.2.3.3.2 for HTTP PATCH message and in clause 8.2.2.3.3.3 for HTTP PUT message.

The PATCH message includes the parameters (location QoS, location granularity, Notification Destination, Reporting requirements and proposed expiry time) that need to be replaced in the existing subscription resource.

The PUT message shall replace all the properties of the existing resource with the location subscription information in the request. The request shall not replace the easId, ueId and groupId properties of the existing resource.

Upon receiving the HTTP PATCH or PUT message from the EAS, the EES shall:

1. check the update subscription message from the EAS to see if the EAS is authorized to modify the requested subscription resource;

2. if the EAS is authorized to update the location information subscription and the easId of the requesting EAS and the easId in the resource match, then the EES shall;

a. update the resource identified by Resource URI of the EAS location information subscription with the updated  information received in the HTTP PATCH or PUT request message;

b. return the updated EAS Location information subscription in the response. In the response message, the EES may provide an updated expiration time to indicate to the EAS when the updated subscription will automatically expire.

If the expiration time is provided, then to maintain the subscription, the EAS shall send a update subscription prior to subscription expiry time. If the update subscription request is not sent before the expiry time, then the EES shall treat EAS subscription as unsubscribed and remove the corresponding EAS location information subscription resource.

#### 5.3.2.6	Eees_UELocation_Unsubscribe

##### 5.3.2.6.1	General

This service operation is used by the EAS to unsubscribe from an existing UE(s) location information subscription.

##### 5.3.2.6.2	EAS unsubscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Unsubscribe operation

To unsubscribe its location information subscription from the EES, the EAS shall send HTTP DELETE message to the EES, on the resource URI identifying the "Individual Location Information Subscription" resource representation as specified in clause 8.2.2.3.3.4. Upon receiving the HTTP DELETE request, the EES shall:

1. verify the identity of the EAS and check if the EAS is authorized to unsubscribe the Individual Location Information Subscription resource;

2. if the EAS is authorized to unsubscribe the Individual Location Information Subscription resource, then the EES shall unsubscribe the EAS subscription identified by the subscriptionId from the EES and delete the resource representing Individual Location Information Subscription resource represented by subscriptionId;

3. return the "204 Not Content" message to the EAS, indicating the successful removal of the subscription information and may unsubscribe to the corresponding 3GPP core network service subscriptions.

## 5.4	Eees_UEIdentifier Service

### 5.4.1	Service Description

The Eees_UEIdentifier API, as defined in 3GPP TS 23.558 [2], allows an Edge Application Server via Eees interface to obtain an identifier uniquely identifying a UE from the EES.

Editor’s Note: Details about EAS security credentials, verification and authorization of Eees_UEIdentifier service operation, by the EES, to be aligned with security aspects defined by SA3.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The service operation defined for Eees_UEIdentifier API is shown in the table 5.4.2.1-1.

Table 5.4.2.1-1: Operations of the Eees_UEIdentifier API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_UEIdentifier_Get | This service operation is used by the EAS to request UE identifier from a given EES. | EAS |

#### 5.4.2.2	Eees_UEIdentifier_Get

##### 5.4.2.2.1	General

This service operation is used by EAS to obtain a UE’s identifier (UE ID, as specified in 3GPP TS 23.558 [2]) from a given EES.

##### 5.4.2.2.2	EAS obtaining UE identifier from EES using Eees_UEIdentifier_Get operation

To obtain an UE’s Identifier from the EES, the EAS shall send a HTTP POST message to the EES on the "Identifier Information of UEs" resource as specified in clause 8.3.2.2.4.2. The request body of the POST message includes the information about the user or UE available with EAS, for which the UE identifier is requested.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1. Process the EAS UE Identifier request;

2. verify the identity of the EAS and check if the EAS is authorized to obtain the UE Identifier;

3. if the EAS is authorized to obtain the UE’s Identifier information, then the EES shall;

a.  obtain the UE identifier based on the user information received;

b.  return the UE identifier to the EAS.

Editor’s Note: How the user’s consent is obtained to share the UE identifier with EAS is FFS and to be aligned with security aspects defined by SA3.

Editor’s Note: How EES determines the UE identifier is FFS and to be aligned with stage 2 aspects.

Editor’s Note: It is FFS, usage of GET method or custom operation without resource, for Eees_UEIdentifier_Get service operation is based on clarification from CT4.

## 5.5	Eees_AppClientInformation Service

### 5.5.1	Service Description

The Eees_AppClientInformation API, as defined in 3GPP TS 23.558 [2], allows an Edge Application Server via Eees interface to obtain the information about the capabilities of the ACs based on certain filter criteria.

Editor’s Note: Details about EAS security credentials, verification and authorization of Eees_AppClientInformation service operations, by the EES, to be aligned with security aspects defined by SA3.

### 5.5.2	Service Operations

#### 5.5.2.1	Introduction

The service operation defined for Eees_AppClientInformation API is shown in the table 5.5.2.1-1.

Table 5.5.2.1-1: Operations of the Eees_AppClientInformation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_AppClientInformation_Subscribe | This service operation is used by the EAS to subscribe to EES, for reporting of AC information. | EAS |
| Eees_AppClientInformation_Notify | This service operation is used by the EES to notify the EAS about the AC information. | EES |
| Eees_AppClientInformation_UpdateSubscription | This service operation is used by the EAS to update its subscription at EES, for reporting of AC information. | EAS |
| Eees_AppClientInformation_Unsubscribe | This service operation is used by the EAS to remove its subscription from EES, for reporting of AC information. | EAS |

#### 5.5.2.2	Eees_AppClientInformation_Subscribe

##### 5.5.2.2.1	General

This service operation is used by the EAS to subscribe for AC(s) information reporting.

##### 5.5.2.2.2	EAS subscribing to AC information reporting from EES using Eees_AppClientInformation_Subscribe operation

To subscribe to AC information reporting at the EES, the EAS shall send a HTTP POST message to the EES on the "Application Client Information Subscriptions" resource. The body of the POST message shall include EAS identifier, criteria information to filter the AC(s) information, Notification Destination URI and may include proposed expiry time of the subscription and reporting requirements, as specified in clause 8.4.2.2.3.1.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1. Process the EAS AC information subscription request;

2. verify the identity of the Edge Application Server and check if the EAS is authorized to subscribe for the AC information reporting;

3. if the EAS is authorized to subscribe for the AC information reporting, then the EES shall;

a.  create a new resource with the AC Information Subscription as specified in clause 8.4.2.1;

b.  return the EAS’s AC information subscription information, the resource URI of the EAS AC information subscription, in the response message.

The EES determines the matching AC information corresponding to the filter criteria information in the subscription request to compose the AC information to the EAS.

If the expiration time is provided, then to maintain the registration, the EAS shall send a subscription update request (as described in clause 5.5.2.4) prior to the expiration time. If the subscription update request is not sent before the expiry time, then the EES shall treat the subscription as unsubscribed and remove the corresponding EAS’s Individual Application Client Information Subscription resource.

#### 5.5.2.3	Eees_AppClientInformation_Notify

##### 5.5.2.3.1	General

This service operation is used by the EES to send AC information notifications to the EAS.

##### 5.5.2.3.2	EES notifying the AC information to EAS using Eees_AppClientInformation_Notify operation

The EES determines to notify the EAS with the AC information matching the filter criteria, when the AC information updates are available to EES, e.g. EES receives registration request from EEC.

To notify the AC(s) information, the EES shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include "ACInfoNotification". "ACInfoNotification" includes AC(s) information matching the filter criteria. The AC(s) information consist of AC(s) profiles, UE identifiers hosting the AC(s) and the location information of the UE(s) hosting the AC(s).

Upon receiving the HTTP POST message, the EAS shall process the AC Information Notification and shall respond to the EES with "204 No Content" message.

#### 5.5.2.4	Eees_AppClientInformation_UpdateSubscription

##### 5.5.2.4.1	General

This service operation is used by the EAS to update its AC information subscription at the EES.

##### 5.5.2.4.2	EAS updating AC information reporting subscription at EES using Eees_AppClientInformation_UpdateSubscribe operation

To update AC information reporting subscription at the EES, the EAS shall send a HTTP PATCH or PUT message to the EES on resource URI identifying the "Individual Application Client Information" Subscription resource representation, as specified in clause 8.4.2.3.3.2 for HTTP PATCH message and in clause 8.4.2.3.3.3 for HTTP PUT message.

The PATCH message includes the parameters (AC filter criteria, Notification Destination, Reporting requirements and proposed expiry time) that need to be replaced in the existing subscription resource.

The PUT message shall replace all the properties of the existing resource with the AC subscription information in the request. The request shall not replace the "easId" property of the existing resource.

Upon receiving the HTTP PATCH or PUT message from the EAS, the EES shall:

1. check the update subscription message from the EAS to see if the EAS is authorized to modify the requested subscription resource;

2. if the EAS is authorized to update the AC information subscription and the easId of the requesting EAS and the easId in the resource match, then the EES shall;

a. update the resource identified by Resource URI of the EAS AC information subscription with the updated  information received in the HTTP PATCH or PUT request message;

b. return the updated EAS AC information subscription in the "200 OK" response message or respond with the "204 No Content" message indicating to the EAS that the EAS AC information subscription is updated successfully. In the response message, the EES may provide an updated expiration time to indicate to the EAS when the updated subscription will automatically expire.

If the expiration time is provided, then to maintain the subscription, the EAS shall send an update subscription prior to subscription expiry time. If the update subscription request is not sent before the expiry time, then the EES shall treat EAS subscription as unsubscribed and remove the corresponding EAS AC information subscription resource.

#### 5.5.2.5	Eees_AppClientInformation_Unsubscribe

##### 5.5.2.5.1	General

This service operation is used by the EAS to unsubscribe from an existing AC information subscription.

##### 5.5.2.5.2	EAS unsubscribing to AC information reporting from EES using Eees_AppClientInformation_Unsubscribe operation

To unsubscribe its AC information subscription from the EES, the EAS shall send HTTP DELETE message to the EES, on the resource URI identifying the "Individual Application Client Information Subscription" resource representation as specified in clause 8.4.2.3.3.4. Upon receiving the HTTP DELETE request, the EES shall:

1. verify the identity of the EAS and check if the EAS is authorized to unsubscribe the Individual Application Client Information Subscription resource;

2. if the EAS is authorized to unsubscribe the Individual Application Client Information Subscription resource, then the EES shall unsubscribe the EAS subscription identified by the "subscriptionId" from the EES and delete the resource representing Individual Application Client Information Subscription resource represented by "subscriptionId";

3. return the "204 Not Content" message to the EAS, indicating the successful removal of the subscription information.

## 5.6	Eees_SessionWithQoS Service

### 5.6.1	Service Description

The Eees_SessionWithQoS API, as defined in 3GPP TS 23.558 [2], allows an Edge Application Server via Eees interface to support the setup of a data session between AC and EAS with a specific QoS and the modification of the QoS of this data session.

### 5.6.2	Service Operations

#### 5.6.2.1	Introduction

The service operation defined for Eees_SessionWithQoS API is shown in the table 5.6.2.1-1.

Table 5.6.2.1-1: Operations of the Eees_SessionWithQoS API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_SessionWithQoS_Create | The service operation is used by the EAS to request reservation of resources for a data session between AC and EAS with a specific QoS and to subscribe to certain session with user plane event notifications. | EAS |
| Eees_SessionWithQoS_Update | The service operation is used by the EAS to request a modification of the QoS of the data session between AC and EAS. | EAS |
| Eees_SessionWithQoS_Revoke | The service operation is used by the EAS to revoke the data session between AC and EAS with a specific QoS and to unsubscribe to the related session with user plane event notifications. | EAS |
| Eees_SessionWithQoS_Notify | The service operation is used by the EES to notify the EAS about a user plane event associated with the established session between an AC and the EAS. | EES |

#### 5.6.2.2	Eees_SessionWithQoS_Create

##### 5.6.2.2.1	General

This service operation is used by EAS to request reservation of resources for a data session between AC and EAS with a specific QoS and to subscribe to certain session with user plane event notifications.

##### 5.6.2.2.2	EAS requesting reservation of resources for a data session between AC and EAS with specific QoS using Eees_SessionWithQoS operation

To request establishment of a data session between AC and EAS with a specific QoS, the EAS shall send a HTTP POST message to the Edge Enabler Server on the "Sessions with QoS" resource as specified in clause 8.5.2.2.3.1. The body of POST message shall include the EAS identifier, only one of the UE’s IP address or the Identifier of the UE or the identifier of the group uniquely identifying a group of UEs, IP flow description, and at least one of requested QoS reference. The body of POST message may include a list of associated events which the EAS subscribes, and if the event list is included, a Notification Destination URI shall also be provided.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1. Process the EAS Session with QoS Create request;

2. verify the identity of the EAS and check if the EAS is authorized to request reservation of resources for a data session between AC and EAS with a specific QoS;

3. if the EAS is authorized, then the EES shall;

a.	create a new resource "Individual Session with QoS";

b.	if the request is for a group of UEs identified by group ID (i.e., via the "intGrpId" or "extGrpId") or for a single UE identified via the "ueId" attribute, interact with the SCEF (as specified in 3GPP TS 29.122 [6]) or the NEF (as specified in 3GPP TS 29.522 [10]) by invoking the MonitroingEvent API with the monitoring type sets to "PDN_CONNECTIVITY_STATUS" to request to be notified when the 3GPP network detects the UE’s PDN connection or PDU session is set up or torn down. If the IP address for the single UE or, the IP address(es) for one or more UEs within the group are received from the 3GPP network, then execute step 3c; and

c.	if the request is for a single UE identified by the IP address or the IP address is obtained in step 3b, interact directly with the PCRF (as specified in 3GPP TS 29.214 [15]) or the PCF (as specified in 3GPP TS 29.514 [16]), or via the SCEF (as specified in 3GPP TS 29.122 [6]) or the NEF (as specified in 3GPP TS 29.522 [10]) by invoking the AsSessionWithQoS API, to provide the specific QoS information to the PCF.

4.	upon receipt of successful response from 3GPP network, respond to the EAS with "201 Created" and include the session with QoS context information. The new created resource URI shall also be included in the Location header field of the HTTP response message.in the response message.

#### 5.6.2.3	Eees_SessionWithQoS_Update

##### 5.6.2.3.1	General

This service operation is used by EAS to request updating QoS of a data session between AC and EAS and to modify the subscription of the session with user plane event notifications.

##### 5.6.2.3.2	EAS updating QoS of a data session between AC and EAS using Eees_SessionWithQoS_Update operation

To request modification of the QoS of the data session between AC and EAS, the EAS shall send a HTTP PATCH or PUT message to the EES on resource URI "Individual Session with QoS" resource as specified in clause 8.5.2.3.3.1 for HTTP PATCH message and in clause 8.5.2.3.3.2 for HTTP PUT message.

The PUT message shall replace all the QoS settings of the data session in the existing context. The request shall not change the values of the "easId", "ueId", "ueIpv4Addr", "ueIpv6Addr", "ipDomain", "intGrpId", "extGrpId", "dnn" and/or "snssai" attributes.

Upon receiving the HTTP PATCH or PUT message from the EAS, the EES shall:

1.	check the update of the existing Individual Session with QoS from the EAS is authorized or not;

2.	if authorized, and the resource exists, then the EES shall;

a.	interact with the 3GPP network to update the associated data session; and

b.	upon receipt of successful response from 3GPP network, respond to the EAS with "204 No Content", or "200 OK" with the updated Individual session with QoS context in the response message.

#### 5.6.2.4	Eees_SessionWithQoS_Revoke

##### 5.6.2.4.1	General

This service operation is used by EAS to revoke the data session between AC and EAS with a specific QoS and to unsubscribe to the related session with user plane event notifications.

##### 5.6.2.4.2	EAS revoking QoS of a data session between AC and EAS using Eees_SessionWithQoS_Revoke operation

To revoke the data session between AC and EAS with a specific QoS and unsubscribe the user plane event notifications, the EAS shall send a HTTP DELETE message to the EES targeting the Individual Session with QoS resource as specified in clause 8.5.2.3.3.3. Upon receiving the HTTP DELETE request, the EES shall:

1.	verify the identity of the EAS and check if the EAS is authorized to revoke the data session between AC and EAS with a specific QoS;

2.	if the EAS is authorized and the resource exists, then the EES shall interact with the 3GPP network to delete the associated data session.

3.	upon receipt of successful response from 3GPP network, delete the Individual Session with QoS resource corresponding to the individual Session with QoS; and

4.	return "204 No Content" message to the EAS, indicating the successful removal.

#### 5.6.2.5	Eees_SessionWithQoS_Notify

##### 5.6.2.5.1	General

This service operation is used by EES to send user plane event notification information of the data session between AC and EAS with a specific QoS to the EAS.

##### 5.6.2.5.2	EAS notifying QoS of a data session between AC and EAS using Eees_SessionWithQoS_Notify operation

The EES determines to notify the user plane event notification information to the EAS, when the EES receives the notification of the user plane event information from the 3GPP core network.

To notify the user plane event notification information, the EES shall send an HTTP POST message using the Notification Destination URI received during the Creation of resource request. The body of POST message shall include the event report information (e.g., resource allocation outcome or information that the QoS targets can no longer (or can again) be fulfilled).

Upon receiving the HTTP POST message, the EAS shall process the event report information and return "204 No Content" message to the EES.

## 5.7	Eees_TargetEASDiscovery Service

### 5.7.1	Service Description

The Eees_TargetEASDiscovery API, as defined in 3GPP TS 23.558 [2], allows S-EAS and S-EES to discover the T-EAS information from the EES.

Editor’s Note: Details about EAS security credentials, verification and authorization of Eees_TargetEASDiscovery service operation, by the EES, to be aligned with security aspects defined by SA3.

### 5.7.2	Service Operations

#### 5.7.2.1	Introduction

The service operation defined for Eees_TargetEASDiscovery API is shown in the table 5.7.2.1-1.

Table 5.7.2.1-1: Operations of the Eees_TargetEASDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_TargetEASDiscovery_Request | This service operation is used by the S-EAS and S-EES to discover the target EAS (T-EAS) information from a given EES. | EAS, EES |

#### 5.7.2.2	Eees_TargetEASDiscovery_Request

##### 5.7.2.2.1	General

This service operation is used by S-EAS and S-EES to fetch T-EAS information, as specified in 3GPP TS 23.558 [2]), from a given EES.

##### 5.7.2.2.2	EAS and EES fetching the T-EAS information from the EES using Eees_TargetEASDiscovery_Request operation

Editor’s Note: The service description of this service operation is FFS.

## 5.8	Eees_ACRManagementEvent Service

### 5.8.1	Service Description

The Eees_ACRManagementEvent API, as defined in 3GPP TS 23.558 [2], allows an Edge Application Server via Eees interface to support (un)subscription to the notifications of ACR management events.

### 5.8.2	Service Operations

#### 5.8.2.1	Introduction

The service operation defined for Eees_ACRManagementEvent API is shown in the table 5.8.2.1-1.

Table 5.8.2.1-1: Operations of the Eees_ACRManagementEvent API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_ACRManagementEvent_Subscribe | The service operation is used by the EAS to subscribe to notifications of ACR management event(s). | EAS |
| Eees_ACRManagementEvent_UpdateSubscription | The service operation is used by the EAS to request a modification of an existing subscription of notifications of ACR management event(s). | EAS |
| Eees_ACRManagementEvent_Unsubscribe | The service operation is used by the EAS to unsubscribe to an existing subscription of notifications of ACR management event(s). | EAS |
| Eees_ACRManagementEvent_Notify | The service operation is used by the EES to notify the EAS about ACR management event(s) if detected. | EES |

#### 5.8.2.2	Eees_ACRManagementEvent_Subscribe

##### 5.8.2.2.1	General

This service operation is used by EAS to subscribe to notifications of ACR management events.

##### 5.8.2.2.2	EAS requesting to get notifications of ACR management events using Eees_ACRManagementEvent_Subscribe service operation

In order to subscribe to notifications of ACR management events, the EAS shall send a HTTP POST message to the EES targeting the "ACR Management Events Subscriptions" resource as specified in clause 8.7.2.2.3.1. The body of POST message shall include the AcrMgntEventsSubscription data type as specified in clause 8.7.5.2.2.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1.	Process the ACR management events subscription request;

2.	verify the identity of the EAS and check if the EAS is authorized to get notifications of ACR management events;

3.	if the EAS is authorized, then if one of the subscribed event(s) is UP path change event, the EES shall,

- if already successful subscribed to the 3GPP core network and there is local cached UP path change event information, perform step 4, and include the local cached UP path change event information in the HTTP POST response; or

- if not subscribed to the 3GPP core network yet, interact with the NEF by invoking the TrafficInfluence API as specified in 3GPP TS 29.522 [10] to request to be notified of the UP path change event. Upon receipt of successful response from the NEF, then the EES shall perform step 4.

The EES may interact with the NEF by invoking the AnalyticsExposure API as specified in 3GPP TS 29.522 [10] to subscribe to notifications of, or retrieve the UE mobility and UE communication events.

4.	create a new resource "Individual ACR Management Events Subscription" and respond to the EAS with "201 Created" and include the Individual ACR Management Events Subscription information. The new created resource URI shall also be included in the Location header field of the HTTP response message;

5.	If the target UE and the 3GPP network supports mobility between 5GC and EPC, interact with the SCEF+NEF by invoking the MonitroingEvent API as specified in 3GPP TS 29.122 [6] with the monitoring type sets to "API_SUPPORT_CAPABILITY" or interact with the CAPIF core function by invoking the CAPIF_Events_API service as specified in 3GPP TS 29.222 [17] to request to be notified the availability of the TrafficInfluence API.

#### 5.8.2.3	Eees_ACRManagementEvent_UpdateSubscription

##### 5.8.2.3.1	General

This service operation is used by EAS to request updating an existing Individual ACR Management Events Subscription.

##### 5.8.2.3.2	EAS updating an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_UpdateSubscription service operation

To request modification of an existing Individual ACR Management Events Subscription, the EAS shall send a HTTP PATCH request (for partial modification) or PUT request (for fully replacement) message to the EES on resource URI "Individual ACR Management Events Subscription" resource as specified in clause 8.7.2.3.3.1 for HTTP PATCH message and in clause 8.7.2.3.3.2 for HTTP PUT message.

Upon receiving the HTTP PATCH or PUT message from the EAS, the EES shall:

1.	check the update of the existing Individual ACR Management Events Subscription from the EAS is authorized or not;

2.	if the EAS is authorized and the resource exists, then if one of the subscribed event(s) is UP path change event, the EES shall;

- if already successful subscribed to the 3GPP core network and there is local cached UP path change event information, perform step 3, and include the local cached UP path change event information in the HTTP PUT or PATCH response; or

- if not subscribed to the 3GPP core network yet, interact with the NEF by invoking the TrafficInfluence API as specified in 3GPP TS 29.522 [10] to request to be notified of the UP path change event. Upon receipt of successful response from the NEF, then the EES shall perform step 3.

The EES may interact with the NEF by invoking the AnalyticsExposure API as specified in 3GPP TS 29.522 [10] to subscribe to notifications of, or retrieve the UE mobility and UE communication events.

3.	update the existing resource "Individual ACR Management Events Subscription", respond to the EAS with "204 No Content", or "200 OK" with the updated Individual ACR Management Events Subscription message.

#### 5.8.2.4	Eees_ACRManagementEvent_Unsubscribe

##### 5.8.2.4.1	General

This service operation is used by EAS to revoke an existing Individual ACR Management Events Subscription.

##### 5.8.2.4.2	EAS revoking an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_Unsubscribe service operation

To revoke an existing Individual ACR Management Events Subscription, the EAS shall send a HTTP DELETE message to the EES targeting the "Individual ACR Management Events Subscription" resource as specified in clause 8.7.2.3.3.3. Upon receiving the HTTP DELETE request, the EES shall:

1.	verify the identity of the EAS and check if the EAS is authorized to revoke the data session between AC and EAS with a specific QoS;

2.	if the EAS is authorized and the resource exists, then the EES shall interact with the 3GPP network to delete the associated Subscription.

3.	upon receipt of successful response from 3GPP network, delete the Individual ACR Management Events Subscription; and return "204 No Content" message to the EAS, indicating the successful removal.

#### 5.8.2.5	Eees_ACRManagementEvent_Notify

##### 5.8.2.5.1	General

This service operation is used by EES to send ACR management event notification information to the EAS.

##### 5.8.2.5.2	EAS notifying ACR management events using Eees_ACRManagementEvent_Notify operation

In order to notify the ACR management event information, the EES shall send an HTTP POST message using the Notification Destination URI received during the creation or modification of subscription. The body of POST message shall include the AcrMgntEventsNotification data type as specified in clause 8.7.5.2.4.

Upon receiving the HTTP POST message, the EAS shall process the event report information and return "204 No Content" message to the EES.

## 5.9	Eees_SelectedTargetEAS Service

### 5.9.1	Service Description

The Eees_SelectedTargetEAS API, as defined in 3GPP TS 23.558 [2], allows S-EAS to declare to the S-EES about the T-EAS selected by the S-EAS.

Editor’s Note: Details about EAS security credentials, verification and authorization of Eees_SelectedTargetEAS by the EES, to be aligned with security aspects defined by SA3.

### 5.9.2	Service Operations

#### 5.9.2.1	Introduction

The service operation defined for Eees_SelectedTargetEAS API is shown in the table 5.9.2.1-1.

Table 5.9.2.1-1: Operations of the Eees_SelectedTargetEAS API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_SelectedTargetEAS_Declare | This service operation is used by the S-EAS to inform the S-EES about selection of the T-EAS. | EAS |

#### 5.9.2.2	Eees_SelectedTargetEAS_Declare

##### 5.9.2.2.1	General

This service operation is used by S-EAS to declare the selected T-EAS information to the S-EES, as specified in 3GPP TS 23.558 [2].

##### 5.9.2.2.2	S-EAS informing the S-EES about the selected T-EAS using Eees_SelectedTargetEAS_Declare operation

To declare the information about the selected T-EAS to the S-EES, the S-EAS shall send a HTTP POST message to the S-EES on the custom URI "/declare" as specified in clause 8.8.3.2. The request body of the POST message includes the information about the UE, selected EAS identifier and end point information.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1. Process the EAS request;

2. verify the identity of the EAS and check if the EAS is authorized to declare the T-EAS information;

3. if the EAS is authorized to declare T-EAS information, then the EES shall;

a.  send successful declaration response message to S-EAS;

b.  additionally, the S-EES determines the T-EES based on the declared T-EAS information and may notify the EEC using the Eees_ACREvents_Notify service operation as specified in 3GPP TS 24.558 [14].

## 5.x	<Eees_xxx> Service

Add a copy of this clause for a new API, adding the overview of the API, Details of service operations, detailed description of each service operation. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

### 5.x.1	Service Description

This clause will provide a general description of the related service, include a description of the functional elements involved in the invocation of the service.

### 5.x.2	Service Operations

One clause per service operation. This clause will include a description of the different service operations supported by the service.

#### 5.x.2.1	Introduction

The service operation defined for <API Name – Eees_xxx> API is shown in the table 5.x.2.1-1.

Table 5.x.2.1-1: Operations of the <API Name> API

| Service operation name | Description | Initiated by |
| --- | --- | --- |

#### 5.x.2.2	<Service operation 1>

##### 5.x.2.2.1	General

Provide the general description of the service operation.

##### 5.x.2.2.2	<Description> <Service Operation Name> operation

#### 5.x.2.3	<Service operation 2>

And so on if there are more than 2 service operations to be described for the service.

# 6	Services offered by Edge Configuration Server

## 6.1	Introduction

This clause will provide the list of Edge Configuration Server services with their respective service operations.

The table 6.1-1 lists the Edge Configuration Server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 6.1-1: List of ECS Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_EESRegistration | Request | Request/Response | EES |
|  | Update | Request/Response | EES |
|  | Deregister | Request/Response | EES |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES |

Table 6.1-2 summarizes the corresponding Edge Configuration Server APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |

## 6.2	Eecs_EESRegistration Service

### 6.2.1	Service Description

The Eecs_EESRegistration API, as defined in 3GPP TS 23.558 [2], allows an Edge Enabler Server via Eecs interface to register, update its registration and deregister at a given Edge Configuration Server.

Editor’s Note: Details about EES security credentials, verification and authorization of Eecs_EESRegistration Request, Update and Delete, by the ECS, to be aligned with security aspects defined by SA3

### 6.2.2	Service Operations

#### 6.2.2.1	Introduction

The service operation defined for Eecs_EESRegistration API is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the Eecs_EESRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_EESRegistraiton_Request | This service operation is used by the EES to register itself to a given ECS. | EES |
| Eecs_EESRegistraiton_Update | This service operation is used by the EES to update its registration information at ECS. | EES |
| Eecs_EESRegistraiton_Deregister | This service operation is used by the EES to deregister itself from a given ECS. | EES |

#### 6.2.2.2	Eecs_EESRegistration_Request

##### 6.2.2.2.1	General

This service operation is used by EES to register itself to a given ECS.

##### 6.2.2.2.2	EES registering to ECS using Eecs_EESRegistration_Request operation

To register itself as an Edge Enabler Server at the ECS, the EES shall send an HTTP POST message to the Edge Configuration Server on the "EES Registrations" collection resource. The body of the HTTP POST message shall include the EES profile information, may include proposed expiration time for the registration, as specified in clause 9.1.2.2.3.1.

Upon receiving the HTTP POST message from the EES, the ECS shall:

1. Process the EES registration request information;

2. verify the identity of the Edge Enabler Server and check if the EES is authorized to register itself at ECS ;

3. if the EES is authorized to register to ECS, then the ECS shall;

a.  store the EES profile and create a new resource with the EES registration information as specified in clause 9.1.2.1;

b. return the EES registration information, the resource URI of the EES registration information, in the response message. The response message may include expiration time to indicate when the EES registration will automatically expire.

If the expiration time is provided, then to maintain the registration, the EES shall send a registration update request (as described in clause 6.2.2.3) prior to the expiration time. If the registration update request is not sent before the expiry time, then the ECS shall treat the EES as deregistered and remove the corresponding EES registration resource.

#### 6.2.2.3	Eecs_EESRegistration_Update

##### 6.2.2.3.1	General

This service operation is used by EES to update its registration information at a given ECS.

##### 6.2.2.3.2	EES updating registration information using Eecs_EESRegistration_Update operation

To update the EES registration information at the ECS, the EES shall send a HTTP PUT or PATCH message to the Edge Configuration Server on resource URI identifying the individual EES registration resource representation, as specified in clause 9.1.2.3.3.2 for HTTP PUT message and in clause 9.1.2.3.3.4 for HTTP PATCH message. The HTTP PUT message shall replace all properties in the existing resource with the EES registration information in the request. The body of the HTTP PUT message shall include the EES profile information, may include proposed expiration time to update the registration. This request shall not replace the eesId property of the existing resource.

The HTTP PATCH message includes parameters (EES Profile, expiry time) that need to be replaced in the existing Individual EAS registration resource.  This request shall not replace the eesId property of the existing resource.

Upon receiving the HTTP PUT or PATCH message from the EES, the ECS shall:

1. check the registration update message from the EES to see if the EES is authorized to modify the requested registration resource;

2. if the EES is authorized to update the registration information and the eesId information in the request and the resource match, then the ECS shall;

a. update the resource identified by Resource URI of the EES registration information with the updated EES registration information received in the HTTP PUT or PATCH request message;

b. return the updated EES registration information in the "200 OK" response message or respond with the "204 No Content" message indicating to the EES that the EES registration information is updated successfully. In the response message, the ECS may provide an updated expiration time to indicate to the EES when the updated registration will automatically expire.

If the expiration time is provided, then to maintain the registration, the EES shall send a registration update prior to registration expiry time. If the registration update request is not sent before the expiry time, then the ECS shall treat EES as deregistered and remove the corresponding EES registration resource.

#### 6.2.2.4	Eecs_EESRegistration_Deregister

##### 6.2.2.4.1	General

This service operation is used by EES to deregister itself from a given ECS.

##### 6.2.2.4.2	EES deregistering from ECS using Eecs_EESRegistration_Deregister operation

To deregister itself from the ECS, the EES shall send HTTP DELETE message to the ECS, on the resource URI identifying the individual EES registration resource representation as specified in clause 9.1.2.3.3.3. Upon receiving the HTTP DELETE request, the ECS shall:

1. verify the identity of the EES and check if the EES is authorized to deregister the EES registration information;

2. if the EES is authorized to deregister the EES registration information, then the ECS shall deregister the EES profile from the ECS and delete the resource representing EES registration information.

3. return the "204 Not Content" message to the EES, indicating the successful deregistration of the EES information.

## 6.3	Eecs_TargetEESDiscovery Service

### 6.3.1	Service Description

The Eecs_TargetEESDiscovery API, as defined in 3GPP TS 23.558 [2], allows an Edge Enabler Server via Eecs interface to retrieve the target EES (T-EES) information at a given ECS.

Editor’s Note: Details about EES security credentials, verification and authorization of Eecs_TargetEESDiscovery_Request, by the ECS, to be aligned with security aspects defined by SA3

### 6.3.2	Service Operations

#### 6.3.2.1	Introduction

The service operation defined for Eecs_TargetEESDiscovery API is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations of the Eecs_TargetEESDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_TargetEESDiscovery_Request | This service operation is used by the S-EES to retrieve the T-EES information. | EES |

#### 6.3.2.2	Eecs_TargetEESDiscovery_Request

##### 6.3.2.2.1	General

This service operation is used by the S-EES to retrieve the T-EES information from the ECS.

##### 6.3.2.2.2	EES fetching the T-EES information from the ECS using Eecs_TargetEESDiscovery_Request operation

To retrieve the T-EES information from the ECS, the S-EES shall send a HTTP GET message to the Edge Configuration Server on the "EES Profiles" collection resource as specified in clause 9.2.2.2.3.1. The S-EES includes the discovery filter information in the query parameters of the GET message, to assist the ECS to determine the T-EES that has the EAS available to serve the given AC in the UE.  The query parameters shall include identifier of the S-EES, identifier of the S-EAS and may include target DNAI, UE location information, and UE identity.

Upon receiving the HTTP GET message from the EES, the ECS shall:

1. Process the EES request information;

2. verify the identity of the Edge Enabler Server and check if the EES is authorized to retrieve the T-EES information;

3. if the EES is authorized to retrieve the T-EES information, then the ECS shall;

a. determine the T-EES(s) as per UE location information, target DNAI and S-EAS in the request. If UE location information is not included or not known to the ECS, then the ECS may fetch the UE location information from the 3GPP core network as specified in 3GPP TS 29.522 [10];

b. returns the 200 OK message including the EDN configuration and the list of T-EES(s) information to the S-EES. The list of T-EES(s) includes the endpoint information to reach the T-EES(s).

## 6.x	<Eecs_xxx> Service

Add a copy of this clause for a new API, adding the overview of the API, Details of service operations, detailed description of each service operation. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

### 6.x.1	Service Description

This clause will provide a general description of the related service, include a description of the functional elements involved in the invocation of the service.

### 6.x.2	Service Operations

One clause per service operation. This clause will include a description of the different service operations supported by the service.

#### 6.x.2.1	Introduction

The service operation defined for <API Name – Eecs_xxx> API is shown in the table 6.x.2.1-1.

Table 6.x.2.1-1: Operations of the <API Name> API

| Service operation name | Description | Initiated by |
| --- | --- | --- |

#### 6.x.2.2	<Service operation 1>

##### 6.x.2.2.1	General

Provide the general description of the service operation.

##### 6.x.2.2.2	<Description> <Service Operation Name> operation

#### 6.x.2.3	<Service operation 2>

And so on if there are more than 2 service operations to be described for the service

# 7	Information applicable to several APIs

This clause will provide the design aspects that are common for several APIs.

## 7.1	General

The EDGEAPP APIs as specified in this document allow secure access to the capabilities provided by the functional entities, Edge Enabler Server and Edge Configuration Server.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.558 [2].

Several design aspects, as mentioned in the following clauses, are specified in 3GPP TS 29.122 [6] and referenced by this specification.

## 7.2	Data Types

### 7.2.1	General

This clause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent clauses.

In addition, data types that are defined in OpenAPI 3.0.0 Specification [3] can also be referenced from data structures defined in the subsequent clauses.

NOTE:	As a convention, data type’s names in the present specification are with an upper-case letter in the beginning. Parameters are with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI 3.0.0 Specification [3] can use a lower-case case letter in the beginning for consistency.

Table 7.2.1-1 specifies data types re-used by APIs in this specification from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the APIs of this specification.

Table 7.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.122 [6] |  |

### 7.2.2	Referenced structured data types

Table 7.2.2-1 lists structured data types defined in this specification referenced by multiple services:

Table 7.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |

### 7.2.3	Referenced simple data types and enumerations

Following simple data types defined in Table 7.2.3-1 are applicable to several APIs in this document:

Table 7.2.3-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |

## 7.3	Usage of HTTP

For all the APIs in this document, the usage of HTTP shall be as specified in clause 5.2.2 of 3GPP TS 29.122 [6], with the clarification that SCA/AS is the functional entity invoking an EDGEAPP API.

Editor’s note: Usage of HTTP over TLS is based on security aspects defined by SA3.

## 7.4	Content type

The content type for the HTTP messages shall be as specified in clause 5.2.3 of 3GPP TS 29.122 [6].

## 7.5	URI structure

### 7.5.1	Resource URI structure

The resource URI structure of all the APIs specified in this document shall be as specified in clause 5.2.4 of 3GPP TS 29.122 [6].

### 7.5.2	Custom operations URI structure

The custom operation definition is in Annex C of 3GPP TS 29.501 [5].

The URI of a custom operation which is associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>/<custOpName>

Custom operations can also be associated with the service instead of a resource. The URI of a custom operation which is not associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<custOpName>

In the above URI structures, "apiRoot", "apiName", "apiVersion" and "apiSpecificResourceUriPart" are as defined in clause 7.5.1 and "custOpName" represents the name of the custom operation as defined in clause 5.1.3.2 of 3GPP TS 29.501 [5].

## 7.6	Notifications

The functional entities

-	shall support the delivery of notifications using a separate HTTP connection towards an address;

-	may support testing delivery of notifications; and

-	may support the delivery of notification using WebSocket protocol (see IETF RFC 6455 [7]),

as described in 3GPP TS 29.122 [6], with the following clarifications:

-	the SCEF is EES for Eees APIs; and

-	the SCS/AS is the Subscriber entity invoking an EDGEAPP API.

Editor’s Note: Applicability of notifications clause for ECS entity to be aligned if Eecs APIs support subscribe/notify type of service operation.

## 7.7	Error handling

Response bodies for error handling, as described in 3GPP TS 29.122 [6], are applicable to all APIs in the present specification unless specified otherwise, with the following clarifications:

-	the SCEF is EES for Eees APIs or ECS for Eecs APIs ; and

-	the SCS/AS is the functional entity invoking an EDGEAPP API

## 7.8 	Feature negotiation

The functional entity invoking an API (i.e. the EAS server) and the EDGEAPP server use feature negotiation procedures defined in 3GPP TS 29.122 [6] to negotiate the supported features, with the following clarifications:

-	description of the SCEF applies to the EES for Eees APIs or ECS for Eecs APIs; and

-	description of the SCS/AS applies to the functional entity invoking an EDGEAPP API.

## 7.9	HTTP headers

The HTTP headers described in 3GPP TS 29.122 [6] are applicable to all APIs in this document.

## 7.10	Conventions for Open API specification files

The conventions for Open API specification files as specified in clause 5.2.9 of 3GPP TS 29.122 [6] shall be applicable for all APIs in this document.

# 8	Edge Enabler Server API Definitions

This clause will provide the definitions of all the EdgeApp APIs from EES.

## 8.1	Eees_EASRegistration API

### 8.1.1	API URI

The Eees_EASRegistration service shall use the Eees_EASRegistration API.

The request URIs used in HTTP requests from the Edge Application Server towards the Edge Enabler Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-easregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.1.2.

### 8.1.2	Resources

#### 8.1.2.1	Overview

Figure 8.1.2.1-1: Resource URI structure of the Eees_EASRegistration API

Table 8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EAS Registrations | /registrations | POST | Registers a new EAS at the Edge Enabler Server. |
| Individual EAS Registration | /registrations/{registrationId} | GET | Fetch an individual EAS registration resource. |
|  |  | PUT | Fully replace an individual EAS registration resource. |
|  |  | DELETE | Remove an individual EAS registration resource. |
|  |  | PATCH | Partially update an individual EAS registration resource. |

Editor’s Note: It is FFS, to obtain the EAS registration information from the "EAS Registrations" collection resource.

Editor’s Note: Inclusion of the EAS identifier in the resource URI structure is FFS.

#### 8.1.2.2	Resource: EAS Registrations

##### 8.1.2.2.1	Description

This resource represents all the Edge Application Servers that are registered at a given Edge Enabler Server.

##### 8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-easregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 8.1.2.2.2-1.

Table 8.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |

##### 8.1.2.2.3	Resource Standard Methods

###### 8.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.1.2.2.3.1-1.

Table 8.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EASRegistration | M | 1 | EAS registration request information. |

Table 8.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 201 Created | EAS information is registered successfully at EES. EAS information registered with EES is provided in the response body. The URI of the created resource shall be returned in the “Location” HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.1.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-easregistration/<apiVersion>/registrations/{registrationId} |

Table 8.1.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.1.2.2.4		Resource Custom Operations

None.

#### 8.1.2.3	Resource: Individual EAS Registration

##### 8.1.2.3.1	Description

This Individual EAS Registration resource represents an individual EAS registered at a given Edge Enabler Server.

##### 8.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-easregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 8.1.2.3.2-1.

Table 8.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |
| registrationId | string | The EAS registration identifier. |

##### 8.1.2.3.3	Resource Standard Methods

###### 8.1.2.3.3.1	GET

This method retrieves the EAS information registered at Edge Enabler Server. This method shall support the URI query parameters specified in table 8.1.2.3.3.1-1.

Table 8.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.1-2 and the response data structures and response codes specified in table 8.1.2.3.3.1-3.

Table 8.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASResgistration | M | 1 | 200 OK | The EAS registration information at the Edge Enabler Server. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.1.2.3.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.1.2.3.3.2	PUT

This method updates the EAS registration information at Edge Enabler Server by completely replacing the existing registration data (except the value of "suppFeat" attribute within the EASRegistration data type, and the value of "easId" attribute within the EASProfile data type). This method shall support the URI query parameters specified in the table 8.1.2.3.3.2-1.

Table 8.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.2-2 and the response data structures and response codes specified in table 8.1.2.3.3.2-3.

Table 8.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EASRegistration | M | 1 | Details of the EAS registration information to be updated |

Table 8.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 200 OK | The EAS registration information updated successfully and the updated EAS registration information is returned in the response. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Editor’s Note: It is FFS, if 204 No Content response message is applicable.

Table 8.1.2.3.3.2-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.2-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.1.2.3.3.3	DELETE

This method deregisters an EAS registration from the EES. This method shall support the URI query parameters specified in the table 8.1.2.3.3.3-1.

Table 8.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.3-2 and the response data structures and response codes specified in table 8.1.2.3.3.3-3.

Table 8.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP DELETE message is FFS and to be updated based on security aspects defined by SA3.

Table 8.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual EAS registration information matching the registrationId is deleted. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.1.2.3.3.3-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.3-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.3-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.1.2.3.3.4	PATCH

This method partially updates the EAS registration information (except the easId) at Edge Enabler Server. This method shall support the URI query parameters specified in the table 8.1.2.3.3.4-1.

Table 8.1.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.4-2 and the response data structures and response codes specified in table 8.1.2.3.3.4-3.

Table 8.1.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EASRegistrationPatch | M | 1 | Details of the EAS registration information to be updated |

Table 8.1.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 200 OK | The Individual EAS registration information was updated successfully and the updated EAS registration information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual EAS registration information was updated successfully. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.1.2.3.3.4-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.4 -5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.4 -6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.1.2.3.4		Resource Custom Operations

None.

### 8.1.3	Custom Operations without associated resources

None.

### 8.1.4	Notifications

None.

### 8.1.5	Data Model

#### 8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.1.5.1-1 specifies the data types defined specifically for the Eees_EASRegistration API service.

Table 8.1.5.1-1: Eees_EASRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EASRegistration | 8.1.5.2.2 | The EAS registration information on EES. |  |
| EASProfile | 8.1.5.2.3 | The profile information related to the EAS in the EASRegistration data type. |  |
| EASServiceKPI | 8.1.5.2.4 | Service characteristics provided by EAS, captured in EAS profile information. |  |
| EndPoint | 8.1.5.2.5 | The end point information of the Edge Application Server in the EAS profile. |  |
| EASRegistrationPatch | 8.1.5.2.6 | To partially update the EAS Registration information. |  |

Table 8.1.5.1-2 specifies data types re-used by the Eees_EASRegistration API service.

Table 8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features defined in table 8.1.7-1. |  |
| DateTime | 3GPP TS 29.122 [6] | Used to capture the expiration time of EAS registration. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [6] | Used to define the schedule of EAS availability. |  |
| RouteToLocation | 3GPP TS 29.571 [8] | Used to define the DNAIs associated with EAS and the corresponding N6 routing information for each EAS DNAI. |  |
| DurationSec | 3GPP TS 29.122 [6] | Duration in seconds, used to define the availability reporting period for EES to check EAS availability. |  |
| LocationArea5G | 3GPP TS 29.122 [6] | Used to defined the geographic and topological area served by EAS. |  |
| BitRate | 3GPP TS 29.571 [8] | Used to express the connection bandwidth of EAS service KPI. |  |
| Fqdn | 3GPP TS 29.510 [9] | Used to express the Fully Qualified Domain Name of the Edge Application server. |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Identifying the IPv4 address of the Edge Application Server. |  |
| Ipv6Addr | 3GPP TS 29.122 [6] | Identifying the IPv6 address of the Edge Application Server. |  |
| ServiceArea | Clause 9.1.5.2.5 | Represents the topological and geographical service area information of the EAS. |  |

#### 8.1.5.2	Structured data types

##### 8.1.5.2.1	Introduction

##### 8.1.5.2.2	Type: EASRegistration

Table 8.1.5.2.2-1: Definition of type EASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easProf | EASProfile | M | 1 | The profile information of the EAS. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the EAS registration. If the expiration time is not present, then it indicates that the registration of EAS never expires. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

Editor’s Note: The data model to be updated with security credentials information, based on security aspects defined by SA3.

##### 8.1.5.2.3	Type: EASProfile

Table 8.1.5.2.3-1: Definition of type EASProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | The identifier of the EAS |  |
| endPt | EndPoint | M | 1 | Endpoint information (URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |  |
| acIds | array(string) | O | 1..N | Identities of the Application Clients that can be served by the EAS |  |
| provId | string | O | 0..1 | Identifier of the ASP that provides the EAS. |  |
| type | string | O | 0..1 | The category or type of EAS. |  |
| scheds | array(ScheduledCommunicationTime) | O | 1..N | The availability schedule of the EAS. |  |
| svcArea | ServiceArea | O | 0..1 | The list of geographical and topological areas that the EAS serves. ACs in the UE that are outside the area shall not be served. |  |
| svcKpi | EASServiceKPI | O | 0..1 | Service characteristics provided by the EAS. |  |
| permLvl | array(string) | O | 1..N | Level of service permissions supported by the EAS. |  |
| easFeats | array(string) | O | 1..N | Service features supported by the EAS. |  |
| svcContSupp | array(ACRScenario) | O | 1..N | The ACR scenarios supported by the EAS for service continuity. If this attribute is not present, then the EAS does not support service continuity. |  |
| appLocs | array(RouteToLocation) | O | 1..N | List of DNAI(s) and the corresponding N6 traffic routing information/routing profile ID, associated with the EAS. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |  |
| avlRep | DurationSec | O | 0..1 | The period indicating to the EES, how often the EES needs to check the EAS's availability after a successful registration. |  |
| status | string | O | 0..1 | EAS status (e.g. Enabled, Disabled etc.) |  |

Editor’s Note: The data type definitions for the attributes, type, permLvl, easFeats, status are FFS.

Editor’s Note: The inclusion of service APIs information in the EAS profile and its usage is FFS.

##### 8.1.5.2.4	Type: EASServiceKPI

Table 8.1.5.2.4-1: Definition of type EASServiceKPI

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxReqRate | string | O | 0..1 | Maximum request rate from the Application Client supported by the EAS. |  |
| maxRespTime | Uinteger | O | 0..1 | The maximum response time, in the units of milliseconds, advertised for the AC’s service requests. This includes the round trip time of the request and response packet, the processing time at the EAS and time required by EAS to consume any 3GPP core network capabilities. |  |
| avail | string | O | 0..1 | Advertised percentage of time the server is available for the AC's use. |  |
| avlComp | string | O | 0..1 | The maximum compute resource available for the AC. |  |
| avlGraComp | string | O | 0..1 | The maximum graphical compute resource available for the AC. |  |
| avlMem | string | O | 0..1 | The maximum memory resource available for the AC. |  |
| avlStrg | string | O | 0..1 | The maximum storage resource available for the AC. |  |
| connBand | BitRate | O | 0..1 | The connection bandwidth in Kbit/s advertised for the AC's use. |  |

Editor’s Note: The data type definitions for the attributes in EASServiceKPI data type are FFS.

##### 8.1.5.2.5	Type: EndPoint

Table 8.1.5.2.5-1: Definition of type EndPoint

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| Fqdn | Fqdn | O | 0..1 | Fully Qualified Domain Name of the Edge server. (NOTE) |  |
| ipv4Addrs | array(Ipv4Addr) | O | 1..N | IPv4 addresses of the Edge server. (NOTE) |  |
| Ipv6Addrs | array(Ipv6Addr) | O | 1..N | IPv6 addresses of the Edge server. (NOTE) |  |
| NOTE: At least one of the addressing parameters (fqdn, ipv4Addrs, ipv6Addrs attributes) shall be included. |  |  |  |  |  |

Editor’s Note: Whether URI information should be part of EndPoint Data type is FFS.

##### 8.1.5.2.6	Type: EASRegistrationPatch

Table 8.1.5.2.6-1: Definition of type EASRegistrationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easProf | EASProfile | O | 0..1 | The profile information of the EAS. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the EAS registration. If the expiration time is not present, then it indicates that the registration of EAS never expires. |  |

#### 8.1.5.3	Simple data types and enumerations

None.

### 8.1.6	Error Handling

General error responses are defined in clause 7.7.

### 8.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.1.7-1 lists the supported features for Eees_EASRegistration API.

Table 8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 8.2	Eees_UELocation API

### 8.2.1	API URI

The Eees_UELocation service shall use the Eees_UELocation API.

The request URIs used in HTTP requests from the Edge Application Server towards the EES shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-uelocation".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.2.2.

### 8.2.2	Resources

#### 8.2.2.1	Overview

Figure 8.2.2.1-1: Resource URI structure of the Eees_UELocation API

Table 8.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Location Information Subscriptions | /subscriptions | POST | Creates a subscription for continuous reporting of UE(s) location information to the EAS. |
| Individual Location Information Subscription | /subscriptions/{subscriptionId} | GET | Retrieves the Individual location information subscription information identified by subscriptionId. |
|  |  | PATCH | Updates the Individual location information subscription identified by subscriptionId. |
|  |  | PUT | Updates the Individual location information subscription identified by subscriptionId. |
|  |  | DELETE | Removes the Individual location information subscription identified by subscriptionId. |

#### 8.2.2.2	Resource: Location Information Subscriptions

##### 8.2.2.2.1	Description

This resource represents all location information subscriptions at a given EES.

##### 8.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-uelocation/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 8.2.2.2.2-1.

Table 8.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.2.1 |

##### 8.2.2.2.3	Resource Standard Methods

###### 8.2.2.2.3.1	POST

This method creates the location information subscription at the EES for continuous reporting of UE(s) location information. This method shall support the URI query parameters specified in the table 8.2.2.2.3.1-1.

Table 8.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.2.3.1-2 and the response data structures and response codes specified in table 8.2.2.2.3.1-3.

Table 8.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscription | M | 1 | Create a new location information subscription. |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP POST message is FFS and to be updated based on security aspects defined by SA3

Table 8.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 201 Created | The individual location information subscription resource created successfully. The information about the confirmed subscription at the EES is provided in the response body. The URI of the created resource shall be retruned in the “Location” HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-uelocation/<apiVersion>/subscriptions/{subscriptionId} |

Table 8.2.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.2.2.2.4		Resource Custom Operations

None.

#### 8.2.2.3	Resource: Individual Location Information Subscription

##### 8.2.2.3.1	Description

This resource represents the individual location information subscription of an EAS at a given EES.

##### 8.2.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-uelocation/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.2.2.3.2-1.

Table 8.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.2.1 |
| subscriptionId | String | Identifies an individual location information subscription. |

##### 8.2.2.3.3	Resource Standard Methods

###### 8.2.2.3.3.1	GET

This method retrieves the location information subscription information at EES. This method shall support the URI query parameters specified in the table 8.2.2.3.3.1-1.

Table 8.2.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP GET message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.2.2.3.3.1-2 and the response data structures and response codes specified in table 8.2.2.3.3.1-3.

Table 8.2.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 200 OK | The location information subscription information is returned by the EES. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.3.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.2.2.3.3.2	PATCH

This method partially updates the location information subscription information at the EES. This method shall support the URI query parameters specified in the table 8.2.2.3.3.2-1.

Table 8.2.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP PATCH message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.2.2.3.3.2-2 and the response data structures and response codes specified in table 8.2.2.3.3.2-3.

Table 8.2.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscriptionPatch | M | 1 | Request to partially update the individual location information subscription matching the subscriptionId at the EES. |

Table 8.2.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 200 OK | The individual Location information subscription matching the subscriptionId was modified successfully and the updated Location subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual Location information subscription matching the subscriptionId was modified successfully. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.3.3.2-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.2-5: Headers supported by the PATCH response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.2.2.3.3.3	PUT

This method updates the location information subscription information at the EES by completely replacing the existing subscription data (except easId, ueId, groupId). This method shall support the URI query parameters specified in the table 8.2.2.3.3.3-1.

Table 8.2.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP PUT message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.2.2.3.3.3-2 and the response data structures and response codes specified in table 8.2.2.3.3.3-3.

Table 8.2.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscription | M | 1 | Details of individual location information subscription matching the subscriptionId to be updated at the EES. |

Table 8.2.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 200 OK | The individual Location information subscription matching the subscriptionId was modified successfully and the updated Location subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual Location information subscription matching the subscriptionId was modified successfully. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.3.3.3-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.3-5: Headers supported by the PUT response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.3-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.2.2.3.3.4	DELETE

This method removes the location information subscription information from the EES. This method shall support the URI query parameters specified in the table 8.2.2.3.3.4-1.

Table 8.2.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP DELETE message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.2.2.3.3.4-2 and the response data structures and response codes specified in table 8.2.2.3.3.4-3.

Table 8.2.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.3.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | The individual Location information subscription matching the subscriptionId is deleted. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.3.3.4-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.4-5: Headers supported by the DELETE response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.4-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.2.2.3.4		Resource Custom Operations

None.

### 8.2.3	Custom Operations without associated resources

#### 8.2.3.1	Overview

The structure of the custom operation URIs of the Eees_UELocation API is shown in Figure 8.2.3.1-1.

Figure 8.2.3.1-1: Custom operation URI structure of the Eees_UELocation API

Custom operations used for this API are summarized in table 8.2.3.1-1.

Table 8.2.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /fetch | POST | Fetch an UE location information. |

#### 8.2.3.2	Operation: Fetch

##### 8.2.3.2.1	Description

This custom operation allows the EAS to fetch an UE’s location information from the EES.

##### 8.2.3.2.2	Operation Definition

This operation shall support the request data structures and response codes and data structures specified in tables 8.2.3.2.2-1 and 8.2.3.2.2-2.

Table 8.2.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationRequest | M | 1 | Parameters to request to fetch the UE location information. |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP POST message is FFS and to be updated based on security aspects defined by SA3.

Table 8.2.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationInfo | M | 1 | 200 OK | Upon success, the UE location information returned by the EES. |
| NOTE: The manadatory HTTP error status code for the POST method listed in the Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

### 8.2.4	Notifications

#### 8.2.4.1	General

Table 8.2.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location Information Notification | {notificationDestination} | POST | Notifies the subscriber EAS the UE(s) location information. |

#### 8.2.4.2	Location Information Notification

##### 8.2.4.2.1	Description

Location Information Notification is used by the EES to notify an EAS with location information of UE(s). The EES shall subscribe to the location information for UE(s) via the Individual Location Information Subscription resource.

##### 8.2.4.2.2	Notification definition

The POST method shall be used by the EES for the notification and the callback URI shall be the one provided by the EAS during the Location Information subscription.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.2.4.2.2-1.

Table 8.2.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.4.2.2-2 and the response data structures and response codes specified in table 8.2.4.2.2-3.

Table 8.2.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationNotification | M | 1 | Notification of UE(s) location information. |

Table 8.2.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

### 8.2.5	Data Model

#### 8.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.2.5.1-1 specifies the data types defined specifically for the Eees_UELocation API service.

Table 8.2.5.1-1: Eees_UELocation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| LocationSubscription | 8.2.5.2.2 | Represents the location information subscription. |  |
| LocationSubscriptionPatch | 8.2.5.2.3 | Used to request the partial update of location information subscription. |  |
| LocationNotification | 8.2.5.2.4 | UE location information notification from EES to EAS. |  |
| LocationEvent | 8.2.5.2.5 | Location information related an Individual UE. |  |
| LocationRequest | 8.2.5.2.6 | UE location information request |  |

Table 8.2.5.1-2 specifies data types re-used by the Eees_UELocation API service.

Table 8.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter, for which location information is queried. |  |
| LocationQoS | 3GPP TS 29.572 [11] | Used to indicate the location quality of service, of the location information queried. |  |
| LocationInfo | 3GPP TS 29.122 [6] | The location information related to the UE . |  |
| DateTime | 3GPP TS 29.122 [6] | Used to capture the expiration time of EAS subscription for location information reporting. |  |
| ReportingInformation | 3GPP TS 29.523 [13] | Used to indicate the reporting requirement, only the following information are applicable: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| TestNotification | 3GPP TS 29.122 [6] | Following differences apply: - The SCEF is the EES; and - The SCS/AS is the subscribing EAS. |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | Following differences apply: - The SCEF is the EES; and - The SCS/AS is the subscribing EAS. |  |
| UeMobilityExposure | 3GPP TS 29.522 [10] | The predictive location information related to the UE, |  |
| GroupId | 3GPP TS 29.571 [8] | Used to present the internal group identifier in location subscription. |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Used to present the external group identifier in location subscription. |  |
| Accuracy | 3GPP TS 29.122 [6] | Used by EAS to indicate the desired granularity of the requested location information. |  |

#### 8.2.5.2	Structured data types

##### 8.2.5.2.1	Introduction

##### 8.2.5.2.2	Type: LocationSubscription

Table 8.2.5.2.2-1: Definition of type LocationSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Identifier of the EAS subscribing for location information report. |  |
| ueId | Gpsi | O | 0..1 | Identifier of the UE for which the location information reporting is subscribed for. (NOTE) |  |
| intGrpId | GroupId | O | 0..1 | The internal group identifier, identifying the group of UEs for which the location information reporting is subscribed for. (NOTE) |  |
| extGrpId | ExternalGroupId | O | 0..1 | The external group identifier, identifying the group of UEs for which the location information reporting is subscribed for. (NOTE) |  |
| expTime | DateTime | O | 0..1 | Indicates the expiration time of the subscription. If the expiration time is not present, then it indicates that the EAS subscription never expires. |  |
| locGran | Accuracy | O | 0..1 | Indicates the format of the location information that the EAS supports. EAS wishes to receive the location information report from the EES in the indicated format. |  |
| locQos | LocationQoS | O | 0..1 | Indicates the location QoS as specified in 3GPP TS 29.572 [11]. |  |
| eventReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the location information event subscription. |  |
| notificationDestination | Uri | O | 1 | URI where the location information notification should be delivered to. This attribute shall be present in HTTP POST message to EES. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the EES to send a test notification as defined in clause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 7.6. | Notification_websocket |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |
| NOTE: Only one of UE Identifier (ueId), Internal group identifier (intGrpId), and External group identifier (extGrpId) shall be included. |  |  |  |  |  |

Editor’s Note: The format of GPSI for ueId attribute is FFS and to be updated based on security aspects defined by SA3.

Editor’s Note: It is FFS, support of civic address format by locGran attribute.

##### 8.2.5.2.3	Type: LocationSubscriptionPatch

Table 8.2.5.2.3-1: Definition of type LocationSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventReq | ReportingInformation | O | 0..1 | The reporting requirements of the location information event subscription to be updated. |  |
| expTime | DateTime | O | 0..1 | Indicates the proposed expiration time of the subscription. |  |
| notificationDestination | Uri | O | 1 | Updated URI where the location information notification should be delivered to. |  |
| locGran | Accuracy | O | 0..1 | Updated format of the location information that the EAS supports. |  |
| locQos | LocationQoS | O | 0..1 | Updated location QoS, as specified in 3GPP TS 29.572 [11]. |  |

##### 8.2.5.2.4	Type: LocationNotification

Table 8.2.5.2.4-1: Definition of type LocationNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 1 | String identifying the individual Location information subscription for which the location notification is delivered. |  |
| locEvs | array(LocationEvent) | M | 1..N | List of notifications that include the location information of the UE(s). |  |

##### 8.2.5.2.5	Type: LocationEvent

Table 8.2.5.2.5-1: Definition of type LocationEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Identifier of the UE for which the location information is reported. |  |
| locInf | LocationInfo | C | 0..1 | Actual Location information of the UE. (NOTE) |  |
| locInfPred | UeMobilityExposure | C | 0..1 | Predictive location information of the UE. (NOTE) |  |
| NOTE: Only one of “locInf” or “locInfPred” shall be included. |  |  |  |  |  |

##### 8.2.5.2.6	Type: LocationRequest

Table 8.2.5.2.6-1: Definition of type LocationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Identifier of the UE for which the location information is requested. |  |
| gran | Accuracy | O | 0..1 | Format in which the location information is requested by the EAS. |  |
| locQos | LocationQoS | O | 0..1 | Indicates the location quality of service of the requested location information. |  |

#### 8.2.5.3	Simple data types and enumerations

None.

### 8.2.6	Error Handling

General error responses are defined in clause 7.7.

### 8.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.2.7-1 lists the supported features for Eees_UELocation API.

Table 8.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.3	Eees_UEIdentifier API

### 8.3.1	API URI

The Eees_UEIdentifier service shall use the Eees_UEIdentifier API.

The request URIs used in HTTP requests from the Edge Application Server towards the Edge Enabler Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-ueidentifier".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.3.2.

### 8.3.2	Resources

#### 8.3.2.1	Overview

Figure 8.3.2.1-1: Resource URI structure of the Eees_UEIdentifier API

Table 8.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.3.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Identifier Information of UEs | /ue-identifier | n/a |  |
|  | /ue-identifier/fetch | fetch (POST) | Fetch the identifier of an UE. |

Editor’s Note: It is FFS, usage of GET method on Identifier Information of UEs resource or custom operation without resource, to fetch the UE identifier, is based on clarification from CT4.

#### 8.3.2.2	Resource: Identifier Information of UEs

##### 8.3.2.2.1	Description

This resource represents identifiers information of all the UEs at a given Edge Enabler Server.

##### 8.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-ueidentifier/<apiVersion>/ue-identifier

This resource shall support the resource URI variables defined in the table 8.3.2.2.2-1.

Table 8.3.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.3.1 |

##### 8.3.2.2.3	Resource Standard Methods

None.

##### 8.3.2.2.4		Resource Custom Operations

###### 8.3.2.2.4.1		Overview

Table 8.3.2.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /fetch | POST | Fetch an UE identifier. |

###### 8.3.2.2.4.2		Operation: Fetch

8.3.2.2.4.2.1	Description

This custom operation allows the EAS to fetch an UE’s identifier, which is UE ID as specified in 3GPP TS 23.558 [2], from the EES for a given UE information.

8.3.2.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 8.3.2.2.4.2.2-1 and the response data structure and response codes specified in table 8.3.2.2.4.2.2-2.

Table 8.3.2.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserInformation | M | 1 | Information about the User or the UE, available at the EAS. |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP POST message is FFS and to be updated based on security aspects defined by SA3.

Table 8.3.2.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Gpsi | M | 1 | 200 OK | The UE Identifier (UE ID), returned by the Edge Enabler Server. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Editor’s Note: The format of GPSI for UE ID is FFS and to be aligned with security aspects defined by SA3.

### 8.3.3	Custom Operations without associated resources

### 8.3.4	Notifications

None.

### 8.3.5	Data Model

#### 8.3.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.3.5.1-1 specifies the data types defined specifically for the Eees_UEIdentifier API service.

Table 8.3.5.1-1: Eees_UEIdentifier API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| UserInformation | 8.3.5.2.2 | Information about the User or the UE, that used by EES to determine the UE identifier. |  |

Table 8.3.5.1-2 specifies data types re-used by the Eees_UEIdentifier API service.

Table 8.3.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter, for which location information is queried. |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Identifying the IPv4 address of the UE. |  |
| Ipv6Addr | 3GPP TS 29.122 [6] | Identifying the IPv6 address of the UE. |  |

#### 8.3.5.2	Structured data types

##### 8.3.5.2.1	Introduction

##### 8.3.5.2.2	Type: UserInformation

Table 8.3.5.2.2-1: Definition of type UserInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acr | String | O | 0..1 | Anonymous Customer Reference as specified in OMA. (NOTE) |  |
| ipv4Addr | Ipv4Addr | O | 0..1 | IPv4 address of the UE. (NOTE) |  |
| Ipv6Addr | Ipv6Addr | O | 0..1 | IPv6 address of the UE. (NOTE) |  |
| NOTE: Only one of the parameters (acr, ipv4Addr, ipv6Addr) shall be included. |  |  |  |  |  |

Editor’s Note: The definition of UserInformation data type is FFS and to be aligned with the SA2 defined service on fetching UE Identifier.

Editor’s Note: Reference to "acr" attribute details is FFS and to be updated based on the alignment with SA2 defined service for fetching UE Identifier.

#### 8.3.5.3	Simple data types and enumerations

None

### 8.3.6	Error Handling

General error responses are defined in clause 7.7.

### 8.3.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.3.7-1 lists the supported features for Eees_UEIdentifier API.

Table 8.3.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 8.4	Eees_AppClientInformation API

### 8.4.1	API URI

The Eees_AppClientInformation service shall use the Eees_AppClientInformation API.

The request URIs used in HTTP requests from the Edge Application Server towards the EES shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-appclientinformation".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.4.2.

### 8.4.2	Resources

#### 8.4.2.1	Overview

Figure 8.4.2.1-1: Resource URI structure of the Eees_AppClientInformation API

Table 8.4.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.4.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Application Client Information Subscriptions | /subscriptions | POST | Creates a subscription for reporting of AC information to the EAS. |
| Individual Application Client Information Subscription | /subscriptions/{subscriptionId} | GET | Retrieves the Individual AC information subscription information identified by subscriptionId. |
|  |  | PATCH | Partially updates the Individual AC information subscription identified by subscriptionId. |
|  |  | PUT | Fully replaces the Individual AC information subscription identified by subscriptionId. |
|  |  | DELETE | Removes the Individual AC information subscription identified by subscriptionId. |

#### 8.4.2.2	Resource: Application Client Information Subscriptions

##### 8.4.2.2.1	Description

This resource represents all AC information subscriptions at a given EES.

##### 8.4.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-appclientinformation/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 8.4.2.2.2-1.

Table 8.4.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.4.1 |

##### 8.4.2.2.3	Resource Standard Methods

###### 8.4.2.2.3.1	POST

This method creates the AC information subscription at the EES for reporting of the AC capabilities. This method shall support the URI query parameters specified in the table 8.4.2.2.3.1-1.

Table 8.4.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.2.3.1-2 and the response data structures and response codes specified in table 8.4.2.2.3.1-3.

Table 8.4.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | Create a new AC information subscription. |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP POST message is FFS and to be updated based on security aspects defined by SA3

Table 8.4.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 201 Created | The individual AC information subscription resource created successfully. The information about the confirmed subscription at the EES is provided in the response body. The URI of the created resource shall be retruned in the “Location” HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-appclientinformation/<apiVersion>/subscriptions/{subscriptionId} |

Table 8.4.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.4.2.2.4		Resource Custom Operations

None.

#### 8.4.2.3	Resource: Individual Application Client Information Subscription

##### 8.4.2.3.1	Description

This resource represents the individual location information subscription of an EAS at a given EES.

##### 8.4.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-appclientinformation/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.4.2.3.2-1.

Table 8.4.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.4.1 |
| subscriptionId | String | Identifies an individual AC information subscription. |

##### 8.4.2.3.3	Resource Standard Methods

###### 8.4.2.3.3.1	GET

This method retrieves the AC information subscription information at EES. This method shall support the URI query parameters specified in the table 8.4.2.3.3.1-1.

Table 8.4.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP GET message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.4.2.3.3.1-2 and the response data structures and response codes specified in table 8.4.2.3.3.1-3.

Table 8.4.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.4.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 200 OK | The AC information subscription information is returned by the EES. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.4.2.3.3.2	PATCH

This method partially updates the individual AC information subscription information at the EES. This method shall support the URI query parameters specified in the table 8.4.2.3.3.2-1.

Table 8.4.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP PATCH message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.4.2.3.3.2-2 and the response data structures and response codes specified in table 8.4.2.3.3.2-3.

Table 8.4.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoSubscriptionPatch | M | 1 | Request to partially update the individual AC information subscription matching the subscriptionId at the EES. |

Table 8.4.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 200 OK | The individual AC information subscription matching the subscriptionId was modified successfully and the updated AC information subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual AC information subscription matching the subscriptionId was modified successfully. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.2-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.2-5: Headers supported by the PATCH response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.4.2.3.3.3	PUT

This method updates the AC information subscription information at the EES by completely replacing the existing subscription data (except easId). This method shall support the URI query parameters specified in the table 8.4.2.3.3.3-1.

Table 8.4.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP PUT message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.4.2.3.3.3-2 and the response data structures and response codes specified in table 8.4.2.3.3.3-3.

Table 8.4.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | Details of individual AC information subscription matching the subscriptionId to be updated at the EES. |

Table 8.4.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 200 OK | The individual AC information subscription matching the subscriptionId was modified successfully and the updated AC subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual AC information subscription matching the subscriptionId was modified successfully. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.3-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.3-5: Headers supported by the PUT response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.3-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.4.2.3.3.4	DELETE

This method removes the AC information subscription information from the EES. This method shall support the URI query parameters specified in the table 8.4.2.3.3.4-1.

Table 8.4.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP DELETE message is FFS and to be updated based on security aspects defined by SA3

This method shall support the request data structures specified in table 8.4.2.3.3.4-2 and the response data structures and response codes specified in table 8.4.2.3.3.4-3.

Table 8.4.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.4.2.3.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | The individual AC information subscription matching the subscriptionId is deleted. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.4-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.4-5: Headers supported by the DELETE response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.4.2.3.3.4-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.4.2.3.4		Resource Custom Operations

None.

### 8.4.3	Custom Operations without associated resources

None.

### 8.4.4	Notifications

#### 8.4.4.1	General

Table 8.4.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AC Information Notification | {notificationDestination} | POST | Notifies the subscriber EAS the AC information. |

#### 8.4.4.2	AC Information Notification

##### 8.4.4.2.1	Description

AC Information Notification is used by the EES to notify an EAS with AC information matching the filter criteria.

##### 8.4.4.2.2	Notification definition

The POST method shall be used by the EES for the notification and the callback URI shall be the one provided by the EAS during the creation or modification of Individual Application Client Information Subscription.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.4.4.2.2-1.

Table 8.4.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.4.2.2-2 and the response data structures and response codes specified in table 8.4.4.2.2-3.

Table 8.4.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoNotification | M | 1 | Notification of AC(s) information. |

Table 8.4.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

### 8.4.5	Data Model

#### 8.4.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.4.5.1-1 specifies the data types defined specifically for the Eees_AppClientInformation API service.

Table 8.4.5.1-1: Eees_AppClientInformation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ACInfoSubscription | 8.4.5.2.2 | Represents the AC information subscription. |  |
| ACInfoSubscriptionPatch | 8.4.5.2.3 | Used to request the partial update of AC information subscription. |  |
| ACFilter | 8.4.5.2.4 | Used to list the set of characteristics to discover the ACs. |  |
| ACInfoNotification | 8.4.5.2.5 | AC information notification matching the filter criteria. |  |
| ACInformation | 8.4.5.2.6 | Used to represent the AC information in the AC information notification. |  |

Table 8.4.5.1-2 specifies data types re-used by the Eees_AppClientInformation API service.

Table 8.4.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [6] | Used to capture the expiration time of EAS subscription for location information reporting. |  |
| ReportingInformation | 3GPP TS 29.523 [13] | Used to indicate the reporting requirement, only the following information are applicable: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | Following differences apply: - The SCEF is the EES; and - The SCS/AS is the subscribing EAS. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| TestNotification | 3GPP TS 29.122 [6] | Following differences apply: - The SCEF is the EES; and - The SCS/AS is the subscribing EAS. |  |
| LocationArea5G | 3GPP TS 29.122 [6] | Used to define the geographic and topological area. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [6] | Used to define the operation schedule of AC. |  |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter, for which location information is queried. |  |
| ACProfile | 3GPP TS 24.558 [14] | Used to represent the profile the information of the Application Client. |  |
| ServiceArea | Clause 9.1.5.2.5 | Used to represent the EAS service area in ACFilters data type. |  |

#### 8.4.5.2	Structured data types

##### 8.4.5.2.1	Introduction

##### 8.4.5.2.2	Type: ACInfoSubscription

Table 8.4.5.2.2-1: Definition of type ACInfoSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Identifier of the EAS subscribing for AC information report. |  |
| acFltrs | array(ACFilters) | O | 1..N | Filters to retrieve the information about particular ACs. |  |
| expTime | DateTime | O | 0..1 | Indicates the expiration time of the subscription. If the expiration time is not present, then it indicates that the EAS subscription never expires. |  |
| eventReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the AC information subscription. (NOTE) |  |
| notificationDestination | Uri | M | 1 | URI where the notification on information about particular ACs should be delivered to. This attribute shall be present in HTTP POST message to EES. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the EES to send a test notification as defined in clause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 7.6. | Notification_websocket |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |
| NOTE: The applicable values of the ReportingInformation data type are, "immRep", "notifMethod", "maxReportNbr", "monDur", "repPeriod". |  |  |  |  |  |

##### 8.4.5.2.3	Type: ACInfoSubscriptionPatch

Table 8.4.5.2.3-1: Definition of type ACInfoSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventReq | ReportingInformation | O | 0..1 | The reporting requirements of the AC information event subscription to be updated. |  |
| expTime | DateTime | O | 0..1 | Indicates the proposed expiration time of the subscription. |  |
| notificationDestination | Uri | O | 0..1 | Updated URI where the AC information notification should be delivered to. |  |
| acFltrs | array(ACFilters) | O | 1..N | Filters to retrieve the information about particular ACs. |  |

##### 8.4.5.2.4	Type: ACFilters

Table 8.4.5.2.4-1: Definition of type ACFilters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acTypes | array(string) | O | 1..N | List of AC types or categories to be matched. |  |
| ecspIds | array(string) | O | 1..N | The list of identifiers of the ECSPs associated with the EEC. |  |
| acIds | array(string) | O | 1..N | List of the identifiers of the AC(s) to be matched. |  |
| svcArea | ServiceArea | O | 0..1 | EAS service area for identifying the UEs with matching expected geographical and topological location(s). |  |
| easKpi | array(EASServiceKPI) | O | 1..N | Service characteristics of the EAS for identifying the matched EEC and ACs. |  |
| prmRng | array(string) | O | 1..N | Parameter ranges to match the ACs profiles. |  |
| opSchds | array(ScheduledCommunicationTime) | O | 1..N | The operation schedule of the EAS to be matched with operation schedule of the AC. |  |
| ueIds | array(Gpsi) | O | 1..N | List of UE identifiers hosting the AC. |  |
| locInfs | LocationArea5G | O | 0..1 | List of location(s) of the UE(s) hosting the AC. |  |

##### 8.4.5.2.5	Type: ACInfoNotification

Table 8.4.5.2.5-1: Definition of type ACInfoNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 1 | String identifying the individual AC information subscription for which the AC information notification is delivered. |  |
| acInfs | array(ACInformation) | M | 1..N | List of notifications that include the information of the ACs matching the filter criteria. |  |

##### 8.4.5.2.6	Type: ACInformation

Table 8.4.5.2.6-1: Definition of type ACInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acProfs | array(ACProfile) | M | 1 | List of ACs profile information. |  |
| ueIds | array(Gpsi) | O | 1..N | List of UE identifiers hosting the ACs. |  |
| ueLocInfs | LocationArea5G | O | 0..1 | Location information of the UEs hosting the AC. |  |

#### 8.4.5.3	Simple data types and enumerations

None.

### 8.4.6	Error Handling

General error responses are defined in clause 7.7.

### 8.4.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.4.7-1 lists the supported features for Eees_AppClientInformation API.

Table 8.4.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.5	Eees_SessionWithQoS API

### 8.5.1	API URI

The Eees_SessionWithQoS service shall use the Eees_SessionWithQoS API.

The request URIs used in HTTP requests from the Edge Application Server towards the Edge Enabler Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-sessionwithqos".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.5.2.

### 8.5.2	Resources

#### 8.5.2.1	Overview

Figure 8.5.2.1-1: Resource URI structure of the Eees_SessionWithQoS API

Table 8.5.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Sessions with QoS | /sessions | POST | Create a new individual Session with QoS |
|  |  | GET | Read all subscription resources for given EAS. |
| Individual Session with QoS | /sessions/{sessionId} | PUT | Fully replace an existing Individual Session with QoS resource identified by a sessionId. |
|  |  | PATCH | Partial update an existing Individual Session with QoS resource identified by a sessionId |
|  |  | DELETE | Remove an Individual Session with QoS resource identified by a sessionId. |
|  |  | GET | Read a subscription resource for a sessionId. |

Editor’s Note: Details of how the EAS security credentials are submitted in various HTTP messages is FFS and to be updated based on security aspects defined by SA3.

#### 8.5.2.2	Resource: Sessions with QoS

##### 8.5.2.2.1	Description

This resource represents session information of all the data sessions with a specific QoS setting at a given Edge Enabler Server.

##### 8.5.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-sessionwithqos/<apiVersion>/sessions

This resource shall support the resource URI variables defined in the table 8.5.2.2.2-1.

Table 8.5.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.5.1 |

##### 8.5.2.2.3	Resource Standard Methods

###### 8.5.2.2.3.1	POST

This method requests resources for a data session between AC and EAS with a specific QoS and may create the session information subscription at the Edge Enabler Server for receiving the user plane event notification of the session information. This method shall support the URI query parameters specified in table 8.5.2.2.3.1-1.

Table 8.5.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.2.3.1-2 and the response data structures and response codes specified in table 8.5.2.2.3.1-3.

Table 8.5.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionWithQoS | M | 1 | Parameters to create a subscription for a session with required QoS for the service requirement. |

Table 8.5.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P |  | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- | --- |
| SessionWithQoS | M |  | 1 | 201 Created | The session is successfully set up with requested QoS, and the session information is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |  |

Table 8.5.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ eees-sessionwithqos/<apiVersion>/sessions/{sessionId} |

Table 8.5.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.5.2.2.3.2	GET

The GET method allows to read all active subscriptions for a given EAS. The EAS shall initiate the HTTP GET request message and the EES shall respond to the message. This method shall support the URI query parameters specified in table 8.5.2.2.3.2-1.

Table 8.5.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| easId | string | M | 1 | Identifier of the EAS which querying the status of subscriptions. |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP GET message is FFS and to be updated based on security aspects defined by SA3.

This method shall support the request data structures specified in table 8.5.2.2.3.2-2 and the response data structures and response codes specified in table 8.5.2.2.3.2-3.

Table 8.5.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SessionWithQoS) | M | 1..N | 200 OK | The subscription information related to the request URI is returned. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.2.3.2-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.2.3.2-5: Headers supported by the GET response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.2.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.5.2.2.4		Resource Custom Operations

None.

#### 8.5.2.3	Resource: Individual Session with QoS

##### 8.5.2.3.1	Description

This resource represents an individual session information of the data session with a specific QoS setting at a given Edge Enabler Server.

##### 8.5.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-sessionwithqos/<apiVersion>/sessions/{sessionId}

This resource shall support the resource URI variables defined in the table 8.5.2.3.2-1.

Table 8.5.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.5.1 |

##### 8.5.2.3.3	Resource Standard Methods

###### 8.5.2.3.3.1	PATCH

This method partially updates the QoS of the data session between AC and EAS. This method shall support the URI query parameters specified in the table 8.5.2.3.3.1-1.

Table 8.5.2.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.3.3.1-2 and the response data structures and response codes specified in table 8.5.2.3.3.1-3.

Table 8.5.2.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionWithQoSPatch | M | 1 | Request to partially update the data session between AC and EAS with a specific QoS |

Table 8.5.2.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 200 OK | The individual Session with QoS is successfully modified and the updated session with QoS context information is returned in the response |
| n/a |  |  | 204 No Content | The individual Session with QoS is successfully modified. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.3.3.1-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.1-5: Headers supported by the PATCH response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.5.2.3.3.2	PUT

This method requests modification of QoS of the data session between AC and EAS and may modify the subscription of the event monitoring by subscribing to new events or removing subscriptions to existing events at the Edge Enabler Server for receiving the user plane event notification of the session information. This method shall support the URI query parameters specified in the table 8.5.2.3.3.2-1.

Table 8.5.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP PUT message is FFS and to be updated based on security aspects defined by SA3.

This method shall support the request data structures specified in table 8.5.2.3.3.2-2 and the response data structures and response codes specified in table 8.5.2.3.3.2-3.

Table 8.5.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionWithQoS | M | 1 | Parameters to create a subscription for a session with required QoS for the service requirement. |

Table 8.5.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 200 OK | The individual Session with QoS is successfully modified and the updated session with QoS context information is returned in the response. |
| n/a |  |  | 204 No Content | The individual Session with QoS is successfully modified. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.3.3.2-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.2-5: Headers supported by the PUT response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.5.2.3.3.3	DELETE

This method revokes the data session between AC and EAS with a specific QoS and unsubscribes to the related session with user plane event notification. This method shall support the URI query parameters specified in table 8.5.2.3.3.3-1.

Table 8.5.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP DELETE message is FFS and to be updated based on security aspects defined by SA3.

This method shall support the request data structures specified in table 8.5.2.3.3.3-2 and the response data structures and response codes specified in table 8.5.2.3.3.3-3.

Table 8.5.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/q |  |  |  |

Table 8.5.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | The individual Session with QoS resource matching the sessionId is successfully deleted. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.3.3.3-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.3-5: Headers supported by the DELETE response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.3-6: Links supported by the 204 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.5.2.3.3.4	GET

The GET method allows to read a subscription. The EAS shall initiate the HTTP GET request message and the EES shall respond to the message. This method shall support the URI query parameters specified in table 8.5.2.3.3.4-1.

Table 8.5.2.3.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP GET message is FFS and to be updated based on security aspects defined by SA3.

This method shall support the request data structures specified in table 8.5.2.3.3.4-2 and the response data structures and response codes specified in table 8.5.2.3.3.4-3.

Table 8.5.2.3.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.3.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 200 OK | The subscription information related to the request URI is returned. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.3.3.4-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.4-5: Headers supported by the GET response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.5.2.3.3.4-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.5.2.3.4		Resource Custom Operations

None.

### 8.5.3	Custom Operations without associated resources

None.

### 8.5.4	Notifications

#### 8.5.4.1	General

Table 8.5.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| User Plane Event Notification | {notificationDestination} | POST | Notifies the EAS the subscribed user plane event(s). |

#### 8.5.4.2	User Plane Event Notification

##### 8.5.4.2.1	Description

##### 8.5.4.2.2	Notification definition

The POST method shall be used by the EES for the notification and the callback URI shall be the one provided by the EAS during the reservation of resources for a data session between AC and EAS with a specific QoS.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.5.4.2.2-1.

Table 8.5.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 8.5.4.2.2-2 and the response data structures and response codes specified in table 8.5.4.2.2-3.

Table 8.5.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserPlaneEventNotification | M | 1 | Notification of the user plane event on the data session. |

Table 8.5.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |

### 8.5.5	Data Model

#### 8.5.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.5.5.1-1 specifies the data types defined specifically for the Eees_SessionWithQoS API service.

Table 8.5.5.1-1: Eees_SessionWithQoS API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SessionWithQoS | 8.5.5.2.2 |  |  |
| SessionWIthQoSPatch | 8.5.5.2.3 |  |  |
| UserPlaneEventNotification | 8.5.5.2.4 |  |  |

Table 8.5.5.1-2 specifies data types re-used by the Eees_SessionWithQoS API service.

Table 8.5.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE, for which the related session with QoS is requested to. |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Identifying the IPv4 address of the UE. |  |
| Ipv6Addr | 3GPP TS 29.122 [6] | Identifying the IPv6 address of the UE. |  |
| UserPlaneEvent | 3GPP TS 29.122 [6] | Indicates the event reported by the EES. |  |
| SponsorInformation | 3GPP TS 29.122 [6] | Indicates a sponsor information |  |
| QosMonitoringInformation | 3GPP TS 29.122 [6] | Indicates the Qos Monitoring information |  |
| DurationSecRm | 3GPP TS 29.571 [8] | This data type is defined in the same way as the "DurationSec" data type, but with the OpenAPI "nullable: true" property. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| TestNotification | 3GPP TS 29.122 [6] | This type represents a notification that can be sent to test whether a chosen notification mechanism works |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | This type represents configuration for the delivery of notifications over Websockets. |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |  |
| GroupId | 3GPP TS 29.571 [8] | Used to present the internal group identifier in the AS session with QoS subscription/request. |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Used to present the external group identifier in the AS session with QoS subscription/request. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies a S-NSSAI |  |
| FlowDescription | 3GPP TS 29.514 [16] | Identifies an IP flow description. |  |
| BitRateRm | 3GPP TS 29.571 [8] | This data type is defined in the same way as the "BitRate" data type, but with the OpenAPI "nullable: true" property. |  |
| AccumulatedUsage | 3GPP TS 29.122 [6] | Contains the applicable information corresponding to the event. |  |
| QosMonitoringReport | 3GPP TS 29.122 [6] | Contains the QoS Monitoring Reporting information. |  |
| UserPlaneEventReport | 3GPP TS 29.122 [6] |  |  |

#### 8.5.5.2	Structured data types

##### 8.5.5.2.1	Introduction

##### 8.5.5.2.2	Type: SessionWithQoS

Table 8.5.5.2.2-1: Definition of type SessionWIthQoS

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Uri | C | 0..1 | Link to the "Individual Session with QoS" resource. Shall only be present in the HTTP GET response on the "Sessions with QoS" resource. |  |
| easId | string | M | 1 | Identifier of the EAS. |  |
| ueIpv4Addr | Ipv4Addr | O | 0..1 | IPv4 address of the UE. (NOTE 1) |  |
| ueIpv6Addr | Ipv6Addr | O | 0..1 | IPv6 address of the UE. (NOTE 1) |  |
| ipDomain | string | O | 0..1 | Identifies the IP domain. The attribute may only be provided if the ueIpv4Addr attribute is present. |  |
| ueId | Gpsi | O | 0..1 | Identifier of the UE for which related session with QoS is requested to. (NOTE 1) |  |
| intGrpId | GroupId | O | 0..1 | The internal group identifier, identifying the group of UEs for which related session with QoS is requested to. (NOTE 1) |  |
| extGrpId | ExternalGroupId | O | 0..1 | The external group identifier, identifying the group of UEs for which the related session with QoS is requested to. (NOTE 1) |  |
| ipFlows | array(FlowDescription) | M | 1..N | Contains the flow description for the Uplink and/or Downlink IP flows. |  |
| qosReference | string | O | 0..1 | Identifies a pre-defined QoS information (NOTE 2) |  |
| altQoSReferences | array(string) | O | 0..N | Identifies an ordered list of pre-defined QoS information. The lower the index of the array for a given entry, the higher the priority. |  |
| events | array(UserPlaneEvent) | M | 1..N | Indicates the events subscribed by the EAS. |  |
| sponsorInformation | SponsorInformation | O | 0..1 | Describes the sponsor information. |  |
| qosMonInfo | QosMonitoringInformation | O | 0..1 | Qos Monitoring information. It can be present when the event "QOS_MONITORING" is subscribed. |  |
| notificationDestination | Uri | C | 0..1 | URI where the event notification shall be delivered to. This attribute shall be present if the "events" attribute is included. |  |
| dnn | Dnn | O | 0..1 | Dnn of the PDU session, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | S-NSSAI of the PDU session. |  |
| maxbrUl | BitRate | O | 0..1 | Indicates the (requested) maximum bandwidth in uplink. (NOTE 2) |  |
| maxbrDl | BitRate | O | 0..1 | Indicates the (requested) maximum bandwidth in downlink. (NOTE 2) |  |
| disUeNotif | boolean | O | 0..1 | Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to "true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile. The default value "false" shall apply, if the attribute is not present and has not been supplied previously. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the EES to send a test notification as defined in 3GPP TS 29.122 [6]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in 3GPP TS 29.122 [6]. | Notification_websocket |
| NOTE 1: Only one of UE IP address (ipv4Addr or ipv6Addr), UE Identifier (ueId), Internal group identifier (intGrpId), or External group identifier (extGrpId) shall be included.. NOTE 2: Only one of requested QoS (qosReference) or Requested bandwidth (maxbuUl and/or maxbtDl) shall be included. |  |  |  |  |  |

##### 8.5.5.2.3	Type: SessionWithQoSPatch

Table 8.5.5.2.3-1: Definition of type SessionWIthQoSPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipFlows | array(FlowDescription) | O | 1..N | Contains the flow description for the Uplink and/or Downlink IP flows. |  |
| qosReference | string | O | 0..1 | Identifies a pre-defined QoS information. |  |
| altQoSReferences | array(string) | O | 1..N | Identifies an ordered list of pre-defined QoS information. The lower the index of the array for a given entry, the higher the priority. |  |
| events | array(UserPlaneEvent) | O | 1..N | Indicates the event subscribed by the EAS. |  |
| sponsorInformation | SponsorInformation | O | 0..1 | Describes the sponsor information such as who is sponsoring the traffic. |  |
| qosMonInfo | QosMonitoringInformationRm | O | 0..1 | Qos Monitoring information. It can be present when the event "QOS_MONITORING" is subscribed. |  |
| notificationDestination | Uri | O | 0..1 | URI where the monitoring event notification should be delivered to. |  |
| maxbrUl | BitRateRm | O | 0..1 | Indicates the (requested) maximum bandwidth in uplink. |  |
| maxbrDl | BitRateRm | O | 0..1 | Indicates the (requested) maximum bandwidth in downlink. |  |
| disUeNotif | boolean | O | 0..1 | Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to "true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile. The default value "false" shall apply, if the attribute is not present and has not been supplied previously. |  |

##### 8.5.5.2.4	Type: UserPlaneEventNotification

Table 8.5.5.2.4-1: Definition of type UserPlaneEventNotifiation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | String identifying the individual data session information for which the QoS event notification is delivered. |  |
| eventReports | array(UserPlaneEventReport) | M | 1..N | List of notifications that include the QoS event information of the data session. |  |

Editor’s Note:	It’s FFS about the reporting of availability of QoS monitoring API.

#### 8.5.5.3	Simple data types and enumerations

None.

### 8.5.6	Error Handling

General error responses are defined in clause 7.7.

### 8.5.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.5.7-1 lists the supported features for Eees_SessionWithQoS API.

Table 8.5.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.6	Eees_TargetEASDiscovery API

### 8.6.1	API URI

The Eees_TargetEASDiscovery service shall use the Eees_TargetEASDiscovery API.

The request URIs used in HTTP requests from the Edge Application Server towards the Edge Enabler Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-targeteasdiscovery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.6.2.

### 8.6.2	Resources

#### 8.6.2.1	Overview

Figure 8.6.2.1-1: Resource URI structure of the Eees_TargetEASDiscovery API

Table 8.6.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.6.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EAS Profiles | /eas-profiles | GET | Discover the T-EAS information. |

#### 8.6.2.2	Resource: EAS Profiles

##### 8.6.2.2.1	Description

This resource allows the source EAS (S-EAS) or source EES (S-EES) to discover the target EAS (T-EAS) information from the EES.

##### 8.6.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-targeteasdiscovery/<apiVersion>/eas-profiles

This resource shall support the resource URI variables defined in the table 8.6.2.2.2-1.

Table 8.6.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.6.1 |

##### 8.6.2.2.3	Resource Standard Methods

###### 8.6.2.2.3.1	GET

This method allows the S-EAS and S-EES to fetch the T-EAS information as specified in 3GPP TS 23.558 [2], from the EES with a given discovery filters. This method shall support the URI query parameters specified in table 8.6.2.2.3.1-1.

Table 8.6.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| TBD |  |  |  |  |

This method shall support the request data structures specified in table 8.6.2.2.3.1-2 and the response data structures and response codes specified in table 8.6.2.2.3.1-3.

Table 8.6.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP GET message is FFS and to be updated based on security aspects defined by SA3.

Table 8.6.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TBD |  |  |  |  |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Editor’s Note: The details of POST Request and Response body is FFS.

Table 8.6.2.2.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.6.2.2.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.6.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 8.6.2.2.4		Resource Custom Operations

None.

### 8.6.3	Custom Operations without associated resources

None.

### 8.6.4	Notifications

None.

### 8.6.5	Data Model

#### 8.6.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.6.5.1-1 specifies the data types defined specifically for the Eees_TargetEASDiscovery API service.

Table 8.6.5.1-1: Eees_TargetEASDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 8.6.5.1-2 specifies data types re-used by the Eees_TargetEASDiscovery API service.

Table 8.6.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

#### 8.6.5.2	Structured data types

Editor’s Note: The definition of structured data types is FFS

#### 8.6.5.3	Simple data types and enumerations

Editor’s Note: The definition of simple data types and enumerations is FFS.

### 8.6.6	Error Handling

General error responses are defined in clause 7.7.

### 8.6.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.6.7-1 lists the supported features for Eees_TargetEASDiscovery API.

Table 8.6.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 8.7	Eees_ACRManagementEvent API

### 8.7.1	API URI

The Eees_ACRManagementEvent service shall use the Eees_ACRManagementEvent API.

The request URIs used in HTTP requests from the Edge Application Server towards the Edge Enabler Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-acrmgntevent".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.w.2.

### 8.7.2	Resources

#### 8.7.2.1	Overview

Figure 8.7.2.1-1: Resource URI structure of the Eees_ACRManagementEvent API

Table 8.7.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.7.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ACR Management Events Subscriptions | /subscriptions | GET | Query all the subscriptions. |
|  |  | POST | Create a new Individual ACR Management Events Subscription resource. |
| Individual ACR Management Events Subscription | /subscriptions/{subscriptionId} | GET | Query an existing Individual ACR Management Events Subscription resource identified by a subscriptionId. |
|  |  | PUT | Fully replace an existing Individual ACR Management Events Subscription resource identified by a subscriptionId. |
|  |  | PATCH | Partial update an existing Individual ACR Management Events Subscription resource identified by a subscriptionId. |
|  |  | DELETE | Remove an Individual ACR Management Events Subscription resource identified by a subscriptionId. |

#### 8.7.2.2	Resource: ACR Management Events Subscriptions

##### 8.7.2.2.1	Description

This resource represents ACR Management Events Subscriptions at a given Edge Enabler Server.

##### 8.7.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-acrmgntevent/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 8.7.2.2.2-1.

Table 8.7.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.7.1 |

##### 8.7.2.2.3	Resource Standard Methods

###### 8.7.2.2.3.1	POST

This method requests to create an Individual ACR Management Event Subscription resource at the EES for receiving the notifications of ACR management events. This method shall support the URI query parameters specified in table 8.7.2.2.3.1-1.

Table 8.7.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.2.3.1-2 and the response data structures and response codes specified in table 8.7.2.2.3.1-3.

Table 8.7.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | Parameters to create a subscription for notifications of ACR management events. |

Table 8.7.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 201 Created | An Individual ACR Management Events Subscription resource is successfully created, and the subscription information is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-acrmgntevent/<apiVersion>/subscriptions/{subscriptionId} |

Table 8.7.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 8.7.2.2.3.2	GET

This method retrieves all the ACR Management Events Subscriptions information at EES. This method shall support the URI query parameters specified in the table 8.7.2.2.3.2-1.

Table 8.7.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the EAS. |

This method shall support the request data structures specified in table 8.7.2.2.3.2-2 and the response data structures and response codes specified in table 8.7.2.2.3.2-3.

Table 8.7.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.7.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AcrMgntEventsSubscription) | M | 1..N | 200 OK | All the ACR Management Events Subscriptions information is returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.2.2.3.2-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.2.3.2-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.2.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.2.3.2-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.7.2.2.3.2-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.7.2.2.4		Resource Custom Operations

None.

#### 8.7.2.3	Resource: Individual ACR Management Events Subscription

##### 8.7.2.3.1	Description

This resource represents an existing Individual ACR Management Events Subscription at a given EES.

##### 8.7.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-acrmgntevent/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.7.2.3.2-1.

Table 8.7.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.7.1 |

##### 8.7.2.3.3	Resource Standard Methods

###### 8.7.2.3.3.1	PATCH

This method partially updates an existing Individual ACR Management Events Subscription. This method shall support the URI query parameters specified in the table 8.7.2.3.3.1-1.

Table 8.7.2.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.3.3.1-2 and the response data structures and response codes specified in table 8.7.2.3.3.1-3.

Table 8.7.2.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsSubscriptionPatch | M | 1 | Request to partially update an existing Individual ACR Management Events Subscription. |

Table 8.7.2.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 200 OK | The Individual ACR Management Events Subscription is successfully modified and the updated subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual ACR Management Events Subscription is successfully modified. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.2.3.3.1-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.1-5: Headers supported by the PATCH response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.1-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.7.2.3.3.1-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.7.2.3.3.2	PUT

This method requests fully replacement of an existing Individual ACR Management Events Subscription at the EES. The request shall not change the values of the "easId", "tgtUeId", "requestTestNotification", "websockNotifConfig" and/or "suppFeat" attributes within the AcrMgntEventsSubscription data type. This method shall support the URI query parameters specified in the table 8.7.2.3.3.2-1.

Table 8.7.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.3.3.2-2 and the response data structures and response codes specified in table 8.7.2.3.3.2-3.

Table 8.7.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | Parameters to replace an existing Individual ACR Management Events Subscription. |

Table 8.7.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 200 OK | The existing Individual ACR Management Events Subscription is successfully replaced and the updated subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The existing Individual ACR Management Events Subscription is successfully modified. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.2.3.3.2-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.2-5: Headers supported by the PUT response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.2-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.7.2.3.3.2-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.7.2.3.3.3	DELETE

This method deletes an existing Individual ACR Management Events Subscription. This method shall support the URI query parameters specified in table 8.7.2.3.3.3-1.

Table 8.7.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.3.3.3-2 and the response data structures and response codes specified in table 8.7.2.3.3.3-3.

Table 8.7.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.7.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The existing Individual ACR Management Events Subscription is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.2.3.3.3-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.3-5: Headers supported by the DELETE response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.3-6: Links supported by the 204 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.3-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.7.2.3.3.3-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.7.2.3.3.4	GET

This method retrieves the location information subscription information at EES. This method shall support the URI query parameters specified in the table 8.7.2.3.3.4-1.

Table 8.7.2.3.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the EAS. |

This method shall support the request data structures specified in table 8.7.2.3.3.4-2 and the response data structures and response codes specified in table 8.7.2.3.3.4-3.

Table 8.7.2.3.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.7.2.3.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 200 OK | The Individual ACR Management Events Subscription is returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.2.3.3.4-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.4-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.4-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.7.2.3.3.4-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.7.2.3.3.4-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.7.2.3.4		Resource Custom Operations

None.

### 8.7.3	Custom Operations without associated resources

None.

### 8.7.4	Notifications

#### 8.7.4.1	General

Table 8.7.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ACR Management Events Notification | {notificationDestination} | POST | Notifies the EAS the subscribed ACR management event(s). |

#### 8.7.4.2	ACR Management Events Notification

##### 8.7.4.2.1	Description

##### 8.7.4.2.2	Notification definition

The POST method is used by the EES for the notifications of subscribed ACR management event(s) and the callback URI shall be provided by the EAS during the creation of the Individual ACR Management Events Subscription resource.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.7.4.2.2-1.

Table 8.7.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 8.7.4.2.2-2 and the response data structures and response codes specified in table 8.7.4.2.2-3.

Table 8.7.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsNotification | M | 1 | Notification of the ACR management events. |

Table 8.7.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.7.4.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

Table 8.7.4.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

### 8.7.5	Data Model

#### 8.7.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.7.5.1-1 specifies the data types defined specifically for the Eees_ACRManagementEvent API service.

Table 8.7.5.1-1: Eees_ACRManagementEvent API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AcrMgntEventsSubscription | 8.7.5.2.2 |  |  |
| AcrMgntEventSubsc | 8.7.5.2.3 |  |  |
| AcrMgntEventsSubscriptionPatch | 8.7.5.2.4 |  |  |
| AcrMgntEventsNotification | 8.7.5.2.5 |  |  |
| AcrMgntEventReport | 8.7.5.2.6 |  |  |
| FailureAcrMgntEventInfo | 8.7.5.2.7 |  |  |
| TargetUeIdentification | 8.7.5.2.8 |  |  |
| AcrMgntEvent | 8.7.5.3.3 |  |  |

Table 8.7.5.1-2 specifies data types re-used by the Eees_ACRManagementEvent API service.

Table 8.7.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [6] |  |  |
| DnaiChangeType | 3GPP TS 29.571 [8] |  |  |
| DnaiChangeTypeRm | 3GPP TS 29.571 [8] |  |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Represents an external group of UEs. |  |
| Gpsi | 3GPP TS 29.571 [8] |  |  |
| GroupId | 3GPP TS 29.571 [8] | Represents an internal group of UEs. |  |
| IpAddr | 3GPP TS 29.571 [8] |  |  |
| ReportingInformation | 3GPP TS 29.523 [TS29523] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| TestNotification | 3GPP TS 29.122 [6] | This type represents a notification that can be sent to test whether a chosen notification mechanism works |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | This type represents configuration for the delivery of notifications over Websockets. |  |

#### 8.7.5.2	Structured data types

##### 8.7.5.2.1	Introduction

##### 8.7.5.2.2	Type: AcrMgntEventsSubscription

Table 8.7.5.2.2-1: Definition of type AcrMgntEventsSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Uri | C | 0..1 | Link to the "Individual ACR Management Events Subscription" resource. Shall only be present in the HTTP GET response on the "ACR Management Events Subscriptions" resource. |  |
| easId | string | M | 1 | Identifier of an EAS. |  |
| tgtUeId | TargetUeIdentification | M | 1 | Identifies the UE identification. |  |
| eventSubscs | array(AcrMgntEventSubsc) | M | 1..N | Indicates the subscribed ACR management events. |  |
| evtReq | ReportingInformation | O | 0..1 | Indicates the event reporting information. |  |
| notificationDestination | Uri | M | 1 | URI where the event notification shall be delivered to. |  |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. Applicable when the "event" attribute within the AcrMgntEventSubsc data type is "UP_PATH_CHG". |  |
| easAckInd | boolean | O | 0..1 | Identifies whether the EAS acknowledgement for UP path change event notification is expected. Set to "true" if the EAS acknowledge is expected. Default value is "false". |  |
| eventReports | array(AcrMgntEventReport) | C | 1..N | Represents ACR management event report(s). Shall be present in the HTTP POST response if the immediate reporting indication in the "immRep" attribute within the "evtReq" attribute sets to true in the event subscription, and the reports are available. May be present in the HTTP PUT or PATCH response when the reports are available. |  |
| easChars | array(EasCharacteristics) | O | 1..N | Represents a list of EAS characteristics. Applicable when the "event" attribute within the AcrMgntEventSubsc data type is "ACR_MONITORING" or "ACR_FACILITATION". |  |
| failEventReports | array(FailureAcrMgntEventInfo) | O | 1..N | Represents the event(s) for which the subscription is not successful including the failure reason(s). |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the EAS to request the EES to send a test notification as defined in 3GPP TS 29.122 [6]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in 3GPP TS 29.122 [6]. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Represents a list of Supported features used as described in clause 8.w.7. Shall be present in the HTTP POST request/response; or in the HTTP GET response if the "supp-feat" attribute query parameter is included in the HTTP GET request. |  |

Editor’s Note:	It's FFS to define the security credentials as described in 3GPP TS 29.558.

Editor’s Note:	It’s FFS that whether the "evtReq" attribute is applicable or not for the UP path change event, since the TrafficInfluence API as defined in 3GPP TS 29.522 does not support the event reporting information yet.

Editor’s Note:	Definition of EasCharacteristics data type is FFS.

##### 8.7.5.2.3	Type: AcrMgntEventSubsc

Table 8.7.5.2.3-1: Definition of type AcrMgntEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| eventFilter | AcrMgntEventFilter | O | 0..1 | Represents the event filter for the subscribed ACR management event. |  |

Editor’s Note: The definition of AcrMgntEventFilter data type is FFS.

##### 8.7.5.2.4	Type: AcrMgntEventsSubscriptionPatch

Table 8.7.5.2.4-1: Definition of type AcrMgntEventsSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubscs | array(AcrMgntEventSubsc) | O | 1..N | Indicates the subscribed ACR management events. |  |
| evtReq | ReportingInformation | O | 1..N | Indicates the event reporting information. |  |
| notificationDestination | Uri | O | 0..1 | URI where the event notification shall be delivered to. |  |
| dnaiChgType | DnaiChangeTypeRm | O | 0..1 | Identifies a type of notification regarding UP path management event. Applicable when the "event" attribute within the AcrMgntEventSubsc data type is "UP_PATH_CHG". |  |
| easAckInd | boolean | O | 0..1 | Identifies whether the EAS acknowledgement for UP path change event notification is expected. Set to "true" if the EAS acknowledge is expected. Default value is "false". |  |
| easChars | array(EasCharacteristics) | O | 1..N | Represents a list of EAS characteristics. Applicable when the "event" attribute within the AcrMgntEventSubsc data type is "ACR_MONITORING" or "ACR_FACILITATION". |  |

##### 8.7.5.2.5	Type: AcrMgntEventsNotification

Table 8.7.5.2.5-1: Definition of type AcrMgntEventsNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subpId | string | M | 1 | String identifying the Individual ACR Management Events Subscription for which the notification is delivered. |  |
| eventReports | array(AcrMgntEventReport) | M | 1..N | Represents a list of ACR management event reports. |  |

##### 8.7.5.2.6	Type: AcrMgntEventReport

Table 8.7.5.2.6-1: Definition of type AcrMgntEventReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| timeStamp | DateTime | O | 0..1 | Represents the time stamp of the detected event. |  |
| easEndPoint | EndPoint | C | 0..1 | Represents the endpoint of the T-EAS. Shall be provided when the "event" attribute is "ACR_MONITORING" or "ACR_FACILITATION". |  |
| upPathChgInfo | UpPathChangeInfo | C | 0..1 | Represents the UP Path change information. Shall be provided when the "event" attribute is "UP_PATH_CHG". |  |

Editor’s Note:	Definition of UpPathChangeInfo data type is FFS.

Editor’s Note:	It’s FFS about the reporting of availability of TrafficInfluence API.

##### 8.7.5.2.7	Type: FailureAcrMgntEventInfo

Table 8.7.5.2.7-1: Definition of type FailureAcrMgntEventInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| failureCode | AcrMgntEventFailureCode | M | 1 | Identifies the failure reason. |  |

Editor’s Note:	Definition of AcrMgntEventFailureCode data type is FFS.

##### 8.7.5.2.8	Type: TargetUeIdentification

Table 8.7.5.2.8-1: Definition of type TargetUeIdentification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | O | 0..1 | Represents external UE identifier. |  |
| intGrpId | GroupId | O | 0..1 | Represents a group of UEs identified by an Internal Group Identifier. |  |
| extGrpId | ExternalGroupId | O | 0..1 | Represents a group of UEs identified by an External Group Identifier. |  |
| ueIpAddr | IpAddr | O | 0..1 | Represents the UE IP address. |  |
| NOTE: Only one of above attributes shall be provided. |  |  |  |  |  |

#### 8.7.5.3	Simple data types and enumerations

##### 8.7.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.7.5.3.2	Simple data types

The simple data types defined in table 8.7.5.3.2-1 shall be supported.

Table 8.7.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 8.7.5.3.3	Enumeration: AcrMgntEvent

Table 8.7.5.3.3-1: Enumeration AcrMgntEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UP_PATH_CHG | The subscribed ACR Management Event is user plane path change. |  |
| ACR_MONITORING | The subscribed ACR Management Event is ACR monitoring. |  |
| ACR_FACILITATION | The subscribed ACR Management Event is ACR facilitation. |  |

### 8.7.6	Error Handling

General error responses are defined in clause 7.7.

### 8.7.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.7.7-1 lists the supported features for Eees_ACRManagementEvent API.

Table 8.7.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.8	Eees_SelectedTargetEAS API

### 8.8.1	API URI

The Eees_SelectedTargetEAS service shall use the Eees_SelectedTargetEAS API.

The request URIs used in HTTP requests from the Edge Application Server towards the Edge Enabler Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-selectedtargeteas".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.8.2.

### 8.8.2	Resources

None.

### 8.8.3	Custom Operations without associated resources

#### 8.8.3.1	Overview

The structure of the custom operation URIs of the Eees_SelectedTargetEAS API is shown in figure 8.8.3.1-1.

Figure 8.8.3.1-1: Custom operation URI structure of the Eees_SelectedTargetEAS API

Table 8.8.3.1-1 provides an overview of the custom operations and the applicable HTTP methods.

Table 8.8.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Declare | /declare | POST | Declare the T-EAS information. |

#### 8.8.3.2	Operation: Declare

##### 8.8.3.2.1	Description

This custom operation allows the S-EAS to declare the selected T-EAS information to the S-EES, as specified in 3GPP TS 23.558 [2].

##### 8.8.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 8.8.3.2.2-1 and 8.8.3.2.2-2.

Table 8.8.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SelectedEAS | M | 1 | Selected T-EAS information. |

Editor’s Note: Details of how the EAS security credentials are submitted in the HTTP POST message is FFS and to be updated based on security aspects defined by SA3

Table 8.8.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The declaration to T-EAS information has been processed successfully. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

### 8.8.4	Notifications

None.

### 8.8.5	Data Model

#### 8.8.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.8.5.1-1 specifies the data types defined specifically for the Eees_SelectedTargetEAS API.

Table 8.8.5.1-1: Eees_SelectedTargetEAS API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SelectedEAS | 8.8.5.2.2 | Used to indicate the information about the selected T-EAS. |  |

Table 8.8.5.1-2 specifies data types re-used by the Eees_SelectedTargetEAS API service.

Table 8.8.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to indicate the identifier of the UE. |  |
| EndPoint | Clause 8.1.5.2.5 | Used to indicate the end point information of the T-EAS. |  |

#### 8.8.5.2	Structured data types

##### 8.8.5.2.1	Introduction

##### 8.8.5.2.2	Type: SelectedEAS

Table 8.8.5.2.2-1: Definition of type SelectedEAS

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Identifier of the selected T-EAS. |  |
| ueId | Gpsi | M | 1 | Identifier of the UE. |  |
| endPt | EndPoint | M | 1 | Selected T-EAS end point information. |  |

Editor’s Note: The format of GPSI for ueId attribute is FFS and to be updated based on security aspects defined by SA3.

#### 8.8.5.3	Simple data types and enumerations

None.

### 8.8.6	Error Handling

General error responses are defined in clause 7.7.

### 8.8.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.8.7-1 lists the supported features for Eees_SelectedTargetEAS API.

Table 8.8.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 8.x	<API Name – Eees_xxx> API

Add a copy of this clause for a new API definition, adding all the clauses below. All the clauses are mandatory for each API. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

### 8.x.1	API URI

### 8.x.2	Resources

#### 8.x.2.1	Overview

Figure 8.x.2.1-1: Resource URI structure of the <API Name> API

Table 8.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

#### 8.x.2.2	Resource: <Resource name>

##### 8.x.2.2.1	Description

##### 8.x.2.2.2	Resource Definition

##### 8.x.2.2.3	Resource Standard Methods

###### 8.x.2.2.3.1	<Method Name>

Table 8.x.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |

This method shall support the request data structures specified in table 8.x.2.2.3.1-2 and the response data structures and response codes specified in table 8.x.2.2.3.1-3.

Table 8.x.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <method 1> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

Table 8.x.2.2.3.1-4: Headers supported by the <e.g. GET> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 8.x.2.2.3.1-5: Headers supported by the <e.g. 200> response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 8.x.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| <link name> e.g. search | <resource 1> e.g. Stored Search (Document) | <method 1> e.g. GET | <parameter> e.g. searchId | <description of the link> |

##### 8.x.2.2.4		Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

###### 8.x.2.2.4.1		Overview

Table 8.x.2.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

###### 8.x.2.2.4.2		Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

8.x.2.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

8.x.2.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 8.x.2.2.4.2.2-1 and the response data structure and response codes specified in table 8.x.2.2.4.2.2-2.

Table 8.x.2.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.2.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

### 8.x.3	Custom Operations without associated resources

#### 8.x.3.1	Overview

This clause will specify custom operations without any associated resource supported by this API.

Table 8.x.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

#### 8.x.3.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

##### 8.x.3.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

##### 8.x.3.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 8.x.3.2.2-1 and 8.x.3.2.2-2.

Table 8.x.3.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.3.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

#### 8.x.3.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 8.x.3.2

### 8.x.4	Notifications

#### 8.x.4.1	General

Table 8.x.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| <notification 1> e.g. Status Change Notification | < Callback URI > e.g. {StatusCallbackUri} | e.g POST | e.g. Notify Event |

#### 8.x.4.2	<notification 1>

##### 8.x.4.2.1	Description

##### 8.x.4.2.2	Notification definition

Callback URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 8.x.4.2.2-1.

Table 8.x.4.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 8.x.4.2.2-2 and the response data structures and response codes specified in table 8.x.4.2.2-3.

Table 8.x.4.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.4.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |

### 8.x.5	Data Model

#### 8.x.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <7.X related to EdgeApp design aspects common for all APIs> apply to this API

Table 8.x.5.1-1 specifies the data types defined specifically for the <API Name> API service.

Table 8.x.5.1-1: <API Name> API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 8.x.5.1-2 specifies data types re-used by the <API Name> API service.

Table 8.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

#### 8.x.5.2	Structured data types

##### 8.x.5.2.1	Introduction

##### 8.x.5.2.2	Type: <Data type name>

Table 8.x.5.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <attribute name> | "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N" | <only if applicable> |  |

#### 8.x.5.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.x.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.x.5.3.2	Simple data types

The simple data types defined in table 8.x.5.3.2-1 shall be supported.

Table 8.x.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

##### 8.x.5.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 8.x.5.3.3-1.

Table 8.x.5.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

### 8.x.6	Error Handling

### 8.x.7	Feature negotiation

General feature negotiation procedures are defined in clause <7.X>. Table 8.x.7-1 lists the supported features for <API name> API.

Table 8.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 9	Edge Configuration Server API Definitions

This clause will provide the definitions of all the EdgeApp APIs from ECS.

## 9.1	Eecs_EESRegistration API

### 9.1.1	API URI

The Eecs_EESRegistration service shall use the Eecs_EESRegistration API.

The request URIs used in HTTP requests from the Edge Enabler Server towards the Edge Configuration Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eecs-eesregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.1.2.

### 9.1.2	Resources

#### 9.1.2.1	Overview

Figure 9.1.2.1-1: Resource URI structure of the Eecs_EESRegistration API

Table 9.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EES Registrations | /registrations | POST | Registers a new EES at the Edge Configuration Server. |
| Individual EES Registration | /registrations/{registrationId} | GET | Fetch an individual EES registration resource. |
|  |  | PUT | Fully replace an individual EES registration resource. |
|  |  | DELETE | Remove an individual EES registration resource. |
|  |  | PATCH | Partially update an individual EES registration resource. |

Editor’s Note: It is FFS, to obtain the EES registration information from the "EES Registrations" collection resource.

Editor’s Note: Inclusion of the EES identifier in the resource URI structure is FFS.

#### 9.1.2.2	Resource: EES Registrations

##### 9.1.2.2.1	Description

This resource represents all the Edge Enabler Servers that are registered at a given Edge Configuration Server.

##### 9.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-eesregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 9.1.2.2.2-1.

Table 9.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 9.1.1 |

##### 9.1.2.2.3	Resource Standard Methods

###### 9.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 9.1.2.2.3.1-1.

Table 9.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.2.3.1-2 and the response data structures and response codes specified in table 9.1.2.2.3.1-3.

Table 9.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EESRegistration | M | 1 | EES registration request information. |

Table 9.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 201 Created | EES information is registered successfully at ECS. EES information registered with ECS is provided in the response body. The URI of the created resource shall be returned in the “Location” HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eecs-eesregistration/<apiVersion>/registrations/{registrationId} |

Table 9.1.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 9.1.2.2.4		Resource Custom Operations

None.

#### 9.1.2.3	Resource: Individual EES Registration

##### 9.1.2.3.1	Description

This Individual EES Registration resource represents an individual EES registered at a given Edge Configuration Server.

##### 9.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/eecs-eesregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 9.1.2.3.2-1.

Table 9.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 9.1.1 |
| registrationId | string | The EES registration identifier. |

##### 9.1.2.3.3	Resource Standard Methods

###### 9.1.2.3.3.1	GET

This method retrieves the EES information registered at Edge Configuration Server. This method shall support the URI query parameters specified in table 9.1.2.3.3.1-1.

Table 9.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.1-2 and the response data structures and response codes specified in table 9.1.2.3.3.1-3.

Table 9.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESResgistration | M | 1 | 200 OK | The EES registration information at the Edge Configuration Server. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 9.1.2.3.3.2	PUT

This method updates the EES registration information at Edge Configuration Server by completely replacing the existing registration data (except the value of "eesId" within EESProfile data type and the value of "suppFeat" attribute within the EESRegistration data type). This method shall support the URI query parameters specified in the table 9.1.2.3.3.2-1.

Table 9.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.2-2 and the response data structures and response codes specified in table 9.1.2.3.3.2-3.

Table 9.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EESRegistration | M | 1 | Details of the EES registration information to be updated |

Table 9.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 200 OK | The EES registration information updated successfully and the updated EES registration information is returned in the response. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Editor’s Note: It is FFS, if 204 No Content response message is applicable.

Table 9.1.2.3.3.2-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.2-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 9.1.2.3.3.3	DELETE

This method deregisters an EES registration from the ECS. This method shall support the URI query parameters specified in the table 9.1.2.3.3.3-1.

Table 9.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.3-2 and the response data structures and response codes specified in table 9.1.2.3.3.3-3.

Table 9.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Editor’s Note: Details of how the EES security credentials are submitted in the HTTP DELETE message is FFS and to be updated based on security aspects defined by SA3.

Table 9.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual EES registration information matching the registrationId is deleted. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.3-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.3-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.3-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 9.1.2.3.3.4	PATCH

This method partially updates the EES registration information at Edge Configuration Server. This method shall support the URI query parameters specified in the table 9.1.2.3.3.4-1.

Table 9.1.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.4-2 and the response data structures and response codes specified in table 9.1.2.3.3.4-3.

Table 9.1.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EESRegistrationPatch | M | 1 | Details of the EES registration information to be updated |

Table 9.1.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 200 OK | The Individual EES registration information was updated successfully and the updated EES registration information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual EES registration information was updated successfully. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.4-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.4-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.1.2.3.3.4-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 9.1.2.3.4		Resource Custom Operations

None.

### 9.1.3	Custom Operations without associated resources

None.

### 9.1.4	Notifications

None.

### 9.1.5	Data Model

#### 9.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 9.1.5.1-1 specifies the data types defined specifically for the Eecs_EESRegistration API service.

Table 9.1.5.1-1: Eecs_EESRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EESRegistration | 9.1.5.2.2 | The EES registration information on ECS. |  |
| EESProfile | 9.1.5.2.3 | The profile information related to the EES in the EESRegistration data type. |  |
| ACRScenario | 9.1.5.3.3 | The ACR scenarios supported by EES. |  |
| EESRegistrationPatch | 9.1.5.2.4 | To partially update the EES Registration information. |  |
| ServiceArea | 9.1.5.2.5 | The topological and geographic areas served by EES. |  |
| TopologicalServiceArea | 9.1.5.2.6 | The topological areas served by EES. |  |
| GeographicalServiceArea | 9.1.5.2.7 | The geographic areas served by EES. |  |

Table 9.1.5.1-2 specifies data types re-used by the Eecs_EESRegistration API service.

Table 9.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features defined in table 9.1.7-1. |  |
| DateTime | 3GPP TS 29.122 [6] | Used to capture the expiration time of EES registration. |  |
| RouteToLocation | 3GPP TS 29.571 [8] | Used to define the DNAIs associated with EES/EAS and the corresponding N6 routing information for each EES/EAS DNAI. |  |
| Ecgi | 3GPP TS 29.571 [8] | Represents an EUTRA cell identifier. |  |
| Ncgi | 3GPP TS 29.571 [8] | Represents an NR cell identifier. |  |
| Tai | 3GPP TS 29.571 [8] | Represents a tracking area identity. |  |
| GeographicArea | 3GPP TS 29.572 [11] | Identifies the geographical information of the user(s). |  |
| CivicAddress | 3GPP TS 29.572 [11] | Identifies the civic address information of the user(s). |  |
| PlmnId | 3GPP TS 29.122 [6] | Used to indicate only the list of PLMN Ids as part of topological service areas. |  |
| Dnai | 3GPP TS 29.571 [8] | Used to represent the list of DNAI(s) information associated with EES. |  |
| DateTimeRm | 3GPP TS 29.571 [8] | Used to capture the expiration time EES registration patch. |  |
| EndPoint | 8.1.5.2.5 | The end point information of the Edge Enabler Server in the EES profile. |  |

#### 9.1.5.2	Structured data types

##### 9.1.5.2.1	Introduction

##### 9.1.5.2.2	Type: EESRegistration

Table 9.1.5.2.2-1: Definition of type EESRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesProf | EESProfile | M | 1 | The profile information of the EES. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the EES registration. If the expiration time is not present, then it indicates that the registration of EES never expires. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

Editor’s Note: The data model to be updated with security credentials information, based on security aspects defined by SA3.

##### 9.1.5.2.3	Type: EESProfile

Table 9.1.5.2.3-1: Definition of type EESProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesId | string | M | 1 | The identifier of the EES |  |
| endPt | EndPoint | M | 1 | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |  |
| easIds | array(string) | O | 1..N | Identities of the Edge Application Servers registered with the EES. |  |
| provId | string | O | 0..1 | Identifier of the ECSP that provides the EES provider. |  |
| svcArea | ServiceArea | O | 0..1 | The list of geographical and topological areas that the EES serves. EECs in the UE that are outside the area shall not be served. |  |
| appLocs | array(Dnai) | O | 1..N | List of DNAI(s) associated with the EES. This is a list of potential locations of the applications. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |  |
| svcContSupp | array(ACRScenario) | O | 1..N | The ACR scenarios supported by the EES for service continuity. If this attribute is not present, then the EES does not support service continuity. |  |
| eecRegConf | boolean | M | 1 | Set to TRUE if the EEC is required to register on the EES to use edge services. Set to FALSE if EEC is not required to register on the EES to use edge services. |  |

##### 9.1.5.2.4	Type: EESRegistrationPatch

Table 9.1.5.2.4-1: Definition of type EESRegistrationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesProf | EESProfile | O | 0..1 | The profile information of the EES. |  |
| expTime | DateTimeRm | O | 0..1 | Identifies the expiration time for the EES registration. If the expiration time is not present, then it indicates that the registration of EES never expires. |  |
| NOTE: The value of the “eesId” attribute within the EESProfile data type shall be the same as the one within the EES Registration data type during the creation of resource procedure. |  |  |  |  |  |

##### 9.1.5.2.5	Type: ServiceArea

Table 9.1.5.2.5-1: Definition of type ServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| topServAr | TopologicalServiceArea | O | 0..1 | The topological service areas. (NOTE) |  |
| geoServAr | GeographicalServiceArea | O | 0..1 | The geographical service areas. |  |
| NOTE: The "topServAr" attribute is not applicable for untrusted functional elements (EESs and/or EASs deployed outside the MNO trust domain). |  |  |  |  |  |

##### 9.1.5.2.6	Type: TopologicalServiceArea

Table 9.1.5.2.6-1: Definition of type TopologicalServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecgis | array(Ecgi) | O | 1..N | This IE contains a list of E-UTRA cell identities. |  |
| ncgis | array(Ncgi) | O | 1..N | This IE contains a list of NR cell identities. |  |
| tais | array(Tai) | O | 1..N | This IE contains a list of tracking area identities. |  |
| plmnIds | array(PlmnId) | O | 1..N | List of PLMN IDs. (NOTE) |  |
| NOTE: A combination of these information elements should not have duplicate or overlapping information for the same topological Service Area. |  |  |  |  |  |

##### 9.1.5.2.7	Type: GeographicalServiceArea

Table 9.1.5.2.7-1: Definition of type GeographicalServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geoArs | array(GeographicArea) | O | 1..N | Identifies a list of geographic area of the user where the UE is located. |  |
| civicAddrs | array(CivicAddress) | O | 1..N | Identifies a list of civic addresses of the user where the UE is located. |  |

#### 9.1.5.3	Simple data types and enumerations

##### 9.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.1.5.3.2	Simple data types

The simple data types defined in table 9.1.5.3.2-1 shall be supported.

Table 9.1.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 9.1.5.3.3	Enumeration: ACRScenario

The enumeration ACRScenario represents the ACR scenarios supported. It shall comply with the provisions defined in table 9.1.5.3.3-1.

Table 9.1.5.3.3-1: Enumeration ACRScenario

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EEC_INITIATED |  |  |
| EEC_EXECUTED_VIA_SOURCE_EES |  |  |
| EEC_EXECUTED_VIA_TARGET_EES |  |  |
| SOURCE_EAS_DECIDED |  |  |
| SOURCE_EES_EXECUTED |  |  |

Editor’s Note: The enumeration values and the descriptions need alignment with ACR scenarios in 3GPP TS 23.558 [2].

### 9.1.6	Error Handling

General error responses are defined in clause 7.7.

### 9.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8 Table 9.1.7-1 lists the supported features for Eecs_EESRegistration API.

Table 9.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 9.2	Eecs_TargetEESDiscovery API

### 9.2.1	API URI

The Eecs_TargetEESDiscovery service shall use the Eecs_TargetEESDiscovery API.

The request URIs used in HTTP requests from the Edge Enabler Server towards the Edge Configuration Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eecs-targeteesdiscovery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.2.2.

### 9.2.2	Resources

#### 9.2.2.1	Overview

Figure 9.2.2.1-1: Resource URI structure of the Eecs_TargetEESDiscovery API

Table 9.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EES Profiles | /ees-profiles | GET | Retrieve the T-EES information. |

#### 9.2.2.2	Resource: EES Profiles

##### 9.2.2.2.1	Description

This resource allows the source EES (S-EES) to retrieve the target (T-EES) information from the ECS.

##### 9.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-targeteesdiscovery/<apiVersion>/ees-profiles

This resource shall support the resource URI variables defined in the table 9.2.2.2.2-1.

Table 9.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 9.2.1 |

##### 9.2.2.2.3	Resource Standard Methods

###### 9.2.2.2.3.1	GET

This method allows the S-EES to fetch the T-EES information as specified in 3GPP TS 23.558 [2], from the ECS with a given discovery filters. This method shall support the URI query parameters specified in table 9.2.2.2.3.1-1.

Table 9.2.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ees-id | string | M | 1 | Unique identifier of the S-EES. |
| eas-id | string | M | 1 | Unique identifier of the S-EAS. |
| target-dnai | Dnai | O | 0..1 | The DNAI information associated with the potential T-EES(s) and/or T-EAS(s). |
| ue-id | Gpsi | O | 0..1 | Identifier of the UE. |
| ue-location | LocationArea5G | O | 0..1 | The location information of the UE. |

Editor’s Note: Details of how the EES security credentials are submitted in the HTTP GET message is FFS and to be updated based on security aspects defined by SA3.

Editor’s Note: The format of GPSI for ue-id parameter is FFS and to be updated based on security aspects defined by SA3.

This method shall support the request data structures specified in table 9.2.2.2.3.1-2 and the response data structures and response codes specified in table 9.2.2.2.3.1-3.

Table 9.2.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.2.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ECSServProvResp | M | 1 | 200 OK | The EDN configuration and the T-EES information determined by the ECS based on the query parameters. |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.2.2.2.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.2.2.2.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 9.2.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 9.2.2.2.4		Resource Custom Operations

None.

### 9.2.3	Custom Operations without associated resources

None.

### 9.2.4	Notifications

None.

### 9.2.5	Data Model

#### 9.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API.

Table 9.2.5.1-1 specifies the data types defined specifically for the Eecs_TargetEESDiscovery API.

Table 9.2.5.1-1: Eecs_TargetEESDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 9.2.5.1-2 specifies data types re-used by the Eecs_TargetEESDiscovery API service.

Table 9.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Dnai | 3GPP TS 29.571 [8] | Used to indicate the target DNAI information. |  |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter. |  |
| LocationArea5G | 3GPP TS 29.122 [6] | Used to indicate the location information of the UE in the query parameter. |  |
| ECSServProvResp | 3GPP TS 24.558 [14] | The response to the target EES discovery request, which includes the EDN configuration along with list of EES(s) information. |  |

#### 9.2.5.2	Structured data types

None.

#### 9.2.5.3	Simple data types and enumerations

None.

### 9.2.6	Error Handling

General error responses are defined in clause 7.7.

### 9.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.2.7-1 lists the supported features for Eecs_TargetEESDiscovery API.

Table 9.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 9.x	<API Name – Eecs_xxx> API

Add a copy of this clause for a new API definition, adding all the clauses below. All the clauses are mandatory for each API. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

### 9.x.1	API URI

### 9.x.2	Resources

#### 9.x.2.1	Overview

Figure 9.x.2.1-1: Resource URI structure of the <API Name> API

Table 9.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

#### 9.x.2.2	Resource: <Resource name>

##### 9.x.2.2.1	Description

##### 9.x.2.2.2	Resource Definition

##### 9.x.2.2.3	Resource Standard Methods

###### 9.x.2.2.3.1	<Method Name>

Table 9.x.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |

This method shall support the request data structures specified in table 9.x.2.2.3.1-2 and the response data structures and response codes specified in table 9.x.2.2.3.1-3.

Table 9.x.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <method 1> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

Table 9.x.2.2.3.1-4: Headers supported by the <e.g. GET> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 9.x.2.2.3.1-5: Headers supported by the <e.g. 200> response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 9.x.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| <link name> e.g. search | <resource 1> e.g. Stored Search (Document) | <method 1> e.g. GET | <parameter> e.g. searchId | <description of the link> |

##### 9.x.2.2.4		Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

###### 9.x.2.2.4.1		Overview

Table 9.x.2.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

###### 9.x.2.2.4.2		Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

9.x.2.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

9.x.2.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 9.x.2.2.4.2.2-1 and the response data structure and response codes specified in table 9.x.2.2.4.2.2-2.

Table 9.x.2.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.2.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

### 9.x.3	Custom Operations without associated resources

#### 9.x.3.1	Overview

This clause will specify custom operations without any associated resource supported by this API.

Table 9.x.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

#### 9.x.3.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

##### 9.x.3.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

##### 9.x.3.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 9.x.3.2.2-1 and 9.x.3.2.2-2.

Table 9.x.3.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.3.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

#### 9.x.3.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 9.x.3.2

### 9.x.4	Notifications

#### 9.x.4.1	General

Table 9.x.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| <notification 1> e.g. Status Change Notification | < Callback URI > e.g. {StatusCallbackUri} | e.g POST | e.g. Notify Event |

#### 9.x.4.2	<notification 1>

##### 9.x.4.2.1	Description

##### 9.x.4.2.2	Notification definition

Callback URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 9.x.4.2.2-1.

Table 9.x.4.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 9.x.4.2.2-2 and the response data structures and response codes specified in table 9.x.4.2.2-3.

Table 9.x.4.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.4.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |

### 9.x.5	Data Model

#### 9.x.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <7.X related to EdgeApp design aspects common for all APIs> apply to this API

Table 9.x.5.1-1 specifies the data types defined specifically for the <API Name> API service.

Table 9.x.5.1-1: <API Name> API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 9.x.5.1-2 specifies data types re-used by the <API Name> API service.

Table 9.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

#### 9.x.5.2	Structured data types

##### 9.x.5.2.1	Introduction

##### 9.x.5.2.2	Type: <Data type name>

Table 9.x.5.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <attribute name> | "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N" | <only if applicable> |  |

#### 9.x.5.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.x.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.x.5.3.2	Simple data types

The simple data types defined in table 9.x.5.3.2-1 shall be supported.

Table 9.x.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

##### 9.x.5.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 9.x.5.3.3-1.

Table 9.x.5.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

### 9.x.6	Error Handling

### 9.x.7	Feature negotiation

General feature negotiation procedures are defined in clause <7.X>. Table 9.x.7-1 lists the supported features for <API name> API.

Table 9.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 10	Using Common API Framework

This clause will provide the EdgeApp APIs aspects when used with Common API Framework

# 11	Security

This clause will provide the security aspects.

Annex A (normative):
OpenAPI specification

This is a normative annex clause to specify the Open API representation of the all the EdgeApp APIs defined in this specification.

## A.1 General

This clause provides the introduction of the Open API specification files and their location.

This annex is based on the OpenAPI 3.0.0 specification [3] and provides corresponding representations of all APIs defined in the present specification in YAML format.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [4] and clause 5.3.1 of the 3GPP TS 29.501 [5] for further information).

## A.1	Eees_ACRManagementEvent API

openapi: 3.0.0

info:

title: EES ACR Management Event_API

description: |

API for EES ACR Management Event.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.0.0-alpha.1

externalDocs:

description: 3GPP TS 29.558 V0.5.0 Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

servers:

- url: '{apiRoot}/eees-acrmgntevent/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/subscriptions:

post:

description: Create an Individual ACR Management Event Subscription resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

callbacks:

ACRManagementEventsNotification:

'{request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: Successful case. The resource has been successfully created and no additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieve all the ACR Management Events Subscriptions information.

parameters:

- name: supp-feat

in: query

description: Features supported by the EAS.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get all of the active subscriptions)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

minItems: 1

description: All the active ACR management events subscriptions

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

description: Retrieve an Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

- name: supp-feat

in: query

description: Features supported by the EAS.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get the active subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Fully replace an existing Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

responses:

'200':

description: OK (Successful get the active subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual ACR Management Events Subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscriptionPatch'

responses:

'200':

description: The Individual ACR Management Events Subscription is successfully modified and the updated subscription information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Delete an existing Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

schemas:

AcrMgntEventsSubscription:

type: object

description: Represents an Individual ACR Management Events Subscription.

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

easId:

type: string

description: Identifier of an EAS.

tgtUeId:

$ref: '#/components/schemas/TargetUeIdentification'

eventSubscs:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventSubsc'

minItems: 1

description: The subscribed ACR management events.

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

easAckInd:

type: boolean

eventReports:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventReport'

minItems: 1

description: The ACR management event report(s).

easChars:

type: array

items:

$ref: '#/components/schemas/EasCharacteristics'

minItems: 1

description: A list of EAS characteristics.

failEventReports:

type: array

items:

$ref: '#/components/schemas/FailureAcrMgntEventInfo'

minItems: 1

description: Failure event reports.

requestTestNotification:

type: boolean

description: Set to true by the EAS to request the EES to send a test notification. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easId

- tgtUeId

- eventSubscs

- notificationDestination

AcrMgntEventSubsc:

type: object

description: Represents an ACR Management Event Subscription.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

eventFilter:

$ref: '#/components/schemas/AcrMgntEventFilter'

required:

- event

- eventFilter

AcrMgntEventsSubscriptionPatch:

type: object

description: Represents a modification request of Individual ACR Management Events Subscription.

properties:

eventSubscs:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventSubsc'

minItems: 1

description: The subscribed ACR management events.

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeTypeRm'

easAckInd:

type: boolean

easChars:

type: array

items:

$ref: '#/components/schemas/EasCharacteristics'

minItems: 1

description: A list of EAS characteristics.

EasCharacteristics:

type: string

# Editor’s Note: Definition of EasCharacteristics data type is FFS.

AcrMgntEventFilter:

type: string

# Editor’s Note: Definition of AcrMgntEventFilter data type is FFS.

AcrMgntEventsNotification:

type: object

description: Represents the ACR management events notification.

properties:

subpId:

type: string

description: String identifying the Individual ACR Management Events Subscription for which the notification is delivered.

eventReports:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventReport'

minItems: 1

description: A list of ACR management event reports.

required:

- subpId

- eventReports

AcrMgntEventReport:

type: object

description: Represents an ACR management event report.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

upPathChgInfo:

$ref: '#/components/schemas/UpPathChangeInfo'

easEndPoint:

type: string

# Editor’s Note: the reference for the EndPoint data tye is defined in Eees_EASRegistration API, and will be updated later.

required:

- event

FailureAcrMgntEventInfo:

type: object

description: Represents a failure ACR management event.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

failureCode:

$ref: '#/components/schemas/AcrMgntEventFailureCode'

required:

- event

- failureCode

TargetUeIdentification:

description: Identifies the target UE information.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

intGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

extGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

AcrMgntEventFailureCode:

type: string

# Editor’s Note: Definition of AcrMgntEventFailureCode data type is FFS.

UpPathChangeInfo:

type: string

# Editor’s Note: Definition of UpPathChangeInfo data type is FFS.

AcrMgntEvent:

anyOf:

- type: string

enum:

- UP_PATH_CHG

- ACR_MONITORING

- ACR_FACILITATION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- UP_PATH_CHG: User plane path change event.

- ACR_MONITORING: ACR monitoring event.

- ACR_FACILITATION: ACR facilitation event.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-01 | CT3#113e | C3-210300 |  |  |  | TS skeleton for Enabling Edge Applications, Application Programming Interface (API) specification; Stage 3. | 0.0.0 |
| 2021-01 | CT3#113e | C3-210322 |  |  |  | Inclusion of documents agreed in CT3#113e: C3-210182, C3-210301, C3-210321 | 0.1.0 |
| 2021-03 | CT3#114e | C3-211508 |  |  |  | Inclusion of documents agreed in CT3#114e: C3-211365, C3-211503, C3-211504, C3-211505, C3-211506, C3-211507 | 0.2.0 |
| 2021-04 | CT3#115e | C3-212520 |  |  |  | Inclusion of documents agreed in CT3#115e: C3-212367, C3-212368, C3-212369, C3-212370 | 0.3.0 |
| 2021-05 | CT3#116e | C3-213328 |  |  |  | Inclusion of documents agreed in CT3#116e: C3-213324, C3-213325, C3-213326, C3-213327, C3-213547 | 0.4.0 |
| 2021-08 | CT3#117e | C3-214571 |  |  |  | Inclusion of documents agreed in CT3#117e: C3-214387, C3-214513, C3-214389, C3-214570, C3-214388, C3-214390, C3-214391, C3-214392 | 0.5.0 |