| draft3GPP TS 29.565 V0.3.0 (2021-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Time Sensitive Communication and Time Synchronization Function Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

Introduction	9

1	Scope	10

2	References	10

3	Definitions, symbols and abbreviations	11

3.1	Definitions	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Overview	11

4.1	Introduction	11

4.2	Service Architecture	11

5	Services offered by the TSCTSF	12

5.1	Introduction	12

5.2	Ntsctsf_TimeSynchronization Service	13

5.2.1	Service Description	13

5.2.1.1	Overview	13

5.2.1.2	Network Functions	13

5.2.1.2.1	TSCTSF	13

5.2.1.2.2	NF Service Consumers	13

5.2.2	Service Operations	14

5.2.2.1	Introduction	14

5.2.2.2	Ntsctsf_TimeSynchronization_CapsSubscribe	14

5.2.2.2.1	General	14

5.2.2.2.2	Creating a new subscription	15

5.2.2.2.3	<Procedure 2 using service operation 1 of service 1>	16

5.2.2.3	Ntsctsf_TimeSynchronization_CapsUnsubscribe service operation	16

5.2.2.3.1	General	16

5.2.2.3.2	Unsubscription from capability notifications	16

5.2.2.4	Ntsctsf_TimeSynchronization_CapsNotify service operation	17

5.2.2.4.1	General	17

5.2.2.4.2	Notification about the capability of time synchronization service	17

5.2.2.5	Ntsctsf_TimeSynchronization_ConfigCreate	17

5.2.2.5.1	General	17

5.2.2.5.2	Creating a new configuration	18

5.2.2.6	Ntsctsf_TimeSynchronization_ConfigUpdate	19

5.2.2.6.1	General	19

5.2.2.6.2	Updating an existing configuration	19

5.2.2.7	Ntsctsf_TimeSynchronization_ConfigDelete	20

5.2.2.7.1	General	20

5.2.2.7.2	Deleting an existing configuration	20

5.2.2.8	Ntsctsf_TimeSynchronization_ConfigUpdateNotify	21

5.2.2.8.1	General	21

5.2.2.8.2	Notifying the current state of an existing configuration	21

5.2.2.9	Nnef_TimeSynchronization_ASTICreate	21

5.2.2.9.1	General	21

5.2.2.9.2	Creating a new configuration	21

5.2.2.10	Ntsctsf_TimeSynchronization_ASTIUpdate	23

5.2.2.10.1	General	23

5.2.2.10.2	Updating an existing configuration	23

5.2.2.11	Ntsctsf_TimeSynchronization_ASTIDelete	23

5.2.2.11.1	General	23

5.2.2.11.2	Delete an existing configuration	23

5.2.2.12	Ntsctsf_TimeSynchronization_ASTIGet	24

5.2.2.12.1	General	24

5.2.2.12.2	Retrieve the status of access stratum time distribution	24

5.3	Ntsctsf_TSCQoSandAssistance Service	25

5.3.1	Service Description	25

5.3.1.1	Overview	25

5.3.1.2	Network Functions	25

5.3.1.2.1	TSCTSF	25

5.3.1.2.2	NF Service Consumers.	26

5.3.2	Service Operations	26

5.3.2.1	Introduction	26

5.3.2.2	Ntsctsf_QoSandTSCAssistance_Create	26

5.3.2.2.1	General	26

5.3.2.2.2	Initial provisioning of TSC related service information	26

5.3.2.3	Ntsctsf_QoSandTSCAssistance_Update	28

5.3.2.3.1	General	28

5.3.2.3.2	Modification of TSC related service information	28

5.3.2.4	Ntsctsf_QoSandTSCAssistance_Delete	30

5.3.2.4.1	General	30

5.3.2.4.2	TSC AF application session context termination	30

5.3.2.6	Ntsctsf_QoSandTSCAssistance_Subscribe	31

5.3.2.6.1	General	31

5.3.2.6.2	Handling of subscription to events for the existing TSC application session context	31

5.3.2.7	Ntsctsf_QoSandTSCAssistance_Unsubscribe	32

5.3.2.7.1.	General	32

5.3.2.7.2	Unsubscription to events	33

5.3.2.5	Ntsctsf_QoSandTSCAssistance_Notify	33

5.3.2.5.1	 General	33

5.3.2.5.2	Notification about TSC application session context event	34

5.2.2.5.3	Notification about TSC application session context termination	34

6	API Definitions	35

6.1	Ntsctsf_TimeSynchronization Service API	35

6.1.1	Introduction	35

6.1.2	Usage of HTTP	36

6.1.2.1	General	36

6.1.2.2	HTTP standard headers	36

6.1.2.2.1	General	36

6.1.2.2.2	Content type	36

6.1.2.3	HTTP custom headers	36

6.1.3	Resources	37

6.1.3.1	Overview	37

6.1.3.2	Resource: Time Synchronization Exposure Subscriptions	38

6.1.3.2.1	Description	38

6.1.3.2.2	Resource Definition	38

6.1.3.2.3	Resource Standard Methods	38

6.1.3.2.4	Resource Custom Operations	39

6.1.3.3	Resource: Individual Time Synchronization Exposure Subscription	39

6.1.3.3.1	Description	39

6.1.3.3.2	Resource Definition	39

6.1.3.3.3	Resource Standard Methods	40

6.1.3.3.3.1	GET	40

6.1.3.3.3.2	DELETE	40

6.1.3.3.4	Resource Custom Operations	41

6.1.3.4	Resource: Time Synchronization Exposure Configurations	41

6.1.3.4.1	Description	41

6.1.3.4.2	Resource Definition	41

6.1.3.4.3	Resource Standard Methods	41

6.1.3.4.3.1	POST	41

6.1.3.4.4	Resource Custom Operations	42

6.1.3.5	Resource: Individual Time Synchronization Exposure Configuration	42

6.1.3.5.1	Description	42

6.1.3.5.2	Resource Definition	42

6.1.3.5.3	Resource Standard Methods	42

6.1.3.5.3.1	GET	42

6.1.3.5.3.2	PUT	43

6.1.3.5.3.3	DELETE	43

6.1.3.5.4	Resource Custom Operations	44

6.1.3.6	Resource: ASTI Configurations	44

6.1.3.6.1	Description	44

6.1.3.6.2	Resource Definition	44

6.1.3.6.3	Resource Standard Methods	44

6.1.3.6.3.1	POST	44

6.1.3.6.4	Resource Custom Operations	45

6.1.3.6.4.1	Overview	45

6.1.3.6.4.2	Operation: retrieve	45

6.1.3.6.4.2.1	Description	45

6.1.3.6.4.2.2	Operation Definition	45

6.1.3.7	Resource: Individual ASTI Configuration	46

6.1.3.7.1	Description	46

6.1.3.7.2	Resource Definition	46

6.1.3.7.3	Resource Standard Methods	46

6.1.3.7.3.2	PUT	46

6.1.3.7.3.3	DELETE	47

6.1.3.7.4	Resource Custom Operations	47

6.1.4	Custom Operations without associated resources	47

6.1.4.1	Overview	47

6.1.4.2	Operation: <operation 1>	48

6.1.4.2.1	Description	48

6.1.4.2.2	Operation Definition	48

6.1.4.3	Operation: < operation 2>	48

6.1.5	Notifications	48

6.1.5.1	General	48

6.1.5.2	Time Synchronization Capability Notification	49

6.1.5.2.1	Description	49

6.1.5.2.2	Target URI	49

6.1.5.2.3	Standard Methods	49

6.1.5.3	Time Synchronization Configuration Notification	50

6.1.5.3.1	Description	50

6.1.5.3.2	Target URI	50

6.1.5.3.3	Standard Methods	50

6.1.6	Data Model	50

6.1.6.1	General	50

6.1.6.2	Structured data types	52

6.1.6.2.1	Introduction	52

6.1.6.2.2	Type: TimeSyncExposureSubsc	53

6.1.6.2.3	Type: TimeSyncExposureSubsNotif	55

6.1.6.2.4	Type SubsEventNotification	55

6.1.6.2.5	Type: TimeSyncCapability	55

6.1.6.2.6	Type: PtpCapabilitiesPerUe	56

6.1.6.2.7	Type: AccessTimeDistributionData	56

6.1.6.2.8	Type: AsTimeDistributionParam	57

6.1.6.2.9	Type: StatusRequestData	57

6.1.6.2.10	Type: StatusResponseData	57

6.1.6.2.11	Type: ActiveUe	58

6.1.6.3	Simple data types and enumerations	58

6.1.6.3.1	Introduction	58

6.1.6.3.2	Simple data types	58

6.1.6.3.3	Enumeration: <EnumType1>	58

6.1.6.3.4	Enumeration: <EnumType2>	58

6.1.6.4	Data types describing alternative data types or combinations of data types	58

6.1.6.4.1	Type: <TypeName 1>	58

6.1.6.4.2	Type: <TypeName 2>	59

6.1.6.5	Binary data	59

6.1.6.5.1	Binary Data Types	60

6.1.6.5.2	< Binary Data 1 >	60

6.1.7	Error Handling	60

6.1.7.1	General	60

6.1.7.2	Protocol Errors	60

6.1.7.3	Application Errors	60

6.1.8	Feature negotiation	60

6.1.9	Security	61

6.2	Ntsctsf_QoSandTSCAssistance Service API	61

6.2.1	Introduction	61

6.2.2	Usage of HTTP	61

6.2.2.1	General	61

6.2.2.2	HTTP standard headers	61

6.2.2.2.1	General	61

6.2.2.2.2	Content type	62

6.2.2.3	HTTP custom headers	62

6.2.3	Resources	62

6.2.3.1	Overview	62

6.2.3.2	Resource: TSC Application Sessions	63

6.2.3.2.1	Description	63

6.2.3.2.2	Resource Definition	63

6.2.3.2.3	Resource Standard Methods	63

6.2.3.2.4	Resource Custom Operations	64

6.2.3.3	Resource: Individual TSC Application Sessions	64

6.2.3.3.1	Description	64

6.2.3.3.2	Resource Definition	64

6.2.3.3.3	Resource Standard Methods	65

6.2.3.3.3.1	GET	65

6.2.3.3.3.2	PATCH	65

6.2.3.3.4	Resource Custom Operations	66

6.2.3.3.4.1	Overview	66

6.2.3.3.4.2	Operation: delete	66

6.2.3.3.4.2.1	Description	66

6.2.3.3.4.2.2	Operation Definition	66

6.2.3.4	Resource: Events Subscription (Document)	67

6.2.3.4.1	Description	67

6.2.3.4.2	Resource definition	67

6.2.3.4.3	Resource Standard Methods	67

6.2.3.4.3.1	PUT	67

6.2.3.4.3.2	DELETE	68

6.2.3.4.4	Resource Custom Operations	68

6.2.4	Custom Operations without associated resources	68

6.2.5	Notifications	68

6.2.5.1	General	68

6.2.5.2	Event Notification	69

6.2.5.2.1	Description	69

6.2.5.2.2	Target URI	69

6.2.5.2.3	Standard Methods	69

6.2.5.3	Termination Request	69

6.2.5.3.1	Description	69

6.2.5.3.2	Target URI	69

6.2.5.3.3	Standard Methods	70

6.2.6	Data Model	70

6.2.6.1	General	70

6.2.6.2	Structured data types	71

6.2.6.2.1	Introduction	71

6.2.6.2.2	Type TscAppSessionContextData	72

6.2.6.2.3	Type EventsSubscReqData	73

6.2.6.2.4	Type TscAppSessionContextUpdateData	73

6.2.6.2.5	Type EventsSubscReqDataRm	73

6.2.6.2.6	Type EventsNotification	74

6.2.6.3	Simple data types and enumerations	74

6.2.6.3.1	Introduction	74

6.2.6.3.2	Simple data types	74

6.2.6.3.3	Enumeration: TscEvent	75

6.2.7	Error Handling	75

6.2.7.1	General	75

6.2.7.2	Protocol Errors	75

6.2.7.3	Application Errors	75

6.2.8	Feature negotiation	76

6.2.9	Security	76

Annex A (normative): OpenAPI specification	77

A.1	General	77

A.2	Ntsctsf_TimeSynchronization API	77

A.3	Ntsctsf_QoSandTSCAssistance API	84

Annex B (informative): Change history	94

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

The present document specifies the stage 3 protocol and data model for the Ntsctsf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the TSCTSF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[16]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[17]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[18]	IEEE 802.1Q: "Virtual Bridged Local Area Networks".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[20]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[21]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[22]	IETF RFC 7396: "JSON Merge Patch".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PTP	Precision Time Protocol

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization function

4	Overview

4.1	Introduction

The Ntsctsf services are offered by the TSCTSF to support the Time Sensitive Communications and Time Synchronization.

The following services are specified:

-	Ntsctsf_TimeSynchronization service;

-	Ntsctsf_QoSandTSCAssistance service.

4.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2].

The known NF service consumers of the Ntsctsf services are the Application Function (AF) within the operator’s trust domain and the Network Exposure Function (NEF).

The Ntsctsf services are provided by the TSCTSF and consumed by the NF service consumers (e.g. AF, NEF), as shown in figure 4.2-1 for the SBI representation model and in figure 4.2-2 for the reference point representation model.

Figure 4.2-1: Ntsctsf services architecture, SBI representation

Figure 4.2-2: Ntsctsf services architecture, reference point representation

5	Services offered by the TSCTSF

5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Ntsctsf_TimeSynchronization | 6.1 | TSCTSF Time Synchronization Service | TS29565_Ntsctsf_TimeSynchronization.yaml | ntsctsf-time-snyc | A.2 |
| Ntsctsf_QoSandTSCAssistance | 6.2 | TSCTSF QoS and TSC Assistance Service | TS29565_Ntsctsf_QoSandTSCAssistance.yaml | ntsctsf-qos-tscai | A.3 |

5.2	Ntsctsf_TimeSynchronization Service

5.2.1	Service Description

5.2.1.1	Overview

This service provides:

-	Authorization of NF Service Consumer requests for the subscription to the notification of the capability of time synchronization service.

-	Authorization of NF Service Consumer requests to create and update time synchronization configuration, and to activate and deactivate the time synchronization service as described in clause 5.27.1.8 of TS 23.501 [2].

-	Authorization of NF Service Consumer requests for the activation, update, and deactivation of the 5G access stratum time distribution.

NOTE:	The AF can use either the procedure specified in bullet 1) and 2) for configuring the (g)PTP instance in 5GS or the procedure specified in bullet 3) for controlling the 5G access stratum time distribution for a particular UE. The procedures are not intended to be used in conjunction.

5.2.1.2	Network Functions

5.2.1.2.1	TSCTSF

The TSCTSF supports to:

-	receive the request to create the subscription to the notification of the capability of time synchronization service from the NEF or AF and store it in the UDR;

-	receive the request to delete the subscription to the notification of the capability of time synchronization service from the NEF or AF and remove the information from the UDR;

-	receive the notification of the availability of the user plane node information from the PCF and subscribe the notification of user plane node related events at the PCF;

-	determine the capability of time synchronization service based on the capability information received from the DS-TT(s) and NW-TT and notify the NEF or AF of the capability;

-	receive the request to create or modify the time synchronization configuration from the NEF or AF, construct a PMIC to each DS-TT/UE to activate the time synchronization service in DS-TT, constructs PMIC(s) and UMIC to NW-TT to activate the time synchronization service in NW-TT and provision them to the PCF;

-	Notify the NEF or AF of the current state of the time synchronization service configuration;

-	receive the request of time synchronization deactivation from the NEF or AF and remove the time synchronization service configuration.

-	receive the request to activate or update the 5G access stratum time distribution configuration from the NEF or AF and store it in the UDR;

-	receive the request to delete the 5G access stratum time distribution configuration from the NEF or AF and remove the information from the UDR;

-	receive the request to query the status of the access stratum time distribution from the NEF or AF and respond to the NEF or AF with the status of the access stratum time distribution.

5.2.1.2.2	NF Service Consumers

The NF service consumer supports to:

-	send the request to create and delete the subscription to the notification of the capability of time synchronization service to TSCTSF;

-	receive the notification of the capability of time synchronization service;

-	send the request to create, modify and delete the time synchronization configuration to the TSCTSF;

-	send the request to create, modify and delete the 5G access stratum time distribution configuration to TSCTSF.

5.2.2	Service Operations

5.2.2.1	Introduction

Service operations defined for the Ntsctsf_TimeSynchronization service are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Ntsctsf_TimeSynchronization Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Ntsctsf_TimeSynchronization_CapsSubscribe | Allows the NF service consumer to create a subscription to the notification about the capability of time synchronization service for a list of UEs or a DNN/S-NSSAI combination. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_CapsUnsubscribe | Allows the NF service consumer to delete the subscription to the notification about capability of time synchronization service for a list of UEs or a DNN/S-NSSAI combination. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_CapsNotify | Allows the TSCTSF to notify the NF service consumer of the capability of time synchronization service. | TSCTSF |
| Ntsctsf_TimeSynchronization_ConfigCreate | Allows the NF service consumer to create a time synchronization configuration. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_ConfigUpdate | Allows the NF service consumers to update the time synchronization configuration. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_ConfigDelete | Allows the NF service consumer to delete the time synchronization configuration. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_ConfigUpdateNotify | Allows the TSCTSF to notify the NF service consumer of the state of time synchronization configuration. | TSCTSF |
| Ntsctsf_TimeSynchronization_ASTICreate | Allows the NF service consumer to create a 5G access stratum time distribution configuration. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_ASTIUpdate | Allows the NF service consumer to update a 5G access stratum time distribution configuration. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_ASTIDelete | Allows the NF service consumer to delete a 5G access stratum time distribution configuration. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_TimeSynchronization_ASTIGet | Allows the NF service consumer to query the status of the 5G access stratum time distribution configuration. | NF service consumer (e.g. AF, NEF) |

NOTE:	The NEF and the AF use the Ntsctsf_TimeSynchronization service in the same way.

5.2.2.2	Ntsctsf_TimeSynchronization_CapsSubscribe

5.2.2.2.1	General

This service operation is used by an NF service consumer to subscribe to notification of capability of time synchronization service for a list of UEs.

The following procedures using the Ntsctsf_TimeSynchronization_CapsSubscribe service operation are supported:

-	creating a new subscription.

5.2.2.2.2	Creating a new subscription

Figure 5.2.2.2.2-1 illustrates the creation of a subscription.

Figure 5.2.2.2.2-1: Creation of a subscription

To subscribe the notification of the capability of time synchronization service, the NF service consumer shall send an HTTP POST message to the TSCTSF to the URI "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions". The HTTP POST message shall include the TimeSyncExposureSubsc data structure as request body. The TimeSyncExposureSubsc data structure shall include:

-	the indication of the UEs to which the time synchronization capabilities is requested via:

-	identification of a list of individual UEs within a "supis" attribute;

-	indication of any UE within the "anyUeInd" attribute; or

-	identification of a group of UE(s) within the "interGroupId" attribute.

-	subscription to event(s) notification as "evSubsc" attribute;

-	notification URI within the "subsNotifUri" attribute;

-	notification correlation Id within the "subsNotifId" attribute;

and my include:

- DNN with the "dnn" attribute;

- S-NSSAI with the "snssai";

- the conditions to match for notifying the event within the "eventFilters" attribute;

- notification methods within the "notifMethods" attribute

- maximum number of reports within the "maxReportNbr" attribute;

- expiry time withinthe "expiry" attribute; and

- report period within the "repPeriod" attribute.

Upon receipt of the HTTP request from the NF service consumer, if the request is authorized, the TSCTSF shall:

-	create a new subscription;

-	assign a subscription correlation ID;

-	select an expiry time that is equal to or less than the expiry time potentially received in the request;

-	store the subscription;

-	interact with the UDR to store the request information in the UDR by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [14].

-	send an HTTP "201 Created" response with TimeSyncExposureSubsc data structure as response body and a Location header field containing the URI of the created individual subscription resource, i.e. "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subcriptionId}".

Editor's Note:	Error and redirection responses are FFS.

5.2.2.2.3	<Procedure 2 using service operation 1 of service 1>

And so on if there are more than 2 procedures that need to be described for the service.

Clauses 5.2.2.2.x are optional to include. They can be specified e.g. if a service operation is implemented using different combinations of resources and methods.

5.2.2.3	Ntsctsf_TimeSynchronization_CapsUnsubscribe service operation

5.2.2.3.1	General

This service operation is used by an NF service consumer to unsubscribe from the notifications.

The following procedure using the Ntsctsf_TimeSynchronization_CapsUnsubscribe service operation is supported:

-	unsubscription from capability notifications.

5.2.2.3.2	Unsubscription from capability notifications

Figure 5.2.2.3.2-1 illustrates the unsubscription from event notifications.

Figure 5.2.2.3.2-1: Unsubscription from capability notifications

To unsubscribe from event notifications, the NF service consumer shall send an HTTP DELETE request with: "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscription correlation ID of the existing subscription that is to be deleted.

Upon the reception of the HTTP DELETE request, if the received HTTP request is successfully processed and accepted, the TSCTSF shall:

-	remove the corresponding subscription; and

-	send an HTTP "204 No Content" response.

Editor's Note:	Error/Redirect responses are FFS

5.2.2.4	Ntsctsf_TimeSynchronization_CapsNotify service operation

5.2.2.4.1	General

This service operation is used by the TSCTSF to send notifications to NF service consumers upon the detection of the capability of the time synchronization service for a list of UEs.

The following procedure using the Ntsctsf_TimeSynchronization_CapsNotify service operation is supported:

-	notification about the capability of time synchronization service.

5.2.2.4.2	Notification about the capability of time synchronization service

Figure 5.2.2.4.2-1 illustrates the notification about the capability of time synchronization service.

Figure 5.2.2.4.2-1: Notification about the capability of time synchronization service

If the TSCTSF detects the capability of time time synchronization service for a list of UEs for which an NF service consumer has subscribed, the TSCTSF shall send an HTTP POST request with "{subsNotifUri}", as previously provided by the NF service consumer within the corresponding subscription, as URI and TimeSyncExposureSubsNotif data structure as request body that shall include:

-	Notification correlation ID provided by the NF service consumer during the subscription within "subsNotifId" attribute; and

-	information about the observed event(s) within the "eventNotifs" attribute that shall contain for each observed event an "SubsEventNotification" data structure that shall include:

1.	the detected event within the  "event" attribute;

2.	the capabilities of time synchronization service for one or more user plane nodes with the "timeSyncCapas" attribute.

Upon the reception of an HTTP POST, the NF service consumer shall send an HTTP "204 No Content" response for a successful processing.

Editor's Note:	Error/Redirect responses are FFS.

5.2.2.5	Ntsctsf_TimeSynchronization_ConfigCreate

5.2.2.5.1	General

This service operation is used by an NF service consumer to create a time synchronization configuration and activate the time synchronization service with the configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ConfigCreate service operation are supported:

-	creating a new configuration..

5.2.2.5.2	Creating a new configuration

Figure 5.2.2.5.2-1 illustrates the creation of a configuration.

Figure 5.2.2.5.2-1: Creation of a configuration

To create a configuration, the NF service consumer shall send an HTTP POST message to the TSCTSF to the URI "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations". The HTTP POST message shall include the TimeSyncExposureConfig data structure as request body, as shown in figure 5.2.2.5.2-1, step 1. The TimeSyncExposureConfig data structure shall include:

-	the user plane node Id within the "upNodeId" attribute;

-	the requested PTP instance within the "reqPtpIns" attribute;

-	the time domian within the "timeDom" attribute;

-	the notification URI within the "configNotifUri" attribute;

-	the notification correlation Id within the "configNotifId" attribute;

and may include:

- the "gmEnable" attribute set to true if the AF requests 5GS to act as a grandmaster for PTP or gPTP;

- the gandmaster priority with the "gmPrio" attribute; and

- the temporal validity condition within the "tempValidity" attribute.

Upon receipt of the HTTP request from the NF service consumer, if the request is authorized, the TSCTSF shall:

-	create a new resource, which represents a new "Individual Time Synchronization Exposure Configuration" instance, addressed by a URI as defined in subclause 6.1.3.5 and containing a TSCTSF created resource identifier;

-	send an HTTP "201 Created" response with TimeSyncExposureConfig data structure as response body and a Location header field containing the URI of the created Individual Time Synchronization Exposure Configuration resource, i.e. "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subcriptionId}/configuration/{configurationId}", as shown in figure 5.2.2.5.2-1, step 2;

-	use the {subscriptionId} within the requested URI and user plane node ID within the "upNodeId" attribute in the request to determine the target UEs and corresponding AF-sessions, and then contact with the PCF(s) to configure and initialize the PTP instance in the DS-TT(s) and NW-TT as defined in 3GPP TS 23.502 [3], clause 4.15.9.3.2, step 5-6.

Editor's Note:	Error responses are FFS.

5.2.2.6	Ntsctsf_TimeSynchronization_ConfigUpdate

5.2.2.6.1	General

This service operation is used by an NF service consumer to update a time synchronization configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ConfigUpdate service operation are supported:

-	Updating an existing  configuration..

5.2.2.6.2	Updating an existing configuration

Figure 5.2.2.6.2-1 illustrates the updating of an existing configuration.

Figure 5.2.2.6.2-1: Update of a configuration

To update a configuration, the NF service consumer shall send an HTTP PUT request to the resource "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations/{configurationId" representing an existing "Individual Time Synchronization Exposure Configuration" resource, as shown in figure 5.2.2.6.2-1, step 1, to modify the configuration.

The TimeSyncExposureConfig data structure provided in the request body shall include:

-	the user plane node Id within the "upNodeId" attribute;

NOTE 1:		The user plane node Id cannot be changed during the modification.

-	the requested PTP instance within the "reqPtpIns" attribute;

-	the time domain within the "timeDom" attribute;

NOTE 2:	The user plane node Id, the requested PTP instance and the time domain cannot be changed during the modification.

-	the notification URI within the "configNotifUri" attribute;

-	the notification correlation Id within the "configNotifId" attribute;

NOTE 2:		If the notification URI or notification correlation Id is not changed the previously value is included.

and may include:

- the "gmEnable" attribute set to true if the AF requests 5GS to act as a grandmaster for PTP or gPTP;

- the gandmaster priority with the "gmPrio" attribute; and

- the temporal validity condition within the "tempValidity" attribute.

Upon receipt of the corresponding HTTP PUT message, if the request is authorized, theTSCTSF shall:

-	update the existing "Individual Time Synchronization Exposure Configuration" resource;

-	send a HTTP response including "200 OK" status code with TimeSyncExposureConfig data structure or "204 No Content" status code, as shown in figure 5.2.2.6.2-1, step 2;

-	use the {subscriptionId} within the requested URI and user plane node ID within the "upNodeId" attribute in the request to determine the target UEs and corresponding AF-sessions, and then contact with the PCF(s) to configure and initialize the PTP instance in the DS-TT(s) and NW-TT as defined in 3GPP TS 23.502 [3], clause 4.15.9.3.2, step 5-6.

Editor's Note:	Error/Redirect responses are FFS.

5.2.2.7	Ntsctsf_TimeSynchronization_ConfigDelete

5.2.2.7.1	General

This service operation is used by an NF service consumer to delete a time synchronization configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ConfigDelete service operation are supported:

-	Deleting an existing  configuration..

5.2.2.7.2	Deleting an existing configuration

Figure 5.2.2.7.2-1 illustrates the deleting of an existing configuration.

Figure 5.2.2.7.2-1: Deletion of a configuration

To delete a configuration, the NF service consumer shall send an HTTP DELETE request to the resource "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations/{configurationId}" representing an existing "Individual Time Synchronization Exposure Configuration" resource, as shown in figure 5.2.2.7.2-1, step 1, to delete the configuration.

Upon the reception of an HTTP DELETE request and if the HTTP DELETE request is accepted by the TSCTSF, the TSCTSF shall:

-	remove the corresponding configuration and respond with "204 No Content" as shown in figure 5.2.2.7.2-1, step 2;

-	use the {configurationId} within the requested URI to identify the time synchronization service configuration and the corresponding AF sessions, and then contact with the PCF(s) to disable the corresponding PTP instance(s) in the DS-TT(s) and NW-TT as defined in 3GPP TS 23.502 [3], clause 4.15.9.3.4, step 5-6.

Editor's Note:	Error/Redirect responses are FFS.

5.2.2.8	Ntsctsf_TimeSynchronization_ConfigUpdateNotify

5.2.2.8.1	General

This service operation is used by an TSCTSF to notify the NF Service Consumer of the current state of the time synchronization configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation are supported:

-	notification about the current state of the time synchronization configuration.

5.2.2.8.2	Notifying the current state of an existing configuration

Figure 5.2.2.8.2-1 illustrates the notification about the current state of the time synchronization configuration.

Figure 5.2.2.8.2-1: Notification about the current state of the time synchronization configuration

When the TSFCST receives the notification for each DS-TT and NW-TT from the PCF, the TSCTSF shall determine the current state of the time synchronization configuration and sends an HTTP POST request with "{configNotifUri}", as previously provided by the NF service consumer within the corresponding configuration, as URI and the TimeSyncExposureConfigNotif data structure as request body, as shown in figure 5.2.2.8.2-1, step 1.

The TimeSyncExposureConfigNotif data structure as request body that shall include:

-	Notification correlation ID provided by the NF service consumer during the subscription within "subsNotifId" attribute;

-	current state of the time synchronization configuration.

Editor's Note:	Error/Redirect responses are FFS.

5.2.2.9	Nnef_TimeSynchronization_ASTICreate

5.2.2.9.1	General

This service operation is used by an NF service consumer to create a 5G access stratum time distribution configuration.

The following procedures using the Nnef_TimeSynchronization_ASTICreate service operation are supported:

-	creating a new configuration..

5.2.2.9.2	Creating a new configuration

Figure 5.2.2.9.2-1 illustrates the creation of a configuration.

Figure 5.2.2.9.2-1: Creation of a configuration

To create a configuration, the NF service consumer shall send an HTTP POST message to the TSCTSF to the URI "{apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations". The HTTP POST message shall include the AccessTimeDistributionData data structure as request body, as shown in figure 5.2.2.9.2-1, step 1. The AccessTimeDistributionData data structure shall include:

-	one of the indication of the UEs to which the 5G access stratum time distribution configuration is requested via:

-	identification of a list of individual UEs within the "supis" attribute;

-	indication of any UE within the "anyUeInd" attribute; or

-	identification of a group of UE(s) within the "interGrpId" attribute.

-	5G access stratum time distribution parameters within the "asTimeDisParam" attribute;

- DNN with the "dnn" attribute if the "anyUeInd" attribute is included and set to "true";

- S-NSSAI with the "snssai" attribute if the "anyUeInd" attribute is included and set to "true".

Within the "asTimeDisParam" attribute inside the AccessTimeDistributionData data structure, the NF service consumer:

- shall include the "asTimeDisEnabled" attribute set to true if the access stratum time distribution via Uu reference point should be activated (otherwise, if the access stratum time distribution via Uu reference point should be inactive, the "asTimeDisEnabled" attribute may either be omitted or included and set to "false");

- may include the time synchronization error budget within the "timeSyncErrBudget" attribute;

- may include the temporal validity condition within the "tempValidity" attribute.

Upon receipt of the HTTP request from the NF service consumer, if the request is authorized, the TSCTSF shall:

-	create a new resource, which represents a new "Individual ASTI Configuration" instance, addressed by a URI as defined in subclause 6.1.3.7 and containing a TSCTSF created resource identifier;

-	send an HTTP "201 Created" response with AccessTimeDistributionData data structure as response body and a Location header field containing the URI of the created Individual ASTI Configuration resource, i.e. "{apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations/{astiConfigId}", as shown in figure 5.2.2.9.2-1, step 2.

Editor's Note:	Error responses are FFS.

Editor's Note:	Interaction with the PCF is FFS.

5.2.2.10	Ntsctsf_TimeSynchronization_ASTIUpdate

5.2.2.10.1	General

This service operation is used by an NF service consumer to update a 5G access stratum time distribution configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ASTIUpdate service operation are supported:

-	Updating an existing  configuration..

5.2.2.10.2	Updating an existing configuration

Figure 5.2.2.10.2-1 illustrates the updating of an existing configuration.

Figure 5.2.2.10.2-1: Update of a configuration

To update a configuration, the NF service consumer shall send an HTTP PUT request to the resource "{apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations/{astiConfigId}" representing an existing "Individual ASTI Configuration" resource, as shown in figure 5.2.2.10.2-1, step 1, to modify the configuration.

The AccessTimeDistributionData data structure provided in the request body shall include the updated information as defined in clause 5.2.2.9.2.

Upon receipt of the corresponding HTTP PUT message, if the request is authorized, theTSCTSF shall update the existing "Individual ASTI Configuration" resource. Then the TSCTSF shall send a HTTP response including "200 OK" status code with AccessTimeDistributionData data structure or "204 No Content" status code, as shown in figure 5.2.2.10.2-1, step 2.

Editor's Note:	Error/Redirect responses are FFS.

Editor's Note:	Interaction with PCF is FFS.

5.2.2.11	Ntsctsf_TimeSynchronization_ASTIDelete

5.2.2.11.1	General

This service operation is used by an NF service consumer to delete a 5G access stratum time distribution configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ASTIDelete service operation are supported:

-	Delete an existing configuration..

5.2.2.11.2	Delete an existing configuration

Figure 5.2.2.11.2-1 illustrates the deleting of an existing configuration.

Figure 5.2.2.11.2-1: Deletion of a configuration

To delete a configuration, the NF service consumer shall send an HTTP DELETE request to the resource "{apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations/{astiConfigId}" representing an existing "Individual ASTI Configuration" resource, as shown in figure 5.2.2.11.2-1, step 1, to delete the configuration.

Upon the reception of an HTTP DELETE request from the NF service consumer, if the HTTP DELETE request is authorized, the TSCTSF shall remove the corresponding configuration and respond with "204 No Content" as shown in figure 5.2.2.11.2-1, step 2.

Editor's Note:	Error/Redirect responses are FFS.

Editor's Note:	Interaction with PCF is FFS.

5.2.2.12	Ntsctsf_TimeSynchronization_ASTIGet

5.2.2.12.1	General

This service operation is used by an NF service consumer to retrieve ths status of the access stratum time distribution for a list of UEs.

The following procedures using the Ntsctsf_TimeSynchronization_ASTIGet service operation are supported:

-	Retrieve the status of access stratum time distribution.

5.2.2.12.2	Retrieve the status of access stratum time distribution

Figure 5.2.2.12.2-1 illustrates the retrieval of the status of access stratum time distribution.

Figure 5.2.2.12.2-1: Retrieval of the status of access stratum time distribution

To retrieve the status of access stratum time distribution, the NF service consumer shall send an HTTP POST request to the resource "{apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations/retrieve". The HTTP POST message shall include the StatusRequestData data structure as request body, as shown in figure 5.2.2.11.2-1, step 1. The StatusRequestData data structure shall include:

-	identification of a list of individual UEs within the "supis" attribute;

and may include:

- DNN with the "dnn" attribute

- S-NSSAI with the "snssai" attribute.

Upon the reception of an HTTP POST request and if the HTTP POST request is accepted by the TSCTSF, the TSCTSF shall send an HTTP "200 OK" response with the StatusResponseData data structure as response body, as shown in figure 5.2.2.11.2-1, step 2.

Within the StatusResponseData data structure, TSCTSF may include:

- a list of UE identifier(s) whose status of the access stratum time distribution is inactive within the "inactiveUes" attribute;

- the "activeUes" attribute containing one or more the ActiveUe instances which includes the UE identifier whose status of the access stratum time distribution is active within the "supi" attribute and optionally the requested time synchronization error budget within the "timeSyncErrBudget" attribute.

Editor's Note:	Error/Redirect responses are FFS.

Editor's Note:	Interaction with PCF is FFS.

Editor's Note:	Whether to consider GET method is FFS.

5.3	Ntsctsf_TSCQoSandAssistance Service

And so on if there are more than two services offered by the NF. Same structure as in clause 5.2.

5.3.1	Service Description

5.3.1.1	Overview

This service provides:

-	Authorization of NF Service Consumer requests for the resource reservation for TSC.

-	NF Service Consumer request to reserve or update resources for handling traffic characterized by TSC QoS parameters as described in clause 6.1.3.22 of TS 23.503 [19].

5.3.1.2	Network Functions

5.3.1.2.1	TSCTSF

The TSCTSF supports to:

-	receive the request to reserve or update a specific QoS or a specific QoS with additional Alternative QoS for an AF session;

-	receive the request to delete the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements

-	receive the request to subscribe or unsubscribe to the event(s) about the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements;

-	determine the requested PDB and construct the TSC Assistance Container; and

-	Notify the NF service consumer of the event(s).

5.3.1.2.2	NF Service Consumers.

The NF service consumer supports to:

-	send the request to reserve or update a specific QoS or a specific QoS with additional Alternative QoS for an AF session;

-	send the request to delete the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements

-	send the request to subscribe or unsubscribe to the event(s); and

-	receive the notification of the event(s).

5.3.2	Service Operations

5.3.2.1	Introduction

Service operations defined for the Ntsctsf_QoSandTSCAssistance service are shown in table 6.3.2.1-1.

Table 5.3.2.1-1: Ntsctsf_TimeSynchronization Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Ntsctsf_QoSandTSCAssistance_Create | Allows the NF service consumer to request the network to provide a specific QoS or a specific QoS with additional Alternative QoS for an AF session. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_QoSandTSCAssistance_Update | Allows the NF service consumer to request the network to update the QoS or the QoS with additional Alternative QoS for an AF session | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_QoSandTSCAssistance_Delete | Allows the NF service consumer to request the network to delete the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements. | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_QoSandTSCAssistance_Notify | Allows the TSCTSF to report the QoS Flow level event(s) to the NF service consumer. | TSCTSF |
| Ntsctsf_QoSandTSCAssistance_Subscribe | Allows the NF service consumer to subscribe to the event(s). | NF service consumer (e.g. AF, NEF) |
| Ntsctsf_QoSandTSCAssistance_Unsubscribe | Allows the NF service consumer to unsubscribe to the event(s). | NF service consumer (e.g. AF, NEF) |

NOTE:	The NEF and the AF use the Ntsctsf_QoSandTSCAssistance service in the same way.

5.3.2.2	Ntsctsf_QoSandTSCAssistance_Create

5.3.2.2.1	General

This service operation is used by an NF service consumer to request the network to provide a specific QoS for an AF session.

The following procedures using the Ntsctsf_QoSandTSCAssistance_Create service operation are supported:

-	Initial provisioning of TSC related service information.

Editor's Note:	The detail procedures, e.g. provisioning of QoS monitoring information, are FFS.

5.3.2.2.2	Initial provisioning of TSC related service information

This procedure is used to set up a TSC AF application session context for the service as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.3.2.2.2-1 illustrates the initial provisioning of TSC related service information.

Figure 5.3.2.2.2-1: Initial provisioning of TSC related service information

When a new TSC AF application session context is being established and media information for this application session context is available at the NF service consumer and the related media requires PCC control, the NF service consumer shall invoke the Ntsctsf_QoSandTSCAssistance_Create service operation by sending the HTTP POST request to the resource URI representing the "TSC Application Sessions" collection resource of the TSCTSF, as shown in figure 5.3.2.2.2-1, step 1.

The NF service consumer shall include the "TscAppSessionContextData" data type in the payload body of the HTTP POST request in order to request the creation of the "Individual TSC Application Session Context" resource. The "Individual TSC Application Session Context" resource and the "Events Subscription" sub-resource are created as described below.

The NF service consumer shall include in the "TscAppSessionContextData" data structure:

-	the AF identifier within the "afId" attribute;

-	either the IP address (IPv4 or IPv6) of the PDU session within the "ueIpAddr" attribute for IP type PDU session or the MAC address of the DS-TT port within the "ueMac" attribute for Ethernet type PDU sessions.;

Editor's Note:	How to get the address of DS-TT by the NF service consumer is FFS.

-	either the flow information within the "flowInfo" or "ethFlowInfo" attribute or the Application Id within the "appId" attribute;

-	the QoS reference within the "qosReference" attribute;

-	the URI where the TSCTSF can request to the NF service consumer to delete the "Individual TSC Application Session Context" resource within the "notifUri" attribute;

and may include:

-	the DNN within the "dnn" attribute;

-	the S-NSSAI within the "snssai" attribute;

-	the domain identity in the "ipDomain" attribute;

-	an ordered list of QoS references within the "altQosReferences" attribute;

-	TSC QoS requirement within the "tscQosReq" attribute; and

-	the request of the notification of certain user plane events within the "evSubsc" attribute. Within the EventsSubscReqData data structure, the NF service consumer shall include:

-	the URI where the TSCTSF sends the event notification to the NF service consumer within the "notifUri" attribute;

-	a Notification Correlation Identifier for the requested notifications within the "notifCorreId" attribute;

-	subscribed the events within the "events" attribute;

-	the usage threshold within the "usgThres" attribute if the "USAGE_REPORT" event is subscribed; and

-	QoS monitoring information within the "qosMon" attribute if the "QOS_MONITORING" event is subscribed.

Upon the reception of this HTTP POST request, the TSCTSF shall:

- interact with the PCF by triggering a Npcf_PolicyAuthorization_Create request to provision the related parameters to the PCF as defined in 3GPP TS 29.514 [20];

- if the Requested 5GS delay is received from NF service consumer, calculate a Requested PDB by subtracting the UE-DS-TT residence time provided by the PCF from the Requested 5GS delay;

-	if receiving a successful response from the PCF, the TSCSTF shall create an "Individual TSC Application Session Context" resource and send to the NF service consumer a "201 Created" response to the HTTP POST request, as shown in figure 5.3.2.2.2-1, step 2. If the "evSubsc" attribute is received, the "Events Subscription" sub-resource shall be created within the "Individual TSC Application Session Context" resource. The TSCTSF shall include in the "201 Created" response:

-	a Location header field; and

-	a "TscAppSessionContextData" data type in the payload body.

The Location header field shall contain the URI of the created "Individual TSC Application Session Context" i.e. "{apiRoot}/ntsctsf-qos-tscai/v1/tsc-app-sessions/{appSessionId}".

When "Events Subscription" sub-resource is created in this procedure, the NF service consumer shall build the sub-resource URI by adding the path segment "/events-subscription" at the end of the URI path received in the Location header field.

Editor's Note:	Error and redirection responses are FFS.

5.3.2.3	Ntsctsf_QoSandTSCAssistance_Update

5.3.2.3.1	General

This service operation is used by an NF service consumer to request the network to update the QoS and/or additional Alternative QoS for an AF session.

The following procedures using the Ntsctsf_QoSandTSCAssistance_Update service operation are supported:

-	Modification of TSC related service information.

Editor's Note:	The detail procedures, e.g. Modification of QoS monitoring information, are FFS.

5.3.2.3.2	Modification of TSC related service information

This procedure is used to modify an existing TSC application session context as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.3.2.3.2-1 illustrates the modification of TSC related service information using HTTP PATCH method.

Figure 5.3.2.3.2-1: Modification of TSC related service information using HTTP PATCH

The NF service consumer may modify the TSC application session context information at any time and invoke the Ntsctsf_QoSandTSCAssistance_Update service operation by sending the HTTP PATCH request message to the resource URI representing the "Individual TSC Application Session Context" resource, as shown in figure 5.3.2.3.2-1, step 1, with the modifications to apply.

The JSON body within the PATCH request shall include the "TscAppSessionContextUpdateData" data type and shall be encoded according to "JSON Merge Patch", as defined in IETF RFC 7396 [22].

The NF service consumer may include in the "TscAppSessionContextUpdateData" data structure:

- the updated flow information within the "flowInfo" or "ethFlowInfo" attribute;

- the updated application Id within the "appId" attribute;

- the updated QoS reference within the "qosReference" attribute;

- the update URI where the TSCTSF can request to the NF service consumer to delete the "Individual TSC Application Session Context" resource within the "notifUri".

- the updated ordered list of QoS references within the "altQosReferences" attribute;

- the updated TSC QoS requirement within the "tscQosReq" attribute; and

- the updated event subscription information within the "evSubsc" attribute. Within the EventsSubscReqDataRm data structure, the NF service consumer shall include:

-	the new complete list of subscribed events within the "events" attribute;

-	when the NF service consumer requests to update the additional information related to an event (e.g. the NF service consumer needs to provide new thresholds to the TSCTSF in the "usgThres" attribute related to the "USAGE_REPORT" event), the additional information within the corresponding attribute(s).

NOTE 1:	Note that when the NF service consumer requests to remove an event, this event is not included in the "events" attribute.

NOTE 2:	When an event is included in the "events" attribute and its related additional information is set to null, the PCF considers the subscription to this event is active, but the related procedures stop applying.

NOTE 3:	When an event is removed from the "events" attribute but its related information is not set to null, the PCF considers the subscription to this event is terminated, the related additional information is removed, and the related procedures stop applying.

The NF service consumer shall remove existing event subscription information by setting to null the "evSubsc" attribute included in "TscAppSessionContextUpdateData".

NOTE 4:	The "notifUri" attribute within the EventsSubscReqData data structure can be modified to request that subsequent notifications are sent to a new NF service consumer.

Upon the reception of this HTTP PATCH request, the TSCTSF shall

-	if the updated Requested 5GS delay is received from NF service consumer, re-calculate a Requested PDB by subtracting the UE-DS-TT residence time provided by the PCF from the Requested 5GS delay;

-	interact with the PCF by triggering a Npcf_PolicyAuthorization_Update request to provision the related parameters to the PCF as defined in 3GPP TS 29.514 [20];

-	if receiving a successful response from the PCF, the TSCSTF shall update the "Individual TSC Application Session Context" resource and send a "200 OK" or "204 No Content" response to the HTTP POST request to the NF service consumer, as shown in figure 5.3.2.3.2-1, step 2.

Editor's Note:	Error and redirection responses are FFS.

5.3.2.4	Ntsctsf_QoSandTSCAssistance_Delete

5.3.2.4.1	General

This service operation is used by an NF service consumer to request the network to delete the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements.

The following procedures using the Ntsctsf_QoSandTSCAssistance_Delete service operation are supported:

-	TSC AF application session context termination.

5.3.2.4.2	TSC AF application session context termination

This procedure is used to terminate an AF application session context for the service as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.3.2.4.2-1 illustrates the application session context termination.

Figure 5.3.2.4.2-1: Application session context termination

When a TSC AF session is terminated, and if the TSC AF application session context was created as described in clause 5.3.2.2, the NF service consumer shall invoke the Ntsctsf_QoSandTSCAssistance_Delete service operation to the TSCTSF using an HTTP POST request, as shown in figure 5.3.2.4.2-1, step 1.

The NF service consumer shall set the request URI to "{apiRoot}/ntsctsf-qos-tscai/v1/tsc-app-sessions/{appSessionId}/delete".

The NF service consumer may include in the body of the HTTP POST the "EventsSubscReqData" data type with the "evSubsc" attribute indicating the corresponding list of events to subscribe to.

When the TSCTSF receives the HTTP POST request from the NF service consumer, indicating the termination of the TSC AF application session context information, the TSCTSF shall acknowledge that request by sending an HTTP response message with the corresponding status code.

If the HTTP POST request from the NF service consumer is accepted, the TSCTSF shall send to the NF service consumer:

a)	if event information is reported, TSCTSF shall defer sending the response to the NF service consumer and shall immediately interact with the PCF to terminate the AF session with the event report, as specified in 3GPP TS 29.514 [20]. After receiving the event information from the PCF, the TSCTSF shall send a "200 OK" response to HTTP POST request, as shown in figure 5.3.2.4.2-1, step 2a, including in the "EventsNotification" to report to the NF service consumer;

b)	otherwise, the TSCTSF shall send to the NF service consumer a "204 No Content".

Editor's Note:	Error and redirection responses are FFS.

5.3.2.6	Ntsctsf_QoSandTSCAssistance_Subscribe

5.3.2.6.1	General

The Ntsctsf_QoSandTSCAssistance_Subscribe service operation enables NF service consumers handling of subscription to events for the existing TSC application session context. Subscription to events shall be created:

-	within the application session context establishment procedure by invoking the Ntsctsf_QoSandTSCAssistance_Subscribe service operation, as described in subclause 5.3.2.2; or

-	within the TSC application session context modification procedure by invoking the Ntsctsf_QoSandTSCAssistance_Subscribe service operation, as described in subclause 5.3.2.3; or

-	by invoking the Ntsctsf_QoSandTSCAssistance_Subscribe service operation for the existing TSC application session context, as described in subclause 5.3.2.6.2.

The following procedures using the Ntsctsf_QoSandTSCAssistance_Subscribe service operation is supported:

-	Handling of subscription to events for the existing TSC application session context.

5.3.2.6.2	Handling of subscription to events for the existing TSC application session context

This procedure is used to create a subscription to events for the existing TSC AF application session context bound to the corresponding PDU session or to modify an existing subscription, as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.3.2.6.2-1 illustrates the creation of events subscription information using HTTP PUT method.

Figure 5.3.2.6.2-1: Creation of events subscription information using HTTP PUT

Figure 5.3.2.6.2-2 illustrates the modification of events subscription information using HTTP PUT method.

Figure 5.3.2.6.2-2: Modification of events subscription information using HTTP PUT

When the NF service consumer decides to create a subscription to one or more events for the existing TSC application session context or to modify an existing subscription previously created by itself at the TSCTSF, the NF service consumer shall invoke the Ntsctsf_QoSandTSCAssistance_Subscribe service operation by sending the HTTP PUT request to the resource URI representing the "Events Subscription" sub-resource in the PCF, as shown in figure 5.3.2.6.2-1, step 1 and figure 5.3.2.6.2-2, step 1. The NF service consumer shall provide in the "EventsSubscReqData" data type of the body of the HTTP PUT request:

-	the "events" attribute with the list of events to be subscribed; and

-	the "notifUri" attribute that includes the Notification URI to indicate to the TSCTSF where to send the notification of the subscribed events if not provided before.

NOTE 1:	The "notifUri" attribute within the EventsSubscReqData data structure can be modified to request that subsequent notifications are sent to a new NF service consumer.

Upon the reception of the HTTP PUT request from the NF service consumer, the PCF shall decide whether the received HTTP PUT request is accepted.

If the PCF accepted the HTTP PUT request to create a subscription to events, the PCF shall create the "Events Subscription" sub-resource and shall send the HTTP response message to the NF service consumer as shown in figure 5.3.2.6.2-1, step 2. The TSCTSF shall include in the "201 Created" response:

-	a Location header field that shall contain the URI of the created "Events Subscription" sub-resource i.e. "{apiRoot}/ntsctsf-qos-tscai/v1/tsc-app-sessions/{appSessionId}/events-subscription"; and

-	a response body with the "EventsSubscReqData" data type representing the created "Events Subscription" sub-resource.

If the TSCTSF accepted the HTTP PUT request to modify the events subscription, the PCF shall modify the "Events Subscription" sub-resource and shall send to the NF service consumer:

-	the HTTP "204 No Content" response (as shown in figure 5.3.2.6.2-2, step 2a); or

-	the HTTP "200 OK" response (as shown in figure 5.3.2.6.2-2, step 2b) including in the "EventsSubscReqData" data type the updated representation of the "Events Subscription" sub-resource.

Editor's Note:	Error and redirection responses are FFS.

5.3.2.7	Ntsctsf_QoSandTSCAssistance_Unsubscribe

5.3.2.7.1.	General

The Ntsctsf_QoSandTSCAssistance_Unsubscribe service operation enables NF service consumers to remove subscription to all subscribed events for the existing TSC application session context. Subscription to events shall be removed:

-	by invoking the Ntsctsf_QoSandTSCAssistance_Unsubscribe service operation for the existing application session context, as described in subclause 5.3.2.7.2; or

-	within the application session context modification procedure by invoking the Ntsctsf_QoSandTSCAssistance_Update service operation, as described in subclause 5.3.2.3; or

-	within the TSC application session context termination procedure by invoking the Ntsctsf_QoSandTSCAssistance_Delete service operation, as described in subclause 5.3.2.4.

The following procedure using the Ntsctsf_QoSandTSCAssistance_Unsubscribe service operation is supported:

-	Unsubscription to events.

5.3.2.7.2	Unsubscription to events

This procedure is used to unsubscribe to all subscribed events for the existing TSC AF application session context, as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.3.2.7.2-1 illustrates the unsubscription to events using the HTTP DELETE method.

Figure 5.3.2.7.2-1: Removal of events subscription information using HTTP DELETE

When the NF service consumer decides to unsubscribe to all subscribed events for the existing TSC application session context, the NF service consumer shall invoke the Ntsctsf_QoSandTSCAssistance_Unsubscribe service operation by sending the HTTP DELETE request message to the resource URI representing the "Events Subscription" sub-resource in the TSCTSF, as shown in figure 5.3.2.7.2-1, step 1.

Upon the reception of the HTTP DELETE request message from the NF service consumer, the TSCTSF shall decide whether the received HTTP request message is accepted.

If the HTTP DELETE request message from the NF service consumer is accepted, the TSCTSF shall delete "Events Subscription" sub-resource and shall send to the NF service consumer a HTTP "204 No Content" response message. The TSCTSF may delete the existing subscription to event notifications for the related PDU session from the PCF as described in 3GPP TS 29.514 [20].

Editor's Note:	Error and redirection responses are FFS.

5.3.2.5	Ntsctsf_QoSandTSCAssistance_Notify

5.3.2.5.1		General

The Ntsctsf_QoSandTSCAssistance_Notify service operation enables notification to NF service consumers that the previously subscribed event for the existing TSC application session context occurred or that the TSC application session context is no longer valid.

The following procedures using the Ntsctsf_QoSandTSCAssistance_Notify service operation are supported:

-	Notification about TSC application session context event.

-	Notification about TSC application session context termination.

5.3.2.5.2	Notification about TSC application session context event

This procedure is invoked by the TSCTSF to notify the NF service consumer when a certain, previously subscribed, application session context event occurs, as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.3.2.5.2-1 illustrates the notification about TSC application session context event.

Figure 5.3.2.5.2-1: Notification about application session context event

When the TSCTSF determines that the event for the existing TSC AF application session context, to which the NF service consumer has subscribed to, occurred e.g. upon reception of an event notification for a PDU session from the PCF as described in 3GPP TS 29.514 [20], the TSCTSF shall invoke the Ntsctsf_QoSandTSCAssistance_Notify service operation by sending the HTTP POST request (as shown in figure 5.3.2.5.2-1, step 1) to the NF service consumer using the notification URI received in the subscription creation (or modification), as specified in subclause 5.3.2.6, and appending the "notify" segment path at the end of the URI. The TSCTSF shall provide in the body of the HTTP POST request the "EventsNotification" data type including:

-	the notification correlation Id within the "notifCorreId"; and

-	the list of the reported events in the "events" attribute. For each reported event, additional event information may be included.

The NF service consumer notification of other specific events using the Ntsctsf_QoSandTSCAssistance_Notify request is described in the related clauses.

Upon the reception of the HTTP POST request from the TSCTSF indicating that the PDU session and/or service related event occurred, the NF service consumer shall acknowledge that request by sending an HTTP response message with the corresponding status code.

If the HTTP POST request from the TSCTSF is accepted, the NF service consumer shall acknowledge the receipt of the event notification with a "204 No Content" response to HTTP POST request, as shown in figure 5.3.2.5.2-1, step 2.

5.2.2.5.3	Notification about TSC application session context termination

This procedure is invoked by the TSCTSF to notify the NF service consumer that the TSC application session context is no longer valid, as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

Figure 5.2.2.5.3-1 illustrates the notification about application session context termination.

Figure 5.2.2.5.3-1: Notification about TSC application session context termination

When the TSCTSF determines that the TSF AF application session context is no longer valid, the TSC shall invoke the Ntsctsf_QoSandTSCAssistance_Notify service operation by sending the HTTP POST request (as shown in figure 5.2.2.5.3-1, step 1) using the notification URI received in the "Individual TSC Application Session Context" context creation, as specified in clause 5.3.2.2, and appending the "termination" segment path at the end of the URI, to trigger the NF service consumer to request the TSC application session context termination (see subclause 4.2.4.2). The TSCTSF shall provide in the body of the HTTP POST request the "TerminationInfo" data type including:

-	the Individual TSC Application Session Context resource identifier related to the termination notification in the "resUri" attribute; and

-	the application session context termination cause in the "termCause" attribute of the "TerminationCause" data type.

Upon the reception of the HTTP POST request from the TSCTSF requesting the TSC application session context termination, the NF service consumer shall acknowledge that request by sending an HTTP response message with the corresponding status code.

If the HTTP POST request from the TSCTSF is accepted, the NF service consumer shall acknowledge the receipt of the application session context termination request with a "204 No Content" response to HTTP POST request (as shown in figure 5.2.2.5.3-1, step 2) and shall invoke the Ntsctsf_QoSandTSCAssistance_Delete service operation to the TSCTSF as described in clause 5.2.2.4.

Editor's Note:	Error and redirection responses are FFS.

6	API Definitions

6.1	Ntsctsf_TimeSynchronization Service API

6.1.1	Introduction

The Ntsctsf_TimeSynchronization service shall use the Ntsctsf_TimeSynchronization API.

The API URI of the Ntsctsf_TimeSynchronization API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be " ntsctsf-time-sync".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.3.

6.1.2	Usage of HTTP

6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Ntsctsf_TimeSynchronization API is contained in Annex A.

6.1.2.2	HTTP standard headers

6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

In this Release of the specification, no specific custom headers are defined for the Ntsctsf_TimeSynchronization API.

6.1.3	Resources

6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the Ntsctsf_TimeSynchronization API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Time Synchronization Exposure | /subscriptions |  |  |
|  |  | POST | Create a new subscription to notification of capability of time synchronization service. |
| Individual Time Synchronization Exposure Subscription | /subscriptions/{subscriptionId} | GET | Read a subscription to notification of capability of time synchronization service. |
|  |  | DELETE | Delete a subscription to notification of capability of time synchronization service. |
| Time Synchronization Exposure Configurations | /subscriptions/{subscriptionId}/configurations | POST | Create a new configuration to time synchronization exposure. |
| Individual Time Synchronization Exposure Configuration | /subscriptions/{subscriptionId}/configurations/{configurationId} | GET | Read a configuration to time synchronization exposure. |
|  |  | PUT | Modify all of the properties of an existing configuration to time synchronization exposure. |
|  |  | DELETE | Delete a configuration to time synchronization exposure. |
| ASTI Configurations | /asti-configurations/ | POST | Create a new configuration of the 5G access stratum time distribution. |
|  |  | retrieve (POST) | Request the status of the access stratum time distribution for a list of UEs. |
| Individual ASTI Configuration | /asti-configurations/{astiConfigId} | PATCH | Request to update the 5G access stratum time distribution configuration. |
|  |  | DELETE | Request to delete the 5G access stratum time distribution configuration. |

6.1.3.2	Resource: Time Synchronization Exposure Subscriptions

6.1.3.2.1	Description

This resource allows a NF service consumer to create a new subscription to notification of the capability for time synchronization service.

6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |

6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

The POST method creates a new subscription resource to time synchronization exposure subscription. The NF service consumer shall initiate the HTTP POST request message and the TSCTSF shall respond to the message. The TSCTSF shall construct the URI of the created resource.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureSubsc | M | "1 | Parameters to request a subscription to notification of the capability for time synchronization service. |

Table 6.1.3.2.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureSubsc | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error responses are FFS.

Table 6.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | "1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ntsctsf-time-sync/{apiVersion}/ subscriptions/{subscriptionId} |

6.1.3.2.4	Resource Custom Operations

None.

6.1.3.3	Resource: Individual Time Synchronization Exposure Subscription

6.1.3.3.1	Description

This resource allows a NF service consumer to read, modify or delete an existing subscription to notification of the capability for time synchronization service.

6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| subscriptionId | string | See clause 6.1.1 |

6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureSubsc | M | 1 | 200 OK | An Individual Time Synchronization Exposure Subscription resource is returned successfully. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error and redirect responses are FFS.

6.1.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The subscription was deleted successfully. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.1.3.3.4	Resource Custom Operations

None.

6.1.3.4	Resource: Time Synchronization Exposure Configurations

6.1.3.4.1	Description

This resource allows a NF service consumer to create a new subscription to notification of the capability for time synchronization service.

6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| subscriptionId | string | See clause 6.1.1 |

6.1.3.4.3	Resource Standard Methods

6.1.3.4.3.1	POST

The POST method creates a new configuration resource to activate time synchronization service. The NF service consumer shall initiate the HTTP POST request message and the TSCTSF shall respond to the message. The TSCTSF shall construct the URI of the created resource.

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | Parameters to create a configuration to activate time synchronization service. |

Table 6.1.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | 201 Created | The configuration was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error and redirect responses are FFS.

Table 6.1.3.4.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ntsctsf-time-sync/{apiVersion}/ subscriptions/{subscriptionId}/configuration/{configurationId} |

6.1.3.4.4	Resource Custom Operations

None.

6.1.3.5	Resource: Individual Time Synchronization Exposure Configuration

6.1.3.5.1	Description

This resource allows a NF service consumer to modify/cancel a configuration to modify/deactivate Time Synchronization service with the TSCTSF

6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| subscriptionId | string | See clause 6.1.1 |
| configurationId | string | See clause 6.1.1 |

6.1.3.5.3	Resource Standard Methods

6.1.3.5.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.5.3.1-1.

Table 6.1.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.5.3.1-2 and the response data structures and response codes specified in table 6.1.3.5.3.1-3.

Table 6.1.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.5.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | 200 OK | The configuration information in the request URI are returned. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.1.3.5.3.2	PUT

This method shall support the URI query parameters specified in table 6.1.3.5.3.2-1.

Table 6.1.3.5.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.5.3.2-2 and the response data structures and response codes specified in table 6.1.3.5.3.2-3.

Table 6.1.3.5.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | Modify an existing Time Synchronization Exposure Configuration. |

Table 6.1.3.5.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | 200 OK | The subscription was updated successfully. |
| n/a |  |  | 204 No Content | The subscription was updated successfully. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.1.3.5.3.3	DELETE

This method shall support the URI query parameters specified in table 6.1.3.5.3.3-1.

Table 6.1.3.5.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.5.3.3-2 and the response data structures and response codes specified in table 6.1.3.5.3.3-3.

Table 6.1.3.5.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.35.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The configuration was deleted successfully. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.1.3.5.4	Resource Custom Operations

None.

6.1.3.6	Resource: ASTI Configurations

6.1.3.6.1	Description

This resource allows a NF service consumer to create a new Individual ASTI Configuration resource.

6.1.3.6.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations

This resource shall support the resource URI variables defined in table 6.1.3.6.2-1.

Table 6.1.3.6.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| apiVersion | string | See clause 6.2.1 |

6.1.3.6.3	Resource Standard Methods

6.1.3.6.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.6.3.1-1.

Table 6.1.3.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.6.3.1-2 and the response data structures and response codes specified in table 6.1.3.6.3.1-3.

Table 6.1.3.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | Contains the information for the creation of a new Individual ASTI Configuration resource. |

Table 6.1.3.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | 201 Created | The resource was created successfully and a representation of the created resource is returned. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error responses are FFS.

Table 6.1.3.6.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations/{astiConfigId} |

6.1.3.6.4	Resource Custom Operations

6.1.3.6.4.1	Overview

Table 6.1.3.6.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| retrieve | /asti-configurations/retrieve | retrieve (POST) | Request the status of the 5G access stratum time distribution for a list of UEs. |

6.1.3.6.4.2	Operation: retrieve

6.1.3.6.4.2.1	Description

6.1.3.6.4.2.2	Operation Definition

This custom operation retrieves the status of the access stratum time distribution for a list of UEs.

This operation shall support the request data structures specified in table 6.1.3.6.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.6.4.2.2-2.

Table 6.1.3.6.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| StatusRequestData | M | 1 | Parameters to be sent by the NF service consumer when the status of the 5G access stratum time distribution for a list of UEs is requested. |

Table 6.1.3.6.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| StatusResponseData | M | 1 | 200 OK | Status of the 5G access stratum time distribution for a list of UEs is returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

Editor's Note:	Whether to consider GET method is FFS.

6.1.3.7	Resource: Individual ASTI Configuration

6.1.3.7.1	Description

This resource allows a NF service consumer to read, modify or delete an existing Individual ASTI Configuration resource.

6.1.3.7.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-time-sync/<apiVersion>/asti-configurations/{astiConfigId}

This resource shall support the resource URI variables defined in table 6.1.3.7.2-1.

Table 6.1.3.7.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| apiVersion | string | See clause 6.2.1 |
| astiConfigId | string | See clause 6.2.1 |

6.1.3.7.3	Resource Standard Methods

6.1.3.7.3.2	PUT

This method shall support the URI query parameters specified in table 6.1.3.7.3.2-1.

Table 6.1.3.7.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.7.3.2-2 and the response data structures and response codes specified in table 6.1.3.7.3.2-3.

Table 6.1.3.7.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | Contains the modification(s) to apply to the Individual ASTI Configuration resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | 200 OK | Successful case. The Individual ASTI Configuration resource was modified and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The Individual ASTI Configuration resource was modified. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [4] for the PATCH method shall also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.1.3.7.3.3	DELETE

This method shall support the URI query parameters specified in table 6.1.3.7.3.3-1.

Table 6.1.3.7.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.7.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.7.3.3-3.

Table 6.1.3.7.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.7.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The configuration was terminated successfully. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [4] for the PATCH method shall also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.1.3.7.4	Resource Custom Operations

None

6.1.4	Custom Operations without associated resources

6.1.4.1	Overview

This clause will specify custom operations without any associated resource (i.e. RPC) supported by this API.

Table 6.1.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

6.1.4.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

6.1.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

6.1.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 6.1.4.2.2-1 and 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

6.1.4.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 6.1.4.2.

6.1.5	Notifications

6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Time Synchronization Capability Notification | {subsNotifUri} | POST | Time Synchronization Capability Notification for a list of UEs. |
| Time Synchronization Configuration Notification | {configNotifUri} | POST | Current state of Time Synchronization Service configuration Notification. |

6.1.5.2	Time Synchronization Capability Notification

6.1.5.2.1	Description

The Time Synchronization Capability Notification is used by the NF service producer to report the capability of the time synchronization service for a list of UEs to a NF service consumer that has subscribed to such Notifications.

6.1.5.2.2	Target URI

The Callback URI "{subsNotifUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| subsNotifUri | String formatted as URI with the Callback Uri. The Callback Uri is assigned within the Time Synchronization Capability Notification and described within the TimeSyncExposureSubsc type (see table 6.1.6.2.2-1). |

6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureSubsNotif | M | 1 | Provides the time synchronization capabilities of a list of UEs by the TSCTSF to the NF service consumer. |

Table 6.1.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/redirection responses are FFS.

6.1.5.3	Time Synchronization Configuration Notification

6.1.5.3.1	Description

The Time Synchronization Configuration Notification is used by the NF service producer to report the current state of configuration of the time synchronization service.

6.1.5.3.2	Target URI

The Callback URI "{configNotifUri}" shall be used with the callback URI variables defined in table 6.1.5.3.2-1.

Table 6.1.5.3.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| configNotifUri | String formatted as URI with the Callback Uri. The Callback Uri is assigned within the Time Synchronization Configuration Notification and described within the TimeSyncExposureConfig type (see table 6.1.6.1-2). |

6.1.5.3.3	Standard Methods

6.1.5.3.3.1	POST

This method shall support the request data structures specified in table 6.1.5.3.3.1-1 and the response data structures and response codes specified in table 6.1.5.3.3.1-1.

Table 6.1.5.3.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureConfigNotif | M | 1 | Provides the current state of time synchronization configuration by the TSCTSF to the NF service consumer. |

Table 6.1.5.3.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/redirection responses are FFS.

6.1.6	Data Model

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Ntsctsf_TimeSynchronization service based interface protocol.

Table 6.1.6.1-1: Ntsctsf_TimeSynchronization specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessTimeDistributionData | 6.1.6.1.7 | Contains the parameters for the creation of 5G access stratum time distribution configuration. |  |
| AsTimeDistributionParam | 6.1.6.1.8 | Contains the 5G access stratum time distribution parameters. |  |
| ActiveUe | 6.1.6.2.11 | Contains the UE identifier whose status of the access stratum time distribution is active and the optional requested time synchronization error budget. |  |
| PtpCapabilitiesPerUe | 6.1.6.2.6 | Contains the PTP capabilities supported by a UE. |  |
| TimeSyncExposureSubsc | 6.1.6.2.2 | Contains the parameters for the subscription to notification of capability of time synchronization service |  |
| TimeSyncCapability | 6.1.6.2.5 | Contains the capability of time synchronization service |  |
| TimeSyncExposureSubsNotif | 6.1.6.2.3 | Contains the notification of time synchronization service. |  |
| StatusRequestData | 6.1.6.2.9 | Contains the parameters for retrieval of the status of the access stratum time distribution for a list of UEs. |  |
| StatusResponseData | 6.1.6.2.10 | Contains the parameters for the status of the access stratum time distribution for a list of UEs. |  |
| SubsEventNotification | 6.1.6.2.4 | Contains the notification of capability of time synchronization for a list of UEs. |  |

Table 6.1.6.1-2 specifies data types re-used by the Ntsctsf_TimeSynchronization service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Ntsctsf_TimeSynchronization service based interface.

Table 6.1.6.1-2: Ntsctsf_TimeSynchronization re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [15] | String with format "date-time" as defined in OpenAPI Specification [6]. |  |
| DistributionMethod | 3GPP TS 29.522 [17] | Identifies the time synchronization distribution methods supported by 5GS. |  |
| Dnn | 3GPP TS 29.571 [15] | The DNN the user is connected to. |  |
| DurationSec | 3GPP TS 29.571 [15] | Identifies a period of time in units of seconds. |  |
| EventFilter | 3GPP TS 29.522 [17] | Contains the conditions to match for notifying the event. |  |
| GmCapable | 3GPP TS 29.522 [17] | Indicates separately whether 5GS supports acting as a gPTP or PTP grandmaster. |  |
| GroupId | 3GPP TS 29.571 [15] | Identifies a group of internal globally unique ID. |  |
| Snssai | 3GPP TS 29.571 [15] | Identifies the S-NSSAI. |  |
| SubscribedEvent | 3GPP TS 29.522 [17] | Indicates the subscribed event. |  |
| Supi | 3GPP TS 29.571 [15] | The identification of the user (i.e. IMSI, NAI). |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |
| TimeSyncExposureConfig | 3GPP TS 29.522 [17] | Contains the parameters of time synchronization configuration. |  |
| Uinteger | 3GPP TS 29.571 [15] | Unsigned integer. |  |
| Uint64 | 3GPP TS 29.571 [15] |  |  |
| Uri | 3GPP TS 29.571 [15] | Identifies a referenced resource. |  |

6.1.6.2	Structured data types

This clause will specify the structured data types.

6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.6.2.2	Type: TimeSyncExposureSubsc

Table 6.1.6.2.2-1: Definition of type TimeSyncExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supis | array(Supi) | C | 1..N | Subscription Permanent Identifier. |  |
| interGrpId | GroupId | C | 0.1 | The internal Group Id(s). |  |
| anyUeInd | boolean | C | 0..1 | Identifies whether the AF request applies to any UE (i.e. all UEs). This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". |  |
| notifMethod | NotificationMethod | O | 0..1 | If "notifMethod" is not supplied, the default value "ON_EVENT_DETECTION" applies. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| afServiceId | string | O | 0..1 | Identifies a service on behalf of which the AF is issuing the request. |  |
| subscribedEvents | array(SubscribedEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). |  |
| eventFilters | array(EventFilter) | O | 1..N | Contains the filter conditions to match for notifying the event(s) of time synchronization capabilities for a list of UE(s). |  |
| subsNotifUri | Uri | M | 1 | Notification URI for time sensitive capability reporting. |  |
| subsNotifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| maxReportNbr | Uinteger | O | 0..1 | If omitted, there is no limit. |  |
| expiry | DateTime | C | 0..1 | This attribute indicates the expiry time of the subscription, after which the NEF shall not send any event notifications and the subscription becomes invalid. It may be included in an event subscription request and may be included in an event subscription response based on operator policies. If an expiry time was included in the request, then the expiry time returned in the response should be less than or equal to that value. If the expiry time is not included in the response, the NF service consumer shall not associate an expiry time for the subscription. |  |
| repPeriod | DurationSec | C | 0..1 | Is supplied for notification Method "periodic". |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the features supported by the NF service consumer. This parameter shall be supplied by the NF service consumer in the POST request and the response that requested the creation of an Individual Time Synchronization Subscription resource. |  |

6.1.6.2.3	Type: TimeSyncExposureSubsNotif

Table 6.1.6.2.3-1: Definition of type TimeSyncExposureSubsNotify

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsNotifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifs | array(SubsEventNotification) | M | 1..N | Notifications about Individual Events |  |

6.1.6.2.4	Type SubsEventNotification

Table 6.1.6.2.4-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | SubscribedEvent | M | 1 | Subscribed events |  |
| timeSyncCapas | array(TimeSyncCapability) | O | 1..N | Contains a list of time syncroniziation capabilities for the List of User-Plane Node IDs. |  |

6.1.6.2.5	Type: TimeSyncCapability

Table 6.1.6.2.5-1: Definition of type TimeSyncCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upNodeId | Uint64 | O | 0..1 | Identifies the applicable NW-TT. Contains a TSC user plane node Id. If integrated with TSN, the user plane node Id is a bridge Id defined in IEEE 802.1Q [18] clause 14.2.5. |  |
| gmCapables | array(GmCapable) | O | 1..N | Indicates whether user plane node supports acting as a gPTP and/or PTP grandmaster. |  |
| ptpCapForUes | map(PtpCapabilitiesPerUe) | M | 1..N | Contains the PTP capabilities supported by the list of UE(s). The key of the map is the SUPI. |  |

Editor's Note:	Data types for notification need further check based on stage 2 progress.

6.1.6.2.6	Type: PtpCapabilitiesPerUe

Table 6.1.6.2.6: Definition of type PtpCapabilitiesPerUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | Identifies the UE to which the reported PTP instance below apply. |  |
| ptpCaps | array(EventFilter) | M | 1..N | Contains the reported PTP capabilities for the UE. |  |

6.1.6.2.7	Type: AccessTimeDistributionData

Table 6.1.6.2.7-1: Definition of type AccessTimeDistributionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supis | array(Supi) | C | 1..N | Subscription Permanent Identifier. (NOTE 1) |  |
| interGrpId | GroupId | C | 0.1 | The internal Group Id(s). (NOTE 1) |  |
| anyUeInd | boolean | C | 0..1 | Identifies whether the AF request applies to any UE (i.e. all UEs). This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". (NOTE 1) |  |
| dnn | Dnn | C | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. (NOTE 2) |  |
| snssai | Snssai | C | 0..1 | Identifies an S-NSSAI. (NOTE 2) |  |
| asTimeDisParam | AsTimeDistributionParam | M | 1 | 5G access stratum time distribution parameters |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the features supported by the NF service consumer. This parameter shall be supplied by the NF service consumer in the POST request and the response that requested the creation of an Individual ASTI Configuration resource. |  |
| NOTE 1: One of "supis", "interGrpId" or "anyUeInd" attribute shall be provided. NOTE 2: Any of it shall be presented if "anyUeInd" attribute is presented. |  |  |  |  |  |

6.1.6.2.8	Type: AsTimeDistributionParam

Table 6.1.6.2.8-1: Definition of type AsTimeDistributionParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asTimeDisEnabled | boolean | O | 0..1 | When this attribute is included and set to true, it indicates that the access stratum time distribution via Uu reference point is activated. When present it shall be set as follows: - true: activated. - false (default): deactivated. |  |
| timeSyncErrBudget | Uinteger | O | 0..1 | Indicates the time synchronization error budget in terms of time units of microseconds. |  |
| tempValidity | TemporalValidity | O | 0..1 | Indicates the time interval during which the AF request is to be applied. |  |

6.1.6.2.9	Type: StatusRequestData

Table 6.1.6.2.9-1: Definition of type StatusRequestData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supis | array(Supi) | M | 1 | Subscription Permanent Identifier. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |

6.1.6.2.10	Type: StatusResponseData

Table 6.1.6.2.10-1: Definition of type StatusResponseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| inactiveUes | array(Supi) | O | 1..N | Indicate the UE(s) whose status of the access stratum time distribution is inactive. |  |
| activeUes | array(ActiveUe) | O | 1..N | Contains the UE identifier(s) whose status of the access stratum time distribution is active and the optional requested time synchronization error budget. |  |

6.1.6.2.11	Type: ActiveUe

Table 6.1.6.2.11-1: Definition of type ActiveUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | Indicate the UE whose status of the access stratum time distribution is active. |  |
| timeSyncErrBudget | Uinteger | O | 0..1 | Indicates the time synchronization error budget in terms of time units of microseconds. |  |

6.1.6.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

6.1.6.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

6.1.6.3.4	Enumeration: <EnumType2>

And so on if there are more enumerations to define.

6.1.6.4	Data types describing alternative data types or combinations of data types

6.1.6.4.1	Type: <TypeName 1>

The data types describing alternative data types or combinations of data types shall represent an OpenAPI schema object using the "oneOf", "anyOf" or "allOf"  keyword to list alternative or to be combined data types (see the OpenAPI specification [4] and https://swagger.io/docs/specification/data-models/oneof-anyof-allof-not/).

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches , i.e. a list of mutually exclusive alternative data types., as described using the OpenAPI "oneOf" keyword, if the instance matches against one and only one of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of non-exclusive alternative data types, as described using the OpenAPI "anyOf" keyword, if the instance matches against one or more of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of to be combined data types, as described using the OpenAPI "allOf" keyword, if the instance matches against all of the to be combined data types."Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON elements in a JSON object.

"Data type": Data type of the alternative or to be combined data type. If the data type is indicated as "<type>", the alternative or to be combined data type shall be of data type <type>. If the data type is indicated as "array(<type>)", the alternative or to be combined data type shall be an array (see IETF RFC 8259 [3]) that contains elements of data type <type>. If the data type is indicated as "map(<type>)", the alternative or to be combined data type shall be an object (see IETF RFC 8259 [3]) encoded in the corresponding OpenAPI specification as a map which values are of data type <type>. <type> can either be "integer", "number", "string" or "boolean" (as defined in the OpenAPI specification [4]), or a data type defined in a 3GPP specification.

"Cardinality": Defines the allowed number of occurrence of data type <type>. A cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)" and indicates the number of elements within the array or map; the values M and N can either be the characters "M" and "N", respectively, or integer numbers; with M being greater than or equal 0, and N being greater than 0. For data type "<type>", the cardinality shall be set to "1".

"Description": Describes the meaning and use of the attribute and may contain normative statements.Applicability: If the attribute is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [4], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the attribute can be used with any feature.

Applicability: If the type is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [2], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the type can be used with any feature. If no optional features are defined for an API, the applicability column can be omitted for that API.

Table 6.1.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

6.1.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

6.1.6.5	Binary data

This clause will specify what is encoded in binary part, if multipart media type is agreed to be supported by CT4 and is supported by the API. It shall be omitted if not applicable.

6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| < Binary Data 1 > e.g. N1 SM Message | < clause > e.g. 6.1.6.5.2 | <content type> e.g. vnd.3gpp.5gnas |

6.1.6.5.2	< Binary Data 1 >

And so on if there are more binary data to specify

6.1.7	Error Handling

6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [5].

For the Ntsctsf_TimeSynchronization API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

Editor's Note:	Error/redirection responses are FFS.

In addition, the requirements in the following clauses are applicable for the Ntsctsf_TimeSynchronization API.

6.1.7.2	Protocol Errors

No specific procedures for the Ntsctsf_TimeSynchronization service are specified.

6.1.7.3	Application Errors

The application errors defined for the Ntsctsf_TimeSynchronization  service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

Editor's Note:	Error/redirection responses are FFS.

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Ntsctsf_TimeSynchronization API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Ntsctsf_TimeSynchronization API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Ntsctsf_TimeSynchronization API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Ntsctsf_TimeSynchronization service.

The Ntsctsf_TimeSynchronization API defines a single scope "ntsctsf-time-sync" for the entire service, and it does not define any additional scopes at resource or operation level.

6.2	Ntsctsf_QoSandTSCAssistance Service API

And so on if there are more than two services supported by the NF. Same structure as in clause 6.1.

6.2.1	Introduction

The Ntsctsf_QoSandTSCAssistance service shall use the Ntsctsf_QoSandTSCAssistance API.

The API URI of the Ntsctsf_QoSandTSCAssistance API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "ntsctsf-qos-tscai".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

6.2.2	Usage of HTTP

6.2.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Ntsctsf_QoSandTSCAssistance API is contained in Annex A.

6.2.2.2	HTTP standard headers

6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

JSON object used in the HTTP PATCH request shall be encoded according to "JSON Merge Patch" and shall be signalled by the content type "application/merge-patch+json", as defined in IETF RFC 7396 [22].

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

In this Release of the specification, no specific custom headers are defined for the Ntsctsf_QoSandTSCAssistance API.

6.2.3	Resources

6.2.3.1	Overview

Figure 6.2.3.1-1: Resource URI structure of the Ntsctsf_QoSandTSCAssistance API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| TSC Application Sessions | /tsc-app-sessions | POST | Ntsctsf_QoSandTSCAssistance_Create. Creates a new Individual TSC Application Session Context resource and may create the child Events Subscription sub-resource. |
| Individual TSC Application Session Context | /tsc-app-sessions/{appSessionId} | PATCH | Ntsctsf_QoSandTSCAssistance_Update. Updates an existing Individual TSC Application Session Context resource. It can also update an Events Subscription sub-resource. |
|  |  | GET | Reads an existing Individual TSC Application Session Context resource. |
|  | /tsc-app-sessions/{appSessionId}/delete | delete (POST) | Ntsctsf_QoSandTSCAssistance_Delete. Deletes an existing Individual TSC Application Session Context resource and the child Events Subscription sub-resource. |
| Events Subscription | /tsc-app-sessions/{appSessionId} /events-subscription | PUT | Ntsctsf_QoSandTSCAssistance_Subscribe. Creates a new Events Subscription sub-resource or modifies an existing Events Subscription sub-resource. |
|  |  | DELETE | Ntsctsf_QoSandTSCAssistance_Unsubscribe. Deletes an Events Subscription sub-resource. |

6.2.3.2	Resource: TSC Application Sessions

6.2.3.2.1	Description

This resource allows a NF service consumer to create a new Individual TSC Application Session Context resource and may create the child Events Subscription sub-resource.

6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-qos-tscai/<apiVersion>/tsc-app-sessions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| apiVersion | string | See clause 6.2.1 |

6.2.3.2.3	Resource Standard Methods

6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TscAppSessionContextData | M | 1 | Contains the information for the creation of a new Individual TSC Application Session Context resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TscAppSessionContextData | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error responses are FFS.

Table 6.2.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ntsctsf-qos-tscai/<apiVersion>/tsc-app-sessions /{appSessionId} |

6.2.3.2.4	Resource Custom Operations

None.

6.2.3.3	Resource: Individual TSC Application Sessions

6.2.3.3.1	Description

This resource allows a NF service consumer to read, modify or delete an existing Individual TSC Application Sessions resource.

6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/ntsctsf-qos-tscai/<apiVersion>/tsc-app-sessions/{appSessionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| apiVersion | string | See clause 6.2.1 |
| appSessionId | string | See clause 6.2.1 |

6.2.3.3.3	Resource Standard Methods

6.2.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TscAppSessionContextData | M | 1 | 200 OK | An Individual TSC Application Sessions resource is returned successfully. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error and redirect responses are FFS.

6.2.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TscAppSessionContextUpdateData | M | 1 | Contains the modification(s) to apply to the Individual TSC Application Session Context resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TscAppSessionContextData | M | 1 | 200 OK | Successful case. The Individual TSC Application Session Context resource was modified and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The Individual TSC Application session context resource was modified. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [4] for the PATCH method shall also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.2.3.3.4	Resource Custom Operations

6.2.3.3.4.1	Overview

Table 6.2.3.3.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| delete | /tsc-app-sessions/{appSessionId}/delete | POST | Ntsctsf_QoSandTSCAssistance_Delete. Deletes an existing Individual TSC Application Session Context resource and the child Events Subscription sub-resource. |

6.2.3.3.4.2	Operation: delete

6.2.3.3.4.2.1	Description

6.2.3.3.4.2.2	Operation Definition

This custom operation deletes an existing Individual TSC Application Session Context resource and the child Events Subscription sub-resource in the TSCTSF.

This operation shall support the request data structures specified in table 6.2.3.3.4.2.2-1 and the response data structure and response codes specified in table 6.2.3.3.4.2.2-2.

Table 6.2.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventsSubscReqData | O | 0..1 | Events subscription information to be sent by the NF service consumer to request event notification when the Individual TSC Application Session Context resource is deleted. |

Table 6.2.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual TSC Application session context resource was deleted. |
| EventsNotification | M | 1 | 200 OK | Successful case. Describes information related to the notification of events. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [4] for the POST method shall also apply. |  |  |  |  |

Editor's Note:	Error/Redirect responses are FFS.

6.2.3.4	Resource: Events Subscription (Document)

6.2.3.4.1	Description

The Events Subscription sub-resource represents a subscription to events for a TSC application session context that exists in the Ntsctsf_QoSandTSCAssistance service.

6.2.3.4.2	Resource definition

Resource URI: {apiRoot}/ntsctsf-qos-tscai/<apiVersion>/tsc-app-sessions/{appSessionId}/events-subscription

This resource shall support the resource URI variables defined in table 6.2.3.4.2-1.

Table 6.2.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| appSessionId | string | Identifies an application session context |

6.2.3.4.3	Resource Standard Methods

6.2.3.4.3.1	PUT

This method shall support the URI query parameters specified in table 6.2.3.4.3.1-1.

Table 6.2.3.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.1-2 and the response data structures and response codes specified in table 6.2.3.4.3.1-3.

Table 6.2.3.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventsSubscReqData | M | 1 | Contains information for the modification of the Events Subscription sub-resource. |

Table 6.2.3.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TscAppSessionContextData | M | 1 | 201 Created | Successful case. The Events Subscription sub-resource was created. |
| TscAppSessionContextData | M | 1 | 200 OK | Successful case. The Events Subscription sub-resource was modified. |
| n/a |  |  | 204 No Content | Successful case. The Events Subscription sub-resource was modified. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [4] for the PUT method shall also apply. |  |  |  |  |

Table 6.2.3.4.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ntsctsf-qos-tscai/<apiVersion>/tsc-app-sessions/{appSessionId}/events-subscription |

6.2.3.4.3.2	DELETE

This method shall support the URI query parameters specified in table 6.2.3.4.3.2-1.

Table 6.2.3.4.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.2-2 and the response data structures and response codes specified in table 6.2.3.4.3.2-3.

Table 6.2.3.4.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.4.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual TSC Application session context resource was deleted. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [4] for the DELETE method shall also apply. |  |  |  |  |

6.2.3.4.4	Resource Custom Operations

None.

6.2.4	Custom Operations without associated resources

None.

6.2.5	Notifications

6.2.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notifUri}/notify | notify (POST) | TSCTSF event notification. |
| Termination Request | {notifUri}/terminate | terminate (POST) | Request for termination of an Individual TSC Application Session Context. |

6.2.5.2	Event Notification

6.2.5.2.1	Description

The Event Notification is used by the TSCTSF to report one or several observed application session context events to the NF service consumer that has subscribed to such notifications via the Events Subscription sub-resource.

6.2.5.2.2	Target URI

The Callback URI "{notifUri}/notify " shall be used with the callback URI variables defined in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification Uri as assigned within the Events Subscription sub-resource and described within the EventsSubscReqData type (see table 6.2.6.2.3-1). |

6.2.5.2.3	Standard Methods

6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-1.

Table 6.2.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventsNotification | M | 1 | Provides Information about observed events. |

Table 6.1.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/redirection responses are FFS.

6.2.5.3	Termination Request

6.2.5.3.1	Description

The Termination Request is used by the TSCTSF to request the NF service consumer the deletion of the Individual TSC Application Session Context resource.

6.2.5.3.2	Target URI

The Callback URI "{notifUri}/terminate " shall be used with the callback URI variables defined in table 6.2.5.3.2-1.

Table 6.2.5.3.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification Uri as assigned within the Individual TSC Application Session Context resource and described within the TscAppSessionContextData Data type (see table 6.2.6.2.2-1). |

6.2.5.3.3	Standard Methods

6.2.5.3.3.1	POST

This method shall support the request data structures specified in table 6.2.5.3.3.1-1 and the response data structures and response codes specified in table 6.2.5.3.3.1-1.

Table 6.2.5.3.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TerminationInfo | M | 1 | Provides information about the deletion of the resource. |

Table 6.2.5.3.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error/redirection responses are FFS.

6.2.6	Data Model

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Ntsctsf_QoSandTSCAssistance service based interface protocol.

Table 6.2.6.1-1: Ntsctsf_QoSandTSCAssistance specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EventsNotification | 6.2.6.2.6 | Describes the notification about the events occurred within an Individual TSC Application Session Context resource. |  |
| EventsSubscReqData | 6.2.6.2.3 | Identifies the events the application subscribes to within an Individual TSC Application Session Context resource |  |
| EventsSubscReqDataRm | 6.2.6.2.5 | This data type is defined in the same way as the "EventsSubscReqData" data type, but with the OpenAPI "nullable: true" property. |  |
| TscAppSessionContextData | 6.2.6.2.2 | Represents the Individual TSC Application Session Context resource data. |  |
| TscAppSessionContextUpdateData | 6.2.6.2.4 | Describes the modifications to an Individual TSC Application Session Context resource. |  |
| TscEvent | 6.2.6.3.3 | Indicates the subscribed event(s). |  |

Table 6.2.6.1-2 specifies data types re-used by the Ntsctsf_QoSandTSCAssistance service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Ntsctsf_QoSandTSCAssistance service based interface.

Table 6.2.6.1-2: Ntsctsf_QoSandTSCAssistance re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccumulatedUsage | 3GPP TS 29.122 [21] | Accumulated Usage. | AccumulatedUsage |
| Dnn | 3GPP TS 29.571 [15] | The DNN the user is connected to. |  |
| EthFlowDescription | 3GPP TS 29.514 [20] | Defines a packet filter for an Ethernet flow. |  |
| FlowInfo | 3GPP TS 29.122 [21] | Contains the IP data flow information. |  |
| IpAddr | 3GPP TS 29.571 [15] | Contains the IP address. |  |
| MacAddr48 | 3GPP TS 29.571 [15] | MAC Address. |  |
| QosMonitoringInformation | 3GPP TS 29.122 [21] | Contains Qos Monitoring information. |  |
| QosMonitoringInformationRm | 3GPP TS 29.122 [21] | This data type is defined in the same way as the "QosMonitoringInformation" data type, but with the OpenAPI "nullable: true" property. |  |
| QosNotificationControlInfo | 3GPP TS 29.514 [20] | Indicates whether the QoS targets related to certain media component are not guaranteed or are guaranteed again. |  |
| ResourcesAllocationInfo | 3GPP TS 29.514 [20] | Indicates the status of the PCC rule(s) related to certain service data flow. |  |
| Snssai | 3GPP TS 29.571 [15] | Identifies the S-NSSAI. |  |
| SubscribedEvent | 3GPP TS 29.522 [17] | Indicates the subscribed event. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |
| TerminationInfo | 3GPP TS 29.514 [20] | Includes information related to the termination of the Individual TSC Application Session Context resource. |  |
| TscQosRequirement | 3GPP TS 29.122 [21] | Contains the QoS requirements for time sensitive communication. |  |
| TscQosRequirementRm | 3GPP TS 29.122 [21] | This data type is defined in the same way as the "TscQosRequirement" data type, but with the OpenAPI "nullable: true" property. |  |
| UsageThreshold | 3GPP TS 29.122 [21] | Time period and/or traffic volume in which the QoS is to be applied. |  |
| UsageThresholdRm | 3GPP TS 29.122 [21] | This data type is defined in the same way as the "UsageThreshold" data type, but with the OpenAPI "nullable: true" property. |  |
| Uri | 3GPP TS 29.571 [15] | Identifies a referenced resource. |  |

6.2.6.2	Structured data types

6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.6.2.2	Type TscAppSessionContextData

Table 6.2.6.2.2-1: Definition of type TscAppSessionContextData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueIpAddr | IpAddr | C | 0..1 | The address of the UE. (NOTE) |  |
| ipDomain | string | C | 0..1 | The IPv4 address domain identifier. The attribute may only be provided if the ueIpv4 attribute is present. |  |
| ueMac | MacAddr48 | C | 0..1 | Identifies the MAC address. (NOTE) |  |
| dnn | Dnn | O | 0..1 | Data Network Name, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies the S-NSSAI. |  |
| notifUri | Uri | M | 1 | Notification URI for Individual TSC Application Session Context termination requests. |  |
| appId | string | C | 0..1 | Identifies the Application Identifier. (NOTE) |  |
| flowInfo | array(FlowInfo) | C | 0..1 | Describe the IP data flow which requires QoS. (NOTE) |  |
| ethFlowInfo | array(EthFlowDescription) | C | 0..1 | Identifies Ethernet packet flows. (NOTE) |  |
| afId | string | M | 1 | Identifies the AF identifier. |  |
| tscQosReq | TscQosRequirement | O | 0..1 | Contains the QoS requirements for time sensitive communication. |  |
| qosReference | string | M | 1 | Identifies a pre-defined QoS information |  |
| altQosReferences | array(string) | O | 0..1 | Identifies an ordered list of pre-defined QoS information. The lower the index of the array for a given entry, the higher the priority. |  |
| evSubsc | EventsSubscReqData | O | 0..1 | Identifies the events the application subscribes to at creation of an Individual TSC Application Session Context resource. |  |
| suppFeat | SupportedFeatures | C | 1 | This IE represents a list of Supported features used as described in clause 6.2.8. It shall be supplied by the NF service consumer in the POST request and response of requests a creation of an Individual TSC Application Session Context resource. |  |
| NOTE: Eirther "ueIpAddr" attribute or "ueMac" attribute shall be included. If IP address is provided, IP flow information shall be provided. If ipv4, the domain identifier may be provided. If mac address is provided, Ethernet flow information shall be provided. One of IP flow information, Ethernet flow information or Application Identifier shall be provided. |  |  |  |  |  |

6.2.6.2.3	Type EventsSubscReqData

Table 6.2.6.2.3-1: Definition of type EventsSubscReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(TscEvent) | M | 1..N | Subscribed Events. |  |
| notifUri | Uri | M | 1 | Notification URI for event notification. |  |
| qosMon | QosMonitoringInformation | C | 0..1 | Qos Monitoring information. It can be present when the event "QOS_MONITORING" is subscribed. |  |
| usgThres | UsageThreshold | C | 0..1 | Includes the volume and/or time thresholds for sponsored data connectivity. It can be present when the event "USAGE_REPORT" is subscribed. |  |
| notifCorreId | string | M | 0..1 | It is used to set the value of Notification Correlation ID in the corresponding notification. |  |

6.2.6.2.4	Type TscAppSessionContextUpdateData

Table 6.2.6.2.4-1: Definition of type TscAppSessionContextUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 1 | Notification URI for Individual TSC Application Session Context termination requests. |  |
| appId | string | O | 0..1 | Identifies the external Application Identifier. (NOTE) |  |
| flowInfo | array(FlowInfo) | O | 0..1 | Describe the IP data flow which requires QoS. (NOTE) |  |
| ethFlowInfo | array(EthFlowDescription) | O | 0..1 | Identifies Ethernet packet flows. (NOTE) |  |
| tscQosReq | TscQosRequirementRm | O | 0..1 | Contains the QoS requirements for time sensitive communication. |  |
| qosReference | string | O | 0..1 | Identifies a pre-defined QoS information |  |
| altQosReferences | array(string) | O | 0..1 | Identifies an ordered list of pre-defined QoS information. The lower the index of the array for a given entry, the higher the priority. |  |
| evSubsc | EventsSubscReqDataRm | O | 0..1 | Identifies the events the application subscribes to at modification of an Individual TSC Application Session Context resource. |  |
| NOTE: One of IP flow information, Ethernet flow information or Application Identifier may be provided. |  |  |  |  |  |

6.2.6.2.5	Type EventsSubscReqDataRm

This data type is defined in the same way as the "EventsSubscReqData" data type, but:

-	with the OpenAPI "nullable: true" property; and

-	the removable attribute "usgThres" is defined with the removable data type "UsageThresholdRm"; and removable attribute "qosMon" is defined with the removable data type "QosMonitoringInformationRm".

Table 6.2.6.2.5-1: Definition of type EventsSubscReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(TscEvent) | M | 1..N | Subscribed Events. |  |
| notifUri | Uri | O | 0..1 | Notification URI for event notification. |  |
| qosMon | QosMonitoringInformationRm | O | 0..1 | Qos Monitoring information. It can be present when the event "QOS_MONITORING" is subscribed. |  |
| usgThres | UsageThresholdRm | O | 0..1 | Includes the volume and/or time thresholds for sponsored data connectivity. |  |
| notifCorreId | string | O | 0..1 | It is used to set the value of Notification Correlation ID in the corresponding notification. |  |

6.2.6.2.6	Type EventsNotification

Table 6.2.6.2.6-1: Definition of type EventsNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorreId | string | M | 1 | It is used to set the value of Notification Correlation ID in the corresponding notification. |  |
| events | array(TscEvent) | M | 1..N | Contains the reported event(s). |  |
| failedResourcAllocReports | array(ResourcesAllocationInfo) | C | 1..N | Indicates the status of the PCC rule(s) related to certain failed media components. It shall be included when the event trigger is "FAILED_RESOURCES_ALLOCATION". |  |
| succResourcAllocReports | array(ResourcesAllocationInfo) | C | 1..N | Indicates the alternative service requirement the NG-RAN can guarantee to certain media components. It may be included when the event trigger is "SUCCESSFUL_RESOURCES_ALLOCATION". |  |
| qncReports | array(QosNotificationControlInfo) | C | 1..N | QoS notification control information. It shall be present when the notified event is "QOS_NOTIF". |  |
| qosMonReports | array(QosMonitoringReport) | C | 1..N | QoS Monitoring reporting information. It shall be present when the notified event is "QOS_MONITORING". |  |
| usgRep | AccumulatedUsage | C | 0..1 | Indicates the measured volume and/or time for sponsored data connectivity. It shall be present when the notified event is "USAGE_REPORT". |  |

6.2.6.3	Simple data types and enumerations

6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

6.2.6.3.3	Enumeration: TscEvent

The enumeration TscEvent represents event for TSC. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration TscEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FAILED_RESOURCES_ALLOCATION | Indicates that one or more of the SDFs of an Individual TSC Application Session Context are deactivated. It also indicates that the resources requested for a particular service information cannot be successfully allocated. |  |
| SUCCESSFUL_RESOURCES_ALLOCATION | Indicates that the resources requested for particular service information have been successfully allocated. |  |
| USAGE_REPORT | Indicates the usage report event. |  |
| QOS_GUARANTEED | The QoS targets of one or more SDFs are guaranteed again. |  |
| QOS_NOT_GUARANTEED | The QoS targets of one or more SDFs are not being guaranteed. |  |
| QOS_MONITORING | Indicates a QoS monitoring event. |  |

6.2.7	Error Handling

6.2.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

For the Ntsctsf_QoSandTSCAssistance API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

Editor's Note:	Error/redirection responses are FFS.

In addition, the requirements in the following clauses are applicable for the Ntsctsf_QoSandTSCAssistance API.

6.2.7.2	Protocol Errors

No specific procedures for the Ntsctsf_QoSandTSCAssistance service are specified.

6.2.7.3	Application Errors

The application errors defined for the Ntsctsf_QoSandTSCAssistance service are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

Editor's Note:	Error/redirection responses are FFS.

6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Ntsctsf_QoSandTSCAssistance API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Ntsctsf_QoSandTSCAssistance API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Ntsctsf_QoSandTSCAssistance API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Ntsctsf_QoSandTSCAssistance service.

The Ntsctsf_QoSandTSCAssistance API defines a single scope "ntsctsf-qos-tscai" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI  specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

A.2	Ntsctsf_TimeSynchronization API

openapi: 3.0.0

info:

title: Ntsctsf_TimeSynchronization Service API

version: 1.0.0-alpha.1

description: |

TSCTSF Time Synchronization Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.565 V0.3.0; 5G System; Time Sensitive Communication and Time Synchronization Function Services; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.565/'

#

servers:

- url: '{apiRoot}/ntsctsf-time-sync/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- ntsctsf-timesynchronization

paths:

/subscriptions:

post:

summary: Creates a new subscription to notification of capability of time synchronization service resource

operationId: TimeSynchronizationExposureSubscriptions

tags:

- Time Synchronization Exposure Subscriptions (Collection)

requestBody:

description: Contains the information for the creation the resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TimeSyncExposureSubsc'

responses:

'201':

description: Successful creation of the resource

content:

application/json:

schema:

$ref: '#/components/schemas/TimeSyncExposureSubsc'

headers:

Location:

description: 'Contains the URI of the created individual time synchronization exposure subscription resource, according to the structure: {apiRoot}/ntsctsf-time-sync/{apiVersion}/
subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

subsEventNotification:

'{$request.body#/subsNotifUri':

post:

requestBody:

description: Notification of an event occurrence in the TSCTSF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TimeSyncExposureSubsNotif'

responses:

'204':

description: The receipt of the notification is acknowledged

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: "Reads an existing Individual Time Synchronization Exposure Subscription"

operationId: GetIndividualTimeSynchronizationExposureSubscription

tags:

- Individual Time Synchronization Exposure Subscription (Document)

parameters:

- name: subscriptionId

description: string identifying an Individual Time Synchronization Exposure Subscription

in: path

required: true

schema:

type: string

responses:

'200':

description: A representation of the resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TimeSyncExposureSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualTimeSynchronizationExposureSubscription

summary: Delete an Individual TimeSynchronization Exposure Subscription

tags:

- Individual Time Synchronization Exposure Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: string identifying an Individual Time Synchronization Exposure Subscription

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}/configurations:

post:

summary: "Craete a new Individual Time Synchronization Exposure Configuration"

operationId: CreateIndividualTimeSynchronizationExposureConfiguration

tags:

- Individual Time Synchronization Exposure Configuration (Document)

parameters:

- name: subscriptionId

description: string identifying an Individual Time Synchronization Exposure Subscription

in: path

required: true

schema:

type: string

requestBody:

description: Contains the information for the creation the resource

required: true

content:

application/json:

schema:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/TimeSyncExposureConfig'

responses:

'201':

description: Successful creation of the resource

content:

application/json:

schema:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/TimeSyncExposureConfig'

headers:

Location:

description: 'Contains the URI of the created individual time synchronization exposure configuration resource, according to the structure: {apiRoot}/ntsctsf-time-sync/{apiVersion}/
subscriptions/{subscriptionId}/configurations/{configurationId}'

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

configEventNotification:

'{$request.body#/configNotifUri':

post:

requestBody:

description: Notification of an event occurrence in the TSCTSF.

required: true

content:

application/json:

schema:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/TimeSyncExposureConfigNotif'

responses:

'204':

description: The receipt of the notification is acknowledged

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}/configurations/{configurationId}:

get:

summary: "Reads an existing Individual Time Synchronization Exposure Configuration"

operationId: GetIndividualTimeSynchronizationExposureConfiguration

tags:

- Individual Time Synchronization Exposure Configuration (Document)

parameters:

- name: subscriptionId

description: string identifying an Individual Time Synchronization Exposure Subscription

in: path

required: true

schema:

type: string

- name: configurationId

description: string identifying an Individual Time Synchronization Exposure Configuration

in: path

required: true

schema:

type: string

responses:

'200':

description: A representation of the resource is returned.

content:

application/json:

schema:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/TimeSyncExposureConfig'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

operationId: ReplaceIndividualTimeSynchronizationExposureConfiguration

summary: Replace an individual Time Synchronization Exposure Configuration

tags:

- IndividualTimeSynchronizationExposureConfiguration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/TimeSyncExposureConfig'

parameters:

- name: subscriptionId

description: string identifying an Individual Time Synchronization Exposure Subscription

in: path

required: true

schema:

type: string

- name: configurationId

description: string identifying an Individual Time Synchronization Exposure Configuration

in: path

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was successfully modified and representation is returned

content:

application/json:

schema:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/TimeSyncExposureConfig'

'204':

description: No Content. Resource was successfully modified

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualTimeSynchronizationExposureConfiguration

summary: Delete an Individual TimeSynchronization Exposure Configuration

tags:

- Individual Time Synchronization Exposure Configuration (Document)

parameters:

- name: subscriptionId

in: path

description: string identifying an Individual Time Synchronization Exposure Subscription

required: true

schema:

type: string

- name: configurationId

description: string identifying an Individual Time Synchronization Exposure Configuration

in: path

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

ntsctsf-timesynchronization: Access to the Ntsctsf_TimeSynchronization API

schemas:

TimeSyncExposureSubsc:

description: Contains the parameters for the subscription to notification of capability of time synchronization service.

type: object

properties:

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

interGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

anyUeInd:

type: boolean

description: Identifies whether the request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

notifMethod:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

subscribedEvents:

type: array

items:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/SubscribedEvent'

minItems: 1

eventFilters:

type: array

items:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/EventFilter'

minItems: 1

subsNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

subsNotifId:

type: string

description: Notification Correlation ID assigned by the NF service consumer.

maxReportNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- subsNotifUri

- subsNotifId

TimeSyncExposureSubsNotif:

description: Contains the notification of time synchronization service.

type: object

properties:

subsNotifId:

type: string

description: Notification Correlation ID assigned by the NF service consumer.

eventNotifs:

type: array

items:

$ref: '#/components/schemas/SubsEventNotification'

minItems: 1

SubsEventNotification:

description: Contains the notification of capability of time synchronization for a list of UEs.

type: object

properties:

event:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/SubscribedEvent'

timeSyncCapas:

type: array

items:

$ref: '#/components/schemas/TimeSyncCapability'

minItems: 1

TimeSyncCapability:

description: Contains the capability of time synchronization service

type: object

properties:

upNodeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

gmCapables:

type: array

items:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/GmCapable'

minItems: 1

ptpCapForUes:

type: object

additionalProperties:

$ref: '#/components/schemas/PtpCapabilitiesPerUe'

minProperties: 1

description: Contains the PTP capabilities supported by each of the UE(s). The key of the map is the supi.

required:

- upNodeId

- ptpCapForUes

PtpCapabilitiesPerUe:

description: Contains the supported PTP capabilities per UE.

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

ptpCaps:

type: array

items:

$ref: 'TS29522_TimeSyncExposure.yaml#/components/schemas/EventFilter'

minItems: 1

required:

- supi

- ptpCaps

A.3	Ntsctsf_QoSandTSCAssistance API

openapi: 3.0.0

info:

title: Ntsctsf_QoSandTSCAssistance Service API

version: 1.0.0-alpha.1

description: |

TSCTSF QoS and TSC Assistance Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.565 V0.3.0; 5G System; Time Sensitive Communication and Time Synchronization function Services; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.565/'

#

servers:

- url: '{apiRoot}/ntsctsf-qos-tscai/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- ntsctsf-qos-tscai

paths:

/tsc-app-sessions:

post:

summary: Creates a new Individual TSC Application Session Context resource

operationId: PostTSCAppSessions

tags:

- TSC Application Sessions (Collection)

requestBody:

description: Contains the information for the creation the resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TscAppSessionContextData'

responses:

'201':

description: Successful creation of the resource

content:

application/json:

schema:

$ref: '#/components/schemas/TscAppSessionContextData'

headers:

Location:

description: 'Contains the URI of the created individual TSC application session context resource, according to the structure: {apiRoot}/ntsctsf-qos-tscai/v1/tsc-app-sessions/{appSessionId} or the URI of the created events subscription sub-resource, according to the structure: {apiRoot}/ntsctsf-qos-tscai/v1/tsc-app-sessions/{appSessionId}/events-subscription}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

terminationRequest:

'{$request.body#/notifUri}/terminate':

post:

requestBody:

description: Request of the termination of the Individual TSC Application Session Context

required: true

content:

application/json:

schema:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TerminationInfo'

responses:

'204':

description: The receipt of the notification is acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

eventNotification:

'{$request.body#/evSubsc/notifUri}/notify':

post:

requestBody:

description: Notification of an event occurrence in the TSCTSF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventsNotification'

responses:

'204':

description: The receipt of the notification is acknowledged

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

#

/tsc-app-sessions/{appSessionId}:

get:

summary: "Reads an existing Individual TSC Application Session Context"

operationId: GetTSCAppSession

tags:

- Individual TSC Application Session Context (Document)

parameters:

- name: appSessionId

description: string identifying the resource

in: path

required: true

schema:

type: string

responses:

'200':

description: A representation of the resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TscAppSessionContextData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: "Modifies an existing Individual TSC Application Session Context"

operationId: ModAppSession

tags:

- Individual TSC Application Session Context (Document)

parameters:

- name: appSessionId

description: string identifying the resource

in: path

required: true

schema:

type: string

requestBody:

description: modification of the resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TscAppSessionContextUpdateData'

responses:

'200':

description: successful modification of the resource and a representation of that resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/TscAppSessionContextData'

'204':

description: The successful modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

eventNotification:

'{$request.body#/evSubsc/notifUri}/notify':

post:

requestBody:

description: Notification of an event occurrence in the TSCTSF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventsNotification'

responses:

'204':

description: The receipt of the notification is acknowledged

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

#

#

/tsc-app-sessions/{appSessionId}/delete:

post:

summary: "Deletes an existing Individual TSC Application Session Context"

operationId: DeleteTSCAppSession

tags:

- Individual TSC Application Session Context (Document)

parameters:

- name: appSessionId

description: string identifying the Individual TSC Application Session Context resource

in: path

required: true

schema:

type: string

requestBody:

description: deletion of the Individual TSC Application Session Context resource, request notification

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/EventsSubscReqData'

responses:

'200':

description: The deletion of the resource is confirmed and a resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/EventsNotification'

'204':

description: The deletion is confirmed without returning additional data.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

#

/tsc-app-sessions/{appSessionId}/events-subscription:

put:

summary: "creates or modifies an Events Subscription subresource"

operationId: putEventsSubsc

tags:

- Events Subscription (Document)

parameters:

- name: appSessionId

description: string identifying the Events Subscription resource

in: path

required: true

schema:

type: string

requestBody:

description: Creation or modification of an Events Subscription resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventsSubscReqData'

responses:

'201':

description: The creation of the Events Subscription resource is confirmed and its representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TscAppSessionContextData'

headers:

Location:

description: 'Contains the URI of the created Events Subscription resource, according to the structure: {apiRoot}/ntsctsf-qos-tscai/v1/tsc-app-sessions/{appSessionId}/events-subscription}'

required: true

schema:

type: string

'200':

description: The modification of the Events Subscription resource is confirmed its representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TscAppSessionContextData'

'204':

description: The modification of the Events Subscription subresource is confirmed without returning additional data.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

eventNotification:

'{$request.body#/notifUri}/notify':

post:

requestBody:

description: Contains the information for the notification of an event occurrence in the TSCTSF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventsNotification'

responses:

'204':

description: The receipt of the notification is acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: deletes the Events Subscription subresource

operationId: DeleteEventsSubsc

tags:

- Events Subscription (Document)

parameters:

- name: appSessionId

description: string identifying the Individual TSC Application Session Context resource

in: path

required: true

schema:

type: string

responses:

'204':

description: The deletion of the of the Events Subscription sub-resource is confirmed without returning additional data.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

ntsctsf-qos-tscai: Access to the Ntsctsf_QoSandTSCAssistance API

schemas:

TscAppSessionContextData:

description: Represents an Individual TSC Application Session Context resource.

type: object

required:

- notifUri

- afId

- qosReference

oneOf:

- required: [ueIpAddr]

- required: [ueMac]

properties:

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

ipDomain:

type: string

description: The IPv4 address domain identifier.

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

appId:

type: string

description: Identifies the Application Identifier.

ethFlowInfo:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

flowInfo:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

afId:

type: string

description: Identifies the AF identifier.

tscQosReq:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/TscQosRequirement'

qosReference:

type: string

description: Identifies a pre-defined QoS information.

altQosReferences:

type: array

items:

type: string

minItems: 1

description: Identifies an ordered list of pre-defined QoS information.

evSubsc:

$ref: '#/components/schemas/EventsSubscReqData'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

TscAppSessionContextUpdateData:

description: Describes the authorization data of an Individual TSC Application Session Context created by the PCF.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

appId:

type: string

description: Identifies the Application Identifier.

ethFlowInfo:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

flowInfo:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

tscQosReq:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/TscQosRequirement'

qosReference:

type: string

description: Identifies a pre-defined QoS information.

altQosReferences:

type: array

items:

type: string

minItems: 1

description: Identifies an ordered list of pre-defined QoS information.

evSubsc:

$ref: '#/components/schemas/EventsSubscReqData'

EventsSubscReqData:

description: Identifies the events the application subscribes to.

type: object

required:

- events

- Uri

- notifCorreId

properties:

events:

type: array

items:

$ref: '#/components/schemas/TscEvent'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

qosMon:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/QosMonitoringInformation'

minItems: 1

usgThres:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

notifCorreId:

type: string

EventsSubscReqDataRm:

description: this data type is defined in the same way as the EventsSubscReqData data type, but with the OpenAPI nullable property set to true.

type: object

required:

- events

properties:

events:

type: array

items:

$ref: '#/components/schemas/TscEvent'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

qosMon:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/QosMonitoringInformationRm'

minItems: 1

usgThres:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThresholdRm'

notifCorreId:

type: string

nullable: true

EventsNotification:

description: describes the notification of a matched event

type: object

required:

- notifCorreId

- events

properties:

events:

type: array

items:

$ref: '#/components/schemas/TscEvent'

minItems: 1

failedResourcAllocReports:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/ResourcesAllocationInfo'

minItems: 1

succResourcAllocReports:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/ResourcesAllocationInfo'

minItems: 1

qncReports:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/QosNotificationControlInfo'

minItems: 1

qosMonReports:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/QosMonitoringReport'

minItems: 1

usgRep:

$ref: 'TS29122_CommonData.yaml#/components/schemas/AccumulatedUsage'

#

# ENUMERATIONS DATA TYPES

#

TscEvent:

description: Represents an event to notify to the AF.

anyOf:

- type: string

enum:

- FAILED_RESOURCES_ALLOCATION

- QOS_MONITORING

- QOS_GUARANTEED

- QOS_NOT_GUARANTEED

- SUCCESSFUL_RESOURCES_ALLOCATION

- USAGE_REPORT

- type: string

#

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-08 |  |  |  |  |  | TS skeleton | 0.0.0 |
| 2021-08 | CT3#117e | C3-214576 |  |  |  | Inclusion of documents agreed in CT3#117e: C3-214145, C3-214149, C3-214154, C3-214466, C3-214467, C3-214468, C3-214469, C3-214505, C3-214506, C3-214507, C3-214508, C3-214509, C3-214510 | 0.1.0 |
| 2012-10 | CT3#118e | C3-215473 |  |  |  | Inclusion of documents agreed in CT3#118e: C3-215347, C3-215348, C3-215349, C3-215350, C3-215351, C3-215352, C3-215353, C3-215354, C3-215356, C3-215357, C3-215358, C3-215470 | 0.2.0 |
| 2012-11 | CT3#119e | C3-216517 |  |  |  | Inclusion of documents agreed in CT3#119e: C3-216114, C3-216115, C3-216116, C3-216121, C3-216397, C3-216398, C3-216399, C3-216400, C3-216401, C3-216402, C3-216426, C3-216594, C3-215357, C3-216595 | 0.3.0 |