3GPP TS 29.155 V17.0.0 (2021-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Traffic steering control;

Representational state transfer (REST) over St reference point

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

HTTP,TSSF,PCRF,JASON

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	St reference point	7

4.1	Overview	7

4.2	Reference model	7

4.3	Functional elements	8

4.3.1	TSSF	8

4.3.2	PCRF	8

4.4	Procedures over St reference point	8

4.4.1	General	8

4.4.2	Traffic Steering Policy Provisioning over St	8

4.4.3	Traffic Steering Rule Error Handling	9

4.4.4	UE IPv4 address provisioning	9

5	St protocol	10

5.1	Introduction	10

5.2	Transport layer	10

5.3	Application delivery layer	10

5.3.1	Methods	11

5.3.2	Resources and URI design	11

5.3.3	HTTP request/response formats	12

5.3.3.1	General	12

5.3.3.2	POST /stapplication/sessions	13

5.3.3.3	PUT /stapplication/sessions/{stsessionid}	15

5.3.3.4	PATCH /stapplication/sessions/{stsessionid}	16

5.3.3.5	DELETE /stapplication/sessions/{stsessionid}	17

5.3.3.6	GET /stapplication/sessions/{stsessionid}	18

5.3.3.7	POST /{notificationbaseurl}/{stsessionid}	19

5.3.4	St session ID	20

5.3.5	HTTP status codes	21

5.3.6	Feature negotiation	22

5.3.6.1	General	22

5.3.7	HTTP custom headers	22

5.3.7.1	3gpp-Optional-Features	22

5.3.7.2	3gpp-Required-Features	22

5.3.7.3	3gpp-Accepted-Features	23

5.3.7.4	3gpp-Notification-Base-URL	23

5.4	Specific application communication	23

5.4.1	General	23

5.4.2	Content type	23

5.4.3	JSON session fields	23

5.4.3.1	General	23

5.4.3.2	session-id	24

5.4.3.3	ue-ipv4	24

5.4.3.4	ue-ipv6-prefix	24

5.4.3.5	tsrules	24

5.4.3.6	ts-rule-name	25

5.4.3.7	precedence	25

5.4.3.8	tdf-application-identifier	25

5.4.3.9	flow-information	25

5.4.3.10	flow-description	26

5.4.3.11	tos-traffic-class	26

5.4.3.12	security-parameter-index	26

5.4.3.13	flow-label	26

5.4.3.14	flow-direction	26

5.4.3.15	ts-policy-identifier-dl	26

5.4.3.16	ts-policy-identifier-ul	26

5.4.3.17	predefined-tsrules	26

5.4.3.18	predefined-group-of-tsrules	27

5.4.3.19	ts-rule-base-name	27

5.4.3.20	called-station-id	27

5.4.4	JSON errors and informational response fields	27

5.4.4.1	General	27

5.4.4.2	errors	28

5.4.4.3	error-type	28

5.4.4.4	error-message	28

5.4.4.5	error-tag	28

5.4.4.6	error-path	28

5.4.4.7	error-info	28

5.4.4.6	success-message	29

5.4.4.7	success-path	29

5.4.4.8	success-info	29

The success-info field is defined as an object type. It may include any additional information under the root level of the JSON content for an informational response.	29

5.4.5	JSON report fields	29

5.4.5.1	General	29

5.4.5.2	ts-rule-reports	29

5.4.5.3	resource-paths	30

5.4.5.4	rule-status	30

5.4.5.5	rule-failure-code	30

5.4.6	JSON notification fields	31

5.4.6.1	General	31

5.4.6.2	notifications	31

]5.4.6.3	notification-type	31

5.4.6.4	notification-message	32

5.4.6.5	notification-tag	32

5.4.6.6	notification-info	32

5.5	TSSF discovery	32

6	Secure communication	32

Annex A (informative):	 Call Flows	32

A.1	General	32

A.2	Traffic Steering Control Information Provisioning	32

A.3	Traffic Steering Control Information Update	33

A.4	Traffic Steering Control Information Removal	34

A.5	Traffic Steering Control Notification	34

Annex B (normative):	JSON Schema	35

B.1	Session body schema	35

B.2	Error and Informational response schema	37

B.3	Report schema	39

B.4	Notification schema	40

Annex C (informative):	 Change history	42

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes the Representational State Transfer (REST) protocol-based St reference point, which is used to provision the traffic steering control information to the TSSF from the PCRF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.203: "Policy and charging control architecture".

[3]	IETF RFC 793: "Transmission Control Protocol".

[4]	Void.

[5]	3GPP TS 29.201: "Representational State Transfer (REST) reference point between Application Function (AF) and Protocol Converter (PC)".

[6]	IETF RFC 1786: "Uniform Resource Locators (URL)".

[7]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[8]	IETF RFC 1983: "Internet Users' Glossary".

[9]	Void.

[10]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	Void.

[12]	IETF RFC 5789: "PATCH Method for HTTP".

[13]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[14]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[15]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[16]	IETF draft-newton-json-content-rules-09: "A Language for Rules Describing JSON Content".

NOTE:	This individual draft will not further progress in IETF. It is available from the following link: https://www.ietf.org/archive/id/draft-newton-json-content-rules-09.txt.

[17]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[18]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[19]	IETF RFC 2818: "HTTP Over TLS".

[20]	3GPP TS 23.003: "Numbering, addressing and identification".

[21]	IETF RFC 6733: "Diameter Base Protocol".

[22]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[23]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[24]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[25]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[26]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[27]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

(S)Gi-LAN: The network infrastructure connected to the 3GPP network over the SGi or Gi reference point that provides various IP-based services.

(S)Gi-LAN service function: A function located in the (S)Gi-LAN that provides value-added IP-based services e.g. NAT, anti-malware, parental control, DDoS protection.

JSON Content Rules: JSON Content Rules (JCR), as defined in IETF draft-newton-json-content-rules [16] is a language specifying the interchange of data in JSON as defined in RFC 7159 [10].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

JCR	JSON Content Rules

JSON	JavaScript Object Notation

PCRF	Policy and Charging Rules Function

TSSF	Traffic Steering Support Function

# 4	St reference point

## 4.1	Overview

The St reference point resides between the PCRF and TSSF. St reference point is used to provision the traffic steering control information from the PCRF to the TSSF.

## 4.2	Reference model

The St reference point resides between the PCRF and TSSFas depicted in figure 4.2.1. The overall PCC architecture is depicted in subclause 3a of 3GPP TS 29.213 [18].

Figure 4.2.1: St reference model

## 4.3	Functional elements

### 4.3.1	TSSF

The TSSF is a function that receives traffic steering control information from the PCRF and ensures that the related traffic steering policy is enforced in the (S)Gi-LAN.

A traffic steering policy is locally configured in TSSF and can be used for uplink, downlink or for both directions. To ensure that the traffic steering policy is enforced, the TSSF performs deployment specific actions as configured for that traffic steering policy.

### 4.3.2	PCRF

The PCRF functionality defined in 3GPP TS 23.203 [2] is applicable. In addition, the PCRF shall be able to make a decision of traffic steering policies used to control the steering of the subscriber’s traffic to appropriate (S)Gi-LAN service functions.

NOTE: In order to allow the PCRF to select and provision an application based traffic steering policy, the reporting of detected applications to the PCRF or any other information can be used.

## 4.4	Procedures over St reference point

### 4.4.1	General

The procedures which can be operated at the St interface are described in the following subclauses.

### 4.4.2	Traffic Steering Policy Provisioning over St

When the PCRF determines the traffic steering control information needed for the IP-CAN session; the PCRF shall send an HTTP POST message to the TSSF to create a new session resource. The PCRF shall provide the representation of the session resource within the body of the HTTP POST. Within the body of the HTTP POST, the PCRF shall provide the St Session ID, the PDN information if required by the operator policies, the UE IPv4 address and/or UE IPv6 prefix and one or more sets of traffic steering control information to the TSSF.

The PCRF may modify or remove traffic steering control information at any time (e.g. due to subscription change or network status change) by sending either an HTTP PUT or HTTP PATCH message to the TSSF including the St session ID within the request URL. When using an HTTP PUT to modify the session resource, the PCRF shall provide the entire state of the session resource within the body of the HTTP PUT. In this case, the TSSF shall replace the existing session information associated with this resource with the information provided in the body of the HTTP PUT. When using an HTTP PATCH to modify the session resource, the PCRF shall provide the modifications within the body of the HTTP PATCH as defined in subclause 5.3.3.3.  In this case, the TSSF shall update the existing session resource based on the content of the body of the HTTP PATCH.

In order to remove all of the traffic steering control information associated with an IP-CAN session when the IP-CAN session is terminated, the PCRF shall send to the TSSF an HTTP DELETE message including the St Session ID within the request URL.

Once the PCRF has requested the creation of a session resource, the PCRF may request the state of the session at any time by sending an HTTP GET request to the TSSF including the Session ID within the request URL. Upon receipt of an HTTP GET from the PCRF, the TSSF shall provide the session representation within the body of the response. Based on the received information, the PCRF may decide whether re-installation, modification, removal of the traffic steering control information or any other action applies.

When a combination of PCEF/TDF with traffic steering control feature and TSSF is deployed, the TSSF shall behave as specified in subclause 6.1.17 of 3GPP TS 23.203 [2]. In this case, the PCRF shall provide the traffic detection information as part of the service data flow information included in the tos-traffic-class within the flow-information field  or within the tdf-application-identifier field. If traffic detection is performed using the flow-information field, the PCRF shall ensure that the relevant value of the tos-traffic-class field included within the flow-information field used for traffic detection is the same as the value provided in the traffic steering policy identifier(s) provided over Gx or Sd reference point. If traffic detection is performed using the tdf-application-identifier field, the PCRF shall ensure that the identifier included within the tdf-application-identifier refers to the configured value(s) in the TSSF that correspond to the value provided in the traffic steering policy identifier(s) over Gx or Sd reference point. See 3GPP TS 29.212 [17].

NOTE 1:	The PCRF and TSSF are configured with the traffic detection mechanism to be applied when the tdf-application-identifier field is used for traffic detection.

NOTE 2:	The tdf-application-identifier can refer to the configured traffic detection information for uplink and/or downlink traffic.

### 4.4.3	Traffic Steering Rule Error Handling

If the installation/activation of one or more traffic steering rules fails, the TSSF shall inform the PCRF of the failed traffic steering rules in the corresponding JSON body of the HTTP response, by including a ts-rule-reports field, and by setting the error-tag to TS_RULE_EVENT. The ts-rule-reports field is an array of rule failure reports. Within each rule failure report, the TSSF shall identify the failed traffic steering rule(s) by including the resource-paths field containing a list of JSON pointer(s) to the relevant traffic steering rule resource(s), shall specify the failed reason code by including a rule-failure-code field and a rule-status as described below:

-	If the installation/activation of one or more new traffic steering rules (i.e., rules which were not previously successfully installed) fails, the TSSF shall set the rule-status field to INACTIVE.

-	If the modification of a currently active traffic steering rule fails, the TSSF shall retain the existing traffic steering rule as active without modification unless the reason for the failure also has an impact on the existing traffic steering rule.

Depending on the value of the rule-failure-code field, the PCRF may decide whether retaining of the old traffic steering rule, re-installation, modification, removal of the traffic steering rule or any other action applies.

If a traffic steering rule was successfully installed/activated, but can no longer be enforced by the TSSF, and if the notification supported feature is supported by the TSSF and PCRF, the TSSF shall notify the PCRF of the rule enforcement failure by sending an  HTTP POST request to the notification URL provided by the PCRF as defined in subclause 5.3.3.2. The JSON body of this HTTP POST shall include a notification-info field containing a ts-rule-reports field and a notification-tag field set to a value of TS_RULE_EVENT filled. Each item of the ts-rule-reports shall include the corresponding rule-failure-code field and rule-status field to INACTIVE.

Note that the PCRF may poll the session state from the TSSF at any time via the HTTP GET method as described in subclause 4.4.2; and take further decisions based on the status of the traffic steering rules reported in the session state.

### 4.4.4	UE IPv4 address provisioning

When the PCRF is notified by the PCEF that either a UE_IP_ADDRESS_ALLOCATE or a UE_IP_ADDRESS_RELEASE event of the IP-CAN session occured in the PCEF, the PCRF shall send an HTTP PATCH or PUT meesage to the TSSF including the St session ID within the request URL.

If the PCRF uses HTTP PUT to update the St session, the PCRF shall provide the entire state of the session resource including the new IPv4 in case a new IPv4 was allocated or omitting the IPv4 in case the IPv4 was released. If the PCRF uses the HTTP PATCH method to update the St session, and if the IPv4 was allocated, the PCRF shall use the “add” operation within the body of the HTTP PATCH to include the new IPv4 as specified in subclause 5.3.3.4; otherwise if the IPv4 was released, the PCRF shall use the “remove” operation within the body of the HTTP PATCH to remove the IPv4 as specified in subclause 5.3.3.4. If the PCRF provides the new UE Ipv4 address to the TSSF, the TSSF shall additionally apply the traffic steering rules to the user plane traffic with the IP address matching the new UE Ipv4 addres. If the PCRF notifies to the TSSF that the UE Ipv4 address has been released, the TSSF shall stop applying the traffic steering rule to the user plane traffic with the IP address matching the released UE Ipv4 address.

# 5	St protocol

## 5.1	Introduction

The following layers of the protocol stack for the St reference point between PCRF and TSSF are described in subclauses:

-	TCP as defined in IETF RFC 793 [3] provides the communication service at the transport layer.

-	An optional communication security layer can be added between the transport and the application delivery layer (see clause 6).

-	The application delivery layer provides the transport of the specific application communication data using HTTP/1.1 as defined in IETF RFC 7230 [22], IETF RFC 7231 [23], IETF RFC 7232 [24], IETF RFC 7233 [25], IETF RFC 7234 [26] and IETF RFC 7235 [27].

-	The specific application communication layer constitutes the transport of the JSON content type.

Figure 5.1.1 illustrates the protocol stack of the RESTful St reference point.

Figure 5.1.1: Protocol stack of the RESTful St reference point

## 5.2	Transport layer

HTTP is layered over TCP, which provides a reliable transport.

For traffic steering policy provisioning and querying, the PCRF acts as an HTTP client and the TSSF acts as an HTTP server. As a result, the PCRF shall initiate a TCP connection with the TSSF.

If the notification feature is supported by TSSF and PCRF, then the TSSF and PCRF shall also respectively act as HTTP client and server. In this case, the TSSF shall initiate a TCP connection with the PCRF.

## 5.3	Application delivery layer

The application delivery layer shall use RESTful HTTP.

The application delivery layer provides the following services:

-	session establishment including the creation of the session resource.

-	session modification and termination procedures.

-	session notification procedures.

In order to establish an St session, the PCRF shall send an HTTP POST message to the TSSF including all session information required. The PCRF shall assign a globally and eternally unique St Session ID and shall include it as part of the session resource representation within the body of the HTTP POST. The St Session ID is defined in subclause 5.3.4. If the session resource is successfully created at the TSSF, the TSSF shall provide the session resource URI within the Location header of the HTTP response.

In order to modify an St session, the PCRF shall send an HTTP PUT  (full replacement of the St session resource) or an HTTP PATCH message, which includes the St session ID as a path element of the URI.

In order to query the state of an St session, the PCRF shall send an HTTP GET message including the St Session ID as a path element of the request URI.

In order to delete an St session, the PCRF shall send an HTTP DELETE message including the St session ID as a path element of the request URI.

In order to report traffic steering rule enforcement failures for an St session, if the notification feature is supported by both the PCRF and TSSF, the TSSF shall send an HTTP POST message including the St session ID as a path element of the notification URI.

Every HTTP message contains the specific communication information required for this case in its body.

### 5.3.1	Methods

Methods indicate to the server what action has to be performed. Every HTTP request message has a method. The following HTTP methods as defined in IETF RFC 7231 [23] and IETF RFC 5789 [12] can be used:

-	POST:

-	Used to create a resource state. The request URI defines the address responsible for the creation of the resource.

-	Used to report traffic steering rule enforcement failures from the TSSF to the PCRF. The notification report is provided in the body of the message. The request URI defines the address of the impacted resource.

-	PUT: Used to replace a resource state. The full state of the resource is provided in the body of the message. The request URI defines the resource which will be replaced.

-	PATCH: The PATCH method applies partial modifications to a resource. The request URI defines the resource which will be modified.

-	GET: Used to retrieve a resource state. The request URI defines the resource which is queried. The server returns the resource state representation within the body of the response.

-	DELETE: Used to delete a resource state. The request URI defines the resource which will be deleted.

Every HTTP request results in a response message that comes back with a status code and further information in its body, if required. The PCRF waits for this response before initiating a further request for the same resource.

### 5.3.2	Resources and URI design

The St session is a RESTful resource that shall be identified by a URI.

The URI design shall be based on the structure defined in IETF RFC 3986 [13]:

scheme ":" hier-part [ "?" query ] [ "#" fragment ]
hier-part   = "//" authority path-abempty
/ path-absolute
/ path-rootless
/ path-empty

The scheme may be HTTP or HTTPS for the REST St interface. Within a scheme the definition of names shall follow the rules of HTTP URIs. Host and port are the main parts of the authority. The path element identifies the resources.

For the REST St interface, the following required parts of the URI shall be used as follows:

-	scheme: The application delivery layer protocol "http" or "https".

-	authority: It includes the server address and optionally a port as follows: host [":" port]

-	path-absolute: The path-absolute should have the following ABNF: "/" mainapp "/" mainresource ["/" resourcepath], where for this release, "mainapp” is "stapplication", "mainresource" is "sessions" and "resourcepath contains the path to identify a session resource, which in this release is the St Session ID as defined in subclause 5.3.4.

NOTE: A different path can be used when the Resource URI is preconfigured in the PCRF.

An example of the URI to identify the "sessions" main resource is: http://tssfserver.example.com/stapplication/sessions.

### 5.3.3	HTTP request/response formats

#### 5.3.3.1	General

Session establishment, modification, termination and query procedures are performed through HTTP transactions consisting of a request initiated by the PCRF and its corresponding response provided by the TSSF.

Table 5.3.3.1-1 summarizes the content of the requests and responses. More detailed information is specified in the corresponding subclauses as indicated in the table.

Table 5.3.3.1-1: St requests/response summary table

| Method | Resource URI’s path (NOTE 1) | Clause Defined | Request body | Response body |
| --- | --- | --- | --- | --- |
| POST | /stapplication/sessions | 5.3.3.2 | Content-Type: application/json The PCRF shall include the state of the session resource using the schema defined in Annex B.1. | Successful response: The TSSF may include informational data in the body of the response as per Annex B.2 Error response: The TSSF should include error data in the body of the response as per Annex B.2 |
| PUT | /stapplication/sessions/{stsessionid} | 5.3.3.3 | Content-Type: application/json The PCRF shall include the state of the session resource using the schema defined in Annex B.1. | Successful response: The TSSF may include informational data in the body of the response as per Annex B.2 Error response: The TSSF should include error data in the body of the response as per Annex B.2 |
| PATCH | /stapplication/sessions/{stsessionid} | 5.3.3.4 | Content-Type: application/json-patch+json The PCRF shall include the partial modifications to the state of the session resource using the schema defined in Annex B.1. | Successful response: The TSSF may include informational data in the body of the response as per Annex B.2 Error response: The TSSF should include error data in the body of the response as per Annex B.2 |
| DELETE | /stapplication/sessions/{stsessionid} | 5.3.3.5 | None | Successful response: The TSSF may include informational data in the body of the response as per Annex B.2 Error response: The TSSF should include error data in the body of the response as per Annex B.2 |
| GET | /stapplication/sessions/{stsessionid} | 5.3.3.6 | None | Successful response: The TSSF shall include the representation of the corresponding session resource in the body of the response as per Annex B.2 Error response: The TSSF should include error data in the body of the response as per Annex B.2 |
| POST | /{notificationbaseurl}/{stsessionid} | 5.3.3.7 | Content-Type: application/json The TSSF shall report traffic steering rule enforcement failures to the PCRF using the JSON format definition in Annex B.4 | Successful response: The PCRF may include informational data in the body of the response as per Annex B.2 Error response: The PCRF should include error data in the body of the response as per Annex B.2 |
| NOTE: A different path from /stapplication/sessions/ may be used when it is configured in the PCRF. In that case the "path" part set in the different methods should use the configured one. |  |  |  |  |

#### 5.3.3.2	POST /stapplication/sessions

The establishment of a session shall be performed by the PCRF by using the POST method as follows:

-	the request URI formatted as defined in subclause 5.3.2 with the "path" part set to: /stapplication/sessions.

-	the Content-Type header field set to "application/json"

-	the body of the message encoded in JSON format as defined in Annex B.1, including the "session-id" field allocated by the PCRF.

The PCRF sets the URI based on the pre-configuration at the PCRF.

If the PCRF supports the Notification feature, it shall follow the procedures defined in subclause 5.3.6.1 to advertise support of the feature. In addition, the PCRF shall include the 3gpp-Notification-Base-URL header as defined in subclause 5.3.7.4.

Upon receipt of the HTTP POST, the TSSF shall respond to the PCRF indicating whether the creation of the resource was successful or not using one of the HTTP status codes as defined in subclause 5.3.5. If the resource creation is successful, the TSSF shall respond with an HTTP 201 (Created) status code, including a Location header field containing the URI for the created session resource based on the St Session ID as defined in subclause 5.3.4. If the creation of the session resource was not successful, the TSSF shall indicate the reason using an appropriate HTTP status code for the St interface as defined in subclause 5.3.5 and optionally additional information in the body of the response as defined in Annex B.2.

If both the PCRF and TSSF support the Notification feature, the TSSF shall remember the value of the 3gpp-Notification-Base-URL for the lifetime of the corresponding St session and shall initiate a TCP connection towards the PCRF (if it is not already established) based on the hostname and optional port elements of the 3gpp-Notification-Base-URL.

NOTE:	Network deployment scenario with overlapping IP address assignment where the PCRF connects to multiple PCEFs connecting to the same PDN and where these multiple PCEFs connect to the same TSSF is not supported in this release.

Below is an example of an HTTP POST and a corresponding successful response:

POST /stapplication/sessions HTTP/1.1

Host: tssfserver.example.com

3gpp-Notification-Base-URL: http://pcrfserver.example.com:8080/stapplication/notification

3gpp-Optional-Features: Notification

Content-Type: application/json

Content-Length: …

{

"session-id": "pcrf.example.com;378388838383;123232",

"ue-ipv4": "10.0.0.2",

"called-station-id": "apncompany.com",

"tsrules": {

"ts-rule-3": {

"ts-rule-name": "ts-rule-3",

"tdf-application-identifier": "ftp-download",

"precedence": 1,

"ts-policy-identifier-dl": "firewall"

}

}

}

Here is an example of a successful response:

HTTP/1.1 201 Created

Location: http:// tssfserver.example.com/stapplication/sessions/pcrf.example.com;378388838383;123232

Date: Mon, 23 Apr 2012 17:10:00 GMT

Server: tssf.example.com

3gpp-Accepted-Features: Notification

Content-Type: application/json

{

"success-message": "Session was created successfully.",

}

#### 5.3.3.3	PUT /stapplication/sessions/{stsessionid}

The full modification of a session resource may be performed by the PCRF using the PUT method. In this case, the PCRF shall send an HTTP PUT request as follows:

-	the request URI formatted as defined in subclause 5.3.2 with the "path" part set to: /stapplication/sessions/{stsessionid}, where the stsessionid is the St Session ID as defined in subclause 5.3.4.

-	the Content-Type header field set to "application/json"

-	the body of the message encoded in JSON format as defined in Annex B.1 specifying the full representation of the session resource

Upon receipt of the HTTP PUT, the TSSF shall respond to the PCRF indicating whether the modification of the resource was successful or not using one of the HTTP status codes as defined in subclause 5.3.5. If the resource modification is successful, the TSSF shall respond with an HTTP 200 OK status code if it includes optional informational data in the body of the response as per Annex B.2 or HTTP 204 No Content status code otherwise. If the modification of the session resource was not successful, the TSSF shall indicate the reason using an appropriate HTTP status code for the St interface as defined in subclause 5.3.5 and optionally additional information in the body of the response as defined in Annex B.2.

Below is an example of an HTTP PUT:

PUT /stapplication/sessions/pcrf.example.com;378388838383;123232 HTTP/1.1

Host: tssfserver.example.com

Content-Type: application/json

Content-Length: …

{

"session-id": "pcrf.example.com;378388838383;123232",

"ue-ipv4": "10.0.0.2",

"tsrules": {

"ts-rule-1": {

"ts-rule-name": "ts-rule-1",

"tdf-application-identifier": "ftp-download",

"precedence": 1,

"ts-policy-identifier-dl": "firewall"

},

"ts-rule-2": {

"ts-rule-name": "ts-rule-2",

"tdf-application-identifier": "application-x",

"precedence": 2,

"ts-policy-identifier-dl": "firewall"

}

}

}

Here is an example of a successful response:

HTTP/1.1 200 OK

Date: Mon, 23 Apr 2012 17:11:00 GMT

Server: tssf.example.com

Content-Type: application/json

{

"success-message": "Session was updated successfully.",

}

#### 5.3.3.4	PATCH /stapplication/sessions/{stsessionid}

The partial modification of a session resource may be performed by the PCRF using the PATCH method as defined in IETF RFC 5789 [12]. In this case, the PCRF shall send an HTTP PATCH request as follows:

-	the request URI formatted as defined in subclause 5.3.2 with the "path" part set to: /stapplication/sessions/{stsessionid}, where the stsessionid is the St Session ID as defined in subclause 5.3.4.

-	the Content-Type header field set to "application/json-patch+json"

-	the body of the message shall be encoded using the JSON Patch format as defined in IETF RFC 6902 [14]. The following operations from [14] may be used:

-	add: to create a new traffic steering rule.

-	remove: to delete a traffic steering rule.

-	replace: to replace an existing rule with a new definition.

Upon receipt of the HTTP PATCH, the TSSF shall respond to the PCRF indicating whether the modification of the resource was successful or not using one of the HTTP status codes as defined in subclause 5.3.5. If the resource modification is successful, the TSSF shall respond with an HTTP 200 OK status code if it includes optional informational data in the body of the response as per Annex B.2 or HTTP 204 No Content status code otherwise. If the modification of the session resource was not successful, the TSSF shall indicate the reason using an appropriate HTTP status code for the St interface as defined in subclause 5.3.5 and optionally additional information in the body of the response as defined in Annex B.2.

Below is an example of an HTTP PATCH that modifies a traffic steering rule and deletes another one:

PATCH /stapplication/sessions/pcrf.example.com;378388838383;123232 HTTP/1.1

Host: tssfserver.example.com

Content-Type: application/json-patch+json

Content-Length: …

[

{

"op": "replace",

"path": "/tsrules/ts-rule-1",

"value": {

"ts-rule-name": "ts-rule-1",

"tdf-application-identifier": "ftp-download",

"precedence": 1,

"ts-policy-identifier-dl": "firewall2"

}

},

{

"op": "remove",

"path": "/tsrules/ts-rule-2",

}

]

Here is an example of a successful response:HTTP/1.1 200 OK

Date: Mon, 23 Apr 2012 17:12:00 GMT

Server: tssf.example.com

Content-Type: application/json

{

"success-message": "Session was patched successfully.",

}

#### 5.3.3.5	DELETE /stapplication/sessions/{stsessionid}

To delete an St session, the PCRF shall send an HTTP DELETE request to the TSSF as follows:

-	the request URI formatted as defined in subclause 5.3.2 with the "path" part set to: /stapplication/sessions/{stsessionid}, where the stsessionid is the St Session ID as defined in subclause 5.3.4.

Upon receipt of the HTTP DELETE, the TSSF shall respond to the PCRF indicating whether the deletion of the resource was successful or not using one of the HTTP status codes as defined in subclause 5.3.5. If the resource deletion is successful, the TSSF shall respond with an HTTP 200 OK status code if it includes optional informational data in the body of the response as per Annex B.2 or HTTP 204 No Content status code otherwise. If the deletion of the session resource was not successful, the TSSF shall indicate the reason using an appropriate HTTP status code for the St interface as defined in subclause 5.3.5 and optionally additional information in the body of the response as defined in Annex B.2.

Below is an example of an HTTP DELETE:

DELETE /stapplication/sessions/pcrf.example.com;378388838383;123232 HTTP/1.1

Host: tssfserver.example.com

Here is an example of a successful response:

HTTP/1.1 204 No Content

Date: Mon, 23 Apr 2012 17:15:00 GMT

Server: tssf.example.com

#### 5.3.3.6	GET /stapplication/sessions/{stsessionid}

To query the state of an St session resource, the PCRF shall send an HTTP GET request to the TSSF as follows:

-	the request URI formatted as defined in subclause 5.3.2 with the "path" part set to: /stapplication/sessions/{stsessionid}, where the stsessionid is the St Session ID as defined in subclause 5.3.4.

Upon receipt of the HTTP GET, the TSSF shall respond to the PCRF indicating whether the querying of the resource was successful or not using one of the HTTP status codes as defined in subclause 5.3.3. If the resource exists, the TSSF shall respond with an HTTP 200 OK status code and include the session representation state within the body of the response as defined in Annex B.2. The TSSF shall also include the Content-Type header field set to "application/json".

The TSSF shall also include the set of features it supports in common with the PCRF within the "3gpp-Accepted-Features" HTTP header in the response to the HTTP GET.

Below is an example of an HTTP GET:

GET /stapplication/sessions/pcrf.example.com;378388838383;123232 HTTP/1.1

Host: tssfserver.example.com

Here is an example of a successful response :

HTTP/1.1 200 OK

Server: tssfserver.example.com

Content-Type: application/json

Content-Length: …

{

"session-id": "pcrf.example.com;378388838383;123232",

"ue-ipv4": "10.0.0.2",

"called-station-id": "apncompany.com",

"tsrules": {

"ts-rule-1": {

"ts-rule-name": "ts-rule-1",

"tdf-application-identifier": "ftp-download",

"precedence": 1,

"ts-policy-identifier-dl": "firewall"

},

"ts-rule-2": {

"ts-rule-name": "ts-rule-2",

"tdf-application-identifier": "application-x",

"precedence": 2,

"ts-policy-identifier-dl": "firewall"

}

}

}

#### 5.3.3.7	POST /{notificationbaseurl}/{stsessionid}

If the notification feature is supported, the TSSF shall inform the PCRF of state changes related to a session resource or sub resources (e.g. traffic steering rules). The TSSF does so by sending an HTTP POST request as follows:

-	the request URI formatted as defined in subclause 5.3.2 with the "path" part set to: /{notificationbaseurl}/ {stsessionid}, where the "stsessionid" is the St Session ID as defined in subclause 5.3.4, and the "notificationbaseurl" is set to the value of the 3gpp-Notification-Base-URL provided by the PCRF.

-	the Content-Type header field set to "application/json"

-	the body of the message encoded in JSON format as defined in Annex B.4 specifying the representation of the notification report.

Upon receipt of the notification HTTP POST, the PCRF shall respond to the TSSF with one of the HTTP status codes as defined in subclause 5.3.5. If the PCRF is able to successfully process the request, the PCRF shall answer with an HTTP 200 OK status code if it includes optional informational data in the body of the response as per Annex B.2 or HTTP 204 No Content status code otherwise. If the PCRF cannot successfully process the request, the PCRF shall indicate the reason using an appropriate HTTP status code for the St interface as defined in subclause 5.3.5 and optionally include additional information in the body of the response as defined in Annex B.2.

Below is an example of an HTTP POST used for notification of traffic steering rule enforcement failure.

POST /stapplication/notification/pcrf.example.com;378388838383;123232 HTTP/1.1

Host: pcrfserver.example.com

Content-Type: application/json

Content-Length: …

{

"notifications": [

{

"notification-message": "Traffic steering rules became inactive.",

"notification-type": "application",

"notification-tag": "TS_RULE_EVENT",

"notification-info": {

"ts-rule-reports": [

{

"resource-paths": [

"/tsrules/ts-rule-1",

"/predefined-tsrules/ts-rule-2",

"/predefined-group-of-tsrules/group-rules-1"

],

"rule-status": "INACTIVE",

"rule-failure-code": "RESOURCES_LIMITATION"

}

]

}

}

]

}

Here is an example of a successful response:

HTTP/1.1 200 OK

Date: Mon, 23 Apr 2012 17:11:00 GMT

Server: pcrfserver.example.com

Content-Type: application/json

{

"success-message": "Notification was processed successfully.",

}

### 5.3.4	St session ID

The PCRF shall allocate an St Session ID that is used to uniquely identify the St session resource between the PCRF and TSSF. The St Session ID is provided as part of the session resource representation within the body of the HTTP message and shall remain unmodified during the lifetime of the St session resource. If the session resource is successfully created at the TSSF, the TSSF shall include the St session ID in the path element of an HTTP request URI to address the corresponding session resource.

NOTE:	The PCRF allocates the St Session ID as opposed to the TSSF to avoid creating duplicate sessions at the TSSF in case an HTTP POST message is received twice by the TSSF (e.g. due to a retry by the PCRF). The PCRF ensures that the St Session ID is globally unique as described in this subclause to avoid conflicts at the TSSF.

The Session ID shall be globally and eternally unique. The Session ID includes a mandatory portion and an implementation specific portion.

The mandatory portion shall start with the PCRF’s FQDN [8]. The remainder of the Session ID is delimited by a ‘;’ character and may be any sequence that the PCRF can guarantee to be eternally unique as follows:

<PCRF FQDN>;<Implementation Specific Sequence>

The implementation specific sequence should follow the recommended implementation specific portion of the Session-Id AVP as defined in section 8.8 of IETF RFC 6733 [21].

Here is an example of the Session ID: pcrf123.acme.com;1876543210;523. Here is an example of a corresponding session URI: http://tssfserver.example.com/stapplication/sessions/pcrf123.acme.com;1876543210;523

Because the Session ID is included as a path element of the request URI, it shall follow the implementation-specific syntax standardized for path elements of URLs as defined in [6].

NOTE:	Characters in the reserved set are protected from normalization and are therefore safe to be used by scheme-specific and producer-specific algorithms for delimiting data subcomponents within a URI [6].

### 5.3.5	HTTP status codes

A subset of the HTTP status codes as defined in IETF RFC 7231 [23] are applicable to the REST based St interface. Table 5.3.5-1 lists the applicable status codes for the St interface for the current release. Note that if the PCRF or TSSF receive a response with a status code not listed in the table below, they shall treat it based on the class of the status code as defined in IETF RFC 7231 [23].

Table 5.3.5-1: St HTTP status codes

| Status Code | Description |
| --- | --- |
| 200 OK | The HTTP request has succeeded. The information returned with the response is dependent on the method used in the request. |
| 201 Created | The HTTP POST request has been fulfilled and resulted in a new resource being created at the TSSF. |
| 204 No Content | The recipient has fulfilled the request but does not need to return an entity-body. |
| 400 Bad Request | The request could not be processed due to something that is perceived to be a client error (e.g., malformed request syntax, invalid request message framing, or deceptive request routing). |
| 403 Forbidden | The recipient understood the request, but is refusing to fulfill it. |
| 404 Not Found | The recipient has not found any resources matching the Request-URI. |
| 405 Method Not Allowed | The method specified in the Request-Line is not allowed for the resource identified by the Request-URI. |
| 408 Request Timeout | The recipient did not respond to the request within the time that the sender is prepared to wait. The sender may repeat the request without modifications at any later time. |
| 412 Precondition Failed | The precondition given in one or more of the request-header fields evaluated to false when it was tested on the recipient. |
| 413 Payload Too Large | The recipient is refusing to process a request because the request entity is larger than the recipient is willing or able to process. |
| 414 URI Too Long | The recipient is refusing to service the request because the Request-URI is longer than the recipient is willing to interpret. |
| 500 Internal Server Error | The recipient encountered an unexpected condition which prevented it from fulfilling the request. |
| 501 Not Implemented | The recipient does not support the functionality required to fulfill the request. |
| 503 Service Unavailable | The recipient is currently out of service. |

### 5.3.6	Feature negotiation

#### 5.3.6.1	General

The REST based St interface needs to provide a mechanism to advertise required and optional features supported by both the PCRF and TSSF for interoperability reasons as the functionality of the REST St based interface is augmented.

The PCRF shall include in the HTTP POST the set of supported features as follows:

-	if a feature is required for the proper operation of the St session, it shall be included within the 3gpp-Required-Features header;

-	if a feature is optional for the proper operation of the St session, it shall be included within the 3gpp-Optional-Features header.

The TSSF shall include, within the 3gpp-Accepted-Features header in the response to the HTTP POST, the set of features it supports in common with the PCRF.

If the TSSF does not support any of the required features advertised by the PCRF within the 3gpp-Required-Features header, the TSSF shall reject the HTTP POST with an HTTP 412 Precondition Failed status code and shall include the commonly supported features with the PCRF within the 3gpp-Accepted-Features.

If the TSSF requires certain features to be supported that aren’t advertised by the PCRF, the TSSF shall reject the HTTP POST with an HTTP 412 Precondition Failed status code and shall include the commonly supported features with the PCRF within the 3gpp-Accepted-Features and the required features in the 3gpp-required-features.

If the PCRF and TSSF successfully negotiate supported features, the list of commonly supported features shall be applicable for the lifetime of the St session. Features that are not advertised as supported shall not be used within the lifetime of the St session.

The table below defines the features applicable to the REST St interface.

- Table 5.3.6.1-1: Features used in REST St Protocol

| Feature | M/O | Description |
| --- | --- | --- |
| Notification | O | This feature indicates the support of the HTTP notification procedure from the TSSF to the PCRF. |
| Feature: A short name that can be used to refer to the bit and to the feature, e.g. "Notification". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O") in this 3GPP Release. Description: A clear textual description of the feature. |  |  |

### 5.3.7	HTTP custom headers

This subclause defines any new HTTP custom headers introduced by this specification.

#### 5.3.7.1	3gpp-Optional-Features

This header is used by the PCRF to advertise the optional features that are supported by the PCRF.

The encoding of the header follows the ABNF as defined in IETF RFC 7231 [23].

3gpp-Optional-Features = "3gpp-Optional-Features" ":" 1#token

An example is: 3gpp-Optional-Features: feature1, feature2

#### 5.3.7.2	3gpp-Required-Features

This header is used by the PCRF to announce the mandatory features that must be supported in TSSF.

This header is also used by the TSSF to indicate the missing features that must be supported in PCRF.

The encoding of the header follows the ABNF as defined in IETF RFC 7231 [23].

3gpp-Required-Features = "3gpp-Required-Features" ":" 1#token

An example is: 3gpp-Required-Features: feature1, feature2

#### 5.3.7.3	3gpp-Accepted-Features

The header is used by the TSSF to confirm the commonly supported set of features with the PCRF.

The encoding of the header follows the ABNF as defined in IETF RFC 7231 [23].

3gpp-Accepted-Features = "3gpp-Accepted-Features" ":" 1#token

An example is: 3gpp-Accepted-Features: feature1, feature2

#### 5.3.7.4	3gpp-Notification-Base-URL

This header is used by the PCRF to provide the notification base URL.

The encoding of the header follows the ABNF as defined in IETF RFC 7231 [23].

3gpp-Notification-Base-URL = "3gpp-Notification-Base-URL " ":" 1#token

An example is: 3gpp-Notification-Base-URL: http://pcrfserver.example.com:8080/stapplication/notification

## 5.4	Specific application communication

### 5.4.1	General

Specific application communication represents the presentation of application data structures by transforming data into the form that the application accepts. It establishes the context between application-layer entities.

Note: This release only supports the content type JSON

### 5.4.2	Content type

The body of  HTTP messages shall be in JSON format. The content of the JSON text is defined in subclause 5.4.3 and Annex B.

The MIME media type that shall be used within the Content-Type header field is “application/json” as defined in  IETF RFC 7159 [10].

## 5.4.3	JSON session fields

### 5.4.3.1	General

Table 5.4.3.1.1 describes the JSON fields used within the body of the HTTP messages representing the session information. The table includes the information about the name of the field, the type of the fields and the applicability of the JSON field.

Table 5.4.3.1.1: St Specific JSON fields

|  |  |  |  | Applicability (NOTE 3) |
| --- | --- | --- | --- | --- |
| Field Name | Subclause defined | JSON Value Type (NOTE 1) | JCR Type (NOTE 2) |  |
| called-station-id | 5.4.3.20 | string | string |  |
| session-id | 5.4.3.2 | string | string |  |
| ue-ipv4 | 5.4.3.3 | string | ipv4 |  |
| ue-ipv6-prefix | 5.4.3.4 | string | Ipv6 |  |
| tsrules | 5.4.3.5 | object | object |  |
| ts-rule-name | 5.4.3.6 | string | string |  |
| Precedence | 5.4.3.7 | number | number: 0..4294967295 |  |
| tdf-application-identifier | 5.4.3.8 | string | string |  |
| flow-information | 5.4.3.9 | array | array |  |
| flow-description | 5.4.3.10 | string | string |  |
| tos-traffic-class | 5.4.3.11 | string | /^[0-9A-Fa-f]{4}$/ (NOTE 4) |  |
| security-parameter-index | 5.4.3.12 | string | /^[0-9A-Fa-f]{8}$/ (NOTE 4) |  |
| flow-label | 5.4.3.13 | string | /^[0-9A-Fa-f]{6}$/ (NOTE 4) |  |
| flow-direction | 5.4.3.14 | string | "BIDIRECTIONAL" "UPLINK" "DOWNLINK" (NOTE 5) |  |
| ts-policy-identifier-dl | 5.4.3.15 | string | string |  |
| ts-policy-identifier-ul | 5.4.3.16 | string | string |  |
| predefined-tsrules | 5.4.3.17 | object | object |  |
| predefined-group-of-tsrules | 5.4.3.18 | object | object |  |
| ts-rule-base-name | 5.4.3.19 | string | string |  |
| NOTE 1: The basic JSON value types are defined in IETF RFC 7159 [15]. NOTE 2: The JCR types are defined in IETF draft-newton-json-content-rules [16]. NOTE 3: Fields marked with a supported feature are applicable as described in subclause 5.3.6. NOTE 4: The regular expression for a string type. NOTE 5: The quoted strings for a string type. |  |  |  |  |

### 5.4.3.2	session-id

The session-id field is of type string, and contains the session id allocated by the PCRF. The session id format is defined in subclause 5.3.4.

### 5.4.3.3	ue-ipv4

The ue-ipv4 field is of type string and ip4 literal as defined in IETF draft-newton-json-content-rules [16].

### 5.4.3.4	ue-ipv6-prefix

The ue-ipv6-prefix field is of type string and ip6 literal as defined in IETF draft-newton-json-content-rules [16]. The PCRF uses it to indicate the IPv6 prefix allocated for the user.

### 5.4.3.5	tsrules

The tsrules field is of type object. It contains all of the dynamic traffic steering rules defined in the JSON body.

The following defines the content of the dynamic traffic steering rule:

The ts-rule-name field shall be included within the dynamic traffic steering rule.

A precedence value may be included.

Either flow-information(s) or the tdf-application-identifier field shall be included within the dynamic traffic steering rule.

The ts-policy-identifier-ul field and/or the ts-policy-identifier-dl field shall be included. If the traffic steering policies are identical in both downlink and uplink directions, the values of the ts-policy-identifier-ul field and the ts-policy-identifier-dl field shall be identical.

Below are the JCR for the dynamic traffic steering rule:

$tsrule =  // : {

$ts-rule-name,

$precedence ?,

( $flow-information | $tdf-application-identifier ),

( $ts-policy-identifier-ul | $ts-policy-identifier-dl | ( $ts-policy-identifier-ul, $ts-policy-identifier-dl ) )

}

### 5.4.3.6	ts-rule-name

The ts-rule-name is of type string, and it defines a name for traffic steering rule. For traffic steering rules provisioned by the PCRF, it uniquely identifies a traffic steering rule within one St session. For traffic steering rules pre-defined at the TSSF, it uniquely identifies a traffic steering rule within the TSSF.

### 5.4.3.7	precedence

The precedence is of type number. Its value ranges from 0 to 4294967295.

Within the tsrule JSON content, the precedence field determines the order for the service data flow detection/the application detection at the TSSF to perform traffic steering. A traffic steering rule with a lower precedence value shall be applied prior to a traffic steering rule with a higher precedence value.

NOTE:	The precedence value range defined within the traffic steering rule is operator configurable.

### 5.4.3.8	tdf-application-identifier

The tdf-application-identifier is of type string. It references the application detection filter defined at the TSSF.

### 5.4.3.9	flow-information

The flow-information field is of type array. It is sent from the PCRF to the TSSF and contains the information for a single IP flow packet filter.

The flow-description, a tos-traffic-class, security-parameter-index and flow-label specify the parameters to be used for matching payload packets. One of these fields shall be present in the object. The flow-direction field shall also be included.

The JCR for the flow-information is as follows:

$flow-information = "flow-information" : [

{

$flow-description ?,

$tos-traffic-class ?,

$security-parameter-index ?,

$flow-label ?,

$flow-direction

} +

]

### 5.4.3.10	flow-description

The flow-description field is of type string, it defines a packet filter for an IP flow.

The content has the same encoding as the IPFilterRule AVP value as defined in IETF RFC 6733 [21]. All limitations applicable to the Flow-Description AVP in subclause 5.4.2 of 3GPP TS 29.212 [17] apply to this field.

### 5.4.3.11	tos-traffic-class

The tos-traffic-class field is of type string. It is encoded on two octets, and represented as a hex string. The first octet contains the IPv4 Type-of-Service or the IPv6 Traffic-Class field and the second octet contains the ToS/Traffic Class mask field. When used for traffic detection, it defines the packet marking information used for traffic detection in the TSSF.

### 5.4.3.12	security-parameter-index

The security-parameter-index field is of type string, and it contains the security parameter index of the IPSec packet.

### 5.4.3.13	flow-label

The flow-label field is of type string, and it contains the IPv6 flow label header field.

### 5.4.3.14	flow-direction

The flow-direction field is of type string with three enumerations. It indicates the direction(s) that a filter is applicable, downlink only, uplink only or both down- and uplink (bidirectional).

DOWNLINK

The corresponding filter applies for traffic to the UE.

UPLINK

The corresponding filter applies for traffic from the UE.

BIDIRECTIONAL

The corresponding filter applies for traffic both to and from the UE.

### 5.4.3.15	ts-policy-identifier-dl

The ts-policy-identifier-dl is of type string and contains a reference to a pre-configured traffic steering policy at the TSSF in the downlink direction.

### 5.4.3.16	ts-policy-identifier-ul

The ts-policy-identifier-ul is of type string and contains a reference to a pre-configured traffic steering policy at the TSSF in the uplink direction.

### 5.4.3.17	predefined-tsrules

The predefined-tsrules contains all of the predefined traffic steering rules in the session.

The ts-rule-name field uniquely identifies the traffic steering rule defined at the TSSF.

The JCR definition for the predefined traffic steering rule is as follows:

$predefined-tsrule = // : {

ts-rule-name

}

### 5.4.3.18	predefined-group-of-tsrules

The predefined-group-of-tsrules field is of type object and contains the groups of the predefined traffic steering rules in the session.

The ts-rule-base-name field uniquely identifies the group of predefined traffic steering rules at the TSSF.

The JCR definition for the group of the dynamic traffic steering rules:

$predefined-group-of-tsrule = // : {

ts-rule-base-name

}

### 5.4.3.19	ts-rule-base-name

The ts-rule-base-name field is of type string, and it indicates the name of a predefined group of traffic steering rules residing at the TSSF.

### 5.4.3.20	called-station-id

The called-station-id is of type string, and it defines the address the user is connected to (i.e. the PDN identifier). For GPRS and EPS the APN. When used to contain the APN, the APN is composed of the APN Network Identifier only, or the APN Network Identifier and the APN Operator Identifier as specified in 3GPP TS 23.003 [20], subclause 9.1. The inclusion of the APN Operator Identifier can be configurable.

## 5.4.4	JSON errors and informational response fields

### 5.4.4.1	General

Table 5.4.4.1.1 describes the JSON fields defined for the errors and informational responsesincluding their types and the filed names.

Table 5.4.4.1.1: JSON fields for errors and informational response

| Field Name | Subclause defined | JSON Value Type (NOTE 1) | JCR Type (NOTE 2) |
| --- | --- | --- | --- |
| errors | 5.4.4.2 | array | array |
| error-type | 5.4.4.3 | string | "application" "interface" "server" "other" (NOTE 3) |
| error-message | 5.4.4.4 | string | string |
| error-tag | 5.4.4.5 | string | string |
| error-path | 5.4.4.6 | string | string |
| error-info | 5.4.4.7 | object | object |
| success-message | 5.4.4.8 | string | string |
| success-path | 5.4.4.9 | string | string |
| success-info | 5.4.3.10 | object | object |
| NOTE 1: The basic JSON value types are defined in IETF RFC 7159 [10]. NOTE 2: The JCR types are defined in IETF draft-newton-json-content-rules [16]. NOTE 3: The quoted strings for a string type. |  |  |  |

### 5.4.4.2	errors

The errors field is of type array, and it contains a list of errors. Each error is defined as an object in the array. This field shall be present when the TSSF responds with error(s) back to an HTTP request initiated by the PCRF.

An error object shall contain the error-type field and the error-message.

Optionally, it may include the error-tag field, error-path field and error-info field for further details. The error tag is used to classify a special error from other errors. The error-path is used to identify which resource has the error. The error-info is provided for additional information.

The JCR format:

e

$errors = "errors" : [

{

$error-type,

$error-message,

$error-tag ?,

$error-path ?,

$error-info ?

} +

]

### 5.4.4.3	error-type

The error-type field is of type string. It can be one of 'application', 'interface', 'server' and 'other'.

When an error is related to the application processing in the TSSF, the error type shall be specified as 'application'. When the error is a protocol compliance error, 'interface' shall be used. A server failure is represented with the 'server' error type. For all other errors, the 'other' error type shall be used.

### 5.4.4.4	error-message

The error-message field is of type string. It is a textual description of the error.

### 5.4.4.5	error-tag

The error-tag field is of type string. It defines a tag for a particular error.

- TS_RULE_EVENT

This error-tag shall be used when the traffic steering rules cannot be installed/activated at the TSSF. Affected traffic steering rules shall be provided in the error-info with a sub field named ts-rule-reports including the reason and status of the rules as described insubclause 5.4.5.

### 5.4.4.6	error-path

The error-path field is of type string. It shall contain a JSON pointer as defined in IETF RFC 6901 [15] pointing to a resource where the error occurs.

### 5.4.4.7	error-info

The error-info field is defined as an object type. It represents informational details regarding the error.

### 5.4.4.6	success-message

The success-message is of type string. When included in the JSON body of an HTTP response, it shall be included under the root level of the JSON content.

### 5.4.4.7	success-path

The success-path field is of type string. It is a JSON pointer as defined in IETF RFC 6901 [15] pointing to the resource that was successfully operated on. It is an optional root level field provided along with a success-message field.

### 5.4.4.8	success-info

### The success-info field is defined as an object type. It may include any additional information under the root level of the JSON content for an informational response.

## 5.4.5	JSON report fields

### 5.4.5.1	General

Table 5.4.5.1.1 describes the JSON fields defined for the report information objects which are included in the error-info field or the notification-info field.

Table 5.4.5.1.1: JSON fields for Report from the TSSF

| Field Name | Subclause defined | JSON Value Type (NOTE 1) | JCR Type (NOTE 2) |
| --- | --- | --- | --- |
| ts-rule-reports | 5.4.5.2 | array | array |
| resource-paths | 5.4.5.3 | array | array |
| rule-status | 5.4.5.4 | string | string |
| rule-failure-code | 5.4.5.5 | string | string |
| NOTE 1: The basic JSON value types are defined in IETF RFC 7159 [10]. NOTE 2: The JCR types are defined in IETF draft-newton-json-content-rules [16]. |  |  |  |

### 5.4.5.2	ts-rule-reports

The ts-rule-reports field is of type array and it contains a list of traffic steering rule reports.

The ts-rule-reports field can be used in an HTTP response to report failures in the installation/activation of traffic steering rules. In this case, it shall be provided as a sub field of the error-info field.

The ts-rule-reports field can also be used in a HTTP request from the TSSF to notify the PCRF about enforcement failures of traffic steering rules. In this case, it shall be provided as a sub field of the notification-info field.

Multiple traffic steering rule report instances shall be provided within the ts-rule-reports field if different rule-status or rule-failure-code values are applicable within the same HTTP response or notification.

A report instance shall contain the resource-paths, rule-status and rule-failure-code fields.

The JCR format for the ts-rule-reports is:

$ts-rule-reports = "ts-rule-reports" : [

{

$resource-paths,

$rule-status,

$rule-failure-code

} +

]

### 5.4.5.3	resource-paths

The resource-paths field is of type array of strings. Each string shall be a JSON pointer as defined in IETF RFC 6901 [15].

Each JSON pointer within the resource-paths field shall be a reference to a specific traffic steering rule or to a group of traffic steering rules which cannot be installed/activated or enforced at the TSSF.

### 5.4.5.4	rule-status

The rule-status is of type string and shall contain one of the string representations of the PCC-Rule-Status AVP values as defined in subclause 5.3.19 of 3GPP TS 29.212 [17].

NOTE: For the current release, only the INACTIVE value is used.

### 5.4.5.5	rule-failure-code

The rule-failure-code is of type string.

The following values defined in the Rule-Failure-Code AVP in subclause 5.3.38 of 3GPP TS 29.212 [17] are applicable for traffic steering rule errors.

- UNKNOWN_RULE_NAME

- GW/PCEF_MALFUNCTION

- RESOURCES_LIMITATION

- MISSING_FLOW_INFORMATION

- INCORRECT_FLOW_INFORMATION

- TDF_APPLICATION_IDENTIFIER_ERROR

- FILTER_RESTRICTIONS

The following value defined in the Rule-Failure-Code AVP is reused for traffic steering rule errors, however with a different explanation:

- RESOURCE_ALLOCATION_FAILURE

This value is used to indicate that the traffic steering rule could not be successfully installed or maintained since the resources or services of the traffic steering control specified by the rule cannot be reserved in the TSSF and/or the (S)Gi-LAN; or those resources or services were released in the TSSF and/or the (S)Gi-LAN, for example, due to the switch eviction mechanism.

In addition, the following values are added:

- RESOURCE_TIMEOUT

This value is used to indicate that the traffic steering rule could not be successfully installed or maintained since the booked resources or services of the traffic steering control specified by the rule have expired in the TSSF and/or the (S)Gi-LAN; for example, due to the flow expiry mechanism in the switches.

- TS_POLICY_IDENTIFIER_ERROR

This value is used to indicate that the rule could not be successfully installed or enforced because both the ts-policy-identifier-dl and ts-policy-identifier-dl are invalid, unknown, or not applicable to the downlink and uplink traffic flows for the traffic steering control.

- TS_POLICY_IDENTIFIER_DL_ERROR

This value is used to indicate that the rule could not be successfully installed or enforced because the ts-policy-identifier-dl is invalid, unknown, or not applicable to the downlink traffic flows for the traffic steering control.

- TS_POLICY_IDENTIFIER_UL_ERROR

This value is used to indicate that the rule could not be successfully installed or enforced because the ts-policy-identifier-ul is invalid, unknown, or not applicable to the uplink traffic flows for the traffic steering control.

## 5.4.6	JSON notification fields

### 5.4.6.1	General

Table 5.4.6.1.1 describes the JSON fields defined for the notification HTTP messages from the TSSF to the PCRF, including their JSON value types, and JCR types as well.

Table 5.4.6.1.1: JSON fields for notifications from the TSSF

| Field Name | Subclause defined | JSON Value Type (NOTE 1) | JCR Type (NOTE 2) |
| --- | --- | --- | --- |
| notifications | 5.4.6.2 | array | array |
| notification-type | 5.4.6.3 | string | "application" "other" (NOTE 3) |
| notification-message | 5.4.6.4 | string | String |
| notification-tag | 5.4.6.5 | string | String |
| notification-info | 5.4.6.6 | object | Object |
| NOTE 1: The basic JSON value types are defined in IETF RFC 7159 [10]. NOTE 2: The JCR types are defined in IETF draft-newton-json-content-rules [16]. NOTE 3: The quoted strings for a string type. |  |  |  |

### 5.4.6.2	notifications

The notifications field is of type array, and it contains a list of notifications provided by the TSSF. Each notification is defined as an object in the array.

A notification object shall contain the notification-type and the notification-message fields.

Optionally, it may include the notification-tag and notification-info fields. The notification-tag is used to distinguish a specific notification from others. The notification-info is provided for additional information.

The JCR format of the notifications is as follows:

$notifications = "notifications" : [

{

$notification-type,

$notification-message,

$notification-tag ?,

$notification-info ?

} +

### ]5.4.6.3	notification-type

The notification-type field is of type string. It can be one of 'application' or 'other'.

When a notification is due to a change to one of the St session resources/sub-resources, the type shall be specified as 'application'. For all other notifications, the 'other' type shall be used.

### 5.4.6.4	notification-message

The notification-message field is of type string. It is a textual description of the notification.

### 5.4.6.5	notification-tag

The notification-tag field is of type string. It defines a tag for a particular notification.

- TS_RULE_EVENT

This notification-tag value shall be used when the traffic steering rules cannot be enforced at the TSSF. Affected traffic steering rules shall be provided by the TSSF in the notification-info with a sub field named ts-rule-reports including the report reason and status of the rules.

### 5.4.6.6	notification-info

The notification-info field is defined as an object type. It represents informational details regarding the notification.

## 5.5	TSSF discovery

The TSSF URI is pre-configured on the PCRF, e.g. per PCEF.

# 6	Secure communication

Either the NDS/IP network layer security defined in 3GPP TS 33.210 [7] or HTTP over TLS as defined in [19] should be used to secure communication over the REST based St interface.

Annex A (informative): 
Call Flows

# A.1	General

This annex describes the procedures for the interactions between the PCRF and the TSSF

# A.2	Traffic Steering Control Information Provisioning

This subclause describes the signalling flow for the traffic steering control information provisioning.

Figure A.2.1: Traffic Steering Control Information Provisioning

1.	The PCRF sends the HTTP POST to the TSSF to initiate the creation of a resource state to provision the traffic steering control information including the parameters defined in subclause 4.4.2.

2. 	The TSSF sends the HTTP 201 CREATED response to the PCRF including the parameters defined in subclause 4.4.2.

# A.3	Traffic Steering Control Information Update

This subclause describes the signalling flow for the traffic steering control information update or the allocation/release notification of the UE IPv4 address.

Figure A.3.1: Traffic Steering Control Information Update

1.	The PCRF sends the HTTP PUT/PATCH to the TSSF to initiate the traffic steering control information update including the parameters defined in subclause 4.4.2. The PCRF also sends the HTTP PUT/PATCH to the TSSF to notify the UE IPv4 address allocation or release including the parameters defined in subclause 4.4.4.

2.	The TSSF sends the HTTP 200 OK to the PCRF.

# A.4	Traffic Steering Control Information Removal

This subclause describes the signalling flow for the traffic steering control information removal.

Figure A.4.1: Traffic Steering Control Information Removal

1.	The PCRF sends the HTTP DELETE to the TSSF to initiate the traffic steering control information removal including the parameters defined in subclause 4.4.2.

2.	The TSSF removes/deactivates all the ADC rules and sends the HTTP 200 OK to the PCRF including the parameters defined in subclause 4.4.2.

# A.5	Traffic Steering Control Notification

This subclause describes the signalling flow for the traffic steering control notification.

Figure A.5.1: Traffic Steering Control Notification

1.	The TSSF sends an HTTP POST to the PCRF to inform it of changes to the state of traffic steering control rules as defined in subclause 4.4.3.

2.	The PCRF acknowledges the notification and sends an HTTP 200 OK to the TSSF.

Annex B (normative): JSON Schema

# B.1	Session body schema

This subclause defines the JSON schema for the body of HTTP requests and responses providing the state of a session resource. The schema is based on IETF draft-newton-json-content-rules [16] and is defined below:

# jcr-version 0.7

# ruleset-id 3gpp.stapplication.session

; A JCR representing the session resource

$session-root = @{root}{

$session-id,

( $ue-ipv4 | $ue-ipv6-prefix | ( $ue-ipv4 , $ue-ipv6-prefix ) ),

$called-station-id ?,

$tsrules ?,

$predefined-tsrules ?,

$predefined-group-of-tsrules ?

}

; Resource fields definitions

; Session ID allocated by the PCRF

$session-id = "session-id" : string

; The UE IPv4 address

$ue-ipv4 = "ue-ipv4" : ipv4

; The UE IPv6 prefix

$ue-ipv6-prefix = "ue-ipv6-prefix" : ipv6

; The PDN the UE accesses

$called-station-id = "called-station-id" : string

; The dynamic traffic steering rules

$tsrules = "tsrules" : { $tsrule + }

; The traffic steering rule

$tsrule = // : {

$ts-rule-name,

$precedence ?,

( $flow-information | $tdf-application-identifier ),

( $ts-policy-identifier-ul | $ts-policy-identifier-dl | ( $ts-policy-identifier-ul, $ts-policy-identifier-dl ) )

}

; The name of the traffic steering rule in the TSSF

$ts-rule-name = "ts-rule-name" : string

; The precedence of the traffic steering rule. From 0 to 4294967295.

$precedence = "precedence" : 0..4294967295

; The flow traffic information

$flow-information = "flow-information" : [

{

$flow-description ?,

$tos-traffic-class ?,

$security-parameter-index ?,

$flow-label ?,

$flow-direction

}

]

; The IP filter rule for a service data flow

$flow-description = "flow-description" : string

; The ToS traffic class is encoded on two octets, and represented in hex string display.

$tos-traffic-class = "tos-traffic-class" : /^[0-9A-Fa-f]{4}$/

; The security parameter index is encoded on four octets, and represented in hex string display.

$security-parameter-index = "security-parameter-index" : /^[0-9A-Fa-f]{8}$/

; The flow label is encoded on three octets, and represented in hex string display.

$flow-label = "flow-label" : /^[0-9A-Fa-f]{6}$/

; The direction of the flow traffic

$flow-direction = "flow-direction" : ( "BIDIRECTIONAL" | "UPLINK" | "DOWNLINK" )

; The detected application traffic id in the TSSF

$tdf-application-identifier = "tdf-application-identifier" : string

; The traffic steering policy id for uplink data

$ts-policy-identifier-ul = "ts-policy-identifier-ul" : string

; The traffic steering policy id for downlink data

$ts-policy-identifier-dl = "ts-policy-identifier-dl" : string

; The predefined traffic steering rules

$predefined-tsrules = "predefined-tsrules" : { $predefined-tsrule +  }

; The predefined traffic steering rule

$predefined-tsrule = // : { $ts-rule-name }

; The predefined traffic steering group rules

$predefined-group-of-tsrules = "predefined-group-of-tsrules" : { $predefined-group-of-tsrule + }

; The predefined traffic steering group rule

$predefined-group-of-tsrule = // : { $ts-rule-base-name }

; The name of the traffic steering group rule in the TSSF

$ts-rule-base-name = "ts-rule-base-name" : string

# B.2	Error and Informational response schema

This subclause defines the JSON schema for the body of HTTP responses in case of errors or success. The schema is based on IETF draft-newton-json-content-rules [16] and is defined below:

# jcr-version 0.7

# ruleset-id 3gpp.stapplication.info

# import 3gpp.stapplication.report as report

; A JCR for the error/successful response body

; Errors information

$errors-root = @{root} { $errors }

; Success information

$success-root = @{root} {

$success-message,

$success-path ?,

$success-info ?

}

; Resource fields definitions

; The list of errors returned in responses sent by the PCRF or TSSF

$errors = "errors" : [

{

$error-type,

$error-message,

$error-tag ?,

$error-path ?,

$error-info ?

} +

]

; The error type for an error. It can be one of 'application', 'interface', 'server' and 'other'.

$error-type = "error-type" : ( "application" | "interface" | "server" | "other" )

; The error text message

$error-message = "error-message" : string

; The error tag for a specific error

$error-tag = "error-tag" : string

; A JSON pointer path to the error resource

$error-path = "error-path" : string

; Any additional information for the error

$error-info = "error-info" : {

$report.ts-rule-reports ?,

// : any *

}

; The successful text message

$success-message = "success-message" : string

; A JSON pointer path to the success resource

$success-path = "success-path" : string

; Any additional information for the success.

$success-info = "success-info" : {  // : any *}

# B.3	Report schema

This clause defines the JSON schema for reports provided by the TSSF to the PCRF. The schema is based on IETF draft-newton-json-content-rules [16] and is defined below:

# jcr-version 0.7

# ruleset-id 3gpp.stapplication.report

; A JCR for the notification body

; Report fields definitions

; The list of ts rule reports sent to the PCRF

$ts-rule-reports = "ts-rule-reports" : [

{

$resource-paths,

$rule-status,

$rule-failure-code

} +

]

; The list of JSON pointers to the resources

$resource-paths = "resource-paths" : [ string + ]

; The status for a traffic steering rule. It can be 'INACTIVE'.

$rule-status = "rule-status" : $rule-status-type

; The string format for the traffic steering rule status

$rule-status-type =: ("INACTIVE")

; The failure code for a traffic steering rule

$rule-failure-code = "rule-failure-code" : $rule-failure-code-type

; The string format for the traffic steering rule failure code

$rule-failure-code-type =: (

"UNKNOWN_RULE_NAME" |

"GW/PCEF_MALFUNCTION" |

"RESOURCES_LIMITATION" |

"MISSING_FLOW_INFORMATION" |

"INCORRECT_FLOW_INFORMATION" |

"TDF_APPLICATION_IDENTIFIER_ERROR" |

"FILTER_RESTRICTIONS" |

"RESOURCE_ALLOCATION_FAILURE" |

"RESOURCE_TIMEOUT" |

"TS_POLICY_IDENTIFIER_ERROR" |

"TS_POLICY_IDENTIFIER_DL_ERROR" |

"TS_POLICY_IDENTIFIER_UL_ERROR"

)

# B.4	Notification schema

This clause defines the JSON schema for the body of the notification from the TSSF to the PCRF via the HTTP POST request. The schema is based on IETF draft-newton-json-content-rules [16] and is defined below:

# jcr-version 0.7

# ruleset-id 3gpp.stapplication.notification

# import 3gpp.stapplication.report as report

; A JCR for the notification body

$notifiction-root = @{root} { $notifications }

; Resource fields definitions

; The list of notifications notified to the PCRF

$notifications = "notifications" : [

{

$notification-type,

$notification-message,

$notification-tag ?,

$notification-info ?

} +

]

; The notification type for a notification. It can be one of 'application' or 'other'.

$notification-type = "notification-type" : ( "application" | "other" )

; The notification text message

$notification-message = "notification-message" : string

; The notification tag for a specific notification

$notification-tag = "notification-tag" : string

; Any additional information for the notification

$notification-info = "notification-info" : { $report.ts-rule-reports ?,  // : any *}

Annex C (informative): 
Change history

| Change history |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR |  | Rev |  | Subject/Comment |  |  | Old | New |
| 2015-08 | CT3#82 |  |  |  |  |  | TS skeleton of St reference point stage 3. |  |  |  | 0.0.0 |
| 2015-08 | CT3#82 | C3-153467 |  |  |  |  | Inclusion of documents agreed in CT3#82. C3-153291, C3-153416, C3-153444, C3-153455, C3-153445, C3-153420, C3-153439, C3-153290. Added reference to IETF RFC 1786: "Uniform Resource Locators (URL).” |  |  | 0.0.0 | 0.1.0 |
| 2015-10 | CT3#82-Bis | C3-154345 |  |  |  |  | Inclusion of documents agreed in CT3#82bis. C3-154231, C3-154049, C3-154231, C3-154217, C3-154218, C3-154319, C3-154139 |  |  | 0.1.0 | 0.2.0 |
| 2015-11 | CT3#83 | C3-155420 |  |  |  |  | Inclusion of documents agreed in CT3#83. C3-155227, C3-155229, C3-155365, C3-155235, C3-155284 |  |  | 0.2.0 | 1.0.0 |
| 2015-11 | CT#70 | CP-150824 |  |  |  |  | TS presented for presentation and approval |  |  | 1.0.0 | 1.0.1 |
| 2015-12 | CT#70 | CP-150824 |  |  |  |  | TS 29.155 upgraded to 13.0.0 after approval |  |  | 1.0.1 | 13.0.0 |
| Change history |  |  |  |  |  |  |  |  |  |  |  |
| Date | TSG # | TSG Doc. |  | CR |  | Rev |  | Cat | Subject/Comment |  | New |
| 2016-03 | CT#71 | CP-160091 |  | 0001 |  | 2 |  | B | Traffic Steering Rule Error handling over the REST based St interface |  | 13.1.0 |
| 2016-03 | CT#71 | CP-160091 |  | 0002 |  | 1 |  | F | Miscellaneous corrections related to the REST based St |  | 13.1.0 |
| 2016-03 | CT#71 | CP-160091 |  | 0003 |  | 3 |  | B | Updates to JSON content rules for the notification procedures from the TSSF to the PCRF |  | 13.1.0 |
| 2016-03 | CT#71 | CP-160091 |  | 0004 |  | 1 |  | B | JSON content rules updates for error responses with traffic steering rule reports over the REST based St interface |  | 13.1.0 |
| 2016-03 | CT#71 | CP-160091 |  | 0005 |  | 3 |  | B | Traffic steering rule report JSON Content Rules over St |  | 13.1.0 |
| 2016-12 | CT#74 | CP-160619 |  | 0008 |  | 4 |  | F | Overlapping IP Addresses with FMSS |  | 13.2.0 |
| 2016-12 | CT#74 | CP-160619 |  | 0009 |  | 3 |  | F | Support of deployments with both PCEF/TDF and TSSF |  | 13.2.0 |
| 2016-12 | CT#74 | CP-160619 |  | 0010 |  | 1 |  | F | Correction of IETF newton json content rule draft version and relevant JCR schemas |  | 13.2.0 |
| 2016-12 | CT#74 | CP-160619 |  | 0013 |  | 2 |  | F | UE IPv4 address provisioning |  | 13.2.0 |
| 2016-12 | CT#74 | CP-160616 |  | 0012 |  | 1 |  | F | Diameter base protocol specification update |  | 14.0.0 |
| 2017-03 | CT#75 | CP-170076 |  | 0014 |  | 1 |  | F | Correction of Diameter base protocol specification update |  | 14.1.0 |
| 2017-06 | CT#76 | CP-171133 |  | 0015 |  | 1 |  | D | Remove the Void reference |  | 14.2.0 |
| 2017-06 | CT#76 | CP-171120 |  | 0017 |  | 1 |  | A | Correction of IETF newton json content rule draft version |  | 14.2.0 |
| 2017-09 | CT#77 | CP-172038 |  | 0021 |  | - |  | A | Correction of resource path |  | 14.3.0 |
| 2018-06 | CT#80 |  |  |  |  |  |  |  | Automatic upgrade from previous Release |  | 15.0.0 |
| 2019-06 | CT#84 | CP-191091 |  | 0024 |  | - |  | A | Reference update: draft-newton-json-content-rules |  | 15.1.0 |
| 2020-07 | SA#88e | - |  | - |  | - |  |  | Update to Rel-16 version (MCC) |  | 16.0.0 |
| 2021-12 | CT#94e | CP-213220 |  | 0025 |  | - |  | B | Reference update for HTTP/1.1 protocol |  | 17.0.0 |