| 3GPP TS 29.538 V0.4.0 (2022-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Enabling MSGin5G Service; Application Programming Interfaces (API) specification; Stage 3; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	12

3.3	Abbreviations	12

4	Overview	12

5	Services offered by the MSGin5G Servers	13

5.1	Introduction	13

5.2	MSGS_ASRegistration Service	13

5.2.1	Service Description	13

5.2.2	Service Operations	13

5.2.2.1	Introduction	13

5.2.2.2	MSGS_ASRegistration_Request	14

5.2.2.2.1	General	14

5.2.2.2.2	Application Server registering to MSGin5G Server using MSGS_ASRegistration_Request operation	14

5.2.2.3	MSGS_ASRegistration_Deregister	14

5.2.2.3.1	General	14

5.2.2.3.2	Application Server deregistering from MSGin5G Server using MSGS_ASRegistration_Deregister operation	14

5.3	MSGS_MSGDelivery Service	15

5.3.1	Service Description	15

5.3.2	Service Operations	15

5.3.2.1	Introduction	15

5.3.2.2	MSGS_MSGDelivery_ASODelivery	16

5.3.2.2.1	General	16

5.3.2.2.2	AS Originating MSGin5G Message Delivery	16

5.3.2.3	MSGS_MSGDelivery_ASODeliveryReport	17

5.3.2.3.1	General	17

5.3.2.3.2	AS Originating Message Delivery Status Report	17

5.3.2.4	MSGS_MSGDelivery_UEODelivery	18

5.3.2.4.1	General	18

5.3.2.4.2	UE Originating Message Delivery	18

5.3.2.5	MSGS_MSGDelivery_UEODeliveryReport	19

5.3.2.5.1	General	19

5.3.2.5.2	UE Originating Message Delivery Status Report	19

6	Services offered by the Message Gateway	20

6.1	Introduction	20

6.2	MSGG_L3GDelivery Service	21

6.2.1	Service Description	21

6.2.2	Service Operations	21

6.2.2.1	Introduction	21

6.2.2.2	MSGG_L3GDelivery_GTDelivery	22

6.2.2.2.1	General	22

6.2.2.2.2	Legacy 3GPP Message Gateway Terminating Message Delivery	22

6.2.2.3	MSGG_L3GDelivery_GTDeliveryReport	23

6.2.2.3.1	General	23

6.2.2.3.2	Legacy 3GPP Message Gateway Terminating Message Delivery Status Report	23

6.3	MSGG_N3GDelivery Service	24

6.3.1	Service Description	24

6.3.2	Service Operations	24

6.3.2.1	Introduction	24

6.3.2.2	MSGG_N3GDelivery_GTDelivery	24

6.3.2.2.1	General	24

6.3.2.2.2	Non-3GPP Message Gateway Terminating Message Delivery	25

6.3.2.3	MSGG_N3GDelivery_GTDeliveryReport	26

6.3.2.3.1	General	26

6.3.2.3.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report	26

7.	Common information applicable to several APIs	26

7.1	General	26

7.2	Data Types	27

7.2.1	General	27

7.2.2	Referenced structured data types	27

7.2.3	Referenced Simple data types and enumerations	27

7.3	Usage of HTTP	28

7.4	Content type	28

7.5	URI structure	28

7.5.1	Resource URI structure	28

7.5.2	Custom operations URI structure	28

7.6	Notifications	28

7.7	Error Handling	29

7.8	Feature negotiation	29

7.9	HTTP headers	29

7.10	Conventions for Open API specification files	29

8	Message Server API definition	29

8.1	MSGS_ASRegistration API	29

8.1.1	API URI	29

8.1.2	Resources	30

8.1.2.1	Overview	30

8.1.2.2	Resource: AS Registrations	30

8.1.2.2.1	Description	30

8.1.2.2.2	Resource Definition	30

8.1.2.2.3	Resource Standard Methods	31

8.1.2.2.3.1	POST	31

8.1.2.3	Resource: AS DeRegistration	32

8.1.2.3.1	Description	32

8.1.2.3.2	Resource Definition	32

8.1.2.3.3	Resource Standard Methods	33

8.1.2.3.3.1	DELETE	33

8.1.3	Custom Operations without associated resources	34

8.1.4	Notifications	34

8.1.5	Data Model	34

8.1.5.1	General	34

8.1.5.2	Structured data types	35

8.1.5.2.1	Introduction	35

8.1.5.2.2	Type: ASRegistration	35

8.1.5.2.3	Type: ASRegistrationAck	35

8.1.5.2.4	Type: ASProfile	35

8.1.5.3	Simple data types and enumerations	35

8.1.6	Error Handling	36

8.1.7	Feature negotiation	36

8.2	MSGS_MSGDelivery API	36

8.2.1	API URI	36

8.2.2	Resources	36

8.2.3	Custom Operations without associated resources	36

8.2.3.1	Overview	36

8.2.3.2	Operation: deliver-as-message	37

8.2.3.2.1	Description	37

8.2.3.2.2	Operation Definition	37

8.2.3.3	Operation: deliver-ue-message	38

8.2.3.3.1	Description	38

8.2.3.3.2	Operation Definition	38

8.2.3.4	Operation: deliver-report	38

8.2.3.4.1	Description	38

8.2.3.4.2	Operation Definition	38

8.2.4	Notifications	39

8.2.5	Data Model	39

8.2.5.1	General	39

8.2.5.2	Structured data types	41

8.2.5.2.1	Introduction	41

8.2.5.2.2	Type: ASMessageDelivery	41

8.2.5.2.3	Type:UEMessageDelivery	43

8.2.5.2.4	Type: MessageDeliveryAck	44

8.2.5.2.5	Type:MessageSegmentParameters	45

8.2.5.2.6	Type:StoreAndForwardParameters	45

8.2.5.2.7	Type:DeliveryStatusReport	46

8.2.5.3	Simple data types and enumerations	46

8.2.5.3.1	Introduction	46

8.2.5.3.2	Simple data types	46

8.2.5.3.3	Enumeration: DeliveryStatus	46

8.2.5.3.4	Enumeration: ReportDeliveryStatus	47

8.2.5.3.5	Enumeration:Priority	47

8.2.6	Error Handling	47

8.2.7	Feature negotiation	47

9	Message Gateway API definition	48

9.1	MSGG_L3GDelivery API	48

9.1.1	API URI	48

9.1.2	Resources	48

9.1.3	Custom Operations without associated resources	48

9.1.3.1	Overview	48

9.1.3.2	Operation: deliver-message	49

9.1.3.2.1	Description	49

9.1.3.2.2	Operation Definition	49

9.1.3.3	Operation: deliver-report	49

9.1.3.3.1	Description	49

9.1.3.3.2	Operation Definition	49

9.1.4	Notifications	50

9.1.5	Data Model	50

9.1.5.1	General	50

9.1.5.2	Structured data types	51

9.1.5.2.1	Introduction	51

9.1.5.2.2	Type: L3gMessageDelivery	51

9.1.5.2.3	Type: Address	52

9.1.5.3	Simple data types and enumerations	52

9.1.5.3.1	Introduction	52

9.1.5.3.2	Enumeration: AddressType	52

9.1.6	Error Handling	52

9.1.7	Feature negotiation	52

9.2	MSGG_N3GDelivery API	53

9.2.1	API URI	53

9.2.2	Resources	53

9.2.3	Custom Operations without associated resources	53

9.2.3.1	Overview	53

9.2.3.2	Operation: deliver-message	54

9.2.3.2.1	Description	54

9.2.3.2.2	Operation Definition	54

9.2.3.3	Operation: deliver-report	54

9.2.3.3.1	Description	54

9.2.3.3.2	Operation Definition	54

9.2.4	Notifications	55

9.2.5	Data Model	55

9.2.5.1	General	55

9.2.5.2	Structured data types	56

9.2.5.2.1	Introduction	56

9.2.5.2.2	Type: N3gMessageDelivery	56

9.2.6	Error Handling	57

9.2.7	Feature negotiation	57

10	Security	57

11	Using Common API Framework	57

11.1	General	57

11.2	Security	58

Annex A (normative): OpenAPI specification	59

A.1	General	59

A.2	MSGS_ASRegistration API	59

A.3	MSGS_MSGDelivery API	63

A.4	MSGG_L3GDelivery API	71

A.5	MSGG_N3GDelivery API	75

Annex B (informative): Change history	79

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specified the Application Programming Interface (API) for enabling the MSGin5G Service over MSGin5G-2/3/4 interfaces. The application layer architecture, functional requirements, procedures and information flows necessary for MSGin5G Service are contained in 3GPP TS 23.554 [2]. The requirements for MSGin5G are specified in 3GPP TS 22.262 [3].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service".

[3]	3GPP TS 22.262: "Message Service within the 5G System".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[8]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[9]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[10]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[11]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[12]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[13]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[14]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[15]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[16]	IETF RFC 7240: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[17]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[18]	3GPP TR 21.900: "Technical Specification Group working methods".

[19]	3GPP TR 33.862: "Study on security aspects of the Message Service for MIoT over the 5G System (MSGin5G)".

[20]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[21]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[22]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and its definitions given in 3GPP TS 23.554 [2] shall apply:

MSGin5G Service

MSGin5G message

MSGin5G UE

MSGin5G Group

MSGin5G Client

MSGin5G Server

Legacy 3GPP Message Gateway

Non-3GPP Message Gateway

Legacy 3GPP UE

Non-3GPP UE

Non-MSGin5G UE

Point-to-Point messaging

Point-to-Application messaging

Application-to-Point messaging

Group messaging

Broadcast messaging

Messaging Topic

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AS	Application Server

CAPIF	Common API Framework

MSG-C	MSGin5G Client

MSG-S	MSGin5G Server

NIDD	Non IP Data Delivery

SEAL	Service Enabler Architecture Layer for Verticals

4	Overview

The MSGin5G Service is designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication and provides messaging capability in 5GS with messaging communication models including point-to-point, application-to-point, group and broadcast messaging.

3GPP TS 23.554 [2] has specified the application layer architecture, architectural requirements, procedures, information flows and some APIs, in order to support the MSGin5G Service. Various features are defined to ensure the efficient use and deployment of MSGin5G Service, including configuration, registration, message delivery, message aggregation, segmentation and reassembly.

The present document specifies MSGin5G services offered by MSGin5G servers and MSGin5G Gateway, and APIs in detail, needed over MSGin5G-2/3/4 interfaces for interworking between MSGin5G Server and Legacy 3GPP UE, Non-3GPP UE or Application Server, with following functionalities need to be supported:

1.	Server-side functionality with the sending and receiving of messages to/from Application Servers and/or other MSGin5G Service endpoints on other UEs, provided by MSGin5G server.

2.	Interconnecting two different messaging delivery mechanisms and assure the message integrity between different message delivery mechanisms, provided by Message Gateway.

And the definition of APIs specified in TS 23.554 [2] clause 9 is introduced in present document.

5	Services offered by the MSGin5G Servers

5.1	Introduction

The Table 5.1-1 lists the services provided by the MSGin5G server and corresponding service operations. A service description subclause for each API gives a general description of the related API.

Table 5.1-1 List of services provided by the MSGin5G Servers

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| MSGS_ASRegistration | MSGS_ASRegistration_Request | Request/Response | AS |
|  | MSGS_ASRegistration_Deregister |  |  |
| MSGS_MSGDelivery | MSGS_MSGDelivery_ASODelivery | Request/ Response | AS, Legacy 3GPP Message Gateway, Non-3GPP Message Gateway |
|  | MSGS_MSGDelivery_ASODeliveryReport |  |  |
|  | MSGS_MSGDelivery_UEODelivery |  |  |
|  | MSGS_MSGDelivery_UEODeliveryReport |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| MSGS_ASRegistration | 8.1 | AS Registration Service | TS29538_MSGS_ASRegistration.yaml | msgs_asregistration | A.2 |
| MSGS_MSGDelivery | 8.2 | Message Delivery Service | TS29538_MSGS_MSGDelivery.yaml | msgs_msgdelivery | A.3 |

5.2	MSGS_ASRegistration Service

5.2.1	Service Description

The MSGS_ASRegistration API, as defined in 3GPP TS 23.554 [2], allows an AS via Mm5s interface to register, and deregister at a given MSGin5G Server.

5.2.2	Service Operations

5.2.2.1	Introduction

The service operation defined for MSGS_ASRegistration API is shown in the Table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the MSGS_ASRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_ASRegistration_Request | This service operation is used by the AS to register itself to MSGin5G Server. | AS |
| MSGS_ASRegistration_Deregister | This service operation is used by the AS to deregister itself from a MSGin5G Server. | AS |

5.2.2.2	MSGS_ASRegistration_Request

5.2.2.2.1	General

This service operation is used by the AS to register itself to MSGin5G Server.

5.2.2.2.2	Application Server registering to MSGin5G Server using MSGS_ASRegistration_Request operation

To register itself at the MSGin5G Server, the AS shall send an HTTP POST message to the MSGin5G Server on the "AS Registrations" collection resource. The body of the HTTP POST message shall include the AS profile information, may include notification target URI for the registration, as specified in clause 8.1.2.2.3.1.

Upon receiving the HTTP POST message from the AS, the MSGin5G Server shall:

1.	process the AS registration request information;

2.	verify the identity of the AS and check if the AS is authorized to register itself at MSGin5G Server; and

3.	if the AS is authorized to register to MSGin5G Server, then the MSGin5G Server shall:

a.	store the AS registration information and create a new resource with the AS registration information as specified in clause 8.1.2.1; and

b.	return the AS registration information, the resource URI of the AS registration information, in the response message.

5.2.2.3	MSGS_ASRegistration_Deregister

5.2.2.3.1	General

This service operation is used by the AS to deregister itself from a MSGin5G Server.

5.2.2.3.2	Application Server deregistering from MSGin5G Server using MSGS_ASRegistration_Deregister operation

To deregister itself from the MSGin5G Server, the AS shall send HTTP DELETE message to the MSGin5G Server on the "AS DeRegistration" collection resource, as specified in clause 8.1.2.3.3.2.

Upon receiving the HTTP DELETE request, the MSGin5G Server shall:

- verify the identity of the AS and check if the AS is authorized to deregister the AS registration information;

- if the AS is authorized to deregister the AS registration information, then the MSGin5G Server shall deregister the AS profile from the MSGin5G Server and delete the resource representing AS registration information; and

- return the "200 OK" message to the AS, indicating the successful deregistration of the AS information.

5.3	MSGS_MSGDelivery Service

5.3.1	Service Description

The MSGS_MSGDelivery Service corresponding to Mm5s as defined in 3GPP TS 23.554 [2], is provided by the MSGin5G Server.

This service:

-	allows AS invokes services provided by MSGin5G Server to send MSGin5G Messages to MSGin5G Server; and

-	allows L3G/N3G invokes services provided by MSGin5G Server to send MSGin5G Messages to MSGin5G Server on behalf of Legacy 3GPP UE or Non-3GPP UE.

5.3.2	Service Operations

5.3.2.1	Introduction

The service operation defined for MSGS_MSGDelivery Service is shown in the Table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the MSGS_MSGDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_MSGDelivery_ASODelivery | This service operation is used by AS to deliver MSGin5G message to the MSGin5G Server. This service operation corresponds to clause 9.1.1.1.2 as defined in 3GPP TS 23.554 [2]. | AS |
| MSGS_MSGDelivery_ASODeliveryReport | This service operation is used by AS to deliver the delivery status report to the MSGin5G Server. This service operation corresponds to clause 9.1.1.2.2 as defined in 3GPP TS 23.554 [2]. | AS |
| MSGS_MSGDelivery_UEODelivery | This service operation is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver MSGin5G message to the MSGin5G Server. This service operation corresponds to clause 9.1.1.3.2 as defined in 3GPP TS 23.554 [2]. | Legacy 3GPP Message Gateway Non-3GPP Message Gateway |
| MSGS_MSGDelivery_UEODeliveryReport | This service operation is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf Legacy 3GPP UE or Non-3GPP UE) to deliver the delivery status report to the MSGin5G Server. This service operation corresponds to clause 9.2.1.4.2 as defined in 3GPP TS 23.554 [2]. | Legacy 3GPP Message Gateway Non-3GPP Message Gateway |

5.3.2.2	MSGS_MSGDelivery_ASODelivery

5.3.2.2.1	General

This service operation corresponding to clause 9.1.1.1.2 as defined in 3GPP TS 23.554 [2], is used by AS to deliver MSGin5G message to the MSGin5G Server.

5.3.2.2.2	AS Originating MSGin5G Message Delivery

Figure 5.3.2.2.2-1: AS Originating MSGin5G Message Delivery

When the AS needs to send the message to the MSGin5G Server, the AS shall send the HTTP POST method as step 1 of the Figure 5.3.2.2.2-1.

The AS shall include ASMessageDelivery data structure in the payload body of the HTTP POST request.

The ASMessageDelivery data structure shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient Address within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the store and forward flag within the "stoAndFwInd" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the security credentials within the "secCred" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the Payload within the "payload" attribute;

-	the priority type within the "priority" attribute;

-	the message segment flag within the "segInd" attribute;

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute;

-	the store and forward parameters within the "stoAndFwParams" attribute, this attribute may include:

-	the message expiration time within the "exprTime" attribute;

-	The latency within the "latency" attribute.

When the MSGin5G Server receives the HTTP POST request from the AS, the MSGin5G server shall make an authorization based on the information received from the AS. If the authorization is successful, the MSGin5G Server shall respond to the AS with a 200 OK message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

5.3.2.3	MSGS_MSGDelivery_ASODeliveryReport

5.3.2.3.1	General

This service operation corresponds to clause 9.1.1.2.2 as defined in 3GPP TS 23.554 [2], is used by AS to deliver the delivery status report to the MSGin5G Server.

5.3.2.3.2	AS Originating Message Delivery Status Report

Figure 5.3.2.3.2-1: AS Originating MSGin5G Delivery Report

When the AS needs to send the delivery report to the MSGin5G Server, the AS shall send the HTTP POST method as step 1 of the Figure 5.3.2.3.2-1.

The AS shall include DeliveryStatusReport data structure in the payload body of the HTTP POST request.

The DeliveryStatusReport data structure shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID/AS Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the delivery status within the "delivSt" attribute; and

may include:

-	the security credentials within the "secCred" attribute; and

-	the failure cause within the "failureCause" attribute;

When the MSGin5G Server receives the HTTP POST request from the AS, the MSGin5G server shall make an authorization based on the information received from the AS. If the authorization is successful, the MSGin5G Server shall respond to the AS with a 200 OK message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

5.3.2.4	MSGS_MSGDelivery_UEODelivery

5.3.2.4.1	General

This service operation corresponds to clause 9.1.1.3.2 as defined in 3GPP TS 23.554 [2], is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver MSGin5G message to the MSGin5G Server.

5.3.2.4.2	UE Originating Message Delivery

Figure 5.3.2.4.2-1: Legacy 3GPP UE or Non-3GPP UE Originating MSGin5G Message Delivery

When the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) needs to send the message to the MSGin5G Server, the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall send the HTTP POST method as step 1of the Figure 5.3.2.4.2-1.

The Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall include UEMessageDelivery data structure in the payload body of the HTTP POST request.

The UEMessageDelivery data structure shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient Address within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

-	the store and forward flag within the "stoAndFwInd" attribute;

and may include:

-	the Application ID within the "appId" attribute;

-	the security credentials within the "secCred" attribute;

-	the Payload within the "payload" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute; and

-	the message segment flag within the "segInd" attribute;

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute;

-	the last segment flag within the "lastSegFlag" attribute;

-	the store and forward parameters within the "stoAndFwParams" attribute, this attribute may include:

-	the message expiration time within the "exprTime" attribute;

When the MSGin5G Server receives the HTTP POST request from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway, the MSGin5G server shall make an authorization based on the information received from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway. If the authorization is successful, the MSGin5G Server shall respond to the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway with a 200 OK message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

5.3.2.5	MSGS_MSGDelivery_UEODeliveryReport

5.3.2.5.1	General

This service operation corresponds to clause 9.2.1.4.2 as defined in 3GPP TS 23.554 [2], is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf Legacy 3GPP UE or Non-3GPP UE) to deliver the delivery status report to the MSGin5G Server.

5.3.2.5.2	UE Originating Message Delivery Status Report

Figure 5.3.2.5.2-1: Legacy 3GPP UE or Non-3GPP UE Originating MSGin5G Delivery Report

When the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) needs to send the delivery report to the MSGin5G Server, the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall send the HTTP POST method as step 1 of the Figure 5.3.2.5.2-1.

The Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall include DeliveryStatusReport data structure in the payload body of the HTTP POST request.

The DeliveryStatusReport data structure shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID/AS Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;and

-	the delivery status within the "delivSt" attribute;

and may include:

-	the security credentials within the "secCred" attribute;

-	The failure cause within the "failureCause" attribute;

When the MSGin5G Server receives the HTTP POST request from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway, the MSGin5G server shall make an authorization based on the information received from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway. If the authorization is successful,  the MSGin5G Server shall respond to the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway with a 200 OK message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

6	Services offered by the Message Gateway

6.1	Introduction

The Table 6.1-1 lists the services provided by the MSGin5G Message Gateway and corresponding service operations. A service description subclause for each API gives a general description of the related API.

Table 6.1-1 List of services provided by the MSGin5G Message Gateway

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| MSGG_L3GDelivery | MSGG_L3GDelivery_GTDelivery | Request/Response | MSGin5G Server |
|  | MSGG_L3GDelivery_GTDeliveryReport |  |  |
| MSGG_N3GDelivery | MSGG_N3GDelivery_GTDelivery | Request/ Response | MSGin5G Server |
|  | MSGG_N3GDelivery_GTDeliveryReport |  |  |

Table 6.1-2 summarizes the corresponding APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| MSGS_L3GDelivery | 9.1 | L3G Message Delivery Service | TS29538_MSGG_ L3GDelivery.yaml | msgg-l3gdelivery | A.4 |
| MSGS_N3GDelivery | 9.2 | N3G Message Delivery Service | TS29538_MSGG_ N3GDelivery.yaml | msgg-n3gdelivery | A.5 |

6.2	MSGG_L3GDelivery Service

6.2.1	Service Description

The MSGG_L3GDelivery Service corresponding to Ml3g as defined in 3GPP TS 23.554 [2], is provided by the Legacy 3GPP Message Gateway.

This service:

-	allows MSGin5G Server invokes services provided by Legacy 3GPP Message Gateway to send MSGin5G Messages to Legacy 3GPP Message Gateway.

6.2.2	Service Operations

6.2.2.1	Introduction

The service operation defined for MSGG_L3GDelivery Service is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the MSGG_L3GDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGG_L3GDelivery_GTDelivery | This service operation is used by MSGin5G Server to deliver MSGin5G message to Legacy 3GPP Message Gateway. This service operation corresponds to clause 9.2.1.1.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |
| MSGG_L3GDelivery_GTDeliveryReport | This service operation is used by MSGin5G Server to deliver the delivery status report to Legacy 3GPP Message Gateway. This service operation corresponds to clause 9.2.1.3.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |

6.2.2.2	MSGG_L3GDelivery_GTDelivery

6.2.2.2.1	General

This service operation corresponds to clause 9.2.1.1.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver MSGin5G message to the Legacy 3GPP Message Gateway.

6.2.2.2.2	Legacy 3GPP Message Gateway Terminating Message Delivery

Figure 6.2.2.2.2-1: Legacy 3GPP Message Gateway Terminating Message Delivery

When the MSGin5G Server needs to send the message to the Legacy 3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-message" resource as shown in Figure 6.2.2.2.2-1.

The MSGin5G Server shall send a POST request to the resource with an L3gMessageDelivery object in the request body.

The L3gMessageDelivery data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID/AS Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the Payload within the "payload" attribute;

-	the Message is segmented within the "segInd" attribute; and

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute.

When the Legacy 3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Legacy 3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If errors occur when processing the HTTP POST request, the Legacy 3GPP Message Gateway shall apply error handling procedures as specified in subclause 9.1.6.

6.2.2.3	MSGG_L3GDelivery_GTDeliveryReport

6.2.2.3.1	General

This service operation corresponds to clause 9.2.1.3.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver the delivery status report to the Legacy 3GPP Message Gateway.

6.2.2.3.2	Legacy 3GPP Message Gateway Terminating Message Delivery Status Report

Figure 6.2.2.3.2-1: Legacy 3GPP Message Gateway Terminating Delivery Status Report

When the MSGin5G Server needs to send the delivery status report to the Legacy 3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-report" resource as shown in Figure 6.2.2.3.2-1.

The MSGin5G Server shall send a POST request to the resource with a DeliveryStatusReport object in the request body.

The DeliveryStatusReport data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID/AS Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the delivery status within the "delivSt" attribute; and

may include:

-	the failure cause within the "failureCause" attribute.

When the Legacy 3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Legacy 3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If errors occur when processing the HTTP POST request, the Legacy 3GPP Message Gateway shall apply error handling procedures as specified in subclause 9.1.6.

6.3	MSGG_N3GDelivery Service

6.3.1	Service Description

The MSGG_N3GDelivery Service corresponding to Mn3g as defined in 3GPP TS 23.554 [2], is provided by the Non-3GPP Message Gateway.

This service:

-	allows MSGin5G Server invokes services provided by Non-3GPP Message Gateway to send MSGin5G Messages to Non-3GPP Message Gateway.

6.3.2	Service Operations

6.3.2.1	Introduction

The service operation defined for MSGG_N3GDelivery Service is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations of the MSGG_N3GDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGG_N3GDelivery_GTDelivery | This service operation is used by MSGin5G Server to deliver MSGin5G message to Non-3GPP Message Gateway. This service operation corresponds to clause 9.2.2.1.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |
| MSGG_N3GDelivery_GTDeliveryReport | This service operation is used by MSGin5G Server to deliver the delivery status report to Non-3GPP Message Gateway. This service operation corresponds to clause 9.2.2.2.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |

6.3.2.2	MSGG_N3GDelivery_GTDelivery

6.3.2.2.1	General

This service operation corresponds to clause 9.2.2.1.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver MSGin5G message to the Non-3GPP Message Gateway.

6.3.2.2.2	Non-3GPP Message Gateway Terminating Message Delivery

Figure 6.3.2.2.2-1: Non-3GPP Message Gateway Terminating Message Delivery

When the MSGin5G Server needs to send the message to the Non-3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-message" resource as shown in figure 6.3.2.2.2-1.

The MSGin5G Server shall send a POST request to the resource with an N3gMessageDelivery object in the request body.

The N3gMessageDelivery data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID/AS Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the Payload within the "payload" attribute;

-	the Message is segmented within the "segInd" attribute; and

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute.

When the Non-3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Non-3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If errors occur when processing the HTTP POST request, the Non-3GPP Message Gateway shall apply error handling procedures as specified in subclause 9.2.6.

6.3.2.3	MSGG_N3GDelivery_GTDeliveryReport

6.3.2.3.1	General

This service operation corresponds to clause 9.2.2.2.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver the delivery status report to the Non-3GPP Message Gateway.

6.3.2.3.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report

Figure 6.3.2.3.2-1: Non-3GPP Message Gateway Terminating Delivery Status Report

When the MSGin5G Server needs to send the delivery status report to the Non-3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-report" resource as shown in figure 6.3.2.3.2-1.

The MSGin5G Server shall send a POST request to the resource with a DeliveryStatusReport object in the request body.

The DeliveryStatusReport data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the delivery status within the "delivSt" attribute; and

may include:

-	the failure cause within the "failureCause" attribute.

When the Non-3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Non-3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If errors occur when processing the HTTP POST request, the Non-3GPP Message Gateway shall apply error handling procedures as specified in subclause 9.2.6.

7.	Common information applicable to several APIs

7.1	General

MSGin5G APIs allow secure access to the capabilities provided by the MSGin5G.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.554 [2].

Several design aspects, as mentioned in the following clauses, are specified in 3GPP TS 29.500 [4] and referenced by this specification.

7.2	Data Types

7.2.1	General

This subclause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent subclauses.

In addition, data types that are defined in OpenAPI Specification [6] can also be referenced from data structures defined in the subsequent subclauses.

NOTE:	As a convention, data types in the present specification are written with an upper-case letter in the beginning. Parameters are written with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI Specification [6] can use a lower-case case letter in the beginning for consistency.

Table 7.2.1-1 specifies data types re-used by the MSGin5G from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the APIs of this specification.

Table 7.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |

7.2.2	Referenced structured data types

Table 7.2.2-1 lists structured data types defined in this specification referenced by multiple services.

Table 7.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| MessageSegmentParameters | Subclause 8.2.5.2.5 | Parameters for message segmentation |
| Address | Subclause 9.1.5.2.3 | Represent an address |

7.2.3	Referenced Simple data types and enumerations

Following simple data types defined in Table 7.2.3.1-1 are applicable to several APIs in this document:

Table 7.2.3.1-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |

7.3	Usage of HTTP

For MSGin5G APIs, support of HTTP/1.1 (IETF RFC 7230 [10], IETF RFC 7231 [11], IETF RFC 7232 [12], IETF RFC 7233 [13], IETF RFC 7234 [14] and IETF RFC 7235 [15]) over TLS is mandatory and support of HTTP/2 (IETF RFC 7540 [16]) over TLS is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 7540 [16].

7.4	Content type

JSON, IETF RFC 8259 [17], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

7.5	URI structure

7.5.1	Resource URI structure

The resource URI structure of all the APIs specified in this document shall be as specified in clause 5.2.4 of 3GPP TS 29.501 [9].

7.5.2	Custom operations URI structure

The custom operation definition is in Annex C of 3GPP TS 29.501 [9].

The URI of a custom operation which is associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>/<custOpName>

Custom operations can also be associated with the service instead of a resource. The URI of a custom operation which is not associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<custOpName>

In the above URI structures, "apiRoot", "apiName", "apiVersion" and "apiSpecificResourceUriPart" are as defined in subclause 7.5.1 and "custOpName" represents the name of the custom operation as defined in subclause 5.1.3.2 of 3GPP TS 29.501 [9].

7.6	Notifications

None.

7.7	Error Handling

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

7.8	Feature negotiation

The procedures in subclause 6.6.2 of 3GPP TS 29.500 [4] shall be applicable for the APIs defined in the present specification. For each of the APIs defined, the applicable list of features is contained in the related API definition.

7.9	HTTP headers

The MSGin5G API shall support mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] and may support HTTP custom header fields specified in subclause 5.2.3.3 of 3GPP TS 29.500 [4]. No specific custom headers are defined for the MSGin5G API in the present specification.

7.10	Conventions for Open API specification files

The conventions for Open API specification files as specified in subclause 5.3 of 3GPP TS 29.501 [9] shall be applicable for all APIs in this document.

8	Message Server API definition

8.1	MSGS_ASRegistration API

8.1.1	API URI

The MSGS_ASRegistration service shall use the MSGS_ASRegistration API.

The request URIs used in HTTP requests from the AS towards the MSGin5G Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs-asregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.1.2.

8.1.2	Resources

8.1.2.1	Overview

Figure 8.1.2.1-1: Resource URI structure of the MSGS_ASRegistration API

Table 8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AS Registrations | /registrations | POST | Registers a new AS at the MSGin5G Server. |
| AS DeRegistration | /registrations/{registrationId} | DELETE | Removes an AS registration resource. |

8.1.2.2	Resource: AS Registrations

8.1.2.2.1	Description

This resource represents all the Application Servers that are registered at a given MSGin5G Server.

8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/msgs-asregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 8.1.2.2.2-1.

Table 8.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |

8.1.2.2.3	Resource Standard Methods

8.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.1.2.2.3.1-1.

Table 8.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ASRegistration | M | 1 | AS registration request information. |

Table 8.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ASRegistrationAck | M | 1 | 201 Created | AS information is registered successfully at MSGin5G Server. The URI of the created resource shall be returned in the “Location” HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 8.1.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/msgs-asregistration/<apiVersion>/registrations/{registrationId} |

Table 8.1.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.1.2.3	Resource: AS DeRegistration

8.1.2.3.1	Description

This resource represents all the Application Servers that are deregistered at a given MSGin5G Server.

8.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/msgs-asregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 8.1.2.3.2-1.

Table 8.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |
| registrationId | string | The AS registration resource id |

8.1.2.3.3	Resource Standard Methods

8.1.2.3.3.1	DELETE

This method deregisters an AS registration from the MSGin5G Server. This method shall support the URI query parameters specified in the table 8.1.2.3.3.1-1.

Table 8.1.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.1-2 and the response data structures and response codes specified in table 8.1.2.3.3.1-3.

Table 8.1.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ASRegistrationAck | M | 1 | 200 OK | The AS DeRegistration information matching the registrationId is deleted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 8.1.2.3.3.1-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.1-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.1.3	Custom Operations without associated resources

None.

8.1.4	Notifications

None.

8.1.5	Data Model

8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API Table 8.1.5.1-1 specifies the data types defined specifically for the MSGS_ASRegistration API service.

Table 8.1.5.1-1: MSGS_ASRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ASRegistration | 8.1.5.2.2 | The AS registration request information. |  |
| ASRegistrationAck | 8.1.5.2.3 | The AS registration response information. |  |
| ASProfile | 8.1.5.2.4 | The profile information related to the AS in the ASRegistration data type. |  |

Table 8.1.5.1-2 specifies data types re-used by the MSGS_ASRegistration API service.

Table 8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [5] |  |  |
| Uri | 3GPP TS 29.571 [5] |  |  |

8.1.5.2	Structured data types

8.1.5.2.1	Introduction

8.1.5.2.2	Type: ASRegistration

Table 8.1.5.2.2-1: Definition of type ASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asSvcId | string | M | 1 | The MSGin5G identifier of the Application Server. |  |
| appId | string | M | 1 | The identifier of the application specified by the application provider |  |
| targetUri | Uri | O | 0..1 | The URL for receiving message, message delivery status report, etc. The MSGin5G Server uses this URL to interact to AS. |  |
| asProf | ASProfile | O | 0..1 | The profile information of the AS. |  |

8.1.5.2.3	Type: ASRegistrationAck

Table 8.1.5.2.3-1: Definition of type ASRegistrationAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asSvcId | string | M | 1 | The MSGin5G identifier of the Application Server. |  |
| result | ProblemDetails | M | 1 | The result of the registration. |  |

8.1.5.2.4	Type: ASProfile

Table 8.1.5.2.4-1: Definition of type ASProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appName | string | O | 1 | The name of the Application Server |  |
| appProviders | array(string) | O | 1..N | The provider of the Application Server |  |
| appSenarios | array(string) | O | 1..N | The application scenario description. |  |
| appCategory | string | O | 0..1 | The category or type of Application Server. |  |
| asStatus | string | O | 0..1 | AS status (e.g. Enabled, Disabled etc.) |  |

8.1.5.3	Simple data types and enumerations

None.

8.1.6	Error Handling

General error responses will be defined in clause 7.7.

8.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.1.7-1 lists the supported features for AS_Registration API.

Table 8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

8.2	MSGS_MSGDelivery API

8.2.1	API URI

The MSGS_MSGDelivery service shall use the MSGS_MSGDelivery API, The MSGS_MSGDelivery API corresponding to Mm5s APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server towards the MSGin5G Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs-msgdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.2.2.

8.2.2	Resources

None.

8.2.3	Custom Operations without associated resources

8.2.3.1	Overview

The structure of the custom operation URIs of the MSGS_MSGDelivery service is shown in Figure 8.2.3.1-1.

Figure 8.2.3.1-1: Custom operation URI structure of the MSGS_MSGDelivery API

Table 8.2.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 8.2.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgs-msgdelivery/<apiVersion>/deliver-as-message | POST | Request of AS to deliver message to a given MSGin5G Server. |
| {apiRoot}/msgs-msgdelivery/<apiVersion>/deliver-ue-message |  | POST |  | Request of Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver message to a given MSGin5G Server. |  |
| {apiRoot}/msgs-msgdelivery/<apiVersion>/deliver-report | POST | Request of Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver status report to a given MSGin5G Server. |

8.2.3.2	Operation: deliver-as-message

8.2.3.2.1	Description

This operation is used by the Application Server to deliver message to a given MSGin5G Server.

8.2.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 8.2.3.2.2-1 and Table 8.2.3.2.2-2.

Table 8.2.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ASMessageDelivery | M | 1 | Represents the data to be used for AS to deliver message. |

Table 8.2.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 200 OK | AS Message is delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

8.2.3.3	Operation: deliver-ue-message

8.2.3.3.1	Description

This operation is used by the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver message to a given MSGin5G Server.

8.2.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 8.2.3.3.2-1 and Table 8.2.3.3.2-2.

Table 8.2.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UEMessageDelivery | M | 1 | Represents the data to be used for Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver message. |

Table 8.2.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 200 OK | UE Message is delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

8.2.3.4	Operation: deliver-report

8.2.3.4.1	Description

This operation is used by the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server to deliver status report to a given MSGin5G Server.

8.2.3.4.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 8.2.3.4.2-1 and Table 8.2.3.4.2-2.

Table 8.2.3.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Represents the data to be used for Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server to deliver status report. |

Table 8.2.3.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 200 OK | The status report is delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

8.2.4	Notifications

None.

8.2.5	Data Model

8.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 8.2.5.1-1 specifies the data types defined specifically for the MSGS_MSGDelivery API service.

Table 8.2.5.1-1:  API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ASMessageDelivery | 8.2.5.2.2 | The AS message delivery request information. |  |
| ReportDeliveryStatus | 8.2.5.3.4 | The delivery status description, including success or failure in delivery. |  |
| DeliveryStatusReport | 8.2.5.2.7 | The message delivery status report request information. |  |
| MessageDeliveryAck | 8.2.5.2.4 | The message delivery response information. |  |
| MessageSegmentParameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |
| Priority | 8.2.5.3.5 | Application priority level requested for this message. |  |
| StoreAndForwardParameters | 8.2.5.2.6 | Contains the store forward information of the message. |  |
| DeliveryStatus | 8.2.5.3.3 | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |  |
| UEMessageDelivery | 8.2.5.2.3 | The UE message delivery request information. |  |

Table 8.2.5.1-2 specifies data types re-used by the MSGS_MSGDelivery API service.

Table 8.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Address | 9.1.5.2.3 | The data type of the oriAddr and destAddr. |  |
| DateTime | 3GPP TS 29.571 [5] | String with format "date-time" as defined in OpenAPI Specification [6]. |  |

8.2.5.2	Structured data types

8.2.5.2.1	Introduction

8.2.5.2.2	Type: ASMessageDelivery

Table 8.2.5.2.2-1: Definition of type ASMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Application Server (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving Legacy 3GPP UE, Non-3GPP UE or MSGin5G UE. The service identifier of the target MSGin5G Group. The service identifier of the Broadcast Service Area where the message needs to be broadcast. Indicates which Messaging Topic this message is related to. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| secCred | string | O | 0..1 | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| priority | Priority | O | 0..1 | Application priority level requested for this message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | O | 0..1 | The message segment parameters. This IE shall be included only if the value of the message Segment Flag IE indicates that message Segment services are requested. |  |
| stoAndFwInd | boolean | M | 1 | An indicator of whether store and forward services are requested for this message. Set to "true" if it is required to store and forward services for this message. otherwise set to "false". Default value is "false". |  |
| stoAndFwParams | StoreAndForwardParameters | O | 0..1 | Parameters used by MSGin5G Server for providing store and forward services, This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. |  |
| latency | integer | O | 0..1 | The latency requirement for the message which only applies to AS Originating MSGin5G Message. Unit: millisecond. |  |
| NOTE: Only "AS" is applicable to the addrType attribute in the Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.2.3	Type:UEMessageDelivery

Table 8.2.5.2.3-1: Definition of type UEMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Legacy 3GPP UE or Non-3GPP UE (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving Application Server or MSGin5G UE. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| secCred | string | O | 0..1 | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | O | 0..1 | The message segment parameters. This IE shall be included only if the value of the message Segment Flag IE indicates that message Segment services are requested. |  |
| stoAndFwInd | boolean | M | 1 | An indicator of whether store and forward services are requested for this message. Set to "true" if it is required to store and forward services for this message. otherwise set to "false". Default value is "false". |  |
| stoAndFwParams | StoreAndForwardParameters | O | 0..1 | Parameters used by MSGin5G Server for providing store and forward services, This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. |  |
| NOTE: Only "UE" is applicable to the addrType attribute in the Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.2.4	Type: MessageDeliveryAck

Table 8.2.5.2.4-1: Definition of type MessageDeliveryAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Legacy 3GPP UE, Non-3GPP UE or the sending Application Server (NOTE). |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| status | DeliveryStatus | O | 0..1 | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |  |
| failureCause | string | C | 0..1 | The reason for failure. May only be present if the DeliveryStatus sets to “DELY_FAILED”. |  |
| NOTE: Either "UE" or "AS" shall be included in the addrType attribute in the Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.2.5	Type:MessageSegmentParameters

Table 8.2.5.2.5-1: Definition of type MessageSegmentParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| segId | string | O | 0..1 | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. |  |
| totalSegCount | integer | O | 0..1 | Indicates the total number of segments for the message. |  |
| segNumb | integer | O | 0..1 | Indicates segmented message number of each segmented message within a set of segmented messages. |  |
| lastSegFlag | boolean | O | 0..1 | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. Set to "true" if the segmented message is the last segment in the set of segmented messages. otherwise set to "false". Default value is "false". |  |

8.2.5.2.6	Type:StoreAndForwardParameters

Table 8.2.5.2.6-1: Definition of type StoreAndForwardParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exprTime | DateTime | O | 0..1 | Indicates message expiration time. |  |

8.2.5.2.7	Type:DeliveryStatusReport

Table 8.2.5.2.5-1: Definition of type DeliveryStatusReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Legacy 3GPP UE, Non-3GPP UE or the sending Application Server (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server (NOTE). |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| secCred | string | O | 0..1 | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |  |
| failureCause | string | C | 0..1 | The Failure Cause indicates the failure reason, if applicable. May only be present if the ReportDeliveryStatus sets to “REPT_DELY_FAILED”. |  |
| delivSt | ReportDeliveryStatus | M | 1 | The delivery status description, including success or failure in delivery. |  |
| NOTE: Either "UE" or "AS" shall be included in the addrType attribute in Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.3	Simple data types and enumerations

8.2.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

8.2.5.3.2	Simple data types

None.

8.2.5.3.3	Enumeration: DeliveryStatus

Table 8.2.5.3.3-1: Enumeration DeliveryStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DELY_FAILED | Indicates that the message delivery is failed. |  |
| DELY_STORED | Indicates that the message is stored for deferred delivery. |  |

8.2.5.3.4	Enumeration: ReportDeliveryStatus

Table 8.2.5.3.4-1: Enumeration ReportDeliveryStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REPT_DELY_SUCCESS | Indicates that the report delivery is successful. |  |
| REPT_DELY_FAILED | Indicates that the report delivery is failed. |  |

8.2.5.3.5	Enumeration:Priority

Table 8.2.5.3.5-1: Enumeration Priority

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HIGH | Indicates the messages should be sent in high priority. |  |
| MIDDLE | Indicates the messages should be sent in middle priority. |  |
| LOW | Indicates the messages should be sent in low priority. |  |

8.2.6	Error Handling

General error responses will be defined in clause 7.7.

8.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.2.7-1 lists the supported features for MSGS_MSGDelivery API.

Table 8.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

9	Message Gateway API definition

9.1	MSGG_L3GDelivery API

9.1.1	API URI

The MSGG_L3GDelivery service shall use the MSGG_L3GDelivery API, The MSGG_L3GDelivery API corresponding to Ml3g APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the MSGin5G Server towards the Legacy 3GPP Message Gateway shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgg-l3gdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.1.3.

9.1.2	Resources

None.

9.1.3	Custom Operations without associated resources

9.1.3.1	Overview

The structure of the custom operation URIs of the MSGG_L3GDelivery service is shown in Figure 9.1.3.1-1.

Figure 9.1.3.1-1: Custom operation URI structure of the MSGG_L3GDelivery API

Table 9.1.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 9.1.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgg-l3gdelivery/<apiVersion>/deliver-message | POST | Request of MSGin5G Server to deliver message to a given Legacy 3GPP Message Gateway. |
| {apiRoot}/msgg-l3gdelivery/<apiVersion>/deliver-report | POST | Request of MSGin5G Server to deliver status report to a given Legacy 3GPP Message Gateway. |

9.1.3.2	Operation: deliver-message

9.1.3.2.1	Description

This operation is used by the MSGin5G Server to deliver message to a given Legacy 3GPP Message Gateway.

9.1.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.1.3.2.2-1 and table 9.1.3.2.2-2.

Table 9.1.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| L3gMessageDelivery | M | 1 | Represents the data to be used for MSGin5G Server to deliver message. |

Table 9.1.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Message is Delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

9.1.3.3	Operation: deliver-report

9.1.3.3.1	Description

This operation is used by the MSGin5G Server to deliver status report to a given Legacy 3GPP Message Gateway.

9.1.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 9.1.3.3.2-1 and Table 9.1.3.3.2-2.

Table 9.1.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Represents the data to be used for MSGin5G Server to deliver status report. |

Table 9.1.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The status report is Delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

9.1.4	Notifications

None.

9.1.5	Data Model

9.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 9.1.5.1-1 specifies the data types defined specifically for the MSGG_L3GDelivery API service.

Table 9.1.5.1-1: MSGG_L3GDelivery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| L3gMessageDelivery | 9.1.5.2.2 | Information within message delivery request. |  |

Table 9.1.5.1-2 specifies data types re-used by the MSGG_L3GDelivery API service.

Table 9.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| MessageSegmentPapameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |

9.1.5.2	Structured data types

9.1.5.2.1	Introduction

9.1.5.2.2	Type: L3gMessageDelivery

Table 9.1.5.2.2-1: Definition of type L3gMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the originating MSGin5G Client or the originating Application Server. This IE is copied from the associated inbound message (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving entity. The receiving entity can only be Legacy 3GPP UE Service ID in MSGG_L3GDelivery API. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. This IE is copied from the associated inbound message request |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. This IE is copied from the associated inbound message. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. This IE is copied from the associated inbound message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | O | 0..1 | The message segment parameters. This IE shall be included only if the value of segInd is true to indicate that message Segment services are requested. |  |
| NOTE: The addrType in Address data type shall only include AS or UE to represent the originating of message request. |  |  |  |  |  |

9.1.5.2.3	Type: Address

Table 9.1.5.2.3-1: Definition of type Address

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| addrType | AddressType | M | 1 | Represent the type of message request. |  |
| addr | string | M | 1 | Refer to UE Service ID or AS Service ID or Group Service ID or Broadcast Area ID or Messaging Topic. |  |

9.1.5.3	Simple data types and enumerations

9.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

9.1.5.3.2	Enumeration: AddressType

Table 9.1.5.3.2-1: Enumeration AddressType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE | The address type is UE. |  |
| AS | The address type is AS. |  |
| GROUP | The address type is GROUP. |  |
| BC | The address type is BC. |  |
| TOPIC | The address type is TOPIC. |  |

9.1.6	Error Handling

General error responses will be defined in clause 7.7.

9.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.1.7-1 lists the supported features for MSGG_L3GDelivery API.

Table 9.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

9.2	MSGG_N3GDelivery API

9.2.1	API URI

The MSGG_N3GDelivery service shall use the MSGG_N3GDelivery API, The MSGG_N3GDelivery API corresponding to Mn3g APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the MSGin5G Server towards the Non-3GPP Message Gateway shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgg-n3gdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.2.3.

9.2.2	Resources

None.

9.2.3	Custom Operations without associated resources

9.2.3.1	Overview

The structure of the custom operation URIs of the MSGG_N3GDelivery service is shown in Figure 9.2.3.1-1.

Figure 9.2.3.1-1: Custom operation URI structure of the MSGG_N3GDelivery API

Table 9.2.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 9.2.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgg-n3gdelivery/<apiVersion>/deliver-message | POST | Request of MSGin5G Server to deliver message to a given Non-3GPP Message Gateway |
| {apiRoot}/msgg-n3gdelivery/<apiVersion>/deliver-report | POST | Request of MSGin5G Server to deliver status report to a given Non-3GPP Message Gateway |

9.2.3.2	Operation: deliver-message

9.2.3.2.1	Description

This operation is used by the MSGin5G Server to deliver message to a given Non-3GPP Message Gateway.

9.2.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.2.3.2.2-1 and table 9.2.3.2.2-2.

Table 9.2.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N3gMessageDelivery | M | 1 | Represents the data to be used for MSGin5G Server to deliver message. |

Table 9.2.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Message is Delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

9.2.3.3	Operation: deliver-report

9.2.3.3.1	Description

This operation is used by the MSGin5G Server to deliver status report to a given Non-3GPP Message Gateway.

9.2.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.2.3.3.2-1 and table 9.2.3.3.2-2.

Table 9.2.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Represents the data to be used for MSGin5G Server to deliver status report. |

Table 9.2.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The status report is Delivered successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

9.2.4	Notifications

None.

9.2.5	Data Model

9.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 9.2.5.1-1 specifies the data types defined specifically for the MSGG_N3GDelivery API service.

Table 9.2.5.1-1: MSGG_N3GDelivery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| N3gMessageDelivery | 9.2.5.2.2 | Information within message delivery request. |  |

Table 9.2.5.1-2 specifies data types re-used by the MSGG_N3GDelivery API service.

Table 9.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| MessageSegmentPapameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |
| Address | 9.1.5.2.3 | The data type of the oriAddr and destAddr. |  |

9.2.5.2	Structured data types

9.2.5.2.1	Introduction

9.2.5.2.2	Type: N3gMessageDelivery

Table 9.2.5.2.2-1: Definition of type N3gMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The oriAddr is the service identity of the originating MSGin5G Client or the originating Application Server. This IE is copied from the associated inbound message (NOTE). |  |
| destAddr | Address | M | 1 | The destAddr is the service identity of the receiving entity. The receiving entity can only be Non-3GPP UE Service ID in MSGG_N3GDelivery API. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. This IE is copied from the associated inbound message request. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. This IE is copied from the associated inbound message. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. This IE is copied from the associated inbound message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | O | 0..1 | The message segment parameters. This IE shall be included only if the value of segInd is true to indicate that message Segment services are requested. |  |
| NOTE: The addrType in Address data type shall only include AS or UE to represent the originating of message request. |  |  |  |  |  |

9.2.6	Error Handling

General error responses will be defined in clause 7.7.

9.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.2.7-1 lists the supported features for MSGG_N3GDelivery API.

Table 9.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

10	Security

TLS shall be used to support the security communication between the MSGin5G server and the application server over MSGin5G-3 interface, and also between MSGin5G server and the Message Gateway over MSGin5G-2 or MSGin5G-4 interface as specified in 3GPP TS 33.862 [19] and 3GPP TS 33.501 [20]. The access to the MSGin5G service APIs shall be authorized by means of OAuth2 protocol (see IETF RFC 6749 [21]), based on local configuration, using the "Client Credentials" authorization grant. If OAuth2 is used, a client, prior to consuming services offered by the MSGin5G service APIs, shall obtain a "token" from the authorization server.

11	Using Common API Framework

11.1	General

When CAPIF is used with a MSGin5G service, the MSGin5G server shall support the following as defined in 3GPP TS 29.222 [8]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and related APIs over CAPIF-4/4e reference point;

-	the API management function and related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [7], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a MSGin5G service, the MSGin5G server shall register all the features for northbound APIs in the CAPIF Core Function.

11.2	Security

When CAPIF is used for external exposure, before invoking the API exposed by the MSGin5G server, the NF service consumer (e.g. the application server) as API invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with CAPIF core function and ensure the MSGin5G server has enough credential to authenticate the NF service consumer (e.g. the application server), see 3GPP TS 29.222 [8], clause 5.6.2.2 and clause 6.2.2.2.

If PKI or TLS-PSK is used as the selected security method between the NF service consumer (e.g. the application server) and the MSGin5G server, upon API invocation, the MSGin5G server shall retrieve the authorization information from the CAPIF core function as described in 3GPP TS 29.222 [8], clause 5.6.2.4.

As indicated in 3GPP TS 33.122 [22], the access to the MSGin5G APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [21]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [8]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth2 is used as the selected security method between the NF service consumer (e.g. the application server) and the MSGin5G server, the the NF service consumer (e.g. the application server), prior to consuming services offered by the MSGin5G APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in 3GPP TS 29.222 [8], clause 5.6.2.3.2.

The MSGin5G APIs do not define any scopes for OAuth2 authorization. It is the MSGin5G server responsibility to check whether the NF service consumer (e.g. the application server) is authorized to use an API based on the "token". Once the MSGin5G server verifies the "token", it shall check whether the MSGin5G server identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the NF service consumer (e.g. the application server) has full authority to access any resource or operation for the invoked API.

NOTE 2:	For aforementioned security methods, the MSGin5G server needs to apply admission control according to access control policies after performing the authorization checks.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex is based on the OpenAPI Specification [6] and provides corresponding representations of all APIs defined in the present specification.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository, that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [18] and clause 5.3.1 of the 3GPP TS 29.501 [9] for further information).

A.2	MSGS_ASRegistration API

openapi: 3.0.0

info:

title: MSGS_ASRegistration

version: 1.0.0-alpha.1

description: |

API for MSGS AS Registration Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V0.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgs-asregistration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- msgs-asregistration

paths:

/registrations:

post:

summary: Registers a new AS at a MSGin5G Server

tags:

- AS registration

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ASRegistration'

responses:

'201':

description: AS information is registered successfully at MSGin5G Server

content:

application/json:

schema:

$ref: '#/components/schemas/ASRegistrationAck'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

delete:

summary: Delete an existing AS registration at MSGin5G Server

tags:

- AS DeRegistration

parameters:

- name: registrationId

in: path

description: AS registration Id

required: true

schema:

type: string

responses:

'200':

description: The individual AS registration is deleted successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ASRegistrationAck'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

msgs-asregistration: Access to the as registration API

schemas:

#

# STRUCTURED DATA TYPES

#

ASRegistration:

description: AS registration data

type: object

required:

- asSvcId

- appId

properties:

asSvcId:

type: string

appId:

type: string

targetUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

asProf:

$ref: '#/components/schemas/ASProfile'

ASRegistrationAck:

description: AS registration response data

type: object

required:

- asSvcId

- result

properties:

asSvcId:

type: string

result:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

ASProfile:

description: AS profile information

type: object

properties:

appName:

type: string

appProviders:

type: array

items:

type: string

minItems: 1

description: The provider of the AS.

appSenarios:

type: array

items:

type: string

minItems: 1

description: The application scenario.

appCategory:

type: string

asStatus:

type: string

A.3	MSGS_MSGDelivery API

openapi: 3.0.0

info:

title: MSGS_MSGDelivery

version: 1.0.0-alpha.1

description: |

API for MSGG MSGin5G Server Message Delivery Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V0.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgs-msgdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- msgs-msgdelivery

paths:

/deliver-as-message:

post:

summary: AS deliver message to MSGin5G Server

tags:

- AS Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ASMessageDelivery'

responses:

'200':

description: OK, AS Message delivery successful

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliveryAck'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/deliver-ue-message:

post:

summary: UE deliver message to MSGin5G Server

tags:

- UE Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UEMessageDelivery'

responses:

'200':

description: OK, UE Message delivery successful

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliveryAck'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/deliver-report:

post:

summary: AS or UE deliver status report to MSGin5G Server

tags:

- AS/UE status report delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DeliveryStatusReport'

responses:

'200':

description: OK, status report delivery successfully

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliveryAck'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

msgs-msgdelivery: Access to the MSGS_MSGDelivery API

schemas:

#

# STRUCTURED DATA TYPES

#

ASMessageDelivery:

description: Contains the AS message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

- stoAndFwInd

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

secCred:

type: string

delivStReqInd:

type: boolean

payload:

type: string

priority:

$ref: '#/components/schemas/Priority'

segInd:

type: boolean

segParams:

$ref: '#/components/schemas/MessageSegmentParameters'

stoAndFwInd:

type: boolean

stoAndFwParams:

$ref: '#/components/schemas/StoreAndForwardParameters'

latency:

type: integer

UEMessageDelivery:

description: Contains the UE message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

- stoAndFwInd

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

secCred:

type: string

delivStReqInd:

type: boolean

payload:

type: string

segInd:

type: boolean

segParams:

$ref: '#/components/schemas/MessageSegmentParameters'

stoAndFwInd:

type: boolean

stoAndFwParams:

$ref: '#/components/schemas/StoreAndForwardParameters'

MessageDeliveryAck:

description: Contains the message delivery ack data

type: object

required:

- oriAddr

- msgId

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

msgId:

type: string

status:

$ref: '#/components/schemas/DeliveryStatus'

failureCause:

type: string

MessageSegmentParameters:

description: Contains the message segment parameters data

type: object

properties:

segId:

type: string

totalSegCount:

type: integer

segNumb:

type: integer

lastSegFlag:

type: boolean

StoreAndForwardParameters:

description: Contains the store and forward parameters data

type: object

properties:

exprTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

DeliveryStatusReport:

description: Contains the delivery status report data

type: object

required:

- oriAddr

- destAddr

- msgId

- delivSt

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

msgId:

type: string

secCred:

type: string

failureCause:

type: string

delivSt:

$ref: '#/components/schemas/ReportDeliveryStatus'

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

DeliveryStatus:

anyOf:

- type: string

enum:

- DELY_FAILED

- DELY_STORED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- DELY_FAILED: Indicates that the message delivery is failed.

- DELY_STORED: Indicates that the message is stored for deferred delivery.

ReportDeliveryStatus:

anyOf:

- type: string

enum:

- REPT_DELY_SUCCESS

- REPT_DELY_FAILED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- REPT_DELY_SUCCESS: Indicates that the report delivery is successful.

- REPT_DELY_FAILED: Indicates that the report delivery is failed.

Priority:

anyOf:

- type: string

enum:

- HIGH

- MIDDLE

- LOW

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- HIGH: Indicates the messages should be sent in high priority.

- MIDDLE: Indicates the messages should be sent in middle priority.

- LOW: Indicates the messages should be sent in low priority.

A.4	MSGG_L3GDelivery API

openapi: 3.0.0

info:

title: MSGG_L3GDelivery

version: 1.0.0-alpha.1

description: |

API for MSGG L3G Message Delivery Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V0.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgg-l3gdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- msgg-l3gdelivery

paths:

/deliver-message:

post:

summary: deliver message to Legacy 3GPP Message Gateway

tags:

- L3G Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/L3gMessageDelivery'

responses:

'204':

description: No Content, Message delivery successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/deliver-report:

post:

summary: deliver status report to Legacy 3GPP Message Gateway

tags:

- L3G status report delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/DeliveryStatusReport'

responses:

'204':

description: No Content, status report delivery successfully

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

msgg-l3gdelivery: Access to the MSGG_L3GDelivery API

schemas:

#

# STRUCTURED DATA TYPES

#

L3gMessageDelivery:

description: Contains the L3G message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

properties:

oriAddr:

$ref: '#/components/schemas/Address'

destAddr:

$ref: '#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

payload:

type: string

segInd:

type: boolean

segParams:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/MessageSegmentParameters'

Address:

description: Contains the Message type data

type: object

required:

- addrType

- addr

properties:

addrType:

$ref: '#/components/schemas/AddressType'

addr:

type: string

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AddressType:

anyOf:

- type: string

enum:

- UE

- AS

- GROUP

- BC

- TOPIC

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- UE: The address type is UE.

- AS: The address type is AS.

- GROUP: The address type is GROUP.

- BC: The address type is BC.

- TOPIC: The address type is TOPIC.

A.5	MSGG_N3GDelivery API

openapi: 3.0.0

info:

title: MSGG_N3GDelivery

version: 1.0.0-alpha.1

description: |

API for MSGG N3G Message Delivery Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V0.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgg-n3gdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- msgg-n3gdelivery

paths:

/deliver-message:

post:

summary: deliver message to NON-3GPP Message Gateway

tags:

- N3G Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/N3gMessageDelivery'

responses:

'204':

description: No Content,Message delivery successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/deliver-report:

post:

summary: deliver status report to NON-3GPP Message Gateway

tags:

- N3G status report delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/DeliveryStatusReport'

responses:

'204':

description: No Content, status report delivery successfully

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

msgg-n3gdelivery: Access to the MSGG_N3GDelivery API

schemas:

#

# STRUCTURED DATA TYPES

#

N3gMessageDelivery:

description: N3G message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

payload:

type: string

segInd:

type: boolean

segParams:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/MessageSegmentParameters'

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-10 | CT3#118-e | C3-215030 |  |  |  | Draft skeleton provided by the rapporteur | 0.0.0 |
| 2021-10 | CT3#118-e | C3-215479 |  |  |  | Inclusion of documents agreed in CT3#118e C3-215116, C3-215165, C3-215206, C3-215432, C3-215441, C3-215462. Editorial change from the rapporteur. Correction from the rapporteur. | 0.1.0 |
| 2021-11 | CT3#119-e | C3-216552 |  |  |  | Inclusion of documents agreed in CT3#119e C3-216175, C3-216525, C3-216528, C3-216529, C3-216545, C3-216546, C3-216581. Correction from the rapporteur. | 0.2.0 |
| 2022-01 | CT3#119bis-e | C3-220458 |  |  |  | Inclusion of documents agreed in CT3#119bis-e C3-220384, C3-220385, C3-220386, C3-220387, C3-220389, C3-220391, C3-220410, C3-220411, C3-220412 and C3-220413. Correction from the rapporteur. | 0.3.0 |
| 2022-02 | CT3#120e |  |  |  |  | Inclusion of documents agreed in CT3#120e C3-221245, C3-221246, C3-221247, C3-221248, C3-221249, C3-221529, C3-221530, C3-221531, C3-221532, C3-221533, C3-222534, C3-221535, C3-221536, C3-221537, C3-221538, C3-221541, C3-221542, C3-221543, C3-221544, C3-221545, C3-221546, C3-221547, C3-221548 and C3-221549. Correction from the rapporteur. | 0.4.0 |