| 3GPP TS 29.576 V17.1.0 (2022-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Messaging Framework Adaptor Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Services offered by the MFAF	9

4.1	Introduction	9

4.2	Nmfaf_3daDataManagement Service	10

4.2.1	Service Description	10

4.2.1.1	Overview	10

4.2.1.2	Service Architecture	11

4.2.1.3	Network Functions	11

4.2.1.3.1	Messaging Framework Adaptor Function (MFAF)	11

4.2.1.3.2	NF Service Consumers	11

4.2.2	Service Operations	12

4.2.2.1	Introduction	12

4.2.2.2	Nmfaf_3daDataManagement_Configure service operation	12

4.2.2.2.1	General	12

4.2.2.2.2	Initial configuration for mapping data or analytics	12

4.2.2.2.3	Update the configuration of existing individual mapping data or analytics	13

4.2.2.3	Nmfaf_3daDataManagement_Deconfigure service operation	14

4.2.2.3.1	General	14

4.2.2.3.2	Stop mapping data or analytics	14

4.3	Nmfaf_3caDataManagement Service	15

4.3.1	Service Description	15

4.3.1.1	Overview	15

4.3.1.2	Service Architecture	15

4.3.1.3	Network Functions	16

4.3.1.3.1	Messaging Framework Adaptor Function (MFAF)	16

4.3.1.3.2	NF Service Consumers	16

4.3.2	Service Operations	16

4.3.2.1	Introduction	16

4.3.2.2	Nmfaf_3caDataManagement_Fetch service operation	17

4.3.2.2.1	General	17

4.3.2.2.2	Retrieves data or analytics from the MFAF	17

4.3.2.2A	Nmfaf_3caDataManagement_Subscribe service operation	17

4.3.2.3	Nmfaf_3caDataManagement_Notify service operation	17

4.3.2.3.1	General	17

4.3.2.3.2	Notification about the subscribed data or analytics	18

5	API Definitions	19

5.1	Nmfaf_3daDataManagement Service API	19

5.1.1	Introduction	19

5.1.2	Usage of HTTP	19

5.1.2.1	General	19

5.1.2.2	HTTP standard headers	19

5.1.2.2.1	General	19

5.1.2.2.2	Content type	19

5.1.2.3	HTTP custom headers	19

5.1.3	Resources	20

5.1.3.1	Overview	20

5.1.3.2	Resource: MFAF Configurations	20

5.1.3.2.1	Description	20

5.1.3.2.2	Resource Definition	20

5.1.3.2.3	Resource Standard Methods	21

5.1.3.2.3.1	POST	21

5.1.3.2.4	Resource Custom Operations	21

5.1.3.3	Resource: Individual MFAF Configuration	21

5.1.3.2.1	Description	21

5.1.3.3.2	Resource Definition	21

5.1.3.3.3	Resource Standard Methods	22

5.1.3.3.3.1	PUT	22

5.1.3.3.3.2	DELETE	23

5.1.4	Custom Operations without associated resources	24

5.1.5	Notifications	24

5.1.6	Data Model	24

5.1.6.1	General	24

5.1.6.2	Structured data types	24

5.1.6.2.1	Introduction	24

5.1.6.2.2	Type: MfafConfiguration	24

5.1.6.2.3	Type: MessageConfiguration	25

5.1.6.2.4	Type: MfafNotiInfo	25

5.1.6.3	Simple data types and enumerations	25

5.1.6.3.1	Introduction	25

5.1.6.3.2	Simple data types	25

5.1.6.4	Data types describing alternative data types or combinations of data types	26

5.1.6.5	Binary data	26

5.1.7	Error Handling	26

5.1.7.1	General	26

5.1.7.2	Protocol Errors	26

5.1.7.3	Application Errors	26

5.1.8	Feature negotiation	26

5.1.9	Security	26

5.2	Nmfaf_3caDataManagement Service API	27

5.2.1	Introduction	27

5.2.2	Usage of HTTP	27

5.2.2.1	General	27

5.2.2.2	HTTP standard headers	27

5.2.2.2.1	General	27

5.2.2.2.2	Content type	27

5.2.2.3	HTTP custom headers	27

5.2.3	Resources	27

5.2.3.1	Overview	27

5.2.3.2	Resource: MFAF Data or Analytics	28

5.2.3.2.1	Description	28

5.2.3.2.2	Resource Definition	28

5.2.3.2.3	Resource Standard Methods	28

5.2.3.2.3.1	POST	28

5.2.4	Custom Operations without associated resources	29

5.2.5	Notifications	29

5.2.5.1	General	29

5.2.5.2	MFAF Notification	29

5.2.5.2.1	Description	29

5.2.5.2.2	Target URI	29

5.2.5.2.3	Standard Methods	30

5.2.5.2.3.1	POST	30

5.2.5.3	Fetch Notification	30

5.2.5.3.1	Description	30

5.2.5.3.2	Target URI	31

5.2.5.3.3	Standard Methods	31

5.2.5.3.3.1	POST	31

5.2.6	Data Model	32

5.2.6.1	General	32

5.2.6.2	Structured data types	32

5.2.6.2.1	Introduction	32

5.2.6.2.2	Type: NmfafDataRetrievalNotification	33

5.2.6.2.3	Type: FetchInstruction	33

5.2.6.2.4	Type: NmfafDataAnaNotification	33

5.2.6.3	Simple data types and enumerations	33

5.2.6.3.1	Introduction	33

5.2.6.3.2	Simple data types	33

5.2.6.4	Data types describing alternative data types or combinations of data types	33

5.2.6.5	Binary data	34

5.2.7	Error Handling	34

5.2.7.1	General	34

5.2.7.2	Protocol Errors	34

5.2.7.3	Application Errors	34

5.2.8	Feature negotiation	34

5.2.9	Security	34

Annex A (normative): OpenAPI specification	35

A.1	General	35

A.2	Nmfaf_3daDataManagement API	35

A.3	Nmfaf_3caDataManagement API	38

Annex B (informative): Change history	41

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

This clause will describe the scope of the corresponding service specification.

The present document specifies the stage 3 protocol and data model for the MFAF Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the MFAF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. The stage 2 definition and procedures of Messaging Framework Adaptation are contained in 3GPP TS 23.288 [14] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services"

[15]	3GPP TS 29.574: "5G System; Data Collection Coordination Services; Stage 3".

[16]	3GPP TS 29.517: "5G System; Application Function Event Exposure Services; Stage 3".

[17]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[18]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[19]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[20]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[21]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[22]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[23] 	3GPP TS 29.575: "5G System; 5G System; Analytics Data Repository Services; Stage 3".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

AMF	Access and Mobility Management Function

CEF	Charging Enablement Function

DCCF	Data Collection Coordination Function

MFAF	Messaging Framework Adaptor Function

NEF	Network Exposure Function

NF	Network Function

NRF	Network Repository Function

NSSF	Network Slice Selection Function

NWDAF	Network Data Analytics Function

OAM	Operation, Administration, and Maintenance

PCF	Policy Control Function

SMF	Session Management Function

UDM	Unified Data Management

4	Services offered by the MFAF

4.1	Introduction

The Messaging Framework Adaptor Services are used for the Messaging Framework Adaptor Function (MFAF) to enable the 5GS to interact with the messaging framework using Nmfaf services. The MFAF offers to other NFs the following services:

-	Nmfaf_3daDataManagement; and

-	Nmfaf_3caDataManagement.

Table 4.1-1: Service provided by MFAF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Nmfaf_3daDataManagement | 3GPP DCCF Adaptor (3DA) Data Management Service enables the DCCF to convey to the messaging framework, information about the data the messaging framework will receive from a Data Source, formatting and processing instructions and the Data Consumer and notification endpoints. | Configure | Request / Response | DCCF |
|  |  | Deconfigure | Request / Response | DCCF |
| Nmfaf_3caDataManagement | 3GPP Consumer Adaptor (3CA) Data Management Service delivers data to each Data Consumer or notification endpoint after formatting and processing of data received by the messaging framework. | Notify | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  |  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
| NOTE: The services correspond to the Nmfaf_3caDataManagement service and Nmfaf_3daDataManagement service as defined in 3GPP TS 23.288 [14]. |  |  |  |  |

Table 4.1-2 summarizes the corresponding APIs defined for this specification.

Table 4.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nmfaf_3daDataManagement | 4.2 | API for Nmfaf_3daDataManagement | Nmfaf_3daDataManagement.yaml | nmfaf_3dadatamanagement | Annex A.2 Nmfaf_3daDataManagement API |
| Nmfaf_3caDataManagement | 4.3 | API for Nmfaf_3caDataManagement | Nmfaf_3caDataManagement.yaml | nmfaf_3cadatamanagement | Annex A.3 Nmfaf_3caDataManagement API |

4.2	Nmfaf_3daDataManagement Service

4.2.1	Service Description

4.2.1.1	Overview

The Nmfaf_3daDataManagement service as defined in 3GPP TS 23.288 [14], is provided by the Messaging Framework Adaptor Function (MFAF).

This service:

-	allows NF consumers to configure or reconfigure the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints; and

-	allows NF consumers to reconfigure the MFAF to stop mapping data or analytics received by the MFAF to out-bound notification endpoints.

4.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14].

The Nmfaf_3daDataManagement service is part of the Nmfaf service-based interface exhibited by the Messaging Framework Adaptor Function (MFAF).

Known consumer of the Nmfaf_3daDataManagement service is:

-	Data Collection Coordination Function (DCCF)

Figure 4.2.1.2-1: Reference Architecture for the Nmfaf_3daDataManagement Service; SBI representation

Figure 4.2.1.2-2: Reference Architecture for the Nmfaf_3daDataManagement Service; reference point representation

4.2.1.3	Network Functions

4.2.1.3.1	Messaging Framework Adaptor Function (MFAF)

The Messaging Framework Adaptor Function (MFAF) provides the functionality to allow NF consumers to configure or reconfigure the behaviour of mapping data or analytics received by the MFAF to out-bound notification endpoints.

4.2.1.3.2	NF Service Consumers

The Data Collection Coordination Function (DCCF):

-	supports configuring the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the out-bound data or analytics; and

-	supports reconfiguring the MFAF to stop the sending of data to consumers.

4.2.2	Service Operations

4.2.2.1	Introduction

Table 4.2.2.1-1: Operations of the Nmfaf_3daDataManagement Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Nmfaf_3daDataManagement_Configure | This service operation is used by an NF to configure or reconfigure the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the outbound data or analytics | NF service consumer (DCCF) |
| Nmfaf_3daDataManagement_Deconfigure | This service operation is used by an NF to stop mapping data or analytics received by the MFAF to one or more outbound notification endpoints. | NF service consumer (DCCF) |

4.2.2.2	Nmfaf_3daDataManagement_Configure service operation

4.2.2.2.1	General

The Nmfaf_3daDataManagement_Configure service operation is used by an NF service consumer to configure or update the configuration of the MFAF for mapping data or analytics received by the MFAF to out-bound notification endpoints, and formatting and processing the out-bound data or analytics.

4.2.2.2.2	Initial configuration for mapping data or analytics

Figure 4.2.2.2.2-1 shows a scenario where the NF service consumer (e.g. DCCF) sends a request to the MFAF to request the configuration of mapping data or analytics (as shown in 3GPP TS 23.288 [14]).

Figure 4.2.2.2.2-1: NF service consumer create the configuration

The NF service consumer shall invoke the Nmfaf_3daDataManagement_Configure service operation to create the configuration(s). The NF service consumer shall send an HTTP POST request with "{apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations" as Resource URI representing the "MFAF Configurations", as shown in figure 4.2.2.2.2-1, step 1, to create a configuration for an "Individual MFAF Configuration" according to the information in message body. The MfafConfiguration data structure provided in the request body

shall include:

-	a description of the configurations as "messageConfigurations" attribute that, for each configuration, the MessageConfiguration data type shall include

1)	a notification URI of Data Consumer or Analytics Consumer or other endpoint where to receive the requested mapping data or analytics as "notificationURI" attribute; and

2)-	if the configuration is used for mapping analytics or data collection, a Notification Correlation ID for the Data or Analytics Consumer (or other endpoint) as "correId" attribute;

and may include:

1)	 the formatting instructions as "formatIstruct" attribute;

2)	the processing instructions as "procIntruct" attribute;

3)	the MFAF notification information to identify the Event Notifications received from the NWDAF or Data Source NF (e.g. AMF, SMF), which can be sent to the consumer or other notification endpoints, as "mfafNotiInfo" attribute; and

X)	NF instance identifier of the ADRF as "adrfId" attribute.

Upon the reception of an HTTP POST request with: "{apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations" as Resource URI and MfafConfiguration data structure as request body, the MFAF shall:

-	create a new configuration;

-	assign a transaction reference id;

-	if no MFAF notification information has been provided in the request, determine the MFAF notification information and add it to the configuration that is created and will be returned to the NF service consumer;

-	store the configuration.

If the MFAF created an "Individual MFAF Configuration" resource, the MFAF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 4.2.2.2.2-1, step 2.

If an error occurs when processing the HTTP POST request, the MFAF shall send an HTTP error response as specified in subclause 5.1.7.

4.2.2.2.3	Update the configuration of existing individual mapping data or analytics

Figure 4.2.2.2.3-1 shows a scenario where the NF service consumer sends a request to the MFAF to update the configuration of mapping data or analytics (as shown in 3GPP TS 23.288 [14])

Figure 4.2.2.2.3-1: NF service consumer updates configuration

The NF service consumer shall invoke the Nmfaf_3daDataManagement_Configure service operation to update the configuration(s). The NF service consumer shall send an HTTP PUT request with "{apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId}" as Resource URI representing the "Individual MFAF Configuration", as shown in figure 4.2.2.2.3-1, step 1, to update the subscription for an "Individual MFAF Configuration" resource identified by the {transRefId}. The MfafConfiguration data structure provided in the request body shall include:

-	a description of the configurations as "messageConfigurations" attribute that, for each configuration, the MfafConfiguration data structure provided in the request body shall include the same contents as described in 4.2.2.2.2.

Upon the reception of an HTTP PUT request with: "{apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId}" as Resource URI and MfafConfiguration data structure as request body, the MFAF shall:

-	update the configuration of corresponding transaction reference Id; and

-	store the configuration.

If the MFAF successfully processed and accepted the received HTTP PUT request, the MFAF shall update an "Individual MFAF Configuration" resource, and shall respond with:

a)	HTTP "200 OK" status code with the message body containing a representation of the updated configuration, as shown in figure 4.2.2.2.3-1, step 2a. or

b)	HTTP "204 No Content" status code, as shown in figure 4.2.2.2.3-1, step 2b.

If an error occurs when processing the HTTP PUT request, the MFAF shall send an HTTP error response as specified in subclause 5.1.7.

If the MFAF determines the received HTTP PUT request needs to be redirected, the MFAF shall send an HTTP redirect response as specified in subclause 6.10.9 of 3GPP TS 29.500 [4].

4.2.2.3	Nmfaf_3daDataManagement_Deconfigure service operation

4.2.2.3.1	General

The Nmfaf_3daDataManagement_Deconfigure service operation is used by an NF service consumer to stop mapping data or analytics received by the MFAF to one or more out-bound notification endpoints.

4.2.2.3.2	Stop mapping data or analytics

Figure 4.2.2.3.2-1 shows a scenario where the NF service consumer sends a request to the MFAF to update the configuration to stop mapping data or analytics (as shown in 3GPP TS 23.288 [14])

Figure 4.2.2.3.2-1: NF service consumer stops mapping data or analytics

The NF service consumer shall invoke the Nmfaf_3daDataManagement_Deconfigure service operation to stop the MFAF to map data or analytics. The NF service consumer shall send an HTTP DELETE request with "{apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId}" as Resource URI, where {transRefId} represents the "Individual MFAF Configuration" to be deleted, as shown in figure 4.2.2.3.2-1, step 1.

Upon the reception of an HTTP DELETE request and if the MFAF successfully processed and accepted the received HTTP DELETE request from the NF service consumer, the MFAF shall acknowledge the request by sending a "204 No Content" response to the NF service consumer, as shown in figure 4.2.2.3.2-1, step 2. Further, the MFAF shall remove the individual resource linked to the delete request.

If errors occur when processing the HTTP DELETE request, the MFAF shall send an HTTP error response as specified in subclause 5.1.7.

If the MFAF determines the received HTTP DELETE request needs to be redirected, the MFAF shall send an HTTP redirect response as specified in subclause 6.10.9 of 3GPP TS 29.500 [4].

4.3	Nmfaf_3caDataManagement Service

4.3.1	Service Description

4.3.1.1	Overview

The Nmfaf_3caDataManagement service as defined in 3GPP TS 23.288 [14], is provided by the Messaging Framework Adaptor Function (MFAF).

This service:

-	allows NF consumers to collect the data or analytics from the MFAF.

4.3.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14].

The Nmfaf_3caDataManagement service is part of the Nmfaf service-based interface exhibited by the Messaging Framework Adaptor Function (MFAF).

Known consumers of the Nmfaf_3caDataManagement service are:

-	Network Data Analytics Function (NWDAF)

-	Policy Control Function (PCF)

-	Network Slice Selection Function (NSSF)

-	Access and Mobility Management Function (AMF)

-	Session Management Function (SMF)

-	Network Exposure Function (NEF)

-	Application Function (AF)

-	Operation, Administration, and Maintenance (OAM)

-	Charging Enablement Function (CEF)

Figure 4.3.1.2-1: Reference Architecture for the Nmfaf_3caDataManagement Service; SBI representation

Figure 4.3.1.2-2: Reference Architecture for the Nmfaf_3caDataManagement Service; reference point representation

4.3.1.3	Network Functions

4.3.1.3.1	Messaging Framework Adaptor Function (MFAF)

The Messaging Framework Adaptor Function (MFAF) provides the functionality to supply data or analytics, or an indication of availability of data or analytics to notification endpoints configured in Nmfaf_3caDataManagement service as described in clause 4.2.1.

4.3.1.3.2	NF Service Consumers

The NWDAF, PCF, NSSF, AMF, AMF, NEF, OAM, CEF and AF:

-	supports retrieving data or analytics from the MFAF.

The MFAF:

-	supports providing data or analytics or notification of availability of data or analytics to notification endpoints.

4.3.2	Service Operations

4.3.2.1	Introduction

Table 4.3.2.1-1: Operations of the Nmfaf_3caDataManagement Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Nmfaf_3caDataManagement_Fetch | This service operation is used by an NF to retrieve stored data or analytics from the MFAF. | NF service consumer (NWDAF, PCF, NSSF, AMF, AMF, NEF, OAM, CEF and AF) |
| Nmfaf_3caDataManagement_Subscribe | This is a pseudo operation, the actual subscription is created via Nmfaf_3daDataManagement Service. (NOTE) |  |
| Nmfaf_3caDataManagement_Notify | This service operation is used by an NF with either data or analytics to provide data or analytics or notification of availability of data or analytics to notification endpoints. | MFAF |
| NOTE: In the current release OpenAPI 3.0.0 is adopted, with OpenAPI 3.0.0 it is not possible to document a stand-alone callback operation, thus the Notify operation has to be defined in combination with a Subscribe operation. |  |  |

NOTE:	Nmfaf_3caDataManagement_Subscribe service operation is not used by any NF service consumers in this release.

4.3.2.2	Nmfaf_3caDataManagement_Fetch service operation

4.3.2.2.1	General

The Nmfaf_3caDataManagement_Fetch service operation allows consumer to retrieves data or analytics from the MFAF as indicated by Nmfaf_3caDataManagement_Notify Fetch Instruction.

4.3.2.2.2	Retrieves data or analytics from the MFAF

Figure 4.3.2.2.2-1 shows a scenario where the NF service consumer (e.g. NWDAF) sends a request to the MFAF to retrieve the data or analytics as indicated by Nmfaf_3caDataManagement_Notify Fetch Instruction.

Figure 4.3.2.2.2-1: NF service consumer retrieve data or analytics from the MFAF

The NF service consumer shall invoke the Nmfaf_3caDataManagement_Fetch service operation to retrieve stored data or analytics. The NF service consumer shall send an HTTP POST request with "{fetchUri}" URI, as shown in figure 4.3.2.2.2-1, step 1, to fetch MFAF data or analytics.

The request body shall include fetch correlation identifies, which was previously provided by the MFAF in the "fetchCorrIds" attribute within fetchInstruction in the MFAF notification.

Upon the reception of the HTTP POST request, the MFAF shall:

-	find the data or analytics according to the requested parameters.

If the requested data is found, the MFAF shall respond with "200 OK" status code with the message body containing the NmfafDataAnaNotification data structure. The NmfafDataAnaNotification data structure in the response body shall include one of the following:

-	information about network data analytics function events that occurred in the "anaNotifications" attribute;

-	network exposure events or

application function events or

access and mobility function events

or session management function events

or monitoring report events that occurred in the "dataNotif " attribute.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.2.7.

If the MFAF determines the received HTTP POST request needs to be redirected, the MFAF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

4.3.2.2A	Nmfaf_3caDataManagement_Subscribe service operation

This is a pseudo operation, the MFAF does not actually provide Subscribe service operation through Nmfaf_3caDataManagement service. The actual subscription is created via Nmfaf_3daDataManagement Service.

4.3.2.3	Nmfaf_3caDataManagement_Notify service operation

4.3.2.3.1	General

The Nmfaf_3caDataManagement_Notify service operation provides data or analytics or notification of availability of data or analytics to notification endpoints.

4.3.2.3.2	Notification about the subscribed data or analytics

Figure 4.3.2.3.2-1 shows a scenario where the MFAF sends a request to the NF service consumer to notify it about data or analytics or fetch instructions.

The subscription corresponding to the notification is created by the service consumer via Nmfaf_3daDataManagement Service.

Figure 4.3.2.3.2-1: MFAF notifies the NF service consumer about subscribed data or analytics or fetch instructions

The MFAF shall invoke the Nmfaf_3caDataManagement_Notify service operation to notify about subscribed data or analytics, or notification about the availability of data or analytics. The MFAF shall send an HTTP POST request to the "{notificationURI}" received in the subscription (see clause 5.2.5 for the definition of this notificationURI), as shown in figure 4.3.2.3.2-1, step 1. The NmfafDataRetrievalNotification data structure provided in the request body shall include:

-	notification correlation Id within the "correId" attribute;

and shall include at least one of the following:

-	fetch instructions indicate whether the data or analytics are to be fetched by the Consumer in the "fetchInstruction" attribute; and

-	information about the MFAF data or analytics in the "dataAnaNotifs" attribute, which contains one of the following:

-	information about network data analytics function events that occurred in the "anaNotifications" attribute;

-	data collected from data sources (e.g. SMF, NEF) in the "dataNotif" attribute.

Upon the reception of an HTTP POST request with "{notificationURI}" as Resource URI and NmfafDataRetrievalNotification data structure as request body, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF Service Consumer shall:

-	store the notification;

-	respond with HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.2.7.

If the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

5	API Definitions

5.1	Nmfaf_3daDataManagement Service API

5.1.1	Introduction

The Nmfaf_3daDataManagement Service shall use the Nmfaf_3daDataManagement API.

The API URI of the Nmfaf_3daDataManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmfaf-3dadatamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.1.3.

5.1.2	Usage of HTTP

5.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmfaf_3daDataManagement API is contained in Annex A.

5.1.2.2	HTTP standard headers

5.1.2.2.1	General

See clause 5.1.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

5.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

5.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.1.3.2 of 3GPP TS 29.500 [4] shall be applicable.

5.1.3	Resources

5.1.3.1	Overview

This clause describes the structure for the Resource URIs, the resources and methods used for the service.

Figure 5.1.3.1-1 depicts the resource URIs structure for the Nmfaf_3daDataManagement API.

Figure 5.1.3.1-1: Resource URI structure of the Nmfaf_3daDataManagement API

Table 5.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| MFAF Configurations | /configurations | POST | Creates a new individual MFAF Configuration resource. |
| Individual MFAF Configuration | /configurations/{transRefId} | PUT | Modifies an existing Individual MFAF Configuration subresource. |
|  |  | DELETE | Deletes an Individual MFAF Configuration identified by subresource {transRefId}. |

5.1.3.2	Resource: MFAF Configurations

5.1.3.2.1	Description

The MFAF Configurations resource represents all configuration to the Nmfaf_3daDataManagement Service at a given MFAF. The resource allows an NF service consumer to create a new Individual MFAF Configuration resource.

5.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.2.2-1.

Table 5.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |

5.1.3.2.3	Resource Standard Methods

5.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.1.3.2.3.1-1.

Table 5.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.2.3.1-2 and the response data structures and response codes specified in table 5.1.3.2.3.1-3.

Table 5.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MfafConfiguration | M | 1 | Create a new Individual MFAF Configuration resource. |

Table 5.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MfafConfiguration | M | 1 | 201 Created | The creation of an Individual MFAF Configuration resource is confirmed and a representation of that resource is returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId} |

5.1.3.2.4	Resource Custom Operations

None in this release of the specification.

5.1.3.3	Resource: Individual MFAF Configuration

5.1.3.2.1	Description

The Individual MFAF Configurations resource represents an individual configuration created in the MFAF and associated with transaction reference Id.

5.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId}

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.3.2-1.

Table 5.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |
| transRefId | string | Unique identifier of the individual MFAF Configurations resource. |

5.1.3.3.3	Resource Standard Methods

5.1.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 5.1.3.3.3.1-1.

Table 5.1.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.1-2 and the response data structures and response codes specified in table 5.1.3.3.3.1-3.

Table 5.1.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MfafConfiguration | M | 1 | The updated MFAF Configuration. |

Table 5.1.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MfafConfiguration | M | 1 | 200 OK | The update of an Individual MFAF Configuration resource is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case: The Individual MFAF Configuration resource was modified. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF (service) instance where the request should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF (service) instance where the request should be sent. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request should be redirected. |

Table 5.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request should be redirected. |

5.1.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 5.1.3.3.3.2-1.

Table 5.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.2-2 and the response data structures and response codes specified in table 5.1.3.3.3.2-3.

Table 5.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual MFAF Configuration resource matching the transRefId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF (service) instance where the request should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF (service) instance where the request should be sent. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request should be redirected. |

Table 5.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request should be redirected. |

5.1.4	Custom Operations without associated resources

None in this release of the specification.

5.1.5	Notifications

None in this release of the specification.

5.1.6	Data Model

5.1.6.1	General

This clause specifies the application data model supported by the API.

Table 5.1.6.1-1 specifies the data types defined for the Nmfaf_3daDataManagement service based interface protocol.

Table 5.1.6.1-1: Nmfaf_3daDataManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MfafConfiguration | 5.1.6.2.2 | The description of MFAF configuration |  |
| MessageConfiguration | 5.1.6.2.3 | The description of the configurations. |  |
| MfafNotiInfo | 5.1.6.2.4 | The MFAF notification information. |  |

Table 5.1.6.1-2 specifies data types re-used by the Nmfaf_3daDataManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nmfaf_3daDataManagement service based interface.

Table 5.1.6.1-2: Nmfaf_3daDataManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| FormattingInstruction | 3GPP TS 29.574 [15] | Contains data or analytics formatting Instructions. |  |
| ProcessingInstruction | 3GPP TS 29.574 [15] | Contains instructions related to the processing |  |
| Uri | 3GPP TS 29.571 [22] | URI. |  |

5.1.6.2	Structured data types

5.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

5.1.6.2.2	Type: MfafConfiguration

Table 5.1.6.2.2-1: Definition of type MfafConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| messageConfigurations | array(MessageConfiguration) | M | 1..N | The configuration of the MFAF for mapping data or analytics. |  |

5.1.6.2.3	Type: MessageConfiguration

Table 5.1.6.2.3-1: Definition of type MessageConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| correId | string | M | 1 | If the configuration is used for mapping analytics or data collection, representing the Analytics Consumer Notification Correlation ID or Data Consumer Notification Correlation ID. |  |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. |  |
| mfafNotiInfo | MfafNotiInfo | C | 0..1 | The MFAF notification information. It shall be provided in a response message if it had not been provided in the respective request message. |  |
| notificationURI | Uri | M | 1 | The notification URI of Data Consumer or Analytics Consumer or other endpoint where to receive the requested mapping data or analytics |  |
| procInstruct | ProcessingInstruction | O | 0..1 | Processing instructions to be used for sending event notifications. |  |
| adrfId | NfInstanceId | O | 0..1 | NF instance identifier of the ADRF in which data and analytics can be stored. |  |

5.1.6.2.4	Type: MfafNotiInfo

Table 5.1.6.2.4-1: Definition of type MfafNotiInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mfafNotifUri | Uri | M | 1 | The notification URI of MFAF Notification Target Address. |  |
| mfafCorreId | string | M | 1 | The MFAF Notification Correlation ID |  |

5.1.6.3	Simple data types and enumerations

5.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.1.6.3.2	Simple data types

The simple data types defined in table 5.1.6.3.2-1 shall be supported.

Table 5.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

5.1.6.4	Data types describing alternative data types or combinations of data types

None in current specification.

5.1.6.5	Binary data

None in current specification.

5.1.7	Error Handling

5.1.7.1	General

For the Nmfaf_3daDataManagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.1.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.1.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmfaf_3daDataManagement API.

5.1.7.2	Protocol Errors

No specific procedures for the Nmfaf_3daDataManagement service are specified.

5.1.7.3	Application Errors

The application errors defined for the Nmfaf_3daDataManagement service are listed in Table 5.1.7.3-1.

Table 5.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

5.1.8	Feature negotiation

The optional features in table 5.1.8-1 are defined for the Nmfaf_3daDataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

5.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmfaf_3daDataManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmfaf_3daDataManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmfaf_3daDataManagement service.

The Nmfaf_3daDataManagement API defines a single scope "nmfaf_3dadatamanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

5.2	Nmfaf_3caDataManagement Service API

5.2.1	Introduction

The Nmfaf_3caDataManagement Service shall use the Nmfaf_3caDataManagement API.

The API URI of the Nmfaf_3caDataManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmfaf-3cadatamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.2.3.

5.2.2	Usage of HTTP

5.2.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmfaf_3caDataManagement API is contained in Annex A.

5.2.2.2	HTTP standard headers

5.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

5.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

5.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

5.2.3	Resources

5.2.3.1	Overview

This clause describes the structure for the Resource URIs, the resources and methods used for the service.

Figure 5.1.3.1-1 depicts the resource URIs structure for the Nmfaf_3caDataManagement API.

Figure 5.2.3.1-1: Resource URI structure of the Nmfaf_3caDataManagement API

Table 5.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| MFAF Data or Analytics(Collection) | /mfaf-data-analytics | POST | This is a pseudo resource. |

5.2.3.2	Resource: MFAF Data or Analytics

5.2.3.2.1	Description

5.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmfaf_3cadatamanagement/<apiVersion>/mfaf-data-analytics

The <apiVersion> shall be set as described in clause 5.2.1.

This resource shall support the resource URI variables defined in table 5.2.3.2.2-1.

Table 5.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.1 |

5.2.3.2.3	Resource Standard Methods

5.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.2.3.2.3.1-1.

Table 5.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.2.3.2.3.1-2 and the response data structures and response codes specified in table 5.2.3.2.3.1-3.

Table 5.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Any |  |  |  |

Table 5.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.2.4	Custom Operations without associated resources

None in this release of the specification.

5.2.5	Notifications

5.2.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 5.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MFAF Notification | {notificationURI} | POST | MFAF Notification |
| NOTE: The notificationURI is not provided by NF service consumer via Nmfaf-3caDataManagement API, it is provided via Nmfaf-3daDataManagement API during the configuration for mapping data or analytics. |  |  |  |

5.2.5.2	MFAF Notification

5.2.5.2.1	Description

The MFAF Notification is used by the MFAF to provide data or analytics or notification of availability of data or analytics to notification endpoints.

5.2.5.2.2	Target URI

The Callback URI "{notificationURI}" shall be used with the callback URI variables defined in table 5.2.5.2.2-1.

Table 5.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationURI | The notification URI of Data Consumer or Analytics Consumer or other endpoint where to receive the requested data or analytics. The notificationURI is not provided by NF service consumer via Nmfaf-3caDataManagement API, it is provided via Nmfaf-3daDataManagement API during the configuration for mapping data or analytics. |

5.2.5.2.3	Standard Methods

5.2.5.2.3.1	POST

This method shall support the request data structures specified in table 5.2.5.2.3.1-1 and the response data structures and response codes specified in table 5.2.5.2.3.1-1.

Table 5.2.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NmfafDataRetrievalNotification | M | 1 | The data or analytics or notification of availability of data or analytics to notification endpoints. |

Table 5.2.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MFAF notification is treated successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during the retrieval notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during the retrieval notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.2.5.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 5.2.5.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

5.2.5.3	Fetch Notification

5.2.5.3.1	Description

The Fetch Notification is used by the NF service consumer to retrieve data or analytics from the MFAF.

5.2.5.3.2	Target URI

The Callback URI "{fetchUri}" shall be used with the callback URI variables defined in table 5.2.5.3.2-1.

Table 5.2.5.3.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| fetchUri | Uri | Fetch Uri as assigned during the procedure of notification about the subscribed data or analytics within the FetchInstruction data type (see table 5.2.6.2.3-1). |

5.2.5.3.3	Standard Methods

5.2.5.3.3.1	POST

This method shall support the URI query parameters specified in table 5.2.5.3.3.1-1.

Table 5.2.5.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.2.5.3.3.1-2 and the response data structures and response codes specified in table 5.2.5.3.3.1-3.

Table 5.2.5.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(string) | M | 1..N | Indicate the fetch correlation identifier(s). |

Table 5.2.5.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NmfafDataAnaNotification | M | 1 | 200 OK | The stored data or analytics related to the "fetch-corr-ids". |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MFAF. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MFAF. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.2.5.3.3.1-4: Headers supported by the by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MFAF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 5.2.3.2.3.1-5: Headers supported by the 308 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MFAF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

5.2.6	Data Model

5.2.6.1	General

This clause specifies the application data model supported by the API.

Table 5.2.6.1-1 specifies the data types defined for the Nmfaf_3caDataManagement service based interface protocol.

Table 5.2.6.1-1: Nmfaf_3caDataManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| FetchInstruction | 5.2.6.2.3 | The fetch instruction indicates whether the data or analytics are to be fetched by the Consumer. |  |
| NmfafDataAnaNotification | 5.2.6.2.4 | MFAF data or analytics. |  |
| NmfafDataRetrievalNotification | 5.2.6.2.2 | The data or analytics or notification of availability of data or analytics to notification endpoints. |  |

Table 5.2.6.1-2 specifies data types re-used by the Nmfaf_3caDataManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nmfaf_3caDataManagement service based interface.

Table 5.2.6.1-2: Nmfaf_3caDataManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DataNotification | 3GPP TS 29.575 [23] | Represents a data subscription notification of one of various possible data sources |  |
| NnwdafEventsSubscriptionNotification | 3GPP TS 29.520 [20] | Represents an NWDAF analytics subscription notification. |  |
| Uri | 3GPP TS 29.571 [22] | URI. |  |

5.2.6.2	Structured data types

5.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

5.2.6.2.2	Type: NmfafDataRetrievalNotification

Table 5.2.6.2.2-1: Definition of type NmfafDataRetrievalNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| correId | string | M | 1 | If the configuration is used for mapping analytics or data collection, representing the Analytics Consumer Notification Correlation ID or Data Consumer Notification Correlation ID. |  |
| dataAnaNotif | NmfafDataAnaNotification | C | 0..1 | Represents notifications of analytics and data. (NOTE) |  |
| fetchInstruction | FetchInstruction | C | 0..1 | The fetch instruction indicate whether the data or analytics are to be fetched by the Consumer. (NOTE) |  |
| NOTE: At least one of the "dataAnaNotif" and "fetchInstruction" shall be included. |  |  |  |  |  |

5.2.6.2.3	Type: FetchInstruction

Table 5.2.6.2.3-1: Definition of type FetchInstruction

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| fetchUri | Uri | M | 1 | The target address used by a data or analytics consumer to fetch the data or analytics. |  |
| fetchCorrIds | array(string) | M | 1..N | the fetch correlation identifier(s) of the MFAF Data or Analytics |  |

5.2.6.2.4	Type: NmfafDataAnaNotification

Table 5.2.6.2.4-1: Definition of type NmfafDataAnaNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaNotifications | array(NnwdafEventsSubscriptionNotification) | C | 1..N | List of analytics subscription notifications. (NOTE) |  |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE) |  |
| NOTE: Exactly one of these attributes shall be provided. |  |  |  |  |  |

5.2.6.3	Simple data types and enumerations

5.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.2.6.3.2	Simple data types

None in this release of the specification.

5.2.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

5.2.6.5	Binary data

None in this release of the specification.

5.2.7	Error Handling

5.2.7.1	General

For the Nmfaf_3caDataManagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmfaf_3caDataManagement API.

5.2.7.2	Protocol Errors

No specific procedures for the Nmfaf_3caDataManagement service are specified.

5.2.7.3	Application Errors

The application errors defined for the Nmfaf_3caDataManagement service are listed in Table 5.2.7.3-1.

Table 5.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

5.2.8	Feature negotiation

The optional features in table 5.2.8-1 are defined for the Nmfaf_3caDataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

5.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmfaf_3caDataManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmfaf_3caDataManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmfaf_3caDataManagement service.

The Nmfaf_3caDataManagement API defines a single scope "nmfaf_3cadatamanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

A.2	Nmfaf_3daDataManagement API

openapi: 3.0.0

info:

version: 1.0.0

title: Nmfaf_3daDataManagement

description: |

MFAF 3GPP DCCF Adaptor (3DA) Data Management Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.576 V17.1.0; 5G System; Messaging Framework Adaptor Services; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.576/'

servers:

- url: '{apiRoot}/nmfaf-3dadatamanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nmfaf-3dadatamanagement

- {}

paths:

/configurations:

post:

summary: Creates a new Individual MFAF Configuration resource.

operationId: CreateMFAFConfiguration

tags:

- MFAF Configuration(Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MfafConfiguration'

required: true

description: >

Contains the information for the creation of a new Individual MFAF

Configuration resource.

responses:

'201':

description: Successful creation of new Individual MFAF Configuration resource.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/MfafConfiguration'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/configurations/{transRefId}:

put:

summary: Updates an existing Individual MFAF Configuration resource.

operationId: UpdateMFAFConfiguration

tags:

- Individual MFAF Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MfafConfiguration'

parameters:

- name: transRefId

in: path

description: Unique identifier of the individual MFAF Configurations resource.

required: true

schema:

type: string

responses:

'200':

description: The updated MFAF Configuration resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MfafConfiguration'

'204':

description: The Individual MFAF Configuration resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual MFAF Configuration resource.

operationId: DeleteMFAFConfiguration

tags:

- Individual MFAF Configuration (Document)

parameters:

- name: transRefId

in: path

description: Unique identifier of the individual MFAF Configurations resource.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual MFAF Configuration resource matching

the transRefId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nmfaf-3dadatamanagement: Access to the nmfaf-3dadatamanagement API

schemas:

MfafConfiguration:

description: Represents an Individual MFAF Configuration.

type: object

required:

- messageConfigurations

properties:

messageConfigurations:

type: array

items:

$ref: '#/components/schemas/MessageConfiguration'

minItems: 1

description: The configuration of the MFAF for mapping data or analytics.

MessageConfiguration:

description: Represents the message configuration.

type: object

required:

- notificationURI

- correId

properties:

correId:

type: string

description: >

If the configuration is used for mapping analytics or data collection,

representing the Analytics Consumer Notification Correlation ID or

Data Consumer Notification Correlation ID.

formatInstruct:

$ref: 'TS29574_Ndccf_DataManagement.yaml#/components/schemas/FormattingInstruction'

mfafNotiInfo:

$ref: '#/components/schemas/MfafNotiInfo'

notificationURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

procInstruct:

$ref: 'TS29574_Ndccf_DataManagement.yaml#/components/schemas/ProcessingInstruction'

adrfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

MfafNotiInfo:

description: >

The MFAF notification information. It shall be provided in a response message

if it had not been provided in the respective request message.

type: object

required:

- mfafNotifUri

- mfafCorreId

properties:

mfafNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

mfafCorreId:

type: string

A.3	Nmfaf_3caDataManagement API

openapi: 3.0.0

info:

version: 1.0.0

title: Nmfaf_3caDataManagement

description: |

MFAF 3GPP Consumer Adaptor (3CA) Data Management Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.576 V17.1.0; 5G System; Messaging Framework Adaptor Services; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.576/'

servers:

- url: '{apiRoot}/nmfaf-3cadatamanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nmfaf-3cadatamanagement

- {}

paths:

/mfaf-data-analytics:

post:

# This is a pseudo operation, clients shall NOT invoke this method!

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- MFAF Data or Analytics Resources (Collection)

requestBody:

required: true

content:

application/json:

schema: {}

responses:

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

Notification:

'{notificationURI}':

# The URI in {notificationURI} is provided via Nmfaf-3daDataManagement API during the configuration for mapping data or analytics.

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NmfafDataRetrievalNotification'

responses:

'204':

description: The receipt of the Notification is acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

Fetch:

'{request.body#/fetchUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

type: string

minItems: 1

description: Indicate the fetch correlation identifier.

responses:

'200':

description: Expected response to a valid request.

content:

application/json:

schema:

$ref: '#/components/schemas/NmfafDataAnaNotification'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nmfaf-3cadatamanagement: Access to the nmfaf-3cadatamanagement API

schemas:

NmfafDataRetrievalNotification:

description: >

Represents the data or analytics or notification of availability

of data or analytics

to notification endpoints.

type: object

required:

- correId

anyOf:

- required: [dataAnaNotif]

- required: [fetchInstruction]

properties:

correId:

type: string

description: >

If the configuration is used for mapping analytics or data collection,

represents the Analytics Consumer Notification Correlation ID

or Data Consumer Notification Correlation ID.

dataAnaNotif:

$ref: '#/components/schemas/NmfafDataAnaNotification'

fetchInstruction:

$ref: '#/components/schemas/FetchInstruction'

FetchInstruction:

description: >

The fetch instructions indicate whether the data or analytics are to be fetched by the

consumer.

type: object

required:

- fetchUri

- fetchCorrIds

properties:

fetchUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

fetchCorrIds:

type: array

items:

type: string

minItems: 1

description: The fetch correlation identifier(s) of the MFAF Data or Analytics.

NmfafDataAnaNotification:

description: MFAF data or analytics.

type: object

oneOf:

- required: [anaNotifications]

- required: [dataNotif]

properties:

anaNotifications:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscriptionNotification'

minItems: 1

description: List of analytics subscription notifications.

dataNotif:

$ref: 'TS29575_Nadrf_DataManagement.yaml#/components/schemas/DataNotification'

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-06 | CT3#116e |  |  |  |  | TS skeleton of Messaging Framework Adaptor Services specification | 0.0.0 |
| 2021-06 | CT3#116e | C3-213502 |  |  |  | Inclusion of documents agreed in CT3#116e C3-213377. | 0.1.0 |
| 2021-08 | CT3#117e | C3-214580 |  |  |  | Inclusion of documents agreed in CT3#117e C3-214479, C3-214358, C3-214359, C3-214360, C3-214361, C3-214362, C3-214363 and C3-214480. | 0.2.0 |
| 2021-11 | CT3#119e | C3-216522 |  |  |  | Inclusion of documents agreed in CT3#119e C3-216422, C3-216423, C3-216441, C3-216465, C3-216467. | 0.3.0 |
| 2022-01 | CT3#119bis-e | C3-220455 |  |  |  | Inclusion of documents agreed in CT3#119bis-e C3-220294, C3-220464, C3-220319, C3-220504, C3-220321, C3-220505. | 0.4.0 |
| 2022-02 | CT3#120e | C3-221516 |  |  |  | Inclusion of documents agreed in CT3#120e C3-221289, C3-221299, C3-221424, C3-221602, C3-221603, C3-221604, C3-221605. | 0.5.0 |
| 2022-03 | CT#95e | CP-220161 |  |  |  | Presentation to TSG CT for approval | 1.0.0 |
| 2022-03 | CT#95e | CP-220161 |  |  |  | Approved by TSG CT | 17.0.0 |
| 2022-06 | CT#96e | CP-221132 | 0001 | 1 | F | Adding 3XX and error response handling support for MFAF services | 17.1.0 |
| 2022-06 | CT#96e | CP-221132 | 0002 | 1 | F | Corrections in the MFAF 3caDataManagement API | 17.1.0 |
| 2022-06 | CT#96e | CP-221129 | 0003 |  | F | Correct the Cardinality and Presence of some attributes | 17.1.0 |
| 2022-06 | CT#96e | CP-221132 | 0004 | 1 | B | Support carrying ADRF ID in Nmfaf_3daDataManagement_Configure service operation | 17.1.0 |
| 2022-06 | CT#96e | CP-221129 | 0005 |  | F | Nmfaf_3daDataManagement API corrections | 17.1.0 |
| 2022-06 | CT#96e | CP-221131 | 0006 | 1 | F | Nmfaf_3caDataManagement API corrections | 17.1.0 |
| 2022-06 | CT#96e | CP-221130 | 0007 |  | F | Handling of the redirection responses | 17.1.0 |
| 2022-06 | CT#96e | CP-221134 | 0009 | 2 | F | Correction on DataNotification type | 17.1.0 |
| 2022-06 | CT#96e | CP-221133 | 0011 |  | F | Removing UDM from the list of MFAF service consumers | 17.1.0 |
| 2022-06 | CT#96e | CP-221135 | 0012 | 1 | F | Update inputs of Nmfaf_3caDataManagement_Notify service | 17.1.0 |
| 2022-06 | CT#96e | CP-221134 | 0014 |  | F | Correction to MFAF notification information | 17.1.0 |
| 2022-06 | CT#96e | CP-221134 | 0015 |  | F | add CEF and OAM as consumers of Ndccf_DataManagement Service | 17.1.0 |
| 2022-06 | CT#96e | CP-221134 | 0016 |  | F | update of Abbreviations | 17.1.0 |
| 2022-06 | CT#96e | CP-221155 | 0018 | 1 | F | Update the apiVersion placeholder | 17.1.0 |
| 2022-06 | CT#96e | CP-221152 | 0019 |  | F | Update of info and externalDocs fields | 17.1.0 |