| 3GPP TS 29.575 V18.0.0 (2022-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Analytics Data Repository Services; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Services offered by the ADRF	9

4.1	Introduction	9

4.2	Nadrf_DataManagement Service	10

4.2.1	Service Description	10

4.2.1.1	Overview	10

4.2.1.2	Service Architecture	10

4.2.1.3	Network Functions	11

4.2.1.3.1	Analytics Data Repository Function (ADRF)	11

4.2.1.3.2	NF Service Consumers	11

4.2.2	Service Operations	12

4.2.2.1	Introduction	12

4.2.2.2	Nadrf_DataManagement_StorageRequest service operation	12

4.2.2.2.1	General	12

4.2.2.2.2	Request Storage of data or analytics	12

4.2.2.3	Nadrf_DataManagement_StorageSubscriptionRequest service operation	13

4.2.2.3.1	General	13

4.2.2.3.2	Requesting subscription and storage of data or analytics	13

4.2.2.4	Nadrf_DataManagement_StorageSubscriptionRemoval service operation	15

4.2.2.4.1	General	15

4.2.2.4.2	Requesting removal of subscription of data or analytics	15

4.2.2.5	Nadrf_DataManagement_RetrievalRequest service operation	15

4.2.2.5.1	General	15

4.2.2.5.2	Request and get stored data or analytics from ADRF Data Store	15

4.2.2.6	Nadrf_DataManagement_RetrievalSubscribe service operation	16

4.2.2.6.1	General	16

4.2.2.6.2	Requesting retrieval and subscription of data or analytics	16

4.2.2.7	Nadrf_DataManagement_RetrievalUnsubscribe service operation	17

4.2.2.7.1	General	17

4.2.2.7.2	Requesting removal of retrieval subscription for data or analytics	18

4.2.2.8	Nadrf_DataManagement_RetrievalNotify service operation	18

4.2.2.8.1	General	18

4.2.2.8.2	Notification about subscribed data or analytics	18

4.2.2.9	Nadrf_DataManagement_Delete service operation	20

4.2.2.9.1	General	20

4.2.2.9.2	Requesting removal of stored data or analytics	20

4.2.2.9.3	Requesting removal of stored data or analytics using data or analytics specification	20

5	API Definitions	21

5.1	Nadrf_DataManagement Service API	21

5.1.1	Introduction	21

5.1.2	Usage of HTTP	22

5.1.2.1	General	22

5.1.2.2	HTTP standard headers	22

5.1.2.2.1	General	22

5.1.2.2.2	Content type	22

5.1.2.3	HTTP custom headers	22

5.1.3	Resources	22

5.1.3.1	Overview	22

5.1.3.2	Resource: ADRF Data Store Records	23

5.1.3.2.1	Description	23

5.1.3.2.2	Resource Definition	23

5.1.3.2.3	Resource Standard Methods	23

5.1.3.2.3.1	POST	23

5.1.3.2.3.2	GET	24

5.1.3.2.4	Resource Custom Operations	24

5.1.3.3	Resource: Individual ADRF Data Store Record	25

5.1.3.3.1	Description	25

5.1.3.3.2	Resource Definition	25

5.1.3.3.3	Resource Standard Methods	25

5.1.3.3.3.1	DELETE	25

5.1.3.3.4	Resource Custom Operations	26

5.1.3.4	Resource: ADRF Data Retrieval Subscriptions	26

5.1.3.4.1	Description	26

5.1.3.4.2	Resource Definition	26

5.1.3.4.3	Resource Standard Methods	26

5.1.3.4.3.1	POST	26

5.1.3.4.4	Resource Custom Operations	27

5.1.3.5	Resource: Individual ADRF Data Retrieval Subscription	27

5.1.3.5.1	Description	27

5.1.3.5.2	Resource Definition	27

5.1.3.5.3	Resource Standard Methods	27

5.1.3.5.3.1	DELETE	27

5.1.3.5.4	Resource Custom Operations	28

5.1.4	Custom Operations without associated resources	28

5.1.4.1	Overview	28

5.1.4.2	Operation: request-storage-sub	29

5.1.4.2.1	Description	29

5.1.4.2.2	Operation Definition	29

5.1.4.3	Operation: request-storage-sub-removal	30

5.1.4.3.1	Description	30

5.1.4.3.2	Operation Definition	30

5.1.4.4	Operation: remove-stored-data-analytics	30

5.1.4.4.1	Description	30

5.1.4.4.2	Operation Definition	30

5.1.5	Notifications	31

5.1.5.1	General	31

5.1.5.2	Retrieval Notification	31

5.1.5.2.1	Description	31

5.1.5.2.2	Target URI	31

5.1.5.2.3	Standard Methods	31

5.1.5.2.3.1	POST	31

5.1.6	Data Model	32

5.1.6.1	General	32

5.1.6.2	Structured data types	35

5.1.6.2.1	Introduction	35

5.1.6.2.2	Type: NadrfDataStoreRecord	35

5.1.6.2.3	Type: NadrfDataStoreSubscription	36

5.1.6.2.4	Type: NadrfDataRetrievalSubscription	37

5.1.6.2.5	Type: NadrfDataRetrievalNotification	38

5.1.6.2.6	Type: NadrfDataStoreSubscriptionRef	38

5.1.6.2.7	Type: NadrfStoredDataSpec	39

5.1.6.2.8	Type: DataSubscription	39

5.1.6.2.9	Type: DataNotification	40

5.1.6.3	Simple data types and enumerations	40

5.1.6.4	Data types describing alternative data types or combinations of data types	40

5.1.7	Error Handling	40

5.1.7.1	General	40

5.1.7.2	Protocol Errors	40

5.1.7.3	Application Errors	40

5.1.8	Feature negotiation	41

5.1.9	Security	41

Annex A (normative): OpenAPI specification	42

A.1	General	42

A.2	Nadrf_DataManagement API	42

Annex B (informative): Change history	52

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

The present document specifies the stage 3 protocol and data model for the ADRF Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the ADRF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. The stage 2 definition and procedures of store and retrieve the collected data and analytics are contained in 3GPP TS 23.288 [14] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[15]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[17]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[20]	3GPP TS 29.517: "5G System; Application Function Event Exposure Services; Stage 3".

[21]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[22]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[23]	3GPP TS 29.574: "5G System; Data Collection Coordination Services; Stage 3".

[24]	3GPP TS 29.576: "5G System; Messaging Framework Adaptor Services; Stage 3".

[25]	3GPP TS 29.536: "5G System; Network Slice Admission Control Services; Stage 3".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

None.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

None.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADRF	Analytics Data Repository Function

AF	Application Function

AMF	Access and Mobility Management Function

DCCF	Data Collection Coordination Function

MFAF	Messaging Framework Adaptor Function

NEF	Network Exposure Function

NF	Network Function

NRF	Network Repository Function

NWDAF	Network Data Analytics Function

NSACF	Network Slice Admission Control Function

SMF	Session Management Function

UDM	Unified Data Management

4	Services offered by the ADRF

4.1	Introduction

The Analytics Data Repository Service is used for the Analytics Data Repository Function (ADRF) to storage and retrieval of data by e.g. Consumers NF(s) (e.g. NWDAF) which access the data using Nadrf service. The ADRF offers to other NFs the following service:

-	Nadrf_DataManagement.

Table 4.1-1: Service provided by ADRF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Nadrf_DataManagement | This service enables the NF service consumers to store, retrieve and remove the data or analytics in an ADRF. | StorageRequest | Request / Response | DCCF, NWDAF, MFAF |
|  |  | StorageSubscriptionRequest | Request / Response | DCCF, NWDAF |
|  |  | StorageSubscriptionRemoval | Request / Response | DCCF, NWDAF |
|  |  | RetrievalRequest | Request / Response | DCCF, NWDAF |
|  |  | RetrievalSubscribe | Subscribe / Notify | DCCF, NWDAF |
|  |  | RetrievalUnsubscribe | Subscribe / Notify | DCCF, NWDAF |
|  |  | RetrievalNotify | Subscribe / Notify | DCCF, NWDAF |
|  |  | Delete | Request / Response | DCCF, NWDAF |
| NOTE: The services correspond to the Nadrf_DataManagement service as defined in 3GPP TS 23.288 [14]. |  |  |  |  |

Table 4.1-2 summarizes the corresponding APIs defined for this specification.

Table 4.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nadrf_DataManagement | 4.2 | API for Nadrf_DataManagement |  | nadrf_datamanagement | Annex A.2 Nadrf_DataManagement API |

4.2	Nadrf_DataManagement Service

4.2.1	Service Description

4.2.1.1	Overview

The Nadrf_DataManagement service as defined in 3GPP TS 23.288 [14], is provided by the Analytics Data Repository Function (ADRF).

This service:

-	allows NF consumers to store data or analytics in the ADRF;

-	allows NF consumers to retrieve data or analytics from an ADRF; and

-	allows NF consumers to delete data or analytics from an ADRF.

NOTE:	Storage of ML models in ADRF is not specified in this Release of the specification.

4.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14].

The Nadrf_DataManagement service is part of the Nadrf service-based interface exhibited by the Analytics Data Repository Function (ADRF).

Known consumers of the Nadrf_DataManagement service are:

-	Data Collection Coordination Function (DCCF)

-	Network Data Analytics Function (NWDAF)

-	Messaging Framework Adaptor Function (MFAF)

The Nadrf_DataManagement service is provided by the ADRF and consumed by the NF service consumers as shown in figure 4.2.1.2-1 for the SBI representation model and in figure 4.2.1.2-2 for the reference point representation model.

Figure 4.2.1.2-1: Reference Architecture for the Nadrf_DataManagement Service; SBI representation

Figure 4.2.1.2-2: Nadrf_DataManagement service architecture, reference point representation

4.2.1.3	Network Functions

4.2.1.3.1	Analytics Data Repository Function (ADRF)

The Analytics Data Repository Function (ADRF) provides the functionality to allow NF consumers to store, retrieve, and remove data or analytics from the ADRF.

4.2.1.3.2	NF Service Consumers

The NWDAF and DCCF:

-	supports storing data or analytics in the ADRF;

-	supports retrieving data or analytics from an ADRF; and

-	supports deletion data or analytics from an ADRF.

The MFAF:

-	supports storing data or analytics in the ADRF.

4.2.2	Service Operations

4.2.2.1	Introduction

Table 4.2.2.1-1: Operations of the Nadrf_DataManagement Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Nadrf_DataManagement_StorageRequest | This service operation is used by an NF to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message. | NF service consumer (DCCF, NWDAF, MFAF) |
| Nadrf_DataManagement_StorageSubscriptionRequest | This service operation is used by an NF to request the ADRF to initiate a subscription for data or analytics. Data or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF. | NF service consumer (DCCF, NWDAF) |
| Nadrf_DataManagement_StorageSubscriptionRemoval | This service operation is used by an NF to request that the ADRF no longer subscribes to data or analytics it is collecting and storing. | NF service consumer (DCCF, NWDAF) |
| Nadrf_DataManagement_RetrievalRequest | This service operation is used by an NF to retrieve stored data or analytics from the ADRF. | NF service consumer (DCCF, NWDAF) |
| Nadrf_DataManagement_RetrievalSubscribe | This service operation is used by an NF to retrieve stored data or analytics from the ADRF and to receive future notifications containing the corresponding data or analytics received by ADRF. | NF service consumer (DCCF, NWDAF) |
| Nadrf_DataManagement_RetrievalUnsubscribe | This service operation is used by an NF to request that the ADRF no longer sends data or analytics to a notification endpoint. | NF service consumer (DCCF, NWDAF) |
| Nadrf_DataManagement_RetrievalNotify | This service operation is used by the ADRF to notify an NF with either data or analytics, or instructions to fetch the data or analytics from the ADRF. | ADRF |
| Nadrf_DataManagement_Delete | This service operation is used by an NF to delete stored data in ADRF. | NF service consumer (DCCF, NWDAF) |

4.2.2.2	Nadrf_DataManagement_StorageRequest service operation

4.2.2.2.1	General

The Nadrf_DataManagement_StorageRequest service operation is used by an NF service consumer to store data or analytics.

4.2.2.2.2	Request Storage of data or analytics

Figure 4.2.2.2.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to store data or analytics.

Figure 4.2.2.2.2-1: NF service consumer requesting to store data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_StorageRequest service operation to store data or analytics. The NF service consumer shall send an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records" as Resource URI representing the "ADRF Data Store Records" resource, as shown in figure 4.2.2.2.2-1, step 1, to create an "Individual ADRF Data Store Record" according to the information in the message body. The NadrfDataStoreRecord data structure provided in the request body shall include:

-	one of the following:

-	analytics subscription notification(s) within the "anaNotifications" attribute together with the corresponding subscription information within the "anaSub" attribute;

-	data subscription notification within the "dataNotif" attribute together with the corresponding subscription information within the " dataSub" attribute.

Upon the reception of an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records" as Resource URI and NadrfDataStoreRecord data structure as request body, the ADRF shall:

-	create a new data store record;

-	assign a storeTransId;

-	store the data or analytics.

NOTE:	If the data and/or analytics is already stored or being stored in the ADRF, the ADRF will still create a new "Individual ADRF Data Store Record" resource and assign a new storeTransId if the ADRF intends to not really store the data again in the memory again based on the implementation.

If the ADRF created an "Individual ADRF Data Store Record" resource, the ADRF shall respond with "201 Created" with the message body containing a representation of the created record, as shown in figure 4.2.2.2.2-1, step 2. The ADRF shall include a Location HTTP header field. The Location header field shall contain the URI of the created record i.e. "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records/{storeTransId}".

If an error occurs when processing the HTTP POST request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

4.2.2.3	Nadrf_DataManagement_StorageSubscriptionRequest service operation

4.2.2.3.1	General

The Nadrf_DataManagement_StorageSubscriptionRequest service operation is used by an NF service consumer to request that the ADRF creates a subscription to data or analytics and subsequently stores notified data or analytics in the ADRF.

4.2.2.3.2	Requesting subscription and storage of data or analytics

Figure 4.2.2.3.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to subscribe for data or analytics to be stored in the ADRF.

Figure 4.2.2.3.2-1: NF service consumer requesting that the ADRF subscribes to and subsequently stores data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_StorageSubscriptionRequest service operation to request the ADRF to subscribe to data or analytics. The NF service consumer shall send an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/request-storage-sub" as URI, as shown in figure 4.2.2.3.2-1, step 1. The NadrfDataStoreSubscription data structure provided in the request body shall include:

-	one of the following subscription attributes:

-	analytics subscription information within the "anaSub" attribute;

-	data subscription information within the "dataSub" attribute;

-	one of the following target identifiers:

-	DCCF or NWDAF instance identifier within the "targetNfId" attribute;

-	DCCF or NWDAF NF set identifier within the "targetNfSetId" attribute;

and may include:

-	formatting instructions within the "formatInstruct" attribute;

-	processing instructions within the "procInstruct" attribute or the "multiProcInstructs" attribute if the "MultiProcessingInstruction" feature is supported.

NOTE:	The parameters provided by the NF service consumer, including Formatting and Processing Instructions (if provided) are used by the ADRF when subscribing to a DCCF or NWDAF for Data or Analytics to be stored.

Upon the reception of an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/request-storage-sub" as URI and NadrfDataStoreSubscription data structure as request body, the ADRF shall assign a transaction reference identifier to this request and, if the request is successfully processed and accepted, the ADRF shall respond with "200 OK" as shown in figure 4.2.2.3.2-1 step 2, with the message body containing an NadrfDataStoreSubscriptionRef data structure, which shall include the assigned transaction reference identifier as "transRefId" attribute.

NOTE:	If the data and/or analytics is already stored or being stored in the ADRF, the ADRF will still assign a new transaction reference identifier if the ADRF intends to not really store the data again in the memory again based on the implementation.

If an error occurs when processing the HTTP POST request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

In the case of a successful response, the ADRF may subsequently create a data or analytics subscription (according to inputs that had been received in the NadrfDataStoreSubscription data structure; this is not performed if the ADRF determines that the data is already being stored based on an existing subscription) with a DCCF as described in 3GPP TS 29.574 [23] or with an NWDAF as described in 3GPP TS 29.520 [15], and create a mapping between the previously assigned and returned transaction reference identifier and the subscription that is used to serve the transaction.

4.2.2.4	Nadrf_DataManagement_StorageSubscriptionRemoval service operation

4.2.2.4.1	General

The Nadrf_DataManagement_StorageSubscriptionRemoval service operation is used by an NF service consumer to request the ADRF to remove a subscription for data or analytics.

4.2.2.4.2	Requesting removal of subscription of data or analytics

Figure 4.2.2.4.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to unsubscribe for storage of data or analytics.

Figure 4.2.2.4.2-1: NF service consumer requesting the removal of a subscription to storage of data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_StorageSubscriptionRemoval service operation to request the ADRF to remove a subscription to data or analytics that are stored in the ADRF. The NF service consumer shall send an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/request-storage-sub-removal" as URI, as shown in figure 4.2.2.4.2-1, step 1. The POST request body shall contain an NadrfDataStoreSubscriptionRef data structure, which shall include a transaction reference identifier as "transRefId" attribute.

Upon the reception of an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/request-storage-sub-removal" as URI, if the ADRF successfully processed and accepted the received HTTP POST request, the ADRF shall respond with HTTP "204 No Content" status. Subsequently, the ADRF shall remove the (DCCF or NWDAF) subscription that had been created and mapped to the received transaction reference identifier as described in clause 4.2.2.3, unless this subscription is mapped to further transaction reference identifiers (of transactions that are still active).

If errors occur when processing the HTTP POST request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

4.2.2.5	Nadrf_DataManagement_RetrievalRequest service operation

4.2.2.5.1	General

The Nadrf_DataManagement_RetrievalRequest service operation is used by an NF service consumer to retrieve stored data or analytics.

4.2.2.5.2	Request and get stored data or analytics from ADRF Data Store

Figure 4.2.2.5.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to retrieve stored data or analytics.

Figure 4.2.2.5.2-1: NF service consumer requesting to retrieve stored data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_RetrievalRequest service operation to retrieve stored data or analytics. The NF service consumer shall send an HTTP GET request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records" as Resource URI representing the "ADRF Data Store Records" resource, as shown in figure 4.2.2.5.2-1, step 1, to request ADRF data store records according to the query parameter value of the store transaction identifier within the "store-trans-id" attribute or the query parameter value of the fetch correlation identifier(s) within the "fetch-correlation-ids" attribute.

Upon the reception of the HTTP GET request, the ADRF shall:

-	find the data or analytics according to the requested parameters.

If the requested data or analytics is found, the ADRF shall respond with "200 OK" status code with the message body containing the NadrfDataStoreRecord data structure. The NadrfDataStoreRecord data structure in the response body shall include:

-	one of the following:

-	information about network analytics function events that occurred in the "anaNotifications" attribute together with the corresponding subscription information within the "anaSub" attribute;

-	information about data event within the "dataNotif" attribute together with the corresponding subscription information within the "dataSub" attribute.

If the requested analytics or data does not exist, the ADRF shall respond with "204 No Content". If an error occurs when processing the HTTP GET request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

4.2.2.6	Nadrf_DataManagement_RetrievalSubscribe service operation

4.2.2.6.1	General

The Nadrf_DataManagement_RetrievalSubscribe service operation is used by an NF service consumer to subscribe to the ADRF to retrieve via notifications data or analytics that is stored in the ADRF and to receive future notifications with data or analytics when they are received by the ADRF.

4.2.2.6.2	Requesting retrieval and subscription of data or analytics

Figure 4.2.2.6.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to retrieve and subscribe to data or analytics.

Figure 4.2.2.6.2-1: NF service consumer requesting to retrieve and subscribe to data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_RetrievalSubscribe service operation to retrieve and subscribe to data or analytics. The NF service consumer shall send an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions" as Resource URI representing the "ADRF Data Retrieval Subscriptions" resource, as shown in figure 4.2.2.6.2-1, step 1, to create an "Individual ADRF Data Retrieval Subscription" according to the information in the message body. The NadrfDataRetrievalSubscription data structure provided in the request body shall include:

-	notification correlation identfier within the "notifCorrId" attribute;

-	one of the following:

-	analytics subscription information within the "anaSub" attribute;

-	data subscription information within the "dataSub" attribute;

-	a notification target address within the "notificationURI" attribute;

-	a time window for the data retrieval and subscription within the "timePeriod" attribute;

and may include:

-	a Consumer triggered Notification indication within the "consTrigNotif" attribute.

Upon the reception of an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions" as Resource URI and NadrfDataRetrievalSubscription data structure as request body, the ADRF shall:

-	create a new subscription;

-	assign a subscriptionId;

-	store the subscription.

If the ADRF created an "Individual ADRF Data Retrieval Subscription" resource, the ADRF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 4.2.2.6.2-1, step 2. The ADRF shall include a Location HTTP header field. The Location header field shall contain the URI of the created record i.e. "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions/{subscriptionId}".

If an error occurs when processing the HTTP POST request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

4.2.2.7	Nadrf_DataManagement_RetrievalUnsubscribe service operation

4.2.2.7.1	General

The Nadrf_DataManagement_RetrievalUnsubscribe service operation is used by an NF service consumer to remove a retrieval subscription to data or analytics.

4.2.2.7.2	Requesting removal of retrieval subscription for data or analytics

Figure 4.2.2.7.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to remove a retrieval subscription for data or analytics.

Figure 4.2.2.7.2-1: NF service consumer requesting to remove retrieval subscription for data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_RetrievalUnsubscribe service operation to remove a retrieval subscription for data or analytics. The NF service consumer shall send an HTTP DELETE request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions/{subscriptionId}" as Resource URI representing an "Individual ADRF Data Retrieval Subscription" resource, as shown in figure 4.2.2.7.2-1, step 1, where "{subscriptionId}" is the identifier of the existing data retrieval subscription that is to be deleted.

Upon the reception of an HTTP DELETE request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions/{subscriptionId}" as Resource URI, if the ADRF successfully processed and accepted the received HTTP DELETE request, the ADRF shall:

-	remove the corresponding subscription;

-	respond with HTTP "204 No Content" status.

If errors occur when processing the HTTP DELETE request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

If the ADRF determines the received HTTP DELETE request needs to be redirected, the ADRF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

4.2.2.8	Nadrf_DataManagement_RetrievalNotify service operation

4.2.2.8.1	General

The Nadrf_DataManagement_RetrievalNotify service operation is used by ADRF to notify NF service consumers about subscribed events related to data or analytics.

4.2.2.8.2	Notification about subscribed data or analytics

Figure 4.2.2.8.2-1 shows a scenario where the ADRF sends a request to the NF service consumer to notify it about data or analytics events.

Figure 4.2.2.8.2-1: ADRF notifies the NF service consumer about subscribed data or analytics

The ADRF shall invoke the Nadrf_DataManagement_RetrievalNotify service operation to notify about subscribed data or analytics events. The ADRF shall send an HTTP POST request to the "{notificationURI}" received in the subscription (see clause 5.1.5 for the definition of this notificationURI), as shown in figure 4.2.2.8.2-1, step 1. The NadrfDataRetrievalNotification data structure provided in the request body shall include:

-	notification correlation Id within the "notifCorrId" attribute;

-	the time stamp which represents the time when ADRF completes preparation of the requested data or analytics within the "timeStamp" attribute;

-	one of the following:

-	information about network data analytics function events that occurred in the "anaNotifications" attribute;

-	data collected from data sources (e.g. SMF, NEF) in the "dataNotif" attribute;

-	information for fetching the data or analytics in the "fetchInstruct" attribute.

NOTE:	The fetch correlation identifiers included in the fetch instructions of the "fetchInstruct" attribute can be used to fetch data or analytics using the Nadrf_DataManagement_RetrievalRequest service operation as described in clause 4.2.2.5.2. The (mandatory) fetch URI included in the fetch instructions of the "fetchInstruct" attribute is expected to be in line with the standard resource URI defined for the Nadrf_DataManagement_RetrievalRequest service operation, i.e. {apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records, but it can be anything because it is actually not needed by the NF service consumer in this case.

The NadrfDataRetrievalNotification data structure provided in the request body may include:

-	a termination request provided by the ADRF within the "terminationReq" attribute.

Upon the reception of an HTTP POST request with "{notificationURI}" as Resource URI and NadrfDataRetrievalNotification data structure as request body, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF Service Consumer shall:

-	store the notification;

-	respond with HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.1.7.

If the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

4.2.2.9	Nadrf_DataManagement_Delete service operation

4.2.2.9.1	General

The Nadrf_DataManagement_Delete service operation is used by an NF service consumer to delete stored data or analytics.

4.2.2.9.2	Requesting removal of stored data or analytics

Figure 4.2.2.9.2-1 shows a scenario where the NF service consumer sends a request to the ADRF to delete stored data or analytics.

Figure 4.2.2.9.2-1: NF service consumer requesting to remove stored data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_Delete service operation to remove stored data or analytics. The NF service consumer shall send an HTTP DELETE request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records/{storeTransId}" as Resource URI representing an "Individual ADRF Data Store Record" resource, as shown in figure 4.2.2.9.2-1, step 1, where "{storeTransId}" is the transaction identifier of the stored record that is to be deleted.

Upon the reception of an HTTP DELETE request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records/{storeTransId}" as Resource URI, if the ADRF successfully processed and accepted the received HTTP DELETE request, the ADRF shall:

-	remove the corresponding stored record;

-	respond with HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

If the ADRF determines the received HTTP DELETE request needs to be redirected, the ADRF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

4.2.2.9.3	Requesting removal of stored data or analytics using data or analytics specification

Figure 4.2.2.9.3-1 shows a scenario where the NF service consumer sends a request to the ADRF to delete stored data or analytics based on a data or analytics specification.

Figure 4.2.2.9.3-1: NF service consumer requesting to remove stored data or analytics

The NF service consumer shall invoke the Nadrf_DataManagement_Delete service operation to remove stored data or analytics based on a data or analytics specification. The NF service consumer shall send an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/remove-stored-data-analytics" as URI, as shown in figure 4.2.2.9.3-1, step 1. The POST request body shall contain an NadrfStoredDataSpec data structure. The NadrfStoredDataSpec data structure provided in the request body shall include:

-	a time window in which the data to be deleted was collected in the "timePeriod" attribute; and

-	one of the following:

-	a data specification in the "dataSpec" attribute;

-	an analytics specification in the "anaSpec" attribute;

Upon the reception of an HTTP POST request with "{apiRoot}/nadrf-datamanagement/<apiVersion>/remove-stored-data-analytics" as URI, if the ADRF successfully processed and accepted the received HTTP POST request, the ADRF shall respond with HTTP "204 No Content" status. The ADRF shall remove any stored analytics or data that match the analytics or data specification received in the request.

If errors occur when processing the HTTP POST request, the ADRF shall send an HTTP error response as specified in clause 5.1.7.

5	API Definitions

5.1	Nadrf_DataManagement Service API

5.1.1	Introduction

The Nadrf_DataManagement service shall use the Nadrf_DataManagement API.

The API URI of the Nadrf_DataManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nadrf-datamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.1.3.

5.1.2	Usage of HTTP

5.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nadrf_DataManagement API is contained in Annex A.

5.1.2.2	HTTP standard headers

5.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

5.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

5.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

5.1.3	Resources

5.1.3.1	Overview

This clause describes the structure for the Resource URIs, the resources and methods used for the service.

Figure 5.1.3.1-1 depicts the resource URIs structure for the Nadrf_DataManagement API.

Figure 5.1.3.1-1: Resource URI structure of the Nadrf_DataManagement API

Table 5.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ADRF Data Store Records | /data-store-records | GET | Retrieve the stored data or analytics |
|  |  | POST | Create a new Individual Data Store resource. |
| Individual ADRF Data Store Record | /data-store-records/{storeTransId} | DELETE | Delete an individual ADRF Data Store Record identified by {storeTransId}. |
| ADRF Data Retrieval Subscriptions | /data-retrieval-subscriptions | POST | Create a new Individual ADRF Data Retrieval Subscription resource. |
| Individual ADRF Data Retrieval Subscription | /data-retrieval-subscriptions/{subscriptionId} | DELETE | Delete an individual ADRF Data Retrieval Subscription identified by {subscriptionId}. |

5.1.3.2	Resource: ADRF Data Store Records

5.1.3.2.1	Description

The ADRF Data Store Records resource represents all data storage records to the Nadrf_DataManagement Service at a given ADRF. The resource allows an NF service consumer to create a new Individual ADRF Data Store Record resource and to retrieve Individual ADRF Data Store Record resources that fulfil certain criteria.

5.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.2.2-1.

Table 5.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |

5.1.3.2.3	Resource Standard Methods

5.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.1.3.2.3.1-1.

Table 5.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.2.3.1-2 and the response data structures and response codes specified in table 5.1.3.2.3.1-3.

Table 5.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NadrfDataStoreRecord | M | 1 | New individual Data Store Record to be created |

Table 5.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NadrfDataStoreRecord | M | 1 | 201 Created | The creation of an Individual Data Store Record resource is confirmed, and a representation of that resource is returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.1.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records/{storeTransId} |

5.1.3.2.3.2	GET

This method shall support the URI query parameters specified in table 5.1.3.2.3.2-1.

Table 5.1.3.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| store-trans-id | string | O | 0..1 | Identifies the "Storage Transaction Identifier" of data store record in ADRF. (NOTE) |
| fetch-correlation-ids | array(string) | O | 1..N | Identifies fetch correlation identifiers received as part of fetch instruction. (NOTE) |
| NOTE: Either "store-trans-id" or "fetch-correlation-ids" shall be provided. |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.2.3.1-2 and the response data structures and response codes specified in table 5.1.3.2.3.1-3.

Table 5.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NadrfDataStoreRecord | M | 1 | 200 OK | Data Store record. |
| n/a |  |  | 204 No Content | If the request ADRF Data Store Record does not exist, the ADRF shall respond with "204 No Content ". |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.1.3.2.4	Resource Custom Operations

None.

5.1.3.3	Resource: Individual ADRF Data Store Record

5.1.3.3.1	Description

The Individual ADRF Data Store Record resource represents data or analytics stored via the Nadrf_DataManagement_StorageRequest in ADRF.

5.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records/{storeTransId}

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.3.2-1.

Table 5.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1. |
| storeTransId | string | Identifies an individual data store record. |

5.1.3.3.3	Resource Standard Methods

5.1.3.3.3.1	DELETE

This method shall support the URI query parameters specified in table 5.1.3.3.3.1-1.

Table 5.1.3.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.1-2 and the response data structures and response codes specified in table 5.1.3.3.3.1-3.

Table 5.1.3.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Individual ADRF Data Store Record resource was deleted successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual ADRF Data Store Record deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADRF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual ADRF Data Store Record deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADRF (service) instance. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADRF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 5.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADRF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

5.1.3.3.4	Resource Custom Operations

None in this release of the specification.

5.1.3.4	Resource: ADRF Data Retrieval Subscriptions

5.1.3.4.1	Description

The ADRF Data Retrieval Subscriptions resource represents all data retrieval subscriptions to the Nadrf_DataManagement Service at a given ADRF. The resource allows an NF service consumer to create a new Individual ADRF Data Retrieval Subscription resource.

5.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.4.2-1.

Table 5.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |

5.1.3.4.3	Resource Standard Methods

5.1.3.4.3.1	POST

This method shall support the URI query parameters specified in table 5.1.3.4.3.1-1.

Table 5.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.4.3.1-2 and the response data structures and response codes specified in table 5.1.3.4.3.1-3.

Table 5.1.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NadrfDataRetrievalSubscription | M | 1 | Individual ADRF Data Retrieval Subscription resource to be created. |

Table 5.1.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NadrfDataRetrievalSubscription | M | 1 | 201 Created | The creation of an Individual ADRF Data Retrieval Subscription resource is confirmed and a representation of that resource is returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.4.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions/{subscriptionId} |

5.1.3.4.4	Resource Custom Operations

None in this release of the specification.

5.1.3.5	Resource: Individual ADRF Data Retrieval Subscription

5.1.3.5.1	Description

The Individual ADRF Data Retrieval Subscription resource represents single ADRF data retrieval subscription to the Nadrf_DataManagement Service at a given ADRF. The resource allows an NF service consumer to delete Individual ADRF Data Retrieval Subscription resource.

5.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.1.3.5.2-1.

Table 5.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |
| subscriptionId | string | Identifies a subscription to the Nadrf_DataManagement service. |

5.1.3.5.3	Resource Standard Methods

5.1.3.5.3.1	DELETE

This method shall support the URI query parameters specified in table 5.1.3.5.3.1-1.

Table 5.1.3.5.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.5.3.1-2 and the response data structures and response codes specified in table 5.1.3.5.3.1-3.

Table 5.1.3.5.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.5.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Individual ADRF Data Retrieval Subscription resource was deleted successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual ADRF Data Retrieval Subscription deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADRF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual ADRF Data Retrieval Subscription deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADRF (service) instance. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADRF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 5.1.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADRF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

5.1.3.5.4	Resource Custom Operations

None in this release of the specification.

5.1.4	Custom Operations without associated resources

5.1.4.1	Overview

The structure of the custom operation URIs of the Nadrf_DataManagement service is shown in Figure 5.1.4.1-1.

Figure 5.1.4.1-1: Custom operation URI structure of the Nadrf_DataManagement API

Table 5.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 5.1.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/nadrf-datamanagement/<apiVersion>/request-storage-sub | POST | Request the ADRF to create a subscription for data or analytics and then store the received data or analytics in the ADRF. |
| {apiRoot}/nadrf-datamanagement/<apiVersion>/request-storage-sub-removal | POST | Request the ADRF to remove a subscription for data or analytics. |
| {apiRoot}/nadrf-datamanagement/<apiVersion>/remove-stored-data-analytics | POST | Request the ADRF to remove already stored data or analytics. |

5.1.4.2	Operation: request-storage-sub

5.1.4.2.1	Description

The operation is used by the NF service consumer to request the ADRF to create a subscription for data or analytics and then store the received data or analytics in the ADRF.

5.1.4.2.2	Operation Definition

This operation shall support the request data structures shown in Table 5.1.4.2.2-1 and the response data structures and error codes specified in Tables 5.1.4.2.2-2.

Table 5.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NadrfDataStoreSubscription | M | 1 | Information about the storage subscription that the ADRF shall create. |

Table 5.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NadrfDataStoreSubscriptionRef | M | 1 | 200 OK | Successful request to trigger the creation of a subscription for data or analytics at the ADRF. A reference is provided. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.1.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.1.4.3	Operation: request-storage-sub-removal

5.1.4.3.1	Description

The operation is used by the NF service consumer to request the ADRF to remove a subscription for data or analytics which was used to store the received data or analytics in the ADRF.

5.1.4.3.2	Operation Definition

This operation shall support the request data structures shown in Table 5.1.4.3.2-1 and the response data structures and error codes specified in Tables 5.1.4.3.2-2.

Table 5.1.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NadrfDataStoreSubscriptionRef | M | 1 | Reference used to identify the subscription that the ADRF shall remove. |

Table 5.1.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful request to trigger the removal of a subscription for data or analytics at the ADRF. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.1.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.1.4.4	Operation: remove-stored-data-analytics

5.1.4.4.1	Description

The operation is used by the NF service consumer to request the ADRF to remove stored data or analytics based on a data or analytics specification.

5.1.4.4.2	Operation Definition

This operation shall support the request data structures shown in Table 5.1.4.4.2-1 and the response data structures and error codes specified in Tables 5.1.4.4.2-2.

Table 5.1.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NadrfStoredDataSpec | M | 1 | Information about the specification of data or analytics stored in ADRF. |

Table 5.1.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful request to remove data or analytics at the ADRF based on a data or analytics specification. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.1.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.1.5	Notifications

5.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 5.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Retrieval Notification | {notificationURI} | POST | Report data or analytics from ADRF. |

5.1.5.2	Retrieval Notification

5.1.5.2.1	Description

The Retrieval Notification is used by the NF service producer to report one or several data retrieval events to an NF service consumer that has subscribed to such notifications.

5.1.5.2.2	Target URI

The Callback URI "{notificationURI}" shall be used with the callback URI variables defined in table 5.1.5.2.2-1.

Table 5.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationURI | String formatted as URI with the Callback Uri |

5.1.5.2.3	Standard Methods

5.1.5.2.3.1	POST

This method shall support the request data structures specified in table 5.1.5.2.3.1-1 and the response data structures and response codes specified in table 5.1.5.2.3.1-2.

Table 5.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NadrfDataRetrievalNotification | M | 1 | Provides information about observed data or analytics. |

Table 5.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during the retrieval notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during the retrieval notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 5.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

5.1.6	Data Model

5.1.6.1	General

This clause specifies the application data model supported by the Nadrf_DataManagement API.

Table 5.1.6.1-1 specifies the data types defined for the Nadrf_DataManagement service based interface protocol.

Table 5.1.6.1-1: Nadrf_DataManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| DataNotification | 5.1.6.2.9 | Represents a data subscription notification of one of various possible data sources. |  |
| DataSubscription | 5.1.6.2.8 | Contains information about Data specification. |  |
| NadrfDataRetrievalNotification | 5.1.6.2.5 | Represents a notification that corresponds with an Individual ADRF Data Retrieval Subscription resource. |  |
| NadrfDataRetrievalSubscription | 5.1.6.2.4 | Represents an Individual ADRF Data Retrieval Subscription resource. |  |
| NadrfDataStoreRecord | 5.1.6.2.2 | Represents an Individual ADRF Data Store Record resource. |  |
| NadrfDataStoreSubscription | 5.1.6.2.3 | Contains information to be used by the ADRF to create a Data or Analytics subscription. |  |
| NadrfDataStoreSubscriptionRef | 5.1.6.2.6 | Contains a reference to a request for a Data or Analytics subscription. |  |
| NadrfStoredDataSpec | 5.1.6.2.7 | Contains information about Data or Analytics specification. |  |

Table 5.1.6.1-2 specifies data types re-used by the Nadrf_DataManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nadrf_DataManagement service based interface.

Table 5.1.6.1-2: Nadrf_DataManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AfEventExposureNotif | 3GPP TS 29.517 [20] | Represents notifications on AF event(s) that occurred for an Individual AF Event Subscription resource. |  |
| AfEventExposureSubsc | 3GPP TS 29.517 [20] | Represents AF event subscription. |  |
| AmfEventNotification | 3GPP TS 29.518 [18] | Represents notifications on AMF event(s) that occurred for an Individual AMF Event Subscription resource. |  |
| AmfEventSubscription | 3GPP TS 29.518 [18] | Represents AMF event subscription. |  |
| DateTime | 3GPP TS 29.571 [16] | Identifies the time. |  |
| EeSubscription | 3GPP TS 29.503 [19] | Represents UDM event subscription. |  |
| FetchInstruction | 3GPP TS 29.576 [24] | The fetch instruction indicates that the data or analytics can be fetched by the consumer. |  |
| FormattingInstruction | 3GPP TS 29.574 [23] | DCCF formatting Instructions. |  |
| MonitoringReport | 3GPP TS 29.503 [19] | UDM Monitoring Report. |  |
| NefEventExposureNotif | 3GPP TS 29.591 [21] | Represents notifications on network exposure event(s) that occurred for an Individual Network Exposure Event Subscription resource. |  |
| NefEventExposureSubsc | 3GPP TS 29.591 [21] | Represents NEF event subscription. |  |
| NfInstanceId | 3GPP TS 29.571 [16] | NF instance identifier. |  |
| NfSetId | 3GPP TS 29.571 [16] | NF set identifier. |  |
| NnwdafEventsSubscription | 3GPP TS 29.520 [15] | Represents an NWDAF analytics subscription. |  |
| NnwdafEventsSubscriptionNotification | 3GPP TS 29.520 [15] | Represents an NWDAF analytics subscription notification. |  |
| NotificationData | 3GPP TS 29.510 [10] | Represents an NRF event notification. |  |
| NsmfEventExposure | 3GPP TS 29.508 [17] | Represents SMF event subscription. |  |
| NsmfEventExposureNotification | 3GPP TS 29.508 [17] | Represents SMF event notification. |  |
| ProcessingInstruction | 3GPP TS 29.574 [23] | DCCF processing Instructions. |  |
| SACEventReport | 3GPP TS 29.536 [25] | Represents an NSACF event notification. |  |
| SACEventSubscription | 3GPP TS 29.536 [25] | Represents and NSACF event subscription. |  |
| SubscriptionData | 3GPP TS 29.510 [10] | Represents an NRF event subscription. |  |
| SupportedFeatures | 3GPP TS 29.571 [6] | Used to negotiate the applicability of the optional features defined in table 5.1.8-1. |  |
| TimeWindow | 3GPP TS 29.122 [22] | Represents a time window. |  |
| Uri | 3GPP TS 29.571 [16] | URI. |  |

5.1.6.2	Structured data types

5.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

5.1.6.2.2	Type: NadrfDataStoreRecord

Table 5.1.6.2.2-1: Definition of type NadrfDataStoreRecord

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE 1) |  |
| anaNotifications | array(NnwdafEventsSubscriptionNotification) | C | 1..N | List of analytics subscription notifications. (NOTE 1) |  |
| anaSub | NnwdafEventsSubscription | C | 0..1 | Represents the subscription information of the corresponding analytics notification. Shall be present if the "anaNotifications" attribute is provided. (NOTE 2) |  |
| dataSub | DataSubscription | C | 0..1 | Represents the subscription information of the corresponding data notification. Shall be present if the "dataNotif" attribute is provided. (NOTE 2) |  |
| NOTE 1: Exactly one of the attributes "anaNotifications" and "dataNotif" shall be provided. NOTE 2: Exactly one of the attributes "anaSub" and "dataSub" shall be provided. |  |  |  |  |  |

5.1.6.2.3	Type: NadrfDataStoreSubscription

Table 5.1.6.2.3-1: Definition of type NadrfDataStoreSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaSub | NnwdafEventsSubscription | C | 0..1 | Subscribed analytics events. (NOTE 1) |  |
| dataSub | DataSubscription | C | 0..1 | Represents requested Events subscription. (NOTE 1) |  |
| targetNfId | NfInstanceId | C | 0..1 | DCCF or NWDAF NF instance identifier to which the ADRF shall create the requested subscription. (NOTE 2) |  |
| targetNfSetId | NfSetId | C | 0..1 | DCCF or NWDAF NF set identifier to which the ADRF shall create the requested subscription. (NOTE 2) |  |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. |  |
| procInstruct | ProcessingInstruction | O | 0..1 | Processing instructions to be used for sending event notifications. (NOTE 3) |  |
| multiProcInstructs | array(ProcessingInstruction) | O | 1..N | Processing instructions to be used for sending event notifications. (NOTE 3) | MultiProcessingInstruction |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features as described in clause 5.1.8. It shall be present if at least one feature defined in clause 5.1.8 is supported. |  |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE 2: One of "targetNfId" and "targetNfSetId" shall be provided. NOTE 3: The "multiProcInstructs" attribute shall be used instead of the "procInstruct" attribute when the "MultiProcessingInstruction" feature is supported. |  |  |  |  |  |

5.1.6.2.4	Type: NadrfDataRetrievalSubscription

Table 5.1.6.2.4-1: Definition of type NadrfDataRetrievalSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaSub | NnwdafEventsSubscription | C | 0..1 | Subscribed analytics events. (NOTE 1) |  |
| dataSub | DataSubscription | C | 0..1 | Represents requested Events subscription. (NOTE 1) |  |
| notificationURI | Uri | M | 1 | Notification target address. |  |
| timePeriod | TimeWindow | M | 1 | Represents a start time and a stop time during which the requested data is collected and/or will be collected at the data source. |  |
| notifCorrId | string | M | 1 | Notification correlation identifier provided by the NF service consumer to be used later by the ADRF in the notifications that correspond with this subscription. The value of this attribute shall be unique per subscription for a given NF service consumer. |  |
| consTrigNotif | boolean | O | 0..1 | If provided and set to "true", it indicates that notifications shall be buffered (sending only fetch instructions to the NF service consumer) until the NF service consumer requests their delivery using Nadrf_DataManagement Service. The default value is "false". |  |
| NOTE 1: Exactly one of these attributes shall be provided. |  |  |  |  |  |

5.1.6.2.5	Type: NadrfDataRetrievalNotification

Table 5.1.6.2.5-1: Definition of type NadrfDataRetrievalNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorrId | string | M | 1 | This attribute indicates the notification correlation identifier provided by the NF service consumer during the data retrieval subscription. This parameter can be useful if the NF service consumer uses a common callback URI for multiple subscriptions. |  |
| anaNotifications | array(NnwdafEventsSubscriptionNotification) | C | 1..N | List of analytics subscription notifications. (NOTE) |  |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE) |  |
| fetchInstruct | FetchInstruction | C | 0..1 | The fetch instruction indicates that the data or analytics can be fetched by the consumer. (NOTE) |  |
| terminationReq | boolean | O | 0..1 | If provided and set to "true", it indicates that the subscription is requested to be terminated, i.e. no further notifications related to this subscription will be provided. The default value is "false". |  |
| timeStamp | DateTime | M | 1 | It represents the time when ADRF completed preparation of the requested data or analytics. |  |
| NOTE: Exactly one of these attributes shall be provided. |  |  |  |  |  |

5.1.6.2.6	Type: NadrfDataStoreSubscriptionRef

Table 5.1.6.2.6-1: Definition of type NadrfDataStoreSubscriptionRef

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transRefId | string | M | 1 | Transaction reference identifier. |  |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features as described in clause 5.1.8. It shall be present in the POST response if the NF service consumer includes the "suppFeat" attribute in the POST request. |  |

5.1.6.2.7	Type: NadrfStoredDataSpec

Table 5.1.6.2.7-1: Definition of type NadrfStoredDataSpec

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSpec | DataSubscription | C | 0..1 | Represents data specification. (NOTE) |  |
| anaSpec | NnwdafEventsSubscription | C | 0..1 | Represents analytics specification. (NOTE) |  |
| timePeriod | TimeWindow | M | 1 | Represents a start time and a stop time during which the requested data to be removed was collected at the data source. |  |
| NOTE: Exactly one of these attributes shall be provided. |  |  |  |  |  |

5.1.6.2.8	Type: DataSubscription

Table 5.1.6.2.8-1: Definition of type DataSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| amfDataSub | AmfEventSubscription | C | 0..1 | Represents requested AMF Events subscription. (NOTE) |  |
| smfDataSub | NsmfEventExposure | C | 0..1 | Represents requested SMF Events subscription. (NOTE) |  |
| udmDataSub | EeSubscription | C | 0..1 | Represents requested UDM Events subscription. (NOTE) |  |
| nefDataSub | NefEventExposureSubsc | C | 0..1 | Represents requested NEF Events subscription. (NOTE) |  |
| afDataSub | AfEventExposureSubsc | C | 0..1 | Represents requested AF Events subscription. (NOTE) |  |
| nrfDataSub | SubscriptionData | C | 0..1 | Represents requested NRF Events subscription. (NOTE) |  |
| nsacfDataSub | SACEventSubscription | C | 0..1 | Represents requested NSACF Events subscription. (NOTE) |  |
| NOTE: Exactly one of these attributes shall be provided. |  |  |  |  |  |

5.1.6.2.9	Type: DataNotification

Table 5.1.6.2.9-1: Definition of type DataNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afEventNotifs | array(AfEventExposureNotif) | C | 1..N | List of notifications on AF event(s). (NOTE 1) |  |
| amfEventNotifs | array(AmfEventNotification) | C | 1..N | List of notifications on AMF event(s). (NOTE 1) |  |
| smfEventNotifs | array(NsmfEventExposureNotification) | C | 1..N | List of notifications on SMF event(s). (NOTE 1) |  |
| udmEventNotifs | array(MonitoringReport) | C | 1..N | List of monitoring reports containing information about UDM event(s). (NOTE 1) |  |
| nefEventNotifs | array(NefEventExposureNotif) | C | 1..N | List of notifications on network exposure event(s). (NOTE 1) |  |
| nrfEventNotifs | array(NotificationData) | C | 1..N | List of notifications on NRF event(s). (NOTE 1) |  |
| nsacfEventNotifs | array(SACEventReport) | C | 1..N | List of notifications on NSACF event(s). (NOTE 1) |  |
| timeStamp | DateTime | O | 0..1 | Indicates the timestamp for the event(s). (NOTE 2) |  |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE 2: The "timeStamp" attribute within the DataNotification data type may be provided if any of the "timeStamp" attribute within AfEventNotification contained in the AfEventExposureNotif, or within AmfEventReport contained in the AmfEventNotification, or within EventNotification contained in the NsmfEventExposureNotification, or within MonitoringReport, or within NefEventNotification contained in the NefEventExposureNotif or within SACEventReportItem contained in the SACEventReport data type is not provided. |  |  |  |  |  |

5.1.6.3	Simple data types and enumerations

None.

5.1.6.4	Data types describing alternative data types or combinations of data types

None.

5.1.7	Error Handling

5.1.7.1	General

For the Nadrf_DataManagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.1.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.1.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nadrf_DataManagement API.

5.1.7.2	Protocol Errors

No specific procedures for the Nadrf_DataManagement service are specified.

5.1.7.3	Application Errors

The application errors defined for the Nadrf_DataManagement  service are listed in Table 5.1.7.3-1.

Table 5.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

5.1.8	Feature negotiation

The optional features in table 5.1.8-1 are defined for the Nadrf_DataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | MultiProcessingInstruction | Indicates the support of multiple processing instructions. |

5.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nadrf_DataManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nadrf_DataManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nadrf_DataManagement service.

The Nadrf_DataManagement API defines a single scope "nadrf-datamanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

A.2	Nadrf_DataManagement API

openapi: 3.0.0

info:

version: 1.1.0-alpha.1

title: Nadrf_DataManagement

description: |

ADRF Data Management Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.575 V18.0.0; 5G System; Analytics Data Repository Services; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.575/'

#

servers:

- url: '{apiRoot}/nadrf-datamanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

#

security:

- oAuth2ClientCredentials:

- nadrf-datamanagement

- {}

#

paths:

/data-store-records:

post:

summary: Creates a new Individual Data Store Record resource.

operationId: CreateADRFDataStoreRecord

tags:

- ADRF Data Store Records (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataStoreRecord'

required: true

description: ADRF data store record to be stored.

responses:

'201':

description: Successful creation of new Individual ADRF Data Store Record resource.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nadrf-datamanagement/<apiVersion>/data-store-records/{storeTransId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataStoreRecord'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves existing Individual ADRF Data Store Records.

operationId: GetAdrfDataStoreRecords

tags:

- ADRF Data Store Records (Collection)

parameters:

- name: store-trans-id

description: A storage transaction identifier of a data store record in ADRF.

in: query

required: false

schema:

type: string

- name: fetch-correlation-ids

description: Fetch correlation identifiers received as part of fetch instruction.

in: query

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

responses:

'200':

description: Data store records are returned.

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataStoreRecord'

'204':

description: No matching ADRF data were found.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/data-store-records/{storeTransId}:

delete:

summary: Delete an existing Individual ADRF Data Store Record.

operationId: DeleteADRFDataStoreRecord

tags:

- Individual ADRF Data Store Record (Document)

parameters:

- name: storeTransId

in: path

description: String identifying a Data Store Record in ADRF.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual ADRF Data Store Record resource matching the

storeTransId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/data-retrieval-subscriptions:

post:

summary: Creates a new Individual ADRF Data Retrieval Subscription resource.

operationId: CreateADRFDataRetrievalSubscription

tags:

- ADRF Data Retrieval Subscriptions (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataRetrievalSubscription'

required: true

description: Individual ADRF Data Retrieval Subscription resource to be created.

responses:

'201':

description: Created a new Individual ADRF Data Retrieval Subscription resource.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nadrf-datamanagement/<apiVersion>/data-retrieval-subscriptions/{subscriptionId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataRetrievalSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

adrfDataRetrievalNotification:

'{$request.body#/notificationURI}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataRetrievalNotification'

responses:

'204':

description: The receipt of the Notification is acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/data-retrieval-subscriptions/{subscriptionId}:

delete:

summary: Delete an existing Individual ADRF Data Retrieval Subscription resource.

operationId: DeleteADRFDataRetrievalSubscription

tags:

- Individual ADRF Data Retrieval Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

String identifying a data retrieval subscription to the Nadrf_DataManagement

Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual ADRF Data Retrieval Subscription resource matching

the subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/request-storage-sub:

post:

summary: Triggers the creation of a new ADRF Storage Subscription.

operationId: CreateADRFStorageSubscription

tags:

- ADRF Storage Subscriptions

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataStoreSubscription'

required: true

responses:

'200':

description: >

Successful response with reference used to identify the subscription at the ADRF.

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataStoreSubscriptionRef'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/request-storage-sub-removal:

post:

summary: Triggers the removal of ADRF storage subscription.

operationId: DeleteADRFStorageSubscription

tags:

- ADRF Storage Subscriptions

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfDataStoreSubscriptionRef'

required: true

responses:

'204':

description: >

No Content. The ADRF Storage Subscription matching the provided reference was deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/remove-stored-data-analytics:

post:

summary: Remove ADRF data based on data or analytics specification.

operationId: DeleteADRFData

tags:

- ADRF Stored Data

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NadrfStoredDataSpec'

required: true

responses:

'204':

description: No Content. The ADRF data matching the provided specification is deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

#

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nadrf-datamanagement: Access to the nadrf-datamanagement API

#

schemas:

#

NadrfDataStoreRecord:

description: Represents an Individual ADRF Data Store Record.

type: object

oneOf:

- allOf:

- required: [anaSub]

- required: [anaNotifications]

- allOf:

- required: [dataSub]

- required: [dataNotif]

properties:

dataNotif:

$ref: '#/components/schemas/DataNotification'

anaNotifications:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscriptionNotification'

minItems: 1

description: List of analytics subscription notifications.

anaSub:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscription'

minItems: 1

description: >

Represents the subscription information of the corresponding analytics notification.

dataSub:

type: array

items:

$ref: '#/components/schemas/DataSubscription'

minItems: 1

description: >

Represents the subscription information of the corresponding data notification.

#

NadrfDataStoreSubscription:

description: >

Contains information to be used by the ADRF to create a Data or Analytics subscription.

type: object

allOf:

- oneOf:

- required: [anaSub]

- required: [dataSub]

- oneOf:

- required: [targetNfId]

- required: [targetNfSetId]

properties:

anaSub:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscription'

dataSub:

$ref: '#/components/schemas/DataSubscription'

targetNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

targetNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

formatInstruct:

$ref: 'TS29574_Ndccf_DataManagement.yaml#/components/schemas/FormattingInstruction'

procInstruct:

$ref: 'TS29574_Ndccf_DataManagement.yaml#/components/schemas/ProcessingInstruction'

multiProcInstructs:

type: array

items:

$ref: 'TS29574_Ndccf_DataManagement.yaml#/components/schemas/ProcessingInstruction'

minItems: 1

description: Processing instructions to be used for sending event notifications.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

#

NadrfDataRetrievalSubscription:

description: Represents an Individual ADRF Data Retrieval Subscription.

type: object

required:

- notifCorrId

- notificationURI

- timePeriod

oneOf:

- required: [anaSub]

- required: [dataSub]

properties:

anaSub:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscription'

dataSub:

$ref: '#/components/schemas/DataSubscription'

notificationURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

timePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

notifCorrId:

type: string

description: Notification correlation identifier.

consTrigNotif:

type: boolean

description: >

It indicates that notifications shall be buffered (sending only fetch instructions

to the NF service consumer) until the NF service consumer requests their delivery

using Nadrf_DataManagement Service.

#

NadrfDataRetrievalNotification:

description: >

Represents a notification that corresponds with an Individual ADRF Data

Retrieval Subscription.

type: object

required:

- notifCorrId

- timeStamp

oneOf:

- required: [anaNotifications]

- required: [dataNotif]

- required: [fetchInstruct]

properties:

notifCorrId:

type: string

description: Notification correlation identifier.

anaNotifications:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscriptionNotification'

minItems: 1

description: List of analytics subscription notifications.

dataNotif:

$ref: '#/components/schemas/DataNotification'

fetchInstruct:

$ref: 'TS29576_Nmfaf_3caDataManagement.yaml#/components/schemas/FetchInstruction'#

terminationReq:

type: boolean

description: >

It indicates the termination of the data management subscription that requested by the

ADRF.

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

#

NadrfDataStoreSubscriptionRef:

description: Contains a reference to a request for a Data or Analytics subscription.

type: object

required:

- transRefId

properties:

transRefId:

type: string

description: Transaction reference identifier.

#

NadrfStoredDataSpec:

description: Contains information about Data or Analytics specification.

type: object

required:

- timePeriod

oneOf:

- required: [dataSpec]

- required: [anaSpec]

properties:

dataSpec:

$ref: '#/components/schemas/DataSubscription'

anaSpec:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscription'

timePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

#

DataSubscription:

description: Contains a data specification.

type: object

oneOf:

- required: [amfDataSub]

- required: [smfDataSub]

- required: [udmDataSub]

- required: [nefDataSub]

- required: [afDataSub]

- required: [nrfDataSub]

- required: [nsacfDataSub]

properties:

amfDataSub:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/AmfEventSubscription'

smfDataSub:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NsmfEventExposure'

udmDataSub:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/EeSubscription'

afDataSub:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEventExposureSubsc'

nefDataSub:

$ref: 'TS29591_Nnef_EventExposure.yaml#/components/schemas/NefEventExposureSubsc'

nrfDataSub:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/SubscriptionData'

nsacfDataSub:

$ref: 'TS29536_Nnsacf_SliceEventExposure.yaml#/components/schemas/SACEventSubscription'

#

DataNotification:

description: Represents a Data Subscription Notification.

type: object

oneOf:

- required: [amfEventNotifs]

- required: [smfEventNotifs]

- required: [udmEventNotifs]

- required: [nefEventNotifs]

- required: [afEventNotifs]

- required: [nrfEventNotifs]

- required: [nsacfEventNotifs]

properties:

amfEventNotifs:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/AmfEventNotification'

minItems: 1

description: List of notifications of AMF events.

smfEventNotifs:

type: array

items:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NsmfEventExposureNotification'

minItems: 1

description: List of notifications of SMF events.

udmEventNotifs:

type: array

items:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/MonitoringReport'

minItems: 1

description: List of notifications of UDM events.

nefEventNotifs:

type: array

items:

$ref: 'TS29591_Nnef_EventExposure.yaml#/components/schemas/NefEventExposureNotif'

minItems: 1

description: List of notifications of NEF events.

afEventNotifs:

type: array

items:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEventExposureNotif'

minItems: 1

description: List of notifications of AF events.

nrfEventNotifs:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NotificationData'

minItems: 1

description: List of notifications of NRF events.

nsacfEventNotifs:

type: array

items:

$ref: 'TS29536_Nnsacf_SliceEventExposure.yaml#/components/schemas/SACEventReport'

minItems: 1

description: List of notifications of NSACF events.

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

#

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-06 | CT3#116e |  |  |  |  | TS skeleton of Analytics Data Repository Services specification | 0.0.0 |
| 2021-06 | CT3#116e | C3-213501 |  |  |  | Inclusion of documents agreed in CT3#116e C3-213537. | 0.1.0 |
| 2021-08 | CT3#117e | C3-214579 |  |  |  | Inclusion of documents agreed in CT3#117e C3-214478. | 0.2.0 |
| 2021-11 | CT3#119e | C3-216521 |  |  |  | Inclusion of documents agreed in CT3#119e C3-216443, C3-216455, C3-216456, C3-216457, C3-216458, C3-216459, C3-216460, C3-216462, C3-216469, C3-216591, C3-216592, C3-216593. | 0.3.0 |
| 2022-01 | CT3#119bis-e | C3-220454 |  |  |  | Inclusion of documents agreed in CT3#119bis-e C3-220500, C3-220512, C3-220513, C3-220440, C3-220441, C3-220369. | 0.4.0 |
| 2022-02 | CT3#120 | C3-221516 |  |  |  | Inclusion of documents agreed in CT3#120-e C3-221091, C3-221288, C3-221620, C3-221621. | 0.5.0 |
| 2022-03 | CT#95e | CP-220160 |  |  |  | Presentation to TSG CT for approval | 1.0.0 |
| 2022-03 | CT#95e | CP-220160 |  |  |  | Approved by TSG CT | 17.0.0 |
| 2022-06 | CT#96 | CP-221132 | 0001 | 1 | F | Adding 3XX response handling support for ADRF services | 17.1.0 |
| 2022-06 | CT#96 | CP-221136 | 0002 | 3 | B | Cleanup of Nadrf_DataManagement data model | 17.1.0 |
| 2022-06 | CT#96 | CP-221132 | 0003 | 1 | F | Corrections in the Nadrf_DataManagement data model | 17.1.0 |
| 2022-06 | CT#96 | CP-221129 | 0004 |  | F | Correct the Cardinality of some attributes | 17.1.0 |
| 2022-06 | CT#96 | CP-221134 | 0005 | 3 | B | Support removal of stored analytics and data from ADRF according to Analytics and Data Specification | 17.1.0 |
| 2022-06 | CT#96 | CP-221132 | 0006 | 1 | B | Support carrying Fetch Instructions in Nadrf_DataManagement_RetrievalNotify service operation | 17.1.0 |
| 2022-06 | CT#96 | CP-221133 | 0008 | 2 | F | Formatting of description fields | 17.1.0 |
| 2022-06 | CT#96 | CP-221130 | 0009 |  | F | Responses on DELETE method | 17.1.0 |
| 2022-06 | CT#96 | CP-221133 | 0011 | 2 | F | Clarification on duplicated data or analytics storage | 17.1.0 |
| 2022-06 | CT#96 | CP-221131 | 0012 | 1 | F | Correction on Nadrf_DataManagement_StorageRequest service operation | 17.1.0 |
| 2022-06 | CT#96 | CP-221133 | 0014 |  | F | Removal of repetitive description in HTTP error response | 17.1.0 |
| 2022-06 | CT#96 | CP-221135 | 0016 | 1 | F | Update inputs of Nadrf_DataManagement_RetrievalNotify service | 17.1.0 |
| 2022-06 | CT#96 | CP-221134 | 0017 |  | F | corrections to Abbreviations and Introduction | 17.1.0 |
| 2022-06 | CT#96 | CP-221134 | 0018 |  | F | correction to time period | 17.1.0 |
| 2022-06 | CT#96 | CP-221155 | 0019 | 1 | F | Update the apiVersion placeholder | 17.1.0 |
| 2022-06 | CT#96 | CP-221152 | 0020 |  | F | Update of info and externalDocs fields | 17.1.0 |
| 2022-09 | CT#97e | CP-222104 | 0021 | 1 | F | Update inputs of Nadrf_DataManagement_RetrievalNotify service | 17.2.0 |
| 2022-09 | CT#97e | CP-222103 | 0022 | 1 | F | Corrections in descriptions of the Nadrf_DataManagement_RetrievalRequest operation | 17.2.0 |
| 2022-09 | CT#97e | CP-222103 | 0023 | 1 | F | ADRF data retrieval notification data model updates | 17.2.0 |
| 2022-09 | CT#97e | CP-222103 | 0024 | 1 | F | Adding NRF and NSACF as data sources | 17.2.0 |
| 2022-09 | CT#97e | CP-222121 | 0027 |  | F | Update of info and externalDocs fields | 17.2.0 |
| 2022-12 | CT#98e | CP-223173 | 0028 | 1 | F | ADRF Retrieval Request inputs | 18.0.0 |
| 2022-12 | CT#98e | CP-223172 | 0029 |  | F | ADRF Storage Subscription Request handling inconsistencies | 18.0.0 |
| 2022-12 | CT#98e | CP-223191 | 0031 |  | F | Adding the mandatory error code 502 Bad Gateway | 18.0.0 |
| 2022-12 | CT#98e | CP-223176 | 0032 | 1 | F | Removal of non-sense statement for notificationURI | 18.0.0 |
| 2022-12 | CT#98e | CP-223176 | 0033 | 1 | F | adding Consumer triggered Notification indication for Nadrf_DataManagement_RetrievalSubscribe | 18.0.0 |
| 2022-12 | CT#98e | CP-223176 | 0034 | 1 | F | Correction of data type of procInstruct | 18.0.0 |
| 2022-12 | CT#98e | CP-223173 | 0035 | 1 | F | The time stamp of data notification | 18.0.0 |
| 2022-12 | CT#98e | CP-223173 | 0036 | 1 | F | Miscellaneous corrections | 18.0.0 |
| 2022-12 | CT#98e | CP-223192 | 0038 | 1 | F | Update the apiVersion in the specification | 18.0.0 |
| 2022-12 | CT#98e | CP-223190 | 0044 |  | F | Update of info and externalDocs fields | 18.0.0 |