3GPP TSG RAN WG1 Ad Hoc meeting for Channel Model	     R1-161677

Ljubljana, Slovenia, March 14-16, 2016

Agenda Item:	6.1

Source:	Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung

Title:	Joint proposal on large-scale parameters for >6 GHz channel model in indoor hotspot

Document for:	Discussion and decision

- 1	Introduction

In the RAN#69, a SID for above 6 GHz channel modelling was approved in [1]. In the RAN1#84 meeting, it was proposed jointly by 23 companies in [2] that three deployment scenarios should be investigated as first priority for the channel model SI. The indoor hotspot scenario is included in the scenario set. Since the indoor scenario was not investigated in the latest 3GPP channel modelling activities, many of the indoor channel characteristics have not been fully studied after LTE-Advanced channel modelling activities. Some recent measurements for indoor channels in above 6 GHz have been reported in [3] and [4]. Considering the agreed requirements for 5G channel modelling in [5], it is necessary to further investigate the channel characteristics of indoor hotspot.

In this contribution, the large-scale channel characteristics are outlined for frequency range up to 100 GHz. These characteristics include delay spread, azimuth angle spread, and elevation angle spread for extension of up to 100 GHz frequency. The frequency-dependent behaviour of the LSPs in the channel model has also been investigated.

- 2	Indoor hotspot large scale channel characteristics study

For InH scenarios, some investigation of fast fading modelling has been conducted based on both measurements and ray-tracing. Both indoor office and shopping mall environments have been investigated at frequencies including 2.9 GHz, 3.5 GHz, 6 GHz, 14 GHz, 15 GHz, 20 GHz, 28 GHz, 29 GHz, 60 GHz, and 73 GHz. Some preliminary analysis on large-scale channel characteristics has been summarized in Table 3 to Table 7 in appendix A. Although it is still too early to apply these results to the full frequency range up to 100 GHz, these preliminary investigations have provided insight into the channel characteristics across the extended frequency range.

The preliminary analysis results shown in Table 1 and Table 2, illustrate the frequency dependency of large-scale channel characteristics across the measured frequency range. For some LSPs, frequency dependency can be observed, but for some other LSPs, no strong frequency dependency is observed. Since for some cases, the number of data samples for frequencies available for analysis is limited, it is not yet easy to come to a conclusion. The observations indicate that, for some of the LSP, frequency dependency should be further studied for channel modelling for the frequency range up to 100 GHz.

Table 1. Analysis on frequency dependency on LSP for indoor office

| Model |  | LOS |  |  | NLOS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fitting | Avg. | IMT-A1 | fitting | Avg. | IMT-A1 |
| DS | Median [ns] | -0.77*log10(1+f)+30.61 | 28.22 | N/A | -11.64*log10(1+f)+54.53 | 38.94 | N/A |
|  | μDS | -0.28*log10(1+f)-7.31 | -7.71 | -7.70 | -0.35*log10(1+f)-7.06 | -7.55 | –7.41 |
|  | σDS | 0.04*log10(1+f)+0.18 | 0.24 | 0.18 | 0.10*log10(1+f)+0.04 | 0.19 | 0.14 |
| ASA | Median [°] | 11.47*log10(1+f)+9.23 | 26.77 | N/A | -17.89*log10(1+f)+71.83 | 44.48 | N/A |
|  | μASA | 0.07*log10(1+f)+1.39 | 1.51 | 1.62 | -0.22*log10(1+f)+1.98 | 1.6 | 1.77 |
|  | σASA | 1.78*log10(1+f)-2.46 | 0.60 | 0.22 | 1.90*log10(1+f) -2.66 | 0.6 | 0.16 |
| ZSA | Median [°] | -5.90*log10(1+f)+29.77 | 7.13 | N/A | -0.56*log10(1+f)+8.14 | 5.97 | N/A |
|  | μZSA | -0.52*log10(1+f)+1.60 | 0.76 | N/A | -0.03*log10(1+f)+0.89 | 0.77 | N/A |
|  | σZSA | 0.07*log10(1+f)+0.005 | 0.12 | N/A | 0.08 | 0.08 | N/A |
| K | μK | 0.84*log10(1+f)+2.12 | 3.38 | 7 | N/A | N/A | N/A |
|  | σK | -0.58*log10(1+f)+6.19 | 4.16 | 4 | N/A | N/A | N/A |
| ASD | μZSD | N/A | 1.08 | 1.60 | N/A | N/A | 1.62 |
|  | σZSD | N/A | 0.23 | 0.18 | N/A | N/A | 0.25 |
| ZSD | μZSD | N/A | 0.40 | N/A | N/A | N/A | N/A |
|  | σZSD | N/A | 0.23 | N/A | N/A | N/A | N/A |

1.The IMT-A results for InH are based on 2D channel model

Table 2. Analysis on frequency dependency on LSP for shopping mall

| Model |  | LOS |  |  | NLOS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fitting | Avg. | IMT-A1 | fitting | Avg. | IMT-A1 |
| DS | Median [ns] | -13.31*log10(1+f)+60.24 | 41.75 | N/A | -23.90*log10(1+f)+89.31 | 56.12 | N/A |
|  | μDS | -0.13*log10(1+f)-7.27 | -7.47 | -7.70 | -0.13*log10(1+f)-7.01 | # -7.21 | –7.41 |
|  | σDS | 0.007*log10(1+f)+0.30 | 0.31 | 0.18 | 0.007*log10(1+f)+0.19 | # 0.21 | 0.14 |
| ASA | Median [°] | -19.01*log10(1+f)+67.41 | 39.06 | N/A | 5.43*log10(1+f)+29.02 | 37.13 | N/A |
|  | μASA | -0.21*log10(1+f)+1.83 | # 1.51 | 1.62 | 0.10*log10(1+f)+1.54 | # 1.7 | 1.77 |
|  | σASA | -0.08*log10(1+f)+0.38 | # 0.25 | 0.22 | -0.13*log10(1+f)+0.35 | # 0.15 | 0.16 |
| ZSA | Median [°] | 9.64*log10(1+f)-8.56 | 5.82 | N/A | 0.46*log10(1+f)+3.35 | 4.05 | N/A |
|  | μZSA | 0.38*log10(1+f)+0.16 | 0.75 | N/A | 0.15*log10(1+f)+0.54 | 0.77 | N/A |
|  | σZSA | -0.33*log10(1+f)+0.73 | 0.23 | N/A | -0.08*log10(1+f)+0.26 | 0.13 | N/A |
| K | μK | -2.53*log10(1+f)+4.34 | # 0.57 | 7 | N/A | N/A | N/A |
|  | σK | -0.44*log10(1+f)+3.85 | 3.19 | 4 | N/A | N/A | N/A |
| ASD | Median [°] | 1.03*log10(1+f)+19.96 | 23.5 | N/A | -19.33*log10(1+f)+86.74 | 57.91 | N/A |
|  | μZSD | 0.18*log10(1+f)+1.09 | 1.36 | 1.60 | -0.14*log10(1+f)+1.93 | 1.72 | 1.62 |
|  | σZSD | -0.11*log10(1+f)+0.55 | 0.17 | 0.18 | 0.04*log10(1+f)+0.06 | 0.14 | 0.25 |
| ZSD | Median [°] | 1.97*log10(1+f)+1.04 | 3.98 | N/A | -5.95*log10(1+f)+16.96 | 8.09 | N/A |
|  | μZSD | 0.18*log10(1+f)+0.38 | 0.67 | N/A | -0.25*log10(1+f)+1.27 | 0.89 | N/A |
|  | σZSD | -0.25*log10(1+f)+0.75 | 0.37 | N/A | 0.01*log10(1+f)+0.17 | 0.19 | N/A |

1.The IMT-A results for InH are based on 2D channel model

- 3	Conclusion

In this contribution, the large-scale channel characteristics have been investigated for indoor hotspot scenarios, including indoor office and shopping mall. In light of these observations, the following is proposed:

Proposal 1: The observations indicate that, for some of the LSP, frequency dependency should be studied for channel modelling for the frequency range up to 100 GHz.

- References

- RP-151606, New SID Proposal: Study on channel model for frequency spectrum above 6 GHz, Samsung, Nokia Networks

- R1-161145, “WF on >6GHz Channel Modeling Scenarios”, NTT DOCOMO, AT&T, ALU, ASB, CATT, Deutsche Telekom, Ericsson, ETRI, Fraunhofer, Huawei, HiSilicon, Intel, InterDigital, KT, LGE, NEC, Mitsubishi, Nokia, NTT, Qualcomm, Samsung, Sharp, Sony, Vodafone, Malta, Feb. 15-19, 2016.

- K. Haneda, et. al., "Indoor 5G 3GPP-like Channel Models for Office and Shopping Mall Environments," in 2016 IEEE International Conference on Communications Workshops (ICCW), May 2016.

- G. MacCartney, et. al., “Indoor Office Wideband Millimeter-Wave Propagation Measurements and Channel Models at 28 and 73 GHz for Ultra-Dense 5G Wireless Networks,” IEEE ACCESS, Vol. 3, Oct. 2015, pp. 2388-2424.

- R1-161142, “WF on channel modeling requirements”, Huawei, HiSilicon, Ericsson, Keysight, CMCC, AT&T, Nokia, NTT DOCOMO, Samsung, CATR, Interdigital, Intel, ALU, China Unicom, ZTE, Malta, Feb. 15-19, 2016.

- Appendix A:	InH Large-scale channel characteristics

Table 3. Preliminary Indoor office large-scale channel characteristics (part 1)

|  |  | 3.5GHz1 |  | 6GHz1 |  | 20 GHz2 | 28GHz3 | 73GHz4 | 73GHz5 |  | 28GHz5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenarios |  | LOS | NLOS | LOS | NLOS | LOS | LOS | Hybrid | LOS | NLOS | LOS | NLOS |
| Delay spread () log10(seconds) | Median(ns) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 14.53 | 23.06 | 10.58 | 23.28 |
|  | DS | TBD | TBD | TBD | TBD | -7.33 | -7.75 | -8.1 | -7.85 | -7.67 | -7.97 | -7.7 |
|  | DS | TBD | TBD | TBD | TBD | 0.1 | 0.13 | 0.4 | 0.18 | 0.2 | 0.25 | 0.2 |
| Delay distribution |  | TBD | TBD | TBD | TBD | TBD | Exponential | Exponential | Exponential |  |  |  |
| AoA spread (ASA) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 30.48 | 37.46 | 28.24 | 47.51 |
|  | ASA | 3GPP | 3GPP | 1.72 | 1.49 | TBD | 1.57 | 1.6 | 1.46 | 1.54 | 1.43 | 1.66 |
|  | ASA | 3GPP | 3GPP | 0.14 | 0.23 | TBD | 0.22 | 0.37 | 0.13 | 0.19 | 0.14 | 0.12 |
| AoD spread (ASD) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASD | 3GPP | 3GPP | 1.34 | 1.49 | 1.8 | 1.08 | 1.5 | TBD | TBD | TBD | TBD |
|  | ASD | 3GPP | 3GPP | 0.15 | 0.08 | 0.09 | 0.23 | 0.26 | TBD | TBD | TBD | TBD |
| ZoA spread (ZSA) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 4.37 | 5.71 | 9.9 | 6.24 |
|  | ZSA | 1.02 | 1.08 | 1.33 | 0.95 | TBD | 0.67 | -0.025d+1.18 | 0.62 | 0.76 | 1 | 0.79 |
|  | ZSA | 0.41 | 0.36 | 0.07 | 0.28 | TBD | 0.17 | 0.30 | 0.14 | 0.08 | 0.05 | 0.08 |
| ZoD spread (ZSD) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD | 1.22 | 1.26 | 1.25 | 1.48 | 0 | 0.40 | -0.040d+1.45 | TBD | TBD | TBD | TBD |
|  | ZSD | 0.23 | 0.67 | 0.25 | 0.09 | 0.48 | 0.23 | 0.33 | TBD | TBD | TBD | TBD |
| AoD and AoA distribution |  | TBD | TBD | TBD | TBD | TBD | Laplacian | Uniform | Wrapped Gaussian |  |  |  |
| ZoD and ZoA distribution |  | Gaussian |  | TBD | TBD | TBD | Laplacian | Laplacian | Laplacian |  |  |  |
| Delay scaling parameter r |  | TBD | TBD | TBD | TBD | TBD | 1.95 | 2.4 | 2.03 | 1.35 | 2.23 | 1.78 |
| XPR (dB) | XPR | TBD | TBD | TBD | TBD | TBD | TBD | 15 | 10 | 6 | TBD | TBD |
|  | XPR | TBD | TBD | TBD | TBD | TBD | TBD | 2 | 7.5 | 10 | TBD | TBD |
| LOS Ricean K factor (dB) * | K | TBD | TBD | TBD | TBD | TBD | TBD | 8 | TBD | N/A | TBD | N/A |
|  | K | TBD | TBD | TBD | TBD | TBD | TBD | 3 | TBD | N/A | TBD | N/A |

- From BUPT based on measurement

- From DOCOMO based on measurement

- From KT based on measurement

- From Nokia/NYU based on ray-tracing

- From Huawei based on measurement

Table 4. Preliminary Indoor office large-scale channel characteristics (part 2)

|  |  | 14GHz1 |  | 28GHz1 |  | 15GHz2 |  | 2.9GHz3 |  | 29GHz3 |  | 28GHz4 |  | 73GHz4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenarios |  | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Delay spread () log10(seconds) | Median(ns) | 27.78 | 36.92 | 29.35 | 43.16 | 22.9 | 42.4 | 24 | 45 | 25 | 36 | TBD | TBD | TBD | TBD |
|  | DS | -7.60 | -7.47 | -7.55 | -7.41 | -7.65 | -7.43 | -7.7 | -7.4 | -7.6 | -7.5 | TBD | TBD | TBD | TBD |
|  | DS | 0.27 | 0.22 | 0.15 | 0.17 | 0.2 | 0.17 | 0.29 | 0.14 | 0.37 | 0.18 | TBD | TBD | TBD | TBD |
| Delay distribution |  | TBD | TBD | TBD | TBD | TBD | TBD | Exponential |  |  |  | TBD | TBD | TBD | TBD |
| XPR (dB) | XPR | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 14 | 10.4 | 22.8 | 15.4 |
|  | XPR | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 1.5 | 9.7 | 2.4 | 8 |

- From CMCC&BUPT based on measurement

- From Ericsson based on measurement

- From Qualcomm based on measurement

- From NYU based on measurement

Table 5. Preliminary Indoor office large-scale channel characteristics (part 3)

| # Parameters |  | # 3.5GHz1 |  | # 20GHz2 | # 28GHz3 | # 73GHz4 | # 28GHz5 |  | # 73GHz5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | LOS | Hybrid | LOS | NLOS | LOS | NLOS |
| Correlation distance (m) | DS [ns] | TBD | TBD | TBD |  | TBD | 2.26 | 2.47 | 1.85 | 1.69 |
|  | ASD [deg] | TBD | TBD | TBD |  | TBD | 2.01 | 1.83 | 1.96 | 0.67 |
|  | ASA [deg] | TBD | TBD | TBD |  | TBD | 2.07 | 2.61 | 2.06 | 1.62 |
|  | ZSD [deg] | 4 | 4 | TBD |  | TBD | 3.01 | 2/23 | 3.03 | 2.02 |
|  | ZSA [deg] | 4 | 4 | TBD |  | TBD | 2.65 | 2.54 | 2.09 | 0.83 |
|  | SF [dB] | TBD | TBD | TBD |  | TBD | 1.64 | 1.14 | 0.28 | 2.45 |
|  | K [dB] | TBD | TBD | TBD |  | TBD | 1.62 | 1.13 | 0.28 | 2.41 |
| Cross Correlation | ASD [deg] vs DS [s] | 3GPP | 3GPP | 0.26 | 0.481 | 0.47 | TBD | TBD | TBD | TBD |
|  | ASA [deg] vs DS [s] | 3GPP | 3GPP | TBD | 0.5433 | 0.28 | 0.83 | -0.03 | 0.69 | -0.1 |
|  | ASA [deg] vs SF [dB] | TBD | TBD | TBD | 0.0454 | TBD | 0.07 | -0.55 | -0.41 | -0.13 |
|  | ASD [deg] vs SF [dB] | TBD | TBD | -0.68 | 0.3111 | TBD | TBD | TBD | TBD | TBD |
|  | DS[s] vs SF[dB] | TBD | TBD | -0.33 | 0.3318 | TBD | 0.44 | 0.35 | -0.24 | 0.6 |
|  | ASD [deg] vs ASA [deg] | 3GPP | 3GPP | TBD | 0.6236 | 0.15 | TBD | TBD | TBD | TBD |
|  | ASD [deg] vs K [dB] | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | TBD |
|  | ASA [deg] vs K [dB] | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | TBD |
|  | DS [s] vs K [dB] | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | TBD |
|  | SF [dB] vs K [dB] | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | TBD |
|  | ZSD [deg] vs SF [dB] | 0.18 | 0 | 0.02 | 0.3749 | TBD | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs SF [dB] | 0.34 | 0 | TBD | 0.3313 | TBD | -0.52 | -0.01 | -0.68 | -0.27 |
|  | ZSD [deg] vs K [dB] | 0 | N/A | TBD |  | TBD | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs K [dB] | 0.1 | N/A | TBD |  | TBD | TBD | TBD | TBD | TBD |
|  | ZSD [deg] vs DS [s] | 0 | -0.27 | 0.24 | 0.1752 | 0.07 | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs DS [s] | 0 | -0.06 | TBD | 0.4963 | -0.01 | -0.2 | -0.12 | 0.41 | -0.24 |
|  | ZSD [deg] vs ASD [deg] | 0.65 | 0.35 | 0.1 | -0.3449 | 0.21 | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs ASD [deg] | 0 | 0.23 | TBD | 0.5116 | 0.1 | TBD | TBD | TBD | TBD |
|  | ZSD [deg] vs ASA [deg] | 0 | -0.08 | TBD | -0.0418 | 0.22 | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs ASA [deg] | 0.62 | 0.43 | TBD | 0.6429 | 0.06 | -0.03 | -0.28 | 0.61 | -0.37 |
|  | ZSD [deg] vs ZSA [deg] | 0.05 | 0.42 | TBD | 0.0975 | 0.44 | TBD | TBD | TBD | TBD |

- From BUPT based on measurement

- From DOCOMO based on measurement

- From KT based on measurement

- From Nokia/NYU based on ray-tracing

- From Huawei based on measurement

Table 6. Preliminary Indoor shopping mall large-scale channel characteristics (part 1)

|  |  | 15GHz1 |  | 28GHz1 |  | 63 GHz1 |  | 2.9GHz2 |  | 29GHz2 |  | 61GHz2 |  | 28GHz3 |  | 38GHz3 |  | 28GHz4 |  | 38GHz4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario |  | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Delay spread () log10(seconds) | Median(ns) | 45.88 | 49.69 | 33.91 | 39.65 | 25.68 | 38.4 | 49 | 80 | 57 | 71 | 39 | 58 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | DS | -7.45 | -7.31 | -7.49 | -7.45 | -7.62 | -7.46 | -7.4 | -7.1 | -7.3 | -7.2 | -7.5 | -7.3 | -7.48 | -7.28 | -7.53 | -7.24 | -7.38 | -6.88 | -7.59 | -6.9 |
|  | DS | 0.31 | 0.24 | 0.16 | 0.25 | 0.2 | 0.22 | 0.3 | 0.2 | 0.3 | 0.23 | 0.34 | 0.26 | 0.37 | 0.18 | 0.34 | 0.13 | 0.3 | 0.2 | 0.49 | 0.19 |
| Delay distribution |  | Exponential |  |  |  |  |  | Exponential |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| AoA spread (ASA) log10(degrees) | Median(deg) | 47.42 | 34.94 | 34.52 | 38.08 | 35.25 | 38.37 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASA | 1.62 | 1.54 | 1.52 | 1.56 | 1.5 | 1.56 | TBD | TBD | TBD | TBD | TBD | TBD | 1.59 | 1.79 | 1.53 | 1.78 | 1.42 | 1.84 | 1.46 | 1.84 |
|  | ASA | 0.26 | 0.23 | 0.15 | 0.2 | 0.16 | 0.17 | TBD | TBD | TBD | TBD | TBD | TBD | 0.31 | 0.08 | 0.27 | 0.11 | 0.26 | 0.18 | 0.37 | 0.09 |
| AoD spread (ASD) log10(degrees) | Median(deg) | 24.06 | 65.3 | 21.31 | 55.24 | 25.19 | 53.2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASD | 1.32 | 1.77 | 1.35 | 1.71 | 1.43 | 1.68 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASD | 0.26 | 0.12 | 0.15 | 0.15 | 0.1 | 0.15 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| ZoA spread (ZSA) log10(degrees) | Median(deg) | 3.69 | 3.92 | 4.43 | 4.04 | 9.34 | 4.2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA | 0.63 | 0.62 | 0.64 | 0.62 | 0.86 | 0.63 | TBD | TBD | TBD | TBD | TBD | TBD | 0.77 | 0.85 | 0.80 | 0.91 | 0.8 | 0.82 | 0.79 | 0.94 |
|  | ZSA | 0.52 | 0.25 | 0.44 | 0.29 | 0.4 | 0.24 | TBD | TBD | TBD | TBD | TBD | TBD | 0.09 | 0.05 | 0.03 | 0.05 | 0.09 | 0.02 | 0.03 | 0.03 |
| ZoD spread (ZSD) log10(degrees) | Median(deg) | 3.36 | 10.26 | 4.02 | 7.45 | 4.56 | 6.56 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD | 0.63 | 0.98 | 0.64 | 0.87 | 0.74 | 0.82 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD | 0.46 | 0.19 | 0.35 | 0.2 | 0.3 | 0.2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| AoD and AoA distribution |  | Wrapped Gaussian |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| ZoD and ZoA distribution |  | Laplacian |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| Delay scaling parameter r |  | 6.79 | 7.17 | 10.23 | 8.62 | 8.7 | 8.31 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| XPR (dB) | XPR | 17.12 | 17.71 | 16.12 | 14.48 | 16.85 | 16.06 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | XPR | 5.85 | 6.74 | 6.22 | 6.25 | 6.62 | 5.34 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| LOS Ricean K factor (dB) * | K | 1.36 | N/A | 0.54 | N/A | -0.18 | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A |
|  | K | 3.49 | N/A | 2.91 | N/A | 3.18 | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A |

- From Aalto University based on measurement

- From Qualcomm based on measurement

- From ETRI based on measurement in Railway station

- From ETRI based on measurement in Airport

Table 7. Preliminary Indoor shopping mall large-scale channel characteristics (part 2)

| # Parameters |  | # 15GHz1 |  | # 28GHz1 |  | # 63GHz1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Correlation distance (m) | DS [ns] | 8 | 2.5 | 8.5 | 2 | 8 | 0.5 |
|  | ASD [deg] | 7 | 6.5 | 5.5 | 1 | 2 | 1 |
|  | ASA [deg] | 17.5 | 7.5 | 6.5 | 6 | 8.5 | 4.5 |
|  | ZSD [deg] | 8.5 | 4.5 | 7.5 | 1.5 | 7.5 | 0.5 |
|  | ZSA [deg] | 12.5 | 3 | 10 | 1.5 | 14.5 | 2 |
|  | SF [dB] | 3 | 7.5 | 6.5 | 7.5 | 15 | 2 |
|  | K [dB] | 2 | N/A | 12.5 | N/A | 16 | N/A |
| Cross Correlation | ASD [deg] vs DS [s] | 0.44 | 0.16 | -0.16 | 0.32 | -0.1 | 0.54 |
|  | ASA [deg] vs DS [s] | 0.31 | 0.51 | 0.46 | 0.36 | 0.62 | 0.21 |
|  | ASA [deg] vs SF [dB] | -0.09 | 0.08 | 0.13 | 0.44 | 0.31 | 0.27 |
|  | ASD [deg] vs SF [dB] | 0.29 | 0.16 | 0.38 | 0.33 | 0.3 | 0.33 |
|  | DS[s] vs SF[dB] | 0.13 | 0.11 | -0.03 | 0.42 | 0.21 | 0.48 |
|  | ASD [deg] vs ASA [deg] | -0.37 | 0.46 | -0.3 | 0.35 | -0.07 | 0.24 |
|  | ASD [deg] vs K [dB] | -0.18 | N/A | 0.34 | N/A | 0.41 | N/A |
|  | ASA [deg] vs K [dB] | -0.13 | N/A | -0.28 | N/A | -0.07 | N/A |
|  | DS [s] vs K [dB] | -0.49 | N/A | -0.46 | N/A | -0.38 | N/A |
|  | SF [dB] vs K [dB] | 0.4 | N/A | 0.49 | N/A | 0.49 | N/A |
|  | ZSD [deg] vs SF [dB] | -0.08 | 0.01 | -0.14 | 0.29 | -0.17 | 0.45 |
|  | ZSA [deg] vs SF [dB] | -0.15 | 0.09 | -0.17 | 0.29 | -0.19 | 0.23 |
|  | ZSD [deg] vs K [dB] | 0.01 | N/A | 0.37 | N/A | 0.56 | N/A |
|  | ZSA [deg] vs K [dB] | -0.07 | N/A | 0.38 | N/A | 0.45 | N/A |
|  | ZSD [deg] vs DS [s] | -0.43 | -0.22 | -0.6 | 0.04 | -0.58 | 0.37 |
|  | ZSA [deg] vs DS [s] | -0.35 | 0.3 | -0.5 | 0.36 | -0.16 | 0.2 |
|  | ZSD [deg] vs ASD [deg] | 0.28 | -0.08 | 0.21 | 0.4 | 0.46 | 0.33 |
|  | ZSA [deg] vs ASD [deg] | 0.3 | -0.2 | 0.27 | 0.03 | 0.34 | -0.1 |
|  | ZSD [deg] vs ASA [deg] | -0.54 | -0.5 | -0.45 | -0.01 | -0.26 | -0.2 |
|  | ZSA [deg] vs ASA [deg] | -0.58 | 0.19 | -0.42 | 0.47 | 0.08 | 0.26 |
|  | ZSD [deg] vs ZSA [deg] | 0.92 | 0.09 | 0.9 | 0.22 | 0.77 | 0.29 |

- From Aalto University based on measurement

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"