3GPP TSG RAN WG1 #AH Channel Model                                               R1-161709

# Ljubljana, Slovenia, March 14-16, 2016

Agenda item:	7.0

Source: 	Samsung

Title: 	Discussion on additional attenuations

Document for: 	Discussion

- Introduction

In [1], the modeling methodologies for the additional features were discussed. This contribution discusses how to model additional attenuations due to rain and atmospheric losses.

- Additional Attenuations for Path Loss Model

In microwave bands, transmission loss is accounted by the free-space loss. However, in >6GHz cm/mm wave bands, additional loss factors come into play, such as atmospheric losses and rain loss in the transmission medium.

Atmospheric attenuations due to e.g. rain, water-vapour and oxygen absorption

These features can be added as an additional loss term, L, to the baseline path loss model. The total path loss (PL) can be described as:

PL [dB] = PL'(f, d) + L(f,  p, t, ρ, R),

where:

- PL'(f, d) is the distance & frequency dependent baseline path loss, which can be the same as the PL in [3] for <6GHz; and

- L(f, p, t, ρ, R, df) is for the additional attenuation;

- f is the carrier frequency;

- d is the link distance;

- p is the air pressure;

- t is the temperature;

- ρ is the water-vapour density;

- R is the rainfall rate; and

The additional attenuations can be described as:

L(f,  p, t, ρ, R) = Lgas(f, p, t, ρ) + Lrain(f, R)

The Lgas [dB/km] is an attenuation term due to atmospheric gaseous losses which is dependent on the carrier frequency, the air pressure, the temperature and the water-vapour density. Figure 1 shows the specific attenuation from 1 to 350 GHz at sea-level for dry air and water vapour with a density of 7.5 g/m3 [4]. The important absorption peaks occur at 24 and 60 GHz due to absorption of the radio signal by water vapour (H2O) and oxygen (O2). Except for these frequencies, there is no significant attenuation for frequencies below 100 GHz. Therefore, for most of the frequencies below 100 GHz, the attenuation due to atmospheric gases can be negligible. The details of Lgas modelling can be found in Appendix A.

In addition, the specific attenuation values due to atmospheric gases at several candidate frequency bands are summarized in Table 1.

Figure 1. Specific attenuation due to atmospheric gases [5]

Table 1. Specific attenuation values due to atmospheric gases at several candidate frequency bands

The additional attenuation Lrain [dB/km] for rain losses was well modelled in [5], as a function of frequency and the rain rate R [mm/h]:

Lrain(f, R) = kRα

The parameter values are available from Appendix B. Figure 2 [6] shows the attenuation per kilometre as a function of rain rate. When the rain rate is high, the corresponding attenuation is not negligible.

Figure 2. Specific attenuation due to rain [6]

- Conclusion

In this contribution, we proposed the additional loss from atmosphere and rain attenuation as an additional path loss term.

Proposal: The additional attenuation due to atmospheric attenuations (water-vapour and oxygen absorption) and rain losses are modelled as an additional path loss term, L(f, p, t, ρ, R), according to [4] and [5]:

L(f, p, t, ρ, R) = Lgas(f, p, t, ρ) + Lrain(f, R)

where

- Lgas(f, p, t, ρ) = γo + γw  [dB/km]

- Lrain(f, R) = kRα  [dB/km]

and

- f is the carrier frequency;

- d is the link distance;

- p is the air pressure;

- t is the temperature;

- ρ is the water-vapour density;

- R is the rainfall rate; and

## References

[1] R1-160590, Discussion on additional features for >6GHz channel model, Samsung

[2] R1-160704, White Paper on Channel Modelling for Bands up to 100 GHz, NTT DOCOMO, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, Nokia, Qualcomm and Samsung

[3] 3GPPTR36.873, 3D Channel Models

[4] ITU-R P.676-10, “Attenuation by atmospheric gases”, International Telecommunication Union Radiocommunication Sector ITU-R, 2013.

[5] ITU-R P.838-3, “Specific attenuation model for rain for use in prediction methods”, International

Telecommunication Union, Radiocommunication Sector, 2005.

[6] M. Marcus and B. Pattan, “Millimeter wave propagation: spectrum management implications,” IEEE Microwave Magazine, vol. 6, no. 2, pp.54-62, June 2005.

## Appendix A. Gas attenuation modeling details in [4]

The specific attenuation due to dry air and water vapour, from sea level to an altitude of 10 km, can be estimated using the following simplified algorithms.

For dry air, the attenuation γo [dB/km] is given by the following equations:

For water vapour, the attenuation γw [dB/km] is given by:

Finally, the specific attenuation due to dry air and water vapour can be modelled as:

Lgas(f, p, t, ρ) = γo + γw  [dB/km]

## Appendix B. Rain attenuation modeling details in [5]

The specific attenuation Lrain[dB/km] due to rain is obtained from the rain rate R [mm/h] using the power-law relationship:

Lrain(f, R) = kRα.

Values for the coefficients k and α are determined as functions of frequency, f (GHz), in the range from 1 to 1 000 GHz, from the following equations, which have been developed from curve-fitting to power-law coefficients derived from scattering calculations:

Values for the constants for the coefficient kH for horizontal polarization, the coefficient kV for vertical polarization, for the coefficient αH for horizontal polarization, and for the coefficient αV for vertical polarization are given in Table 2.

Table 2. Values for the constants for the coefficients for kH, kV, αH, αV

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"