3GPP TSG-RAN WG1 NB-IoT Ad-Hoc Meeting	R1-162056

Sophia Antipolis, France, 22nd – 24th March 2016

Agenda item:		2.2.3

Source:	Ericsson, Panasonic, LGE, Nokia, ALU, ASB

Title:	WF for TBS/MCS table design for NB-PUSCH

Document for:		Discussion

# WF

Multi-tone IMCS to ITBS mapping table

For multi-tone, the IMCS to ITBS mapping table is:

| IMCS | Modulation | ITBS |
| --- | --- | --- |
| 0 | QPSK | 0 |
| 1 | QPSK | 1 |
| 2 | QPSK | 2 |
| 3 | QPSK | 3 |
| 4 | QPSK | 4 |
| 5 | QPSK | 5 |
| 6 | QPSK | 6 |
| 7 | QPSK | 7 |
| 8 | QPSK | 8 |
| 9 | QPSK | 9 |
| 10 | QPSK | 10 |
| 11 | QPSK | 11 |
| 12 | QPSK | 12 |

Sinle-tone IMCS to ITBS mapping table

For single-tone, IMCS= 0 and 1 points to pi/2-BPSK. For single-tone, the IMCS to ITBS mapping table is:

| IMCS | Modulation | ITBS |
| --- | --- | --- |
| 0 | pi/2-BPSK | 0 |
| 1 | pi/2-BPSK | 2 |
| 2 | pi/4-QPSK | 1 |
| 3 | pi/4-QPSK | 3 |
| 4 | pi/4-QPSK | 4 |
| 5 | pi/4-QPSK | 5 |
| 6 | pi/4-QPSK | 6 |
| 7 | pi/4-QPSK | 7 |
| 8 | pi/4-QPSK | 8 |
| 9 | pi/4-QPSK | 9 |
| 10 | pi/4-QPSK | 10 |

TBS look-up table

The TBS look-up table for both multi-tone and single-tone is:

|  | NRU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 696 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 328 88 | 176 | 256 | 392 | 504 | 600 | 808 | 10321000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 968 1000 | N/A |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | N/A | N/A |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | N/A | N/A |
| 10 | 144 | 328 | 504 | 680 | 872 | 10321000 | N/A | N/A |
| 11 | 176 | 376 | 584 | 776 | 1000 | N/A | N/A | N/A |
| 12 | 208 | 440 | 680 | 9041000 | N/A | N/A | N/A | N/A |

# Annex

Agreements:

- For multi-tone and single-tone,

  - RV0 or RV2 is separately indicated by 1 bit DCI. RV2 is supported in all ITBS

- Starting point is to reuse TBS/MCS table for DL

- ITBS is 4 bits indication in DCI

- NPRB is 3 bits indication in DCI. NPRB indicates the number of resource unit

- For multi-tone, support ITBS equals 0 to at least 10

- For single-tone, support ITBS equals 0 to 10

- For single-tone cases,

  - Pi/2 BPSK is used for the lowest one ITBS entry or lowest two ITBS entries. Pi/4 QPSK is used in the other ITBS entries.

Possible agreements:

- For single-tone cases

  - Alt A: Pi/2 BPSK for NPRB is supported 1,2, 3,4,5,6,8, 10. One row of ITBS indexes are used.

  - Alt B: Pi/2 BPSK for NPRB is supported 1,2, 3,4,5,6,8, 10, 11,12, 13,14,15,16,18, 20. Two rows of ITBS indexes are used.

  - Alt C: Pi/2 BPSK for NPRB is supported 2, 4,8,10,16, 20, 24, 28. Two rows of ITBS indexes are used.

Continue offline discussion until Thursday – Suzuki (Panasonic)