3GPP TSG-WG2 Meeting #102AH	R2-1810388

# Montreal, Canada, 2nd – 6th of July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 100 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SA. |
| --- | --- |
| Source to WG: | Ericsson (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is the result of Rapporteur Implementation of RAN2 agreements at the RAN2#102AH. The text on the CR cover page is the same as earlier submitted in Rev 1 of this CR (R2-1809239). Changes related to NR-Standalone: At RAN2#101bis, the following CRs were to be added to the Rapporteur CR for SA, either complete, or as specific agreements, see the Report of RAN2#102: R2-1805664 NR reconfiguration message structure for NSA & SA R2-1805225 MSG3 based other SI acquisition R2-1806418 Standalone NR full configuration (discussion and TP to 38.331) At RAN2#102, the following CRs were agreed to be added to the Rapporteur CR for SA, either complete, or as specific agreements, see Chairman’s notes of main session and breakout sessions for details (later available in the Report of RAN2#102): R2-1808961 TP on Connection Control R2-1807911 Summary of email discussion [101bis#51][NR] Connection control open issues R2-1809111 Offline discussion #34 on Msg3 R2-1808986 [TP for SDAP configuration] R2-1807929 TP on UL/DL Information Transfer R2-1809060 Remaining issue on Events and measurements for handover from NR to E-UTRA R2-1809077 Draft CR on ANR for 38.331 R2-1809109 38 331 TP on SI procedure text R2-1809108 [101bis#13][NR] Text Proposal on the ASN.1 for SIB content. R2-1809088 Report of [101bis#45][NR] TP on AC R2-1808964 OTDOA measurement gap request R2-1808784 TP on considering good beams in cell reselection (option 2.3) R2-1806796 On RSSI related parameter configuration in SIBs (implemented with User set to SA R2-1808796) R2-1807101 Essential System Information in NR R2-1807120 Adding Access Type to Paging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes related to NR-Standalone: Implemented the TPs, CRs and other agreements listed in the "reason for change". Impact Analysis Impacted functionality: NR for standalone operation Inter-operability: This CR implements NR Standalone functionality on top of the current (EN-DC) baseline (CR 42). This CR adds functionality in a backwards compatible manner. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR Standalone functionality is not available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 5, 6, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR ... CR ... All 38-series specs |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ... All 38-series specs |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Radio Resource Control protocol for the radio interface between UE and NG-RAN.

The scope of the present document also includes:

-	the radio related information transported in a transparent container between source gNB and target gNB upon inter gNB handover;

-	the radio related information transported in a transparent container between a source or target gNB and another system upon inter RAT handover.

-	the radio related information transported in a transparent container between a source eNB and target gNB during E-UTRA-NR Dual Connectivity.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[6]	ITU-T Recommendation X.680 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[7]	ITU-T Recommendation X.681 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[8]	ITU-T Recommendation X.691 (08/2015) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[9]	3GPP TS 38.215: "NR; Physical layer measurements".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[14]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[15]	3GPP TS 38.101: "NR; User Equipment (UE) radio transmission and reception".

[16]	3GPP TS 38.211: "NR; Physical channels and modulation".

[17]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[18]	ITU-T Recommendation X.683 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications" (Same as the ISO/IEC International Standard 8824-4).

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	3GPP TS 36.101: " E-UTRA; User Equipment (UE) radio transmission and reception".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[xx]	3GPP TS 37.324: “Service Data Adaptation Protocol (SDAP) specification”.

[25]	3GPP TS 22.261: "Service requirements for the 5G System"

[26]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities"

[27]	3GPP TS 36.304: “E-UTRA; User Equipment (UE) procedures in idle mode”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Field: The individual contents of an information element are referred as fields.

Floor: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Information element: A structural element containing a single or multiple fields is referred as information element.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.”

PUCCH SCell: An SCell configured with PUCCH.

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

SRB1S: The SCG part of MCG split SRB1 for EN-DC.

SRB2S: The SCG part of MCG split SRB2 for EN-DC.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BLER	Block Error Rate

BWP	Bandwidth Part

CA	Carrier Aggregation

CCCH	Common Control Channel

CG	Cell Group

CMAS	Commercial Mobile Alert Service

CP	Control Plane

C-RNTI	Cell RNTI

CSI	Channel State Information

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

EN-DC	E-UTRA NR Dual Connectivity

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC      E-UTRA connected to 5GC

E-UTRA/EPC      E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

MAC	Medium Access Control

MCG	Master Cell Group

MIB	Master Information Block

N/A	Not Applicable

NR/5GC          	NR connected to 5GC

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PSCell	Primary Secondary Cell

PTAG	Primary Timing Advance Group

PWS	Public Warning System

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	RObust Header Compression

RRC	Radio Resource Control

RS	Reference Signal

SCell	Secondary Cell

SCG	Secondary Cell Group

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SpCell	Special Cell

SRB	Signalling Radio Bearer

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDD	Time Division Duplex

TM	Transparent Mode

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

# 4	General

## 4.1	Introduction

This specification is organised as follows:

-	sub-clause 4.2 describes the RRC protocol model;

-	sub-clause 4.3 specifies the services provided to upper layers as well as the services expected from lower layers;

-	sub-clause 4.4 lists the RRC functions;

-	clause 5 specifies RRC procedures, including UE state transitions;

-	clause 6 specifies the RRC messages in ASN.1 and description;

-	clause 7 specifies the variables (including protocol timers and constants) and counters to be used by the UE;

-	clause 8 specifies the encoding of the RRC messages;

-	clause 9 specifies the specified and default radio configurations;

-	clause 10 specifies generic error handling;

-	clause 11 specifies the RRC messages transferred across network nodes;

-	clause 12 specifies the UE capability related constraints and performance requirements.

## 4.2	Architecture

### 4.2.1	UE states and state transitions including inter RAT

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors a Paging channel for CN paging using 5G-S-TMSI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information and can send SI request (if configured).

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	UE controlled mobility based on network configuration;

- 	The UE stores the AS context;

-	A RAN-based notification area is configured by RRC layer;

The UE:

-	Monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using I-RNTI;

-	Performs neighbouring cell measurements and cell (re-)selection;

- 	Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area;

-	Acquires system information and can send SI request (if configured).

-	RRC_CONNECTED:

-	The UE stores the AS context;

-	Transfer of unicast data to/from UE;

-	At lower layers, the UE may be configured with a UE specific DRX;

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility within NR and to/from E-UTRA;

-	The UE:

-	Monitors a Paging channel, if configured;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell measurements and measurement reporting;

-	Acquires system information.

Editor’s Note:	FFS Whether UE in RRC_CONNECTED monitors paging channel..

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/5GC E-UTRA/EPC and E-UTRA/5GC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/5GC, E-UTRA/EPC and and E-UTRA/5GC

### 4.2.2	Signalling radio bearers

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel;

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel;

-	SRB2 is for NAS messages, all using DCCH logical channel. SRB2 has a lower-priority than SRB1 and is always configured by the network after security activation;

-	SRB3 is for specific RRC messages when UE is in EN-DC, all using DCCH logical channel.

In downlink piggybacking of NAS messages is used only for  bearer establishment/modification/release. In uplink piggybacking of NAS message is used only for transferring the initial NAS message during connection setup and connection resume.

Editor’s Note: FFS Piggybacking of NAS messages in other procedures than bearer establishment/ modification/ release.

NOTE 1:	The NAS messages transferred via SRB2 are also contained in RRC messages, which however do not include any RRC protocol control information.

Once security is activated, all RRC messages on SRB1, SRB2 and SRB3, including those containing NASmessages, are integrity protected and ciphered by PDCP. NAS independently applies integrity protection and ciphering to the NAS messages.

Editor’s Note:	FFS Which SRBs are used for NE-DC, NR-NR DC.

Editor’s Note:	FFS Duplication in UL, for split SRB and DRBs.

## 4.3	Services

### 4.3.1	Services provided to upper layers

The RRC protocol offers the following services to upper layers:

-	Broadcast of common control information;

-	Notification of UEs in RRC_IDLE, e.g. about a terminating call;

-    Notification of UEs about ETWS and/or  CMAS

-	Transfer of dedicated control information, i.e. information for one specific UE.

### 4.3.2	Services expected from lower layers

In brief, the following are the main services that RRC expects from lower layers:

-	Integrity protection, ciphering and loss-less in-sequence delivery of information without duplication;

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE and RRC_INACTIVE (e.g. cell (re-)selection parameters, neighbouring cell information) and information (also) applicable for UEs in RRC_CONNECTED (e.g. common channel configuration information);

-	Including ETWS notification, CMAS notification.

-	RRC connection control:

-	Paging;

-	Establishment/modification/suspension/resumption/release of RRC connection, including e.g. assignment/modification of UE identity (C-RNTI, I-RNTI, etc.), establishment/modification/suspension/resumption/release of SRBs (except for SRB0);

-	Access class barring;

-	Initial security activation, i.e. initial configuration of AS integrity protection (SRBs, DRBs ) and AS ciphering (SRBs, DRBs);

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, associated security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes;

-	Establishment/modification/suspension/resumption/release of RBs carrying user data (DRBs);

-	Radio configuration control including e.g. assignment/modification of ARQ configuration, HARQ configuration, DRX configuration;

-	In case of DC, cell management including e.g. change of PSCell, addition/modification/release of SCG cell(s);

-	In case of CA, cell management including e.g. addition/modification/release of SCell(s);

-	QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration  and configured grant configuration for DL and UL repectfully, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB.

-	Recovery from radio link failure.

-	Inter-RAT mobility including e.g. security activation, transfer of RRC context information;

-	Measurement configuration and reporting:

-	Establishment/modification/release of measurement configuration (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting.

-	Other functions including e.g. generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information.

# 5	Procedures

## 5.1	General

### 5.1.1	Introduction

This section covers the general requirements.

### 5.1.2	General requirements

The UE shall:

1>	process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE:	Network may initiate a subsequent procedure prior to receiving the UE's response of a previously initiated procedure.

1>	within a sub-clause execute the steps according to the order specified in the procedural description;

1>	consider the term 'radio bearer' (RB) to cover SRBs and DRBs unless explicitly stated otherwise;

1>	set the rrc-TransactionIdentifier in the response message, if included, to the same value as included in the message received from NR that triggered the response message;

1>	upon receiving a choice value set to setup:

2>	apply the corresponding received configuration and start using the associated resources, unless explicitly specified otherwise;

1>	upon receiving a choice value set to release:

2>	clear the corresponding configuration and stop using the associated resources;

1>	in case the size of a list is extended, upon receiving an extension field comprising the entries in addition to the ones carried by the original field (regardless of whether NR signals more entries in total); apply the following generic behaviour unless explicitly stated otherwise:

2>	create a combined list by concatenating the additional entries included in the extension field to the original field while maintaining the order among both the original and the additional entries;

2>	for the combined list, created according to the previous, apply the same behaviour as defined for the original field.

## 5.2	System information

Editor’s Note: Targeted for completion in September2018. For EN_DC, only parts related to MIB acquisition, in sub-clauses 5.2.2.3.1 and 5.2.2.4.1, are applicable.

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs where:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms [17, Section 7.1] and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes defined by [TS 38.211, 7.4.3.2] of radio frames for which the SFN mod 8 = 0, and repetitions are scheduled in other radio frames according to the period of SSB;

-	the SIB1 is transmitted on the DL-SCH with a periodicity of 160ms and variable transmission repetition periodicity as specified in TS 38.213 [13, Section 13]. The default transmission repetition periodicity of SIB1 is 20ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period [13]. SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs having the same periodicity can be mapped to the same SI message. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Any SIB except SIB1 can be configured to be cell specific or area specific. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells and is identified by systemInformationAreaID;

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, RAN releases and adds the concerned SCell. For PSCell, SI can only be changed with Reconfiguration with Sync.

### 5.2.2	System information acquisition

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS- and NAS information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4 and SIB5 (if the UE supports E-UTRA).

Editor’s Note: [FFS

if different versions of SIBs are provided].

#### 5.2.2.2	SI validity and need to (re)-acquire SI

##### 5.2.2.2.1	SI validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification; whenever the UE does not have a valid version of a stored SI.

Editor’s Note: [FFS if upon receiving HO command the SI acquisition depend on stored SI]

When the UE acquires a MIB or a SIB1 or a SI message in a currently camped/serving cell as described in clause 5.2.2.3, the UE shall store the acquired SI. A version of the SI that the UE stored is no longer valid 3 hours after acquisition. The UE may use a valid stored version of the SI except MIB and SIB1 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication.

NOTE: 	The storage and management of the stored SI in addition to the SI valid for the current camped/serving cell is left to UE implementation.

##### 5.2.2.2.1	SI validity

The UE shall:

1>	delete any stored version of aZ SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the stored SIB has an area scope and if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the SIB1 received from the currently camped/serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB; or

2>	if the stored SIB is cell specific and if valueTag and CellIdentity included in the

SIB1 received from the currently camped/serving cell is identical to the valueTag

and CellIdentity associated with stored version of that SIB;

3> consider the stored SIB as valid for the cell;

##### 5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications   using Short Message transmitted with P-RNTI over DCI (see section 6.5). Repetitions of SI change indication may occur within preceding modification period.

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion if the UE is provided with common search space to monitor paging, as specified in TS 38.213 [13, section 13].

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion if the UE is provided with common search space to monitor paging.

If the UE receives aShort Message, the UE shall:

1>	if the UE is ETWS capable or CMAS capable, and the etwsAndCmasIndication bit of Short Message is set:

2> immediately re-acquire the SIB1;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6:

3>	acquire SIB6, as specified in sub-clause 5.2.2.3, immediately;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7:

3>	acquire SIB7, as specified in sub-clause 5.2.2.3, immediately;

2>	if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8:

3>	acquire SIB8, as specified in sub-clause 5.2.2.3, immediately;

1> the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in sub-clause 5.2.2.3 from the start of the next modification period.

Editor’s Note: [FFS Whether to make a generic bit to indicate immediate acquisition of SI will be considered after AC discussion has progressed]

Editor’s Note: [FFS if terminology to be used for PWS Notification]

#### 5.2.2.3	Acquisition of System Information

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.X;

1>	if the UE is in RRC_CONNECTED and the cell is a PSCell:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	perform the actions specified in section 5.2.2.4.1;

1> else if the UE is in RRC_CONNECTED with an active BWP with common search space configured or for which the conditions defined in FFS_Spec [X], section FFS_Section are satisfied, and has received an indication about change of system information:

2>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

NOTE:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

2>	if the UE is unable to acquire the SIB1:

3> perform the actions as specified in clause 5.2.2.5;

2>	else:

3> perform the actions specified in section 5.2.2.4.2.

1>	else:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2> if the UE is unable to acquire the MIB;

3> perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in section 5.2.2.4.1.

2>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the SIB1:

3> perform the actions as specified in clause 5.2.2.5;

2>	else:

3> perform the actions specified in section 5.2.2.4.2.

##### 5.2.2.3.2	Acquisition of an SI message

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1;

2>	determine the integer value x = (n – 1)*w, where w is the si-WindowLength;

2>	the SI-window starts at the subframe #a, where a = x mod 10, in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

1> if SI message acquisition is not triggered due to UE request:

2>	receive DL-SCH using the SI-RNTI from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received;

2>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message;

1> else if SI message acquisition is triggered due to UE request:

2> [FFS receive DL-SCH using the SI-RNTI from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received];

2>	[FFS if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message];

Editor’s Note: [FFS_Standalone on the details of from which SI-window the UE shall receive the DL-SCH upon triggering the SI request.

Editor’s Note: [FFS on the details of how many SI-windows the UE should monitor for SI message reception if transmission triggered by UE request]

1>	perform the actions for the acquired SI message as specified in sub-clause 5.2.2.4.

Editor’s Note: FFS The procedural text for SI message acquisition triggered by UE request will be updated upon finalizing the details.

##### 5.2.2.3.3	Request for on demand system information

The UE shall:

1>	if SIB1 includes si-SchedulingInfo containing si-Request-Config for the SI message(s) that the UE requires to operate within the cell:

2>		trigger the lower layer to initiate the Random Access procedure in accordance with  [3] using the PRACH preamble and PRACH resource corresponding to the SI message(s) that the UE requires to operate within the cell;

2>	if acknowledgement for SI request is received from lower layers;

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2;

1>	else

2>

initiate transmission of the RRCSystemInfoRequest message in accordance with 5.2.2.3.4;

2>	if acknowledgement for RRCSystemInfoRequest message is received from lower layers;

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2;

NOTE: 	1>	else

there  is no need to differentiate the SIB acquisiton failure is caused by RACH failure or reception failure of on –demand SI. After RACH failure for SI request it is UE implementation when to retry the SI request.

##### 5.2.2.3.4	Actions related to transmission of RRCSystemInfoRequest message

The UE shall set the contents of RRCSystemInfoRequest message as follows:

1>	set the request-SIType-List to indicate the SI message(s) that the UE requires to operate within the cell.

The UE shall submit the RRCSystemInfoRequest message to lower layers for transmission.

#### 5.2.2.4	Actions upon receipt of System Information

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2> if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if intraFreqReselection is set to notAllowed:

4> consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20].

3>	else:

4> consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20].

2>	else:

3>	apply the received pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position and acquire SIB1, if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell [13].

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfo containing the selected PLMN;

1>	forward the cellIdentity to upper layers;

1>	forward the trackingAreaCode to upper layers;

1>	forward the ims-EmergencySupport to upper layers, if present;

1>	forward the eCallOverIMS-Support to upper layers, if present;

1>	apply the configuration included in the servingCellConfigCommonSIB;

1>	if the UE has a stored valid version of a SIB that the UE requires to operate within the cell in accordance with sub-clause 5.2.2.2.1:

2>	use the stored version of the required SIB;

1>	if the UE has not stored the valid version of one or several required SIB(s), in accordance with sub-clause 5.2.2.2.1:

2>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

3> acquire the SI message(s) as defined in sub-clause 5.2.2.3.2;

2>	for the SI message(s) that, according to the the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

3>	trigger a request to acquire the SI message(s) as defined in sub-clause 5.2.2.3.3.

Editor’s Note: To be further updated when content of the SIB1 has been completed.

##### 5.2.2.4.3	Actions upon reception of SIB2

No UE requirements related to the contents of this SIB2 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

Editor’s Note: To be further updated when content of the SIB2 has been completed.

##### 5.2.2.4.4	Actions upon reception of SIB3

No UE requirements related to the contents of this SIB3 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

Editor’s Note: To be further updated when content of the SIB3 has been completed.

##### 5.2.2.4.5	Actions upon reception of SIB4

Upon receiving the SIB4 the UE shall:Editor’s Note: FFS the UE behavior related to multiFrequencyBandListNR.

Editor’s Note: To be updated when content of the SIB4 has been agreed.

##### 5.2.2.4.6	Actions upon reception of SIB5

No UE requirements related to the contents of this SIB5 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

Editor’s Note: To be updated when content of the SIB5 has been agreed.

##### 5.2.2.4.7	Actions upon reception of SIB6

Upon receiving the SIB6 the UE shall:

1>	forward the received warningType, messageIdentifier and serialNumber to upper layers;

##### 5.2.2.4.8	Actions upon reception of SIB7

Upon receiving the SIB7 the UE shall:

1>	if there is no current value for messageIdentifier and serialNumber for SIB7; or

1>	if either the received value of messageIdentifier or of serialNumber or of both are different from the current values of messageIdentifier and serialNumber for SIB7:

2>	use the received values of messageIdentifier and serialNumber for SIB7 as the current values of messageIdentifier and serialNumber for SIB7;

2>	discard any previously buffered warningMessageSegment;

2>	if all segments of a warning message have been received:

3>	assemble the warning message from the received warningMessageSegment;

3>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

3>	stop reception of SIB7;

3>	discard the current values of messageIdentifier and serialNumber for SIB7;

2>	else:

3>	store the received warningMessageSegment;

3>	continue reception of SIB7;

1>	else if all segments of a warning message have been received:

2>	assemble the warning message from the received warningMessageSegment;

2>	forward the received complete warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2>	stop reception of SIB7;

2>	discard the current values of messageIdentifier and serialNumber for SIB7;

1>	else:

2>	store the received warningMessageSegment;

2>	continue reception of SIB7;

The UE should discard any stored warningMessageSegment and the current value of messageIdentifier and serialNumber for SIB7 if the complete warning message has not been assembled within a period of 3 hours.

##### 5.2.2.4.9	Actions upon reception of SIB8

Upon receiving the SIB8 the UE shall:

1>	if the SIB8 contains a complete warning message:

2>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2>	continue reception of SIB8;

1>	else:

2>	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message is currently being assembled:

3>	store the received warningMessageSegment;

3>	if all segments of a warning message have been received:

4>	assemble the warning message from the received warningMessageSegment;

4>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

4>	stop assembling a warning message for this messageIdentifier and serialNumber and delete all stored information held for it;

3>	continue reception of SIB8;

2>	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3>	start assembling a warning message for this messageIdentifier and serialNumber pair;

3>	store the received warningMessageSegment;

3>	continue reception of SIB8;

The UE should discard warningMessageSegment and the associated values of messageIdentifier and serialNumber for SIB8 if the complete warning message has not been assembled within a period of 3 hours.

NOTE: 	The number of warning messages that a UE can re-assemble simultaneously is a function of UE implementation.

##### 5.2.2.4.10	Actions upon reception of SIB9

No UE requirements related to the contents of this SIB9 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

Editor’s Note: To be extended with further sub-clauses as more SIBs are defined. FFS_Standalone

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection is set to allowed;

2>	else if the UE is unable to acquire the SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20].

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful security activation. However, the UE only accepts a re-configuration with sync message when security has been activated.

Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial security activation procedure. The RRC messages to activate security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered. After having initiated the initial security activation procedure, the network initiates the establishment of SRB2 and DRBs, i.e. the network may do this prior to receiving the confirmation of the initial security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2 and DRBs. The network should release the RRC connection if the initial security activation and/ or the radio bearer establishment fails.

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an EUTRA carrier frequency.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE AS context and any configuration received from the network, and transits to RRC_INACTIVE state. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates security and re-establishes SRB(s) and DRB(s).

In response to a request to resume the RRC connection, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to discard the stored context and initiate NAS level recovery (in this case the network sends an RRC setup message).

Editor’s Note	FFS NE-DC, NR-NR-DC related aspects.

#### 5.3.1.2	Security

AS security comprises of the integrity protection and ciphering of RRC signalling (SRBs) and user data (DRBs).

RRC handles the configuration of the security parameters which are part of the AS configuration: the integrity protection algorithm, the ciphering algorithm, if integrity and/or ciphering is enabled for a DRB and two parameters, namely the keySetChangeIndicator and the nextHopChainingCount, which are used by the UE to determine the AS security keys upon reconfiguration with sync (with key change), connection re-establishment and/or connection resume.

The integrity protection and ciphering algorithm is common for signalling radio bearers SRB1 and SRB2. When not configured with any kind of DC, the ciphering and integrity protection algorithm is common for all radio bearers (i.e. SRB1, SRB2 and DRBs). All DRBs related to the same PDU session have the same security configuration.Neither integrity protection nor ciphering applies for SRB0.

Editor’s note:	FFS NE-DC, NR-NR-DC related security parameters such as SK-counter and S-KgNB.

RRC integrity and ciphering are always activated together, i.e. in one message/ procedure. RRC integrity and ciphering for SRBs are never de-activated. However, it is possible to switch to a 'NULL' ciphering algorithm (nea0).

The 'NULL' integrity protection algorithm (nia0) is used only for SRBs and for the UE in limited service mode [11]. In case the 'NULL' integrity protection algorithm is used, 'NULL' ciphering algorithm is also used.

NOTE 1:	Lower layers discard RRC messages for which the integrity check has failed and indicate the integrity verification check failure to RRC.

The AS applies four different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc), one for integrity protection of user data (KUPint) and one for the ciphering of user data (KUPenc). All four AS keys are derived from the KgNB key. The KgNB is based on the KAMF key (as specified in TS 33.501 [11]), which is handled by upper layers.

The integrity and ciphering algorithms can only be changed with reconfiguration with sync. The AS keys (KgNB, KRRCint, KRRCenc, KUPint  and KUPenc) change upon reconfiguration with sync (if keyRefresh is included), and upon connection re-establishment and connection resume.

For each radio bearer an independent counter (COUNT, as specified in TS 38.323 [5]) is maintained for each direction. For each radio bearer, the COUNT is used as input for ciphering and integrity protection. It is not allowed to use the same COUNT value more than once for a given security key. In order to limit the signalling overhead, individual messages/ packets include a short sequence number (PDCP SN, as specified in TS 38.323 [5]). In addition, an overflow counter mechanism is used: the hyper frame number (TX_HFN and RX_HFN, as specified in TS 38.323 [5]). The HFN needs to be synchronized between the UE and the network. The network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same key, e.g. due to the transfer of large volumes of data, release and establishment of new RBs. In order to avoid such re-use, the network may e.g. use different RB identities for successive RB establishments, trigger an intra cell reconfiguration with sync, or an RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE and then to RRC_CONNECTED transition.

For each SRB, the value provided by RRC to lower layers to derive the 5-bit BEARER parameter used as input for ciphering and for integrity protection is the value of the corresponding srb-Identity with the MSBs padded with zeroes.

Editor’s note:	FFS Handling of keys in NE-DC and NR-NR-DC.

### 5.3.2	Paging

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit paging information to a UE in RRC_IDLE or RRC_INACTIVE.

#### 5.3.2.2	Initiation

The network initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in TS 38.304 [20]. The network may address multiple UEs within a Paging message by including one PagingRecord for each UE.

#### 5.3.2.3	Reception of the Paging message by the UE

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity and accessType (if present) to the upper layers;

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE’s stored I-RNTI:

3>	 initiate the RRC connection resumption procedure according to 5.3.13;

2>	else if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3>	forward the ue-Identity to upper layers and accessType (if present) to the upper layers;

3> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ‘CN paging’;

### 5.3.3	RRC connection establishment

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.3.3.1	General

Figure 5.3.3.1-1: RRC connection establishment, successful

Figure 5.3.3.1-2: RRC connection establishment, network reject

The purpose of this procedure is to establish an RRC connection. RRC connection establishment involves SRB1 establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network.

The network applies the procedure as follows:

-	When establishing an RRC connection;

-	When UE is resuming or re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context. In this case, UE receives RRCSetup and responds with RRCSetupComplete.

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

NOTE: 	As one UE implementation option, upper layers may request access barring check as specified in the unified acccess control procedure in 5.3.14 and only request the RRC establishment when the access attempt is considered allowed. As another UE implementation option, upper layers may instead provide an Access Category and one or more Access Identities upon requesting the RRC establishment so that RRC initiates the access barring check.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified acccess control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

Editor’s Note: FFS How to capture the different options for AS/NAS modeling for barring check for connection establishment.

1>	apply the default physical channel configuration as specified in 9.2.x;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.x;

1>	apply the default MAC main configuration as specified in 9.2.x;

1>	apply the CCCH configuration as specified in 9.1.1.X;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

Editor’s Note: FFS Details regarding default L1/L2 configurations (e.g. CCCH, physical channel, MAC, scheduling, etc.).

Editor’s Note: FFS Whether NR supports a timeAlignmentTimerCommon, whether is transmitted in SIB2 and UE behavior associated).

Editor’s Note: FFS Requirements on up to date system information acquisiton before connection setup.

#### 5.3.3.3	Actions related to transmission of RRCSetupRequest message

The UE shall set the contents of RRCSetupRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an 5G-S-TMSI:

3>	set the ue-Identity to ng-5G-s-tmsi-part1;

2>	else:

3>	draw a 39 bit random value in the range 0 .. 239-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the 5G-S-TMSI if the UE is registered in the TA of the current cell.

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.6.

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest:

2> discard the stored UE AS context, fullI-RNTI and shortI-RNTI, if stored;

2>	indicate to upper layers fallback of the RRC connection;

1> perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301 or T319 if running;

Editor’s Note: FFS Whether there is a need to define UE actions related to access control timers (equivalent to T302, T303, T305, T306, T308 in LTE). For example, informing upper layers if a given timer is not running.

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCSetupRequest:

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCSetupComplete message as follows:

2> if upper layers provide an 5G-S-TMSI:

3> if the RRCSetup is received in response to an RRCSetupRequest:

4>  set the ng-5g-s-tmsi-bits to ng-5G-s-tmsi-part2;

3> else:

4>  set the ng-5g-s-tmsi-bits set to ng-5g-s-tmsi;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroupConfig contains the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

2>	if upper layers provide the 'Registered AMF':

3> include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Region, amf-SetId, amf-Pointer to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

Editor’s Note: FFS Confirm whether the guami-Type is included and set in the abovementioned condition.

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [20]):

3>	include the s-nssai-list and set the content to the values provided by the upper layers;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends

#### 5.3.3.5	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15;

#### 5.3.3.6	Cell re-selection while T300 is running

Editor’s Note: FFS  Whether cell reselection actions need to be defined for other timers e.g. access control timers equivalent to T302, T303, T305, T306 and T308 in LTE).

The UE shall:

1>	if cell reselection occurs while T300 is running:

2>	stop timer T300;

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	inform upper layers about the failure to establish the RRC connection;

#### 5.3.3.7	T300 expiry

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2> if the T300 has expired a consecutive connEstFailCount times on the same cell for which connectionEstablishmentFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE :	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

#### 5.3.3.8	Abortion of RRC connection establishment

If upper layers abort the RRC connection establishment procedure while the UE has not yet entered RRC_CONNECTED, the UE shall:

1>	stop timer T300, if running;

1>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

Editor’s Note: FFS Discuss whether abortion of RRC connection establishment triggered upper layers is needed.

### 5.3.4	Initial security activation

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.3.4.1	General

Figure 5.3.4.1-1: Security mode command, successful

Figure 5.3.4.1-2: Security mode command, failure

The purpose of this procedure is to activate AS security upon RRC connection establishment.

#### 5.3.4.2	Initiation

The network initiates the security mode command procedure to a UE in RRC_CONNECTED. Moreover, the network applies the procedure as follows:

-	when only SRB1, is established, i.e. prior to establishment of SRB2 and/ or DRBs.

#### 5.3.4.3	Reception of the SecurityModeCommand by the UE

The UE shall:

1>	derive the KgNB key, as specified in TS 33.501 [11];

1>	derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	configure lower layers to apply SRB integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2>	configure lower layers to apply SRB ciphering using the indicated algorithm, the KRRCenc keyafter completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	consider AS security to be activated;

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	continue using the configuration used prior to the reception of the SecurityModeCommand message, i.e. neither apply integrity protection nor ciphering.

2>	submit the SecurityModeFailure message to lower layers for transmission, upon which the procedure ends;

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

In EN-DC, SRB3 can be used for measurement configuration and reporting, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, provided that the (re-)configuration does not require any MeNB involvement.

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one DRB is setup in SCG.

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration:

1>	if the RRCReconfiguration includes the fullConfig:

2>	perform the radio configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1> if the RRCReconfiguration includes the keyRefresh:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration message contains the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	set the content of RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent, or;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

1>  if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2> if RRCReconfiguration was received via SRB1:

3> submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

3> if reconfigurationWithSync was included in spCellConfig of an SCG:

4> initiate the random access procedure on the SpCell, as specified in TS 38.321 [3];

3> else:

4>  the procedure ends;

NOTE:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2> else (RRCReconfiguration was received via SRB3):

3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE:	For EN-DC, in the case of SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case of SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1 > else:

2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

1>  if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a random access procedure triggered above;

2>  stop timer T304 for that cell group;

2>  apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>  apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>  if the reconfigurationWithSync was included in spCellConfig of an MCG:

3> if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured or if the conditions defined in FFS_Spec [X], section FFS_Section are satisfied:

4> acquire the SIB1 of the target SpCell of the MCG, as specified in 5.2.2.3.1;

2>  the procedure ends.

NOTE:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA:

2> reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2> release the SCG configuration;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running.

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released.

#### 5.3.5.5	Cell Group configuration

##### 5.3.5.5.1	General

The network configures the UE with Master Cell Group (MCG), and zero or one Secondary Cell Group (SCG). For EN-DC, the MCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2> perform Reconfiguration with sync according to 5.3.5.5.2;

2> resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended;

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2> perform SCell addition/modification as specified in 5.3.5.5.9.

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	stop timer T310 for the corresponding SpCell, if running;

1>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

1>	if the frequencyInfoDL is included:

2>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

1>	else:

2>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

1>	start synchronising to the DL of the target SpCell and acquire the MIB of the target SpCell as specified in 5.2.2.3.1;

NOTE:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset the MAC entity of this cell group;

1>	consider the SCell(s) of this cell group, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

Editor’s Note: Verify that this does not configure some common parameters which are later discarded due to e.g. SCell release or due to LCH release.

1>	configure lower layers in accordance with the received spCellConfigCommon;

1>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id to be the active uplink bandwidth part;

1>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the active downlink bandwidth part;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

##### 5.3.5.5.3	RLC bearer release

The UE shall:

1>	for each logicalChannelIdentity value included in the rlc-BearerToReleaseList that is part of the current UE configuration (LCH release); or

1>	for each logicalChannelIdentity value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4, section 5.1.3] ;

2>	release the corresponding logical channel.

##### 5.3.5.5.4	RLC bearer addition/modification

For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE’s current configuration contains a RLC bearer with the received logicalChannelIdentity:

2> if reestablishRLC is received:

3> re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2> reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

NOTE:	The network does not re-associate an already configured logical channel with another radio bearer. Hence servedRadioBearer is not present in this case.

1> else (a logical channel with the given logicalChannelIdentity was not configured before):

2> if the logicalChannelIdentity corresponds to an SRB and rlc-Config is not included:

3> establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB;

2> else:

3> establish an RLC entity in accordance with the received rlc-Config;

2> if the logicalChannelIdentity corresponds to an SRB and if mac-LogicalChannelConfig is not included:

3> configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3> configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig;

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer.

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if MCG MAC is not part of the current UE configuration (i.e. MCG establishment):

2>	create an MCG MAC entity;

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

1>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig other than tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer.

##### 5.3.5.5.6	RLF Timers & Constants configuration

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

NOTE:	In EN-DC, rlf-TimersAndConstants cannot be released.

Editor’s Note: Standalone part to be complete by Sept 2018.

2>	if the cell group is MCG:

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

2>	else:

3>	stop timer T310 for this cell group, if running, and

3>	release the value of timer t310 as well as constants n310 and n311 for this cell group;

1>	else:

2>	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstants.

2>	stop timer T310 for this cell group, if running, and

2>	reset the counters N310 and N311

##### 5.3.5.5.7	SPCell Configuration

The UE shall:

1>	if the SpCellConfig contains the rlf-TimersAndConstants:

2>	configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6.

1>  if the SpCellConfig contains spCellConfigDedicated:

2> configure the SpCell in accordance with the spCellConfigDedicated;

2> if the any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

3>	stop timer T310 for the corresponding SpCell, if running;

3>	reset the counters N310 and N311.

##### 5.3.5.5.8	SCell Release

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell.

##### 5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

Editor’s Note: FFS Check automatic measurement handling for SCells.

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated.

#### 5.3.5.6	Radio Bearer configuration

##### 5.3.5.6.1	General

The UE shall perform the following actions based on a received RadioBearerConfig IE:

1>	if the RadioBearerConfig includes the srb3-ToRelease and set to true:

2>	perform the SRB release as specified in 5.3.5.6.2;

1>	if the RadioBearerConfig includes the srb-ToAddModList:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5.

1>	release all SDAP entities, if any, that have no associated DRB as specified in TS 37.324 [xx] section 5.1.2.

##### 5.3.5.6.2	SRB release

The UE shall:

1>	release the PDCP entity of the SRB3.

##### 5.3.5.6.3	SRB addition/modification

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity and configure it with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB), as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	re-establish the PDCP entity of this SRB as specified in 38.323 [5];

2>	else, if the discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

##### 5.3.5.6.4	DRB release

Editor’s Note: FFS / TODO: Add handling for the new QoS concept (mapping of flows; configuration of QFI-to-DRB mapping; reflective QoS...) but keep also EPS-Bearer handling for the EN-DC case

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration (DRB release):

2>	release the PDCP entity;

2> indicate the release of the DRB to SDAP entity associated with this DRB.( TS 37.324 [xx] section 5.3.3);

2>	if the UE is operating in EN-DC:

3> if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	if the procedure was triggered due to reconfiguration with sync:

5>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers after successful reconfiguration with sync;

4>	else:

5>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers immediately.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2> if an sdap-Config is included:

3> if an SDAP entity with the received pdu-Session does not exist:

4> establish an SDAP entity as specified in TS 37.324 [xx] section 5.1.1;

3> configure the SDAP entity in accordance with the received sdap-Config  as specified in TS 37.324 [xx] and associate the DRB with the SDAP entity;

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

3>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2> if the PDCP entity of this DRB is configured with integrityProtection:

3> configure the PDCP entity with the integrity algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2>	if the UE is operating in EN-DC:

3>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

4> associate the established DRB with the corresponding eps-BearerIdentity;

3> else:

4>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

- for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2> if an sdap-Config is included, reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [xx];

2>	if the reestablishPDCP is set:

3>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

4>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master or secondary key (KeNB/S-KgNB/KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3> if the PDCP entity of this DRB is configured with integrityProtection:

4> configure the PDCP entity with the integrity algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	re-establish the PDCP entity of this DRB as specified in 38.323 [5], section 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in 38.323;

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

NOTE 1:	Removal and addition of the same drb-Identity in a single radioResourceConfig is not supported. In case drb-Identity is removed and added due to reconfiguration with sync or re-establishment with the full configuration option, the network can use the same value of drb-Identity.

NOTE 2:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in.  To re-associate a DRB with a different key (KeNB to S-KeNB or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE 3:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 4: 	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 5: Ciphering and integrity protection can be enabled or disabled for a DRB. The enabling/disabling of cipheriong or integrity protection can be changed only by releasing and adding the DRB.

#### 5.3.5.7	Security key update

The UE shall:

1>	if the UE is operating in EN-DC:

2> upon reception of sk-Counter as specified in TS 36.331 [10]:

3>	update the S-KgNB key based on the KeNB key and using the received sk-Counter value, as specified in TS 33.401 [11];

3>	derive KRRCenc and KUPenc key as specified in TS 33.401 [11];

3>	derive the KRRCint and KUPint key as specified in TS 33.401 [11].

1>	else:

2 > if the keySetChangeIndicator is included in the received keyRefresh:

3>	if the keySetChangeIndicator is set to TRUE:

4>	derive or update the KgNB key based on the KAMF key, as specified in TS 33.501 [11];

3>	else:

4>	derive or update the KgNB key based on the current KgNB or the NH, using the nextHopChainingCount value indicated in the received keyRefresh, as specified in TS 33.501 [11];

2>	if the n2ModeNAS-Container is included in the received keyRefresh:

3>	forward the n2ModeNAS-Container to the upper layers;

3>	derive the KgNB key as specified in TS 33.501 [11];

2> store the nextHopChainingCount value;

2> derive the keys associated with KgNB as follows:

2> if the securityAlgorithmConfig is included in the received SecurityConfig:

3>	derive KRRCenc and KUPenc key associated with the cipheringAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

3>	derive the KRRCint and KUPint key associated with the integrityProtAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

2> else:

3>	derive KRRCenc and KUPenc key associated with the current cipheringAlgorithm, as specified in TS 33.501 [11];

3>	derive the KRRCint and KUPint key associated with the current integrityProtAlgorithm, as specified in TS 33.501 [11];

NOTE:	Ciphering and integrity protection are optional to configure for the DRBs.

#### 5.3.5.8	Reconfiguration failure

##### 5.3.5.8.1	Integrity check failure

Editor’s Note: Removed "SIB3" from heading so that this sub-section can easily be expanded to stand-alone case (if considered necessary). FFS_Standalone

The UE shall:

1>	upon integrity check failure indication from NR lower layers for SRB3:

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SRB3 integrity check failure.

##### 5.3.5.8.2	Inability to comply with RRCReconfiguration

The UE shall:

1>	if the UE is operating in EN-DC:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

2>	else, if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over MCG SRB1;

3> continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the connection re-establishment procedure as specified in TS 36.331 [10, 5.3.7], upon which the connection reconfiguration procedure ends.

1>	else:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause other;

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends;

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure)

The UE shall:

1>	if T304 of a secondary cell group expires:

2>  release dedicated preambles provided in rach-ConfigDedicated;

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

#### 5.3.5.9	Other configuration

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.3.5.10	EN-DC release

The UE shall:

1>	as a result of EN-DC release triggered by E-UTRA:

2> release SRB3 (configured according to radioBearerConfig), if present;

2>	release measConfig;

2> release the SCG configuration as specified in section 5.3.5.4.

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except the MCG C-RNTI and the security configurations associated with the master key;

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig.

1>	if the spCellConfig in the masterCellGroupConfig includes the reconfigurationWithSync (handover):

2>	release/ clear all current common radio configurations;

2>	use the default values specified in 9.2.x for timer T310, T311 and constant N310, N311;

1>	else (full configuration after re-establishment):

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1

1>	apply the default physical channel configuration as specified in 9.2.x;

1>	apply the default semi-persistent scheduling/configured grant configuration as specified in 9.2.x;

1>	apply the default MAC main configuration as specified in 9.2.x;

1>	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

2>	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

2>	apply the corresponding default PDCP configuration for the SRB specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2;

2>	apply the corresponding default RLC configuration for the SRB specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2;

2>	apply the corresponding default logical channel configuration for the SRB as specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2;

NOTE 2: 	This is to get the SRBs (SRB1 and SRB2 for handover and SRB2 for reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

### 5.3.6	Counter check

FFS

### 5.3.7	RRC connection re-establishment

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.3.7.1	General

Figure 5.3.7.1-1: RRC connection re-establishment, successful

Figure 5.3.7.1-2: RRC re-establishment, fallback to RRC establishment, successful

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which security has been activated, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to section 5.3.3.4. If AS security has not been activated, the UE does not initiate the procedure but instead moves to RRC_IDLE directly.

The network applies the procedure as follows:

-	When AS security has been activated and the network retrieves or verifies the UE context:

-	to re-activate AS security without changing algorithms;

-	to re-establish and resume the SRB1;

-	When UE is re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context:

-	to discard the stored AS Context and release all RB;

-	fallback to establish a new RRC connection.

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause  5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2;

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1> stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured, in accordance with sub-clause 5.3.5.5.8;

1>	apply the default physical channel configuration as specified in sub-clause x.x.x;

1>	apply the default semi-persistent scheduling configuration as specified in sub-clause  x.x.x;

1>	apply the default MAC main configuration as specified in x.x.x;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [21];

Editor’s Note: FFS How to handle L1/L2 default configurations for RRC re-establishment.

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	stop timer T311;

1>	start timer T301;

1>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4

NOTE:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

Editor’s Note: FFS Whether NR supports a timeAlignmentTimerCommon, whether is transmitted in SIB2 and UE behavior associated).

FFS Whether NR supports a timeAlignmentTimerCommon, whether is transmitted in SIB2 and UE behavior associated).

#### 5.3.7.4	Actions related to transmission of RRCReestablishmentRequest message

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source PCell (reconfiguration with sync or mobility from NR failure) or used in the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the X least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	 if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.8.2:

3>	set the reestablishmentCause to the value reconfigurationFailure;

2>	else if the re-establishment procedure was initiated due to reconfiguration with sync failure as specified in 5.3.5.8.3 (intra-NR handover failure) or 5.4.3.5 (inter-RAT mobility from NR failure):

3>	set the reestablishmentCause to the value handoverFailure;

2>	else:

3>	set the reestablishmentCause to the value otherFailure;

1> restore the RRC configuration and security context from the stored UE AS context;

1>	restore the PDCP state and re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	resume SRB1;

1>	The UE shall submit the RRCReestablishmentRequest message to lower layers for transmission.

#### 5.3.7.5	Reception of the RRCReestablishment by the UE

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1> perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

Editor’s Note: FFS Which parts of the mastercellGroup and radioBearerConfig IEs are applicable to the re-establishment case

1>	store the nextHopChainingCount value indicated in the RRCReestablishment message;

1>	update the KgNB key based on the current KgNB or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.501 [11];

1>	derive the KRRCint key and the KUPint key associated with the previously configured integrity protection algorithm, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the RRCReestablishment message, using the previously configured algorithm and the KRRCint key;

1>	if the integrity protection check of the RRCReestablishment message fails:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other', upon which the procedure ends;

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	the procedure ends;

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

#### 5.3.7.7	T301 expiry or selected cell no longer suitable

The UE shall:

1>	if timer T301 expires; or

1>	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in TS 38.304 [21]:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

#### 5.3.7.8	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection establishment procedure as specified in 5.3.3.4.

### 5.3.8	RRC connection release

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.3.8.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	 to release the RRC connection, which includes the release of the established radio bearers as well as all radio resources; or

-	to suspend the RRC connection, which includes the suspension of the established radio bearers.

#### 5.3.8.2	Initiation

The network initiates the RRC connection release procedure to transit a UE in RRC_CONNECTED to RRC_IDLE; or to transit a UE in RRC_CONNECTED to RRC_INACTIVE; or to transit a UE in RRC_INACTIVE  back to RRC_INACTIVE when the UE tries to resume; or to transit a UE in RRC_INACTIVE to RRC_IDLE when the UE tries to resume. The procedure can also be used to release and redirect a UE to another frequency.

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

Editor’s Note: FFS Whether RRCRelease supports a mechanim equivalent to loadBalancingTAURequired.

1>	if deprioritisationReq is included:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

1>	if the RRCRelease includes suspendConfig:

2> if UE has stored fullI-RNTI, shortI-RNTI, nextHopChainingCount, periodic-RNAU-timer and ran-PagingCycle:

3> release the previously stored fullI-RNTI, shortI-RNTI, nextHopChainingCount and ran-PagingCycle provided in suspendConfig;

2> store fullI-RNTI, shortI-RNTI, nextHopChainingCount, periodic-RNAU-timer and ran-PagingCycle provided in suspendConfig;

2> if suspendConfig includes ran-NotificationAreaInfo;

3> if the UE has stored ran-NotificationAreaInfo;

4> release the previously stored ran-NotificationAreaInfo;

3> store the ran-NotificationAreaInfo provided in suspendConfig;

2>	reset MAC;

2>	re-establish RLC entities for all SRBs and DRBs;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest:

3> stop the timer T319 if running;

3> replace any previously stored security context with newly received security context in the suspendConfig;

3> replace the previously stored C-RNTI with the temporary C-RNTI in the cell the UE has received the RRCRelease message;

3> replace the previously stored cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

3> replace the previously stored physical cell identity with the physical cell identity of  the cell the UE has received the RRCRelease message;

2>	else:

3> store the UE AS Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, SDAP configuration, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

2>	suspend all SRB(s) and DRB(s), except SRB0;

2>	start timer T380, with the timer value set to periodic-RNAU-timer;

2>	indicate the suspension of the RRC connection to upper layers;

2> enter RRC_INACTIVE and perform procedures as specified in TS 38.304 [21]

1>	else

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11;

Editor’s Note: FFS Whether there needs to be different release causes and actions associated.

#### 5.3.8.4	T320 expiry

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information;

#### 5.3.8.5	UE actions upon the expiry of DataInactivityTimer

Upon receiving the expiry of DataInactivityTimer from lower layers while in RRC_CONNECTED, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

### 5.3.9	RRC connection release requested by upper layers

#### 5.3.9.1	General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

NOTE:	Upper layers invoke the procedure, e.g. upon determining that the network has failed an authentication check, see TS 24.501 [23].

#### 5.3.9.2	Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>	if the upper layers indicate barring of the PCell:

2>	treat the PCell used prior to entering RRC_IDLE as barred according to TS 38.304 [20];

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';

### 5.3.10	Radio link failure related actions

#### 5.3.10.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while T311 is not running:

2>	start timer T310 for the corresponding SpCell.

#### 5.3.10.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell.

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by layer 1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while T311 is not running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

Editor’s Note: FFS whether maximum ARQ retransmission is only criteria for RLC failure.

2>	consider radio link failure to be detected for the MCG i.e. RLF;

Editor’s Note: FFS Whether indications related to beam failure recovery may affect the declaration of RLF.

Editor’s Note: FFS: How to handle RLC failure in CA duplication for MCG DRB and SRB.

Editor’s Note: FFS: RLF related measurement reports e.g. VarRLF-Report is supported in NR.

2>	if AS security has not been activated:

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

Editor’s Note: FFS: How to handle RLC failure in CA duplication for SCG DRB and SRB.

2>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

### 5.3.11	UE actions upon going to RRC_IDLE

Editor’s Note: Targeted for completion in Sept 2018.

UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320 and T325;

1>	discard any stored AS context, fullI-RNTI, shortIRNTI, ran-PagingCycle and ran-NotificationAreaInfo;

1>	discard the security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key, if stored;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	enter RRC_IDLE and perform procedures as specified in TS 38.304 [21], except if going to RRC_IDLE was triggered by reception of the MobilityFromNRCommand message or by selecting an inter-RAT cell while T311 was running;

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources c1onfigured in CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances configured in SRS-Config.

### 5.3.13	RRC connection resume

#### 5.3.13.1	General

Figure 5.3.13.1-1: RRC connection resume, successful

Figure 5.3.13.1-2: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.13.1-3: RRC connection resume followed by network release, successful

Figure 5.3.13.1-4: RRC connection resume followed by network suspend, successful

Figure 5.3.13.1-5: RRC connection resume, network reject

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s) and DRB(s) or perform an RNA update.

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to NG-RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

NOTE: 	As one UE implementation option, upper layers may request access barring check as specified in the unified acccess control procedure in 5.3.14 and only request the RRC resumption when the access attempt is considered allowed. As another UE implementation option, upper layers may instead provide an Access Category and one or more Access Identities upon requesting the RRC resumption so that RRC initiates the access barring check.

Upon initiation of the procedure, the UE shall:

Editor’s Note: FFS Whether SCG configuration should be released or whether that should be treated as any other configuration (i.e. with delta signalling).

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting the resumption of an RRC connection:

2>	perform the unified acccess control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

Editor’s Note: FFS How to capture the different options for AS/NAS modeling for barring check for connection resume.

1>	if the resumption of the RRC connection is triggered due to an RNA update:

2>	if an emergency service is ongoing:

NOTE: 	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

2>	else:

3>	select [the Access Category corresponding to RNA update] as the Access Category;

Editor’s note: FFS whether to use access category 3 for MO-signalling or a standardised RAN specific access category for RNA update.

2>	perform the unified acccess control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to 'TRUE';

4>	the procedure ends;

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified acccess control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	apply the default Serving Cell configuration as specified in 9.2.x3;

1>	apply the default physical Cell Group configuration as specified in 9.2.x2;

1>	apply the default MAC Cell Group configuration as specified in 9.2.x1;

1>	apply the CCCH configuration as specified in 9.1.1.2;

Editor’s Note: FFS Whether NR supports a timeAlignmentTimerCommon, whether is transmitted in SIB2 and U	E behavior associated).

1>	start timer T319;

1>	stop timer T380;

1>	initiate transmission of the RRCResumeRequest message in accordance with 5.3.13.3;

Editor’s Note: FFS Requirements on up to date system information acquisiton before connection resumption.

#### 5.3.13.3	Actions related to transmission of RRCResumeRequest message

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the shortResumeIdentity to the stored shortI-RNTI value;

1>	set the resumeCause in accordance with the information received from upper layers or from AS layer;

1>	set the resumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarResumeMAC-Input;

2>	with the KRRCint key and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

Editor’s Note: FFS Additional input to VarResumeMAC-Input (replay attacks mitigation).

1>	restore the RRC configuration and security context from the stored UE AS context except the cellGroupConfig;

1>	update the KgNB key based on the current KgNB or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key;

1>	configure lower layers to resume integrity protection for all radio bearers except SRB0 using the previously configured algorithm and the KRRCint key and KUPint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

NOTE 1:	Only DRBs with previously configured UP integrity protection shall resume integrity protection.

1>	configure lower layers to resume ciphering for all radio bearers except SRB0 and to apply the previously configured ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	restore the PDCP state and re-establish PDCP entities for SRB1;

1>	resume SRB1;

1> submit the RRCResumeRequest message to lower layers for transmission;

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running, perform actions specified in 5.3.13.5.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	restore the PDCP state, reset COUNT value and re-establish PDCP entities for SRB2 and all DRBs;

2>	restore the cellGroupConfig from the stored UE AS context;

2>	if drb-ContinueROHC is included:

3>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

3>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

2>	else:

3>	indicate to lower layers that stored UE AS context is used;

3>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

1>	discard the fullI-RNTI, shortI-RNTI and the stored UE AS context, except ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

Editor’s Note: FFS Whether it is supported to configure secondaryCellGroup at Resume.

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

Editor’s Note: FFS Whether there needs to be a second radioBearerConfig.

1>	resume SRB2 and all DRBs;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

Editor’s Note: FFS Whether there is a need to define UE actions related to access control timers (equivalent to T302, T303, T305, T306, T308 in LTE). For example, informing upper layers if a given timer is not running.

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>  if the upper layer provides NAS PDU, set the dedicatedInfoNAS to include the information received from upper layers;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

#### 5.3.13.5	T319 expiry or Integrity check failure from lower layers while T319 is running

The UE shall:

1>	if timer T319 expires or upon receiving Integrity check failure indication from lower layers while T319 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause RRC Resume failure;

#### 5.3.13.6	Cell re-selection while T319 or T302 is running

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running:

2>	set the variable pendingRnaUpdate to 'FALSE', if that is set to TRUE;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause RRC Resume failure;

#### 5.3.13.7	Reception of the RRCSetup by the UE

The UE shall:

1>	set the variable pendingRnaUpdate to 'FALSE';

1>	perform the RRC connection setup procedure as specified in 5.3.3.4;

#### 5.3.13.8	RNA update

Upon entering RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if upon cell reselection the UE enters an RNA not belonging to the configured ran-NotificationAreaInfo:

2>	if upper layers request resumption of an RRC connection;

3>	initiate RRC connection resume procedure in 5.3.13 with cause value set in accordance with the information received from upper layers;

2>	else:

3>	initiate RRC connection resume procedure in 5.3.13 with cause value set to ‘rna-Update’;

Editor’s note: FFS How to handle simultaneous NAS triggered events and AS triggered events (except TAU and RNAU, which has been explicitly agreed).

1> if barring is alleviated for Access Category [the Access Category corresponding to RNA update], as specified in 5.3.14.4:

Editor’s note: FFS whether to use access category 3 for MO-signalling or a standardised RAN specific access category for RNA update.

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRnaUpdate is set to 'TRUE':

3> set the variable pendingRnaUpdate to 'FALSE';

3>	initiate RRC connection resume procedure in 5.3.13.2 with cause value set to ‘rna-Update’;

### 5.3.14	Unified Access Control

#### 5.3.14.1	General

The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer.

#### 5.3.14.2	Initiation

Upon initiation of the procedure, the UE shall:

1>	if timer [T30x] is running for the Access Category:

2>	consider the access attempt as barred;

1>	if timer T302 is running and the Access Category is neither '2' nor '0':

2>	consider the access attempt as barred;

1>	else:

Editor’s note: FFS whether indication/selection of the Access Category for RRC Resume is described in this section or not.

2>	if the Access Category is ‘0’:

3>	consider the access attempt as allowed;

2>	else:

3>	if SIB1 includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 24.501 [23]):

4>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SIB1;

3>	else

4>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SIB1;

3>	if the uac-ACBarringListType indicated that uac-ExplicitACBarringList is used:

4>	if the corresponding UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

5>	if the uac-BarringInfoSetList contain a UAC-BarringInfoSet entry corresponding to the uac-barringInfoSetIndex in the UAC-BarringPerCat:

6>	select the UAC-BarringInfoSet entry;

6> perform access barring check for the Access Category as specified in 5.3.14.5, using the UAC-BarringInfoSet as "UAC barring parameter";

5>	else:

6> consider the access attempt as allowed;

4>	else:

5> consider the access attempt as allowed;

3>	select the UAC-BarringInfoSetIndex entry from uac-ImplicitACBarringList entry corresponding to the Access Category;

4>	perform access barring check for the Access Category as specified in 5.3.14.5, using  uac-BarringInfo corresponding to UAC-BarringInfoSetIndex as "UAC barring parameter";

1>	if the access barring check was requested by RRC for an RRC establishment or for an RRC resumption requested by upper layers; or

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends;

#### 5.3.14.3	Cell re-selection while T30x is running

The UE shall:

1>	if cell reselection occurs while [T30x] is running:

2>	stop timer [T30x];

2>	perform the actions as specified in 5.3.14.4.

Editor’s note: FFS whether T30x is stopped due to cell reselection (e.g. as in LTE).

#### 5.3.14.4	Barring alleviation

The UE shall:

1>	if timer T302 expires or is stopped, and if timer [T30x] corresponding to an Access Category is not running; or

1>	if timer [T30x] corresponding to an Access Category expires or is stopped, and if timer T302 is not running:

2>	consider the barring for this Access Category to be alleviated;

1>	When barring for an access category is considered being alleviated:

2>	if the Access Category was provided upon access barring check requested by upper layers:

3>	inform upper layers about barring alleviation for the Access Category;

#### 5.3.14.5	Access barring check

The UE shall:

1>	if one or more Access Identities are indicated by upper layers according to TS 24.501 [23] or obtained by the RRC layer, and

1>	if for at least one of these Access Identities the corresponding bit in the uac-BarringForAccessIdentity contained in "UAC barring parameter" is set to zero:

2>	consider the access attempt as allowed;

1>	else:

2>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

2>	if 'rand' is lower than the value indicated by uac-BarringFactor included in "UAC barring parameter":

3>	consider the access attempt as allowed;

2>	else:

3>	consider the access attempt as barred;

1>	if the access attempt is considered as barred:

2>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2>	start timer [T30x] for the Access Category with the timer value calculated as follows, using the uac-BarringTime included in "AC barring parameter":

"Tbarring" = (0.7+ 0.6 * rand) * uac-BarringTime;

### 5.3.15	RRC connection reject

#### 5.3.15.1	Initiation

The UE initiates the procedure upon the reception of RRCReject when the UE tries to establish or resume an RRC connection.

#### 5.3.15.2	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if runing;

1>	stop timer T319, if running;

1>	reset MAC and release the MAC configuration;

1>	start timer T302, with the timer value set to the waitTime;

1>	set the variable pendingRnaUpdate to 'FALSE';

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories ‘0’and '2';

1>	if RRCReject is received in response to an RRCSetupRequest:

2>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

1>	if RRCReject is received in response to an RRCResumeRequest:

2>	if resume is triggered by upper layers:

2>	inform upper layers about the failure to resume the RRC connection;

Editor’s note: FFS In which cases upper layers are informed that a resume failure occurred upon the reception of RRC Reject.

2>	if resume is triggered by RRC:

3>	set the variable pendingRnaUpdate to 'TRUE';

2>	discard the security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key;

2>	suspend SRB1, upon which the procedure ends;

Editor’s note: FFS Handling of timer T380 upon Reject e.g. stop, re-start, etc.

The UE shall continue to monitor RAN and CN paging while the timer T302 is running.

## 5.4	Inter-RAT mobility

#### 5.4.1	Introduction

NR support network controlled inter-RAT mobility between NR and E-UTRA which can be connected to either EPC or 5GC.

#### 5.4.2	Handover to NR

##### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA.

##### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to NR procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

The network applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to re-establish SRBs and one or more DRBs;

##### 5.4.2.3	Reception of the RRCReconfiguration by the UE

The UE shall:

1>	start timer T304 with the timer value set to t304, as included in the reconfigurationWithSync;

- perform RRC reconfiguration procedure as specified in 5.3.5;

NOTE: 	In the case that UE is connected to 5GC in the source E-UTRA cell, the delta configuration can be carried in RRCReconfiguration, e.g. PDCP-config IE and SDAP-config IE can be absent, which means the PDCP entity and SDAP entity established in source cell of the DRBs admitted by target cell could be maintained in the target cell, in order to perform lossless intra-system inter-RAT handover.

##### 5.4.2.4	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with any part of the configuration included in the RRCReconfiguration message:

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

#### 5.4.2.5	Expiry of T304

The UE shall:

1>	upon T304 expiry (handover to NR failure):

2>	reset MAC;

#### 2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

#### 5.4.3	Mobility from NR

##### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from NR, successful

Figure 5.4.3.1-2: Mobility from NR, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using other RAT, e.g. E-UTRA. The mobility from NR procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell;

##### 5.4.3.2	Initiation

The network initiates the mobility from NR procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message, by sending a MobilityFromNRCommand message. The network applies the procedure as follows:

-	the procedure is initiated only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended;

#### 5.4.3.3	Reception of the MobilityFromNR by the UE

The UE shall:

1>	if the targetRAT-Type is set to eutra:

2>	consider inter-RAT mobility as initiated towards E-UTRA;

1>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

#### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';

##### 5.4.3.5	Mobility from NR failure

The UE shall:

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

- if the UE is unable to comply with any part of the configuration included in the MobilityFromNRCommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromNRCommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2>	revert back to the configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in subclause 5.3.7;

## 5.5	Measurements

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements and report them in accordance with the measurement configuration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

- 	The measObjectIdof the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single EUTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description;.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements, i.e. no (UL, DL) transmissions are scheduled.

AUE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) - these are the SpCell and one or more SCells.

2.	Listed cells - these are cells listed within the measurement object(s).

3.	Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells.

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

### 5.5.2	Measurement configuration

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig, it includes a measObject for the SpCell and for each NR SCell to be measured;

-	to configure at most one measurement identity using a reporting configuration with the reportType set to reportCGI;

-	to ensure that, for all SSB based reporting configurations have at most one measurement object with the same ssbFrequency and ssbSubcarrierSpacing;

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the quantityConfig:

2>	perform the quantity configuration procedure as specified in 5.5.2.8;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

1>	if the received measConfig includes the measGapSharingConfig:

2>	perform the measurement gap sharing configuration procedure as specified in 5.5.2.11;

1>	if the received measConfig includes the s-MeasureConfig:

2>	if s-MeasureConfig is set to ssb-RSRP, set parameter ssb-RSRP of s-MeasureConfig within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-MeasureConfig;

2>	else, set parameter csi-RSRP of s-MeasureConfig within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-MeasureConfig.

#### 5.5.2.2	Measurement identity removal

The UE shall:

1>	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer if running and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

#### 5.5.2.3	Measurement identity addition/modification

The network applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured.

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId.

2>	if the reportType is set to reportCGI in the reportConfig associated with this measId;

3>	if the measObject associated with this measId concerns E-UTRA:

4>	start timer T321 with the timer value set to X seconds for this measId;

3>	if the measObject associated with this measId concerns NR:

4>	start timer T321 with the timer value set to Y seconds for this measId;

#### 5.5.2.4	Measurement object removal

The UE shall:

1>	for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig:

2>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

#### 5.5.2.5	Measurement object addition/modification

The UE shall:

1>	for each measObjectId included in the received measObjectToAddModList:

2>	if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, blackCellsToAddModList, whiteCellsToAddModList, cellsToRemoveList, blackCellsToRemoveList and whiteCellsToRemoveList;

3>	if the received measObject includes the cellsToRemoveList:

4>	for each physCellId included in the cellsToRemoveList:

5>	remove the entry with the matching physCellId from the cellsToAddModList;

3>	if the received measObject includes the cellsToAddModList:

4>	for each physCellId value included in the cellsToAddModList:

5>	if an entry with the matching physCellId exists in the cellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the cellsToAddModList;

3>	if the received measObject includes the blackCellsToRemoveList:

4>	for each pci-RangeIndex included in the blackCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the blackCellsToAddModList;

NOTE:	For each pci-RangeIndex included in the blackCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the black list of cells only if all cell indexes containing it are removed.

3>	if the received measObject includes the blackCellsToAddModList:

4>	for each pci-RangeIndex included in the blackCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the blackCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the blackCellsToAddModList;

3>	if the received measObject includes the whiteCellsToRemoveList:

4>	for each pci-RangeIndex included in the whiteCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the whiteCellsToAddModList;

3>	if the received measObject includes the whiteCellsToAddModList:

4>	for each pci-RangeIndex included in the whiteCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the whiteCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the whiteCellsToAddModList;

3>	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received measObject to the measObjectList within VarMeasConfig.

#### 5.5.2.6	Reporting configuration removal

The UE shall:

1>	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

#### 5.5.2.7	Reporting configuration addition/modification

The UE shall:

1>	for each reportConfigId included in the received reportConfigToAddModList:

2>	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this reportConfig;

3>	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig.

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId.

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	if mgta is configured, apply the specified timing advance to the gap occurences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurences);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of SCG cells on FR2):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	if mgta is configured, apply the specified timing advance to the gap occurences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	if mgta is configured, apply the specified timing advance to the gap occurences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration.

NOTE 2: For gapFR1 or gapUE configuration, the SFN and subframe of the PCell is used in the gap calculation.

#### 5.5.2.10	Reference signal measurement timing configuration

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10)) mod T;

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition:

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS.

#### 5.5.2.11	Measurement gap sharing configuration

The UE shall:

1>	if gapSharingFR1 is set to setup:

2>	if an FR1 measurement gap sharing configuration is already setup, release the measurement gap sharing configuration;

2>	setup the FR1 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received measGapSharingScheme as defined in TS 38.133 [14];

1>	else if gapSharingFR1 is set to release:

2>	release the FR1 measurement gap sharing configuration;

1>	if gapSharingFR2 is set to setup:

2>	if an FR2 measurement gap sharing configuration is already setup, release the measurement gap sharing configuration;

2>	setup the FR2 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received measGapSharingScheme as defined in TS 38.133 [14];

1>	else if gapSharingFR2 is set to release:

2>	release the FR2 measurement gap sharing configuration.

1>	if gapSharingUE is set to setup:

2>	if a per UE measurement gap sharing configuration is already setup, release the per UE measurement gap sharing configuration;

2>	setup the per UE measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received measGapSharingScheme as defined in TS 38.133 [14];

1>	else if gapSharingUE is set to release:

2>	release the per UE measurement gap sharing configuration.

### 5.5.3	Performing measurements

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results in RRC_CONNECTED the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria and measurement reporting. For cell measurements, the network can configure RSRP, RSRQ or SINR as trigger quantity. Reporting quantities can be the same as trigger quantity or combinations of quantities (i.e. RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR).

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact layer 1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb:

3>	if at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRsIndexes and maxNrofRSIndexesToReport:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

2>	if at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs:

3>	if at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRsIndexes and maxNrofRSIndexesToReport:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	if at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the associated reportConfig contains rsType set to ssb:

3>	if the measId contains a reportQuantityRsIndexes and maxNrofRSIndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the associated reportConfig contains rsType set to csi-rs:

3>	if the measId contains a reportQuantityRsIndexes and maxNrofRSIndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is set to reportCGI:

3>	perform the corresponding measurements on the frequency and RAT indicated

3>	try to acquire the global cells identity of the cells indicated by the cellForWhichToReportCGI by acquiring the relevant system information from the concerned cell;

3>	if an entry in the cellAccessRelatedInfoList includes the selected PLMN, acquire the relevant system information from the concerned cell;

3>	if the cells indicated by the cellslForWhichToReportCGI is an NR cell:

4>	try to acquire the trackingAreaCode in the concerned cell;

4>	try to acquire the list of additional PLMN Identities, as included in the plmn-IdentityList, if multiple PLMN identities are broadcast in the concerned cell;

4>	try to acquire the list of additional frequency band, as included in the frequencyBandList, if multiple frequency bands are broadcast in the concerned cell;

4>	if cellAccessRelatedInfoList is included, use trackingAreaCode and plmn-IdentityList from the entry of cellAccessRelatedInfoList containing the selected PLMN;

Editor’s Note: FFS Capture inter-RAT EUTRAN CGI reporting when ASN.1 for measObjectEUTRA and reportConfig-IRAT is finalized.

2>	if the reportType for the associated reportConfig is periodical or eventTriggered:

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP,or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRsIndexes and maxNrofRSIndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRsIndexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for each trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRsIndexes and maxNrofRSIndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRsIndexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for each trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject;

2>	perform the evaluation of reporting criteria as specified in 5.5.4.

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity and for each beam measurement quantity that the UE performs measurements according to 5.5.3.1:

2>	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1is the old filtered measurement result, where F0is set to M1 when the first measurement result from the physical layer is received; and

a = 1/2(ki/4) , where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in 38.331 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria or for measurement reporting, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133[14]. For further details about the physical layer measurements, see TS 38.133 [14].

#### 5.5.3.3	Derivation of cell measurement results

The network may configure the UE to derive RSRP, RSRQ and SINR measurement results per cell associated to NR measurement objects based on parameters configured in the measObject (e.g. maximum number of beams to be averaged and beam consolidation thresholds) and in the reportConfig (rsType to be measured, SS/PBCH block 	or CSI-RS).

The UE shall:

1>	for each cell measurement quantity to be derived based on SS/PBCH block:

2>	if nrofSS-BlocksToAverage in the associated measObject is not configured; or

2>	if absThreshSS-BlocksConsolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below absThreshSS-BlocksConsolidation:

3>	derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on SS/PBCH block as the linear power scale average of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage;

2>	apply layer 3 cell filtering as described in 5.5.3.2;

1>	for each cell measurement quantity to be derived based on CSI-RS:

2>	consider a CSI-RS resource to be applicable for deriving cell measurements when the concerned CSI-RS resource is included in the csi-rs-ResourceCellMobilityincludingthe physCellId of the cell in theCSI-RS-ConfigMobility in the associated measObject;

2>	if nrofCSI-RS-ResourcesToAverage in the associated measObject is not configured; or

2>	if absThreshCSI-RS-Consolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below absThreshCSI-RS-Consolidation:

3>	derive each cell measurement quantity based on applicable CSI-RS resources for the cell as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on CSI-RS as the linear power scale average of the highest beam measurement quantity values above absThreshCSI-RS-Consolidation where the total number of averaged beams shall not exceed nrofCSI-RS-ResourcesToAverage;

2>	apply layer 3 cell filtering as described in 5.5.3.2.

#### 5.5.3.3a	Derivation of layer 3 beam filtered measurement

The UE shall:

1>	for each layer 3 beam filtered measurement quantity to be derived based on SS/PBCH block;

2>	derive each configured beam measurement quantity based on SS/PBCH block as described in TS 38.215[9], and apply layer 3 beam filtering as described in 5.5.3.2;

1>	for each layer 3 beam filtered measurement quantity to be derived based on CSI-RS;

2>	derive each configured beam measurement quantity based on CSI-RS as described in TS 38.215 [9], and apply layer 3 beam filtering as described in 5.5.3.2.

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfigincludes a reportType set to eventTriggered or periodical;

3>	if the corresponding measObject concerns NR;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	for events involving a serving cell associated with a measObjectNRand neighbours  associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

5>	if useWhiteCellList is set to TRUE:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNRto be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNRto be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA;

4>		consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModListEUTRAN defined within the VarMeasConfig for this measId;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	if the reportAmount exceeds 1:

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell;

4>	else (i.e. the reportAmount is equal to 1):

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and for the strongest cell among the applicable cells;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	if reportType is set to reportCGI;

3>	if the UE acquired the information needed to set all fields of cgi-Info for the requested cell; or

3>	if the UE detects that the requested NR cell is not transmitting SIB1 (see TS 38.213 [13], section 13):

4>	stop timer T321;

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

4>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon the expiry of T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType or the corresponding reportConfig is set to reportCGI:

3>	consider any neighbouring cell detected on the associated measurement object which has a physical cell identity matching the value of the cellForWhichToReportCGI to be applicable;

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the NR serving cell corresponding to the associated measObjectNR associated with this event.

Inequality A1-1 (Entering condition)

Inequality A1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresisas defined within reportConfigNRfor this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined withinreportConfigNRfor this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated bythe measObjectNRassociated to this event.

Inequality A2-1 (Entering condition)

Inequality A2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	use the PSCell for Mp, Ofp and Ocp.

NOTE	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCellmeasObjectNR.

Inequality A3-1 (Entering condition)

Inequality A3-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement objectspecific offset of the reference signalof the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement objectspecific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined withinreportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled.

Inequality A4-1 (Entering condition)

Inequality A4-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement objectspecific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour/SCell becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the PSCell for Mp.

NOTE:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5-1 (Entering condition 1)

Inequality A5-2 (Entering condition 2)

Inequality A5-3 (Leaving condition 1)

Inequality A5-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mnis the measurement result of the neighbouring cell/SCell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour/SCell cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell/SCell).

Ocn is the cell specific offset of the neighbour cell/SCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell/SCell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNRfor this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell corresponding to the measObjectNRassociated to this event to be the serving cell.

NOTE:	The reference signal(s) of the neighbour(s) and the reference signal(s) of the SCell are both indicated in the associated measObjectNR.

Inequality A6-1 (Entering condition)

Inequality A6-2 (Leaving condition)

The variables in the formula are defined as follows:

Mnis the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within the associatedmeasObjectNR ), and set to zero if not configured for the neighbour cell.

Msis the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within the associatedmeasObjectNR), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined withinreportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined withinreportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

#### 5.5.4.8	Event B1 (Inter RAT neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition B1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B1-2, as specified below, is fulfilled;

Inequality B1-1 (Entering condition)

Inequality B1-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. b1-ThresholdEUTRA as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm or in dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.9	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition B2-1 and condition B2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B2-3 or condition B2-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality B2-1 (Entering condition 1)

Inequality B2-2 (Entering condition 2)

Inequality B2-3 (Leaving condition 1)

Inequality B2-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the inter-RAT neighbour cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. b2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. b2-Threshold2EUTRA as defined within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mn is expressed in dBm or dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR for each configured serving cell derived based on the rsType indicated in the associated reportConfig;

1>	set the measResultServingCell within measResultServingMOList to include for each NR serving cell that is configured with servingCellMO, if any, the servCellId;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRsIndexes and maxNrofRSIndexesToReport:

2>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>for each serving cell measObjectId referenced in the measIdList, other than the measObjectIdcorresponding with the measId that triggered the measurement reporting:

3>	set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCelland rsTypeindicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNRwith the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

3>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRsIndexes and maxNrofRSIndexesToReport:

4>	for each best non-serving cell included in the measurement report:

5>include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	if reportQuantityRsIndexesand maxNrofRSIndexesToReport areconfigured, include beam measurement information as described in 5.5.5.2;

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the reportType is set to eventTriggered:

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns NR:

6>	if rsType in the associated reportConfig is set to ssb:

7> set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in order of decreasing trigger quantity, i.e. the best cell is included first:

8>	if reportQuantityRsIndexesand maxNrofRSIndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

6>	else if rsType in the associated reportConfig is set to csi-rs:

7> set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in order of decreasing trigger quantity, i.e. the best cell is included first:

8> if reportQuantityRsIndexesand maxNrofRSIndexesToReport are, include beam measurement information as described in 5.5.5.2;

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in order of decreasing triggerQuantity, i.e. the best cell is included first;

3>	if the reportType is set to reportCGI:

4>	if the cell indicated by cellForWhichToReportCGI is a NR cell:

5>	if the UE has acquired the mandatory present fields of the cgi-Info for the cell indicated by cellForWhichToReportCGI in the associated reportConfig:

6>	include the global cell identity, tracking are code and RAN area code of the cell indicated by the cellForWhichToReportCGI;

6>	include the list of additional PLMN Identities, if multiple PLMN identities are broadcast in the concerned cell;

6>	include the list of frequency bands, if multiple frequency bands are broadcast in the concerned cell;

5>	else if the requested cell is not broadcasting SIB1:

6>	include the noSIB1 indication;

Editor’s Note: FFS Capture inter-RAT EUTRAN CGI reporting when ASN.1 for measObjectEUTRA and reportConfig-IRAT is finalized.

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1> if the UE is configured with EN-DC:

2>	if SRB3 is configured:

3> submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>else:

3> submit the MeasurementReport message via the EUTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else:

2>submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity;

1>	if reportType is set to periodical:

2> if a single reporting quantity is set to TRUE in reportQuantityRsIndexes;

3> consider the configured single quantity as the sorting quantity;

2> else:

3> if rsrp is set to TRUE;

4> consider RSRP as the sorting quantity;

3> else:

4> consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRsIndexesToReportSS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting 	quantity and if absThreshSS-BlocksConsolidationis included in the VarMeasConfig for the corresponding measObject, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if includeBeamMeasurements is configured, include the SS/PBCH based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each SS/PBCH blockindex;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and, if  absThreshCSI-RS-Consolidation is included in the VarMeasConfig for the corresponding measObject,the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if includeBeamMeasurementsis configured, include the CSI-RS based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each CSI-RS index.

### 5.5.6	Location measurement indication

#### 5.5.6.1	General

Figure 5.5.5.1-1: Location measurement indication

The purpose of this procedure is to indicate to the network that the UE is going to start/stop location related measurements which require measurement gaps.

NOTE: 	It is a network decision to configure the measurement gap.

#### 5.5.6.2	Initiation

The UE shall:

1>	if and only if upper layers indicate to start performing location measurements and the UE requires measurement gaps for these measurements while measurement gaps are either not configured or not sufficient:

2>	initiate the procedure to indicate start;

NOTE 1:	The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient.

1>	if and only if upper layers indicate to stop performing location measurements:

2>	initiate the procedure to indicate stop;

NOTE 2:	The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

#### 5.5.6.3	Actions related to transmission of LocationMeasurementIndication message

The UE shall set the contents of LocationMeasurementIndication message as follows:

1>	set the measurementIndication as follows:

2>	if the procedure is initiated to indicate start of location related measurements:

3>	set the measurementIndication to the value start;

3>	if the procedure is initiated for RSTD measurements towards E-UTRA:

4>	set the locationMeasurementInfoto the value eutra-RSTDaccording to the information received from upper layers;

Editor’s Note: Initiation of the procedure to start measurements other than RSTD measurements towards E-UTRA is FFS.

2>	else if the procedure is initiated to indicate stop of location related measurements:

3>	set the measurementIndication to the value stop;

1>	submit the LocationMeasurementIndication message to lower layers for transmission, upon which the procedure ends;

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

Editor’s Note: Targeted for completion in Sept 2018

#### 5.6.1.2	Initiation

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.6.1.4	Compilation of band combinations supported by the UE

The UE shall:

1>	if FreqBandList is received:

2>	if the received FreqBandListcontains atleast one of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL for atleast one of the bands:

3>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,  only consisting of bands included in FreqBandList, where for each band in the band combination, the parameters of the band do not exceed the corresponding parameters provided by the IEs maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequested, whichever are recevied.

2>	else:

3>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,only consisting of bands included in FreqBandList, and prioritized in the order of FreqBandList, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

2>	for each band combination included in the candidate list:

3>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of candidates as specified in TS 38.306 [xx]:

4>	remove the band combination from the list of candidates;

2>	include all band combinations in the candidate list into supportedBandCombination;

2>	include the received FreqBandList in the field appliedFreqBandListFilter of the requested UE capability;

1>	else:

2> include all band combinations supported by the UE into supportedBandCombination, excluding fallback band combinations with the same capabilities of another band combination included in the list of band combinations supported by the UE.

#### 5.6.1.5	Void

## 5.7	Other

### 5.7.1	DL information transfer

Editor’s Note: Targeted for completion in Sept 2018.

#### 5.7.1.1	General

NG-RAN

UE

DLInformationTransfer

Figure 5.7.1.1-1: DL information transfer

The purpose of this procedure is to transfer NAS dedicated information from NG-RAN to a UE in RRC_CONNECTED.

#### 5.7.1.2 	Initiation

The nework initiates the DL information transfer procedure whenever there is a need to transfer NAS dedicated information. The network initiates the DL information transfer procedure by sending the DLInformationTransfer message.

#### 5.7.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

1>	if dedicatedNAS-Message is included:

2>	forward dedicatedNAS-Message to upper layers.

### 5.7.2	UL information transfer

Editor’s Note: It is assumed that NAS triggers the Unified Access Control specified in 5.3.x before initiating this procedure. UE performs this procedure if the access attempt is allowed according to 5.3.14.

#### 5.7.2.1	General

NG-RAN

UE

DLInformationTransfer

Figure 5.7.2.1-1: UL information transfer

The purpose of this procedure is to transfer NAS dedicated information from the UE to the network.

#### 5.7.2.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer NAS dedicated information. The UE initiates the UL information transfer procedure by sending the ULInformationTransfer message.

#### Actions related to transmission of ULInformationTransfer message

The UE shall set the contents of the ULInformationTransfer message as follows:

1> if the upper layer provides NAS PDU:

2> set the dedicatedNAS-Messageto include the information received from upper layers

1> submit the ULInformationTransfer message to lower layers for transmission, upon which the procedure ends;

#### 5.7.2.4	Failure to deliver ULInformationTransfer message

The UE shall:

1>	if AS security is not started and radio link failure occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers; or

1>	if mobility (i.e. handover, RRC connection re-establishment) occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers:

2>	inform upper layers about the possible failure to deliver theinformation contained in the concerned ULInformationTransfer messages;

Editor’s Note: Targeted for completion in Sept 2018.

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform EUTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3, SCG integrity check failure and exceeding the maximum uplink transmission timing difference.

Editor’s Note: SCG failure considers the case of exceeding the maximum uplink transmission timing difference if RAN1 decides that EN-DC supports the synchronised operation case. FFS how to capture

Editor’s Note: FFS whether to include the handling of SCell Failure in CA duplication case in SCGfailureinformation procedure and whether to rename SCGfailureinformation.

#### 5.7.3.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

1>	upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.8.3;

1>	upon SCG configuration failure, in accordance with subclause 5.3.5.8.2;

1>	upon integrity check failure indication from SCG lower layers, in accordance with subclause 5.3.5.8.1.

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG-MAC;

1>	stop T304, if running;

1>	if the UE is operating in EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10, 5.6.13a].

Editor’s Note:The section for transmission of SCGFailureInformation in NR RRC entity for SA is FFS_Standalone.

#### 5.7.3.3	Failure type determination

Editor’s Note: FFS / TODO: Either use this section also for NR-DC or change section title (add "for EN-DC").

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationNR message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as scg-ChangeFailure;

Editor’s Note: FFS whether to change scg-ChangeFailure to synchronousReconfigurationFailure-SCG.

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else, if the UE initiates transmission of the SCGFailureInformationNR message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1> else, if the UE initiates transmission of the SCGFailureInformationNR message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure.

Editor’s Note: FFS: whether to include rrc-TransactionIdentifier information.

#### 5.7.3.4	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failureas follows:

1>	for each MeasOjectNR for which a measId is configured and measurement results are available;

2>	include an entry in measResultsPerMOList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which hasrsType set to ssb:

3>	set ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which hasrsType set to csi-rs:

3>	set refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCellto include the available quantities of the concerned cell and in accordance with the performance requirements in [FFS_Ref];

2>	set the measResultNeighCellList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available available and otherwise based on CSI-RS,

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise usingSINR,

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

### 6.1.1	Introduction

The contents of each RRC message is specified in sub-clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in sub-clause 6.3.

### 6.1.2	Need codes and conditions for optional downlink fields

The need for fields to be present in a message or an abstract type, i.e., the ASN.1 fields that are specified as OPTIONAL in the abstract notation (ASN.1), is specified by means of comment text tags attached to the OPTIONAL statement in the abstract syntax. All comment text tags are available for use in the downlink direction only. The meaning of each tag is specified in table 6.1.2-1.

If conditions are used, a conditional presence table is provided for the message or information element specifying the need of the field for each condition case. The table also specifies whether UE maintains or releases the value in case the field is not present. The conditions clarify what the UE may expect regarding the setting of the message by the network. Violation of conditions is regarded as invalid network behaviour, which the UE is not required to cope with. Hence the general error handling defined in 10.4 does not apply in case a field is absent although it is mandatory according to the CondC or CondM condition.

For guidelines on the use of need codes and conditions, see Annex A.6 and A.7.

Table 6.1.2-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

## 6.2	RRC messages

### 6.2.1	General message structure

#### –	NR-RRC-Definitions

This ASN.1 segment is the start of the NR RRC PDU definitions.

-- ASN1START

-- TAG-NR-RRC-DEFINITIONS-START

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-RRC-DEFINITIONS-STOP

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the network to the UE via BCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-BCH-MESSAGE-START

BCCH-BCH-Message ::= SEQUENCE {

message											BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::= CHOICE {

mib												MIB,

messageClassExtension	SEQUENCE {}

}

-- TAG-BCCH-BCH-MESSAGE-STOP

-- ASN1STOP

#### –	BCCH-DL-SCH-Message

The BCCH-DL-SCH-Message class is the set of RRC messages that may be sent from the network to the UE via DL-SCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-DL-SCH-MESSAGE-START

BCCH-DL-SCH-Message ::=			SEQUENCE {

message							BCCH-DL-SCH-MessageType

}

BCCH-DL-SCH-MessageType ::=		CHOICE {

c1								CHOICE {

systemInformation				SystemInformation,

systemInformationBlockType1		SIB1

},

messageClassExtension			SEQUENCE {}

}

-- TAG-BCCH-DL-SCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-CCCH-Message

The DL-CCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the downlink CCCH logical channel.

-- ASN1START

-- TAG-DL-CCCH-MESSAGE-START

DL-CCCH-Message ::= SEQUENCE {

message					DL-CCCH-MessageType

}

DL-CCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcReject								RRCReject,

rrcSetup								RRCSetup,

spare2 NULL,

spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-DL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::= SEQUENCE {

message											DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcReconfiguration							RRCReconfiguration,

rrcResume									RRCResume,

rrcRelease									RRCRelease,

rrcReestablishment							RRCReestablishment,

securityModeCommand							SecurityModeCommand,

dlInformationTransfer						DLInformationTransfer,

ueCapabilityEnquiry							UECapabilityEnquiry,

spare9 NULL, spare8 NULL, spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

#### –	PCCH-Message

The PCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the PCCH logical channel.

-- ASN1START

-- TAG-PCCH-PCH-MESSAGE-START

PCCH-Message ::= SEQUENCE {

message					PCCH-MessageType

}

PCCH-MessageType ::= CHOICE {

c1						CHOICE {

paging									Paging,

spare1	NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-PCCH-PCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH-Message

The UL-CCCH-Message class is the set of RRC messages that may be sent from the UE to the Network on the uplink CCCH logical channel.

-- ASN1START

-- TAG-UL-CCCH-MESSAGE-START

UL-CCCH-Message ::= SEQUENCE {

message					UL-CCCH-MessageType

}

UL-CCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcSetupRequest						RRCSetupRequest,

rrcResumeRequest				RRCResumeRequest,

rrcReestablishmentRequest		RRCReestablishmentRequest,

rrcSystemInfoRequest			RRCSystemInfoRequest

},

messageClassExtension	SEQUENCE {}

}

-- TAG-UL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH1-Message

The UL-CCCH1-Message class is the set of 64bit RRC messages that may be sent from the UE to the Network on the uplink CCCH1 logical channel.

-- ASN1START

-- TAG-UL-CCCH1-MESSAGE-START

UL-CCCH1-Message ::= SEQUENCE {

message               UL-CCCH1-MessageType

}

UL-CCCH1-MessageType ::= CHOICE {

c1                    CHOICE {

rrcResumeRequest1            RRCResumeRequest1,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension SEQUENCE {}

}

-- TAG-UL-CCCH1-MESSAGE-STOP

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::= SEQUENCE {

message											UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

measurementReport							MeasurementReport,

rrcReconfigurationComplete					RRCReconfigurationComplete,

rrcSetupComplete							RRCSetupComplete,

rrcReestablishmentComplete					RRCReestablishmentComplete,

rrcResumeComplete							RRCResumeComplete,

securityModeComplete						SecurityModeComplete,

securityModeFailure							SecurityModeFailure,

ulInformationTransfer						ULInformationTransfer,

locationMeasurementIndication				LocationMeasurementIndication,

ueCapabilityInformation					UECapabilityInformation,

spare6 NULL,

spare5 NULL, spare4 NULL, spare3 NULL,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

### 6.2.2	Message definitions

#### –	DLInformationTransfer

The DLInformationTransfer message is used for the downlink transfer of NAS or non-3GPP dedicated information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet. If SRB2 is suspended, the network does not send this message until SRB2 is resumed.)

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLInformationTransfer message

-- ASN1START

DLInformationTransfer ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

dlInformationTransfer 			DLInformationTransfer-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

DLInformationTransfer-IEs ::=	SEQUENCE {

dedicatedNAS-Message					DedicatedInfoNAS  					OPTIONAL,	-- Need N

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}	OPTIONAL

}

-- ASN1STOP

#### –	LocationMeasurementIndication

The LocationMeasurementIndication message is used to indicate that the UE is going to either start or stop location related measurement which requires measurement gaps.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

LocationMeasurementIndication message

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINDICATION-START

LocationMeasurementIndication ::=	SEQUENCE {

criticalExtensions						CHOICE {

locationMeasurementIndication				LocationMeasurementIndication-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

LocationMeasurementIndication-IEs ::=		SEQUENCE {

measurementIndication		SetupRelease {

LocationMeasurementInfo

},

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-LOCATIONMEASUREMENTINDICATION-STOP

-- ASN1STOP

#### –	MIB

The MIB includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: Network to UE

MIB

-- ASN1START

-- TAG-MIB-START

MIB ::= 							SEQUENCE {

systemFrameNumber					BIT STRING (SIZE (6)),

subCarrierSpacingCommon				ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset				INTEGER (0..15),

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1					PDCCH-ConfigSIB1,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

spare								BIT STRING (SIZE (1))

}

-- TAG-MIB-STOP

-- ASN1STOP

MIB field descriptions

cellBarred Indicates whether the cell allows UEs to camp on this cell, as specified in TS 38.304 [20].

dmrs-TypeA-Position Position of (first) DL DM-RS, see 38.211 [16].

intraFreqReselection Controls cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 38.304 [20}.

pdcch-ConfigSIB1 See TS 38.213 [13]. Determines a common ControlResourceSet (CORESET) a common search space and necessary PDCCH parameters. If the field ssb-SubcarrierOffset indicates that SIB1 is not present, the field pdcch-ConfigSIB1 indicate the frequency positions where the UE may find SS/PBCH block with SIB1 or the frequency range where the network does not provide SS/PBCH block with SIB1 (see TS 38.213 [13], section 13).

ssb-SubcarrierOffset Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See 38.211). The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in 38.213 [13]. This field may indicate that this cell does not provide SIB1 and that there is hence no common CORESET (see TS 38.213 [13], section 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see 38.213 [13], section 13).

subCarrierSpacingCommon Subcarrier spacing for SIB1, Msg.2/4 for initial access and broadcast SI-messages. If the UE acquires this MIB on a carrier frequency <6GHz, the value scs15or60 corresponds to 15 Khz and the value scs30or120 corresponds to 30 kHz. If the UE acquires this MIB on a carrier frequency >6GHz, the value scs15or60 corresponds to 60 Khz and the value scs30or120 corresponds to 120 kHz.

systemFrameNumber The 6 most significant bit (MSB) of the 10 bit System Frame Number. The 4 LSB of the SFN are conveyed in the PBCH transport block as part of channel coding (i.e. outside the MIB encoding).

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReport message

-- ASN1START

-- TAG-MEASUREMENTREPORT-START

MeasurementReport ::=				SEQUENCE {

criticalExtensions					CHOICE {

measurementReport					MeasurementReport-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

MeasurementReport-IEs ::=			SEQUENCE {

measResults							MeasResults,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-MEASUREMENTREPORT-STOP

-- ASN1STOP

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA (connected to EPC or 5GC).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

MobilityFromNRCommand ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

mobilityFromNRCommand				MobilityFromNRCommand-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=	SEQUENCE {

targetRAT-Type						ENUMERATED {

eutra, spare2, spare1, ...},

targetRAT-MessageContainer			OCTET STRING,

lateNonCriticalExtension				OCTET STRING								OPTIONAL,

nonCriticalExtension					SEQUENCE {}									OPTIONAL

}

-- ASN1STOP

MobilityFromNRCommand field descriptions

targetRAT-Type Indicates the target RAT type.

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. NOTE 1. A complete message is included, as specified in the other standard.

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | 3GPP TS 36.331 (clause 5.4.2) | RRCCONNECTIONreconfiguration |

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::= 							SEQUENCE {

pagingRecordList						PagingRecordList														OPTIONAL, -- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

PagingRecordList ::=				SEQUENCE(SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=					SEQUENCE{

ue-Identity							PagingUE-Identity,

accessType							ENUMERATED {non3GPP}	OPTIONAL,	-- Need N

...

}

PagingUE-Identity ::=				CHOICE{

ng-5G-S-TMSI						NG-5G-S-TMSI,

i-RNTI								I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

Paging field descriptions

accessType It indicates whether Paging is originated due to the PDU sessions from the non-3GPP access.

#### –	RRCReestablishment

The RRCReestablishment message is used to re-establish SRB1.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReestablishment message

-- ASN1START

-- TAG-RRCREESTABLISHMENT-START

RRCReestablishment ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcReestablishment		RRCReestablishment-IEs,

spare3 NULL, spare2 NULL, spare1	NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReestablishment-IEs ::= SEQUENCE {

nextHopChainingCount				NextHopChainingCount,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- TAG-RRCREESTABLISHMENT-STOP

-- ASN1STOP

#### –	RRCReestablishmentComplete

The RRCReestablishmentComplete message is used to confirm the successful completion of an RRC connection re-establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReestablishmentComplete message

-- ASN1START

-- TAG-RRCREESTABLISHMENTCOMPLETE-START

RRCReestablishmentComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReestablishmentComplete

RRCReestablishmentComplete-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReestablishmentComplete-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- TAG-RRCREESTABLISHMENTCOMPLETE-STOP

-- ASN1STOP

#### –	RRCReestablishmentRequest

The RRCReestablishmentRequest message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCReestablishmentRequest message

-- ASN1START

-- TAG-RRCREESTABLISHMENTREQUEST-START

RRCReestablishmentRequest ::= SEQUENCE {

rrcReestablishmentRequest

RRCReestablishmentRequest-IEs

}

RRCReestablishmentRequest-IEs ::= SEQUENCE {

ue-Identity							ReestabUE-Identity,

reestablishmentCause				ReestablishmentCause,

spare								BIT STRING (SIZE (1))

}

ReestabUE-Identity ::=				SEQUENCE {

c-RNTI								RNTI-Value,

physCellId							PhysCellId,

shortMAC-I							ShortMAC-I

}

ReestablishmentCause ::=			ENUMERATED {

reconfigurationFailure, handoverFailure,

otherFailure, spare1}

-- TAG-RRCREESTABLISHMENTREQUEST-STOP

-- ASN1STOP

RRCReestablishmentRequest field descriptions

physCellId The Physical Cell Identity of the PCell the UE was connected to prior to the failure.

reestablishmentCause Indicates the failure cause that triggered the re-establishment procedure. gNB is not expected to reject a RRCReestablishmentRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to retrieve UE context and to facilitate contention resolution by lower layers.

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) including and security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					RRCReconfiguration-vxx-IEs																	OPTIONAL

}

RRCReconfiguration-vxx-IEs ::= 			SEQUENCE {

masterCellGroup							OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

fullConfig								ENUMERATED {true} 														OPTIONAL, -- Need N

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedInfoNAS							OPTIONAL, -- Cond nonHO

keyRefresh								KeyRefresh																OPTIONAL, -- 	Cond MasterKeyChange

dedicatedSIB1-Delivery				OCTET STRING (CONTAINING SIB1)										OPTIONAL,

dedicatedSystemInformationDelivery	OCTET STRING (CONTAINING SystemInformation)							OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

KeyRefresh ::=					SEQUENCE {

keySetChangeIndicator			BOOLEAN															OPTIONAL,	-- Cond MasterKeyChange

nextHopChainingCount			NextHopChainingCount											OPTIONAL,	-- Cond MasterKeyChangeNCC

n2ModeNAS-Container				OCTET STRING													OPTIONAL,	-- Cond InterSystemHO

...

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message.

keySetChangeIndicator True is used in an intra-cell handover when a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. False is used in an intra-NR handover when the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

nas-securityParamToNGRAN This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS- security after inter-system handover to NR. The content is defined in TS 24.501.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

secondaryCellGroup Configuration of secondary cell group (EN-DC).

| Conditional presence | Explanation |
| --- | --- |
| nonHO | The field is not present in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| InterSystemHO | This field is mandatory present in case of inter system handover. Otherwise the field is absent. |
| MasterKeyChange | If ReconfigurationWithSync is included, this field is optionally present, need N, otherwise the field is absent. The field is not included during inter-system handover. |
| MasterKeyChangeNCC | If ReconfigurationWithSync is included, this field is optionally present, need M, otherwise the field is absent, . The field is not included during inter-system handover. |

Editor’s Note: FFS: the details of the conditional presence of fullConfig.

#### –	RRCReconfigurationComplete

The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::= 			SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

rrcReconfigurationComplete				RRCReconfigurationComplete-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

RRCReconfigurationComplete-IEs ::= SEQUENCE {

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					RRCReconfigurationComplete-v15xy-IEs																OPTIONAL

}

RRCReconfigurationComplete-v15xy-IEs ::= SEQUENCE {

uplinkTxDirectCurrentList				UplinkTxDirectCurrentList												OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

RRCReconfigurationComplete-v15xy-IEs field descriptions

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs.

#### –	RRCReject

The RRCReject message is used to reject an RRC connection establishment or an RRC connection resumption.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCReject message

-- ASN1START

-- TAG-RRCREJECT-START

RRCReject ::=				SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

rrcReject				RRCReject-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReject-IEs ::=		SEQUENCE {

waitTime							RejectWaitTime															OPTIONAL,	-- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

RejectWaitTime ::=						INTEGER (1..16)

-- TAG-RRCREJECT-STOP

-- ASN1STOP

RRCReject field descriptions

waitTime Wait time value in seconds.

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcRelease					RRCRelease-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCRelease-IEs ::=		SEQUENCE {

redirectedCarrierInfo				RedirectedCarrierInfo				OPTIONAL,	-- Need N

cellReselectionPriorities			CellReselectionPriorities							OPTIONAL,	-- Need M

suspendConfig						SuspendConfig						OPTIONAL,	-- Need R

deprioritisationReq				SEQUENCE {

deprioritisationType				ENUMERATED {frequency, nr},

deprioritisationTimer				ENUMERATED {min5, min10, min15, min30}

}																												OPTIONAL,	-- Need N

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

RedirectedCarrierInfo ::=			CHOICE {

nr									ARFCN-ValueNR,

eutra								ARFCN-ValueEUTRA,

...

}

SuspendConfig ::= SEQUENCE {

fullI-RNTI							I-RNTI-Value,

shortI-RNTI-Value					ShortI-RNTI-Value,

ran-PagingCycle						ENUMERATED{ rf32, rf64, rf128, rf256},

ran-NotificationAreaInfo			RAN-NotificationAreaInfo 	OPTIONAL,	-- Need M,

periodic-RNAU-Timer					ENUMERATED {min5, min10, min30, min60, 																min120, min360, min720, infinity},

nextHopChainingCount				NextHopChainingCount

}

CellReselectionPriorities ::=		SEQUENCE {

freqPriorityListEUTRA				FreqPriorityListEUTRA			OPTIONAL,		-- Need M

freqPriorityListNR					FreqPriorityListNR				OPTIONAL, 		-- Need M

t320								ENUMERATED {

min5, min10, min20, min30, min60, min120, min180,

spare1}						OPTIONAL,		-- Need R

...

}

-- FFS Maximum number of frequency in priority list

FreqPriorityListEUTRA ::=			SEQUENCE (SIZE (1..ffsValue)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=				SEQUENCE (SIZE (1..ffsValue)) OF FreqPriorityNR

FreqPriorityEUTRA ::=				SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityNR ::=				SEQUENCE {

carrierFreq							ARFCN-ValueNR,

cellReselectionPriority				CellReselectionPriority

}

RAN-NotificationAreaInfo ::=  CHOICE {

cellList				PLMN-RAN-AreaCellList,

ran-AreaConfigList		PLMN-RAN-AreaConfigList

}

PLMN-RAN-AreaCellList ::=			SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

-- Sum of cells from all PLMNs does not exceed 32

PLMN-RAN-AreaCell ::=	SEQUENCE {

plmn-Identity				PLMN-Identity													OPTIONAL,	-- Need S

ran-AreaCells				SEQUENCE (SIZE (1..32)) OF	CellIdentity

}

PLMN-RAN-AreaConfigList	::=			SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=	SEQUENCE {

plmn-Identity				PLMN-Identity													OPTIONAL,	-- Need S

ran-Area					SEQUENCE (SIZE (1..16)) OF	RAN-AreaConfig

}

RAN-AreaConfig ::=	SEQUENCE {

trackingAreaCode			TrackingAreaCode,

--Sum of RAN-AreaCodes all PLMNs does not exceed 32

ran-AreaCodeList			SEQUENCE (SIZE (1..32)) OF	RAN-AreaCode		OPTIONAL

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

Editor’s Note: FFS Whether fulll and/or short I-RNTI(s) are defined as optional fields in SuspendConfig.

Editor’s Note: FFS Signalling optimizations and/or default configuration for RNA Area and/or corrections (TAI vs. TAC).

Editor’s Note: FFS Whether RejectWaitTimer is needed in RRCRelease message.

RRCRelease field descriptions

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised. The UE shall be able to store a depriotisation request for up to X frequencies (applicable when receiving another frequency specific deprioritisation request before T325 expiry).

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

suspendConfig Indicates configuration for the RRC_INACTIVE state.

RAN-NotificationAreaInfo field descriptions

cellList A list of cells configured as RAN area.

ran-AreaConfigList A list of RAN area codes or RA code(s) as RAN area.

PLMN-RAN-AreaConfig field descriptions

plmn-Identity PLMN Identity to which the cells in ran-AreaCells belong. If the field is absent the UE uses the ID of the registered PLMN.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or RAN area code(s) to configure a UE.

PLMN-RAN-AreaCell field descriptions

plmn-Identity PLMN Identity to which the cells in ran-AreaCells belong. If the field is absent the UE uses the ID of the registered PLMN.

Editor’s Note: FFS Confirm the number X of deprioritisation frequencies the UE shall be able to store.

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=						SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcResume							RRCResume-IEs,

spare3 								NULL,

spare2 								NULL,

spare1 								NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCResume-IEs ::=		SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

-- Configuration of master cell group (NR Standalone):

masterCellGroup							OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

-- Configuration of master cell group:

measConfig								MeasConfig																OPTIONAL, -- Need M

drb-ContinueROHC						ENUMERATED {true}																OPTIONAL, -- Need M

fullConfig								ENUMERATED {true} 														OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

Editor’s Note: FFS Whether secondary group can be resumed.

#### –	RRCResumeRequest

The RRCResumeRequest message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCResumeRequest message

-- ASN1START

-- TAG-RRCRESUMEREQUEST-START

RRCResumeRequest ::=	SEQUENCE {

rrcResumeRequest			RRCResumeRequest-IEs

}

RRCResumeRequest-IEs ::=		SEQUENCE {

shortResumeIdentity							ShortI-RNTI-Value,

resumeMAC-I									BIT STRING (SIZE (16)),

resumeCause									ResumeCause,

spare										BIT STRING (SIZE (1))

}

-- FFS Which additional resume causes are supported: delayTolerantAccess, MO video, MO SMS, etc.

ResumeCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, rna-Update, spare1, spare2, spare3, spare4,

spare5, spare6, spare7, spare8, spare9 }

-- TAG-RRCRESUMEREQUEST-STOP

-- ASN1STOP

RRCResumeRequest field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. Network is not expected to reject a RRCResumeRequest due to unknown cause value being used by the UE.

shortResumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the security configuration as specified in 5.3.13.3.

#### –	RRCResumeRequest1

The RRCResumeRequest1 is the 64 bit message used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH1

Direction: UE to Network

RRCResumeRequest1 message

-- ASN1START

-- TAG-RRCRESUMEREQUEST1-START

RRCResumeRequest1 ::= SEQUENCE {

rrcResumeRequest1      RRCResumeRequest1-IEs

}

RRCResumeRequest1-IEs ::=    SEQUENCE {

resumeIdentity                      I-RNTI-Value,  --40bits

resumeMAC-I                         BIT STRING (SIZE (16)),

resumeCause                         ResumeCause,

spare                               BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST1-STOP

-- ASN1STOP

RRCResumeRequest1 field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. gNB is not expected to reject a RRCResumeRequest due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Security token used to identify and verify the UE at RRC Resume Request. The 16 least significant bits of the MAC-I calculated using the security configuration as specified in 5.3.13.3.

#### –	RRCResumeComplete

The RRCResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::= SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions							CHOICE {

rrcResumeComplete				RRCResumeComplete-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

RRCResumeComplete-IEs ::= SEQUENCE {

dedicatedInfoNAS					DedicatedInfoNAS						OPTIONAL,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

Editor’s Note: FFS Need for selectedPLMN-Identity in RRCResumeComplete.

Editor’s Note: FFS Whether the NSSAI info needs to be included in MSG5 in the case of resume.

#### –	RRCSetup

The RRCSetup message is used to establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCSetup message

-- ASN1START

-- TAG-RRCSETUP-START

RRCSetup ::=						SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcSetup				RRCSetup-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCSetup-IEs ::=		SEQUENCE {

radioBearerConfig					RadioBearerConfig,

masterCellGroup						OCTET STRING (CONTAINING CellGroupConfig),

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-RRCSETUP-STOP

-- ASN1STOP

Editor’s Note: FFS Which IEs in radioBearerConfig and masterCellGroup are applicable for RRCSetup message.

#### –	RRCSetupComplete

The RRCSetupComplete message is used to confirm the successful completion of an RRC connection establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCSetupComplete message

-- ASN1START

-- TAG-RRCSETUPCOMPLETE-START

RRCSetupComplete ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcSetupComplete			RRCSetupComplete-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCSetupComplete-IEs ::= SEQUENCE {

selectedPLMN-Identity				INTEGER (1..maxPLMN),

registeredAMF						RegisteredAMF									OPTIONAL,

guami-Type							ENUMERATED {native, mapped}						OPTIONAL,

s-nssai-list						SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI	OPTIONAL,

dedicatedInfoNAS					DedicatedInfoNAS,

ng-5G-S-TMSI-Value					CHOICE {

ng-5g-s-tmsi						NG-5G-S-TMSI								,

ng-5g-s-tmsi-part2					BIT STRING (SIZE (9))

}																					OPTIONAL,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-RRCSETUPCOMPLETE-STOP

-- ASN1STOP

Editor’s Note: FFS Field description of 5GC identifiers and other other information.

RRCSetupComplete field descriptions

ng-5g-s-tmsi-part2 The leftmost 9 bits of 5G-S-TMSI.

#### –	RRCSetupRequest

The RRCSetupRequest message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=			SEQUENCE {

rrcSetupRequest							RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause

}

InitialUE-Identity ::=				CHOICE {

ng-5g-s-tmsi-part1					BIT STRING (SIZE (39)),

randomValue							BIT STRING (SIZE (39)),

spare								BIT STRING (SIZE (1))

}

-- FFS Which additional cause values are supported: delayTolerantAccess, MO videop, MO SMS, etc.

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall,spare10, spare9, spare8,spare7,

spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

RRCSetupRequest field descriptions

establishmentCause Provides the establishment cause for the RRC request in accordance with the information received from upper layers. gNB is not expected to reject a RRCSetupRequest due to unknown cause value being used by the UE.

randomValue Integer value in the range 0 to 239 1.

ue-Identity UE identity included to facilitate contention resolution by lower layers.

ng-5g-s-tmsi-part1 The rightmost 39 bits of 5G-S-TMSI.

#### –	RRCSystemInfoRequest

The RRCSystemInfoRequest message is used to request the other SI.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to NR

RRCSystemInfoRequest message

-- ASN1START

-- TAG-RRCSYETEMINFOREQUEST-START

RRCSystemInfoRequest ::= SEQUENCE {

criticalExtensions					CHOICE {

rrcSystemInfoRequest-r15			RRCSystemInfoRequest-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCSystemInfoRequest-r15-IEs ::= SEQUENCE {

requested-SIList					BIT STRING (SIZE (maxSI-Message)),	--32bits

spare 								BIT STRING (SIZE (12))

}

-- TAG-RRCSYETEMINFOREQUEST-STOP

-- ASN1STOP

RRCSystemInfoRequest field descriptions

Requested-SI-List Contains a list of requested SI messages. First but corresponds to SI message 0, second to SI message 1 and so on.

#### –	SecurityModeCommand

The SecurityModeCommand message is used to command the activation of AS security.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

SecurityModeCommand message

-- ASN1START

-- TAG-SECURITYMODECOMMAND-START

SecurityModeCommand ::=				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

securityModeCommand				SecurityModeCommand-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SecurityModeCommand-IEs ::=		SEQUENCE {

securityConfigSMC					SecurityConfigSMC,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

SecurityConfigSMC ::=					SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig,

...

}

-- TAG-SECURITYMODECOMMAND-STOP

-- ASN1STOP

#### –	SecurityModeComplete

The SecurityModeComplete message is used to confirm the successful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeComplete message

-- ASN1START

-- TAG-SECURITYMODECOMPLETE-START

SecurityModeComplete ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

securityModeComplete					SecurityModeComplete-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

SecurityModeComplete-IEs ::=		SEQUENCE {

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-SECURITYMODECOMPLETE-STOP

-- ASN1STOP

#### –	SecurityModeFailure

The SecurityModeFailure message is used to indicate an unsuccessful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeFailure message

-- ASN1START

-- TAG-SECURITYMODEFAILURE-START

SecurityModeFailure ::=				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

securityModeFailure				SecurityModeFailure-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

SecurityModeFailure-IEs ::= SEQUENCE {

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-SECURITYMODEFAILURE-STOP

-- ASN1STOP

#### –	SIB1

Editor’s Note: Targeted for completion in September 2018. Not used in EN-DC.

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=		SEQUENCE {

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)		OPTIONAL, 		-- Need R

q-RxLevMinSUL						Q-RxLevMin			OPTIONAL, 		-- Need R

q-QualMin							Q-QualMin			OPTIONAL,		-- Need R

q-QualMinOffset					INTEGER (1..8)		OPTIONAL		-- Need R

}																				OPTIONAL,			-- Need S

cellAccessRelatedInfo				CellAccessRelatedInfo,

connEstFailureControl 	ConnEstFailureControl											OPTIONAL,			-- Need R

si-SchedulingInfo					SI-SchedulingInfo							OPTIONAL,			-- Need R

servingCellConfigCommon				ServingCellConfigCommonSIB					OPTIONAL,			-- Need R

ims-EmergencySupport					ENUMERATED {true}							OPTIONAL,			-- Need R

eCallOverIMS-Support					ENUMERATED {true}							OPTIONAL, 			-- Cond Absent

ue-TimersAndConstants				UE-TimersAndConstants						OPTIONAL,			-- Need R

uac-BarringInfo 						SEQUENCE {

uac-BarringForCommon					UAC-BarringPerCatList			OPTIONAL,			-- Need S

uac-BarringPerPLMN-List				UAC-BarringPerPLMN-List			OPTIONAL,		-- Need S

uac-BarringInfoSetList				UAC-BarringInfoSetList

}									OPTIONAL,														-- Need R

useFullResumeID							ENUMERATED {true}				OPTIONAL,	-- Need N

lateNonCriticalExtension			OCTET STRING															OPTIONAL,

nonCriticalExtension				SEQUENCE{}																OPTIONAL

}

UAC-BarringPerPLMN-List ::= 		SEQUENCE (SIZE (1.. maxPLMN)) OF UAC-BarringPerPLMN

UAC-BarringPerPLMN ::=			SEQUENCE {

plmn-IdentityIndex					INTEGER (1..maxPLMN),

uac-ACBarringListType		CHOICE{

uac-ImplicitACBarringList		SEQUENCE (SIZE(maxAccessCat-1)) OF UAC-BarringInfoSetIndex,

uac-ExplicitACBarringList				UAC-BarringPerCatList

}

}

UAC-BarringPerCatList ::= SEQUENCE (SIZE (1..maxAccessCat-1)) OF UAC-BarringPerCat

UAC-BarringPerCat ::= SEQUENCE {

accessCategory             		INTEGER (1..maxAccessCat-1),

uac-barringInfoSetIndex  	    		UAC-BarringInfoSetIndex

}

UAC-BarringInfoSetIndex  ::=				INTEGER (1..maxBarringInfoSet)

UAC-BarringInfoSetList ::= SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet

UAC-BarringInfoSet ::= SEQUENCE {

uac-BarringFactor			ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime				ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity			BIT STRING (SIZE(7))

}

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

accessCategory The Access Category according to [TS 22.261]

q-QualMin Parameter “Qqualmin” in TS 38.304 [4], applicable for serving cell. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter “Qrxlevmin” in TS 38.304 [4], applicable for serving cell.

q-RxLevMinSUL Parameter “QrxlevminSUL” in TS 38.304 [4], applicable for serving cell

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs, unless overwritten by the PLMN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in section 5.3.14.2.

uac-BarringPerPLMN-List Access control parameters for each access category valid only for a specific PLMN.

uac-barringInfoSetIndex Index of the entry in field uac-BarringInfoSetList. Value 1 corresponds to the first entry in uac-BarringInfoSetList, value 2 corresponds to the second entry in this list and so on.

uac-BarringInfoSetList List of access control parameter sets. Each access category can be configured with access parameters corresponding to a particular set.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, and so on. Value 0 means that access attempt is allowed for the corresponding access identity.

useFullResumeID This field indicates if the UE indicates full resume ID of 40 bits in RRCResumeRequest.

UAC-BarringPerCat field descriptions

accessCategory The Access Category according to [TS 22.261]

UAC-BarringInfoSet field descriptions

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringTime The minimum time before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category.

| Conditional presence | Explanation |
| --- | --- |
| Absent | The field is not used in this version of the specification, if received the UE shall ignore. |

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction:- Network to UE

SystemInformation message

-- ASN1START

SystemInformation ::=				SEQUENCE {

criticalExtensions					CHOICE {

systemInformation-r15				SystemInformation-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

SystemInformation-IEs ::=		SEQUENCE {

sib-TypeAndInfo						SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2								SIB2,

sib3								SIB3,

sib4								SIB4,

sib5								SIB5,

sib6								SIB6,

sib7								SIB7,

sib8								SIB8,

sib9								SIB9,

...

},

nonCriticalExtension				SystemInformation-v15x0-IEs			OPTIONAL

}

SystemInformation-v15x0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

#### –	UECapabilityEnquiry

The UECapabilityEnquiry message is used to request UE radio access capabilities for NR as well as for other RATs.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UECapabilityEnquiry information element

-- ASN1START

-- TAG-UECAPABILITYENQUIRY-START

UECapabilityEnquiry ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueCapabilityEnquiry					UECapabilityEnquiry-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UECapabilityEnquiry-IEs ::=		SEQUENCE {

ue-CapabilityRAT-RequestList		UE-CapabilityRAT-RequestList,

lateNonCriticalExtension			OCTET STRING															OPTIONAL,

nonCriticalExtension				SEQUENCE{}																OPTIONAL

}

-- TAG-UECAPABILITYENQUIRY-STOP

-- ASN1STOP

#### –	UECapabilityInformation

The IE UECapabilityInformation message is used to transfer UE radio access capabilities requested by the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UECapabilityInformation information element

-- ASN1START

-- TAG-UECAPABILITYINFORMATION-START

UECapabilityInformation ::=			SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

ueCapabilityInformation					UECapabilityInformation-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

UECapabilityInformation-IEs ::=		SEQUENCE {

ue-CapabilityRAT-ContainerList			UE-CapabilityRAT-ContainerList											OPTIONAL,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-UECAPABILITYINFORMATION-STOP

#### -- ASN1STOP

#### –	ULInformationTransfer

The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

ULInformationTransfer message

-- ASN1START

ULInformationTransfer ::=			SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulInformationTransfer			ULInformationTransfer-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULInformationTransfer-IEs ::=	SEQUENCE {

dedicatedInfoNAS					DedicatedInfoNAS					OPTIONAL,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

## 6.3	RRC information elements

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUP-RELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release			NULL,

setup			ElementTypeParam

}

-- TAG-SETUP-RELEASE-STOP

-- ASN1STOP

### 6.3.1	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=		SEQUENCE {

cellReselectionInfoCommon			SEQUENCE {

q-Hyst								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars			SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

q-HystSF						SEQUENCE {

sf-Medium						ENUMERATED {

dB-6, dB-4, dB-2, dB0},

sf-High							ENUMERATED {

dB-6, dB-4, dB-2, dB0}

}

}																OPTIONAL		-- Need R

},

cellReselectionServingFreqInfo		SEQUENCE {

s-NonIntraSearchP						ReselectionThreshold		OPTIONAL,		-- Need R

s-NonIntraSearchQ						ReselectionThresholdQ		OPTIONAL,		-- Need R

threshServingLowP						ReselectionThreshold,

threshServingLowQ						ReselectionThresholdQ		OPTIONAL, 		-- Need R

cellReselectionPriority					CellReselectionPriority,

nrofSS-BlocksToAverage					INTEGER (2..maxNrofSS-BlocksToAverage)	OPTIONAL,		-- Need R

absThreshSS-BlocksConsolidation			ThresholdNR									OPTIONAL, 	-- Need R

rangeToBestCell						RangeToBestCell							OPTIONAL	-- Need R

},

intraFreqCellReselectionInfo		SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinSUL						Q-RxLevMin					OPTIONAL, 		-- Need R

q-QualMin							Q-QualMin					OPTIONAL,		-- Need S

s-IntraSearchP						ReselectionThreshold,

s-IntraSearchQ						ReselectionThresholdQ		OPTIONAL,	-- Cond RSRQ

t-ReselectionNR						T-Reselection,

p-Max								P-Max						OPTIONAL,		-- Need R

smtc								SSB-MTC						OPTIONAL,		-- Need R

ss-RSSI-Measurement				SS-RSSI-Measurement		OPTIONAL,		-- Need R

ssb-ToMeasure						SSB-ToMeasure				OPTIONAL,		-- Need R

deriveSSB-IndexFromCell			BOOLEAN

},

useFullResumeID							ENUMERATED {true}				OPTIONAL,	-- Need N

...

}

RangeToBestCell	::= Q-OffsetRange

IntraFreqBlackCellList ::=		SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

intraFreqcellReselectionInfo Cell re-selection information common for intra-frequency cells.

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

rangeToBestCell Parameter “rangeToBestCell” in TS 38.304 [4].

p-Max Value applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to the UE capability.

q-Hyst Parameter Qhyst in TS 38.304 [4], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-QualMin Parameter “Qqualmin” in TS 38.304 [4], applicable for intra-frequency neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter “Qrxlevmin” in TS 38.304 [4], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter “QrxlevminSUL” in TS 38.304 [4], applicable for intra-frequency neighbour cells.

s-IntraSearchP Parameter “SIntraSearchP” in TS 38.304 [4]. If this field is not present, the UE applies the (default) value of infinity for SIntraSearchP.

s-IntraSearchQ Parameter “SIntraSearchQ” in TS 38.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter “SnonIntraSearchP” in TS 38.304 [4]. If this field is not present, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter “SnonIntraSearchQ” in TS 38.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

threshServingLowP Parameter “ThreshServing, LowP” in TS 38.304 [4].

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see 38.215). When the field is absent the UE measures on all SS-blocks.

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequnecy cells.

threshServingLowQ Parameter “ThreshServing, LowQ” in TS 38.304 [4].

t-ReselectionNR Parameter “TreselectionNR” in TS 38.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| RSRQ | The field is optionally present, Need R, if threshServingLowQ is present in SIB2; otherwise it is not present. |

#### –	SIB3

SIB3 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells.

SIB3 information element

-- ASN1START

-- TAG-SIB3-START

SIB3 ::=		SEQUENCE {

intraFreqNeighCellList				IntraFreqNeighCellList		OPTIONAL,	-- Need R

intraFreqBlackCellList				IntraFreqBlackCellList		OPTIONAL,	-- Need R

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

...

lateNonCriticalExtension				OCTET STRING				OPTIONAL

}

IntraFreqNeighCellList ::=		SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellInfo ::=		SEQUENCE {

physCellId							PhysCellId,

q-OffsetCell						Q-OffsetRange,

...

}

IntraFreqBlackCellList ::=		SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

-- TAG-SIB3-STOP

-- ASN1STOP

SIB3 field descriptions

intraFreqBlackCellList List of blacklisted intra-frequency neighbouring cells.

intraFreqNeighCellList List of intra-frequency neighbouring cells with specific cell re-selection parameters.

q-OffsetCell Parameter “Qoffsets,n” in TS 38.304 [4].

#### –	SIB4

SIB4 contains information relevant only for inter-frequency cell re-selection i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=		SEQUENCE {

interFreqCarrierFreqList			InterFreqCarrierFreqList,

lateNonCriticalExtension			OCTET STRING		OPTIONAL,

...

lateNonCriticalExtension			OCTET STRING		OPTIONAL

}

InterFreqCarrierFreqList ::=             SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqInfo ::=	 SEQUENCE {

dl-CarrierFreq					ARFCN-ValueNR,

multiFrequencyBandListNR 		MultiFrequencyBandListNR,

nrofSS-BlocksToAverage			INTEGER (2..maxNrofSS-BlocksToAverage)		OPTIONAL,

absThreshSS-BlocksConsolidation	ThresholdNR									OPTIONAL,

smtc								SSB-MTC										OPTIONAL,	-- Need R

ssbSubcarrierSpacing				SubcarrierSpacing,

ssb-ToMeasure					SSB-ToMeasure 								OPTIONAL,	-- Need R

deriveSSB-IndexFromCell			BOOLEAN,

ss-RSSI-Measurement				SS-RSSI-Measurement

OPTIONAL,

q-RxLevMin							Q-RxLevMin,

q-RxLevMinSUL						Q-RxLevMin			OPTIONAL, 		-- Need R

q-QualMin							Q-QualMin			OPTIONAL,		-- Need R,

p-Max								P-Max	 						OPTIONAL,		-- Need R

t-ReselectionNR					T-Reselection,

t-ReselectionNR-SF					SpeedStateScaleFactors OPTIONAL,		-- Need N

threshX-HighP						ReselectionThreshold,

threshX-LowP						ReselectionThreshold,

threshX-Q						SEQUENCE {

threshX-HighQ						ReselectionThresholdQ,

threshX-LowQ						ReselectionThresholdQ

}																	OPTIONAL, -- Cond RSRQ

cellReselectionPriority			CellReselectionPriority		OPTIONAL,		-- Need R

q-OffsetFreq						Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList			InterFreqNeighCellList		OPTIONAL, 		-- Need R

interFreqBlackCellList			InterFreqBlackCellList		OPTIONAL, 		-- Need R

...

}

InterFreqNeighCellList ::=			SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellInfo ::=			SEQUENCE {

physCellId								PhysCellId,

q-OffsetCell							Q-OffsetRange,

...

}

InterFreqBlackCellList ::=			SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

-- TAG-SIB4-STOP

-- ASN1STOP

SIB4 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency.

interFreqBlackCellList List of blacklisted inter-frequency neighbouring cells.

interFreqCarrierFreqList List of neighbouring carrier frequencies and frequency specific cell re-selection information.

interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

p-Max Value applicable for the neighbouring NR cells on this carrier frequency. [FFS = Ref]

q-OffsetCell Parameter “Qoffsets,n” in TS 38.304 [4].

q-OffsetFreq Parameter “Qoffsetfrequency” in TS 38.304 [4].

q-QualMin Parameter “Qqualmin” in TS 38.304 [4].

smtc Measurement timing configuration for inter-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms in this frequency.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see 38.215). When the field is absent the UE measures on all SS-blocks.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 or 30 (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable.

threshX-HighP Parameter “ThreshX, HighP” in TS 38.304 [4].

threshX-HighQ Parameter “ThreshX, HighQ” in TS 38.304 [4].

threshX-LowP Parameter “ThreshX, LowP” in TS 38.304 [4].

threshX-LowQ Parameter “ThreshX, LowQ” in TS 38.304 [4].

t-ReselectionNR Parameter “TreselectionNR” in TS 38.304 [4].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [4]. If the field is not present, the UE behaviour is specified in TS 38.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if threshServingLowQ is present in SIB2; otherwise it is not present. |

#### –	SIB5

SIB5 contains information relevant only for inter-RAT cell re-selection i.e. information about E-UTRA frequencies and E-UTRAs neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

SIB5 information element

-- ASN1START

-- TAG-SIB5-START

SIB5 ::=		SEQUENCE {

carrierFreqListEUTRA			CarrierFreqListEUTRA			OPTIONAL, 		-- Need R

t-ReselectionEUTRA				T-Reselection,

t-ReselectionEUTRA-SF				SpeedStateScaleFactors			OPTIONAL,		-- Need N

lateNonCriticalExtension		OCTET STRING				OPTIONAL,

...

lateNonCriticalExtension		OCTET STRING				OPTIONAL

}

CarrierFreqListEUTRA ::=		SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA

CarrierFreqEUTRA ::=				SEQUENCE {

carrierFreq						ARFCN-ValueEUTRA,

eutra-multiBandInfoList			EUTRA-MultiBandInfoList			OPTIONAL,	-- Need R

eutra-FreqNeighCellList			EUTRA-FreqNeighCellList			OPTIONAL, 		-- Need R

eutra-BlackCellList				EUTRA-FreqBlackCellList			OPTIONAL, -- Need R

allowedMeasBandwidth				EUTRA-AllowedMeasBandwidth,

presenceAntennaPort1				EUTRA-PresenceAntennaPort1,

cellReselectionPriority			CellReselectionPriority			OPTIONAL, 		-- Need R

threshX-High						ReselectionThreshold,

threshX-Low						ReselectionThreshold,

q-RxLevMin							INTEGER (-70..-22),

q-QualMin							INTEGER (-34..-3),

p-MaxEUTRA							INTEGER (-30..33),

threshX-Q							SEQUENCE {

threshX-HighQ				ReselectionThresholdQ,

threshX-LowQ				ReselectionThresholdQ

}																OPTIONAL		-- Cond RSRQ

}

EUTRA-FreqBlackCellList ::=			SEQUENCE (SIZE (1..maxEUTRA-CellBlack)) OF EUTRA-PhysCellIdRange

EUTRA-FreqNeighCellList ::=			SEQUENCE (SIZE (1..maxCellEUTRA)) OF EUTRA-FreqNeighCellInfo

EUTRA-FreqNeighCellInfo ::=			SEQUENCE {

physCellId								EUTRA-PhysCellId,

q-OffsetCell							EUTRA-Q-OffsetRange

}

-- TAG-SIB5-STOP

-- ASN1STOP

SIB5 field descriptions

carrierFreqListEUTRA List of carrier frequencies of EUTRA.

eutra-BlackCellList List of blacklisted EUTRA neighbouring cells.

eutra-multiBandInfoList Indicates the list of frequency bands in addition to the band represented by carrierFreq for which cell reselection parameters are common, and a list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [xx, table 6.2.4-1], for the frequency bands in eutra-multiBandInfoList

p-MaxEUTRA The maximum allowed transmission power on the (uplink) carrier frequency, see TS 36.304 [21]. In dBm

q-QualMin Parameter “Qqualmin” in TS 36.304 [21]. Actual value Qqualmin = field value [dB].

q-RxLevMin Parameter “Qrxlevmin” in TS 36.304 [21]. Actual value Qrxlevmin = field value * 2 [dBm].

t-ReselectionEUTRA Parameter “TreselectionEUTRA” in TS 38.304 [20].

threshX-High Parameter “ThreshX, HighP” in TS 38.304 [20].

threshX-HighQ Parameter “ThreshX, HighQ” in TS 38.304 [20].

threshX-Low Parameter “ThreshX, LowP” in TS 38.304 [20].

threshX-LowQ Parameter “ThreshX, LowQ” in TS 38.304 [20].

t-ReselectionEUTRA-SF Parameter "Speed dependent ScalingFactor for TreselectionEUTRA" in TS 38.304 [4]. If the field is not present, the UE behaviour is specified in TS 38.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if the threshServingLowQ is present in SIB2; otherwise it is not present. |

#### –	SIB6

SIB6 contains an ETWS primary notification.

SIB6 information element

-- ASN1START

-- TAG-SIB6-START

SIB6 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningType						OCTET STRING (SIZE (2)),

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- TAG-SIB6-STOP

-- ASN1STOP

SIB6 field descriptions [FFS - detailed field descriptions with references to RAN3 specs will be added later]

messageIdentifier Identifies the source and type of ETWS notification.

serialNumber Identifies variations of an ETWS notification.

warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup.

#### –	SIB7

SIB7 contains an ETWS secondary notification.

SIB7 information element

-- ASN1START

-- TAG-SIB7-START

SIB7 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningMessageSegmentType			ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber		INTEGER (0..63),

warningMessageSegment				OCTET STRING,

dataCodingScheme					OCTET STRING (SIZE (1)) 		OPTIONAL, 	-- Cond Segment1

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- TAG-SIB7-STOP

-- ASN1STOP

SIB7 field descriptions [FFS - detailed field descriptions with references to RAN3 specs will be added later]

dataCodingScheme Identifies the alphabet/coding and the language applied variations of an ETWS notification.

messageIdentifier Identifies the source and type of ETWS notification.

serialNumber Identifies variations of an ETWS notification.

warningMessageSegment Carries a segment of the Warning Message Contents IE

warningMessageSegmentNumber Segment number of the ETWS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on.

warningMessageSegmentType Indicates whether the included ETWS warning message segment is the last segment or not.

| Conditional presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB7, otherwise it is not present. |

#### –	SIB8

SIB8 contains a CMAS notification.

SIB8 information element

-- ASN1START

-- TAG-SIB8-START

SIB8 ::=	SEQUENCE {

messageIdentifier				BIT STRING (SIZE (16)),

serialNumber					BIT STRING (SIZE (16)),

warningMessageSegmentType		ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber	INTEGER (0..63),

warningMessageSegment			OCTET STRING,

dataCodingScheme				OCTET STRING (SIZE (1)) 		OPTIONAL, 	-- Cond Segment1

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

lateNonCriticalExtension		OCTET STRING					OPTIONAL

}

-- TAG-SIB8-STOP

-- ASN1STOP

SIB8 field descriptions [FFS - detailed field descriptions with references to RAN3 specs will be added later]

dataCodingScheme Identifies the alphabet/coding and the language applied variations of a CMAS notification.

messageIdentifier Identifies the source and type of CMAS notification.

serialNumber Identifies variations of a CMAS notification.

warningMessageSegment Carries a segment of the Warning Message Contents IE.

warningMessageSegmentNumber Segment number of the CMAS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on.

warningMessageSegmentType Indicates whether the included CMAS warning message segment is the last segment or not.

| Conditional presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB8, otherwise it is not present. |

#### –	SIB9

SIB9 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

NOTE:	The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock (a.o. to determine MBMS session start/ stop).

SIB9 information element

-- ASN1START

-- TAG-SIB9-START

SIB9 ::=		SEQUENCE {

timeInfo							SEQUENCE {

timeInfoUTC					INTEGER (0..549755813887),

dayLightSavingTime				BIT STRING (SIZE (2))			OPTIONAL,	-- Need R

leapSeconds					INTEGER (-127..128)			OPTIONAL,	-- Need R

localTimeOffset				INTEGER (-63..64)				OPTIONAL	-- Need R

}																OPTIONAL,	-- Need R

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- TAG-SIB9-STOP

-- ASN1STOP

SIB9 field descriptions

dayLightSavingTime It indicates if and how daylight saving time (DST) is applied to obtain the local time.

leapSeconds Number of leap seconds offset between GPS Time and UTC. UTC and GPS time are related i.e. GPS time -leapSeconds = UTC time.

localTimeOffset Offset between UTC and local time in units of 15 minutes. Actual value = field value * 15 minutes. Local time of the day is calculated as UTC time + localTimeOffset.

timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). NOTE 1. This field is excluded when estimating changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of SIBValueTag in SIB1.

NOTE 1:	The UE may use this field together with the leapSeconds field to obtain GPS time as follows: GPS Time (in seconds) = timeInfoUTC (in seconds) -  2,524,953,600 (seconds) + leapSeconds, where 2,524,953,600 is the number of seconds between 00:00:00 on Gregorian calendar date 1 January, 1900 and 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time).

### 6.3.2	Radio resource control information elements

#### –	AdditionalSpectrumEmission

The IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by the UE (see 38.101, section FFS_Section)

AdditionalSpectrumEmission information element

-- ASN1START

-- TAG-ADDITIONALSPECTRUMEMISSION-START

AdditionalSpectrumEmission ::=				INTEGER (0..7)

-- TAG-ADDITIONALSPECTRUMEMISSION-STOP

-- ASN1STOP

#### –	Alpha

The IE Alpha defines possible values for uplink power control.

-- ASN1START

-- TAG-ALPHA-START

Alpha ::=						ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1}

-- TAG-ALPHA-STOP

-- ASN1STOP

#### –	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in 23.003, section 2.10.1.

AMF-Identifier information element

-- ASN1START

-- TAG-AMF-IDENTIFIER-START

AMF-Identifier ::=						BIT STRING (SIZE (24))

-- TAG-AMF-IDENTIFIER-STOP

-- ASN1STOP

#### –	ARFCN-ValueEUTRA

The IE ARFCN-ValueEUTRA is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) E-UTRA carrier frequency, as defined in TS 36.101 [22].

ARFCN-ValueEUTRA information element

-- ASN1START

-- TAG-ARFCN-VALUEEUTRA-START

ARFCN-ValueEUTRA ::=				INTEGER (0..maxEARFCN)

-- TAG-ARFCN-VALUEEUTRA-STOP

-- ASN1STOP

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101- [15], section 5.4.2.

-- ASN1START

-- TAG-ARFCN-VALUE-NR-START

ARFCN-ValueNR ::=				INTEGER (0..maxNARFCN)

-- TAG-ARFCN-VALUE-NR-STOP

-- ASN1STOP

#### –	BWP

The BWP IE is used to configure a bandwidth part as defined in 38.211, section 4.2.2.

For each serving cell the network configures at least an initial bandwidth part comprising of at least a downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The bandwidth part configuration is split into uplink and downlink parameters and into common and dedicated parameters. Common parameters (in BWP-UplinkCommon and BWP-DownlinkCommon) are ”cell specific” and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP information element

-- ASN1START

-- TAG-BANDWIDTH-PART-START

BWP ::= 							SEQUENCE {

locationAndBandwidth				INTEGER (0..37949),

subcarrierSpacing					SubcarrierSpacing,

cyclicPrefix						ENUMERATED { extended }													OPTIONAL	-- Need R

}

BWP-Uplink ::= 						SEQUENCE {

bwp-Id								BWP-Id,

bwp-Common							BWP-UplinkCommon														OPTIONAL,	-- Cond SetupOtherBWP

bwp-Dedicated						BWP-UplinkDedicated														OPTIONAL,	-- Need M

...

}

BWP-UplinkCommon ::=				SEQUENCE {

genericParameters					BWP,

rach-ConfigCommon					SetupRelease { RACH-ConfigCommon }										OPTIONAL, 	-- Need M

pusch-ConfigCommon					SetupRelease { PUSCH-ConfigCommon }										OPTIONAL, 	-- Need M

pucch-ConfigCommon					SetupRelease { PUCCH-ConfigCommon }										OPTIONAL, 	-- Need M

...

}

BWP-UplinkDedicated ::= 			SEQUENCE {

pucch-Config						SetupRelease { PUCCH-Config }											OPTIONAL, 	-- Need M

pusch-Config						SetupRelease { PUSCH-Config }											OPTIONAL, 	-- Need M

configuredGrantConfig				SetupRelease { ConfiguredGrantConfig }									OPTIONAL, 	-- Need M

srs-Config							SetupRelease { SRS-Config }												OPTIONAL, 	-- Need M

beamFailureRecoveryConfig			SetupRelease { BeamFailureRecoveryConfig }								OPTIONAL,	-- Cond SpCellOnly

...

}

BWP-Downlink ::= 					SEQUENCE {

bwp-Id								BWP-Id,

bwp-Common							BWP-DownlinkCommon														OPTIONAL,	-- Cond SetupOtherBWP

bwp-Dedicated						BWP-DownlinkDedicated													OPTIONAL,	-- Need M

...

}

BWP-DownlinkCommon ::=				SEQUENCE {

genericParameters					BWP,

pdcch-ConfigCommon					SetupRelease { PDCCH-ConfigCommon }										OPTIONAL,	-- Need M

pdsch-ConfigCommon					SetupRelease { PDSCH-ConfigCommon }										OPTIONAL,	-- Need M

...

}

BWP-DownlinkDedicated ::= 			SEQUENCE {

pdcch-Config						SetupRelease { PDCCH-Config }											OPTIONAL,	-- Need M

pdsch-Config						SetupRelease { PDSCH-Config }											OPTIONAL,	-- Need M

sps-Config							SetupRelease { SPS-Config }												OPTIONAL, 	-- Need M

radioLinkMonitoringConfig			SetupRelease { RadioLinkMonitoringConfig }								OPTIONAL,	-- Need M

...

}

-- TAG-BANDWIDTH-PART-STOP

-- ASN1STOP

BWP field descriptions

cyclicPrefix Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Normal CP is supported for all numerologies and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. (see 38.211, section 4.2.2)

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 with assumptions as described in TS 38.213, section 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see 38.213, section 12)

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to 38.211, Table 4.2-1. The value kHz15 corresponds to µ=0, kHz30 to µ=1, and so on. Only the values 15, 30, or 60 kHz (<6GHz), and 60 or 120 kHz (>6GHz) are applicable. For the initial DL BWP this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell.

BWP-Downlink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The BWP ID=0 is always associated with the initial BWP and may hence not be used here (in other bandwidth parts). The NW may trigger the UE to swtich UL or DL BWP using a DCI field. The four code points in that DCI field map to the RRC-configured BWP-ID as follows: For up to 3 configured BWPs (in addition to the initial BWP) the DCI code point is equivalent to the BWP ID (initial = 0, first dedicated = 1, ...). If the NW configures 4 dedicated bandwidth parts, they are identified by DCI code points 0 to 3. In this case it is not possible to switch to the initial BWP using the DCI field. (see 38.211, 38.213, section 12)

BWP-DownlinkCommon field descriptions

pdcch-ConfigCommon Cell specific parameters for the PDCCH of this BWP

pdsch-ConfigCommon Cell specific parameters for the PDSCH of this BWP

BWP-DownlinkDedicated field descriptions

pdcch-Config UE specific PDCCH configuration for one BWP

pdsch-Config UE specific PDSCH configuration for one BWP

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP.

radioLinkMonitoringConfig UE specific configuration of radio link monitoring for detecting cell- and beam radio link failure occasions.

BWP-Uplink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The BWP ID=0 is always associated with the initial BWP and may hence not be used here (in other bandwidth parts). The NW may trigger the UE to swtich UL or DL BWP using a DCI field. The four code points in that DCI field map to the RRC-configured BWP-ID as follows: For up to 3 configured BWPs (in addition to the initial BWP) the DCI code point is equivalent to the BWP ID (initial = 0, first dedicated = 1, ...). If the NW configures 4 dedicated bandwidth parts, they are identified by DCI code points 0 to 3. In this case it is not possible to switch to the initial BWP using the DCI field. Corresponds to L1 parameter 'UL-BWP-index'. (see 38.211, 38.213, section 12)

BWP-UplinkCommon field descriptions

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs allows the UE to acquire the SSB associated to the serving cell.

BWP-UplinkDedicated field descriptions

beamFailureRecoveryConfig Determines how the UE performs Beam Failure Recovery upon detection of a Beam Failure (see RadioLinkMonitoringConfig)

configuredGrantConfig A Configured-Grant of typ1 or type2. It may be configured for UL or SUL but in case of type1 not for both at a time.

pucch-Config PUCCH configuration for one BWP of the regular UL or SUL of a serving cell. If the UE is configured with SUL, the network configures PUCCH only on the BWPs of one of the uplinks (UL or SUL).The network configures PUCCH-Config for each SpCell. If supported by the UE, the network may configure at most one additional SCell of a cell group with PUCCH-Config (i.e. PUCCH SCell). For EN-DC, The NW configures at most one serving cell per frequency range with PUCCH. And for EN-DC, if two PUCCH groups are configured, the serving cells of the NR PUCCH group in FR2 use the same numerology. The NW configures PUCCH for a BWP only when setting up the BWP. Afterwards, only modifications of the pucch-Config are allowed. If one (S)UL BWP of a serving cell is configured with PUCCH, all other (S)UL BWPs must be configured with PUCCH, too.”

pusch-Config PUSCH configuration for one BWP of the regular UL or SUL of a serving cell. If the UE is configured with SUL and if it has a PUSCH-Config for both UL and SUL, a carrier indicator field in DCI indicates for which of the two to use an UL grant. See also L1 parameter 'dynamicPUSCHSUL' (see 38.213, section FFS_Section)

srs-Config Uplink sounding reference signal configuration

| Conditional Presence | Explanation |
| --- | --- |
| SetupOnly | The field is optionally present, Need M, upon configuration of a new SCell. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need M, in the BWP-UplinkDedicated of an SpCell. It is absent otherwise. |
| SetupOtherBWP | The field is mandatory present, Need M, upon configuration of a new BWP if the parent IE is included (if configured with UL/DL). The field is optionally present, Need M, otherwise. |

#### –	BWP-Id

The IE BWP-Id is used to refer to Bandwidth Parts (BWP). The initial BWP is referred to by BWP-Id 0. The other BWPs are referred to by BWP-Id 1 to maxNrofBWPs.

BWP-Id information element

-- ASN1START

-- TAG-BWP-ID-START

BWP-Id ::=							INTEGER (0..maxNrofBWPs)

-- TAG-BWP-ID-STOP

-- ASN1STOP

#### –	BeamFailureRecoveryConfig

The BeamFailureRecoveryConfig IE is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also 38.321, section 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAM-FAILURE-RECOVERY-CONFIG-START

BeamFailureRecoveryConfig ::= 		SEQUENCE {

rootSequenceIndex-BFR				INTEGER (0..137)															OPTIONAL,	-- Need M

rach-ConfigBFR						RACH-ConfigGeneric															OPTIONAL,	-- Need M

rsrp-ThresholdSSB				RSRP-Range																	OPTIONAL,	-- Need M

candidateBeamRSList					SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR		OPTIONAL,	-- Need M

ssb-perRACH-Occasion				ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} 	OPTIONAL,	-- Need M

ra-ssb-OccasionMaskIndex			INTEGER (0..15)																OPTIONAL, 	-- Need M

recoverySearchSpaceId				SearchSpaceId																OPTIONAL,	-- Cond CF-BFR

ra-Prioritization					RA-Prioritization															OPTIONAL,	-- Need R

beamFailureRecoveryTimer			ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}			OPTIONAL,	-- Need M

...,

[[

msg1-SubcarrierSpacing				SubcarrierSpacing															OPTIONAL	-- Need M

]]

}

PRACH-ResourceDedicatedBFR ::= 		CHOICE {

ssb									BFR-SSB-Resource,

csi-RS								BFR-CSIRS-Resource

}

BFR-SSB-Resource ::= 			SEQUENCE {

ssb								SSB-Index,

ra-PreambleIndex				INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=			SEQUENCE {

csi-RS							NZP-CSI-RS-ResourceId,

ra-OccasionList					SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)	OPTIONAL,	-- Need R

ra-PreambleIndex				INTEGER (0..63)																	OPTIONAL,	-- Need R

...

}

-- TAG-BEAM-FAILURE-RECOVERY-CONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. ms10 corresponds to 10ms, ms20 to 20ms, and so on.

candidateBeamRSList A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters

msg1-SubcarrierSpacing Subcarrier spacing for contention based- and contention free beam failure recovery procedure. Only the values 15 or 30 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section).

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free Random Access to recover from beam failure. The signalled threshold is applied directly for SSB; a threshold for CSI-RS is determined by linearly scaling singalled value based on Pc_ss corresponding to the CSI-RS resource. (see FFS_Specification, FFS_Section)

ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see 38.321, section 5.1.1).

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 36.321. The mask is valid for all SSB resources

rach-ConfigBFR Configuration of contention free random access occasions for BFR

recoverySearchSpaceId Search space to use for BFR RAR.

ssb-perRACH-Occasion Number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion')

BFR-CSIRS-Resource field descriptions

csi-RS The ID of a NZP-CSI-RS-Resource configured in the CSI-MeasConfig of this serving cell. This reference signal determines a candidate beam for beam failure recovery (BFR).

ra-OccasionList RA occasions that the UE shall use when performing BFR upon selecting the candidate beam identified by this CSI-RS. If the field is absent the UE uses the RA occasion associated with the SSB that is QCLed with this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS. If the field is absent, the UE uses the preamble index associated with the SSB that is QCLed with this CSI-RS.

BFR-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing BFR upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell. It determines a candidate beam for beam failure recovery (BFR)

| Conditional Presence | Explanation |
| --- | --- |
| CF-BFR | The field is mandatory present, Need R, if contention free random access resources for BFR are configured. It is optionally present otherwise. |

#### –	CellAccessRelatedInfo

The IE CellAccessRelatedInfo indicates cell access related information for this cell.

CellAccessRelatedInfo information element

-- ASN1START

-- TAG-CELL-ACCESS-RELATED-INFO-START

CellAccessRelatedInfo	::=			SEQUENCE {

plmn-IdentityList					PLMN-IdentityInfoList,

cellIdentity								CellIdentity,

cellReservedForOtherUse            		ENUMERATED {true}  OPTIONAL,

...

}

-- TAG- CELL-ACCESS-RELATED-INFO-STOP

-- ASN1STOP

CellAccessRelatedInfo field descriptions

plmn-IdentityList The PLMN-IdentityList is used to configure a set of PLMN-IdentityInfo elements. Each of those elements contains a list of one or more PLMN Identities and additional information associated with this PLMN. The total number of PLMNs in the PLMNIdentitynfoList does not exceed 12. The PLMN index is defined as b1+b2+…+b(n-1)+i If this PLMN is included at the n-th entry of PLMN-IdentityInfoList and the i-th entry of its corresponding PLMN-IdentityInfo, where b(j) is the number of PLMN-Identity entries in each PLMN-IdentityInfo respectively.

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELL-GROUP-CONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig	::= 						SEQUENCE {

cellGroupId									CellGroupId,

rlc-BearerToAddModList 						SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig				OPTIONAL,   -- Need N

rlc-BearerToReleaseList						SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity			OPTIONAL,   -- Need N

mac-CellGroupConfig							MAC-CellGroupConfig												OPTIONAL,	-- Need M

physicalCellGroupConfig						PhysicalCellGroupConfig											OPTIONAL,	-- Need M

spCellConfig								SpCellConfig													OPTIONAL, 	-- Need M

sCellToAddModList							SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig				OPTIONAL,	-- Need N

sCellToReleaseList							SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex				OPTIONAL,	-- Need N

...,

[[

reportuUplinkTxDirectCurrent			ENUMERATED {true}												OPTIONAL	-- Cond BWP-Reconfig

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=						SEQUENCE {

servCellIndex						ServCellIndex															OPTIONAL,	-- Cond SCG

reconfigurationWithSync 			ReconfigurationWithSync 												OPTIONAL,	-- Cond ReconfWithSync

rlf-TimersAndConstants				SetupRelease { RLF-TimersAndConstants }									OPTIONAL,	-- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                         								OPTIONAL,	-- Need S

spCellConfigDedicated				ServingCellConfig														OPTIONAL,	-- Need M

...

}

ReconfigurationWithSync ::=			SEQUENCE {

spCellConfigCommon					ServingCellConfigCommon													OPTIONAL,	-- Need M

newUE-Identity						RNTI-Value,

t304								ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated				CHOICE {

uplink								RACH-ConfigDedicated,

supplementaryUplink				RACH-ConfigDedicated

}																											OPTIONAL,	-- Need N

...

}

SCellConfig ::=						SEQUENCE {

sCellIndex							SCellIndex,

sCellConfigCommon					ServingCellConfigCommon													OPTIONAL,	-- Cond SCellAdd

sCellConfigDedicated				ServingCellConfig														OPTIONAL,	-- Cond SCellAddMod

...

}

-- TAG-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportuUplinkTxDirectCurrent Enables reporting of uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 ([14], Table 8.1.1-1). n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets on-going RLF timers and counter. In EN-DC, rlf-TimersAndConstants cannot be released.

sCellToAddModList List of seconary serving cells (SCells) to be added or modified.

sCellToReleaseList List of seconary serving cells (SCells) to be released

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

SpCellConfig field descriptions

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. In EN-DC, rlf-TimersAndConstants cannot be released.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed in the same message. Otherwise it is absent. |
| ReconfWithSync | The field is mandatory present in case of SpCell change, PSCell addition and security key change; otherwise it is optionally present, need M. |
| SCellAdd | The field is mandatory present, need M, upon SCell addition; otherwise it is not present |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

#### –	CellGroupId

The IE CellGroupId is used to identify a cell group. 0 identifies the master cell group. Other values identify secondary cell groups. In this version of the specification only values 0 and 1 are supported.

CellGroupId information element

-- ASN1START

-- TAG-CELLGROUPID-START

CellGroupId ::=								INTEGER (0.. maxSecondaryCellGroups)

-- TAG-CELLGROUPID-STOP

-- ASN1STOP

#### –	CellIdentity

The IE CellIdentity is used to unambiguously identify a cell within a PLMN.

CellIdentity information element

-- ASN1START

CellIdentity ::=					BIT STRING (SIZE (36))

-- ASN1STOP

#### –	CellGlobalIdNR

The IE CellGlobalIdNR specifies the NR Cell Global Identifier (NR-CGI), the globally unique identity of a cell in NR.

CellGlobalIdNR information element

-- ASN1START

-- TAG-CGI-NR-START

CellGlobalIdNR ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellIdentityNR							CellIdentity

}

-- TAG-CGI-NR-STOP

-- ASN1STOP

#### –	CellReselectionPriority

The IE CellReselectionPriority concerns the absolute priority of the concerned carrier frequency, as used by the cell reselection procedure. Corresponds with parameter "priority" in TS 38.304 [21]. Value 0 means: lowest priority. The UE behaviour for the case the field is absent, if applicable, is specified in TS 38.304 [21].

CellReselectionPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONPRIORITY-START

CellReselectionPriority ::=				INTEGER (0..ffsValue)

-- TAG-CELLRESELECTIONPRIORITY-STOP

-- ASN1STOP

Editor’s Note: FFS Maximum number of cell reselection priorities and whether sub-priorities are necessary.

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see 38.214, section 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::= 									SEQUENCE {

codebookType 										CHOICE {

type1 												SEQUENCE {

subType										CHOICE {

typeI-SinglePanel									SEQUENCE {

nrOfAntennaPorts									CHOICE {

two													SEQUENCE {

twoTX-CodebookSubsetRestriction						BIT STRING (SIZE (6))

},

moreThanTwo											SEQUENCE {

n1-n2												CHOICE {

two-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction			BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction			BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction			BIT STRING (SIZE (64))

},

typeI-SinglePanel-codebookSubsetRestriction-i2		BIT STRING (SIZE (16))	OPTIONAL	-- Need R

}

},

typeI-SinglePanel-ri-Restriction					BIT STRING (SIZE (8))

},

typeI-MultiPanel									SEQUENCE {

ng-n1-n2												CHOICE {

two-two-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (8)),

two-four-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (16)),

four-two-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (8)),

two-two-two-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (64)),

two-eight-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (32)),

four-four-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (16)),

two-four-two-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (128)),

four-two-two-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (64))

},

ri-Restriction							BIT STRING (SIZE (4))

}

},

codebookMode										INTEGER (1..2)

},

type2									SEQUENCE {

subType									CHOICE {

typeII									SEQUENCE {

n1-n2-codebookSubsetRestriction			CHOICE {

two-one									BIT STRING (SIZE (16)),

two-two									BIT STRING (SIZE (43)),

four-one								BIT STRING (SIZE (32)),

three-two								BIT STRING (SIZE (59)),

six-one									BIT STRING (SIZE (48)),

four-two								BIT STRING (SIZE (75)),

eight-one								BIT STRING (SIZE (64)),

four-three								BIT STRING (SIZE (107)),

six-two									BIT STRING (SIZE (107)),

twelve-one								BIT STRING (SIZE (96)),

four-four								BIT STRING (SIZE (139)),

eight-two								BIT STRING (SIZE (139)),

sixteen-one								BIT STRING (SIZE (128))

},

typeII-RI-Restriction					BIT STRING (SIZE (2))

},

typeII-PortSelection					SEQUENCE {

portSelectionSamplingSize				ENUMERATED {n1, n2, n3, n4}					OPTIONAL,		-- Need M

typeII-PortSelectionRI-Restriction		BIT STRING (SIZE (2))

}

},

phaseAlphabetSize						ENUMERATED {n4, n8},

subbandAmplitude						BOOLEAN,

numberOfBeams							ENUMERATED {two, three, four}

}

}

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

CodebookConfig field descriptions

codebookMode CodebookMode as specified in 38.214 section 5.2.2.2.2

codebookType CodebookType including possibly sub-types and the corresponding parameters for each. Corresponds to L1 parameter 'CodebookType' (see 38.214, section 5.2.2.2)

n1-n2-codebookSubsetRestriction Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction. Corresponds to L1 parameters 'CodebookConfig-N1', 'CodebookConfig-N2' The CHOICE name indicates the value of n1 and n2, the CHOICE contents is the codebook subset restriction bitmap Corresponds to L1 parameter ' TypeII-CodebookSubsetRestriction' (see 38.214 section 5.2.2.2.3) Number of bits for codebook subset restriction is ceil(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!)

n1-n2 Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction. Corresponds to L1 parameters 'CodebookConfig-N1', 'CodebookConfig-N2' 'TypeI-SinglePanel-CodebookSubsetRestriction ' (see 38.214 section 5.2.2.2.1)

ng-n1-n2 Codebook subset restriction for Type I Multi-panel codebook Corresponds to L1 parameter 'TypeI-MultiPanel-CodebookSubsetRestriction' (see 38.214, section 5.2.2.2.2)

numberOfBeams Number of beams, L, used for linear combination

phaseAlphabetSize The size of the PSK alphabet, QPSK or 8-PSK

portSelectionSamplingSize The size of the port selection codebook (parameter d)

ri-Restriction Restriction for RI for TypeI-MultiPanel-RI-Restriction Corresponds to L1 parameter 'TypeI-MultiPanel-RI-Restriction' (see 38.214, section 5.2.2.2.2)

subbandAmplitude If subband amplitude reporting is activated (true)

twoTX-CodebookSubsetRestriction Codebook subset restriction for 2TX codebook Corresponds to L1 parameter ' TypeI-SinglePanel-2Tx-CodebookSubsetRestriction' (see 38.214 section 5.2.2.2.1)

typeI-SinglePanel-codebookSubsetRestriction-i2 i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction-i2' (see 38.214 section 5.2.2.2.1)

typeI-SinglePanel-ri-Restriction Restriction for RI for TypeI-SinglePanel-RI-Restriction Corresponds to L1 parameter 'TypeI-SinglePanel-RI-Restriction' (see 38.214, section 5.2.2.2.1)

typeII-PortSelectionRI-Restriction Restriction for RI for TypeII-PortSelection-RI-Restriction Corresponds to L1 parameter 'TypeII-PortSelection-RI-Restriction' (see 38.214, section 5.2.2.4)

typeII-RI-Restriction Restriction for RI for TypeII-RI-Restriction Corresponds to L1 parameter 'TypeII-RI-Restriction' (see 38.214, section 5.2.2.2.3)

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2).

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=			SEQUENCE {

frequencyHopping						ENUMERATED {intraSlot, interSlot} 									OPTIONAL,	-- Need S,

cg-DMRS-Configuration				DMRS-UplinkConfig,

mcs-Table							ENUMERATED {qam256, qam64LowSE}									OPTIONAL, 	-- Need S

mcs-TableTransformPrecoder			ENUMERATED {qam256, qam64LowSE}									OPTIONAL,	-- Need S

uci-OnPUSCH							SetupRelease { CG-UCI-OnPUSCH }											OPTIONAL,	-- Need M

resourceAllocation					ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size								ENUMERATED {config2}											OPTIONAL,	-- Need S

powerControlLoopToUse				ENUMERATED {n0, n1},

p0-PUSCH-Alpha						P0-PUSCH-AlphaSetId,

transformPrecoder					ENUMERATED {enabled}											OPTIONAL,	-- Need S

nrofHARQ-Processes					INTEGER(1..16),

repK									ENUMERATED {n1, n2, n4, n8},

repK-RV								ENUMERATED {s1-0231, s2-0303, s3-0000}						OPTIONAL,	-- Cond RepK

periodicity							ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer					INTEGER (1..64)													OPTIONAL,	-- Need R

rrc-ConfiguredUplinkGrant				SEQUENCE {

timeDomainOffset							INTEGER  (0..5119),

timeDomainAllocation						INTEGER  (0..15),

frequencyDomainAllocation				BIT STRING (SIZE(18)),

antennaPort								INTEGER (0..31),

dmrs-SeqInitialization					INTEGER (0..1)											OPTIONAL,	-- Cond NoTransformPrecoder

precodingAndNumberOfLayers				INTEGER (0..63),

srs-ResourceIndicator					INTEGER (0..15),

mcsAndTBS								INTEGER (0..31),

frequencyHoppingOffset					INTEGER (1.. maxNrofPhysicalResourceBlocks-1) 			OPTIONAL,	-- Need M

pathlossReferenceIndex					INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...

}																									OPTIONAL		-- Need R

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic									SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic								BetaOffsets

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig field descriptions

antennaPort Indicates the anntenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214, section 6.1.2, and TS 38.212, section 7.3.1.

cg-DMRS-Configuration DMRS configuration, corresponds to L1 parameter ‘UL-TWG-DMRS’ (see TS 38.214, section 6.1.2).

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321,) in number of periodicities.

frequencyDomainAllocation Indicates the frequency domain resource allocation, see TS 38.214, section 6.1.2, and TS 38.212, section 7.3.1).

frequencyHopping The value intraSlot enables ‘Intra-slot frequency hopping’ and the value interSlot enables ‘Inter-slot frequency hopping’. If the field is absent, frequency hopping is not configured.

frequencyHoppingOffset Enables intra-slot frequency hopping with the given frequency hopping offset. Frequency hopping offset used when frequency hopping is enabled. Corresponds to L1 parameter 'Frequency-hopping-offset' (see TS 38.214, section 6.1.2).

mcs-Table Indicates the MCS table the UE shall use for PUSCH without transform precoding. If the field is absent the UE applies the value 64QAM.

mcs-TableTransformPrecoder Indicates the MCS table the UE shall use for PUSCH with transform precoding. If the field is absent the UE applies the value 64QAM.

mcsAndTBS The modulation order, target code rate and TB size (see TS38.214, section 6.1.2).

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321, section 5.4.1.

p0-PUSCH-Alpha Index of the P0-PUSCH-AlphaSet to be used for this configuration.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2. Corresponds to L1 parameter 'UL-TWG-periodicity' (see TS 38.321, section 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60kHz with normal CP: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} (see 38.214, Table 6.1.2.3-1)

powerControlLoopToUse Closed control loop to apply. Corresponds to L1 parameter 'PUSCH-closed-loop-index' (see TS 38.213, section 7.7.1).

rbg-Size Selection between config 1 and config 2 for RBG size for PUSCH. When the field is absent the UE applies the value config1. Note: rbg-Size is used when the transformPrecoder parameter is disabled.

repK-RV If repetitions is used, this field indicates the redundancy version (RV) sequence to use. See TS 38.214, section 6.1.2.

repK The number or repetitions of K.

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1. For Type 1 UL data transmission without grant, "resourceAllocation" should be resourceAllocationType0 or resourceAllocationType1.

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2). Type 1 confgured grant may be configured for UL or SUL, but not for both simultaneously.

srs-ResourceIndicator Indicates a the SRS resource to be used. If no SRS resources are configured in the current UL BWP, the UE shall ignore this field.

timeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214, section 6.1.2 and TS 38.212, section 7.3.1.

timeDomainOffset Offset related to SFN=0, see TS 38.321, section 5.8.2.

transformPrecoder Enables transformer precoder for type1 and type2. If the field is absent, the UE enables or disables transformer precoder in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon, see 38.214, section 6.1.3.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic.

| Conditional Presence | Explanation |
| --- | --- |
| RepK | The field is mandatory present if repK is set to n2, n4,or n8. It is not present if repK is set to n1. |
| NoTransformPrecoder | The field is mandatory present if transformPrecoder is disabled. It is absent otherwise. |

#### –	ConnEstFailureControl

The IE ConnEstFailureControl is used to configure parameters for connection establishment failure control.

ConnEstFailureControl information element

-- ASN1START

-- TAG-CONNESTFAILURECONTROL-START

ConnEstFailureControl ::= 	SEQUENCE {

connEstFailCount					ENUMERATED {n1, n2, n3, n4},

connEstFailOffsetValidity			ENUMERATED {s30, s60, s120, s240, s300, s420, s600, s900},

connEstFailOffset					INTEGER (0..15)																OPTIONAL	-- Need S

}

-- TAG-CONNESTFAILURECONTROL-STOP

-- ASN1STOP

ConnEstFailureControl field descriptions

connEstFailCount Number of times that the UE detects T300 expiry on the same cell before applying connEstFailOffset.

connEstFailOffset Parameter “Qoffsettemp” in TS 38.304 [4]. If the field is not present the value of infinity shall be used for “Qoffsettemp”.

connEstFailOffsetValidity Amount of time that the UE applies connEstFailOffset before removing the offset from evaluation of the cell. Value s30 corresponds to 30 seconds, s60 corresponds to 60 seconds, and so on.

#### –	ControlResourceSet

The IE ControlResourceSet is used to configure a time/frequency control resource set (CORESET) in which to search for downlink control information (see 38.213, section FFS_Section).

ControlResourceSet information element

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::= 					SEQUENCE {

controlResourceSetId					ControlResourceSetId,

frequencyDomainResources				BIT STRING (SIZE (45)),

duration								INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType						CHOICE {

interleaved								SEQUENCE {

reg-BundleSize							ENUMERATED {n2, n3, n6},

interleaverSize							ENUMERATED {n2, n3, n6},

shiftIndex								INTEGER(0..maxNrofPhysicalResourceBlocks-1) 						 OPTIONAL 	-- Need S

},

nonInterleaved 							NULL

},

precoderGranularity						ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList					SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId				OPTIONAL,	-- Need N

tci-StatesPDCCH-ToReleaseList			SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId				OPTIONAL,	-- Need N

tci-PresentInDCI						ENUMERATED {enabled}													OPTIONAL, 	-- Need S

pdcch-DMRS-ScramblingID					INTEGER (0..65535)													OPTIONAL, 	-- Need S

...

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

ControlResourceSet field descriptions

cce-REG-MappingType Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG). Corresponds to L1 parameter 'CORESET-CCE-REG-mapping-type' (see 38.211Section sections 7.3.2.2 and 7.4.1.3.2).

controlResourceSetId Corresponds to L1 parameter 'CORESET-ID'. Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon (controlResourceSetZero) and is hence not used here in the ControlResourceSet IE. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling. The controlResourceSetId is unique among the BWPs of a ServingCell.

duration Contiguous time duration of the CORESET in number of symbols. Corresponds to L1 parameter 'CORESET-time-duration' (see 38.211, section 7.3.2.2FFS_Section)

frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from CRB#0.. The first (left-most / most significant) bit corresponds to CRB#0, and so on.. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero. Corresponds to L1 parameter 'CORESET-freq-dom'(see 38.211, section 7.3.2.2).

interleaverSize Corresponds to L1 parameter 'CORESET-interleaver-size' (see 38.211, 38.213, section FFS_Section).

pdcch-DMRS-ScramblingID PDCCH DMRS scrambling initalization. Corresponds to L1 parameter 'PDCCH-DMRS-Scrambling-ID' (see 38.211, section 7.4.1). When the field is absent the UE applies the value of the physCellId configured for this serving cell.

precoderGranularity Precoder granularity in frequency domain. Corresponds to L1 parameter 'CORESET-precoder-granuality' (see 38.211, sections 7.3.2.2 and 7.4.1.3.2).

reg-BundleSize Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles. Corresponds to L1 parameter 'CORESET-REG-bundle-size' (see 38.211, section FFS_Section).

shiftIndex Corresponds to L1 parameter 'CORESET-shift-index'. When the field is absent the UE applies the value of the physCellId configured for this serving cell (see 38.211, section 7.3.2.2).

tci-PresentInDCI If at least spatial QCL is configured/indicated, this field indicates if TCI field is present or not present in DL-related DCI. When the field is absent the UE considers the TCI to be absent/disabled. Corresponds to L1 parameter 'TCI-PresentInDCI' (see 38,213, section 5.1.5).

tci-StatesPDCCH-ToAddList, tci-StatesPDCCH-ToReleaseList A subset of the TCI states defined in TCI-States used for providing QCL relationships between the DL RS(s) in one RS Set (TCI-State) and the PDCCH DMRS ports. Corresponds to L1 parameter 'TCI-StatesPDCCH' (see 38.213, section10.). The network configures at most maxNrofTCI-StatesPDCCH entries.

#### –	ControlResourceSetId

The ControlResourceSetId IE concerns a short identity, used to identify a control resource set within a serving cell. The ControlResourceSetId = 0 identifies the ControlResourceSet#0 configured via PBCH (MIB) and in controlResourceSetZero (ServingCellConfigCommon). The ID space is used across the BWPs of a Serving Cell. The number of CORESETs per BWP is limited to 3 (including the initial CORESET).

ControlResourceSetId information element

-- ASN1START

-- TAG-CONTROL-RESOURCE-SET-ID-START

ControlResourceSetId ::=				INTEGER (0..maxNrofControlResourceSets-1)

-- TAG-CONTROL-RESOURCE-SET-ID-STOP

-- ASN1STOP

#### –	ControlResourceSetZero

The IE ControlResourceSetZero is used to configure CORESET#0 of the initial BWP (see TS 38.213 [13], section 13).

ControlResourceSetZero information element

-- ASN1START

-- TAG-CONTROLRESOURCESETZERO-START

ControlResourceSetZero ::=					INTEGER (0..15)

-- TAG-CONTROLRESOURCESETZERO-STOP

-- ASN1STOP

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information elements

-- ASN1START

CrossCarrierSchedulingConfig ::=		SEQUENCE {

schedulingCellInfo						CHOICE {

own										SEQUENCE {					-- No cross carrier scheduling

cif-Presence							BOOLEAN

},

other									SEQUENCE {					-- Cross carrier scheduling

schedulingCellId						ServCellIndex,

cif-InSchedulingCell					INTEGER (1..7)

}																	-- Cond SCellOnly

},

...

}

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

cif-Presence The field is used to indicate whether carrier indicator field is present (value TRUE) or not (value FALSE) in PDCCH/EPDCCH DCI formats, see TS 38.213 [REF, SECTION].

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [REF, SECTION]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

pdsch-Start The starting OFDM symbol of PDSCH for the concerned SCell, see TS [REF]. Values 1, 2, 3 are applicable when dl-Bandwidth for the concerned SCell is greater than 10 resource blocks, values 2, 3, 4 are applicable when dl-Bandwidth for the concerned SCell is less than or equal to 10 resource blocks, see TS [REF].

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell.

| Conditional Presence | Explanation |
| --- | --- |
| SCellOnly | This field is optionally present, Need M, for SCells. It is absent otherwise |

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of aperiodic trigger states. Each codepoint of the DCI field "CSI request" is associated with one trigger state. Upon reception of the value associated with a trigger state, the UE will perform measurement of aperiodic CSI-RS (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

CSI-AperiodicTriggerStateList information element

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

CSI-AperiodicTriggerStateList ::=	SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=		SEQUENCE {

associatedReportConfigInfoList		SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...

}

CSI-AssociatedReportConfigInfo ::=		SEQUENCE {

reportConfigId						CSI-ReportConfigId,

resourcesForChannel					CHOICE {

nzp-CSI-RS							SEQUENCE {

resourceSet							INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info							SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId	OPTIONAL -- Cond Aperiodic

},

csi-SSB-ResourceSet					INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference		INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)			OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesforInterference	INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)		OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...

}

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

CSI-AssociatedReportConfigInfo field descriptions

csi-IM-ResourcesForInterference CSI-IM-ResourceSet for interference measurement. Entry number in csi-IM-ResourceSetList in the CSI-ResourceConfig indicated by csi-IM-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (1 corresponds to the first entry, 2 to the second entry, and so on). The indicated CSI-IM-ResourceSet should have exactly the same number of resources like the NZP-CSI-RS-ResourceSet indicated in nzp-CSI-RS-ResourcesforChannel.

csi-SSB-ResourceSet CSI-SSB-ResourceSet for channel measurements. Entry number in csi-SSB-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (1 corresponds to the first entry, 2 to the second entry, and so on).

nzp-CSI-RS-ResourcesforInterference NZP-CSI-RS-ResourceSet for interference measurement. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by nzp-CSI-RS-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (1 corresponds to the first entry, 2 to the second entry, and so on).

qcl-info List of references to TCI-States for providing the QCL source and QCL type for for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet indicated by nzp-CSI-RS-ResourcesforChannel. Each TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resourcesForChannelMeasurement (in the CSI-ReportConfig indicated by reportConfigId above) belong to. First entry in qcl-info-forChannel corresponds to first entry in nzp-CSI-RS-Resources of that NZP-CSI-RS-ResourceSet, second entry in qcl-info-forChannel corresponds to second entry in nzp-CSI-RS-Resources, and so on. Corresponds to L1 parameter 'QCL-Info-aPeriodicReportingTrigger' (see 38.214, section 5.2.1.5.1)

reportConfigId The reportConfigId of one of the CSI-ReportConfigToAddMod configured in CSI-MeasConfig

resourceSet NZP-CSI-RS-ResourceSet for channel measurements. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (1 corresponds to the first entry, 2 to thesecond entry, and so on).

| Conditional Presence | Explanation |
| --- | --- |
| Aperiodic | The field is mandatory present if the NZP-CSI-RS-Resources in the associated resourceSet have the resourceType aperiodic. The field is absent otherwise. |
| CSI-IM-ForInterference | This field is optional need M if the CSI-ReportConfig identified by reportConfigId is configured with csi-IM-ResourcesForInterference; otherwise it is absent. |
| NZP-CSI-RS-ForInterference | This field is optional need M if the CSI-ReportConfig identified by reportConfigId is configured with nzp-CSI-RS-ResourcesForInterference; otherwise it is absent. |

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=			SEQUENCE {

startingRB							INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs								INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

CSI-FrequencyOccupation field descriptions

nrofRBs Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.

startingRB PRB where this CSI resource starts in relation tocommon resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...)

#### –	CSI-IM-Resource

The IE CSI-IM-Resource is used to configure one CSI Interference Management (IM) resource.

CSI-IM-Resource information element

-- ASN1START

-- TAG-CSI-IM-RESOURCE-START

CSI-IM-Resource ::= 				SEQUENCE {

csi-IM-ResourceId					CSI-IM-ResourceId,

csi-IM-ResourceElementPattern			CHOICE {

pattern0								SEQUENCE {

subcarrierLocation-p0					ENUMERATED { s0, s2, s4, s6, s8, s10 },

symbolLocation-p0						INTEGER (0..12)

},

pattern1								SEQUENCE {

subcarrierLocation-p1					ENUMERATED { s0, s4, s8 },

symbolLocation-p1						INTEGER (0..13)

}

}																									OPTIONAL,	-- Need M

freqBand							CSI-FrequencyOccupation											OPTIONAL,	-- Need M

periodicityAndOffset				CSI-ResourcePeriodicityAndOffset								OPTIONAL,	-- Cond PeriodicOrSemiPersistent

...

}

-- TAG-CSI-IM-RESOURCE-STOP

-- ASN1STOP

CSI-IM-Resource field descriptions

csi-IM-ResourceElementPattern The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters. Corresponds to L1 parameter 'CSI-IM-RE-pattern' (see 38.214, section 5.2.2.3.4)

freqBand Frequency-occupancy of CSI-IM. Corresponds to L1 parameter 'CSI-IM-FreqBand' (see 38.214, section 5.2.2.3.2)

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent CSI-IM. Corresponds to L1 parameter 'CSI-IM-timeConfig'

subcarrierLocation-p0 OFDM subcarrier occupancy of the CSI-IM resource for Pattern0. Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p1 OFDM subcarrier occupancy of the CSI-IM resource for Pattern1. Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p0 OFDM symbol location of the CSI-IM resource for Pattern0. Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p1 OFDM symbol location of the CSI-IM resource for Pattern1. Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is mandatory present, Need M, for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::= 				INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSet

The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSet information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESET-START

CSI-IM-ResourceSet ::=				SEQUENCE {

csi-IM-ResourceSetId				CSI-IM-ResourceSetId,

csi-IM-Resources					SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId,

...

}

-- TAG-CSI-IM-RESOURCESET-STOP

-- ASN1STOP

CSI-IM-ResourceSet field descriptions

csi-IM-Resources CSI-IM-Resources associated with this CSI-IM-ResourceSet. Corresponds to L1 parameter 'CSI-IM-ResourceConfigList' (see 38.214, section 5.2)

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=			INTEGER (0..maxNrofCSI-IM-ResourceSets-1)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-MeasConfig

The CSI-MeasConfig IE is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also 38.214, section 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEAS-CONFIG-START

CSI-MeasConfig ::=					SEQUENCE {

nzp-CSI-RS-ResourceToAddModList		SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource					OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId				OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet			OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId			OPTIONAL, -- Need N

csi-IM-ResourceToAddModList			SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource							OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId						OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet					OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId					OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet					OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddReleaseList	SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId				OPTIONAL, -- Need N

csi-ResourceConfigToAddModList 		SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig			OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList 	SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId			OPTIONAL, -- Need N

csi-ReportConfigToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig				OPTIONAL, -- Need N

csi-ReportConfigToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId				OPTIONAL, -- Need N

reportTriggerSize					INTEGER (0..6)																			OPTIONAL, -- Need M

aperiodicTriggerStateList			SetupRelease { CSI-AperiodicTriggerStateList }											OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList			SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }					OPTIONAL, -- Need M

...

}

-- TAG-CSI-MEAS-CONFIG-STOP

-- ASN1STOP

CSI-MeasConfig field descriptions

aperiodicTriggerStateList Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement. FFS: How to address the MAC-CE configuration

csi-IM-ResourceSetToAddModList Pool of CSI-IM-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs

csi-IM-ResourceToAddModList Pool of CSI-IM-Resource which can be referred to from CSI-IM-ResourceSet

csi-ReportConfigToAddModList Configured CSI report settings as specified in TS 38.214 section 5.2.1.1

csi-ResourceConfigToAddModList Configured CSI resource settings as specified in TS 38.214 section 5.2.1.2

csi-SSB-ResourceSetToAddModList Pool of CSI-SSB-ResourceSet which can be referred to from CSI-ResourceConfig

nzp-CSI-RS-ResourceSetToAddModList Pool of NZP-CSI-RS-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs

nzp-CSI-RS-ResourceToAddModList Pool of NZP-CSI-RS-Resource which can be referred to from NZP-CSI-RS-ResourceSet

reportTriggerSize Size of CSI request field in DCI (bits). Corresponds to L1 parameter 'ReportTriggerSize' (see 38.214, section 5.2)

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See 38.214, section 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=				SEQUENCE {

reportConfigId							CSI-ReportConfigId,

carrier									ServCellIndex					OPTIONAL,	-- Need S

resourcesForChannelMeasurement			CSI-ResourceConfigId,

csi-IM-ResourcesForInterference			CSI-ResourceConfigId		OPTIONAL,	-- Need R

nzp-CSI-RS-ResourcesForInterference		CSI-ResourceConfigId		OPTIONAL, 	-- Need R

reportConfigType						CHOICE {

periodic								SEQUENCE {

reportSlotConfig						CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH					SEQUENCE {

reportSlotConfig						CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH					SEQUENCE {

reportSlotConfig						ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList				SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha									P0-PUSCH-AlphaSetId

},

aperiodic								SEQUENCE {

reportSlotOffsetList				SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity							CHOICE {

none									NULL,

cri-RI-PMI-CQI							NULL,

cri-RI-i1								NULL,

cri-RI-i1-CQI							SEQUENCE {

pdsch-BundleSizeForCSI					ENUMERATED {n2, n4}		OPTIONAL 	-- Need S

},

cri-RI-CQI								NULL,

cri-RSRP								NULL,

ssb-Index-RSRP							NULL,

cri-RI-LI-PMI-CQI						NULL

},

reportFreqConfiguration					 SEQUENCE {

cqi-FormatIndicator						ENUMERATED { widebandCQI, subbandCQI }										OPTIONAL, 	-- Need R

pmi-FormatIndicator						ENUMERATED { widebandPMI, subbandPMI }										OPTIONAL, 	-- Need R

csi-ReportingBand						CHOICE {

subbands3								BIT STRING(SIZE(3)),

subbands4								BIT STRING(SIZE(4)),

subbands5								BIT STRING(SIZE(5)),

subbands6								BIT STRING(SIZE(6)),

subbands7								BIT STRING(SIZE(7)),

subbands8								BIT STRING(SIZE(8)),

subbands9								BIT STRING(SIZE(9)),

subbands10								BIT STRING(SIZE(10)),

subbands11								BIT STRING(SIZE(11)),

subbands12								BIT STRING(SIZE(12)),

subbands13								BIT STRING(SIZE(13)),

subbands14								BIT STRING(SIZE(14)),

subbands15								BIT STRING(SIZE(15)),

subbands16								BIT STRING(SIZE(16)),

subbands17								BIT STRING(SIZE(17)),

subbands18								BIT STRING(SIZE(18)),

...,

[[

subbands19-v15xy						BIT STRING(SIZE(19))

]]

}	OPTIONAL	-- Need S

}																														OPTIONAL, 	-- Need R

timeRestrictionForChannelMeasurements				ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements			ENUMERATED {configured, notConfigured},

codebookConfig								CodebookConfig																OPTIONAL, 	-- Need R

nrofCQIsPerReport							ENUMERATED {n1, n2}															OPTIONAL, 	-- Need R

groupBasedBeamReporting						CHOICE {

enabled										NULL,

disabled 									SEQUENCE {

nrofReportedRS							ENUMERATED {n1, n2, n3, n4}											OPTIONAL	-- Need S

}

},

cqi-Table								ENUMERATED {table1, table2, table3, spare1}									OPTIONAL, 	-- Need R

subbandSize								ENUMERATED {value1, value2},

non-PMI-PortIndication					SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks	OPTIONAL,	-- Need R

...

}

CSI-ReportPeriodicityAndOffset ::=	CHOICE {

slots4								INTEGER(0..3),

slots5								INTEGER(0..4),

slots8								INTEGER(0..7),

slots10								INTEGER(0..9),

slots16								INTEGER(0..15),

slots20								INTEGER(0..19),

slots40								INTEGER(0..39),

slots80								INTEGER(0..79),

slots160							INTEGER(0..159),

slots320							INTEGER(0..319)

}

PUCCH-CSI-Resource ::= 				SEQUENCE {

uplinkBandwidthPartId				BWP-Id,

pucch-Resource						PUCCH-ResourceId

}

PortIndexFor8Ranks ::=				CHOICE {

portIndex8							SEQUENCE{

rank1-8								PortIndex8															OPTIONAL,	-- Need R

rank2-8								SEQUENCE(SIZE(2)) OF PortIndex8									OPTIONAL, 	-- Need R

rank3-8								SEQUENCE(SIZE(3)) OF PortIndex8									OPTIONAL, 	-- Need R

rank4-8								SEQUENCE(SIZE(4)) OF PortIndex8									OPTIONAL, 	-- Need R

rank5-8								SEQUENCE(SIZE(5)) OF PortIndex8									OPTIONAL, 	-- Need R

rank6-8								SEQUENCE(SIZE(6)) OF PortIndex8									OPTIONAL, 	-- Need R

rank7-8								SEQUENCE(SIZE(7)) OF PortIndex8									OPTIONAL, 	-- Need R

rank8-8								SEQUENCE(SIZE(8)) OF PortIndex8									OPTIONAL	-- Need R

},

portIndex4							SEQUENCE{

rank1-4								PortIndex4															OPTIONAL, 	-- Need R

rank2-4								SEQUENCE(SIZE(2)) OF PortIndex4									OPTIONAL, 	-- Need R

rank3-4								SEQUENCE(SIZE(3)) OF PortIndex4									OPTIONAL, 	-- Need R

rank4-4								SEQUENCE(SIZE(4)) OF PortIndex4									OPTIONAL	-- Need R

},

portIndex2							SEQUENCE{

rank1-2								PortIndex2															OPTIONAL, 	-- Need R

rank2-2								SEQUENCE(SIZE(2)) OF PortIndex2									OPTIONAL	-- Need R

},

portIndex1							NULL

}

PortIndex8::=						INTEGER (0..7)

PortIndex4::=						INTEGER (0..3)

PortIndex2::=						INTEGER (0..1)

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

CSI-ReportConfig field descriptions

carrier Indicates in which serving cell the CSI-ResourceConfig indicated below are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

codebookConfig Codebook configuration for Type-1 or Type-II including codebook subset restriction

cqi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI. (see 38.214, section 5.2.1.4)

cqi-Table Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-table' (see 38.214, section 5.2.2.1)

csi-IM-ResourcesForInterference CSI IM resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only CSI-IM resources. The bwp-Id in that CSI-ResourceConfig is the same value like the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

csi-ReportingBand Indicates a contiguous or non-contigous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see 38.214, section 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise, the number of sub bands can be from 3 (24 PRBs, sub band size 8) to 18 (72 PRBs, sub band size 4).

groupBasedBeamReporting Turning on/off group beam based reporting (see 38.214, section 5.2.1.4)

non-PMI-PortIndication Port indication for RI/CQI calculation. For each CSI-RS resource in the linked ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback. Corresponds to L1 parameter 'Non-PMI-PortIndication' (see 38.214, section FFS_Section). The first entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the CSI-ResourceConfig whose CSI-ResourceConfigId is indicated in a CSI-MeasId together with the above CSI-ReportConfigId; the second entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the second entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig, and so on until the NZP-CSI-RS-Resource indicated by the last entry in nzp-CSI-RS-Resources in the in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig. Then the next entry corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the second entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig and so on.

nrofCQIsPerReport Maximum number of CQIs per CSI report (cf. 1 for 1-CW, 2 for 2-CW)

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. FFS: The signaling mechanism for the gNB to select a subset of N beams for the UE to measure and report. FFS: Note: this parameter may not be needed for certain resource and/or report settings FFS_ASN1: Change groupBasedBeamReporting into a CHOICE and include this field into the "no" option? (see 38.214, section FFS_Section) When the field is absent the UE applies the value 1

nzp-CSI-RS-ResourcesForInterference NZP CSI RS resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources. The bwp-Id in that CSI-ResourceConfig is the same value like the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

p0alpha Index of the p0-alpha set determining the power control for this CSI report transmission. Corresponds to L1 parameter 'SPCSI-p0alpha' (see 38.214, section FFS_Section)

pdsch-BundleSizeForCSI PRB bundling size to assume for CQI calcuation when reportQuantity is CRI/RI/i1/CQI. If the field is absent, the UE assumes that no PRB bundling is applied. Corresponds to L1 parameter 'PDSCH-bundle-size-for-CSI' (see 38.214, section 5.2.1.4)

pmi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see 38.214, section 5.2.1.4)

pucch-CSI-ResourceList Indicates which PUCCH resource to use for reporting on PUCCH.

reportConfigType Time domain behavior of reporting configuration

reportFreqConfiguration Reporting configuration in the frequency domain. (see 38.214, section 5.2.1.4)

reportQuantity The CSI related quanities to report. Corresponds to L1 parameter 'ReportQuantity' (see 38.214, section REF)

reportSlotConfig Periodicity and slot offset. Corresponds to L1 parameter 'ReportPeriodicity'and 'ReportSlotOffset' (see 38.214, section section 5.2.1.4) as well as to L1 parameter 'Reportperiodicity-spCSI'. (see 38.214, section 5.2.1.1?FFS_Section)

reportSlotOffsetList Timing offset Y for semi persistent reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on. The first report is transmitted in slot n+Y, second report in n+Y+P, where P is the configured periodicity. Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on (see 38.214, section 5.2.3).

resourcesForChannelMeasurement Resources for channel measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources. This CSI-ReportConfig is associated with the DL BWP indicated by bwp-Id in that CSI-ResourceConfig.

subbandSize Indicates one out of two possible BWP-dependent values for the subband size as indicated in 38.214 table 5.2.1.4-2 Corresponds to L1 parameter 'SubbandSize' (see 38.214, section 5.2.1.4)

timeRestrictionForChannelMeasurements Time domain measurement restriction for the channel (signal) measurements. Corresponds to L1 parameter 'MeasRestrictionConfig-time-channel' (see 38.214, section 5.2.1.1)

timeRestrictionForInterferenceMeasurements Time domain measurement restriction for interference measurements. Corresponds to L1 parameter 'MeasRestrictionConfig-time-interference' (see 38.214, section 5.2.1.1)

PortIndexFor8Ranks field descriptions

portIndex8 Port-Index configuration for up to rank 8. If present, the network configures port indexes for at least one of the ranks.

portIndex4 Port-Index configuration for up to rank 4. If present, the network configures port indexes for at least one of the ranks.

portIndex2 Port-Index configuration for up to rank 2. If present, the network configures port indexes for at least one of the ranks.

portIndex1 Port-Index configuration for rank 1.

PUCCH-CSI-Resource field descriptions

pucch-Resource PUCCH resource for the associated uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID.

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::= 				INTEGER (0..maxNrofCSI-ReportConfigurations-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::= 				SEQUENCE {

csi-ResourceConfigId				CSI-ResourceConfigId,

csi-RS-ResourceSetList 	 			CHOICE {

nzp-CSI-RS-SSB 						SEQUENCE {

nzp-CSI-RS-ResourceSetList			SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId OPTIONAL,	-- Need R

csi-SSB-ResourceSetList				SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId	OPTIONAL	-- Need R

},

csi-IM-ResourceSetList				SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id								BWP-Id,

resourceType						ENUMERATED { aperiodic, semiPersistent, periodic },

...

}

-- TAG-CSI-RESOURCECONFIGTOADDMOD-STOP

-- ASN1STOP

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in. Corresponds to L1 parameter 'BWP-Info' (see 38.214, section 5.2.1.2

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig

csi-RS-ResourceSetList Contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if ResourceConfigType is 'aperiodic' and 1 otherwise. Corresponds to L1 parameter 'ResourceSetConfigList' (see 38.214, section 5.2.1.3.1)

csi-SSB-ResourceSetList List of SSB resources used for beam measurement and reporting in a resource set Corresponds to L1 parameter 'resource-config-SS-list' (see 38,214, section FFS_Section)

resourceType Time domain behavior of resource configuration. Corresponds to L1 parameter 'ResourceConfigType' (see 38.214, section 5.2.2.3.5). It does not apply to resources provided in the csi-SSB-ResourceSetList.

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::= 			INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, slots5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=	CHOICE {

slots4								INTEGER (0..3),

slots5								INTEGER (0..4),

slots8								INTEGER (0..7),

slots10								INTEGER (0..9),

slots16								INTEGER (0..15),

slots20								INTEGER (0..19),

slots32								INTEGER (0..31),

slots40								INTEGER (0..39),

slots64								INTEGER (0..63),

slots80								INTEGER (0..79),

slots160							INTEGER (0..159),

slots320							INTEGER (0..319),

slots640							INTEGER (0..639)

}

-- TAG-CSI-RESIYRCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CSI-RS-ResourceConfigMobility

The IE CSI-RS-ResourceConfigMobility is used to configure CSI-RS based RRM measurements.

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::= 	SEQUENCE {

subcarrierSpacing					SubcarrierSpacing,

csi-RS-CellList-Mobility 			SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM))	OF CSI-RS-CellMobility,

... ,

[[refServCellIndex					ServCellIndex															OPTIONAL	-- Need S

]]

}

CSI-RS-CellMobility ::=				SEQUENCE {

cellId								PhysCellId,

csi-rs-MeasurementBW				SEQUENCE {

nrofPRBs							ENUMERATED { size24, size48, size96, size192, size264},

startPRB							INTEGER(0..2169)

},

density								ENUMERATED {d1,d3}												OPTIONAL,	-- Need R

csi-rs-ResourceList-Mobility 		SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM))	OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=		SEQUENCE {

csi-RS-Index						CSI-RS-Index,

slotConfig							CHOICE {

ms4									INTEGER (0..31),

ms5									INTEGER (0..39),

ms10								INTEGER (0..79),

ms20								INTEGER (0..159),

ms40								INTEGER (0..319)

},

associatedSSB						SEQUENCE {

ssb-Index							SSB-Index,

isQuasiColocated					BOOLEAN

}																										OPTIONAL, -- Need R

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain			INTEGER (0..13),

sequenceGenerationConfig			INTEGER (0..1023),

...

}

CSI-RS-Index ::= 					INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per frequency layer depends on the configuration of associatedSSB (see 38.214, section 5.1.6.1.3).

density Frequency domain density for the 1-port CSI-RS for L3 mobility Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section).

nrofPRBs Allowed size of the measurement BW in PRBs Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section).

startPRB Starting PRB index of the measurement bandwidth Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section) FFS_Value: Upper edge of value range unclear in RAN1.

CSI-RS-ResourceConfigMobility field descriptions

csi-RS-CellList-Mobility List of cells

refServCellIndex Indicates the serving cell providing the timing reference for CSI-RS resources without associatedSSB. The field may be present only if there’s at least one CSI-RS resource configured without associatedSSB. In case there’s at least one CSI-RS resource configured without associatedSSBand this field is absent, the UE shall use the timing of the PCell. The CSI-RS resources and the serving cell indicated by refServCellIndex for timing reference should be located in the same band.

subcarrierSpacing Subcarrier spacing of CSI-RS. Only the values 15, 30 or 60 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. Corresponds to L1 parameter 'Numerology' (see 38.211, section FFS_Section).

CSI-RS-Resource-Mobility field descriptions

associatedSSB If this field is present, the UE may base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the cell indicated by the cellId in the CSI-RS-CellMobility. In this case,the UE is not required to monitor that CSI-RS resource if the UE can’t detect the SS/PBCH block indicated by this associatedSSB and cellId. If this field is absent, the UE shall base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobilityon the timing of the serving cell. In this case, the UE is required to measure the CSI-RS resource even if SS/PBCH block(s) with cellIdin the CSI-RS-CellMobilityare not detected. CSI-RS resources with and without associatedSSB may be configured in accordance with the rules in 38.214, section 5.1.6.1.3.

csi-RS-Index CSI-RS resource index associated to the CSI-RS resource to be measured (and used for reporting).

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. Parameter l0 in 38.211, section 7.4.1.5.3. Value 2 is supported only when DL-DMRS-typeA-pos equals 3.

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with 38.211, section 7.4.1.5.3 including table 7.4.1.5.2-1. The number of bits that may be set to one depend on the chosen row in that table. For the choice "other", the row can be determined from the parmeters below and from the number of bits set to 1 in frequencyDomainAllocation.

isQuasiColocated The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters Corresponds to L1 parameter 'QCLed-SSB' (see FFS_Spec, section FFS_Section).

sequenceGenerationConfig Scrambling ID for CSI-RS (see 38.211, section 7.4.1.5.2).

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacingCSI-RS is set to 15kHZ, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 3/4/9/19/39 slots. When subcarrierSpacingCSI-RS is set to 30kHZ, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 7/9/19/39/79 slots. When subcarrierSpacingCSI-RS is set to 60kHZ, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 15/19/39/79/159 slots. When subcarrierSpacingCSI-RS is set 120kHZ, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 31/39/79/159/319 slots.

#### –	CSI-RS-ResourceMapping

The IE CSI-RS-ResourceMapping is used to configure the resource element mapping of a CSI-RS resource in time- and frequency domain.

CSI-RS-ResourceMapping information element

-- ASN1START

-- TAG-CSI-RS-RESOURCEMAPPING-START

CSI-RS-ResourceMapping ::= 			SEQUENCE {

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12)),

row4								BIT STRING (SIZE (3)),

other								BIT STRING (SIZE (6))

},

nrofPorts							ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain			INTEGER (0..13),

firstOFDMSymbolInTimeDomain2		INTEGER (2..12)																OPTIONAL,	-- Need R

cdm-Type							ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density								CHOICE {

dot5								ENUMERATED {evenPRBs, oddPRBs},

one									NULL,

three								NULL,

spare								NULL

},

freqBand							CSI-FrequencyOccupation,

...

}

-- TAG-CSI-RS-RESOURCEMAPPING-STOP

-- ASN1STOP

CSI-RS-ResourceMapping field descriptions

cdm-Type CDM type (see 38.214, section 5.2.2.3.1)

density Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density' (see 38.211, section 7.4.1.5.3). Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, value 1 (one) is allowed for X=4, 8, 12. For density = 1/2, includes 1 bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS.

firstOFDMSymbolInTimeDomain2 Time domain allocation within a physical resource block. Parameter l1 in 38.211, section 7.4.1.5.3.

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. Parameter l0 in 38.211, section 7.4.1.5.3. Value 2 is supported only when DL-DMRS-typeA-pos equals 3.

freqBand Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1)

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with 38.211, section 7.4.1.5.3. The applicable row number in table 7.4.1.5.3-1 is determined by the frequencyDomainAllocation for rows 1, 2 and 4, and for other rows by matching the values in the column Ports, Density and CDMtype in table 7.4.1.5.3-1 with the values of nrofPorts, cdm-Type and density below and, when more than one column has the 3 values matching, by selecting the row where the column (k bar, l bar) in table 7.4.1.5.3-2 has indexes for k ranging from 0 to 2*n-1 where n is the number of bits set to 1 in frequencyDomainAllocation.

nrofPorts Number of ports (see 38.214, section 5.2.2.3.1)

#### –	CSI-SemiPersistentOnPUSCH-TriggerStateList

The CSI-SemiPersistentOnPUSCH-TriggerStateList IE is used to configure the UE with list of trigger states for semi-persistent reporting of channel state information on L1.  . See also 38.214, section 5.2.

CSI-SemiPersistentOnPUSCH-TriggerStateList information element

-- ASN1START

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-START

CSI-SemiPersistentOnPUSCH-TriggerStateList ::=	SEQUENCE(SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-SemiPersistentOnPUSCH-TriggerState

CSI-SemiPersistentOnPUSCH-TriggerState ::=		SEQUENCE {

associatedReportConfigInfo						CSI-ReportConfigId,

...

}

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-STOP

-- ASN1STOP

#### –	CSI-SSB-ResourceSetId

The IE CSI-SSB-ResourceSetId is used to identify one SS/PBCH block resource set.

CSI-SSB-ResourceId information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESETID-START

CSI-SSB-ResourceSetId ::=			INTEGER (0..maxNrofCSI-SSB-ResourceSets-1)

-- TAG-CSI-SSB-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-SSB-ResourceSet

The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon.

CSI-SSB-ResourceSet information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESET-START

CSI-SSB-ResourceSet ::=				SEQUENCE {

csi-SSB-ResourceSetId				CSI-SSB-ResourceSetId,

csi-SSB-ResourceList				SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...

}

-- TAG-CSI-SSB-RESOURCESET-STOP

-- ASN1STOP

#### –	DedicatedInfoNAS

The IE DedicatedInfoNAS is used to transfer UE specific NAS layer information between the 5GC CN and the UE. The RRC layer is transparent for this information.

DedicatedInfoNASinformation element

-- ASN1START

-- TAG-DEDICATED-INFO-NAS-START

DedicatedInfoNAS ::=		OCTET STRING

-- TAG-DEDICATED-INFO-NAS-STOP

-- ASN1STOP

#### –	DMRS-DownlinkConfig

The IE DMRS-DownlinkConfig is used to configure downlink demodulation reference signals for PDSCH.

DMRS-DownlinkConfig information element

-- ASN1START

-- TAG-DMRS-DOWNLINKCONFIG-START

DMRS-DownlinkConfig ::=				SEQUENCE {

dmrs-Type							ENUMERATED {type2}														OPTIONAL,	-- Need S

dmrs-AdditionalPosition				ENUMERATED {pos0, pos1, pos3}											OPTIONAL,	-- Need R

maxLength							ENUMERATED {len2}														OPTIONAL, 	-- Need S

scramblingID0						INTEGER (0..65535)														OPTIONAL, 	-- Need S

scramblingID1						INTEGER (0..65535)														OPTIONAL, 	-- Need S

phaseTrackingRS						SetupRelease { PTRS-DownlinkConfig	}									OPTIONAL,	-- Need M

...

}

-- TAG-DMRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

DMRS-DownlinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. If the field is absent, the UE applies the value pos2.

dmrs-Type Selection of the DMRS type to be used for DL (see 38.211, section 7.4.1.1.1). If the field is absent, the UE uses DMRS type 1.

maxLength The maximum number of OFDM symbols for DL front loaded DMRS. 'len1' corresponds to value 1. 'len2 corresponds to value 2. If the field is absent, the UE applies value len1. Corresponds to L1 parameter 'DL-DMRS-max-len' (see 38.214, section 5.1)

phaseTrackingRS Configures downlink PTRS. If absent of released, the UE assumes that downlink PTRS are not present. See 38.214 section 5.1.6.3

scramblingID0 DL DMRS scrambling initalization Corresponds to L1 parameter 'n_SCID 0' (see 38.211, section 7.4.1). When the field is absent the UE applies the value Physical cell ID (physCellId) configured for this serving cell."

scramblingID1 DL DMRS scrambling initalization. Corresponds to L1 parameter 'n_SCID 1' (see 38.211, section 7.4.1). When the field is absent the UE applies the value (physCellId) configured for this serving cell.

#### –	DMRS-UplinkConfig

The IE DMRS-UplinkConfig is used to configure  uplink demodulation reference signals for PUSCH.

DMRS-UplinkConfig information element

-- ASN1START

-- TAG-DMRS-UPLINKCONFIG-START

DMRS-UplinkConfig ::=				SEQUENCE {

dmrs-Type							ENUMERATED {type2} 													OPTIONAL,	-- Need S

dmrs-AdditionalPosition				ENUMERATED {pos0, pos1, pos3}										OPTIONAL,	-- Need R

phaseTrackingRS						SetupRelease { PTRS-UplinkConfig }									OPTIONAL,	-- Need M

maxLength							ENUMERATED {len2}													OPTIONAL, 	-- Need S

transformPrecodingDisabled							SEQUENCE {

scramblingID0						INTEGER (0..65535)												OPTIONAL,	-- Need S

scramblingID1						INTEGER (0..65535)												OPTIONAL,	-- Need S

...

}																										OPTIONAL, 	-- Need R

transformPrecodingEnabled								SEQUENCE {

nPUSCH-Identity						INTEGER(0..1007)												OPTIONAL,	-- Need S

sequenceGroupHopping			ENUMERATED {disabled}											OPTIONAL,	-- Need S

sequenceHopping				ENUMERATED {enabled}											OPTIONAL,	-- Need S

...

}																										OPTIONAL, 	-- Need R

...

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

DMRS-UplinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in UL. Corresponds to L1 parameter 'UL-DMRS-add-pos' (see Table 7.4.1.1.2-4 in 38.211) The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for UL. If the field is absent, the UE applies the value pos2.

dmrs-Type Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.3) If the field is absent, the UE uses DMRS type 1.

maxLength The maximum number of OFDM symbols for UL front loaded DMRS. 'len1' corresponds to value 1. 'len2 corresponds to value 2. If the field is absent, the UE applies value len1. Corresponds to L1 parameter 'UL-DMRS-max-len' (see 38.214, section 6.4.1.1.2)

nPUSCH-Identity Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS. If the value is absent or released, the UE uses the Physical cell ID. Corresponds to L1 parameter 'nPUSCH-Identity-Transform precoding' (see 38.211, section FFS_Section)

phaseTrackingRS Configures uplink PTRS (see 38.211, section x.x.x.x) FFS_Ref

scramblingID0 UL DMRS scrambling initalization for CP-OFDM Corresponds to L1 parameter 'n_SCID 0' (see 38.214, section 6.4.1.1.2) When the field is absent the UE applies the value Physical cell ID (physCellId)

scramblingID1 UL DMRS scrambling initalization for CP-OFDM. Corresponds to L1 parameter 'n_SCID 1' (see 38.214, section 6.4.1.1.2) When the field is absent the UE applies the value Physical cell ID (physCellId)

sequenceGroupHopping For DMRS transmission with transform precoder the NW may configure sequence-group hopping by the cell-specific parameter groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon. In this case, the NW may include this UE specific field to disable sequence group hopping, i.e., to override the configuration in PUSCH-ConfigCommon (see 38.211)

sequenceHopping Determines if sequence hopping is enabled for DMRS transmission with transform precoder. If the field is absent, the UE considers sequence hopping to be disabled. Corresponds to L1 parameter 'Sequence-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section)

transformPrecodingDisabled DMRS related parameters for Cyclic Prefix OFDM

transformPrecodingEnabled DMRS related parameters for DFT-s-OFDM (Transform Precoding)

#### –	DownlinkConfigCommon

The IE DownlinkConfigCommon provides common downlink parameters of a cell.

DownlinkConfigCommon information element

-- ASN1START

-- TAG-DOWNLINK-CONFIG-COMMON-START

DownlinkConfigCommon ::=		SEQUENCE {

frequencyInfoDL					FrequencyInfoDL									OPTIONAL,	-- Cond InterFreqHOAndServCellAdd

initialDownlinkBWP				BWP-DownlinkCommon								OPTIONAL,	-- Cond ServCellAdd

...

}

-- TAG-DOWNLINK-CONFIG-COMMON-STOP

-- ASN1STOP

DownlinkConfigCommon field descriptions

frequencyInfoDL Basic parameters of a downlink carrier and transmission thereon

initialDownlinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover, and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell). It is optionally present, Need M otherwise. |

#### –	DownlinkConfigCommonSIB

The IE DownlinConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-START

DownlinkConfigCommonSIB ::=		SEQUENCE {

frequencyInfoDL					FrequencyInfoDL-SIB,

initialDownlinkBWP				BWP-DownlinkCommon,

bcch-Config 						BCCH-Config,

pcch-Config 						PCCH-Config,

...

}

BCCH-Config ::=					SEQUENCE {

modificationPeriodCoeff			ENUMERATED {n2, n4, n8, n16}

}

PCCH-Config ::=					SEQUENCE {

defaultPagingCycle					ENUMERATED {

rf32, rf64, rf128, rf256},

n									ENUMERATED {

halfT, quarterT, oneEighthT,

oneSixteenthT},

ns									ENUMERATED {four, two, one},

pf-Offset							INTEGER (0..255)

}

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon

initialDownlinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

bcch-Config The modification period related configuration.

pcch-Config The paging related configuration.

PCCH-Config field descriptions

defaultPagingCycle Default paging cycle, used to derive ‘T’ in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

N Number of total paging frames in T

Ns Number of paging occasions in paging frame

PF-Offset Paging frame offset, corresponding to parameter PF_offset in TS 38.304 [20].

#### –	DownlinkPreemption

The IE DownlinkPreemption is used to configure the UE to monitor PDCCH for the INT-RNTI (interruption).

DownlinkPreemption information element

-- ASN1START

-- TAG-DOWNLINKPREEMPTION-START

DownlinkPreemption ::=				SEQUENCE {

int-RNTI							RNTI-Value,

timeFrequencySet					ENUMERATED {set0, set1},

dci-PayloadSize						INTEGER (0..maxINT-DCI-PayloadSize),

int-ConfigurationPerServingCell		SEQUENCE (SIZE (1..maxNrofServingCells)) OF INT-ConfigurationPerServingCell,

...

}

INT-ConfigurationPerServingCell ::= SEQUENCE {

servingCellId						ServCellIndex,

positionInDCI						INTEGER (0..maxINT-DCI-PayloadSize-1)

}

-- TAG-DOWNLINKPREEMPTION-STOP

-- ASN1STOP

DownlinkPreemption field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with INT-RNTI. Corresponds to L1 parameter 'INT-DCI-payload-length' (see 38.213, section 11.2)

int-ConfigurationPerServingCell Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload. Corresponds to L1 parameter 'INT-cell-to-INT' and 'cell-to-INT' (see 38.213, section 11.2)

int-RNTI RNTI used for indication pre-emption in DL. Corresponds to L1 parameter 'INT-RNTI', where ”INT” stands for ”interruption” (see 38.213, section 10)

timeFrequencySet Set selection for DL-preemption indication. Corresponds to L1 parameter 'int-TF-unit' (see 38.213, section 10.1) The set determines how the UE interprets the DL preemption DCI payload.

INT-ConfigurationPerServingCell field descriptions

positionInDCI Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI payload. Must be multiples of 14 (bit). Corresponds to L1 parameter 'INT-values' (see 38.213, section 11.2)

#### –	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

DRB-Identity information elements

-- ASN1START

-- TAG-DRB-IDENTITY-START

DRB-Identity ::=					INTEGER (1..32)

-- TAG-DRB-IDENTITY-STOP

-- ASN1STOP

#### –	EUTRA-MBSFN-SubframeConfigList

The IE EUTRA-MBSFN-SubframeConfigList is used to define an E-UTRA MBSFN subframe pattern (for the purpose of NR rate matching).

EUTRA-MBSFN-SubframeConfigList information element

-- ASN1START

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-START

EUTRA-MBSFN-SubframeConfigList ::= 	SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF EUTRA-MBSFN-SubframeConfig

EUTRA-MBSFN-SubframeConfig ::=		SEQUENCE {

radioframeAllocationPeriod			ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset			INTEGER (0..7),

subframeAllocation					CHOICE {

oneFrame							BIT STRING (SIZE(6)),

fourFrames							BIT STRING (SIZE(24))

},

subframeAllocation-v1430			CHOICE {

oneFrame-v1430						BIT STRING (SIZE(2)),

fourFrames-v1430					BIT STRING (SIZE(8))

}																														OPTIONAL,	-- Need R

...

}

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-STOP

-- ASN1STOP

EUTRA-MBSFN-SubframeConfig field descriptions

fourFrames-v1430 Field as defined in MBSFN-SubframeConfig in 36.331

fourFrames Field as defined in MBSFN-SubframeConfig in 36.331

oneFrame-v1430 Field as defined in MBSFN-SubframeConfig in 36.331

oneFrame Field as defined in MBSFN-SubframeConfig in 36.331

radioframeAllocationOffset Field as defined in MBSFN-SubframeConfig in 36.331

radioframeAllocationPeriod Field as defined in MBSFN-SubframeConfig in 36.331

subframeAllocation Field as defined in MBSFN-SubframeConfig in 36.331

#### –	EUTRA-Q-OffsetRange

The IE EUTRA-Q-OffsetRange is used to indicate a cell, or frequency specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

EUTRA-Q-OffsetRange information element

-- ASN1START

EUTRA-Q-OffsetRange ::=						ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- ASN1STOP

#### –	FilterCoefficient

The IE FilterCoefficient specifies the measurement filtering coefficient. Value fc0 corresponds to k = 0, fc1 corresponds to k = 1, and so on.

FilterCoefficient information element

-- ASN1START

-- TAG-FILTERCOEFFICIENT-START

FilterCoefficient ::=				ENUMERATED { fc0, fc1, fc2, fc3, fc4, fc5, fc6, fc7, fc8, fc9, fc11, fc13, fc15, fc17, fc19, spare1, ...}

-- TAG-FILTERCOEFFICIENT-STOP

-- ASN1STOP

Editor’s Note: Values should be checked.

#### –	FreqBandIndicatorNR

The IE FreqBandIndicatorNR is used to convey an NR frequency band number as defined in 38.101.

FreqBandIndicatorNR information element

-- ASN1START

-- TAG-FREQBANDINDICATORNR-START

FreqBandIndicatorNR ::=     		INTEGER (1..1024)

-- TAG-FREQBANDINDICATORNR-STOP

-- ASN1STOP

#### –	FrequencyInfoDL

The IE FrequencyInfoDL provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCY-INFO-DL-START

FrequencyInfoDL ::= 				SEQUENCE {

absoluteFrequencySSB				ARFCN-ValueNR																OPTIONAL,	-- Cond SpCellAdd

frequencyBandList					MultiFrequencyBandListNR,

absoluteFrequencyPointA				ARFCN-ValueNR,

scs-SpecificCarrierList				SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...

}

-- TAG-FREQUENCY-INFO-UL-STOP

-- ASN1STOP

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList. Corresponds to L1 parameter 'offset-ref-low-scs-ref-PRB' (see 38.211, section FFS_Section)

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The frequency provided in this field identifies the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see 38.101). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell. This is only supported in case the Scell is in the same frequency band as the SpCell.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. Multiple values are only supported in system information but not when the FrequencyInfoDL is provided in dedicated signalling (HO or S(p)Cell addition).

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need R. |

#### –	FrequencyInfoDL-SIB

The IE FrequencyInfoDL-SIB provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL-SIB information element

-- ASN1START

-- TAG-FREQUENCY-INFO-DL-SIB-START

FrequencyInfoDL-SIB ::= 				SEQUENCE {

frequencyBandList					MultiFrequencyBandListNR,

offsetToPointA						INTEGER (0..2199)				,

scs-SpecificCarrierList				SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier

}

-- TAG-FREQUENCY-INFO-DL-SIB-STOP

-- ASN1STOP

FrequencyInfoDL-SIB field descriptions

offsetToPointA The offset in PRB between the Point A and the lowest subcarrier of the lowest PRB of the cell-defining SSB after floating SSB is resolved [FFS Ref]

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. Multiple values are only supported in system information but not when the FrequencyInfoDL is provided in dedicated signalling (HO or S(p)Cell addition).

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCY-INFO-UL-START

FrequencyInfoUL ::= 				SEQUENCE {

frequencyBandList					MultiFrequencyBandListNR										OPTIONAL,	-- Cond FDD-OrSUL

absoluteFrequencyPointA				ARFCN-ValueNR													OPTIONAL,	-- Cond FDD-OrSUL

scs-SpecificCarrierList				SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission			AdditionalSpectrumEmission										OPTIONAL,	-- Need S

p-Max								P-Max															OPTIONAL,	-- Need S

frequencyShift7p5khz				ENUMERATED {true}												OPTIONAL,	-- Cond FDD-OrSUL-Optional

...

}

-- TAG-FREQUENCY-INFO-UL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList. Corresponds to L1 parameter 'offset-ref-low-scs-ref-PRB' (see 38.211, section FFS_Section)

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE applies the value FFS_RAN4. (see FFS_section, section FFS_Section)

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. Multiple values are only supported in system information but not when the FrequencyInfoDL is provided in dedicated signalling (HO or S(p)Cell addition).

frequencyShift7p5khz Enable the NR UL transmission with a 7.5KHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. If absent, the UE applies the maximum power according to TS 38.101 [15].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise. |

#### –	Hysteresis

The IE Hysteresis is a parameter used within the entry and leave condition of an event triggered reporting condition. The actual value is field value * 0.5 dB.

Hysteresis information element

-- ASN1START

Hysteresis ::=						INTEGER (0..30)

-- ASN1STOP

Editor’s Note: Values should be checked.

#### –	I-RNTI-Value

The I-RNTI-Value IE is used to identify the suspended UE context of a UE in RRC_INACTIVE.

I-RNTI-Value information element

-- ASN1START

-- TAG-I-RNTI-VALUE-START

I-RNTI-Value ::=						BIT STRING (SIZE(40))

-- TAG-I-RNTI-VALUE-STOP

-- ASN1STOP

#### –	LocationMeasurementInfo

The IE LocationMeasurementInfo defines the information sent by the UE to the network to assist with the configuration of measurement gaps for location related measurements.

-- ASN1START

-- TAG-LOCATION-MEASUREMENT-INFO-START

LocationMeasurementInfo ::=		CHOICE {

eutra-RSTD						EUTRA-RSTD-InfoList,

...

}

EUTRA-RSTD-InfoList ::= SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info

EUTRA-RSTD-Info ::= SEQUENCE {

carrierFreq					ARFCN-ValueEUTRA,

measPRS-Offset				INTEGER (0..39),

...

}

-- TAG-LOCATION-MEASUREMENT-INFO-STOP

-- ASN1STOP

LocationMeasurementInfo field descriptions

carrierFreq The EARFCN value of the carrier received from upper layers for which the UE needs to perform the inter-RAT RSTD measurements.

measPRS-Offset Indicates the requested gap offset for performing RSTD measurements towards E-UTRA. It is the smallest subframe offset from the beginning of subframe 0 of SFN=0 of the serving cell of the requested gap for measuring PRS positioning occasions in the carrier frequency carrierFreq for which the UE needs to perform the inter-RAT RSTD measurements. The PRS positioning occasion information is received from upper layers. The value of measPRS-Offset is obtained by mapping the starting subframe of the PRS positioning occasion in the measured cell onto the corresponding subframe in the serving cell and is calculated as the serving cell’s number of subframes from SFN=0 mod 40. The UE shall take into account any additional time required by the UE to start PRS measurements on the other carrier when it does this mapping for determining the measPRS-Offset. NOTE: Figure 6.2.2-1 in TS 36.331[10] illustrates the measPRS-Offset field.

#### -	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICAL-CHANNEL-CONFIG-START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (1..16),

prioritisedBitRate					ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration					ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells					SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex					OPTIONAL,	-- Need R

allowedSCS-List						SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing							OPTIONAL,	-- Need R

maxPUSCH-Duration					ENUMERATED { ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1 }

OPTIONAL,	-- Need R

configuredGrantType1Allowed			ENUMERATED {true}															OPTIONAL,	-- Need R

logicalChannelGroup					INTEGER (0..maxLCG-ID)														OPTIONAL, 	-- Need R

schedulingRequestID					SchedulingRequestId															OPTIONAL,	-- Need R

logicalChannelSR-Mask				BOOLEAN,

logicalChannelSR-DelayTimerApplied	BOOLEAN,

...

}																													OPTIONAL,	-- Cond UL

...

}

-- TAG-LOGICAL-CHANNEL-CONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 KHz (for FR1) and 60/120 KHz (for FR2) are applicable. Corresponds to ‘allowedSCS-List’ as specified in TS 38.321 [3].

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3].

bucketSizeDuration Value in ms. ms5 corresponds to 5ms, ms10 corresponds to 10ms, and so on.

configuredGrantType1Allowed If present, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Indicates whether SR masking is configured for this logical channel. See TS 38.321 [3]

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to FALSE if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration' in TS 38.321 [3].

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. 0kBps corresponds to 0, 8kBps corresponds to 8 kiloBytes/s,16 kBps corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB. It is optionally present for a logical channel with uplink if it serves an SRB. otherwise it is not present. |

#### –	LogicalChannelIdentity

The IE LogicalChannelIdentity is used to identify one logical channel (LogicalChannelConfig) and the corresponding RLC bearer (RLC-BearerConfig).

LogicalChannelIdentity information element

-- ASN1START

-- TAG-LOGICALCHANNELIDENTITY-START

LogicalChannelIdentity ::= 			INTEGER (1..maxLC-ID)

-- TAG-LOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELL-GROUP-CONFIG-START

MAC-CellGroupConfig ::= 			SEQUENCE {

drx-Config							SetupRelease { DRX-Config }													OPTIONAL, -- Need M

schedulingRequestConfig				SchedulingRequestConfig														OPTIONAL, -- Need M

bsr-Config							BSR-Config																	OPTIONAL, -- Need M

tag-Config							TAG-Config																	OPTIONAL, -- Need M

phr-Config							SetupRelease { PHR-Config }													OPTIONAL, -- Need M

skipUplinkTxDynamic					BOOLEAN,

...,

[[

csi-Mask							ENUMERATED {true}															OPTIONAL -- Need R

]]

}

DRX-Config ::=						SEQUENCE {

drx-onDurationTimer					CHOICE {

subMilliSeconds	INTEGER (1..31),

milliSeconds	ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer					ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL				INTEGER (0..56),

drx-HARQ-RTT-TimerUL				INTEGER (0..56),

drx-RetransmissionTimerDL			ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL			ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset			CHOICE {

ms10								INTEGER(0..9),

ms20								INTEGER(0..19),

ms32								INTEGER(0..31),

ms40								INTEGER(0..39),

ms60								INTEGER(0..59),

ms64								INTEGER(0..63),

ms70								INTEGER(0..69),

ms80								INTEGER(0..79),

ms128								INTEGER(0..127),

ms160								INTEGER(0..159),

ms256								INTEGER(0..255),

ms320								INTEGER(0..319),

ms512								INTEGER(0..511),

ms640								INTEGER(0..639),

ms1024								INTEGER(0..1023),

ms1280								INTEGER(0..1279),

ms2048								INTEGER(0..2047),

ms2560								INTEGER(0..2559),

ms5120								INTEGER(0..5119),

ms10240								INTEGER(0..10239)

},

-- FFS need for finer offset granulary

-- FFS need for shorter values for long and short cycles

shortDRX							SEQUENCE {

drx-ShortCycle						ENUMERATED	{

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer					INTEGER (1..16)

}																													OPTIONAL,	-- Need R

drx-SlotOffset						INTEGER (0..31)

}

PHR-Config ::=						SEQUENCE {

phr-PeriodicTimer					ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity},

phr-ProhibitTimer					ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

phr-Tx-PowerFactorChange			ENUMERATED {dB1, dB3, dB6, infinity},

multiplePHR							BOOLEAN,

phr-Type2SpCell						BOOLEAN,

phr-Type2OtherCell					BOOLEAN,

phr-ModeOtherCG						ENUMERATED {real, virtual},

...

}

TAG-Config ::=						SEQUENCE {

tag-ToReleaseList					SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-Id										OPTIONAL,	-- Need N

tag-ToAddModList					SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG								OPTIONAL	-- Need N

}

TAG ::= 					SEQUENCE {

tag-Id								TAG-Id,

timeAlignmentTimer					TimeAlignmentTimer,

...

}

TAG-Id ::=							INTEGER (0..maxNrofTAGs-1)

TimeAlignmentTimer ::= 				ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

BSR-Config ::=						SEQUENCE {

periodicBSR-Timer					ENUMERATED { sf1, sf5, sf10, sf16, sf20, sf32, sf40, sf64,

sf80, sf128, sf160, sf320, sf640, sf1280, sf2560, infinity },

retxBSR-Timer						ENUMERATED { sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, sf2560,

sf5120, sf10240, spare5, spare4, spare3, spare2, spare1},

logicalChannelSR-DelayTimer			ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}			OPTIONAL,	-- Need R

...

}

-- TAG-MAC-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

csi-Mask Limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

drx-Config Used to configure DRX as specified in TS 38.321 [3].

drx-HARQ-RTT-TimerDL Value in number of symbols.

drx-HARQ-RTT-TimerUL Value in number of symbols.

drx-InactivityTimer Value in multiple integers of 1ms. ms0 corresponds to 0, ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1ms.

drx-RetransmissionTimerDL Value in number of slot lengths. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle. A value of 1 corresponds to drx-ShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0ms, value 1 corresponds to 1/32ms, value 2 corresponds to 2/32ms, and so on.

logicalChannelSR-DelayTimer Value in number of subframes. sf1 corresponds to one subframe, sf2 corresponds to 2 subframes, and so on.

multiplePHR Indicates if power headroom shall be reported using the Single Entry PHR MAC control element or Multiple Entry PHR MAC control element defined in TS 38.321 [3]. True means to use Multiple Entry PHR MAC control element and False means to use the Single Entry PHR MAC control element defined in TS 38.321 [3].

periodicBSR-Timer Value in number of subframes. Value sf1 corresponds to 1 subframe, sf5 corresponds to 5 subframes and so on.

phr-Tx-PowerFactorChange Value in dB for PHR reporting as specified in TS 38.321 [3]. Value dB1 corresponds to 1 dB, dB3 corresponds to 3 dB and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

phr-ModeOtherCG Indicates the mode (i.e. real or virtual) used for the PHR of the activated cells that are part of the other Cell Group (i.e. MCG or SCG), when DC is configured. If the UE is configured with only one cell group (no DC), it ignores the field.

phr-PeriodicTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. sf10 corresponds to 10 subframes, sf20 corresonds to 20 subframes, and so on.

phr-ProhibitTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. sf0 corresponds to 0 subframe, sf10 corresponds to 10 subframes, sf20 corresponds to 20 subframes, and so on.

phr-Type2SpCell Indicates whether or not PHR type 2 is reported for the SpCell of the MAC entity. It is set to false in this release of the specification.

phr-Type2OtherCell Indicates whether or not PHR type 2 is reported for the SpCell of the other MAC entity or PUCCH SCells of the MAC entity.

retxBSR-Timer Value in number of subframes. Value sf10 corresponds to 10 subframes, sf20 corresponds to 20 subframes and so on.

skipUplinkTxDynamic If set to true, the UE skips UL transmissions for an uplink grant other than a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 38.321 [3]. FFS : configurable per SCell?

tag-ID Indicates the TAG of an SCell, see TS 38.321 [3]. Uniquely identifies the TAG within the scope of a Cell Group (i.e. MCG or SCG). If the field is not configured for an SCell, the SCell is part of the PTAG.

timeAlignmentTimer Value in ms of the timeAlignmentTimer for TAG with ID tag-Id, as specified in TS 38.321 [3].

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=						SEQUENCE {

measObjectToRemoveList				MeasObjectToRemoveList												OPTIONAL,	-- Need N

measObjectToAddModList				MeasObjectToAddModList												OPTIONAL,	-- Need N

reportConfigToRemoveList			ReportConfigToRemoveList											OPTIONAL,	-- Need N

reportConfigToAddModList			ReportConfigToAddModList											OPTIONAL,	-- Need N

measIdToRemoveList					MeasIdToRemoveList													OPTIONAL,	-- Need N

measIdToAddModList					MeasIdToAddModList													OPTIONAL,	-- Need N

s-MeasureConfig						CHOICE {

ssb-RSRP							RSRP-Range,

csi-RSRP							RSRP-Range

} 																										OPTIONAL,	-- Need M

quantityConfig						QuantityConfig														OPTIONAL,	-- Need M

measGapConfig						MeasGapConfig														OPTIONAL,	-- Need M

measGapSharingConfig					MeasGapSharingConfig												OPTIONAL,	-- Need M

...

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

Editor’s Note: FFS Whether UE speed based TTT scaling (e.g. speedStatePars) is supported in Rel-15 (not applicable for EN-DC).

Editor’s Note: FFS Whether measScaleFactor (or equivalent) is supported in Rel-15 (not applicable for EN-DC).

Editor’s Note: FFS How to support allowInterruptions in NR (RAN4 input needed) in Rel-15.

MeasConfig field descriptions

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

MeasGapSharingConfig The IE MeasGapSharingConfig specifies the measurement gap sharing scheme

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfig information element

-- ASN1START

--TAG-MEAS-GAP-CONFIG-START

MeasGapConfig ::=					SEQUENCE {

gapFR2 								SetupRelease { GapConfig }														OPTIONAL,	-- Need M

...,

[[

gapFR1							SetupRelease { GapConfig }														OPTIONAL,	-- Need M

gapUE							SetupRelease { GapConfig }														OPTIONAL	-- Need M

]]

}

GapConfig ::=						SEQUENCE {

gapOffset 							INTEGER (0..159),

mgl 								ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp 								ENUMERATED {ms20, ms40, ms80, ms160},

mgta								ENUMERATED {ms0, ms0dot25, ms0dot5},

...

}

-- TAG-MEAS-GAP-CONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In the case of EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap). gapFR1 can not be configured together with gapUE. The applicability of the measurement gap is according to Table 9.1.2-2 in TS 38.133 [14].

gapFR2 Indicates measurement gap configuration applies to FR2 only. gapFR2 cannot be configured together with gapUE. The applicability of the measurement gap is according to Table 9.1.2-2 in TS 38.133 [14].

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In the case of EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap). If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. The applicability of the measurement gap is according to Table 9.1.2-2 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range should be from 0 to mgrp-1.

mgl Value mgl is the measurement gap length in ms of the measurement gap. The applicability of the measurement gap is according to in Table 9.1.2-1 and Table 9.1.2-2 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5ms, ms3 corresponds to 3ms and so on.

mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The applicability of the measurement gap is according to in Table 9.1.2-1 and Table 9.1.2-2 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to section 9.1.2 of TS 38.133 [14]. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25ms and ms0dot5 corresponds to 0.5ms. For FR2, the network only configures 0 and 0.25ms.

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

--TAG-MEAS-GAP-SHARING-CONFIG-START

MeasGapSharingConfig ::=		SEQUENCE {

gapSharingFR2 				SetupRelease { MeasGapSharingScheme }		OPTIONAL,	-- Need M

...,

[[

gapSharingFR1 				SetupRelease { MeasGapSharingScheme }		OPTIONAL,	--Need M

gapSharingUE 				SetupRelease { MeasGapSharingScheme }		OPTIONAL	--Need M

]]

}

MeasGapSharingScheme ::=		ENUMERATED { scheme00, scheme01, scheme10, scheme11 }

--TAG-MEAS-GAP-SHARING-CONFIG-STOP

-- ASN1STOP

MeasGapSharingConfig field descriptions

gapSharingFR1 Indicates the measurement gaps sharing scheme that applies to the gap set for FR1 only. In the case of EN-DC, gapSharingFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap sharing). gapSharingFR1 can not be configured together with gapSharingUE. For the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to "00", value scheme01 corresponds to "01", and so on.

gapSharingFR2 Indicates the measurement gaps sharing scheme that applies to the gap set for FR2 only. gapSharingFR2 cannot be configured together with gapSharingUE. For the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to "00", value scheme01 corresponds to "01", and so on.

gapSharingUE Indicates the measurement gaps sharing scheme that applies to the gap set per UE. In EN-DC, gapSharingUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap sharing). If gapSharingUE is configured, then neither gapSharingFR1 nor gapSharingFR2 can be configured. For the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to "00", value scheme01 corresponds to "01", and so on.

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEAS-ID-START

MeasId ::=							INTEGER (1..maxNrofMeasId)

-- TAG-MEAS-ID-STOP

-- ASN1STOP

#### –	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEAS-ID-TO-ADD-MOD-LIST-START

MeasIdToAddModList ::=				SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=					SEQUENCE {

measId								MeasId,

measObjectId						MeasObjectId,

reportConfigId						ReportConfigId

}

-- TAG-MEAS-ID-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

-- TAG-MEAS-OBJECT-EUTRA-NR-START

MeasObjectEUTRA ::=							SEQUENCE {

carrierFreq									ARFCN-ValueEUTRA,

allowedMeasBandwidth						EUTRA-AllowedMeasBandwidth	,

cellsToRemoveListEUTRAN						EUTRA-CellIndexList			OPTIONAL,	-- Need N

cellsToAddModListEUTRAN						SEQUENCE (SIZE (1.. maxCellMeasEUTRA)) OF EUTRA-Cell		    OPTIONAL,	-- Need N

blackCellsToRemoveListEUTRAN				EUTRA-CellIndexList				OPTIONAL,	-- Need N

blackCellsToAddModListEUTRAN				SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-BlackCell	OPTIONAL,	-- Need N

cellForWhichToReportCGI						EUTRA-PhysCellId				OPTIONAL,	-- Need R

eutra-PresenceAntennaPort1					EUTRA-PresenceAntennaPort1 	,

eutra-Q-OffsetRange							EUTRA-Q-OffsetRange 	OPTIONAL,	-- Need R

...

}

EUTRA-CellIndexList ::=						SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-CellIndex

EUTRA-CellIndex ::=					INTEGER (1..maxCellMeasEUTRA)

EUTRA-Cell ::=	SEQUENCE {

cellIndex							EUTRA-CellIndex,

physCellId							EUTRA-PhysCellId,

cellIndividualOffset				EUTRA-Q-OffsetRange

}

EUTRA-BlackCell ::=	SEQUENCE {

cellIndex							EUTRA-CellIndex,

physCellIdRange						EUTRA-PhysCellIdRange

}

-- TAG-MEAS-OBJECT-EUTRA-NR-STOP

-- ASN1STOP

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEAS-OBJECT-ID-START

MeasObjectId ::=					INTEGER (1..maxNrofObjectId)

-- TAG-MEAS-OBJECT-ID-STOP

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=					SEQUENCE {

ssbFrequency						ARFCN-ValueNR													OPTIONAL,	-- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing					SubcarrierSpacing											OPTIONAL,	-- Cond SSBorAssociatedSSB

smtc1								SSB-MTC                                                			OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2								SSB-MTC2														OPTIONAL,	-- Cond IntraFreqConnected

refFreqCSI-RS						ARFCN-ValueNR															OPTIONAL,

referenceSignalConfig				ReferenceSignalConfig,

absThreshSS-BlocksConsolidation		ThresholdNR																OPTIONAL,	-- Need R

absThreshCSI-RS-Consolidation		ThresholdNR																OPTIONAL,	-- Need R

nrofSS-BlocksToAverage				INTEGER (2..maxNrofSS-BlocksToAverage)									OPTIONAL,	-- Need R

nrofCSI-RS-ResourcesToAverage		INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)							OPTIONAL,	-- Need R

quantityConfigIndex					INTEGER (1..maxNrofQuantityConfig),

offsetMO							Q-OffsetRangeList,

cellsToRemoveList					PCI-List																OPTIONAL,	-- Need N

cellsToAddModList					CellsToAddModList														OPTIONAL,	-- Need N

blackCellsToRemoveList				PCI-RangeIndexList														OPTIONAL,	-- Need N

blackCellsToAddModList				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement			OPTIONAL,	-- Need N

whiteCellsToRemoveList				PCI-RangeIndexList														OPTIONAL,	-- Need N

whiteCellsToAddModList				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement		OPTIONAL,	-- Need N

...

}

ReferenceSignalConfig::=     		SEQUENCE {

ssb-ConfigMobility					SSB-ConfigMobility														OPTIONAL,	-- Need M

csi-rs-ResourceConfigMobility		SetupRelease { CSI-RS-ResourceConfigMobility } 							OPTIONAL	-- Need M

}

SSB-ConfigMobility ::= 				SEQUENCE {

ssb-ToMeasure							SetupRelease { SSB-ToMeasure }										OPTIONAL,	-- Need M

deriveSSB-IndexFromSCell								BOOLEAN,

ss-RSSI-Measurement							SS-RSSI-Measurement											OPTIONAL,	-- Need M

...

}

Q-OffsetRangeList ::=				SEQUENCE {

rsrpOffsetSSB						Q-OffsetRange				DEFAULT dB0,

rsrqOffsetSSB						Q-OffsetRange				DEFAULT dB0,

sinrOffsetSSB						Q-OffsetRange				DEFAULT dB0,

rsrpOffsetCSI-RS					Q-OffsetRange				DEFAULT dB0,

rsrqOffsetCSI-RS					Q-OffsetRange				DEFAULT dB0,

sinrOffsetCSI-RS					Q-OffsetRange				DEFAULT dB0

}

ThresholdNR ::=						SEQUENCE{

thresholdRSRP						RSRP-Range					OPTIONAL,	-- Need R

thresholdRSRQ						RSRQ-Range					OPTIONAL, 	-- Need R

thresholdSINR						SINR-Range					OPTIONAL	-- Need R

}

CellsToAddModList ::=				SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddMod ::=					SEQUENCE {

physCellId							PhysCellId,

cellIndividualOffset				Q-OffsetRangeList

}

-- TAG-MEAS-OBJECT-NR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

physCellId Physical cell identity of a cell in the cell list.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per CSI-RS resource as described in 5.5.3.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3and the L3 filter(s) per SS/PBCH block index as described in 5.5.3.2.

blackCellsToAddModList List of cells to add/modify in the black list of cells. This list applied to both SSB and CSI-RS resource(s).

blackCellsToRemoveList List of cells to remove from the black list of cells.

cellsToAddModList List of cells to add/modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

nrofCSInrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration (e.g. SMTC window, CSI-RS resource, etc.)

refFreqCSI-RS Point A which is used for maping of CSI-RS to physical resources according to TS 38.211 section 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see section 5.5.2.10).

smtc2 Secondary measurement timing confguration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value stricty shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured).

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 or 30 (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable.

whiteCellsToAddModList List of cells to add/modify in the white list of cells. This list applied to both SSB and CSI-RS resource(s).

whiteCellsToRemoveList List of cells to remove from the white list of cells.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration)

SSB-ConfigMobility field descriptions

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured. (see 38.215) When the field is absent the UE measures on all SS/PBCH blocks. Regardless of the value of this field, SS/PBCH block outside of the applicable smtc are not to be measured. See TS 38.215 section 5.1.1.

derive-SSBIndexFromCell If the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

Editor’s Note: FFS How to support CGI reporting and whether changes are required in MeasObjectNR (e.g. introduction of cellForWhichToReportCGI). Not applicable for EN-DC.

Editor’s Note: FFS Whether alternative TTT is supported in Rel-15 (not applicable for EN-DC).

Editor’s Note: FFS measCycleSCell. (not applicable for EN-DC)

Editor’s Note: FFS reducedMeasPerformance (not applicable for EN-DC).

| Conditional presence | Explanation |
| --- | --- |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell, otherwise, it is absent. |
| IntraFreqConnected | This field is optionally present Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |

#### –	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEAS-OBJECT-TO-ADD-MOD-LIST-START

MeasObjectToAddModList ::=					SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=						SEQUENCE {

measObjectId								MeasObjectId,

measObject									CHOICE {

measObjectNR								MeasObjectNR,

...,

measObjectEUTRA								MeasObjectEUTRA

}

}

-- TAG-MEAS-OBJECT-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, and inter-RAT mobility.

MeasResults information element

-- ASN1START

-- TAG-MEAS-RESULTS-START

MeasResults ::=								SEQUENCE {

measId										MeasId,

measResultServingMOList					MeasResultServMOList,

measResultNeighCells						CHOICE {

measResultListNR							MeasResultListNR,

...,

measResultListEUTRA							MeasResultListEUTRA

}																														OPTIONAL,

...

}

MeasResultServMOList ::=					SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=						SEQUENCE {

servCellId									ServCellIndex,

measResultServingCell						MeasResultNR,

measResultBestNeighCell						MeasResultNR															OPTIONAL,

...

}

MeasResultListNR ::=						SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=							SEQUENCE {

physCellId									PhysCellId																	OPTIONAL,

measResult									SEQUENCE {

cellResults									SEQUENCE{

resultsSSB-Cell								MeasQuantityResults													OPTIONAL,

resultsCSI-RS-Cell							MeasQuantityResults													OPTIONAL

},

rsIndexResults								SEQUENCE{

resultsSSB-Indexes							ResultsPerSSB-IndexList												OPTIONAL,

resultsCSI-RS-Indexes						ResultsPerCSI-RS-IndexList											OPTIONAL

}																													OPTIONAL

},

...,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdNR,

trackingAreaCode					TrackingAreaCode,

plmn-IdentityList					PLMN-IdentityInfoList 																OPTIONAL,

frequencyBandList					MultiFrequencyBandListNR														OPTIONAL,

noSIB1								SEQUENCE {

ssb-SubcarrierOffset				INTEGER (0..15),

pdcch-ConfigSIB1					PDCCH-ConfigSIB1

}																													OPTIONAL

}																														OPTIONAL

}

MeasResultListEUTRA ::=						SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=							SEQUENCE {

physCellId									PhysCellId,

measResult									MeasQuantityResultsEUTRA,

--FFS: Details of cgi info

...

}

MeasQuantityResults ::= 			SEQUENCE {

rsrp								RSRP-Range													OPTIONAL,

rsrq								RSRQ-Range													OPTIONAL,

sinr								SINR-Range													OPTIONAL

}

MeasQuantityResultsEUTRA ::= 			SEQUENCE {

rsrp									RSRP-RangeEUTRA											OPTIONAL,

rsrq									RSRQ-RangeEUTRA											OPTIONAL,

sinr									SINR-RangeEUTRA											OPTIONAL

}

ResultsPerSSB-IndexList::= 				SEQUENCE (SIZE (1..maxNrofSSBs)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::= 					SEQUENCE {

ssb-Index									SSB-Index,

ssb-Results									MeasQuantityResults							OPTIONAL

}

ResultsPerCSI-RS-IndexList::= 				SEQUENCE (SIZE (1..maxNrofCSI-RS)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::= 					SEQUENCE {

csi-RS-Index								CSI-RS-Index,

csi-RS-Results								MeasQuantityResults							OPTIONAL

}

-- TAG-MEAS-RESULTS-STOP

-- ASN1STOP

MeasResultServFreq field descriptions

measResultBestNeighCell Measured results of the best detected neighbour cell on the corresponding serving frequency.

Editor’s Note: FFS locationInfo.

MeasResults field descriptions

csi-rs-Index CSI-RS resource index associated to the measurement information to be reported.

measId Identifies the measurement identity for which the reporting is being performed.

measResult Measured results of an NR cell.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultServingMOList Measured results of measured cells with reference signals indicated in the serving cell measurement objects including measurement results of SpCell, configured SCell(s) and best neighbouring cell within measured cells with reference signals indicated in on each serving cell measurement object.

noSib1 True indicates that SIB1 is not broadcasted for the concern cell.

resultsCSI-RS-Indexes List of measurement information per CSI-RS resource index of an NR cell.

resultsSSB-Indexes List of measurement information per SS/PBCH index of an NR cell.

resultsCSI-RS-Cell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived from CSI-RS measurements.

resultsSSB-Cell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived on SS/PBCH block measurements.

rsrp Measured SS-RSRP or CSI-RSRP resultsas defined in TS 38.215 [9], either per NR cell from the L1 filter(s) or per (SS/PBCH)/(CSI-RS) index as specified in 5.5.3.3a.

rsrq Measured SS-RSRQ or CSI-RSRQ results as defined in TS 38.215 [9], either per NR cell from the L1 filter(s) or per (SS/PBCH)/(CSI-RS) index as specified in 5.5.3.3a.

sinr Measured SS-SINR or CSI-SINR results as defined in TS 38.215 [9], either per NR cell from the L1 filter(s) or per (SS/PBCH)/(CSI-RS) index as specified in 5.5.3.3a.

ssb-Index SS/PBCH block index associated to the measurement information to be reported.

#### –	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in case of EN-DC.

MeasResultSCG-Failure information element

-- ASN1START

-- TAG-MEAS-RESULT-SCG-FAILURE-START

MeasResultSCG-Failure ::= 			SEQUENCE {

measResultPerMOList						MeasResultList2NR,

...

}

MeasResultList2NR ::=				SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

MeasResult2NR ::=					SEQUENCE {

ssbFrequency						ARFCN-ValueNR							OPTIONAL,

refFreqCSI-RS						ARFCN-ValueNR							OPTIONAL,

measResultServingCell				MeasResultNR							OPTIONAL,

measResultNeighCellListNR			MeasResultListNR

}

-- TAG-MEAS-RESULT-SCG-FAILURE-STOP

-- ASN1STOP

#### –	MeasResultCellListSFTD

The IE MeasResultCellListSFTD consists of SFN and radio frame boundary difference between the PCell and an NR  cell as specified in TS 38.215 [9] and TS 38.133 [14].

MeasResultCellListSFTD information element

-- ASN1START

-- TAG-MEASRESULT-CELL-LIST-SFTD-START

MeasResultCellListSFTD ::=			SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD

MeasResultCellSFTD ::=				SEQUENCE {

physCellId							PhysCellId,

sfn-OffsetResult					INTEGER (0..1023),

frameBoundaryOffsetResult			INTEGER (-30720..30719),

rsrp-Result							RSRP-Range						OPTIONAL

}

-- TAG-MEASRESULT-CELL-LIST-SFTD-STOP

-- ASN1STOP

MeasResultSFTD field descriptions

sfn-OffsetResult Indicates the SFN difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

#### –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

-- TAG-MOBILITY-STATE-PARAMETERS-START

MobilityStateParameters ::=			SEQUENCE {

t-Evaluation						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium					INTEGER (1..16),

n-CellChangeHigh					INTEGER (1..16)

}

-- TAG-MOBILITY-STATE-PARAMETERS-STOP

-- ASN1STOP

MobilityStateParameters field descriptions

n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in TS 38.304 [4].

n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in TS 38.304 [4].

t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in TS 38.304 [4]. Value in seconds, s30 corresponds to 30 s and so on.

t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in TS 38.304 [4]. Value in seconds, s30 corresponds to 30 s and so on.

#### –	MultiFrequencyBandListNR

The IE MultiFrequencyBandListNR is used to configure a list of one or multiple NR frequency bands.

MultiFrequencyBandListNR information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-START

MultiFrequencyBandListNR ::=		SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR

-- TAG-MULTIFREQUENCYBANDLISTNR-STOP

-- ASN1STOP

#### –	NextHopChainingCount

The IE NextHopChainingCount is used to update the KgNB key and corresponds to parameter NCC: See TS 33.501 [11].

NextHopChainingCount information element

-- ASN1START

-- TAG-NEXTHOPCHAININGCOUNT-START

NextHopChainingCount ::=					INTEGER (0..7)

-- TAG-NEXTHOPCHAININGCOUNT-STOP

-- ASN1STOP

Editor’s Note: FFS Confirm that same value range from LTE for the NextHopChainingCountis reused in NR (i.e. 0 to 7).

#### –	NG-5G-S-TMSI

The IE NG-5G-TMSI contains a 5G S-Temporary Mobile Subscription Identifier (5G-S-TMSI), a temporary UE identity provided by the 5GC which uniquely identifies the UE within the tracking area, see TS 23.003 [20].

NG-5G-S-TMSI information element

-- ASN1START

-- TAG-NG-5G-S-TMSI-START

NG-5G-S-TMSI::=							SEQUENCE {

amf-SetId								AMF-SetID,

amf-Pointer								AMF-Pointer,

ng-5g-TMSI								BIT STRING (SIZE (32))

}

-- TAG-NG-5G-S-TMSI-STOP

-- ASN1STOP

NG-5G-S-TMSI field descriptions

ng-5g-TMSI Indicates the5G-TMSI as defined in ts 23.003 [20].

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RStransmitted in the cell where the IE is included, which the UE may be configured to measure on (see 38.214, section 5.2.2.3.1).

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=				SEQUENCE {

nzp-CSI-RS-ResourceId				NZP-CSI-RS-ResourceId,

resourceMapping						CSI-RS-ResourceMapping,

powerControlOffset					INTEGER(-8..15),

powerControlOffsetSS				ENUMERATED {db-3, db0, db3, db6}							OPTIONAL,	-- Need R

scramblingID						ScramblingId,

periodicityAndOffset				CSI-ResourcePeriodicityAndOffset							OPTIONAL,	-- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS				TCI-StateId													OPTIONAL,	-- Cond Periodic

...

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

NZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots. Corresponds to L1 parameter 'CSI-RS-timeConfig' (see 38.214, section 5.2.2.3.1)

powerControlOffset Power offset of NZP CSI-RS RE to PDSCH RE. Value in dB. Corresponds to L1 parameter Pc (see 38.214, sections 5.2.2.3.1 and 4.1)

powerControlOffsetSS Power offset of NZP CSI-RS RE to SS RE. Value in dB. Corresponds to L1 parameter 'Pc_SS' (see 38.214, section 5.2.2.3.1)

qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belong to. Corresponds to L1 parameter 'QCL-Info-PeriodicCSI-RS' (see 38.214, section 5.2.2.3.1)

resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource

scramblingID Scrambling ID (see 38.214, section 5.2.2.3.1)

| Conditional Presence | Explanation |
| --- | --- |
| Periodic | The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise |
| PeriodicOrSemiPersistent | The field is mandatory present, Need M, for periodic and semi-persistent NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::= 			INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::= 			SEQUENCE {

nzp-CSI-ResourceSetId				NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources				SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition							ENUMERATED { on, off }															OPTIONAL,	-- Need R

aperiodicTriggeringOffset			INTEGER(0..4)																	OPTIONAL,	-- Need S

trs-Info							ENUMERATED {true}																OPTIONAL,	-- Need R

...

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

NZP-CSI-RS-ResourceSet field descriptions

aperiodicTriggeringOffset Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. When the field is absent the UE applies the value 0. Corresponds to L1 parameter 'Aperiodic-NZP-CSI-RS-TriggeringOffset' (see 38,214, section FFS_Section)

nzp-CSI-RS-Resources NZP-CSI-RS-Resources assocaited with this NZP-CSI-RS resource set. Corresponds to L1 parameter 'CSI-RS-ResourceConfigList' (see 38.214, section 5.2). For CSI, there are at most 8 NZP CSI RS resources per resource set

repetition Indicates whether repetition is on/off. If set to set to 'OFF', the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter and with same NrofPorts in every symbol. Corresponds to L1 parameter 'CSI-RS-ResourceRep' (see 38.214, sections 5.2.2.3.1 and 5.1.6.1.2). Can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP or “no report”

trs-Info Indicates that the antenna port for all NZP-CSI-RS resources in the CSI-RS resource set is same. If the field is absent or released the UE applies the value "false". Corresponds to L1 parameter 'TRS-Info' (see 38.214, section 5.2.2.3.1)

#### –	NZP-CSI-RS-ResourceSetId

The IE NZP-CSI-RS-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESETID-START

NZP-CSI-RS-ResourceSetId ::=		INTEGER (0..maxNrofNZP-CSI-RS-ResourceSets-1)

-- TAG-NZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

#### –	P-Max

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency, see TS 38.101 [14].

P-Max information element

-- ASN1START

-- TAG-P-MAX-START

P-Max ::=							INTEGER (-30..33)

-- TAG-P-MAX-STOP

-- ASN1STOP

#### –	PCI-List

The IE PCI-List concerns a list of physical cell identities, which may be used for different purposes.

PCI-List information element

-- ASN1START

-- TAG-PCI-LIST-START

PCI-List ::=						SEQUENCE (SIZE (1..maxNrofCellMeas)) OF PhysCellId

-- TAG-PCI-LIST-STOP

-- ASN1STOP

#### –	PCI-Range

The IE PCI-Range is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PCI-Range, the Network may configure overlapping ranges of physical cell identities.

PCI-Range information element

-- ASN1START

-- TAG-PCI-RANGE-START

PCI-Range ::=						SEQUENCE {

start								PhysCellId,

range								ENUMERATED { n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008, spare1} 					OPTIONAL	 -- Need S

}

-- TAG-PCI-RANGE-STOP

-- ASN1STOP

PCI-Range field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

#### –	PCI-RangeElement

The IE PCI-RangeElement is used to define a PCI-Range as part of a list (e.g. AddMod list).

PCI-RangeElement information element

-- ASN1START

-- TAG-PCI-RANGEELEMENT-START

PCI-RangeElement ::=				SEQUENCE {

pci-RangeIndex						PCI-RangeIndex,

pci-Range							PCI-Range

}

-- TAG-PCI-RANGEELEMENT-STOP

-- ASN1STOP

PCI-RangeElement field descriptions

pci-Range Physical cell identity or a range of physical cell identities.

#### –	PCI-RangeIndex

The IE PCI-RangeIndex identifies a physical cell id range, which may be used for different purposes.

PCI-RangeIndex information element

-- ASN1START

-- TAG-PCI-RANGE-INDEX-START

PCI-RangeIndex ::=					INTEGER (1..maxNrofPCI-Ranges)

-- TAG-PCI-RANGE-INDEX-STOP

-- ASN1STOP

#### –	PCI-RangeIndexList

The IE PCI-RangeIndexList concerns a list of indexes of physical cell id ranges, which may be used for different purposes.

PCI-RangeIndexList information element

-- ASN1START

-- TAG-PCI-RANGE-INDEX-LIST-START

PCI-RangeIndexList ::=				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeIndex

-- TAG-PCI-Range-INDEX-LIST-STOP

-- ASN1STOP

#### –	PDCCH-Config

The PDCCH-Config IE is used to configure UE specific PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::= 					SEQUENCE {

controlResourceSetToAddModList		SEQUENCE(SIZE (1..3)) OF ControlResourceSet 						OPTIONAL,	-- Need N

controlResourceSetToReleaseList		SEQUENCE(SIZE (1..3)) OF ControlResourceSetId						OPTIONAL, 	-- Need N

searchSpacesToAddModList			SEQUENCE(SIZE (1..10)) OF SearchSpace								OPTIONAL, 	-- Need N

searchSpacesToReleaseList			SEQUENCE(SIZE (1..10)) OF SearchSpaceId								OPTIONAL, 	-- Need N

downlinkPreemption					SetupRelease { DownlinkPreemption }									OPTIONAL,	-- Need M

tpc-PUSCH							SetupRelease { PUSCH-TPC-CommandConfig }							OPTIONAL,   -- Need M

tpc-PUCCH							SetupRelease { PUCCH-TPC-CommandConfig }							OPTIONAL,   -- Cond PUCCH-CellOnly

tpc-SRS								SetupRelease { SRS-TPC-CommandConfig}								OPTIONAL,	-- Need M

...

}

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

PDCCH-Config field descriptions

controlResourceSetToAddModList List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE. The network configures at most 3 CORESETs per BWP per cell (including UE-specific and common CORESETs).

downlinkPreemption Configuration of downlink preemtption indications to be monitored in this cell. Corresponds to L1 parameter 'Preemp-DL' (see 38.214, section 11.2) FFS_RAN1: LS R1-1801281 indicates this is "Per Cell (but association with each configured BWP is needed)" => Unclear, keep on BWP for now.

searchSpacesToAddModList List of UE specifically configured Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces).

tpc-PUCCH Enable and configure reception of group TPC commands for PUCCH

tpc-PUSCH Enable and configure reception of group TPC commands for PUSCH

tpc-SRS Enable and configure reception of group TPC commands for SRS

| Conditional Presence | Explanation |
| --- | --- |
| PUCCH-CellOnly | The field is optionally present, Need M, for the PDCCH-Config of an SpCells as well as for PUCCH SCells. The field is absent otherwise. |

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as during handover and PSCell/SCell addition.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=				SEQUENCE {

controlResourceSetZero				ControlResourceSetZero														OPTIONAL, 	-- Cond InitialBWP-Only

commonControlResourceSet			ControlResourceSet										OPTIONAL, 	-- Need R

searchSpaceZero						SearchSpaceZero													OPTIONAL, 	-- Cond InitialBWP-Only

commonSearchSpaceList					SEQUENCE (SIZE(1..4)) OF SearchSpace								OPTIONAL,	-- Need R

searchSpaceSIB1						SearchSpaceId														OPTIONAL,	-- Need S

searchSpaceOtherSystemInformation	SearchSpaceId														OPTIONAL,	-- Need S

pagingSearchSpace					SearchSpaceId														OPTIONAL, 	-- Need S

ra-SearchSpace						SearchSpaceId														OPTIONAL, 	-- Need S

...

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource setwhich may be configured and used for RAR (see ra-SearchSpace). If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet.

commonSearchSpaceList A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0.

controlResourceSetZero Parameters of the common CORESET#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) the UE operating in a different BWP acquires the CORESET#0 if the conditions defined in 38.213, section 10 are satisfied.

pagingSearchSpace ID of the Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10). If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13.

ra-SearchSpace ID of the Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13.

searchSpaceSIB1 ID of the search space for SIB1 message. If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13. Corresponds to L1 parameter 'rmsi-SearchSpace' (see 38.213, section 10)

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) the UE acquires the SearchSpace#0 irrespective of the currently active BWP as described in FFS_Spec, section FFS_Section).

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in dedicated signalling. It is absent in other BWPs and when sent in system information. |

#### –	PDCCH-ConfigSIB1

The IE PDCCH-ConfigSIB1 is used to configure the initial DL BWP.

PDCCH-ConfigSIB1 information element

-- ASN1START

-- TAG-PDCCH-CONFIGSIB1-START

PDCCH-ConfigSIB1 ::=				SEQUENCE {

controlResourceSetZero				ControlResourceSetZero,

searchSpaceZero						SearchSpaceZero

}

-- TAG-PDCCH-CONFIGSIB1-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

controlResourceSetZero Corresponds to the 4 LSB RMSI-PDCCH-Config in TS 38.213 [13], section 13. Determines a common ControlResourceSet (CORESET) of initial DL BWP.

searchSpaceZero Corresponds to 4 MSB of RMSI-PDCCH-Config in TS 38.213 [13], section 13. Determines a common search space of initial DL BWP

#### –	PDCCH-ServingCellConfig

The IE PDCCH-ServingCellConfig is used to configure UE specific PDCCH parameters applicable across all bandwidth parts of a serving cell.

PDCCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDCCH-SERVINGCELLCONFIG-START

PDCCH-ServingCellConfig	::=			SEQUENCE {

slotFormatIndicator					SetupRelease { SlotFormatIndicator }								OPTIONAL,	-- Need M

...

}

-- TAG-PDCCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDCCH-ServingCellConfig field descriptions

slotFormatIndicator Configuration of Slot-Format-Indicators to be monitored in the correspondingly configured PDCCHs this serving cell.

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=			SEQUENCE {

drb						SEQUENCE {

discardTimer			ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} 													OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL			ENUMERATED {len12bits, len18bits}				OPTIONAL, -- Cond Setup2

pdcp-SN-SizeDL			ENUMERATED {len12bits, len18bits}				OPTIONAL, -- Cond Setup2

headerCompression		CHOICE {

notUsed					NULL,

rohc					SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0001			BOOLEAN,

profile0x0002			BOOLEAN,

profile0x0003			BOOLEAN,

profile0x0004			BOOLEAN,

profile0x0006			BOOLEAN,

profile0x0101			BOOLEAN,

profile0x0102			BOOLEAN,

profile0x0103			BOOLEAN,

profile0x0104			BOOLEAN

},

drb-ContinueROHC			ENUMERATED { true }												OPTIONAL	-- Need R

},

uplinkOnlyROHC			SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0006			BOOLEAN

},

drb-ContinueROHC			ENUMERATED { true }												OPTIONAL	-- Need R

},

...

},

integrityProtection		ENUMERATED { enabled }														OPTIONAL,	-- Cond ConnectedTo5GC

statusReportRequired	ENUMERATED { true }															OPTIONAL,	-- Cond Rlc-AM

outOfOrderDelivery		ENUMERATED { true }													OPTIONAL	-- Need R

}																											OPTIONAL,	-- Cond DRB

moreThanOneRLC			SEQUENCE {

primaryPath				SEQUENCE {

cellGroup				CellGroupId											OPTIONAL,	-- Need R

logicalChannel			LogicalChannelIdentity								OPTIONAL	-- Need R

},

ul-DataSplitThreshold	UL-DataSplitThreshold 									OPTIONAL, -- Cond SplitBearer

pdcp-Duplication			BOOLEAN													OPTIONAL	-- Need R

}																										OPTIONAL, -- Cond MoreThanOneRLC

t-Reordering				ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300,	ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24, spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03, spare02, spare01 }		OPTIONAL, -- Need S

...,

[[

cipheringDisabled		ENUMERATED {true} 														OPTIONAL 	-- Cond ConnectedTo5GC

]]

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, cipherng is disabled for this DRB and the UE applies the NULL ciphering algorithm (nea0) regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms50 corresponds to 50 ms, ms100 corresponds to 100 ms and so on.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of reconfiguration with sync where the PDCP termination point is not changed.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configure ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment. Network configures headerCompression to notUsed when outOfOrderDelivery is configured.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The value of integrityProtection for a DRB can only be changed using reconfiguration with sync. FFS: text to indicate where to find the key.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5] FFS: need to specify something with respect to UE capabilities.

moreThanOneRLC FFS / TODO: Handle more than two secondary cell groups

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. Out-of-order delivery is configured only when the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates whether duplication is configured. The value of this field, when the field is present, indicates whether duplication is activated. The value of this field is always TRUE, when configured for a SRB.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value 12 is applicable.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value 12 is applicable.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 clause 5.2.1 for UL data tranmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs.

pdcp-SN-Size PDCP sequence number size, 12 or 18 bits.

statusReportRequired For AM DRBs, indicates whether the DRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For UL DRBs, the value shall be ignored by the UE.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0ms, value ms20 corresponds to 20ms, value ms40 corresponds to 40ms, and so on. When the field is absent the UE applies the value infinity.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 byte, value b100 corresponds to 100 byte, value b200 corresponds to 200 byte, and so on.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB is being set up, not present for SRBs. Otherwise this field is optionally present, need M. |
| DRB-SetupOnly | This field is optionally present, Need R, upon DRB addition if the UE is connected to 5GC. Otherwise the field is absent. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of an additional logical channel to the PDCP entity. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent and all its included parameters are released. |
| Rlc-AM | For RLC AM, the field is optionally present, need R. Otherwise, the field is not present. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is optional present, need M, n case of radio bearer with more than one associated RLC mapped to different cell groups. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC and the DRB is being setup. Otherwise the field is absent. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM.. Otherwise, ths field is not present. |

#### –	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::= 						SEQUENCE {

dataScramblingIdentityPDSCH				INTEGER (0..1023)														OPTIONAL,	-- Need S

dmrs-DownlinkForPDSCH-MappingTypeA		SetupRelease { DMRS-DownlinkConfig }									OPTIONAL,	-- Need M

dmrs-DownlinkForPDSCH-MappingTypeB		SetupRelease { DMRS-DownlinkConfig } 									OPTIONAL,	-- Need M

tci-StatesToAddModList					SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State						OPTIONAL,	-- Need N

tci-StatesToReleaseList					SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId					OPTIONAL,	-- Need N

vrb-ToPRB-Interleaver					ENUMERATED {n2, n4}														OPTIONAL, 	-- Need S

resourceAllocation						ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList			SetupRelease { PDSCH-TimeDomainResourceAllocationList }					OPTIONAL,	-- Need M

pdsch-AggregationFactor					ENUMERATED { n2, n4, n8 }												OPTIONAL,	-- Need S

rateMatchPatternToAddModList			SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern		OPTIONAL, 	-- Need N

rateMatchPatternToReleaseList			SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId		OPTIONAL, 	-- Need N

rateMatchPatternGroup1					RateMatchPatternGroup		OPTIONAL, 	-- Need R

rateMatchPatternGroup2					RateMatchPatternGroup		OPTIONAL, 	-- Need R

rbg-Size								ENUMERATED {config1, config2},

mcs-Table								ENUMERATED {qam256, qam64LowSE}										OPTIONAL,	-- Need S

maxNrofCodeWordsScheduledByDCI			ENUMERATED {n1, n2}														OPTIONAL, 	-- Need R

prb-BundlingType						CHOICE {

staticBundling							SEQUENCE {

bundleSize								ENUMERATED { n4, wideband }										OPTIONAL		-- Need S

},

dynamicBundling 						SEQUENCE {

bundleSizeSet1							ENUMERATED { n4, wideband, n2-wideband, n4-wideband }			OPTIONAL,	-- Need S

bundleSizeSet2							ENUMERATED { n4, wideband }										OPTIONAL		-- Need S

}

},

zp-CSI-RS-ResourceToAddModList					SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource		  OPTIONAL, -- Need N

zp-CSI-RS-ResourceToReleaseList					SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId		  OPTIONAL, -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList 	SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet	  OPTIONAL, -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList	SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId OPTIONAL,

-- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList	SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet			  OPTIONAL,	-- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList	SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId		  OPTIONAL,	-- Need N

p-ZP-CSI-RS-ResourceSet					SetupRelease { ZP-CSI-RS-ResourceSet }												  OPTIONAL,	-- Need M

...

}

RateMatchPatternGroup ::=				SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {	cellLevel								RateMatchPatternId,

bwpLevel								RateMatchPatternId

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3. Corresponds to L1 parameter ' Aperiodic-ZP-CSI-RS-Resource-List' (see 38.214, section 5.1.4.2)

dataScramblingIdentityPDSCH Identifer used to initalite data scrambling (c_init) for PDSCH. If the field is absent, the UE applies the physical cell ID. (see 38.211, section 7.3.1.1).

dmrs-DownlinkForPDSCH-MappingTypeA DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

dmrs-DownlinkForPDSCH-MappingTypeB DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH. Corresponds to L1 parameter 'MCS-Table-PDSCH' (see 38.214, section 5.1.3.1). If the field is absent the UE applies the value 64QAM.

pdsch-AggregationFactor Number of repetitions for data. Corresponds to L1 parameter 'aggregation-factor-DL' (see 38.214, section FFS_Section) When the field is absent the UE applies the value 1

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data. If configured, the values provided herein override the values received in corresponding PDSCH-ConfigCommon for PDCCH scrambled with C-RNTI or CS-RNTI but not for CORESET#0 for which the default values in 38.214, table 5.1.2.1.1-1 apply.

prb-BundlingType Indicates the PRB bundle type and bundle size(s). Corresponds to L1 parameter 'PRB_bundling' (see 38.214, section 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 ([19], section 5.1.2.3). If a bundleSize(Set) value is absent, the UE applies the value n2.

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set.

rateMatchPatternGroup1 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config -> rateMatchPatternToAddModList (BWP level) or in ServingCellConfig -> rateMatchPatternToAddModList (cell level). Corresponds to L1 parameter 'Resource-set-group-1'. (see 38.214, section FFS_Section)

rateMatchPatternGroup2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config -> rateMatchPatternToAddModList (BWP level) or in ServingCellConfig -> rateMatchPatternToAddModList (cell level). Corresponds to L1 parameter 'Resource-set-group-2'. (see 38.214, section FFS_Section)

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the nexted bitmaps. Corresponds to L1 parameter 'Resource-set-BWP' (see 38.214, section 5.1.2.2.3) FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary.

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH. Corresponds to L1 parameter 'RBG-size-PDSCH' (see 38.214, section 5.1.2.2.1)

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 5.1.2)

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). Corresponds to L1 parameter 'ZP-CSI-RS-ResourceSetConfigList' (see 38.214, section FFS_Section).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see 38.214, section 5.1.4)

vrb-ToPRB-Interleaver Interleaving unit configurable between 2 and 4 PRBs Corresponds to L1 parameter 'VRB-to-PRB-interleaver' (see 38.211, section 6.3.1.7). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigList' (see 38.214, section FFS_Section)

#### –	PDSCH-ConfigCommon

The IE PDSCH-ConfigCommon is used to configure FFS

PDSCH-ConfigCommon information element

-- ASN1START

-- TAG-PDSCH-CONFIGCOMMON-START

PDSCH-ConfigCommon ::=					SEQUENCE {

pdsch-TimeDomainAllocationList					PDSCH-TimeDomainResourceAllocationList			OPTIONAL,	-- Need R

...

}

-- TAG-PDSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDSCH-ConfigCommon field descriptions

pdsch-AllocationList List of time-domain configurations for timing of DL assignment to DL data. The configuration applies for PDCCH scrambled with C-RNTI or CS-RNTI but not for CORESET#0 for which the default values in 38.214, table 5.1.2.1.1-1 apply.

#### –	PDSCH-ServingCellConfig

The IE PDSCH-ServingCellConfig is used to configure UE specific PDSCH parameters that are common across the UE's BWPs of one serving cell.

PDSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDSCH-SERVINGCELLCONFIG-START

PDSCH-ServingCellConfig ::= 			SEQUENCE {

codeBlockGroupTransmission				SetupRelease { PDSCH-CodeBlockGroupTransmission }						OPTIONAL,	-- Need M

xOverhead								ENUMERATED { xOh6, xOh12, xOh18 }										OPTIONAL,	-- Need S

nrofHARQ-ProcessesForPDSCH				ENUMERATED {n2, n4, n6, n10, n12, n16}									OPTIONAL,	-- Need S

pucch-Cell								ServCellIndex															OPTIONAL	,	-- Cond SCellAddOnly

...

}

PDSCH-CodeBlockGroupTransmission ::=	SEQUENCE {

maxCodeBlockGroupsPerTransportBlock		ENUMERATED {n2, n4, n6, n8},

codeBlockGroupFlushIndicator			BOOLEAN,

...

}

-- TAG-PDSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDSCH-CodeBlockGroupTransmission field descriptions

codeBlockGroupFlushIndicator Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see 38.212, section 7.3.1.2.2)

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1)

PDSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see 38.213, section 9.1.1)

nrofHARQ-ProcessesForPDSCH The number of HARQ processes to be used on the PDSCH of a serving cell. n2 corresponds to 2 HARQ processes, n4 to 4 HARQ processes and so on. If the field is absent, the UE uses 8 HARQ processes. Corresponds to L1 parameter 'number-HARQ-process-PDSCH' (see 38.214, section REF)

pucch-Cell The ID of the serving cell (of the same cell group) to use for PUCCH. If the field is absent, the UE sends the HARQ feedback on the PUCCH of the SpCell of this cell group.

xOverhead Accounts for overhead from CSI-RS, CORESET, etc. If the field is absent, the UE applies value xOh0. Corresponds to L1 parameter 'Xoh-PDSCH' (see 38.214, section 5.1.3.2)

| Conditional Presence | Explanation |
| --- | --- |
| SCellAddOnly | It is optionally present, Need M, for (non-PUCCH) SCells when adding a new SCell. The field is absent when reconfiguring SCells. The field is also absent for the SpCells as well as for a PUCCH SCell. |

#### –	PDSCH-TimeDomainResourceAllocationList

The IE PDSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PDSCH. The PDSCH-TimeDomainResourceAllocationList contains one or more of such PDSCH-TimeDomainResourceAllocations. 	The network indicates in the DL assignment which of the configued time domain allocations the UE shall apply for that DL assignment. The UE determines the bit width of the DCI field based on the number of entries in the PDSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PDSCH-TimeDomainResourceAllocationList information element

-- ASN1START

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PDSCH-TimeDomainResourceAllocationList ::=	SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::= 	SEQUENCE {

k0										INTEGER(0..32)						OPTIONAL,	-- Need S

mappingType								ENUMERATED {typeA, typeB},

startSymbolAndLength					INTEGER (0..127)

}

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PDSCH-TimeDomainResourceAllocation field descriptions

k0 The n1 corresponds to the value 1, n2 corresponds to value 2, and so on. Corresponds to L1 parameter 'K0' (see 38.214, section 5.1.2.1) When the field is absent the UE applies the value 0.

mappingType PDSCH mapping type. (see 38.214, section 5.3)

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary. Corresponds to L1 parameter 'Index-start-len' (see 38.214, section 5.1.2.1)

#### –	PhysCellId

The PhysCellId identifies the physical cell identity (PCI).

PhysCellId information element

-- ASN1START

-- TAG-PHYS-CELL-ID-START

PhysCellId ::=						INTEGER (0..1007)

-- TAG-PHYS-CELL-ID-STOP

-- ASN1STOP

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=			SEQUENCE {

harq-ACK-SpatialBundlingPUCCH		ENUMERATED {true}													OPTIONAL,	-- Need S

harq-ACK-SpatialBundlingPUSCH		ENUMERATED {true}													OPTIONAL,	-- Need S

p-NR								P-Max																OPTIONAL,	-- Need R

pdsch-HARQ-ACK-Codebook				ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI						RNTI-Value															OPTIONAL,	-- Need R

tpc-PUCCH-RNTI						RNTI-Value															OPTIONAL,	-- Need R

tpc-PUSCH-RNTI						RNTI-Value															OPTIONAL,	-- Need R

sp-CSI-RNTI							RNTI-Value															OPTIONAL,	-- Cond SP-CSI-Report

cs-RNTI								SetupRelease { RNTI-Value }											OPTIONAL,	 -- Need M

...,

[[

mcs-C-RNTI							RNTI-Value															OPTIONAL	-- Need R

]]

}

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the fidld is absent, the spatial bundling is disabled. Corresponds to L1 parameter 'HARQ-ACK-spatial-bundling' (see 38.213, section FFS_Section)

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the fidld is absent, the spatial bundling is disabled. Corresponds to L1 parameter 'HARQ-ACK-spatial-bundling' (see 38.213, section FFS_Section)

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the MCS-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

p-NR The maximum transmit power to be used by the UE in this NR cell group.

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation. Corresponds to L1 parameter 'HARQ-ACK-codebook' (see 38.213, section FFS_Section)

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig). Corresponds to L1 parameter 'SPCSI-RNTI' (see 38.214, section 5.2.1.5.2)

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI. Corresponds to L1 parameter 'TPC-PUCCH-RNTI' (see 38.213, section 10).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10)

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI. Corresponds to L1 parameter 'TPC-SRS-RNTI' (see 38.213, section 10)

| Conditional Presence | Explanation |
| --- | --- |
| SP-CSI-Report | The field is mandatory present, Need R, when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured; otherwise it is optionally present, need M. |

#### –	PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE is specified in TS 23.003 [20].

PLMN-Identityinformation element

-- ASN1START

-- TAG-PLMN-IDENTITY-INFORMATION-START

PLMN-Identity ::=					SEQUENCE {

mcc									MCC					OPTIONAL,					-- Cond MCC

mnc									MNC

}

MCC ::=								SEQUENCE (SIZE (3)) OF	MCC-MNC-Digit

MNC ::=								SEQUENCE (SIZE (2..3)) OF	MCC-MNC-Digit

MCC-MNC-Digit ::=					INTEGER (0..9)

-- TAG-PLMN-IDENTITY-INFORMATION-STOP

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [20].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [20].

Editor’s Note: FFS Whether there is a conditional presence in the case of MCC e.g. when PLMN identity is included in the Cell Global Id NR.

#### –	PLMN-IdentityInfoList

Includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITY-LIST-START

PLMN-IdentityInfoList ::=				SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode							TrackingAreaCode OPTIONAL,	-- Need R

ranac									RAN-AreaCode						OPTIONAL,		-- Need R

cellIdentity								CellIdentity,

cellReservedForOperatorUse 					ENUMERATED {reserved, notReserved},

...

}

-- TAG-PLMN-IDENTITY-LIST-STOP

-- ASN1STOP

#### –	PRB-Id

The PRB-Id indentifies a Physical Resource Block (PRB) position within a carrier.

PRB-Id information element

-- ASN1START

-- TAG-PRB-ID-START

PRB-Id ::=							INTEGER (0..maxNrofPhysicalResourceBlocks-1)

-- TAG-PRB-ID-STOP

-- ASN1STOP

#### –	PTRS-DownlinkConfig

The IE PTRS-DownlinkConfig is used to configure downlink phase tracking reference signals (PTRS) (see 38.214 section5.1.6.3)

PTRS-DownlinkConfig information element

-- ASN1START

-- TAG-PTRS-DOWNLINKCONFIG-START

PTRS-DownlinkConfig ::= 			SEQUENCE {

frequencyDensity					SEQUENCE (SIZE (2)) OF INTEGER (1..276)									OPTIONAL,	-- Need S

timeDensity							SEQUENCE (SIZE (3)) OF INTEGER (0..29)									OPTIONAL, 	-- Need S

epre-Ratio						INTEGER (0..3)															OPTIONAL,	-- Need S

resourceElementOffset				ENUMERATED { offset01, offset10, offset11 }								OPTIONAL,	-- Need S

...

}

-- TAG-PTRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

PTRS-DownlinkConfig field descriptions

epre-Ratio EPRE ratio between PTRS and PDSCH. Value 0 correspond to the codepoint ”00” in table 4.1-2. Value 1 corresponds to codepoint ”01” If the field is not provided, the UE applies value 0. Corresponds to L1 parameter 'DL-PTRS-EPRE-ratio' (see 38.214, section 4.1)

frequencyDensity Presence and frequency density of DL PT-RS as a function of Scheduled BW. If the field is absent, the UE uses K_PT-RS = 2. Corresponds to L1 parameter ‘DL-PTRS-frequency-density-table’ (see 38.214, section 5.1.6.3, Table 5.1.6.3-2)

resourceElementOffset Indicates the subcarrier offset for DL PTRS. If the field is absent, the UE applies the value offset00. Corresponds to L1 parameter ‘DL-PTRS-RE-offset’ (see 38.214, section 5.1.6.3)

timeDensity Presence and time density of DL PT-RS as a function of MCS. The value 29 is only applicable for MCS Table 5.1.3.1-1 (38.214). If the field is absent, the UE uses L_PT-RS = 1. Corresponds to L1 parameter 'DL-PTRS-time-density-table' (see 38.214, section 5.1.6.3, Table 5.1.6.3-1)

#### –	PTRS-UplinkConfig

The IE PTRS-UplinkConfig is used to configure uplink Phase-Tracking-Reference-Signals (PTRS).

PTRS-UplinkConfig information element

-- ASN1START

-- TAG-PTRS-UPLINKCONFIG-START

PTRS-UplinkConfig ::= 					SEQUENCE {

modeSpecificParameters					CHOICE {

transformPrecoderDisabled									SEQUENCE {

frequencyDensity						SEQUENCE (SIZE (2)) OF INTEGER (1..276)					OPTIONAL,	-- Need S

timeDensity								SEQUENCE (SIZE (3)) OF INTEGER (0..29)					OPTIONAL, 	-- Need S

maxNrofPorts							ENUMERATED {n1, n2},

resourceElementOffset					ENUMERATED {offset01, offset10, offset11 }				OPTIONAL,	-- Need S

ptrs-Power								ENUMERATED {p00, p01, p10, p11}

},

transformPrecoderEnabled								SEQUENCE {

sampleDensity						SEQUENCE (SIZE (5)) OF INTEGER (1..276),

timeDensityTransformPrecoding		ENUMERATED {d2}												OPTIONAL	-- Need S

}

}																										OPTIONAL,	-- Need M

...,

[[

transformPrecodingEnabled								SEQUENCE {

sampleDensity						SEQUENCE (SIZE (5)) OF INTEGER (1..276),

timeDensityTransformPrecoder		ENUMERATED {d2}													OPTIONAL	-- Need S

}																										OPTIONAL	-- Need R

]]

}

-- TAG-PTRS-UPLINKCONFIG-STOP

-- ASN1STOP

PTRS-UplinkConfig field descriptions

frequencyDensity Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW If the field is absent, the UE uses K_PT-RS = 2. Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1)

maxNrofPorts The maximum number of UL PTRS ports for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.2.3.1)

ptrs-Power UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1, table 6.2.3-5)

resourceElementOffset Indicates the subcarrier offset for UL PTRS for CP-OFDM. If the field is absent, the UE applies the value offset00. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1)

sampleDensity Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn,n=0,1,2,3,4}, that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should use depending on the scheduled BW according to the table in 38.214 FFS_Section. Corresponds to L1 parameter 'UL-PTRS-pre-DFT-density' (see 38.214, section 6.1, 6.2.3-3)

timeDensity Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS If the field is absent, the UE uses L_PT-RS = 1. Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1)

timeDensityTransformPrecoding Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM. If the field is absent, the UE applies value d1. Corresponds to L1 parameter 'UL-PTRS-time-density-transform-precoding' (see 38.214, section 6.1)

transformPrecoderDisabled Configuration of UL PTRS without transform precoder (with CP-OFDM).

transformPrecoderEnabled Configuration of UL PTRS with transform precoder (DFT-S-OFDM).

#### –	PUCCH-Config

The IE PUCCH-Config is used to configure UE specific PUCCH parameters (per BWP).

PUCCH-Config information element

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-Config ::= 						SEQUENCE {

resourceSetToAddModList					SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet				OPTIONAL,	-- Need N

resourceSetToReleaseList				SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId			OPTIONAL,	-- Need N

resourceToAddModList					SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource					OPTIONAL,	-- Need N

resourceToReleaseList					SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId					OPTIONAL,	-- Need N

format1									SetupRelease { PUCCH-FormatConfig }												OPTIONAL,	-- Need M

format2									SetupRelease { PUCCH-FormatConfig }												OPTIONAL,	-- Need M

format3									SetupRelease { PUCCH-FormatConfig }												OPTIONAL,	-- Need M

format4									SetupRelease { PUCCH-FormatConfig }												OPTIONAL,	-- Need M

schedulingRequestResourceToAddModList	SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig		OPTIONAL, -- Need N

schedulingRequestResourceToReleaseList	SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId			OPTIONAL, -- Need N

multi-CSI-PUCCH-ResourceList			SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId										OPTIONAL,-- Need M

dl-DataToUL-ACK							SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)											OPTIONAL,	-- Need M

spatialRelationInfoToAddModList			SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo	OPTIONAL,	-- Need N

spatialRelationInfoToReleaseList		SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId	OPTIONAL,	-- Need N

pucch-PowerControl						PUCCH-PowerControl																OPTIONAL,	-- Need M

...

}

PUCCH-FormatConfig ::=					SEQUENCE {

interslotFrequencyHopping				ENUMERATED {enabled}														OPTIONAL, 	-- Need R

additionalDMRS							ENUMERATED {true}															OPTIONAL,	-- Need R

maxCodeRate								PUCCH-MaxCodeRate															OPTIONAL,	-- Need R

nrofSlots								ENUMERATED {n2,n4,n8}														OPTIONAL, 	-- Need S

pi2BPSK									ENUMERATED {enabled}														OPTIONAL, 	-- Need R

simultaneousHARQ-ACK-CSI				ENUMERATED {true}															OPTIONAL	-- Need R

}

PUCCH-MaxCodeRate ::= 					ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}

-- A set with one or more PUCCH resources

PUCCH-ResourceSet ::=					SEQUENCE {

pucch-ResourceSetId						PUCCH-ResourceSetId,

resourceList					SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId,

maxPayloadMinus1						INTEGER (4..256)														OPTIONAL	-- Need R

}

PUCCH-ResourceSetId ::=					INTEGER (0..maxNrofPUCCH-ResourceSets-1)

PUCCH-Resource ::= 						SEQUENCE {

pucch-ResourceId						PUCCH-ResourceId,

startingPRB								PRB-Id,

intraSlotFrequencyHopping				ENUMERATED { enabled }													OPTIONAL,	-- Need R

secondHopPRB							PRB-Id																	OPTIONAL,	-- Need R

format									CHOICE {

format0									PUCCH-format0,														-- Cond InFirstSetOnly

format1									PUCCH-format1,														-- Cond InFirstSetOnly

format2									PUCCH-format2,														-- Cond NotInFirstSet

format3									PUCCH-format3,														-- Cond NotInFirstSet

format4									PUCCH-format4														-- Cond NotInFirstSet

}

}

PUCCH-ResourceId ::=					INTEGER (0..maxNrofPUCCH-Resources-1)

PUCCH-format0 ::=								SEQUENCE {

initialCyclicShift								INTEGER(0..11),

nrofSymbols										INTEGER (1..2),

startingSymbolIndex								INTEGER(0..13)

}

PUCCH-format1 ::= 								SEQUENCE {

initialCyclicShift								INTEGER(0..11),

nrofSymbols										INTEGER (4..14),

startingSymbolIndex								INTEGER(0..10),

timeDomainOCC									INTEGER(0..6)

}

PUCCH-format2 ::= 								SEQUENCE {

nrofPRBs										INTEGER (1..16),

nrofSymbols										INTEGER (1..2),

startingSymbolIndex								INTEGER(0..13)

}

PUCCH-format3 ::= 								SEQUENCE {

nrofPRBs										INTEGER (1..16),

nrofSymbols										INTEGER (4..14),

startingSymbolIndex								INTEGER(0..10)

}

PUCCH-format4 ::= 								SEQUENCE {

nrofSymbols										INTEGER (4..14),

occ-Length										ENUMERATED {n2,n4},

occ-Index										ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex								INTEGER(0..10)

}

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

PUCCH-Config field descriptions

dl-DataToUL-ACK List of timing for given PDSCH to the DL ACK. In this version of the specification only the values [0..8] are applicable. Corresponds to L1 parameter 'Slot-timing-value-K1' (see TS 38.213, section FFS_Section).

format1 Parameters that are common for all PUCCH resources of format 1.

format2 Parameters that are common for all PUCCH resources of format 2.

format3 Parameters that are common for all PUCCH resources of format 3.

format4. Parameters that are common for all PUCCH resources of format 4

resourceSetToAddModList Lists for adding and releasing PUCCH resource sets (see TS 38.213, section 9.2).

resourceToAddModList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report.

spatialRelationInfoToAddModList Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321, section FFS_Section and TS 38.213, section 9.2.2).

PUCCH-format3 field descriptions

nrofPRBs The supported values are 1,2,3,4,5,6,8,9,10,12,15 and 16.

PUCCH-FormatConfig field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS sybmols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 1 and 2. See TS 38.213, section 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 1, 3 or 4 is repetead over multiple slots. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The field is not applicable for format 2. See TS 38.213, section 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 1. See TS 38.213, section 9.2.5.

nrofSlots Number of slots with the same PUCCH F1, F3 or F4. When the field is absent the UE applies the value n1. The field is not applicable for format 2. See TS 38.213, section 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 1 and 2. See TS 38.213, section 9.2.5.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213, section 9.2.5. When the field is absent the UE applies the value OFF The field is not applicable for format 1.

PUCCH-Resource field descriptions

format Selection of the PUCCH format (format 0 - 4) and format-specific parameters, see TS 38.213, section 9.2.

intraSlotFrequencyHopping Enabling intra-slot frequency hopping, applicable for all types of PUCCH formats. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213, section 9.2.1.

secondHopPRB Index of first PRB after frequency hopping (for second hop) of PUCCH. This value is appliable for intra-slot frequency hopping. Ssee TS 38.213, section 9.2.1.

PUCCH-ResourceSet field descriptions

maxPayloadMinus1 Maximum number of payload bits minus 1 that the UE may transmit using this PUCCH resource set. In a PUCCH occurrence, the UE chooses the first of its PUCCH-ResourceSet which supports the number of bits that the UE wants to transmit. The field is not present in the first set (Set0) since the maximum Size of Set0 is specified to be 3 bit. The field is not present in the last configured set since the UE derives its maximum payload size as specified in 38.213. This field can take integer values that are multiples of 4. Corresponds to L1 parameter 'N_2' or 'N_3' (see TS 38.213, section 9.2).

resourceList PUCCH resources of format0 and format1 are only allowed in the first PUCCH resource set, i.e., in a PUCCH-ResourceSet with pucch-ResourceSetId = 0. This set may contain between 1 and 32 resources. PUCCH resources of format2, format3 and format4 are only allowed in a PUCCH-ResourceSet with pucch-ResourceSetId > 0. If present, these sets contain between 1 and 8 resources each. The UE chooses a PUCCH-Resource from this list as speciied in TS 38.213, section 9.2.3. Note that this list contains only a list of resource IDs. The actual resources are configured in PUCCH-Config.

#### –	PUCCH-ConfigCommon

The PUCCH-ConfigCommon IE is used to configure the cell specific PUCCH parameters.

PUCCH-ConfigCommon information element

-- ASN1START

-- TAG-PUCCH-CONFIGCOMMON-START

PUCCH-ConfigCommon ::=				SEQUENCE {

pucch-ResourceCommon				INTEGER (0..15)															OPTIONAL,	-- Need R

pucch-GroupHopping					ENUMERATED { neither, enable, disable },

hoppingId							INTEGER (0..1024)															OPTIONAL, 	-- Need R

p0-nominal							INTEGER (-202..24)																OPTIONAL, 	-- Need R

...

}

-- TAG-PUCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUCCH-ConfigCommon field descriptions

hoppingId Cell-Specific scrambling ID for group hoppping and sequence hopping if enabled. Corresponds to L1 parameter 'HoppingID' (see 38.211, section 6.3.2.2)

p0-nominal Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed. Corresponds to L1 parameter 'p0-nominal-pucch' (see 38.213, section 7.2)

pucch-GroupHopping Configuration of group- and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. "neither" implies neither group or sequence hopping is enabled. "enable" enables group hopping and disables sequence hopping. "disable"” disables group hopping and enables sequence hopping. Corresponds to L1 parameter 'PUCCH-GroupHopping' (see 38.211, section 6.4.1.3)

pucch-ResourceCommon An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters. The UE uses those PUCCH resources during initial access on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field. Corresponds to L1 parameter 'PUCCH-resource-common' (see 38.213, section 9.2)

#### –	PUCCH-PathlossReferenceRS-Id

The IE PUCCH-PathlossReferenceRS-Id is an ID for a referemce signal (RS) configured as PUCCH pathloss reference. It corresponds to L1 parameter 'pucch-pathlossreference-index' (see 38.213, section 7.2).

PUCCH-PathlossReferenceRS-Id information element

-- ASN1START

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-START

PUCCH-PathlossReferenceRS-Id ::=			INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1)

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-STOP

-- ASN1STOP

#### –	PUCCH-PowerControl

The IE PUCCH-PowerControl is used to configure FFS

PUCCH-PowerControl information element

-- ASN1START

-- TAG-PUCCH-POWERCONTROL-START

PUCCH-PowerControl ::= 				SEQUENCE {

deltaF-PUCCH-f0						INTEGER (-16..15)																	OPTIONAL, 	-- Need R

deltaF-PUCCH-f1						INTEGER (-16..15)																	OPTIONAL, 	-- Need R

deltaF-PUCCH-f2						INTEGER (-16..15)																	OPTIONAL, 	-- Need R

deltaF-PUCCH-f3						INTEGER (-16..15)																	OPTIONAL, 	-- Need R

deltaF-PUCCH-f4						INTEGER (-16..15)																	OPTIONAL, 	-- Need R

p0-Set								SEQUENCE (SIZE (1..maxNrofPUCCH-P0-PerSet)) OF P0-PUCCH								OPTIONAL, -- Need M

pathlossReferenceRSs				SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS	OPTIONAL, -- Need M

twoPUCCH-PC-AdjustmentStates		ENUMERATED {twoStates}																OPTIONAL, -- Need S

...

}

P0-PUCCH ::=							SEQUENCE {

p0-PUCCH-Id								P0-PUCCH-Id,

p0-PUCCH-Value							INTEGER (-16..15)

}

P0-PUCCH-Id ::=							INTEGER (1..8)

PUCCH-PathlossReferenceRS ::=					SEQUENCE {

pucch-PathlossReferenceRS-Id 				PUCCH-PathlossReferenceRS-Id,

referenceSignal								CHOICE {

ssb-Index									SSB-Index,

csi-RS-Index								NZP-CSI-RS-ResourceId

}

}

-- TAG-PUCCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUCCH field descriptions

p0-PUCCH-Value P0 value for PUCCH with 1dB step size.

PUCCH-PowerControl field descriptions

deltaF-PUCCH-f0 deltaF for PUCCH format 0 with 1dB step size (see 38.213, section 7.2)

deltaF-PUCCH-f1 deltaF for PUCCH format 1 with 1dB step size (see 38.213, section 7.2)

deltaF-PUCCH-f2 deltaF for PUCCH format 2 with 1dB step size (see 38.213, section 7.2)

deltaF-PUCCH-f3 deltaF for PUCCH format 3 with 1dB step size (see 38.213, section 7.2)

deltaF-PUCCH-f4 deltaF for PUCCH format 4 with 1dB step size (see 38.213, section 7.2)

p0-Set A set with dedicated P0 values for PUCCH, i.e., {P01, P02,... }. Corresponds to L1 parameter 'p0-pucch-set' (see 38.213, section 7.2)

pathlossReferenceRSs A set of Reference Signals (e.g. a CSI-RS config or a SSblock) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured FFS_CHECK: Is it possible not to configure it at all? What does the UE use then? Any SSB? Corresponds to L1 parameter 'pucch-pathlossReference-rs-config' (see 38.213, section 7.2)

twoPUCCH-PC-AdjustmentStates Number of PUCCH power control adjustment states maintained by the UE (i.e., g(i)). If the field is present (n2) the UE maintains two power control states (i.e., g(i,0) and g(i,1)). If the field is absent, it applies one (i.e., g(i,0)). Corresponds to L1 parameter 'num-pucch-pcadjustment-states' (see 38.213, section 7.2)

#### –	PUCCH-SpatialRelationInfo

The IE PUCCH-SpatialRelationInfo is used to configure FFS

PUCCH-SpatialRelationInfo information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfo ::=			SEQUENCE {

pucch-SpatialRelationInfoId			PUCCH-SpatialRelationInfoId,

servingCellId							ServCellIndex										OPTIONAL,	-- Need S

referenceSignal 						CHOICE {

ssb-Index								SSB-Index,

csi-RS-Index								NZP-CSI-RS-ResourceId,

srs										SEQUENCE {

resource							SRS-ResourceId,

uplinkBWP							BWP-Id

}

},

pucch-PathlossReferenceRS-Id 			PUCCH-PathlossReferenceRS-Id,

p0-PUCCH-Id								P0-PUCCH-Id,

closedLoopIndex							ENUMERATED { i0, i1 }

}

PUCCH-SpatialRelationInfoId ::= 		INTEGER (1..maxNrofSpatialRelationInfos)

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

PUCCH-SpatialRelationInfo field descriptions

servingCellId If the field is absent, the UE applies the ServCellId of the serving cell in which this PUCCH-SpatialRelationInfo is configured

#### –	PUCCH-TPC-CommandConfig

The IE PUCCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUCCH from a group-TPC messages on DCI.

PUCCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUCCH-TPC-COMMANDCONFIG-START

PUCCH-TPC-CommandConfig ::=				SEQUENCE {

tpc-IndexPCell							INTEGER (1..15)										OPTIONAL,	-- Cond PDCCH-OfSpcell

tpc-IndexPUCCH-SCell					INTEGER (1..15)										OPTIONAL,	-- Cond PDCCH-ofSpCellOrPUCCH-Scell

...

}

-- TAG-PUCCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUCCH-TPC-CommandConfig field descriptions

tpc-IndexPCell An index determining the position of the first bit of TPC command (applicable to the SpCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-SCell An index determining the position of the first bit of TPC command (applicable to the PUCCH SCell) inside the DCI format 2-2 payload.

| Conditional Presence | Explanation |
| --- | --- |
| PDCCH-OfSpcell | The field is mandatory present, need R, if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent. |
| PDCCH-ofSpCellOrPUCCH-Scell | The field is mandatory present, need R, if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the PUCCH-SCell. The field is optionally present, need R, if the UE is configured with a PUCCH SCell in this cell group and if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent. |

#### –	PUSCH-Config

The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::= 						SEQUENCE {

dataScramblingIdentityPUSCH				INTEGER (0..1023)															OPTIONAL,	-- Need S

txConfig								ENUMERATED {codebook, nonCodebook}										OPTIONAL, 	-- Need S

dmrs-UplinkForPUSCH-MappingTypeA		SetupRelease { DMRS-UplinkConfig }											OPTIONAL,	-- Need M

dmrs-UplinkForPUSCH-MappingTypeB		SetupRelease { DMRS-UplinkConfig }											OPTIONAL,	-- Need M

pusch-PowerControl						PUSCH-PowerControl															OPTIONAL, -- Need M

frequencyHopping						ENUMERATED {intraSlot, interSlot}													OPTIONAL,	-- Need S

frequencyHoppingOffsetLists				SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)		OPTIONAL,	-- Need M

resourceAllocation						ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList			SetupRelease { PUSCH-TimeDomainResourceAllocationList }						OPTIONAL, 	-- Need M

pusch-AggregationFactor					ENUMERATED { n2, n4, n8 }										OPTIONAL, 	-- Need S

mcs-Table								ENUMERATED {qam256, qam64LowSE}											OPTIONAL,	-- Need S

mcs-TableTransformPrecoder				ENUMERATED {qam256, qam64LowSE}										OPTIONAL,	-- Need S

transformPrecoder						ENUMERATED {enabled, disabled}									OPTIONAL,	-- Need S

codebookSubset							ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,

nonCoherent}										OPTIONAL,	-- Cond codebookBased

maxRank									INTEGER (1..4) 													OPTIONAL, 	-- Cond codebookBased

rbg-Size								ENUMERATED { config2}												OPTIONAL,	-- Need S

uci-OnPUSCH								SetupRelease { UCI-OnPUSCH}									OPTIONAL, -- Need M

tp-pi2BPSK								ENUMERATED {enabled}											OPTIONAL,	-- Need S

...

}

UCI-OnPUSCH ::= 						SEQUENCE {

betaOffsets								CHOICE {

dynamic								SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic							BetaOffsets

}																													OPTIONAL, -- Need M

scaling									ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

PUSCH-Config field descriptions

codebookSubset Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities Corresponds to L1 parameter 'ULCodebookSubset' (see 38.211, section 6.3.1.5).

dataScramblingIdentityPUSCH Identifer used to initalite data scrambling (c_init) for PUSCH. If the field is absent the UE applies the physical cell ID. (see 38.211, section 6.3.1.1).

dmrs-UplinkForPUSCH-MappingTypeA DMRS configuration for PUSCH transmissions using PUSCH mapping type A (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

dmrs-UplinkForPUSCH-MappingTypeB DMRS configuration for PUSCH transmissions using PUSCH mapping type B (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

frequencyHopping The value intraSlot enables ‘Intra-slot frequency hopping’ and the value interSlot enables ‘Inter-slot frequency hopping’. If the field is absent, frequency hopping is not configured. Corresponds to L1 parameter 'Frequency-hopping-PUSCH' (see 38.214, section 6).

frequencyHoppingOffsetLists Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 Corresponds to L1 parameter 'Frequency-hopping-offsets-set' (see 38.214, section 6.3).

maxRank Subset of PMIs addressed by TRIs from 1 to ULmaxRank. Corresponds to L1 parameter 'ULmaxRank' (see 38.211, section 6.3.1.5).

mcs-Table Indicates which MCS table the UE shall use for PUSCH without transform precoder Corresponds to L1 parameter 'MCS-Table-PUSCH' (see 38.214, section 6.1.4) If the field is absent the UE applies the value 64QAM

mcs-TableTransformPrecoder Indicates which MCS table the UE shall use for PUSCH with transform precoding Corresponds to L1 parameter 'MCS-Table-PUSCH-transform-precoding' (see 38.214, section 6.1.4) If the field is absent the UE applies the value 64QAM

pusch-AggregationFactor Number of repetitions for data. Corresponds to L1 parameter 'aggregation-factor-UL' (see 38.214, section FFS_Section). If the field is absent the UE applies the value 1.

pusch-AllocationList List of time domain allocations for timing of UL assignment to UL data. If configured, the values provided herein override the values received in corresponding PUSCH-ConfigCommon for PDCCH scrambled with C-RNTI or CS-RNTI but not for CORESET#0 (see 38.214, table 6.1.2.1.1-1)”.

rbg-Size Selection between config 1 and config 2 for RBG size for PUSCH. When the field is absent the UE applies the value config1. Corresponds to L1 parameter 'RBG-size-PUSCH' (see 38.214, section 6.1.2.2.1).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 6.1.2).

tp-pi2PBSK Enables pi/2-BPSK modulation with transform precoding if the field is present and disables it otherwise.

transformPrecoder The UE specific selection of transformer precoder for PUSCH. When the field is absent the UE applies the value msg3-tp. Corresponds to L1 parameter 'PUSCH-tp' (see 38.211, section 6.3.1.4).

txConfig Whether UE uses codebook based or non-codebook based transmission. Corresponds to L1 parameter 'ulTxConfig' (see 38.214, section 6.1.1). If the field is absent, the UE transmits PUSCH on one antenna port, see 38.214, section 6.1.1.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. If the field is absent or released, the UE applies the value 'semiStatic' and the BetaOffsets according to FFS [BetaOffsets and/or section 9.x.x). Corresponds to L1 parameter 'UCI-on-PUSCH' (see 38.213, section 9.3).

UCI-OnPUSCH field descriptions

scaling Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on. Corresponds to L1 parameter 'uci-on-pusch-scaling' (see 38.212, section 6.3).

betaOffsets Selection between and configuration of dynamic and semi-static beta-offset. If the field is absent or released, the UE applies the value 'semiStatic' and the BetaOffsets according to FFS [BetaOffsets and/or section 9.x.x). Corresponds to L1 parameter 'UCI-on-PUSCH' (see 38.213, section 9.3).

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | The field is mandatory present if txConfig is set to codebook and absent otherwise. |

#### –	PUSCH-ConfigCommon

The IE PUSCH-ConfigCommon IE is used to configure the cell specific PUSCH parameters.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIGCOMMON-START

PUSCH-ConfigCommon ::= 					SEQUENCE {

groupHoppingEnabledTransformPrecoding	ENUMERATED {enabled}													OPTIONAL,	-- Need R

pusch-TimeDomainAllocationList			PUSCH-TimeDomainResourceAllocationList								OPTIONAL,	-- Need R

msg3-DeltaPreamble						INTEGER (-1..6)															OPTIONAL,	-- Need R

p0-NominalWithGrant						INTEGER (-202..24)														OPTIONAL,	-- Need R

...

}

-- TAG-PUSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUSCH-ConfigCommon field descriptions

groupHoppingEnabledTransformPrecoding Sequence-group hopping can be enabled or disabled by means of this cell-specific parameter. Corresponds to L1 parameter 'Group-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) This field is Cell specific

msg3-DeltaPreamble Power offset between msg3 and RACH preamble transmission. Actual value = field value * 2 [dB]. Corresponds to L1 parameter 'Delta-preamble-msg3' (see 38.213, section 7.1)

p0-NominalWithGrant P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed. Corresponds to L1 parameter 'p0-nominal-pusch-withgrant' (see 38.213, section 7.1) This field is cell specific

pusch-AllocationList List of time domain allocations for timing of UL assignment to UL data

#### –	PUSCH-PowerControl

The IE PUSCH-PowerControl is used to configure UE specific power control parameter for PUSCH.

PUSCH-PowerControl information element

-- ASN1START

-- TAG-PUSCH-POWERCONTROL-START

PUSCH-PowerControl ::= 				SEQUENCE {

tpc-Accumulation					ENUMERATED { disabled }														OPTIONAL,	-- Need S

msg3-Alpha							Alpha																		OPTIONAL, 	-- Need S

p0-NominalWithoutGrant				INTEGER (-202..24)															OPTIONAL,	-- Need M,

p0-AlphaSets						SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet			OPTIONAL,	-- Need M,

pathlossReferenceRSToAddModList		SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS

OPTIONAL,	-- Need N

pathlossReferenceRSToReleaseList	SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id

OPTIONAL,	-- Need N

twoPUSCH-PC-AdjustmentStates		ENUMERATED {twoStates}														OPTIONAL, -- Need S

deltaMCS							ENUMERATED {enabled}														OPTIONAL, -- Need S

sri-PUSCH-MappingToAddModList		SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl	OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList		SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId	OPTIONAL -- Need N

}

-- A set of p0-pusch and alpha used for PUSCH with grant. 'PUSCH beam indication' (if present) gives the index of the set to

-- be used for a particular PUSCH transmission.

(see 38.213, section 7.1.1)

P0-PUSCH-AlphaSet ::= 				SEQUENCE {

p0-PUSCH-AlphaSetId 				P0-PUSCH-AlphaSetId,

p0									INTEGER (-16..15)															OPTIONAL,	-- Need S

alpha 								Alpha																		OPTIONAL	-- Need S

}

-- ID for a P0-PUSCH-AlphaSet. Corresponds to L1 parameter 'p0alphasetindex' (see 38.213, section 7.1)

P0-PUSCH-AlphaSetId ::= 			INTEGER (0..maxNrofP0-PUSCH-AlphaSets-1)

-- A reference signal (RS) configured as pathloss reference signal for PUSCH power control

-- Corresponds to L1 parameter 'pusch-pathlossReference-rs' (see 38.213, section 7.1)

PUSCH-PathlossReferenceRS ::=		SEQUENCE {

pusch-PathlossReferenceRS-Id 		PUSCH-PathlossReferenceRS-Id,

referenceSignal						CHOICE {

ssb-Index							SSB-Index,

csi-RS-Index						NZP-CSI-RS-ResourceId

}

}

-- ID for a referemce signal (RS) configured as PUSCH pathloss reference

-- Corresponds to L1 parameter 'pathlossreference-index' (see 38.213, section 7.1)

PUSCH-PathlossReferenceRS-Id ::=	INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)

-- A set of PUSCH power control parameters associated with one SRS-ResourceIndex (SRI)

SRI-PUSCH-PowerControl ::=			SEQUENCE {

sri-PUSCH-PowerControlId			SRI-PUSCH-PowerControlId,

sri-PUSCH-PathlossReferenceRS-Id	PUSCH-PathlossReferenceRS-Id,

sri-P0-PUSCH-AlphaSetId 			P0-PUSCH-AlphaSetId,

sri-PUSCH-ClosedLoopIndex			ENUMERATED { i0, i1 }

}

SRI-PUSCH-PowerControlId ::=		INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

-- A set of beta-offset values

BetaOffsets ::= 					SEQUENCE {

betaOffsetACK-Index1				INTEGER(0..31)															OPTIONAL, -- Need S

betaOffsetACK-Index2				INTEGER(0..31)															OPTIONAL, -- Need S

betaOffsetACK-Index3				INTEGER(0..31)															OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1			INTEGER(0..31)															OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2			INTEGER(0..31)															OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1			INTEGER(0..31)															OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2			INTEGER(0..31)															OPTIONAL -- Need S

}

-- TAG-PUSCH-POWERCONTROL-STOP

-- ASN1STOP

BetaOffsets field descriptions

betaOffsetACK-Index1 Up to 2 bits HARQ-ACK. Corresponds to L1 parameter 'betaOffset-ACK-Index-1' (see 38.213, section 9.3) When the field is absent the UE applies the value 11

betaOffsetACK-Index2 Up to 11 bits HARQ-ACK. Corresponds to L1 parameter 'betaOffset-ACK-Index-2' (see 38.213, section 9.3) When the field is absent the UE applies the value 11

betaOffsetACK-Index3 Above 11 bits HARQ-ACK. Corresponds to L1 parameter 'betaOffset-ACK-Index-3' (see 38.213, section 9.3) When the field is absent the UE applies the value 11

betaOffsetCSI-Part1-Index1 Up to 11 bits of CSI part 1 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-1-Index-1' (see 38.213, section 9.3) When the field is absent the UE applies the value 13

betaOffsetCSI-Part1-Index2 Above 11 bits of CSI part 1 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-1-Index-2' (see 38.213, section 9.3) When the field is absent the UE applies the value 13

betaOffsetCSI-Part2-Index1 Up to 11 bits of CSI part 2 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-2-Index-1' (see 38.213, section 9.3) When the field is absent the UE applies the value 13

betaOffsetCSI-Part2-Index2 Above 11 bits of CSI part 2 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-2-Index-2' (see 38.213, section 9.3) When the field is absent the UE applies the value 13

P0-PUSCH-AlphaSet field descriptions

alpha alpha value for PUSCH with grant (except msg3) (see 38.213, section 7.1) When the field is absent the UE applies the value 1

p0 P0 value for PUSCH with grant (except msg3) in steps of 1dB. The UE shall use P0-nominal when UE-specific P0 is not configured (p0 = 0). Corresponds to L1 parameter 'p0-pusch' (see 38,213, section 7.1)

PUSCH-PowerControl field descriptions

deltaMCS Indicates whether to apply dela MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH. Corresponds to L1 parameter 'deltaMCS-Enabled' (see 38.213, section 7.1)

msg3-Alpha Dedicated alpha value for msg3 PUSCH. Corresponds to L1 parameter 'alpha-ue-pusch-msg3' (see 38.213, section 7.1) When the field is absent the UE applies the value 1.

p0-AlphaSets configuration {p0-pusch,alpha} sets for PUSCH (except msg3), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...}. Corresponds to L1 parameter 'p0-push-alpha-setconfig' (see 38,213, section 7.1)

p0-NominalWithoutGrant P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed. Corresponds to L1 parameter 'p0-nominal-pusch-withoutgrant' (see 38.213, section 7.1)

pathlossReferenceRSToAddModList A set of Reference Signals (e.g. a CSI-RS config or a SSblock) to be used for PUSCH path loss estimation. Up to maxNrofPUSCH-PathlossReferenceRSs may be configured when 'PUSCH beam indication' is present. Otherwise, there may be only one entry. Corresponds to L1 parameter 'pusch-pathlossReference-rs-config' (see 38.213, section 7.1)

sri-PUSCH-MappingToAddModList A list of SRI-PUSCH-PowerControl elements among which one is selected by the SRI field in DCI. Corresponds to L1 parameter 'SRI-PUSCHPowerControl-mapping' (see 38.213, section 7.1)

tpc-Accumulation If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation. If the field is absent, TPC accumulation is enabled. Corresponds to L1 parameter 'Accumulation-enabled' (see 38.213, section 7.1)

twoPUSCH-PC-AdjustmentStates Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains two power control states (i.e., fc(i,1) and fc(i,2)). If the field is absent, it applies one (i.e., fc(i,1)). Corresponds to L1 parameter 'num-pusch-pcadjustment-states' (see 38.213, section 7.1)

SRI-PUSCH-PowerControl field descriptions

sri-P0-PUSCH-AlphaSetId The ID of a P0-PUSCH-AlphaSet as configured in p0-AlphaSets in PUSCH-PowerControl.

sri-PUSCH-ClosedLoopIndex The index of the closed power control loop associated with this SRI-PUSCH-PowerControl

sri-PUSCH-PathlossReferenceRS-Id The ID of PUSCH-PathlossReferenceRS as configured in the pathlossReferenceRSToAddModList in PUSCH-PowerControl.

sri-PUSCH-PowerControlId The ID of this SRI-PUSCH-PowerControl configuration. It is used as the codepoint (payload) in the SRI DCI field.

#### –	PUSCH-ServingCellConfig

The IE PUSCH-ServingCellConfig is used to configure UE specific PUSCH parameters that are common across the UE's BWPs of one serving cell.

PUSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PUSCH-SERVINGCELLCONFIG-START

PUSCH-ServingCellConfig ::=				SEQUENCE {

codeBlockGroupTransmission				SetupRelease { PUSCH-CodeBlockGroupTransmission	}							OPTIONAL,	-- Need M

rateMatching							ENUMERATED {limitedBufferRM}												OPTIONAL,	-- Need S

xOverhead								ENUMERATED {xoh6, xoh12, xoh18}												OPTIONAL,	-- Need S

...

}

PUSCH-CodeBlockGroupTransmission ::=	SEQUENCE {

maxCodeBlockGroupsPerTransportBlock		ENUMERATED {n2, n4, n6, n8},

...

}

-- TAG-PUSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PUSCH-CodeBlockGroupTransmission field descriptions

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) For 2 codewords, only the values { n2, n4 } are valid.

PUSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see 38.214, section FFS_Section)

rateMatching Enables LBRM (Limited buffer rate-matching). When the field is absent the UE applies FBRM (Full buffer rate-matchingLBRM). Corresponds to L1 parameter 'LBRM-FBRM-selection' (see 38.212, section 5.4.2)

xOverhead Accounts for overhead from CSI-RS, CORESET, etc. If the field is absent, the UE applies the value 'xoh0'. Corresponds to L1 parameter 'Xoh-PUSCH' (see 38.214, section 5.1.3.2)

#### –	PUSCH-TimeDomainResourceAllocationList

The IE PUSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PUSCH. PUSCH-TimeDomainResourceAllocationList contains one or more of such PUSCH-TimeDomainResourceAllocations. The network indicates in the UL grant which of the configued time domain allocations the UE shall apply for that UL grant. The UE determines the bit width of the DCI field based on the number of entries in the PUSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PUSCH-TimeDomainResourceAllocation information element

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::= 	SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::= 	SEQUENCE {

k2										INTEGER(0..32)			OPTIONAL,	-- Need S

mappingType								ENUMERATED {typeA, typeB},

startSymbolAndLength					INTEGER (0..127)

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PUSCH-TimeDomainResourceAllocationList field descriptions

k2 Corresponds to L1 parameter 'K2' (see 38.214, section 6.1.2.1) When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30KHz; 2 when PUSCH SCS is 60KHz and 3 when PUSCH SCS is 120KHz.

mappingType Mapping type. Corresponds to L1 parameter 'Mapping-type' (see 38.214, section 6.1.2.1)

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV) The network configures the field so that the allocation does not cross the slot boundary. (see 38.214, section 6.1.2.1)

#### –	PUSCH-TPC-CommandConfig

The IE PUSCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUSCH from a group-TPC messages on DCI.

PUSCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUSCH-TPC-COMMANDCONFIG-START

PUSCH-TPC-CommandConfig ::=			SEQUENCE {

tpc-Index							INTEGER (1..15)															OPTIONAL,	-- Cond SUL

tpc-IndexSUL						INTEGER (1..15)															OPTIONAL,	-- Cond SUL-Only

targetCell							ServCellIndex															OPTIONAL,	-- Need S

...

}

-- TAG-PUSCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUSCH-TPC-CommandConfig field descriptions

targetCell The serving cell to which the acquired power control commands are applicable. If the value is absent, the UE applies the TPC commands to the serving cell on which the command has been received.

tpc-Index An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

tpc-IndexSUL An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

| Conditional Presence | Explanation |
| --- | --- |
| SUL-Only | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink (SUL). It is absent otherwise. |
| SUL | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink (SUL). It is mandatory present otherwise. |

#### –	Q-OffsetRange

The IE Q-OffsetRange is used to indicate a cell, beam or measurement object specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

Q-OffsetRange information element

-- ASN1START

-- TAG-Q-OFFSET-START

Q-OffsetRange ::=					ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-Q-OFFSET-STOP

-- ASN1STOP

Editor’s Note: FFS Confirm the exact values that are supported.

#### –	Q-QualMin

The IE Q-QualMin is used to indicate for cell selection/ re-selection the required minimum received RSRQ level in the (NR) cell. Corresponds to parameter Qqualmin in TS 38.304 [4]. Actual value Qqualmin = field value [dB].

Q-QualMin information element

-- ASN1START

-- TAG-Q-QUALMIN-START

Q-QualMin ::=					INTEGER (-34..-3)		-- FFS range

-- TAG-Q-QUALMIN-STOP

-- ASN1STOP

#### –	Q-RxLevMin

The IE Q-RxLevMin is used to indicate for cell selection/ re-selection the required minimum received RSRP level in the (NR) cell. Corresponds to parameter Qrxlevmin in TS 38.304 [4]. Actual value Qrxlevmin = field value * 2 [dBm].

Q-RxLevMin information element

-- ASN1START

-- TAG-Q-RXLEVMIN-START

Q-RxLevMin ::=						INTEGER (-70..-22)		-- FFS range

-- TAG-Q-RXLEVMIN-STOP

-- ASN1STOP

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITY-CONFIG-START

QuantityConfig ::=					SEQUENCE {

quantityConfigNR-List				SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR			OPTIONAL,	-- Need M

...,

[[	quantityConfigEUTRA                    FilterConfig                                                           		OPTIONAL	-- Need M

]]

}

QuantityConfigNR::=					SEQUENCE {

quantityConfigCell					QuantityConfigRS,

quantityConfigRS-Index				QuantityConfigRS														OPTIONAL	-- Need M

}

QuantityConfigRS ::=				SEQUENCE {

ssb-FilterConfig					FilterConfig,

cs-RS-FilterConfig					FilterConfig

}

FilterConfig ::=					SEQUENCE {

filterCoefficientRSRP				FilterCoefficient										DEFAULT fc4,

filterCoefficientRSRQ				FilterCoefficient										DEFAULT fc4,

filterCoefficientRS-SINR			FilterCoefficient										DEFAULT fc4

}

-- TAG-QUANTITY-CONFIG-STOP

-- ASN1STOP

QuantityConfigNR field descriptions

quantityConfigCell Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

quantityConfigRS-Index Specifies L3 filter configurations for measurement results per RS index for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

QuantityConfigRS field descriptions

cs-RS-FilterConfig CSI-RS basedL3 filter configurations: Specifies L3 filter configurations for CSI-RSRP, CSI-RSRQ and CSI-SINR measurement results from the L1 filter(s), as defined in 38.215 [9].

ssb-FilterConfig SS Block based L3 filter configurations: Specifies L3 filter configurations for SS-RSRP, SS-RSRQ and SS-SINR measurement results from the L1 filter(s), as defined in 38.215 [9].

#### –	RACH-ConfigCommon

The RACH-ConfigCommon IE is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

rach-ConfigGeneric			RACH-ConfigGeneric,

totalNumberOfRA-Preambles			INTEGER (1..63)																		OPTIONAL,	-- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																														OPTIONAL,	-- Need M

groupBconfigured 					SEQUENCE {

ra-Msg3SizeGroupA					ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, spare7, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			INTEGER (1..64)

}																														OPTIONAL,	-- Need R

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB						RSRP-Range																		OPTIONAL,	-- Need R

rsrp-ThresholdSSB-SUL					RSRP-Range																		OPTIONAL,	-- Cond SUL

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

},

msg1-SubcarrierSpacing					SubcarrierSpacing														OPTIONAL,   -- Cond L139Need S

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder					ENUMERATED {enabled}															OPTIONAL,	-- Need R

...

}

-- TAG-RACH-CONFIG-COMMON-STOP

-- ASN1STOP

RACH-ConfigCommon field descriptions

messagePowerOffsetGroupB Threshold for preamble selection. Value in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see 38.321, section 5.1.2)

msg1-SubcarrierSpacing Subcarrier spacing of PRACH. Only the values 15 or 30 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section). If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1 and Table 6.3.3.2-2, 38.211). The value also applies to contention free random access (RACH-ConfigDedicated) but not for beam failure recovery (BeamFailureRecoveryConfig).

msg3-transformPrecoder Enables the transform precoder for Msg3 transmission. If the field is absent, the UE disables the transformer precoder..Corresponds to L1 parameter 'msg3-tp' (see 38.213, section 8.1)

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A. This determines implicitly the number of CB preambles per SSB available in group B. (see 38.321, section 5.1.1). The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

prach-RootSequenceIndex PRACH root sequence index. Corresponds to L1 parameter 'PRACHRootSequenceIndex' (see 38.211, section 6.3.3.1). The value range depends on whether L=839 or L=139. The short/long preamble format indicated in this IE should be consistent with the one indicated in prach-ConfigurationIndex in the RACH-ConfigDedicated (if configured).

ra-ContentionResolutionTimer The initial value for the contention resolution timer (see 38.321, section 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on.

ra-Msg3SizeGroupA Transport Blocks size threshold in bit below which the UE shall use a contention based RA premable of group A. (see 38.321, section 5.1.2)

rach-ConfigGeneric Generic RACH parameters

restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see 38.211 6.3.3.1

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see 38.213)

rsrp-ThresholdSSB-SUL The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321, section 5.1.1).

ssb-perRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion'). Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicatesthe number of Contention Based preambles per SSB (L1 parameter 'CB-preambles-per-SSB'). Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1,SSB-per-rach-occasion).

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free random access in the RACH resources defined in RACH-ConfigCommon, excluding preambles used for other purposes (e.g. for SI request). If the field is absent, the UE may use all 64 preambles for RA. The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, i.e. it should be a multiple of the number of SSBs per RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent. |
| SUL | The field is mandatory present in initialUplinkBWP if supplementaryUplinkConfig is present; otherwise, the field is absent. |

#### –	RACH-ConfigGeneric

The RACH-ConfigGeneric IE is used to specify the cell specific random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigGeneric information element

-- ASN1START

-- TAG-RACH-CONFIG-GENERIC-START

RACH-ConfigGeneric ::= 				SEQUENCE {

prach-ConfigurationIndex			INTEGER (0..255),

msg1-FDM							ENUMERATED {one, two, four, eight},

msg1-FrequencyStart					INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig			INTEGER(0..15),

preambleReceivedTargetPower			INTEGER (-202..-60),

preambleTransMax					ENUMERATED {n3, n4, n5, n6, n7,	n8, n10, n20, n50, n100, n200},

powerRampingStep					ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow					ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...

}

-- TAG-RACH-CONFIG-GENERIC-STOP

-- ASN1STOP

RACH-ConfigGeneric field descriptions

msg1-FDM The number of PRACH transmission occasions FDMed in one time instance. (see 38.211, section 6.3.3.2)

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see 38,211, section 6.3.3.2)

powerRampingStep Power ramping steps for PRACH (see 38.321,5.1.3)

prach-ConfigurationIndex PRACH configuration index. For prach-ConfigurationIndex configured under beamFailureRecovery-Config, the prach-ConfigurationIndex can only correspond to the short preamble format. Corresponds to L1 parameter 'PRACHConfigurationIndex' (see 38.211, section 6.3.3.2)

preambleReceivedTargetPower The target power level at the network receiver side (see 38.213, section 7.4, 38.321, section 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...).

preambleTransMax Max number of RA preamble transmission perfomed before declaring a failure (see 38.321, section 5.1.4, 5.1.5)

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or euqal to 10 ms (see 38.321, section 5.1.4)

zeroCorrelationZoneConfig N-CS configuration, see Table 6.3.3.1-5 in 38.211

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIG-DEDICATED-START

RACH-ConfigDedicated ::=		SEQUENCE {

cfra							CFRA																	OPTIONAL,	-- Need N

ra-Prioritization				RA-Prioritization																OPTIONAL,	--Need N

...

}

CFRA ::=					SEQUENCE {

occasions						SEQUENCE {

rach-ConfigGeneric				RACH-ConfigGeneric,

ssb-perRACH-Occasion			ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}	OPTIONAL	-- Cond SSB-CFRA

}																												OPTIONAL,	-- Need S

resources						CHOICE {

ssb								SEQUENCE {

ssb-ResourceList				SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex		INTEGER (0..15)

},

csirs							SEQUENCE {

csirs-ResourceList				SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS			RSRP-Range

}

},

...

}

CFRA-SSB-Resource ::= 			SEQUENCE {

ssb								SSB-Index,

ra-PreambleIndex				INTEGER (0..63),

...

}

CFRA-CSIRS-Resource ::= 		SEQUENCE {

csi-RS							CSI-RS-Index,

ra-OccasionList					SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex				INTEGER (0..63),

...

}

-- TAG-RACH-CONFIG-DEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions assoicated with this CSI-RS.

CFRA field descriptions

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 36.321. The mask is valid for all SSB resources signalled in ssb-ResourceList

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA.

ssb-perRACH-Occasion Number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion').

CFRA-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

RACH-ConfigDedicated field descriptions

cfra Parametersfor contention free random access to a given target cell. If the field is absent, the UE performs contention based random access.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see 38.321, section 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| SSB-CFRA | The field is mandatory present if the field resources in CFRA is set to ssb; otherwise it is not present. |

#### –	RA-Prioritization

The IE RA-Prioritization is used to configure prioritized random access.

RA-Prioritization information element

-- ASN1START

-- TAG-RA-PRIORITIZATION-START

RA-Prioritization ::=			SEQUENCE {

powerRampingStepHighPriority		ENUMERATED {dB0, dB2, dB4, dB6},

scalingFactorBI					ENUMERATED {zero, dot25, dot5, dot75}								OPTIONAL,	-- Need R

...

}

-- TAG-RA-PRIORITIZATION-STOP

-- ASN1STOP

RA-Prioritization field descriptions

powerRampingStepHighPrioritiy Power ramping step applied for prioritized random access procedure.

scalingFactorBI Scaling factor for the backoff indicator (BI) for the prioritized random access procedure. (see 38,321, section 5.1.4). Value zero corresponds to 0, value dot25 corresponds to 0.25 and so on.

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=					SEQUENCE {

srb-ToAddModList						SRB-ToAddModList										OPTIONAL,	-- Cond HO-Conn

srb3-ToRelease							ENUMERATED{true}										OPTIONAL,	-- Need N

drb-ToAddModList						DRB-ToAddModList										OPTIONAL,	-- Cond HO-toNR

drb-ToReleaseList						DRB-ToReleaseList										OPTIONAL,	-- Need N

securityConfig 							SecurityConfig											OPTIONAL,	-- Need M

...

}

SRB-ToAddModList ::=					SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=						SEQUENCE {

srb-Identity							SRB-Identity,

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 	-- Need N

discardOnPDCP                           ENUMERATED{true}										OPTIONAL,	-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,	-- Cond PDCP

...

}

DRB-ToAddModList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=						SEQUENCE {

cnAssociation							CHOICE {

eps-BearerIdentity						INTEGER (0..15),									-- EPS-DRB-Setup

sdap-Config								SDAP-Config											-- 5GC

} 										OPTIONAL, -- Cond DRBSetup

drb-Identity							DRB-Identity,

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 	-- Need N

recoverPDCP								ENUMERATED{true}										OPTIONAL, 	-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,	-- Cond PDCP

...

}

DRB-ToReleaseList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=						SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig									OPTIONAL,	-- Cond RBTermChange

keyToUse								ENUMERATED{master, secondary}								OPTIONAL,	-- Cond RBTermChange

...

}

-- TAG-RADIO-BEARER-CONFIG-STOP

-- ASN1STOP

DRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to TRUE whenever the security key used for this radio bearer changes, resuming an RRC connection, or the first reconfiguration after reestablishment. It may only be set if the cell groups of all linked logical channels are reset or released.

recoverPDCP Indicates that PDCP should perform recovery according to TS38.323.

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this radioBearerConfig When the field is not included, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this radioBearerConfig.

srb3-ToRelease Release SRB3. SRB3 release can only be done at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this radioBearerConfig is using the master keyor the secondary key for deriving ciphering and/or integrity protection keys. For EN-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this radioBearerConfig. If EN-DC is not configured, this field is set to master.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this radioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this radioBearerConfig.

| SRB-ToAddMod field descriptions |  |
| --- | --- |
| discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS38.323. |
| reestablishPDCP Indicates that PDCP should be re-established. Network sets this to TRUE whenever the security key used for this radio bearer changes, resuming an RRC connection, or the first reconfiguration after reestablishment. It may only be set if the cell groups of all linked logical channels are reset or released. |  |
| srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only. |  |

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of set up of signalling and data radio bearer and change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB is being setup or corresponding RB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| interSystemHO | This field is mandatory present in case of inter system handover, and optionally present in case of NG based HO. Otherwise, the field is absent. |
| HO-Conn | The field is mandatory present in case of inter-system handover from E-UTRA (connected to EPC) to E-UTRA (connected to 5GC) or NR, or when the fullConfig is included in the RRCReconfiguration message, or in case of RRCSetup; otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | The field is mandatory present in case of inter-system handover from E-UTRA (connected to EPC) to E-UTRA (connected to 5GC) or NR, or when the fullConfig is included in the RRCReconfiguration message. In case of RRCSetup and RRCReestablishment, the field is not present; otherwise the field is optionally present, need N. |

#### –	RadioLinkMonitoringConfig

The RadioLinkMonitoringConfig IE is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also 38.321, section 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=		SEQUENCE {

failureDetectionResourcesToAddModList	SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS	OPTIONAL, 	-- Need N

failureDetectionResourcesToReleaseList	SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id	OPTIONAL,-- Need N

beamFailureInstanceMaxCount				ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}									OPTIONAL,	-- Need S

beamFailureDetectionTimer      			ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}			OPTIONAL,	-- Need R

...

}

RadioLinkMonitoringRS ::=			SEQUENCE {

radioLinkMonitoringRS-Id				RadioLinkMonitoringRS-Id,

purpose								ENUMERATED {beamFailure, rlf, both},

detectionResource					CHOICE {

ssb-Index							SSB-Index,

csi-RS-Index						NZP-CSI-RS-ResourceId

},

...

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

beamFailureDetectionTimer Timer for beam failure detection (see 38.321, section 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "periods of Beam Failure Detection" Reference Signal. Value pbfd1 corresponds to 1 period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount This field determines after how many beam failure events the UE triggers beam failure recovery (see 38.321, section 5.17). Value n1 corresponds to 1 beam failure instance, n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The network configures at most two detectionResources per BWP for the purpose "beamFailure" or "both". If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213, section 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCHWhen the RS(s) for beam failure detection (BFD) is reconfigured by the network, the UE resets the on-going beamFailureDetectionTimerand the counter related to beamFailureInstanceMaxCount.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose).

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection.

#### –	RadioLinkMonitoringRSId

The IE RadioLinkMonitoringRSId is used to identify one RadioLinkMonitoringRS.

RadioLinkMonitoringRSId information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGRSID-START

RadioLinkMonitoringRS-Id ::= 			INTEGER (0..maxNrofFailureDetectionResources-1)

-- TAG-RADIOLINKMONITORINGRSID-STOP

-- ASN1STOP

#### –	RAN-AreaCode

The IE RAN-AreaCode is used to identify a RAN area within the scope of a tracking area.

RAN-AreaCode information element

-- ASN1START

-- TAG-RAN-AREACODE-START

RAN-AreaCode ::=				INTEGER (0..255)

-- TAG-RAN-AREACODE-STOP

-- ASN1STOP

#### –	RateMatchPattern

The IE RateMatchPattern is used to configure one rate matching pattern for PDSCH. Corresponds to L1 IE 'rate-match-PDSCH-resource-set', see 38.214, section FFS_Section.

RateMatchPattern information element

-- ASN1START

-- TAG-RATEMATCHPATTERN-START

RateMatchPattern ::= 				SEQUENCE {

rateMatchPatternId					RateMatchPatternId,

patternType							CHOICE {

bitmaps								SEQUENCE {

resourceBlocks						BIT STRING (SIZE (275)),

symbolsInResourceBlock				CHOICE {

oneSlot								BIT STRING (SIZE (14)),

twoSlots							BIT STRING (SIZE (28))

},

periodicityAndPattern				CHOICE {

n2									BIT STRING (SIZE (2)),

n4									BIT STRING (SIZE (4)),

n5									BIT STRING (SIZE (5)),

n8									BIT STRING (SIZE (8)),

n10									BIT STRING (SIZE (10)),

n20									BIT STRING (SIZE (20)),

n40									BIT STRING (SIZE (40))

}																										OPTIONAL, 	-- Need S

...

},

controlResourceSet					ControlResourceSetId

},

subcarrierSpacing					SubcarrierSpacing															OPTIONAL,	-- Cond CellLevel

mode								ENUMERATED { dynamic, semiStatic },

...

}

-- TAG-RATEMATCHPATTERN-STOP

-- ASN1STOP

RateMatchPattern field descriptions

controlResourceSet This ControlResourceSet us used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it.

mode FFS_Description, FFS_Section

periodicityAndPattern A time domain repetition pattern. at which the symbolsInResourceBlock pattern recurs. This slot pattern repeats itself continuously. Absence of this field indicates the value n1, i.e., the symbolsInResourceBlock recurs every 14 symbols. Corresponds to L1 parameter 'rate-match-PDSCH-bitmap3' (see 38.214, section 5.1.4.1)

resourceBlocks A resource block level bitmap in the frequency domain. A bit in the bitmap set to 1 indicates that the UE shall apply rate matching in the corresponding resource block in accordance with the symbolsInResourceBlock bitmap. If used as cell-level rate matching pattern, the bitmap identifies “common resource blocks (CRB)”. If used as BWP-level rate matching pattern, the bitmap identifies “physical resource blocks” inside the BWP. The first/ leftmost bit corresponds to resource block 0, and so on. Corresponds to L1 parameter 'rate-match-PDSCH-bitmap1' (see 38.214, section 5.1.4.1)

subcarrierSpacing The SubcarrierSpacing for this resource pattern. If the field is absent, the UE applies the SCS of the associcated BWP. The value kHz15 corresponds to µ=0, kHz30 to µ=1, and so on. Only the values 15 or 30 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. Corresponds to L1 parameter 'resource-pattern-scs' (see 38.214, section 5.1.4.1)

symbolsInResourceBlock A symbol level bitmap in time domain. It indicates with a bit set to true that the UE shall rate match around the corresponding symbol.. The first/left-most bit in the bitmap corresponds to the first symbol in the slot, and so on. This pattern recurs (in time domain) with the configured periodicityAndOffset. Corresponds to L1 parameter 'rate-match-PDSCH-bitmap2' (see 38.214, section 5.1.4.1)

| Conditional Presence | Explanation |
| --- | --- |
| CellLevel | The field is mandatory present if the RateMatchPattern is defined on cell level. The field is absent when the RateMatchPattern is defined on BWP level. If the RateMatchPattern is defined on BWP level, the UE applies the SCS of the BWP. |

#### –	RateMatchPatternId

The IE RateMatchPatternId identifies one RateMatchMattern. Corresponds to L1 parameter 'resource-set-index' (see 38.214, section 5.1.4.2)

RateMatchPatternId information element

-- ASN1START

-- TAG-RATEMATCHPATTERNID-START

RateMatchPatternId ::=				INTEGER (0..maxNrofRateMatchPatterns-1)

-- TAG-RATEMATCHPATTERNID-STOP

-- ASN1STOP

#### –	RateMatchPatternLTE-CRS

The IE RateMatchPatternLTE-CRS is used to configure a pattern to rate match around LTE CRS. See TS 38214 Section 5.1.4.2.

RateMatchPatternLTE-CRS information element

-- ASN1START

-- TAG-RATEMATCHPATTERNLTE-CRS-START

RateMatchPatternLTE-CRS ::=			SEQUENCE {

carrierFreqDL						INTEGER (0..16383),

carrierBandwidthDL					ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1},

mbsfn-SubframeConfigList			EUTRA-MBSFN-SubframeConfigList												OPTIONAL,	-- Need M

nrofCRS-Ports						ENUMERATED {n1, n2, n4},

v-Shift								ENUMERATED {n0, n1, n2, n3, n4, n5}

}

-- TAG-RATEMATCHPATTERNLTE-CRS-STOP

-- ASN1STOP

RateMatchPatternLTE-CRS field descriptions

carrierBandwidthDL BW of the LTE carrier in numbewr of PRBs. Corresponds to L1 parameter 'BW' (see 38.214, section 5.1.4.2)

carrierFreqDL Center of the LTE carrier. Corresponds to L1 parameter 'center-subcarrier-location' (see 38.214, section 5.1.4.2)

mbsfn-SubframeConfigList LTE MBSFN subframe configuration. Corresponds to L1 parameter 'MBSFN-subframconfig' (see 38.214, section 5.1.4.2) FFS_ASN1: Import the LTE MBSFN-SubframeConfigList

nrofCRS-Ports Number of LTE CRS antenna port to rate-match around. Corresponds to L1 parameter 'rate-match-resources-numb-LTE-CRS-antenna-port' (see 38.214, section 5.1.4.2)

v-Shift Shifting value v-shift in LTE to rate match around LTE CRS Corresponds to L1 parameter 'rate-match-resources-LTE-CRS-v-shift' (see 38.214, section 5.1.4.2)

#### –	ReportConfigId

The IE ReportConfigId is used to identify a measurement reporting configuration.

ReportConfigId information element

-- ASN1START

-- TAG-REPORT-CONFIG-ID-START

ReportConfigId ::=							INTEGER (1..maxReportConfigId)

-- TAG-REPORT-CONFIG-ID-STOP

-- ASN1STOP

#### –	ReportConfigInterRAT

The IE ReportConfigInterRAT specifies criteria for triggering of an inter-RAT measurement reporting event. The inter-RAT measurement reporting events for EUTRA are labelled BN with N equal to 1, 2 and so on.

Event B1:	Neighbour becomes better than absolute threshold;

Event B2:	PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2;

ReportConfigInterRAT information element

-- ASN1START

-- TAG-REPORT-CONFIG-INTER-RAT-START

ReportConfigInterRAT ::=					SEQUENCE {

reportType									CHOICE {

periodical									PeriodicalReportConfigInterRAT,

eventTriggered								EventTriggerConfigInterRAT,

reportCGI									ReportCGI-EUTRA,

...

}

}

ReportCGI-EUTRA ::=							 SEQUENCE {

cellForWhichToReportCGI			EUTRA-PhysCellId,

...

}

EventTriggerConfigInterRAT ::=				SEQUENCE {

eventId										CHOICE {

eventB1										SEQUENCE {

b1-ThresholdEUTRA							ThresholdEUTRA,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

...

},

eventB2										SEQUENCE {

b2-Threshold1								MeasTriggerQuantity,

b2-Threshold2EUTRA							ThresholdEUTRA,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

...

},

...

},

rsType							 	NR-RS-Type,

triggerQuantity						ENUMERATED {rsrp, rsrq, sinr},

reportInterval								ReportInterval,

reportAmount						ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity						MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

...

}

PeriodicalReportConfigInterRAT ::=				SEQUENCE {

reportInterval									ReportInterval,

reportAmount									ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity  								MeasReportQuantity,

maxReportCells									INTEGER (1..maxCellReport),

...

}

ThresholdEUTRA ::=							CHOICE {

rsrp										RSRP-RangeEUTRA,

rsrq										RSRQ-RangeEUTRA,

sinr										SINR-RangeEUTRA

}

RSRP-RangeEUTRA ::=					INTEGER (0..97)

RSRQ-RangeEUTRA ::=					INTEGER (0..34)

SINR-RangeEUTRA ::=					INTEGER (0..127)

-- TAG-REPORT-CONFIG-INTER-RAT-STOP

-- ASN1STOP

EventTriggerConfigInterRAT field descriptions

b2-Threshold1 NR threshold to be used in inter RAT measurement report triggering condition for event b2.

bN-ThresholdEUTRA E-UTRA threshold to be used in inter RAT measurement report triggering condition for event number bN.

eventId Choice of inter RAT event triggered reporting criteria.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantity The cell measurement quantities to be included in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

triggerQuantity The quantity used to evaluate the triggering condition for the event. Set the value according to the quantity of the bN-ThresholdEUTRA for this event. The values rsrp, rsrq and sinr correspond to Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ) and Reference Signal Signal to Noise and Interference Ratio (RS-SINR), see TS 38.214 [19].

PeriodicalReportConfigInterRAT field descriptions

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantityCell The cell measurement quantities to be included in the measurement report.

ThresholdEUTRA field descriptions

EUTRA-RSRP Corresponds to RSRP-Range in TS 36.331 [10].

EUTRA-RSRQ Corresponds to RSRQ-Range in TS 36.331 [10].

EUTRA-SINR Corresponds to RS-SINR-Range in TS 36.331 [10].

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event. Measurement reporting events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS. These events are labelled AN with N equal to 1, 2 and so on.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2.

Event A6:	Neighbour becomes amount of offset better than SCell.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORT-CONFIG-START

ReportConfigNR ::=							SEQUENCE {

reportType									CHOICE {

periodical									PeriodicalReportConfig,

eventTriggered								EventTriggerConfig,

-- reportCGI is to be completed before the end of Rel-15.

...,

[[

cellForWhichToReportCGI						PhysCellId

]]

}

}

-- FFS / TODO: Consider separating trgger configuration (trigger, periodic, ...) from report configuration.

-- Current structure allows easier definiton of new events and new report types e.g. CGI, etc.

EventTriggerConfig::=						SEQUENCE {

eventId										CHOICE {

eventA1										SEQUENCE {

a1-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger

},

eventA2										SEQUENCE {

a2-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger

},

eventA3										SEQUENCE {

a3-Offset									MeasTriggerQuantityOffset,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA4										SEQUENCE {

a4-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA5										SEQUENCE {

a5-Threshold1								MeasTriggerQuantity,

a5-Threshold2								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA6										SEQUENCE {

a6-Offset									MeasTriggerQuantityOffset,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

...

},

rsType										NR-RS-Type,

reportInterval								ReportInterval,

reportAmount								ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell							MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL,	-- Need R

maxNrofRSIndexesToReport					INTEGER (1..maxNrofIndexesToReport) 								OPTIONAL,	-- Need R

includeBeamMeasurements						BOOLEAN,

reportAddNeighMeas							ENUMERATED {setup}													OPTIONAL,	-- Need R

...

}

PeriodicalReportConfig ::=					SEQUENCE {

rsType										NR-RS-Type,

reportInterval								ReportInterval,

reportAmount								ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell							MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL,	-- Need R

maxNrofRsIndexesToReport					INTEGER (1..maxNrofIndexesToReport) 								OPTIONAL,	-- Need R

includeBeamMeasurements						BOOLEAN,

useWhiteCellList							BOOLEAN,

...

}

NR-RS-Type ::= 								ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=						CHOICE {

rsrp										RSRP-Range,

rsrq										RSRQ-Range,

sinr										SINR-Range

}

MeasTriggerQuantityOffset ::=				CHOICE {

rsrp										INTEGER (-30..30),

rsrq										INTEGER (-30..30),

sinr										INTEGER (-30..30)

}

MeasReportQuantity ::=						SEQUENCE {

rsrp										BOOLEAN,

rsrq										BOOLEAN,

sinr										BOOLEAN

}

-- TAG-REPORT-CONFIG-START

-- ASN1STOP

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6.The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. The network configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5.

eventId Choice of NR event triggered reporting criteria.

maxNrofRsIndexesToReport Max number of measurement information per RS index to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall includes the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRsIndexes Indicates which measurement information per RS index the UE shall include in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

PeriodicalReportConfig field descriptions

maxNrofRsIndexesToReport Max number of measurement information per RS index to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRsIndexes Indicates which measurement information per RS index the UE shall include in the measurement report.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

#### –	ReportConfigToAddModList

The IE ReportConfigToAddModList concerns a list of reporting configurations to add or modify.

ReportConfigToAddModList information element

-- ASN1START

-- TAG-REPORT-CONFIG-TO-ADD-MOD-LIST-START

ReportConfigToAddModList ::=		SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod

ReportConfigToAddMod ::=			SEQUENCE {

reportConfigId						ReportConfigId,

reportConfig						CHOICE {

reportConfigNR						ReportConfigNR,

...,

reportConfigInterRAT				ReportConfigInterRAT

}

}

-- TAG- REPORT-CONFIG-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

#### –	ReportInterval

The ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1), for triggerTypeevent as well as for triggerTypeperiodical. Value ms120 corresponds to 120 ms, ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

ReportInterval ::=					ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

-- ASN1STOP

#### –	ReselectionThreshold

ReselectionThreshold is used to indicate an Rx level threshold for cell reselection. Actual value of threshold = field value * 2 [dB].

ReselectionThreshold information element

-- ASN1START

-- TAG-RESELECTION-THRESHOLD-START

ReselectionThreshold ::=				INTEGER (0..31)

-- TAG-RESELECTION-THRESHOLD-STOP

-- ASN1STOP

#### –	ReselectionThresholdQ

The IE ReselectionThresholdQ is used to indicate a quality level threshold for cell reselection. Actual value of threshold = field value [dB].

ReselectionThresholdQ information element

-- ASN1START

-- TAG-RESELECTION-THRESHOLDQ-START

ReselectionThresholdQ ::=			INTEGER (0..31)

-- TAG-RESELECTION-THRESHOLDQ-STOP

-- ASN1STOP

#### –	RLC-BearerConfig

The IE RLC-BearerConfig is used to configure an RLC entity, a corresponding logical channel in MAC and the linking to a PDCP entity (served radio bearer).

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=						SEQUENCE {

logicalChannelIdentity						LogicalChannelIdentity,

servedRadioBearer							CHOICE {

srb-Identity								SRB-Identity,

drb-Identity								DRB-Identity

}																											OPTIONAL,	-- Cond LCH-SetupOnly

reestablishRLC								ENUMERATED {true}												OPTIONAL, 	-- Need N

rlc-Config									RLC-Config														OPTIONAL,	-- Cond LCH-Setup

mac-LogicalChannelConfig					LogicalChannelConfig											OPTIONAL,	-- Cond LCH-Setup

...

}

-- TAG-RLC-BEARERCONFIG-STOP

-- ASN1STOP

RLC-BearerConfig field descriptions

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer.

servedRadioBearer Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling restrictions ('moreThanOneRLC' in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present, Need M, upon creation of a new logical channel. It is optionally present otherwise. |
| LCH-SetupOnly | This field is mandatory present, Need M, upon creation of a new logical channel. It is absent otherwise. |

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=						CHOICE {

am									SEQUENCE {

ul-AM-RLC							UL-AM-RLC,

dl-AM-RLC							DL-AM-RLC

},

um-Bi-Directional					SEQUENCE {

ul-UM-RLC							UL-UM-RLC,

dl-UM-RLC							DL-UM-RLC

},

um-Uni-Directional-UL				SEQUENCE {

ul-UM-RLC							UL-UM-RLC

},

um-Uni-Directional-DL				SEQUENCE {

dl-UM-RLC							DL-UM-RLC

},

...

}

UL-AM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthAM									OPTIONAL,	-- Cond Reestab

t-PollRetransmit					T-PollRetransmit,

pollPDU								PollPDU,

pollByte							PollByte,

maxRetxThreshold					ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthAM									OPTIONAL,	-- Cond Reestab

t-Reassembly						T-Reassembly,

t-StatusProhibit					T-StatusProhibit

}

UL-UM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthUM									OPTIONAL	-- Cond Reestab

}

DL-UM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthUM									OPTIONAL,	-- Cond Reestab

t-Reassembly						T-Reassembly

}

T-PollRetransmit ::=				ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, spare5, spare4, spare3,

spare2, spare1}

PollPDU ::=							ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384, p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=						ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=					ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=				ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=				ENUMERATED {size6, size12}

SN-FieldLengthAM ::=				ENUMERATED {size12, size18}

-- TAG-RLC-CONFIG-STOP

-- ASN1STOP

RLC-Configfield descriptions

maxRetxThreshold Parameter for RLC AM in TS 38.322 [4]. Value t1 corresponds to 1 retransmission, t2 to 2 retransmissions and so on.

pollByte Parameter for RLC AM in TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, kB50 to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.

pollPDU Parameter for RLC AM in TS 38.322 [4]. Value p4 corresponds to 4 PDUs, p8 to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.

sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, size12 means 12 bits, size18 means 18 bits.The value of sn-FieldLength for a DRB shall be changed only using reconfiguration with sync.

t-PollRetransmit Timer for RLC AM inTS 38.322 [4], in milliseconds. Value ms5 means 5ms, ms10 means 10ms and so on.

t-Reassembly Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0ms, ms5 means 5ms and so on.

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0ms, ms5 means 5ms and so on.

| Conditional Presence | Explanation |
| --- | --- |
| Reestab | The field is mandatory present at bearer setup. It is optionally present, need M, at RLC re-establishment. Otherwise it is not present. |

#### –	RLF-TimersAndConstants

Editor’s Note: FFS / TODO: Insert the RLF timers and related functionality. Check what is needed for EN-DC.

The RLF-TimersAndConstants IE is used to configure UE specific timers and constants.

RLF-TimersAndConstants information element

-- ASN1START

-- TAG-RLF-TIMERS-AND-CONSTANTS-START

RLF-TimersAndConstants ::= 			SEQUENCE {

t310								ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310								ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311								ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...

}

-- TAG-RLF-TIMERS-AND-CONSTANTS-STOP

-- ASN1STOP

RLF-TimersAndConstants field descriptions

n3xy Constants are described in section 7.3.n1 corresponds with 1, n2 corresponds to 2 and so on.

t3xy Timers are described in section 7.1. Value ms0 corresponds with 0 ms, ms50 corresponds to 50 ms and so on.

#### –	RNTI-Value

The RNTI-ValueIE represents a Radio Network Temporary Identity.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=						INTEGER (0..65535)

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

#### –	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. Integer value for RSRP measurements according to mapping table in TS 38.133 [14].

RSRP-Range information element

-- ASN1START

-- TAG-RSRP-RANGE-START

RSRP-Range ::=						INTEGER(0..127)

-- TAG-RSRP-RANGE-STOP

-- ASN1STOP

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. Integer value for RSRQ measurements is according to mapping table in TS 38.133 [14].

RSRQ-Range information element

-- ASN1START

-- TAG-RSRQ-RANGE-START

RSRQ-Range ::=						INTEGER(0..127)

-- TAG-RSRQ-RANGE-STOP

-- ASN1STOP

#### –	SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell.The value range is shared across the Cell Groups.

SCellIndex information element

-- ASN1START

-- TAG-SCELL-INDEX-START

SCellIndex ::=						INTEGER (1..31)

-- TAG-SCELL-INDEX-STOP

-- ASN1STOP

#### –	SchedulingRequestConfig

The IE SchedulingRequestConfig is used to configure the parameters, for the dedicated scheduling request (SR) resources.

SchedulingRequestConfig information element

-- ASN1START

-- TAG-SCHEDULING-REQUEST-CONFIG-START

SchedulingRequestConfig ::= 		SEQUENCE {

schedulingRequestToAddModList		SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod		OPTIONAL, -- Need N

schedulingRequestToReleaseList		SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestId			OPTIONAL-- Need N

}

SchedulingRequestToAddMod ::=		SEQUENCE {

schedulingRequestId		SchedulingRequestId,

sr-ProhibitTimer					ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128}							OPTIONAL,	-- Need S

sr-TransMax							ENUMERATED { n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

-- TAG-SCHEDULING-REQUEST-CONFIG-STOP

-- ASN1STOP

SchedulingRequestConfig field descriptions

schedulingRequestToAddModList List of Scheduling Request configurations to add or modify.

schedulingRequestToReleaseList List of Scheduling Request configurations to release

SchedulingRequestToAddMod field descriptions

schedulingRequestId Used to modify a SR configuration and to indicate, in LogicalChannelConfig, the SR configuration to which a logical channel is mapped and to indicate, in SchedulingRequestresourceConfig, the SR configuration for which a scheduling request resource is used.

sr-ProhibitTimer Timer for SR transmission on PUCCH in TS 38.321 [3]. Value in ms. ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on. When the field is absent, the UE applies the value 0.

sr-TransMax Maximum number of SR transmissions as described in 38.321 [3]. n4 corresponds to 4, n8 corresponds to 8, and so on.

#### –	SchedulingRequestId

The IE SchedulingRequestId is used to identify a Scheduling Request instance in the MAC layer.

SchedulingRequestId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTID-START

SchedulingRequestId ::=				INTEGER (0..7)

-- TAG-SCHEDULINGREQUESTID-STOP

-- ASN1STOP

#### –	SchedulingRequestResourceConfig

The IE SchedulingRequestResourceConfig determines physical layer resources on PUCCH where the UE may send the dedicated scheduling request (D-SR) (see 38.213, section 9.2.4).

SchedulingRequestResourceConfig information element

-- ASN1START

-- TAG-SCHEDULING-REQUEST-RESOURCE-CONFIG-START

SchedulingRequestResourceConfig ::=		SEQUENCE {

schedulingRequestResourceId				SchedulingRequestResourceId,

schedulingRequestID						SchedulingRequestId,

periodicityAndOffset					CHOICE {

sym2									NULL,

sym6or7									NULL,

sl1										NULL,						-- Recurs in every slot

sl2										INTEGER (0..1),

sl4										INTEGER (0..3),

sl5										INTEGER (0..4),

sl8										INTEGER (0..7),

sl10									INTEGER (0..9),

sl16									INTEGER (0..15),

sl20									INTEGER (0..19),

sl40									INTEGER (0..39),

sl80									INTEGER (0..79),

sl160									INTEGER (0..159),

sl320									INTEGER (0..319),

sl640									INTEGER (0..639)

}																													OPTIONAL,	-- Need M

resource								PUCCH-ResourceId															OPTIONAL	-- Need M

}

-- TAG-SCHEDULING-REQUEST-RESOURCE-CONFIG-STOP

-- ASN1STOP

SchedulingRequestResourceConfig field descriptions

periodicityAndOffset SR periodicity and offset in number of slots. Corresponds to L1 parameter 'SR-periodicity' and 'SR-offset' (see 38.213, section 9.2.2) The following periodicities may be configured depending on the chosen subcarrier spacing: SCS = 15 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl SCS = 30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl, 160sl SCS = 60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 16sl, 20sl, 40sl, 80sl, 160sl, 320sl SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, sl640 sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols. For periodicities sym2, sym7 and sl1 the UE assumes an offset of 0 slots.

resource ID of the PUCCH resource in which the UE shall send the scheduling request. The actual PUCCH-Resource is configured in PUCCH-Config of the same UL BWP and serving cell as this SchedulingRequestResourceConfig. The network configures a PUCCH-Resource of PUCCH-format0 or PUCCH-format1 (other formats not supported). Corresponds to L1 parameter 'SR-resource' (see 38.213, section 9.2.2)

schedulingRequestID The ID of the SchedulingRequestConfig that uses this scheduling request resource.

#### –	SchedulingRequestResourceId

The IE SchedulingRequestResourceId is used to identify scheduling request resources on PUCCH.

SchedulingRequestResourceId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCEID-START

SchedulingRequestResourceId ::=		INTEGER (1..maxNrofSR-Resources)

-- TAG-SCHEDULINGREQUESTRESOURCEID-STOP

-- ASN1STOP

#### –	ScramblingId

The IE ScramblingID is used for scrambling channels and reference signals.

-- ASN1START

-- TAG-SCRAMBLING-ID-START

ScramblingId ::=					INTEGER(0..1023)

-- TAG-SCRAMBLING-ID-STOP

-- ASN1STOP

#### –	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

-- ASN1START

-- TAG-SCS-SPECIFIC-CARRIER-START

SCS-SpecificCarrier ::=				SEQUENCE {

offsetToCarrier						INTEGER (0..2199),

subcarrierSpacing					SubcarrierSpacing,

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation			INTEGER (0..4095) 										OPTIONAL			-- Need S

]]

}

-- TAG-SCS-SPECIFIC-CARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) Corresponds to L1 parameter 'BW' (see 38.211, section FFS_Section)

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. Corresponds to L1 parameter 'offset-pointA-low-scs' (see 38.211, section FFS_Section)

txDirectCurrentLocation Indicates the downlink Tx Direct Current location for the carrier. A value in the range 0..3299 indicates the subcarrier index within the carrier. The values in the value range 3301..4095 are reserved and ignored by the UE. If this field is absent, the UE assumes the default value of 3300 (i.e. "Outside the carrier").

subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the values 15 or 30 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. The network configures all SCSs of configured BWPs configured in this serving cell.Corresponds to L1 parameter 'ref-scs' (see 38.211, section FFS_Section)

| Conditional Presence | Explanation |
| --- | --- |
| OnePerServCell | This field must be present for exactly one SCS-SpecificCarrier of a serving cell. |

#### –	SDAP-Config

The IE SDAP-Config is used to set the configurable SDAP parameters for a data radio bearer. All configured instances of SDAP-Config with the same value of pdu-Session correspond to the same SDAP entity as specified in TS 37.324 [FFS_Ref].

SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=						SEQUENCE {

pdu-Session							PDU-SessionID,

sdap-HeaderDL 						ENUMERATED {present, absent},

sdap-HeaderUL 						ENUMERATED {present, absent},

defaultDRB							BOOLEAN,

mappedQoS-FlowsToAdd				SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI 									OPTIONAL, -- Need N

mappedQoS-FlowsToRelease			SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI 									OPTIONAL, -- Need N

...

}

QFI ::= 							INTEGER (0..maxQFI)

PDU-SessionID ::= 					INTEGER (0..255)

-- TAG-SDAP-CONFIG-STOP

-- ASN1STOP

SDAP-Config field descriptions

defaultDRB Indicates whether or not this is the default DRB for this PDU session. Among all configured instances of SDAP-Config with the same value of pdu-Session, this field shall be set to TRUE in at most one instance of SDAP-Config and to FALSE in all other instances.

mappedQoS-FlowsToAdd Indicates the list of QFIs of QoS flows of the PDU session to be additionally mapped to this DRB. A QFI value can be included at most once in all configured instances of SDAP-Config with the same value of pdu-Session.

mappedQoS-FlowsToRelease Indicates the list of QFIs of QoS flows of the PDU session to be released from existing QoS flow to DRB mapping of this DRB.

pdu-Session Identity of the PDU session whose QoS flows are mapped to the DRB

sdap-HeaderUL Indicates whether or not a SDAP header is present for UL data on this DRB.The field cannot be changed after a DRB is established.

sdap-HeaderDL Indicates whether or not a SDAP header is present for DL data on this DRB. The field cannot be changed after a DRB is established.

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::= 						SEQUENCE {

searchSpaceId							SearchSpaceId,

controlResourceSetId					ControlResourceSetId														OPTIONAL, 	-- Cond SetupOnly

monitoringSlotPeriodicityAndOffset		CHOICE {

sl1										NULL,

sl2										INTEGER (0..1),

sl4										INTEGER (0..3),

sl5 									INTEGER (0..4),

sl8										INTEGER (0..7),

sl10 									INTEGER (0..9),

sl16 									INTEGER (0..15),

sl20 									INTEGER (0..19),

sl40 									INTEGER (0..39),

sl80 									INTEGER (0..79),

sl160 									INTEGER (0..159),

sl320									INTEGER (0..319),

sl640									INTEGER (0..639),

sl1280									INTEGER (0..1279),

sl2560									INTEGER (0..2559)

}																													OPTIONAL,	-- Cond Setup

duration								INTEGER (2..2559)															OPTIONAL,	-- Need R

monitoringSymbolsWithinSlot				BIT STRING (SIZE (14))														OPTIONAL, 	-- Cond Setup

nrofCandidates							SEQUENCE {

aggregationLevel1						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}																													OPTIONAL,	-- Cond Setup

searchSpaceType							CHOICE {

common									SEQUENCE {

dci-Format0-0-AndFormat1-0				SEQUENCE {

...

}																											OPTIONAL,	-- Need R

dci-Format2-0							SEQUENCE {

nrofCandidates-SFI						SEQUENCE {

aggregationLevel1						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel2						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel4						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel8						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel16						ENUMERATED {n1, n2}											OPTIONAL	-- Need R

},

...

}																											OPTIONAL,	-- Need R

dci-Format2-1							SEQUENCE {

...

}																											OPTIONAL,	-- Need R

dci-Format2-2							SEQUENCE {

...

}																											OPTIONAL,	-- Need R

dci-Format2-3							SEQUENCE {

monitoringPeriodicity					ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}					OPTIONAL, 	-- Cond Setup

nrofPDCCH-Candidates					ENUMERATED {n1, n2},

...

}																											OPTIONAL	-- Need R

},

ue-Specific								SEQUENCE {

dci-Formats								ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...

}

}																													OPTIONAL	-- Cond Setup

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, CS-RNTI (if configured), SP-CSI-RNTI (if configured), RA-RNTI, TC-RNTI, P-RNTI, SI-RNTI

dci-Format2-0 If configured, UE monitors the DCI format format 2_0 with CRC scrambled by SFI-RNTI

dci-Format2-1 If configured, UE monitors the DCI format format 2_1 with CRC scrambled by INT-RNTI

dci-Format2-2 If configured, UE monitors the DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI

dci-Format2-3 If configured, UE monitors the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

monitoringPeriodicity Monitoring periodicity of SRS PDCCH in number of slots for DCI format 2-3. Corresponds to L1 parameter 'SRS-monitoring-periodicity' (see 38.212, 38.213, section 7.3.1, 11.3)

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. If UE is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. Corresponds to L1 parameters 'Montoring-periodicity-PDCCH-slot' and 'Montoring-offset-PDCCH-slot' (see 38.213, section 10)

monitoringSymbolsWithinSlot Symbols for PDCCH monitoring in the slots configured for PDCCH monitoring (see monitoringSlotPeriodicityAndOffset). The most significant (left) bit represents the first OFDM in a slot. The least significant (right) bit represents the last symbol. Corresponds to L1 parameter 'Montoring-symbols-PDCCH-within-slot' (see 38.213, section 10)

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. Corresponds to L1 parameters 'SFI-Num-PDCCH-cand' and 'SFI-Aggregation-Level' (see 38.213, section 11.1.1).

nrofCandidates Number of PDCCH candidates per aggregation level. Corresponds to L1 parameter 'Aggregation-level-1' to 'Aggregation-level-8'. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). (see 38.213, section 10)

nrofPDCCH-Candidates The number of PDCCH candidates for DCI format 2-3 for the configured aggregation level. Corresponds to L1 parameter 'SRS-Num-PDCCH-cand' (see 38.212, 38.213, section 7.3.1, 11.3)

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent otherwise. |

#### –	SearchSpaceId

The IE SearchSpaceId is used to identify Search Spaces. The search space with the SearchSpaceId = 0 identifies the search space configured via PBCH (MIB) and in ServingCellConfigCommon (searchSpaceZero). The number of Search Spaces per BWP is limited to 10 including the initial Search Space.

SearchSpaceId information element

-- ASN1START

-- TAG-SEARCHSPACEID-START

SearchSpaceId ::= 					INTEGER (0..maxNrofSearchSpaces-1)

-- TAG-SEARCHSPACEID-STOP

-- ASN1STOP

#### –	SearchSpaceZero

The IE SearchSpaceZero is used to configure SearchSpace#0 of the initial BWP (see TS 38.213 [13], section 13).

SearchSpaceZero information element

-- ASN1START

-- TAG-SEARCHSPACEZERO-START

SearchSpaceZero ::=					INTEGER (0..15)

-- TAG-SEARCHSPACEZERO-STOP

-- ASN1STOP

#### –	SecurityAlgorithmConfig

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm and AS ciphering algorithm for SRBs and DRBs.

SecurityAlgorithmConfig information element

-- ASN1START

-- TAG-SECURITY-ALGORITHM-CONFIG-START

SecurityAlgorithmConfig ::=			SEQUENCE {

cipheringAlgorithm					CipheringAlgorithm,

integrityProtAlgorithm				IntegrityProtAlgorithm			OPTIONAL,	-- Need R

...

}

IntegrityProtAlgorithm ::=			ENUMERATED {

nia0, nia1, nia2, nia3, spare4, spare3,

spare2, spare1, ...}

CipheringAlgorithm ::=				ENUMERATED {

nea0, nea1, nea2, nea3, spare4, spare3,

spare2, spare1, ...}

-- TAG-SECURITY-ALGORITHM-CONFIG-STOP

-- ASN1STOP

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nea0-nea3 are identical to the LTE algorithms eea0-3. For EN-DC, the algorithms configured for bearers using KeNB shall be the same as for all bearers using KeNB and the algorithms configured for bearers using S-KgNB shall be the same as for all bearers using S-KgNB. If EN-DC is not configured, the algorithm shall be the same for all bearers.

integrityProtAlgorithm For EN-DC, this IE indicates the integrity protection algorithm to be used for SRBs, as specified in TS 33.501 [11]. The algorithms nia0-nia3 is identical to the LTE algorithms eia0-3. For EN-DC, the algorithms configured for SRBs using KeNB shall be the same as for all SRBs using KeNB and the algorithms configured for bearers using S-KgNB shall be the same as for all bearers using S-KgNB.The network does not configure nia0 for SRB3. If EN-DC is not configured, the algorithm shall be the same for all bearers.

#### –	ServCellIndex

The IE ServCellIndex concerns a short identity, used to identify a serving cell (i.e. the PCell, the PSCell or an SCell). Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

-- TAG-SERV-CELL-INDEX-START

ServCellIndex ::=					INTEGER (0..maxNrofServingCells-1)

-- TAG-SERV-CELL-INDEX-STOP

-- ASN1STOP

#### –	ServingCellConfig

The ServingCellConfig IE is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts).

ServingCellConfig information element

-- ASN1START

-- TAG-SERVING-CELL-CONFIG-START

ServingCellConfig ::=				SEQUENCE {

tdd-UL-DL-ConfigurationDedicated	TDD-UL-DL-ConfigDedicated												OPTIONAL,	-- Cond TDD

initialDownlinkBWP					BWP-DownlinkDedicated													OPTIONAL,	-- Cond ServCellAdd

downlinkBWP-ToReleaseList			SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id								OPTIONAL,	-- Need N

downlinkBWP-ToAddModList			SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink						OPTIONAL, 	-- Need N

firstActiveDownlinkBWP-Id			BWP-Id																	OPTIONAL,	-- Cond SyncAndCellAdd

bwp-InactivityTimer					ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }	OPTIONAL,	--Need R

defaultDownlinkBWP-Id				BWP-Id																	OPTIONAL, 	-- Need S

uplinkConfig						UplinkConfig															OPTIONAL,	-- Need M

supplementaryUplink					UplinkConfig 															OPTIONAL, 	-- Need M

pdcch-ServingCellConfig				SetupRelease { PDCCH-ServingCellConfig }								OPTIONAL,	-- Need M

pdsch-ServingCellConfig				SetupRelease { PDSCH-ServingCellConfig }								OPTIONAL,	-- Need M

csi-MeasConfig						SetupRelease { CSI-MeasConfig }											OPTIONAL,	-- Need M

sCellDeactivationTimer				ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}					OPTIONAL,	-- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig		CrossCarrierSchedulingConfig											OPTIONAL,	-- Need M

tag-Id								TAG-Id,

ue-BeamLockFunction					ENUMERATED {enabled}													OPTIONAL,	-- Need R

pathlossReferenceLinking			ENUMERATED {pCell, sCell}												OPTIONAL,	-- Cond SCellOnly

servingCellMO						MeasObjectId															OPTIONAL,	-- Cond MeasObject

...

}

UplinkConfig ::=					SEQUENCE {

initialUplinkBWP					BWP-UplinkDedicated														OPTIONAL, 	-- Cond ServCellAdd

uplinkBWP-ToReleaseList				SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id								OPTIONAL,	-- Need N

uplinkBWP-ToAddModList				SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink							OPTIONAL, 	-- Need N

firstActiveUplinkBWP-Id				BWP-Id																	OPTIONAL,	-- Cond SyncAndCellAdd

pusch-ServingCellConfig				SetupRelease { PUSCH-ServingCellConfig }								OPTIONAL,	-- Need M

carrierSwitching					SetupRelease { SRS-CarrierSwitching	}									OPTIONAL,	-- Need M

...

}

-- TAG-SERVING-CELL-CONFIG-STOP

-- ASN1STOP

ServingCellConfig field descriptions

bwp-InactivityTimer The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.321, section 5.15) The value 0.5 ms is only applicable for carriers >6 GHz. When the network releases the timer configuration, the UE stops the timer without swithching to the default BWP.

crossCarrierSchedulingConfig Indicates whether this SCell is cross-carrier scheduled by another serving cell.

defaultDownlinkBWP-Id Corresponds to L1 parameter 'default-DL-BWP'. The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of txxx. This field is UE specific. When the field is absent the UE uses the the initial BWP as default BWP. (see 38.211, 38.213, section 12 and 38.321, section 5.15)

downlinkBWP-ToAddModList List of additional downlink bandwidth parts to be added or modified. (see 38.211, 38.213, section 12).

downlinkBWP-ToReleaseList List of additional downlink bandwidth parts to be released. (see 38.211, 38.213, section 12).

firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated upon performing the reconfiguration in which it is received. If the field is absent, the RRC reconfiguration does not impose a BWP switch (corresponds to L1 parameter 'active-BWP-DL-Pcell'). If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon MAC-activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0.

initialDownlinkBWP The dedicated (UE-specific) configuration for the initial downlink bandwidth-part.

pathlossReferenceLinking Indicates whether UE shall apply as pathloss reference either the downlink of PCell or of SCell that corresponds with this uplink (see 38.213, section 7)

pdsch-ServingCellConfig PDSCH releated parameters that are not BWP-specific.

sCellDeactivationTimer SCell deactivation timer in TS 38.321 [3]. If the field is absent, the UE applies the value infinity.

servingCellMO measObjectIdof the MeasObjectNR in MeasConfigwhich is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon of the serving cell: if ssbFrequency is configured, its value is the same aslike the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellList-Mobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellList-Mobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

tag-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell belongs to.

ue-BeamLockFunction Enables the "UE beam lock function (UBF)", which disable changes to the UE beamforming configuration when in NR_RRC_CONNECTED. FFS: Parameter added preliminary based on RAN4 LS in R4-1711823. Decide where to place it (maybe ServingCellConfigCommon or in a BeamManagement IE??)

UplinkConfig field descriptions

carrierSwitching Includes parameters for configuration of carrier based SRS switching Corresponds to L1 parameter 'SRS-CarrierSwitching' (see 38,214, section FFS_Section)

firstActiveUplinkBWP-Id If configured for an SpCell, this field contains the ID of the UL BWP to be activated upon performing the reconfiguration in which it is received. If the field is absent, the RRC reconfiguration does not impose a BWP switch (corresponds to L1 parameter 'active-BWP-UL-Pcell'). If configured for an SCell, this field contains the ID of the uplink bandwidth part to be used upon MAC-activation of an SCell. The initial bandwidth part is referred to by BandiwdthPartId = 0.

initialUplinkBWP The dedicated (UE-specific) configuration for the initial uplink bandwidth-part.

pusch-ServingCellConfig PUSCH related parameters that are not BWP-specific.

supplementaryUplink The field is optionally present if supplementaryUplinkConfig is configured in ServingCellConfigCommon and absent otherwise.

uplinkBWP-ToReleaseList The additional bandwidth parts for uplink. In case of TDD uplink- and downlink BWP with the same bandwidthPartId are considered as a BWP pair and must have the same center frequency.

uplinkConfig The field is optionally present if uplinkConfigCommon is configured in ServingCellConfigCommon, and absent otherwise.

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject | This field is mandatory present for the SpCell, it is optionally present, Need R, for SCells. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell). It is optionally present, Need M otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present, Need N, for a SpCell upon reconfigurationWithSync (PCell handover, PSCelladdition/change). The field is mandatory present, Need M, for an SCellupon addition. In all other cases the field is absent. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |

#### –	ServingCellConfigCommon

The ServingCellConfigCommon IE is used to configure cell specific parameters of a UE’s serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVING-CELL-CONFIG-COMMON-START

ServingCellConfigCommon ::=			SEQUENCE {

physCellId							PhysCellId													OPTIONAL, -- Cond HOAndServCellAdd,

downlinkConfigCommon				DownlinkConfigCommon										OPTIONAL, -- Cond HOAndServCellAdd

uplinkConfigCommon					UplinkConfigCommon											OPTIONAL, -- Need M

supplementaryUplinkConfig					UplinkConfigCommon									OPTIONAL, -- Need M

n-TimingAdvanceOffset				ENUMERATED { n0, n25600, n39936 }								OPTIONAL,-- Need S

ssb-PositionsInBurst				CHOICE {

shortBitmap							BIT STRING (SIZE (4)),

mediumBitmap						BIT STRING (SIZE (8)),

longBitmap							BIT STRING (SIZE (64))

}																								OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell			ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }		OPTIONAL,	-- Need S

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround				SetupRelease { RateMatchPatternLTE-CRS } 								OPTIONAL,	-- Need M

rateMatchPatternToAddModList		SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern		OPTIONAL, -- Need N

rateMatchPatternToReleaseList		SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId		OPTIONAL, -- Need N

subcarrierSpacing					SubcarrierSpacing														OPTIONAL, -- Cond HOAndServCellAdd

tdd-UL-DL-ConfigurationCommon		TDD-UL-DL-ConfigCommon													OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower					INTEGER (-60..50),

...

}

-- TAG-SERVING-CELL-CONFIG-COMMON-STOP

-- ASN1STOP

ServingCellConfigCommon field descriptions

dmrs-TypeA-Position Position of (first) DL DM-RS (see 38.211, section 7.4.1.1.1)

initialDownlinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG). The parameters provided herein should match the parameters configured by MIB and SIB1 of the serving cell.

longBitmap bitmap for above 6 GHz

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap bitmap for 3-6 GHz

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency rangeof this serving cell. See 38.133, table 7.1.2-2.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the nested bitmaps. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology. Corresponds to L1 parameter 'Resource-set-cekk' (see 38.214, section 5.1.2.2.3)

shortBitmap bitmap for sub 3 GHz

ss-PBCH-BlockPower TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power. (see 38.213, section 7.4)

ssb-periodicityServingCell The SSB periodicity in msec for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see 38.211, section [7.4.3.1])

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in an SS-burst. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. Corresponds to L1 parameter 'SSB-Transmitted' (see 38.213, section 5.1)

subcarrierSpacing Subcarrier spacing of SSB. Used only for non-initial access (e.g. SCells, PCell of SCG). Only the values 15 or 30 kHz (<6GHz), 120 or 240 kHz (>6GHz) are applicable.

supplementaryUplinkConfig This field is optionally present if uplinkConfigCommon is configured, and absent otherwise.

tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see 38.213, section 11.1.

| Conditional Presence | Explanation |
| --- | --- |
| AbsFreqSSB | The field is absent when absoluteFrequencySSB in frequencyInfoDL is absent, oherwise the field is mandatory present. |
| HOAndServCellAdd | This field is mandatory present for inter-cell handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell). It is optionally present, Need M otherwise. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is not present. |

#### –	ServingCellConfigCommonSIB

The ServingCellConfigCommonSIB IE is used to configure cell specific parameters of a UE’s serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=		SEQUENCE {

downlinkConfigCommon					DownlinkConfigCommonSIB,

uplinkConfigCommon					UplinkConfigCommon											OPTIONAL,	-- Need R

supplementaryUplink					UplinkConfigCommon											OPTIONAL,	-- Need R

n-TimingAdvanceOffset				ENUMERATED { n25560, n39936 }								OPTIONAL,	-- Need S

ssb-PositionsInBurst					SEQUENCE {

inOneGroup							BIT STRING (SIZE (8)),

groupPresence						BIT STRING (SIZE (8))				OPTIONAL -- Cond Above6GHzOnly

},

ssb-PeriodicityServingCell			ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2},

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround				RateMatchPatternLTE-CRS 				OPTIONAL,	-- Need R

rateMatchPatternList					SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern		OPTIONAL, -- Need N

tdd-UL-DL-ConfigurationCommon		TDD-UL-DL-ConfigCommon				OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower					INTEGER (-60..50),

...

}

-- TAG-SERVINGCELLCONFIGCOMMONSIB-STOP

-- ASN1STOP

ServingCellConfigCommonSIB field descriptions

groupPresence This field is present when the carrier frequency is above 6GHz. The first/leftmost bit corresponds to the SS/PBCH index 0-7, the second bit corresponds to SS/PBCH block 8-15, and so on. Value 0 in the bitmap indicates that the SSBs according to inOneGroup are not present. Value 1 indcates that the SS/PBCH blocks are transmitted in accrodance with inOneGroup.

inOneGroup when carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bit are vaild; the UE ignores the 4 righmost bit. When the carrier frequency is larger than 3 GHz and smaller than or equal to 6 GHz, all 8 bit are vaild. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. For carrier frequencies larger than 6 GHz, all 8 bit are valid; The first/ leftmost bit corresponds to the first SS/PBCH block index in the group (i.e., to SSB index 0, 8, and so one); the second bit corresponds to the second SS/PBCH block index in the group (i.e., to SSB index 1, 9, and so one), and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst. Corresponds to L1 parameter 'SSB-Transmitted' (see 38.213, section 4.1)

#### –	ShortI-RNTI-Value

The ShortI-RNTI-Value IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using fewer bits compared to I-RNTI-Value.

ShortI-RNTI-Value information element

-- ASN1START

-- TAG-ShortI-RNTI-VALUE-START

ShortI-RNTI-Value ::=	BIT STRING (SIZE(24))

-- TAG-ShortI-RNTI-VALUE-STOP

-- ASN1STOP

#### –	SINR-Range

The IE SINR-Range specifies the value range used in SINR measurements and thresholds. Integer value for SINR measurements is according to mapping table in TS 38.133 [14].

SINR-Range information element

-- ASN1START

-- TAG-SINR-RANGE-START

SINR-Range ::=						INTEGER(0..127)

-- TAG-SINR-RANGE-STOP

-- ASN1STOP

#### –	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG-OTHER-SI-INFO-START

SI-SchedulingInfo ::=				SEQUENCE {

schedulingInfoList 					SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength						ENUMERATED {s5, s10, s20, s40, s80, s160},

si-RequestConfig					SI-RequestConfig													OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL				SI-RequestConfig													OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID				BIT STRING (SIZE (24))												OPTIONAL,	-- Need R

...

}

SchedulingInfo ::=					SEQUENCE {

-- Indicates whether the SI is currently being broadcast. The value of the indication is valid until the end of the BCCH modification period. Change of si-BroadcastStatus should not result in system information change notifications in paging message or Direct Indication Information.

si-BroadcastStatus					ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity						ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-Mapping

}

SIB-Mapping ::= 						SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-TypeInfo

SIB-TypeInfo ::=						SEQUENCE {

type									ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1,... },

valueTag 							INTEGER (0..31)													OPTIONAL, -- Cond SIB-TYPE

areaScope							ENUMERATED {true}													OPTIONAL -- Cond AREA-ID

}

-- Configuration for Msg1 based SI Request

SI-RequestConfig::=				SEQUENCE {

dedicatedResourceConfig			SEQUENCE {

rach-OccasionsSI						SEQUENCE {

rach-ConfigSI						RACH-ConfigGeneric,

ssb-perRACH-Occasion					ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}																										OPTIONAL, 	-- Need R

si-RequestPeriod					ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}					OPTIONAL,

si-RequestResources 	SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}	OPTIONAL 	-- Cond DEDICATED

}

SI-RequestResources ::= 		SEQUENCE {

ra-PreambleStartIndex 		INTEGER (0..63),

ra-ConfigurationPeriodIndex		INTEGER (0..15)		OPTIONAL,			-- Need R

ra-ssb-OccasionMaskIndex		INTEGER (0..15)

}

-- TAG-OTHER-SI-INFO-STOP

-- ASN1STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occassions for SI

si-RequestPeriod Periodicity of the SI-Request configuration in number of PRACH configuration periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages which are provided on demand. Otherwise the 1st entry in the list corresponds to the first on demand SI message in schedulingInfoList, 2nd entry in the list corresponds to the second on demand SI message in schedulingInfoList and so on.

SI-RequestResources field descriptions

ra-ConfigurationPeriodIndex Index of the PRACH configuration period in the si-RequestPeriod from which the dedicated preamble indices can be used for this SI request. If the overall number of SSBs (NTOT) is higher than the number of SSBs associated with one RACH occasion (N), the dedicated preamble indices can be used for NTOT/N consecutive RACH occasions.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the ith SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

SI-SchedulingInfo field descriptions

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to onDemand. If the field is not present the UE uses Msg3 to request SI-messages for which si-BroadcastStatus is set to onDemand (if any).

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to onDemand. If the field is not present the UE uses Msg3 to request SI-messages for which si-BroadcastStatus is set to onDemand (if any) on supplementary uplink.

si-WindowLength The length of the SI scheduling window. s5 corresponds to 5 slots, s10 to 10 slots and so on.

systemInformationAreaID Indicates the system information area that the cell belongs to. A SIB that is area specific may be applicable within an area referred to as a system information area, where the area is identified by systemInformationAreaID. The systemInformationAreaID is unique within a PLMN and is associated with the first PLMN-Identity included in the PLMN-IdentityInfoList.

SchedulingInfo field descriptions

si-BroadcastStatus Indicates the SI message is being broadcasted or on-demand. Change of si-BroadcastStatus should not result in system information change notifications in paging message or Direct Indication Information. The value of the indication is valid until the end of the BCCH modification period.

| Conditional presence | Explanation |  |
| --- | --- | --- |
| AREA-ID | The field is present if systemInformationAreaID is presentand the SIB is valid within the area identified by systemInformationAreaID,otherwise it is not present. |  |
| MSG-1 | The field is optionally present, Need R, if si-BroadcastStatus is onDemand for any SI-message included in SchedulingInfo. It is absent otherwise. |  |
| SIB-TYPE | The field is mandatory present if the SIB type is different from SIB6, SIB7 or SIB8. |
| SUL-MSG-1 | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink and if si-BroadcastStatus is onDemand for any SI-message included in SchedulingInfo. It is absent otherwise. |
| DEDICATED | The field is present if dedicated RACH resources are configured for SI request. If absent the UE uses common RACH reseourcse. |

#### –	SlotFormatCombinationsPerCell

The IE SlotFormatCombinationsPerCell is used to configure the SlotFormatCombinations applicable for one serving cell. Corresponds to L1 parameter 'cell-to-SFI' (see 38.213, section 11.1.1).

SlotFormatCombinationsPerCell information element

-- ASN1START

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-START

SlotFormatCombinationsPerCell ::=	SEQUENCE {

servingCellId						ServCellIndex,

subcarrierSpacing					SubcarrierSpacing,

subcarrierSpacing2					SubcarrierSpacing																	OPTIONAL,	-- Need R

slotFormatCombinations				SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination	OPTIONAL, 	-- Need M

positionInDCI						INTEGER(0..maxSFI-DCI-PayloadSize-1)												OPTIONAL, 	-- Need M

...

}

SlotFormatCombination ::= 			SEQUENCE {

slotFormatCombinationId				SlotFormatCombinationId,

slotFormats							SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255)

}

SlotFormatCombinationId	::=			INTEGER (0..maxNrofSlotFormatCombinationsPerSet-1)

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-STOP

-- ASN1STOP

SlotFormatCombination field descriptions

slotFormatCombinationId This ID is used in the DCI payload to dynamically select this SlotFormatCombination. Corresponds to L1 parameter 'SFI-index' (see 38.213, section FFS_Section)

slotFormats Slot formats that occur in consecutive slots in time domain order as listed here. The the slot formats are defined in 38.211, table 4.3.2-3 and numbered with 0..255.

SlotFormatCombinationsPerCell field descriptions

positionInDCI The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload. Corresponds to L1 parameter 'SFI-values' (see 38.213, section FFS_Section)

servingCellId The ID of the serving cell for which the slotFormatCombinations are applicable

slotFormatCombinations A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see 38.211, section 4.3.2). The total number of slotFormats in the slotFormatCombinations list does not exceed 512. FFS_CHECK: RAN1 indicates that the combinations could be of two different types... but they don't specify the second

subcarrierSpacing2 Reference subcarrier spacing for a Slot Format Combination on an FDD or SUL cell. Corresponds to L1 parameter 'SFI-scs2' (see 38.213, section FFS_Section). For FDD, subcarrierSpacing (SFI-scs) is the reference SCS for DL BWP and subcarrierSpacing2 (SFI-scs2) is the reference SCS for UL BWP. For SUL, subcarrierSpacing (SFI-scs) is the reference SCS for non-SUL carrier and subcarrierSpacing2 (SFI-scs2) is the reference SCS for SUL carrier. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of theserving cell which the UE monitors for SFI indications.

subcarrierSpacing Reference subcarrier spacing for this Slot Format Combination. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of theserving cell which the UE monitors for SFI indications.Corresponds to L1 parameter 'SFI-scs' (see 38.213, section FFS_Section)

#### –	SlotFormatIndicator

The IE SlotFormatIndicator is used to configure monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI).

SlotFormatIndicator information element

-- ASN1START

-- TAG-SLOTFORMATINDICATOR-START

SlotFormatIndicator ::= 		SEQUENCE {

sfi-RNTI						RNTI-Value,

dci-PayloadSize					INTEGER (1..maxSFI-DCI-PayloadSize),

slotFormatCombToAddModList		SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell	OPTIONAL,	-- Need N

slotFormatCombToReleaseList		SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex					OPTIONAL,	-- Need N

...

}

-- TAG-SLOTFORMATINDICATOR-STOP

-- ASN1STOP

SlotFormatIndicator field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with SFI-RNTI. Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section 11.1.1)

sfi-RNTI RNTI used for SFI on the given cell Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section 11.1.1)

slotFormatCombToAddModList A list of SlotFormatCombinations for the UE's serving cells. Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section 11.1.1)

#### –	S-NSSAI

The IE S-NSSAI (Single Network Slice Selection Assistance Information) identifies a Network Slice end to end and comprises a slice/service type and a slice differentiator, see TS 23.003 [20].

S-NSSAI information element

-- ASN1START

-- TAG-S-NSSAI-START

S-NSSAI  ::=						CHOICE{

sst									BIT STRING (SIZE (8)),

sst-SD								BIT STRING (SIZE (32))

}

-- TAG-S-NSSAI-STOP

-- ASN1STOP

S-NSSAI field descriptions

sst-SD Indicates the S-NSSAI consists of Slice/Service Type and Slice Differentiator, see TS 23.003 [20].

sst Indicates the S-NSSAI consists of Slice/Service Type, see TS 23.003 [20].

#### –	SpeedStateScaleFactors

The IE SpeedStateScaleFactors concerns factors, to be applied when the UE is in medium or high speed state, used for scaling a mobility control related parameter.

SpeedStateScaleFactors information element

-- ASN1START

-- TAG-SPEEDSTATESCALEFACTORS-START

SpeedStateScaleFactors ::=			SEQUENCE {

sf-Medium							ENUMERATED {oDot25, oDot5, oDot75, lDot0},

sf-High								ENUMERATED {oDot25, oDot5, oDot75, lDot0}

}

-- TAG-SPEEDSTATESCALEFACTORS-STOP

-- ASN1STOP

SpeedStateScaleFactors field descriptions

sf-High The concerned mobility control related parameter is multiplied with this factor if the UE is in High Mobility state as defined in TS 36.304 [4]. Value oDot25 corresponds to 0.25, oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on8

sf-Medium The concerned mobility control related parameter is multiplied with this factor if the UE is in Medium Mobility state as defined in TS 38.304 [4]. Value oDot25 corresponds to 0.25, oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

#### –	SS-RSSI-Measurement

The IE SS-RSSI-Measurement is used to configure RSSI measuremens based on synchronization reference signals.

SS-RSSI-Measurement information element

-- ASN1START

-- TAG-SS-RSSI-MEASUREMENT-START

SS-RSSI-Measurement ::=				SEQUENCE {

measurementSlots					BIT STRING (SIZE (1..80)),

endSymbol							INTEGER(0..3)

}

-- TAG-SS-RSSI-MEASUREMENT-STOP

-- ASN1STOP

SS-RSSI-Measurement field descriptions

endSymbol Within a slot that is configured for RSSI measurements (see measurementSlots) the UE measures the RSSI from symbol 0 to symbol endSymbol. This field identifies the entry in Table 5.1.3-1 in TS 38.215 which determines the actual end symbol.

measurementSlots Indicates the slots in which the UE can perform RSSI measurements. The length of the BIT STRING is equal to the number of slots in the configured SMTC window (determined by the duration and by the subcarrierSpacing). The first (left-most / most significant) bit in the bitmap corresponds to the first slot in the SMTC window, the second bit in the bitmap corresponds to the second slot in the SMTC window, and so on. The UE measures in slots for which the corresponding bit in the bitmap is set to 1.

#### –	SPS-Config

The SPS-Config IE is used to configure downlink semi-persistent transmission. Downlink SPS may be configured on the PCell as well as on SCells. But it shall not be configured for more than one serving cell of a cell group at once.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::= 							SEQUENCE {

periodicity								ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes						INTEGER (1..8),

n1PUCCH-AN								PUCCH-ResourceId														OPTIONAL,	-- Need M

mcs-Table								ENUMERATED {qam64LowSE}												OPTIONAL,	-- Need S

...

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

SPS-Config field descriptions

mcs-Table Indicates the MCS table the UE shall use for DL SPS. Corresponds to L1 parameter ‘mcs-Table’ in section 5.1.3.1 of 38.214. If present, the UE shall use the MCS table of low-SE 64QAM table indicated in Table 5.1.3.1-3 of 38.214. If this field is absent and field mcs-table in PDSCH-Config is set to ‘qam256’ and the activating DCI is of format 1_1, the UE applies the 256QAM table indicated in Table 5.1.3.1-2 of 38.214. Otherwise, the UE applies the non-low-SE 64QAM table indicated in Table 5.1.3.1-1 of 38.214.

n1PUCCH-AN HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. See 38.214, section FFS_Section.

nrofHARQ-Processes Number of configured HARQ processes for SPS DL. Corresponds to L1 parameter 'numberOfConfSPS-Processes' (see 38.214, section FFS_Section)

periodicity Periodicity for DL SPS Corresponds to L1 parameter 'semiPersistSchedIntervalDL' (see 38.214 and 38.321, section FFS_Section)

#### –	SRB-Identity

The IE SRB-Identity is used to identify a Signalling Radio Bearer (SRB) used by a UE.

-- ASN1START

-- TAG-SRB-IDENTITY-START

SRB-Identity ::=					INTEGER (1..3)

-- TAG-SRB-IDENTITY-STOP

-- ASN1STOP

#### –	SRS-Config

The SRS-Config IE is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources. The network triggers the transmission of the set of SRS-Resources using a configured aperiodicSRS-ResourceTrigger (L1 DCI).

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::= 							SEQUENCE {

srs-ResourceSetToReleaseList 			SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId			OPTIONAL, 	-- Need N

srs-ResourceSetToAddModList 			SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet				OPTIONAL, 	-- Need N

srs-ResourceToReleaseList 				SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId					OPTIONAL,	-- Need N

srs-ResourceToAddModList 				SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource					OPTIONAL, 	-- Need N

tpc-Accumulation						ENUMERATED {disabled}														OPTIONAL,	-- Need S

...

}

SRS-ResourceSet ::= 					SEQUENCE {

srs-ResourceSetId						SRS-ResourceSetId,

srs-ResourceIdList						SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId		OPTIONAL,	-- Cond Setup

resourceType							CHOICE {

aperiodic								SEQUENCE {

aperiodicSRS-ResourceTrigger			INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS									NZP-CSI-RS-ResourceId											OPTIONAL,	-- Cond NonCodebook

slotOffset								INTEGER (1..32)													OPTIONAL,	-- Need S

...,

[[

aperiodicSRS-ResourceTriggerList		SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2)) OF INTEGER (1..maxNrofSRS-TriggerStates-1)																															OPTIONAL		-- Need M

]]

},

semi-persistent							SEQUENCE {

associatedCSI-RS						NZP-CSI-RS-ResourceId											OPTIONAL, -- Cond NonCodebook

...

},

periodic								SEQUENCE {

associatedCSI-RS						NZP-CSI-RS-ResourceId											OPTIONAL, -- Cond NonCodebook

...

}

},

usage									ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha									Alpha																	OPTIONAL, -- Need S

p0										INTEGER (-202..24)														OPTIONAL, -- Cond Setup

pathlossReferenceRS						CHOICE {

ssb-Index								SSB-Index,

csi-RS-Index							NZP-CSI-RS-ResourceId

}																												OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates		ENUMERATED { sameAsFci2, separateClosedLoop}							OPTIONAL, -- Need S

...

}

SRS-ResourceSetId ::= 					INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-Resource ::= 						SEQUENCE {

srs-ResourceId							SRS-ResourceId,

nrofSRS-Ports							ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex							ENUMERATED {n0, n1 }													OPTIONAL,	-- Need R

transmissionComb						CHOICE {

n2										SEQUENCE {

combOffset-n2							INTEGER (0..1),

cyclicShift-n2							INTEGER (0..7)

},

n4										SEQUENCE {

combOffset-n4							INTEGER (0..3),

cyclicShift-n4							INTEGER (0..11)

}

},

resourceMapping							SEQUENCE {

startPosition							INTEGER (0..5),

nrofSymbols								ENUMERATED {n1, n2, n4},

repetitionFactor						ENUMERATED {n1, n2, n4}

},

freqDomainPosition						INTEGER (0..67),

freqDomainShift							INTEGER (0..268),

freqHopping								SEQUENCE {

c-SRS									INTEGER (0..63),

b-SRS									INTEGER (0..3),

b-hop									INTEGER (0..3)

},

groupOrSequenceHopping					ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType							CHOICE {

aperiodic								SEQUENCE {

...

},

semi-persistent							SEQUENCE {

periodicityAndOffset-sp						SRS-PeriodicityAndOffset,

...

},

periodic								SEQUENCE {

periodicityAndOffset-p						SRS-PeriodicityAndOffset,

...

}

},

sequenceId								BIT STRING (SIZE (10)),

spatialRelationInfo						SRS-SpatialRelationInfo											OPTIONAL,	-- Need R

...

}

SRS-SpatialRelationInfo ::=		SEQUENCE {

servingCellId						ServCellIndex										OPTIONAL,	-- Need S

referenceSignal 					CHOICE {

ssb-Index							SSB-Index,

csi-RS-Index						NZP-CSI-RS-ResourceId,

srs									SEQUENCE {

resourceId							SRS-ResourceId,

uplinkBWP							BWP-Id

}

}

}

SRS-ResourceId ::= 						INTEGER (0..maxNrofSRS-Resources-1)

SRS-PeriodicityAndOffset ::=			CHOICE {

sl1										NULL,

sl2										INTEGER(0..1),

sl4										INTEGER(0..3),

sl5										INTEGER(0..4),

sl8										INTEGER(0..7),

sl10									INTEGER(0..9),

sl16									INTEGER(0..15),

sl20									INTEGER(0..19),

sl32									INTEGER(0..31),

sl40									INTEGER(0..39),

sl64									INTEGER(0..63),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319),

sl640									INTEGER(0..639),

sl1280									INTEGER(0..1279),

sl2560									INTEGER(0..2559)

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) Corresponds to L1 parameter 'Accumulation-enabled-srs' (see 38,213, section 7.3)

SRS-Resource field descriptions

cyclicShift-n2 Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1)

cyclicShift-n4 Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1)

freqDomainPosition Parameter(s) defining frequency domain position and configurable shift to align SRS allocation to 4 PRB grid. Corresponds to L1 parameter 'SRS-FreqDomainPosition' (see 38.214, section 6.2.1)

freqHopping Includes parameters capturing SRS frequency hopping Corresponds to L1 parameter 'SRS-FreqHopping' (see 38.214, section 6.2.1)

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping Corresponds to L1 parameter 'SRS-GroupSequenceHopping' (see 38.211, section FFS_Section)

periodicityAndOffset-p Periodicity and slot offset for for this SRS resource. All values in "number of slots" sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots. Corresponds to L1 parameter 'SRS-SlotConfig' (see 38.214, section 6.2.1)

periodicityAndOffset-sp Periodicity and slot offset for for this SRS resource. All values in "number of slots". sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots. Corresponds to L1 parameter 'SRS-SlotConfig' (see 38.214, section 6.2.1)

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than or equal to the maxNnrofPorts configured in the PTRS-UplinkConfig. Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1)

resourceMapping OFDM symbol location of the SRS resource within a slot including number of OFDM symbols (N = 1, 2 or 4 per SRS resource), startPosition (SRSSymbolStartPosition = 0..5; "0" refers to the last symbol, "1" refers to the second last symbol) and RepetitionFactor (r = 1, 2 or 4). Corresponds to L1 parameter 'SRS-ResourceMapping' (see 38.214, section 6.2.1 and 38.211, section 6.4.1.4). FFS: Apparently, RAN1 considers replacing these three fields by a table in RAN1 specs and a corresponding index in ASN.1?!

resourceType Time domain behavior of SRS resource configuration. Corresponds to L1 parameter 'SRS-ResourceConfigType' (see 38.214, section 6.2.1). For codebook based uplink transmission, the network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS. FFS: Add configuration parameters for the different SRS resource types?

sequenceId Sequence ID used to initialize psedo random group and sequence hopping. Corresponds to L1 parameter 'SRS-SequenceId' (see 38.214, section 6.2.1)

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS Corresponds to L1 parameter 'SRS-SpatialRelationInfo' (see 38.214, section 6.2.1)

transmissionComb Comb value (2 or 4) and comb offset (0..combValue-1). Corresponds to L1 parameter 'SRS-TransmissionComb' (see 38.214, section 6.2.1)

SRS-ResourceSet field descriptions

alpha alpha value for SRS power control. Corresponds to L1 parameter 'alpha-srs' (see 38.213, section 7.3) When the field is absent the UE applies the value 1

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration. Corresponds to the second to last entries of L1 parameter 'AperiodicSRS-ResourceTrigger' (see 38.214, section 6.1.1.2).

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration. Corresponds to L1 parameter 'AperiodicSRS-ResourceTrigger' (see 38.214, section 6.1.1.2)

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation. Corresponds to L1 parameter 'SRS-AssocCSIRS' (see 38.214, section 6.2.1)

csi-RS ID of CSI-RS resource associated with this SRS resource set. (see 38.214, section 6.1.1.2)

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed. Corresponds to L1 parameter 'p0-srs' (see 38.213, section 7.3)

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SSblock) to be used for SRS path loss estimation. Corresponds to L1 parameter 'srs-pathlossReference-rs-config' (see 38.213, section 7.3)

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0)

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or serarate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 Corresponds to L1 parameter 'srs-pcadjustment-state-config' (see 38.213, section 7.3)

srs-ResourceIdList The IDs of the SRS-Resources used in this SRS-ResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries.

srs-ResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined.

usage Indicates if the SRS resource set is used for beam management vs. used for either codebook based or non-codebook based transmission. The network configures at most one resource set with usage set to codebook and at most one with usage set to nonCodebook. Corresponds to L1 parameter 'SRS-SetUse' (see 38.214, section 6.2.1)

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optional (Need M) otherwise |
| NonCodebook | This field is optionally present, Need M, in case of non-codebook based transmission, otherwise the field is absent. |

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure FFS

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=			SEQUENCE {

srs-SwitchFromServCellIndex			INTEGER (0..31)																	OPTIONAL,	-- Cond Setup

srs-SwitchFromCarrier				ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group					CHOICE {

typeA								SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB								SRS-TPC-PDCCH-Config

}																													OPTIONAL, 	-- Cond Setup

monitoringCells						SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex						OPTIONAL,	-- Cond Setup

...

}

-- One trigger configuration for SRS-Carrier Switching. (see 38.212, 38.213, section 7.3.1, 11.3)

SRS-TPC-PDCCH-Config ::=			SEQUENCE {

srs-CC-SetIndexlist					SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex 										OPTIONAL	-- Cond Setup

}

SRS-CC-SetIndex ::=					SEQUENCE {

cc-SetIndex							INTEGER (0..3)																	OPTIONAL, 	-- Cond Setup

cc-IndexInOneCC-Set					INTEGER (0..7)																	OPTIONAL	-- Cond Setup

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

SRS-CC-SetIndex field descriptions

cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A (see 38.212, 38.213, section 7.3.1, 11.3)

cc-SetIndex Indicates the CC set index for Type A associated (see 38.212, 38.213, section 7.3.1, 11.3)

SRS-CarrierSwitching field descriptions

monitoringCells A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI Corresponds to L1 parameter 'SRS-monitoring-cells' (see 38.212, 38.213, section 7.3.1, 11.3)

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less cell. During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less cell to transmit SRS. (see 38.214, section 6.2.1.3)

srs-TPC-PDCCH-Group Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.

typeA Type A trigger configuration for SRS transmission on a PUSCH-less SCell. Corresponds to L1 parameter 'typeA-SRS-TPC-PDCCH-Group' (see 38.212, 38.213, section 7.3.1, 11.3)

typeB Type B trigger configuration for SRS transmission on a PUSCH-less SCell. Corresponds to L1 parameter 'typeB-SRS-TPC-PDCCH-Config' (see 38.212, 38.213, section 7.3.1, 11.3)

SRS-TPC-PDCCH-Config field descriptions

srs-CC-SetIndexlist A list of pairs of [cc-SetIndex; cc-IndexInOneCC-Set] (see 38.212, 38.213, section 7.3.1, 11.3)

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon configuration of SRS-CarrierSwitching or SRS-TPC-PDCCH-Config and optional (Need M) otherwise |

#### –	SRS-TPC-CommandConfig

The IE SRS-TPC-CommandConfig is used to configure the UE for extracting TPC commands for SRS from a group-TPC messages on DCI

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=				SEQUENCE {

startingBitOfFormat2-3					INTEGER (1..31)																OPTIONAL, 	-- Cond Setup

fieldTypeFormat2-3						INTEGER (0..1)																OPTIONAL,	-- Cond Setup

...

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

SRS-TPC-CommandConfig field descriptions

fieldTypeFormat2-3 The type of a field within the group DCI with SRS request fields (optional), which indicates how many bits in the field are for SRS request (0 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 38.212. (see 38.212, 38.213, section 7.3.1, 11.3)

startingBitOfFormat2-3 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands (see 38.212, 38.213, section 7.3.1, 11.3).

#### –	SSB-Index

The IE SSB-Index identifies an SS-Block within an SS-Burst. See FFS_Ref, section FFS_Section.

SSB-Index information element

-- ASN1START

-- TAG-SSB-INDEX-START

SSB-Index ::=						INTEGER (0..63)

-- TAG-SSB-INDEX-STOP

-- ASN1STOP

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC	::=								SEQUENCE {

periodicityAndOffset					CHOICE {

sf5									INTEGER (0..4),

sf10									INTEGER (0..9),

sf20									INTEGER (0..19),

sf40									INTEGER (0..39),

sf80									INTEGER (0..79),

sf160								INTEGER (0..159)

},

duration								ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=						SEQUENCE {

pci-List							SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId						OPTIONAL,	-- Need M

periodicity							ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

-- TAG-SSB-MTC-STOP

-- ASN1STOP

SSB-MTCfield descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see 38.213, section 4.1)

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks. Periodicity and offset are given in number of subframes. FFS_FIXME: This does not match the L1 parameter table! They seem to intend an index to a hidden table in L1 specs. (see 38.213, section REF): Periodicity for the given PCIs. Timing offset and Duration as provided in smtc1.

SSB-MTC2 field descriptions

pci-List PCIs that are known to follow this SMTC.

#### –	SSB-ToMeasure

The IE SSB-ToMeasure is used to configure a pattern of SSBs.

SSB-ToMeasure information element

-- ASN1START

-- TAG-SSB-TOMEASURE-START

SSB-ToMeasure ::=					CHOICE {

shortBitmap							BIT STRING (SIZE (4)),

mediumBitmap						BIT STRING (SIZE (8)),

longBitmap							BIT STRING (SIZE (64))

}

-- TAG-SSB-TOMEASURE-STOP

-- ASN1STOP

SSB-ToMeasure field descriptions

longBitmap bitmap for above 6 GHz

mediumBitmap bitmap for 3-6 GHz

shortBitmap bitmap for sub 3 GHz

#### –	SubcarrierSpacing

The SubcarrierSpacing IE determines the subcarrier spacing. Restrictions applicable for certain frequencies, channels or signals are clarified in the fields that use this IE.

SubcarrierSpacing information element

-- ASN1START

-- TAG-SUBCARRIER-SPACING-START

SubcarrierSpacing ::= 				ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1}

-- TAG-SUBCARRIER-SPACING-STOP

-- ASN1STOP

#### –	TCI-State

The TCI-State IE associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::= 						SEQUENCE {

tci-StateId							TCI-StateId,

qcl-Type1							QCL-Info,

qcl-Type2							QCL-Info																OPTIONAL,	-- Need R

...

}

QCL-Info ::=						SEQUENCE {

cell								ServCellIndex															OPTIONAL,	-- Need R

bwp-Id								BWP-Id																	OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal						CHOICE {

csi-rs								NZP-CSI-RS-ResourceId,

ssb									SSB-Index

},

qcl-Type							ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

QCL-Info field descriptions

bwp-Id The DL BWP which the RS is located in.

cell The UE’s serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeD. See TS 38.214 section 5.1.5.

referenceSignal Reference signal with which quasi-collocation information is provided as specified in TS 38.3214 subclause 5.1.5.

qcl-Type QCL type as specified in TS 38.214 subclause 5.1.5.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RS-Indicated | This field is mandatory present if csi-rs or csi-RS-for-tracking is included, absent otherwise |

#### –	TCI-StateId

The IE TCI-StateId is used to identify one TCI-State configuration.

TCI-StateId information element

-- ASN1START

-- TAG-TCI-STATEID-START

TCI-StateId ::=						INTEGER (0..maxNrofTCI-States-1)

-- TAG-TCI-STATEID-STOP

-- ASN1STOP

#### –	TDD-UL-DL-Config

The TDD-UL-DL-Config IEs determines the Uplink/Downlink TDD configuration. There are both, UE- and cell specific IEs.

TDD-UL-DL-Config information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIG-START

TDD-UL-DL-ConfigCommon ::=			SEQUENCE {

referenceSubcarrierSpacing			SubcarrierSpacing,

pattern1							TDD-UL-DL-Pattern,

pattern2							TDD-UL-DL-Pattern															OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::= 				SEQUENCE {

dl-UL-TransmissionPeriodicity		ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots					INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols					INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots						INTEGER (0..maxNrofSlots),

nrofUplinkSymbols					INTEGER (0..maxNrofSymbols-1),

...

}

TDD-UL-DL-ConfigDedicated ::=		SEQUENCE {

slotSpecificConfigurationsToAddModList		SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig					OPTIONAL, -- Need N

slotSpecificConfigurationsToreleaseList		SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex					OPTIONAL,-- Need N

...

}

TDD-UL-DL-SlotConfig ::=			SEQUENCE {

slotIndex							TDD-UL-DL-SlotIndex,

symbols								CHOICE {

allDownlink							NULL,

allUplink							NULL,

explicit							SEQUENCE {

nrofDownlinkSymbols					INTEGER (1..maxNrofSymbols-1)												OPTIONAL,	-- Need S

nrofUplinkSymbols					INTEGER (1..maxNrofSymbols-1)												OPTIONAL	-- Need S

}

}

}

TDD-UL-DL-SlotIndex ::=				INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIG-STOP

-- ASN1STOP

TDD-UL-DL-ConfigCommon field descriptions

referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the values 15, 30 or 60 kHz (<6GHz) and 60 or 120 kHz (>6GHz) are applicable. The network configures a not larger than any SCS of configured BWPs for the serving cell. Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section)

TDD-UL-DL-Pattern field descriptions

dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see 38.211, section FFS_Section.

nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see 38.213, Table 4.3.2-1. In this release, the maximum value for this field is 80.

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see 38.211¨3, section FFS_Section).

nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see 38.213, Table 4.3.2-1. In this release, the maximum value for this field is 80.

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see 38.213, section FFS_Section)

TDD-UL-DL-ConfigDedicated field descriptions

slotSpecificConfigurationsToAddModList The slotSpecificConfiguration allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon.

TDD-UL-DL-SlotConfig field descriptions

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see 38.213, section FFS_Section)

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see 38.213, section FFS_Section)

slotIndex Identifies a slot within a dl-UL-TransmissionPeriodicity (given in tdd-UL-DL-configurationCommon)

symbols The direction (downlink or uplink) for the symbols in this slot. "allDownlink" indicates that all symbols in this slot are used for downlink; "allUplink" indicates that all symbols in this slot are used for uplink; "explicit" indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively.

#### –	TrackingAreaCode

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN, see TS 24.501 [FFS_Ref].

FFS whether CHOICE of 16 bit TAC is also needed.

TrackingAreaCodeinformation element

-- ASN1START

TrackingAreaCode ::= BIT STRING (SIZE (24))

-- ASN1STOP

#### –	T-Reselection

Editor's Note: Text and value converted from 36.331.

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselectioninformation element

-- ASN1START

T-Reselection ::=					INTEGER (0..7)

-- ASN1STOP

#### –	TimeToTrigger

The IE TimeToTrigger specifies the value range used for time to trigger parameter, which concerns the time during which specific criteria for the event needs to be met in order to trigger a measurement report. Value ms0 corresponds to 0 ms and behaviour as specified in 7.1.2 applies, ms40 corresponds to 40 ms, and so on.

TimeToTrigger information element

-- ASN1START

TimeToTrigger ::=					ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

-- ASN1STOP

Editor's Note:: Values should be checked.

#### –	UplinkConfigCommon

The IE UplinkConfigCommonprovides common uplink parameters of a cell.

UplinkConfigCommoninformation element

-- ASN1START

-- TAG-UPLINK-CONFIG-COMMON-START

UplinkConfigCommon ::=				SEQUENCE {

frequencyInfoUL						FrequencyInfoUL										OPTIONAL,	-- Cond InterFreqHOAndServCellAddAndSIB1

initialUplinkBWP						BWP-UplinkCommon										OPTIONAL,	-- Cond ServCellAddAndSIB1

timeAlignmentTimerCommon				TimeAlignmentTimer

}

-- TAG-UPLINK-CONFIG-COMMON-STOP

-- ASN1STOP

UplinkConfigCommon field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

initialUplinkBWP The initial uplink BWP configuration for a SpCell (PCell of MCG or SCG). Corresponds to L1 parameter 'initial-UL-BWP'. (see 38.331, section FFS_Section).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAddAndSIB1 | This field is mandatory present for inter-frequency handover, SIB1 and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAddAndSIB1 | This field is mandatory present for SIB1 and upon serving cell addition (for PSCell and SCell). It is optionally present, Need M otherwise. |

#### –	UplinkTxDirectCurrentList

The IE UplinkTxDirectCurrentList indicates the Tx Direct Current locations per serving cell for each configured UL BWP in the serving cell, based on the BWP numerology and the associated carrier bandwidth.

UplinkTxDirectCurrentList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTLIST-START

UplinkTxDirectCurrentList ::=			SEQUENCE (SIZE (1..maxNrofServingCells)) OF UplinkTxDirectCurrentCell

UplinkTxDirectCurrentCell ::=			SEQUENCE {

servCellIndex							ServCellIndex,

uplinkDirectCurrentBWP					SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP,

...

}

UplinkTxDirectCurrentBWP ::=			SEQUENCE {

bwp-Id									BWP-Id,

shift7dot5kHz							BOOLEAN,

txDirectCurrentLocation				INTEGER (0..3301)

}

-- TAG-UPLINKTXDIRECTCURRENTLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentBWP field descriptions

bwp-Id The BWP-Id of the corresponding uplink BWP.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to TRUE. Otherwise 7.5 kHz shift is not applied.

txDirectCurrentLocation The uplink Tx Direct Current location for the carrier. Only values in the value range of this field between 0 and 3299, which indicate the subcarrier index within the carrier corresponding to the numerology of the corresponding uplink BWP and value 3300, which indicates "Outside the carrier" and value 3301, which indicates "Undetermined position within the carrier" are used in this version of the specification.

UplinkTxDirectCurrentCell field descriptions

servCellIndex The serving cell ID of the serving cell corresponding to the uplinkDCLocationsPerBWP.

uplinkDirectCurrentBWP The Tx Direct Current locations for all the uplink BWPs configured at the corresponding serving cell.

#### –	UE-TimersAndConstants

The IE UE-TimersAndConstants contains timers and constants used by the UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstants information element

-- ASN1START

-- TAG-UE-TIMERS-AND-CONSTANTS-START

UE-TimersAndConstants ::=			SEQUENCE {

t300								ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t301								ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t310								ENUMERATED { ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310								ENUMERATED { n1, n2, n3, n4, n6, n8, n10, n20},

t311								ENUMERATED { ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000},

n311								ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10},

t319								ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms1500,	ms2000},

...

}

-- TAG-UE-TIMERS-AND-CONSTANTS-STOP

-- ASN1STOP

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfig' (see 38.214, section 5.1.4.2).

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=				SEQUENCE {

zp-CSI-RS-ResourceId				ZP-CSI-RS-ResourceId,

resourceMapping						CSI-RS-ResourceMapping,

periodicityAndOffset				CSI-ResourcePeriodicityAndOffset							OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=			INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

ZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS. Corresponds to L1 parameter 'ZP-CSI-RS-timeConfig' (see 38.214, section 5.1.4.2)

resourceMapping OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot

zp-CSI-RS-ResourceId ZP CSI-RS resource configuration ID. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigId' (see 38.214, section 5.1.4.2)

#### –	ZP-CSI-RS-ResourceSet

The IE ZP-CSI-RS-ResourceSet refers to a set of ZP-CSI-RS-Resources using their ZP-CSI-RS-ResourceIds. It corresponds to the L1 parameter 'ZP-CSI-RS-ResourceSetConfigList'.

ZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESET-START

ZP-CSI-RS-ResourceSet ::= 			SEQUENCE {

zp-CSI-RS-ResourceSetId				ZP-CSI-RS-ResourceSetId,

zp-CSI-RS-ResourceIdList			SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId,

...

}

-- TAG-ZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

ZP-CSI-RS-ResourceSet field descriptions

zp-CSI-RS-ResourceIdList The list of ZP-CSI-RS-ResourceId identifying the ZP-CSI-RS-Resource elements belonging to this set.

#### –	ZP-CSI-RS-ResourceSetId

The IE ZP-CSI-RS-ResourceSetId identifies a ZP-CSI-RS-ResourceSet.

ZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESETID-START

ZP-CSI-RS-ResourceSetId ::= 					INTEGER (0..maxNrofZP-CSI-RS-ResourceSets-1)

-- TAG-ZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

### 6.3.3	UE capability information elements

#### –	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=			SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombination ::= 				SEQUENCE {

bandList								SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination				FeatureSetCombinationId,

ca-ParametersEUTRA					CA-ParametersEUTRA					OPTIONAL,

ca-ParametersNR						CA-ParametersNR						OPTIONAL,

mrdc-Parameters						MRDC-Parameters						OPTIONAL,

supportedBandwidthCombinationSet		BIT STRING (SIZE (1..32))			OPTIONAL

}

BandParameters ::=					CHOICE {

eutra							SEQUENCE {

bandEUTRA						FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA		CA-BandwidthClassEUTRA				OPTIONAL,

ca-BandwidthClassUL-EUTRA		CA-BandwidthClassEUTRA				OPTIONAL

},

nr								SEQUENCE {

bandNR							FreqBandIndicatorNR,

ca-BandwidthClassDL-NR			CA-BandwidthClassNR					OPTIONAL,

ca-BandwidthClassUL-NR			CA-BandwidthClassNR					OPTIONAL

}

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

BandCombination field descriptions

supportedBandwidthCombinationSet For NR SA and for inter-band EN-DC, the field defines the bandwidth combinations for the NR part of the band combination. For intra-band EN-DC, the field indicates the supported bandwidth combination set applicable to the NR and LTE band combinations. The first (left-most) bit in the bitmap corresponds to the BWCS#1 and so on. If the bit is set to 1, the UE supports the corresponding BWCS.

#### –	CA-BandwidthClassNR

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

CA-BandwidthClassNR ::= ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...}

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

#### –	CA-BandwidthClassEUTRA

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=	ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

#### –	CA-ParametersNR

The IE CA-ParametersNR  contains carrier aggregation related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=	SEQUENCE {

multipleTimingAdvances				ENUMERATED {supported}		OPTIONAL,

parallelTxSRS-PUCCH-PUSCH			ENUMERATED {supported}		OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH		ENUMERATED {supported}		OPTIONAL,

simultaneousRxTxInterBandCA			ENUMERATED {supported}		OPTIONAL,

simultaneousRxTxSUL					ENUMERATED {supported}		OPTIONAL,

diffNumerologyAcrossPUCCH-Group		ENUMERATED {supported}		OPTIONAL,

diffNumerologyWithinPUCCH-Group		ENUMERATED {supported}		OPTIONAL,

supportedNumberTAG					ENUMERATED {n2, n3, n4}		OPTIONAL,

...

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

#### –	CA-ParametersEUTRA

The IE CA-ParameterEUTRA contains the EUTRA part of band combination parameters for a given MR-DC band combination.

NOTE:	If an additional EUTRA band combonation parameters are defined in TS 36.331 [10], which are supported for MR-DC, they will be defined here as well.

-- ASN1START

-- TAG-CA-PARAMETERSEUTRA-START

CA-ParametersEUTRA ::=	SEQUENCE {

multipleTimingAdvance					ENUMERATED {supported}							OPTIONAL,

simultaneousRx-Tx						ENUMERATED {supported}							OPTIONAL,

supportedNAICS-2CRS-AP					BIT STRING (SIZE (1..8))						OPTIONAL,

additionalRx-Tx-PerformanceReq			ENUMERATED {supported}							OPTIONAL,

ue-CA-PowerClass-N						ENUMERATED {class2}								OPTIONAL,

...,

[[

supportedBandwidthCombinationSetEUTRA	BIT STRING (SIZE (1..32))						OPTIONAL

]]

}

-- TAG-CA-PARAMETERSEUTRA-STOP

-- ASN1STOP

CA-ParametersEUTRA field descriptions

supportedBandwidthCombinationSetEUTRA Indicates the set of supported bandwidth combinations for the LTE part for inter-band EN-DC. The first (left-most) bit in the bitmap corresponds to the BWCS#1 and so on. If the bit is set to 1, the UE supports the corresponding BWCS.

#### –	FeatureSetCombination

The IE FeatureSetCombination is a two dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

Each FeatureSet contains either a pair of NR- or EUTRA feature set IDs for UL and DL.

In case of NR, the actual feature sets for UL and DL are defined in the FeatureSets IE and referred to from here by their ID, i.e., their position in the featureSetsUplink / featureSetsDownlink list in the FeatureSet IE.

In case of EUTRA, the feature sets referred to from this list are defined in TS 36.331 and conveyed as part of the UE-EUTRA-Capability container. The FeatureSetUL-Id-r15 and FeatureSetDL-Id-r15 in the EUTRA feature sets correspond to the FeatureSetEUTRA-DownlinkId and FeatureSetEUTRA-UplinkId, respectively.

The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Id:s and FeatureSetDownlinkPerCC-Id:s. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of FeatureSetUplink-Id:s/DownlinkPerCC-Id:s shall not exceed the number of carrier supported according to the BWC indicated in the associated BandCombination, if present.

FeatureSetCombination information element

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=	SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::= 		SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=					CHOICE {

eutra							SEQUENCE {

downlinkSetEUTRA				FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA					FeatureSetEUTRA-UplinkId

},

nr								SEQUENCE {

downlinkSetNR					FeatureSetDownlinkId,

uplinkSetNR						FeatureSetUplinkId

}

}

-- ASN1STOP

-- TAG-FEATURESETCOMBINATION-STOP

#### –	FeatureSetCombinationId

The IE FeatureSetCombinationId identifies a FeatureSetCombination. The FeatureSetCombinationId of a FeatureSetCombination is the position of the FeatureSetCombination in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability).

FeatureSetCombinationId information element

-- ASN1START

-- TAG-FEATURESET-COMBINATION-ID-START

FeatureSetCombinationId ::= 				INTEGER (0.. maxFeatureSetCombinations)

-- TAG-FEATURESET-COMBINATION-ID-STOP

-- ASN1STOP

#### –	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=				SEQUENCE {

featureSetListPerDownlinkCC			SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL				FreqSeparationClass														OPTIONAL,

scalingFactor							ENUMERATED {f0p4, f0p75, f0p8}											OPTIONAL,

crossCarrierSchedulingDL-OtherSCS		ENUMERATED {supported}													OPTIONAL,

scellWithoutSSB							ENUMERATED {supported}													OPTIONAL,

csi-RS-MeasSCellWithoutSSB				ENUMERATED {supported}													OPTIONAL,

srs-AssocCSI-RS							ENUMERATED {supported}													OPTIONAL,

type1-3-CSS								ENUMERATED {supported}													OPTIONAL,

pdcchMonitoringAnyOccasions				ENUMERATED {withoutDCI-Gap, withDCI-Gap}								OPTIONAL,

pdcchMonitoringAnyOccasionsWithSpanGap	ENUMERATED {supported}													OPTIONAL,

ue-SpecificUL-DL-Assignment				ENUMERATED {supported}													OPTIONAL,

searchSpaceSharingCA-DL					ENUMERATED {supported}													OPTIONAL,

timeDurationForQCL						SEQUENCE {

scs-60kHz								ENUMERATED {s7, s14, s28}											OPTIONAL,

sch-120kHz								ENUMERATED {s14, s28}												OPTIONAL

}																												OPTIONAL,

pdsch-DifferentTB-PerSlot				SEQUENCE {

scs-15kHz								ENUMERATED {upto2, upto4, upto7}									OPTIONAL,

scs-30kHz								ENUMERATED {upto2, upto4, upto7}									OPTIONAL,

scs-60kHz								ENUMERATED {upto2, upto4, upto7}									OPTIONAL,

scs-120kHz								ENUMERATED {upto2, upto4, upto7}									OPTIONAL

}																												OPTIONAL,

csi-RS-IM-ReceptionForFeedback			CSI-RS-IM-ReceptionForFeedback											OPTIONAL,

typeI-SinglePanelCodebookList			SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF TypeI-SinglePanelCodebook	OPTIONAL,

typeI-MultiPanelCodebookList			SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF TypeI-MultiPanelCodebook		OPTIONAL,

typeII-CodebookList						SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF TypeII-Codebook				OPTIONAL,

typeII-CodebookPortSelectionList		SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF TypeII-CodebookPortSelection	OPTIONAL

}

CSI-RS-IM-ReceptionForFeedback ::=	SEQUENCE {

maxNumberNZP-CSI-RS-PerCC					INTEGER (1..32),

maxNumberPortsAcrossNZP-CSI-RS-PerCC			ENUMERATED {p2, p4, p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256},

maxNumberCS-IM-PerCC							ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousCSI-RS-ActBWP-AllCC		ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26,

n28, n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52,

n54, n56, n58, n60, n62, n64},

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC	ENUMERATED {p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256}

}

TypeI-SinglePanelCodebook ::=		SEQUENCE {

maxNumberTxPortsPerResource			ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources					INTEGER (1..64),

totalNumberTxPorts					INTEGER (2..256),

supportedCodebookMode				ENUMERATED {mode1, mode1AndMode2},

maxNumberCSI-RS-PerResourceSet		INTEGER (1..8)

}

TypeI-MultiPanelCodebook ::=		SEQUENCE {

maxNumberTxPortsPerResource			ENUMERATED {p8, p16, p32},

maxNumberResources					INTEGER (1..64),

totalNumberTxPorts					INTEGER (2..256),

supportedCodebookMode				ENUMERATED {mode1, mode2, both},

supportedNumberPanels				ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet		INTEGER (1..8)

}

TypeII-Codebook ::=					SEQUENCE {

maxNumberTxPortsPerResource			ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources					INTEGER (1..64),

totalNumberTxPorts					INTEGER (2..256),

parameterLx							INTEGER (2..4),

amplitudeScalingType				ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction			ENUMERATED {supported}							OPTIONAL,

maxNumberCSI-RS-PerResourceSet		INTEGER (1..8)

}

TypeII-CodebookPortSelection ::=	SEQUENCE {

maxNumberTxPortsPerResource			ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources					INTEGER (1..64),

totalNumberTxPorts					INTEGER (2..256),

parameterLx							INTEGER (2..4),

amplitudeScalingType				ENUMERATED {wideband, widebandAndSubband},

maxNumberCSI-RS-PerResourceSet		INTEGER (1..8)

}

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

FeatureSetDownlink field descriptions

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassDL. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

#### –	FeatureSetDownlinkId

The IE FeatureSetDownlinkId identifies a downlink feature set. The FeatureSetDownlinkId of a FeatureSetDownlink is the index position of the FeatureSetDownlink in the featureSetsDownlink list in the FeatureSets IE. The first element in that list is referred to by FeatureSetDownlinkId = 1. The FeatureSetDownlinkId=0 is not used by an actual FeatureSetDownlink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetDownlinkId information element

-- ASN1START

-- TAG-FEATURESET-DOWNLINK-ID-START

FeatureSetDownlinkId ::= 				INTEGER (0..maxDownlinkFeatureSets)

-- TAG-FEATURESET-DOWNLINK-ID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-DownlinkId

The IE FeatureSetEUTRA-DownlinkId identifies a downlink feature set in EUTRA. The FeatureSetEUTRA-DownlinkId=0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-DownlinkId information element

-- ASN1START

-- TAG-FEATURESET-EUTRA-DOWNLINK-ID-START

FeatureSetEUTRA-DownlinkId ::= 				INTEGER (0..maxEUTRA-DL-FeatureSets)

-- TAG-FEATURESET-EUTRA-DOWNLINK-ID-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetDownlinkPerCC information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-START

FeatureSetDownlinkPerCC ::=		SEQUENCE {

supportedSubcarrierSpacingDL	SubcarrierSpacing,

supportedBandwidthDL			SupportedBandwidth,

channelBW-90mhz					ENUMERATED {supported}														OPTIONAL,

maxNumberMIMO-LayersPDSCH			MIMO-LayersDL															OPTIONAL,

supportedModulationOrderDL			ModulationOrder															OPTIONAL

}

-- TAG-FEATURESETDOWNLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC-Id

The IE FeatureSetDownlinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetDownlinkPerCC-Id of a FeatureSetDownlinkPerCC is the index position of the FeatureSetDownlinkPerCC in the featureSetsDownlinkPerCC. The first element in the list is referred to by FeatureSetDownlinkPerCC-Id = 1, and so on.

FeatureSetDownlinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESET-DOWNLINK-PER-CC-ID-START

FeatureSetDownlinkPerCC-Id ::=			INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESET-DOWNLINK-PER-CC-ID-STOP

-- ASN1STOP

#### –	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=					SEQUENCE {

featureSetListPerUplinkCC				SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor						ENUMERATED {f0p4, f0p75, f0p8}				OPTIONAL,

crossCarrierSchedulingUL-OtherSCS	ENUMERATED {supported}						OPTIONAL,

intraBandFreqSeparationUL				FreqSeparationClass					OPTIONAL,

searchSpaceSharingCA-UL				ENUMERATED {supported}						OPTIONAL,

srs-TxSwitch						SRS-TxSwitch								OPTIONAL,

supportedSRS-Resources				SRS-Resources								OPTIONAL,

twoPUCCH-Group						ENUMERATED {supported}						OPTIONAL,

dynamicSwitchSUL					ENUMERATED {supported}						OPTIONAL,

pusch-DifferentTB-PerSlot			SEQUENCE {

scs-15kHz							ENUMERATED {upto2, upto4, upto7}			OPTIONAL,

scs-30kHz							ENUMERATED {upto2, upto4, upto7}			OPTIONAL,

scs-60kHz							ENUMERATED {upto2, upto4, upto7}			OPTIONAL,

scs-120kHz							ENUMERATED {upto2, upto4, upto7}			OPTIONAL

}																				OPTIONAL,

csi-ReportFramework					CSI-ReportFramework							OPTIONAL

}

CSI-ReportFramework ::=					SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP			INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP			INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP		INTEGER (0..4),

simultaneousCSI-ReportsAllCC					INTEGER (5..32)

}

-- TAG- FEATURESETUPLINK-STOP

-- ASN1STOP

FeatureSetUplink field descriptions

featureSetsPerUplinkCC Indicates which features the UE supports on the individual carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassUL. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

#### –	FeatureSetUplinkId

The IE FeatureSetUplinkId identifies a downlink feature set. The FeatureSetUplinkId of a FeatureSetUplink is the index position of the FeatureSetUplink in the featureSetsUplink list in the FeatureSets IE. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.The FeatureSetUplinkId =0 is not used by an actual FeatureSetUplink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetUplinkId information element

-- ASN1START

-- TAG-FEATURESET-UPLINK-ID-START

FeatureSetUplinkId ::= 					INTEGER (0..maxUplinkFeatureSets)

-- TAG-FEATURESET-UPLINK-ID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-UplinkId

The IE FeatureSetEUTRA-UplinkId identifies an uplink feature set. The FeatureSetEUTRA-UplinkId =0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-UplinkId information element

-- ASN1START

-- TAG-FEATURESET-EUTRA-UPLINK-ID-START

FeatureSetEUTRA-UplinkId ::= 					INTEGER (0..maxEUTRA-UL-FeatureSets)

-- TAG-FEATURESET-EUTRA-UPLINK-ID-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetUplinkPerCC information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-START

FeatureSetUplinkPerCC ::=			SEQUENCE {

supportedSubcarrierSpacingUL			SubcarrierSpacing,

supportedBandwidthUL					SupportedBandwidth,

channelBW-90mhz						ENUMERATED {supported}						OPTIONAL,

mimo-CB-PUSCH						SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL							OPTIONAL,

maxNumberSRS-ResourcePerSet			INTEGER (1..2)

}																				OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL								OPTIONAL,

supportedModulationOrderUL			ModulationOrder								OPTIONAL,

simultaneousTxSUL-NonSUL			ENUMERATED {supported}						OPTIONAL

}

-- TAG-FEATURESETUPLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC-Id

The IE FeatureSetUplinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetUplinkPerCC-Id of a FeatureSetUplinkPerCC is the index position of the FeatureSetUplinkPerCC in the featureSetsUplinkPerCC. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.

FeatureSetUplinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESET-UPLINK-PER-CC-ID-START

FeatureSetUplinkPerCC-Id ::=			INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESET-UPLINK-PER-CC-ID-STOP

-- ASN1STOP

#### –	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports fot that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=	SEQUENCE {

featureSetsDownlink					SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink				OPTIONAL,

featureSetsDownlinkPerCC			SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC			OPTIONAL,

featureSetsUplink					SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink					OPTIONAL,

featureSetsUplinkPerCC				SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC				OPTIONAL,

...

}

-- ASN1STOP

-- TAG-FEATURESETS-STOP

#### –	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQ-BAND-INDICATOR-EUTRA-START

FreqBandIndicatorEUTRA ::=	INTEGER (1..maxBandsEUTRA)

-- TAG-FREQ-BAND-INDICATOR-EUTRA-STOP

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used by the network to request NR CA and/or MR-DC band combinations for specific NR and/or E-UTRA frequency bands and/or up to a specific number of carriers and/or up to a specific aggregated bandwidths.

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=				SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::= 		CHOICE {

bandInformationEUTRA			FreqBandInformationEUTRA,

bandInformationNR				FreqBandInformationNR

}

FreqBandInformationEUTRA ::=	SEQUENCE {

bandEUTRA						FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA		CA-BandwidthClassEUTRA					OPTIONAL,	-- Need N

ca-BandwidthClassUL-EUTRA		CA-BandwidthClassEUTRA					OPTIONAL	-- Need N

}

FreqBandInformationNR ::=		SEQUENCE {

bandNR							FreqBandIndicatorNR,

maxBandwidthRequestedDL			AggregatedBandwith			OPTIONAL, 	-- Need N

maxBandwidthRequestedUL			AggregatedBandwith			OPTIONAL, 	-- Need N

maxCarriersRequestedDL			INTEGER (1.. maxNrofServingCells)		OPTIONAL, 	-- Need N

maxCarriersRequestedUL			INTEGER (1.. maxNrofServingCells)		OPTIONAL	-- Need N

}

AggregatedBandwith ::=			ENUMERATED {mhz50, mhz100, mhz150, mhz200, mhz250, mhz300, mhz350,

mhz400, mhz450, mhz500, mhz550, mhz600, mhz650, mhz700, mhz750, mhz800}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

#### –	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::=	ENUMERATED {c1, c2, c3, ...}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	MIMO-Layers

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=	ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=	ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

#### –	ModulationOrder

-- ASN1START

-- TAG-MODULATION-ORDER-START

ModulationOrder ::=	ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATION-ORDER-STOP

-- ASN1STOP

#### –	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::=	SEQUENCE {

singleUL-Transmission				ENUMERATED {supported}		OPTIONAL,

dynamicPowerSharing					ENUMERATED {supported}		OPTIONAL,

tdm-Pattern							ENUMERATED {supported}		OPTIONAL,

ul-SharingEUTRA-NR					ENUMERATED {tdm, fdm, both}		OPTIONAL,

ul-SwitchingTimeEUTRA-NR			ENUMERATED {type1, type2}	OPTIONAL,

simultaneousRxTxInterBandENDC		ENUMERATED {supported}		OPTIONAL,

asyncIntraBandENDC					ENUMERATED {supported}		OPTIONAL,

...

}

-- TAG-MRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, eutra, spare1, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	SupportedBandwidth

The IE SupportedBandwidth is used to indicate the maximum channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth ::=		CHOICE {

fr1							ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2							ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of radio access technology specific capability containers.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ue-CapabilityRAT-Container			OCTET STRING

}

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For rat-Type set to nr: the encoding of UE capabilities is defined in UE-NR-Capability. For rat-Type set to eutra-nr: the encoding of UE capabilities is defined in UE-MRDC-Capability. For rat-Type set to eutra: the encoding of UE capabilities is defined in UE-EUTRA-Capability specified in 36.331.

#### –	UE-CapabilityRAT-RequestList

The IE UE-CapabilityRAT-RequestList is used to request UE capabilities for one or more RATs from the UE.

UE-CapabilityRAT-RequestList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-START

UE-CapabilityRAT-RequestList ::=		SEQUENCE (SIZE (1..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request

UE-CapabilityRAT-Request ::=			SEQUENCE {

rat-Type								RAT-Type,

capabilityRequestFilter				OCTET STRING					OPTIONAL,	-- Need N

...

}

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-Request field descriptions

capabilityRequestFilter Information by which the network requests the UE to filter the UE capabilities. For ratType set to nr: the encoding of the capabilityRequestFilter is defined in UE-CapabilityRequestFilterNR.

rat-Type The RAT type for which the NW requests UE capabilities.

#### –	UE-CapabilityRequestFilterNR

The IE UE-CapabilityRequestFilterNR is used to request filtered UE capabilities.

UE-CapabilityRequestFilterNR information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERNR-START

UE-CapabilityRequestFilterNR ::=			SEQUENCE {

frequencyBandList							FreqBandList				OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERNR-STOP

#### -- ASN1STOP

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=	SEQUENCE {

measParametersMRDC					MeasParametersMRDC					OPTIONAL,

rf-ParametersMRDC					RF-ParametersMRDC,

generalParametersMRDC				GeneralParametersMRDC-XDD-Diff		OPTIONAL,

fdd-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddXDD-Mode		OPTIONAL,

tdd-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddXDD-Mode		OPTIONAL,

fr1-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddFRX-Mode		OPTIONAL,

fr2-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddFRX-Mode		OPTIONAL,

featureSetCombinations				SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination			OPTIONAL,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=	SEQUENCE {

measParametersMRDC-XDD-Diff		MeasParametersMRDC-XDD-Diff		OPTIONAL,

generalParametersMRDC-XDD-Diff		GeneralParametersMRDC-XDD-Diff		OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=	SEQUENCE {

measParametersMRDC-FRX-Diff		MeasParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path				ENUMERATED {supported}		OPTIONAL,

splitDRB-withUL-Both-MCG-SCG		ENUMERATED {supported}		OPTIONAL,

srb3								ENUMERATED {supported}		OPTIONAL,

...

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for MR-DC. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

#### –	RF-ParametersMRDC

The IE RF-ParametersMRDC is used to convey RF related capabilities for MR-DC.

RF-ParametersMRDC information element

-- ASN1START

-- TAG-RF-PARAMETERSMRDC-START

RF-ParametersMRDC ::= SEQUENCE {

supportedBandCombinationList		BandCombinationList					OPTIONAL,

appliedFreqBandListFilter			FreqBandList							OPTIONAL

}

-- TAG-RF-PARAMETERSMRDC-STOP

-- ASN1STOP

RF-ParametersMRDC field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for MR-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

#### –	MeasParametersMRDC

The IE MeasParametersMRDC is used to configure FFS

MeasParametersMRDC information element

-- ASN1START

-- TAG-MEASPARAMETERSMRDC-START

MeasParametersMRDC ::= SEQUENCE {

measParametersMRDC-Common		MeasParametersMRDC-Common				OPTIONAL,

measParametersMRDC-XDD-Diff		MeasParametersMRDC-XDD-Diff				OPTIONAL,

measParametersMRDC-FRX-Diff		MeasParametersMRDC-FRX-Diff				OPTIONAL

}

MeasParametersMRDC-Common ::=	SEQUENCE {

independentGapConfig			ENUMERATED {supported}				OPTIONAL

}

MeasParametersMRDC-XDD-Diff ::=	SEQUENCE {

sftd-MeasPSCell					ENUMERATED {supported}				OPTIONAL,

sftd-MeasNR-Cell				ENUMERATED {supported}				OPTIONAL

}

MeasParametersMRDC-FRX-Diff ::=	SEQUENCE {

simultaneousRxDataSSB-DiffNumerology	ENUMERATED {supported}		OPTIONAL

}

-- TAG-MEASPARAMETERSMRDC-STOP

-- ASN1STOP

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

accessStratumRelease			AccessStratumRelease,

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters						OPTIONAL,

mac-Parameters					MAC-Parameters						OPTIONAL,

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

featureSets						FeatureSets 						OPTIONAL,

featureSetCombinations			SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination			OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			UE-NR-Capability-vxy							OPTIONAL

}

UE-NR-Capability-vxy ::= 			SEQUENCE {

voiceOverMCGBearer					ENUMERATED {supported}			OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff			OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff			OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for NR (not for MR-DC). The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

#### –	Phy-Parameters

The IE Phy-Parameters is used to convey the physical layer capabilities.

Phy-Parameters information element

-- ASN1START

-- TAG-PHY-PARAMETERS-START

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff				OPTIONAL,

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff				OPTIONAL,

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL

}

Phy-ParametersCommon ::=	SEQUENCE {

csi-RS-CFRA-ForHO					ENUMERATED {supported}						OPTIONAL,

dynamicPRB-BundlingDL				ENUMERATED {supported}						OPTIONAL,

sp-CSI-ReportPUCCH					ENUMERATED {supported}						OPTIONAL,

sp-CSI-ReportPUSCH					ENUMERATED {supported}						OPTIONAL,

nzp-CSI-RS-IntefMgmt				ENUMERATED {supported}						OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH		ENUMERATED {supported}						OPTIONAL,

precoderGranularityCORESET			ENUMERATED {supported}						OPTIONAL,

dynamicHARQ-ACK-Codebook			ENUMERATED {supported}						OPTIONAL,

semiStaticHARQ-ACK-Codebook			ENUMERATED {supported}						OPTIONAL,

spatialBundlingHARQ-ACK				ENUMERATED {supported}						OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI	ENUMERATED {supported}						OPTIONAL,

pucch-Repetition-F1-3-4				ENUMERATED {supported}						OPTIONAL,

ra-Type0-PUSCH						ENUMERATED {supported}						OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH		ENUMERATED {supported}						OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH		ENUMERATED {supported}						OPTIONAL,

pdsch-MappingTypeA					ENUMERATED {supported}						OPTIONAL,

pdsch-MappingTypeB					ENUMERATED {supported}						OPTIONAL,

interleavingVRB-ToPRB-PDSCH			ENUMERATED {supported}						OPTIONAL,

interSlotFreqHopping-PUSCH			ENUMERATED {supported}						OPTIONAL,

type1-PUSCH-RepetitionMultiSlots	ENUMERATED {supported}						OPTIONAL,

type2-PUSCH-RepetitionMultiSlots	ENUMERATED {supported}						OPTIONAL,

pusch-RepetitionMultiSlots			ENUMERATED {supported}						OPTIONAL,

pdsch-RepetitionMultiSlots			ENUMERATED {supported}						OPTIONAL,

downlinkSPS							ENUMERATED {supported}						OPTIONAL,

configuredUL-GrantType1				ENUMERATED {supported}						OPTIONAL,

configuredUL-GrantType2				ENUMERATED {supported}						OPTIONAL,

pre-EmptIndication-DL				ENUMERATED {supported}						OPTIONAL,

cbg-TransIndication-DL				ENUMERATED {supported}						OPTIONAL,

cbg-TransIndication-UL				ENUMERATED {supported}						OPTIONAL,

cbg-FlushIndication-DL				ENUMERATED {supported}						OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL	ENUMERATED {supported}						OPTIONAL,

rateMatchingResrcSetSemi-Static		ENUMERATED {supported}						OPTIONAL,

rateMatchingResrcSetDynamic			ENUMERATED {supported}						OPTIONAL,

bwp-SwitchingDelay					ENUMERATED {type1, type2}					OPTIONAL,

...

}

Phy-ParametersXDD-Diff ::=	SEQUENCE {

dynamicSFI							ENUMERATED {supported}						OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUSCH			ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUCCH			ENUMERATED {supported}						OPTIONAL,

...

}

Phy-ParametersFRX-Diff ::=	SEQUENCE {

dynamicSFI							ENUMERATED {supported}						OPTIONAL,

oneFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

twoFL-DMRS							BIT STRING (SIZE (2))						OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

supportedDMRS-TypeDL				ENUMERATED {type1, type2}					OPTIONAL,

supportedDMRS-TypeUL				ENUMERATED {type1, type2}					OPTIONAL,

semiOpenLoopCSI						ENUMERATED {supported}						OPTIONAL,

csi-ReportWithoutPMI				ENUMERATED {supported}						OPTIONAL,

csi-ReportWithoutCQI				ENUMERATED {supported}						OPTIONAL,

onePortsPTRS						BIT STRING (SIZE (2))						OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

pucch-F2-WithFH						ENUMERATED {supported}						OPTIONAL,

pucch-F3-WithFH						ENUMERATED {supported}						OPTIONAL,

pucch-F4-WithFH						ENUMERATED {supported}						OPTIONAL,

freqHoppingPUCCH-F0-2				ENUMERATED {notSupported}					OPTIONAL,

freqHoppingPUCCH-F1-3-4				ENUMERATED {notSupported}					OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH			ENUMERATED {supported}						OPTIONAL,

uci-CodeBlockSegmentation			ENUMERATED {supported}						OPTIONAL,

onePUCCH-LongAndShortFormat			ENUMERATED {supported}						OPTIONAL,

twoPUCCH-AnyOthersInSlot			ENUMERATED {supported}						OPTIONAL,

intraSlotFreqHopping-PUSCH			ENUMERATED {supported}						OPTIONAL,

pusch-LBRM							ENUMERATED {supported}						OPTIONAL,

dummy1							ENUMERATED {supported}						OPTIONAL,

tpc-PUSCH-RNTI						ENUMERATED {supported}						OPTIONAL,

tpc-PUCCH-RNTI						ENUMERATED {supported}						OPTIONAL,

tpc-SRS-RNTI						ENUMERATED {supported}						OPTIONAL,

absoluteTPC-Command					ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUSCH			ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUCCH			ENUMERATED {supported}						OPTIONAL,

pusch-HalfPi-BPSK					ENUMERATED {supported}						OPTIONAL,

pucch-F3-4-HalfPi-BPSK				ENUMERATED {supported}						OPTIONAL,

almostContiguousCP-OFDM-UL			ENUMERATED {supported}						OPTIONAL ,

sp-CSI-RS							ENUMERATED {supported}						OPTIONAL,

sp-CSI-IM							ENUMERATED {supported}						OPTIONAL,

tdd-MultiDL-UL-SwitchPerSlot		ENUMERATED {supported}						OPTIONAL,

multipleCORESET						ENUMERATED {supported}						OPTIONAL,

...,

[[

pdcch-BlindDetectionCA				INTEGER (4..16)								OPTIONAL

]]

}

Phy-ParametersFR1 ::=	SEQUENCE {

pdcchMonitoringSingleOccasion		ENUMERATED {supported}						OPTIONAL,

scs-60kHz							ENUMERATED {supported}						OPTIONAL,

pdsch-256QAM-FR1					ENUMERATED {supported}						OPTIONAL,

pdsch-RE-MappingFR1					ENUMERATED {n10, n20}						OPTIONAL,

...

}

Phy-ParametersFR2 ::=	SEQUENCE {

calibrationGapPA					ENUMERATED {supported}						OPTIONAL,

pdsch-RE-MappingFR2					ENUMERATED {n6, n20}						OPTIONAL,

...

}

-- TAG-PHY-PARAMETERS-STOP

-- ASN1STOP

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::= SEQUENCE {

supportedBandListNR				SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList		BandCombinationList						OPTIONAL,

appliedFreqBandListFilter			FreqBandList							OPTIONAL

}

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicatorNR,

modifiedMPR-Behaviour			BIT STRING (SIZE (8))						OPTIONAL,

mimo-ParametersPerBand			MIMO-ParametersPerBand						OPTIONAL,

extendedCP						ENUMERATED {supported}						OPTIONAL,

multipleTCI						ENUMERATED {supported}						OPTIONAL,

bwp-WithoutRestriction			ENUMERATED {supported}						OPTIONAL,

bwp-SameNumerology					ENUMERATED {upto2, upto4}				OPTIONAL,

bwp-DiffNumerology					ENUMERATED {upto4}						OPTIONAL,

crossCarrierSchedulingDL-SameSCS		ENUMERATED {supported}						OPTIONAL,

crossCarrierSchedulingUL-SameSCS		ENUMERATED {supported}						OPTIONAL,

pdsch-256QAM-FR2				ENUMERATED {supported}						OPTIONAL,

pusch-256QAM					ENUMERATED {supported}						OPTIONAL,

ue-PowerClass					ENUMERATED {pc2, pc3}				OPTIONAL,

rateMatchingLTE-CRS				ENUMERATED {supported}						OPTIONAL,

...,

[[

channelBWs-DL		CHOICE {

fr1					SEQUENCE {

scs-15kHz        	BIT STRING (SIZE (10))   			OPTIONAL,

scs-30kHz      		BIT STRING (SIZE (10))   			OPTIONAL,

scs-60kHz 			BIT STRING (SIZE (10))				OPTIONAL

},

fr2					SEQUENCE {

scs-60kHz        	BIT STRING (SIZE (3)) 				OPTIONAL,

scs-120kHz     	 	BIT STRING (SIZE (3)) 				OPTIONAL

}

} 																OPTIONAL,

channelBWs-UL		CHOICE {

fr1					SEQUENCE {

scs-15kHz        	BIT STRING (SIZE (10))   			OPTIONAL,

scs-30kHz      		BIT STRING (SIZE (10))   			OPTIONAL,

scs-60kHz 			BIT STRING (SIZE (10))				OPTIONAL

},

fr2					SEQUENCE {

scs-60kHz        	BIT STRING (SIZE (3))   			OPTIONAL,

scs-120kHz     	 	BIT STRING (SIZE (3))   			OPTIONAL

}

} 																OPTIONAL

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for NR (without MR-DC). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE.

#### –	MIMO-ParametersPerBand

The IE MIMO-ParametersPerBand is used to convey MIMO related parameters specific for a certain band (not per feature set or band combination).

MIMO-ParametersPerBand information element

-- ASN1START

-- TAG-MIMO-PARAMETERSPERBAND-START

MIMO-ParametersPerBand ::= SEQUENCE {

tci-StatePDSCH						SEQUENCE {

maxNumberConfiguredTCIstatesPerCC	ENUMERATED {n4, n8, n16, n32, n64}							OPTIONAL,

maxNumberActiveTCI-PerBWP			ENUMERATED {n1, n2, n4, n8}									OPTIONAL

}																									OPTIONAL,

additionalActiveTCI-StatePDCCH		ENUMERATED {supported}											OPTIONAL,

pusch-TransCoherence				ENUMERATED {nonCoherent, partialNonCoherent, fullCoherent}	OPTIONAL,

beamCorrespondence					ENUMERATED {supported}											OPTIONAL,

periodicBeamReport					ENUMERATED {supported}											OPTIONAL,

aperiodicBeamReport					ENUMERATED {supported}											OPTIONAL,

sp-BeamReportPUCCH					ENUMERATED {supported}											OPTIONAL,

sp-BeamReportPUSCH					ENUMERATED {supported}											OPTIONAL,

beamManagementSSB-CSI-RS			BeamManagementSSB-CSI-RS										OPTIONAL,

maxNumberRxBeam						INTEGER (2..8)													OPTIONAL,

maxNumberRxTxBeamSwitchDL			SEQUENCE {

scs-15kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-30kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-60kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-120kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-240kHz							ENUMERATED {n4, n7, n14}									OPTIONAL

}																									OPTIONAL,

maxNumberNonGroupBeamReporting		ENUMERATED {n1, n2, n4}											OPTIONAL,

groupBeamReporting					ENUMERATED {supported}											OPTIONAL,

uplinkBeamManagement				SEQUENCE {

maxNumberSRS-ResourcePerSet-BM			ENUMERATED {n2, n4, n8, n16, dummy},

maxNumberSRS-ResourceSet			INTEGER (1..8)

}																									OPTIONAL,

maxNumberCSI-RS-BFR					INTEGER (1..64)													OPTIONAL,

maxNumberSSB-BFR					INTEGER (1..64)													OPTIONAL,

maxNumberCSI-RS-SSB-BFR				INTEGER (1..256)												OPTIONAL,

twoPortsPTRS-DL						ENUMERATED {supported}											OPTIONAL,

twoPortsPTRS-UL						ENUMERATED {supported}											OPTIONAL,

supportedSRS-Resources			SRS-Resources														OPTIONAL,

srs-TxSwitch					SRS-TxSwitch														OPTIONAL,

maxNumberSimultaneousSRS-PerCC		INTEGER (1..4)													OPTIONAL,

beamReportTiming					SEQUENCE {

scs-15kHz							ENUMERATED {sym2, sym4, sym8}								OPTIONAL,

scs-30kHz							ENUMERATED {sym4, sym8, sym14}								OPTIONAL,

scs-60kHz							ENUMERATED {sym8, sym14, sym28}								OPTIONAL,

scs-120kHz							ENUMERATED {sym14, sym28, sym56}							OPTIONAL

}																									OPTIONAL,

ptrs-DensityRecommendationSetDL		SEQUENCE {

scs-15kHz							PTRS-DensityRecommendationDL								OPTIONAL,

scs-30kHz							PTRS-DensityRecommendationDL								OPTIONAL,

scs-60kHz							PTRS-DensityRecommendationDL								OPTIONAL,

scs-120kHz							PTRS-DensityRecommendationDL								OPTIONAL

}																									OPTIONAL,

ptrs-DensityRecommendationSetUL		SEQUENCE {

scs-15kHz							PTRS-DensityRecommendationUL								OPTIONAL,

scs-30kHz							PTRS-DensityRecommendationUL								OPTIONAL,

scs-60kHz							PTRS-DensityRecommendationUL								OPTIONAL,

scs-120kHz							PTRS-DensityRecommendationUL								OPTIONAL

}																									OPTIONAL,

csi-RS-ForTracking					CSI-RS-ForTracking												OPTIONAL,

aperiodicTRS						ENUMERATED {supported}											OPTIONAL,

...

}

BeamManagementSSB-CSI-RS ::=	SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx	ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx	ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density				ENUMERATED {one, three, oneAndThree}

}

CSI-RS-ForTracking ::=				SEQUENCE {

burstLength							INTEGER (1..2),

maxSimultaneousResourceSetsPerCC	INTEGER (1..8),

maxConfiguredResourceSetsPerCC		INTEGER (1..64),

maxConfiguredResourceSetsAllCC		INTEGER (1..128)

}

PTRS-DensityRecommendationDL ::=	SEQUENCE {

frequencyDensity1					INTEGER (1..276),

frequencyDensity2					INTEGER (1..276),

timeDensity1						INTEGER (0..29),

timeDensity2						INTEGER (0..29),

timeDensity3						INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=	SEQUENCE {

frequencyDensity1					INTEGER (1..276),

frequencyDensity2					INTEGER (1..276),

timeDensity1						INTEGER (0..29),

timeDensity2						INTEGER (0..29),

timeDensity3						INTEGER (0..29),

sampleDensity1						INTEGER (1..276),

sampleDensity2						INTEGER (1..276),

sampleDensity3						INTEGER (1..276),

sampleDensity4						INTEGER (1..276),

sampleDensity5						INTEGER (1..276)

}

SRS-Resources ::=	SEQUENCE {

maxNumberAperiodicSRS-PerBWP					ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot			INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP						ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot				INTEGER (1..6),

maxNumberSemiPersitentSRS-PerBWP				ENUMERATED {n1, n2, n4, n8, n16},

maxNumberSP-SRS-PerBWP-PerSlot					INTEGER (1..6),

maxNumberSRS-Ports-PerResource					ENUMERATED {n1, n2, n4}

}

SRS-TxSwitch ::=	SEQUENCE {

supportedSRS-TxPortSwitch			ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, tr-equal},

txSwitchImpactToRx					ENUMERATED {true}										OPTIONAL

}

-- ASN1STOP

-- TAG-MIMO-PARAMETERSPERBAND-STOP

#### –	PDCP-Parameters

The IE PDCP-Parameters is used to convey capabilities related to PDCP.

PDCP-Parameters information element

-- ASN1START

-- TAG-PDCP-PARAMETERS-START

PDCP-Parameters ::= SEQUENCE {

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64,

cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context				ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL,

...

}

-- TAG-PDCP-PARAMETERS-STOP

-- ASN1STOP

#### –	RLC-Parameters

The IE RLC-Parameters is used to convey capabilities related to RLC.

RLC-Parameters information element

-- ASN1START

-- TAG-RLC-PARAMETERS-START

RLC-Parameters ::= SEQUENCE {

am-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WithLongSN					ENUMERATED {supported}	OPTIONAL,

...

}

-- TAG-RLC-PARAMETERS-STOP

-- ASN1STOP

#### –	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon			MAC-ParametersCommon	OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff	OPTIONAL

}

MAC-ParametersCommon ::=	SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

pucch-SpatialRelInfoMAC-CE		ENUMERATED {supported}	OPTIONAL,

lch-ToSCellRestriction			ENUMERATED {supported}	OPTIONAL,

...

}

MAC-ParametersXDD-Diff ::=	SEQUENCE {

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

multipleSR-Configurations		ENUMERATED {supported}	OPTIONAL,

multipleConfiguredGrants	ENUMERATED {supported}	OPTIONAL,

...

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

MAC-ParametersCommon field descriptions

lch-ToSCellRestriction Indicates whether the UE supports restricting data transmission from a given LCH to a configured (sub-)set of serving cells (see allowedServingCells in LogicalChannelConfig). A UE supporting pdcp-Duplication (see PDCP-Config) shall also support lch-ToSCellRestriction.

#### –	MeasParameters

The IE MeasParameters is used to convey UE capabilities related to measurements for radio resource management (RRM) and radio link monitoring (RLM).

MeasParameters information element

-- ASN1START

-- TAG-MEASPARAMETERS-START

MeasParameters ::= SEQUENCE {

measParametersCommon				MeasParametersCommon			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff		OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff		OPTIONAL

}

MeasParametersCommon ::= SEQUENCE {

supportedGapPattern				BIT STRING (SIZE (22))			OPTIONAL,

...

}

MeasParametersXDD-Diff ::=	SEQUENCE {

intraAndInterF-MeasAndReport	ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL,

...

}

MeasParametersFRX-Diff ::=	SEQUENCE {

ss-SINR-Meas						ENUMERATED {supported}		OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB		ENUMERATED {supported}		OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB		ENUMERATED {supported}		OPTIONAL,

csi-SINR-Meas						ENUMERATED {supported}		OPTIONAL,

csi-RS-RLM							ENUMERATED {supported}		OPTIONAL,

...

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

### 6.3.4	Other information elements

#### –	EUTRA-AllowedMeasBandwidth

The IE EUTRA-AllowedMeasBandwidth is used to indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [47]. The values mbw6, mbw15, mbw25, mbw50, mbw75, mbw100 indicate 6, 15, 25, 50, 75 and 100 resource blocks respectively.

EUTRA-AllowedMeasBandwidth information element

-- ASN1START

-- TAG-EUTRA-ALLOWED-MEAS-BANDWIDTH-START

EUTRA-AllowedMeasBandwidth ::=				ENUMERATED {mbw6, mbw15, mbw25, mbw50, mbw75, mbw100}

-- TAG-EUTRA-ALLOWED-MEAS-BANDWIDTH-STOP

-- ASN1STOP

#### –	EUTRA-MultiBandInfoList

The IE EUTRA-MultiBandInfoList indicates the list of frequency bands in addition to the band represented by CarrierFreq for which cell reselection parameters are common, and a list of additionalPmax and additionalSpectrumEmission.

EUTRA-MultiBandInfoList information element

-- ASN1START

-- TAG-EUTRA-MULTI-BAND-INFO-LIST-START

EUTRA-MultiBandInfoList ::=	SEQUENCE (SIZE (1..maxMultiBands)) OF EUTRA-MultiBandInfo

EUTRA-MultiBandInfo ::= SEQUENCE {

eutra-FreqBandIndicator FreqBandIndicatorEUTRA,

eutra-NS-PmaxList EUTRA-NS-PmaxList		OPTIONAL

}

maxMultiBands	INTEGER ::= ffsValue

-- TAG-EUTRA-MULTI-BAND-INFO-LIST-STOP

-- ASN1STOP

#### –	EUTRA-NS-PmaxList

The IE EUTRA-NS-PmaxList concerns a list of additionalPmax and additionalSpectrumEmission, as defined in TS 36.101 [22, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [22, table 6.2.4E-1] for UEs in CE or BL UEs, for a given frequency band.

EUTRA-NS-PmaxList information element

-- ASN1START

-- TAG-EUTRA-NS-PMAX-LIST-START

EUTRA-NS-PmaxList ::=				SEQUENCE (SIZE (1..maxEUTRA-NS-Pmax)) OF EUTRA-NS-PmaxValue

EUTRA-NS-PmaxValue ::=			SEQUENCE {

additionalPmax						INTEGER (-30..33)						OPTIONAL,	-- Need N

additionalSpectrumEmission		INTEGER (1..288)						OPTIONAL	-- Need N

}

-- TAG-EUTRA-NS-PMAX-LIST-STOP

-- ASN1STOP

#### –	EUTRA-PhysCellId

The IE EUTRA-PhysCellId is used to indicate the physical layer identity of the cell, as defined in TS 36.211 [21].

EUTRA-PhysCellId information element

-- ASN1START

-- TAG-EUTRA-PHYS-CELL-ID-START

EUTRA-PhysCellId ::=						INTEGER (0..503)

-- TAG-EUTRA-PHYS-CELL-ID-STOP

-- ASN1STOP

#### –	EUTRA-PhysCellIdRange

The IE EUTRA-PhysCellIdRange is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of EUTRA-PhysCellIdRange, NW may configure overlapping ranges of physical cell identities.

EUTRA-PhysCellIdRange information element

-- ASN1START

-- TAG-EUTRA-PHYS-CELL-ID-RANGE-START

EUTRA-PhysCellIdRange ::=			SEQUENCE {

start							EUTRA-PhysCellId,

range							ENUMERATED {

n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, spare2,

spare1} 					OPTIONAL	-- Need N

}

-- TAG-EUTRA-PHYS-CELL-ID-RANGE-STOP

-- ASN1STOP

#### –	EUTRA-PresenceAntennaPort1

The IE EUTRA-PresenceAntennaPort1 is used to indicate whether all the neighbouring cells use Antenna Port 1. When set to TRUE, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

EUTRA-PresenceAntennaPort1 information element

-- ASN1START

-- TAG-EUTRA-PRESENCE-ANTENNA-PORT1-START

EUTRA-PresenceAntennaPort1 ::=				BOOLEAN

-- TAG-EUTRA-PRESENCE-ANTENNA-PORT1-STOP

-- ASN1STOP

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type, for the identification of an RRC procedure (transaction).

RRC-TransactionIdentifier information element

-- ASN1START

-- TAG-RRC-TRANSACTIONIDENTIFIER-START

RRC-TransactionIdentifier ::=		INTEGER (0..3)

-- TAG-RRC-TRANSACTIONIDENTIFIER-STOP

-- ASN1STOP

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxBandComb								INTEGER ::=	65536	-- Maximum number of DL band combinations

maxCellBlack							INTEGER ::= ffsValue	 -- Maximum number of NR blacklisted cell ranges in SIB3, SIB4

maxCellInter 							INTEGER ::= ffsValue 	-- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra 							INTEGER ::= ffsValue 	-- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA						INTEGER ::= ffsValue 	-- Maximum number of cells in EUTRAN

maxEARFCN								INTEGER ::= 262143		-- Maximum value of EUTRA carrier frequency

maxEUTRA-CellBlack						INTEGER ::= ffsValue 	-- Maximum number of EUTRA-blacklisted physical cell identity ranges in SIB5

maxEUTRA-FBI							INTEGER ::= 256			-- Highest value extended EUTRA- FBI range

maxEUTRA-NS-Pmax						INTEGER ::= 8			-- Maximum number of NS and P-Max values per band

maxNARFCN								INTEGER ::= 3279165		-- Maximum value of NR carrier frequency

maxNrofServingCells						INTEGER ::=	32		-- Max number of serving cells (SpCell + SCells) per cell group

maxNrofServingCells-1					INTEGER ::= 31		-- Max number of serving cells (SpCell + SCells) per cell group minus 1

maxNrofAggregatedCellsPerCellGroup		INTEGER ::= 16

maxNrofSCells							INTEGER ::=	31		-- Max number of secondary serving cells per cell group

maxNrofCellMeas							INTEGER ::=	32		-- Maximum number of entries in each of the cell lists in a measurement object

maxNrofSS-BlocksToAverage				INTEGER ::= 16		-- Max number for the (max) number of SS blocks to average to determine cell

-- measurement

maxNrofCSI-RS-ResourcesToAverage			INTEGER ::= 16		-- Max number for the (max) number of CSI-RS to average to determine cell

-- measurement

maxNrofDL-Allocations 					INTEGER ::= 16		-- Maximum number of PDSCH time domain resource allocations

maxNrofSR-ConfigPerCellGroup				INTEGER ::= 8		-- Maximum number of SR configurations per cell group

maxLCG-ID								INTEGER ::= 7		-- Maximum value of LCG ID

maxLC-ID								INTEGER ::= 32		-- Maximum value of Logical Channel ID

maxNrofTAGs								INTEGER ::=	4		-- Maximum number of Timing Advance Groups

maxNrofTAGs-1							INTEGER ::=	3		-- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs								INTEGER ::= 4		-- Maximum number of BWPs per serving cell

maxNrofSymbols-1						INTEGER ::= 13		-- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots							INTEGER ::= 320		-- Maximum number of slots in a 10 ms period

maxNrofSlots-1							INTEGER ::= 319		-- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks			INTEGER ::= 275		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1			INTEGER ::= 274		-- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1		INTEGER ::= 276		-- Maximum number of PRBs plus 1

maxNrofControlResourceSets 				INTEGER ::= 12 		-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= 11  	-- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetDuration						INTEGER ::= 3		-- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces						INTEGER ::= 40		-- Max number of Search Spaces

maxNrofSearchSpaces-1					INTEGER ::= 39		-- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize					INTEGER ::= 128		-- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1				INTEGER ::= 127		-- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize					INTEGER ::= 126		-- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1				INTEGER ::= 125		-- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns				INTEGER ::= 4		-- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1				INTEGER ::= 3		-- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup			INTEGER ::= 8		-- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations			INTEGER ::= 48 		-- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1		INTEGER ::= 47		-- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations		INTEGER ::= 112		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= 111		-- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet		INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers			INTEGER ::= 128		-- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger	INTEGER ::= 16		-- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources				INTEGER ::= 192		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= 191		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet		INTEGER ::= 64		-- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets			INTEGER ::= 64		-- Maximum number of NZP CSI-RS resources per cell

maxNrofNZP-CSI-RS-ResourceSets-1		INTEGER ::= 63		-- Maximum number of NZP CSI-RS resources per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig	INTEGER ::= 16		-- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig		INTEGER ::=	128		-- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources				INTEGER ::= 32		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= 31		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1			INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet		INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets			INTEGER ::= 16

maxNrofCSI-IM-Resources					INTEGER ::= 32		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= 31		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214

maxNrofCSI-IM-ResourceSets 				INTEGER ::=	64		-- Maximum number of NZP CSI-IM resources per cell

maxNrofCSI-IM-ResourceSets-1			INTEGER ::=	63		-- Maximum number of NZP CSI-IM resources per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig 	INTEGER ::= 16		-- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet 			INTEGER ::= 64		-- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets 			INTEGER ::=	64		-- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1 			INTEGER ::=	63		-- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig 	INTEGER ::= 1		-- Maximum number of CSI SSB resource sets per resource configuration

maxNrofFailureDetectionResources		INTEGER ::= 10		-- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1		INTEGER ::= 9		-- Maximum number of failure detection resources minus 1

maxNrofObjectId							INTEGER ::= 64		-- Maximum number of measurement objects

maxNrofPageRec							INTEGER ::= 32-- Maximum number of page records

maxNrofPCI-Ranges						INTEGER ::= 8		-- Maximum number of PCI ranges

maxPLMN									INTEGER ::=	12		-- Maximum number of PLMNs broadcast and reported by UE at establisghment

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= 96		-- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1			INTEGER ::= 95		-- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofMeasId							INTEGER ::= 64		-- Maximum number of configured measurements

maxNrofQuantityConfig					INTEGER	::= 2		-- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM 					INTEGER ::= 96 		-- Maximum number of FFS

maxNrofSRS-ResourceSets					INTEGER ::= 16		-- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1				INTEGER ::= 15		-- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-Resources					INTEGER ::= 64		-- Maximum number of SRS resources in an SRS resource set.

maxNrofSRS-Resources-1					INTEGER ::= 63		-- Maximum number of SRS resources in an SRS resource set minus 1.

maxNrofSRS-TriggerStates-1 				INTEGER ::= 3		-- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2 				INTEGER ::= 2		-- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers				INTEGER ::= 8		-- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands					INTEGER ::= 32		-- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerCell	INTEGER ::= 16		-- Maximum number of

maxNrofSlotFormatCombinationsPerSet		INTEGER ::= 512	-- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1	INTEGER ::= 511	-- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofPUCCH-Resources					INTEGER ::= 128

maxNrofPUCCH-Resources-1				INTEGER ::= 127

maxNrofPUCCH-ResourceSets				INTEGER ::= 4		-- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1				INTEGER ::= 3		-- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet			INTEGER ::= 32		-- Maximum number of PUCCH Resources per PUCCH-Resour©ceSet

maxNrofPUCCH-ResourcesPerSet-1			INTEGER ::= 31		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet minus 1.

maxNrofPUCCH-P0-PerSet					INTEGER ::= 8		-- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1		INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofP0-PUSCH-AlphaSets				INTEGER ::= 30		-- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1)

maxNrofP0-PUSCH-AlphaSets-1				INTEGER ::= 29		-- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1)

maxNrofPUSCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1		INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxBands 								INTEGER ::= 1024		-- Maximum number of supported bands in UE capability.

maxBandsMRDC							INTEGER	::= 1280

maxBandsEUTRA							INTEGER ::=	256

maxCellReport		 					INTEGER ::= 8

maxDRB		 							INTEGER ::= 29			-- Maximum number of DRBs (that can be added in DRB-ToAddModLIst).

maxFreq		 							INTEGER ::= 8			-- Max number of non-serving frequencies in MeasResultSCG-Failure.

maxNrofCSI-RS	 						INTEGER ::= 64

maxNrofCandidateBeams 					INTEGER ::= 16			-- Max number of PRACH-ResourceDedicatedBFR that in BFR config.

maxNrofPCIsPerSMTC 						INTEGER ::= 64			-- Maximun number of PCIs per SMTC.

maxNrofQFIs 							INTEGER ::= 64

maxNrOfSemiPersistentPUSCH-Triggers		INTEGER ::= 64			-- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources		 				INTEGER ::= 8			-- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination 		INTEGER ::= 256

maxNrofSpatialRelationInfos 			INTEGER ::= 8

maxNrofSRS-ResourcesPerSet 				INTEGER ::= 16

maxNrofIndexesToReport 					INTEGER ::= 32

maxNrofSSBs 							INTEGER ::= 64 			-- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1							INTEGER ::= 63			-- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI							INTEGER ::= 8			-- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH 					INTEGER ::= 64

maxNrofTCI-States						INTEGER ::= 64			-- Maximum number of TCI states.

maxNrofTCI-States-1						INTEGER ::= 63			-- Maximum number of TCI states minus 1.

maxNrofUL-Allocations 					INTEGER ::= 16			-- Maximum number of PUSCH time domain resource allocations.

maxQFI 									INTEGER ::= 63

maxRA-CSIRS-Resources 					INTEGER ::= 96

maxRA-OccasionsPerCSIRS					INTEGER ::= 64			-- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1						INTEGER ::=	511			-- Maximum number of RA occasions in the system

maxRA-SSB-Resources 					INTEGER ::= 64

maxSCSs									INTEGER ::= 5

maxSecondaryCellGroups 					INTEGER ::= 3

maxNrofServingCellsEUTRA				INTEGER ::= 32

maxMBSFN-Allocations					INTEGER ::= 8

maxNrofMultiBands						INTEGER ::= 8

maxNrofMultiBandsEUTRA				INTEGER ::= ffsValue		-- Maximum number of EUTRA multi frequency bands

maxCellSFTD    	                    	INTEGER ::= 3  			-- Maximum number of cells for SFTD reporting

maxReportConfigId                 	 	INTEGER ::= 64

maxNrofCodebooks						INTEGER ::= 16			-- Maximum number of codebooks suppoted by the UE

maxNrofSRI-PUSCH-Mappings				INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1				INTEGER ::= 15

maxSIB									INTEGER::= 32			-- Maximum number of SIBs

maxSIB-1								INTEGER::= 31

maxSI-Message							INTEGER::= 32			-- Maximum number of SI messages

maxAccessCat-1							INTEGER ::= 63				-- Maximum number of Acccess Categories minus 1

maxBarringInfoSet						INTEGER ::= 8				-- Maximum number of Acccess Categories

maxCellEUTRA							INTEGER ::= ffsValue		-- Maximum number of EUTRA cells in SIB list

maxEUTRA-Carrier						INTEGER ::= ffsValue		-- Maximum number of EUTRA carriers in SIB list

maxPLMNIdentities						INTEGER ::= 8		-- Maximum number of PLMN identites in RAN area configurations

maxDownlinkFeatureSets				INTEGER ::= 1024		-- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets				INTEGER ::= 1024		-- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets				INTEGER ::= 256		-- (for EUTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets				INTEGER ::= 256		-- (for EUTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand				INTEGER ::= 128			-- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets				INTEGER ::= 1024		-- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations 			INTEGER ::= 1024		-- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq					INTEGER ::= 3

-- Editor’s Note: Targeted for completion in Sept 2018. Not used in EN-DC drop.

ShortMAC-I ::=							ENUMERATED {ffsTypeAndValue}

ffsValue								INTEGER ::= 64

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

### –	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

## 6.5	Short message

Short messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17, 7.3.1.2.1]).

Table 6.x-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short messages

| Bit | Short message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 – [8] | Not used in this release of the specification, and shall be ignored by UE if received. |

# 7	Variables and constants

## 7.1	Timers

### 7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Upon transmission of RRCSetupRequest. | Upon reception of RRCSetup or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.3.6. |
| T301 | Upon transmission of RRCReestabilshmentRequest | Upon reception of RRCReestablishment or RRCSetupmessage as well as when the selected cell becomes unsuitable | Go to RRC_IDLE |
| T302 | Upon reception of RRCReject while performing RRC connection establishment or resume. | Upon entering RRC_CONNECTED and upon cell re-selection. | Inform upper layers about barring alleviation as specified in 5.3.x. (FFS) |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, and upon initiating the connection re-establishment procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T311 | Upon initiating the RRC connection re-establishment procedure | Upon selection of a suitable NR cell or a cell using another RAT. | Enter RRC_IDLE |
| T319 | Upon transmission of RRCResumeRequest. | Upon reception of RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.13.5. |
| T320 | Upon reception of t320 or upon cell (re)selection to NR from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, when PLMN selection is performed on request by NAS, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT). | Discard the cell reselection priority information provided by dedicated signalling. |
| T325 | Upon reception of RRCRelease message with deprioritisationTimer. |  | Stop deprioritisation of all frequencies or NR signalled by RRCRelease. |
| T380 | Upon reception of RRCRelease including suspendConfig . | Upon initiation of RRC resume procedure; upon entering RRC_IDLE and upon initiation of the RRC resume procedure. | Perform the actions as specified in 5.3.13. |
| [T30x] | When access attempt is barred at access barring check for an Access Category. |  | Perform the actions as specified in 5.3.14.4. |

### 7.1.2	Timer handling

When the UE applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

## 7.2	Counters

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |

#### –	NR-UE-Variables

This ASN.1 segment is the start of the NR UE variable definitions.

-- ASN1START

NR-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CellIdentity,

MeasId,

MeasIdToAddModList,

MeasObjectToAddModList,

PhysCellId,

RNTI-Value,

ReportConfigToAddModList,

RSRP-Range,

QuantityConfig,

maxNrofCellMeas,

maxNrofMeasId

FROM NR-RRC-Definitions;

-- ASN1STOP

## 7.3	Constants

| Constant | Usage |
| --- | --- |
| N310 | Maximum number of consecutive "out-of-sync" indications for the PCell received from lower layers |
| N311 | Maximum number of consecutive "in-sync" indications for the PCell received from lower layers |

## 7.4	UE variables

NOTE: 	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	NR-UE-Variables

This ASN.1 segment is the start of the NR UE variable definitions.

-- ASN1START

NR-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CellIdentity,

MeasId,

MeasIdToAddModList,

MeasObjectToAddModList,

PhysCellId,

RNTI-Value,

ReportConfigToAddModList,

RSRP-Range,

QuantityConfig,

maxNrofCellMeas,

maxNrofMeasId

FROM NR-RRC-Definitions;

-- ASN1STOP

#### –	VarPendingRnaUpdate

The UE variable VarPendingRnaUpdate indicates whether there is a pending RNA update procedure or not. The setting of this BOOLEAN variable to TRUE means that there is a pending RANU procedure.

VarPendingRnaUpdate UE variable

-- ASN1START

-- TAG-VAR-PENDING-RNAU-START

VarPendingRnaProcedure ::=					SEQUENCE {

pendingRnaProcedure					BOOLEAN								OPTIONAL

}

-- TAG-VAR-PENDING-RNAU-STOP

-- ASN1STOP

#### –	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements.

VarMeasConfig UE variable

-- ASN1START

-- TAG-VAR-MEAS-CONFIG-START

VarMeasConfig ::=					SEQUENCE {

-- Measurement identities

measIdList							MeasIdToAddModList					OPTIONAL,

-- Measurement objects

measObjectList						MeasObjectToAddModList				OPTIONAL,

-- Reporting configurations

reportConfigList					ReportConfigToAddModList			OPTIONAL,

-- Other parameters

quantityConfig						QuantityConfig						OPTIONAL,

s-MeasureConfig							CHOICE {

ssb-RSRP								RSRP-Range,

csi-RSRP								RSRP-Range

} 																		OPTIONAL

}

-- TAG-VAR-MEAS-CONFIG-STOP

-- ASN1STOP

Editor’s Note: FFS Revisit whether we really need VarMeasConfig.

#### –	VarMeasReportList

The UE variable VarMeasReportList includes information about the measurements for which the triggering conditions have been met.

VarMeasReportList UE variable

-- ASN1START

-- TAG-VAR-MEAS-REPORT-START

VarMeasReportList ::=				SEQUENCE (SIZE (1..maxNrofMeasId)) OF VarMeasReport

VarMeasReport ::=					SEQUENCE {

-- List of measurement that have been triggered

measId								MeasId,

cellsTriggeredList					CellsTriggeredList				OPTIONAL,

numberOfReportsSent					INTEGER

}

CellsTriggeredList ::=				SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CHOICE {

physCellId							PhysCellId,

--	Not needed for EN-DC.

physCellIdEUTRA						ENUMERATED {ffsTypeAndValue}

}

-- TAG-VAR-MEAS-REPORT-STOP

-- ASN1STOP

#### –	VarResumeMAC-Input

The UE variable VarResumeMAC-Input specifies the input used to generate the resumeMAC-I during RRC Connection Resume procedure.

VarResumeMAC-Inputvariable

-- ASN1START

-- TAG-VAR-RESUMEMACINPUT-START

VarResumeMAC-Input	::=		SEQUENCE {

sourcePhysCellId						PhysCellId,

targetCellIdentity						CellIdentity,

source-I-RNTI							RNTI-Value,

resumeDiscriminator						BIT STRING(SIZE(1))

}

-- TAG-VAR-RESUMEMACINPUT-STOP

-- ASN1STOP

Editor’s Note: FFS Additional input to VarResumeMAC-Input (replay attacks mitigation).

Editor’s Note: FFS Whether we need the resumeDiscriminator in VarResumeMAC-Input.

VarShortResumeMAC-Input field descriptions

targetCellIdentity Set to CellIdentity of the target cell i.e. the cell the UE is trying to resume.

source-c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to suspension of the RRC connection.

sourcePhysCellId Set to the physical cell identity of the PCell the UE was connected to prior to suspension of the RRC connection.

resumeDiscriminator A constant that allows differentiation in the calculation of the MAC-I for ResumeMAC-I. The resumeDiscriminator is set to ‘1’.

#### –	End of NR-UE-Variables

-- ASN1START

END

-- ASN1STOP

# 8	Protocol data unit abstract syntax

## 8.1	General

The RRC PDU contents in clause 6 and clause 10 are described using abstract syntax notation one (ASN.1) as specified in ITU-T Rec. X.680 [6] and X.681 [7]. Transfer syntax for RRC PDUs is derived from their ASN.1 definitions by use of Packed Encoding Rules, unaligned as specified in ITU-T Rec. X.691 [8].

The following encoding rules apply in addition to what has been specified in X.691:

-	When a bit string value is placed in a bit-field as specified in 15.6 to 15.11 in X.691, the leading bit of the bit string value shall be placed in the leading bit of the bit-field, and the trailing bit of the bit string value shall be placed in the trailing bit of the bit-field;

NOTE:	The terms 'leading bit' and 'trailing bit' are defined in ITU-T Rec. X.680. When using the 'bstring' notation, the leading bit of the bit string value is on the left, and the trailing bit of the bit string value is on the right.

-	When decoding types constrained with the ASN.1 Contents Constraint ("CONTAINING"), automatic decoding of the contained type should not be performed because errors in the decoding of the contained type should not cause the decoding of the entire RRC message PDU to fail. It is recommended that the decoder first decodes the outer PDU type that contains the OCTET STRING or BIT STRING with the Contents Constraint, and then decodes the contained type that is nested within the OCTET STRING or BIT STRING as a separate step;

-	When decoding a) RRC message PDUs, b) BIT STRING constrained with a Contents Constraint, or c) OCTET STRING constrained with a Contents Constraint, PER decoders are required to never report an error if there are extraneous zero or non-zero bits at the end of the encoded RRC message PDU, BIT STRING or OCTET STRING.

## 8.2	Structure of encoded RRC messages

An RRC PDU, which is the bit string that is exchanged between peer entities/across the radio interface contains the basic production as defined in X.691.

RRC PDUs shall be mapped to and from PDCP SDUs (in case of DCCH) or RLC SDUs (in case of PCCH, BCCH or CCCH) upon transmission and reception as follows:

-	when delivering an RRC PDU as an PDCP SDU to the PDCP layer for transmission, the first bit of the RRC PDU shall be represented as the first bit in the PDCP SDU and onwards; and

-	when delivering an RRC PDU as an RLC SDU to the RLC layer for transmission, the first bit of the RRC PDU shall be represented as the first bit in the RLC SDU and onwards; and

-	upon reception of an PDCP SDU from the PDCP layer, the first bit of the PDCP SDU shall represent the first bit of the RRC PDU and onwards; and

-	upon reception of an RLC SDU from the RLC layer, the first bit of the RLC SDU shall represent the first bit of the RRC PDU and onwards.

## 8.3	Basic production

The 'basic production' is obtained by applying UNALIGNED PER to the abstract syntax value (the ASN.1 description) as specified in X.691. It always contains a multiple of 8 bits.

## 8.4	Extension

The following rules apply with respect to the use of protocol extensions:

-	A transmitter compliant with this version of the specification shall, unless explicitly indicated otherwise on a PDU type basis, set the extension part empty. Transmitters compliant with a later version may send non-empty extensions;

-	A transmitter compliant with this version of the specification shall set spare bits to zero.

## 8.5	Padding

If the encoded RRC message does not fill a transport block, the RRC layer shall add padding bits. This applies to PCCH and BCCH.

Padding bits shall be set to 0 and the number of padding bits is a multiple of 8.

Figure 8.5-1: RRC level padding

# 9	Specified and default radio configurations

Specified and default configurations are configurations of which the details are specified in the standard. Specified configurations are fixed while default configurations can be modified using dedicated signalling.

Editor’s Note: FFS / FIXME: Default configurations

## 9.1	Specified configurations

Editor’s Note: FFS

### 9.1.1	Logical channel configurations

### 9.1.2	SRB configurations

#### 9.1.2.1	SRB1/SRB1S

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  |  |  |
| logicalChannelIdentity | 1 |  |  |

#### 9.1.2.2	SRB2/SRB2S

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  |  |  |
| logicalChannelIdentity | 2 |  |  |

#### 9.1.2.3	SRB3

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  |  |  |
| logicalChannelIdentity | 3 |  |  |

## 9.2	Default radio configurations

### 9.2.1	SRB configurations

#### 9.2.1.1	SRB1/SRB1S

Parameters (FFS)

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| PDCP-Config >t-Reordering | infinity |  |  |
| RLC-Config CHOICE | am |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t4 |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms35 ms0 |  |  |
| LogicalChannelConfig |  |  |  |
| >priority | 1 | Highest priority |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | N/A |  |  |
| >allowedSubCarrierSpacing | FFS |  |  |
| >allowedTiming | FFS |  |  |
| >logicalChannelGroup | 0 |  |  |
| >logicalChannelSR-DelayTimerApplied | false |  |  |

#### 9.2.1.2	SRB2/SRB2S

Parameters (FFS)

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| PDCP-Config >t-Reordering | infinity |  |  |
| RLC-Config CHOICE | am |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t4 |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms35 ms0 |  |  |
| LogicalChannelConfig |  |  |  |
| >priority | 3 |  |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | N/A |  |  |
| >allowedSubCarrierSpacing | FFS |  |  |
| >allowedTiming | FFS |  |  |
| >logicalChannelGroup | 0 |  |  |
| >logicalChannelSR-DelayTimerApplied | false |  |  |

#### 9.2.1.3	SRB3

Parameters (FFS)

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| PDCP-Config >t-Reordering | infinity |  |  |
| RLC-Config CHOICE | am |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t4 |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms35 ms0 |  |  |
| LogicalChannelConfig |  |  |  |
| >priority | 1 | Highest priority |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | N/A |  |  |
| >allowedSubCarrierSpacing | FFS |  |  |
| >allowedTiming | FFS |  |  |
| >logicalChannelGroup | 0 |  |  |
| >logicalChannelSR-DelayTimerApplied | false |  |  |

Editor’s Note: FFS General structure for L1/L2 default configurations e.g. MAC configuration, scheduling configuration, physical channel configuration, etc.

# 10	Generic error handling

## 10.1	General

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE;

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved field.

## 10.2	ASN.1 violation or encoding error

The UE shall:

1>	when receiving an RRC message on the [BCCH] for which the abstract syntax is invalid [6]:

2>	ignore the message.

NOTE:	This section applies in case one or more fields is set to a value, other than a spare, reserved or extended value, not defined in this version of the transfer syntax. E.g. in the case the UE receives value 12 for a field defined as INTEGER (1..11). In cases like this, it may not be possible to reliably detect which field is in the error hence the error handling is at the message level.

## 10.3	Field set to a not comprehended value

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 10.4.

## 10.4	Mandatory field missing

The UE shall:

1>	if the message includes a field that is mandatory to include in the message (e.g. because conditions for mandatory presence are fulfilled) and that field is absent or treated as absent:

2>	if the RRC message was received on DCCH or CCCH:

3>	ignore the message;

2>	else:

3>	if the field concerns a (sub-field of) an entry of a list (i.e. a SEQUENCE OF):

4>	treat the list as if the entry including the missing or not comprehended field was not present;

3>	else if the field concerns a sub-field of another field, referred to as the 'parent' field i.e. the field that is one nesting level up compared to the erroneous field:

4>	consider the 'parent' field to be set to a not comprehended value;

4>	apply the generic error handling to the subsequent 'parent' field(s), until reaching the top nesting level i.e. the message level;

3>	else (field at message level):

4>	ignore the message.

NOTE 1:	The error handling defined in these sub-clauses implies that the UE ignores a message with the message type or version set to a not comprehended value.

NOTE 2:	The nested error handling for messages received on logical channels other than DCCH and CCCH applies for errors in extensions also, even for errors that can be regarded as invalid network operation e.g. the network not observing conditional presence.

The following ASN.1 further clarifies the levels applicable in case of nested error handling for errors in extension fields.

-- /example/ ASN1START

-- Example with extension addition group

ItemInfoList ::=					SEQUENCE (SIZE (1..max)) OFItemInfo

ItemInfo ::=						SEQUENCE {

itemIdentity						INTEGER (1..max),

field1								Field1,

field2								Field2					OPTIONAL,			-- Need N

...

[[	field3-r9						Field3-r9				OPTIONAL,			-- Cond Cond1

field4-r9						Field4-r9				OPTIONAL			-- Need N

]]

}

-- Example with traditional non-critical extension (empty sequence)

BroadcastInfoBlock1 ::=				SEQUENCE {

itemIdentity						INTEGER (1..max),

field1								Field1,

field2								Field2					OPTIONAL,			-- Need N

nonCriticalExtension				BroadcastInfoBlock1-v940-IEs	OPTIONAL

}

BroadcastInfoBlock1-v940-IEs::=	SEQUENCE {

field3-r9							Field3-r9				OPTIONAL,			-- Cond Cond1

field4-r9							Field4-r9				OPTIONAL,			-- Need N

nonCriticalExtension				SEQUENCE {}				OPTIONAL			-- Need S

}

-- ASN1STOP

The UE shall, apply the following principles regarding the levels applicable in case of nested error handling:

-	an extension additon group is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire itemInfo entry to be ignored (rather than just the extension addition group containing field3 and field4);

-	a traditional nonCriticalExtension is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire BroadcastInfoBlock1 to be ignored (rather than just the non critical extension containing field3 and field4).

## 10.5	Not comprehended field

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that the UE does not comprehend:

2>	treat the rest of the message as if the field was absent.

NOTE:	This section does not apply to the case of an extension to the value range of a field. Such cases are addressed instead by the requirements in section 10.3.

# 11	Radio information related interactions between network nodes

## 11.1	General

This section specifies RRC messages that are transferred between network nodes. These RRC messages may be transferred to or from the UE via another Radio Access Technology. Consequently, these messages have similar characteristics as the RRC messages that are transferred across the NR radio interface, i.e. the same transfer syntax and protocol extension mechanisms apply.

## 11.2	Inter-node RRC messages

### 11.2.1	General

This section specifies RRC messages that are sent either across the X2-, Xn- or the NG-interface, either to or from the gNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The information could originate from or be destined for another RAT.

-- ASN1START

-- TAG_NR-INTER-NODE-DEFINITIONS-START

NR-InterNodeDefinitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

CellIdentity,

CSI-RS-Index,

GapConfig,

maxBandComb,

maxFeatureSetsPerBand,

maxNrofSCells,

maxNrofServingCells-1,

maxNrofIndexesToReport,

MeasQuantityResults,

MeasResultSCG-Failure,

MeasResultCellListSFTD,

MeasResultList2NR,

P-Max,

PhysCellId,

RadioBearerConfig,

RAN-NotificationAreaInfo,

RRCReconfiguration,

ServCellIndex,

SetupRelease,

SSB-Index,

SSB-MTC,

ShortMAC-I,

SubcarrierSpacing,

UE-CapabilityRAT-ContainerList

FROM NR-RRC-Definitions;

-- TAG_NR-INTER-NODE-DEFINITIONS-STOP

-- ASN1STOP

### 11.2.2	Message definitions

#### –	HandoverCommand

Editor’s Note: Targeted for completion in Sept 2018.

This message is used to transfer the handover command as generated by the target gNB.

Direction: target gNB to source gNB/source RAN.

HandoverCommand message

-- ASN1START

-- TAG-HANDOVER-COMMAND-START

HandoverCommand ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverCommand					HandoverCommand-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverCommand-IEs ::=				SEQUENCE {

handoverCommandMessage				OCTET STRING (CONTAINING RRCReconfiguration),

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- TAG-HANDOVER-COMMAND-STOP

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the RRCReconfiguration message used to perform handover within NR or handover to NR, as generated (entirely) by the target gNB.

#### –	HandoverPreparationInformation

Editor’s Note: Targeted for completion in Sept 2018.

This message is used to transfer the NR RRC information used by the target gNB during handover preparation, including UE capability information.

Direction: source gNB/source RAN to target gNB.

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation		HandoverPreparationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-IEs ::= SEQUENCE {

ue-CapabilityRAT-List				UE-CapabilityRAT-ContainerList,

sourceConfig						AS-Config		OPTIONAL,

rrm-Config							RRM-Config				OPTIONAL,

as-Context							AS-Context				OPTIONAL,

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

AS-Config ::=             SEQUENCE {

rrcReconfiguration					OCTET STRING (CONTAINING RRCReconfiguration),

...

}

AS-Context ::=							SEQUENCE {

reestablishmentInfo						ReestablishmentInfo

OPTIONAL,

configRestrictInfo					ConfigRestrictInfoSCG						OPTIONAL,

...,

[[	ran-NotificationAreaInfo			RAN-AreaInfo	OPTIONAL

]]

}

ReestablishmentInfo ::=				SEQUENCE {

sourcePhysCellId						PhysCellId,

targetCellShortMAC-I					ShortMAC-I,

additionalReestabInfoList				ReestabNCellInfoList					OPTIONAL

}

ReestabNCellInfoList ::=		SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo

ReestabNCellInfo::=	SEQUENCE{

cellIdentity							CellIdentity,

key-gNodeB-Star							BIT STRING (SIZE (256)),

shortMAC-I								ShortMAC-I

}

RRM-Config ::=				SEQUENCE {

ue-InactiveTime				ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20c, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}		OPTIONAL,

candidateCellInfoList		MeasResultList2NR		OPTIONAL,

...

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Context Local RAN context required by the target gNB.

sourceConfig The radio resource configuration as used in the source cell.

rrm-Config Local RAN context used mainly for RRM purposes.

ue-CapabilityRAT-List The UE radio access related capabilities concerning RATs supported by the UE. FFS whether certain capabilities are mandatory to provide by source e.g. of target and/or source RAT.

NOTE 2:	The following table indicates per source RAT whether RAT capabilities are included or not.

| Source RAT | NR capabilites | E-UTRA capabilities | MR-DC capabilities |
| --- | --- | --- | --- |
| NR | Included | May be included | May be included |
| E-UTRAN | Included | May be included | May be included |

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB.

Direction: Secondary gNB to master gNB or eNB.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

cg-Config					CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CG-Config-IEs ::=			SEQUENCE {

scg-CellGroupConfig					OCTET STRING (CONTAINING RRCReconfiguration)	OPTIONAL,

scg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfig)		OPTIONAL,

configRestrictModReq				ConfigRestrictModReqSCG							OPTIONAL,

drx-InfoSCG							DRX-Info										OPTIONAL,

candidateCellInfoListSN				OCTET STRING (CONTAINING MeasResultList2NR)	OPTIONAL,

measConfigSN						MeasConfigSN									OPTIONAL,

selectedBandCombinationNR			BandCombinationInfoSN							OPTIONAL,

fr-InfoListSCG						FR-InfoList										OPTIONAL,

nonCriticalExtension				SEQUENCE {}										OPTIONAL

}

MeasConfigSN ::= SEQUENCE {

measuredFrequenciesSN				SEQUENCE (SIZE (1..maxMeasFreqsSN))	OF NR-FreqInfo	OPTIONAL,

...

}

NR-FreqInfo ::= SEQUENCE {

measuredFrequency 					ARFCN-ValueNR									OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=			SEQUENCE {

requestedBC-MRDC					BandCombinationInfoSN							OPTIONAL,

requestedP-MaxFR1				P-Max												OPTIONAL,

...

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

BandCombinationInfoSN ::=	SEQUENCE {

bandCombinationIndex				BandCombinationIndex,

requestedFeatureSets				FeatureSetEntryIndex

}

FR-InfoList ::=	SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::=	SEQUENCE {

servCellIndex		ServCellIndex,

fr-Type				ENUMERATED {fr1, fr2}

}

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

fr-InfoListSCG Contains information of FR information of serving cells.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

requestedP-MaxFR1 IRequested value for the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

requestedBC-MRDC Used to request configuring an NR band combination and corresponding feature sets which are forbidden to use by MN.

scg-CellGroupConfig Contains the RRCReconfiguration message, used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB

scg-RB-Config Contains the IE RadioBearerConfig, used to establish or reconfigure the SCG configuration, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB

selectedBandCombinationNR Indicates the band combination selected by SN for the EN-DC.

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can used to request configuring an NR band combination whose use MN has previously forbidden.

BandCombinationInfoSN field descriptions

bandCombinationIndex The position of a band combination in the supportedBandCombinationList

requestedFeatureSets The position in the FeatureSetCombination which identifies one FeatureSetUplink/Downlink for each band entry in the associated band combination

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB to set the SCG configuration.It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=				SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE{

cg-ConfigInfo				CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=		SEQUENCE {

ue-CapabilityInfo			OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)		OPTIONAL,-- Cond SN-Addition

candidateCellInfoListMN			MeasResultList2NR									OPTIONAL,

candidateCellInfoListSN			OCTET STRING (CONTAINING MeasResultList2NR)			OPTIONAL,

measResultCellListSFTD			MeasResultCellListSFTD									OPTIONAL,

scgFailureInfo					SEQUENCE {

failureType						ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG					OCTET STRING (CONTAINING MeasResultSCG-Failure)

}																						OPTIONAL,

configRestrictInfo			ConfigRestrictInfoSCG										OPTIONAL,

drx-InfoMCG					DRX-Info													OPTIONAL,

measConfigMN				MeasConfigMN												OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)				OPTIONAL,

scg-RB-Config             	OCTET STRING (CONTAINING RadioBearerConfig)        			OPTIONAL,

mcg-RB-Config				OCTET STRING (CONTAINING RadioBearerConfig)					OPTIONAL,

nonCriticalExtension		SEQUENCE {}													OPTIONAL

}

ConfigRestrictInfoSCG ::=		SEQUENCE {

allowedBC-ListMRDC				BandCombinationInfoList								OPTIONAL,

powerCoordination-FR1				SEQUENCE {

p-maxNR							P-Max												OPTIONAL,

p-maxEUTRA						P-Max												OPTIONAL

}																						OPTIONAL,

servCellIndexRangeSCG			SEQUENCE {

lowBound						ServCellIndex,

upBound							ServCellIndex

}																						OPTIONAL,   -- Cond SN-Addition

maxMeasFreqsSCG-NR					INTEGER(1..maxMeasFreqsMN)							OPTIONAL,

maxMeasIdentitiesSCG-NR				INTEGER(1..maxMeasIdentitiesMN)						OPTIONAL,

...

}

BandCombinationInfoList ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationInfo

BandCombinationInfo ::=		SEQUENCE {

bandCombinationIndex		BandCombinationIndex,

allowedFeatureSetsList		SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex

}

FeatureSetEntryIndex ::=	INTEGER (1.. maxFeatureSetsPerBand)

DRX-Info ::=					SEQUENCE {

drx-LongCycleStartOffset		CHOICE {

ms10							INTEGER(0..9),

ms20							INTEGER(0..19),

ms32							INTEGER(0..31),

ms40							INTEGER(0..39),

ms60							INTEGER(0..59),

ms64							INTEGER(0..63),

ms70							INTEGER(0..69),

ms80							INTEGER(0..79),

ms128							INTEGER(0..127),

ms160							INTEGER(0..159),

ms256							INTEGER(0..255),

ms320							INTEGER(0..319),

ms512							INTEGER(0..511),

ms640							INTEGER(0..639),

ms1024							INTEGER(0..1023),

ms1280							INTEGER(0..1279),

ms2048							INTEGER(0..2047),

ms2560							INTEGER(0..2559),

ms5120							INTEGER(0..5119),

ms10240							INTEGER(0..10239)

},

shortDRX							SEQUENCE {

drx-ShortCycle						ENUMERATED	{

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer					INTEGER (1..16)

}																					OPTIONAL

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN				SEQUENCE (SIZE (1..maxMeasFreqsMN))	OF NR-FreqInfo	OPTIONAL,

measGapConfig					SetupRelease { GapConfig }							OPTIONAL,

gapPurpose							ENUMERATED {perUE, perFR1}							OPTIONAL,

...

}

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBandCombinationListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select an NR band combination. Each entry refers to a band combination numbered according to supportedBandCombination in the UE-MRDC-Capability and the Feature Sets allowed for each band entry. All MR-DC band combinations indicated by this field comprise the LTE band combination, which is a superset of the LTE band(s) selected by MN.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB to consider configuring. Including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification.

maxMeasFreqsSCG-NR Indicates the maximum number of NR inter-frequency carriers the SN is allowed to configure with PSCell for measurements.

maxMeasIdentitiesSCG-NR Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure.

measuredFrequenciesMN Used by MN to indicate a list of frequencies measured by the UE.

measGapConfig Indicates the measurement gap configuration configured by MN.

mcg-RB-Config Contains the IE RadioBearerConfig of the MN, used to support delta configuration for bearer type change between MN terminated to SN terminated bearer and SN change. It is also used to indicate the PDCP duplication related information (whether duplication is configured and if so, whether it is initially activated) in SN Addition/Modification procedure.

p-maxEUTRA Indicates the maximum power for EUTRA (see TS 36.104 [XX]) the UE can use in LTE MCG.

p-maxNR Indicates the maximum power for NR (see TS 38.104 [12]) the UE can use in NR SCG.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

scg-RB-Config Contains the IE RadioBearerConfig of the SN, used to support delta configuration e.g. during SN change. This field is absent when master eNB uses full configuration option.

sourceConfigSCG Includes the current dedicated SCG configuration in the same format as the RRCReconfiguration message, i.e. not only CellGroupConfig but also e.g. measConfig. This field is absent when master eNB uses full configuration option.

ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

BandCombinationInfo field descriptions

allowedFeatureSetsList Defines a subset of the entries in a FeatureSetCombination. Each index identifies one FeatureSetUplink/Downlink for each band entry in the associated band combination.

bandCombinationIndex The position of a band combination in the supportedBandCombinationList

| Conditional Presence | Explanation |
| --- | --- |
| SN-Addition | The field is mandatory present upon SN addition. |

#### –	MeasurementTimingConfiguration

Editor’s Note: Targeted for completion in Sept 2018. Usage and Direction need further RAN2 discussions.

The MeasurementTimingConfigurationmessage is used to convey assistance information for measurement timing betwen master eNB and secondary gNB.

Direction: en-gNB to eNB, eNB to en-gNB, gNB DU to gNB CU, and gNB CU to gNB DU.

MeasurementTimingConfiguration message

-- ASN1START

-- TAG-MEASUREMENT-TIMING-CONFIGURATION-START

MeasurementTimingConfiguration ::=				SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE{

measTimingConf					MeasurementTimingConfiguration-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

MeasurementTimingConfiguration-IEs ::=	SEQUENCE {

measTiming								MeasTimingList							OPTIONAL,

nonCriticalExtension					SEQUENCE {}								OPTIONAL

}

MeasTimingList ::= SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF MeasTiming

MeasTiming ::= SEQUENCE {

frequencyAndTiming						SEQUENCE {

carrierFreq								ARFCN-ValueNR,

ssbSubcarrierSpacing						SubcarrierSpacing,

ssb-MeasurementTimingConfiguration		SSB-MTC

}																				OPTIONAL,

...

}

-- TAG-MEASUREMENT-TIMING-CONFIGURATION-STOP

-- ASN1STOP

MeasurementTimingConfiguration field descriptions

measTiming A list of SMTC information and associated NR frequency that SN informs MN via EN-DC X2 Setup and EN-DC Configuration Update procedures, or F1 messages from gNB DU to gNB CU.

## 11.3	Inter-node RRC information element definitions

## 11.4	Inter-node RRC multiplicity and type constraint values

#### –	Multiplicity and type constraints definitions

-- ASN1START

-- TAG_NR-MULTIPLICITY-AND-CONSTRAINTS-START

maxMeasFreqsMN				INTEGER ::= 32	-- Maximum number of MN-configured measurement frequencies

maxMeasFreqsSN				INTEGER ::= 32	-- Maximum number of SN-configured measurement frequencies

maxMeasIdentitiesMN			INTEGER ::= 62	-- Maximum number of measurement identities that a UE can be configured with

maxCellPrep					INTEGER ::= 32	-- Maximum number of cells prepared for handover

-- TAG_NR-MULTIPLICITY-AND-CONSTRAINTS-STOP

-- ASN1STOP

#### –	End of NR-InterNodeDefinitions

-- ASN1START

-- TAG_NR-INTER-NODE-DEFINITIONS-END-START

END

-- TAG_NR-INTER-NODE-DEFINITIONS-END-STOP

-- ASN1STOP

# 12	Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).

Figure 11.2-1: Illustration of RRC procedure delay

Table 11.2-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | X |  |

Annex A (informative):	Guidelines, mainly on use of ASN.1

# A.1	Introduction

The following clauses contain guidelines for the specification of RRC protocol data units (PDUs) with ASN.1.

# A.2	Procedural specification

## A.2.1	General principles

The procedural specification provides an overall high level description regarding the UE behaviour in a particular scenario.

It should be noted that most of the UE behaviour associated with the reception of a particular field is covered by the applicable parts of the PDU specification. The procedural specification may also include specific details of the UE behaviour upon reception of a field, but typically this should be done only for cases that are not easy to capture in the PDU section e.g. general actions, more complicated actions depending on the value of multiple fields.

Likewise, the procedural specification need not specify the UE requirements regarding the setting of fields within the messages that are sent to the network i.e. this may also be covered by the PDU specification.

## A.2.2	More detailed aspects

The following more detailed conventions should be used:

-	Bullets:

-	Capitals should be used in the same manner as in other parts of the procedural text i.e. in most cases no capital applies since the bullets are part of the sentence starting with 'The UE shall:'

-	All bullets, including the last one in a sub-clause, should end with a semi-colon i.e. an ';.

-	Conditions:

-	Whenever multiple conditions apply, a semi-colon should be used at the end of each conditions with the exception of the last one, i.e. as in 'if cond1, or cond2.

# A.3	PDU specification

## A.3.1	General principles

### A.3.1.1	ASN.1 sections

The RRC PDU contents are formally and completely described using abstract syntax notation (ASN.1), see X.680 [13], X.681 (02/2002) [14].

The complete ASN.1 code is divided into a number of ASN.1 sections in the specifications. In order to facilitate the extraction of the complete ASN.1 code from the specification, each ASN.1 section begins with the following:

- 	a first text paragraph consisting entirely of an ASN.1 start tag, which consists of a double hyphen followed by a single space and the text string "ASN1START" (in all upper case letters);

-	a second text paragraph consisting entirely of a block start tag is included, which consists of a double hyphen followed by a single space and the text string "TAG-NAME-START" (in all upper case letters), where the "NAME" refers to the main name of the paragraph (in all upper-case letters).

Similarly, each ASN.1 section ends with the following:

-	a first text paragraph consisting entirely of a blockstop tag, which consists of a double hyphen followed by a single space and the text string "TAG-NAME-STOP" (in all upper-case letters), where the "NAME" refers to the main name of the paragraph (in all upper-case letters);

-	a second text paragraph consisting entirely of an ASN.1 stop tag, which consists of a double hyphen followed by a singlespace and the text "ASN1STOP" (in all upper case letters).

This results in the following tags:

-- ASN1START

-- TAG-NAME-START

-- TAG-NAME-STOP

-- ASN1STOP

The text paragraphs containing either of thestart and stop tags should not contain any ASN.1 code significant for the complete description of the RRC PDU contents. The complete ASN.1 code may be extracted by copying all the text paragraphs between an ASN.1 start tag and the following ASN.1 stop tag in the order they appear, throughout the specification.

NOTE:	A typical procedure for extraction of the complete ASN.1 code consists of a first step where the entire RRC PDU contents description (ultimately the entire specification) is saved into a plain text (ASCII) file format, followed by a second step where the actual extraction takes place, based on the occurrence of the ASN.1 start and stop tags.

### A.3.1.2	ASN.1 identifier naming conventions

The naming of identifiers (i.e., the ASN.1 field and type identifiers) should be based on the following guidelines:

-	Message (PDU) identifiers should be ordinary mixed case without hyphenation. These identifiers, e.g., the RRCConnectionModificationCommand, should be used for reference in the procedure text. Abbreviations should be avoided in these identifiers and abbreviated forms of these identifiers should not be used.

-	Type identifiers other than PDU identifiers should be ordinary mixed case, with hyphenation used to set off acronyms only where an adjacent letter is a capital, e.g., EstablishmentCause, SelectedPLMN (not Selected-PLMN, since the "d" in "Selected" is lowercase), InitialUE-Identity and MeasSFN-SFN-TimeDifference.

-	Field identifiers shall start with a lowercase letter and use mixed case thereafter, e.g., establishmentCause. If a field identifier begins with an acronym (which would normally be in upper case), the entire acronym is lowercase (plmn-Identity, not pLMN-Identity). The acronym is set off with a hyphen (ue-Identity, not ueIdentity), in order to facilitate a consistent search pattern with corresponding type identifiers.

-	Identifiers should convey the meaning of the identifier and should avoid adding unnecessary postfixes (e.g. abstractions like 'Info') for the name.

-	Identifiers that are likely to be keywords of some language, especially widely used languages, such as C++ or Java, should be avoided to the extent possible.

-	Identifiers, other than PDU identifiers, longer than 25 characters should be avoided where possible. It is recommended to use abbreviations, which should be done in a consistent manner i.e. use 'Meas' instead of 'Measurement' for all occurrences. Examples of typical abbreviations are given in tableA.3.1.2.1-1 below.

-	For future extension: When an extension is introduced a suffix is added to the identifier of the concerned ASN.1 field and/or type. A suffix of the form "-rX" is used, with X indicating the release, for ASN.1 fields or types introduced in a later release (i.e. a release later than the original/first release of the protocol) as well as for ASN.1 fields or types for which a revision is introduced in a later release replacing a previous version, e.g., Foo-r9 for the Rel-9 version of the ASN.1 type Foo. A suffix of the form "-rXb" is used for the first revision of a field that it appears in the same release (X) as the original version of the field, "-rXc" for a second intra-release revision and so on. A suffix of the form "-vXYZ" is used for ASN.1 fields or types that only are an extension of a corresponding earlier field or type (see sub-clause A.4), e.g., AnElement-v10b0 for the extension of the ASN.1 type AnElement introduced in version 10.11.0 of the specification. A number 0...9, 10, 11, etc. is used to represent the first part of the version number, indicating the release of the protocol. Lower case letters a, b, c, etc. are used to represent the second (and third) part of the version number if they are greater than 9. In the procedural specification, in field descriptions as well as in headings suffices are not used, unless there is a clear need to distinguish the extension from the original field.

-	More generally, in case there is a need to distinguish different variants of an ASN.1 field or IE, a suffix should be added at the end of the identifiers e.g. MeasObjectUTRA, ConfigCommon. When there is no particular need to distinguish the fields (e.g. because the field is included in different IEs), a common field identifier name may be used. This may be attractive e.g. in case the procedural specification is the same for the different variants.

-	It should be avoided to use field identifiers with the same name within the elements of a CHOICE, including using a CHOICE inside a SEQUENCE (to avoid certain compiler errors).

TableA.3.1.2-1: Examples of typical abbreviations used in ASN.1 identifiers

| Abbreviation | Abbreviated word |
| --- | --- |
| Config | Configuration |
| DL | Downlink |
| Ext | Extension |
| Freq | Frequency |
| Id | Identity |
| Ind | Indication |
| Meas | Measurement |
| MIB | MasterInformationBlock |
| Neigh | Neighbour(ing) |
| Param(s) | Parameter(s) |
| Phys | Physical |
| PCI | Physical Cell Id |
| Proc | Process |
| Reconfig | Reconfiguration |
| Reest | Re-establishment |
| Req | Request |
| Rx | Reception |
| Sched | Scheduling |
| SIB | SystemInformationBlock |
| Sync | Synchronisation |
| Thr | Threshold |
| Tx | Transmission |
| UL | Uplink |

NOTE:	The tableA.3.1.2.1-1 is not exhaustive. Additional abbreviations may be used in ASN.1 identifiers when needed.

### A.3.1.3	Text references using ASN.1 identifiers

A text reference into the RRC PDU contents description from other parts of the specification is made using the ASN.1 field identifier of the referenced type. The ASN.1 field and type identifiers used in text references should be in the italic font style. The "do not check spelling and grammar" attribute in Word should be set. Quotation marks (i.e., "") should not be used around the ASN.1 field or type identifier.

A reference to an RRC PDU should be made using the corresponding ASN.1 field identifier followed by the word "message", e.g., a reference to the RRCRelease message.

A reference to a specific part of an RRC PDU, or to a specific part of any other ASN.1 type, should be made using the corresponding ASN.1 field identifier followed by the word "field", e.g., a reference to the prioritisedBitRate field in the example below.

-- /example/ ASN1START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							Priority,

prioritisedBitRate					PrioritisedBitRate,

bucketSizeDuration					BucketSizeDuration,

logicalChannelGroup					INTEGER (0..3)

}		OPTIONAL

}

-- ASN1STOP

NOTE:	All the ASN.1 start tags in the ASN.1 sections, used as examples in this annex to the specification, are deliberately distorted, in order not to include them when the ASN.1 description of the RRC PDU contents is extracted from the specification.

A reference to a specific type of information element should be made using the corresponding ASN.1 type identifier preceded by the acronym "IE", e.g., a reference to the IE LogicalChannelConfig in the example above.

References to a specific type of information element should only be used when those are generic, i.e., without regard to the particular context wherein the specific type of information element is used. If the reference is related to a particular context, e.g., an RRC PDU type (message) wherein the information element is used, the corresponding field identifier in that context should be used in the text reference.

A reference to a specific value of an ASN.1 field should be made using the corresponding ASN.1 value without using quotation marks around the ASN.1 value, e.g., 'if the status field is set to value true'.

## A.3.2	High-level message structure

Within each logical channel type, the associated RRC PDU (message) types are alternatives within a CHOICE, as shown in the example below.

-- /example/ ASN1START

DL-DCCH-Message ::= SEQUENCE {

message					DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

dlInformationTransfer					DLInformationTransfer,

handoverFromEUTRAPreparationRequest		HandoverFromEUTRAPreparationRequest,

mobilityFromEUTRACommand				MobilityFromEUTRACommand,

rrcConnectionReconfiguration			RRCConnectionReconfiguration,

rrcConnectionRelease					RRCConnectionRelease,

securityModeCommand						SecurityModeCommand,

ueCapabilityEnquiry						UECapabilityEnquiry,

spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

A nested two-level CHOICE structure is used, where the alternative PDU types are alternatives within the inner level c1 CHOICE.

Spare alternatives (i.e., spare1 in this case) may be included within the c1 CHOICE to facilitate future extension. The number of such spare alternatives should not extend the total number of alternatives beyond an integer-power-of-two number of alternatives (i.e., eight in this case).

Further extension of the number of alternative PDU types is facilitated using the messageClassExtension alternative in the outer level CHOICE.

## A.3.3	Message definition

Each PDU (message) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfiguration ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReconfiguration-r8		RRCConnectionReconfiguration-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

-- Enter the IEs here.

...

}

-- ASN1STOP

Hooks for critical and non-critical extension should normally be included in the PDU type specification. How these hooks are used is further described in sub-clause A.4.

Critical extensions are characterised by a redefinition of the PDU contents and need to be governed by a mechanism for protocol version agreement between the encoder and the decoder of the PDU, such that the encoder is prevented from sending a critically extended version of the PDU type, which is not comprehended by the decoder.

Critical extension of a PDU type is facilitated by a two-level CHOICE structure, where the alternative PDU contents are alternatives within the inner level c1 CHOICE. Spare alternatives (i.e., spare3 down to spare1 in this case) may be included within the c1 CHOICE. The number of spare alternatives to be included in the original PDU specification should be decided case by case, based on the expected rate of critical extension in the future releases of the protocol.

Further critical extension, when the spare alternatives from the original specifications are used up, is facilitated using the criticalExtensionsFuture in the outer level CHOICE.

In PDU types where critical extension is not expected in the future releases of the protocol, the inner level c1 CHOICE and the spare alternatives may be excluded, as shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

-- Enter the fields here.

...

}

-- ASN1STOP

Non-critical extensions are characterised by the addition of new information to the original specification of the PDU type. If not comprehended, a non-critical extension may be skipped by the decoder, whilst the decoder is still able to complete the decoding of the comprehended parts of the PDU contents.

Non-critical extensions at locations other than the end of the message or other than at the end of a field contained in a BIT or OCTET STRING are facilitated by use of the ASN.1 extension marker "...". The original specification of a PDU type should normally include the extension marker at the end of the sequence of information elements contained.

Non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING may be facilitated by use of an empty sequence that is marked OPTIONAL e.g. as shown in the following example:

-- /example/ ASN1START

RRCMessage-r8-IEs ::= 					SEQUENCE {

field1									InformationElement1,

field2									InformationElement2,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

The ASN.1 section specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.

%PDU-TypeIdentifier% field descriptions

%field identifier% Field description.

%field identifier% Field description.

The field description table has one column. The header row shall contain the ASN.1 type identifier of the PDU type.

The following rows are used to provide field descriptions. Each row shall include a first paragraph with a field identifier (in bold and italic font style) referring to the part of the PDU to which it applies. The following paragraphs at the same row may include (in regular font style), e.g., semantic description, references to other specifications and/or specification of value units, which are relevant for the particular part of the PDU.

The parts of the PDU contents that do not require a field description shall be omitted from the field description table.

## A.3.4	Information elements

Each IE (information element) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

PRACH-ConfigSIB ::=					SEQUENCE {

rootSequenceIndex					INTEGER (0..1023),

prach-ConfigInfo					PRACH-ConfigInfo

}

PRACH-Config ::=					SEQUENCE {

rootSequenceIndex					INTEGER (0..1023),

prach-ConfigInfo					PRACH-ConfigInfo					OPTIONAL	-- Need N

}

PRACH-ConfigInfo ::=				SEQUENCE {

prach-ConfigIndex					ENUMERATED {ffs},

highSpeedFlag						ENUMERATED {ffs},

zeroCorrelationZoneConfig			ENUMERATED {ffs}

}

-- ASN1STOP

IEs should be introduced whenever there are multiple fields for which the same set of values apply. IEs may also be defined for other reasons e.g. to break down a ASN.1 definition in to smaller pieces.

A group of closely related IE type definitions, like the IEs PRACH-ConfigSIB and PRACH-Config in this example, are preferably placed together in a common ASN.1 section. The IE type identifiers should in this case have a common base, defined as the generic type identifier. It may be complemented by a suffix to distinguish the different variants. The "PRACH-Config" is the generic type identifier in this example, and the "SIB" suffix is added to distinguish the variant. The sub-clause heading and generic references to a group of closely related IEs defined in this way should use the generic type identifier.

The same principle should apply if a new version, or an extension version, of an existing IE is created for critical or non-critical extension of the protocol (see sub-clause A.4). The new version, or the extension version, of the IE is included in the same ASN.1 section defining the original. A suffix is added to the type identifier, using the naming conventions defined in sub-clause A.3.1.2, indicating the release or version of the where the new version, or extension version, was introduced.

Local IE type definitions, like the IE PRACH-ConfigInfo in the example above, may be included in the ASN.1 section and be referenced in the other IE types defined in the same ASN.1 section. The use of locally defined IE types should be encouraged, as a tool to break up large and complex IE type definitions. It can improve the readability of the code. There may also be a benefit for the software implementation of the protocol end-points, as these IE types are typically provided by the ASN.1 compiler as independent data elements, to be used in the software implementation.

An IE type defined in a local context, like the IE PRACH-ConfigInfo, should not be referenced directly from other ASN.1 sections in the RRC specification. An IE type which is referenced in more than one ASN.1 section should be defined in a separate sub-clause, with a separate heading and a separate ASN.1 section (possibly as one in a set of closely related IE types, like the IEs PRACH-ConfigSIB and PRACH-Config in the example above). Such IE types are also referred to as 'global IEs'.

NOTE:	Referring to an IE type, that is defined as a local IE type in the context of another ASN.1 section, does not generate an ASN.1 compilation error. Nevertheless, using a locally defined IE type in that way makes the IE type definition difficult to find, as it would not be visible at an outline level of the specification. It should be avoided.

The ASN.1 section specifying the contents of one or more IE types, like in the example above, may be followed by a field description table, where a further description of, e.g., the semantic properties of the fields of the information elements may be included. This table may be absent, similar as indicated in sub-clause A.3.3 for the specification of the PDU type. The general format of the field description table is the same as shown in sub-clause A.3.3 for the specification of the PDU type.

## A.3.5	Fields with optional presence

A field with optional presence may be declared with the keyword DEFAULT. It identifies a default value to be assumed, if the sender does not include a value for that field in the encoding:

-- /example/ ASN1START

PreambleInfo ::=					SEQUENCE {

numberOfRA-Preambles				INTEGER (1..64)						DEFAULT 1,

...

}

-- ASN1STOP

Alternatively, a field with optional presence may be declared with the keyword OPTIONAL. It identifies a field for which a value can be omitted. The omission carries semantics, which is different from any normal value of the field:

-- /example/ ASN1START

PRACH-Config ::=				SEQUENCE {

rootSequenceIndex					INTEGER (0..1023),

prach-ConfigInfo					PRACH-ConfigInfo					OPTIONAL	-- Need N

}

-- ASN1STOP

The semantics of an optionally present field, in the case it is omitted, should be indicated at the end of the paragraph including the keyword OPTIONAL, using a short comment text with a need code. The need code includes the keyword "Need", followed by one of the predefined semantics tags (S, M, N or R) defined in sub-clause 6.1. If the semantics tag S is used, the semantics of the absent field are further specified either in the field description table following the ASN.1 section, or in procedure text.

The addition of OPTIONAL keywords for capability groups is based on the following guideline. If there is more than one field in the lower level IE, then OPTIONAL keyword is added at the group level. If there is only one field in the lower level IE, OPTIONAL keyword is not added at the group level.

## A.3.6	Fields with conditional presence

A field with conditional presence is declared with the keyword OPTIONAL. In addition, a short comment text shall be included at the end of the paragraph including the keyword OPTIONAL. The comment text includes the keyword "Cond", followed by a condition tag associated with the field ("UL" in this example):

-- /example/ ASN1START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (0),

...

}		OPTIONAL																	-- Cond UL

}

-- ASN1STOP

When conditionally present fields are included in an ASN.1 section, the field description table after the ASN.1 section shall be followed by a conditional presence table. The conditional presence table specifies the conditions for including the fields with conditional presence in the particular ASN.1 section.

| Conditional presence | Explanation |
| --- | --- |
| UL | Specification of the conditions for including the field associated with the condition tag = "UL". Semantics in case of optional presence under certain conditions may also be specified. |

The conditional presence table has two columns. The first column (heading: "Conditional presence") contains the condition tag (in italic font style), which links the fields with a condition tag in the ASN.1 section to an entry in the table. The second column (heading: "Explanation") contains a text specification of the conditions and requirements for the presence of the field. The second column may also include semantics, in case of an optional presence of the field, under certain conditions i.e. using the same predefined tags as defined for optional fields in A.3.5.

Conditional presence should primarily be used when presence of a field depends on the presence and/or value of other fields within the same message. If the presence of a field depends on whether another feature/function has been configured, while this function can be configured independently e.g. by another message and/or at another point in time, the relation is best reflected by means of a statement in the field description table.

If the ASN.1 section does not include any fields with conditional presence, the conditional presence table shall not be included.

Whenever a field is only applicable in specific cases e.g. TDD, use of conditional presence should be considered.

## A.3.7	Guidelines on use of lists with elements of SEQUENCE type

Where an information element has the form of a list (the SEQUENCE OF construct in ASN.1) with the type of the list elements being a SEQUENCE data type, an information element shall be defined for the list elements even if it would not otherwise be needed.

For example, a list of PLMN identities with reservation flags is defined as in the following example:

-- /example/ ASN1START

PLMN-IdentityInfoList ::=				SEQUENCE (SIZE (1..6)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=				SEQUENCE {

plmn-Identity						PLMN-Identity,

cellReservedForOperatorUse			ENUMERATED {reserved, notReserved}

}

-- ASN1STOP

rather than as in the following (bad) example, which may cause generated code to contain types with unpredictable names:

-- /bad example/ ASN1START

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..6)) OFSEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

-- ASN1STOP

## A.3.8	Guidelines on use of parameterised SetupRelease type

The usage of the parameterised SetupRelease type is like a function call in programming languages where the element type parameter is passed as a parameter. The parameterised type only implies a textual change in abstract syntax where all references to the parameterised type are replaced by the compiler with the release/setup choice. Two examples of the usage are shown below:

-- /example/ ASN1START

RRCMessage-r15-IEs ::= SEQUENCE {

field-r15				SetupRelease { IE-r15 }					OPTIONAL,	--	Need M

...

}

RRCMessage-r15-IEs ::= SEQUENCE {

field-r15		SetupRelease { Element-r15 }

}																	OPTIONAL,	-- Need M

Element-r15 ::= SEQUENCE {

field1-r15					IE1-r15,

field2-r15					IE2-r15								OPTIONAL	-- Need N

}																	OPTIONAL,	-- Need M

-- /example/ ASN1STOP

The SetupRelease is always be used with only named IEs, i.e. the example below is not allowed:

-- /example/ ASN1START

RRCMessage-r15-IEs ::= SEQUENCE {

field-r15		SetupRelease { SEQUENCE { 	-- Unnamed SEQUENCEs are not allowed!

field1-r15					IE1-r15,

field2-r15					IE2-r15							OPTIONAL	-- Need N

}

}																	OPTIONAL,	-- Need M

}

-- /example/ ASN1STOP

If a field defined using the parameterized SetupRelease type requires procedural text, the field is referred to using the values defined for the type itself, namely, "setup" and "release". For example, procedural text for field-r15 above could be as follows:

1> if field-r15 is set to "setup":

2> do something;

1> else (field-r15 is set to "release"):

2> release field-r15 (if appropriate).

## A.3.9	Guidelines on use of ToAddModList and ToReleaseList

In order to benefit from delta signalling when modifying lists with many and/or large elements, so-called add/mod- and release- lists should be used. Instead of a single list containing all elements of the list, the ASN.1 provides two lists. One list is used to convey the actual elements that are to be added to the list or modified in the list. The second list conveys only the identities (IDs) of the list elements that are to be released from the list. In other words, the ASN.1 defines only means to signal modifications to a list maintained in the receiver (typically the UE). An example is provided below:

-- /example/ ASN1START

AnExampleIE ::= 		SEQUENCE {

elementsToAddModList	SEQUENCE (SIZE (1..maxNrofElements)) OF Element										OPTIONAL,	--	Need N

elementsToReleaseList	SEQUENCE (SIZE (1..maxNrofElements)) OF ElementId									OPTIONAL,	--	Need N

...

}

Element ::=			SEQUENCE {

elementId				ElementId,

aField					INTEG ER (0..16777215),

anotherField			OCTET STRING,

...

}

ElementId ::=			INTEGER (0..maxNrofElements-1)

maxNrofElements 		INTEGER ::= 50

maxNrofElements-1 		INTEGER ::= 49

-- /example/ ASN1STOP

As can be seen, the elements of the list must contain an identity (INTEGER) that identifies the elements unambiguously upon addition, modification and removal. It is recommended to define an IE for that identifier (here ElementId) so that it can be used both for a field inside the element as well as in the elementsToReleaseList.

Both lists should be made OPTIONAL and flagged as ”Need N”. The need code reflects that the UE does not maintain the received lists as such but rather updates its configuration using the information therein. In other words, it is not possible to provide via delta signalling an update to a previously signalled elementsToAddModList or elementsToReleaseList (which Need M would imply). The update is always in relation to the UE's internal configuration.

If no procedural text is provided for a set of ToAddModList and ToReleaseList, the following generic procedure applies:

The UE shall:

1>	for each ElementId in the elementsToReleaseList,:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	release the Element from the current UE configuration;

1>	for each Element in the elementsToAddModList:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	modify the configured Element in accordance with the received Element;

2>	else:

3>	add received Element to the UE configuration.

# A.4	Extension of the PDU specifications

## A.4.1	General principles to ensure compatibility

It is essential that extension of the protocol does not affect interoperability i.e. it is essential that implementations based on different versions of the RRC protocol are able to interoperate. In particular, this requirement applies for the following kind of protocol extensions:

-	Introduction of new PDU types (i.e. these should not cause unexpected behaviour or damage).

-	Introduction of additional fields in an extensible PDUs (i.e. it should be possible to ignore uncomprehended extensions without affecting the handling of the other parts of the message).

-	Introduction of additional values of an extensible field of PDUs. If used, the behaviour upon reception of an uncomprehended value should be defined.

It should be noted that the PDU extension mechanism may depend on the logical channel used to transfer the message e.g. for some PDUs an implementation may be aware of the protocol version of the peer in which case selective ignoring of extensions may not be required.

The non-critical extension mechanism is the primary mechanism for introducing protocol extensions i.e. the critical extension mechanism is used merely when there is a need to introduce a 'clean' message version. Such a need appears when the last message version includes a large number of non-critical extensions, which results in issues like readability, overhead associated with the extension markers. The critical extension mechanism may also be considered when it is complicated to accommodate the extensions by means of non-critical extension mechanisms.

## A.4.2	Critical extension of messages and fields

The mechanisms to critically extend a message are defined in A.3.3. There are both "outer branch" and "inner branch" mechanisms available. The "outer branch" consists of a CHOICE having the name criticalExtensions, with two values, c1 and criticalExtensionsFuture. The criticalExtensionsFuture branch consists of an empty SEQUENCE, while the c1 branch contains the "inner branch" mechanism.

The "inner branch" structure is a CHOICE with values of the form "MessageName-rX-IEs" (e.g., "RRCConnectionReconfiguration-r8-IEs") or "spareX", with the spare values having type NULL. The "-rX-IEs" structures contain the complete structure of the message IEs for the appropriate release; i.e., the critical extension branch for the Rel-10 version of a message includes all Rel-8 and Rel-9 fields (that are not obviated in the later version), rather than containing only the additional Rel-10 fields.

The following guidelines may be used when deciding which mechanism to introduce for a particular message, i.e. only an 'outer branch', or an 'outer branch' in combination with an 'inner branch' including a certain number of spares:

-	For certain messages, e.g. initial uplink messages, messages transmitted on a broadcast channel, critical extension may not be applicable.

-	An outer branch may be sufficient for messages not including any fields.

-	The number of spares within inner branch should reflect the likelihood that the message will be critically extended in future releases (since each release with a critical extension for the message consumes one of the spare values). The estimation of the critical extension likelyhood may be based on the number, size and changeability of the fields included in the message.

-	In messages where an inner branch extension mechanism is available, all spare values of the inner branch should be used before any critical extensions are added using the outer branch.

The following example illustrates the use of the critical extension mechanism by showing the ASN.1 of the original and of a later release

-- /example/ ASN1START					-- Original release

RRCMessage ::=							SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcMessage-r8						RRCMessage-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

-- ASN1STOP

-- /example/ ASN1START					-- Later release

RRCMessage ::=							SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcMessage-r8						RRCMessage-r8-IEs,

rrcMessage-r10						RRCMessage-r10-IEs,

rrcMessage-r11						RRCMessage-r11-IEs,

rrcMessage-r14						RRCMessage-r14-IEs

},

later							CHOICE {

c2									CHOICE{

rrcMessage-r16						RRCMessage-r16-IEs,

spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

}

-- ASN1STOP

It is important to note that critical extensions may also be used at the level of individual fields i.e. a field may be replaced by a critically extended version. When sending the extended version, the original version may also be included (e.g. original field is mandatory, EUTRAN is unaware if UE supports the extended version). In such cases, a UE supporting both versions may be required to ignore the original field. The following example illustrates the use of the critical extension mechanism by showing the ASN.1 of the original and of a later release.

-- /example/ ASN1START					-- Original release

RRCMessage ::=							SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcMessage-r8						RRCMessage-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCMessage-rN-IEs ::= SEQUENCE {

field1-rN							ENUMERATED {

value1, value2, value3, value4}	OPTIONAL,	-- Need N

field2-rN							InformationElement2-rN				OPTIONAL,	-- Need N

nonCriticalExtension				RRCConnectionReconfiguration-vMxy-IEs	OPTIONAL

}

RRCConnectionReconfiguration-vMxy-IEs ::= SEQUENCE {

field2-rM							InformationElement2-rM			OPTIONAL, -- Cond NoField2rN

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NoField2rN | The field is optionally present, need N, if field2-rN is absent. Otherwise the field is not present |

Finally, it is noted that a critical extension may be introduced in the same release as the one in which the original field was introduced e.g. to correct an essential ASN.1 error. In such cases a UE capability may be introduced, to assist the network in deciding whether or not to use the critically extension.

## A.4.3	Non-critical extension of messages

### A.4.3.1	General principles

The mechanisms to extend a message in a non-critical manner are defined in A.3.3. W.r.t. the use of extension markers, the following additional guidelines apply:

-	When further non-critical extensions are added to a message that has been critically extended, the inclusion of these non-critical extensions in earlier critical branches of the message should be avoided when possible.

-	The extension marker ("...") is the primary non-critical extension mechanism that is used but empty sequences may be used if length determinant is not required. Examples of cases where a length determinant is not required:

-	at the end of a message;

-	at the end of a structure contained in a BIT STRING or OCTET STRING.

-	When an extension marker is available, non-critical extensions are preferably placed at the location (e.g. the IE) where the concerned parameter belongs from a logical/ functional perspective (referred to as the 'default extension location').

-	It is desirable to aggregate extensions of the same release or version of the specification into a group, which should be placed at the lowest possible level.

-	In specific cases it may be preferrable to place extensions elsewhere (referred to as the 'actual extension location') e.g. when it is possible to aggregate several extensions in a group. In such a case, the group should be placed at the lowest suitable level in the message.<TBD: ref to seperate example>

-	In case placement at the default extension location affects earlier critical branches of the message, locating the extension at a following higher level in the message should be considered.

-	In case an extension is not placed at the defaultextension location, an IE should be defined. The IE's ASN.1 definition should be placed in the same ASN.1 section as the default extension location. In case there are intermediate levels in-between the actual and the defaultextension location, an IE may be defined for each level. Intermediate levels are primarily introduced for readability and overview. Hence intermediate levels need not allways be introduced e.g. they may not be needed when the default and the actual extension location are within the same ASN.1 section. <TBD: ref to seperate example>

### A.4.3.2	Further guidelines

Further to the general principles defined in the previous section, the following additional guidelines apply regarding the use of extension markers:

-	Extension markers within SEQUENCE:

-	Extension markers are primarily, but not exclusively, introduced at the higher nesting levels.

-	Extension markers are introduced for a SEQUENCE comprising several fields as well as for information elements whose extension would result in complex structures without it (e.g. re-introducing another list).

-	Extension markers are introduced to make it possible to maintain important information structures e.g. parameters relevant for one particular RAT.

-	Extension markers are also used for size critical messages (i.e. messages on BCCH, BR-BCCH, PCCH and CCCH), although introduced somewhat more carefully.

-	The extension fields introduced (or frozen) in a specific version of the specification are grouped together using double brackets.

-	Extension markers within ENUMERATED:

-	Spare values may be used until the number of values reaches the next power of 2, while the extension marker caters for extension beyond that limit, given that the use of spare values in a later Release is possible without any error cases.

-	A suffix of the form "vXYZ" is used for the identifier of each new value, e.g. "value-vXYZ".

-	Extension markers within CHOICE:

-	Extension markers are introduced when extension is foreseen and when comprehension is not required by the receiver i.e. behaviour is defined for the case where the receiver cannot comprehend the extended value (e.g. ignoring an optional CHOICE field). It should be noted that defining the behaviour of a receiver upon receiving a not comprehended choice value is not required if the sender is aware whether or not the receiver supports the extended value.

-	A suffix of the form "vXYZ" is used for the identifier of each new choice value, e.g. "choice-vXYZ".

Non-critical extensions at the end of a message/ of a field contained in an OCTET or BIT STRING:

-	When a nonCriticalExtension is actually used, a "Need" code should not be provided for the field, which always is a group including at least one extension and a field facilitating further possible extensions. For simplicity, it is recommended not to provide a "Need" code when the field is not actually used either.

Further, more general, guidelines:

-	In case a need code is not provided for a group, a "Need" code is provided for all individual extension fields within the group i.e. including for fields that are not marked as OPTIONAL. The latter is to clarify the action upon absence of the whole group.

### A.4.3.3	Typical example of evolution of IE with local extensions

The following example illustrates the use of the extension marker for a number of elementary cases (sequence, enumerated, choice). The example also illustrates how the IE may be revised in case the critical extension mechanism is used.

NOTE	In case there is a need to support further extensions of release n while the ASN.1 of release (n+1) has been frozen, without requiring the release n receiver to support decoding of release (n+1) extensions, more advanced mechanisms are needed e.g. including multiple extension markers.

-- /example/ ASN1START

InformationElement1 ::= 			SEQUENCE {

field1								ENUMERATED {

value1, value2, value3, value4-v880,

..., value5-v960 },

field2								CHOICE {

field2a								BOOLEAN,

field2b								InformationElement2b,

...,

field2c-v960						InformationElement2c-r9

},

...,

[[	field3-r9							InformationElement3-r9		OPTIONAL		-- Need R

]],

[[	field3-v9a0							InformationElement3-v9a0	OPTIONAL,		-- Need R

field4-r9							InformationElement4			OPTIONAL		-- Need R

]]

}

InformationElement1-r10 ::=			SEQUENCE {

field1								ENUMERATED {

value1, value2, value3, value4-v880,

value5-v960, value6-v1170, spare2, spare1, ... },

field2								CHOICE {

field2a								BOOLEAN,

field2b								InformationElement2b,

field2c-v960						InformationElement2c-r9,

...,

field2d-v12b0						INTEGER (0..63)

},

field3-r9							InformationElement3-r10			OPTIONAL,	-- Need R

field4-r9							InformationElement4				OPTIONAL,	-- Need R

field5-r10							BOOLEAN,

field6-r10							InformationElement6-r10			OPTIONAL,	-- Need R

...,

[[	field3-v1170					InformationElement3-v1170		OPTIONAL	-- Need R

]]

}

-- ASN1STOP

Some remarks regarding the extensions of InformationElement1 as shown in the above example:

–	The InformationElement1 is initially extended with a number of non-critical extensions. In release 10 however, a critical extension is introduced for the message using this IE. Consequently, a new version of the IE InformationElement1 (i.e. InformationElement1-r10) is defined in which the earlier non-critical extensions are incorporated by means of a revision of the original field.

–	The value4-v880 is replacing a spare value defined in the original protocol version for field1. Likewise value6-v1170 replaces spare3 that was originally defined in the r10 version of field1.

–	Within the critically extended release 10 version of InformationElement1, the names of the original fields/IEs are not changed, unless there is a real need to distinguish them from other fields/IEs. E.g. the field1 and InformationElement4 were defined in the original protocol version (release 8) and hence not tagged. Moreover, the field3-r9 is introduced in release 9 and not re-tagged; although, the InformationElement3 is also critically extended and therefore tagged InformationElement3-r10 in the release 10 version of InformationElement1.

### A.4.3.4	Typical examples of non critical extension at the end of a message

The following example illustrates the use of non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING i.e. when an empty sequence is used.

-- /example/ ASN1START

RRCMessage-r8-IEs ::=			SEQUENCE {

field1							InformationElement1,

field2							InformationElement2,

field3							InformationElement3					OPTIONAL,	-- Need N

nonCriticalExtension			RRCMessage-v860-IEs					OPTIONAL

}

RRCMessage-v860-IEs ::=			SEQUENCE {

field4-v860						InformationElement4					OPTIONAL,	-- Need S

field5-v860						BOOLEAN								OPTIONAL,	-- Cond C54

nonCriticalExtension			RRCMessage-v940-IEs					OPTIONAL

}

RRCMessage-v940-IEs ::=			SEQUENCE {

field6-v940						InformationElement6-r9				OPTIONAL,	-- Need R

nonCriticalExtensions			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

Some remarks regarding the extensions shown in the above example:

–	The InformationElement4 is introduced in the original version of the protocol (release 8) and hence no suffix is used.

### A.4.3.5	Examples of non-critical extensions not placed at the default extension location

The following example illustrates the use of non-critical extensions in case an extension is not placed at the defaultextension location.

#### –	ParentIE-WithEM

The IE ParentIE-WithEMis an example of a high level IE including the extension marker (EM). The root encoding of this IE includes two lower level IEs ChildIE1-WithoutEM and ChildIE2-WithoutEM which not include the extension marker. Consequently, non-critical extensions of the Child-IEs have to be included at the level of the Parent-IE.

The example illustrates how the two extension IEs ChildIE1-WithoutEM-vNx0 and ChildIE2-WithoutEM-vNx0 (both in releaseN) are used to connect non-critical extensions with a default extension location in the lower level IEs to the actual extension location in this IE.

ParentIE-WithEM information element

-- /example/ ASN1START

ParentIE-WithEM ::=					SEQUENCE {

-- Root encoding, including:

childIE1-WithoutEM					ChildIE1-WithoutEM				OPTIONAL,		-- Need N

childIE2-WithoutEM					ChildIE2-WithoutEM				OPTIONAL,		-- Need N

...,

[[	childIE1-WithoutEM-vNx0				ChildIE1-WithoutEM-vNx0		OPTIONAL,		-- Need N

childIE2-WithoutEM-vNx0				ChildIE2-WithoutEM-vNx0		OPTIONAL		-- Need N

]]

}

-- ASN1STOP

Some remarks regarding the extensions shown in the above example:

–	The fields childIEx-WithoutEM-vNx0 may not really need to be optional (depends on what is defined at the next lower level).

–	In general, especially when there are several nesting levels, fields should be marked as optional only when there is a clear reason.

#### –	ChildIE1-WithoutEM

The IE ChildIE1-WithoutEM is an example of a lower level IE, used to control certain radio configurations including a configurable feature which can be setup or released using the local IE ChIE1-ConfigurableFeature. The example illustrates how the new field chIE1-NewField is added in release N to the configuration of the configurable feature. The example is based on the following assumptions:

–	When initially configuring as well as when modifying the new field, the original fields of the configurable feature have to be provided also i.e. as if the extended ones were present within the setup branch of this feature.

–	When the configurable feature is released, the new field should be released also.

–	When omitting the original fields of the configurable feature the UE continues using the existing values (which is used to optimise the signalling for features that typically continue unchanged upon handover).

–	When omitting the new field of the configurable feature the UE releases the existing values and discontinues the associated functionality (which may be used to support release of unsupported functionality upon handover to an eNB supporting an earlier protocol version).

The above assumptions, which affect the use of conditions and need codes, may not always apply. Hence, the example should not be re-used blindly.

ChildIE1-WithoutEM information elements

-- /example/ ASN1START

ChildIE1-WithoutEM ::=				SEQUENCE {

-- Root encoding, including:

chIE1-ConfigurableFeature			ChIE1-ConfigurableFeature		OPTIONAL		-- Need N

}

ChildIE1-WithoutEM-vNx0 ::=		SEQUENCE {

chIE1-ConfigurableFeature-vNx0		ChIE1-ConfigurableFeature-vNx0	OPTIONAL	-- Cond ConfigF

}

ChIE1-ConfigurableFeature ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

-- Root encoding

}

}

ChIE1-ConfigurableFeature-vNx0 ::=	SEQUENCE {

chIE1-NewField-rN					INTEGER (0..31)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ConfigF | The field is optional present, need R, in case of chIE1-ConfigurableFeature is included and set to "setup"; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	ChildIE2-WithoutEM

The IE ChildIE2-WithoutEM is an example of a lower level IE, typically used to control certain radio configurations. The example illustrates how the new field chIE1-NewField is added in release N to the configuration of the configurable feature.

ChildIE2-WithoutEM information element

-- /example/ ASN1START

ChildIE2-WithoutEM ::=				CHOICE {

release								NULL,

setup								SEQUENCE {

-- Root encoding

}

}

ChildIE2-WithoutEM-vNx0 ::=			SEQUENCE {

chIE2-NewField-rN					INTEGER (0..31)					OPTIONAL	-- Cond ConfigF

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ConfigF | The field is optional present, need R, in case of chIE2-ConfigurableFeature is included and set to "setup"; otherwise the field is not present and the UE shall delete any existing value for this field. |

# A.5	Guidelines regarding inclusion of transaction identifiers in RRC messages

The following rules provide guidance on which messages should include a Transaction identifier

1:	DL messages on CCCH that move UE to RRC-Idle should not include the RRC transaction identifier.

2:	All network initiated DL messages by default should include the RRC transaction identifier.

3:	All UL messages that are direct response to a DL message with an RRC Transaction identifier should include the RRC Transaction identifier.

4:	All UL messages that require a direct DL response message should include an RRC transaction identifier.

5:	All UL messages that are not in response to a DL message nor require a corresponding response from the network should not include the RRC Transaction identifier.

# A.6	Guidelines regarding use of need codes

The following rule provides guidance for determining need codes for optional downlink fields:

- if the field needs to be stored by the UE (i.e. maintained) when absent:

- use Need M (=Maintain);

- else, if the field needs to be released by the UE when absent:

- use Need R (=Release);

- else, if UE shall take no action when the field is absent (i.e. UE does not even need to maintain any existing value of the field):

- use Need N (=None);

- else (UE behaviour upon absence doesn’t fit any of the above conditions):

- use Need S (=Specified);

- specify the UE behaviour upon absence of the field in the procedural text or in the field description table.

# A.7	Guidelines regarding use of conditions

Conditions are primarily used to specify network restrictions, for which the following types can be distinguished:

-	CondM: Message Contents related constraints e.g. that a field B is mandatory present if the same message includes field A and when it is set value X.

-	CondC: Configuration Constraints e.g. that a field D can only be signalled if field C is configured and set to value Y. (i.e. regardless of whether field C is present in the same message or previously configured).

The use of these conditions is illustrated by an example.

-- /example/ ASN1START

RRCMessage-IEs ::= SEQUENCE {

fieldA							FieldA					OPTIONAL,	-- Need M

fieldB							FieldB					OPTIONAL,	-- CondM-FieldAsetToX

fieldC							FieldC					OPTIONAL,	-- Need M

fieldD							FieldD					OPTIONAL,	-- CondC-FieldCsetToY

nonCriticalExtension			SEQUENCE {}				OPTIONAL

}

-- /example/ ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Message (content) constraints |  |
| CondM-FieldAsetToX | The field is mandatory present if fieldA is included and set to valueX. Otherwise the field is optional present, need R. |
| Configuration constraints |  |
| CondC- FieldCsetToY | The field is optional present, need M, if fieldC is configured and set to valueY. Otherwise the field is not present and the UE does not maintain the value |

# A.8	Protection of RRC messages (informative)

The following list provides information which messages can be sent (unprotected) prior to security activation and which messages can be sent unprotected after security activation. Those messages indicated "-" in "P" column should never be sent unprotected by gNB or UE. Further requirements are defined in the procedural text.

P…Messages that can be sent (unprotected) prior to security activation

A - I…Messages that can be sent without integrity protection after security activation

A - C…Messages that can be sent unciphered after security activation

NA… Message can never be sent after security activation

| Message | P | A-I | A-C | Comment |
| --- | --- | --- | --- | --- |
| DLInformationTransfer | + | - | - |  |
| LocationMeasurementIndication | - | - | - |  |
| MIB | + | + | + |  |
| MeasurementReport | - | - | - | Measurement configuration may be sent prior to security activation. But: In order to protect privacy of UEs, MEASUREMENT REPORT is only sent from the UE after successful security activation. |
| Paging | + | + | + |  |
| RRCReconfiguration | + | - | - | The message shall not be sent unprotected before security activation if it is used to perform handover or to establish SRB2 and DRBs |
| RRCReconfigurationComplete | + | - | - | Unprotected, if sent as response to RRCConnectionReconfiguration which was sent before security activation |
| RRCReestablishment | - | - | + | Integrity protection applied, but no ciphering. |
| RRCReestablishmentComplete | - | - | - |  |
| RRCReestablishmentRequest | - | - | + | This message is not protected by PDCP operation. However, a short MAC-I is included. |
| RRCReject | + | NA | NA |  |
| RRCRelease | + | - | - | Justification for P: If the RRC connection only for signalling not requiring DRBs or ciphered messages, or the signalling connection has to be released prematurely, this message is sent as unprotected. |
| RRCRequest | + | NA | NA |  |
| RRCResume | - | - | - |  |
| RRCResumeRequest | - | - | + | This message is not protected by PDCP operation. However, a short MAC-I is included. |
| RRCResumeComplete | - | - | - |  |
| RRCSetup | + | NA | NA |  |
| RRCSetupComplete | + | NA | NA |  |
| RRCSystemInfoRequest | + | NA | NA |  |
| SecurityModeCommand | + | NA | NA | Integrity protection applied, but no ciphering (integrity verification done after the message received by RRC) |
| SecurityModeComplete | - | NA | NA | Integrity protection applied, but no ciphering. Ciphering is applied after completing the procedure. |
| SecurityModeFailure | + | NA | NA | Neither integrity protection nor ciphering applied. |
| SystemInformation | + | + | + |  |
| SIB1 | + | + | + |  |
| UECapabilityEnquiry | + | - | - |  |
| UECapabilityInformation | + | - | - |  |
| ULInformationTransfer | + | - | - |  |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04/2017 | RAN2#97bis | R2-1703395 |  |  |  |  | 0.0.1 |
| 04/2017 | RAN2#97bis | R2-1703922 |  |  |  |  | 0.0.2 |
| 05/2017 | RAN2#98 | R2-1705815 |  |  |  |  | 0.0.3 |
| 06/2017 | RAN2#NR2 | R2-1707187 |  |  |  |  | 0.0.4 |
| 08/2017 | RAN2#99 | R2-1708468 |  |  |  |  | 0.0.5 |
| 09/2017 | RAN2#99bis | R2-1710557 |  |  |  |  | 0.1.0 |
| 11/2017 | RAN2#100 | R2-1713629 |  |  |  |  | 0.2.0 |
| 11/2017 | RAN2#100 | R2-1714126 |  |  |  |  | 0.3.0 |
| 12/2017 | RAN2#100 | R2-1714259 |  |  |  |  | 0.4.0 |
| 12/2017 | RAN#78 | RP-172570 |  |  |  | Submitted for Approval in RAN#78 | 1.0.0 |
| 12/2017 | RAN#78 |  |  |  |  | Upgraded to Rel-15 (MCC) | 15.0.0 |
| 03/2018 | RAN#79 | RP-180479 | 0008 | 1 | F | Corrections for EN-DC (Note: the clause numbering between 15.0.0 and 15.1.0 has changed in some cases). | 15.1.0 |

Annex XYZ (to be deleted before approval of the spec):
Extended ASN.1 RIL comments