SA WG2 Meeting #127	S2-184077

Apr 16 - 20, 2018, Sanya, P.R.China	(revision of S2-183483)

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0318 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slicing information as the service input parameter of BSF service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-04-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Concerning the case that there might be multiple network slice instances deployed in a single DNN, each contains a PCF and the same private IPv4 address may be allocated to the UE in different network slice instances, it is necessary to additionally consider the network slicing information for binding the AF request to the PCF, and the slicing information may correspond to S-NNSAI based on operator configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add S-NNSAI as the input parameters of the BSF service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insufficient information to bind the AF request to the PCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

- 5.2.13	BSF Services

- 5.2.13.1	General

The following table shows the BSF Services and Service Operations:

Table 5.2.13.1-1: NF services provided by the BSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nbsf_management | Register | Request/Response | PCF |
|  | Deregister | Request/Response | PCF |
|  | Discovery | Request/Response | NEF, AF |

- 5.2.13.2	Nbsf_Management service

- 5.2.13.2.1	General

The Nbsf provides the Nbsf Management Register, Nbsf Management Remove and the Nbsf Management Discovery service operations.

- 5.2.13.2.2	Nbsf_Management_Register service operation

Service Operation name: Nbsf_Management_Register.

Description: Registers the tuple (UE IP address(es), SUPI, GPSI, DNN, S-NNSAI, PCF id).

Inputs, Required: UE IP address(es), PCF id

Inputs, Optional: DNN, SUPI, GPSI, S-NNSAI,

Outputs, Required: Result indication.

Outputs, Optional: None.

- 5.2.13.2.3	Nbsf_Management_Deregister service operation

Service Operation name: Nbsf_Management_Deregister

Description: Removes the tuple (UE IP address(es), SUPI, GPSI, DNN, , S-NNSAI, PCF id).

Inputs, Required: UE IP address(es)

Inputs, Optional: DNN, SUPI, GPSI, S-NNSAI,

Outputs, Required: Result indication.

Outputs, Optional: None.

- 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF selected for the tuple (UE IP address(es), SUPI, GPSI, DNN, S-NNSAI,).

Inputs, Required: UE IP address or tuple (UE IP address, DNN, S-NNSAI,) or tuple (SUPI, DNN, S-NNSAI,) or tuple ( GPSI, DNN, S-NNSAI,) or tuple (UE IP address,, DNN, SUPI/GPSI, S-NNSAI,)

Inputs, Optional: None

Outputs, Required: PCF id

Outputs, Optional: None.

END OF CHANGES