3GPP TSG-SA3 Meeting #115 	S3-24xxxx

Athens, Greece, 26 Feb - 01 March 2024

Source:	                       Nokia, Nokia Shanghai Bell

Title:	Key issue on 256-bit algorithm MAC size negotiation in NAS procedures

Document for:	Approval

Agenda Item:	X

# 1	Decision/action requested

It is proposed to approve the key isuue.

# 2	References

[1]	S3-235091 New SID on study on enabling a cryptographic algorithm transition to 256-bits.

[2]	TS 33.501 Security architecture and procedures for 5G system.

# 3	Rationale

Current 5G 3gpp security specification [2] describes the procedures of how the different variants of 128-bit algorithm for both ciphering and integrity protection is negotiated (with UE security capabilities) and selected algorithms exchanged in a secured way in Security mode command between UE and AMF. With 128-bit integrity algorithm, the UE and AMF computes a 32-bit message authentication code (MAC-I/NAS-MAC).

One of the objectives from Rel-19 study S3-235091[1] is to study the impacts to the current 5GS system when supporting both 128-bit and 256-bit algorithms in parallel and the adoption of 256-bit algorithms in existing deployments where 128 bits is already supported. Below key issue proposes to study this further.

# 4	Detailed proposal

*** 1st CHANGE ***

## X.1	Key issue # X: 256-bit algorithm MAC size negotiation in NAS procedures

### X.1.1	Key issue details

This key issue studies the potential support of the extension of different MAC sizes between UE and AMF. Also, the extension of the NAS security mode command procedure is adapted with newly extended MAC sizes. With extension of the new algorithms, the impact of truncations of MAC sizes to be analyzed.

### X.1.2	Threats

The adversary is replaying and/or altering NAS procedures and is therefore driving a bidding down attack, i.e., the UE/CN is selecting and configuring MAC size with weaker level of security.

The adversary is replaying and/or altering NAS procedures and is therefore driving a denial-of-service attack, i.e., the UE/CN is selecting an invalid MAC size and is driving the peer side into a denial-of-service.

### X.1.3	Potential security requirements

The UE and the AMF shall support hardened NAS procedures for the selection of MAC sizes for ciphering and integrity protection.

*** END OF 1st CHANGE***