3GPP TSG-SA3 Meeting #115 	S3-24xxxx

- Athens, Greece, 26 Feb - 01 March 2024

- Source:	Nokia, Nokia Shanghai Bell

- Title:	pCR on Non3GPPMob_Sec  TR Security assumption

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is requested to approve the chapter 4 content.

# 2	References

[x] 3GPP TR 33.702: "Study on Security for mobility over non-3GPP access to avoid full primary authentication."

# 3	Rationale

This pCR introduces the architecture assumption for FS_Non3GPPMob_Sec study

# 4	Detailed proposal

**** START OF CHANGE ****

# 4	Security Assumptions

Editor’s Note: This clause contains security assumptions to be considered for the study (e.g., per work task).

## 4.X	Task 1: UE connecting to the new TNAP within the same TNGF

- When user disconnect from a TNAP and connects to a new TNAP within the same TNGF, primary authentication can be skipped. However, if new TNAP is not connected to the same TNGF,  then full primary authentication is performed.

- In Rel 18, TR 33.887 [x], KI 4 and their solutions 5, 6, 7, 8, 10, and 13 will be considered for evaluation, along with any new solution if any. There is no need to resubmit the solutions if there is no change in the solution. If there is a change in the solution, new solution needs to be submitted in the TR and old solution is no longer valid for evaluation.

## 4.Y	Task 2: AUN3 device connecting to a new 5G-RG under the same W-AGF

The AUN3 device does not support NAS over non 3GPP Access and connect to 5GC via RG. 5G-RG creates NAS Registration on half of AUN3 device. Authentication is defined in TS 33.501, section 7B-7.

- When AUN3 device disconnect from a 5G-RG and connects to a new 5G-RG  within the same W-AGF, primary authentication can be skipped. However, if new 5G-RG is not connected to the same W-AGF,  then full primary authentication is performed.

## 4.Z	Task 3: N5CW device connecting to a new 5G-RG under the same W-AGF

The N5CW device does not support NAS over non 3GPP Access and connect to 5GC via RG/W-AGF. W-AGF creates NAS Registration on behalf of N5CW device. Authentication of N5CW device via trusted WLAN is defined in TS 33.501, section 7A.2.4.

When N5CW device disconnect from a 5G-RG and connects to a new 5G-RG  within the same W-AGF, primary authentication can be skipped. However, if new 5G-RG is not connected to the same W-AGF,  then full primary authentication is performed.

## 4.I	Task 4: UE connecting to a new WLAN AP connected via the same NSWOF

TBD.

**** END OF CHANGE ****