3GPP TSG-SA3 Meeting #115 	draft_S3-240718-1

Athens, Greece, 26th February - 1st March 2024    merger of S3-240415, S3-240718						, S3-240721

- Source:	China Mobile, China Telecom, ZTE, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- Title:	New KI on security protection for signaling message on GTP interface

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add new KI on security protection for GTP interfaces.

# 2	References

# 3	Rationale

The contribution proposes to add new KI on security protection for GTP interfaces.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the change ****************

## 5.X	Key Issue #X: Security for dedicated UPF interacting with PLMN through N4GTP interfaces

### 5.X.1	Key issue details

In the scenario where the dedicated UPFs are deployed in customer premise, the compromised UPF in NPN might launch signaling attacks towards the SMF in PLMN 5GC network.

If the dedicated UPF is compromised, attackers may utilize compromised dedicated UPF to collect PLMN’s topology, send malformed messages or launch DDoS attacks to PLMN etc. This also applies if the SMF in the operator premises compromised. Other scenarios are possible for attacks through GTP-based interfaces, such as N3, N4, N6, and N9.

For thisthese scenarios, NDS/IP shall be supported to ensure confidentiality, integrity and replay protection as described in clause 9.9 in TS 33.501[1].

However, existing NDS/IP cannot protect PLMN or NPN from attacks from a compromised dedicated UPF or SMFNFs, such as DDoS, malformed signaling messages, topology information exposure etc.

- X.2	Security threats

If a dedicated UPF or SMF in the operator premises isNFs are compromised by an attacker, the following problems may occur:

-	The attacker may collect topology information of the PLMN and use the information to direct further attacks at the PLMN.

-	The attacker may send malformed signaling messages to NFs in operator premises or customer premises to degrade NFs’ ability to process normal signaling messages.

-	The attacker may send messages to the NFs in the operator premises or customer premises with wrong NF types according to 3GPP specifications. For example, a comprised dedicated UPF may send messages to the SMF in the operator premises to discover vulnerabilities of the SMF and vice versa.

-	The attacker may launch DDoS attacks to flood and disrupt the PLMN or NPN.

### 5.X.3	Potential security requirements

5GS shall support mutual topology information hiding of the PLMN and the customer premises network.

5GS shall support the means to block malformed GTP signaling messages sent from dedicated UPF in the customer premises and compromised SMF in the operator premises.

5GS shall support the means to block GTP messages with wrong NF types sent from dedicated UPF in the customer premises or SMF in the operator premises according to 3GPP specifications.

5GS shall support the means to mitigate DDoS attacks from dedicated UPF in the customer premises to the PLMN and from the SMF to the customer premises network.

5GS shall support the means to authenticate and authorize the dedicated NFs in the customer premises and operator premises.

5GS shall support the detection and the security protection of the signaling messages being transferred through GTP-based interfaces.

*************** End of the change ****************