3GPP TSG-SA3 Meeting #115 	S3-23xxxx

- Athens, February 26 - March 01, 2024               			   		(revision of S3-yyxxxx)

- Source:	China Telecom, ZTE, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- Title:	New KI on dedicated UPF interacting with PLMN through N4 interface

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add new KI on dedicated UPF interacting with PLMN through N4 interface.

# 2	References

[1]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

# 3	Rationale

The contribution proposes to add new KI on dedicated UPF interacting with PLMN through N4 interface.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the change ****************

## 5.X	Key Issue #X: Dedicated UPF interacting with PLMN through N4 interface

### 5.X.1	Key issue details

When only dedicated UPF is deployed in the customer premises, the interface between the dedicated UPF in the customer premises and NFs in the operator premises is N4 interface.

If the dedicated UPF is compromised, attackers may utilize compromised dedicated UPF to collect PLMN’s topology, send malformed messages or launch DDoS attacks to PLMN etc. This also applies if the SMF in the operator premises compromised.

For this scenario, NDS/IP shall be supported to ensure confidentiality, integrity and replay protection as described in clause 9.9 in TS 33.501[1].

However, existing NDS/IP cannot protect PLMN or NPN from attacks from a compromised dedicated UPF or SMF, such as DDoS, malformed signaling messages, topology information exposure etc.

### 5.X.2	Security threats

If a dedicated UPF or SMF in the operator premises is compromised by an attacker, the following problems may occur:

-	The attacker may collect topology information of the PLMN and use the information to direct further attacks at the PLMN.

-	The attacker may send malformed signaling messages to NFs in operator premises or customer premises to degrade NFs’ ability to process normal signaling messages.

-	The attacker may send messages to the NFs in the operator premises or customer premises with wrong NF types according to 3GPP specifications. For example, a comprised dedicated UPF may send messages to the SMF in the operator premises to discover vulnerabilities of the SMF and vice versa.

-	The attacker may launch DDoS attacks to flood and disrupt the PLMN or NPN.

5.X.3	Potential security requirements

5GS should support mutual topology information hiding of the PLMN and the customer premises network.

5GS should support the means to block malformed signaling messages sent from dedicated UPF in the customer premises and compromised SMF in the operator premises.

5GS should support the means to block messages with wrong NF types sent from dedicated UPF in the customer premises or SMF in the operator premises according to 3GPP specifications.

5GS should support the means to mitigate DDoS attack from dedicated UPF in the customer premises to the PLMN and from the SMF to the customer premises network.

5GS should support the means to authenticate and authorize the dedicated NFs in the customer premises and operator premises.

*************** End of the change ****************