3GPPSA3-e(AH) for Rel-19 SID/WID workshop 	S3-******

- Online 26-27 September 2023                     	      			                  		(revision of S3-yyxxxx)

- Source:	China Telecom

- Title:	New SID on security for PLMN hosting a NPN

- Document for:	Approval

Agenda Item:	3

- 3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

- Title:	Study on security for PLMN hosting a NPN

- Acronym:	PLMNNPN_SEC

- Unique identifier:

{A number to be provided by MCC at the plenary}

- Potential target Release:	Rel-19

- 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | x | x | X |  |  |
| Don't know |  |  |  |  |  |

- 2	Classification of the Work Item and linked work items

- 2.1	Primary classification

- This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

- 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

- 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1000027 | NPN security considerations (SecNPN) | WID:SecNPN proposes requirements to address aspects related to the security of the PLMN when it hosts an NPN |

- 3	Justification

TS 22.261 documented the scenario where dedicated network entities of NPN can be deployed in customer premises that are outside the control of the PLMN operator and proposed the requirement that the 5G system shall enable a PLMN to host an NPN without compromising the security of that PLMN.

There are two scenarios: 1) The interface between customer premises and operator premises is non-SBA interface, i.e. N4; 2) The interface between customer premises and operator premises is SBA interface.

Dedicated NFs deployed in customer premises could be compromised due to several reasons, e.g., weaker physical perimeters, misoperations, etc. Currently NDS/IP and SBA security are able to provide authentication, confidentiality, integrity and anti-replay protection. However, existing NDS/IP or SBA security cannot protect PLMN from attacks that attackers exploit compromised dedicated NFs to launch, such as DDoS, malformed signaling messages, topology information exposure etc.

It’s therefore proposed to study how to guarantee the security level of PLMN considering the potential attacks from the compromised dedicated NFs.

# 4	Objective

Based on the above justification, the following objectives will be studied:

- Identify key issues and potential security requirements for the scenario of PLMN hosting a NPN with dedicated NFs deployed in customer premises.

- Develop solutions to address the identified requirements.

## TU estimates

Total TU estimates for the study phase: 1.5 TUs (3 meeting cycles)

Total TU estimates for the normative phase: 1.5 TUs (3 meeting cycles)

Total TU estimates: 3

- 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 33.XXX | Study on security for PLMN hosting a NPN | TSG#103 | TSG#104 | Jun Shen China Telecom Shenjun6@chinatelecom.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

- 6	Work item Rapporteur(s)

- 7	Work item leadership

SA3

- 8	Aspects that involve other WGs

- 9	Supporting Individual Members

Supporting IM name

China Telecom

ZTE

CATT

China Unicom

APPLE

Nokia

China Mobile

OPPO

CableLabs

Lenovo