3GPP TSG-SA3 Meeting #104-e 	S3-211XYZ

- e-meeting, 16 – 27 August 2021

- Source:	Philips International B.V.

- Title:	Brief assessment related to resistance of Solution 20 to quantum computers

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Acceptance of additions in Solution #20 of TR33.809.

# 2   References

[x] NISTIR 8309: “Status Report on the Second Round of the NIST Post-Quantum Cryptography Standardization Proces”. July 2020.  Available online at https://doi.org/10.6028/NIST.IR.8309

# 3   Rationale

This proposal includes a brief assessment related to resistance of Solution 20 to quantum computers.

# 4	Detailed proposal

***	BEGIN OF 1st CHANGE	***

### 6.20.3	Assessment using clause A.3

#### 6.20.3.1	UE aspects

UE needs to be provisioned of a list of trust anchors (see clause 6.20.2.5.1).

UE should support secure storage of trusted anchors.

UE needs to take into consideration results from signature and freshness verification of SI for cell selection and reselection (see clause 6.20.2.5).

Editor Note: further assessment is required, e.g., SIB acquisition, (gNB) certificate/key acquisition depending on the solution variants.

#### 6.20.3.2	UE actions upon detection of invalid signature

Upon detection of invalid signature, UE should not select the cell as described in clause 6.20.2.5.5.

#### 6.20.3.3	Threats that are mitigated by signed SI messages

Man-on-the-side attacks (e.g., SigOver), man-in-the-middle attacks (e.g., replay and relay), and some denial of services (e.g., from tampering with SI) are mitigated.

#### 6.20.3.4	Threats that are not mitigated by signed SI messages

Some denial of services (such as from bitflipping or radio jamming of all available cells) cannot be mitigated.

#### 6.20.3.5	Provisioning of keys

Trusted anchors need to be provisioned into UE during manufacture, onboarding, or after registration.

#### 6.20.3.6	RAN aspects

gNB needs to request digital signatures and obtain short-lived keys from DSnF.

gNB needs to sign dynamic fields (e.g., SFN) using short-lived keys.

gNB needs to broadcast digital signatures along with SIs

Editor Note: further assessment is required, e.g., certificate/key broadcast depending on the solution variants.

#### 6.20.3.7	VPLMN aspects

If the trust anchor of VPLMN is provisioned into a UE, the UE is protected when accessing the VPLMN.

#### 6.20.3.8	HPLMN aspects

If the trust anchor of HPLMN is provisioned into a UE, the UE is protected when accessing the HPLMN.

#### 6.20.3.9	Network sharing aspects

When a gNB is shared by multiple PLMNs, the operator owning the gNB can sign SIs as long as the trust anchor of the gNB operator is provisioned into a UE.

#### 6.20.3.10	Roaming aspects

See clause 6.20.3.6 VPLMN aspects.

#### 6.20.3.11	Regulatory aspects

Regulatory requirements, if there are any, can be supported.

#### 6.20.3.12	Signature schemes

Potential signature schemes include:

-	ECDSA (recommended with named curves)

Editor's Note: the ECDSA profile for SUCI can be reused.

-	RSA

-	others

NOTE: The hash based approach in 6.20.2.2.4 only requires a hash function. For instance, SHA-256 could be used.

#### 6.20.3.13	Signature length

RSA: 256 bytes

ECDSA: 64 bytes

#### 6.20.3.14	Resistance against Quantum Computing

Quantum computers will severely affect digital signing algorithm based on factoring, distrete logarithms, and elliptic cureve cryptography [x] such as RSA or ECDSA. Therefore, resistance against quantum computing requires using quantum resistant solutions. For instance, NIST is standarizing quantum resistant signature algorithms in its Post-Quantum Cryptography project [x]. Quantum resistant signature algorithms have bigger public keys and signatures than ECDSA or RSA as reflected in the following table that includes the key sizes of the three NIST Round 3 PQC finalists for their smallest security parameters. These bigger key/signature sizes might affect the performance of the solution once the solution (or other digital signature solutions) is required to start using quantum-resistant signature algorithms.

| Algorithm | Public key size | Signature size | Source |
| --- | --- | --- | --- |
| Dilithium | 1312 bytes | 2420 bytes | https://pq-crystals.org/dilithium/index.shtml |
| Falcon | 897 bytes | 666 bytes | https://falcon-sign.info/ |
| Rainbow | 58.8 kB | 66 bytes | https://www.pqcrainbow.org/ |

Hash functions are only mildly affected by quantum computers [x]. Thus, the hash based approach in 6.20.2.2.4 is a potential quantum resistant solution for suitable parameters.

The expected performance and properties of quantum resistance algorithms and solutions should be considered to facilitate interoperability in the future.

***	END OF 1st CHANGE	***