3GPP TSG-SA3 Meeting #104-e 	S3-21xyzb

- e-meeting, 16th – 27th August 2021

- Source:	Samsung

- Title:	Proposed Way Forward

- Document for:	Discussion

Agenda Item:

# 1	Decision/action requested

It is requested to endorse this contribution.

# 2	Introduction

In SA3 #103e, Samsung presented Discussion Paper S3-212099, suggesting a number of criteria for evaluating the different solutions to KI#1 in 3GPP TR 33.864. This discussion paper intends to introduce an additional criteria - “Isolated operation” for evaluating the different solutions for AMF Reallocation, as there are different understanding among the delegates.

In this document, the isolation criteria is categorized into four cases and it is shown which category each solution belongs to. Also, in this document, the context consists of security context and any other UE related data (e.g. SUCI or 5G-GUTI included in the UE request message).

1. Isolation Case 1: No context transfer

No context is transferred from iAMF to tAMF.

2. Isolation Case 2: Redirection of the UE request message (w/o security context transfer)

The whole UE request is redirected from iAMF to tAMF via RAN, but the security context is not transferred.

3. Isolation Case 3: Redirection of the UE request message (with protected security context transfer)

The whole UE request is redirected from iAMF to tAMF via RAN. The security context is also transferred from iAMF to tAMF, in a protected form.

4. Isolation Case 4: Redirection of the UE request message (with unprotected SC transfer)

The whole UE request is redirected from iAMF to tAMF via RAN. The security context is also transferred from iAMF to tAMF, in an unprotected form.

The meaning of protection is that

- in the case of Sol #10, the security context is protected using asymmetric cryptography, and only iAMF and tAMF can read it.

- in the case of Sol #9, the security context is protected using symmetric cryptography, and only iAMF, tAMF, and NSSF can read it.

# 3	Discussions

The table below shows which category each solution belongs to.

| Description/Solution # | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Isolation Case 1 |  |  |  |  |  |  |  |  |  |  |  |
| Isolation Case 2 |  |  |  |  |  |  |  |  |  |  |  |
| Isolation Case 3 |  |  |  |  |  |  |  |  |  |  |  |
| Isolation Case 4 |  |  |  |  |  |  |  |  |  |  |  |

Green: 	The solution belongs to the case.

Red:	 	The solution does not belong to the case

# 4	Evaluation

Based on the discussions in the previous meeting, it was emphasized to define the requirement on the isolation criteria. But, as of now it was not defined and it is one of the major lacking factors to decide on the solution. With the definition detailed in clause 1 of this document, Case 1 provides the strictest isolation and therefore from the security perspective the solutions addressing Case 1 need to be considered and concluded.

Additionally, if operators are fine with the other isolation cases (Case 2 and/or Case 3), then the solutions addressing these two cases are also to be considered.

# 5	Conclusion

It is proposed to endorse a solution which can also fulfil the isolation requirements from operators, apart from other must-have evaluation criteria discussed in the previous meeting.

Additionally, it is proposed to endorse more than one solution, if some operators are willing to accept more than one isolation cases and no single solution meets all the evaluation criteria identified.

# 6	Proposal

It is requested to endorse this contribution and agree with the above-mentioned proposal.