- 3GPP TSG-SA3 Meeting #103-e 	S3-21xxxx

- e-meeting, 16 – 27 August 2021, Online

- Source:	Huawei, HiSilicon

- Title:	EC: Conclusion for Key issue #2

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes the conclusion for KI#2.

# 2	References

[1] TR 33.839 v040

# 3	Rationale

5.2.3	Potential Security Requirements

Edge Configuration Server(ECS) Requirements:

Edge Configuration Server(ECS) shall be able to provide mutual authentication with Edge Enabler Client over EDGE-4 Interface.

Edge Configuration Server(ECS) shall be able to determine whether Edge Enabling the client is authorized to access provisioning services offered by Edge Configuration Server(ECS).

As described in TS 23.558-200, the ECS may utilize the 3GPP core network capabilities. Hence, solution #10 proposes to use UDM for EEC authorization is reasonable.

On the other hand, ss described in TS 23.548-010 clause 6.5.2, the SMF may derive the ECS Address Configuration Information based on local configuration, the UE's location, and/or UE subscription information, which means the UE subscription information stored in the UDM could have the ECS service information. Hence, solution #10 proposed to use UDM for EEC authorization is applicable.

When GPSI is used as a user identifier for authorization, the GPSI also requires authentication. As solution#23 has no impact on AKMA procedure and user consent, it’s proposed to use solution#23 as basis for normative work.

As discussed in S3-21xxxxx, this contribution proposes a conclusion for key issue #2.

On the other hand, the EEC may have to know whether to use the AKMA for the data protection. Solution #11 proposed that the 5GC could send an AKMA indication to notify the EEC on the usage of AKMA. Hence, it is proposed to use the AKMA indication for the AKMA usage negotiation.

# 4	Detailed proposal

It is suggested to approve the following changes.

***************************Start of change ****************************

### 7.2	Conclusions for Key Issue #2

It is concluded are as follows:

Authentication issue between EEC and ECS:

- AKMA based authentication and data protection.

  - Solution #3, and #24 are recommended for the normative work to include that AKMA is used for generating the shared key between EEC and ECS, and TLS is used to authenticate with each other based on the shared key. The version for appling AKMA with TLS is left for the normative work.

- GBA based authentication and data protection

  - Solution #29 is recommended as the baseline for the normative work

- Certificate based mutual authentication and data protection

  - Solution #4, and #17 are recommended for the normative work to include the certificate based TLS mutual authentication between EEC and EES

NOTE 1: Certificate allocation for the EEC is out of scope of the present document.

- Kamf based authentication and data protection

  - Solution #7 are recommended as the baseline for the normative work

- 	EES authentication using TLS based the server’s certificate + EEC authentication with the token generated by the ECSP or a third party

  - Solution #17 are recommended as the baseline for the normative work.

NOTE 2: Regarding that EEC ID generation is out of scope of TS 23.558 [2], EEC ID verification is out of the scope of the present documents as well.

NOTE 3: EEC ID verification needs further evaluation.

Authorization issue between EEC and ECS

- Using the static authorization by the ECS for the ECS service authorization.

NOTE 4: Authorization mechanisms based on the application layer is out scope of this document.

GPSI verification

- Solution #17 are recommended as the baseline for the normative work.

- Solution #24 are recommended as the baseline for the normative work.

***************************End of change****************************