3GPP TSG-SA Meeting #95e	SP-220075

Electronic Meeting, 14 – 23 March 2022

Title:	Summary for Network Slice charging based on 5G Data Connectivity

Type:	Work Item Summary

Agenda Item:	21.5

Source:	MATRIXX Software

Contact:	Gerald.Goermer@matrixx.com

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

Summary based on the input provided by MATRIXX Software in SP-220075.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 9500xx | Network Slice charging based on 5G Data Connectivity | NETSLICE_DC_CH | S5 | SP-22xxxx (S5-221655) | MATRIXX Software, Gerald Görmer |

Introduction

In 5G system when Network Slices allocated to third party providers are deployed by Mobile Network Operators, how these third-party providers can be charged for usage of assigned Network Slice(s), is not described. This work item introduces a description focusing on one particular type of Network Slice usage, leveraging from capabilities specified from Rel-15.

Description

5G Data Connectivity charging specified in 5GS between SMF and CHF for individual UEs PDU sessions from Rel-15, is used with Converged Charging System (CCS) hosting the CHF extended to also cover the third-party provider (addressed under the tenant concept).

By receiving the S-NSSAI for each UE PDU session, the enhanced CCS is able to perform per tenant charging based on tenant's Network Slice total UEs data connectivity usage, the Network Slice being identified by the S-NSSAI.

The internal behaviour of the enhanced CCS is not specified in this release.

References

TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

List of related CRs:

[1]	SP-22xxxx (S5-221674): "Introduction of Annex on Network slice charging"