| 3GPP TR 28.840 V0.5.0 (2023-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on CHF Segmentation; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	8

5	CHF Selection Scenarios and Key Issues	8

5.1	Topic #1: CHF Selection by NF Consumers Information	8

5.1.1	General description and assumptions	8

5.1.2	Potential charging requirements	9

5.1.3	Key issues#1.1: Charging events and charging information required	9

5.1.4	Possible Solutions	9

5.1.4.1	CHF Selection and Discovery for NF Service Consumers Solution based on UDM	9

5.1.4.2	 CHF Selection by using CHF Default Instance Solution	12

5.1.4.3	Solution #1.3 CHF selection based on location	14

5.1.4.4	Solution #1.4: CHF selection based on NF instance	14

5.1.4.5	Solution #1.5: Using NRF locality information	15

5.1.5	Evaluation	15

5.1.6	Conclusion	15

5.2	Topic #2: CHF Selection based on SUPI or Group ID	15

5.2.1	 General description and assumptions	15

5.2.2	Potential charging requirements	16

5.2.3	Key issues#2.1: CHF Instance Selection optimization	16

5.2.4	Possible Solutions	16

5.2.4.1	Location Optimization Solution	16

5.2.4.2	Solution #2.2 CHF selection based on user group	17

5.2.4.3	Solution #2.3: Use full set of NRF discovery	17

5.2.5	Evaluation	18

5.2.6	Conclusion	18

5.3	Key Issue #3 CHF Selection with Extended Subscription Data	18

5.4	Topic #4 CHF Discovery by Charging Domains	18

5.4.1 	General description and assumptions	18

5.4.2	Potential charging requirements	18

5.4.3 	Key Issues	18

5.4.4	Possible Solutions	19

5.4.4.1	Solution #4.1: Use NF type	19

5.4.5	Evaluation	19

5.4.6 	Conclusion	19

6	Evaluation	19

7	Conclusion	19

Annex <A>: Change history	20

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the CHF discovery and selection in order to support the discovery of a CHF by the NF consumers like SMF, AMF, SMSF and PCF based on information about network segments that the CHF belongs to.

The following are studied:

- possible charging scenarios and requirements which makes use of CHF Segmentation.

- potential charging solutions which make use of CHF Segmentation.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.510: "Network Function Repository Services".

[3]	3GPP TS 32.290: " Services, operations and procedures of charging using Service Based Interface (SBI) ".

[4]	3GPP TS 32.255 “5G data connectivity domain charging; Stage 2”

[5]	3GPP TS 23.501 “System architecture for the 5G System (5GS);Stage 2”

[6]	3GPP TS 29.503 “Unified Data Management Services; Stage 3”

[7]	3GPP TS 29.505 “Usage of the Unified Data Repository services for Subscription Data; Stage 3”

[8]	3GPP TS 29.525 “Network Data Analytics Services; Stage 3”

[9]	3GPP TS 32.240 “Charging architecture and principles”

[10]	3GPP TS 29.571 “5G System; Common Data Types for Service Based Interfaces; Stage 3”

[11]	3GPP TS 32.291 “Telecommunication management; Charging management; 5G system, charging service; Stage 3”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AMF	Access and Mobility management Function

CHF	Charging Function

IP	Internet Protocol

NF	Network Function

NRF	Network Repository Function

PCF	Policy Control Function

PDU	Protocol Data Unit

SMF	Session Management Function

SMSF	Short Message Service Function

SUPI	Subscription Permanent Identifier

UDM	Unified Data Maangement

UDR	Unified Data Repository

UE	User Equipment

# 4	Overview

The current NRF Based Discovery can use the SUPI, GPSI PLMN and CHF Group ID for CHF Discovery and Selection according to TS 29.510 [2] clause 6.1.6.2.32, and TS 32.290[3] clause 6.1. The CHF(s) can be selected taking in account the business model in use by a Customer, for instance its possible to group the CHF instance(s) for enterprises customers and another one for non-enterprise customers, and taking in account the deployment model in place (e.g. central or local/edge).

The present document aims to study on how to enhance CHF Discovery and selection by the NF Consumers like SMF, AMF, SMSF, PCF and other NFs, as specified in various 3GPP Charging Specifications.

# 5	CHF Selection Scenarios and Key Issues

## 5.1	Topic #1: CHF Selection by NF Consumers Information

### 5.1.1	General description and assumptions

The CHF Selection, in a non-roaming scenario can be done by the SMF at the PDU Session establishment through the following priority (according to 3GPP TS 32.255 [4] clause 5.1.8)

- CHF address(es) with possible associated CHF instance ID(s) and/or CHF set ID(s) provided by the PCF for the PDU session.

- UDM provided charging characteristics.

- NRF based discovery.

- SMF locally provisioned charging characteristics.

On a roaming scenario the CHF Selection is supported taking in consideration the above point. The following exceptions apply (according to 3GPP TS 32.255 [4] clause 5.1.9.2) – Edge emphasize:

In local breakout scenario, at PDU session establishment, the CHF selection mechanism specified in clause 5.1.8 applies to:

- The V-SMF for CHF selection in VPLMN, is the same as in the home routed scenario.

- The V-SMF for CHF selection in HPLMN, with the following differences.

- CHF address(es) selection mechanisms based on PCF, UDM, and local configuration are not applicable.

- NRF based discovery, the H-CHF can be selected based on the H-PLMN of the UE.

Therefore, the CHF Instance should be also selected upon the NF Consumer derived information.

### 5.1.2	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 32.255 [4].

REQ-CH_ SELCHF_LO -01: The 5GS should be able to provide a CHF Instance based on the NF Consumer Location.

### 5.1.3	Key issues#1.1: Charging events and charging information required

This key issue is for investigating how to support the selection of a CHF Instance based on the Location considering REQ-CH_ SELCHF_LO -01. This investigation covers the following:

-	identification of the CHF Instance based on NF Consumer derived information;

### 5.1.4	Possible Solutions

### 5.1.4.1	CHF Selection and Discovery for NF Service Consumers Solution based on UDM

A possible solution for key issue #1.1 covers the requirement REQ-CH_ SELCHF_LO -01.

SUPI is the parameter for CHF discovery through NRF is not efficient in such network deployments and a Group ID based CHF discovery is needed. Although in 3GPP Rel-17 the CHF can register with the NRF using CHF-Group-ID (along other parameters such as CHF-Hostname, IP Address etc.) and an NF consumer can use CHF-Group-ID during CHF discovery process through the NRF:

- The NF consumers like the SMF, the AMF, the SMSF and the PCF does not have CHF-Group-ID information associated with UE.

- Group ID Mapping stored in UDR is not accessible to all NF consumers (e.g. AMF, SMF etc.). This also requires the UDR to store/maintain the mapping of all the subscribers in the network to the CHF-Group-ID

Therefore, it’s proposed to to have ChfSubsriptionData maintained at UDM which enables the current services of UDM and UDR to fetch the ChfSubscriptionData which would consists of the CHF Group ID.

This is new in 3GPP TS 29.505 [6] and 3GPP TS 29.505 [7].

This ChfSubscriptionData can be for a group or per individual UE, in the same way other SubscriptionData are maintained in the UDM / UDR.

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ChfGroupID | NfGroupId | C | 0..1 | CHF instances that belong to the CHF Group Id that would serve the specific UE or a group of UEs defined by the group |

Table 5.1.4.1-1: Definition of type ChfSubscriptionData

Next, Nudr interface shall be extended, in 3GPP TS 29.505 [7] with CHF, and CHF shall be able to fetch the ChfSubscriptoinData for each of the UE’s.

1.	Any NF Service Consumer that would like to access the Nchf service shall discover the CHF. SUPI is one of the parameter that is include as part of the query parameters amongst others.

2.	As part of the discovery response, NRF shall provide the list of CHFs that match the discovery criteria, and it shall also include the CHF Group ID for each of the CHF.

3.	The NF Service Consumer shall invoke the Nchf Service API towards the selected CHF instance Id.

3a. The NF Service Producer (CHF) shall query UDM / UDR requesting the ChfSubsrciptionData for the corresponding SUPI.

3b. UDM / UDR shall provide the CHF Group ID along as part of the ChfSubscriptionData.

3c. CHF shall verify if it is the defined instance, as part of CCS, to handle the request for that particular SUPI.

3d. If the CHF identifies that it is the defined instance, as part of CCS, than it shall process the request and respond to the NF Service Consumer.

3e. If the CHF identifies that the CHF Group ID of self and the SUPI are different, it shall respond to the the NF Service Consumer with an error code 307 including the CHF Group ID of the SUPI as part of the header.

4.	NF Service Consumer based on the discovery result in step 2 and also the CHF Group ID received in step 3b, shall carry out the reselection of CHF.

5.	NF Service Consumer shall invoke the Nchf Service API towards the CHF selected as part of the reselection logic. NF Service Consumer shall then use the selected CHF for the forthcoming requests.

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| CHF Group | OC | - | CHF instances that belong to the CHF Group Id that would serve the specific UE or a group of UEs defined by the group |

Table 5.1.4.1-2: Extend to Charging Data Request Message (3GPP TS 32.255 [4] – Table 6.1.1.2.1)

Editor’s Note: Trigger and CHF Group Priority Information Elements availability is for FFS

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Multiple Unit Information | OC | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Result Code | OC | OC | Described in TS 32.290 [57] |
| Rating Group | M | M | Described in TS 32.290 [57] |
| UPF ID | OC | OC | This field holds the UPF identifier used for quota granted per UPF by CHF |
| Granted Unit | OC | - | Described in TS 32.290 [57] |
| Validity Time | OC | - | Described in TS 32.290 [57] |
| Final Unit Indication | OC | - | Described in TS 32.290 [57] |
| Time Quota Threshold | OC | - | Described in TS 32.290 [57] |
| Volume Quota Threshold | OC | - | Described in TS 32.290 [57] |
| Unit Quota Threshold | OC | - | Described in TS 32.290 [57] |
| Quota Holding Time | OC | - | Described in TS 32.290 [57] |
| Triggers | OC | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| CHF Subscription Data | OC | - | CHF Instance to be used for a UE / Subscriber |

Table 5.1.4.1-3: Extend to Charging Data Response Message (3GPP TS 32.255 [4] – Table 6.1.1.2.2)

This solution would bring the following advantages for CHF Selection:

- Enable the possibility to for CHF Selection and Discovery by NF Consumers like SMF, AMF, SMSF and PCF

- The solution proposes a more accurate CHF selection taking in account the details provided.

### 5.1.4.2	 CHF Selection by using CHF Default Instance Solution

A possible solution for key issue #1.1 covers the requirement REQ-CH_ SELCHF_LO -01.

The solution proposed is following what is described in detail in NF Service Consumers Selection and Discovery Solution based on UDM (clause 5.1.4.1). The following points differentiate the solution from the one described there. The goal is to avoid using NRF Lookup for Selection and intiate the CHF Selection and Discovery from the NF Service Consumer to the CHF.

Therefore, it’s proposed to to have ChfSubsriptionData maintained at UDM which enables the current services of UDM and UDR to fetch the ChfSubscriptionData which would consists of the CHF Group ID. It’s proposed that a CHF Iinstance is used by default once Nchf service is called, avoiding CHF discovery and selection oprations with NRF.

This is new in 3GPP TS 29.503 [6] (Unified Data Management Services; Stage 3) and 3GPP TS 29.505 [7] (Usage of the Unified Data Repository services for Subscription Data; Stage 3)

This ChfSubscriptionData can be for a group or per individual UE, in the same way other SubscriptionData are maintained in the UDM / UDR.

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ChfGroupID | NfGroupId | C | 0..1 | CHF instances that belong to the CHF Group Id that would serve the specific UE or a group of UEs defined by the group |

Table 5.1.4.2-1: Definition of type ChfSubscriptionData

Next, Nudr interface shall be extended (3GPP TS 29.505 [7]) to the CHF, and CHF shall be able to fetch the ChfSubscriptoinData for each one of the UE’s. Aditionally, Nchf interface shall be extended, in 3GPP TS 32.290 [3], to acoomodate a default CHF instance that is used in the first step once called by a NF Consumer (e.g. SMF).

1.	The NF Service Consumer shall invoke the Nchf Service API towards the selected CHF instance Id.

1a. The NF Service Producer (CHF) shall query UDM / UDR requesting the ChfSubsrciptionData for the corresponding SUPI.

1b. UDM / UDR shall provide the CHF Group ID along as part of the ChfSubscriptionData.

1c. CHF shall verify if it is the defined instance, as part of CCS, to handle the request for that particular SUPI.

1d. If the CHF identifies that it is the defined instance, as part of CCS, to handle the request, than it shall process the request as is currently and respond to the NF Service Consumer.

1e. If the CHF identifies that the CHF Group ID of self and the SUPI are different, it shall respond to the the NF Service Consumer with an error code 307 including the CHF Group ID of the SUPI as part of the header.

2.	NF Service Consumer based on the discovery result in step 2 and also the CHF Group ID received in step 3b, shall carry out the reselection of CHF.

3.	NF Service Consumer shall accordingly invoke the Nchf Service API towards the CHF selected as part of the reselection logic.  NF Service Consumer shall then use the selected CHF for the forthcoming requests.

Table 5.1.4.2-2: Extend to Charging Data Request Message (3GPP TS 32.255 [4] – Table 6.1.1.2.1)

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| CHF Group | OC | - | CHF instances that belong to the CHF Group Id that would serve the specific UE or a group of UEs defined by the group |

Editor’s Note: Trigger and CHF Group Priority Information Elements availability is for FFS

This solution would bring the following advantages for CHF Selection:

- Enable the possibility for CHF Selection and Discovery by NF Consumers like SMF, AMF, SMSF and PCF

- CHF Selection Time is reduced by enabling the selection to be done directly between the NF Service Consumer and the CHF.

### 5.1.4.3	Solution #1.3 CHF selection based on location

The solution addresses key issue #1.1 of topic #1 on CHF selection by NF Consumers Information.

NF(CTF) may discover and select the CHF instance(s) that is deployed geographically close to the UE(s), e.g. in edge computing scenario. To support the CHF discovery and selection based on location, CHF can provide the location information during NRF registration, i.e. the scope of geographical areas that can be served by the CHF instance.

The CHF attributes registered with NRF per Chfinfo specified in TS 29.510 [2] Table 6.1.6.2.32-1 should be enhanced by including location information, e.g. reuse the “servingScope” attribute.

The attribute “servingScope” refers to the served area(s) of the NF instance. As specified in TS 29.510 [2], the servingScope may indicate geographical areas, it may be used e.g. to discover and select NFs in centralized Data Centers that are expected to serve users located in specific region(s) or province(s).

An example of the extension to Chfinfo is shown in table 5.1.4.3-1.

Table 5.1.4.3-1: Extended attribute to the Chfinfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ServingScope | array(string) | O | 1..N | The scope of geographical areas that can be served by the CHF instance. |

### 5.1.4.4	Solution #1.4: CHF selection based on NF instance

The solution addresses key issue #1.1 of topic #1 on CHF selection by NF Consumers Information.

NF(CTF) may discover and select the CHF instance(s) based on the serving NF instances. To support the CHF discovery and selection based on NF instance, CHF can provide the information of NFinstances during NRF registration, i.e. the list of NF instance(s) that can be served by the CHF instance.

The CHF attributes registered with NRF per Chfinfo specified in TS 29.510 [2] Table 6.1.6.2.32-1 should be enhanced by including the identifiers of NF instance(s), for instance, a list of “nfInstanceId”, or a list of “NFIdentification”.

The attribute “nfInstanceId” uniquely identifies an NF instance as specified in TS 29.571 [10].

The attribute “NFIdentification” is a common data type defined in Nchf_ConvergedCharging Service API, which contains a set of information identifying the NF consumer, i.e. nodeFunctionality, nFName, nFIPv4Address, nFIPv6Address, nFFqdn, nFPLMNID, as specified in TS 32.291 [11].

The selection of attributes for identifying the NF instance(s) is up to operator.

An example of the extension to Chfinfo is shown in table 5.1.4.4-1. To support multiple occurency of NF instances that can be supported by one CHF instance, the registered attribute in chfinfo can be “nfInstanceIdList”.

Editor’s Note: How the list of nfInstandId or nFIdentification is obtained is FFS.

Table 5.1.4.4-1: Extended attribute to the Chfinfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nfInstanceIdList | Array(NfInstanceId) | O | 1..N | The list of NF instances that can be served by the CHF instance. |

#### 5.1.4.5	Solution #1.5: Using NRF locality information

A possible solution for key issue #1.1 covering requirements REQ-CH_SELCHF_LO-01, CHF instance based on NF Service Consumer (CTF) location.

The CHF can today register in the NRF using the NFRegister service operation in the Nnrf_NFManagement Service. The NFRegister service operation will include the NFProfile. In the NFProfile there are CHF specific attributes defined in the ChfInfo, 3GPP TS 29.510 [2] clause 6.1.6.2.32, currently it contains: supiRangeList, gpsiRangeList, plmnRangeList, groupId, primaryChfInstance, and secondaryChfInstance. The NFProfile also contain attributes common for all NFs that register. For locating an NF there are several attributes, but there are two that allows operator defined information of the NF instance location of these are locality and extLocality. The extLocality can be used to describe location of different granularity, 3GPP TS 29.510 [2] clause 6.1.6.2.2.

An NF Service Consumer can today use the Nnrf_NFDiscovery to find the CHF, this can be based on several query parameters. Two of these matches the locality and extLocality in the NFProfile these are preferred-locality and the ext-preferred-locality, where the last can set a priority on the locations listed.

For an NF using the subscription to subscribe to events there are also two attributes in the SubscriptionData that can be used LocalityDescriptionItem 3GPP TS 29.510 [2] clause 6.1.6.2.111 and LocalityDescription 3GPP TS 29.510 [2] clause 6.1.6.2.112

The above allows an NF Service Consumer to find an NF instance close to it or at a specific location.

### 5.1.5	Evaluation

### 5.1.6	Conclusion

## 5.2	Topic #2: CHF Selection based on SUPI or Group ID

### 5.2.1		General description and assumptions

In TS 23.501 [5] clause 6.3.11 it states that discovered using NRF as described in in clause 6.1 of TS 32.290 [3]. It further states that the CHF selection functionality in NF consumer or in SCP should consider one of the following factors:

- CHF Group ID of the UE's SUPI.

- SUPI; the NF consumer selects a CHF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for CHF discovery.

In the case of delegated discovery and selection in SCP, the NF consumer shall include all available factors in the request towards SCP.

Therefore, it should be improved the time required for such operation, for use cases which make use of latency as one the main requirements.

### 5.2.2	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 32.255 [4].

REQ-CH_ SELCHF_LA -01: The 5GS should be able to optimize the CHF Selection during the PDU Session Establishment

### 5.2.3	Key issues#2.1: CHF Instance Selection optimization

This key issue is for investigating how to support the selection of a CHF Instance based on SUPI or Group ID considering REQ-CH_ SELCHF_LA-01. This investigation covers the following:

-	CHF Instance Selection optimization.

-	identification and classification of the information required to reduce the time of CHF Instance Selection.

### 5.2.4	Possible Solutions

### 5.2.4.1	Location Optimization Solution

A possible solution for key issue #2.1 covers the requirement REQ-CH_ SELCHF_LA -01.

Currently, SUPI/Group ID is used for NRF-based discovery of CHF. The CHF discovery mechanism needs to support the discovery of a CHF by SMF, or AMF, or SMSF or PCF based on information about network segments that the CHF belongs to.

By using CHF selection based on location information, we enable supporting the most suitable CHF for efficient processing based on the location . Potentially selecting most suitable CHF based on the location, may reduce also latency for NF that are running PDU sessions with the Charging, such as SMF and PCF.

Network Integration before the suggested change for a service operation discovers the set of CHF Instances (and its associated CHF Service Instances), that is currently registered in NRF and is selected based on the  SUPI or group ID.

This operation using NF type of CHF, retrieves a list of CHF Instances, and their offered services, currently registered in the NRF, satisfying a filter criteria of SUPI  or group ID. Since the CHF selection is based on SUPI or Group ID only, the selected CHF may be distant from the consumer NF.

Network Integration after CHF selection based on location, will be able to get a CHF Instance based on a location. There are two options to implement the location information:

1.	UE serving area, as received from AMF (TAI).

2.	NF consumer location that is set during NF deployment.

The following procedure shall be followed for CHF Registration (with Location Data), which shall be used for CHF Selection:

1.	CHF registers to NRF with served location area (such as TAI or list of TAI). The registration will be using a new  data type CHFCond (to be included in Nnrf_NFManagement service-based interface protocol as described in 3GPP TS 29.510).

2.	CHF discovery by the NF consumers (like the SMF, the AMF, the SMSF and the PCF) will retrieve relevant CHF per the location information (UE serving area or NF consumer deployment location).

3.	CHF sends a response to the consumer. CHF discovery by the NF consumers (like the SMF, the AMF, the SMSF and the PCF) will retrieve relevant CHF per the location information (UE serving area or NF consumer deployment location). This operation using NF type of CHF ,retrieves a list of CHF Instances, and their offered services, currently registered in the NRF, satisfying a filter criteria of UE serving area or NF consumer location.

This solution would bring the following advantages for CHF Selection:

- Extend the criteria that can be used with SUPI to optimize time for CHF Selection

### 5.2.4.2	Solution #2.2 CHF selection based on user group

The solution addresses key issue #2.1 of topic #2 on CHF selection by SUPI or Group ID.

NF(CTF) can discover and select the CHF instance(s) based on the UE’s group information, e.g. for enterprise customers. To support the CHF discovery and selection based on group, CHF can provide the UE group information during NRF registration, i.e. the group of individual UE(s) that can be served by the CHF instance.

The CHF attributes registered with NRF per Chfinfo specified in TS 29.510 [2] Table 6.1.6.2.32-1 should be enhanced by including group information, e.g. reuse the “externalGroupIdentifiersRanges” attribute defined in NEFinfo in TS 29.510 [2] Table 6.1.6.2.48-1.

The “externalGroupIdentifiersRanges” is made up of a list of “externalGroupIdentifier” with data type “externalGroupId”. According to the definition in TS 29.571 [10], the data type “externalGroupId” refers to string identifying External Group Identifier that identifies a group made up of one or more subscriptions associated to a group of IMSIs.

An example of the extension to Chfinfo is shown in table 5.2.4.2-1.

Table 5.2.4.2-1: Extended attribute to the Chfinfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| externalGroupIdentifiersRanges | array(IdentityRange) | C | 1..N | The range of external group identifier representing a group of individual UE(s) that can be served by the CHF instance. |

### 5.2.4.3	Solution #2.3: Use full set of NRF discovery

A possible solution for key issue #2.1 covering requirements REQ-CH_SELCHF_LA-01, optimize CHF selection.

The CHF can today register in the NRF using the NFRegister service operation in the Nnrf_NFManagement Service. The NFRegister service operation will include the NFProfile. In the NFProfile there are CHF specific attributes defined in the ChfInfo, 3GPP TS 29.510 [2] clause 6.1.6.2.32. The NFProfile also contain attributes common for all NFs that register.

An NF Service Consumer can today use the Nnrf_NFDiscovery to find the CHF, this can be based on several query parameters, 3GPP TS 29.510 [2] clause A.2. These query parameters can be dynamically set based on internal configuration of the NF Service Consumer.

### 5.2.5	Evaluation

### 5.2.6	Conclusion

## 5.3	Key Issue #3 CHF Selection with Extended Subscription Data

## 5.4	Topic #4 CHF Discovery by Charging Domains

### 5.4.1 	General description and assumptions

This topic is for investigating how to support the discovery of a Charging Function segregated by one or more charging domains or subsytems. For example, there can be separate CHF NFs deployed:

- 5G connection and mobility domain charging

- IP Multimedia System (IMS) domain charging

- Edge Computing domain charging

- Proximity-based Services (ProSe) charging

- Exposure function Northbound Application Program Interfaces (APIs) charging.

Sometimes a CHF NF instance could support a set of related domains: e.g.

- IMS and MMTel domains

- Network Slice Management, Network Slice Admission Control, and Network Slice Performance and Analytics domains.

Therefore, it should be possible to discover and select a CHF instance based on the charging domain(s) or subsystem(s) that the charging is required for.

### 5.4.2	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 32.255 [4]

REQ-CH_ SELCHF_CD -01: The 5GS should be able to provide a CHF instance based on the functional domain or subsystem that requires charging.

### 5.4.3 	Key Issues

The following key issues are identified:

- Key Issue #4a: Definition of charging domain(s) or subsystem(s) for distributed charging deployments

- Key Issue #4b: Determination of methods and procedures by which a CHF service producer can indicate the supported charging domain(s) or subsystem(s)

- Key Issue #4c: Determination of methods and procedures by which a CHF service consumer can indicate the charging domain(s) or subsystem(s) for which it requests charging

- Key Issue #4d: Determination of methods and procedures by which a CHF service consumer can discover a CHF service producer providing charging service for one or more charging domain(s) or subsystem(s)

### 5.4.4	Possible Solutions

#### 5.4.4.1	Solution #4.1: Use NF type

A possible solution for key issue #4.1, discovery by charging domains.

Reuse the NF type to cover the requirements on domain based discovery and charging. In most cases one NF type is connected to a domain the service specific charging information is also connected to the NF type rather than the domain. This means that in most cases the domain and required support of service specific charging information can be inferred by using the NF type.

Editor’s note: CHF to CHF connection and reference to the NF type definition are ffs.

### 5.4.5	Evaluation

### 5.4.6 	Conclusion

# 6	Evaluation

# 7	Conclusion

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-02 | SA5#147 | S5-232095 S5-232096 S5-232918 |  |  |  | Add the skeleton to TR 28.844 Add scope and reference to TR 28.844 Some editorial changes including aligning TR front page title with SA5 official title are made as well | 0.1.0 |
| 2023-04 | SA5#148e | S5-233668 S5-233678 |  |  |  | Document Structure Abbreviations Chapter Update | 0.2.0 |
| 2023-04 | SA5#149 | S5-234010 S5-234632 |  |  |  | Abbreviations Chapter Update Update of the Overview | 0.3.0 |
| 2023-08 | SA5#150 | S5-235884 S5-235885 S5-235887 |  |  |  | CHF Selection by NF Consumers Information CHF Selection based on SUPI ID or Group ID CHF Selection per Charging Domain | 0.4.0 |
| 2023-10 | SA5#151 | S5-236386 S5-236985 S5-236388 S5-236998 S5-236986 S5-236987 S5-236988 S5-236989 S5-236990 S5-236991 S5-236992 S5-236993 |  |  |  | Abbreviations Chapter Update References Chapter Update The followinCHF Selection and Discovery for NF Service Consumers Solution based on UDM CHF Selection by using CHF Default Instance Solution Location Optimization Solution CHF Selection by Charging Domains CHF Selection by Location CHF Selection by NF Instance CHF Selection by User Group CHF Selection by NF Instance Location CHF Selection CHF Selection for domain based discovery | 0.5.0 |