3GPP TSG-SA5 Meeting #153 	S5-240705

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0514 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Network slice charging into generic business charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MATRIXX Software, Huawei, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE_CH_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Network slice charging is one case of business charging.The flows for Network slice charging need to be more generic to also cover other business converged charging based on 5G data connectivity The UE CHF and Tenant CHF refer to the C-CHF and B-CHF and need to be replaced. Introduce study TR 32.847 conclusion clause 6.6.7 on solutions #6.1 (MnS solution), #6.2 (NWDAF solution), #6.3 and #6.4 (NSACF solutions) for Key Issue #6: UE Converged Charging based on network slice charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update flows for Network Slice charging to generic business converged charging based on 5G data connectivity Change the name of UE CHF to C-CHF, Tenant CHF to B-CHF. Introduce a new flow combining solutions #6.1 (MnS solution), #6.2 (NWDAF solution), #6.3 and #6.4 (NSACF solutions) of TR 32.847 conclusion for Key Issue #6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Business converged charging not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.21, 5.2.2.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Merge of S5-240320 and S5-240550 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-240670 |  |  |  |  |  |  |  |  |

First change

#### 5.2.2.21	Business charging based on 5G data connectivity

##### 5.2.2.21.1	General

The clause below describes business converged charging based on PDU session charging, using the C-CHF and B-CHF architecture defined in clause 4.3.

The flows are applicable for Network slice converged charging based on 5G data connectivity where the B-CHF handles the Tenant the Network Slice is assigned to.

The flows are applicable for TSN converged charging based on 5G data connectivity where the B-CHF handles the TSN service provider.

##### 5.2.2.21.2	PDU session establishment

The clause describes the high level charging procedure for business converged charging based on 5G data connectivity with PDU session SCUR scenario.

The following figure 5.2.2.21.2-1 describes a UE PDU session establishment for business converged charging based on 5G data connectivity

Figure 5.2.2.21.2-1: business converged charging based on 5G data connectivity – UE PDU session establishment

0.	Business dependent charging information is configured in C-CHF and B-CHF, which could include e.g.

subscriber identifier,

Identification of the business subscriber.

1 to 9ch-b. PDU session establishment for individual UE per Figure 5.2.2.2.2-1.

9ch-ba.	Business charging based on 5G data connectivity is enabled.

9ch-bc.  C-CHF sends the Charging Data Request [Initial] to B-CHF with UE PDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID. Account, Rating and reservation control for the Tenant.

9ch-bd.	 The B-CHF opens or updates B-CDR for this business specific identifier with the PDU session.

9ch-be.	 B-CHF provides appropriate Charging Data Response [Initial] for Tenant.

9ch-cf.	C-CHF provides appropriate Charging Data Response [Initial] for the individual UE.

9ch-c to 16ch-b. PDU session establishment for individual UE per Figure 5.2.2.2.2-1.

16ch-ba.	Business charging based on 5G data connectivity is enabled.

16ch-bc. C-CHF sends the Charging Data Request [Update] to B-CHF with PDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID.

16ch-bd.	 The B-CHF updates B-CDR for this business specific identifier.

16ch-be.	 B-CHF provides appropriate Charging Data Response [Update].

17 to 19. PDU session establishment for individual UE per Figure 5.2.2.2.2-1.

##### 5.2.2.21.3	PDU session modification

The following figure 5.2.2.21.3-1 describes a UE PDU session modification for business converged charging based on 5G data connectivity:

Figure 5.2.2.21.3-1: business converged charging based on 5G data connectivity – UE PDU session modification

1 to 2ch-b. PDU session modification for individual UE per Figure 5.2.2.2.3-1.

2ch-ba.	Business charging based on 5G data connectivity is enabled.

2ch-bc.  C-CHF sends the Charging Data Request [Update] to B-CHF with UE PDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID. Account, Rating and reservation control for the Tenant.

2ch-bd.	 The B-CHF updates B-CDR for this business specific identifier with the PDU session.

2ch-be.	 B-CHF provides appropriate Charging Data Response [Update] for Tenant.

2ch-c to 13. PDU session modification for individual UE per Figure 5.2.2.2.2-1.

##### 5.2.2.21.4	PDU session release

The following figure 5.2.2.21.4-1 describes a UE PDU session release for business converged charging based on 5G data connectivity:

Figure 5.2.2.21.4-1: business converged charging based on 5G data connectivity – UE PDU session release

1 to 2ch-b. PDU session release for individual UE per Figure 5.2.2.2.4-1.

2ch-ba.	Business charging based on 5G data connectivity is enabled.

2ch-bc.  C-CHF sends the Charging Data Request [Termination] to B-CHF with UE PDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID. Account, Rating and reservation control for the Tenant.

2ch-bd.	 The B-CHF updates or closes B-CDR for this business specific identifier.

2ch-be.	 B-CHF provides appropriate Charging Data Response [Termination] for Tenant.

2ch-c to 13. PDU session release for individual UE per Figure 5.2.2.2.4-1.

Next change

#### 5.2.2.X	UE PDU session charging based on Business converged charging

##### 5.2.2.X.1	General

The clause below describes UE PDU session converged charging influenced by business converged charging based the C-CHF and B-CHF architecture defined in clause 4.3.

The flows are applicable for Network slice converged charging based on 5G data connectivity where the B-CHF handles the Tenant the Network Slice is assigned to.

##### 5.2.2.X.2	PDU session establishment

The following figure 5.2.2.X.2-1 describes a UE PDU session converged influenced by business converged charging:

Figure 5.2.2.X.2-1: UE PDU session converged Charging influenced by business charging

0ch. Business charging information obtained by B-CHF from NWDAF, MnS Producer, NSACF, which could be e.g:

-	S-NSSAI service profile: e.g. "Max nb of PDU sessions", "Max nb of UEs”…

-	S-NSSAI KPIs;

- 	load level.

1 to 9ch-b. PDU session establishment for individual UE per Figure 5.2.2.2.2-1.

9ch-ba.	UE PDU session charging needs input from business charging

9ch-bc.	C-CHF sends the Charging Data Request [Initial] to B-CHF with UE PDU session charging information received from SMF, and "UE requested business charging information".

9ch-bd.	Based on received "UE requested business charging information", B-CHF determines based on business charging information the UE charging information to be supplied.

9ch-be.	B-CHF sends the Charging Data Response [Initial] to C-CHF with the "UE supplied business charging information".

9ch-bf.	Account, Rating and reservation control for individual UE based on "UE supplied business charging information".

9ch-c.	C-CHF provides appropriate answer for the individual UE.