| 3GPP TR 28.871 V0.1.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Based Management Architecture (SBMA) enhancement phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Concepts and background	7

5	Use cases and potential requirements	7

5.1	NRM investigation	7

5.1.1	Description	7

5.1.2 Potential requirement	8

5.2 Use case#2: Connection between TS 28.537 and TS 28.622 for generic control NRM capabilities	8

5.2.1 Description	8

5.2.2 Potential requirements	8

5.2.3 Potential solutions	8

5.2.3.1 Potential solution#1 Add reference for solution description (NRM fragment) for each management capability in TS 28.537.	8

5.2.3.2 Potential solution#2 Add reference for management capability description for each NRM fragment in TS 28.622	9

5.2.3.3 Potential solution#3 Add reference for management capability requirements for each NRM fragment in TS 28.622	9

5.2.4 Evaluation of potential solutions	10

5.3	 Support IOCs in SBMA	10

5.3.1	Description	10

5.3.2 	Potential requirements	10

5.3.3	Potential solutions	10

6	Potential solutions	11

7	Conclusions and recommendations	11

Annex <X> (informative): Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

With the progression in the use of service-based management services by service providers and operators, new and updated use cases and potential requirements are introduced for managing 5G network and services within the service-based management architecture (SBMA). The present document studies the corresponding updates for the SBMA.

# 1	Scope

The present document describes use cases, potential requirements, and solutions for the enhancement of the Service-Based Management Architecture (SBMA). The document provides conclusions and recommendations on the next steps in the standardization.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[3]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[4]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".[5]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[6]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA) "

[7]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management"

[8]	3GPP TS 28.111: "Fault management"

[9]	3GPP TS 28.311: "Management and orchestration; Network policy management for mobile networks based on Network Function Virtualization (NFV) scenarios"

[10]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

[11]	3GPP TS 28.317: "Management and orchestration; Self-configuration of Rad io Access Network Entities (RAN NEs)"

[12]	3GPP TS 28.318: "Management and Orchestration; Network and services operations for energy utilities"

[13]	3GPP TS 28.319: "Management and orchestration; Access Control for Management services"

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services"

[15]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3"

[16]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management"

[17]	3GPP TS 28.556: "Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and background

# 5	Use cases and potential requirements

## 5.1	NRM investigation

### 5.1.1	Description

One important argument when introducing SBMA was to get away from the fragmented specification structure that IRP architecture has.

The following TSs contain stage 2 NRM fragments specified for 5G:

28.622 Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) [2]

28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 [5]

28.104 Management and orchestration; Management Data Analytics (MDA) [6]

28.105 Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management [7]

28.111 Fault management [8]

28.311 Management and orchestration; Network policy management for mobile networks based on Network Function Virtualization (NFV) scenarios [9]

28.312 Management and orchestration; Intent driven management services for mobile networks [10]

28.317 Management and orchestration; Self-configuration of Radio Access Network Entities (RAN NEs) [11]

28.318 Management and Orchestration; Network and services operations for energy utilities [12]

28.319 Management and orchestration; Access Control for Management services [13]

28.532 Management and orchestration; Generic management services [14]

28.536 Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3 [15]

28.538 Management and orchestration; Edge Computing Management [16]

28.556 Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3 [17]

### 5.1.2 Potential requirement

NA.

## 5.2 Use case#2: Connection between TS 28.537 and TS 28.622 for generic control NRM capabilities

### 5.2.1 Description

Several control NRM Fragments (e.g. MnS Registry NRM fragment, File retrieval NRM fragment, File download NRM fragment and Management data collection NRM fragment) defined in TS 28.622 [2]. However, corresponding stage1 (including description, use case and requirements) is defined in TS 28.537 [3]. It is not clear which management capabilities/features such control NRM Fragments are used to support. The connection between TS 28.622 [2] and TS 28.537[3] for each control NRM fragment needs to be added to improve the readability.

### 5.2.2 Potential requirements

NA

### 5.2.3 Potential solutions

Following potential solution use Capability of discovery of management services defined in TS 28.537[3] and MnS Registry NRM fragment defined in TS 28.622[2] for example, same solution also applied for other management capabilities in TS 28.537 [3], including but not limited to Heartbeat, Managing management data, File management.

#### 5.2.3.1 Potential solution#1 Add reference for solution description (NRM fragment) for each management capability in TS 28.537.

Following clause 5.1 overview and 5.2 specification level requirements are the existing clauses, clause 5.3 solution is proposed to be added in clause 5 in TS 28.537 [3].

5	Discovery of Management Services

5.1	Overview

5.2	Specification level requirements

5.3	Solution

The information model for Registry NRM fragment to support above management capabilities for discovery of Management Services is defined in TS 28.622 [3].

The YAML and YANG solution set for NRM fragment to support above management capabilities for discovery of Management Services is defined in TS 28.623 [4]

#### 5.2.3.2 Potential solution#2 Add reference for management capability description for each NRM fragment in TS 28.622

Following clause 4.3.41 is the existing clauses, the first sentence is proposed to be added in clause 4.3.41.1 in TS 28.537 [3].

4.3.41	MnsRegistry

4.3.41.1	Definition

The MnSRegistry IOC and contained MnsInfo IOC is used to support the management capabilities for Discovery of Management Services in clause 5 in TS 28.537 [2].

This IOC is a container for MnsInfo IOC-s. It can be contained only by SubNetwork IOC. A SubNetwork IOC can contain only one instance of MnsRegistry.The IOC is instantiated by the system.

4.3.41.2	Attributes

The MnsRegistry IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29).

4.3.41.3	Attribute constraints

None.

4.3.41.4	Notifications

None.

#### 5.2.3.3 Potential solution#3 Add reference for management capability requirements for each NRM fragment in TS 28.622

Following clause 4.3.41 and 4.3.42 is the existing clauses, the highlight sentence and table is proposed to be added in clause 4.3.42.1 in TS 28.537 [3].

4.3.41	MnsRegistry

4.3.41.1	Definition

This IOC is a container for MnsInfo IOC-s. It can be contained only by SubNetwork IOC. A SubNetwork IOC can contain only one instance of MnsRegistry.The IOC is instantiated by the system.

4.3.41.2	Attributes

The MnsRegistry IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29).

4.3.41.3	Attribute constraints

None.

4.3.41.4	Notifications

None.

4.3.42	MnsInfo

4.3.42.1	Definition

This IOC represents an available Management Service (MnS) and provides the data required to support its discovery.  It is name-contained by MnsRegistry.

This information is used by the consumer to discover the producers of specific Management Services and to derive the addresses of the Management Service.

Attributes mnsLabel, mnsType, and mnsVersion are used to describe the Management Service.

Attribute mnsAddress is used to provide addressing information for the Management Service operations.

Attribute mnsScope is used to provide information about the management scope of the Management Service. The management scope is defined as the set of managed object instances that can be accessed using the Management Service.

Following requirements related to management capabilities for Discovery of Management Services supported by the MnsInfo IOC.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.537 [2] | REQ-DMS-1 |  |
| TS 28.537 [2] | REQ-DMS-2 |  |
| TS 28.537 [2] | REQ-DMS-3 |  |
| TS 28.537 [2] | REQ-DMS-4 |  |

### 5.2.4 Evaluation of potential solutions

TBD

## 5.3		Support IOCs in SBMA

### 5.3.1	Description

SBMA is based on data that is visible, readable, and depending on its nature, potentially writable using the CRUD operations defined in TS 28.532[3].  Support IOCs cannot be handled by the CRUD operations.

The only reason to model data by a real or support IOC is because that data is available in the MnS producer. While it is possible to provide this information via notifications, files or to set the data via dedicated operations SBMA follows the principles:

- if the data is interesting for the consumer, it should be readable via CRUD, because the consumer might not be available when the data is produced. The consumer should be able to fetch the data on demand when the consumer needs it and when the consumer is capable of receiving it.

- it is easier for a consumer to handle a few simple CRUD operations with configuration specific data models than to implement and handle a potentially big number of different dedicated operations and access methods.

Based on the above CRUD operations should be available for modelled data. CRUD operations are available for normal IOCs but not for support IOC, hence specifying new support IOCs should be avoided.

Removing existing support IOCs (if any) is out of scope for this proposal.

### 5.3.2 	Potential requirements

NA.

### 5.3.3	Potential solutions

Clarify in TS 32.156[2] that Support IOCs are not used when the MnS is designed based on a model driven approach using a NRM and CRUD operations.

# 6	Potential solutions

# 7	Conclusions and recommendations

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 |  | - | - | - | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5‑241379 | - | - | - | pCR TR 28.871 Add scope for SBMA study report | 0.1.0 |
| 2024-04 | SA5#154 | S5‑241380 | - | - | - | pCR TR 28.871 Add structure for SBMA study report | 0.1.0 |
| 2024-04 | SA5#154 | S5‑241381 | - | - | - | pCR TR 28.871 Add introduction for SBMA study report | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242077 | - | - | - | pCR TR 28.871 Add use case, requirement and solution for connection between TS 28.537 and TS 28.622 for control NRM capabilities | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242080 | - | - | - | Rel-19 pCR TR 28.871 Dont use support IOCs in SBMA | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242081 | - | - | - | pCR 28.871-000 Inventory of 5G NRM specifications | 0.1.0 |