| 3GPP TR 28.849 V0.1.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on charging Aspects for Common API Framework (CAPIF) phase2; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	  Objective	8

5	  Concepts and background	8

5.1	General Description	8

5.2	Background	9

5.2.1	API Exposure	9

5.3	Business Roles related to CAPIF charging	10

5.3.1	Basic CAPIF business relationships	10

5.3.2	CAPIF business relationships for RNAA	10

5.4	Charging aspect requirements by SA6	11

6	  CAPIF Charging Scenarios and Topics	12

6.1	Topic #1 CAPIF Converged Charging support of Service APIs Operation and Management	12

6.1.1	General description and assumptions	12

6.1.2	Use Case	12

6.1.2.1	Use Case #1.1: Service API Operation and management via CAPIF	12

6.1.2.2	Use Case #1.2: API invoker management via CAPIF	12

6.1.3	Potential charging requirements	13

6.1.4	Key issues	13

6.1.4.1	Key issue#1.1: Charging events and charging information required	13

6.1.4.2	Key issue#1.2: Charging events and charging information required	13

6.1.5	Possible Solutions	13

6.1.5.1	Solution #1.1: API invocation via CAPIF	13

6.1.5.1.1	General description	13

6.1.5.1.2	Architecture description	13

6.1.5.1.3	Procedures description	14

6.1.6	Evaluation	15

6.1.7	Conclusion	15

6.2	Topic #2 CAPIF Converged Charging of multiple API Providers	15

6.2.1	General description and assumptions	15

6.2.2	Use Case	15

6.2.2.1	Use Case #2.1: API Provider Converged Charging	15

6.2.2.2	Use Case #2.2: API Invoker Converged Charging	16

6.2.3	Potential charging requirements	16

6.2.4	Key issues	16

6.2.4.1	Key issue#1.1: Charging events and charging information required	16

6.2.5	Possible Solutions	16

6.2.6	Evaluation	16

6.2.7	Conclusion	16

6.3	Topic #3 API Service usage charging of CAPIF	16

6.3.1	General description and assumptions	17

6.3.2	Use Case	17

6.3.2.1	Use Case #3.1: API Service Usage via CAPIF	17

6.3.3	Potential charging requirements	17

6.3.4	Key issues	17

6.2.4.1	Key issue#1.1: Charging events and charging information required	17

6.3.5	Possible Solutions	18

6.3.6	Evaluation	18

6.3.7	Conclusion	18

7	  Evaluation	18

8	  Conclusion	18

Annex <A>: Change history		19

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document on how CAPIF can be supported by the current Converged charging Architecture.

The following are studied:

- possible charging scenarios and requirements for supporting CAPIF.

- identify NEF functionalities which can implement CAPIF functionalities for charging scenarios

- potential charging solutions for supporting CAPIF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[4]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP		3rd Generation Partnership Project

5GS	5g System

AEF	API Exposing Function

API	Application Programming Interface

AS	Application Server

BSS	Business Support Systems

CAPIF	Common API Framework

CCF	CAPIF Core Function

CDR	Charging Data Record

CHF	Charging Function

CTF		Charging Trigger Function

ECUR	Event Charging with Unit Reservation

EPS	Evolved Packet System

IEC	Immediate Event Charging

NEF	Network Exposure Function

NF	Network Function

OCS	Online Charging System

OSS	Operational Support Systems

PEC	Post Event Charging

RNAA	Resource owner-aware Northbound API Access

SCEF	Service Capabilities Exposure Function

SCS	Service Capability Server

# 4			Objective

# 5			Concepts and background

## 5.1	General Description

The Common API framework (CAPIF) architecture is defined as service-based and interactions between the CAPIF functions are represented in two ways:

-	A service-based representation, where CAPIF functions enable other authorized CAPIF functions to access their services;

-	A reference point representation, where interactions between any two CAPIF functions (e.g. CCF, AEF) is shown by an appropriate point-to-point reference point (e.g. CAPIF-3).

The CAPIF functional architecture can be adopted by any 3GPP functionality providing 3GPP northbound service APIs.

Figure 5.1-1 shows the reference point based functional model for the CAPIF.

Figure 5.1-1: Functional model for the CAPIF

## 5.2	Background

### 5.2.1	API Exposure

According to the CAPIF relationship with network exposure aspects of 3GPP systems in the clause Annex B described in the TS 23.222 [2], the details of SCEF which is the role in exposing network capabilities of EPS to 3rd party applications and NEF which is the role in exposing network capabilities of EPS to 3rd party applications are present in the following clause:

-	B.1 	CAPIF relationship with 3GPP EPS network exposure

-	B.2	 	CAPIF relationship with 3GPP 5GS network exposure

According to the CAPIF role in charging in the clause Annex C described in the TS 23.222 [2], the common architecture for charging, including the offline charging and online charging, which is the role of CAPIF in charging service API invocations, is introduced in the figure Figure C.1-1 and following clauses in the TS 23.222 [2]:

-	C.2 	CAPIF role in online charging

-	C.3 	CAPIF role in offline charging

According to the high-level Service Exposure Function architecture described in the TS 32.254 [3], the offline, online and converged charging description for Northbound Application Program Interfaces (API), based on the transaction over T8 reference point between SCEF and SCS/AS and Network Exposure Function (NEF) are introduced:

-	Clause 4.2 and 4.3: the details of SCEF functioning as the CTF, may generate accounting metrics sets for Northbound Application Program Interfaces (APIs) CDRs in the figure 4.2.1, and the details of SCEF utilizing Ro interface and application towards the OCS in the figure 4.2.2 are described:

-	Clause 4.4: the details of NEF functioning as the CTF, using Nchf to perform converged charging for the northbound API access.

## 5.3	Business Roles related to CAPIF charging

### 5.3.1	Basic CAPIF business relationships

As per the TS 23.222 [2] clause 5.1, the basic business roles related to CAPIF charging include:

Figure 5.3.1-1: Business relationships in CAPIF

-	API invoker: typically provided by a 3rd party application provider, which is the charged party for CAPIF API invocation charging by CAPIF provider.

-	CAPIF provider: the charging party, which embeds the CTF to perform the charging to API invoker.

### 5.3.2	CAPIF business relationships for RNAA

As per the TS 23.222 [2] clause 5.2, the business roles related CAPIF for RNAA charging include:

Figure 5.3.2-1: CAPIF business relationships for RNAA

-	API invoker: the same as the basic CAPIF business relationships in clause 5.2.1.

-	CAPIF provider:the same as the basic CAPIF business relationships in clause 5.2.1.

## 5.4	Charging aspect requirements by SA6

As per the charging aspect requirements specified in TS  23.222 [2], the requirement related CAPIF Charging are present as following:

-	 [AR-5.4.2-a] The CAPIF shall support charging for service APIs usage.

-	 [AR-5.4.2-b] The CAPIF shall provide mechanisms to record the usage (e.g. invocation count) of the service APIs for charging purpose, on a per API invoker basis.

-	 [AR-5.4.2-c] The CAPIF shall provide mechanisms to record timestamp of the service API invocation.

-	 [AR-5.4.2-d] The CAPIF shall provide mechanisms to record the service API related information, e.g. API location.

-	 [AR-5.4.3-a] The CAPIF shall support online and offline charging for 3rd party API providers' service APIs -usage.

-	 [AR-5.4.3-b] The CAPIF shall provide mechanisms to query charging related information of the 3rd party service APIs by the authorized users.

The above charging requirements are defined by SA6, which need to be studied from SA5 charging aspect.

# 6			CAPIF Charging Scenarios and Topics

## 6.1	Topic #1 CAPIF Converged Charging support of Service APIs Operation and Management

### 6.1.1	General description and assumptions

Informative Annex B.2.2 of 3GPP TS 23.222 [2] describes on how the NEF can implement the specific service aspect and how its aligned with the CAPIF Architecture. Therefore, it would be necessary to evaluate the possibility to support Converged charging in those deployment models.

### 6.1.2	Use Case

#### 6.1.2.1	Use Case #1.1: Service API Operation and management via CAPIF

The service API invokers (e.g. the 3rd party application provider) and the CAPIF providers (i.e. Operators) has the service agreement about the API invocations. The CAPIF providers and API providers has the Service API arrangement about the API publishing, API exposure and API management.

The API invoker can perform one or multiple service API invocation to CAPIF. The CAPIF Core Function acknowledges the API invocation request and provide the API invocation response to the API invoker. CAPIF can collect the charging information based on the following chargeable events, for example:

-	the service API publish, as specified in the TS 23.222 [2] clause 8.3

-	the service API unpublish, as specified in the TS 23.222 [2] clause 8.4

-	the service API retrieve, as specified in the TS 23.222 [2] clause 8.5

-	the service API update, as specified in the TS 23.222 [2] clause 8.6

The charging party: the CAPIF providers

The charged party: the API provider

The potential charging requirements for this UC are: REQ-3GPPCH-APIM-01.

#### 6.1.2.2	Use Case #1.2: API invoker management via CAPIF

The service API invokers (e.g. the 3rd party application provider) and the CAPIF providers (i.e. Operators) has the service agreement about the API invoker management. The CAPIF providers the Service API about API invoker management.

As part of onboarding an API Provider is provisioned to get access to the service APIs. From a high-level perspective, the corresponding process has implications on the provisioning on the Operator’s BSS/OSS Systems that will also be responsible for triggering the chargeable events. CAPIF can collect the charging information based on the following chargeable events, for example:

-	the service activation: Onboarding the API invoker to the CAPIF, as specified in TS 23.222 [2] Clause 8.1

-	the service in-activation: Offboarding the API invoker from the CAPIF, as specified in TS 23.222 [2] Clause 8.2

The charging party: the CAPIF providers

The charged party: the API invoker

The potential charging requirements for this UC are: REQ-3GPPCH-IVKM-01.

### 6.1.3	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 23.222 [2].

REQ-3GPPCH-APIM-01: The CAPIF should support converged charging for APIs service management per API provider.

REQ-3GPPCH-IVKM-01: The CAPIF should support converged charging for APIs invoker management.

### 6.1.4	Key issues

#### 6.1.4.1	Key issue#1.1: Charging events and charging information required

This key issue is for investigating how to support the charging considering REQ-3GPPCH-APIM -01. This investigation covers the following:

-	identification and classification of the charging information for APIs service Operation and Management;

-	identification and classification of the chargeable event for APIs service Operation and Management.

#### 6.1.4.2	Key issue#1.2: Charging events and charging information required

This key issue is for investigating how to support the charging considering REQ-3GPPCH-IVKM-01. This investigation covers the following:

-	identification of the charging information for API invoker management;

-	identification of the chargeable event for API invoker management;

### 6.1.5	Possible Solutions

#### 6.1.5.1	Solution #1.1: API invocation via CAPIF

##### 6.1.5.1.1	General description

This solution #1.1 which relying on CHF/5G Converged Charging System for CAPIF Converged Charging, addresses the Key Issue #1.2.

##### 6.1.5.1.2	Architecture description

Figure 6.1.5.1.2-1 shows the 5G System high level charging architecture for CAPIF Core Function (CCF) Charging, in the reference point representation for non-roaming:

Figure 6.1.5.1.2-1: 5G CCF converged charging architecture non-roaming

##### 6.1.5.1.3	Procedures description

The figure 6.1.5.1.3-1 describes the high-level charging procedure for CCF Converged charging for API invocation via CAPIF-2/CAPIF-2e and CAPIF-3, based on the figure 8.20.3.1 Procedure for charging the invocation of service APIs in the TS 23.222 [2]. The event-based charging. i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Figure 6.1.5.1.3-1: Charging Procedure for API invocation to the CAPIF via AEP (PEC)

1.	One or more API invoker send the invocation of service API(s) request to AEF.

2.	Upon invocation of service API(s) from one more API invokers, the AEF triggers an API invocation request and includes API invoker information (e.g. invoker's ID and IP address, location, timestamp) and service API information (e.g. service API name and version, invoked operation, input parameters, invocation result), Network Slice Info of the service API towards the CAPIF core function.

3.	The CAPIF core function performs a API invocation handling procedure which includes storing the information for access by authorized API management.

4.	The AEF receives the API invocation response from the CAPIF core function.

4ch-a, CAPIF sends the Charging Data Request [Event] to CHF for the service API(s) invocation to be granted authorization, and to allow the number of units, if determined by CAPIF, to be rated and accounted.

4ch-b.	The CHF performs the Account, Rating Control procedure and based on policies, the CHF creates a CDR related to the service API(s) invocation.

4ch-c. The CHF grants authorization to CAPIF for the service API(s) invocation to start, with a number of granted units.

5.	The AEF sent the API invocation response to the API invoker.

The figure 6.1.5.1.3-2 describes the high-level charging procedure for CCF Converged charging for API invocation via CAPIF-1 or CAPIF-1e. The event-based charging. i.e. IEC, PEC and ECUR specified in TS 32.290 [4] are supported.

Figure 6.1.5.1.3-2: Charging Procedure for API invocation to the CAPIF (PEC)

The steps description of figure 6.1.5.1.3-2 is similar with the figure 6.1.5.1.3-1, with the difference as the following:

-	The step 2 and step 4 in the figure 6.1.5.1.3-1 are not applicable.

### 6.1.6	Evaluation

### 6.1.7	Conclusion

## 6.2	Topic #2 CAPIF Converged Charging of multiple API Providers

### 6.2.1	General description and assumptions

Informative Annex B.3.2 of 3GPP TS 23.222 [2] describes on how the NEF (and SCEF) can implement the specific service aspect and how its aligned with the CAPIF Architecture. Therefore, it would be necessary to evaluate the possibility to support Converged charging in those deployment models.

### 6.2.2	Use Case

#### 6.2.2.1	Use Case #2.1: API Provider Converged Charging

The service API provider (e.g. the 3rd party application provider) can access Service APIs (i.e. available from the API Provider) through NEF. CAPIF Provider is not only managing the Service APIs that are available to API invokers, but also able to support converged charging of one or more API Providers.

The charging party: CAPIF provider

The charged party: API providers

The potential charging requirements for this UC is: REQ-CH_CAPIF_NEF-01.

#### 6.2.2.2	Use Case #2.2: API Invoker Converged Charging

The service API invokers (e.g. the 3rd party application provider) can access Service APIs (i.e. available from the API Provider) through NEF. CAPIF Provider is not only managing the exposed APIs that are available to API invokers, but also able to support converged charging either one or more API Invokers.

The charging party: CAPIF provider

The charged party: API Invokers

The potential charging requirements for this UC is: REQ-CH_CAPIF_NEF-01.

### 6.2.3	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 23.222 [2].

REQ-CH_ CAPIF_NEF-01: The 5GS should be able to provide NEF Converged Charging support to CAPIF.

### 6.2.4	Key issues

#### 6.2.4.1	Key issue#1.1: Charging events and charging information required

This key issue is for investigating how Converged Charging can support CAPIF deployment models considering REQ-CH_ CAPIF_NEF-01. This investigation covers the following:

-	identification of the CAPIF deployment models that shall be supported taking in consideration NEF specific service aspects.

-	identification of charging information required for CAPIF deployment models identified.

### 6.2.5	Possible Solutions

### 6.2.6	Evaluation

### 6.2.7	Conclusion

## 6.3	Topic #3 API Service usage charging of CAPIF

### 6.3.1	General description and assumptions

### 6.3.2	Use Case

#### 6.3.2.1	Use Case #3.1: API Service Usage via CAPIF

The service API invokers (e.g. the 3rd party application provider) and the CAPIF providers (e.g. Operators) has the service agreement about the API invocations.

The API invoker can perform one or multiple service API invocation to CAPIF. The CAPIF Core Function acknowledges the API invocation request and provide the API invocation response to the API invoker. CAPIF can collect the charging information based on the following chargeable events, according the service procedure specified in the TS 23.222[2] Service API invocation, for example:

-	the API invocation, e.g. API invoker identifier, timestamp of API invocations

-	the result of the API invocation response, e.g. the success or failure of API invocation request.

-	Subscription, un-subscription and notifications for the CAPIF events (e.g. API notification for the Monitoring service API invocation) as specified in the TS 23.222[2] clause 8.8

-	Revoking subscription of the CAPIF events, as specified in the TS 23.222[2] clause 8.9

The charging party: the CAPIF providers

The charged party: the API invoker for the chargeable event of API invocations

The potential charging requirements for this UC are: REQ-3GPPCH-APIInvo-01 and REQ-3GPPCH-APIInvo-02.

### 6.3.3	Potential charging requirements

REQ-3GPPCH-APIInvo-01: The CAPIF should support converged charging for service APIs invocations, on a per API invoker basis.

REQ-3GPPCH-APIInvo-02: The CAPIF should support the converged charging based on the resource usage (e.g. invocation count, invocation duration, invocation data volume) of the service APIs, on a per API invoker basis.

### 6.3.4	Key issues

#### 6.3.4.1	Key issue#1.1: Charging events and charging information required

This key issue is for investigating how to support the charging considering REQ-3GPPCH-APIInvo-01 and REQ-3GPPCH-APIInvo-02. This investigation covers the following:

-	identification of the charging information for API invocation;

-	identification of the chargeable event for API invocation;

### 6.3.5	Possible Solutions

### 6.3.6	Evaluation

### 6.3.7	Conclusion

# 7			Evaluation

# 8			Conclusion

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-8 | SA5#156 | S5-243527 S5-243716 S5-244511 S5-244512 S5-243719 S5-244513 S5-244515 S5-244514 S5-244516 S5-244517 S5-244518 |  |  |  | Initial skeleton Document Structure Update Skeleton Update Update of the Scope Abbreviations Update Introduce the background of CAPIF charging CAPIF Charging Scenarios and KI Addition of use cases for the API operation and management Introduce the topic on API invoker management Introduce the topic on API invocation charging Introduce the solution for API invocation charging | 0.1.0 |