3GPP TR 28.872 V0.3.0 (2024-08)

Technical Report .

3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management of planned configurations (Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Concepts and background	6

5	Use cases	6

5.1	Use case #1: Managing planned configurations	6

5.1.1	Description	6

5.1.2	Potential requirements	6

5.1.3	Potential solutions	7

5.1.3.1	Potential solution #1: Data node tree	7

5.1.4	Evaluation of potential solutions	11

5.2.4	Evaluation of potential solutions	17

5.3	Use case #3: Activating planned configurations	17

5.3.1	Description	17

5.3.2	Potential requirements	18

5.3.3	Potential solutions	18

5.3.3.1	Potential solution #1: Copy plan into current configuration	18

5.3.3.1	Potential solution #2: Dedicated job for activation	18

5.3.4	Evaluation of potential solutions	21

5.4	Use case #4: Managing transactions	21

5.4.1	Description	21

5.4.2	Potential requirements	21

5.4.3	Potential solutions	21

5.4.3.1	Potential solution #1: Grouping plans into transactions	21

5.4.3.1	Potential solution #2: Creating new plans for transactions	22

5.4.4	Evaluation of potential solutions	22

5.5	Use case #5: Fallback	22

5.5.1	Description	22

5.5.2	Potential requirements	23

5.5.3	Potential solutions	23

5.5.3.1	Potential solution #1: Fallback facility	23

5.5.3.1.1	Introduction	23

5.5.3.1.2	Description	23

5.5.4	Evaluation of potential solutions	24

5.6	Use case #6: Conditional activation of planned configurations	25

5.6.1	Description	25

5.6.2	Potential requirements	25

5.6.3	Potential solutions	25

5.6.3.1	Potential solution #1: Dedicated job	25

5.6.4	Evaluation of potential solutions	26

6	Conclusion and recommendation	26

Annex <X> (informative): Change history	27

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document contains a study on the management of planned configurations.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Management and orchestration; Management services".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Concepts and background

A telecommunication network is represented for management purposes by a data node tree on a MnS producer. When configuration changes are applied to this data node tree it is expected that the data node tree and the represented resources are updated (in synchronous or asynchronous fashion). So far there is no concept of a place, where configuration plans can be maintained.

This study investigates possibilities to manage configuration plans. These plans are activated only on explicit request. This basic functionality can be complemented with additional features like the validation of configuration plans, the conditional activation of configuration plans, or the possibility to tag plans as alternative configurations (e.g. for weekdays and weekends), or to tag them as plans that need to be activated together (transactions).

# 5	Use cases

## 5.1	Use case #1: Managing planned configurations

### 5.1.1	Description

The information architecture specified in SA5 knows only one type of data node tree on a MnS producer. This data node tree includes configuration data nodes and state data nodes. When writing to configuration data nodes on a MnS producer it is normally expected that the new values are applied to the managed system. The data node tree represents hence the best information that the management system has about the current configuration really running in the managed system and the state of the managed system.

Many management problems would benefit from the possibility of creating planned configurations that are not active yet and that can be manipulated without changing the current configuration of the managed system. When ready, the planned configurations can be activated.

Planned configurations do not contain any state data, only configuration data.

Planned configurations have meta data such as a name, a description, or the date and time of the last modification. Furthermore, the activation mode (see clause 5.3) is part of the meta data.

### 5.1.2	Potential requirements

Req-1: The 3GPP management system should support creating, reading, updating, and deleting planned configurations for managed systems.

Req-2: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configurations.

### 5.1.3	Potential solutions

#### 5.1.3.1	Potential solution #1: Data node tree

A new capability is introduced that allows to create, update, and delete planned configurations. Planned configurations can be manipulated without impacting the current configuration of the managed system. They contain only information relating to configuration data nodes. Information relating to state data nodes is omitted.

A planned configuration may include only the configuration for the part of the managed system that shall be reconfigured. For example, when a cell is to be reconfigured, the planned configuration may contain the configuration of the complete BTS supporting the cell or only the configuration of the cell itself (with required containment nodes).

A planned configuration is represented by a data node tree. The data node tree is compliant to the NRM schema that also describes the data node tree of the corresponding current configuration. The values of the configuration data nodes in the data node tree are those values with which the managed system shall be reconfigured.

Note that NRM schemas currently specified in 3GPP SA5 are OpenAPI definitions and YANG definitions.

The normal CRUD operations specified in TS 28.532 [2] are used for manipulating the data node trees of planned configurations.

Multiple alternative data node trees with planned configurations may be instantiated.

The planned configuration is merged into the current configuration.

The planned configurations and the current configuration need to be distinguished. The solution for this depends on the protocol.

HTTP/JSON

The path component "plans" is inserted between the path components "/{MnSName}/{MnSVersion}" and the URI-LDN. The "plans" resource shall exist on MnS producers supporting planned configurations. The "plans" resource is a collection resource.

An item of the collection resource "plans/<planId>" is created by MnS consumers using HTTP PUT. The following example demonstrates the creation of a new item of a "plans" collection resource.

PUT 3gpp/ProvMnS/1900/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json { "name": "NewBts10Plan", "description": "This is the plan for the new BTS 10.", "activation-mode": "BEST_EFFORT" }

After applying the creation request the collection resource for planned configurations has a new item for a planned configuration. At this point in time, it contains just the plan meta data but not the new planned configuration.

{ "plans": { "p1": { "name": "NewBts10Plan", "description": "This is the plan for the new BTS 10.", "activation-mode": "BEST_EFFORT", "last-modified": "2024-08-21T16:39:57-08:00" } } }

The data node tree of the planned configuration is described by the definition of the NRM representing the system for which is the planned configuration. The planned configuration is described by the same definition as the current data node tree. The data nodes of the planned configuration are accessed using the methods described in clause 12.1.1 of TS 28.532 [2].

The planned data node tree contains the data nodes that are to be reconfigured. Deletion of data nodes requires special consideration. Attributes and attribute fields are marked for deletion by setting their value to the literal name "null". Objects are marked for deletion by setting the value of the "attributes" container to the literal name "null". Attribute elements cannot be marked for deletion. Instead, the complete array with all items needs to be present in the planned data node tree.

The following example shows how the new item for a planned configuration created in the example above can be populated with data. In this case a new "ManagedElement" object is added to the planned configuration data tree. The empty "SubNetwork" represents an object existing in the current configuration and is required for containment purposes. The planned configuration is manipulated using HTTP PATCH with JSON Patch.

PATCH 3gpp/ProvMnS/1900/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "add", "path": "/SubNetwork=SN1", "value": { "id": "SN1" } }, { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ]

After applying the request, the planned configuration looks like following.

{ "plans": { "p1": { "name": "NewBts10Plan", "description": "This is the plan for the new BTS 10.", "activation-mode": "BEST_EFFORT", "last-modified": "2024-08-21T16:39:57-08:00", "SubNetwork": [ { "id": "SN1", "ManagedElement": [ { "id": "ME10", "attributes": { "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } ] } } }

To read the planned configuration "p1" a MnS consumer may send the following request.

GET 3gpp/ProvMnS/1900/plans/p1 HTTP/1.1 Host: example.org Accept: application/json

The MnS producer may send the following response.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2023 16:50:26 GMT Content-Type: application/json { "name": "NewBts10Plan", "description": "This is the plan for the new BTS 10.", "last-modified": "2024-08-21T16:39:57-08:00", "SubNetwork": [ { "id": "SN1", "ManagedElement": [ { "id": "ME10", "attributes": { "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } ] }

The next example shows how an attribute is marked for deletion.

PATCH 3gpp/ProvMnS/1900/plans/p1/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/attributes/attrA", "value": null } ]

After applying the PATCH request the data node looks as follows in the planned configuration.

{ "id": "XYZF1", "attributes": { "attrA": null, } }

Note that the attribute "attrA" is not deleted but only marked for deletion.

The following example shows how a planned configuration is manipulated using HTTP PATCH with JSON Merge Patch.

PATCH 3gpp/ProvMnS/1900/plans/p1/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "id": "XYZF1", "attributes": { "attrA": null } }

#### 5.1.3.2	Potential solution #2: CRUD operations

In this solution a planned configuration is not represented by a data node tree. Instead, it is represented by a set of operations that, when applied to the current configuration (data node tree), transform the current configuration into the planned configuration.

The information model for the configuration plans is as follows:

plans

<plan-id>

meta-data

name

version

activation-mode

last-modified

operations

<plan-id>

meta-data

name

version

activation-mode

last-modified

operations

The information model is manipulated using normal CRUD operations.

The optional information element "meta-data" contains information describing the plan, such as the plan name, a version, or the time the plan was updated the last time. The information element "operations" contains the operations transforming the current configuration into the planned configuration.

The activation mode allows to specify if the operations in the "operations" information element shall be activated in atomic or best effort mode.

The following table specifies the information elements of the information model for plans.

| Information element name | Description |
| --- | --- |
| name | The name of the plan. |
| version | The version of the plan |
| activation-mode | The activation mode for the operatios, either "ATOMIC" or "BEST_EFFORT" |
| last-modified | Date and time of the last plan modification. |

HTTP/JSON

The stage 2 information model is mapped to a resource model specified by OpenAPI definitions. It is ffs if the CRUD operations specified in TS 32.158 can be used for manipulating the resource model or if some changes are needed.

The operations in the "operations" property are described by JSON Patch. JSON Patch is described in RFC 6902 and TS 32.158. A JSON Patch document is JSON array of JSON objects. Each object represents a single operation. Therefore, the operations in a JSON Patch document are ordered. They shall be applied sequentially in the order they appear in the array.

The first example shows how the first operation is created in a planned configuration.

PATCH 3gpp/ProvMnS/1900/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "add", "path": "/operations/-", "value": { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } } ]

The operation in the request is stored in the planned configurations. Operations are also retrieved in that format.

{ "plans": { "p1": { "meta-data": { "name": "NewBts10Plan", "description": "This is the plan for the new BTS 10.", "activation-mode": "BEST_EFFORT", "content-type": "application/vnd.3gpp.json-patch+json", "last-modified": "2024-08-21T16:39:57-08:00", }, "operations": [ { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } } }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg".

PATCH 3gpp/ProvMnS/1900/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/operations/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

### 5.1.4	Evaluation of potential solutions

It is recommended to select potential solution #2.

## 5.2	Use case #2: Validating planned configurations

### 5.2.1	Description

Planned configurations may have inconsistencies or may not fit to the current configuration. Validation is the process to unveil and report these problems. Validation may make a subsequent activation of the planned configuration smoother. Validation is requested by MnS consumers and performed by MnS producers.

More specifically, validation may check for the following problems:

- Information model (protocol layer) errors: After applying the planned configuration to the current configuration the result must respect all the constraints defined in the information model, such as attribute properties or cardinality requirements. Note that some of these constrains may be expressed in the NRM schema, and some only in the stage 2 definitions of the NRM.

- Application layer errors: These are errors that do not relate to the information model, but to the related application (business logic). For example, an attribute value in the planned configuration may be in the value range allowed by the NRM schema but be not supported or not allowed in the current state of the application. Application layer errors are implementation specific.

The MnS consumer should be able to request validation of multiple plans using a single request. When multiple plans are validated questions about the relationship between these plans arise and the MnS consumer needs to be able to inform the MnS producer about the type of the relationship.

Furthermore, when multiple plans are validated, plan conflicts may arise and the MnS consumer needs to tell the MnS producer how to deal with these conflicts.

The MnS producer should make the validation result available to MnS consumers.

### 5.2.2	Potential requirements

Req-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to validate a planned configuration.

Req-2: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to validate multiple planned configurations with a single request.

Req-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to MnS producers the type of relationship between the plans, for the case where multiple plans are requested to be validated with a single request.

Req-4: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to MnS producers the behavior when plan conflicts are detected, for the case where multiple plans are requested to be validated with a single request.

Req-5: The 3GPP management system shall validate a planned configuration automatically when activation of the planned configuration is requested without prior validation.

Req-6: The 3GPP management system shall made the result of the validation available for retrieval.

Req-4: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously validated.

### 5.2.3	Potential solutions

#### 5.2.3.1	Potential solution #1: Dedicated job for validation

Stage 2

A dedicated information element named "plan-validation-jobs" is introduced on the MnS producer for plan validation. This information element is located at the same level as the "plans" information element. To request plan validation, a MnS consumer needs to create a validation job below "plan-validation-jobs". Plan validation is an asynchronous process. The result is made available in the information element "results".

Completed validation jobs can be deleted by MnS consumers. The MnS producer may delete validation jobs as well to limit the number of old jobs stored on the MnS producer. The deletion policy used by the MnS producer is implementation specific.

Validation jobs are created, updated, deleted and read using normal CRUD operations.

The information model for plan validation is as follows:

plan-validation-jobs

<job-id>

request

plan-ids

plans-relationship

conflict-mode

result

<plan-id>

state

time

error-details

MnS consumers should request validation prior to activation. When a planned configuration is activated without prior validation the MnS producer shall validate the planned configuration automatically.

One or more plans can be validated with a single request. The plans to be validated are specified with the "plan-ids" attribute or the "transaction-id" attribute. The possibility to validate more than one plan with a single request is optional.

When more than one plan is requested to be validated the MnS consumer needs to indicate to the MnS producer the type of relationship between the plans:

- independent plans

- ordered dependent plans

- unordered dependent plans

The specified relationship applies to all plans. It is not possible to specify more than one relationship.

Independent plans are validated individually against the current configuration. A validation result is produced for each plan. The order of the plan validation is irrelevant. Independent plans are validated as follows:

- The plan p1 is validated against the current configuration c0. The result is made available.

- The plan p2 is validated against the current configuration c0. The result is made available.

- and so forth until all plans are processed.

Ordered dependent plans are processed in the specified order as follows:

- The plan p1 is validated against the current configuration c0.

- The plan p1 is applied to a copy of the current configuration and stored internally as output configuration c1.

- The plan p2 is validated against the output configuration c1.

- The plan p2 is applied to the output configuration c1 and stored internally as output configuration c2.

- and so forth until all plans are processed.

Example for ordered dependent plans: Plan pB creates an object B under a parent object A that does not exist in the current configuration, and another plan pA creates the object A. In this case plan pA must be validated before plan pB.

Unordered dependent plans are processed as follows:

- The plans p1, p2,…, px are added to form a new plan pn.

- The plan pn is validated against the current configuration c0. The result is made available.

Example for unordered dependent plans: Plan pA creates an object A with a reference to an object B that does not exist in the current configuration and another plan pB creates object B with a reference to the object A that does not exist in the current configuration. In this case plan A and Plan B must be validated together against the current configuration.

Note that a plan may be valid when validated in the independent mode and invalid when validated in one of the dependent modes. For example, plan pA removes the leaf object B. This plan is valid. When validated together with plan pB, that adds a new object A under object B then object A is no leaf anymore and cannot be removed. Plan pA is hence invalid.

Multiple plans may exhibit conflicting configurations.

Example for a plan conflict: One of the validated plans sets an attribute to some value and another validated plan sets the same attribute to some other value.

In the unordered dependent and the independent mode, upon detection of a conflict, the Mns producer shall stop with error. In the ordered dependent mode, upon detection of a conflict, the MnS producer may stop or continue. The MnS consumer should indicate to the MnS producer its preference using the "conflict-mode" information element.

The following table specifies the information elements of the information model for plan validation.

| Information element name | Description |
| --- | --- |
| plan-ids | The ordered list of the identifiers of the plans to be validated. |
| transaction-id | or alternatively, the transaction id for the transaction to be validated. |
| plans-relationship | When more than one plan shall be validated, this attribute allows to specify the relationship between the plans to be validated. Allowed values are "INDEPENDENT", "ORDERED_DEPENDENT and "UNORDERED_DEPENDENT". |
| conflict-mode | When more than one plan shall be validated, this attribute allows to specify for the "ORDERED-DEPENDENT" mode, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. Allowed values are "STOP_ON_CONFLICT" or "CONTINUE_ON_CONFLICT". |
| state | The validation state, either "VALIDATING", "VALIDATED" or "VALIDATION_FAILED". |
| time | The start of the validation when in state "VALIDATING", or the end of the validation when in state "VALIDATED" or "VALIDATION FAILED". |
| error-details | In case validation failed, error reasons are specified here. |

HTTP/JSON

The stage 2 information model is mapped to a resource model specified by OpenAPI definitions. It is ffs if the CRUD operations specified in TS 32.158 can be used for manipulating the resource model or if some changes are needed.

The resource "plan-validation-jobs" is introduced at the same level as the resource "plans". Validation is requested by sending a PUT request to the resource "plan-validation-jobs/<jobId>".

The validation result is made available in the resource "result".

The format of the validation result depends on the format of the planned configuration. If the format is a data node tree, then it is equal to the error response format used with (3GPP) JSON Merge Patch (clause 6.6 of TS 32.158 [?]). If the format is a set of operations, then it is equal to the error response format used with (3GPP) JSON Patch (clause 6.6 of TS 32.158 [?]).

Example:

A MnS consumer may send the following message to the MnS producer for requesting validation of the plan "p1".

PUT 3gpp/ProvMnS/1900/plan-validation-jobs/pvj1 HTTP/1.1 Host: example.org Content-Type: application/json { "request": { "plan-ids": ["p1", "p2"], "plans-relationship": "ORDERED_DEPENDENT", "conflict-mode": "IGNORE_CONFLICT" } }

After applying the PUT request the resource structure on the MnS producer is as follows.

{ "plans": { "p1": {...}, "p2": {...} }, "plan-validation-jobs": { "pvj1": { "request": { "plan-ids": ["p1", "p2"], "plans-relationship": "ORDERED-DEPENDENT", "conflict-mode": "CONTINUE_ON_CONFLICT" }, "result": { "p1": { "state": "VALIDATING", "time": "2024-08-21T16:39:55-08:00" }, "p2": { "state": "VALIDATING", "time": "2024-08-21T16:39:55-08:00" } } } } }

The validation result is made available in the resource "result". Successful validation may be advertised as follows.

{ "plans": { "p1": {...}, "p2": {...}, "p3": {...} }, "plan-validation-jobs": { "pvj1": { "request": {...}, "result": { "p1": { "state": "VALIDATED", "time": "2024-08-21T16:39:57-08:00" }, "p2": { "state": "VALIDATED", "time": "2024-08-21T16:39:57-08:00" } } } }

The following examples show how this resource may look like for validation failures in the different cases.

Case 1: The planned configuration is a data node tree.

In this case the validation errors are made available in the error format used with (3GPP) JSON Merge Patch.

When an attribute is bad it may look like the following.

{ "plan-validation-jobs": { "pvj1": { "request": {...}, "result": { "p1": { "state": "VALIDATION_FAILED", "time": "2024-08-21T16:39:57-08:00" "error-details": { "type": "MODIFICATION_NOT_ALLOWED", "reason": "ATTRIBUTE_INVARIANT", "title": "The attribute field, whose value shall be replaced, is invariant.", "badAttributes": [ "#/attributes/attrA/attrB" ] } } } } } }

When an object is bad it may look like the following.

{ "plan-validation-jobs": { "pvj1": { "request": {...}, "result": { "p1": { "state": "VALIDATION_FAILED", "time": "2024-08-21T16:39:57-08:00" "error-details": { "type": "REQUEST_OBJECTS_MISMATCH", "reason": "NEW_OBJECT_PARENT_NOT_FOUND", "title": "The object, below which objects shall be created, does not exist.", "badObjects": [ "/ManagedElement=ME3/XyzFunction=XYZF1", "/ManagedElement=ME3/XyzFunction=XYZF2" ] } } } } } }

Case 2: The planned configuration is a set of operations.

In this case the validation errors are made available in the error format used with (3GPP) JSON Patch.

When an attribute is bad the "error-details" property may look like the following.

{ "type": "MODIFICATION_NOT_ALLOWED", "reason": "ATTRIBUTE_INVARIANT", "title": "The attribute, whose value is requested to be replaced, is invariant.", "badOp": "/0" }

When an object is bad the "error-details" property may look like the following.

{ "type": "VALIDATION_ERROR", "reason": "NEW_OBJECT_CLASS_NAME_INVALID", "title": "The class of the new object to be created is invalid.", "badOp": "/1" }

For reporting more than one problem the "otherProblems" property is used.

{ "type": "VALIDATION_ERROR", "reason": "NEW_OBJECT_CLASS_NAME_INVALID", "title": "The class of the new object to be created is invalid.", "badOp": "/1", "otherProblems": [ { "type": "REQUEST_OBJECTS_MISMATCH", "reason": "NEW_OBJECTS_PARENT_NOT_FOUND", "title": "The parent object of the new object to be created does not exist." "badOp": "/2" } ] }

To find out if validation results are available for the plan "p1", a MnS consumer may send the following:

GET 3gpp/ProvMnS/1900/plan-validation-jobs?filter=/*/p1 HTTP/1.1 Host: example.org Accept: application/json

### 5.2.4	Evaluation of potential solutions

It is recommended to select potential solution #1.

## 5.3	Use case #3: Activating planned configurations

### 5.3.1	Description

Activation is the process of writing a planned configuration into the managed system, so that the new configuration is actually used by the system.

Activation may be performed in two modes:

- Best effort mode: The updates that fail simply fail without impact on the successful updates.

- Atomic mode: The complete planned configuration shall be applied to the system. If only one update fails, all updates shall not be performed.

Activation of a new configuration might come with impairments and disruptions of the provided service. Usually, it is possible to tradeoff between the time needed for the activation and the service impairments due to the activation: a short activation time results in more service impact than a longer activation time and vice versa. Therefore, a MnS consumer should be able to indicate to the MnS producer if his preference is fast execution or minimization of service impairments.

The MnS producer should make the validation progress and the validation result available to MnS consumers.

Instead of referencing an existing planned configuration, the configuration to be activated may be provided also together with the activation request.

### 5.3.2	Potential requirements

Req-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to activate a planned configuration.

Req-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of the activation process.

Req-3: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the result of the activation process.

Req-4: The 3GPP management system should support a capability allowing a MnS consumer to select atomic activation or best effort activation.

Req-5: The 3GPP management system should support a capability allowing a MnS consumer to select if activation should be done with the least possible service impact or within the shortest possible time.

Req-6: The 3GPP management system should allow multiple non concurrent activations of the same plan.

Req-7: The 3GPP management system should store the results of plan activations for later retrieval.

### 5.3.3	Potential solutions

#### 5.3.3.1	Potential solution #1: Copy plan into current configuration

The planned configuration is copied into the current configuration.

A MnS consumer can discover successful updates and failed updates by comparing the planned configuration with the current configuration.

Note that this solution works only when the format of the planned configuration is a data node tree.

HTTP/JSON

The "copy" operation of JSON Patch can be used to copy the planned configuration into the activated configuration.

#### 5.3.3.1	Potential solution #2: Dedicated job for activation

A dedicated information element named "plan-activation-jobs" is introduced on the MnS producer for plan activation. This information element is located at the same level as the "plans" information element. To request plan activation, a MnS consumer needs to create an activation job below "plan-activation-jobs". Plan activation is an asynchronous process. The result is made available in the information element "results".

Completed activation jobs can be deleted by MnS consumers. The MnS producer may delete activation jobs as well to limit the number of old jobs stored on the MnS producer. The deletion policy used by the MnS producer is implementation specific.

Activation jobs are created, updated, deleted and read using normal CRUD operations.

The information model for plan activation is as follows:

plan-activation-jobs

<job-id>

request

plan-ids

transaction-id

operations

plans-relationship

conflict-mode

execution-mode

enable-fallback

progress

result

<plan-id>

state

time

error-details

The following table specifies the information elements of the information model for plan activation.

| Information element name | Description |
| --- | --- |
| plan-ids | The ordered list of the identifiers of the plans to be activated. |
| transaction-id | or, alternatively, the transaction id for the transaction to be validated. |
| operations | or,alternatively, the operations to be activated, when no plan was created earlier for them. |
| plans-relationship | When more than one plan shall be validated, this attribute allows to specify the relationship between the plans to be validated. Allowed values are "INDEPENDENT", "ORDERED_DEPENDENT and "UNORDERED_DEPENDENT". |
| conflict-mode | When more than one plan shall be validated, this attribute allows to specify for the "ORDERED-DEPENDENT" mode, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. Allowed values are "STOP_ON_CONFLICT" or "CONTINUE_ON_CONFLICT". |
| execution-mode | The execution mode, either "LEAST_SERVICE_IMPACT" or "SHORTEST_TIME". |
| enable-fallback | This Boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. |
| state | The validation state, either "ACTIVATING", "ACTIVATED" or "ACTIVATION_FAILED". |
| time | The start of the activation when in state "ACTIVATING", or the end of the validation when in state "ACTIVATED" or "ACTIVATION_FAILED". |
| error-details | In case validation failed, error reasons are specified here. |

HTTP/JSON

The task resource "configuration-activation-jobs" is introduced for activation. This resource is located in the resource tree at the same level as the "plans" resource.

To start a job, the MnS producer shall send a PUT request to the resource "configuration-activation-jobs/<jobId>". The plans to be activated are identified with the "plan-ids" property. Alternatively, the plans to be activated can be identified with a transaction identifier. Numerous parameters for controlling the execution of the activation are available.

In case the plan to be activated has not been created yet under the "plans" resource, the "operations" property in the request shall be present (instead of the "plan-ids" property) and specify the plan to be activated.

The MnS producer publishes the progress of a job in the resource "progress". The result is made available in the resource "results".

Activation errors are reported using the same format as validation errors.

Example:

A MnS consumer may send the following message to the MnS producer for requesting activation of the plan "p1".

PUT 3gpp/ProvMnS/1900/configuration-activation-jobs/caj1 HTTP/1.1 Host: example.org Content-Type: application/json { "request": { "plan-ids": ["p1", "p2"], "plans-relationship": "INDEPENDENT", "execution-mode": "LEAST_SERVICE_IMPACT", "enable-fallback": true } }

The activation progress is made available in the resource "progress". A GET on this resource will return the progress of the job. When finished, the MnS consumer should consult the resource "results", where the result is available.

The resources on the MnS producer may look as follows.

{ "plans": { "p1": {}, "p2": {}, "p3": {} }, "configuration-activation-jobs": { "caj1": { "request": { "plan-ids": ["p1", "p2"], "plans-relationship": "INDEPENDENT", "execution-mode": "LEAST_SERVICE_IMPACT", "enable-fallback": true }, "progress": {}, "result": { "p1": { "state": "ACTIVATED", "time": "2024-08-21T16:39:57-08:00" }, "p2": { "state": "ACTIVATED", "time": "2024-08-21T16:39:57-08:00" } } } } }

A MnS consumer can also include the plan to be activated in the activation request (rather than referring to an already existing plan). The plan is contained in the "operations" property.

PUT 3gpp/ProvMnS/1900/configuration-activation-jobs/px HTTP/1.1 Host: example.org Content-Type: application/json { "request": { "operations": [], "execution-mode": "LEAST_SERVICE_IMPACT" } }

The resources on the MnS producer may look  as follows.

{ "plans": { "p1": {}, "p2": {}, "p3": {} }, "configuration-activation-jobs": { "caj1": { "request": { "operations": [], "execution-mode": "LEAST_SERVICE_IMPACT" }, "progress": {}, "result": {} } } }

### 5.3.4	Evaluation of potential solutions

Because potential solution #2 is recommended for use case 1, also here potential solution #2 needs to be selected.

## 5.4	Use case #4: Managing transactions

### 5.4.1	Description

A transaction is a set of plans that need to be handled (validated, activated) together. For example, one plan may create a new cell and another plan may start the collection of measurements for the new cell. A transaction may also contain the control parameters specified for validation and activation. When specified for a transaction they can be omitted in the validation and activation requests.

The 3GPP management should enable to manage transactions.

### 5.4.2	Potential requirements

Req-1: The 3GPP management system should allow MnS consumers to group plans into transactions.

Req-2: The 3GPP management system should allow MnS consumers to specify validation and activation control parameters for the transaction.

Req-3: The 3GPP management system should allow building transactions from transactions.

Req-4: The 3GPP management system should allow MnS consumers to validate and activate the plans pertaining to a transaction.

### 5.4.3	Potential solutions

#### 5.4.3.1	Potential solution #1: Grouping plans into transactions

A transaction is described with a set of properties. A MnS consumer can create the description of a transaction on MnS producers.

The main property of a transaction is the "plan-ids" property that identifies the planned configurations pertaining to the transaction. The control parameters available for validation and activation ("plan-relationship", "conflict-mode", "fallback") can be specified also for a transaction. If done, they do not need to be specified in the validation and activation request. In addition, a parameter must be provided for specifying if all plans need to validate or activate successfully or if best effort mode is acceptable.

It is possible to build transactions with transactions (instead of plans).

A transaction can be validated using the normal validation mechanism (specified in clause 5.2) and activated using the normal activation mechanism (specified in clause 5.3). To do so, the "transaction-id" property needs to be specified instead of the "plan-ids" property or the "operations" property.

HTTP/JSON

The collection resource "configuration-transactions" is introduced at the same level as the resource "plans". An item of this collection resource describes a transaction.

Editor's note: It is ffs how to apply activation control parameters to transactions. For example, is it possible that a planned configuration is activated in best-effort mode whereas another one in all-or-nothing mode?

Example:

The following example demonstrates how a new transaction is created. The planned configurations "p1" and "p2" are part of this transaction. Both need to be activated successfully to create the URLLC slice.

POST 3gpp/ProvMnS/1900/configuration-transactions HTTP/1.1 Host: example.org Content-Type: application/json { "plan-ids": [p1, p7], "plan-relationship": "ORDERED_DEPENDENT", "conflict-mode": "CONTINUE_ON_CONFLICT", "enable-fallback": true "label": "urllcSlice", "description": "This transaction creates a URLLC slice." }

After applying the POST request, the resource structure on the MnS producer looks as follows.

{ "configuration-transactions": { "t1": { "plan-ids": ["p1", "p7"], "plan-relationship": "ORDERED_DEPENDENT", "conflict-mode": "CONTINUE_ON_CONFLICT", "enable-fallback": true "label": "urllcSlice", "description": "This transaction creates a URLLC slice." } } }

For requesting activation of the transaction "t1" a MnS consumer may send the following message to the MnS producer.

PUT 3gpp/ProvMnS/1900/configuration-activation-jobs/caj1 HTTP/1.1 Host: example.org Content-Type: application/json { "request": { "transaction-id": "t1" } }

#### 5.4.3.1	Potential solution #2: Creating new plans for transactions

In this solution a new plan is created for a transaction. This plan is built from the other existing plans that form the set of plans pertaining to the transaction.

### 5.4.4	Evaluation of potential solutions

It is recommended to select potential solution #1.

## 5.5	Use case #5: Fallback

### 5.5.1	Description

Sometimes activating a plan (i.e. configuring the network according to the plan) has unintended consequences. The operator may be monitoring the performance of the network. If the activation of the plan results in degraded performance or even alarms, the operator may want to return the network to its previous configuration. The operator may want to "FALLBACK" to the previous configuration.

### 5.5.2	Potential requirements

Req-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to undo the configuration updates that are the result of the previous plan activation-job. A fallback capability should be available for this purpose. Fallback should always operate according to the version of the plan that was last activated. Fallback should apply to activation jobs both if they were activated in atomic mode or in best-effort mode.

### 5.5.3	Potential solutions

#### 5.5.3.1	Potential solution #1: Fallback facility

##### 5.5.3.1.1	Introduction

The producer shall implement a fallback facility. It should provide status, progress and result information.

Whether the producer keeps track of the complete configuration immediately before activation or it keeps a list of configuration updates included in the plan or whether it uses some other internal mechanism to execute the "fallback" is implementation specific.

##### 5.5.3.1.2	Description

The producer needs to be prepared for a fallback after activation.

The producer shall automatically create a fallback-plan that reverses the changes included in the original plan. The consumer may read or delete the fallback-plan, but it may not change its content. The fallback-plan shall include a reference to the activation-job and the original plan and the timestamp of the creation of the fallback-plan.

The purpose of the fallback-plan is to facilitate the fallback and allow the consumer to review the operations included in the fallback before executing it. For each plan only one fallback-plan should be maintained by the producer. When a plan is activated any previously created fallback-plan for that planned configuration shall be deleted by the producer.

This preparation consumes resources. However, fallback might not be needed for every plan and every activation. To allow the producer to save resources, the producer shall only prepare for fallback if the activation-job explicitly requests this. Activation jobs shall have an input parameter fallbackEnabled (boolean, default=FALSE) that indicates to the producer that it should be prepared for a fallback operation for that specific activation-job. A fallback request for an activation-job without the fallbackEnabled=TRUE parameter shall be rejected.

The solution shall reuse the information elements defined for managing and activating configuration plans. The meaning and the behavior of the “enable-fallback” information element defined by plan activation shall be modified to define the fallback capability.

No new information elements or operations are defined.

A fallback shall be executed by activating the fallback-plan. For such a (fallback) activation no fallback is possible or prepared. Activation of a fallback plan shall always be done in ATOMIC mode.

PUT 3gpp/ProvMnS/1900/configuration-activation-jobs/fallback-job1 HTTP/1.1 Host: example.org Content-Type: application/json { "request": { "planId": ["fallback-p1"] } }

A fallback shall be rejected if the

- original plan is in the process of being activated. It shall be avoided to execute multiple parallel actions on a specific plan.

- if an activation of the original plan did not include the enable-fallback = TRUE parameter.

### 5.5.4	Evaluation of potential solutions

It is recommended to select potential solution #1.

## 5.6	Use case #6: Conditional activation of planned configurations

### 5.6.1	Description

A planned configuration may have to be activated automatically when certain conditions are met. For example, performance metric collection jobs or assurance functions shall be created when metrics that are always collected cross some thresholds.

### 5.6.2	Potential requirements

Req-1: The 3GPP management system should allow MnS consumers to create conditions on MnS producers that specify when planned configurations are activated.

### 5.6.3	Potential solutions

#### 5.6.3.1	Potential solution #1: Dedicated job

MnS consumers can specify conditions on MnS producers. Planned configurations are automatically activated when the conditions evaluate to true.

Conditions shall be specified using the exact same mechanism as the "ConditionMonitor", i.e. Jex for the HTTP/JSON solution and XPath for the NETCONF/YANG solution. Note though that the "ConditionMonitor" is designed to switch on and off a process: When the condition evaluates to false the process is (switched) off, when the condition evaluates to true, the process is (switched) on. For the present use case the condition needs to trigger an action. The triggering condition is therefore the transition of the condition evaluation result from false to true or true to false. The triggering condition needs to be specified together with the condition expression.

Furthermore, the case where the condition expression toggles between true and false needs consideration. For example, a condition may become inactive when it triggered once the activation of a plan. This work is left for the normative phase.

The information model for conditional plan activation is as follows:

conditional-plan-activation-jobs

<job-id>

request

plan-ids

here are the activation controls

condition-expression

conflict-mode

results

<plan-id>

state

time

error-details

The following table specifies the attributes of the validation job.

| Attribute name | Description |
| --- | --- |
| plan-ids | The identifiers of the plans to be activated. |
| condition-expression | The condition expression. |
| trigger-condition | The trigger condition, either "FALSE_TO_TRUE" or "TRUE_TO_FALSE". |

HTTP/JSON

The "configuration-conditions" collection resource is introduced. Conditions are specified using Jex. When the evaluation result of the condition transits from false to true the MnS producer shall automatically create a configuration activation job for the planned configurations specified by "plan-ids".

Example:

POST 3gpp/ProvMnS/1900/conditional-plan-activation-jobs HTTP/1.1 Host: example.org Content-Type: application/json { "plan-ids": ["p1", "p7"], "condition-expression": "some Jex expression", "trigger-condition": "FALSE_TO_TRUE" }

After applying the POST request, the resource structure on the MnS producer looks as follows.

{ "conditional-plan-activation-jobs": { "cpaj1": { "plan-ids": ["p1", "p7"], "condition-expression": "some Jex expression", "trigger-condition": "FALSE_TO_TRUE" } } }

### 5.6.4	Evaluation of potential solutions

It is recommended to select potential solution #1.

# 6	Conclusion and recommendation

This study investigates methods for managing planned configurations. Five use cases are described. For each use case there are one or two solution proposals and a clear recommendation on which solution proposal should be selected.

It is recommended to start normative work on planned configurations and to specify solutions for the six use cases described in this document,

3GPP

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 |  |  |  |  |  | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5-242085 |  |  |  | Add skeleton | 0.1.0 |
| 2024-04 | SA5#154 | S5-242084 |  |  |  | Add key issue on Planned configurations | 0.1.0 |
| 2024-05 | SA5#155 | S5-243216 |  |  |  | Clarify potential solution 1 for Managing planned configurations | 0.2.0 |
| 2024-05 | SA5#155 | S5-243217 |  |  |  | Add potential solution 2 for Managing planned configurations | 0.2.0 |
| 2024-05 | SA5#155 | S5-243218 |  |  |  | Add use case on Validating planned configurations | 0.2.0 |
| 2024-05 | SA5#155 | S5-243219 |  |  |  | Add use case on Activating planned configurations | 0.2.0 |
| 2024-05 | SA5#155 | S5-243220 |  |  |  | Add use case on Managing transactions | 0.2.0 |
| 2024-05 | SA5#155 | S5-243221 |  |  |  | Add use case on Managing alternative planned configurations | 0.2.0 |
| 2024-05 | SA5#155 | S5-243222 |  |  |  | Add use case on Conditional activation of planned configurations | 0.2.0 |
| 2024-08 | SA5#156 | S5-244803 |  |  |  | Improve description of Managing planned configurations | 0.3.0 |
| 2024-08 | SA5#156 | S5-244804 |  |  |  | Fallback for planned config | 0.3.0 |
| 2024-08 | SA5#156 | S5-244805 |  |  |  | Improve description of Validating planned configurations | 0.3.0 |
| 2024-08 | SA5#156 | S5-244806 |  |  |  | Improve description of Activating planned configurations | 0.3.0 |
| 2024-08 | SA5#156 | S5-244807 |  |  |  | Improve description of Managing transactions | 0.3.0 |
| 2024-08 | SA5#156 | S5-244808 |  |  |  | Improve description of Managing alternative planned configurations | 0.3.0 |
| 2024-08 | SA5#156 | S5-244809 |  |  |  | Improve description of Conditional activation of planned configurations | 0.3.0 |
| 2024-08 | SA5#156 | S5-244810 |  |  |  | Add clause on Scope and clause on Concept and Background | 0.3.0 |
| 2024-08 | SA5#156 | S5-244811 |  |  |  | Add clause on Conclusions and recommendations | 0.3.0 |