| 3GPP TR 28.866 V1.1.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management Data Analytics (MDA) – Phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Concept and background	9

5	Use cases	9

5.1	Energy efficiency analytics	9

5.1.1	Use case 1: Energy Saving based on throughput requirements	9

5.1.1.1	Description	9

5.1.1.2	Potential requirements	10

5.1.1.3	Potential solutions	10

5.1.1.4	Evaluation of solutions	11

5.1.2	Use case 2: Extension of Cell Energy Saving analytics	11

5.1.2.1	Description	11

5.1.2.2	Potential requirements	12

5.1.2.3	Potential solutions	12

5.1.2.4	Evaluation of solution	12

5.2	End-to-End performance analytics including Edge computing domain	13

5.2.1	Use case 1: Edge computing performance analytics	13

5.2.1.1	Description	13

5.2.1.2	Potential requirements	13

5.2.1.3	Potential solutions	13

5.2.1.4	Evaluation of solutions	14

5.2.2	Use case 2: Edge application deployment location analytics	14

5.2.2.1 Description	14

5.2.2.2	Potential Requirements	14

5.2.2.3	Potential solutions	14

5.2.2.4	Evaluation of solutions	15

5.2.3	Use case 3: Edge network topology mapping	15

5.2.3.1	Description	15

5.2.3.2	Potential Requirements	16

5.2.3.3	Potential solutions	16

5.3	Data correlation analytics	16

5.3.1	Description	16

5.3.2	Use case 1: Measurement data correlation analytics for ML training	17

5.3.2.1	Description	17

5.3.2.2	Potential requirements	17

5.3.2.3	Potential solutions	17

5.3.2.3.1	Possible solution for measurement data correlation analytics for ML training	17

5.3.2.4	Evaluation of solutions	17

5.3.3	Use case 2: Measurement data correlation analytics for threshold assessment and adjustment	17

5.3.3.1	Description	17

5.3.3.2	Potential requirement	18

5.3.3.3	Potential solutions	18

5.3.3.4	Evaluation of solutions	18

5.3.4 	Use case 3: Multi-domain resource optimization (MARO)	18

5.3.4.1	Description	18

5.3.4.2	Potential requirements	18

5.3.4.3	Potential solutions	18

5.3.4.4	Evaluation of solutions	19

5.3.5	Use case 4: Handover and service data correlation analytics	19

5.3.5.1	Description	19

5.3.5.2	Potential requirements	19

5.3.5.3	Potential solutions	19

5.3.5.3.1	Possible solution for handover and service data correlation analytics	19

5.3.5.4	Evaluation of solutions	20

5.3.6	Use case 5: Correlation analytics for NF Scaling and dimensioning	20

5.3.6.1	Description	20

5.3.6.2	Potential requirements	20

5.3.6.3	Potential solution #1	20

5.3.6.4	Evaluation of solutions	20

5.4	ATSSS performance analytics	20

5.4.1	Use case 1: Traffic steering analysis	20

5.4.1.1	Description	20

5.4.1.2	Potential requirements	21

5.4.1.3	Potential solutions	21

5.4.1.3.1	Solution description	21

5.4.1.3.2	Data required	21

5.4.1.3.3	Analytics Report	21

5.4.1.4	Evaluation of solutions	22

5.5	UE throughput analytics	22

5.5.1	Use case 1: UE throughput analysis	22

5.5.1.1	Description	22

5.5.1.2	Potential requirements	23

5.5.1.3	Potential solutions	23

5.5.1.4	Evaluation of solutions	24

5.5.2	Use case 2: Network congestion analytics based on UE throughput	24

5.5.2.1 Description	24

5.5.2.2 Requirements	24

5.5.2.3	Potential solutions	24

5.5.2.4	Evaluation of solutions	26

5.6	Fault management related analytics and alarm prediction	26

5.6.1	Use case 1: Providing threshold statistics information for fault management	26

5.6.1.1	Description	26

5.6.1.2	Potential Requirements	26

5.6.1.3	Potential Solutions	27

5.6.1.4	Evaluation of solutions	27

5.6.2	Use case 2: Enhancing failure prediction	27

5.6.2.1	Description	27

5.6.2.2	Potential Requirements	27

5.6.2.3	Potential solutions	28

5.6.2.4	Evaluation of solutions	28

5.6.3	Use case 3: Enhancing service failure recovery	28

5.6.3.1	Description	28

5.6.3.2	Potential requirements	28

5.6.3.3	Potential solutions	29

5.6.3.4	Evaluation of solutions	29

5.6.4	Use case 5: NF failure evaluation	29

5.6.4.1	Description	29

5.6.4.2	Potential Requirements	29

5.6.4.3	Potential solutions	29

5.6.4.4	Evaluation of solutions	30

5.7	Software upgrade validation	30

5.7.1	Use case 1: Software upgrade validation	30

5.7.1.1	Description	30

5.7.1.2	Potential requirements	30

5.7.1.3	Potential solutions	30

5.7.1.4	Evaluation of solutions	31

5.8	Control plane congestion analytics	31

5.8.1	Use case 1: Enhancing control plane congestion analysis	31

5.8.1.1	Description	31

5.8.1.2	Potential requirements	31

5.8.1.3	Potential solutions	31

5.8.1.4	Evaluation of solutions	32

6	Conclusions and recommendations	32

6.1	Energy efficiency analytics	32

6.2	End-to-End performance analytics including Edge computing domain	32

6.3	Data correlation analytics	32

6.4	ATSSS performance analytics	33

6.5	UE throughput analytics	33

6.6	Fault management related analytics and alarm prediction	33

6.8	Control plane congestion analytics	33

Annex <A> (informative): Change history	34

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document investigates new capabilities and improvements to the Management Data Analytics (MDA) which are defined in TS 28.104 [2].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[8]	3GPP TS 28.538: " Management and orchestration; Edge Computing Management (ECM)".

[9]	3GPP TS 37.817: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancement for data collection for NR and EN-DC".

[10]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[11]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[12]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[13]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[14]	3GPP TS 28.622: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[15]	3GPP TS 28.558: "Management and orchestration; UE level measurements for 5G system".

[16]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally

3.2	Symbols

Void.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MDA	Management Data Analytics

MDA MnS	MDA Management service

MDAS	Management Data Analytics Service

4	Concept and background

Management Data Analytics (MDA) is defined in TS 28.104 [2].

MDA provides a capability of processing and analysing data related to network and service events and status to provide analytics output, and may also provide recommendations to enable necessary actions for network and service operations.

5	Use cases

5.1	Energy efficiency analytics

5.1.1	Use case 1: Energy Saving based on throughput requirements

5.1.1.1	Description

Directional beams are formed using multiple antenna elements and directional beams are used in both common channels for initial access and in RRC_CONNECTED state. Common signals/channels used for UE initial access are transmitted in synchronization signal block (SSB). Each SSB associated with a beam is designed at different directions to cover the intended coverage area of a cell as depicted in Figure 5.1.1.1-1.

| Figure 5.1.1.1-1: Illustration of SSB beams covering full cell coverage area. | Figure 5.1.1.1-2: Illustration of reduced number of SSB beams covering only hotspot area. |
| --- | --- |

To reduce energy consumption, SSB beams which are not required based on traffic demand can be modified or deactivated. For example, as depicted in Figure 5.1.1.1-2, if the expected traffic (hotspot area) can be covered by 3 beams, then the remaining beams can be deactivated. This energy saving techniques depend on the accuracy of where the expected traffic demand comes from geographically, so that it can be correlated with SSB beam coverage areas.

This use case considers throughput as main criteria to define the traffic load. It is desirable to use MDA analytics to get throughput prediction for traffic load at the granular level of geographical coordinate.  This information can be then used to reduce the coverage of the beam resulting in energy savings.

5.1.1.2	Potential requirements

REQ-TLM-FUN-01: MDA capability for energy saving analysis shall include providing the predicted throughput requirements for the area which is the candidate for the energy saving mechanism.

5.1.1.3	Potential solutions

The enabling data for MDAAssistedEnergySaving.EnergySavingAnalysis is defined in TS 28.104 [2]. This solution proposes additional performance metrics into this table to enrich UE throughput and location information so that throughput metrics can be processed with UE location to generate throughput requirement, at the granular level of geographical coordinate, for more accurate energy saving decisions.

| Data category | Description | References |
| --- | --- | --- |
| MDT Reports | PDCP Data Volume per UE | PDCP SDU Data volume measurement separately for DL and UL, per DRB per UE by gNB of M4 measurements in TS 32.422 [12] and TS 32.423 [10]. |
|  | Average UE throughput measurement | Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB of M5 measurements in TS 32.422 [12] and TS 32.423 [10]. |
|  | Packet Delay measurement | Packet delay measurement, separately for DL and UL, per DRB per UE by gNB of M6 measurements in TS 32.422 [12] and TS 32.423 [10]. |
| UE location reports | UE location information provided by the LMF services. | The UE location information provided by LMF via service-based interface (see TS 23.273 [7]). The mapping between the UE location and the SSB coverage need to be decided and up to the producer implementation. |

The analytics output for MDAAssistedEnergySaving.EnergySavingAnalysis is defined in TS 28.104 [2]. This solution proposes that throughput information at the granular level of geographical coordinate is added to the analytics output. Additionally the recommendation to activate the SSB is also provided based on the throughput requirements at the granular level of geographical coordinate. This recommendation can be proposed to gNB which can decide to implement the recommendation based on local policies.

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| rANenergySavingRecommendationSSB | For ES on NR cell SSBs. It may contain a set of: - Recommended NRCell SSB to be activated/de-activated. - Recommended candidate cell SSBs with precedence for taking over the traffic of the ES-Cell. - The load threshold to activate/de-activate the NR Cell SSB. | M | type: EsRecommendationOnNRcellSSB multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

5.1.1.4	Evaluation of solutions

The solution proposed in clause 5.1.1.3 satisfies the requirements and this solution is feasible for normative work.

5.1.2	Use case 2: Extension of Cell Energy Saving analytics

5.1.2.1	Description

Energy efficiency can be achieved by moving certain cells to energy saving state (see TS 28.104 [2]). When moved to energy saving state, such cells or beams will have different impacts to the neighbour cells or beams depending on how much the coverage of those cells overlap

RAN3 has a use case to configure unified mapping rule for a group of cells. This implies that a mapping rule related to EE are same across a group of cells for a given area. It is desirable to use MDA analytics to get a group of cells that should be managed together for energy saving, computed, or learned depending on their proximity and likely impact on one another.

Then, for the cells in the group, it is desirable to use MDA analytics to get the sequence in which the beams can be activated/de-activated in order to maximize the throughput of the users while minimizing the energy consumption. Note that the sequence in which activation of the beams happen activated are necessarily not in the same sequence as deactivation. The sequence of activation/de-activation of the beams do matter in real time scenarios. For instance, there are 4 beams in a cell serving the same geo area at different directions. To maximize the energy efficiency, if it has been decided to switch two beams off for energy saving purposes, in general the sequence of the deactivation of the beams can be random or all the identified beams can be deactivated together. In both these cases, the sharing of load among the other beams and the handover performances between the beams would not be optimal. If all the identified beams are deactivated at the same time, the remaining beams would have to share the load of the beams those are deactivated at the same time. This would cause call drops and throughput impacts. Alternatively, if a random sequence is followed, the impact and the performance of the beams sharing the load of the deactivated beams would be random as well. To avoid this scenario, it is recommended to follow a sequence in which the beams are activated/deactivated.

RAN3 has identified the need for configuring a unified mapping rule among gNBs. These set of gNBs to which unified mapping rule to be applied form a group of cells that need to be managed together for energy efficiency purposes as shown in the below figure.

Figure 5.1.2.1-1: Example of Energy Cost exchange between gNBs.

The enabling data of this use case may include Mobility Management related measurements (clause 5.1.1.6 from TS 28.552 [5]) including intra and inter gNB handovers. These also include handover measurements per beam (in clauses 5.1.1.6.1.12 and 5.1.1.6.1.13 from TS 28.552 [5]) and intra-NR cell SSB beam switch measurements (in clause 5.1.1.21 from TS 28.552 [5]).

5.1.2.2	Potential requirements

REQ-TLM-FUN-01: MDA capability for energy saving analysis shall include providing, for a given area, the recommended grouping of cells that should be managed together for energy saving.

REQ-TLM-FUN-02: MDA capability for energy saving analysis shall include providing the recommended sequence in which the beams are switched off/on in order to maximize the throughput of the users while minimizing the energy consumption.

5.1.2.3	Potential solutions

This potential solution proposes to extend the existing energy saving analytics for RAN to include the datatype that may contain the groups and the cells in each of the groups. RAN3 has already identified the need for the groups of cells that can be managed together and this recommendation of energy saving analytics proposes the optimal number of groups and the cells in each of those groups. The cells belonging to each group can be identified by the datatype NCGI (clause 5.3.212 from [13]).

The potential solution also proposes to extend the cell energy saving analytics for gNBto indicate the sequence in which the beams may be activated or de-activated. This recommendation can be proposed to gNB which can decide to implement the recommendation based on the local policies. The beams can be identified with the “beamIndex” (clause 4.3.40 from [13]).

5.1.2.4	Evaluation of solution

The solution proposed in clause 5.1.2.3 satisfies the requirements and this solution is feasible for normative work.

5.2	End-to-End performance analytics including Edge computing domain

5.2.1	Use case 1: Edge computing performance analytics

5.2.1.1	Description

For edge applications such as remote control and automation vehicles, the end-to-end performance (e.g. latency) to an end user is contributed by both the network side and the Edge Computing side. We can guarantee the end-to-end latency between UE and EAS to satisfy the consumer requirements for guarantee user service experience. In R18, the MDA capability include E2E latency analysis, including CN latency analysis and RAN latency analysis, but not include latency between UE and EAS.

It is desirable that the end-to-end latency can be predicted by MDA. MDA consumer sends the request for end-to-end latency analytics to MDA producer, MDA producer correlates and analyses multi-fold data (such as EDN NF (e.g. EAS, EES) performance measurements, 5GC NF measurement and alarm related to edge computing performance, together with the geographical and configuration data of edge computing). The MDAS producer provides the analytics report that include predicting end-to-end latency for edge application. The MDAS producer may provide the recommendations that may be for example to adjust the configuration data of EDN NF.

Note: if the UPF and DN are not co-located, the input of MDA can require consultation with SA6.

5.2.1.2	Potential requirements

REQ-EDGE-CON-1:	MDA capability for edge computing performance analytics should provide the prediction related to E2E latency between UE and EAS.

5.2.1.3	Potential solutions

The solution is to introduce the enabling data for edge computing performance analytics, the enabling data are provided in table 5.2.1.3-1

Table 5.2.1.3-1: Enabling data for edge computing performance analytics

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Packet Delay on air-interface | Average delay DL air-interface(clause 5.1.1.1.1 TS 28.552 [5]);(Average delay UL on over-the-air interface(clause 5.1.1.1.3 TS 28.552 [5]) |
|  | Round-trip GTP Data Packet Delay on N3 interface | Round-trip delay on a N3 interface on PSA UPF(see clause 5.4.1.9 in TS 28.552 [5]) |
|  | GTP packets delay in UPF | GTP packets delay within the PSA UPF(see clause 5.4.5 in TS 28.552[5]). |
|  | Round-trip Packet Delay | Round-trip packet delay between PSA UPF and NG-RAN (see clause 5.4.8 in TS 28.552 [5]). |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (clause 6.3.1.2 in TS 28.554 [6]); Integrated uplink delay in RAN (see clause 6.3.1.7 in TS 28.554 [6]). |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [7]). |
| EAS Service location | It defines the location where the EAS service should be available | requiredEASservingLocation（see clause 7.3.3.6 in TS 28.538 [8] |

The solution is also to introduce a data type for edge computing performance analytics, called edgeComputingPerformanceAnalysis. The data type can be the contents of the analytics report that representing the MDA for edge computing performance correlation analytics

Attributes may configure the analytics context in MDARequest. The context may include attributes.

- An attribute may indicate the EAS identifier that identifies a particular application (e.g. Video, Game, … etc.).

- An attribute may indicate the latency of UE and EAS.

5.2.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

5.2.2	Use case 2: Edge application deployment location analytics

5.2.2.1 Description

With the rapid development of edge computing, an increasing number of applications require deployment at the network edge to meet the demands for low latency, high bandwidth, and localized data processing. However, selecting the optimal edge deployment location is a complex issue that involves considering multiple factors such as QoS  requirements, network resource information and UE distribution. Currently, edge application deployment locations primarily rely on operator experience and manual configuration, lacking automated and intelligent decision support. This can lead to deployment location that fail to fully leverage the advantages of edge computing.

The use case aims to utilize MDA analytics capabilities to provide data-driven decision support for edge application deployment location selection. MDA can collect and analyse related data (including performance measurement from RAN(e.g. UE throughput, latency, coverage), EDNs connection information, available EDNs virtual resource information, UE location information, QoE data) to determine the deployment location that satisfies user experience.

5.2.2.2	Potential Requirements

REQ-EDGE-APP-LOCATION-01: MDA capability for edge application deployment location analysis should be able to recommend the deployment location that meets user experience.

5.2.2.3	Potential solutions

The solution is to introduce the enabling data for edge application deployment location analytics, the enable data are provided in table 5.2.2.3-1.

Table 5.2.2.3-1: Enabling data for edge application deployment location analytics

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Packet Delay on air-interface | Average delay DL air-interface (clause 5.1.1.1.1 TS 28.552 [5]); (Average delay UL on over-the-air interface (clause 5.1.1.1.3 TS 28.552 [5]) |
|  | Round-trip Packet Delay | Round-trip packet delay between PSA UPF and NG-RAN (clause 5.4.8 TS 28.552 [5]). |
|  | Registered subscribers measurement | Registered subscribers measurement as defined in clause 5.2.1 in Ts 28.552[5] |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (clause 6.3.1.2 in TS 28.554 [6]); Integrated uplink delay in RAN (clause 6.3.1.7 in TS 28.554 [6]). |
|  | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance as defined in clause 6.3.2 in TS 28.554 [5]; Downstream throughput for Single Network Slice Instance as defined in clause 6.3.3 in TS 28.554 [5]. |
|  | RAN UE Throughput | RAN UE Throughput as defined in clause 6.3.6 in TS 28.554 [5]. |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface as defined in clause 6.3.4 in TS 28.554 [5]; Downstream Throughput at N3 interface as defined in clause 6.3.5 in TS 28.554 [5]. |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [7]). |
| QoE Data | The QoE data of the different services | QoE data (TS 26.247 [22] and TS 26.114 [23] can be acquired through the procedures defined in TS 28.405 [8]). |
| EDN Identifier | It defines the identifier of the edge data network | ednIdentifier as defined in clause 6.3.10 in TS 28.538[8] |
| EDN Connection Information | It defines the set of information needed to connect to an EDN. | eDNConnectionInfo as defined in clause 6.3.10 in TS28.538 [8] |
| Available Edge Virtual Resources | It defines the available edge virtual resources managed by an EDN | availableEdgeVirtualResources as defined in clause 6.3.10 in TS28.538 [8] |

The analytics output of MDA for Edge application deployment location analytics, are provided in table 5.2.3.3-2.

Table 5.2.2.3-2: Analytics output for edge application deployment location analytics

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| edgeApplicationDeplymentLocationRecommendations | It defines geographic locations where are recommended for edge application deployment | M | type: GeoArea (see TS 28.622 [14]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

5.2.2.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

5.2.3	Use case 3: Edge network topology mapping

5.2.3.1	Description

This use case assumes that the 5GC nodes are deployed on the edge of the network to provide better and reliable services to the end users. With edge network deployment, we can guarantee the latency and reliability for critical applications such as factory automation, automation vehicles, remote control and virtual/augmented reality. As depicted in the following figure multiple instances of the 5GC nodes may be deployed on the edge network to mitigate the load. Selecting the set of nodes and their instances to be deployed in the edge site is crucial for better performance.

5.2.3.2	Potential Requirements

REQ-FUN-EdgePM-01: It should be possible for MDA analytics to suggest the components of an edge site.

5.2.3.3	Potential solutions

The solution involves performing heuristics learnings on measurement data related with RTT and Packet Loss on various interfaces including N3 (gNB UPF), N4 (SMF UPF), N10 (SMF  UDM), N11(AMF SMF) and N7 (SMF  PCF). Thereafter, deploying automation functionalities (e.g. AI/ML Time Series Methods) to analyse the above parameters and subsequently chooses the best components to be part of an edge site. The analytics will provide projected RTT and Packet Loss information, as peer information, for each related interface. Based on the peer information the best possible components can be decided.

The solution proposes a new MDA type for edge network topology mapping.

The analytics scope of this MDA type would indicate the following:

- The DNs of the interface (EP_N3, EP_N11) for which the peer information is requested.

The enabling data for this MDA type would include the following:

- The average round-trip delay on a N3 interface of this gNB on PDU Session Anchor (PSA) for available instances of core UPF. This measurement is split into sub-counters per DSCP (Differentiated Services Code Point). See clause 5.4 of TS 28.552 [5].

- Number of GTP data packets of this gNB which are not successfully received at UPF. See clause 5.4 of TS 28.552 [5].

- Number of GTP data packets of this gNB which are not successfully received at gNB over N3. See clause 5.4 of TS 28.552 [5].

The analytics output for this MDA type would include the following:

- Member NF: The DN of the selected VNF to be deployed on the edge.

5.3	Data correlation analytics

5.3.1	Description

This MDA capability is for data correlation analytics on network, computing and slice resources across different aspects consisting of network domains(RAN/ core/ transport), resource domain  (network and cloud) and slice / slice subnet aspects.

5.3.2	Use case 1: Measurement data correlation analytics for ML training

5.3.2.1	Description

For a ML model training, a large amount of measurement data instances may be collected and does not necessarily add value to training performance. The collected measurement data for ML training can be highly correlated (linear or non-linear), hence there may be high redundancy among the collected measurement data. Hence using all measurement data for a model training (and inference) can be a waste of computing resources and energy. Therefore, it's necessary to optimize the training data preparation based on the correlation analytics and redundancy information. A correlation analytics may help to detect the redundancy pattern among the measurement data for ML training, such as:

-	For a given task (e.g. analytics, model training), analyses the correlation among the given set of measurement data, the output can be a (much) smaller set of measurement data, with which ML model (re-)training could be much more efficient with limited (or managed) impact to model training performance (comparing to use full set of data). The output may include a recommendation, for example recommendation to optimize measurement data collection for the model training.

-	Regularly renew the correlation analytics as time progresses, since the correlation relationship might change; this is especially useful when there is a need to regularly re-train the ML model.

5.3.2.2	Potential requirements

REQ-DATA-CORRELATION-1:	MDA capability for Measurement data correlation analytics for ML training should include the capability to provide the measurement data redundancy analysis including which measurement data correlate to which measurement data, the rate of redundancy, and recommendation to optimize measurement data collection for the model training.

5.3.2.3	Potential solutions

5.3.2.3.1	Possible solution for measurement data correlation analytics for ML training

- Introduce a data type for measurement data correlation analytics recommendation, called measurementDataCorrelationRecommendation. The data type can be the contents of the analytics report representing the recommendations from MDA for the measurement data correlation analytics for ML training.

  - The contents of this data type may be a set of 3GPP and non-3GPP recommendations.

- An optional attribute may be used to configure the analytics context in MDARequest to indicate the training performance requirement.

5.3.2.4	Evaluation of solutions

Only one potential solution in 5.3.2.3.1 is proposed, the requirement is satisfied and this solution is feasible for normative work.

5.3.3	Use case 2: Measurement data correlation analytics for threshold assessment and adjustment

5.3.3.1	Description

To monitor the threshold crossings of performance metric values related to specified managed objects, the consumer may firstly configure the performance metric threshold.

Current MDA supports recommendation threshold for ES use cases, but recommending or assessing the threshold may need to be more generic.

If the threshold is configured too high or too low or not reflect with fluctuation of the actual traffic, there may be alarms generated. Hence the threshold related alarms may have potential correlation with the threshold configuration of the performance metric. The potential correlation information, may be used to assess the configuration of threshold and possible to support the auto-configure the threshold.

After getting the threshold assessment result, MDA may predict the recommended actions to optimize threshold configuration for possible auto adjustment of the threshold configuration.

5.3.3.2	Potential requirement

REQ-DATA-CORRELATION-2:	MDA capability for threshold assessment and adjustment should include the capability to provide the threshold configuration assessment analytics indicating if the threshold instance was set properly based on that correlation data. The analytics should also include recommendations of how to optimize the threshold.

5.3.3.3	Potential solutions

- Enhance the existing data type Recommended3GPPAction to support the recommendations from MDA for the threshold assessment and adjustment optimization actions.

5.3.3.4	Evaluation of solutions

Only one potential solution in 5.3.3.3 is proposed, the requirement is satisfied and this solution is feasible for normative work.

5.3.4 	Use case 3: Multi-domain resource optimization (MARO)

5.3.4.1	Description

MDA may provide analytics on different domains of the network, including radio, core and transport domains and on different functions in these domains including physical and virtual network functions. These different domains and functions may be following different architectures ( e.g. the split RAN architecture (CU-CP/ CU-UP/ DU).

The behaviour of these network domains and functions may be correlated, requiring that any optimization actions need to be correlated between these domains or functions. The MDA may need to support a multi-domain resource optimization (MARO) capability that provides information on degree of correlation observed in the data from these network domains / functions.

Consider an example where the performance expectations for end-to-end network slice need to be fulfilled. To fulfil these requirements, the performance at relevant radio network, core network and transport network need to be considered and these different performance data from different domains need to be correlated. The information on the correlation between these domains provided by MARO capability can provide insights on how to fulfil the end-to-end performance expectations.

MARO could also evaluate the related impact of one domain on the other based on which it derives recommendations for how to optimize resources among these domains in a correlated way. For example, to optimize the allocation of load among cells, MARO may provide a report that contains recommendations on changes to be concurrently made to the edge cloud resources hosting the cells functions (CU-CP/ CU-UP/ DU) as well as on the handover parameters of the cells to allow a set of users to be moved among those cells.

5.3.4.2	Potential requirements

REQ-MDA-MARO-1:	The MnS for MDA should include the capability to provide a report on the coordinated simultaneous optimization actions to be undertaken in multiple network domains including physical and virtual infrastructure.

REQ-MDA-MARO-2:	The MnS for MDA should have the capability to get data from different domains to prepare the correlation analytics across these domains.

5.3.4.3	Potential solutions

- Enhance the existing data type RecommendedAction to support the recommendations from MDA for the set of simultaneous changes to be made to different domains, including 3GPP recommendations and non-3GPP recommendations. It may introduce a timewindow attribute for a recommended action in the involved domain. A same timewindow for multi-domain may be used for simultaneous operations and different timewindows may be used for sequencing recommendations operations.

5.3.4.4	Evaluation of solutions

Only one potential solution in 5.3.4.3 is proposed, the requirements are satisfied and this solution is feasible for normative work.

5.3.5	Use case 4: Handover and service data correlation analytics

5.3.5.1	Description

Handover performance may be affected by multiple aspects including:

- Signal measurements, either as measured in the network or as predicted.

- User speeds which may be measured or predicted.

User paths or trajectories, by predicting the likely destination based on the user current path and learning from previous user trajectories. These may be combined in different forma to either derive the optimal handover settings or to also determine the related service configurations. It is desired that the MDA supports analytics that enables the consumer to request analytics on real or predicted signal measurements, on real or predicted user speeds, on learned and predicted user trajectories or paths, or any combination of these aspects. The predicted user speed and predicted trajectory mentioned here are referring to the RAN3 agreed predicted outputs on the same as described in TS 37.817 [9] and TS 38.423 [10].

Figure 5.3.5.1-1: Examples of various context information and HO performance metrics can be used for handover optimization through different combinations A1 to A5.

5.3.5.2	Potential requirements

REQ-MDA-CORR-1:	The MDA producer for handover and service data correlation analytics should have a capability of allowing the use the predicted signal measurements, predicted user speeds and predicted user trajectories for providing recommendations for Handover analytics.

REQ-MDA-CORR-2:	The MDA producer for handover and service data correlation analytics should have a capability of allowing the consumer to use the service characteristics.

Note: The service characteristics may include information such as QoE for handover and service data correlation analytics.

5.3.5.3	Potential solutions

5.3.5.3.1	Possible solution for handover and service data correlation analytics

- Extend the existing use case on Handover optimization analytics by enhancing the enabling data to include predicted signalling measurements, predicted user trajectories and predicted signal measurements.

- Extend the existing usecase on Handover optimization analytics by enhancing the enabling data to include service characteristics such as QoE.

5.3.5.4	Evaluation of solutions

The solution proposed in clause 5.3.5.3.1 satisfies the requirements and this solution is feasible for normative work.

5.3.6	Use case 5: Correlation analytics for NF Scaling and dimensioning

5.3.6.1	Description

Performance metrics (performance measurements, KPIs) generated by a Network Function (NF) or related NFs may exhibit correlation and association patterns. These patterns can be leveraged to derive NF correlation or dependency relationships, which are valuable inputs for network performance optimization scenarios such as scaling and dimensioning.

-	For performance optimization scenarios (e.g. NF scaling, dimensioning), correlation/association analysis of NF measurement data can yield NF dependency/correlation patterns related to NF scaling and network resource utilization. The analysis output may include recommendations for performance optimization (e.g. optimizing NF scaling and dimensioning).

-	Correlation analytics may be refreshed regularly as correlation relationships can vary across different locations and time periods.

5.3.6.2	Potential requirements

REQ-MDA-CORE-X:	MDA capability for correlation analytics for NF scaling and dimensioning should include the capability for NF dependency/correlation pattern recognition with respect to NF scaling or NF resource usage for dimensioning, and predict recommendations of NF scaling and dimensioning optimization in a coordinated manner.

5.3.6.3	Potential solution #1

- Introduce new information attributes in MDA analytics output (e.g. RecommendedAction) to encapsulate recommendations for NF scaling and dimensioning optimization. The solution may include the following attributes:

  - Information Attributes indicating the proportional information among the NF for scaling or dimentioning optimization.

  - Optional information attribute indicating the ordering information for NF scaling

5.3.6.4	Evaluation of solutions

Potential solution #1 is proposed, the requirement is satisfied and this solution is feasible for normative work.

5.4	ATSSS performance analytics

5.4.1	Use case 1: Traffic steering analysis

5.4.1.1	Description

This MDA capability is for the analytics on the usage of ATSSS and N4 rules and provide recommendations on most suitable ATSSS/N4 rules based on the analytics and predictions performed.

ATSSS feature introduced the concept of Multi Access PDU session, a PDU session for which the data traffic can be served over one or more concurrent accesses (3GPP access, trusted non-3GPP access and untrusted non-3GPP access). Using these services, the UE can access 5G services via 3GPP/non-3GPP access methods.

After the establishment of a MA PDU Session, and when there are user-plane resources on both access networks, the UE applies network-provided policy (i.e. ATSSS rules) and considers local conditions (such as network interface availability, signal loss conditions, user preferences, etc.) for deciding how to distribute the uplink traffic across the two access networks. Similarly, the UPF anchor of the MA PDU session applies network-provided policy (i.e. N4 rules) and feedback information received from the UE via the user-plane (such as access network Unavailability or Availability) for deciding how to distribute the downlink traffic across the two access networks. When there are user-plane resources on only one access network, the UE applies the ATSSS rules and considers local conditions for triggering the establishment or activation of the user plane resources over another access.

In order to optimally steer the traffic across different accesses and based on the load conditions, the network derives a set of rules that are communicated to UE (ATSSS rules) and UPF (N4 rules) as guidance on how to steer the traffic under certain conditions. The PCF derives the ATSSS policy and sends it to SMF in case of dynamic PCC (policy and charging control) enabled. SMF then converts it to ATSSS rules for UEs (UL traffic) and N4 rules for the UPF (DL traffic). These rules are then enforced by the UPF in DL and by the UE in UL to send the PDU packets. The rules contain thresholds values for selection of certain access types including the instruction of which steering modes to be used. If the PCC is not used, then the rules are framed by the SMF based on the local configurations. The structure of the ATSSS rules is described in the table 5.32.8-1 of TS 23.501 [4].

The network derives these rules only based on immediate performance measurements (e.g. RTT and packet loss) and local configuration at SMF. i.e. besides considering the radio link performance data, the traffic steering analytics solution derives traffic steering decisions based on the Quality of Service or Quality of Experience (QoS/E) characteristics of the related service at the UE. The rules do not capture holistic view on the past network traffic trends, the history, and trends of the performance measurements (RTT and packet loss) from the past and situation on different accesses, ATSSS/N4 rules usage statistics, impact on available accesses, nor the predictions of the network behaviour in the future. Therefore, the derived rules may be suboptimal and may lead to network degradation. The MDA can predict the optimal rules and the consumer (e.g. UPF or SMF) decides to apply these rules for the MAPDU sessions.

5.4.1.2	Potential requirements

REQ-TRF_STR_MDA-01: MDA capability for traffic steering analysis should be able to provide the following analytics information of the configured ATSSS rules.

- the percentage of time the network defined rules are used to steer the traffic.

- the percentage of time a particular steering mode in a rule is used to steer the traffic.

- the number of instances the threshold values have crossed for the requested rules.

REQ-TRF_STR_MDA-02: MDA capability for traffic steering analysis should include providing recommendations on ATSSS rules and N4 rules and their precedence for a (set of) network slice(s) or network slice subnet(s).

5.4.1.3	Potential solutions

5.4.1.3.1	Solution description

This potential solution proposes a new MDA type traffic steering analytics. This potential solution also proposes to define a new data type that may be used by the MDA producer to recommend the optimal ATSSS rule to MDA MnS consumer.

5.4.1.3.2	Data required

The input data needed for these analytics are described below.

- Performance measurements on DL packet loss rate on Uu interface (clause 5.1.1.35 from [5]), UL PDCP SDU loss rate and DL/UL F1-U packet loss rate (clause 5.1.3.1 from [5]), Incoming and outgoing GTP packet loss rate (clauses 5.4.1.7, 5.4.1.8 from [5])

- Performance measurements on Packet delay (clause 5.1.3.3 from [5], GTP packet delay (clause 5.4.1.9 from [5])

- UE level performance measurements on packet delay and packet loss rate (clauses 6.2.2.1, 6.3.1.1, 6.3.1.2, 6.3.1.3 from [15])

- M6 and M7 measurements as defined in [16].

5.4.1.3.3	Analytics Report

The analytics report may contain the new datatype proposed as part of this solution. This proposed new datatype may contain the following attributes.

- The precedence of this predicted ATSSS rule. This precedence value may be used to specify the precedence of the ATSSS rule among all ATSSS rules. This field may follow the rules as specified in table 5.32.8-1 of TS 23.501 [4].

- The steering mode. This attribute recommends the optimal steering mode. Steering mode determines how the traffic of the matching SDF may be distributed across 3GPP and non-3GPP accesses. The possible values of the steering mode are Active-Standby, Smallest delay, Load-balancing, Priority-based, Redundant.

- The steering mode indicator. This attribute indicates that the UE may change the default steering parameters provided in the Steering Mode component and may adjust the traffic steering based on its own decisions. The following are the possible values for this attribute, autonomous load-balance indicator and UE assistance indicator.

- Steering mode assistance information. If steering mode is recommended as active-standby, the active and standby components between 3GPP and non-3GPP is recommended. For instance, one possible option may be "Active 3GPP and non-3GPP standby". This indicates that the active steering mode is 3GPP and the non-3GPP is used as standby mode. If the steering mode is recommended as "load-balancing, the split of load between 3GPP and non-3GPP access methods may be recommended. For instance, the following may be an option. "90% over 3GPP and 10% over non-3GPP" or "0% over 3GPP and 100% over non-3GPP".

- The threshold values. this attribute may contain two kinds of information. Maximum RTT value and maximum packet loss. Both information may be reported as a percentage value from 0 to 100%. These threshold values indicate the level if crossed, the UE can decide to autonomously change the rules to maximize the UL bandwidth. Both threshold values are valid if the steering mode is recommended as load-balancing, priority-based or redundant.

5.4.1.4	Evaluation of solutions

The solution proposed in clause 5.4.1.3 satisfies the requirements and this solution is feasible for normative work.

5.5	UE throughput analytics

5.5.1	Use case 1: UE throughput analysis

5.5.1.1	Description

With the development of diverse communication services and the increasing number of connections, user data volume demanded by end users grows rapidly. As the user traffic increases, user experience deteriorates significantly. Therefore, UE throughput related measurements need to be analysed to identify the potential traffic congestion time and place by monitoring the network and provide the corresponding recommendation optimization that can be implemented to avoid potential traffic congestion.

Two types of traffic congestion as follows.

- Non-burst traffic congestion: Congestion that have heavy traffic which occurs at a fixed time or place every day, for example, during peak hours of daily morning, heavy traffic will resulting in a degraded user experience

- Burst traffic congestion:  user traffic increases sharply in a specific area, for example, in commercial activities and sports events, a large number of users gather for a period of time, resulting in rate experience deterioration.

The occurrence of such traffic congestion problem may seriously affect user experience and can be detected based on the UE throughput related measurements analysis.

Figure 5.5.1.1-1  UE throughput analytics

Due to the complexity of 5G network and wireless environment, multiple types of performance deteriorate are related with high user traffic volume, e.g. the related performance measurements such as UE throughput, Radio resource utilization(e.g. PRB utilization), RRC connection number (e.g. number of RRC Connections) can be used to analyze, Once the preset threshold is reached, it may be considered that there is a traffic congestion problem as shown in the Figure 5.5.1.1-1. The user experience related performance should be analysed and identified to help to resolve improve the user experience. In this case, the consumer may need to be able to set the policy to help producer to analyze the congestion problems. The policy could be, for example, the threshold of the UE throughput related performance measurements (threshold 1 and threshold 2 in the figure).

The MDAS producer for radio network may also provide the recommendations to solve the UE throughput issues identified by the analytics congestion scenarios. The recommended actions may be for example to change the mobility parameters (e.g. MRO or LBO) or antenna RF parameters (e.g. CCO) such as downtilt and azimuth to where they are needed.

5.5.1.2	Potential requirements

REQ-TRAFFCONG-1: The MDAS producer for radio network should be able to analyze and identify the traffic congestion based on the threshold of UE throughput related performance measurements and provide the analysis report to the MnS consumer. The traffic congestion problems should contain the following information:

-	The traffic congestion type and its severity level;

-	The start time and end time of the traffic congestion;

-	The geographical area and location where the traffic congestion has/may happen;

-	The recommended actions to solve the potential traffic congestion problem.

5.5.1.3	Potential solutions

This potential solution proposes a new MDA type: traffic congestion analytics.

Enhance MDARequest IOC with the threshold of traffic related performance measurements for this MDA type, including:

- Threshold of PRB utilization

- Threshold of number of RRC Connections

- Threshold of UE throughput

The enabling data for this MDA type would include the following:

- UE throughput including DL and UL throughput in gNB. See clause 5.1.1.3 of TS 28.552 [5].

- Radio resource utilization including DL and UL PRB usage. See clause 5.1.1.2 of TS 28.552 [5].

- RRC connection number. See clause 5.1.1.4 of TS 28.552 [5].

The analytics output for this MDA type would include the following:

-	The traffic congestion type including Non-burst traffic congestion and Burst traffic congestion

-	The traffic congestion severity level, e.g. level 1, level 2, and level 3, which may respectively indicate slight congestion, moderate congestion, and severe congestion.

-	The start time and end time of the traffic congestion;

-	The geographical area and location where the traffic congestion has been happened;

-	The recommended actions to resolve the congestion issue (e.g. MRO, MLB or CCO optimization etc.).

5.5.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work

5.5.2	Use case 2: Network congestion analytics based on UE throughput

5.5.2.1 Description

The use case focuses on proactive identification and mitigation of network congestion by analysing UE throughput data.

A surge in user traffic within a specific area, such as a shopping mall, concert venue, or stadium, can lead to network congestion. As user density and data demands increase, cell resources become strained, resulting in a decline in UE throughput. Without timely intervention, congestion intensifies, leading to service degradation and impacting user experience.

The use case leverages MDA to analyse real-time and historical UE throughput performance data to detect and predict congestion. Based on prediction results, the MDA may recommend appropriate mitigation measures (e.g. transfer some UE traffic from congested cells to neighbouring cells with lighter loads) to maintain network performance and ensure user experience.

5.5.2.2 Requirements

REQ-CONG-MDA-01: MDA capability for network congestion analytics based on UE throughput should provide the predict of network congestion for various time granularities (e.g. seconds, minutes, hours) and geographical scopes (e.g. cell, cluster, area).

REQ-CONG-MDA-02: MDA capability for network congestion analytics based on UE throughput should recommend appropriate mitigation measures based on prediction results.

5.5.2.3	Potential solutions

The solution is to introduce the enabling data for network congestion analytics based on UE throughput, the enabling data are provided in table 5.5.2.3-1.

Table 5.5.2.3-1: Enabling data for network congestion analytics based on UE throughput

| Data category | Description | References |
| --- | --- | --- |
| Performance measurement | Radio resource utilization | Radio resource utilization as defined in clause 5.1.1.2 in TS 28.552 [5]. |
|  | RRC connection establishment related measurements | RRC connection establishment related measurements as defined in clause 5.1.1.15 in TS 28.552 [5]. |
|  | RAN UE Throughput | RAN UE Throughput as defined in clause 6.3.6 in TS 28.554 [6]. |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [7]). |
| Configuration data | cellIndividualOffset, isHOAllowed and isMLBAllowed of corresponding NRCellRelation(s). | NRM information TS 28.541 [13]. |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | The geographical information (longitude, latitude, altitude) information (see the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [14]). |

The analytics output of MDA for network congestion analytics based on UE throughput, are provided in table 5.5.2.3-2.

Table 5.5.2.3-2: Analytics output for network congestion analytics based on UE throughput

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| congestionPredictionId | The identifier of the network congestion prediction. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| congestionPredictionArea | Geographical location areas where the congestion is predicted to occur. | M | type: GeoArea (see TS 28.622 [14]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| congestionPredictionAffectedCells | The CGIs of cells where the congestion is predicted to occur. | M | type: NRCGI (see TS28.541 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| congestionPredictionStartTime | The predicted start time of the network congestion. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| congestionPredictionEndTime | The predicted end time of the network congestion . | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| congestionRootCause | The root cause of the predicted network congestion issues. The allowed value is one of the enumerated values: unbalanced load between different cells, frequently handover among several cells, other. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| congestionPredictionRecommendations | The recommended actions to mitigate the predicted congestion. The recommended action may be (but not limited to): - Load balancing; - adjusting and configuring the RAN UE throughput related parameters (e.g. cellIndividualOffset, isHOAllowed and isMLBAllowed of corresponding NRCellRelation(s), maximumDeviationHoTrigger of corresponding DMROFunction).scale in a list of NFs. | M | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

NOTE:	Recommendation for plan from plan management can be considered during normative work.

5.5.2.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

5.6	Fault management related analytics and alarm prediction

5.6.1	Use case 1: Providing threshold statistics information for fault management

5.6.1.1	Description

Threshold crossing statistics information is very helpful to know the running state of a NF. However, the threshold which impacts the network and service may not be known by the MDAS producer. In this use case, the consumer (e.g. Operator) provides the performance crossing threshold information and resource usage threshold information to MDAS producer for calculating and collecting the statistics information, e.g. percentage of time when the performance KPI exceeds the threshold, and percentage of time when resource usage of the NF is beyond the threshold. Such statistics information helps to tell whether the NF is in the overloaded state. Furthermore, the statistics information can be combined with historical data and AI/ML model for MDAS producer to perform fault prediction of the NF.

5.6.1.2	Potential Requirements

REQ-STATISTICS_MDA-01: The MDAS producer should have a capability allowing the consumer to provide threshold information for calculating and collecting statistics information.

REQ-STATISTICS_MDA-02: The MDAS producer should have a capability to predict a fault in a network function based on calculated statistics information.

5.6.1.3	Potential Solutions

It is proposed to enhance the current MDAS solution as specified in TS 28.104 [2], following are the proposed enhancements to support the potential requirements as defined in clause 5.6.1.2:

Proposal to support REQ-STATISTICS_MDA-01:

Introduce new attribute in the MDARequest IOC. The new attribute indicates the threshold information that the consumer cares about. The threshold information could be the thresholds of one or more performance metrics or the DN of already existed ThresholdMonitor instance created by the consumer.

Proposal to support REQ-STATISTICS_MDA-02:

Extend the Analytics output for fault prediction analysis as specified in Table 8.4.3.1.3-1 in TS 28.104 [2] to include:

new information element to express the calculated statistics information which are used for fault prediction, e.g. percentage of time when the performance KPI exceeds the threshold.

5.6.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

5.6.2	Use case 2: Enhancing failure prediction

5.6.2.1	Description

This use case is to enhance the existing MDA capability for failure prediction. The enhancements to the existing use case on failure prediction is targeted towards effective correlation of the predicted alarms and the actual alarms which has resulted as predicted.

The existing use case of failure prediction provides a prediction of failure with some additional information, see TS 28.104 [2]. This includes information such as predicted time of failure occurrence, perceived severity, and the recommended actions. These additional information helps the consumer to understand the effect of the failure in the network to a basic extent. However, the current solution does not provide additional information (e.g. predicted failure recovery time or the trend of the problem causing the failure) which may enable the consumer to anticipate the impacts to the network more effectively.

The predicted failure recovery time indicates the time by which the target entity recovers from the predicted failure. For instance, MDA can predict the time to recover for a given failure given the predicted root cause of the predicted failure and the history information.

The attribute "trend" indicates the trend of the failure, whether the failure will intensify or weaken in the future. In case of periodic analytics reports, the trend information can provide information on how the identified issue might transform in the future. This is similar to the "trend" attribute in the alarm solution in 28.532 [3].

Also, for the consumers to effectively use the output of MDA assisted failure prediction, it is required by the consumer to seamlessly correlate the predicted failure and the actual failure when it occurs in the system. The alarm stage 2 solution is defined in 28.532 [3]. To utilize the failure prediction output effectively for this purpose, it is required to enhance the existing failure prediction to include additional information that may help in effective correlation. To achieve this, it is required to enhance the existing solution towards the alarm solution in 28.532 [3].

5.6.2.2	Potential Requirements

REQ-FAILURE_PRED_ENHANCE_MDA-01: MDA capability for enhanced failure prediction should be able to provide the following information as part of the failure prediction analytics.

- a predictedRootCauseIndicator indicating the possible root cause of the predicted failure.

- Trend indication about the predicted failure in case of periodic reporting.

- Predicted end time of the failure

5.6.2.3	Potential solutions

The analytics output for MDA assisted fault management may be enhanced with the following attributes.

-	predictedRootCauseIndicator. This attribute may indicate the predicted root cause of the potential failure that has been predicted. This attribute may be of type string.

-	predictedTrendIndication – this attribute may indicate the predicted trend of the failure that has been predicted. In case of periodic MDA outputs, this attribute may indicate the trend of the predicted failure is worsening or getting better or not changing. This attribute can be an enumerated value with possible allowed values of "less severe", "no change" and "more severe". The detailed definition of these enumerated values can be found in clause 8.1.2.6 of ITU-T Recommendation X.733 [11].

-	predictedRecoveryTime – this attribute may indicate the predicted recovery time of the predicted failure.

5.6.2.4	Evaluation of solutions

The solution proposed in clause 5.6.2.3 satisfies the requirements and this solution is feasible for normative work.

5.6.3	Use case 3: Enhancing service failure recovery

5.6.3.1	Description

TS 28.104 [2] clause 7.2.3.2 describes a use case for service failure recovery. To explore possible issues, this use case is elaborated in more detail as follows:

1. The network operator discovers that a service problem exists. This may be a failed service or a degraded service. The network operator may discover the issue because of an alarm or because of performance measurements.

2. The network operator requests MDA to analyse the service problem.

3. MDA produces a report, which includes recommendations to resolve the issue.

4. The network operator evaluates the recommendations.

5. If the network operator accepts the recommendations, the network operator implements the recommended actions.

The above use case raises the following issues:

It is not clear how the MDA MnS consumer should request analysis of a particular service problem. The MDARequest IOC contains an attribute analyticsScope which may be expressed as managedEntitiesScope. The definition of managedEntitiesScope says that it "carries the DN(s) of SubNetwork MOI(s), ManagedElement MOI(s), and/or the MOI(s) of the derivative IOCs of ManagedFunction". It is not obvious how a service problem can be identified by any of the allowed DN types. For example, if the MDA MnS consumer wishes to use an alarm to identify a service failure, it is not possible to express an alarm identifier in managedEntitiesScope.

It is not clear which MDA type supports the use case for service failure recovery. The analytics output for MDA type MDAAssistedFaultManagement.FailurePrediction includes an information element recommendedActions which mentions "recovery", but there is no description of what this means. Also, the related analytics output contains mandatory information elements which are not appropriate for service failure recovery.

MDA type MDAAssistedFaultManagement.FailurePrediction contains recommendations on how to resolve a predicted failure, but there is not enough information for the MDA MnS consumer to evaluate whether the recommendations are valid or not. Missing important information includes the root cause of the service problem and demarcation information. This information is essential to allow the MDA MnS consumer to trust the recommendations and to discover underlying issues that could cause a similar service problem to occur in the future.

5.6.3.2	Potential requirements

REQ-FAILURE_RECORV_MDA-01: MDA capability for service failure recovery shall allow a consumer to select an alarm for analysis.

REQ-FAILURE_RECORV_MDA-02: MDA capability for service failure recovery shall be able to provide the analytics output including the probable cause of the failure and recommended actions to resolve the service failure.

5.6.3.3	Potential solutions

Update TS 28.104 [2] to add a new MDA type MDAAssistedFaultManagement.FailureRecovery to allow an MnS consumer to request analytics related to service failure recovery. The attribute analyticsScope may need to be extended to allow the MDA MnS consumer to use an alarm identifier to indicate the scope of the analysis.

Update TS 28.104 [2] to add a new analytics output for service failure recovery analysis, including (but not limited to) information related to

- related alarm(s) which are likely to be correlated to the service failure

- recommended actions to resolve the service failure

Update TS 28.104 [2] to remove the term "recovery" in the failure prediction use case and MDA type MDAAssistedFaultManagement.FailurePrediction.

5.6.3.4	Evaluation of solutions

There is one proposed solution which satisfies the requirements, and the solution is feasible.

5.6.4	Use case 5: NF failure evaluation

5.6.4.1	Description

The MDARequest IOC as defined in clause 9.3.2 in TS 28.104 [2] is used to request MDA analysis service by the consumer. In the MDARequest, it includes analyticsScope attribute used to indicate the scope of the analytics requested by the MnS consumer. The analyticsScope attribute can be expressed as managedEntitiesScope which possibly can carry the DN of one or more ManagedFunction (see Table 9.5.1 in TS 28.104 [2]). In case the analyticsScope only contains the DN of a ManagedFunction, it means that the analytics only relate to a specific ManagedFunction.

The existing MDA capability for failure prediction is mainly used to analysis and predict whether a failure will occur in the 5G network. The analytics output for failure prediction analysis mainly contains failurePredictionObject, potentialFailureType, eventTime, perceivedSeverity and recommendedActions (see table 8.4.3.1.3-1 in TS 28.104 [2]). Those information elements in the analytics output are about to express the predicted faults that may occur. As described in the above scenario that the analyticsScope only contains the DN of a ManagedFunction and the target ManagedFunction running well, then the current analytics output is not applicable.

When consumer requests failure prediction analysis for a specific ManagedFunction, it is not clearly specified in TS 28.104 [2] how the MDA report can indicate that no failure is predicted. In addition, when a failure is predicted to occur, quick and accurate identification of the affected users, services, cells, and geographical areas is also crucial for fault management and subsequent handling. The MDAS producer needs to identify the affected network entities, users, and geographical areas related to the predicted failure by correlating with the analysing data from multiple sources. The analysis results can assist the consumer to quickly assess the impact of failures and take appropriate actions for failure recovery and service assurance, thereby minimizing the negative impact caused by the failure.

5.6.4.2	Potential Requirements

REQ-FAULT_EVALUATION_MDA-01: The MDAS producer should have a capability to provide the information of network impacts related to a predicted failure, including:

- A geographical information description of the affected areas, such as longitude and latitude ranges, polygon areas, etc.

5.6.4.3	Potential solutions

The MDA analytics output for failure prediction analysis may be enhanced with the following attributes.

-	predictedImpactArea. This attribute may indicate the predicted network impact of the potential failure. This attribute may be described as geographical information of the affected areas, such as longitude and latitude ranges, polygon areas, etc.

5.6.4.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

5.7	Software upgrade validation

5.7.1	Use case 1: Software upgrade validation

5.7.1.1	Description

Upgrades in the network including software upgrades are typically conducted during less busy times such as night time. Even though the software upgrade task is performed during the night (anticipating that as a lean period), the validation window following the upgrade can be large because it involves a series of pre-and post-checks. These checks include the evaluation of a set of Key Performance Indicators (KPIs). In this process the operations team has to carefully scrutinize hundreds of such KPI's. This team has to monitor trends and detect spikes/falls in the data to find anomalous KPI's after the NF is upgraded. This whole process of finding outlying KPIs can take anywhere from a few hours to a few days depending upon the circle/area under analysis. Finding root cause of problem by correlating KPI with FM/CM changes and see whether the problem is due to software upgrade has impacted performance.

If the above validation of the software upgrade fails, a neighbour NF is selected to take the load until the re-upgrade and re-validation is done. Typically, it takes weeks or even longer to validate the new software upgrade for the complete network because the upgrade needs to be monitored for weeks before it is considered valid. The validation process requires checking for performance measurements and KPIs for any degrade. If degradation has occurred the validation will be considered as failed, resulting in the re-upgrade making the entire management process inefficient. MDAS analytics can be used to validate the SW upgrade, for a future point of time, right after the upgrade is done. The validation involves monitoring and evaluating the set of Key Performance Indicators (KPIs) with the help of AI/ML technologies to create analytic results that can be used to predict the success or failure of a software upgrade.  If the MDAS analytics indicate the unsuccessful upgrade then a neighbour NF can be decided to take the load of the target NF while it goes into re-upgrade and validation cycle.

5.7.1.2	Potential requirements

REQ-SWA-FUN-01: The MDA MnS producer should enable an authorized consumer to request for an analytics report validating a software upgrade.

NOTE: Validation of a software upgrade involves monitoring and evaluating the set of Key Performance Indicators (KPIs) to create analytic results that can be used to predict the success or failure of a software upgrade.

5.7.1.3	Potential solutions

The solution requires collecting the performance data of the target NF (i.e. the NF which got software upgraded) analysing it using time series model by deploying appropriate algorithms; and predict the PM data. If the predicted PM data degrades the software upgrade is considered un-successful.

The solution proposes a new MDA type for software upgrade validation.

The analytics scope of this MDA type would indicate the following:

- The DNs of the NF (NRCellCU, AMF, SMF etc) for which the upgrade validation analytics is requested.

The enabling data for this MDA type would include the following:

- Performance Data (performance measurement and KPI) for the entities, provided by the analytics scope, as defined in 3GPP TS 28.552 [5] and TS 28.554 [6].

- Provisioning Data:  This refers to the operation state of a cell. It shows whether the cell is Enabled or Disabled for service as defined in 3GPP TS 28.541 [13].

- Alarm Information: This refers to the active alarm information (AlarmInformation as defined in 3GPP TS 28.532 [3]) in the network.

NOTE: aforementioned data will be collected for the new software.

The analytics output for this MDA type would include the following:

- UpgradeStatus: This indicates if the upgrade is predicted to be successful in future. This will be a Boolean attribute with default value as TRUE. The value FALSE indicate the un-successful upgrade

5.7.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirement is satisfied and this solution is feasible for normative work.

5.8	Control plane congestion analytics

5.8.1	Use case 1: Enhancing control plane congestion analysis

5.8.1.1	Description

The use case for control plane congestion analysis is described in TS 28.104 [2]. It proposes to use MDA to assist control plane congestion analysis in order to detect, prevent or resolve congestion issue happened at the control plane. NWDAF as an MDA MnS consumer may need to take further analysis on control plane congestion issue based on the analytics output from MDAF. Existing MDA analytics output for 5GC control plane congestion analysis only includes 5GC NFs affected by congestion issues, analysis report identifier and the recommended actions, which may need enhancement to support further analytics of NWDAF. Missing attributes include the root cause of the congestion issue and the predicted duration of congestion issue.

The examples of root cause could be the huge amount of UE access network in short period of time. Congestion issues with different root causes need different mitigation solutions. For example, if congestion issue is caused by network failure, it is possible for OAM to resolve the issue with analytics performed by MDAF and may not need NWDAF to take further actions. If congestion issue is caused by signalling overload, the MDA report may include the identified object which is the bottleneck of network and the recommended action to eliminate the bottleneck issue. NWDAF may perform further analytics to recommend actions to eliminate the congestion issue, such as adjusting the backoff timer or controlling the signalling access rate.

Besides, information about predicted duration of identified congestion issue may be also needed by NWDAF. For example, if the congestion is predicted to be self-recovered in a short time, it may not necessary for NWDAF to take actions. If the predicted duration of congestion issue is long, timely actions may need to be taken by NWDAF to resolve the issue.

5.8.1.2	Potential requirements

REQ-CP_ANA-ENHANCE-01: MDA capability for control plane congestion analysis should be able to provide the analytics output including root cause of the congestion issue and the predicted duration of congestion issue to the consumer.

5.8.1.3	Potential solutions

The analytics output for MDA assisted control plane congestion analysis may be enhanced with the following attributes.

- rootCause. This attribute may indicate the root cause of control plane congestion issue, such as network failure, signalling overload, etc.- predictedDuration. This attribute may indicate the predicted time duration of control plane congestion issue.

5.8.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

6	Conclusions and recommendations

6.1	Energy efficiency analytics

The use case, requirements and solution for Energy Saving based on throughput requirements is described in clause 5.1.1. It is recommended to update the inputs and outputs for MDAAssistedEnergySaving.EnergySavingAnalysis as proposed in clause 5.1.1.3.

The use case, requirements and solution for the capability of Extension of Cell Energy Saving analytics is described in clause 5.1.2.  It is recommended to update the inputs and outputs for MDAAssistedEnergySaving.EnergySavingAnalysis as proposed in clause 5.1.2.3.

Note: It is assumed that the gNB would consume the beam level information provided as part of MDAAssistedEnergySaving.EnergySavingAnalysis. The gNB can manage beams as suggested in analytics output.

6.2	End-to-End performance analytics including Edge computing domain

The use case, requirements and solution for Use case: Edge computing performance analytics in clause 5.2.1. It is recommended to add new capability in TS 28.104 [2] to support MDA analytics for Edge computing performance analytics, which can be used to predict related to E2E latency between UE and EAS. The detailed solution is described in clause 5.2.1.3.

The use case, requirements and solution for Use case: Edge application deployment location analytics in clause 5.2.2. It is recommended to add new capability in TS 28.104 [2] to support MDA analytics for edge application deployment location, which can be used to recommend the deployment location that meets user experience. The detailed solution is described in clause 5.2.2.3.

6.3	Data correlation analytics

The use case, requirements and solution for use case: Measurement data correlation analytics for ML training is described in clause 5.3.2. It is recommended to add new capability in TS 28.104 [2] to support MDA analytics (e.g. for providing ML model training information). The detailed solution is described in clause 5.3.2.3.

The use case, requirements and solution for use case: Measurement data correlation analytics for threshold assessment and adjustment is described in clause 5.3.3. It is recommended to add new attribute in the Recommended3GPPAction for management data correlation analytics in TS 28.104 [2] to support providing threshold assessment and adjustment information in the analytics output. The detailed solution is described in clause 5.3.3.3.

The use case, requirements and solution for use case: Multi-domain resource optimization is described in clause 5.3.4. It is recommended to add new attribute in the RecommendedAction for management data correlation analytics in TS 28.104 [2] to support providing Multi-domain resource optimization information in the analytics output. The detailed solution is described in clause 5.3.4.3.

The use case, requirements and solution for the capability of handover and service data correlation analytics is described in clause 5.3.5. It is recommended to extend the input data in TS 28.104 [2] as described in clause 5.3.5.3.

The use case, requirements and solution for Use case: correlation analytics for NF Scaling and dimensioning is described in clause 5.3.6. It is recommended to add new information elements in the MDA analytics output in TS 28.104 [2] to support providing NF Scaling and dimensioning information in the analytics output. The detailed solution is described in clause 5.3.6.3.

6.4	ATSSS performance analytics

The use case, requirements and solution for the capability of Traffic Steering Analytics is described in clause 5.4.1. It is recommended to add new capability in TS 28.104 [2] to support Traffic Steering Analytics as proposed in the solution in clause 5.4.1.3.

6.5	UE throughput analytics

The use case, requirements and solution for Use case: NF fault evaluation is described in clause 5.5.1. It is recommended to add new attribute in the MDA analytics output for UE throughput analysis in TS 28.104 [2] to support providing traffic congestion information in the analytics output. The detailed solution is described in clause 5.5.1.3.

The use case, requirements and solution for Use case 2: Network congestion analytics based on UE throughput is described in clause 5.5.2. It is recommended to add new capability in TS 28.104 [2] to support MDA analytics for UE throughput, which can be used to predict network congestion. The detailed solution is described in clause 5.5.2.3.

6.6	Fault management related analytics and alarm prediction

The use case, requirements and solution for Use case: Providing threshold statistics information for fault management is described in clause 5.6.1. It is recommended to add new attribute for threshold information in the MDARequest in TS 28.104 [2] to support providing statistics information in the analytics output. The detailed solution is described in clause 5.6.1.3.

The use case, requirements and solution for the capability of enhancing failure prediction is described in clause 5.6.2. It is recommended to extend the output of failure prediction in TS 28.104 [2] as described in clause 5.6.2.3.

The use case, requirements and solution for the capability of service failure recovery is described in clause 5.6.3. It is recommended to add new capability in TS 28.104 [2] to support service failure recovery. The detailed solution is described in clause 5.6.3.3.

The use case, requirements and solution for Use case: NF failure evaluation is described in clause 5.6.4. It is recommended to add new attribute in the MDA analytics output for failure prediction analysis in TS 28.104 [2] to support providing predicted network impact information in the analytics output. The detailed solution is described in clause 5.6.4.3.

6.8	Control plane congestion analytics

The use case, requirements and solution for Use case: Enhancing control plane congestion analysis is described in clause 5.8.1. It is recommended to add new attributes in the MDA analytics output for control plane congestion analysis in TS 28.104 [2] to support providing root cause and predicted time duration of control plane congestion issue in the analytics output. The detailed solution is described in clause 5.8.1.3.

Annex <A> (informative):
Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2024-02 |  |  | New draft document | 0.0.0 |
| 2024-04 | SA5#154 | S5‑242036 | TR28.866 MDA management data correlation analytics | 0.1.0 |
|  |  | S5‑242037 | Rel19 pCR TR 28.866 Add Use case for providing statistic information for fault management |  |
|  |  | S5‑242041 | Rel-19 pCR Use case and requirements for Enhancement of Failure Prediction |  |
|  |  | S5‑242042 | pCR 28.866 Add fault management use case |  |
|  |  | S5‑242045 | Rel-19 pCR 28.866 Add use case for Edge computing performance analytics |  |
|  |  | S5‑242046 | Rel-19 pCR Use case and requirements for Traffic Steering analytics |  |
|  |  | S5‑242047 | pCR Add structure for TR 28.866 |  |
|  |  | S5‑242048 | Rel-19 pCR 28.866 Throughput and Latency map for EE |  |
| 2024-05 | SA5#155 | S5‑242507 | Rel19 pCR TR 28.866 Add potential solution for providing statistic information for fault management | 0.2.0 |
|  |  | S5‑242563 | Rel-19 pCR 28.866 Editorial improvements |  |
|  |  | S5‑242827 | Rel-19 pCR 28.866 Add use case for enhancing control plane congestion analysis |  |
|  |  | S5‑243073 | Rel-19 pCR 28.866 Software Upgrade Validation |  |
|  |  | S5‑243075 | Rel-19 pCR 28.866 solution for energy saving based on throughput requirements |  |
|  |  | S5‑243076 | Rel-19 pCR 28.866 Add use case for edge application deployment location analytics |  |
|  |  | S5‑243077 | Rel-19 pCR 28.866 update use case for edge computing performance analytics |  |
|  |  | S5‑243078 | Rel-19 pCR 28.866 Add solution for edge computing performance analytics |  |
|  |  | S5‑243079 | Rel-19 pCR 28.866 network assisted topology mapping for edge network |  |
|  |  | S5‑243080 | TR28.866 pCR add Multi-domain resource optimization use case in MDA management data correlation analytics |  |
|  |  | S5‑243081 | TR28.866 pCR add threshold assessment use case in MDA management data correlation analytics |  |
|  |  | S5‑243082 | Rel-19 pCR Usecase, requirement and solution for Handover and service data correlation analytics |  |
|  |  | S5‑243083 | Rel-19 pCR Potential solution for Traffic Steering analytics |  |
|  |  | S5‑243084 | Rel-19 pCR TR 28.866 Add use case and requirements for UE Throughput analysis |  |
|  |  | S5‑243085 | Rel-19 pCR 28.866 Add use case for network congestion analytics based on UE throughput |  |
|  |  | S5‑243086 | Rel19 pCR TR 28.866 Add UC for NF fault evaluation |  |
|  |  | S5‑243088 | Rel-19 pCR 28.866 Add solution for service failure recovery |  |
|  |  | S5‑243089 | Rel-19 pCR Potential solution for enhancing failure prediction |  |
|  |  | S5‑243325 | Rel-19 pCR Usecase, requirement and solution for extension for cell energy saving analytics |  |
| 2024-08 | SA5#156 | S5‑243950 | Rel-19 pCR TR 28.866 Add solution for UE Throughput analysis | 0.3.0 |
|  |  | S5‑244090 | Rel-19 pCR 28.866 Editorial improvements |  |
|  |  | S5‑244092 | Rel-19 pCR 28.866 Correction of energy saving terminology |  |
|  |  | S5‑244093 | Rel-19 pCR 28.866 Conclusion for service failure recovery |  |
|  |  | S5‑244620 | Rel-19 pCR 28.866 Scope and background |  |
|  |  | S5‑244622 | Rel-19 pCR TR 28.866 Evaluation and conclusion of extension of cell energy saving analytics |  |
|  |  | S5‑244624 | Rel-19 pCR 28.866 Add solution for edge application deployment location analytics |  |
|  |  | S5‑244625 | Rel-19 pCR TR28.866 adding NF Scaling&dimensioning use case in MDA management data correlation analytics |  |
|  |  | S5‑244626 | Rel-19 pCR TR28.866 adding Evaluation and conclusion for 3 management data correlation analytics use cases |  |
|  |  | S5‑244627 | Rel-19 pCR TR 28.866 Evaluation and conclusion for HO and service correlation analytics |  |
|  |  | S5‑244629 | Rel-19 pCR 28.866 Remove unnecessary attribute |  |
|  |  | S5‑244630 | Rel-19 pCR TR 28.866 Evaluation and conclusion of ATSSS analytics |  |
|  |  | S5‑244631 | Rel-19 pCR 28.866 Add solution for network congestion analytics based on UE throughput |  |
|  |  | S5‑244632 | pCR TR 28.866 Add solution for NF fault evaluation |  |
|  |  | S5‑244633 | Rel-19 pCR TR 28.866 Evaluation and conclusion of enhancement of failure prediction analytics |  |
|  |  | S5‑244640 | pCR TR 28.866 Add solution evaluation and conclusion for threshold statistics UC |  |
|  |  | S5‑244818 | Rel-19 pCR TR 28.866 Add solution for enhancing control plane congestion analysis |  |
| 2024-09 | SA#105 | SP-241138 | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  | After editHelp cleanup | 1.0.1 |
| 2024-10 | SA5#157 | S5-245221 | Rel-19 pCR TR 28.866 Add solution evaluation and conclusion for Edge computing performance analytics UC | 1.1.0 |
|  |  | S5‑245362 | Rel-19 pCR 28.866 Remove WT-5 |  |
|  |  | S5‑245363 | Rel-19 pCR 28.866 WT alignment for control plane congestion analytics |  |
|  |  | S5‑246035 | Rel-19 pCR 28.866 Improve wording of conclusions |  |
|  |  | S5‑245365 | Rel-19 pCR 28.866 Editorial improvements |  |
|  |  | S5‑246036 | Rel-19 pCR 28.866 EE Conclusions |  |