| 3GPP TR 28.879 V1.1.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on OAM for service management and exposure to external consumers (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	10

3.3	Abbreviations	10

4	Concepts and Background	10

4.1	Exposure of management services	10

4.1.1	Overview	10

4.1.2	Background of existing SA5 solutions related to exposure	11

4.1.3	Background on existing telco exposure initiatives	12

4.1.3.1	3GPP exposure framework	12

4.1.3.2	GSMA Open Gateway	14

4.1.4	Examples of external MnS consumers	15

5	Use Cases	16

5.1	Exposure of management services	16

5.1.0	Exposure of management services through the CAPIF framework	16

5.1.1	Use case #1: MnS producer registration into CAPIF	17

5.1.1.1	Description	17

5.1.1.2	Potential requirements	18

5.1.1.3	Potential solutions	19

5.1.1.3.1	Potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails	19

5.1.1.4	Evaluation of potential solutions	23

5.1.1.4.1	Evaluation of potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails	23

5.1.2	Use case #2: Publishing of management services to the CCF	23

5.1.2.1	Description	23

5.1.2.2	Potential requirements	24

5.1.2.3	Potential solutions	24

5.1.2.3.1	Potential solution #1: Publishing of management services into the CCF	24

5.1.2.3.2	Potential solution #2: Mapping MnS API URI to Service API URI.	31

5.1.2.4	Evaluation of potential solutions	31

5.1.2.4.1	Evaluation of potential solution #1	31

5.1.2.4.2	Evaluation of potential solution #2	32

5.1.3	Use case #3: Configuring discovery information of an external MnS consumer	32

5.1.3.1	Description	32

5.1.3.2	Potential requirements	33

5.1.3.3	Potential solutions	33

5.1.3.3.1	Potential solution #1: Using AccessRule class to support discovery information definition	33

5.1.3.3.2	Potential solution #1b: Using Role class for discovery information definition	34

5.1.3.3.3	Potential solution #3: Mapping policy definition into OAuth2.0 access token	34

5.1.3.4	Evaluation of potential solutions	35

5.1.3.4.1	Evaluation of potential solutions #1 and #1b	35

5.1.4	Use case #4: Authorization of the external MnS consumer to access the management service API	35

5.1.4.1	Description	35

5.1.4.2	Potential requirements	36

5.1.4.3	Potential solutions	36

5.1.4.3.1	Potential solution #1: <Potential Solution i Title>	36

5.1.4.4	Evaluation of potential solutions	41

5.1.5	Use case #5: Logging the management service API invocations to the CCF	41

5.1.5.1	Description	41

5.1.5.2	Potential requirements	41

5.1.5.3	Potential solutions	41

5.1.5.3.1	Potential solution #1: Creation and logging of the management service API invocations	41

5.1.5.4	Evaluation of potential solutions	44

5.1.x	Use case #<X>: <Use case title>	45

5.1.X.1	Description	45

5.1.X.2	Potential requirements	45

5.1.X.3	Potential solutions	45

5.1.X.3.i	Potential solution #<i>: <Potential Solution i Title>	45

5.1.X.4	Evaluation of potential solutions	45

6	Conclusions and recommendations	46

Annex A: 3GPP management capabilities	47

Annex B: Change history	49

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is to study exposure of management services to external consumers.

# 1	Scope

The present document identifies potential use cases and requirements for a generic approach for exposure of management services to external consumers. It documents and evaluates potential solutions and provides recommendations for the normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture Framework".

[3]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[4]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[5]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[6]	SP-231669: "LS on collaboration and alignment of 3GPP defined application enablers with GSMA Open Gateway".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.255: "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows".

[9]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows".

[10]	3GPP TS 23.545: "Application layer support for Factories of the Future (FF)".

[11]	3GPP TS 23.542: "Application layer support for Personal IoT Networks".

[12] 	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[13]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

[14]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[15]	"The Ecosystem for Open Gateway NaaS API Development", white paper, June 2023.

[16]	GSMA Official Document OPG.02: "Operator Platform Group:- Requirements and Architecture", version 5.0, 26 July 2023.

[17]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[18]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[19]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[20]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[21]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[22]	3GPP TS 28.111: "Fault Management".

[23]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/Machine Learning (AI/ML) management".

[24]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[25]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and 3".

[26]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[27]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[28]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

[29]	3GPP TS 28.319: "Management and orchestration; Access Control for Management services".

[30]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[31]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[32]	IETF RFC 7519: "JSON Web Token (JWT)".

[33]

3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MnS consumer: Defined in 3GPP TS 28.533 [2].

MnS producer: Defined in 3GPP TS 28.533 [2].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core

AAA	Authentication, Authorization and Accounting

AEF	API Exposing Function

AMF	API Management Function

APF	API Publishing Function

CAPIF	Common API Framework

CCF	CAPIF Core Function

CSP	Communication Service Provider

EAS	Edge Application Server

ECS	Edge Configuration Server

EDN	Edge Data Network

EEC	Edge Enabler Client

EES	Edge Enabler Server

FF	Factories of the Future

GSMA	GSM Association

MnS	Management Service

NOTE:	See 3GPP TS 28.533 [2].

NaaS	Network as a Service

NEF	Network Exposure Function

NOP	Network Operator

NSACF	Network Slice Access Control Function

NSCE 	Network Slice Capability Enablement

NWDAF	Network Data Analytics Function

OAM	Operation, Administration and Maintenance

OPAG	Operator Platform API Group

OPG	Operator Platform Group

SEAL	Service Enabler Abstraction Layer

UAS	Uncrewed Aerial Systems

V2X	Vehicle-to-Everything

VAE	Vertical App Enabler

WAS	Whole Agreement Services

WG	Working Group

# 4	Concepts and Background

## 4.1	Exposure of management services

### 4.1.1	Overview

The present document studies a generic approach to expose management services to external MnS consumers. An external MnS consumer is a MnS consumer that has discovered an MnS via a discovery mechanism which is not defined in 3GPP SA5.

NOTE:	CAPIF [5] is an example of a framework providing such discovery mechanism.

A management service (MnS) is identified by different component types, i.e. MnS component type A (management operations or notifications), MnS component type B (managed objects), and MnS component type C (performance and fault information) (as defined in clause 4.2 of 3GPP TS 28.533 [2]). Accordingly, in order for the external MnS consumer to consume management services, the access rights have to be configured for the external MnS consumer. These access rights determine which MnS resources (i.e. component type B and component type C) can be accessed. In addition, the access rights also determine what operations (i.e. MnS component A) can or cannot be performed on the MnS resources. For example, considering Figure 4.1.1-1, the MnS A producer produces MnS A that is directly consumed by the internal MnS consumer and the external MnS consumers 1 and 2 respectively. Depending on the different access rights assigned to the external MnS consumers 1 and 2, it's possible that:

-	The internal MnS Consumer and external MnS Consumer 1 and 2 can access the same or different managed objects under the management scope of MnS A producer.

-	The operations (i.e. CRUD) or notifications that internal MnS Consumer and external MnS Consumer 1 and 2 can perform on the accessed managed objects are the same or different.

-	The internal MnS Consumer and external MnS Consumer 1 and 2 can or cannot access the same or different performance and fault information associated with the accessible managed objects of MnS A.

Figure 4.1.1-1: Example of Exposing Management Services concept

To provide a MnS to be consumed, it first needs to be published to the MnS discovery producer entity. After it is published, the MnS will be available to be discovered by the external MnS consumers. Following discovery, authentication and authorization mechanisms need to be applied to ensure that the external MnS consumers only have access to the allowed MnS component type A, B, or C for the MnS.

### 4.1.2	Background of existing SA5 solutions related to exposure

This study will focus on how the external MnS consumer can leverage external exposure frameworks (e.g. the CAPIF framework [5]) to access and consume management services with regards to all the stages involved: publishing of new MnS, discovery of MnS and access control for MnS.

A MnS can be available (i.e. instantiated, and ready to be exposed) and hence discoverable by internal and external MnS consumers using 3GPP standard as defined in following specifications:

-	An available MnS is described by the MnsInfo IOC (see clause 4.3.42 of 3GPP TS 28.622 [3]).

-	The available MnSs in the 3GPP management system can be found in the MnSRegistry IOC (see clause 4.3.41 of 3GPP TS 28.622 [3]).

-	The use case procedures for discovery are described in clause 5 of 3GPP TS 28.537 [4].

The access control framework is defined in 3GPP TS 28.319 [29]:

-	The concept and solution for access control are described in clause 4 and clause 7 of 3GPP TS 28.319 [29].

### 4.1.3	Background on existing telco exposure initiatives

#### 4.1.3.1	3GPP exposure framework

3GPP standard exposure technologies hide the complexity of 5G and offer 3rd party applications a simple, secure, use-case-oriented configuration interface to the 5G system. The exposure interfaces will be quite valuable to a multitude of industrial use cases (i.e. non-telco use cases, with requirements beyond secure and reliable connectivity), allowing industry verticals to make use of the key features and performance that 5G has to offer in a simple and straightforward manner.

The development of solutions for exposure in 3GPP is led by SA6 WG. Figure 4.1.3.1-1 pictures the latest status of the 3GPP exposure framework. As seen, the framework consists of a set of Application Enablement Services ("Edge", "SEAL", "Vertical App Enabler") that pursue one mission: to provide means to 3rd parties to rapidly develop and deploy new vertical-oriented applications ("Application Layer") over 3GPP system ("Network and OAM/CH"). To that end, Application Enablement services offer industry-tailored APIs ("Northbound Interface - service APIs") that build on 3GPP system APIs ("Southbound Interface - network APIs"). All these APIs are documented in 3GPP Technical Specifications and made available under the 3GPP GitLab repository.

Figure 4.1.3.1-1: 3GPP exposure framework
(Source: SP-231669 [6])

SA6 WG realized soon that managing all these APIs, especially in an environment where 3rd parties are developing applications, will also require a management layer that enforces the strong security policies defined by SA3 WG (e.g. Mutual TLS Authentication). This is where "CAPIF" comes to the picture. Common API Framework (CAPIF) is defined to manage access to all 3GPP APIs, in a consistent and uniform way with regards to publication, discovery and access control, among other functionalities. The operator can use CAPIF as an entry point for the Application Layer to gain access to 3GPP APIs, including not only the APIs offered by Application Enablement services (SA6), but also the APIs offered by the 3GPP system (SA2 and SA5).

Table 4.1.3.1-1 provides a more detailed description of all the components mentioned above.

Table 4.1.3.1-1: Components of the 3GPP exposure framework

| Component | Description |
| --- | --- |
| 3GPP system | Also referred to "5G Network Services" in Figure 4.1.3.1-1, it includes: - Network Services: groups all the capabilities related to 3GPP SA2. These are made available through Core network functions, e.g. Network Exposure Function (NEF). - OAM/CH services: groups all the capabilities related to 3GPP SA5. These capabilities are offered through MnSs by producers. |
| Edge Services | It includes services for hosting edge computing applications, while consolidating edge computing standardization in 3GPP. These services provide various capabilities such as rich discovery of the Edge Application Servers (EAS), service continuity between multiple Edge Data Networks (EDN), interworking with the core network, and APIs for EASs to integrate with the edge hosting environments. The main components building out this layer are specified in 3GPP TS 23.558 [20]: - Edge Enabler Server (EES), primarily responsible for enabling discovery of EASs. - Edge Enabler Client (EEC), providing support functions, such as EAS discovery to the application clients in the device. - Edge Configuration Server (ECS), providing configurations to the EEC to connect with targeted EAS(s). See note. |
| Service Enabler Abstraction Layer (SEAL) | It provides a set of core services that are common to industry verticals. The motivation is largely to avoid redefining the individual services for each vertical industry, thereby lowering the deployment costs for operators, and significantly reducing the barrier of adoption and the time-to-market for integrating new verticals to the 3GPP ecosystem. SEAL services are specified in 3GPP TS 23.434 [7], and include location management, group management, configuration management, identity management, key management, network resource management, data delivery, notification management, Network Slice Capability Enablement (NSCE) and application data analytics enablement. |
| Vertical App Enabler (VAE) | It provides vertical-specific service enablers. In contrast to the SEAL, VAE targets service specific vertical applications. As of today, the verticals service enablers are defined for: - Automotive applications referred to as Vehicle-to-anything (V2X) communications, see 3GPP TS 23.255 [8]. - Drone applications known as Uncrewed Aerial Systems (UAS), see 3GPP TS 23.286 [9]. - Industry 4.0/OT applications, also referred to as Factories of the Future (FF), see 3GPP TS 23.545 [10] - Personal IoT networks, see 3GPP TS 23.542 [11]. - Message communication in massive IoT, also referred to as MSGIn5G, see 3GPP TS 23.554 [12]. |
| Common API Framework (CAPIF) | Started in Release 15, CAPIF services are listed in 3GPP TS 23.222 [5] and specified in 3GPP TS 29.222 [13] with security aspects being addressed in 3GPP TS 33.122 [14]. It provides a unified Northbound API framework across network/application functions, to facilitate a harmonized approach for API exposure within 3GPP. This framework builds upon three main components: - API invokers: they represent consumers of 3GPP APIs. - CAPIF Core Function (CCF): responsible for managing onboarding of API invokers, and access control (authentication, authorization) when trying to gain access to 3GPP APIs. - API provider domain: collection of functions (discovery, registration, publishing, auditability) required to allow authorized API invokers to consume 3GPP APIs. They implement agents that allow API producers to make APIs available through CAPIF. |
| Application Layer | This layer represents the 3rd party applications that want to gain access (discover and consume) 3GPP APIs, to develop and deploy new vertical services. |
| NOTE: The Edge Computing Layer only consumes network capability set from the 3GPP system (see Figure 4.1.3.1-1) |  |

#### 4.1.3.2	GSMA Open Gateway

The development of telco capability exposure (also coined "Network as a Service" (NaaS)) requires a collaborative workspace that bring together incumbent telco standard bodies with IT/cloud communities, industry associations and open-source projects. An effective collaboration among organizations needs to be based on a clear demarcation on their scope of work, avoiding their participating organizations running overlapping activities or duplicate efforts; otherwise, NaaS may risk ending up with a fragmented ecosystem. To that end, the GSM Association (GSMA) launched Open Gateway in MWC Barcelona 23. GSMA Open Gateway mission is twofold:

i)	to provide a governance framework for NaaS, covering technical and business aspects;

ii)	to get operator commitment to launch universal NaaS API services in 2023.

Open Gateway initiative recognizes that NaaS the concept builds on the work developed by three organizations - see Table 4.1.3.2-1. The role of the different organizations and their relationship is graphically shown in Figure 4.1.3.2-1.

Table 4.1.3.2-1: Organizations participating in GSMA Open Gateway initiative

| Organization | Description of Activity |
| --- | --- |
| Linux Foundation's CAMARA | it represents the "exposure" doctrine, i.e. how capabilities are exposed for external consumption through 3rd party facing APIs. CAMARA defines these APIs and is responsible for their hosting and release management. 3rd party facing APIs are dev-friendly (semantics tailored to service and business needs of 3rd parties) and open (following Apache2.0 license). |
| GSMA | it represents: i) the "technical" doctrine, by specifying how 3rd party facing APIs are to be supported by underlying telco capabilities; and ii) the "business" doctrine, with the definition of agreement templates for federation between the operator networks and for relationship with 3rd parties, ensuring a consistent yet fair commercial framework for exposing services. GSMA conducts the technical workstream through OPG/OPAG (Operator Platform Group / Operator Platform API Group) and the business workstream through WAS (Wholesale Agreement Services) group. |
| TM Forum | It represents the "operational" doctrine, i.e. how 3rd party facing APIs are to be operated and managed, to make a commercial product out of them. Aspects such as 3rd party onboarding, application registration, access control and billing aspects are in scope. |

On the one hand, scope of "GSMA" is restricted to the telco domain. GSMA prescribes the capabilities that all operators can make available for 3rd parties, to ensure global reach and scale. These capabilities are referred to as Open Gateway services. The GSMA is also responsible for:

i)	the prioritization and roadmap management of Open Gateway services, according to market needs and commercial readiness of underlying technologies; and

ii)	architecting the platform that individual operators will use to realize, federate, and expose Open Gateway services.

On the other hand, the focus of "CAMARA" and "TM Forum is on the dev-friendly APIs that allows programmatic access to Open Gateway services. As seen in the bottom of Figure 4.1.3.2-1, these APIs can be clustered into three groups:

-	service APIs;

-	service management APIs; and

-	operate APIs.

For further information on these APIs, see [15].

Each Communication Service Provider (CSP) exposes APIs through the Open Gateway Transformation Function (Figure 4.1.3.2-1). Deployed as an internal component of GSMA Operator Platform [16] the Open Gateway Transformation Function is tasked with defining and enforcing the mapping between the dev-friendly APIs (towards 3rd party applications) and network APIs (towards 3GPP system). The mapping logic is not standardized, though GSMA provide non-prescriptive guidelines on a per API basis. For the interaction with OAM systems, the white paper in [15] notes that transformation function may interact with TM Forum (Open API portfolio) and 3GPP SA5 (management services).

Figure 4.1.3.2-1: GSMA Open Gateway ecosystem

### 4.1.4	Examples of external MnS consumers

Figure 4.1.4-1 provides examples of functional entities that can become external MnS consumers. Table 4.1.4-1 elaborates on the rationale.

Figure 4.1.4-1: Examples of external MnS consumers

Table 4.1.4-1: Examples of external MnS consumers

| Functional Entity | Justification |
| --- | --- |
| Application Layer Server | Any 3rd party application that gains access (discover and consume) to MnSs using a discovery mechanism defined outside SA5 is an external MnS consumer. The logic of this application is on the 3rd party and outside standardization. |
| SEAL's NSCE server | Network Slice Capability Enablement (NSCE) is a SEAL service that provides add-on slicing capabilities to vertical customers' applications. NSCE has a server and multiple clients (installed on vertical customer's devices). NSCE server consumes slicing capabilities related to OAM (i.e. MnSs) and 5G network services (i.e. NEF APIs, NWDAF APIs, NSACF APIs), and process them (aggregation, abstraction, filtering, etc.) in order to build vertical-oriented slicing functionality to applications. The set of operations/notifications related to OAM that are eligible for consumption by NSCE server are specified in 3GPP TS 28.531 [18], and conceptually grouped under the NSCE-OAM interface in 3GPP TS 23.435 [19]. To gain access to these capabilities, NSCE server can use a discovery mechanism defined outside SA5. In this regard, the NSCE server becomes an external (network slice / network slice subnet) MnS consumer. |
| Open Gateway Transformation Function | Open Gateway services (defined by GSMA) are offered through dev-friendly APIs (specified and maintained by CAMARA and TM Forum). Some services provisioning and monitoring actions on 5G managed resources, including network slicing. In such a case, the invocation of these dev-friendly APIs needs to be mapped into one or more calls to MnSs. The Open Gateway Transformation Function is in charge of this mapping and MnS invocation. To that end, the Open Gateway Transformation needs to be able to discover MnS. In this regard, one can realize that the Open Gateway Transformation Function complies with the external MnS consumer when it gains access to MnSs using a discovery mechanism defined outside SA5. |

It is worth noting that the functional entities represented as examples of external MnS consumers:

-	Provides a non-exhaustive list; the only aim is to provide clarity on how external MnS consumer concept fits with the background of telco exposure initiatives reported in the background.

-	Are all optional; the decision to deploy these functional entities or not is up to operator discretion.

-	Perform the role of "API invokers", when the discovery mechanism these entities use to gain access to MnSs is the mechanism provided by CAPIF.

# 5	Use Cases

## 5.1	Exposure of management services

### 5.1.0	Exposure of management services through the CAPIF framework

To expose management services through the CAPIF framework, the 3GPP management system should define the functional entity to provide the API provider domain functions (i.e., API Exposing Function (AEF), API management function (AMF), API publishing function (APF)) as defined by the CAPIF framework in clause 6.3 of TS 23.222[5].

For the present document, the functional entity is called the management services exposure function (MSEF). The MSEF can consume CCF APIs (i.e., CAPIF-3, CAPIF-4 and CAPIF-5 interfaces) and allow API invokers to access management services through service APIs (using CAPIF-2/2e interface).

Figure 5.1.0-1: MSEF implementing the API provider domain functions as defined by the CAPIF framework.

The MSEF implements the CAPIF API provider domain functions, and the following can apply for this functional entity:

-	The MSEF is a non-binding name. The final name will be discussed on normative phase.

-	The MSEF is not a management function (MnF) as defined in TS 28.533[2] because:

-	The MSEF requires implementing CAPIF interfaces, which are not defined in the 3GPP management system.

-	The MSEF publishes the management service APIs for discovery through the CAPIF core function (CCF) and exposes the management services for invocation by external MnS consumers through the AEF.

-	The MSEF is not an MnS producer as defined in TS 28.533[2].

-	The possible deployment options of the MSEF will be agreed at the normative phase.

-	The MSEF should be able to expose one or more management services.

-	Whether the MSEF needs to implement the required mappings between SA5 and SA6 data types will be discussed in normative phase.

Editor’s Note 1: The content in this clause is subject to change

Editor's Note 2: Clarification on how the MSEF differs from the EGMF and if EGMF can be enhanced is FFS.

### 5.1.1	Use case #1: MnS producer registration into CAPIF

#### 5.1.1.1	Description

For a MnS to be made available for consumption through an exposure framework, it is needed that this MnS producer registers into that exposure framework. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS from external MnS consumers. These procedures include publication, discovery and access control, among others.

In the CAPIF, the scope of registration is limited to API provider domain functions, which include API Exposing Function (AEF), API Publishing Function (APF) and API Management Function (AMF). The registration is a procedure whereby these functions become recognized users of the CCF. This procedure is described in clause 8.28 of 3GPP TS 23.222 [5], with stage 3 solution set detailed in clause 8.9 of 3GPP TS 29.222 [13].

To register (and/or update the registration information of) an API provider domain on the CCF, the AMF from this API provider domain communicates with the CCF over CAPIF-5 interface. This means that:

-	This API provider domain should have an AMF.

-	The AMF of this API provider domain should be an authorized user of CCF.

-	The AMF security information for CCF to validate the registration request is provisioned by the CAPIF administrator.

To register a MnS producer on the CCF, it is needed for the MnS producer to support some API provider domain functionality, as detailed below:

-	AEF. If supported, this means that the MnS producer will need to support:

-	CAPIF-2/2e interface, so that the API invokers acting as external MnS consumers can access service APIs, when required. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clause 9.1) can be configured later, depending on the intended usage of MnS producer.

NOTE:	The service APIs can carry different MnS information for different API invokers. This issue is discussed in use case #3 (clause 5.1.3).

-	CAPIF-3 interface, so that the MnS producer can communicate with CCF to exercise access and policy related control for service API invocations initiated by the API invoker. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clauses 8.3, 8.5, 8.7 and 8.6) can be configured later, depending on the intended usage of MnS producer.

-	APF. If supported, this means that the MnS producer will need to support CAPIF-4 interface, so that it can communicate with CCF to publish (and manage the published) MnS information. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clause 8.2) can be configured later, depending on the intended usage of MnS producer.

-	AMF. If supported, this means that MnS producer will need to support CAPIF-5 interface. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clauses 8.3 and 8.9) can be configured later, depending on the intended usage of MnS producer.

The MnS producer could support one or more API provider domain functions, with at least one of them being AEF; otherwise, the MnS producer is not eligible for integration into the CAPIF.

The AMF communicates the MnS producer details (including supported API provider domain functions) to the CCF using CAPIF_API_Provider_Management_API (see 3GPP TS 29.222 [13], clause 8.9). Upon reception of these details, the CCF registers the MnS producer with the supported API provider domain function(s).

#### 5.1.1.2	Potential requirements

PREQ-FS_MExpo-Reg-01: The 3GPP management system shall provide the capability to register a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-02: The 3GPP management system shall provide the capability to deregister a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-03: The 3GPP management system shall provide the capability to update the registration details of a MnS producer on the CCF.

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	Potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

5.1.1.3.1.1	Introduction

This solution focuses on meeting the requirements PREQ-FS_MExpo-Reg-01, PREQ-FS_MExpo-Reg-02 and PREQ-FS_MExpo-Reg-03.

In the CAPIF, the registration procedure is executed with the CAPIF_API_Provider_Management API (see 3GPP TS 29.222 [13], clause 8.3), initiated by the AMF over the CAPIF-5 interface. The AMF sends a HTTP POST message to the CCF with a request body containing the following dataType: "APIProviderEnrolmentDetails" (see 3GPP TS 29.222 [13], clause 8.9.4.2.2).

The solution proposes to capture MnS producer information with "APIProviderEnrolmentDetails" and reuse the CAPIF_API_Provider_Management API to register, de-register and update registration information related to MnS producer.

5.1.1.3.1.2	Description

How the MnS producer information relevant for registration can be captured with "APIProviderEnrolmentDetails" is described in tables 5.1.1.3.1.2-1, 5.1.1.3.1.2-2 and 5.1.1.3.1.2-3.

Table 5.1.1.3.1.2-1 lists the attributes of type APIProviderEnrolmentDetails (see clause 8.9.4.2.2 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.9.4.2.2-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type APIProviderEnrolmentDetails.

Table 5.1.1.3.1.2-1: Representing MnS producer registration information with
APIProviderEnrolmentDetails attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiProvDomId | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). |  |
| regSec | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | It can be used to store the credentials of the MnS producer. |
| apiProvFuncs | The data type of this attribute is defined as "array(APIProviderFunctionDetails)" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | See Table 5.1.1.3.1.2-2. |
| apiProvDomInfo | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). |  |
| suppFeat | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "C" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | Only applicable when API provider is SCEF/NEF, and thus not applicable when API provider is MnS producer. |
| failReason | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.9.4.2.2-1 of TS 29.222 [13]). |  |
| apiProvName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | RNAA. |

Table 5.1.1.3.1.2-2 lists the attributes of type APIProviderFunctionDetails (see clause 8.9.4.2.3 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.9.4.2.3-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type APIProviderFunctionDetails.

Table 5.1.1.3.1.2-2: Representing MnS producer registration information with
APIProviderFunctionDetails attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiProvFuncId | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.9.4.2.3-1 of TS 29.222 [13]). |  |
| regInfo | The data type of this attribute is defined as "RegistrationInformation" and presence qualifier is defined as "M" (see table 8.9.4.2.3-1 of TS 29.222 [13]). | See Table 5.1.1.3.1.2-3. |
| apiProvFuncRole | The data type of this attribute is defined as "ApiProviderFuncRole" and presence qualifier is defined as "M" (see table 8.9.4.2.3-1 of TS 29.222 [13]). | This data type serves to specify, for the MnS producer to be registered, which CAPIF API provider domain function(s) will be supported. |
| apiProvFuncInfo | See clause 8.9.4.3.3 of TS 29.222 [13] for the enumeration values and description of type ApiProviderFuncRole. |  |

Table 5.1.1.3.1.2-3 lists the attributes of type RegistrationInformation (see clause 8.9.4.2.4 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.9.4.2.4-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type RegistrationInformation.

Table 5.1.1.3.1.2-3: Representing MnS producer registration information with
RegistrationInformation attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiProvPubKey | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.9.4.2.4-1 of TS 29.222 [13]). | It can be used to store the public key of the MnS producer. |
| apiProvCert | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.4-1 of TS 29.222 [13]). | It can be used to store the client certificate of the MnS producer, if existing. |

#### 5.1.1.4	Evaluation of potential solutions

##### 5.1.1.4.1	Evaluation of potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

To manage the registration of an MnS producer on the CCF using CAPIF_API_Provider_Management API, it is needed to populate the "APIProviderEnrolmentDetails" datatype with the registration information of the MnS producer. Detailed in clause 5.1.1.3.1.2, the mapping solution is summarized below:

-	The credentials of the MnS producer can be mapped to "APIProviderEnrolmentDetails /regSec".

-	The different roles played for the MnS producer can be mapped to "APIProviderEnrolmentDetails /apiProvFuncs". Note that the MnS producer will always play AEF, with the option to also play APF and/or AMF, if needed.

-	The public key of the MnS producer can be mapped to "APIProviderEnrolmentDetails/ apiProvFuncs/regInfo/apiProvPubKey".

The solution is feasible.

### 5.1.2	Use case #2: Publishing of management services to the CCF

#### 5.1.2.1	Description

Upon registration of MnS producer into the CAPIF, then the MnS producer can be enabled to decide what management service information will be made available for external consumption (i.e. the entire MnS, or a subset of it).

Once this decision is made, then this information needs to be published into CCF, so that it can later be used by the external MnS consumers to discover and subsequently invoke the MnS. To this end, the following should occur:

-	First, the management services are mapped into service APIs that can be published to the CCF. This is due to the fact that the information published into CAPIF is information related to service APIs. This information is represented with "ServiceAPIDescription" (see clause 8.2.4.2.2 in 3GPP TS 29.222 [13]). However, in the 3GPP management system, the IOC that describes the information related to an MnS's is MnSInfo IOC (see clause 4.3.42 in 3GPP TS 28.622 [3]). The MnS producer posts and manage MnSInfo instances into MnSRegistry IOC (see clause 4.3.41 in 3GPP TS 28.622 [3]). In this regard, there is a need to map MnSInfo elements into serviceAPIDescription and to determine who in the 3GPP management system does the map.

-	Secondly, the mapped management services (now service APIs) will be published in the CCF. In CAPIF, the API Publishing Function (APF) is the API provider domain's entity responsible for publishing and managing service API information (see clause 6.3.5 in 3GPP TS 23.222 [5]). The APF supports the CAPIF-4 interface (see clause 6.4.7 in 3GPP TS 23.222 [5]). However, in the 3GPP management system, there is a need to determine who will play the APF role, for example:

-	MnS registry (represented by the MnSRegistry IOC): The issue is that MnS registry is a managed object and therefore as a managed object, it is not capable of either the mapping of management service information into service API information or the actual publishing of the mapped management service information to the CCF. Accordingly, the MnSRegistry IOC cannot play the role of the CAPIF APF. There is an option of defining a new Management Function (MnF) that consumes the MnSInfo IOCs data from MnSRegistry IOC and, in turn, takes on the role of APF and publishes the management services to the CCF via the CAPIF-4 interface. This option has the advantage of simultaneously publishing all the management services to the CCF. However, considering SBMA principles, the definition of a new MnF would mean going back to IRP definitions.

-	MnS producer: the MnS producers are already capable of providing the functionalities offered by the CAPIF APFs. Accordingly, the MnS producers can be enhanced to support the CAPIF-4 interface.

#### 5.1.2.2	Potential requirements

PREQ-FS_MExpo-Pub-01: The 3GPP management system shall have the capability to map management services information (described by the MnSInfo IOC) into service API information (described by the CAPIF ServiceAPIDescription information element).

PREQ-FS_MExpo-Pub-02: The 3GPP management system shall have the capability to publish the mapped management service information to the CCF.

#### 5.1.2.3	Potential solutions

##### 5.1.2.3.1	Potential solution #1: Publishing of management services into the CCF

5.1.2.3.1.1	Introduction

This potential solution describes how to publish the management services to the CCF.

5.1.2.3.1.2	Description

To publish a management service to the CCF, there is a need to map the MnsInfo IOC attributes into the ServiceAPIDescription data type. The potential solution for this mapping is given in Table 5.1.2.3.1.2-1.

Table 5.1.2.3.1.2-1 lists the attributes of type ServiceAPIDescription (see clause 8.2.4.2.2 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.2.4.2.2-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type ServiceAPIDescription.

Table 5.1.2.3.1.2-1: Mapping of MnSInfo information attributes to
the ServiceAPIDescription information elements

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| apiName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | Corresponds to the following IOC attribute: mnsType |
| apiId | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.2-1 of TS 29.222 [13]). |  |
| aefProfiles | The data type of this attribute is defined as " array(AefProfile)" and presence qualifier is defined as "C" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | See Table 5.1.2.3.1.2-2 |
| description | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.2-1 of TS 29.222 [13]). |  |
| supportedFeatures | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS. |
| shareableInfo | The data type of this attribute is defined as "ShareableInformation" and presence qualifier is defined as "O" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS. |
| serviceAPICategory | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.2.4.2.2-1 of TS 29.222 [13]). |  |
| ccfId | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS. |
| apiSuppFeats | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS. |
| pubApiPath | The data type of this attribute is defined as "PublishedApiPath" and presence qualifier is defined as "C" (see table 8.2.4.2.2-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS. |

Table 5.1.2.3.1.2-2 lists the attributes of type AefProfile (see clause 8.2.4.2.4 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.2.4.2.4-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for attributes of type AefProfile.

Table 5.1.2.3.1.2-2: Transformation of MnSInfo IOC attributes to AefProfile data type attributes

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| aefId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.2.4.2.4-1 of TS 29.222 [13]). | Corresponds to the AEF identifier provided by the CCF upon MnS producer registration (use case 5.1.1). |
| versions | The data type of this attribute is defined as "array(Version)" and presence qualifier is defined as "M" (see table 8.2.4.2.4-1 of TS 29.222 [13]). | See Table 5.1.2.3.1.2-3. |
| protocol | The data type of this attribute is defined as "Protocol" and presence qualifier is defined as "O" (see table 8.2.4.2.4-1 of TS 29.222 [13]). | Only "HTTP_1_1" and "HTTP_1_2" are applicable in the context of SA5 MnS. |
| dataFormat | The data type of this attribute is defined as "DataFormat" and presence qualifier is defined as "O" (see table 8.2.4.2.4-1 of TS 29.222 [13]). | Only "JSON" value is applicable in the context of SA5 MnS. |
| securityMethods | The data type of this attribute is defined as "array(SecurityMethod)" and presence qualifier is defined as "O" (see table 8.2.4.2.4-1 of TS 29.222 [13]). | Only "OAUTH" value (i.e. TLS with OAuth token) is applicable in the context of SA5 MnS. |
| domainName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.4-1 of TS 29.222 [13]). |  |
| interfaceDescriptions | The data type of this attribute is defined as "array(InterfaceDescription)" and presence qualifier is defined as "O" (see table 8.2.4.2.4-1 of TS 29.222 [13]). | See Table 5.1.2.3.1.2-5. |
| aefLocation | The data type of this attribute is defined as "AefLocation" and presence qualifier is defined as "O" (see table 8.2.4.2.4-1 of TS 29.222 [13]). |  |

Table 5.1.2.3.1.2-3 lists the attributes of type Version (see clause 8.2.4.2.5 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.2.4.2.5-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for attributes of type Version.

Table 5.1.2.3.1.2-3: Mapping of MnSInfo IOC attributes to Version datatype

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| apiVersion | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.2.4.2.5-1 of TS 29.222 [13]). | Corresponds to the following MnSInfo IOC attribute: mnsVersion |
| expiry | The data type of this attribute is defined as "DateTime" and presence qualifier is defined as "O" (see table 8.2.4.2.5-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS. |
| resources | The data type of this attribute is defined as "array(Resource)" and presence qualifier is defined as "O" (see table 8.2.4.2.5-1 of TS 29.222 [13]). | See Table 5.1.2.3.1.2-4. Each resource corresponds to a service API endpoint. |
| custOperations | The data type of this attribute is defined as "array(CustomOperation)" and presence qualifier is defined as "O" (see table 8.2.4.2.5-1 of TS 29.222 [13]). |  |

Table 5.1.2.3.1.2-4 lists the attributes of type Resource (see clause 8.2.4.2.6 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.2.4.2.6-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for attributes of type Resource.

Table 5.1.2.3.1.2-4: Mapping of MnSInfo IOC attributes to Resource data type

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| resourceName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.2.4.2.6-1 of TS 29.222 [13]). | {className}. |
| commType | The data type of this attribute is defined as "CommunicationType" and presence qualifier is defined as "M" (see table 8.2.4.2.6-1 of TS 29.222 [13]). | Only "REQUEST_RESPONSE" value is applicable for SA5 MnS of type Provisioning. |
| uri | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.2.4.2.6-1 of TS 29.222 [13]). | Corresponds to the following URI component in a MnS endpoint: {URI-LDN-first-part}/{className} = {id}. |
| custOpName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.6-1 of TS 29.222 [13]). |  |
| custOperations | The data type of this attribute is defined as "array(CustomOperation)" and presence qualifier is defined as "O" (see table 8.2.4.2.6-1 of TS 29.222 [13]). |  |
| operations | The data type of this attribute is defined as "array(Operation)" and presence qualifier is defined as "C" (see table 8.2.4.2.6-1 of TS 29.222 [13]). | HTTP methods specified under {URI-LDN-first-part}/{className} = {id}. |
| description | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.6-1 of TS 29.222 [13]). |  |

Table 5.1.2.3.1.2-5 lists the attributes of type InterfaceDescription (see clause 8.2.4.2.3 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.2.4.2.3-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for attributes of type InterfaceDescription.

Table 5.1.2.3.1.2-5: Mapping of MnSInfo IOC attributes to InterfaceDescription datatype

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| ipv4Addr | The data type of this attribute is defined as "Ipv4Addr" and presence qualifier is defined as "C" (see table 8.2.4.2.3-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS |
| ipv6Addr | The data type of this attribute is defined as "Ipv6Addr" and presence qualifier is defined as "C" (see table 8.2.4.2.3-1 of TS 29.222 [13]). | Not applicable in the context of SA5 MnS |
| fqdn | The data type of this attribute is defined as "Fqdn" and presence qualifier is defined as "C" (see table 8.2.4.2.3-1 of TS 29.222 [13]). | Corresponds to the following URI component in a MnS: {URI-DN-prefix} This FQDN can be constructed from the DN prefix as detailed in clause 4.2.3, 3GPP TS 32.158 [30]. |
| port | The data type of this attribute is defined as "Port" and presence qualifier is defined as "O" (see table 8.2.4.2.3-1 of TS 29.222 [13]). |  |
| apiPrefix | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.3-1 of TS 29.222 [13]). | Corresponds to the following URI component in a MnS: "/" + {root} |
| securityMethods | The data type of this attribute is defined as "array(SecurityMethod)" and presence qualifier is defined as "O" (see table 8.2.4.2.3-1 of TS 29.222 [13]). | Only "OAUTH" value (i.e. TLS with OAuth token) is applicable in the context of SA5 MnS. |

With this mapping, the management service information can be represented by the service API description information element.

##### 5.1.2.3.2	Potential solution #2: Mapping MnS API URI to Service API URI.

5.1.2.3.2.1	Introduction

When publishing on the CCF, MnS APIs need to be mapped to service APIs, so that the latter can be accessed by external MnS consumers. This potential solution compares the URI structure of a MnS API and a service API, to help understand how this mapping looks like.

5.1.2.3.2.2	Description

The table below compares the URI structure for Service API and MnS API.

Table 5.1.2.3.2.2-1: URI structure for Service API and MnS API

| Service API | MnS API (see 3GPP TS 32.158) |
| --- | --- |
| URI: = <apiRoot>/<apiName>/<apiVersion>/<APISpecificSuffixes> See NOTE 1 | URI: = {MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN-first-part}/{className} = {id} See NOTE 2 |
| NOTE 1: <apiRoot>:= https://<authority>/<API-prefix>, with <API-prefix> being optional. NOTE 2: {MnSRoot} = https://{URI-to-DN-prefix}/{root}, with {root} being optional. |  |

It is needed that the URI structure for MnS APIs follow the same format as defined for service APIs. This means:

-	{MnSRoot} corresponds to the <apiRoot>. The {apiRoot} variable of the URI structure for the service API can be constructed by the API invoker based on the "interfaceDescriptions" attribute of the AefProfile data type (Table 5.1.2.3.1.2-2). For further information, see clause 5.2.2.2.2 in 3GPP TS 29.222 [13].

-	{URI-to-DN-prefix} corresponds to the <authority> (host and optional TCP port). The host name is constructed from the DN prefix as detailed in clause 4.2.3, 3GPP TS 32.158 [30].

-	{root} corresponds to <API-prefix>.

-	{MnSName} corresponds to <apiName>. For example, when <MnSName> := ProvMnS, then the apiName in the service API URI shall be ProvMnS.

-	{MnSVersion} corresponds to <apiVersion>.

-	{URI-LDN-first-part}/{className} = {id} corresponds to <apiSpecificSuffixes>

#### 5.1.2.4	Evaluation of potential solutions

##### 5.1.2.4.1	Evaluation of potential solution #1

The potential solution #1 shows that it is feasible to map MnSInfo IOC attributes into ServiceAPIDescription data type attributes. The solution fulfils PREQ- PREQ-FS_MExpo-Pub-01.

For discovery purposes, the external MnS consumer can read the following attribute of the ServiceAPIDescription data type:

-	"resources" (see Table 5.1.2.3.1.1-4). This attribute provides information on the scope of a service API, i.e. constituent service API resources. This solution requires that the external MnS consumer understand the NRM tree.

##### 5.1.2.4.2	Evaluation of potential solution #2

The potential solution #2 shows that the URI structure of MnS APIs can be mapped into the URI structure of service APIs.

The potential solution identifies that {MnSVersion} in MnS corresponds to <apiVersion> in service API. However, it is worth noting that <apiVersion> represents only the major release, e.g. "v1", while the <MnSVersion> follow "v<major><minor><patch>" format, e.g. "v1501". It shall be clarified how the versioning format in MnS API matches with the major release format in service API.

### 5.1.3	Use case #3: Configuring discovery information of an external MnS consumer

#### 5.1.3.1	Description

The operator may not want that all service APIs published into CAPIF are discoverable to all API invokers; in fact, the operator may want to limit the visibility that certain API invokers have over published API information, according to the business agreements settled with the stakeholder owning the API invoker. To that end, the CCF can be configured with discovery information on a per API invoker basis. The discovery information, allows to specify the visibility for an API invoker, and therefore filter which service API information this API invoker can:

-	Subscribe to, during the API invoker onboarding.

-	Discover and subsequently access, once API invoker gets onboarded. Upon receiving a discovery request from the API invoker with certain query information (i.e. criteria for discovery matching service APIs), the CCF will apply the configured discovery information on the search results matching the query criteria and filter them accordingly.

When using the CAPIF as the framework to expose MnSs, the external MnS consumer plays the role of API invoker, and the published API information corresponds to the MnS information published into CCF compliant with ServiceAPIDescription (clause 8.2.4.2.2 from 3GPP TS 23.222 [5]). Likewise, for the cases where CAPIF and 3GPP management system belong both to the same administrative domain, the 3GPP management system administrator [29] also acts as CAPIF administrator.

However, there are two issues that deserve further analysis.

On the one hand, it is needed to discuss how 3GPP management system can help define discovery information for each external MnS consumer. The aim of the configured discovered information is to limit their visibility over the published MnS information, either at onboarding time (when the external MnS consumer becomes an onboarded API invoker) or at operation time (when the external MnS consumer issues discovery requests). The configured discovered information can define filters at two levels:

-	First, the discovery information allows configuring which resources are made discoverable to the external MnS consumer. For each MOI which is under the management scope of the published MnS, the discovery information specifies whether the MOI is visible or not.

-	Secondly, for each visible MOI, the discovery information allows limiting what the external MnS consumer can see regarding:

-	CRUD operations and/or notifications associated to the MOI.

-	Management data, e.g. performance and fault information, associated to the MOI.

On the other hand, it is needed to discuss how a defined discovery information can be configured on the CCF. This association allows setting the visibility that this API invoker will have over its lifetime, from onboarding to offboarding, including all stages in between. If needed, the discovery information can be updated over the API invoker's lifetime.

#### 5.1.3.2	Potential requirements

PREQ-FS_MExpo-Disc-01: The 3GPP management system shall provide the capability to configure which MOIs, among those ones that are within the scope of a published MnSs, are visible to an external MnS consumer. The filtered set of MOIs is referred to as discoverable MOIs.

PREQ-FS_MExpo-Disc-02: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which attributes supported by this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-03: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which CRUD operations associated to this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-04: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which notifications associated to this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-05: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which management data associated to this MOI are visible to an external MnS consumer.

NOTE 1:	Management data includes for example performance measurements, KPIs and alarm information.

NOTE 2:	The above listed requirements are based on those originally defined in 3GPP TS 28.319 [29], but now applicable for external MnS consumers.

PREQ-FS_MExpo-Disc-06: The 3GPP management system shall provide the means to map the configuration associated to discoverable MOIs to appropriate OAuth 2.0 access token.

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.1	Potential solution #1: Using AccessRule class to support discovery information definition

5.1.3.3.1.1	Introduction

AccessRule class (3GPP TS 28.319 [29], clause 7.3.3) enables providing granular control on what actions are visible over which resources, when certain conditions (filter criteria) are met. See clause 7.3.3.2 of TS 28.319 [29] for the summary of the class attributes.

The different AccessRule instances are defined at design time and stored in a database accessible by 3GPP management system. An example of this database can be the operator's Authentication, Authorization and Accounting (AAA) server.

This solution proposes using the existing AccessRule class to define discovery information for external MnS consumer. With this solution, it is proposed to fulfil the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05.

5.1.3.3.1.2	Description

It is proposed to use AccessRule to generate discovery information as follows:

-	"dataNodeSelector": it can be used not only to configure which MOI are visible (first level of filter), but also the visibility over the attributes of these MOIs. This allows fulfilling PREQ-FS_MExpo-Disc-01 and PREQ-FS_MExpo-Disc-02.

-	"operations": it can be used to specify which CRUD operations are visible for each MOI listed in the dataNodeSelector. This allows fulfilling PREQ-FS_MExpo-Disc-03.

-	"componentCData": it can be used to specify which management data (e.g. performance measurements, KPIs and alarm information) are visible for each MOI listed in the dataNodeSelector. This allows fulfilling PREQ-FS_MExpo-Disc-05.

-	"actions": The usage of this attribute is to grant authorization to the external MnS consumer when accessing service APIs over CAPIF-2/2e interface. However, this is a separate use case, which is described in clause 5.1.4.

##### 5.1.3.3.2	Potential solution #1b: Using Role class for discovery information definition

5.1.3.3.2.1	Introduction

Role class (3GPP TS 28.319 [29], clause 7.3.2) enables capturing multiple access rules. See clause 7.3.2.2 of TS 28.319 [29] for the class attributes.

The different Role instances are defined at design time and stored in a database accessible by 3GPP management system. Examples of this database can be the operator's Authentication, Authorization and Accounting (AAA) server.

This solution proposes using the existing Role class to define discovery information for external MnS consumer. With this solution, it is proposed to fulfil the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05.

5.1.3.3.2.2	Description

This solution builds on top of solution #1, by associating roles instead of individual access rules. Solution #1b requires the underlying implementation of solution #1.

##### 5.1.3.3.3	Potential solution #3: Mapping policy definition into OAuth2.0 access token

5.1.3.3.3.1	Introduction

This solution focuses on meeting the requirement PREQ-FS_MExpo-Disc-06.

5.1.3.3.3.2	Description

The discovery policy is configured on the CCF, so the CCF can limit what the external MnS consumer can subscribe to (during the API invoker onboarding) and discover and subsequently access (once the API invoker gets onboarded). This configuration can be accomplished through OAuth 2.0 access token. The details and usage of this token when used in CAPIF are defined in table C.2.2-1 of 3GPP TS 33.122 [14].

To configure the policy on the CCF, it is proposed to use scope parameter, which is a string containing a space-delimited list. The configuration can be done, for example by the CAPIF administrator, for the cases where this administrator is also administrator of the 3GPP management system (see 3GPP TS 28.319 [29]).

Once configured with the discovery policy, the CCF can use the policy with the external MnS consumer over CAPIF-1 interface.

#### 5.1.3.4	Evaluation of potential solutions

##### 5.1.3.4.1	Evaluation of potential solutions #1 and #1b

Solutions #1 and #1b allow fulfilling the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03 and PREQ-FS_MExpo-Disc-05, providing controllable granularity on which published MnS information can be consumed by an external MnS consumer.

However, there are two gaps with the proposed solutions.

Firstly, it is noteworthy that PREQ-FS_MExpo_Disc-04 cannot be fulfilled with the current MSAC solution. The "operations" attribute only supports CRUD operations, but no notifications. Before working on fulfilling PREQ-FS_MExpo_Disc-04, there are open questions that need to be answered first:

-	Notification types: it should be clarified whether the intention is to set the visibility over certain notification types, and if so, which notification types are eligible. Are Configuration Management related notifications (e.g. notifyMOICreation, notifyMOIAttributeValueChange, notifyFileReady) included? Are Fault Management related notifications (e.g. notifyNewAlarm, notifyAckStateChanged, notifyClearedAlarm) included? Are Threshold Crossing notifications included?

-	Notification content: it should be clarified whether it is the intention to set the visibility over notification content; accessing to notification content can reveal secret.

-	"dataNodeSelector" attribute in AccessRule class: it should be clarified the role of this attribute with notification support: In every notification, there is parameter "objectInstance" in the notification header. Is "dataNodeSelector" attribute evaluated against this parameter "objectInstance"?

Secondly, it is not clear what is the default system behavior when the operator does not configure any rule, and therefore any policy. For example, what happens when a node (specified by "dataNodeSelector") is not covered by any rule?. Should the system allow or deny visibility over such a node? The default system behavior for this situation is not described in the present document nor in 3GPP TS 28.319 [29].

### 5.1.4	Use case #4: Authorization of the external MnS consumer to access the management service API

#### 5.1.4.1	Description

After the external MnS consumer has discovered the available management services (now service APIs) at the CCF, the external MnS consumer proceeds to request the CCF for the authorization information to access the service API (see clause 8.11 of 3GPP TS 23.222 [5] and clause 6.5.2.3 of 3GPP TS 33.122 [14]) via the CAPIF-1e interface. This authorization information contains the actions (e.g. ALLOW, DENY) the external MnS consumer can perform on the discovered MnS producer. These actions might differ from what the external MnS consumer is authorized to discover from the CCF (depending on the configured discovery policy). However, currently, the CCF does not have this authorization information in order to generate the access token to grant external MnS consumers access to the discovered service APIs (i.e. the management services).

Furthermore, after receiving the authorization information from the CCF, the external MnS consumer will proceed to access the management service at the MnS producer (the MnS producer is the CAPIF API Exposing Function (AEF)) via the CAPIF-2e interface. This implies that the MnS producer should be able to support the CAPIF-2e interface; however, it is safe to assume that this is already supported.

#### 5.1.4.2	Potential requirements

PREQ-FS_MExpo-Auth-01: The 3GPP management system shall provide the CCF with the authorization information for an external MnS consumer.

#### 5.1.4.3	Potential solutions

#### 5.1.4.3.1	Potential solution #1: <Potential Solution i Title>

5.1.4.3.1.1	Introduction

This potential solution describes how the CCF can authorize the external MnS consumer to access the management service API.

5.1.4.3.1.2	Description

To authorize an external MnS consumer to consume management service(s) at the MnS producer, the CCF can use the AccessRule class (defined in clause 7.3.3 in 3GPP TS 28.319 [29]) to define the access rule for authorization. This access rule is identified by the ruleName attribute. The dataNodeSelector attribute of the AccessRule class can be used to select the resources to which the access rule will apply. These resources are either the managed objects (IOCs), instances of managed objects (MOIs), or their corresponding attribute(s).

For the selected resources under the dataNodeSelector attribute, the operations attribute is used to specify the operations (as specified in clause 11.1.1 in 3GPP TS 28.532 [17]) that can be applied to these resources.

For each resource and operation combination, the actions attribute permits what operation(s) the external MnS consumer can perform on the resource.

To request for authorization to consume a specific service API, the external MnS consumer sends a "service API authorization request" (see clause 8.11 of 3GPP TS 23.222 [5]) to the CCF. The request should include the information provided in Table 5.1.4.3.1.2-1.

Table 5.1.4.3.1.2-1 lists the attributes of type AccessTokenReq (see clause 8.5.4.2.6 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.5.4.2.6-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type AccessTokenReq.

Table 5.1.4.3.1.2-1: Definition of type AccessTokenReq

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| grant_type | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | Should be set to “client_credentials” |
| client_id | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | This is the same as the API invoker Id. |
| resOwnerId | The data type of this attribute is defined as "ResOwnerId" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). |  |
| client_secret | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). |  |
| scope | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). |  |
| authCode | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | RNAA |
| redirect_uri | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.6-1 of TS 29.222 [13]). | RNAA |

When the CCF receives the "service API authorization request" (see clause 8.11 of 3GPP TS 23.222 [5]) from the external MnS consumer, the CCF first validates the request and generates the access token. Upon generating the access token, the CCF sends the "service API authorization response" with the access token information to the external MnS consumer. The generated access token is structured according to the information provided in Table 5.1.4.3.1.2-2.

Table 5.1.4.3.1.2-2 lists the attributes of type AccessTokenRsp (see clause 8.5.4.2.7 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.5.4.2.7-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type AccessTokenRsp.

Table 5.1.4.3.1.2-2: Access token response message parameters

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| access_token | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.7-1 of TS 29.222 [13]). |  |
| token_type | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.5.4.2.7-1 of TS 29.222 [13]). |  |
| expires_in | The data type of this attribute is defined as "DurationSec" and presence qualifier is defined as "M" (see table 8.5.4.2.7-1 of TS 29.222 [13]). |  |
| scope | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.5.4.2.7-1 of TS 29.222 [13]). | This scope contains information that will be used by the management system to enforce authorization to consume management services. |

To consume the management service, the external MnS consumer provides the generated access token to the MnS producer via the CAPIF-2 interface (following the procedures described in clause 8.16 of 3GPP TS 23.222 [5]).

#### 5.1.4.4	Evaluation of potential solutions

The proposed solution covers the requirement PREQ-FS_MExpo-Auth-01 of the use case. However, the proposed solution has the following limitations when it comes OAuth2.0 scopes:

1)	Given that the access token is carried in the HTTP authorization header (see IETF RFC 6750 [31]), size limitations are introduced on how big the access token could be. In the 3GPP management system, the access control for each MnS consumer is at the granularity of the resources (i.e. MOIs, IOCs, or their corresponding attributes), the operations that can be performed on these resources, and the actions that are allowed on the permitted operations. This implies that as the number of resources the external MnS consumer is authorized to access increases, so does the size of the access token since the access rule(s) are stored in the "scope" attribute of the access token.

2)	At the MnS producer, as the enforcer, the MnS producer is in charge of validating the access token provided by the external MnS consumer (see clause C.7, 3GPP TS 33.122 [14]). The validation process of the access token involves ensuring that the access token is a valid (see IETF RFC 7519 [32]). Upon validation, the MnS producer needs to map the scope value of the access token to the access rules (see clause C.5, 3GPP TS 33.122 [14]) and ensure that the access rules are valid for the external MnS consumer. To do this, the MnS producer can contact the authorization server. These checks (i.e. the access token validation, the mapping and the validation of the access rules applicable to the external MnS consumer) contribute to the processing time of the access token on the MnS producer.

### 5.1.5	Use case #5: Logging the management service API invocations to the CCF

#### 5.1.5.1	Description

When exposing management services for consumption by the external MnS consumers (referred to as the API invoker in CAPIF terms), it is crucial to monitor, for example, what management service API was invoked (i.e. the MOIs, attributes or operations), who invoked the API (i.e. the ID of the external MnS consumer), the result of the invocation (e.g. success, or failure) and at what time it was invoked.

Accordingly, the MnS producer (which acts as the CAPIF AEF) should be able to create the management service API (now service API) invocation log(s) (see clause 8.7 of 3GPP TS 29.222 [13]) with the desired information. Subsequently, the MnS producer should be able to send the invocation log(s) to the CCF via the CAPIF-3 interface. The stored logs of the service API invocations can be consumed by authorized consumers (e.g. CAPIF API AMF for auditing purposes and the charging functions). However, currently, the MnS producers do not support these functionalities.

#### 5.1.5.2	Potential requirements

PREQ-FS_MExpo-Log-01: The 3GPP management system should support the capability to create logs based on the management service API invocations by external MnS consumers.

PREQ-FS_MExpo-Log-02: The 3GPP management system should support the capability to log the management service API invocations to the CCF.

#### 5.1.5.3	Potential solutions

##### 5.1.5.3.1	Potential solution #1: Creation and logging of the management service API invocations

5.1.5.3.1.1	Introduction

The potential solution assumes that the MnS producer is the AEF and interacts with the CCF via the CAPIF-3 interface. Accordingly, this potential solution describes how the MnS producer can create the management service API invocation log. Secondly, the solution describes how the MnS producer can send these logging data to the CCF for authorized consumers to consume (e.g. AMF and charging functions).

5.1.5.3.1.2	Description

To log the management service API invocations to the CCF, the MnS producer should create an invocation log (see clause 8.7 in 3GPP TS 29.222 [13]). Table 5.1.5.3.1.2-1 maps the CAPIF InvocationLog data type and the MnSInfo IOC (see clause 4.3.42 of 3GPP TS 28.622 [3]) attributes.

Table 5.1.5.3.1.2-1 lists the attributes of type InvocationLog (see clause 8.7.4.2.2 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.7.4.2.2-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type InvocationLog.

Table 5.1.5.3.1.2-1: Mapping between CAPIF InvocationLog data type and the MnSInfo IOC attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| aefId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.2-1 of TS 29.222 [13]). | MnS producer obtains this information from the CCF after MnS producer registration |
| apiInvokerId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.2-1 of TS 29.222 [13]). | Provided by the external MnS consumer when invoking the management service API |
| logs | The data type of this attribute is defined as "array(Log)" and presence qualifier is defined as "M" (see table 8.7.4.2.2-1 of TS 29.222 [13]). | See Table 5.1.5.3.1.2-2 |
| supportedFeatures | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O" (see table 8.7.4.2.2-1 of TS 29.222 [13]). |  |

Table 5.1.5.3.1.2-2 lists the attributes of type Log (see clause 8.7.4.2.3 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.7.4.2.3-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type Log.

Table 5.1.5.3.1.2-2: Mapping between CAPIF Log data type and the MnSInfo IOC attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | The MnS producer receives this attribute from the CCF after publishing the MnS information. |
| apiName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Same as mnsLabel. |
| apiVersion | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Same as mnsVersion. |
| resourceName | The data type of this attribute is defined as "String" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | No possible mapping to MnSInfo attributes (could be the IOC name of the MOI). |
| uri | The data type of this attribute is defined as "Uri" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Can be the DN that uniquely identifies the MOI. The DN-to-URI mapping mechanism is defined in clause 4.2.3, 3GPP TS 32.158 [30]. |
| protocol | The data type of this attribute is defined as "Protocol" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | HTTPS?? |
| operation | The data type of this attribute is defined as "Operation" and presence qualifier is defined as "C" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | E.g. PUT, POST, GET, DELETE, PATCH, etc. |
| result | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | HTTP status codes. |
| invocationTime | The data type of this attribute is defined as "DateTime" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Is this time when the MnS producer receives the service API invocation request from the external MnS consumer? Needs further clarification. |
| invocationLatency | The data type of this attribute is defined as "DurationMs" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Is this the duration of the service API invocation (i.e. from the time the external MnS consumer sends the invocation request to the MnS producer to the time when the MnS producer sends the response back to the external MnS consumer)? Needs further clarification. |
| inputParameters | The data type of this attribute is defined as "ANY TYPE" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| OutputParameters | The data type of this attribute is defined as "ANY TYPE" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| srcInterface | The data type of this attribute is defined as "InterfaceDescription" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| destInterface | The data type of this attribute is defined as "InterfaceDescription" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| fwdInterface | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |

|  |  |  |  |  | -- |
| --- | --- | --- | --- | --- | --- |

After creating the invocation log, the MnS producer can log these data to the CCF via the CAPIF-3 interface.

#### 5.1.5.4	Evaluation of potential solutions

The proposed solution satisfies the requirement PREQ-FS_MExpo-Log-01. To fulfil the use case requirement PREQ-FS_MExpo-Log-02, the proposed solution requires that the 3GPP management entity providing the AEF role implements the CAPIF-3 interface and the associated functionalities.

### 5.1.x	Use case #<X>: <Use case title>

#### 5.1.X.1	Description

Editor's note: This clause provides a description of the use case, that includes problem statement(s) and/or gap(s) in the existing solution(s).

#### 5.1.X.2	Potential requirements

Editor's note: This clause captures potential requirements.

#### 5.1.X.3	Potential solutions

##### 5.1.X.3.i	Potential solution #<i>: <Potential Solution i Title>

5.1.X.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

5.1.X.3.i.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

#### 5.1.X.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

# 6	Conclusions and recommendations

Annex A:
3GPP management capabilities

SA5 WG is responsible for developing, maintaining, and evolving solutions building up the 3GPP management system. The relationship of the 3GPP management system with the state-of-the-art initiatives (clause 4.1.3) can be summarized as follows:

-	Corresponds to the "OAM/CH services" box inside 3GPP system (see Figure 4.1.3.1-1).

-	Is a collection of solutions, from one or more vendors, that reside within the CSP domain (see Figure 4.1.3.2-1).

-	Provides management capabilities which are realized via network APIs (see Figure 4.1.3.2-1).

The table below provides a (non-exhaustive) list of the management capabilities that can be provided by 3GPP management system. This list builds on the information available in 3GPP TS 28.533 [2], Annex F, complementing it with mechanisms that are non-CRUD based.

Table A-1: 3GPP management system capabilities

| Management Capability | Mechanisms |  | Solutions |  |
| --- | --- | --- | --- | --- |
|  |  |  | RESTFUL | NETCONF/ YANG |
| Performance data collection control | CRUD operations (3GPP TS 28.532 [17]) + PM control NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
|  | CRUD operation (3GPP TS 28.532 [17]) + ManagementDataCollection NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
|  | Measurement job control (3GPP TS 28.550 [21]) |  | X |  |
| Performance data report | CRUD operations (3GPP TS 28.532 [17]) + PM control NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
|  | File data reporting service (3GPP TS 28.532 [17]) |  | X |  |
|  | Streaming data reporting service (3GPP TS 28.532 [17]) |  | X |  |
| Performance data monitoring | CRUD operations (3GPP TS 28.552 [33]) + Threshold monitoring control NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
| Trace/MDT data collection control | CRUD operations (3GPP TS 28.532 [17]) + Trace control NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
|  | CRUD operation + ManagementDataCollection NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
| Trace/MDT data report | File data reporting service (3GPP TS 28.532 [17]) |  | X |  |
|  | CRUD operations (3GPP TS 28.532 [17]) + File retrieval NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
|  | Streaming data reporting service (3GPP TS 28.532 [17]) |  |  |  |
| Fault data control | CRUD operations (3GPP TS 28.532 [17]) + FM control NRM fragment (3GPP TS 28.111 [22]) |  | X | X |
|  | Fault supervision data control service (3GPP TS 28.532 [17]) |  |  |  |
| Fault data report | CRUD operations (3GPP TS 28.532 [17]) + FM control NRM fragment (3GPP TS 28.111 [22]) |  | X | X |
|  | Fault supervision data report service (3GPP TS 28.532 [17]) |  |  |  |
| QoE control | CRUD operations (3GPP TS 28.532 [17]) + QMC control NRM fragment (3GPP TS 28.532 [17]) |  | X | X |
| Service registry and discovery | CRUD operations (3GPP TS 28.532 [17]) + MnS Registry NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
| Subscription | CRUD operations (3GPP TS 28.532 [17]) + Notification subscription control NRM fragment (3GPP TS 28.622 [3]) |  | X | X |
| AI/ML management | CRUD operations (3GPP TS 28.532 [17]) + ML Training NRM fragment (3GPP TS 28.105 [23]) |  | X |  |
| Management Data Analytics | CRUD operations (3GPP TS 28.532 [17]) + MDA request & report NRM fragment (3GPP TS 28.104 [24]) |  | X |  |
| NR Provisioning | CRUD operations (3GPP TS 28.532 [17]) + NR NRM fragment (3GPP TS 28.541 [25]) |  | X | X |
| 5GC Provisioning | CRUD operations (3GPP TS 28.532 [17]) + 5GC NRM fragment (3GPP TS 28.541 [25]) |  | X | X |
| Network Slice Provisioning | CRUD operations (3GPP TS 28.532 [17]) + Network slice NRM fragment (3GPP TS 28.541 [25]) |  | X | X |
|  | Network slicing provisioning service (3GPP TS 28.531 [18]) | X |  |  |
| SON Policy | CRUD operations (3GPP TS 28.532 [17]) + NRM fragments for DANR/DES/DRACH/DMRO/DPCI/CES/CPCI/DLBO/CCO management (3GPP TS 28.541 [25]) |  | X | X |
| Intent Driven Management | CRUD operations (3GPP TS 28.532 [17]) + Intent NRM fragment (3GPP TS 28.312 [26]) |  | X |  |
| Edge Computing Provisioning | CRUD operations (3GPP TS 28.532 [17]) + Edge NRM fragment (3GPP TS 28.538 [27]) |  | X |  |
| Communication Service Assurance | CRUD operations (3GPP TS 28.532 [17]) + Assurance NRM fragment (3GPP TS 28.536 [28]) |  | X |  |

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-02 | - | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2024-02 | SA5#153 | S5-241089 |  |  |  | Implements S5-241060, S5-241061 | 0.1.0 |
| 2024-04 | SA5#154 | S5-242008 S5-242005 S5-242010 S5-242009 S5-242163 | pCR pCR pCR pCR pCR |  |  | Add Concepts and background for Exposure of Management Services in Clause 4 Update use case template in Clause 5 Add definition of external MnS consumer concept Add background on existing telco exposure initiatives Add list with the different types of external MnS consumers | 0.2.0 |
| 2024-05 | SA5#155 | S5-243264 S5-243250 S5-243252 S5-243253 S5-243256 S5-243259 S5-243262 S5-243263 | pCR pCR pCR pCR pCR pCR pCR pCR |  |  | Add and correct references Registration of MnS producer into CAPIF Add use case for publishing management services to the CCF Add potential solutions for publishing management services to the CCF Configuring discovery policy for an external MnS consumer Add use case for authorization of the external MnS consumer to access the management service API Add use case for logging the management service API invocations Add a new potential solution for logging the management service API invocations to the CCF | 0.3.0 |
| 2024-08 | SA5#156 | S5-244746 S5-244747 S5-244751 S5-244752 S5-244755 S5-244756 S5-244757 | pCR pCR pCR pCR pCR pCR pCR |  |  | pCR TR 28.879 Update concepts pCR TR 28.879 Registration use case - update agreed content and add evaluation of solution 1 Rel-19 pCR TR 28.879 Updating the configuring discovery policy for an external MnS consumer use case pCR TR 28.879 Discovery policy configuration - update solution 1 description and add evaluation Rel-19 pCR TR 28.879 Add solution for authorization of the external MnS consumer to access the management service API use case Rel-19 pCR TR 28.879 Evaluation analysis for authorization of the external MnS consumer to access the management service API use case Rel-19 pCR TR 28.879 Evaluation analysis for Logging the management service API invocations to the CCF use case |  |
| 2024-09 | SA#105 | SP-241137 |  |  |  | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  |  |  |  | After editHelp cleanup | 1.0.1 |
| 2024-10 | SA5#157 | S5-245616 S5-245617 S5-246038 S5-246283 | pCR pCR pCR pCR |  |  | Rel-19 pCR TR 28.879 Update Introduction, add scope and remove clause 5.2 and 5.3 Rel-19 pCR TR 28.879 Addressing editHelp comments and Rapporteur cleanup Rel-19 pCR TR 28.879 Clean up to clarify the roles of the MnS producer vs the MnF and differentiates between discovery info configuration and authorization pCR TR 28.879 pCR TR 28.879 Publishing UC – updates on solution and evaluation | 1.1.0 |