| 3GPP TR 28.908 V0.5.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Artificial Intelligence / Machine Learning (AI/ML) management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Concepts and overview	10

4.1	Overview	10

4.2	AI/ML workflow for 5GS	11

4.2.1	AI/ML operational workflow	11

4.2.2	AI/ML management capabilities	12

5	Use cases, potential requirements and possible solutions	12

5.1	AI/ML performance evaluation	12

5.1.1	Description	12

5.1.2	Use cases	12

5.1.2.1	Performance indicator selection for ML model training	12

5.1.2.2	Monitoring and control of AI/ML behavior	13

5.1.2.3	AI/ML performance evaluation in inference phase	13

5.1.2.4	AI/ML abstract performance	14

5.1.2.5	ML entity performance indicators query and selection	14

5.1.2.6	AI/ML entity performance indicators selection based on MnS consumer policy	14

5.1.3	Potential requirements	15

5.1.4	Possible solutions	16

5.4.4.1	Potential solutions for performance indicator selection for ML model training	16

5.4.4.2	Potential solutions for monitoring and control of AI/ML behavior	16

5.4.4.3	Potential solutions for AI/ML performance evaluation in inference phase	17

5.4.4.4	Potential solutions for AI/ML performance abstraction	18

5.4.4.5	Potential solutions for ML entity performance indicators query and selection	19

5.4.4.6	Potential solutions for policy-based performance indicator selection	20

5.1.5	Evaluation	20

5.2	Event data for ML training	21

5.2.1	Description	21

5.2.2	Use cases	21

5.2.2.1	Pre-processed event data for ML training	21

5.2.3	Potential requirements	22

5.2.4	Possible solutions	22

5.2.5	Evaluation	23

5.3	ML entity validation	24

5.3.1	Description	24

5.3.2	Use cases	24

5.3.2.1	ML entity validation performance reporting	24

5.3.3	Potential requirements	24

5.3.4	Possible solutions	24

5.3.4.1	Validation performance reporting by enhancing the existing IOC	24

5.3.5	Evaluation	25

5.4	ML entity testing	25

5.4.1	Description	25

5.4.2	Use cases	25

5.4.2.1	Consumer-requested ML entity testing	25

5.4.2.2	Control of ML entity testing	26

5.4.3	Potential requirements	26

5.4.4	Possible solutions	26

5.4.4.1	NRM based solution	26

5.4.5	Evaluation	28

5.5	ML entity loading	28

5.5.1	Description	28

5.5.2	Use cases	28

5.5.2.1	ML entity loadingcontrol and monitoring	28

5.5.3	Potential requirements	29

5.5.4	Possible solutions	29

5.5.4.1	NRM based solution	29

5.5.5	Evaluation	30

5.6	AI/ML Inference History	31

5.6.1	Description	31

5.6.2	Use cases	31

5.6.2.1	Tracking AI/ML inference decision and context	31

5.6.3	Potential requirements	31

5.6.4	Possible solutions	31

5.6.5	Evaluation	32

5.7	ML context	32

5.7.1	Description	32

5.7.2	Use cases	33

5.7.2.1	ML context monitoring and reporting	33

5.7.2.2	Mobility of ML Context	33

5.7.2.3	Standby mode for AI/ML inference functionality	33

5.7.3	Potential requirements	34

5.7.4	Possible solutions	34

5.7.4.1	MLContext datatype on MLEntity	34

5.7.4.2	Mobility of MLContext	35

5.7.5	Evaluation	35

5.8	ML Entity Capability Discovery and Mapping	35

5.8.1	Description	35

5.8.2	Use cases	36

5.8.2.1	Identifying capabilities of ML entities	36

5.8.2.2	Mapping of the capabilities of ML entities	36

5.8.3	Potential requirements	37

5.8.4	Possible solutions	37

5.8.5	Evaluation	38

5.9	AI/ML update management	38

5.9.1	Description	38

5.9.2	Use cases	38

5.9.2.2	AI/ML entities updating initiated by producer	38

5.9.3	Potential requirements	39

5.9.4	Possible solutions	39

5.9.4.1	Possible solution#1	39

5.9.5	Evaluation	39

5.10	AI/ML configuration management	39

5.10.1	Description	39

5.10.2	Use cases	40

5.10.2.1	ML entity configuration for RAN domain ES initiated by consumer	40

5.10.2.2	ML entity configuration for RAN domain ES initiated by producer	40

5.10.2.3	Partial activation of AI/ML inference capabilities	40

5.10.2.4	Configuration for AI/ML inference initiated by consumer	41

5.10.2.5	Configuration for AI/ML inference initiated by producer	41

5.10.2.6	Enabling policy-based activation of AI/ML capabilities	41

5.10.3	Potential requirements	42

5.10.4	Possible solutions	42

5.10.4.1	ML training configuration	42

5.10.4.2	AI/ML inference function configuration	42

5.10.4.3	AI/ML activation	43

5.10.4.3.1	General framework for activation and deactivation	43

5.10.4.3.2	Instant activation and deactivation	43

5.10.4.3.3	Policy based activation	43

5.10.4.3.4	Schedule based activation	43

-	The schedule for activation/deactivation.	43

5.10.5	Evaluation	43

5.11	Orchestrating AI/ML Inference	44

5.11.1	Description	44

5.11.2	Use cases	44

5.11.2.1	Knowledge sharing on executed actions	44

5.11.2.2	Knowledge sharing on impacts of executed actions	44

5.11.2.3	Abstract information on impacts of executed actions	45

5.11.2.4	Triggering execution of AI/ML inference functions or ML entities	45

5.11.2.5	Orchestrating decisions of AI/ML inference functions or ML entities	46

5.11.3	Potential requirements	46

5.11.4	Possible solutions	47

5.11.5	Evaluation	53

5.12	Coordination between the AI/ML capabilities	53

5.12.1	Description	53

5.12.2	Use cases	53

5.12.2.1	Alignment of the AI/ML capability between 5GC/RAN and 3GPP management system	53

5.12.3	Potential requirements	53

5.12.4	Possible solutions	54

5.12.5	Evaluation	54

5.13	ML entity re-training	54

5.13.1	Description	54

5.13.2	Use cases	54

5.13.2.1	Producer-initiated threshold-based ML Retraining	54

5.13.2.2	Efficient ML entity re-training	54

5.13.3	Potential requirements	54

5.13.4	Possible solutions	55

5.13.4.1	Producer Initiated Retraining	55

5.13.4.2	Efficient AI/ML entity re-training	55

5.13.5	Evaluation	56

5.14	ML entity joint training	56

5.14.1	Description	56

5.14.2	Use cases	56

5.14.2.1	Support for ML entity modularity – joint training of ML entities	56

5.14.3	Potential requirements	56

5.14.4	Possible solutions	57

5.14.4.1	Support for AI/ML Entity modularity - joint training of ML entities	57

5.14.5	Evaluation	57

5.15	ML Inference emulation	57

5.15.1	Description	57

5.15.2	Use cases	58

5.15.2.1	AI/ML Inference emulation	58

5.15.2.2	Orchestrating ML Inference emulation	58

5.15.3	Potential requirements	58

5.15.4	Possible solutions	59

5.15.5	Evaluation	59

5.16	Training data effectiveness reporting and analytics	59

5.16.1	Description	59

5.16.2	Use cases	59

5.16.2.1	 Training data effectiveness reporting	59

5.16.2.2	Training data effectiveness analytics	60

5.16.2.3	Measurement data correlation analytics for ML training	60

5.16.3	Potential requirements	60

5.17	Trustworthy Machine Learning	61

5.17.1	Description	61

5.17.2	Use cases	61

5.17.2.1	AI/ML trustworthiness indicators	61

5.17.2.2	AI/ML data trustworthiness	62

5.17.2.3	ML training trustworthiness	62

5.17.2.4	AI/ML inference trustworthiness	63

5.17.3	Potential requirements	63

5.17.4	Possible solutions	64

5.17.5	Evaluation	64

Annex X (informative): Change history	65

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document studies the Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services for 5GS where AI/ML is used, including management and orchestration (e,g., MDA, see TS 28.104 [2]) and 5G networks (e.g., NWDAF, see TS 23.288 [3]).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.105: " Artificial Intelligence / Machine Learning (AI/ML) management ".

[5]	IBM Watson Studio, “Model Drift” [Online]. Link: https://www.ibm.com/cloud/watson-studio/drift.

[6]	3GPP TR 28.864: "Study on Enhancement of the management aspects related to NetWork Data Analytics Functions (NWDAF)".

[7]	3GPP TR 28.310: "Management and orchestration; Energy efficiency of 5G".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[10] 	European Commission (21.04.2021): “Proposal for a Regulation laying down harmonized rules on artificial intelligence”.

[11]	High-level Expert Group on Artificial Intelligence setup by the European Commission (08.04.2019): "Ethical Guidelines for Trustworthy AI".

[12]	ISO/IEC TR 24028:2020, Information technology - Artificial intelligence - Overview of trustworthiness in artificial intelligence.

[13]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 28.105 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and overview

## 4.1	Overview

Artificial Intelligence/Machine Learning (AI/ML) techniques are being embraced by telecommunication service providers around the world to facilitate enabling the existing and the new challenging use cases that 5G offers. AI/ML capabilities are being increasingly adopted in mobile networks as a key enabler for wide range of features and functionalities that maximise efficiency and bring intelligence and automation in various domains of the 5GS. For example, these include the Management Data Analytics (MDA) in the management and orchestration [1], the Network Data Analytics Function in the 5G core network domain [3].

The AI/ML inference functions in the 5GS use the ML model for inference and in order to enable and facilitate the AI/ML adoption, the ML model needs to be created, trained and then managed during its entire lifecycle.

To enable, facilitate and support AI/ML-capabilities in the 5GS, the following management capabilities are studied in this report:

- Validation of ML model or entity

- Testing of ML model or entity (before deployment)

- Deployment of ML model or entity (new or updated model/entity)

- Configuration of ML training and AI/ML inference

- Performance evaluation of ML training and AI/ML inference

NOTE: The ML model training capability is specified in TS 28.105 [4].

## 4.2	AI/ML workflow for 5GS

### 4.2.1	AI/ML operational workflow

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN and management system), and the generic workflow of the operational steps in the lifecycle of an ML model or entity, is depicted in the figure 4.2.1-1.

Figure 4.2.1-1: AI/ML operational workflow

The workflow involves 3 main phases; the training, deployment and inference phase, including the main operational tasks for each phase. These are briefly described below:

Training phase:

-	ML Training: Learning by the Machine from the training data to generate the (new or updated) ML entity (see TS 28.105 [4]) that could be used for inference. The ML Training may also include the validation of the generated ML entity to evaluate the performance variance of the ML entity when performing on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the ML entity needs to be re-trained. This is the initial step of the workflow. The ML Training MnS is specified in TS 28.105 [4].

-	ML Testing: Testing of the validated ML entity with testing data to evaluate the performance of the trained ML entity for selection for inference. When the performance of the trained ML entity meets the expectations on both training data and validation data, the ML entity is finally tested to evaluate the performance on testing data. If the testing result meets the expectation, the ML entity may be counted as a candidate for use towards the intended use case or task, otherwise the ML entity may need to be further (re)trained. In some cases, the ML entity may need to be verified which is the special case of testing to check whether it works in the AI/ML inference  function or the target node. In other cases, the verification step may be skipped, for instance in case the input and output data, data types and formats, have been unchanged from the last ML entity.

Deployment phase:

-	AI/ML Deployment: Deployment of the trained and tested ML entity to the target inference function which will use the subject ML entity for inference.

NOTE: 	The deployment phase may not be needed in some cases, for example when the training function and inference function are in the same entity.

Inference phase:

-	AI/ML Inference: Performing inference using the ML entity by the inference function.

In telco-grade environments, it is worth noting that the selected learning method (see examples of learning methods  captured in Table 4.1-1 in [2]) can influence on how AI/ML operational workflow executes. In some cases (e.g., when using supervised learning methods), the inference phase cannot start until training phase gets ended. In other cases (e.g., when using reinforcement learning methods), the inference phase can start while training phase is still in progress.

### 4.2.2	AI/ML management capabilities

Each operational step in the workflow (as depicted in clause 4.2.1)  is supported by the specific AI/ML management capabilities, including:

Management capabilities for training phase

-	ML training management: allowing the consumer to request and/or manage the model training/retraining based on the performance evaluation results observed by model performance monitoring (performance data and/or feedback). For example, if the model performance degrades, the ML performance management capability may trigger the ML training to start re-training.

-	ML testing management:  allowing the consumer to request the ML entity testing, and receive the testing results for a trained ML model.

Management capabilities for deployment phase

-	AI/ML deployment control and monitoring:

-	ensuring availability of the desired ML entity and allowing the consumer to request the deployment of the ML entity to be conducted by the producer,

-	allowing the consumer to set the policy for deployment of the ML entity to be initiated and conducted by the producer

-	allowing the consumer to monitor the deployment process.

Management capabilities for inference phase

-	AI/ML inference activation/deactivation: allowing the consumer to activate/deactivate the inference that uses an ML entity.

-	AI/ML inference monitoring: allowing the consumer to monitor and evaluate the inference performance of an ML entity when used by an AI/ML inference function..

Editor’s note: more details and/or clarifications to the management tasks can be added later.

# 5	Use cases, potential requirements and possible solutions

## 5.1	AI/ML performance evaluation

### 5.1.1	Description

In ML model training phase (including training, validation and testing), the performance of ML model needs to be evaluated. The related performance indicators need to be collected and analyzed.

In the AI/ML inference phase, the performance of the inference function and ML model need to be evaluated against consumer's provided performance expectations/targets, in order to identify and timely fix any problem. Actions to fix any problem would be e.g., to trigger the ML model/entity re-training, testing, and re-deployment.

### 5.1.2	Use cases

#### 5.1.2.1	Performance indicator selection for ML model training

The ML training function may support training for single or different kinds of ML models and may support to evaluate each kind of ML model by one or more specific corresponding performance indicators.

The MnS consumer may use some performance indicator(s) over the others to evaluate one kind of ML model. The performance indicators for training mainly include the following aspects:

-	Model training resource performance indicators:  the performance indicators of the system that the model trains. e.g., "training duration" etc.

-	Model performance indicators: performance indicators of the model itself, e.g., "accuracy", "precision", "F1 score", etc..

Therefore, the MLT MnS producer needs to provide the name(s) of supported performance indicator(s) for the MnS consumer to query and select for ML model performance evaluation. The MnS consumer may also need to provide the performance requirements of the ML model using the selected performance indicators.

The MLT MnS producer uses the selected performance indicators for evaluating ML model training, and reports with the corresponding performance score in the ML training report when the training is completed.

#### 5.1.2.2	Monitoring and control of AI/ML behavior

In a typical network operation, an operator configures and operates an ML entity according to the corresponding manual of the entity. Usually, the operator does not need to know the details of the ML entity’s internal-decision making process and implementations, simply due to “too many” ML entities running for inference in the network and also due to too much details and information that are deemed to be redundant or unnecessary for the operator, plus it is in the vendor's interest not to disclose any internal aspects of the implementation of their automation solutions.

However, the operator may still need to guide and evaluate the solutions and to configure/reconfigure them to achieve the desired outcomes in an ML entity-agnostic manner. For example, consider the load balancing automation use case (AutoLB) summarized by Table 1. An AutoLB ML entity helps to decide how to distribute load among network objects. The ML entity has specific actions that it can take while the operator also has operational actions that it needs to take to customize or steer the solution, e.g. to switch off the solution, to reconfigure the solution, to change the solutions input. As such, the behavior of the ML entity in terms of the actions it takes under any given conditions needs to be related to the configuration actions from the MnS consumer (e.g., the operator)

Table 5.1.2.2-1: Operability of the Automated load balancing

| Automation use case | Description | ML entity's context | Example ML entity's decisions | Operator's Actions |
| --- | --- | --- | --- | --- |
| load balancing (AutoLB) | Distribute load among different network objects, e.g. among cells. | - Amount of traffic, No. of users, etc | - select CIO values | - set the maximum CIO values, - Deactivate AutoLB |

So, it is important that even without knowing the details of the ML entity, the operator needs to have the understanding of the ML entity’s overall behaviour. And, if a part of the ML entity’s decisions/actions are not what is preferred by the operator, the operator needs to customise the ML entity in order to produce the expected or optimum solutions. Additionally, some measures or means are needed to enable the operators to associate their actions to the context and to enable the operator to provide information regarding the expected behaviour of, and facilitate, the AI/ML capabilities or solution. This could result in a re-design or re-training of the ML entity, according to the workflow pictured in Figure 4.2.1-1.

#### 5.1.2.3	AI/ML performance evaluation in inference phase

In the inference phase, the inference function (including MDAF, NWDAF and RAN intelligence functions) uses one or more ML entities for inference and generate the inference output.

The consumer (e.g., a Network or Management function) may take some actions according to the inference output provided by the inference function. If the actions are taken accordingly, the network performance is expected to be optimized. Each inference function has its specific focus and will impact the network performance from different perspectives.

The consumer may choose to not take any actions by various reasons, for instance lacking confidence on the inference output, avoiding potential conflict with other actions or when no actions are needed or recommended at all according to the inference output.

For evaluating the performance of the AI/ML inference function and ML entity, the operator needs to be able to 	get the inference output generated by each inference function and	be informed if actions are taken based or according to the inference output.

In the inference phase, the performance of a running ML entity may degrade over time due to changes in network state, which will affect the related network performance and service. Thus, it is necessary to evaluate performance of the ML entity during the inference process. If the inference output is executed, the network performance related to each inference function also needs to be evaluated.

Depending on the performance, the operator may request the ML training function to re-train the ML model(s).

#### 5.1.2.4	AI/ML abstract performance

The AI/ML inference MnS consumer is typically interested in understanding the performance of a given AI/ML inference instance, but it is not guaranteed that the MnS consumer understands the applicable AI/ML performance metrics, i.e., it is not always the case that the AI/ML MnS consumer is able to interpret the various metrics on performance KPIs (accuracy, confidence etc) speed, computational resource usage, etc. Relatedly, it may be necessary to provide means to abstract the measured metrics into indices that can be standardized, as such can be easily interpreted by any MnS consumer of AI/ML-related performance management. Thereby, the AI/ML inference function can request for qualification and abstraction of its performance by which a report is generated indicating the qualified abstract performance. Relatedly, an AI/ML inference MnS consumer can request the AI/ML inference MnS producer or the performance abstraction MnS producer for the abstract performance of a specific ML Entity or AI/ML inference function. This allows the MnS consumer to interpret the performance even without knowing the details of the specific applicable metrics.

#### 5.1.2.5	ML entity performance indicators query and selection

The ML entity performance evaluation and management is needed during training, test and inference phases. The related performance indicators need to be collected and analyzed. The MnS producer of ML training, ML testing or ML inference should determine which indicators are needed, i.e., select some indicators based on the use case and select use these indicators for performance evaluation.

The AI/ML MnS consumer may have different requests on AI/ML performance, depending on its use case and requirements, which may imply that different performance indicators may be relevant for performance evaluation. MnS producer for ML training, ML testing or ML inference can be queried to provide the information on supported performance indicators referring to ML entity training, testing and inference phases. Such performance indicators in training phase may be for example accuracy/precision/recall/F1-score/MSE/MAE /confusion matrix, in test phase may be data drift in data statistics, in inference phase may be confidence.  Based on supported performance indicators in different phases as well as based on consumer’s requirements, the  MnS consumer for ML training, ML testing or ML inference may request a sub-set of supported performance indicators to be monitored and used for performance evaluation. Management capabilities are needed to enable the  MnS consumer for ML training, ML testing or ML inference to query the supported performance indicators and select a sub-set of performance indicators in training, test and deployment phases to be used for performance evaluation.

#### 5.1.2.6	AI/ML entity performance indicators selection based on MnS consumer policy

As described in TR 28.908 ML entity performance evaluation and management is needed during training, test and inference phases. The related performance indicators need to be collected and analyzed. The  MnS producer for ML training, testing or inference should determine which indicators are needed or may be reported, i.e. select some indicators based on the service and use these indicators for performance evaluation.

The MnS consumer for ML training, testing or inference may have differentiated levels of interest in the different performance dimensions or metrics. Thus, depending on its use case, the AI/ML MnS consumer may indicate the preferred behaviour and performance requirement that shall be considered during training, testing and inference of/from the ML entity by the ML MnS Producer for ML training, testing or inference. These performance requirements need not indicate the technical performance indicators used for ML training, testing or inference, such as "accuracy" or "precision" or "recall" or "Mean Squared Error" etc. The ML MnS consumer for ML training, testing or inference may not be capable enough to indicate the performance metrics that shall be used for training and testing. Instead, the AI/ML MnS consumer may indicate the requirement using a policy or guidance that reflects the preferred performance characteristics of the ML entity. Based on the indicated policy/ guidance, the AI/ML MnS producer for ML training, testing may then deduce and apply the appropriate performance indicators for training, testing or inference. Management capabilities are needed to enable the ML MnS consumer for ML training, testing or inference to indicate the behavioural and performance policy/guidance that may be transformed by the MnS producer to a technical performance indicator during training, testing or inference.

### 5.1.3	Potential requirements

REQ-MODEL_PERF-TRAIN-1: The MLT MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the training function is able to train.

REQ-MODEL_PERF-TRAIN-2: The MLT MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML training function for each kind of ML model.

REQ-MODEL_PERF-TRAIN-3: The MLT MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each kind of ML model.

REQ-MODEL_PERF-TRAIN-4: The MLT MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function.

REQ-AI/ML_BEH-1: The 3GPP management system should have a capability to inform an authorized AI/ML MnS consumer (e.g. the operator) about the behavior of the ML entity, in an ML entity agnostic manner without the need to expose its internal characteristics.

REQ-AI/ML_BEH-2: The 3GPP management system should have a capability that enables an authorized AI/ML MnS consumer (e.g. the operator) to configure the behavior of the ML entity, in an ML entity agnostic manner that does need to expose its internal characteristics.

REQ- AI/ML_PERF-INF-1: The MnS producer responsible for AI/ML inference performance management should have a capability to allow an authorized consumer to get the inference output provided by an inference function (MDAF, NWDAF or RAN intelligence function).

REQ- AI/ML_PERF-INF-2: The MnS producer responsible for AI/ML inference performance management should have a capability to allow an authorized consumer to provide feedback about an inference output.

REQ- AI/ML_PERF-INF-3: The MnS producer responsible for AI/ML inference performance management should have a capability to allow an authorized consumer to be informed about the actions taken that were triggered by the inference output provided by an inference function (MDAF, NWDAF or RAN intelligence function).

REQ- AI/ML_PERF-INF-4: The MnS producer responsible for AI/ML inference performance management should have a capability to allow an authorized consumer to collect the performance data related to an inference function (MDAF, NWDAF or RAN intelligence function).

REQ- AI/ML_PERF-INF-5: The MnS producer responsible for AI/ML inference performance management should have a capability to allow an authorized consumer to collect the performance data for evaluating the performance of an ML entity during inference.

REQ-AI/ML_PERF-ABS-1: The 3GPP management system should have a capability for an authorized MnS consumer (e.g., an operator) to configure an abstract performance range that defines the minimum and maximum performance as expressed on an abstract performance index.

REQ-AI/ML_PERF-ABS-2: The 3GPP management system should have a capability for an authorized MnS consumer (e.g., the producer of AI/ML services such as the producer of AI/ML training or AI/ML inference services) to request the MnS producer to abstract and qualify one or more ML performance metrics of one or more specific ML entities.

REQ-AI/ML_PERF-ABS-3: The 3GPP management system should have a capability for an authorized MnS consumer (e.g., the MnS consumer of AI/ML services such as the MnS consumer of AI/ML training) to request the abstract performance of one or more specific ML entities.

REQ-AI/ML_PERF-ABS-4: The 3GPP management system should have a capability for an authorized MnS consumer (e.g., the operator) to request the  MnS producer to provide the abstract performance as a performance abstraction report of one or more ML performance metrics of one or more ML entities.

REQ-AI/ML_PERF -SEL-1: The MLT MnS producer should have a capability allowing the authorized MnS consumer to discover supported AI/ML performance indicators related to AI/ML training, testing and inference and select some the desired indicators based on the MnS consumer’s requirements.

REQ-AI/ML_PERF-POL-2: The AI/ML MnS producer should have a capability allowing the authorized MnS consumer to indicate a performance policy related to ML model training, testing and inference phases.

### 5.1.4	Possible solutions

#### 5.4.4.1	Potential solutions for performance indicator selection for ML model training

This solution uses the instances of following IOCs or attribute for interaction between MnS producer and consumer to support the performance indicator selection for ML model training:

1) 	The IOC or attribute representing the ML training capability, for example named as MLTrainingCapability, contained by MLTrainingFunction (see TS 28.105 [4]).

This IOC or attribute is created by the MnS producer and contains the following attributes:

- 	inference type of the ML model that the ML training function supports to train;

-	supported performance metrics (see performanceMetric in TS 28.105 [4]).

2) 	The IOC MLTrainingRequest (see TS 28.105 [4]) with the existing performanceRequirements attribute. The performanceMetric element of the ModelPerformance  data type for the performanceRequirements attribute is semantically extended to indicate the MnS consumer selected performance indicator/metric.

NOTE: 	The name of the IOCs and attributes are to be decided in normative phase.

#### 5.4.4.2	Potential solutions for monitoring and control of AI/ML behavior

To allow for monitoring and control of AI/ML behavior :

- 	The contexts and actions of the AI/ML MnS provider shall be grouped into operational modes represented by abstract states that are understood by both the AI/ML MnS provider and the AI/ML MnS Consumer.

> 	For example, the Robocar may be considered to have a few (e.g. two) abstract states

>>	normal operations, where the Robocar may be simply given a destination and let to act as it wishes

>>	extraneous circumstances, which represents unusual conditions such an accident on the road (as learned from the radio), abnormal street conditions such an unusually wet street due to pipe splashing water onto the street or a street power line bent into the road. In such cases the operator actions may be different , e.g. to ask the car to make a sudden stop or sudden turn.

> 	The expected number of abstract states depends on use case but is in general a small number. So, the maximum number of abstract states may be set to a small value but large enough to support most use cases (e.g. a set of states numbered 0-16 or 0-63).

- 	Each ML entity or AI/ML inference function should have an object say called abstract behavior that contains characteristics of the abstract behavior of the ML entity or AI/ML inference function. The abstract behavior may be an IOC names say, abstractBehavior and name contained on the ML entity or AI/ML inference function. The abstract behavior contains 2 attributes- the candidate abstract states and the applied abstract states.

- 	A list of candidate abstract states and their candidate actions and a list of the selected and configured abstract states and their respective selected actions.

- 	Introduce a datatype for the candidate abstract state, say called candidateAbstractState.

> 	Introduce candidateAbstractStates as an attribute of the abstract behavior. The candidateAbstractState is a list of abstract states and where each state has a list of candidate abstract actions for that abstract state.

> 	Each candidateAbstractState may have a string identifier of the abstract state, a human readable description and a list of possible actions that may be selected for that state. As such there should be an attribute for possible actions, say called possibleActions that holds the possible actions for that state. The possibleActions attribute may be an enumeration of the actions from which the MnS consumer may chose those to be applied.

- 	Introduce a datatype for the applied abstract states, say called appliedAbstractStates.

> 	The appliedAbstractStates is a list of state-action tuples. Each state may be represented by an identifier for the respective state as listed in the candidateAbstractStates. Similarly, each action may be represented by an identifier for the respective action as listed in the possibleActions of the respective candidateAbstractState.

#### 5.4.4.3	Potential solutions for AI/ML performance evaluation in inference phase

This solution comprises of the following aspects:

- 	For getting the inference output, the MDA MnS (see TS 28.104 [2]) already supports MDA reporting by notifications, file and data streaming. The same approach can be applied to reporting other kinds of inference output (NWDAF analytics report, RAN intelligence output). A common data format may be defined for all kinds of inference outputs, and the format will be decided in normative phase.

- 	For providing the feedback about the inference output, the IOC representing the feedback, for example named as InferenceFeedback, can be used to allow the MnS consumer to create an instance on the producer. This IOC contains the following attributes:

-	inference report id;

-	indication of whether there are actions to be taken triggered by the inference report;

-	feedback for the inference report, e.g., lack of confidence or accuracy for a specific output information element.

-	For being informed about the actions taken trigged by the inference output, the NRM notification representing the already taken actions triggered by the inference is used. For example defining a new IOC named as ActionsTriggeredByInferenceOutput, or enhancing the existing notifications for the NRMs. The notification contains the following information:

-	inference report id that triggers the action;

-	actions taken (this information is already supported when enhancing the existing notifications).

- 	For monitoring the network performance related to each inference function, the performance measurements related to each inference function need to be defined to allow the MnS consumer to collect:

-	For the performance measurements related to MDAF, the performance measurements listed in the analytics enabling data for each MDA capability can be used for performance evaluation of MDAF (see TS 28.104 [2]);

-	For the performance measurements related to NWDAF, the studies are described in TR 28.864 [6];

-	For the performance data related to RAN intelligence functions, including RAN intelligence ES function, RAN intelligence MRO function, RAN intelligence MLB function, the MDT data and following performance measurements for MRO, Energy Efficiency and MLB respectively can be reused:

-	for RAN intelligence ES function, the measurements related to distributed energy saving (see clause 6.2.3.1.3.2 of TS 28.310 [7], TS 28.552[8]) for NG-RAN can be reused;

-	for RAN intelligence MRO function, the measurements related to D-MRO (see clauses 7.1.2.3.1 and 7.1.6.3.1 of TS 28.313 [9], TS 28.552 [8]) can be reused;

-	for RAN intelligence MLB function, the measurements related to D-MLB (see clauses 7.1.5.3.1 of TS 28.313 [9], TS 28.552 [8]) can be reused.

#### 5.4.4.4	Potential solutions for AI/ML performance abstraction

Introduce an IOC for AI/ML performance abstraction as the entity that is the producer of AI/ML performance abstraction and supports all the related services for request and delivery of qualified ML performance Abstraction. The IOC may be named MLPerformanceAbstraction.

MLPerformanceAbstraction may be name-contained in either a Subnetwork, a ManagedFunction or a ManagementFunction.

-	The MLPerformanceAbstraction receives a request for the qualification and abstraction of one or more ML Performance metric(s) of a specific ML Entity.

The request might be an IOC and may be named MLPerfQualRequest.

-	The request may contain the raw metrics (Confusion Matrix, Precision and Recall, F1-score, AU-ROC, …….) or the input(s) and the expected output(s) of the stated ML entity for which performance abstraction is desired.

-	For each request, the MLPerformanceAbstraction provides a response that contains the report on the qualified abstract performance. The report might be named MLAbstractPerfReport.

Abstraction of ML Performance

An IOC is introduced to support ML performance abstraction. It might be named mlPerformanceIndex. The mlPerformanceIndex has a pre-defined index range that specifies the absolute minimum and maximum performance. It is introduced as an attribute to the mlPerformanceIndex and might be named mlPerformanceIndexRange.

-	The mlPerformanceIndexRange is standardized and known by both the consumers and the producers of AI/ML services and may be applied for different performance metrics.

-	For each performance metric, the performance abstraction producer should map the specific performance value to the predefined mlPerformanceIndexRange to generate the specific mlAbstractPerfIndex value for that performance metric value. This can then be communicated to the consumers, who do not need to know the original performance metric value or its interpretation but can still make sense of the achieved performance.

-	The mlPerformanceIndex may be computed based on only one performance metric. However, an aggregate index may also be computed for a combination of multiple performance metrics, to generate the specific mlAggregatePerfIndex value.

Requesting and Reporting on ML Performance Abstraction

The MLPerformanceAbstraction has the capability to compute an abstraction of the performance of a given ML Entity given the achieved performance of the ML Entity on the specific metrics. A mlPerformanceIndexRange is configured onto the MLPerformanceAbstraction to indicate the fixed range on which all performances must be mapped.

>	For each request to abstract and qualify the performance of the a given ML Entity, an MnS consumer creates a new request, might be named MLPerfQualRequest, on the MLPerformanceAbstraction, i.e., MLPerfQualRequest should be an IOC that is instantiated for each request to abstract and qualify performance.

>	Any request for qualifying and abstracting performance must state the following:

>>	mLFunctionID: the identifier of the specific AI/ML inference function the MnS consumer wishes to have performance qualified and abstracted. In some cases, the request may be submitted by the network function  having ML capabilities itself, in such a case the network function submits its own DN.

>>	MLEntityID: The request may optionally state the identifier of the specific ML Entity for which the MnS consumer wishes to have performance qualified and abstracted.

>>	mlPerformanceMetrics: The request must indicate the specific one or more ML-related performance metrics and their values that should be evaluated by the MLPerformanceAbstraction for generating the abstract performance index.

>	Following the request, the MLPerformanceAbstraction computes the mlPerformanceIndex as the abstraction of the performance metric values as fitted to the specified mlPerformanceIndexRange.

For the computed mlPerformanceIndex, the MLPerformanceAbstraction compiles report containing the computed mlPerformanceIndex. Then it forwards it to the MnS consumer (the function that requested for the performance abstraction) to notify the MnS consumer about the outcomes of the performance abstraction. Subsequent to reporting the MLPerformanceAbstraction may also publish the abstract performance to some shared publication space. The report is a data type and might be named MLAbstractPerfReport.

#### 5.4.4.5	Potential solutions for ML entity performance indicators query and selection

This solution extends the ModelPerformance data type to specify which ML performance indicators can be supported by ML Entity or its hosting function (e.g., MLTrainingFunction or MLInferenceFunction). The same data type can be used to activate the notification on specific ML performance indicators based on the request by the authorized MnS consumer.

-  SupportedMlPerformance <<dataType>>

This data type specifies the performance indicator which can be supported by an AI/ML entity or a function (e.g., MLTrainingFunction or MLInferenceFunction).  It contains the tuples of supportedPerformanceMetric and activatedPerformanceMetric attributes. The supportedPerformanceMetric indicates performance metric which AIML entity or a function is capable of providing e.g., accuracy/precision/recall/F1-score/MSE/MAE. The authorized MnS consumer should be notified only on a specific subset of such performance metrics for which the activatedPerformanceMetric indicator is set.

-  Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedPerformanceMetric | M | T | F | F | T |
| activatedPerformanceMetric | M | T | F | F | T |

-  Attribute definitions

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| SupportedPerformanceMetric | It indicates the performance metric which AIML entity or a function is capable of providing e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ActivatedPerformanceMetric | It indicates whether the ML MnS consumer activated the notifications on specific performance metric. Setting this attribute to "TRUE" the SupportedPerformanceMetric will be notified to the consumer. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |

#### 5.4.4.6	Potential solutions for policy-based performance indicator selection

Following is the proposed solution based on information model defined in TS28.105 [2].

- 	Existing ModelPerformance <<datatype>> as part of MLTrainingRequest IOC may be extended optionally with attribute that represents the behavioural requirements as a policy. This attribute may be named as "trainingPolicyIndicator".

- 	This attribute may contain information on the magnitude of the sensitive inference as a triplet. Some examples may look like following.

Example 1:

False Positives, less, 10

- 	The above example indicates the training to be performed such that the probability of false positives is less than 10% in case of classification.

Example 2:

Over-Prediction, high, 80

- 	The above example indicates the training to be performed such that the probability of over-prediction is greater than 80% in case of regression.

This information along with the existing "performanceScore" and "performanceMetric" may help the ML Training Producer to train the ML entity efficiently for the specific use case. This attribute may be applicable when configured in MLTrainingRequest IOC and not applicable in MLTrainingReport IOC.

A similar solution can be applied to the testing and inference functions.

### 5.1.5	Evaluation

The solution described in clause 5.4.4.1 uses the NRM based solution for the consumer to query the supported ML training capabilities, and reuses the existing IOC and attributes in maximum extend. The new and existing NRMs can be easily and clearly correlated in this solution. Therefore, the solution described in clause 5.4.4.1 is a feasible solution.

The solution described in clause 5.4.4.2 enhances the existing information model for the MLEntity with 1 IOC that contains two attributes. These allow to hold the candidate abstract behavior for the ML entity and the applied, e.g., as set by the AI/ML MnS consumer. These information elements should support management and control of the abstract behavior of the ML entity or the related AI/ML inference function. Therefore, the solution described in clause 5.4.4.2 is a feasible solution for monitoring and control of AI/ML behavior.

The solutions described in clause 5.4.4.3:

-	reuses already defined MDA reporting mechanisms for inference output reporting, which may require some minimal change to make the solution (e.g., reporting format) applicable to all kinds of inference functions;

-	uses NRM based solution for providing the feedback and informing the taken actions, which makes the new and existing NRMs can be easily and clearly correlated;

-	reuses the existing performance measurements for monitoring the network performance related to each inference function.

Therefore, the solutions described in clause 5.4.4.3 are feasible.

The solution described in clause 5.4.4.4 reuses the existing provisioning MnS operations and notifications in combination with extensions of the NRM. Indeed, requests for qualifying and abstracting performance of ML training, AI/ML inference function or an ML entity may be instantiated using provisining Management service implemented via CRUD (Create, Read, Update, Delete) operations on the request objects. The solution provides the flexibility to allow any function to be the MnS producer for ML performance abstraction, e.g., the training function or the inference function. It also allows any function that utilizes ML related results to consume that resulting report for the performance abstraction.

Therefore, the solution described in clause 5.4.4.4 is a feasible solution to be developed further in the normative specifications.

The solution described in 5.4.4.5 is NRM-based approach and reuses the existing provisioning MnS operations. It is consistent with the ML entity definitions and enhances its existing attributes. It provides the means to facilitate both capturing the information on the supported performance indicators in different ML phases as well as selecting the performance indicators to be provided using the consistent NRM-based approach.

## 5.2	Event data for ML training

### 5.2.1	Description

In analytics solutions, Performance Measurements (PMs) and Fault Reports (FRs) from various network function are collected and analytics applied on the PMs and FRs to derive events (statistical insights and predictions). For most algorithms, the prediction accuracy depends upon the amount of available relevant historical data, motivating the need to store ever more data, which correspondingly increases the storage and processing resource requirements. However, not all recorded data is useful as the derived events, e.g., captured through analytics processes, may have loss of information OR misinformation e.g., with respect to time of the event.

### 5.2.2	Use cases

#### 5.2.2.1	Pre-processed event data for ML training

For AI/ML algorithms, a large amount of data points does not necessarily add value, e.g., if most of it includes biased data which ends up getting discarded during the pre-processing stages. Instead, the AI/ML algorithms need to have information-rich events data that is condensed but with most of it useful for the required training. For example, one could train an interference optimization solution that learns the best way to combat interference by looking at correlated event which derived from counters of handover failures correlated with signal quality.

It is as such necessary to provide means to isolate and store the information rich network events in the network, to ensure that minimizing storage and processing costs by discarding the unnecessary raw data does not compromise the ability to and still avails adequate historical information to adequately train ML entities. In other words it is necessary for network functions and their management system to generate data about the observed network events, e.g., based on the criteria set by the operator. The network events can then be stored to be used later to train ML entities.

Here, a network event represents a specific combination of activities executed on the network or occurrences in the network identified say by a set of instances of threshold crossings.

Fig 2.2.1-1. Exposing and storing network events data

As maybe seen in Fig 2.2.1-1, there could be multiple sources for the same event data e.g., either the source network function or a related management function. Correspondingly, it may be necessary to eliminate duplications of events or event data that is exposed to the AI/ML inference functions. It may in such cases make sense to have a single events data exposure entity, say called a network events aggregator that exposes data from multiple sources but without duplications.

Relatedly, when an MnS consumer for ML training producer wishes to trigger a training based on such events and not on specific raw data, such a consumer should be enabled to request for such training and clarify that they have training executed based on network events.

### 5.2.3	Potential requirements

REQ-EVENT-DATA-1: The 3GPP management system should enable an authorized consumer to request from the network data producer for network events corresponding to the data produced by that network data producer.

REQ-EVENT-DATA-2: The 3GPP management system should enable a network data producer to generate network events.

REQ-EVENT-DATA-3: The 3GPP management system should enable a  network events aggregator to take the events from different network entities and re-expose them in an aggregated way that eliminates duplications.

REQ-EVENT-DATA-4: The 3GPP management system should enable an authorized MnS consumer to configure the ML Training MnS producer to provide training based on a set of network events.

REQ-EVENT-DATA-5: The 3GPP management system should have a capability to inform an authorized MnS consumer when training is triggered based on an MnS consumer’s request for training based on a set of network events.

### 5.2.4	Possible solutions

1)	Introduce an IOC for processing data to identify network events. The IOC may be called a Network Events Processing Agent (named e.g., NetworkEventsProcessor or NEPA). The NEPA may be associated with data collection and exposure services and may be contained in any ManagedFunction, ManagementFunction or subnetwork. This NEPA is responsible for generating the events data which can be availed to the AI/ML training function, i.e. the training function may configure the NEPA to compile and generate network events with certain characteristics.

a. 	The ML training functions may configure the events on any one or more data sources according to the kinds of events needed by the ML training functions(s)

b.	The data source provides the events to the ML training functions(s) as configured, ML training functions(s) executes the training using those events.

2)	Introduce an information Model for a Network Event as the datatype that can be exposed by an NEPA. The model may distinguish between Metric-Threshold Crossing events and Object-Status Change Events

a.	Metric Threshold Crossing events derived from a thresholds or a combination of multiple thresholds, i.e., the Metric Threshold Crossing events are the events captured when one or more metrics cross the one or more configured thresholds. The definition of the Metric Threshold Crossing events may reuse the threshold monitoring MnS defined in TS 28.550 as well as the related NRMs defined in TS28.622 [13]. Thereby, new parameters and metrics for which thresholds can be set may have to be defined.

b.	Object Status Change Events are the events captured when the status of a managed object instance changes be it due to internal processes within the MOI or due to external actors doing something on the MOI.

3)	Introduce a Status-Change-monitoring IOC contained by any ManagedElement, ManagedFunction, or ManagementFunction, to enable an MnS consumer to define Object Status Change Events.

4)	Define/specify a set of network events for different managed objects. Example network events include.

Table 5.2.4-1: Examples of potential Metric Threshold Crossing events

| Event- source Function | Event Name | Event description | Input/Metric | MOI | Condition | Threshold | Monitoring Period |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BTSs; NBs; eNBs; gNBs; BSC; RNC | High Call Drop Rate event | Call Drop Rate more than a configurable threshold | Call Drop Rate | Cell | greater than | 2% | 15 minutes |
|  | Low Availability KPIs event | Availability KPIs dropping below a configurable threshold | Availability | Cell | less than | 99% | 30 minutes |
|  | Low Retainability KPIs event | Retainability KPIs dropping below a configurable threshold | Retainability | Cell | less than | 98% | 30 minutes |
|  | High Traffic event | Traffic greater than a configurable threshold | Cell load / PRB Utilization | Cell | greater than | 80% | 15 minutes |
|  | Interference event | User has experienced interference | SINR | Cell | less than | X dB | 15 minutes |
|  |  |  | Serving cell RSRP | Cell | greater than | Y dBm | 15 minutes |

Note that some events are the results of combinations of multiple thresholds (derived from the success of multiple threshold monitors).

Table Table 5.2.4-2: Examples of potential Object-Status Change events

| Event-source Function | Event Name | Event description | MOI | Affected unit /parameter | Change/ value |
| --- | --- | --- | --- | --- | --- |
| BTSs; NBs; eNBs; gNBs; BSC; RNC; NMS | HW Upgrade event | System Module HW version upgraded | BTS | System Module, Radio Module, … | HW version |
|  | SW Upgrade event | System Software Upgraded | BTS | System Module, Radio Module, … | SW version |
|  | Capability Enablement event | A specific Capability Enabled on the MOI | BTS | Spectrum Sharing | Spectrum Range for affected RATs |
|  | New Sector Addition Event | A new Sector getting added to a Site | Cell | Capacity and Coverage | Number of Sectors / Cells |
| Network Management | Parameter change event | CM Parameter changes applied for specific network element | Cell | Configuration Parameter | Parameter value |
|  | Home status event | MOI (e.g. site) Rehoming | Site | BSC/RNC, OSS | New BSC/ RNC/ OSS |
| SON, Analytics function | New Site event | New Site Integrated | Site/ gNB | C-SON Functions | Optimization Parameters in C-SON |
|  | - Predicted Congestion | - Trigger for Load Balancing detected | - Cell | - C-SON LBO | - Mobility Parameter changes |
|  | - Frequent Handover Failures | - Trigger for MRO detected | - Cell | - C-SON MRO | - Handover Parameter Changes |
|  | - PCI Conflict | - PCI conflict detected | - Cell | - C-SON PCI | - PCI Changes |
|  | - PRACH Conflict | - PRACH conflict detected | - Cell | - C-SON PRACH | - PRACH related parameter Changes |
|  | - NCR Change | - New First tier neighbour getting added | - Cell | - C-SON ANR | - NCR Changes |
|  | - Frequency Layer Change | - New Frequency Layer added onto a site | - BTS | - C-SON | - Frequency Layer Addition |

### 5.2.5	Evaluation

The solution described in clause 5.2.4.1 reuses the existing provisioning MnS operations and notifications in combination with extensions of the NRM. This solution is also consistent with the approach used by AI/ML training MnS in TS 28.105 [4] which shall be training MnS producer to be configured with the needs to train using events data but shall also be the MnS consumer for the data services that provide events data. The solution provides the flexibility to allow any function to be the MnS producer for the network events data and for any training producer to consume that data for its training.

Therefore, the solution described in clause 5.2.4 is a feasible solution to be developed further in the normative specifications.

## 5.3	ML entity validation

### 5.3.1	Description

During the ML training process, the generated ML entity (see TS 28.105 [4]) needs to be validated. The purpose of ML validation is to evaluate the performance of the ML entity when performing on the validation data, and to identify the variance of the performance on the training data and the validation data. If the variance is not acceptable, the entity would need to be tuned (re-trained) before being made available to the consumer and used for inference.

The training data and validation data are normally split from the same data set with a certain ratio in terms of the quantity of the data examples, therefore they have the same pattern.

### 5.3.2	Use cases

#### 5.3.2.1	ML entity validation performance reporting

In the ML training, the ML entity is generated based on the learning from the training data, and validated using validation data. The performance of the ML entity has tight dependency on the data (i.e., training data) from which the ML entity is generated. Therefore, an ML entity performing well on the training data may not necessarily perform well on other data e.g., while conducting inference.. If the performance of ML entity is not good enough as result of ML validation, the ML entity will be tuned (re-trained) and validated again. The process of ML entity generation and validation is repeated by the ML training function, until the performance of the ML entity meets the expectation on both training data and validation data. The producer in the end selects one or more ML entities with the best level performance on both training data and validation data as the result of the ML training, and reports to the consumer. The performance of each selected ML entity on both training data and validation data also need to be reported.

The performance result of the validation may also be impacted by the ratio of the training data and validation data. Consumer needs to be aware of the ratio of training data and validation data, besides the performance score on each data set, in order to be confident about the performance of ML entity.

### 5.3.3	Potential requirements

REQ-MODEL_VLD-CON-1	The MLT MnS producer should have a capability to validate the ML entities during the training process and report the performance of the ML entities on both the training data and validation data to the authorized consumer.

REQ-MODEL_VLD-CON-2	The MLT MnS producer should have a capability to report the ratio (in terms of the quantity of the data examples) of the training data and validation data used for training of an ML entity during the training process.

### 5.3.4	Possible solutions

#### 5.3.4.1	Validation performance reporting by enhancing the existing IOC

In TS 28.105 [4], the ML entity training report is provided by MLTrainingReport IOC, which includes the attribute indicating the performance of the ML entity when performing on the training data.

To support the ML entity validation performance reporting, a new optional attribute can be defined in the MLTrainingReport IOC to indicate the performance of the ML entity when performing on the validation data.

### 5.3.5	Evaluation

The possible solution described in clause 5.3.4.1 enhances the existing MLTrainingReport IOC with a new optional attribute indicating the performance of the ML entity when performing on the validation data, the change is lightweight and it is backward compatible, therefore it is a feasible solution.

## 5.4	ML entity testing

### 5.4.1	Description

After an ML Entity is trained, validation is done to ensure the training process is completed successfully. However, even when validation is conducted successfully during development, it is necessary to test and check if the ML entity is working correctly under certain runtime contexts or using certain testing data set. Testing may involve interaction with third parties (besides the developer of the ML training function), e.g., the operators may use the ML training function or third-party systems/functions that may rely on the results computed by the ML entity for testing.

After completing the ML entity training, and when the performance of the trained ML entity meets the expectations on both training and validation data, the ML entity is made available to the consumer(s) via the ML training report (see MLTrainingReport IOC in TS 28.105 [4]). Before applying the ML entity to the target AI/ML inference function, the ML training MnS producer may need to allow the consumer to evaluate the performance of the ML entity via the ML testing process using the consumer’s provided testing data. The testing data have the same pattern as the input part of the training data.

For these reasons, provision of ML entity testing and its control need to be standardized to enable the multi-vendor interaction among the different systems. If the testing performance is not acceptable or does not meet the pre-defined requirements, the consumer may request the ML training producer to re-train the ML entity with specific training data and/or performance requirements.

### 5.4.2	Use cases

#### 5.4.2.1	Consumer-requested ML entity testing

After receiving an ML training report about a trained ML entity from the ML training MnS producer, the consumer may request the testing MnS producer to test the ML entity before applying it to the target inference function. In the ML testing request, the consumer provides the testing data which have the same pattern as the input part of the training data.

Any ML entity needs to be tested with specific inputs and features that are applicable to the use case and the applicable deployment environment.

The ML testing MnS producer performs the ML testing using the consumer’s provided testing data. The ML testing is to conduct inference on the tested ML entity using the testing data as the inference inputs and produce the inference output for each testing dataset example.

The AML testing MnS producer may be the same as or different from the ML training MnS producer.

After completing the ML testing, the ML testing MnS producer provides the testing report indicating the success or failure of the ML testing to the consumer. For a successful ML testing, the testing report contains the testing results, i.e., the inference output for each testing dataset example.

The ML testing MnS producer needs to have the capabilities to provide the services needed to enable the consumer to request testing and receive results on the testing of a specific ML entity or of an application or function that contains an ML entity.

To achieve the desired outcomes, any ML entity needs to be tested with the appropriate testing data (e.g. batch data or continuous data streams), which can reflect the current status of the network where the ML entity is expected to be deployed. Correspondingly, the ML testing MnS producer needs to support the required management services to test the ML entities.

#### 5.4.2.2	Control of ML entity testing

Given a testing capability as provided by a given ML testing MnS producer, a consumer (e.g., an operator) may wish to control and manage that testing process capability. For example, the operator may wish to define policies on how frequent testing for a given ML entity may be executed. Correspondingly, the 3GPP management system needs to provide the capability to allow the ML entity testing to be configured.

Figure 5.4.2.2-1: ML entity testing and control

### 5.4.3	Potential requirements

REQ-AI/MLTEST-1: The ML testing MnS producer should have a capability to enable  an authorized consumer to request the testing of a specific ML entity.

REQ-AI/MLTEST-2: The ML testing MnS producer should have a capability to create a testing process instance per the testing request for an authorized consumer.

REQ-AI/MLTEST-3: The ML testing MnS producer should have a capability to report to an authorized consumer the results of a specific instance of ML testing process with the result of a successful ML entity testing containing the inference output for each testing data example.

REQ-AI/MLTEST-4: The ML testing MnS producer should have a capability to enable  an authorized consumer (e.g. the operator) to configure or modify an instance of ML testing process.

REQ-AI/MLTEST-5: The ML testing MnS producer should have a capability to test a specific ML entity using specific data specified by the consumer, using data at a location address specified by the consumer, using data with specific characteristics defined by the consumer, or using continuous data streams provided from the consumer.

REQ-AI/MLTEST-6: The ML testing MnS producer should have a capability to test a specific ML entity for a specified expected runtime context as may be stated by the consumer.

REQ-AI/MLTEST-8: The ML testing MnS producer should support a capability to enable  an authorized consumer to define the reporting characteristics related to a specific instance of ML testing request.

REQ-AI/MLTEST-9 The ML testing MnS producer should support a capability for an authorized consumer to manage the AI/ML testing request, including suspending, resuming, cancelling the request, or adjusting the desired runtime context of the testing.

### 5.4.4	Possible solutions

#### 5.4.4.1	NRM based solution

This solution uses the instances of following IOCs for interaction between ML testing MnS producer and consumer to support the ML entity testing:1) 	The IOC representing the ML entity testing request, for example named as MLTestingRequest.

This IOC is created by the ML entity testing MnS consumer on the producer, and it contains the following attributes:

- 	identifier of the ML entity to be tested;

-	testing environment requirements, e.g., expected runtime context;

-	testing data.

2) 	The IOC representing the ML entity testing policy, for example named as MLTestingPolicy.

This IOC is created by the ML entity testing MnS consumer on the producer to control the testing initiated by the producer, and it contains the following attributes:

- 	identifier or inference type of the ML entity to be tested;

-	testing environment requirements, e.g., expected runtime context;

-	testing triggers, i.e., the conditions that would trigger the testing of an ML entity.

3) 	The IOC representing the ML entity testing process, for example named as MLTestingProcess. This MOI is created for the ML entity testing process corresponding to the testing requested by the consumer per the IOC described in 1), or the testing initiated by the producer based on the given testing policy per the IOC described in 2).

This IOC is created by the ML entity testing MnS producer and reported to the consumer, and it contains the following attributes:

- 	identifier of the ML entity being tested;

-	the associated ML entity testing request;

-	the associated ML entity testing policy;

-	testing progress;

-	testing environment, e.g., the testing runtime context;

-	testing data to be used;

-	control of the process, like cancel, suspend and resume.

4) 	The IOC representing the ML testing report, for example named as MLTestingReport.

This IOC is created by the ML testing MnS producer and reported to the consumer, and it contains the following attributes:

- 	identifier of the tested ML entity;

-	the associated ML entity testing request;

-	the associated ML entity testing process;

-	testing result indicating the success or failure and containing the inference output for each testing data example for successful case, and the failure reason for the failed case.

The examples of IOCs and their relations between the IOCs are depicted in the figure below.

Figure 5.4.4.1-1: Example of ML entity testing related NRMs

NOTE: The name of the IOCs and attributes are to be decided in normative phase.

### 5.4.5	Evaluation

The solution described in clause 5.4.4.1 adopts the NRM-based approach, which reuses the existing provisioning MnS operations and notifications. This solution is also consistent with the approach used by ML training MnS defined in TS 28.105 [4]. It does not only reuse the existing capabilities (provisioning MnS operations and notifications) to a greater extent, but also provides the flexibility to facilitate both co-located and distributed implementation and deployment of ML training MnS and ML testing MnS producers by using the consistent NRM-based approach.

Therefore, the solution described in clause 5.4.4.1 is a feasible solution.

## 5.5	ML entity loading

### 5.5.1	Description

ML entity loading refers to the process of making an ML entity available in the operational environments, where it could start adding value by conducting inference (e.g., prediction). After a trained ML entity meets the performance criteria per the ML entity testing, the ML entity could be loaded in target inference function(s) in 3GPP system, e.g., via a software installation, file transfer, or a configuration management procedure and subsequently activated. The ML entity loading may be requested by the consumer, or initiated by the producer based on the loading policy (e.g., the threshold of the testing performance of the ML entity, threshold of the inference performance of the existing ML model, predefined time schedule, etc.) provided by the consumer.

The loading of ML entity has no implication about "push" or "pull" method.

After an ML entity is loaded in the  target inference function, the data fed to the ML entity may change to the level where it is different from the data used in the initial prior training of the respective ML entity. To improve model performance with the changed data, the ML entity therein may need to be retrained and reloaded.

### 5.5.2	Use cases

#### 5.5.2.1	ML entity loadingcontrol and monitoring

This use case is appliable to the deployment scenario where the ML training function and inference function are not co-located.

After the ML entity is trained and tested, the ML entity needs to be loaded by the ML entity loading MnS producer to the target inference function(s) per the request from the MnS consumer or initiated based on a consumer predefined loading policy.

NOTE: ML entity loading MnS producer may be a separate entity or co-located with the MnS producer of the inference function or training function.

One potential reflection of loading policy is to enable a scheduled loading. ML models are typically trained and tested to meet specific requirements for inference, addressing a specific use case or task. Inference requirements could change regularly. For example, a network node supported by AI/ML capability may require employing a specifically trained/different type of ML entity at different time of day, or a specific day in the week with an already known repeated pattern. For example, a gNB providing coverage for a specific location is scheduled to accommodate different load level and/or pattern of services at different time of the day. A dedicated ML model (specifically trained and/or varying type altogether) may be required.

Once the ML entity has been loaded in the target inference function(s), some MnS consumers may need to know the available information of ML entity and to determine the next appropriate action. In this case the MnS consumer needs to be notified about the ML entity loading or be able to retrieve the loading information of the ML entity.  This would allow the consumer to e.g., request ML entity re-training if e.g., performance fall below certain threshold or request the loading of different ML entity altogether, etc.).

The general information used to describe a loaded ML entity may include:

-	Resource information, which describes the static parameters of the ML entity (e.g., mLEntityVersion, mLEntityId, trainingContext, see TS 28.105 [4])

-	Management information, which describes the information model that is used for ML entity lifecycle management (e.g., activation flag, status, creation time, last update time).

-	Capability information, which describes the capability information (e.g., inference type, performance metrics).

Editor's Note: the liability aspect on loading the ML entity to inference function is FFS.

### 5.5.3	Potential requirements

REQ-MODEL_DPL-CON-1: The ML entity loading MnS producer should have a capability allowing the consumer to request and retrieve loadinginformation of an AI/ML entity.

REQ-MODEL_DPL-CON-2: The ML entity loading MnS producer should have a capability to notify the consumer about the loadinginformation of an ML entity.

REQ-MODEL_DPL-CON-3: The ML entity loading MnS producer should have a capability allowing the consumer to request the loading of an ML entity to the target inference function(s).

REQ-MODEL_DPL-CON-4: The ML entity loading MnS producer should have a capability allowing the consumer to provide the loading policy for an ML entity.

### 5.5.4	Possible solutions

#### 5.5.4.1	NRM based solution

This solution uses the instances of following IOCs for interaction between ML loading MnS producer and consumer to support the ML entity loading, where the ML loading MnS producer could be part or a separate entity of the inference function:

1) 	The IOC representing the ML entity loading request, named for example as MLEntityLoadingRequest.

This IOC is created by the ML entity loading MnS consumer on the producer, and it contains the following attributes:

- 	identifier of the ML entity to be loaded;

-	the identifier (e.g., DN) of target inference functions where the ML entity is loaded to. This attribute is optional if the target inference function is itself that provides the ML entity loading MnS.

2) 	The IOC representing the ML entity loading policy, for example named as MLEntityLoadingPolicy.

This IOC is created by the ML entity loading MnS consumer on the producer, so that the producer can load the ML entity according to the policy without an explicit loading request from the consumer, and it contains the following attributes:

- 	identifier or inference type of the ML entity to be loaded;

-	trigger of ML entity loading, including e.g., pre-defined scheduled loading, a threshold of the testing performance of the ML entity and/or a threshold of the inference performance of the existing ML entity in the target inference function(s);

-	identifier (e.g., DN) of target inference functions where the ML entity is loaded to. This attribute is optional if the target inference function is itself that provides the ML entity loading MnS.

3) 	The IOC representing the ML entity loading process, for example named as MLEntityLoadingProcess.

This IOC is created by the ML entity loading MnS producer and reported to the consumer, and it contains the following attributes:

- 	identifier of the ML entity being loaded;

-	associated ML entity loading request;

-	associated ML entity loading policy;

-	identifier (e.g., DN) of the target inference function; This attribute is optional if the target inference function is itself that provides the ML entity loading MnS.

-	loading progress;

-	control of the loading process, like cancel, suspend and resume.

How to load the ML entity by the MnS producer is vendor specific.

4) 	The IOC representing the ML entity loaded in the inference function, for example by extension of the existing IOC (MLEntity) representing the ML entity, or by a new IOC.

This IOC is created by the ML loading MnS producer and reported to the consumer, and it contains the following attributes:

- 	identifier of the loaded ML entity;

-	associated trained ML entity (e.g., DN of the MOI representing the trained ML entity), which is to be loaded to the inference function;

-	associated ML entity loading process;

-	status (such as activated, de-activated, etc) of the loaded ML entity.

The examples of IOCs and their relations between the IOCs are depicted in the figure below.

Figure 5.5.4.1-1: Example of ML entity loading related NRMs

NOTE: Further details including e.g., the name of the IOCs and corresponding attributes are to be decided in

normative phase.

### 5.5.5	Evaluation

The solution described in clause 5.5.4.1 adopts the NRM-based approach, which to a great extent reuses the existing provisioning MnS operations and notifications. This solution is also consistent with the approach used by ML training MnS defined in TS 28.105 [4]. It does not only reuse the existing capabilities (provisioning MnS operations and notifications), but also cater for the flexibility that is needed to facilitate both co-located and separate implementation and deployment options of ML training and/or testing MnS and ML loading MnS by using the consistent NRM-based approach.

Therefore, the solution described in clause 5.5.4.1 is considered a feasible solution.

## 5.6	AI/ML Inference History

### 5.6.1	Description

For different automation requirements, network and management automation functions (e.g. gNB, MDAS, SON) may apply Machine Learning functionality to make the appropriate inferences in different contexts. Depending on the contexts, the ML entity may take different decisions at inference with different outcomes. The history of such inference decisions and the context within which they are taken may be of interest to different consumers.

### 5.6.2	Use cases

#### 5.6.2.1	Tracking AI/ML inference decision and context

The deployed ML entity may take different decisions at inference in different contexts and with different outcomes. The selected decisions may need to be tracked for future reference, e.g., to evaluate the appropriateness/ effectiveness of the decisions for those contexts or to evaluate degradations in the ML entity's decision-making capability. For this, the network not only needs to have the required inference capabilities but needs also to have the means to track and enable usage of the history of the inferences made by the ML entity.

Figure 5.6.2.1-1: Example use and control of ML inference history request and reporting.

### 5.6.3	Potential requirements

REQ-MLHIST-1 The producer of ML inference history should have a capability allowing an authorized consumer to request the inference history of a specific ML entity.

REQ-MLHIST-2 The producer of ML inference history should support a capability to enable an authorized consumer (e.g. the function/entity that generated the Request for ML inference history) to define the reporting characteristics related to a specific instance of ML inference history or the reporting thereof.

### 5.6.4	Possible solutions

-	Introduce ML Inference History (named e.g. MLInferenceHistory) as an IOC, which may be contained in a ManagedFunction, ManagementFunction or subnetwork. The MLInferenceHistory then may contain or be associated with the critical properties and modules needed to accomplish ML Testing, including:

-	the list of MLInferenceHistoryRequests,

-	the list of ML entities either under Testing or to be considered for Testing,

-	MLInferenceHistoryReporting to report on MLInferenceHistoryRequests or their related outcomes.

-	Introduce ML Inference History Request (named e.g., MLInferenceHistoryRequest) as an IOC, which may be instantiated by the consumer (e.g. an operator, a managed functions or a management function) for any required history. Each MLInferenceHistoryRequest

-	May be associated to exactly one deployed ML entity

-	may contain specific reporting requirements, e.g., one attribute ReportingPeriod may define how the MLInferenceHistory may report about the MLInferenceHistoryRequest,

-	may have a source to identify where its coming from. The sources may for example be an enumeration defined for network functions, operator roles, or other functional differentiations.

-	The MLInferenceHistoryRequest may prescribe a list of Reporting-Context, which describes the list of constraints or conditions that may evaluate to true when the Reporting is executed. The Reporting-Context may be a triple <Attribute, Condition, ValueRange >  where:

-	Attribute: describes a specific attribute of or related to the object or the use case that relates to the ML entity on which reporting is executed. It may also refer to the characteristics of such object (e.g., its control parameter, gauge, counter, KPI, weighted metric, etc.). It may also refer to an attribute related to the operating conditions of the object or use case (such as weather conditions, load conditions, etc.).

-	Condition: expresses the limits within which the Attribute is allowed/supposed to be. The allowed values for the condition may include: "is equal to"; "is less than"; "is greater than"; "is within the range"; "is outside the range".

-	ValueRange: describes the range of values that are applicable to the Attribute as constrained by the Condition.

-	Introduce a ML Inference History Reporting (named e.g., MLInferenceHistoryReporting) as an IOC, which may be used to model the capability of compiling and delivering reports and notifications about MLInferenceHistory or its associated MLInferenceHistoryRequests. The MLInferenceHistory may generate one or more ML Inference History Reports via one or more instances of MLInferenceHistoryReporting.

-	Each ML Inference History Report may be associated to one or more ML entities for which InferenceHistory is requested and/or reported

Editor’s note: there may be potential alignment with the solution for historical data handling from  Rel-18 Study on management data collection study.

### 5.6.5	Evaluation

The solution described in clause 5.6.4 adopts the NRM-based approach, proposing three new information object classes with clear association relationship internally and with existing information element “MLEntity”. It fully reuses the existing provisioning MnS Operations and notifications for control of Inference History Request and reporting. The implementation of this NRM-based solution is straightforward.

Therefore, the solution described in clause 5.6.4 is a feasible solution for AI/ML Inference History.

## 5.7	ML context

### 5.7.1	Description

MLContext attribute represents the status and conditions related to the ML entity (cf. TS 28.105[4]). This may include the network context as defined in TS 28.104 [2] as well as other conditions that may be applicable to the ML entity but are not part of network characteristics e.g. the time of day, season of the year. As part of ML model performance management there is the identification of the problem that the ML model is meant to address or deal with. As described in TS 28.104[2], the differences in the network context, i.e., network status, under which data is collected to produce analytics, significantly affect the produced analytics. Similarly, the changes in the AI/ML context, e.g., the characteristics of the data related to the network status and conditions used for ML model training, testing and deployment may affect the ML entity performance, thus may represent a problem for the ML entity. Thus management capabilities are needed to enable awareness of the AI/ML context in terms of the identification as well as monitoring and reporting of changes in AI/ML context as part of the identification of the problem that the ML entity is meant to address or deal with.

### 5.7.2	Use cases

#### 5.7.2.1	ML context monitoring and reporting

AI/ML context related to ML model training, testing and deployment needs to be identified by characterizing the input data used by the ML model is targeted to work. As an example, such characterization may be done based on the statistical properties of data. Monitoring of such AI/ML context serves to detect the changes and anomalies in the AI/ML context. Some anomalies may be considered as a problem that ML entity is facing as it may lead to its performance degradation. Therefore, the consumer of the related AI/ML service needs to be informed about such observed AI/ML context change.

#### 5.7.2.2	Mobility of ML Context

In several network automation use cases, the respective AI/ML inference function cannot cover the complete network by employing single ML entity instance. An ML entity may be trained for a specific local context, and similarly, a different context may be applicable for inference, so the ML entity may be characterized by different trainingContext and an expecetdInferenceContext. However, the network scopes where the data used for training and inference is collected does not always necessarily overlap with the network scopes in which the function makes decisions. The context of ML entities or AI/ML inference function may need to distinguish between context for generating decisions or insights, the context from which it generates measurements or data as well as the context in which it is prepared before being active for inference. So, the characteristics of the respective AI/ML inference function need to be distinguished depending on the different contexts of the AI/ML inferencefunction. As such besides the validity scope defined by the trainingContext and an expecetdInferenceContext, the ML entity should also be characterized by specific measurement scopes, where the input measurements are collected. And these may also be separately defined for the 2 use cases.

#### 5.7.2.3	Standby mode for AI/ML inference functionality

Where the respective AI/ML inference function cannot cover the complete network in one ML entity instance, multiple instances of ML entities may be required, one for each specific network scope, such as a cell. When a network automation use case requires several ML entities instances, where each has its own limited validity scope (a geographical area or a subnetwork), transfers of machine learning context, i.e. “handovers” between the ML entities covering different validity scopes (not necessarily identical to cell coverage area), are needed. Accordingly, the AI/MLinferencefunction or the ML entities therein may have different roles, either as active or standby decision makers.

Consider the use case where a different ML entity instance is needed for each Base Station, i.e. the validity scope defined by the expecetdInferenceContext is a specific gNB. An instance of this is predictive ML-driven handover where an ML entity  is trained to decide the optimal handover point and target cell based on the UE measurements. Furthermore, the model inference is done in the UE. When the UE hands over to a cell in another gNB, which is in another validity scope, a new ML entity  instance fitting the new validity scope needs to be deployed in the UE. This is illustrated by Figure 5.7.2.3-1 a) where the UE uses ML entity instance 1 in both cells 1 & 2 but when the UE hands over to cell 3, which is outside the validity area of ML entity instance 2 needs to be deployed to the UE.

However, the deployment may require uploading the required ML entity instance into the UE and initializing the ML entity, for example to collect and feed the necessary input data to setup the required internal states, such as in Long-Short Term Memory (LSTM) Recurrent Neural Networks (RNNs). Accordingly, it may take significant time before the new ML entity becomes active and operational. Moreover, if the UE hands over back to cell 2 after a short stay in cell 3 (ping pong), the UE needs to immediately re-deploy ML entity instance 1, compounding the problem further.

To minimize this risk, there should be a "prepared scope" defined for each ML entity, which is the scope within which the ML entity  is deployed and initialized but not activated for inference.

- b)

Figure 5.7.2.3-1: Example mobility of AI/ML context - a) validity scopes, b) validity and standby scopes

This is illustrated by Figure 5.7.2.3-1 b) where besides the validity areas, standby areas are defined for each ML entity  instance, e.g., ML entity  1 is active in cells 1 and 2 but standby in cell 3. This implies that the ML entity  1 should be availed to the UE in cell 3 even if the UE cannot use ML entity  instance 1 in cell 3. To support this, it must be possible to configure both, the validity areas and the standby areas for ML entities and to define their role in them, i.e., either active, or prepared.

### 5.7.3	Potential requirements

REQ-ML_CTX -1: The MLT MnS producer  should have a capability to identify and monitor the AI/ML context, as well as to inform the MnS consumer about observed changes in AI/ML context.

REQ-ML_CTX-2: The MLT MnS producer should have a capability for an authorized MnS consumer to configure or read the measurement scope of an ML entity for ML training.

REQ-ML_CTX-3: The 3GPP Management system should have a capability for an authorized MnS consumer to configure or read the validity scope of an ML entity for AI/ML inference.

REQ-ML_CTX-4: The  MnS producer responsible for interface configuration should have a capability for an authorized MnS consumer to read the ML entity's inference prepared scope that defines the network scope within which the ML entity is prepared to be in standby mode in preparation for elevating to active mode.

### 5.7.4	Possible solutions

#### 5.7.4.1	MLContext datatype on MLEntity

The IOC MLContext is a <<datatype >> attribute on the MLEntity. The  MLContext is notifiable, so that any interestd party can subscribe to a notification on the MLContext.

when there is a change in the MLContext, e.g. as observed from the statistical properties of data, the notification is sent to the entity that subscribed to the notification.

The MLContext has the following attributes which can be configured by the MnS consumer when defining an MLContext to be moitored.

-	Attribute “area of interest” identifying a scope e.g., the geographical area to be taken into account.

-	Attribute "area granularity" defining the size of the sub-areas of the area of interest for which the statistical properties of data should be identified. It can be expressed for example in km or as a description of a relevant part of the network (e.g., building, street, block, district, city, or state). In case area granularity attribute is not specified by the MnS consumer, contexts related to different areas are determined according to the data distribution detected in the area of interest.

-	Attribute " reporting_threshold " indicating when the deviation in data statistics compared to previously determined context shall be reported. It can be numeric attribute, e.g., indicating the percentage of changes between the currently monitored data statistics and previously identified data statistics.

The notification delivers the MLContextReport that contains the information on partitioning of area of interest into smaller areas (i.e. sub-areas) based on statistical properties of data. The report may also comprise the statistical properties of identified sub-areas. Furthermore, the report may include the information on detected changes in data statistics. Hereby, either the complete information on current data statistics or the actual “delta” compared to previous data statistics may be indicated to the MnS consumer.

The MLContextReport MOI is contained by the MLTrainingFunction or MLInferenceFunction MOI.

#### 5.7.4.2	Mobility of MLContext

To support Mobility of MLContext, extend MLContext (TS 28.105 [4], clause 7.4.3) with additional parameters monitoringScope, validityScope and preparedScope. The monitoringScope is where the data used for training and inference is collected, the validityScope is the network scopes in which the function makes decisions while the preparedScope is the network scopes in which the function is prepared to be ready for inference.

Table 5.7.4.2-1: Extended attributes for MLContext

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| monitoringScope | ? | T | T | ? | ? |
| validityScope | ? | T | T | ? | ? |
| preparedScope | ? | T | T | ? | ? |

### 5.7.5	Evaluation

The solution described in clause 5.7.4.1 is consistent with the MLEntity <<IOC>>  and enhances the existing information element MLContext with three attributes, which are to configure & monitor three types of context. It is a fully NRM-based approach, and reuses the existing provisioning MnS operations and notifications for context configuration and monitoring. It introduces the MLContextReport information class to enable a versatile solution for deliveries of context notifications. It  provides the means to facilitate both capturing the information on the context of the MLEntity, as well enabling the notifications on the context change using the consistent NRM-based approach. Therefore, the solution described in clause 5.7.4.1 is a feasible solution for ML context.

The solution described in clause 5.7.4.2 enhances the MLContext datatype with attributes that characterize the scope of the ML entity. This enables the network or management functions to read the scope and determine the respective scope for which the ML entity supports. It also enables the consumers to configure the scopes differently even where the ML entity changes contexts. Therefore, the solution described in clause 5.7.4.2 is a feasible solution for Mobility of ML context.

## 5.8	ML Entity Capability Discovery and Mapping

### 5.8.1	Description

A network or management function that applies AI/ML to accomplish specific tasks may be considered to have one or more ML entities each having specific  capabilities. The capabilities are either of:

-	a decision-making capability which is in the form of triple <x,y,z> indicating

-	x: the object or object types for which the ML entity can undertake optimization or control

-	y: the configurable attributes on object or object types x, which the ML entity optimizes or controls to achieve the desired outcomes

-	z: the performance metrics which the ML entity optimizes through its actions

-	an analysis capability which is in the form of tuple <x,z> indicating

-	x: the object or object types for which the ML entity can undertake analysis

-	z: the network context (on object x) for which the ML entity produces analysis

Different network functions may need to rely on existing AI/ML capabilities to accomplish the desired automation. However, the applicability of the ML-based solutions and the details of such ML-based solutions (i.e., which ML entities shall be applied and how) for accomplishing those automation functionalities is not obvious. On a high-level, such ML-based solutions may be categorized into cases with or those without ML orchestration. In both cases, management services are required to identify the capabilities of the involved ML entities and to map those capabilities to the desired logic.

### 5.8.2	Use cases

#### 5.8.2.1	Identifying capabilities of ML entities

Network functions, especially network automation functions, may need to rely on AI/ML capabilities that are not internal to those network functions to accomplish the desired automation. For example, as stated in TS 28.104, “an MDA Function may optionally be deployed as one or more AI/ML inference function(s) in which the relevant models are used for inference per the corresponding MDA capability.” Similarly, owing to the differences in the kinds and complexity of intents that need to be fulfilled, an intent fulfilment solution may need to employ the capabilities of existing AI/ML to fulfil the intents. In any such case, management services are required to identify the capabilities of those existing ML entities.

Figure 5.8.2.1-1: Request and reporting on AI/ML capabilities

Figure 5.8.2.1-1 shows that the consumer may wish to obtain AI/ML capabilities to determine how to use them for the consumer's needs, e.g., for its intent targets or other automation targets.

#### 5.8.2.2	Mapping of the capabilities of ML entities

Besides the discovery of the capabilities of ML entities, services are needed for mapping the ML entities and capabilities. In other words, instead of the consumer discovering specific capabilities, the consumer may want to know the ML entities than can be used to achieve a certain outcome. For this, the producer should be able to inform the consumer of the set of ML entities that together achieve the consumer's automation needs.

In the case of intents for example, the complexity of the stated intents may significantly vary - from simple intents which may be fulfilled with a call to a single ML entity to complex intents that may require an intricate orchestration of multiple ML entities. For simple intents, it may be easy to map the execution logic to the one or multiple ML entities. For complex intents, it may be required to employ multiple ML entities along with a corresponding functionality that manages their inter-related execution. The usage of the ML entities requires the awareness of the capabilities of their capabilities and interrelations.

Moreover, given the complexity of the required mapping to the multiple ML entities, services should be supported to provide the mapping of ML entities and capabilities.

Figure 5.8.2.2-1: Mapping execution logic to AI/ML Capabilities

NOTE: 	Figure 5.8.2.2-1 shows that the consumer may wish to obtain the mapping of AI/ML capabilities to some management tasks to determine how to use them for the consumer's needs, e.g., for its intent targets or other automation targets. The management tasks may for example include specific metrics to be optimized, but the candidate tasks to be considered shall be agreed at the normative phase.

### 5.8.3	Potential requirements

REQ-ML_CAP-1 The 3GPP Management system should have a capability for an authorized MnS consumer to request the AI/ML MnS Producer for the capabilities of existing ML entities available within the provider of AI/ML inference.

REQ-ML_CAP-2 The AI/ML MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as a decision described as a triplet <object(s), parameters, metrics> with the entries respectively indicating: the object or object types for which the ML entity can undertake optimization or control; the configuration parameters on the stated object or object types, which the ML entity optimizes or controls to achieve the desired outcomes; and the network metrics which the ML entity optimizes through its actions.

REQ-ML_CAP-3 The AI/ML MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as an analysis described as a tuple <object(s), characteristics> with the entries respectively indicating: the object or object types for which the ML entity can undertake analysis; and the network characteristics (related to the stated object or object types) for which the ML entity produces analysis.

REQ-ML_CAP-4 The 3GPP Management system should have a capability to enable an authorized MnS consumer to request an AI/ML MnS Producer for a mapping of the consumer's targets to the capabilities of one or more ML entities.

### 5.8.4	Possible solutions

The network functions may rely on available AI/ML capabilities to achieve the desired outcome. Such available AI/ML capabilities may need to be discovered as a first step.

The following solution (related to the workflow depicted in Figure 1) may be applicable:

-	When the AI/ML MnS Consumer requests for information on available ML entities and their supported AI/ML capabilities from the AI/ML MnS Producer (e.g., inference producer), the AI/ML MnS Producer provides the AIML_capability in the following form:

-	a decision-making capability in the form of triple <x,y,z> indicating

x: the object or object types for which the ML entity can undertake optimization or control

y: the configurable attributes on object or object types x, which the ML entity optimizes or controls to achieve the desired outcomes

z: the performance metrics which the ML entity optimizes through its actions

-	an analysis capability in the form of tuple <x,z> indicating

x: the object or object types for which the ML entity can undertake analysis

z: the network context (on object x) for which the ML entity produces analysis

-	Introduce the <<datatype>> attribute representing the AI/ML capability e.g. named as  AIML_capability as The attribute is aproperty of any AI/ML MnS Producer or any function that has or contains ML entity e.g. for any AI/ML inference function, ML testing function or ML training function. The AIML_capability may also be added to the ML entity, and

-	The attribute for the AI/ML capability will as such have three attributes

-	An attribute for the managed object: This is conditionally mandatory as either the object or the object type should be stated. It is mandatory if the managed object type is not included.

-	An attribute for the managed object type: This is also conditionally mandatory as either the object or the object type should be stated. It is mandatory if the managed object is not included.

-	An attribute for the configurable attributes on the managed object or managed object types:  This is optional as it only applies for decisions and not for analysis type capabilities.

-	An attribute for the metrics: This which includes either the performance for the decision or the analyses metrics or context should be mandatory.

Figure 5.8.4-1: AI/ML capability request and report

### 5.8.5	Evaluation

The solution described in clause 5.8.4 adopts the NRM-based approach, which reuses the existing provisioning MnS operations and notifications. Moreover, the solution enables reuse of the Discovery MnS to discover both the AI/ML functionality and their capabilities. Introducing the AI/ML_capability <<datatype>> enables a working solution with or without the discovery MnS. Via the discovery MnS this AI/ML_capability will be the returned outcome and without the discovery MnS, the consumer can instead read this AI/ML_capability attribute off the MOI.

Therefore, the solution described in clause 5.8.4 is a feasible solution.

## 5.9	AI/ML update management

### 5.9.1	Description

Due to the complexity and time-varying nature of network, the ML entities previously deployed may no longer be applicable to the current network after running for a period of time. Typically, the performance of a trained model may degrade over time (this is referred to as model drift [5]).

The ML entity needs to be updated timely to ensure the performance of inference and analysis.

### 5.9.2	Use cases

#### 5.9.2.2	AI/ML entities updating initiated by producer

The ML entity updating may be initiated by the AI/ML MnS producer. In order to keep the model at a requested level, the AI/ML MnS producer may periodically conduct ML entity retraining with new available training data. Once a new version ML entity is obtained after the training is finished, it can be used to update the current ML entity with this new version. In another condition, the AI/ML MnS producer may initiate ML entity updating based on the running model performance. For example, if the performance of the running ML model is decreased under a predefined threshold, the AI/ML MnS producer may decide to start re-training and then update the ML entity to a new version which performs better.

### 5.9.3	Potential requirements

REQ-AIML_UPD-CON-1: The AI/ML MnS producer should have a capability to update the ML entities and inform an authorized consumer about the update status.

### 5.9.4	Possible solutions

#### 5.9.4.1	Possible solution#1

Following is the proposed solution based on information model defined in TS 28.105 [2].

-	Extend the MLEntity <<dataType>> with an attribute "updatedTime", which indicates the time that the ML entity is updated. For a trained ML entity, the value of "mLEntityVersion" indicates the version number of the ML entity. When the ML entities updating is initiated and successfully executed, the value of "mLEntityVersion" is modified to be the new version number, and the value of "updatedTime" is the time that the updating is finished. Then AI/ML MnS producer can use the "notifyMOIAttributeValueChange" operation to inform the authorized MnS consumer about the AIML update Status including the updated mLEntity version number and the corresponding updating time.

### 5.9.5	Evaluation

TBD.

## 5.10	AI/ML configuration management

### 5.10.1	Description

The AI/ML inference function (e.g., NG-RAN intelligence ES function as described in TR 37.817 [b]) may use the ML entity for inference.

The AI/ML inference function needs to be configured (e.g., with policies, target, conditions where applicable) in order to conduct inference in the 5G system aligning with operator's expectation.

To enable the AI/ML inference function to perform inference using the preferred ML entity, the relevant ML entity needs to be able to be activated and deactivated.

As described in clause 4.7 in TR 28.813[3], RAN domain ES can use AI to formulate energy saving solutions. Therefore, the AI/ML entities which enabled RAN domain ES function should be controlled by 3GPP management system. The AI/ML entity configuration needs to be triggered to enable RAN domain ES function.

The AI/ML configuration can be initiated by the MnS consumer or initiated by the MnS producer.

The following aspects are described for AI/ML configuration:

- 	Configuration for AI/ML inference function,

-	Configuration for AI/ML entity for RAN domain ES function,

-	Activation for AI/ML inference capabilities on ML entities and inference functions.

### 5.10.2	Use cases

#### 5.10.2.1	ML entity configuration for RAN domain ES initiated by consumer

The AI/ML entity configuration may be initiated by the AI/ML MnS consumer of Cross domain management. AI/ML MnS Consumer monitor network performance and determine whether to trigger the ML entity configuration. For example, for ES purpose, AI/ML MnS Consumer collects the information of the capacity booster cells and coverage cells inside the RAN domain area, then makes the decision for activation ML entity, e.g. for enabled RAN domain ES function, consumer can configure the training/retraining activation/deactivation, related policy at training phase, and activation/deactivation, related policy for inference phase etc. In this case, the consumer can trigger the ML entities so as to get better network performance.

Figure 5.10.2.1-1: ML entity configuration initiated by consumer

#### 5.10.2.2	ML entity configuration for RAN domain ES initiated by producer

The ML entity configuration may be initiated by the AI/ML MnS producer. AI/ML MnS producer can determines whether to trigger ML entity configuration based on network performance and service requirements. For example, after receiving an RAN domain ES requirement, the AI/ML MnS producer can decide to trigger training of the ML entity, e.g. RAN domain ES function. In this case, the AI/ML MnS producer responsible for AI/ML management shall have a capability to trigger the ML entities and inform an authorized consumer about the ML entity status.

Figure 5.10.2.2-1: ML entity configuration initiated by producer

#### 5.10.2.3	Partial activation of AI/ML inference capabilities

An ML entity may provide the AI/ML inference capabilities for a scope (e.g., a specific list of NR cells) of the radio coverage area as either of a decision-making capability or an analysis capability. For a given AI/ML inference function, first, it can be very difficult to accurately “predict” or quantify the benefits of using an ML entity or an inference capability for the ML entity or inference function in a given context of operational system, before using it. Secondly, testing the ML capabilities and entities using test data does not necessarily give a “full picture” on how the ML model will impact the performance of the network or system when it is activated in the operational environment. For example, testing may provide some insights on the accuracy of the ML entity that can be expected once the ML entity is deployed and activated.

Furthermore, it is also necessary to ensure that AI/ML inference capabilities of an ML entity or an inference function that are being activated in operational system will bring the expected/planned benefits and will not further downgrade the existing network performance. Moreover, it is important to provide means to check which particular AI/ML inference capabilities of an ML entity or an inference function are beneficial to be activated in a given context of operational network. Correspondingly, the MnS producer for AI/ML inference management may provide different steps through which the capabilities of an ML entity or inference function may be activated progressively. This abstraction phased activation of the scope of the ML entities may be referred to as “Abstract activation steps”.  For example, with such Abstract activation steps technique, the producer may support a capability to allow only a sub-scope to be activated e.g., to only allow inference activation for a limited or specific number of cells covering part of a geographical coverage area and not the whole city or only for a certain limited period of time (say between 18:00 and 6:00) rather than for the entire operation time.

Another approach to implement partial or progressive activation of AI/ML inference capabilities for an ML entity or an inference function would be through a predefined policy which may include e.g., time scheduled or conditional progressive or phased activation of the inference capabilities or the scope of activation.

So, it is possible that the AI/ML inference function is configured to start using a newly deployed ML entity for one part (e.g., one NR cell of the gNB) of the function but the existing ML entity for the rest parts, and then gradually switch to use the new AI/ML entities for the larger or full scope, by activating/deactivating the AI/ML inference capabilities in the corresponding scope for the AI/ML entities.

Together, these imply that it is important to ensure that the MnS consumer has a finer control on activation and de-activation of AI/ML inference capabilities for an ML entity or an inference function.

#### 5.10.2.4	Configuration for AI/ML inference initiated by consumer

The MnS consumer monitors the network performance and determines on whether to, and when to trigger the AI/ML inference configuration or re-configuration. For example, for NG-RAN intelligence ES function (as described in TR 37.817 [b]), the MnS consumer collects the performance data of the capacity booster cells and coverage cells, then makes the decision for configuring or re-configuring the inference function with a policy may include e.g., performance targets of the inference function, or the activation/deactivation of the ES function and/or the associated ML entities.

In this case, the consumer may need to initiate the AI/ML inference configuration/reconfiguration.

#### 5.10.2.5	Configuration for AI/ML inference initiated by producer

The MnS producer monitors the network performance and determines on whether to, and when to trigger the AI/ML inference configuration or re-configuration. For example, NG-RAN intelligence ES function (as described in TR 37.817 [b]), per the performance of the energy efficiency result by execution of the inference output, the MnS producer may decide to activate or deactivate the inference function, or decide to use another ML entity for inference. In this case, the MnS producer may initiate the configuration and inform an authorized consumer about the configurations. The configuration actions conducted by the MnS producer may also be triggered by a predefined configuration policy.

#### 5.10.2.6	Enabling policy-based activation of AI/ML capabilities

If the activation procedure is entirely relying on the AI/ML MnS consumer to micro-manage every activation step, such process may require extensive signaling between the AI/ML MnS consumer and producer and intrinsically lacks the automation potential. On the other hand, the activation procedure cannot be left fully to the producer either, as the producer may not have a “full picture” on other ML entities/capabilities that are currently in operation, activated by different producers on the request from consumer. The producer needs to be instructed by the consumer on the ways to perform the adequate activation of AI/ML capabilities.

The activation may be instructed via one or more AI/ML activation policies, where an AI/ML activation policy is a sequence of tuples of conditions and activation settings that may be executed by the AI/ML producer. Conditions may define specific outcomes on performance metrics for which a particular activation may be executed while activation settings define specific attributes of the AI/ML capability activation scope (e.g., object or object type, network context, activation time window) for which AI/ML capability should be activated.

### 5.10.3	Potential requirements

REQ-MLTRAIN_CFG-1: The AI/ML MnS producer should have a capability to allow the authorized consumer to trigger AI/ML training configuration.

REQ-MLTRAIN_CFG--2: The AI/ML MnS producer responsible for AI/ML management shall have a capability to configure the AI/ML training capability.

REQ-AIML_INF_CFG- -1: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized consumer to configure the inference function.

REQ-AIML_INF_CFG -2: The MnS producer responsible for AI/ML inference management should have a capability to configure inference function and inform an authorized MnS consumer about the configurations of the AI/ML inference function.

REQ-AIML_ INF_ACT-1: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized MnS consumer to activate an AI/ML inference function.

REQ-AIML_ INF_ACT-2: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized MnS consumer to deactivate an AI/ML AI/ML inference function.

REQ-AIML_ INF_ACT-3: The MnS producer responsible for AI/ML inference management should have a capability to inform an authorized MnS consumer about the activation and deactivation of an AI/ML inference function.

REQ-AIML_ INF_ACT-4: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized MnS consumer to partially or progressively activate/deactivate the AI/ML inference capabilities for an inference function.

REQ-AI/ML_ENTITY_ACT-1: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized MnS consumer to activate an ML entity.

REQ-AI/ML_ENTITY_ACT-2: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized MnS consumer to deactivate an ML entity.

REQ-AI/ML_ENTITY_ACT-3: The MnS producer responsible for AI/ML inference management should have a capability to inform an authorized MnS consumer about the activation and deactivation of an ML entity.

REQ-AIML_ENTITY_ACT-4: The MnS producer responsible for AI/ML inference management should have a capability to allow an authorized MnS consumer to partially or progressively activate/deactivate the AI/ML inference capabilities for an ML entity. REQ-AIML_ENTITY_ACT-5: The 3GPP management system shall have a capability to allow an authorized MnS consumer to define the policies for activation of AI/ML capabilities in order to instruct the AI/ML MnS producer on how to perform the AI/ML capability activation (e.g., when and where to activate which AI/ML capabilities).

REQ-AIML_ENTITY_ACT-6: the 3GPP management system shall have a capability to allow a producer to activate the AI/ML capabilities based on the policies specified by the AI/ML MnS consumer.

### 5.10.4	Possible solutions

#### 5.10.4.1	ML training configuration

TBD

#### 5.10.4.2	AI/ML inference function configuration

TBD

#### 5.10.4.3	AI/ML activation

##### 5.10.4.3.1	General framework for activation and deactivation

This subclause describes the general framework for activation and deactivation of AI/ML inference capabilities, ML entities and inference function in inference phase.

A data type or abstract class describing the activation properties, and this data type or abstract class can be used or inherited by the MOI representing the inference function and ML entity.

This general framework supports the general properties for all types of activation/deactivation including:

-	Instant activation and deactivation: The AI/ML inference capabilities to be instantly activated/deactivated on the ML entity or inference function.

-	Policy-based activation and deactivation: The AI/ML inference capabilities to be activated/deactivated on the ML entity or inference function based on a given policy.

-	Schedule-based activation and deactivation: The AI/ML inference capabilities to be activated/deactivated on the ML entity or inference function based on a given schedule.

-	Gradual activation and deactivation: The AI/ML inference capabilities to be activated/deactivated on the ML entity or inference function based on a given scope.

And this data type or abstract class is extended with the attributes supporting these specific types of activation.

##### 5.10.4.3.2	Instant activation and deactivation

The generic framework described in clause 5.10.4.3.1 is extended with the following attributes to support instant activation and deactivation:

- 	The AI/ML inference capabilities to be instantly activated/deactivated on the ML entity or inference function.

##### 5.10.4.3.3	Policy based activation

The generic framework described in clause 5.10.4.3.1 is extended with the following attributes to support the policy-based activation and deactivation:

- 	AI/ML inference capabilities to be activated/deactivation on the ML entity or inference function based on the given policy.

-	The policy (e.g., condition) for activation/deactivation.

##### 5.10.4.3.4	Schedule based activation

The generic framework described in clause 5.10.4.3.1 is extended with the following attributes to support the schedule-based activation and deactivation:

- 	AI/ML inference capabilities to be activated/deactivation on the ML entity or inference function based on the given schedule.

### -	The schedule for activation/deactivation.5.10.5	Evaluation

The solution described in clause 5.10.4 uses the NRMs of the inference function and ML entity for AI/ML inference configuration. This approach supports both MnS consumer-initiated and MnS producer-initiated configuration based on the existing provisioning MnS operations and notifications.

Therefore, the solution described in clause 5.10.4 is a feasible solution.

## 5.11	Orchestrating AI/ML Inference

### 5.11.1	Description

A network automation system may involve or apply multiple  AI/ML inference functions and/or ML entities each of which only has a limited view of the network scope. For their effective operation, it may be necessary to apply orchestration mechanisms (be it centralized or otherwise) to orchestrate both the operation of the AI/ML inference functions as well as the execution of the actions recommended by the AI/ML inference  functions.

Note: 	The AI/ML inference function is of any function that employs the capabilities of a trained mathematical entity (the ML model) or Decision Matrix to make inferences for a specific use case. Such a function may for example optimize load distribution among cells, detect anomalies from data or evaluate the likelihood interference among a set of cells.

### 5.11.2	Use cases

#### 5.11.2.1	Knowledge sharing on executed actions

The actions and effects of employing and applying AI/ML inference cannot be known beforehand since they are based on the learnings of the AI/ML entities. An AI/ML inference function may be to optimize one set of parameters but its actions may impact another function. In that case mechanisms are needed to counteract conflicts and or minimize potential negative impacts resulting from  conflicting actions brought up by  applying AI/ML inference.

When an ML entity A executes an action on the network, that action may affect other network functions. Most critical is that those actions may affect the learning environment (i.e., the training data) of another ML entity, say ML entity B. Correspondingly, the ML entity B needs to be informed when such actions are taken by any ML entity A.

#### 5.11.2.2	Knowledge sharing on impacts of executed actions

AI/ML inference functions are able to adjust to adjust their behavior depending on context and on all the information they receive. When an ML entity in function A executes an action on the network that affect other network functions, the ML entity in function A may be able to adjust its behavior to minimize its impact on the other network functions if such an ML entity is informed of its impact on the other network functions. To account for such impacts, the network functions that are affected or the 3GPP management system, needs to inform the ML entity in function A of the observed impacts of the action of the ML entity in function A on the other network functions.

In otherwards, it is necessary that when an action is taken by ML entity in function A, after an appropriate interval (specific to either A or B as may be needed), the network function B (and the other network functions that notice impacts on their metrics or input data) should report their metrics to A. Correspondingly, ML entity in function A may aggregate the reported observations with its own metrics to evaluate the global effect of its actions. In doing so, ML entity in function A is able to learn the best actions that concurrently optimize it’s (A’s) objective(s) and also minimize the effects on the peers.

The report from B to all may contain values on known KPIs and metrics, e.g., those standardized in TS 28.552 [8] and TS 28.554 [14].

Figure 5.11.2.2-1: Distributed coordination of Cognitive Network Automation Functions (NAF)

#### 5.11.2.3	Abstract information on impacts of executed actions

In a multivendor environment, the KPIs semantics differ and KPIs that measure one event may be named and computed differently by two vendors.  e.g., the Handover rate (H) could be Handovers per user per unit time or Handovers per cell per unit time. Consequently, there is no guarantee that the exchanged KPI or metric values as described in clause 5.C.2.2. will be interpretable by the ML entity A when it receives that metric.

Instead, it is better when the ML entity B expresses its level of dissatisfaction or impact of the action that was taken by ML entity A. The level of dissatisfaction or impact may be expressed in terms of an Action Quality Indicator (AQI) that is a generic measure that uses a fixed scale to quantify the effect of one function on another. This is similar to the way the Composite Available Capacity (CAC) was specified for cell load to communicate used vs. available cell capacity among cells from different vendors and with different total resources.

For the AQI, if ML entity A takes an action, its effects on the peers will range from an extremely negative impact, e.g., like Mobility Load balancing causing too many mobility related Radio Link Failures; through mild effects that are insignificant (like MLB causing a few handover ping pongs) and to very positive effects (like MRO unexpectedly removing overload in a cell). Consequently, a simple linear measure can easily be used to capture these effects.

Figure 5.11.2.3-1: Multi-vendor coordination of AI/ML inference network automation functions

#### 5.11.2.4	Triggering execution of AI/ML inference functions or ML entities

A network automation system may involve or apply multiple AI/ML inference functions or ML entities. These ML entities may not conflict with one another but may focus on only a subset of the problems and may propose changes that are suboptimal since each focus on only a subset of the network control parameters. It may happened frequently that the individual ML entities do not know the expected end-to-end performance of the network, i.e., the ML entities need to be explicitly called to act by an entity which has a wider view of the network problems and the capabilities of the ML entities. For example, consider the distributed AI/ML inference functions (i.e., those instantiated within the gNB) with effects across multiple managed objects such as interference management which may impact multiple cells. Such AI/ML inference functions may not be able to have a wider view of the network state. As such, a centralized controller (i.e., a controller that with a wider and common view to the set of managed objects) is needed to control and coordinate both centralized and distributed AI/ML inference functions. Specifically, the controller may (based on received network data and analytics insight):

-	diagnose network problem(s) to identify the nature of the problem, and

-	receive the capabilities of the available AI/ML inference functions either directly from the AI/ML inference functions or from a Capability Library that acts as a registry to which the capability of each NAF is added each time a new NAF introduced into the system, and

-	evaluate the capabilities of the AI/ML inference functions to identify the best (set and sequence of) AI/ML inference functions to address the identified problem(s), and

-	trigger the ML entities to act, providing at trigger time any required extra generalized or specific information.

#### 5.11.2.5	Orchestrating decisions of AI/ML inference functions or ML entities

Given the multiple ML entities which may differ in terms of source vendors and behavior characteristics, the operator may not find it appropriate to grant access to the network to all the different ML entities (both for security and operability reasons).

In that case, there is a need for an orchestration functionality that takes responsibility for the end-to-end performance of the Autonomous Network and that supervises the ML entities to guarantee the end-to-end performance. The orchestration functionality receives the recommendation changes from the ML entities, evaluates the proposed changes and their likely effects, decides the changes that should be executed on the network (e.g. to minimize concurrent changes on the same network resources) and informs the ML entities of the respective feedback related to their recommended actions. The orchestration function may also (re)configure the ai based on the observed effects of the actions of the ML entities (e.g. to redefine the control parameter space of the individual ML entities). In either cases, the orchestration function may rely on network states analytics functions which may provide insights that characterize the state of the network into specific states. Such insights may for example characterize whether the network is experiencing low traffic states or anomaly states.

Figure 5.11.2.5-1: Orchestrating AI/ML

### 5.11.3	Potential requirements

REQ-ML_ORCH-1: The AI/ML inference MnS producer should have a capability to inform an authorized consumer (e.g., another AI/ML inference function) of actions undertaken by the producer of AI/ML inference.

REQ-ML_ORCH-2: The AI/ML inference MnS producer should support the capability to request a producer of AI/ML-Action-evaluation (e.g., another AI/ML inference function) to evaluate one or more actions undertaken by the producer of AI/ML inference.

REQ-ML_ORCH-3:  The AI/ML inference MnS producer should support the capability to specify to the producer of AI/ML-Action-evaluation (e.g., another AI/ML inference function) requested to evaluate one or more actions undertaken by the producer of AI/ML inference the timing within which the consumer should report the observed effects of that evaluated actions.

REQ-ML_ORCH-4: The AI/ML inference MnS producer should support the capability to report the metrics of another AI/ML inference MnS producer that are affected by the one or more actions undertaken by a specific AI/ML- inference producer.

REQ-ML_ORCH-5: The AI/ML inference MnS producer should support the capability to report an Action Quality Indicator as the abstraction of the impacts of the one or more actions undertaken by a specific first AI/ML inference producer on a specific metric of the first AI/ML inference producer.

REQ-ML_ORCH-6: The 3GPP Management system should have a capability for an authorized consumer to configure a producer of AI/ML Orchestration to monitor recommendations of multiple AI/ML inference functions and decide on the appropriate recommendation to activate on the network.

### 5.11.4	Possible solutions

A single solution may be provided to support the different requirements in clause 5.11.3 as follows:

Information elements:

- 	Introduce an <<IOC>> for centralized Orchestration of AI/ML inference functions. The <<IOC>> which may be named AIMLOrchestration, would function as the centralized Automation Controller that takes responsibility for the end-to-end performance of the complete set of network functions that apply AI/ML capabilities.

- 	Introduce an <<IOC>> for a function that contains AI/ML to be used for network automation. This may be called an AI/ML inference function or network automation function since it is likely to have similar features with or without AI/ML. This <<IOC>> may be named as a NetworkAutomationFunction <<IOC>>.

- 	Introduce an <<IOC>> for a Network Automation Capability Library as an attribute of the AIMLOrchestration. The <<IOC>> which may be named a CapabilityLibrary, stores the capabilities of the AI/ML inference functions or ML entities that the AIMLOrchestration shall need to orchestrate. The AIMLOrchestration uses a Network Automation Capability Library as a database in which it registers the capabilities of the different network automation functions available, i.e., when an AI/ML inference function or ML entity is added to the system, its capabilities or the problems it can solve as well as the KPIs it optimizes are registered with the CapabilityLibrary The AIMLOrchestration may populate the CapabilityLibrary by querying the individual AI/ML inference  functions or ML entities for their capabilities.

- 	Introduce a <<dataType>> for the network performance Targets.  Through the network performance Targets, the AIMLOrchestration receives the technical objectives that are expected to be achieved. These are set either by the human operator or by a network automation function (as shown in Figure 5.11.2.2-1) responsible for deriving concrete objectives from the operators desired goals. Such an objectives-setting function is here referred to as the Network Objectives Manager. The AIMLOrchestration monitors the NAFs to ensure that all are contributing towards achieving the objectives and not towards impeding objective achievement.

- 	Introduce a <<datatype>> on the AIMLOrchestration for the Network state. The datatype which may be called the NetworkState indicates, and labels specific unique states of the network as derived from specific combinations of raw network data. The state may be derived from an external entity that shares that state with all interested entities or it may be derived by the AIMLOrchestration. The Network state aids the AIMLOrchestration not only to relate states observed in different time periods but to also reference states in a way that is understandable to other entities, e.g., while communicating to the AI/ML inference functions or ML entities.

- 	Introduce a <<datatype>> on the AIMLOrchestration for a recommended action from a network automation function. The datatype which may be called the recommendedAction<<datatype>> captures the recommended policy and configuration change of a specific network automation function, e.g., an AI/ML inference function or ML entity towards the AIMLOrchestration. - All recommendations for policy changes as computed by the network automation functions for their respective objectives are communicated via this recommendedAction. Such a recommendedAction may be a hash function of parameter to parameter-value annotated with an indication of the time within which the change should be activated or otherwise discarded.

- 	The recommendedAction <<datatype>> may also include a field for the eventual action that is selected by the AIMLOrchestration, say called the selectedAction. This is written by the AIMLOrchestration with the specific values that have been applied by the AIMLOrchestration following which a notification may be sent to the network automation function that generated the recommendedAction. The selectedAction may also be used by the AIMLOrchestration to inform the network automation functions if the recommended policy change has been activated or not and possibly the reason thereof. The respective message sent to a network automation function may for example be verbalized as follows:

-	“In network state A1, when you changed policy X from configuration X1 to X2, the observed effect on the network KPI vector v exceeded a predefined threshold. Consequently, policy configuration X2 is now barred from your applicable control and operational parameter spaces.”

NOTE 1: 	the AIMLOrchestration is also tasked with reconfigurations of the control and operational parameter spaces of the network automation functions to adjust the limits within which the network automation functions may operate. For example, for a load balancing function, the AIMLOrchestration may adjust the limits to which the load balancing function may adjust the cell individual offset (CIO) by setting the maximum or minimum CIO or steps within which the CIO may be changed. For these reconfigurations, the AIMLOrchestration may use existing NRMs for the network automation functions, e.g., the MLEntity NRM, to change the attributes of the network automation functions. For example, the AIMLOrchestration may mask a part of the control and operational parameter spaces such that those masked values become inaccessible for the network automation function. It may also use the existing NRMs to activate or deactivate particular network automation functions as may be necessary (e.g., based on network context).

NOTE 2: 	The AIMLOrchestration may be the only responsible entity for activating the selected inference decisions onto the network. For this, the AIMLOrchestration may activate the successful recommended policies and/or configurations on the network via the existing NRMs for the network objects or existing CM capabilities.

- 	Introduce an IOC on the network automation function, e.g., on the AI/ML inference function or on the ML entity for a request for monitoring. The IOC which may called metricMonitoringRequest, may be used by the AIMLOrchestration or by any network automation function A to request another network automation function B to start a monitoring of the metrics of the of network automation function B and subsequently report the outcomes of the monitoring.

- 	Introduce a datatype on the AIMLOrchestration for an indication of the observed effect of a given action on the metrics of given network automation function. The IOC which may be called the Action Quality Indicator, provides information to the source network automation function (i.e., the function that generated the action) about how good or bad that action was to the metrics of the reporting function.

Usage of the information Elements:

1) 	To Identify and trigger Automation capabilities, e.g., AI/ML inference functions or ML entities:

- 	The AIMLOrchestration or a multi-functional analytics service of the AIMLOrchestration evaluates the network state to diagnose what the network or network resource problem might be. In general, such a problem cannot be concluded from a single KPI, otherwise, the NAF responsible for that KPI should be triggered by default. Instead, it is typically a rare event which can only be determined from multiple KPIs. For identifying the problem, the AIMLOrchestration may collect data on KPIs. Counters, CM values etc. The combination of KPIs. Counters, CM values, etc may be correlated e.g., using an analytics service to identify the problem and the specific combination may be labelled as a specific network state. Note: the problem may also be pointed to using analytics service such as MDAS.

- 	For the identified problem, the AIMLOrchestration finds the most appropriate network automation function to trigger. It could also be the case that there are multiple network automation functions responsible for a given KPI, which could say happen if there is an open network automation platform to which multiple vendors have supplied network automation functions. In such a case, the network automation function to be triggered by default is not obvious and either the AIMLOrchestration or an analytics function must figure out the best network automation function to trigger. The AIMLOrchestration queries the Network Automation Capability Library to match the identified problem to one of the sets of network automation functions that are registered in the library.

- 	The AIMLOrchestration then triggers the identified network automation function to find an appropriate action for the problem. The trigger may be sent via a ProblemResolutionRequest sent by the AIMLOrchestration to the network automation function. The ProblemResolutionRequest may include an identifier for the managed object related to the problem as well as the KPIs. Counters, CM values related to the observed problem.

- 	The selected network automation function submits a proposed recommended action to the AIMLOrchestration for execution. The AIML orchestrator may also undertake coordination action (as described next) to ensure the action is not  opposite to the interests of other network automation functions.

- 	At the end of the cycle, the AIMLOrchestration determines the next action, either to recall the previous network automation function to find a new configuration, or to call a different network automation function to attempt the same or related problem or to move to start a new cycle for a completely different problem if the previous problem has been successfully solved.

@startuml Procedure 2. Requesting and Instantiating an MLInferenceEmulationJobs

skinparam Shadowing false

skinparam Monochrome true

!pragma teoz true

'Autonumber

skinparam maxMessageSize 130

participant "Active, unapproved \nNAF e.g., A" as NAFA

participant "Active, approved \n NAF e.g., B" as NAFB

participant "AIML \nOrchestration" as Orch

collections "Peer \n NAFs e.g., C, D,.. " as NAFC

Orch -> Orch: 1. Identify network problem

Orch -> NAFA: 2.

& Orch -> NAFC: 2. Request Capabilities

NAFA -> Orch: 3.

& NAFB -> Orch: 3. providecapabilites.

Orch -> Orch: 4. Select appropriate NAF

Orch -> NAFB: 5. Trigger NAFto find best action.

Note over NAFB, NAFC: 6. May need to coordinate effect of the action

@enduml

Figure 5.11.4-1: Identifying and triggering Automation capabilities among multiple network automation functions.

2) 	To Orchestrate the decisions of AI/ML network automation functions, e.g., AI/ML inference functions or ML entities ( see Figure 5.11.4-2):

- 	Each network automation function generates a recommendation which it proposes to the AIMLOrchestration for implementation. The AIMLOrchestration takes recommendations for changes from the network automation functions and takes a decision whether to implement the policy changes or not. The policy changes may be stated by the network automation functions as hash functions of parameter to parameter-values annotated with the time within which the values should be activated or else be discarded.

- 	The AIMLOrchestration undertake control tasks for the recommended and approved configuration changes, e.g., concurrency control, to ensure that their action will not conflict with other ongoing or proposed actions. In case of conflicts the AIMLOrchestration may choose to schedule the action to a different time from when it is proposed.

- 	At the right time, the AIMLOrchestration implements the action onto the network and subsequently manages the coordination.

@startuml Procedure 2. Requesting and Instantiating an MLInferenceEmulationJobs

skinparam Shadowing false

skinparam Monochrome true

!pragma teoz true

'Autonumber

skinparam maxMessageSize 130

participant "Active, unapproved \nNAF e.g., A" as NAFA

participant "Active, approved \n NAF e.g., B" as NAFB

participant "AIML \nOrchestrator" as Orch

collections "Peer \n NAFs e.g., C, D,.. " as NAFC

NAFA -> NAFA: 1. Compute configuration, e.g. select action a

& NAFB -> NAFB: 1. Compute configuration, e.g. select action a

NAFA -> Orch: 2.

& NAFB -> Orch: 2. Request action a, PM interval t sec.

Orch -> Orch: 3. Select action to execute

Orch -> NAFA: 4.

& Orch -> NAFB: 4. Notify NAFs of selected action.

Orch -> Orch: 5. Execute selected action

@enduml

Figure 5.11.4-2: Orchestrating the decision among multiple network automation functions.

3) 	To share knowledge and coordinate the impacts executed decisions of AI/ML network automation functions, e.g., AI/ML inference functions or ML entities ( see Figure 5.11.4-3):

- 	The AIMLOrchestration is a meta-learning agent responsible for learning if any of the availed network automation functions is behaving outside its expected region and for taking the accordingly appropriate counter-measures. So, for the activated configuration changes, the AIMLOrchestration manages the transaction among network automation functions that are intended to coordinate their actions and executions.

- 	Following the execution of changes, the AIMLOrchestration triggers the other network automation functions (besides the one requesting the change) to start a monitoring period to identify any negative effects on their metrics.

- 	At the end of the observation period the network automation functions evaluate the effects of the changes and report to the AIMLOrchestration their network (metric) status observations in form of Action Quality Indicators. The network automation functions consume performance assurance (PM), fault supervision (FM) and provisioning (CM) services on their respective network elements and domains to evaluate the effect of the executed policy changes or configurations.

- 	The network automation functions report the observed effects in terms of the Action Quality Indicators to the AIMLOrchestration for aggregation. Note that, in a distributed implementation of the coordination, the Action Quality Indicators may also be reported directly to the network automation function which generated the action that was executed.

- 	In the subsequent AQI handling, the AIMLOrchestration evaluates the network status inputs from the multiple network automation functions and network domains to learn the effects of the configuration changes, i.e., the AIMLOrchestration determines if the effects are acceptable or not.

- 	Where a given change is determined to be out of the expected range, such a change needs to be labelled accordingly. For example, the change may be barred from ever being re-applied or from being reused in the specific context.

- 	The AIMLOrchestration informs the respective network automation function of the evaluation outcome (e.g., by sending the aggregate AQI). The AIMLOrchestration also accordingly re-configures the network automation functions, when necessary, e.g., by changing the network automation function’s applicable control parameter spaces or its performance targets.

@startuml Procedure 2. Requesting and Instantiating an MLInferenceEmulationJobs

skinparam Shadowing false

skinparam Monochrome true

!pragma teoz true

'Autonumber

skinparam maxMessageSize 130

participant "Active, unapproved \nNAF e.g., A" as NAFA

participant "Active, approved \n NAF e.g., B" as NAFB

participant "AIML \nOrchestrator" as Orch

collections "Peer \n NAFs e.g., C, D,.. " as NAFC

Note over NAFB, Orch: Action from B executed

Orch -> NAFA: 6.

& Orch -> NAFC: 6. Notify execution, trigger PM for interval t sec.

NAFA -> NAFA: 7. Collect PM for t sec

& NAFC -> NAFC: 7. Collect PM for t sec

NAFA -> NAFA: 8. compute & interpret KPIs to compute AQI

& NAFC -> NAFC: 8. compute & interpret KPIs to compute AQI

Alt distributed coordination

NAFA -> NAFB: 9.

& NAFC -> NAFB: 9. Report AQI

NAFB -> NAFB: 10. AQI handling

NAFB -> NAFB: 10. Revise NAF-B configuration if needed

Else centralized coordination

NAFA -> Orch: 11.

& NAFC -> Orch: 11. Report AQI

Orch -> Orch: 12. AQI handling

Orch -> NAFB: 13. Report aggregate AQI

Orch -> Orch: 14. Compute NAF configuration decision

Orch -> NAFB: 15. Configure NAF(s)

end

@enduml

Figure 5.11.4-3: The control and Coordination transaction of network automation functions requests and actions.

### 5.11.5	Evaluation

The solution described in clause 5.11.4 introduces new information elements that together solve all three aspects for the orchestration of AI/ML inference, i.e., the identification of the right AI/ML inference function or ML entity to solve a given problem, the orchestration of the actions of the AI/ML inference function or ML entities and the sharing of knowledge among the different AI/ML inference functions or ML entities about the performance of the individual AI/ML inference functions or ML entities.

Therefore, the solution described in clause 5.11.4 is a feasible solution for Orchestrating AI/ML inference.

## 5.12	Coordination between the AI/ML capabilities

### 5.12.1	Description

For AI/ML in 5GC or RAN, the AI/ML capabilities in 5GC or RAN may be needed to coordinate and possibly other aspects in order to improve the overall performance.

Typically, due to the collected data used for model training/inference for 5GC or RAN, performance of a model in 5GC or RAN may be biased in some respects. On the other hand, the 3GPP management system collects data of from a wide scope of RAN nodes/5GC covering e.g., wider geographical network and radio coverage areas, which as a consequence implies that the prediction will be unbiased towards the overall RAN nodes/5GC. However, 3GPP management system prediction may lack insight of patterns related to specific node or finer granularity of time. Hence with coordination or alignment of the AI/ML capability between 5GC/RAN and 3GPP management system, the overall performance may be optimised.

To enable the coordination between the AI/ML capabilities, the configuration (e.g., a triggering condition, i.e., when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting in order to capture the deviation of predictions (and / or the related context) so the consumer can gain a better understanding regarding the coordination of AI/ML capabilities.

### 5.12.2	Use cases

#### 5.12.2.1	Alignment of the AI/ML capability between 5GC/RAN and 3GPP management system

Generally, the typical data from 5GS data is measurements (PM, KPI), which may be modeled as “time series data” and the analytics of “time series data” is normally to learn the seasonality, trend, etc., patterns. There is different type of seasonality patterns, e.g., daily, weekly, monthly, seasonally, annually, etc., patterns. A RAN node collects finer granularity data for short duration. The finer seasonality pattern will be well captured in a timely manner, while the 3GPP management system with longer range of data (which is more aggregated) will more accurately capture the higher level of seasonality patterns. Hence, combing the analytics results from RAN, 5G core and 3GPP management system or between RAN and 5G core may improve the accuracy for overall predictions.

On another matter, the data collected from one RAN or 5G core node would tend to be biased for that specific RAN node or NF, which implies that the prediction with the data learned and inferred will tend to be biased for that specific node. On the other hand, the 3GPP management system collects data of longer range and from a large amount of different RAN nodes or NFs, which as a consequence implies that the prediction will be unbiased towards the overall RAN nodes or 5G core area. Hence combining the results from both the RAN or 5G core and 3GPP management system, or between NWDAF and RAN may also improve the overall predictions accuracy (and mitigate the bias).

### 5.12.3	Potential requirements

REQ-AIML_COORD-01:	3GPP management system shall have the capability to allow an authorized consumer to configure an AI/ML capability regarding the correlation of predictions between MDAS and RAN function, or MDAS and NWDAF.

REQ-AIML_COORD-02:	3GPP management system shall have the capability to report the result of correlation of predictions between MDAS and RAN function, or MDAS and NWDAF.

### 5.12.4	Possible solutions

TBD.

### 5.12.5	Evaluation

TBD.

## 5.13	ML entity re-training

### 5.13.1	Description

TBD.

### 5.13.2	Use cases

#### 5.13.2.1	Producer-initiated threshold-based ML Retraining

The performance of the ML entity depends on the commonality of the distribution of the data used for training and the distribution of the data used for inference. Typically, the model performance would be good soon after deployment. This is because the chances of the distributions of the data used for training and the samples picked for inference are same. As the time progresses, the distribution of the network data might change as compared to the distribution of the train data. In such scenario, the performance of the ML entity degrades over time. Hence there is a need for monitoring the network counters and thresholds such as PMs, KPIs, alarms etc., and use this information in the producer to decide on the retraining.

#### 5.13.2.2	Efficient ML entity re-training

During inference phase of ML entity, a lot of potentially new data samples are processed and some of them are useful for a re-training and should therefore be labelled and added to the training set. However, using all inference data samples for retraining generates a high effort for data labelling, data provision (signalling) and model training, and this effort is not feasible in environments with limited resources.

In the case that re-training/model adaptation is performed at the entity with low processing power or limited energy the amount of data used for re-training and the time needed for model to converge towards the maximum performance is critical and needs to be minimized.

In order to optimize the re-training, it is necessary to reduce of the number of training samples, by extracting the most supporting data samples for re-training from all data samples (pool samples) that have been used for inference.

### 5.13.3	Potential requirements

REQ-AIML_RETRAIN -1: The AI/ML MnS producer should have the capability of allowing an authorized AI/ML MnS consumer to provide the counters and thresholds to be monitored to trigger the retraining of an ML entity.

REQ-AIML_RETRAIN -2: The AI/ML MnS producer should have the capability of allowing an authorized AI/ML MnS consumer to update the ML entity with the data counters and thresholds to be monitored to trigger the retraining of that ML entity.

REQ-AIML_RETRAIN -3: The 3GPP management system should have a capability for the he authorized MnS consumer to request and receive from the AI/ML inference producer the most supporting data samples for re-training from all data samples (pool samples) that have been used for AI/ML inference.

REQ-AIML_RETRAIN -4: The 3GPP management system should have a capability for the AI/ML MnS inference producer to provide to the ML training MnS producer the most supporting data samples for re-training.

### 5.13.4	Possible solutions

#### 5.13.4.1	Producer Initiated Retraining

Following is the proposed solution based on information model defined in TS28.105 [4].

-	Extend the existing MLTrainingRequest IOC with an optional <<datatype>> attribute on monitored data events. The attribute may be called "monitoredDataEvents" and is a list of monitored data events each of which may be of <<datatype>>  "monitoredDataEvent" that contains the following information:

-	An attribute called "ThresholdInfoList" as a list of threshold information with each entry a "ThresholdInfo" <<datatype as defined in TS28.622 [13]. The ThresholdInfo is an array containing: 1) the performance metrices to be measured and monitored by the AIML producer, 2) the threshold value, 3) threshold directions indicating the direction for which a threshold crossing triggers a threshold and 4) the threshold hysteresis indicating hysteresis of a threshold, if configured, the PM is not compared only against the threshold value but also considering the hysteresis value.

-	An attribute called "MonitoredkPIList" as a list of KPIs t be monitored for the particular data event. Each entry of the "MonitoredkPIList" is a "kPIName" indicating the name of the KPI as defined in TS 28.554 [14] to be monitored for this ML training.

-	Existing MLEntity <<datatype>> is extended with the same information mentioned above. This is needed to ensure an MnS consumer can configure the MLEntity and by doing so trigger the ML retraining. ML training producer may monitor the information available at MLEntity <<datatype>> and when any of the thresholds is crossed, retraining may be performed by the ML training producer. The threshold crossing may be identified via direct monitoring of the ML entity by the retraining producer e.g. via data monitoring IOC or via a notification to the retraining producer.

#### 5.13.4.2	Efficient AI/ML entity re-training

This solution uses the instances of following IOCs for interaction between ML inference MnS producer and MnS consumer (e.g. the ML training function) to support efficient re-training of ML entity:

- 	MLDataSamplesRequest - this IOC represents the request for obtaining the data samples that are likely to have more value for re-training from all data samples that have been used for inference. This IOC allows an MOI to be created on the ML inference MnS Producer and may contain the following attributes:

-	definition of one or more data features

-	minimum number of data samples to be obtained

-	criteria for obtaining the most supporting data samples

All data samples that have been used for inference are filtered in accordance with the one or more requested features and other provided  criteria in order to obtain the most supporting data samples for re-training.

- 	MLDataSamplesResponse - this IOC represents the response  indicating the data obtained according to the MLDataSamplesRequest. This IOC is created by the AI/ML MnS inference producer towards the MnS consumer and includes at least the requested minimum number of data samples or pointers to them that satisfy criteria specified in MLDataSamplesRequest. The response may further include additional information quantifying the supportiveness for each data sample.

### 5.13.5	Evaluation

The solution described in clause 5.13.4.1 adopts the NRM-based approach, which reuses the existing provisioning MnS operations and notifications. This solution is also consistent with the approach used by ML training MnS defined in TS 28.105 [4]. Moreover, it also enables the MnS consumer to configure existing management data relevant for ML training like PMs and KPIs. Therefore, the solution described in clause 5.13.4.1 is a feasible solution.

The solution described in 5.13.4.2 is consistent with NRM- based approach and reuses existing provisioning MnS operations. The solution is also consistent with the approach used by ML training MnS described in TS 28.105 [4]. It  provides the means for obtaining the data samples that are likely to have more value for re-training using the consistent NRM-based approach.

## 5.14	ML entity joint training

### 5.14.1	Description

TBD

### 5.14.2	Use cases

#### 5.14.2.1	Support for ML entity modularity – joint training of ML entities

Besides the discovery of the capabilities of ML entities, services are needed for identifying which AI/ML capabilities shall be used in specific use case and how. The TS 28.105 [4] defines the inferenceType which indicates the type of inference, i.e., the use case that the ML model supports. This indicator may be represented by the MDA type (see TS 28.104 [2]), Analytics ID(s) of NWDAF (see TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions.

In order to address complex use cases, applying multiple, cooperative ML entities might be necessary. There are different ways in which the ML entities may cooperate. An example is the case where the output of one ML entity can be used as input to another ML entity forming a sequence of interlinked ML entities. Another example is the case where multiple ML entities provide the output in parallel (either the same output type where outputs may be merged (e.g., using weights), or their outputs are needed in parallel as input in the automation process or as input to another ML entity. Such modular approach in building a single AI/ML inference function for a given use case facilitates the reusability of ML entities in different use cases. Furthermore, it facilitates the replacement, changes and improvements of individual ML entities ties within complex use cases.

Based on the use case complexity, a single or multiple ML entities may be needed in order to provide a complete solution for a given use case. For simple use cases, it may be sufficient to apply only a single ML entity. For complex use cases or vendor specific extensions, multiple ML entities need to be employed in potentially cooperative way, e.g., ML entities employed in sequence, parallel or any structure thereof. Given the complexity of the required mapping between the use cases and potentially multiple ML entities, services should be supported to facilitate this mapping, e.g., determination if the specific use case can be realized by a single ML entity or a group of ML entities, configuration of individual ML entities based on their interdependencies in the group, enabling joint training of interdependent ML entities, etc.

### 5.14.3	Potential requirements

REQ-ML_MOD-1: The 3GPP Management system should have a capability for an authorized MnS consumer to request the training of a group of ML entities working together to address specific use case.

REQ-ML_MOD-2: 3GPP management system shall have the capability to enable an authorized MnS consumer to manage and configure multiple training processes, e.g., to start, suspend or restart the training; or to adjust the training conditions and/or characteristics based on the ML entity group.

REQ-ML_MOD-3: The MLT MnS producer shall have a capability to provide the ML entities group training result (including the location of the trained ML entities in the group) to the authorized MnS consumer.

### 5.14.4	Possible solutions

#### 5.14.4.1	Support for AI/ML Entity modularity - joint training of ML entities

The IOC MLTrainingRequest represents the ML model training request that is created by the ML training MnS consumer. In order to support joint training of a group of ML entities this IOC needs to capture the information on the ML entities group and the relation among the ML entities, i.e. MLEntityGroupProfile datatype. Such data type may contain following attributes:

-	MLEntityGroup ID- Unique identity value identifies the ML entity group instance.

-	JointTrainingIndicator - which indicates if the ML entity group instance needs to be trained (perform joint training of ML entities in the group)

-	Levels - An integer range (1, n) that indicate the ML entity group has n levels, where an integer in the range indicates a specific level, starting from 1. Each level would consist of an ML entity. The output data of one level are used as the input data of the next level.

-	Parallels - a sub integer range (1, m) may be added to indicate the series or the parallel arrangement of ML entities inside a given level, starting from 1.  The output data of parallel ML entities inside a given level are used as the input data of the next level.

-	expectedRunTimeContext - This may include information related to specific extraction, transformation, and load of data as input to a specific ML entity inside a given level and parallel.

-	MLEntitiyID inside a given level and parallel.

If multiple ML models need to be trained jointly (in relation which each other) the MnS producer needs to start the training based on the information obtained in the MLEntityGroupProfile. The ML training MnS producer instantiates multiple MLTrainingProcess MOI(s) that are responsible to perform the following:

-	collects data for training, taking into account the inter-relation among ML entities in the MLEntityGroupProfile, e.g., if output of first ML entity is used as input to the second ML entity, the data for training of the second model needs to be collected accordingly,

-	prepares the training data for each ML entity in the group, based on the information in MLEntityGroupProfile. I.e., based on expectedRunTimeContext contained in MLEntityGroupProfile the specific extraction, transformation, and load of data as input to a specific ML entity inside a given level and parallel needs to be performed.

-	trains the ML entities based on the JointTrainingIndicator.

### 5.14.5	Evaluation

The solution described in clause 5.14.5.1 is consistent with the MLTrainingRequest IOC and enhances it in order to support joint training of a group of ML entities. The new attributes added to the MLTrainingRequest IOC are to configure the joint training of a group of ML entities. It is a fully NRM-based approach, and reuses the existing provisioning MnS operations. It  provides the means to facilitate both capturing the information on the group of ML entities working together to address specific use case, as well as enabling the configuration of joint training of such group of ML entities using the consistent NRM-based approach.

## 5.15	ML Inference emulation

### 5.15.1	Description

A trained ML entity can be used for inference within the stated scope e.g., on a managed function or in a management function. Accordingly, there may be an AI/ML inference MnS producer that is responsible for executing the inference.

### 5.15.2	Use cases

#### 5.15.2.1	AI/ML Inference emulation

After an ML entity is trained, validation is done to ensure the training process is completed successfully. Typically, validation is done by preserving part of the training data set and using it after training to check whether the ML entity has been trained correctly or not. However, even after the ML entity is validated during development, inference emulation is necessary to check if the ML entity containing the ML entity is working correctly under certain runtime context or using certain inference emulation data set. In principle, the two operations are similar on a functional level, where both of them check the ML performance against given context or data to ensure the ML functionality is functioning correctly. But inference emulation involves interaction with third parties, e.g., the operators who shall use the ML entity or third-party systems that may rely on the results computed by the ML entity. For these reasons, it is necessary to support inference emulation, specifically to support means:

-	For a given MnS consumer to request for a specific AI/ML capability to be executed in ML inference emulator environment,

-	For a given MnS consumer to request a specific ML inference emulator to execute a given AI/ML capability,

-	For a managed function to act as a ML inference emulator and execute AI/ML capabilities in a controlled way.

The network or its management system needs to have the capabilities and provide the services needed to enable the MnS consumer to request inference emulation and receive feedback on the inference emulation of a specific ML entity or of an application or function that contains an ML entity.

#### 5.15.2.2	Orchestrating ML Inference emulation

The 3GPP management system may have resources for multiple emulation environments to be used depending on need. These may include simulation environments, a digital twin of the network, a test network or the real network under curtain constrained conditions, e.g., for a selected set of UEs. The multiple emulation environments may represent different levels of trust that the operator or management system has in the ML entity or AI/ML inference functions. Correspondingly, 3GPP management system needs to have means and method for Orchestrating the inference emulation i.e., say called the inference emulation orchestrator. Accordingly:

-	the emulation orchestration process involves choosing the right type and instance of an emulation environment to which an ML entity, AI/ML inference Function or the action thereof may be tested depending on the needs of the function to be tested and the available emulation environments and their resources,

-	the emulation process may also involve executing the ML entity, AI/ML inference Function or its action on the real network but in a controlled fashion, e.g., only within certain hours or only on cells with a particular kind of load or only on cells in a particular area or in limited subscriber groups.

Relatedly, the actions taken by the inference emulation orchestrator may include:

-	Controlling the allowed parameter space/ranges of the parameters optimized by the ML entity or AI/ML inference Function depending on the emulation environment to which the ML entity, AI/ML inference function or the actions are being executed,

-	Adjusting the parameter space in consideration of the observed behaviour of the ML entity or AI/ML inference function,

-	Deploying the actions of the ML entity or AI/ML inference Function on a selected emulation environment or on the real network,

-	Blocking the ML entity or AI/ML inference function from being used on the network.

### 5.15.3	Potential requirements

REQ-AI/ML_EMUL-1: The 3GPP management system shall have a capability to allow an authorized MnS consumer to request an ML Inference Emulator to execute ML inference emulation for a specific ML entity.

REQ-AI/ML_EMUL-2: The 3GPP management system shall have a capability to allow an authorized MnS consumer to request for ML Inference Emulation for a specific ML entity using specified data or data with specifically stated characteristics and inference emulation features.

REQ-AI/ML_EMUL-3: The 3GPP management system shall have a capability to inform authorized AI/ML MnS consumer about the ML entity under execution in a specific ML inference emulator.

REQ-AI/ML_EMUL-4: The 3GPP management system shall have a capability to allow an authorized AI/ML MnS consumer (e.g., an operator) to manage or control a specific ML inference emulator request or ML inference emulation process, e.g., to start, suspend or restart the inference emulation; or to adjust the inference emulation conditions or characteristics.

REQ-AI/ML_EMUL-5: The 3GPP management system shall have a capability to allow an authorized AI/ML MnS consumer to request reporting, and for the ML inference emulator  to report on a specific ML inference emulation process or on the outcomes of any such ML inference emulation request or process.

REQ-AI/ML_EMUL-6: The 3GPP management system shall have a capability to allow an authorized AI/ML MnS consumer to configure an ML entity or AI/ML inference function supporting with the level of trust that expresses the degree to which the ML entity or AI/ML inference function or the different action thereof have been confirmed as trusted.

REQ-AI/ML_EMUL-7: The producer of ML inference emulation orchestration MnS shall have a capability to graduate an ML entity, AI/ML inference function or the different action thereof through different levels of trust each expressing a different degree to which the ML entity, AI/ML inference function or action has been confirmed as trusted.

### 5.15.4	Possible solutions

TBD

### 5.15.5	Evaluation

TBD

## 5.16	Training data effectiveness reporting and analytics

### 5.16.1	Description

For AI/ML model training, a large amount of data instances does not necessarily add value, e.g., if only a portion contribute to actual model training, the other portion will be discarded by some well-designed algorithms. During ML model training the information can be provided on whether a specific sample is useful for the training or not and on how much such a sample is useful for the training.

### 5.16.2	Use cases

#### 5.16.2.1	 Training data effectiveness reporting

To train a ML model, high quality and large volume of training data instances are mandatory. The general practice is to collect as much data as possible and feed them to the ML model for pre-processing and training, in the hope to get high quality of trained model. This is usually done without considering the possibly different contributions of the different portions of input data samples to the accuracy of the trained model.  However, this open use of all available data is expensive both on data collection and on computation resources as even the unnecessary data samples must be computed through the ML model. One solution is to leave the challenge  to the specific ML model training function to optimize the training data usage during model training, or before every training or retraining, e.g., by simply resampling the training data to only use part of the collected training data. However, in mobile networks where the amount of data is large, resources are constrained yet models need to be very accurate, this method is inappropriate. Instead, it is better that the training function evaluates the usefulness of different data samples or features and indicated that usefulness to the consumer so that the data used for retraining can be optimized.

The 3GPP management system needs to support means to report the contributions to model training of the different training data instances/samples based on insight of how the different portion of data contribute differently to the model accuracy.

#### 5.16.2.2	Training data effectiveness analytics

A single/independent observation on whether a certain sample or feature at a given timestamp contributed to model gradients cannot provide understanding on whether using such a sample or feature will contribute to model accuracy of the same model in further training/re-trainings, or if it will contribute to efficiency of training of further models related to the same use case.

In order to have such understanding, further analysis of the data related to the importance of the data instances during training is needed. The patterns of the most effective training data generated from the analytics would be very helpful to improve data collection in order to optimize the quantity and quality of the data to be used for training.

#### 5.16.2.3	Measurement data correlation analytics for ML training

For AI/ML model training, a large amount of measurement data points collected and does not necessarily add value, e.g., due to the complexity and time-varying nature of network, the measurement data collected can be highly correlated (linear or non-linear), using all measurement data for model training (and inference) is a waste of computing resource. Hence there is a need to have a solution:

-	For a give task (e.g., analytics, model training), automatically analyses the correlation among the given set of all measurement data, the output can be a much smaller set of measurement data, with which ML model training could be much more efficient with limited (or managed) impact to model training performance,

-	Regularly renew the correlation analytics as the time progress the correlation relationship might change; this is especially useful when there is regular ML model retraining needed.

### 5.16.3	Potential requirements

REQ-TRAIN_EFF-01: The 3GPP management system shall have the capability to allow an authorized consumer to configure an AI/ML training function to report the effectiveness of data used for model training.

REQ-TRAIN_EFF-02:	The 3GPP management system shall have the capability to report the effectiveness of data used in the training, including providing an indication or label of which data instance is useful or not useful and an indication of which data feature is useful or not useful, or is contributing negatively to the model training

REQ-TRAIN_EFF-03:	The 3GPP management system shall have the capability to allow the authorized consumer to activate the AI/ML training function /Entity to label the effectiveness of data used for training.

REQ-TRAIN_EFF-04:	The 3GPP management system shall have the capability to allow authorized consumer to request analytics for data used for model training with respect to effectiveness of such data during model training.

REQ-TRAIN_EFF-05:	The 3GPP management system shall have the capability to generate and to provide to consumer a pattern of highly effective data for training of either specific version of a model, all versions of a single model, or all models related to certain use case.

REQ-MEAS-DATA-1	the 3GPP management system shall have a capability to enable an authorized MnS consumer (e.g. an MLT function) to request the analysis of the correlation of measurement data used for training an ML entity.

REQ-MEAS-DATA-2	The AI/ML MnS producer shall have the capability enabling an authorized AI/ML MnS consumer to configure scheduling of the analysis of the correlation of measurement data.

REQ-MEAS-DATA-3 	The AI/ML MnS producer shall have the capability enabling an authorized AI/ML MnS consumer to manage the scheduled analysis of the correlation of measurement data, e.g., to suspend an ongoing scheduled MDCA activity, to resume a suspended scheduled MDCA activity, to cancel a scheduled MDCA activity, to configure the cycle of MDCA activity.

## 5.17	Trustworthy Machine Learning

### 5.17.1	Description

During ML training, testing and inference, the AI/ML trustworthiness management is needed. Based on the risk level (e.g., unacceptable, high, minimal) of the use case, the trustworthiness requirements for ML training, testing and inference may vary and therefore the related trustworthiness mechanisms need to be configured and  monitored. The purpose of AI/ML trustworthiness is to ensure that the model being trained, tested, and deployed is explainable, fair and robust.

Note: In the context of SA5, explainability of a model refers to explaining individual decisions predicted by the model and not explaining the internal behavior of the model itself.

The EU has proposed an AI regulation act for AI/ML consisting of several key requirements that the AI/ML systems should meet (based on the risk level of the use case) for them to be considered trustworthy [10]. These requirements include human agency and oversight, technical robustness and safety, privacy and data governance, transparency, diversity, non-discrimination and fairness, accountability, societal and environmental well-being. More details on each of these seven requirements are described in [11]. Furthermore, ISO/IEC analyses the factors that can impact the trustworthiness of systems providing or using AI and possible approaches or requirements to improving their trustworthiness that can be used by any business regardless of its size or sector [12].

Three well known categories under the umbrella of Trustworthy Machine Learning are as follows:

Explainable Machine Learning: Explainability in machine learning refers to the ability of ML models to enable humans to understand decisions or predictions made by them.

Fair Machine Learning: Fairness in machine learning refers to the process of correcting and eliminating bias in machine learning models.

Robust Machine Learning: Robustness in machine learning refers to the process of handling various forms of errors/corruptions in machine learning models as well as changes in the underlying data distribution in an automatic way.

These features apply to the four aspects of the ML process:

-	Data processing for use towards training, testing and inference,

-	The training of ML entities,

-	The testing of ML entities,

-	The use of ML entities for inference.

### 5.17.2	Use cases

#### 5.17.2.1	AI/ML trustworthiness indicators

The AI/ML trustworthiness indicators related to AI/ML training, testing and inference need to be precisely defined. The indicators mainly include three aspects:

Explainability-related indicators: the explainability indicators of the ML entity. For example, the AI/ML MnS consumer may indicate the AI/ML MnS producer to:

-	provide local explanation for one particular instance predicted by the ML entity without disclosing the ML entity internals.

-	provide global explanation for a group of instances predicted by the ML entity without disclosing the ML entity internals.

-	evaluate monotonicity - a quantitative metric for explainability - that measures the effect of individual features on ML entity performance by evaluating the effect on ML entity performance by incrementally adding each feature in order of increasing importance.

Fairness-related indicators: the fairness indicators of the data or the ML entity. For example, the AI/ML MnS consumer may indicate the AI/ML MnS producer to:

-	evaluate disparate impact - a quantitative measure for fairness - that measures the ratio of rate of favorable outcome for the unprivileged group to that of the privileged group.

-	evaluate Manhattan distance - a quantitative measure for fairness - that measures the average distance between the samples from two datasets.

-	evaluate average odds difference - a quantitative measure for fairness - that measures the average difference of false positive rate and true positive rate between unprivileged and privileged groups.

Robustness-related indicators: the robustness  indicators of the data or the ML entity. For example, the AI/ML MnS consumer may indicate the AI/ML MnS producer to:

-	evaluate missingness ratio - a quantitative measure for robustness - that measures the percentage of missing values in the training dataset.

Depending on the use case, some or all trustworthiness indicators can be selected for monitoring and evaluation. The AI/ML MnS consumer should first determine which indicators are needed and then request the AI/ML MnS producer to monitor and evaluate the requested indicators.

#### 5.17.2.2	AI/ML data trustworthiness

The training data, testing data and inference data used for ML training, testing and inference, respectively, may need to be pre-processed according to the desired trustworthiness measure of the ML model. For example,

-	The samples in the training data and testing data can be labeled to include the ground-truth explanation label (in addition to the ground-truth class label). Therefore, the ML model can be trained to predict both ground-truth explanations label and ground-truth class label for an inference sample.

-	The samples in the training data and testing data can be assigned weights to ensure individual or group fairness in the ML model.

-	The missing features in the training data, testing data and inference data can be imputed with mean values to ensure the ML model is technically robust.

-	Noise can be added to the training data and testing data to ensure that the data samples are free from any kind of poisoning attacks.

Depending on the use case, some or all data trustworthiness pre-processing techniques can be applied before training, testing and deployment of the ML model. The MnS consumer should be enabled to receive information on the supported trustworthiness-related data processing capabilities for training, testing or inference. Moreover the producer of data processing be it for training, testing or inference should enable the MnS consumer to provide requirements for trustworthiness which should then be considered in the data processing. And the MnS consumer should be enabled to define their reporting characteristics for ML trustworthiness.

#### 5.17.2.3	ML training trustworthiness

The ML training may need to be performed according to the desired trustworthiness measure of the ML model. For example,

-	The ML model can be trained to generate explanations for the predictions.

-	The ML model can be trained to detect and mitigate biased outcomes.

-	The ML model can be trained to perform well on unseen or missing data.

-	The ML model can be trained together with adversarial input samples so that the trained model can detect adversaries.

Depending on the use case, one or more training trustworthiness techniques can be applied during training the ML model. Therefore, the ML training producer can be queried to provide information on the supported training trustworthiness capabilities enabling the ML training MnS consumer to request for a subset of supported training trustworthiness characteristics to be configured, measured, and reported.

#### 5.17.2.4	AI/ML inference trustworthiness

The AI/ML inference may need to be performed according to the desired trustworthiness measure of the ML model. For example,

-	Post-processing explanations can be generated based on one or multiple inferences generated by the ML model.

-	The ML model can be trained to flip biased outcomes during inference using post-processing fairness techniques, for e.g., based on confidence value of a prediction.

-	The ML model can be trained to infer well on unseen or missing inference data.

-	Perturbing model predictions to obfuscate labels/confidence information to protect them from model inversion or model extraction attacks.

Depending on the use case, one or more inference trustworthiness techniques can be applied on the deployed ML model. Therefore, the AI/ML inference producer can be queried to provide information on the supported inference trustworthiness capabilities enabling the AI/ML inference consumer to request for a subset of supported inference trustworthiness characteristics to be configured, measured, and reported.

### 5.17.3	Potential requirements

REQ-ML_TRUST_IND-1 The AI/ML MnS producer should have a capability to define trustworthiness indicators for AI/ML data or ML entity and select some indicators based on the use case.

REQ-ML_TRUST_IND-2 The AI/ML MnS producer should have a capability to define a common trustworthiness measure covering main aspects of trustworthiness indicators of AI/ML data or ML entity.

REQ-ML_TRUST_IND-3 The AI/ML MnS producer should have a capability to enable the authorized MnS consumer to request for the desired individual or common trustworthiness measure of AI/ML data or ML entity.

REQ-ML_TRUST_IND-4 The AI/ML MnS producer should have a capability to report to the authorized MnS consumer the achieved individual or common trustworthiness measure of AI/ML data or ML entity.

REQ-ML_DATA_TRUST-1 The producer(s) of ML training, ML testing and AI/ML inference service(s) should support a capability to enable an authorized MnS consumer to request reporting on the supported data trustworthiness related pre-processing capabilities of an ML entity.

REQ-ML_DATA_TRUST-2 The producer(s) of ML training, ML testing and AI/ML inference service(s) should have a capability to pre-process the training data, testing data and inference data of an ML entity to satisfy the desired data trustworthiness measure.

REQ-ML_DATA_TRUST-3 The  producer(s)of ML training, ML testing and AI/ML inference service(s) should support a capability to enable an authorized MnS consumer to define the reporting characteristics related to the data trustworthiness reports of an ML entity.

REQ-ML_TRAIN_TRUST-1 The ML training MnS producer should support a capability to enable an authorized MnS consumer to request reporting on the supported training explainability capabilities of an ML entity.

REQ-ML_TRAIN_TRUST-2 The ML training MnS producer should have a capability to train a specific ML entity using training data with explainability characteristics as defined by the MnS consumer.

REQ-ML_TRAIN_TRUST-3 The ML training MnS producer should support a capability to enable an authorized MnS consumer to define the reporting characteristics related to the training explainability reports  of an ML entity.

REQ-ML_TRAIN_TRUST-4 The ML training MnS producer should support a capability to enable an authorized MnS consumer to request reporting on the supported training fairness capabilities of an ML entity.

REQ-ML_TRAIN_TRUST-5 The ML training MnS producer should have a capability to train a specific ML entity using training data with fairness characteristics as defined by the MnS consumer.

REQ-ML_TRAIN_TRUST-6 The ML training MnS producer should support a capability to enable an authorized MnS consumer to define the reporting characteristics related to the training fairness reports of an ML entity.

REQ-ML_TRAIN_TRUST-7 The ML training MnS producer should support a capability to enable an authorized MnS consumer to request reporting on the supported training robustness capabilities of an ML entity.

REQ-ML_TRAIN_TRUST-8 The ML training MnS producer should have a capability to train a specific ML entity using training data with robustness characteristics as defined by the MnS consumer.

REQ-ML_TRAIN_TRUST-9 The ML training MnS producer should support a capability to enable an authorized MnS consumer to define the reporting characteristics related to the training robustness  reports  of an ML entity.

REQ-ML_INF_TRUST-1 The provider of AI/ML inference should have a capability to infer using a specific ML entity rained and tested with explainability characteristics as defined by the MnS consumer.

REQ-ML_INF_TRUST-2 The provider of AI/ML inference should support a capability for an authorized MnS consumer to define the reporting characteristics related to the inference explainability reports of an ML entity.

REQ-ML_INF_TRUST-3 The provider of AI/ML inference should have a capability to infer using a specific ML entity trained and tested with fairness characteristics as defined by the MnS consumer.

REQ-ML_INF_TRUST-4 The provider of AI/ML inference should support a capability for an authorized MnS consumer to define the reporting characteristics related to the inference fairness reports of an ML entity.

REQ-ML_INF_TRUST-5 The provider of AI/ML inference should have a capability to infer using a specific ML entity trained and tested with robustness characteristics as defined by the MnS consumer.

REQ-ML_INF_TRUST-6 The provider of AI/ML inference should support a capability for an authorized MnS consumer to define the reporting characteristics related to the inference robustness reports of an ML entity.

### 5.17.4	Possible solutions

TBD

### 5.17.5	Evaluation

TBD

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-03 | SA5#142e | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222188 | - | - | - | Add scope | 0.1.0 |
| 2022-04 | SA5#142e | S5-222477 | - | - | - | Add AI/ML management overview | 0.1.0 |
| 2022-05 | SA5#143e | S5-223576 | - | - | - | Add use case on AI/ML model performance | 0.2.0 |
| 2022-07 | SA5#144e | S5-224364 | - | - | - | Requirements on AIML context | 0.3.0 |
| 2022-07 | SA5#144e | S5-224365 | - | - | - | Add AI-ML workflow for 5GS | 0.3.0 |
| 2022-07 | SA5#144e | S5-224366 | - | - | - | Clarifications into the overview and terminologies updates | 0.3.0 |
| 2022-07 | SA5#144e | S5-224367 | - | - | - | Requirements on AIML Inference History | 0.3.0 |
| 2022-07 | SA5#144e | S5-224368 | - | - | - | Add use case on AI-ML entity validation | 0.3.0 |
| 2022-07 | SA5#144e | S5-224369 | - | - | - | Requirements on AIML Testing | 0.3.0 |
| 2022-07 | SA5#144e | S5-224386 | - | - | - | Requirements on AIMLEntity Capability Discovery | 0.3.0 |
| 2022-07 | SA5#144e | S5-224370 | - | - | - | Requirements on Pre-processed event data for ML training | 0.3.0 |
| 2022-07 | SA5#144e | S5-224371 | - | - | - | Add use case on AI-ML model update | 0.3.0 |
| 2022-07 | SA5#144e | S5-224372 | - | - | - | Add use case on AI-ML model deployment | 0.3.0 |
| 2022-08 | SA5#145e | S5-225693 | - | - | - | Enhance AI-ML workflow | 0.4.0 |
| 2022-08 | SA5#145e | S5-225694 | - | - | - | Potential solution on AIML Inference History | 0.4.0 |
| 2022-08 | SA5#145e | S5-225695 | - | - | - | Potential Solution on AIMLEntity Capability Discovery | 0.4.0 |
| 2022-08 | SA5#145e | S5-225696 | - | - | - | Clarification on AIMLEntity Capability Mapping | 0.4.0 |
| 2022-08 | SA5#145e | S5-225520 | - | - | - | Add possible solution for AI-ML entity validation | 0.4.0 |
| 2022-08 | SA5#145e | S5-225697 | - | - | - | Add possible solution for AI-ML entity testing | 0.4.0 |
| 2022-08 | SA5#145e | S5-225698 | - | - | - | Add possible solution on AI-ML model update | 0.4.0 |
| 2022-08 | SA5#145e | S5-225699 | - | - | - | Use case on Coordination of the AIML capability between 5GC/RAN and 3GPP management system | 0.4.0 |
| 2022-08 | SA5#145e | S5-225700 | - | - | - | Add use case on AI-ML model configuration | 0.4.0 |
| 2022-08 | SA5#145e | S5-225701 | - | - | - | Use case on Abstract AIML Behavior | 0.4.0 |
| 2022-08 | SA5#145e | S5-225702 | - | - | - | Use case on impacts of AIML inference actions | 0.4.0 |
| 2022-08 | SA5#145e | S5-225703 | - | - | - | Use case on ML gradual activation | 0.4.0 |
| 2022-08 | SA5#145e | S5-225704 | - | - | - | Requirements on Mobility of AIML Context | 0.4.0 |
| 2022-08 | SA5#145e | S5-225705 | - | - | - | Potential solutions on AIML context | 0.4.0 |
| 2022-08 | SA5#145e | S5-225802 | - | - | - | Use case on orchestrating AIML inference | 0.4.0 |
| 2022-08 | SA5#145e | S5-225043 | - | - | - | Use case on improving AI/ML (re-)training efficiency | 0.4.0 |
| 2022-11 | SA5#146 | S5-226916 | - | - | - | Addressing wording issues | 0.5.0 |
| 2022-11 | SA5#146 | S5-226917 | - | - | - | pCR 28.809 Clarifying simultaneous and separate execution of training and inference phases | 0.5.0 |
| 2022-11 | SA5#146 | S5-226918 | - | - | - | Corrections on terms and definitions | 0.5.0 |
| 2022-11 | SA5#146 | S5-226558 | - | - | - | Corrections including editorials | 0.5.0 |
| 2022-11 | SA5#146 | S5-226924 | - | - | - | Add use cases for AI/ML performance evaluation | 0.5.0 |
| 2022-11 | SA5#146 | S5-226927 | - | - | - | Add potential solutions for AI-ML performance evaluation | 0.5.0 |
| 2022-11 | SA5#146 | S5-226919 | - | - | - | Use case and potential solution on Abstraction of AIML performance | 0.5.0 |
| 2022-11 | SA5#146 | S5-226920 | - | - | - | Selection of AIML performance indicators | 0.5.0 |
| 2022-11 | SA5#146 | S5-226921 | - | - | - | Policy based selection of Performance Indicators | 0.5.0 |
| 2022-11 | SA5#146 | S5-226928 | - | - | - | Terminology on Trustworthy AI/ML | 0.5.0 |
| 2022-11 | SA5#146 | S5-226929 | - | - | - | Add use case on AIML data trustworthiness | 0.5.0 |
| 2022-11 | SA5#146 | S5-226934 | - | - | - | Add use case on AIML training trustworthiness | 0.5.0 |
| 2022-11 | SA5#146 | S5-226935 | - | - | - | Add use case on AIML trustworthiness indicators | 0.5.0 |
| 2022-11 | SA5#146 | S5-226936 | - | - | - | Add use case on AIML inference trustworthiness | 0.5.0 |
| 2022-11 | SA5#146 | S5-226937 | - | - | - | Add use case and potential solution on AIML modularity | 0.5.0 |
| 2022-11 | SA5#146 | S5-226940 | - | - | - | Add use case on AIML inference emulation | 0.5.0 |
| 2022-11 | SA5#146 | S5-226372 | - | - | - | Correct the illustration of AIML inference history request and control | 0.5.0 |
| 2022-11 | SA5#146 | S5-226373 | - | - | - | Correct the illustration of AIML entity testing and control | 0.5.0 |
| 2022-11 | SA5#146 | S5-226942 | - | - | - | Add possible solution for AI/ML configuration | 0.5.0 |
| 2022-11 | SA5#146 | S5-226941 | - | - | - | Potential solution on ML Gradual Activation | 0.5.0 |
| 2022-11 | SA5#146 | S5-226943 | - | - | - | Use case on policy-based Activation | 0.5.0 |
| 2022-11 | SA5#146 | S5-226945 | - | - | - | Potential solution on improving retraining efficiency | 0.5.0 |
| 2022-11 | SA5#146 | S5-226986 | - | - | - | Potential solution on event data for ML training | 0.5.0 |
| 2022-11 | SA5#146 | S5-226371 | - | - | - | Correct the illustration of Event data for ML training | 0.5.0 |
| 2022-11 | SA5#146 | S5-226385 | - | - | - | Add use case on measurement data for ML training | 0.5.0 |
| 2022-11 | SA5#146 | S5-226984 | - | - | - | Potential solution on Mobility of AI/ML Context | 0.5.0 |
| 2022-11 | SA5#146 | S5-226985 | - | - | - | Potential solution on Producer Initiated AIML Training | 0.5.0 |
| 2022-11 | SA5#146 | S5-226048 | - | - | - | Potential solution on Abstract AIML Behavior | 0.5.0 |
| 2022-11 | SA5#146 | S5-226987 | - | - | - | Potential solution on Orchestrating AIML inference | 0.5.0 |
| 2022-11 | SA5#146 | S5-226988 | - | - | - | Evaluations for Solution 5.6 and 5.7 | 0.5.0 |
| 2022-11 | SA5#146 | S5-227011 | - | - | - | Add possible solutions for AI/ML entity deployment | 0.5.0 |
| 2022-11 | SA5#146 | S5-226997 | - | - | - | Update description and requirements on supporting online AIML entity testing | 0.5.0 |
| 2022-11 | SA5#146 | S5-226555 | - | - | - | Add use case on training data effectiveness report and analytics | 0.5.0 |