| 3GPP TR 28.835 V0.5.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on Management Aspect Enhancement of 5G MOCN Network Sharing Phase2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Concepts and overview	6

5	Key issues and potential solutions	6

5.1	Issue #1: Management requirement for different POP’s network operation	6

5.1.1	Description	6

5.1.2	Potential solution	6

5.2	Issue #2: Performance measurements without PLMN ID at NRCellCU	7

5.2.1	Description	7

5.2.2	Potential solutions	7

5.2.2.1	Solution 1	7

5.2.2.2	Solution 2	8

5.3	Issue #3:  Management requirement between MOP-NM and MOP-SR-DM	9

5.3.1	Description	9

5.3.2	Potential solution	10

5.4	Issue #4:  Service-based management architecture for MOCN	11

5.4.1	Description	11

5.4.2	Potential solution	12

5.5	Issue #5:  Remote access requirement of MOP-SR-DM for each POP	12

5.5.1	Description	12

5.5.2	Potential solution	12

5.6	Issue #6:  PLMN-related attribute requirement for operator specific IOC	13

5.6.1	Description	13

5.6.2	Potential solution	13

5.7	Issue #7:  5QI-related attribute requirement for operator specific IOC	13

5.7.1	Description	13

5.7.2	Potential solutions	13

5.7.2.1	Solution 1	13

5.7.2.2	Solution 2	14

5.7.2.3	Solution 3	14

5.8	Issue #8:  PLMN granularity requirement of performance measurements for MOCN	15

5.8.1	Description	15

5.8.2	Potential solution	15

6	Conclusion and Recommendation	16

6.X	Issue #X	16

6.Y	Issue #Y	16

Annex <X> (informative): Change history	17

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This report is to study on management aspect enhancement of 5G MOCN network sharing Phase2.

1	Scope

The present document studies on potential requirements, impact on interaction between MOP and POP/NEP, and management architecture and more potential performance measurements of 5G MOCN network sharing.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                        3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3".

[3]                        3GPP TS 28.552: "5G performance measurements".

[4]                        3GPP TS 32.130: "Network sharing; Concepts and requirements".

[5]                        3GPP TS 28.533: " Architecture framework".

[6]                        3GPP TS 28.622: " Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[7]                        3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[8]                       3GPP TS 28.532: " Management and orchestration; Generic management services" .

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Concepts and overview

5G network sharing, especially the 5G MOCN scenario, is essential to save construction costs for network operators. So far, requirements of configuration, management authority and performance measurement for network sharing have been covered in TS 32.130[4]. The NRM and part of performance measurement for 5G MOCN network sharing have been defined in TS 28.541 and TS 28.552. With the deployment and operation of 5G MOCN network sharing, more and more new detailed management requirements need be investigated for 5G MOCN network sharing. For example, preventing data modification by MOP-NM and ensuring data fairness per POP, MOP-NM needs to get different data from MOP-SR-DM, and put different data to different POP-NM. The performance measurement in TS 28.552[3] cannot fully evaluate the network performance of 5GMOCN network sharing.

To enhance the sharing mechanism, the present document studies on management aspects of 5G MOCN network sharing, including potential requirements, impact on interaction between MOP and POP/NEP, and management architecture, and more potential performance measurements in management system.

5	Key issues and potential solutions

5.1	Issue #1: Management requirement for different POP’s network operation

5.1.1	Description

For MOCN Networking Sharing, different POP needs different data for personalized operation and maintenance services (e.g. mobility management measurements focused by POP A, packet delay measurements focused by POP B). POP-NM needs to get different data from MOP-NM for POPs’ network operation.

Figure 5.1-1: Management architecture for MOCN

5.1.2	Potential solution

1. According to different POPs’ operation requirements, the 3GPP management system of the MOP shall have capability to report different configuration and performance measurements (e.g. Mobility management measurements focused by POP A, packet delay measurements focused by POP B) for each POP.

2. Ensuring alarm continuity, the 3GPP management system of the MOP shall have capability to provide full fault measurements for each POP.

5.2	Issue #2: Performance measurements without PLMN ID at NRCellCU

5.2.1	Description

In TS 28.541[2], the NRCellCU IOC with pLMNInfoList attribute can naturally distinguish PLMN granularity. However, some performance measurements that donot contain PLMN information are collected at NRCellCU in TS 28.552[3]. These measurements, which are based on the message without PLMN Id information, are inappropriate to be collected from NRCellCU. For example, the NRCellCU which distinguishs PLMN granularity cannot collect RRC.ConnEstabAtt and RRC.ReEstabAtt (See TS 28.522 [3]) measurements.

For 5G MOCN network sharing, some potential solutions need to be studied to support above performance measurements without PLMN ID.

5.2.2	Potential solutions

5.2.2.1	Solution 1

- The following enhanced NR NRM solution is proposed:

Introduce new OperatorCU <<IOC>> name-contained by GNBCUCPFunction<<IOC >> to represent operator specific gNB level information (including operator specific gNB Id, gNBIdlength, pLMNId), and new NROperatorCellCU<<IOC>> name-contained by OperatorCU<<IOC>> to represent operator specific cell level information (including operator specific CelllocalId, PLMNInfoList). Which means individual OperatorCU instance and NROperatorCellCU needs to be created and configured for each POP.

Figure 5.2-1 NR NRM enhancement to suport NG-RAN sharing MOCN network sharing

The OperatorCU <<IOC>> contains attributes to support operator specific gNB-CU level information (including gNB Id, gNBIdLength, PLMNId) to support 5G Multi-Operator Core Network (5G MOCN) network sharing. Individual instance of OperatorCU <<IOC>> should be created and configured for each POP. When configured the attributes override those in parent GNBCUCPFunction instance.

The OperatorCU<<IOC>> is only used to support MOCN. If the MOCN is not supported, this IOC is not used.

The OperatorCU<<IOC>> includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | F | T |

NROperatorCellCU<<IOC>>

The NROperatorCellCU <<IOC>> contains attributes to support operator specific cell level information (including cellLocalId, pLMNInfoList) to support 5G Multi-Operator Core Network (5G MOCN) network sharing. Individual instance of NROperatorCellCU <<IOC>> should be created and configured for each POP. When configured the attributes override those in associated NRCellCU instance.

The NROperatorCellCU <<IOC>> is only used to support MOCN. If the MOCN is not supported, this IOC is not used.

The NROperatorCellCU <<IOC>> includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellCURef | M | T | T | F | T |

Some updates are needed for NRCellCU IOC, NRCellRelation IOC and ExternalNRCellCU IOC in the existing NR NRM.

- The Support Qualifier for attribute "cellLocalId " and "pLMNInfoList" in existing NRCellCU IOC needs to be changed to "CM", the condition is " If MOCN Network Sharing is configured the value is not used ".

- The define of exisiting NRCellRelation IOC needs to be added: In the MOCN networking sharing scenario, the NRCellCU instance of the target cell and source cell need to be changed an NROperatorCU instance.

- The define of exisiting ExternalNRCellCU IOC needs to be added: In the MOCN networking sharing scenario, ExternalNRCellCU IOC represents the properties of an NROperatorCellCU controlled by another Management Service Provider.

- Operator specific measurements can be collected and reported within OperatorCU<<IOC>> and NROperatorCellCU<<IOC>>, Operator agnostic measurements can be collected and reported within GNBCUCPFunction<<IOC>> and NRCellCU<<IOC>>.

- The following NR NRM fragments need to be added.

Figure 5.2-2 NRM for Eps to support <<IOC>>OperatorCU for NG-RAN MOCN network sharing

For the scenario of individual interfaces for NG-RAN MOCN network sharing, EP_XnC, EP_E1, EP_F1C, EF_NgC and EP-X2C name contained by OperatorCU can be used to represent individual interfaces for each POP.

5.2.2.2	Solution 2

In this solution, NRCellDU is added as the measurement object class to the attempt and failure measurements of RRC connection related measurements. Detailedly, the following modification is applied to the measurements “5.1.1.15.1 Attempted RRC connection establishments”,  “5.1.1.15.3 Failed RRC connection establishments”,  “5.1.1.17.1 Number of RRC connection re-establishment attempts”,  “5.1.1.17.4	Number of RRC connection re-establishment attempts followed by RRC Setup”, “5.1.1.18.1 Number of RRC connection resuming attempts”, “5.1.1.18.4	RRC connection resuming followed by network release”, “5.1.1.18.5	RRC connection resuming followed by network suspension”, “5.1.1.18.6	Number of RRC connection resuming attempts followed by RRC Setup”.

******** the proposed modification: *********

f)	NRCellCU (for all scenarios except MOCN network sharing with multiple Cell Identity broadcast scenario),

NRCellDU (for MOCN network sharing with multiple Cell Identity broadcast scenario).

******** the proposed modification end *********

Based on the above mentioned modificaitons, for MOCN network sharing with multiple Cell Identity broadcast scenario, the corresponding KPI formulas can be:

- RRC connection setup success rate

RRC connection setup success rate = ∑NRCellCU.RRC.ConnEstabSucc/NRCellDU.RRC.ConnEstabAtt.

That is RRC connection setup success rate on NRCellDU level the measurement shall be the averaged over all NRCellCUs Associated NRCellDU where NRCellCU.cellLocalId equeals NROperatorCellDU.cellLocalId and NROperatorCellDU.nRCellDURef equeals NRCellDU

- RRC connection re-establishment success rate

RRC connection re-establishment success rate = (∑NRCellCU.RRC.ReEstabSuccWithUeContext+∑NRCellCU.RRC.ReEstabSuccWithoutUeContext + NRCellDU. RRC.ReEstabFallbackToSetupAtt) / NRCellDU.RRC.ReEstabAtt.

That is RRC connection re-establishment success rate on NRCellDU level the measurement shall be the averaged over all NRCellCUs Associated NRCellDU where NRCellCU.cellLocalId equeals NROperatorCellDU.cellLocalId and NROperatorCellDU.nRCellDURef equeals NRCellDU

- RRC connection resuming success rate

RRC connection resuming success rate = (∑NRCellCU.RRC.ResumeSucc + ∑NRCellCU.RRC.ResumeSuccByFallback+ NRCellDU.RRC.ResumeFollowedbyNetworkRelease +NRCellDU.RRC.ResumeFollowedbySuspension +NRCellDU.ResumeFallbackToSetupAtt) / NRCellDU.RRC.ResumeAtt.

That is RRC connection resuming success rate on NRCellDU level the measurement shall be the averaged over all NRCellCUs Associated NRCellDU where NRCellCU.cellLocalId equeals NROperatorCellDU.cellLocalId and NROperatorCellDU.nRCellDURef equeals NRCellDU

5.3	Issue #3:  Management requirement between MOP-NM and MOP-SR-DM

5.3.1	Description

For MOCN Networking Sharing, different POP needs different data for personalized operation and maintenance services (e.g. mobility management measurements focused by POP A, packet delay measurements focused by POP B). MOP-NM needs to sent different data to POP-NM for POPs’ network operation.

Figure 5.3-1: Data flow of configuration and performance measurements for MOCN

The data flow of configuration and performance measurements is depicted in figure 5.3-1. Preventing data modification by MOP-NM and ensuring data fairness per POP, different configuration and performance measurements for each POP on MOP-SR-DM need to be forwarded to MOP-NM. Then the MOP-NM sent these data to POP-NM directly.

The data flow of alarm data is depicted in figure 5.3-2. Ensuring alarm ID continuity, different alarm data for each POP on MOP-NM need to be forwarded to each POP.

Figure 5.3-2: Data flow of alarm data for MOCN

Editor's Note: The above figures will be modified on service-based management architecture if service-based management architecture for MOCN is determined.

5.3.2	Potential solution

- Individual PerfMetricJob instance can be created and configured for each POP.

- The following attribute pLMNId can be added to PerfMetricJob IOC defined in TS 28.622[6] and used to identify the POP.

The optional attribute pLMNId can be used to establish operator-instance of PerfMetricJob IOC. The attribute performanceMetrics and optional attribute pLMNId together define the performance metrics to be produced according to individual POP’s requirements for MOCN scenario.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | CO | T | T | F | T |

The attribute constraint of pLMNId shall be added as follows:

| Name | Definition |
| --- | --- |
| pLMNId | This attribute should be supported, when the MnS producer supports the PLMN specific jobs for MOCN scenario. |

The definition of pLMNId can see in the following table.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| pLMNId | This parameter defines the information of a PLMN identification and is used to distinguish operator granularity. It is defined in TS 28.658[7]. allowedValues: N/A | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

- The getMOIAttribute operation defined in TS 28.532[8] shall have capability to filter opetrator-specific managed objects.

The optional parameter "filter" shall support operator-instance filtering. The pLMNId can be one filter criteria, which is applied to the objects in the subtree of baseObjectInstance.

- The getAlarmList operation defined in TS 28.532[8] shall have capability to filter opetrator-specific alarms.

The optional parameter "filter" shall support operator-instance filtering. The pLMNId can be one filter criteria, which is applied to the objects selected by the "baseObjectInstance" parameter. MnS producer shall apply pLMNId on AlarmInformation instances in AlarmList when constructing its output parameter AlarmInformationList.

5.4	Issue #4:  Service-based management architecture for MOCN

5.4.1	Description

The management architecture of network sharing in TS 32.130 [4] is not service-based management architecture, which cannot meet the management requirements of 5G MOCN network sharing. The Service Based Management Architecture (SBMA) need to be provided for MOCN networking sharing.

5.4.2	Potential solution

The service-based management architecture for MOCN is depicted in figure 5.4-1. It is compliant with 3GPP management reference models (TS 28.533 [5] and TS 32.130 [4]).

Figure 5.4-1: Service-based management architecture for MOCN

The service-based management architecture for MOCN shows that management functions may interact by consuming management services produced by other management functions:

-	POP-NM can consume MnS(s) provided by MOP-NM and POP-RAN-DM.

-	MOP-NM provides a set of MnS(s) for POP-NM and consumes MnS(s) provided by MOP-SR-DM.

-	MOP-SR-DM provides a set of MnS(s) for MOP-NM and POP-RAN-DM provides a set of MnS(s) for POP-NM.

5.5	Issue #5:  Remote access requirement of MOP-SR-DM for each POP

5.5.1	Description

For MOCN Networking Sharing and POP’s network operation requirements, each POP needs to get and view some individual requirement data from MOP-SR-DM directly. In TS 32.130[4], the data of MOP-SR-DM is transmitted to POP-NM through MOP-NM, which cannot meet the real-time acquisition and viewing data requirements of each POP. This data of MOP-SR-DM contain configuration data, performance data and alarms here. According to the POP’s real-time data requirements, MOP-SR-DM needs to allow each POP to be able to access remotely and provide relevant system user permissions for each POP.

5.5.2	Potential solution

1. The MOP-SR-DM shall have capability to provide remote access for each POP to log in.

2. The MOP-SR-DM shall have capability to provide view permission of different data (e.g. configuration, performance measurements and alarms) for each POP, when POP login MOP-SR-DM.

3. The MOP-SR-DM shall have capability to provide get permission of different data (e.g. configuration, performance measurements and alarms) for each POP, when POP login MOP-SR-DM.

4. The MOP-SR-DM shall have capability to provide prohibit-modification permission of different data (e.g. configuration, performance measurements and alarms) for each POP, when POP login MOP-SR-DM.

5.6	Issue #6:  PLMN-related attribute requirement for operator specific IOC

5.6.1	Description

For MOCN networking sharing and POP’s network operation requirements, each POP needs to get some individual operator-specific data from MOP-SR-DM or MOP-NM. Some IOCs defined in TS 28.541[2] should create and configure an instance for each POP, but the PLMN-related attribute of these IOCs are not defined. Some of the operations defined in TS 28.532[8] shall have capability to filter opetrator-specific object instances via the PLMN-related attribute. According to the POP’s data requirements and supporting the operation PLMN-filtering capability, some operator specific IOCs need to add PLMN-related attribute for creating different POPs’ object instances. In addition, the operator specific IOCs mainly include two categories: specific IOCs for NRM NG-RAN MOCN network sharing(e.g. OperatorDU) and operator-related IOCs whose instances or attributes can be configured for each POP(e.g.  configurable5QISet).

5.6.2	Potential solution

1. The OperatorDU IOC defined in TS 28.541[2] shall add PLMN-related attributes.

The pLMNId attribute shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |

- The GNBDUFunction IOC defined in TS 28.541[2] shall add PLMN-related attributes.

  - The pLMNId attribute shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | O | T | T | F | T |

5.7	Issue #7:  5QI-related attribute requirement for operator specific IOC

5.7.1	Description

For MOCN RAN network sharing and POP’s network operation requirements, each POP needs to get some individual operator-specific data from MOP-SR-DM or MOP-NM.

5.7.2	Potential solutions

To configure operator specific 5QI an optional attribute can be added to the Configurable5QISet to identify the operator to which the 5QI set applies.

5.7.2.1	Solution 1

- The Configurable5QISet IOC defined in TS 28.541[2] could be updated to include PLMN-related attributes identifying each POP.

- The potential NRM changes to support above could include:

  - New attribute added to Configurable5QISet:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | CM | T | T | F | T |

  - with constraint:

| Name | Definition |
| --- | --- |
| pLMNId S | Condition: The NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature is supported. |

- Such updated Configurable5QISet could be used to create a 5QI set for each POP’s pre-configured 5QIs in MOCN networking sharing scenario.

5.7.2.2	Solution 2

To configure operator specific 5QI sets, new attributes could be added to the OperatorDU to refer to operator specific 5QI sets.

This would not impact the Configurable5QISet definition and keeps the 5QI set configuration the same for RAN MOCN network sharing, and non-sharing scenarios.

- The OperatorDU IOC defined in TS 28.541[2] could be updated to include a reference to a specific 5QI set.

- The potential NRM changes to support above could include:

New attributes added to OperatorDU:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| configurable5QISetRef | CO | T | T | F | T |
| dynamic5QISetRef | CO | T | F | F | T |

with constraints:

| Name | Definition |
| --- | --- |
| configurable5QISetRef | Condition: The NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported. |
| dynamic5QISetRef | Condition: The NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported. |

- One instance of Configurable5QISet IOC defined in TS 28.541[2] is created for each POP to support different POP’s pre-configured 5QIs in MOCN RAN network sharing scenario.

- The Configurable5QISet instance should be contained under the same GNBDUFunction MOI as the operator specific OperatorDU is contained.

- Each operator’s OperatorDU instance references their specific 5QI set via the new reference attribute (configurable5QISetRef).

5.7.2.3	Solution 3

To configure operator specific 5QI sets, new attributes could be added to the Configurable5QISet to refer to operator specific DUs.

This would impact the Configurable5QISet definition and make the 5QI set configuration different for RAN MOCN network sharing, and non-sharing scenarios.

- The Configurable5QISet IOC defined in TS 28.541[2] could be updated to include a reference to a specific OperatorDU instance.

- The potential NRM changes to support above could include:

New attributes added to Configurable5QISet:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operatorDURef | CO | T | T | F | T |

with constraints:

| Name | Definition |
| --- | --- |
| operatorDURef | Condition: The NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported. |

- One instance of Configurable5QISet IOC defined in TS 28.541[2] is created for each POP to support different POP’s pre-configured 5QIs in MOCN RAN network sharing scenario.

- Each Configurable5QISet instance should be contained under the same GNBDUFunction MOI as the operator specific OperatorDU is contained.

- Each operator’s Configurable5QISet instance references their specific OperatorDU instance via the new reference attribute (operatorDURef).

5.8	Issue #8:  PLMN granularity requirement of performance measurements for MOCN

5.8.1	Description

For management requirements of MOCN networking sharing, MOP-NM needs to get some PLMN granularity performance measurements to support the POP’s network operation. For an operator in a shared network, cell-level metrics cannot meet the requirements of a single operator for network evaluation and optimization. The 3GPP management system of the MOP shall have the capability to collect and report some measurements in PLMN granularity for each POP. However, in TS 28.552, some measurements(e.g. PDU session management measurements, DRB related measurements) have not yet been differentiated the PLMN granularity. So some non-PLMN performance measurements need add PLMN granularity in MOCN networking sharing scenario.

5.8.2	Potential solution

1. At NRCellCU, some performance measurements defined in TS 28.522 need to add PLMN granularity.

The PLMN granularity need to be added in the following performance measurements:

| Category | Measurement Name |
| --- | --- |
| RRC connection number | Max number of stored inactive RRC Connections |
| PDU Session Management | Number of PDU Sessions requested to setup |
|  | Number of PDU Sessions successfully setup |
|  | Number of PDU Sessions failed to setup |
| DRB related measurements | Mean number of DRBs being allocated |
|  | Peak number of DRBs being allocated |
| QoS flow setup | Number of QoS flow attempted to setup |
|  | Number of QoS flow successfully established |
|  | Number of QoS flow failed to setup |
| QoS flow modification | Number of QoS flows attempted to modify |
|  | Number of QoS flows successfully modified |
|  | Number of QoS flows failed to modify |
| QoS flow release | Number of released active QoS flows |
| 5QI 1 QoS Flow Duration Monitoring | Average Normally Released Call (5QI 1 QoS Flow) Duration |
|  | Average Abnormally Released Call (5QI 1 QoS Flow) Duration |
|  | Distribution of Normally Released Call (5QI 1 QoS Flow) Duration |
|  | Distribution of Abnormally Released Call (5QI 1 QoS Flow) Duration |

2. At NRCellDU, some performance measurements defined in TS 28.522 need to add PLMN granularity.

The PLMN granularity need to be added in the following performance measurements:

| Category | Measurement Name |
| --- | --- |
| Number of Active Ues | Max number of Active UEs in the UL per cell |
| Paging Measurement | Number of paging records received by the NRCellDU |

6	Conclusion and Recommendation

6.X	Issue #X

6.Y	Issue #Y

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222297 S5-222308 S5-222343 S5-222663 |  |  |  | 1. TR 28.835-0.0.0 initial skeleton 2. pCR Add scope to TR 28.835 3. pCR Add key issue management requirement for different POP’s network operation 4. pCR Add key issue performance measurements without PLMN ID at NRcellCU | 0.1.0 |
| 2022-05 | SA5#143e | S5-223602 S5-223603 S5-223604 |  |  |  | 1. pCR TR 28.835 Add concepts and overview 2. pCR TR 28.835 Add key issue management requirement between MOP-NM and MOP-SR-DM 3. pCR TR 28.835 Add key issue service-based management architecture for MOCN | 0.2.0 |
| 2022-06 | SA5#144e | S5-224162 S5-224165 S5-224432 |  |  |  | 1.pCR TR 28.835 Add key issue remote access requirement of MOP-SR-DM for each POP 2.pCR TR 28.835 Add potential solution for remote access requirement of MOP-SR-DM for each POP 3.pCR TR 28.835 Add potential solution for management requirement between MOP-NM and MOP-SR-DM | 0.3.0 |
| 2022-08 | SA5#145e | S5-225413 S5-225815 S5-225816 S5-225817 |  |  |  | - pCR TR 28.835 Add potential solutions for performance measurements without PLMN ID at NRcellCU - pCR TR 28.835 Add potential solution for PLMN-related attribute requirement for operator specific IOC - pCR TR 28.835 Add key issue PLMN-related attribute requirement for operator specific IOC - pCR TR 28.835 Update potential solution for management requirement between MOP-NM and MOP-SR-DM | 0.4.0 |
| 2022-11 | SA5#146 | S5-226527 S5-226989 S5-226991 |  |  |  | - pCR TR 28.835 Add key issue on PLMN granularity requirement of performance measurements for MOCN - pCR TR 28.835 Add Issue for Operator Specific 5QI - pCR TR 28.835 Add potential solution for issue on PLMN granularity requirement of performance measurements for MOCN | 0.5.0 |